National Competency Standards for

Electric Vehicle (EV) Assistant Technician,

Level-2

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

FOREWORDS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification of Electric Vehicles (EV) Assistant Technician Level 2. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director   (NAVTTC)

Table of Contents

1.	Introduction	4

1.1. Date of Development:	4

1.2. Date of Validation/NQAC::	4

1.3. Entry Requirement:	4

2.	Code of Qualification 0716EVA	4

3.	Summary of Competency Standards	4

4.	Detail of Competency Standards	5

0716MSA191	Perform Basic Computer Operations	5

0716MSA192	Maintain Health, Safety and Environment.	6

0716MSA193	Perform Basic Electrical Measurements	9

0716MSA194	Maintain EV Battery System	11

0716MSA195	Perform EV Battery Charging	13

0716MSA196	Maintain EVs Motor Drive and Brake system	16

0716MSA197	Maintain EV Controller and Power Electronics	18

0716MSA198	Maintain EVs Thermal and Cooling System	20

0716MSA199	Maintain 2-Wheeler EVs	22

0716MSA1910	Maintain 3-Wheeler EVs	25

0716MSA1911	Maintain 4-Wheeler EVs	28

5.	List of Machines, Tools and Equipment	31

6.	List of Consumables	33

7.	Members of Qualification Development Committee	36

# Introduction

An Electric Vehicle (EV) is any vehicle that uses one or more electric motors for propulsion, drawing power from an energy storage system—most commonly a rechargeable battery. Unlike traditional internal combustion engine (ICE) vehicles that burn fossil fuels like petrol or diesel, EVs use electricity as their primary energy source. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Electric Vehicle (EV) under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the Auto Mobile industry of the country.

| ## 1.1. Date of Development: | May 2026 |
| --- | --- |
| ## 1.2. Date of Validation/NQAC: | June 18, 2026 |
| ## 1.3. Entry Requirement: | Middle or Equivalent |

# Code of Qualification 0716EVA

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| Code | Qualification Title |
| --- | --- |
| 0716MSA19 | National Vocational Certificate Level-2 in Electric Vehicle (EV) Assistant Technician |
| 0716MSA20 | National Vocational Certificate Level-3 in Electric Vehicle (EV) Technician. |
| 0716MSA21 | National Vocational Certificate Level-4 in Electric Vehicle (EV) Supervisor. |

# Summary of Competency Standards

| Code | Competency Standards | Category | Level | Th | Pr | Total | Cr.H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Perform Basic Computer Operations | Generic | 2 | 15 | 24 | 39 | 2.7 |
|  | Maintain Health, Safety and Environment | Functional | 2 | 15 | 24 | 39 | 2.7 |
|  | Perform Basic Electrical Measurements | Technical | 2 | 7 | 21 | 28 | 1.8 |
|  | Maintain EV Battery System | Technical | 2 | 7 | 21 | 28 | 1.8 |
|  | Perform EVs Battery Charging | Technical | 2 | 8 | 24 | 32 | 2.0 |
|  | Maintain EVs Motor Drive and Brake system | Technical | 2 | 9 | 24 | 33 | 2.1 |
|  | Maintain EV Controller and Power Electronics | Technical | 2 | 10 | 30 | 40 | 2.5 |
|  | Maintain EVs Thermal and Cooling System | Technical | 2 | 10 | 30 | 40 | 2.5 |
|  | Maintain 2-Wheeler EVs | Technical | 2 | 10 | 30 | 40 | 2.5 |
|  | Maintain 3-Wheeler EVs | Technical | 2 | 10 | 30 | 40 | 2.5 |
|  | Maintain 4-Wheeler EVs | Technical | 2 | 10 | 30 | 40 | 2.5 |
| On Job Training (OJT) |  |  |  | 0 | 180 | 180 | 4.5 |
| Total |  |  |  | 111 | 468 | 579 | 30.0 |

# Detail of Competency Standards

## Perform Basic Computer Operations

Overview: - This competency standard covers the skills and knowledge required to Operate MS word, Operate MS Excel, Operate MS Power Point, Perform Browsing and Print Document.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Operate MS word | P1. Perform Microsoft basic commands in MS word P2. Open File P3. Format a file - Font (Type/size/bold/Italic) - Header Footer - Page number - Insert pics / table/hyperlink P4. Save a File P5. Save a folder |
| CU2. Operate MS Excel | P1. Perform basic commands in Microsoft MS Excel P2. Open a worksheet P3. Sum functions P4. IF functions P5. Basic calculations P6. Table and graphs P7. Save a worksheet/folder |
| CU3. Operate MS Power Point | P1. Prepare Microsoft power point presentation with basic commands P2. Make a power point file P3. Insert pics/table/hyperlink P4. Design a theme for slides P5. Save a power point file |
| CU4. Perform Browsing | P1. Perform browsing on the internet as per needs P2. Perform search online on new trends in the market with the help of internet. P3. Create Email Account P4. Compose Email |
| CU5. Print Document | - Select Printer - Select page setup - Print relevant pages |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Basic parts of computers

- Importance and uses of MS Word

- Importance and uses of MS Excel

- Importance and uses of MS Power Point

- Importance of Internet.

- Use of various search engines like Google, U-tube etc.

- Printing Procedure.

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Operate MS word

- Operate MS Excel

- Operate MS Power Point

Tools & Equipment

- Computer sets

- Multimedia

- Computer Tables

- Chairs

- Printer.

- Internet facility.

## Maintain Health, Safety and Environment.

Overview: This competency standard covers the skills and knowledge required to Follow health and safety protocols, Maintain fire safety measures, Identify HV risks in 2W to 4W EVs, Handle hazardous materials Safely, Follow emergency measures, Maintain PPEs for EV works, Apply waste disposal system and Maintain personal hygiene and discipline

| Competency Units | Performance Criteria |
| --- | --- |
| Follow health and safety protocols | - Identify hazards at workplace during daily inspection - Wear PPE according to safety guidelines - Report incidents to supervisor immediately - Place warning signs near hazardous areas properly - Maintain safety records as per SOP |
| Maintain fire safety measures | - Check fire extinguishers for validity regularly - Place firefighting equipment at designated locations - Inspect emergency exits for accessibility daily - Report faulty safety equipment to supervisor promptly - Conduct basic fire drills under supervision |
| Identify HV risks in 2W to 4W EVs | - Inspect high-voltage components as per safety guidelines. - Detect exposed cables and connectors during visual safety checks. - Assess battery pack condition for leakage, swelling, or damage. - Verify isolation status before starting maintenance activities. - Report identified hazards to supervisor as per SOP |
| Handle hazardous materials Safely | - Store chemicals in labelled containers properly - Dispose waste according to environmental regulations - Handle battery chemicals Safely Using gloves - Clean spills using appropriate materials safely - Report leakage incidents immediately |
| Follow emergency measures | - Identify emergency exits during shift start - Inform emergency services during incidents quickly - Assist evacuation during emergency situations - Follow supervisor instructions during crises - Maintain calm behavior during emergencies |
| Maintain PPEs for EV works | - Dress personal protective equipment before each work assignment. - Clean insulated gloves and face shields with approved materials. - Store protective equipment in designated dry storage areas. - Replace damaged PPEs immediately according to safety standards. - Record PPE inspection details in maintenance logbooks regularly. |
| Apply waste disposal system | - Segregate waste materials into categories according to disposal policy - Collect hazardous waste through labelled containers safely - Store recyclable materials in designated area properly - Transport waste to disposal points following safety measures - Record waste disposal activities in logbook as per SOP |
| Maintain personal hygiene and discipline | - Wear clean uniform during duty hours as per workplace rules - Wear protective equipment properly according to task requirements - Wash hands before and after work with hygiene standards - Follow workplace discipline policies during working hours - Report health issues to supervisor as per SOP |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Safety protocols in EV workplace

- Importance of safety compliance

- Important fire safety measures

- Types of fire extinguishers

- HV risks in 2W and 4W EV systems

- Hazard of electric shock in EV

- Hazardous materials in EV workshop

- Safe handling of hazardous materials

- Emergency response steps

- Emergency shutdown procedure

- PPE for EV work

- Importance of using PPE correctly

- Waste types in EV workshop

- Basic waste disposal system

- Importance of personal hygiene and discipline in workshop

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Follow safety protocols during EV work

- Wear correct PPE before starting tasks

- Identify fire safety equipment correctly

- Operate fire extinguisher in basic simulated condition

- Identify high voltage risks in 2W and 4W EV

- Maintain safe distance from HV components

- Identify hazardous materials in workshop

- Handle hazardous materials using safe method

- Demonstrate correct emergency response steps

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | First aid kit | 2 |
|  | Fire extinguisher (CO2 / dry powder) | 3 |
|  | Digital multimeter | 5 |
|  | Clamp meter | 5 |
|  | Insulated screwdriver set | 5 Sets |
|  | Safety goggles | 5 Packet |
|  | Insulated gloves | 5 Packet |
|  | Spill containment kit | 3 |
|  | Warning tags and lockout tools | Different |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical insulation tape | 5 |
|  | Heat shrink tubing | 5 Packet |
|  | Cable ties | 2 Packet |
|  | Cleaning cloths | 5 Packet |
|  | Contact cleaner spray | 3 |
|  | Hand sanitizer | 3 |
|  | Soap or cleaning detergent | 5 |
|  | Waste disposal bags | 10 |
|  | Absorbent pads for spill control | 5 |

## Perform Basic Electrical Measurements

Overview: This competency standard covers the skills and knowledge required to Operate electrical measuring instruments, Measure electrical Quantities along with its units, Construct simple series and parallel circuit, Detect high & Low voltage components in EV, Detect AC/DC wire cables & Insolation resistance and Diagnose basic wiring faults

| Competency Units | Performance Criteria |
| --- | --- |
| Operate electrical measuring instruments | - Set multimeter for voltage, current, and resistance measurement. - Apply clamp meter for non-contact current testing. - Select insulation tester according to insulation resistance requirements. - Separate oscilloscope features for waveform analysis - Verify instrument condition before performing electrical measurements. |
| Measure electrical Quantities along with its units. | - Measure voltage values with volt meter settings. - Measure current flow (amperes) through correct circuit connection. - Measure resistance values in ohms with resistance mode. - Record power readings in watts according to test results. - Confirm frequency values in hertz during AC supply checks. |
| Construct simple series and parallel circuit | - Assemble series circuit components according to circuit diagram. - Connect parallel circuit branches following electrical design. - Install resistors and loads with proper wiring methods. - Test circuit continuity before applying power supply. - Verify circuit operation under safe working conditions. |
| Detect high & Low voltage components in EV | - Identify high-voltage battery components through EV system layout. - Locate low-voltage accessories with vehicle wiring diagram. - Separate HV cables by color coding standards. - Recognize DC-DC converter during component inspection. - Verify component voltage classification according to manufacturer specifications. |
| Detect AC/DC wire cables & Insolation resistance | - Identify AC and DC cables through insulation markings. - Check cable insulation condition before electrical testing. - Measure insulation resistance with approved insulation tester. - Identify damaged cables through resistance test results. - Record cable specifications according to SOP |
| Diagnose basic wiring faults | - Check wiring connections for looseness or damage. - Detect short circuits with continuity testing methods. - Identify open circuits through resistance measurement. - Trace faulty wiring paths with diagnostic tools. - Report wiring defects according to maintenance procedures. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Basic electrical measuring instruments

- Purpose of multimeter

- Clamp meter use

- Ammeter and voltmeter function

- Ohm law

- Ohmmeter function

- Measurement of voltage and its unit

- Measurement of current and its unit

- Measurement of resistance and its unit

- SI units of electrical quantities

- Construction of simple series circuit

- Construction of simple parallel circuit

- Difference between series and parallel circuits

- High voltage components in EV

- Low voltage components in EV

- AC and DC cables and insulation resistance concept

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify electrical measuring instruments correctly

- Select correct instrument for voltage, current and resistance measurement

- Measure voltage using multimeter safely

- Measure current using clamp meter correctly

- Measure resistance and interpret reading

- Identify correct SI units of electrical quantities

- Construct a simple series circuit

- Construct a simple parallel circuit

- Verify circuit continuity and function

- Identify high voltage components in EV system

- Identify low voltage components in EV system List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | Clamp meter | 5 |
|  | Analog multimeter | 5 |
|  | Test leads and probes | 5 |
|  | Insulated screwdriver set | 5 Sets |
|  | Banana plug connectors | 3 |
|  | Test lamp (12V/24V) | 5 |
|  | Continuity tester | 5 |
|  | Battery tester | 2 |
|  | Insulation resistance tester (Megger) | 2 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical insulation tape | 5 |
|  | Heat shrink tubing | 5 |
|  | Test wires | 5 Meter |
|  | Alligator clips | 1 Box |
|  | Connectors and terminals | 2 Box |
|  | Fuse set | 2 Box |
|  | Contact cleaner spray | 3 |

## Maintain EV Battery System

Overview: This competency standard covers the skills and knowledge required to Adopt Safety Measures Before Handling EV Battery, Identify Basic EV Types (BEV, HEV, PHEV, REEV), Label various types of EV Battery, Identify Battery Pack of 2 to 4-wheeler EVs, Identify Battery Terminals and Connectors, Secure Battery Mounting and Measure Battery Voltage with Multimeter

| Competency Units | Performance Criteria |
| --- | --- |
| Adopt Safety Measures Before Handling EV Battery | - Wear insulated PPE before touching battery components. - Disconnect power source with approved isolation Techniques - Check work area for electrical and fire hazards. - Apply insulated tools during battery handling operations. - Place warning signs around EV maintenance zone. |
| Identify Basic EV Types (BEV, HEV, PHEV, REEV) | - Separate EV types based on powertrain design. - Identify battery-only system in BEV models. - Recognize hybrid engine-battery system in HEV models. - Identify plug-in charging system in PHEV models. - Categorize vehicle type with manufacturer specifications - Identify Major Components of REEV |
| Label various types of EV Battery | - Recognize lithium-ion battery by high energy density. - Identify NiMH battery through hybrid vehicle application. - Separate lead-acid battery by low-cost design. - Compare battery types using voltage and capacity ratings. - Label battery chemistry according to manufacturer markings |
| Identify Battery Pack of 2 to 4-wheeler EVs | - Locate battery pack position in 2-wheeler EVs. - Locate battery pack position in 4-wheeler EVs. - Identify battery pack casing for physical protection. - Recognize battery module arrangement within pack. - Verify pack type using vehicle service manual |
| Identify Battery Terminals and Connectors | - Locate positive terminal using polarity marking. - Locate negative terminal using polarity marking. - Identify high-voltage connectors by color coding. - Check terminal condition for corrosion signs. - Verify connector locking mechanism for secure fitment |
| Secure Battery Mounting | - Check battery brackets for secure attachment. - Tighten mounting bolts according to torque specification. - Check battery alignment within compartment frame. - Verify vibration pads for proper installation. - Confirm battery pack stability. |
| Measure Battery Voltage with Multimeter | - Set multimeter to correct DC voltage range. - Connect test probes to battery terminals safely. - Measure battery voltage under no-load condition. - Record voltage reading according to specification. - Compare measured voltage with manufacturer standard. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- EV battery safety practices

- Precautions before handling EV battery

- PPE required for battery work

- Basic EV types BEV, HEV, PHEV, REEV

- Difference between BEV, HEV, PHEV, REEV

- Common EV battery types

- Function of Generator in REEV

- Characteristics of lithium-ion battery

- Battery pack structure in 2W EV

- Battery pack structure in 4W EV

- Battery terminals positive and negative

- Battery connectors and types

- Correct battery mounting position

- Measuring battery voltage with multimeter

- Interpretation of basic battery voltage reading

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Follow safety procedures before handling EV battery

- Waer corrects PPE during battery inspection

- Identify BEV, HEV, PHEV and REEV correctly

- Identify EV battery types used in vehicles

- Identify battery pack layout in 2W and 4W EV

- Locate battery terminals safely

- Identify battery connectors correctly

- Check battery mounting condition

- Inspect battery for loose fitting or damage

- Measure battery voltage using multimeter

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | insulation tester (Megger) | 3 |
|  | Insulated screwdriver set | 3 |
|  | Combination pliers | 5 Sets |
|  | Socket wrench set | 5 Sets |
|  | Torque wrench | 5 |
|  | Wire stripper | 5 |
|  | Crimping tool | 5 |
|  | Clamp meter | 5 |
|  | PPE kit (insulated gloves, safety goggles, face shield) | 5 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical insulation tape | 5 |
|  | Heat shrink tubing | 5 |
|  | Cable ties | 2 Pack |
|  | Terminal lugs and connectors | 2 Sets |
|  | Dielectric grease | 3 |
|  | Cleaning spray for terminals | 5 |
|  | Contact cleaner | 5 |
|  | Fuse set (low voltage control circuits) | 1 Box |
|  | Battery terminal protectors | 5 |
|  | Microfiber cleaning cloths | 5 |

## Perform EV Battery Charging

Overview: This competency standard covers the skills and knowledge required to Identify Charging Station Components (Connector, Display, Cable), Identify EV Charging Port, Connect & Disconnect Charging Cable Safely, Identify EV AC Charging and DC Fast Charging, Identify Charging Status Indicators in 2-Wheeler to 4-Wheeler EV, Identify Charging System of 2 to 4-Wheeler EVs, Detect Charging Faults in EVs and Keep Charging Records

| Competency Units | Performance Criteria |
| --- | --- |
| Identify Charging Station Components (Connector, Display, Cable) | - Identify charging connector by plug design type. - Recognize charging display for operational information. - Check charging cable for insulation condition. - Identify emergency stop button on charging station. - Verify charging station components using user manual |
| Identify EV Charging Port | - Locate charging port on 2-wheeler EV body. - Locate charging port on 4-wheeler EV body. - Identify charging port type by connector shape. - Check charging port cover for physical damage. - Verify charging port compatibility with charger type |
| Connect & Disconnect Charging Cable Safely | - Switch off vehicle before connecting charger. - Insert charging connector into port securely. - Confirm charging initiation through system indicator. - Disconnect charging cable after charging completion. - Store charging cable properly after use. |
| Identify EV AC Charging and DC Fast Charging | - Recognize AC charging by onboard charger practice. - Identify DC fast charging by direct battery supply. - Separate charging speed between AC and DC systems. - Verify charging method through charger specification. - Select appropriate charging mode for vehicle type. |
| Identify Charging Status Indicators in 2-Wheeler to 4-Wheeler EV | - Detect charging light color during charging process. - Identify full-charge indicator on dashboard display. - Recognize fault warning through error symbols. - Monitor charging progress with battery percentage display. - Verify charging status through vehicle manual guide. |
| Identify Charging System of 2 to 4-Wheeler EVs | - Identify onboard charger in 2-wheeler EV system. - Identify charging controller in 4-wheeler EV system. - Locate charging inlet in vehicle structure. - Recognize battery management interface during charging. - Verify charging system components using technical manual |
| Detect Charging Faults in EVs | - Check charger connection for loose fitment. - Identify charging interruption through warning alerts. - Check cable condition for physical damage. - Detect charging port faults using visual inspection. - Report charging system errors according to procedure |
| Keep Charging Records | - Record charging date in maintenance logbook. - Note charging duration for battery monitoring. - Document charging type used for each session. - Record battery status before and after charging. - Maintain charging history for service reference. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- EV charging system

- Types of EV charging systems

- Main charging components

- Charging port and connector types

- Basic working Principle of charging process

- AC charging system

- DC fast charging system

- Difference between AC and DC charging

- Safety features in charging system

- Safe connection of charging cable

- Safe disconnection of charging cable

- Charging status indicators

- Interpretation of charging lights and signals

- Charging differences in 2W and 4W EV

- Importance of safe charging practices

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify EV charging system components

- Identify AC and DC charging equipment

- Differentiate AC charging and DC fast charging

- Locate charging port on 2W and 4W EV

- Connect charging cable safely to vehicle

- Disconnect charging cable safely from vehicle

- Follow correct charging sequence steps

- Identify charging status indicators correctly

- Interpret indicator lights during charging

- Verify proper earthing and safety conditions

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | Insulation tester (Megger) | 3 |
|  | Clamp meter | 5 |
|  | Insulated screwdriver set | 5 Sets |
|  | Socket wrench set | 5 Sets |
|  | Combination pliers | 5 Sets |
|  | Wire stripper | 5 |
|  | Crimping tool | 5 |
|  | EV charging cable tester | 3 |
|  | PPE kit (gloves, safety goggles) | 5 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical insulation tape | 5 |
|  | Heat shrink tubing | 5 |
|  | Cable ties | 2 Pack |
|  | Cleaning spray for connectors | 3 |
|  | Contact cleaner | 3 |
|  | Dielectric grease | 2 |
|  | Fuse set (low voltage) | 5 |
|  | Replacement O-rings for connectors | 1 Box |
|  | Microfiber cleaning cloths | 5 |
|  | Terminal connectors and lugs | 5 |

## Maintain EVs Motor Drive and Brake system

Overview: This competency standard covers the skills and knowledge required to Identify Components of Motor Drive System, Detect hub motor in 02 to 03-Wheeler EVs, Identify Traction Motor in 04-Wheeler EVs, Detect Signs of Motor Fault, Identify basic regenerative system and Identify Braking System in 2-Wheeler and 4-Wheeler EVs

| Competency Units | Performance Criteria |
| --- | --- |
| Identify Components of Motor Drive System | - Identify electric motor within drive assembly. - Locate motor controller in drive system. - Identify inverter for power conversion function. - Identify transmission linkage in drivetrain layout. - Verify drive system components using service manual |
| Detect hub motor in 02 to 03-Wheeler EVs | - Identify hub motor in 2-3-wheeler EVs. - Locate hub motor in 2-3-wheeler EVs. - Check motor casing for identification markings - Separate motor types by construction design. - Inspect motor label for technical specifications. |
| Identify Traction Motor in 04-Wheeler EVs | - Identify traction motor in 4-wheeler EVs - Locate traction motor in 4-wheeler EVs - Check motor casing for identification markings - Inspect motor label for technical specifications. - Follow basic safety while identifying motors |
| Detect Signs of Motor Fault | - Identify abnormal sound from motor - Detect loss of vehicle performance - Detect overheating of motor - Identify unusual electrical signs - Take initial action on motor fault signs |
| Identify basic regenerative system | - Identify regenerative braking components - Interpret regenerative braking function - Detect energy recovery process - Compare regenerative and conventional braking - Check regenerative system operation. |
| Identify Braking System in 2-Wheeler and 4-Wheeler EVs | - Identify disc brake system in EV models. - Identify drum brake system in applicable vehicles. - Identify regenerative braking system function. - Locate brake pedal or lever mechanism. - Verify braking system type with vehicle manual. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Motor drive system in EV

- Main components of motor drive system

- Types of EV motors used in 2W and 4W

- Difference between hub motor and traction motor

- Mounting location of hub motor in 2W

- Placement of traction motor in 4W

- Basic working principle of electric motor

- Common signs of motor fault

- Causes of overheating in motor

- Basic principle of regenerative braking

- Braking systems in 2W and 4W EV

- Common brake faults and safety issues

- Hydraulic and electronic brake systems

- Basic maintenance of brake systems

- Torque and power delivery in EV

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

  - Identify motor drive system components in EV

  - Identify motor type used in 2W and 4W EV

  - Differentiate hub motor and traction motor

  - Locate motor position in vehicle

  - Clarify basic working of EV motor

  - Identify visible signs of motor fault

  - Recognize basic regenerative braking operation

  - Identify braking system in 2W and 4W EV

  - Identify common brake faults

  - Perform basic inspection of brake system

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | Clamp meter | 5 |
|  | Insulated screwdriver set | 5 Sets |
|  | Socket wrench set | 5 Sets |
|  | Allen key set | 5 Sets |
|  | Insulated Combination pliers | 5 Sets |
|  | Wire stripper | 5 Sets |
|  | Crimping tool | 5 |
|  | Brake bleeding kit | 2 |
|  | Feeler gauge | 5 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Brake fluid (DOT 3 / DOT 4 as per vehicle) | 5 |
|  | Cleaning spray for brake parts | 3 |
|  | Electrical insulation tape | 5 |
|  | Heat shrink tubing | 5 |
|  | Cable ties | 2 Pack |
|  | Copper grease | 3 |
|  | Grease for bearings | 2 |
|  | Brake cleaner spray | 5 |
|  | Hydraulic seals and O-rings | 15 |
|  | Shop rags / wiping cloths | 5 |

## Maintain EV Controller and Power Electronics

Overview: This competency standard covers the skills and knowledge required to Measure Power Electronics Fundamentals, Apply Motor Controlling Techniques, Implement EV Controller Policy, Identify Power Electronic Components, Identify EV Controller of 2-4-wheeler and Interpret accelerator signals in EV system

| Competency Units | Performance Criteria |
| --- | --- |
| Measure Power Electronics Fundamentals | - Identify basic components - Clarify working of components - Perform basic power conversion - Identify losses and heating - Recognize switching operation |
| Apply Motor Controlling Techniques | - Recognize motor control - Identify motor types - Recognize basic motor operation - Recognize commutation - Control motor direction - Identify controller role |
| CU3 Implement EV Controller Policy | - Identify system inputs and outputs - Clarify controller working - Integrate sensors correctly - Implement protection logic - Recognize communication system |
| CU4 Identify Power Electronic Components | - Correct identification of components - Recognize basic function - Identify power devices - Recognize circuit roles - Apply Safe handling Techniques |
| CU5 Identify EV Controller of 2-4-wheeler | - Measure input voltage with calibrated testing equipment. - Verify output signals according to system specifications. - Check continuity of controller circuits before energizing. - Assess sensor connections for correct electrical response. - Compare test readings with manufacturer reference values |
| CU6 Interpret accelerator signals in EV system | - Identify accelerator pedal sensor in EV system as input device - Recognize accelerator position signal sent to controller as voltage or digital signal - Identify how throttle signal controls motor speed through controller - Check basic wiring and connector of accelerator signal circuit - Identify safety function that limits power if accelerator signal is abnormal |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the identify basic EV controller components including ECU, inverter, sensors, connectors, fuse, and relay

- Basic Working of power electronics

- Main power electronic components

- Basic Principle of Controller

- Concept of motor controller

- Type of EV controller

- PWM control

- EV powertrain structure

- Energy flow in EV

- Regenerative braking

- Roll of EV controller

- Sensor inputs in EV

- Accelerator signal

- Communication system

- Protection systems

- EV controller integration system

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify EV controller components on real unit or diagram

- Identify 2-wheeler and 4-wheeler EV controller types

- Identify basic power electronic components in controller circuit

- Clarify basic function of inverter and motor controller

- Identify accelerator pedal signal and its role in motor control

- Clarify main connectors, wiring harness, and terminals in controller

- Identify safety components like fuse, relay, and protection devices

- Identify heat sink and cooling elements used in EV controller

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | Insulated screwdriver set | 5 Sets |
|  | Insulated Combination pliers | 5 Sets |
|  | Wire stripper | 5 |
|  | Crimping tool | 5 |
|  | Insulated Socket wrench set | 5 Sets |
|  | Allen key set | 5 Sets |
|  | Clamp meter | 5 |
|  | Test light | 3 |
|  | PPP Kits | 5 Sets |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical insulation tape | 5 |
|  | Heat shrink tubing | 5 |
|  | Cable ties | 2 Pack |
|  | Electrical wires | 5 Meters |
|  | Wire connectors | 2 Box |
|  | Crimp terminals | 5 |
|  | Solder wire | 2 |
|  | Thermal paste | 3 |

## Maintain EVs Thermal and Cooling System

Overview: This competency standard covers the skills and knowledge required to Manage Thermal Systems in EV, Identify EV cooling system and Types, Identify components of cooling system, Identify liquid cooling system, Identify Air Cooling System of HVAC and Identify EV overheating signs

| Competency Units | Performance Criteria |
| --- | --- |
| Manage Thermal Systems in EV | - Identify Heat Source - Monitor Temperature and Sensors - Select Cooling Method - Implement Thermal Protection - Check Thermal Performance - Identify cause of overheating |
| Identify EV cooling system and Types | - Identify EV cooling system - Identify types of cooling systems in EV - Identify cooling of battery pack - Inspect cooling of motor and inverter - Follow basic safety during inspection |
| Identify components of cooling system | - Identify radiator in cooling system - Identify cooling fan - Identify coolant pump - Identify coolant reservoir - Identify hoses and pipes |
| CU4 Identify liquid cooling system | - Identify liquid cooling system - Identify coolant in system - Identify main parts of system - Identify coolant flow path |
| CU5 Identify Air Cooling System of HVAC | - Identify air cooling in HVAC system - Identify airflow path in air cooling - Identify blower fan in air cooling - Identify temperature control in air cooling - Check basic faults in air cooling system |
| CU6 Identify EV overheating signs | - Check temperature warning light on dashboard. - Check unusual noisy component of EV cooling system. - Notice reduced vehicle performance or power drop. - Check unusual smell /smoke in EV component. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Basic concept of EV cooling system.

- Importance of cooling system in EV.

- Main types of EV cooling systems.

- Basic role of temperature control in EV.

- EV cooling system components are identified.

- Important of radiator in cooling system.

- Important of coolant and its circulation.

- Function of HVAC system

- Role of cooling fan in temperature control.

- Basic concept of air-cooling system in HVAC.

- Air cooling components and airflow process.

- Basic concept of liquid cooling system.

- Basic working principle of liquid cooling.

- Importance of cooling system for battery, controller and motor.

- Common overheating signs in EV.

- Basic safety response to overheating condition

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify EV cooling system type on a vehicle correctly.

- Identify main cooling components such as radiator, fan, and coolant lines.

- Demonstrate basic inspection of cooling system for visible faults.

- Check coolant level and condition using safe method.

- Identify air cooling system parts in HVAC system correctly.

- Identify liquid cooling system components and flow path.

- Detect basic EV overheating signs during practical observation.

- Follow safety procedures while working near cooling and high temperature components.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | Basic hand tool kit | 5 Tools Kit |
|  | Insulated tools set | 5 Tools Kit |
|  | Coolant tester or hydrometer | 3 |
|  | Funnel | 3 |
|  | Coolant drain pan | 3 |
|  | Inspection lamp | 3 |
|  | Air blower or cleaning brush | 5 |
|  | Thermometer or infrared temperature gun | 5 |
|  | Torque wrench | 5 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Coolant | 25 Liters |
|  | Distilled water | 10 Liters |
|  | Electrical insulation tape | 5 |
|  | Cable ties | 2 Pack |
|  | Cleaning cloth or rags | 5 |
|  | Sealant or gasket paste | 5 |
|  | Contact cleaner spray | 3 |
|  | Rust remover spray | 3 |

## Maintain 2-Wheeler EVs

Overview: This competency standard covers the skills and knowledge required to Check Battery Pack and Charging System, Maintain Electric Motor and Drivetrain System, Service Thermal Management and Cooling System, Inspect Braking, Suspension, and Tire System and Perform Electrical Safety and Software Maintenance

| Competency Units | Performance Criteria |
| --- | --- |
| Check Battery Pack and Charging System | - Check battery pack voltage with approved diagnostic equipment before maintenance. - Check charging inlet for damage, contamination, or loose fittings carefully. - Check battery terminals for corrosion and secure electrical connections thoroughly. - Test onboard charger under specified charging conditions accurately. - Monitor battery temperature during charging and operation cycles regularly. |
| Maintain Electric Motor and Drivetrain System | - Check traction motor for abnormal noise, vibration, or overheating routinely. - Check reduction gearbox for lubrication level and mechanical wear carefully. - Check drive shafts for alignment and physical damage thoroughly. - Test accelerator system under controlled operating conditions safely. - Verify motor controller with diagnostic software for optimal power delivery |
| Service Thermal Management and Cooling System | - Check coolant level within specified range for thermal system efficiency. - Check coolant hoses for leaks, cracks, or wear carefully. - Check radiator assembly for blockage or physical damage thoroughly. - Test cooling pump under operational conditions for proper circulation. - Verify temperature sensors with diagnostic tools for accurate readings |
| Inspect Braking, Suspension, and Tire System | - Check brake pads for wear and replacement requirements periodically. - Check regenerative braking for effective energy recovery performance accurately. - Check suspension components for leaks or mechanical damage carefully. - Check tire pressure according to manufacturer load specifications regularly. - Adjust wheel alignment with service equipment precisely. |
| Perform Electrical Safety and Software Maintenance | - Check high-voltage cables for insulation integrity and secure routing thoroughly. - Test safety interlocks for proper high-voltage isolation reliably. - Update vehicle software with authorized service platforms correctly. - Verify dashboard indicators for accurate system status communication properly. - Document maintenance records according to workshop and manufacturer standards. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Basic concept of 2W EV electrical and battery system.

- Main battery components such as cells, modules, and BMS.

- Function of battery in vehicle operation.

- Basic concept of hub motor and drive system.

- Role of controller in motor operation.

- Power flow from battery to wheel.

- Basic concept of braking system.

- Main brake and suspension components.

- Importance of brake safety and suspension function.

- Basic structure of tire and wheel assembly.

- Tools used for tire removal and fitting.

- Safe method of tire disassembly and assembly.

- Basic concept of instrument cluster.

- Common warning lights and fault indicators.

- Importance of fault signals for safety and action.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify 2W EV battery system components and electrical parts correctly.

- Demonstrate safe handling of electrical and battery system during inspection.

- Identify hub motor, drive system, and controller on a 2W EV correctly.

- Interpret basic power flow from battery to wheel during practical work.

- Perform basic inspection of motor, drive system, and connections.

- Identify brake system and suspension components correctly on a 2W EV.

- Safely disassemble and assemble EV tire using correct tools and method.

- Recognize dashboard cluster warning lights and respond correctly to faults.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | Basic hand tool kit | 5 Tools Kit |
|  | Insulated tool set | 5 Tools Kit |
|  | Hub motor lifting stand or wheel stand | 5 |
|  | Tire lever set | 5 |
|  | Tire pressure gauge | 5 |
|  | Spanner set for fasteners | 5 Sets |
|  | OBD or diagnostic scanner | 3 |
|  | Torque wrench | 5 |
|  | Pliers for gripping and cutting wires | 5 Sets |
|  | Jack stand or lift for scooter support | 2 |
|  | Inspection lamp | 2 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical insulation tape | 5 |
|  | Cable ties | 2 Pack |
|  | Fuses | 2 Box |
|  | Connectors and terminals | 1 Box |
|  | Heat shrink sleeves | 3 |
|  | Contact cleaner spray | 3 |
|  | Grease or lubricant | 3 |
|  | Brake pads | 5 Set |
|  | Valve caps | 5 |
|  | Tire patch kit | 5 |
|  | Lubricating oil for motor and brake parts | 24 Liters |

## Maintain 3-Wheeler EVs

Overview: This competency standard covers the skills and knowledge required to Check Battery and Charging System of 3-Wheeler, Maintain Motor and Powertrain System, Maintain Brake and Suspension System of 3-Wheeler, Check Body, Tires, and Chassis Components of 3-Wheeler and Interpret faults signs on cluster of 3-Wheeler

| Competency Units | Performance Criteria |
| --- | --- |
| Check Battery and Charging System of 3-Wheeler | - Check battery voltage with calibrated diagnostic equipment before operation. - Check charging socket for cleanliness, damage, or loose connections carefully. - Check battery terminals for corrosion and secure attachment thoroughly. - Test charger performance under standard charging conditions accurately. - Monitor battery temperature during charging cycles for safety compliance |
| Maintain Motor and Powertrain System | - Check traction motor for overheating, noise, or vibration regularly. - Check drive axle for alignment and wear during maintenance procedures. - Lubricate transmission components according to manufacturer specifications properly. - Test accelerator response under controlled operating conditions safely. - Verify motor controller using diagnostic software for efficient performance |
| Maintain Brake and Suspension System of 3-Wheeler | - Check brake linings for wear and serviceability periodically. - Check brake fluid within specified levels for hydraulic systems. - Test braking efficiency under load conditions for operational safety. - Check suspension assemblies for leaks, wear, or mechanical damage carefully. - Adjust steering linkage according to service manual standards precisely |
| Check Body, Tires, and Chassis Components of 3-Wheeler | - Check tire pressure according to recommended load specifications regularly. - Check tire condition for tread wear and sidewall damage thoroughly. - Check chassis frame for cracks, rust, or structural weakness carefully. - Tighten wheel fasteners with approved torque settings accurately. - Clean vehicle body to maintain operational appearance and prevent corrosion |
| Interpret faults signs on cluster of 3-Wheeler | - Identify common warning lights on 3-wheeler cluster. - Recognize color signals on cluster. - Interpret basic symbols and indicators. - Detect abnormal readings on gauges. - Document take initial action based on fault signs. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Basic concept of 3W EV battery and charging system.

- Main battery components such as cells, modules, and BMS.

- Basic charging process and AC, DC charging difference.

- Charging ports and connectors used in 3W EV.

- Basic concept of traction motor and its function in vehicle movement.

- Power flow from battery to wheels through powertrain system.

- Main parts of powertrain such as motor, controller, and related system.

- Basic maintenance needs of motor and powertrain system.

- Basic concept of brake system and its working.

- Main components of braking and suspension systems.

- Importance of brake fluid, shocks, and suspension parts.

- Basic concept of vehicle body, chassis, and structural parts.

- Importance of tire condition, wears, and corrects pressure.

- Basic concept of dashboard cluster and warning lights.

- Meaning of common fault signs and need for action.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify 3W EV battery pack, charging port, and charging system components correctly.

- Demonstrate safe handling and basic observation of charging process.

- Identify traction motor and main powertrain components on a 3W EV.

- Clarify basic power flow from battery to wheels during practical work.

- Perform basic inspection of motor and powertrain for visible faults.

- Identify brake system and suspension components correctly on a vehicle.

- Inspect tire condition, wear, and pressure using correct method.

- Recognize dashboard warning lights and basic fault signs correctly.

- Follow safety procedures during all inspection and maintenance tasks.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | Basic hand tool kit | 5 Tool Kits |
|  | Screwdriver set | 5 Sets |
|  | Pliers for gripping and cutting wires | 5 |
|  | Insulated tools set | 5 Tools Kit |
|  | OBD scanner | 3 |
|  | Tire pressure gauge | 3 |
|  | Air compressor | 1 |
|  | Hydraulic jack | 2 |
|  | Jack stands | 4 |
|  | Torque wrench | 5 |
|  | Inspection lamp or work light | 3 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical insulation tape | 5 |
|  | Cable ties | 2 Pack |
|  | Connectors and terminals | 5 |
|  | Contact cleaner spray | 3 |
|  | Cleaning cloth or waste rags | 3 |
|  | Grease or lubricant | 3 |
|  | Brake fluid | 5 liters |
|  | Lubricating oil for motor and brake parts | 24 Liters |
|  | Brake pads for wear replacement | 2 |
|  | Coolant | 25 Liters |
|  | Tire soap or cleaning solution | 5 |

## Maintain 4-Wheeler EVs

Overview: This competency standard covers the skills and knowledge required to Maintain Battery Pack and Charging System of 4-Wheeler, Maintain Electric Motor and Drivetrain System of 4-Wheeler, Identify HVAC of 4-Wheeler, Inspect Suspension, steering, Brake, and Tire of 4-Wheeler and Interpret fault codes on cluster of 4-Wheeler

| Competency Units | Performance Criteria |
| --- | --- |
| Maintain Battery Pack and Charging System of 4-Wheeler | - Check battery pack voltage with approved diagnostic equipment before maintenance. - Check charging inlet for damage, contamination, or loose fittings carefully. - Check battery terminals for corrosion and secure electrical connections thoroughly. - Test onboard charger under specified charging conditions accurately. - Monitor battery temperature during charging and operation cycles regularly. |
| Maintain Electric Motor and Drivetrain System of 4-Wheeler | - Check traction motor for abnormal noise, vibration, or overheating routinely. - Check reduction gearbox for lubrication level and mechanical wear carefully. - check drive shafts for alignment and physical damage thoroughly. - Test accelerator system under controlled operating conditions safely. - Verify motor controller with diagnostic software for optimal power delivery. |
| Identify HVAC of 4-Wheeler | - Identify the main parts of a 4-wheeler HVAC system. - Clarify basic working of HVAC in a vehicle. - Locate HVAC components in a 4-wheeler. - Identify control functions of HVAC system. - Follow basic safety while working near HVAC system. |
| Inspect Suspension, steering, Brake, and Tire of 4-Wheeler | - Check brake pads for wear and replacement requirements periodically. - Check regenerative braking for effective energy recovery performance accurately. - Check suspension components for leaks or mechanical damage carefully. - Check tire pressure according to manufacturer load specifications regularly. - Adjust wheel alignment with service equipment precisely |
| Interpret fault codes on cluster of 4-Wheeler | - Identify warning lights on the vehicle cluster. - Interpret basic fault symbols and indicators. - Locate fault messages on digital instrument cluster. - Apply basic scan tool to read fault codes. - Identify initial action based on fault indication. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Basic concept and function of EV battery pack.

- Main components of battery system, cells, modules, and BMS.

- Basic difference between AC and DC EV charging.

- Charging ports and charging cables used in 4W EVs.

- Role of electric motor in vehicle propulsion.

- Basic power flow from battery to wheels.

- Main parts of drivetrain system such as motor, controller, and gearbox.

- Basic maintenance requirements of motor and drivetrain system.

- Roll of HVAC system in vehicle.

- Main HVAC components and their basic functions.

- Impact of HVAC on vehicle comfort and battery.

- Key components of suspension, steering, and braking systems.

- Importance of correct tire condition and pressure.

- Basic dashboard warning lights and symbols.

- Basic concept of fault codes and required action.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify battery pack and charging system components on a 4W EV.

- Safely demonstrate basic charging procedure using proper steps.

- Identify electric motor and drivetrain parts on the vehicle.

- Perform basic inspection of motor and drivetrain for visible faults.

- Identify HVAC components and explain their basic function.

- Inspect suspension, steering, brakes, and tires using a checklist.

- Identify common faults such as worn tires, loose parts, or leaks.

- Read and recognize dashboard warning lights and fault codes.

- Follow safety rules while working on EV systems and tools.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | Digital multimeter | 5 |
|  | Basic hand tool kit | 5 Sets |
|  | Insulated tools | 5 Sets |
|  | Tire pressure gauge | 3 |
|  | Insulated Screw Driver Set | 5 |
|  | Hydraulic jack | 2 |
|  | Jack stands | 4 |
|  | OBD scanner | 3 |
|  | Battery charger | 1 |
|  | Work light or inspection lamp | 3 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Cleaning cloth or waste rags | 5 |
|  | Electrical insulation tape | 5 |
|  | Cable ties | 2 Pack |
|  | Contact cleaner spray | 3 |
|  | Grease or lubricant | 3 |
|  | Brake fluid | 5 liters |
|  | Coolant | 25 Liters |
|  | Brake Pad set (Front and Rear) | 2 Sets Each |
|  | Engine Oil and Oil Filter | 24 Liters,6 |
|  | Soap solution or tire polish | 5 |

# List of Machines, Tools and Equipment

List of Machines, Tools and Equipment for the entire qualification of 25 Trainees.

Electric Vehicle (EV) Specifications

- 2-Wheeler EV	1 No.

- 3-Wheeler EV	1 No.

- 4-Wheeler EV	1 No.

| Specification | 2-Wheeler EV | 3-Wheeler EV | 4-Wheeler EV |
| --- | --- | --- | --- |
| Vehicle Type | Electric Motorcycle/ Scooter | Electric Rickshaw/ Cargo Loader | Electric Car/SUV/Bus |
| Number of Wheels | 2 | 3 | 4 |
| Seating Capacity | 1–2 | 3–6 | 4–7 (cars) |
| Motor Type | BLDC Hub or Mid-drive | BLDC/PMSM | PMSM or Induction Motor |
| Motor Power | 0.5–8 kW | 1–12 kW | 30–250+ kW |
| Maximum Speed | 25–120 km/h | 25–60 km/h | 100–220 km/h |
| Driving Range | 50–180 km | 70–180 km | 200–700 km |
| Charging Time | 3–6 hours | 4–8 hours | 30 minutes (DC Fast) to 12 hours (AC) |
| Transmission | Single Speed | Single Speed | Single Speed Reduction Gear |
| Regenerative Braking | Optional | Optional | Standard |
| Cooling System | Air cooled | Air/Liquid cooled | Liquid cooled |

| Name of Trade |  | 0716MSA19 Electric Vehicle (EV) Assistant Technician Level-2 |  |  |
| --- | --- | --- | --- | --- |
| Duration of the Course |  |  | 6 Months |  |
| S. No | Tools and Equipment |  |  | Quantity |
| 1 | Screwdriver set |  |  | 5Sets |
| 2 | Flat screwdriver |  |  | 5 Sets |
| 3 | Phillips screwdriver |  |  | 5 Sets |
| 4 | Insulated screwdriver |  |  | 5 Sets |
| 5 | Combination plier |  |  | 5 Sets |
| 6 | Wire stripper |  |  | 5 Packets |
| 7 | Long nose plier |  |  | 5 Pcs |
| 8 | Side cutter |  |  | 5 Pcs |
| 9 | Crimping tool |  |  | 5 Pcs |
| 10 | Adjustable spanner |  |  | 5 pcs |
| 11 | Ring spanner sets |  |  | 5Sets |
| 12 | Open end spanner |  |  | 5 Sets |
| 13 | Socket set |  |  | 5 Sets |
| 14 | Ratchet handle |  |  | 5 pcs |
| 15 | Allen key set |  |  | 5 Sets |
| 16 | Torque wrench |  |  | 5 Pcs |
| 17 | Hammer |  |  | 3 Pcs |
| 18 | Rubber mallet |  |  | 5 Pcs |
| 19 | Punch set |  |  | 5 Sets |
| 20 | Chisel |  |  | 5 Sets |
| 21 | Bench vice |  |  | 5 Sets |
| 22 | Hand drill |  |  | 5 Pcs |
| 23 | Drill bits |  |  | 5 Kits |
| 24 | Hacksaw |  |  | 5 Pcs |
| 25 | File set |  |  | 5 Sets |
| 26 | Measuring tape |  |  | 5 Pcs |
| 27 | Brake puller (Disc and Durm) |  |  | 2 Pcs |
| 28 | Steel ruler |  |  | 5 Pcs |
| 29 | Vernier caliper |  |  | 5 Pcs |
| 30 | Multimeter |  |  | 5 Sets |
| 31 | Clamp meter |  |  | 5 Pcs |
| 32 | Test lamp |  |  | 5 Pcs |
| 33 | Continuity tester |  |  | 5 Pcs |
| 34 | Insulation tester |  |  | 10 Pcs |
| 35 | Battery tester |  |  | 5 Sets |
| 36 | Oscilloscope basic |  |  | 2 Pcs |
| 37 | Temperature gun |  |  | 5 Pcs |
| 38 | Test light |  |  | 5 Pcs |
| 39 | Fuse puller |  |  | 5 Pcs |
| 40 | Trim removal tool Kits |  |  | 5 Pcs |
| 41 | Inspection mirror |  |  | 5 Pcs |
| 42 | Magnetic pickup tool |  |  | 5 Pcs |
| 43 | Flashlight, rechargeable |  |  | 5 Pcs |
| 44 | Creeper seat |  |  | 5 Pcs |
| 45 | Parts tray, magnetic |  |  | 15 Pcs |
| 46 | DC power supply |  |  | 5 Pcs |
| 47 | Insulation resistance tester |  |  | 5 Pcs |
| 48 | Torque wrench Big Size |  |  | 5 Pcs |
| 49 | Clamp meter (DC) |  |  | 5 Pcs |
| 50 | Test leads replacement |  |  | 2 Kits |
| 51 | Extension lead |  |  | 5 Pcs |
| 52 | Cable reel |  |  | 5 Pcs |
| 53 | Soldering iron |  |  | 5 Pcs |
| 54 | DE soldering pump |  |  | As per Uses |
| 55 | Heat gun |  |  | 5 Pcs |
| 56 | Utility knife |  |  | 5 Pcs |
| 57 | Electrical tape |  |  | 3 Role |
| 58 | Charging cable |  |  | 2 Sets |
| 59 | Battery charger |  |  | 2 Pcs |
| 60 | Portable EV charger |  |  | 2 pcs |
| 61 | EV training board |  |  | 2 Pcs |
| 62 | Motor controller trainer |  |  | 2 |
| 63 | BLDC motor model |  |  | 2 Station |
| 64 | Hub motor model |  |  | 2 Station |
| 65 | EV battery pack model |  |  | 2 |
| 66 | Lithium-ion cell sample |  |  | 5 Sets |
| 67 | Battery management system trainer |  |  | 2 |
| 68 | Accelerator throttle |  |  | 2 |
| 69 | Brake switch sensor |  |  | 5 Pcs |
| 70 | Temperature sensor |  |  | 5 Pcs |
| 71 | DC motor |  |  | 2 Pcs |
| 72 | AC motor model |  |  | 2 Pcs |
| 73 | Inverter trainer |  |  | 2 |
| 74 | Converter trainer |  |  | 2 |
| 75 | Wiring harness |  |  | 2 Sets |
| 76 | Connector set |  |  | 5 |
| 77 | Electrical panel board |  |  | 5 |
| 78 | Junction box |  |  | 2 Station |
| 79 | Circuit breaker |  |  | 10 Pcs |
| 80 | Insulated gloves |  |  | 15 Packets |
| 81 | Safety shoes |  |  | 25 Packets |
| 82 | Fire extinguisher |  |  | 3 Pcs |
| 83 | Lockout tag |  |  | 2 Sets |
| 84 | Stethoscope |  |  | 5 Pcs |
| 85 | Cable cutter |  |  | 5 Pcs |
| 86 | Fire suppression system (lithium battery specific) |  |  | 2 |
| 87 | Explosion proof storage cabinet (damaged battery) |  |  | 1 |
| 88 | Air blower |  |  | 2 |
| 89 | Laptop for diagnostics |  |  | 2 System |
| 90 | Diagnostic scanner basic |  |  | 2 System |
| 91 | HV voltage detector |  |  | 5 Pcs |
| 92 | Ventilation system |  |  | 2 |
| 93 | AVC Gas Recycling Machine |  |  | 1 |
| 94 | Soldering Iron Kit |  |  | 3 Kits |
| 95 | Bearing puller |  |  | 3 Sets |
| 96 | Battery Pack Analyzer |  |  | 5 Pcs |
| 97 | Hydraulic jack |  |  | 5 Pcs |
| 98 | Jack stand |  |  | 20 Pcs |
| 99 | Wheel spanner |  |  | 5 Pcs |
| 100 | Air compressor |  |  | 1 Pcs |

# List of Consumables

List of consumables for the entire qualification of 25 Trainees

Battery Specifications

- 2-Wheeler EV	1 No.

- 3-Wheeler EV	1 No.

- 4-Wheeler EV	1 No.

| Parameter | Typical Specification |  |  |
| --- | --- | --- | --- |
|  | 2-Wheeler EV | 3-Wheeler EV | 4-Wheeler EV |
| Battery Chemistry | Lithium-ion (LFP/NMC) | Lithium-ion or LFP | Lithium-ion (NMC/LFP) |
| Battery Voltage | 48V, 60V, 72V | 48V, 60V, 72V, 96V | 200V–800V |
| Battery Capacity | 20–80 Ah | 100–250 Ah | 100–250 Ah |
| Energy Capacity | 1–5 kWh | 5–15 kWh | 20–120 kWh |
| Battery Type | Mostly Low Voltage | Low Voltage (48–72V) | High Voltage (200–800V |
| BMS | Yes | Yes | Advanced BMS |
| Charger | 48V/60V/72V Smart Charger | AC Charger | DC CCS/NACS (Fast Charging) |
| Cooling | Air cooling | Air cooling | Liquid Cooling |

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Electrical insulation tape | 5 Packs |
| 2 | Cable ties | 2 Packs |
| 3 | Cleaning cloth | 25 Pcs |
| 4 | Marker and labels | 40 Pcs |
| 5 | Disposable gloves | 10 Sets |
| 6 | Sandpaper | 15 Pcs |
| 7 | Wire connectors | 2 Packs |
| 8 | Fuse wire | 50 Pcs |
| 9 | Lubricant spray | 5 Pcs |
| 10 | Battery terminal grease | 2 Pcs |
| 11 | Distilled water | 10 Liters |
| 12 | Notebook and recording sheets | 20 pcs |
| 13 | Masking tape | 20 pcs |
| 14 | Cleaning brush | 5 Pcs |
| 15 | Cotton waste | 10 Pack |
| 16 | PVC tape | 10 Pcs |
| 17 | Safety gloves | 50 Pcs |
| 18 | Face masks | 100 Pieces |
| 19 | Safety glasses wipes | 2 Bulk Packs |
| 20 | Hand cleaner | 3 Bottles |
| 21 | Disposable gloves | 20 Pcs |
| 22 | Permanent markers | 15 Pieces |
| 23 | Whiteboard markers | 1 dozen |
| 24 | Labels and Tag stickers | - Pcs |
| 25 | Recording sheets and Inspection sheets | 50 Pcs |
| 26 | Log book | 10 Pcs |
| 27 | Pens, Pencils, Erasers | As Required |
| 28 | Sharpeners | As Required |
| 29 | Printing paper and Clip boards | As Required |
| 30 | Carbon paper | As Required |
| 31 | Wire brush | 5 Pcs |
| 32 | Lubricant spray | 5 Rolls |
| 33 | Rust remover spray | 5 Cans |
| 34 | Degreaser | 5 Pcs |
| 35 | Glass cleaner | 3 Cans |
| 36 | Distilled water | 10 Liters |
| 37 | Battery water | As required |
| 38 | Coolant liquid | 15 Liters |
| 39 | Grease | 3 Pcs |
| 40 | Contact cleaner spray | 5 Pcs |
| 41 | Condenser cleaning solvent | 15 Liters |
| 42 | Silicone sealant | 2 Bottles |
| 43 | Adhesive glue | 5 Bottles |
| 44 | Epoxy adhesive | 5 Pcs |
| 45 | Thread seal tape | 5 pcs |
| 46 | Wire connectors | 5 Sets |
| 47 | Cable lugs | 5 Pcs |
| 48 | Crimp terminals | 10 Pcs |
| 49 | Ring terminals | 10 Pcs |
| 50 | Spade terminals | 5 Pcs |
| 51 | Heat shrink sleeves | 5 Packets |
| 52 | Ferrules | 5 Pcs |
| 53 | Fuse wire | 5 Role |
| 54 | Glass fuses | 15 Pcs |
| 55 | Electrical sleeves | 5 Role |
| 56 | Plastic covers | 5 Role |
| 57 | Storage boxes | 5 Pcs |
| 58 | Water bottle | 5 Sets |
| 59 | Mop cloth | 5 Sets |
| 60 | Floor cleaner | 5 Pcs |
| 61 | Dustpan bags | 5 Pcs |
| 62 | Waste bags | 10 Packets |
| 63 | Measuring tape stickers | 25 Pcs |
| 64 | Warning labels | 25 Pcs |
| 65 | Danger tags | 15 Pcs |
| 66 | Reflective tape | 15 Pcs |
| 67 | Cable markers | 15 Pcs |
| 68 | Battery labels | 20 Rolls |
| 69 | Microfiber cloth | 5 Packs |
| 70 | O rings | 10 Packs |
| 71 | Rubber seals | 2 Boxes |
| 72 | Foam tape | 15 Role |
| 73 | Electrical sleeves | 5 Sets |
| 74 | Hose clamps | 20 Pcs |
| 75 | Nuts, Bolts, Washers, Screws | 200 Pcs |
| 76 | Self-tapping screws | 2 Box |
| 77 | Rivets | Assorted kit |
| 78 | Battery Storage Containers | 5 Pcs |
| 79 | Straps / Securing Materials | 5 Sets |
| 80 | Hazard Labels (UN Markings) | 25 Pcs |
| 81 | Adhesive-lined Heat Shrink Tubing | 3 Role |
| 82 | Contact Cleaner Spray | 3 cans |
| 83 | Cleaning wipes for tools and PPE | 10 packs |
| 84 | Battery terminal covers and caps | 10 Sets |
| 85 | Warning labels and HV safety stickers | 25 Pcs |
| 86 | Absorbent pads for electrolyte leakage | 5 Packs |
| 87 | Cleaning solvent for workshop equipment | 10 liters |
| 88 | Refrigerant filter drier | 5 Pcs |
| 89 | Insulation tape (HVAC grade) | 50 Rolls |
| 90 | UV leak detection dye | 10 bottles |

# Members of Qualification Development Committee

| Sr.# | Name | Designation | Department |
| --- | --- | --- | --- |
|  | Inayat ur Rehman | DACUM Expert | KPK-TEVTA |
|  | Mohammad Ishaq | Deputy Director | NAVTTC |
|  | Shehzad Ahmed | Director | NAVTTC |
|  | Abdul Hafeez Abbasi | Director Operations | S-TEVTA |
|  | Mohammad Yousif Baloch | Director Academic | S-TEVTA |
|  | Syed Sultan Shah | Field Officer | Ministry of Federal Education and Professional Training |
|  | Rehman Ali | Sr: Executive | HYNDAI/ Nishat Motor Lahore |
|  | Syed Umar Azhar Bukhari | EV Technician | Yutong EV Buses R. Y. Khan |
|  | Shabaz Haider | Assistant Director | KP-TEVTA |
|  | Mohammad Zulqarnain | EV Technician | BYD Bahawalpur |
|  | Mohammad Zeeshan | Maintenance Supervisor | MG Service Center Faisalabad |
|  | Engr Zohaib Afzal | Head of R& D | Pak zone Lahore |
|  | Engr Saqib Abdullah | GM Plant | Jolta Electric Lahore |
|  | Zahid Razzaq | MM Tech | WESCON, Engg. Multan |
|  | Ijaz Hamid | Ex- Chief Instructor | P-TEVTA |
|  | Dr. Maliha Bakhs | Asst: Professor | UET Lahore |
|  | Abdul Haseeb Imtiaz | Service Engineer | KTS, Green Buses. Bahawalpur |
|  | Mian Abdul Atique | Service Manager | HONDA Center. Lahore |
|  | Muhammad Naveed | CEO | Vision Plus Automotive. Faisalabad |
|  | Muzammal Hussain | Assistant Director | P-TEVTA. R. Yar Khan |
|  | Mohid Ali | Service Advisor | Max Power. Lahore |
|  | Abdul Mannan | Instructor | P-TEVTA. Gujrat |
|  | Aftab Hussain Soomro | Program Coordinator | RDF. Sindh |