National Competency Standards for

Electric Vehicles (EV) Supervisor Level-4

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

FOREWORDS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification of Electric Vehicle (EV) Supervisor Level-4. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director   (NAVTTC)

Table of Contents

1.	Introduction	4

1.1. Date of Development:	4

1.2. Date of Validation/NQAC	4

1.3. Entry Requirement:	4

2.	Code of Qualification	4

3.	Summary of Competency Standards	4

4.	Detail of Competency Standards	5

0716MSA211	Supervise Workshop Safety	5

0716MSA212	Diagnose of HV System	8

0716MSA213	Diagnose and repair Battery system	11

0716MSA214	Maintain EV HVAC system	13

0716MSA215	Maintain EV Inverter & Converter system	16

0716MSA216	Service Electric Motor/Drive Unit	19

0716MSA217	Analysis CAN Bus & Network Communication Practices	22

0716MSA218	Conduct ADAS & Autonomous Sensor Integration	27

0716MSA219	Perform HV Safety Isolation & Lockout-Tagout (LOTO)	34

0716MSA2110	Repair On-Board Charging System (OBC)	38

0716MSA2111	Perform Sensor and actuator Calibration	42

0716MSA2112	Perform HV Component Sealing & IP Rating Validation	49

0716MSA2113	Maintain Regenerative Braking System	55

0716MSA2114	Test Insulation Resistance (Megohmmeter)	60

0716MSA2115	Maintain Technical Documentation & Compliance Reporting	63

0716MSA2116	Perform HV Cabling & Connector Specialized Crimping	69

5.	List of Machines, Tools and Equipment	76

6.	List of Consumables	78

7.	Members of Qualification Development & Review Committee	81

8.	Qualification Review Committee	82

# Introduction

An Electric Vehicle (EV) is any vehicle that uses one or more electric motors for propulsion, drawing power from an energy storage system—most commonly a rechargeable battery. Unlike traditional internal combustion engine (ICE) vehicles that burn fossil fuels like petrol or diesel, EVs use electricity as their primary energy source. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Electric Vehicle (EV) under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the Auto Mobile industry of the country.

| ## 1.1. Date of Development: | March 2026 |
| --- | --- |
| ## 1.2. Date of Validation/NQAC | June 18, 2026 |
| ## 1.3. Entry Requirement: | Electric Vehicle (EV) Technician Level-3 |

# Code of Qualification

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| Code | Qualification Title |
| --- | --- |
| 0716MSA19 | National Vocational Certificate Level-2 in Electric Vehicle (EV) Assistant Technician |
| 0716MSA20 | National Vocational Certificate Level-3 in Electric Vehicle (EV) Technician. |
| 0716MSA21 | National Vocational Certificate Level-4 in Electric Vehicle (EV) Supervisor. |

# Summary of Competency Standards

| Code | Competency Standards | Category | Level | Th | Pr | Total | Cr.H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Supervise Workshop Safety | Generic | 4 | 9 | 30 | 39 | 2.4 |
|  | Diagnose High-Voltage (HV) System | Technical | 4 | 14 | 36 | 50 | 3.2 |
|  | Diagnose and repair Battery system | Technical | 4 | 12 | 36 | 48 | 3.0 |
|  | Maintain (EV) HVAC system | Technical | 4 | 12 | 33 | 45 | 2.9 |
|  | Maintain Inverter & Converter system | Technical | 4 | 12 | 33 | 45 | 2.9 |
|  | Service Electric Motor/Drive Unit | Technical | 4 | 12 | 33 | 45 | 2.9 |
|  | Analysis CAN Bus & Network Communication | Technical | 4 | 14 | 33 | 47 | 3.1 |
|  | Conduct ADAS & Autonomous Sensor Integration | Technical | 4 | 14 | 33 | 47 | 3.1 |
|  | Perform HV Safety Isolation & Lockout-Tagout (LOTO) | Technical | 4 | 12 | 33 | 45 | 2.9 |
|  | Repair On-Board Charging System (OBC) | Technical | 4 | 10 | 33 | 43 | 2.7 |
|  | Perform Sensor and actuator Calibration | Technical | 4 | 10 | 33 | 43 | 2.7 |
|  | Perform HV Component Sealing and IP Rating Validation | Technical | 4 | 10 | 33 | 43 | 2.7 |
|  | Maintain Regenerative Braking System | Technical | 4 | 9 | 33 | 42 | 2.6 |
|  | Test Insulation Resistance (Megohmmeter) | Technical | 4 | 9 | 33 | 42 | 2.6 |
|  | Maintain Technical Documentation & Compliance Reporting | Generic | 4 | 9 | 30 | 39 | 2.4 |
|  | Perform HV Cabling & Connector Specialized Crimping | Technical | 4 | 9 | 30 | 39 | 2.4 |
| On Job Training (OJT) |  |  |  | 0 | 240 | 240 | 6.0 |
| Total |  |  |  | 177 | 765 | 942 | 50.0 |

# Detail of Competency Standards

## Supervise Workshop Safety

Overview: This competency standard covers the skills and knowledge required to Conduct daily safety briefing on High-Voltage (HV), Inspect Technician Personal Protective Equipment (PPE), Supervise Initial High-Voltage De- Energization Process, Manage High-Voltage Battery "Quarantine Area" and Perform Emergency Response Drill for EV fires

| Competency Units | Performance Criteria |
| --- | --- |
| Conduct daily safety briefing on High-Voltage (HV) | - Conduct safety briefing at the start of each shift - Identify High-Voltage hazards to all technicians - Assign safety roles to team members - Communicate emergency procedures to all staff - Record briefing details in logbook |
| Inspect Technician Personal Protective Equipment (PPE) | - Identify required PPE for each task - Inspect PPE of technicians before work - Replace defective PPE immediately - Monitor PPE usage during work - Document PPE audit results |
| Supervise Initial High-Voltage De- Energization Process | - Confirm EV system shutdown status - Implement Lockout Tagout procedure - Confirm absence of voltage - Confirm High-Voltage battery isolation - Record de-energization steps |
| Manage High-Voltage Battery "Quarantine Area" | - Mark quarantine area boundaries - Store damaged High-Voltage batteries safely - Maintain fire safety equipment in area - Monitor safety conditions continuously - Label quarantined batteries clearly |
| Perform Emergency Response Drill for EV fires | - Plan emergency response drill - Train personnel on fire response - Demonstrate fire suppression techniques - Conduct evacuation drill - Evaluate drill performance |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- High-Voltage safety for workshop team

- Inspection of Personal Protective Equipment of multiple technicians

- Supervision of High-Voltage de-energization on an EV system

- Management of High-Voltage battery quarantine area setup

- Implementation of emergency response drill for EV fire scenario

- Lockout Tagout techniques during isolation process

- Confirmation of nonappearance of voltage using testing equipment

- Safe handling of damaged High-Voltage batteries

- Fire safety equipment in EV case of fire

- Maintaining of safety records and documentation for all activities

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Conduct daily High-Voltage (HV) safety briefing and communicate hazards clearly to all technicians

- Identify High-Voltage risks associated with EV workshop activities

- Assign and communicate safety roles and responsibilities to workshop personnel

- Inspect technician Personal Protective Equipment (PPE) and ensure compliance with HV safety requirements

- Identify non-compliant, damaged, or defective PPE and initiate corrective action

- Supervise High-Voltage de-energization process on an EV system safely

- Apply Lockout Tagout (LOTO) procedure during High-Voltage isolation

- Confirm absence of voltage using approved testing instruments before work commencement

- Create and control High-Voltage battery quarantine area for damaged or suspect units

- Confirm safe storage, labeling, and segregation of High-Voltage batteries

- Monitor environmental and safety conditions within the quarantine area continuously

- Implement emergency response drill for EV fire scenario in a controlled environment

- Demonstrate correct use of fire suppression equipment for EV-related fire incidents

- Apply evacuation Techniques effectively during emergency drill situations

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | High-Voltage insulated gloves | 25 pairs |
| 2 | Arc-rated protective clothing | 25 sets |
| 3 | Safety helmet with face shield | 25 sets |
| 4 | Insulated safety boots | 25 pairs |
| 5 | Lockout Tagout (LOTO) kit | 5 sets |
| 6 | HV warning signs | 10 sets |
| 7 | Barricade tape | 10 rolls |
| 8 | Voltage tester (CAT III / CAT IV rated) | 5 units |
| 9 | Digital multimeter for HV systems | 5 units |
| 10 | Insulated hand tools set | 5 sets |
| 11 | HV battery lifting trolley | 2 units |
| 12 | Battery isolation mat | 5 units |
| 13 | Fire extinguisher (Class D – lithium battery) | 4 units |
| 14 | CO₂ fire extinguisher | 4 units |
| 15 | Fire blanket (EV rated) | 5 units |
| 16 | First aid kit (electrical burn response) | 2 units |
| 17 | Gas detector (workshop safety monitoring) | 2 units |
| 18 | Workshop communication system (walkie-talkie/intercom) | 5 units |
| 19 | Safety inspection checklist forms (pads) | 25 pads |
| 20 | Emergency stop switch testing kit | 2 units |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | High-Voltage insulating tape rolls | 20 rolls |
| 2 | Cable ties (assorted sizes) | 5 packs |
| 3 | Cleaning wipes for tools and PPE | 10 packs |
| 4 | Disposable nitrile gloves | 5 boxes |
| 5 | Battery terminal covers and caps | 25 sets |
| 6 | Warning labels and HV safety stickers | 100 pieces |
| 7 | Absorbent pads for electrolyte leakage | 10 packs |
| 8 | Cleaning solvent for workshop equipment | 10 liters |
| 9 | Permanent marker pens for labeling | 25 pieces |
| 10 | Printed safety briefing sheets and log forms | 200 sheets |

- Diagnose High-Voltage (HV) System

Overview: This competency standard covers the skills and knowledge required to Perform Bidirectional Component Test through Scan Tool, Measure Live Sensor Data for Low Voltage System, Measure Live Sensor Data for High Voltage System, Perform Isolation & Verification of HV System, Perform Insulation Resistance Test on HV Cables and Record CAN-Bus Communication Signals with Oscilloscope

| Competency Units | Performance Criteria |
| --- | --- |
| Perform Bidirectional Component Test through Scan Tool | - Identify vehicle configuration system and software version prior to testing - Start communication between scan tool and vehicle control modules - Select relevant module and execute bidirectional actuation tests as per OEM procedure. - Operate actuators/components and monitor/ record their response. - Compare actual actuator values with standard values of service Manual. - Identify abnormal behavior and record any DTCs (if appearing). - Confirm component functionality or diagnose fault based on test results - Record and document test outcomes. |
| Measure Live Sensor Data for Low Voltage System | - Access and monitor live data parameters for Low Voltage sensors using diagnostic equipment. - Measure and interpret sensor outputs against OEM reference values - Detect signal deviations, fluctuations, or inconsistencies - Compare multiple sensor inputs for accurate system diagnosis - Identify sensor, wiring, and ECU-related faults. |
| Measure Live Sensor Data for High Voltage System | - Confirm HV safety compliance (de-energized vehicle) before accessing system data. - Access and monitor live data parameters for High Voltage components, sensors and control units using diagnostic equipment. - Compare data with OEM thresholds and specifications - Identify and monitor abnormal patterns or values. - Analyze system behavior to determine root cause - Confirm diagnostic conclusion with supporting data |
| Perform Isolation & Verification of HV System | - Identify HV system layout and isolation points - Apply Megaohm meter to test isolation of HV system. - Discharge capacitors following OEM specified procedures - Confirm zero-voltage condition at all required points - Recheck isolation status prior to system handling. - Document isolation and verification process |
| Perform Insulation Resistance Test on HV Cables | - Select appropriate insulation resistance testing equipment like Mega Ohm Meter. - Prepare system ensuring complete HV isolation - Apply specified test voltage according to OEM standards - Measure insulation resistance across cables and components - Compare readings with acceptable limits - Identify insulation degradation or leakage paths - Confirm results through repeat or comparative testing |
| Record CAN-Bus Communication Signals with Oscilloscope | - Identify CAN network structure and access points - Connect oscilloscope probes correctly to CAN High and CAN Low lines - Capture waveform signals under operating conditions - Measure voltage levels and analyze signal patterns - Identify abnormalities such as noise, distortion, or signal loss - Diagnose communication faults (open, short, termination issues) - Confirm findings by cross-checking with system behavior - Document waveform analysis and diagnostic results |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- OEM scan tool operation and bidirectional control strategies

- ECU architecture, control logic, and communication protocols (CAN, LIN)

- Diagnostic Trouble Codes (DTCs) and advanced live data analysis

- Sensor operating principles (LV & HV) and signal behavior characteristics

- High Voltage (HV) system architecture, components, and operation

- Battery Management System (BMS) parameters (SOC, SOH, cell voltage, temperature)

- HV safety standards and PPE usage.

- Electrical measurement principles (voltage, current, resistance, insulation)

- Insulation resistance testing techniques and evaluation criteria

- Oscilloscope operation and waveform interpretation for diagnostics

- CAN-bus communication principles, signaling, and network diagnostics

- Wiring diagram analysis and circuit-level fault isolation

- Failure modes and diagnostics of EV electrical/electronic systems

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform bidirectional tests and accurately verify actuator functionality.

- Analyze LV and HV live data and identify system abnormalities.

- Diagnose faults using scan tool data, DTCs, and system behavior

- Execute HV system isolation and confirm zero-voltage condition

- Apply correct PPE and follow HV safety procedures

- Conduct insulation resistance testing and interpret results accurately

- Capture and analyze CAN-bus signals using oscilloscope

- Identify electrical and communication faults using wiring diagrams

- Complete diagnostic reports with findings and corrective actions

- Demonstrate safe handling of HV systems and diagnostic equipment

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | OEM Diagnostic Scan Tool | 5 |
| 2 | Digital Multimeter (True RMS) | 10 |
| 3 | Insulation Resistance Tester (Megger) | 5 |
| 4 | Oscilloscope (2 Channel) | 3 |
| 5 | HV Insulated Tool Kit | 5 sets |
| 6 | HV Voltage Tester | 5 |
| 7 | Laptop/Diagnostic Workstation | 5 |
| 8 | CAN Breakout Box / Interface | 3 |
| 9 | Test Leads & Probes Kit | 10 sets |
| 10 | HV Training Simulator / Bench | 2 |
| 11 | PPE Kit (HV Rated) | 10 sets |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Electrical Contact Cleaner | 15 cans |
| 2 | Insulation Tape | 25 rolls |
| 3 | Cable Ties | 25 pcs |
| 4 | Marker Pens / Labels | 25 pcs |
| 5 | Disposable Gloves | 5 boxes |
| 6 | Cleaning Cloths | 40 pcs |
| 7 | Probe Tips Replacement | 20 sets |
| 8 | Fuse Assortment (Practice) | 25 pcs |

## Diagnose and repair Battery system

Overview: This competency Standards cover the skill and Knowledge required to Inspect Battery Management System (BMS), Balance Individual Cell Voltages within the Battery Pack, Replace / Repair Internal High-Voltage Contactors, Fuses, and Relays, Inspect Internal Thermal Management Plates for Leaks, Reseal Battery Enclosure with OEM-Approved Silicone and Isolate and Replace Faulty Battery Module

| Competency Units | Performance Criteria |
| --- | --- |
| Inspect Battery Management System (BMS) | - Identify vehicle configuration system and Access BMS data using OEM-approved diagnostic tools - Confirm communication between BMS and vehicle control systems - Monitor key parameters (cell voltage, temperature, SOC, SOH) - Identify abnormal readings or fault codes - Compare data with OEM specifications and thresholds - Analyze BMS behavior for fault identification - Confirm fault through cross-checking with system data - Record inspection results as per OEM standards |
| Balance Individual Cell Voltages within the Battery Pack | - Access cell voltage data via diagnostic system - Identify imbalance among battery cells - Initiate cell balancing process using OEM method - Monitor balancing operation and progress - Confirm uniformity of cell voltages post-balancing - Detect cells not responding to balancing - Confirm system stability after balancing - Document balancing results. |
| Replace / Repair Internal High-Voltage Contactors, Fuses, and Relays | - Identify faulty HV components using diagnostic methods - Confirm complete HV isolation before component handling - Access battery pack and locate defective components - Remove and replace/repair contactors, fuses, or relays - Inspect connections and ensure proper installation - Reassemble battery system as per OEM guidelines - Confirm component functionality after replacement |
| Inspect Internal Thermal Management Plates for Leaks | - Access battery thermal management system components - Inspect cooling plates and channels for leakage - Identify signs of coolant loss or contamination - Check pressure or flow where applicable - Determine source and severity of leakage - Assess impact on battery performance and safety |
| Reseal Battery Enclosure with OEM-Approved Silicone | - Prepare battery enclosure surface for resealing - Remove old sealing material without damaging components - Apply OEM-approved sealant as per specification - Confirm uniform application and proper sealing - Reassemble enclosure with correct torque procedures - Confirm sealing integrity (visual/functional check) - Confirm no contamination inside enclosure - Document resealing process |
| Isolate and Replace Faulty Battery Module | - Identify faulty module using diagnostic data - Perform complete HV system isolation - Access battery pack and locate defective module - Remove faulty module Safely following OEM Techniques - Replace module with correct alignment and connections - Reassemble battery pack and restore system integrity - Perform post-replacement confirmation and system checks - Record replacement and test results |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Battery Management System (BMS) architecture and functions

- Battery parameters (SOC, SOH, cell voltage, temperature)

- Cell balancing principles and methods

- High Voltage (HV) battery pack construction and components

- Functions of HV contactors, fuses, and relays

- HV safety procedures and isolation techniques

- Battery thermal management system and cooling mechanisms

- Leak detection methods and coolant properties

- Battery enclosure design, sealing methods, and IP standards

- OEM-approved sealants and application procedures

- Insulation and electrical safety within battery systems

- Battery module configuration and replacement procedures

- Common battery failure modes and diagnostic techniques

- OEM repair standards and documentation practices

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Retrieval and analysis of BMS data for fault identification

- Identification and correction of cell voltage imbalance

- Safe isolation of HV battery system before repair

- Replacement/repair of HV contactors, fuses, or relays

- Detection and assessment of thermal system leaks

- Proper resealing of battery enclosure using OEM materials

- Safe removal and replacement of faulty battery module

- Verification of system performance after repair

- Compliance with HV safety and OEM procedures

- Accurate documentation of diagnostics, repairs, and results

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | OEM Diagnostic Scan Tool | 5 |
| 2 | Digital Multimeter (True RMS) | 10 |
| 3 | Insulation Resistance Tester | 5 |
| 4 | HV Insulated Tool Kit | 5 sets |
| 5 | Torque Wrench Set | 5 |
| 6 | HV Voltage Tester | 5 |
| 7 | Cooling System Pressure Tester | 3 |
| 8 | Sealant Application Gun | 5 |
| 9 | PPE Kit (HV Rated) | 10 sets |
| 10 | Workstation/Laptop | 5 |

List of Consumable

The consumables required for this competency standard are provided below:

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | OEM Silicone Sealant | 20 tubes |
| 2 | Electrical Cleaner | 15 cans |
| 3 | Lint-Free Cloths | 50 pcs |
| 4 | Insulation Tape | 30 rolls |
| 5 | Cable Ties | 200 pcs |
| 6 | Replacement Fuses | 50 pcs |
| 7 | Replacement Relays | 20 pcs |
| 8 | Gaskets / Sealing Kits | 20 sets |
| 9 | Battery Coolant | 20 liters |
| 10 | Abrasive Pads | 20 pcs |
| 11 | Disposable Gloves | 5 boxes |
| 12 | Cleaning Brushes | 20 pcs |

## Maintain EV HVAC system

Overview: This competency standard covers the skills and knowledge required to Locate HVAC components, Diagnose faults in HVAC system of EV through scanner, Perform Service of HVAC Condenser, Repair/Replace faulty components of compressor, Test climate control sensors and Calibrate electronic expansion valves (EXV)

| Competency Unite | Performance Criteria |
| --- | --- |
| Locate HVAC components | - Identify HVAC components - Confirm component locations - Inspect component condition - Trace system connections - Confirm components identification |
| Diagnose faults in HVAC system of EV through scanner | - Connect diagnostic scanner - Retrieve diagnostic codes - Interpret diagnostic data - Monitor system parameters - Document fault information |
| Perform Service of HVAC Condenser | - Clean condenser surface - Inspect condenser condition - Straighten condenser fins - Check airflow passage - Secure condenser assembly |
| Repair/Replace faulty components of compressor | - Diagnose compressor faults - Inspect compressor assembly - Repair compressor components - Replace faulty components of compressor - Test compressor efficiency |
| Test climate control sensors | - Identify system sensors - Measure sensor signals - Assess sensor readings - Detect sensor faults - Confirm sensor performance |
| Calibrate electronic expansion valves (EXV) | - Inspect EXV operation - Calibrate EXV settings - Monitor Valve Responses - Confirm refrigerant control - Confirm system operation |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Components and functions of EV HVAC system

- Layout and refrigerant flow in system

- Working principle of refrigeration cycle

- Operation of electric compressor

- Function of condenser and evaporator

- Operation of electronic expansion valve (EXV)

- Types and working of HVAC sensors

- HVAC control system and ECU basics

- Diagnostic scanner operation and fault codes

- Basic electrical connections in HVAC system

- Common faults and causes in HVAC system

- Safety precautions in EV HVAC systems

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify HVAC components

- Diagnose HVAC system faults

- Interpret diagnostic fault codes

- Monitor HVAC live data parameters

- Service condenser unit

- Inspect condenser airflow condition

- Diagnose compressor system faults

- Repair compressor components

- Replace compressor components

- Test compressor operation

- Test climate control sensors

- Identify faulty sensors

- Confirm sensor signals

- Calibrate electronic expansion valve (EXV)

- Confirm refrigerant flow control

- Apply safety procedures during HVAC maintenance

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | Diagnostic scanner | 5 |
| 2 | Manifold gauge set (HVAC) | 5 |
| 3 | Vacuum pump | 3 |
| 4 | Refrigerant recovery machine | 2 |
| 5 | Refrigerant charging station | 2 |
| 6 | Digital multimeter | 10 |
| 7 | Clamp meter | 5 |
| 8 | Infrared thermometer | 5 |
| 9 | Temperature sensor probe set | 5 |
| 10 | Pressure sensor testing kit | 3 |
| 11 | Electronic leak detector | 5 |
| 12 | UV leak detection lamp | 3 |
| 13 | Nitrogen cylinder with regulator | 2 |
| 14 | EXV diagnostic/calibration interface tool | 3 |
| 15 | Service hose set (HVAC) | 5 sets |
| 16 | Torque wrench set | 5 |
| 17 | Socket and spanner set (insulated) | 10 sets |
| 18 | Screwdriver set (insulated) | 10 sets |
| 19 | Pliers set (insulated) | 10 sets |
| 20 | PPE safety kit (gloves, goggles, shoes, face shield) | 25 sets |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Refrigerant gas (R134a / R1234yf) | 10 cylinders |
| 2 | Compressor oil (PAG / POE oil) | 5 liters |
| 3 | O-rings and sealing gaskets | 100 pcs |
| 4 | Refrigerant filter drier | 25 pcs |
| 5 | Nitrogen gas cylinder (leak testing) | 3 cylinders |
| 6 | Condenser cleaning solvent | 10 liters |
| 7 | Electrical contact cleaner spray | 15 cans |
| 8 | Cable ties (nylon) | 500 pcs |
| 9 | Insulation tape (HVAC grade) | 50 rolls |
| 10 | UV leak detection dye | 10 bottles |

## Maintain EV Inverter & Converter system

Overview: This competency standard covers the skills and knowledge required to Locate Fundamental Architecture of EV Inverter/Converter System, Diagnose Faults in EV Inverter & Converter System, Analyze Converter/Inverter Modules Data, Test Actuators of Inverter/Converter System and Inspect Wiring Harness of Inverter/Converter System

| Competency Units | Performance Criteria |
| --- | --- |
| Locate Fundamental Architecture of EV Inverter/Converter System | - Check Safety & EMI Shielding Architecture - Detect System Layout & Physical Integration - Locate Inverter Power Stage Components - Locate AC/DC Converter - Locate DC/AC Inverter - Detect DC-DC Converter (Step-Down) Architecture - Record Thermal Management Architecture |
| Diagnose Faults in EV Inverter & Converter System | - Conduct Preliminary Diagnostic Scanning, DTC Analysis and Freeze frame data, - Diagnose Trouble Code (DTC) as per repair Manual - Conduct High-Voltage Isolation (Insulation) Testing - Analyze sensor, actuator and Switching Signals - Assess Thermal Performance via Infrared (IR) Thermography - Diagnose DC-DC Converter Output and Efficiency |
| Analyze Converter/Inverter Modules Data | - Analyze Live Data Streams via Diagnostic Interface as per manual - Compare DC-Bus Voltage Stability Against Battery Management (BMS) Data - Assess sensors Switching Frequencies and Phase Currents - Cross-Reference Thermal Sensor Data with Cooling System Status - Validate DC-DC Conversion Efficiency and Low-Voltage Output Logic |
| Test Actuators of Inverter/Converter System | - Conduct actuation test using diagnostic tools - Cycle the Low-Temperature Coolant Pump (PWM). - Actuate the High-Voltage System Main Relays (SMR). - Test Power Electronics Cooling Fans |
| Inspect Wiring Harness of Inverter/Converter System | - Tactile Check for Connector Seating and Lock Engagement - Conduct Visual inspection of High-Voltage Insulation - Check the High-Voltage Interlock Loop (HVIL) Pilot Circuit - Inspect the resistance values of high/low voltage circuits wiring. - Inspect Low-Voltage Signal Wire "Bend Radii" and Stress Point - Conduct Physical Probe of Connector Seals for Moisture Ingress - Repair or replace faulty wirings and relevant components - Test wiring after repair work |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- EV high-voltage safety protocols, PPE, Lockout/Tagout (LOTO) procedures

- EMI shielding principles, and insulation requirements.

- Function of inverter power stage components

- AC/DC converters and their role in EV charging systems.

- working principle of DC/AC inverters and their role in motor control.

- PWM (Pulse Width Modulation) techniques used in inverter switching.

- Diagnostic procedures including scanning tools, DTC structure

- Types of diagnostic data (live data, freeze frame, historical data).

- Principles of high-voltage insulation resistance and isolation testing.

- DC-DC converter operation and evaluate its output performance and efficiency.

- CAN Bus communication and its role in inverter/converter control systems.

- Communication faults between EV control modules (ECU, BMS, inverter).

- Thermal management systems and analyze temperature impact on performance.

- Working principles of actuators used in inverter/converter systems.

- Diagnostic tool functions for actuator testing and activation.

- Operation of high-voltage system main relays (SMR).

- Wiring harness design, connector types, and locking mechanisms.

- High-voltage insulation standards and safety requirements.

- Function and operation of the High-Voltage Interlock Loop (HVIL).

- Importance of wire routing, bend radius, and mechanical stress factors.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be declared competent in this competency standard:

- Perform high-voltage insulation resistance test with Mega-ohmmeter between the DC bus and chassis ground, and verify the continuity of the High-Voltage Interlock Loop (HVIL) pilot circuit

- Probe the inverter gate driver signals with Digital Storage Oscilloscope (DSO) to measure Pulse Width Modulation (PWM) duty cycles and phase-to-phase output consistency.

- Command Low-Temperature (LT) coolant pump and fans via a diagnostic scan tool and trace the heat dissipation across the inverter and DC-DC converter using an Infrared (IR) thermographic camera.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | Class 0 HV Gloves | 15 Pairs |
| 2 | Insulated Tool Kit | 5 Sets |
| 3 | Arc Flash Face Shield | 5 Units |
| 4 | HV Rescue Hook | 5 Units |
| 5 | LOTO Kit | 5 Kits |
| 6 | Insulated Floor Mat | 5 Mats |
| 7 | Insulation Tester | 5 Units |
| 8 | Digital Multimeter | 10 Units |
| 9 | Automotive Oscilloscope | 5 Units |
| 10 | HV Differential Probes | 5 Units |
| 11 | EV Scan Tool | 3 Units |
| 12 | IR Thermal Camera | 5 Units |
| 13 | Milliohm Meter | 10 Units |
| 14 | AC/DC Clamp Meter | 10 Units |
| 15 | Back-Probe Kit | 10 Sets |
| 16 | Pin Tension Gauges | 5 Sets |
| 17 | Terminal Repair Kit | 5 Sets |
| 18 | Functional EV Rig | 5 Unit |
| 19 | Sectioned Inverter | 5 Units |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Qty required |
| --- | --- | --- |
| 1 | EV-Specific Coolant | 20 Liters |
| 2 | Dielectric Grease | 10 Tubes (3oz) |
| 3 | Electrical Contact Cleaner | 5 Cans |
| 4 | Orange Shielded Cable | 10 Meters |
| 5 | High-Temp Loom/Tape | 10 Rolls |
| 6 | Heat Shrink Tubing | 2 Assorted Kits |
| 7 | Nitrile Gloves | 5 Boxes (100ct) |
| 8 | Terminal Pins/Sockets | 200 Pieces |
| 9 | Cotton Swabs/Lint-free Wipes | 2 Bulk Packs |
| 10 | Glove Dusting Powder | 3 Bottles |
| 11 | Distilled Water | 10 Gallons |
| 12 | Nylon Cable Ties | 500 Pieces |

- Service Electric Motor/Drive Unit

Overview: This competency standard covers the skills and knowledge required to Test Insulation Resistance of Stator, Test motor stator for phase-to-phase continuity, Inspect Internal Rotor Position Sensor (Resolver), Inspect Bearings of High-Speed Drive Unit and Maintain Reduction Gear, Differential & Drive Axle Assemblies

| Competency Units | Performance Criteria |
| --- | --- |
| CU1 Test Insulation Resistance of Stator | - Confirm readiness of HV system after proper isolation. - Confirm availability of non-conductive work mat. - Confirm availability of HV-specific tools and equipment. - Select appropriate insulation resistance tester (Megger) - Measure insulation resistance between U, V, W phases and ground. - Apply correct test voltage as per OEM standards (500V/1000V DC). - Interpret insulation values against specified limits. - Identify insulation degradation or dielectric breakdown. - Detect moisture ingress or coolant contamination Clear workspace of any flammable materials - Compare readings under different environmental conditions. - Confirm repeatability and accuracy of measurements - Record results and conclude system health. |
| CU2 Test motor stator for phase-to-phase continuity | - Confirm milli-ohm meter calibrated for low resistance measurement. - Measure resistance between U–V, V–W, and U–W. - Confirm symmetry across all phases. - Identify open circuit conditions. - Detect short circuit or partial winding faults. - Analyze imbalance indicating internal damage. - Inspect terminal connections and contact points. - Compare readings with diagnostic trouble codes (DTCs) & Compare with OEM reference values. |
| CU3 Inspect Internal Rotor Position Sensor (Resolver) | - Inspect wiring harness and shielding condition. - Confirm excitation voltage supply. - Measure resolver output signals - Analyze waveform using oscilloscope - Detect signal distortion or phase shift - Confirm synchronization with control unit - Diagnose resolver-related DTCs. - Perform calibration/relearn procedure |
| CU4 Inspect Bearings of High-Speed Drive Unit | - Conduct operational noise analysis under varying conditions - Identify abnormal sounds (whining, grinding) - Perform vibration/NVH analysis. - Apply diagnostic tools (stethoscope/NVH analyzer). - Inspect lubrication condition and contamination. - Measure axial and radial play. - Identify overheating or discoloration on the bearing and motor housing. - Assess impact on performance and efficiency. - Regulate replacement criteria. |
| CU5 Maintain Reduction Gear, Differential & Drive Axle Assemblies | - Inspect gear teeth for wear, pitting, or scoring - Check gear oil level and condition - Identify contamination or metal particles - Assess differential operation - Inspect axle shafts and CV joints - Diagnose abnormal noise during operation - Measure backlash and gear clearances - Inspect seals and leakage points - Confirm smooth torque transmission. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Introduction to E-Drive unit.

- Electric Drive Control System Assembly

- Electric motor construction and working principles.

- Diagnostic Trouble Codes and live data interpretation.

- Root Cause Analysis methodology.

- High voltage branch connectors and AC input/output connector terminals for Power Distribute Unit

- Power Distribute Unit & Junction box.

- Stator winding and insulation systems.

- Insulation Resistance Testing

- Importance of insulation, resistance and leakage of current.

- importance of resistance between U, V, W phases and ground.

- Resolver operation and signal generation.

- NVH analysis techniques.

- Bearing types, lubrication, and possible failure causes

- Personal Protective Equipment (PPE)

- Multimeter Safety

- Emergency Response

- EV Battery Safety

- Safety in Workshops

- Risk Assessment

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Apply OHS and high-voltage safety techniques and use correct PPE throughout all practical tasks.

- Apply electrical concepts including insulation resistance, current leakage, and phase-to-phase (U, V, W) and phase-to-ground resistance characteristics

- Apply clear understanding of E-Drive unit architecture by identifying key components.

- Clarify Electric Drive Control System Assembly, including signal flow, power distribution, and control logic, with reference to actual system components.

- Clarify electric motor construction and working principles, including stator, rotor, and electromagnetic torque generation, supported by practical examples.

- Identify and demonstrate correct handling of high-voltage branch connectors and AC input/output terminals within the Power Distribution Unit (PDU), ensuring safe working practices.

- Identify different bearing types, explain lubrication methods, and analyze possible causes of bearing failure in electric motors.

- Conduct a basic risk assessment before performing tasks, identifying potential hazards and implementing control measures.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | OEM Diagnostic Scanner / OBD Tool | 5 |
|  | Data Logging Tool | 3 |
|  | Digital Multimeter (HV Rated) | 5 |
|  | Oscilloscope | 2 |
|  | Insulation Tester (Megohmmeter) | 3 |
|  | HV Test Leads/Probes | 5 sets |
|  | DC Clamp Meter | 5 |
|  | Battery Load Tester | 2 |
|  | Insulated Hand Tools (1000V Rated) | 5 sets |
|  | Torque Wrench (Calibrated) | 3 |
|  | Socket & Ratchet Set | 5 sets |
|  | Cable Cutter/Crimping Tool | 3 |
|  | Trim Removal Tools | 3 sets |
|  | Battery Lift/Hoist | 2 |
|  | Insulated Rubber Gloves | 10 pairs |
|  | Leather Protector Gloves | 10 pairs |
|  | Safety Helmet with Face Shield | 5 |
|  | Safety Goggles / Arc Flash Visor | 5 |
|  | Flame-Resistant Clothing | 5 sets |
|  | Lock-Out/Tag-Out (LOTO) Kit | 3 sets |
|  | Insulated Safety Mat | 5 |
|  | Fire Extinguisher (Li-ion Compatible) | 5 |
|  | Spill Kit | 2 |
|  | First Aid Kit | 3 |
|  | Warning Signs / Barricades | 5 sets |
|  | HV Rescue Hook | 5 |
|  | Infrared Thermometer / Thermal Camera | 2 |
|  | Temperature & Humidity Meter | 2 |
|  | Differential oil seal tool | 1 |
|  | Inspection Forms / Checklists | As required |
|  | Rubber hammer | 3 |
|  | Allen key set | 2 |
|  | Workshop Software / PLM Access | 1 system |
|  | Wall mounted Charger along with Charging Kit | 1 |
|  | Work Benches | 10 |
|  | Trays for Tools and Parts | 10 |
|  | Seat and Steering Covers | 5 Set |
|  | Fender and Bumper Covers | 5 Set |
|  | Safety Shoes (Insulated) | 25 pairs |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | heat shrink Tubes | 3 dozen |
| 2 | Insulated Tap | 3 dozen |
| 3 | Thermal paste (for cooling systems) | 10 |
| 4 | WD-40 | 12 |
| 5 | Contact cleaners | 12 |
| 6 | Coolant | As required |
| 7 | Cable ties | 4 dozen |
| 8 | E-drive assembly lubricating oil | As required |

## Analyse Controller Area Network (CAN) Bus & Network Communication Practices

Overview: This competency standard covers the skills and knowledge required to Analyze CAN-Bus Communication Signals through Oscilloscope, Measure Terminating Resistance of CAN-Bus Network, Detect Malfunctioning ECU Node by Bus Isolation, Repair Damaged Twisted-Pair Communication Wiring and Analyze Gateway Module Data Routing Efficiency

| Competency Units | Performance Criteria |
| --- | --- |
| Analyze CAN-Bus Communication Signals through Oscilloscope | - Identify CAN system issues with Oscilloscopes for CAN-Bus decode functions. - Set the oscilloscope to CAN-Bus protocol decoding mode. - Connect probes to CAN-H and CAN-L lines (differential signal) - Check CAN-H and CAN-L signal voltage levels (as per standard guidelines) - Assess CAN Dominant (0) and recessive (1) Values - Assess CAN graph Bit timing and baud rate - Identify CAN-H and CAN-L Error frames or corrupted data |
| Measure Terminating Resistance of CAN-Bus Network | - Disconnect power from the Vehicle network. - Set the multimeter to the resistance mode (Ohms/Ω) ideally on the 200Ω or 600Ω range (if auto-ranging not available) - Identify CAN-H and CAN-L pins at DLC connector for measuring termination resistance - Measure the CAN-H and CAN-L terminating resistance values. - Assess standard termination resistance values of CAN-H and CAN-L as per repair manual guidelines. |
| Detect Malfunctioning ECU Node by Bus Isolation | - Connect a Vehicle diagnostic tool to the vehicle's OBD Port. - Perform DTC interrogation on the malfunctioning ECU using the diagnostic Tool - Conduct a comprehensive diagnostic scan to inspect all effected ECUs within the vehicular network topology. - Access the live data stream or parameter list on the diagnostic Tool. - Perform an in-depth analysis of the DTC freeze frame data set. - Confirm the signal voltage parameters in accordance with the prescribed DTC remediation procedure. - Disconnect ECUs individually from the CAN-Bus network to facilitate fault isolation and identification. in case completely BUS failure. - Check network communication after each disconnection. - Identify aberrant ECU, compromised wiring segment, or defective connector precipitating the CAN-Bus network outage. |
| Repair Damaged Twisted-Pair Communication Wiring | - Check relevant repair manual or electrical wiring diagram to Identify the defective area. - Analysis and modify defective ECU component, wiring segment, or connectors with precision tools to extract defects. - Perform a multimeter-based resistance measurement across the wiring or joint connector to detect potential open-circuit discontinuities or short-circuit anomalies. - Identify and isolate the defected area - Strip back damaged insulation and trim wires. - Apply solder or crimp connectors to establish robust, low-resistance joints, ensuring mechanical integrity and reliable electrical continuity. - Re-twist the wires, maintaining the original twist rate if feasible, to restore the twisted pair's electromagnetic interference (EMI) rejection properties. - Apply heat shrink tubing or wrap with electrical tape to provide insulation, moisture resistance, and mechanical protection to the repaired joint - Conduct a comprehensive wiring validation to confirm conductor continuity, inter-wire and ground-plane isolation, and adherence to termination and shielding protocols (if applicable) |
| Analyse Gateway Module Data Routing Efficiency | - Consult the repair manual to identify the Gateway Module's location, pinout, and communication protocols, facilitating targeted diagnostic access - Connect the diagnostic Tool to the OBD-II port and establish communication with all Electronic Control Units (ECUs) via the Gateway Module. - Conduct a thorough diagnostic scan via the Gateway Module to retrieve fault codes and live data from affected ECUs, analyzing the vehicle's network topology for anomalies - Check message prioritization (like safety-critical vs. comfort systems) - Perform assessment of the Gateway Module's Techniques including Power Management, Wake-up and Sleep Modes, ECUs Firmware Updates, and ECUs Communication to assess system operation. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Introduction to network and CAN Communication

- Analysis of automotive CAN Network Protocol Decoding and waveform

- Identification of CAN-H and CAN-L Error frames

- Fundamentals of CAN-Bus Network Topology and Termination Requirements

- Testing CAN-Bus Termination Resistance and Physical Layer Specifications

- Standard termination resistance values of CAN-H and CAN-L as per manual

- Diagnostic Systematic CAN-Bus Network and Fault Isolation

- CAN-Bus Network Failure Modes and Diagnostic Classifications

- Live data parameters.

- Network topology

- Divide and isolation of fault

- Precautions for Vehicle Wiring Harnesses, Modules and Actuators

- Introduction to Wiring diagram

- Precautions for Vehicle Wiring Harnesses

- Twisted pair principles

- Techniques of wiring Harness, Fault Diagnosis and CAN Twisted Pair Repair.

- Techniques for Multimeter-Based Diagnosis of Electrical Circuits.

- Post-repair validation

- Wastage Disposal for Environmental Protection

- Gateway Module Architecture

- Gateway Module role and function

- Common Gateway DTCs

- Multi-Network Diagnostics in Modern Vehicles

- Gateway Module Message prioritization

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identification of CAN High and CAN Low wires

- Measurement of CAN bus voltage levels

- Troubleshooting of simple CAN communication issues

- CAN bus termination resistance checks

- Identification of ECU connections and pinouts

- Simple network topology diagram

- CAN bus signal waveforms (oscilloscope screenshots)

- Documentation of findings and repairs

- Create a diagnosis report including all captured data

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | OEM Diagnostic Scanner / OBD Tool | 5 |
|  | Laptop/Tablet with Software | 5 |
|  | Data Logging Tool | 3 |
|  | Digital Multimeter (HV Rated) | 5 |
|  | Oscilloscope | 2 |
|  | Insulation Tester (Megohmmeter) | 3 |
|  | HV Test Leads/Probes | 5 sets |
|  | DC Clamp Meter | 5 |
|  | Non-contact Voltage Tester | 5 |
|  | Battery Load Tester | 2 |
|  | Insulated Hand Tools (1000V Rated) | 5 sets |
|  | Torque Wrench (Calibrated) | 3 |
|  | Socket & Ratchet Set | 5 sets |
|  | Cable Cutter/Crimping Tool | 3 |
|  | Trim Removal Tools | 3 sets |
|  | Battery Lift/Hoist | 2 |
|  | Insulated Rubber Gloves | 10 pairs |
|  | Leather Protector Gloves | 10 pairs |
|  | Safety Helmet with Face Shield | 5 |
|  | Safety Goggles / Arc Flash Visor | 5 |
|  | Flame-Resistant Clothing | 5 sets |
|  | Lock-Out/Tag-Out (LOTO) Kit | 3 sets |
|  | Insulated Safety Mat | 5 |
|  | Fire Extinguisher (Li-ion Compatible) | 5 |
|  | Spill Kit | 2 |
|  | First Aid Kit | 3 |
|  | Warning Signs / Barricades | 5 sets |
|  | HV Rescue Hook | 5 |
|  | Infrared Thermometer / Thermal Camera | 2 |
|  | Temperature & Humidity Meter | 2 |
|  | Tire Pressure Gauge | 1 |
|  | Inspection Forms / Checklists | As required |
|  | Clipboard / Digital Tablet | 5 |
|  | Camera / Mobile Device | 3 |
|  | Printer/Scanner | 1 |
|  | Workshop Software / PLM Access | 1 system |
|  | Secure Internet Connection | 1 |
|  | Data Backup Devices | 2 |
| 39 | 22KW wall mounted Charger along with Charging Kit | 1 |
| 40 | Work Benches | 10 |
| 41 | Trays for Tools and Parts | 10 |
| 42 | Seat and Steering Covers | 5 Set |
| 43 | Fender and Bumper Covers | 5 Set |
| 44 | Safety Shoes (Insulated) | 25 pairs |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | heat shrink Tubes | 3 dozen |
| 2 | Insulated Tap | 3 dozen |
| 3 | Thermal paste (for cooling systems) | 10 |
| 4 | WD-40 | 12 |
| 5 | Contact cleaners | 12 |
| 6 | Label makers and labels (for wiring and components) | 12 |
| 7 | Cable ties | 4 dozen |
| 8 | Wire connectors (various sizes) | As required |
| 9 | Fuses for different rating (Blade fuses, Bolt-down fuses) | As required |

## Conduct Advanced Driver Assistance Systems (ADAS) & Autonomous Sensor Integration

Overview: This competency standard covers the skills and knowledge required to Calibrate Forward-Facing Long-Range Radar (LRR), Align Blind Spot Monitoring (BSM) Sensors, Program Front View Camera (FVC) Module, Troubleshoot Ultrasonic Park Assist Sensors and Synchronize LiDAR and Camera Data Streams

| Competency Units | Performance Criteria |
| --- | --- |
| Calibrate Forward-Facing Long-Range Radar (LRR) | - Position the OEM radar target board at the correct distance, offset, and height per the calibration procedure. - Confirm target board alignment using a calibrated measurement reference system before initiating the calibration routine. - Connect the OEM diagnostic platform to the vehicle CAN/Ethernet interface and navigate to the LRR calibration function. - Confirm all pre-calibration conditions (ride height, steering angle at zero, tyre pressures) are within OEM specification before starting the static calibration sequence. - Execute the LRR static calibration sequence to completion and confirm azimuth and elevation offsets are within OEM acceptance thresholds. - Conduct a controlled road test at OEM-specified speed to confirm the radar correctly detects and tracks targets within the specified range and angular window. - Log dynamic road test results using the diagnostic platform's data recording function and sign the road test record. - Clear all LRR-related DTCs and verify zero active codes on a final diagnostic scan. - Document all calibration parameter values on the job card and obtain assessor sign-off before returning the vehicle to service. |
| Align Blind Spot Monitoring (BSM) Sensors | - Retrieve all active and stored BSM DTCs using the OEM diagnostic platform. - Inspect BSM sensor housings and mounting brackets for physical damage, corrosion, and deformation, and verify sensor faces are clean and unobstructed. - Confirm mounting fastener torque meets OEM specification using a calibrated torque wrench. - Input all required BSM module configuration parameters (vehicle width, sensor variant, tow package status) via the diagnostic platform before initiating the alignment routine. - Run the BSM alignment calibration sequence and confirm left and right detection-zone angles are within OEM symmetry tolerance. - Adjust sensor mounting brackets per OEM guidance where angle values exceed tolerance, and re-run the alignment sequence to confirm acceptance. - Pass a reference obstacle alongside the vehicle at OEM-specified speed and lateral distance, confirming both BSM sensors activate the warning indicator within the correct detection zone. - Clear all BSM DTCs and confirm zero active codes on a final diagnostic scan. - Record alignment results and sensor IDs on the completed job card. |
| Program Front View Camera (FVC) Module | - Confirm FVC module part number and hardware version compatibility with the vehicle ADAS network by cross-referencing the module label against OEM fitment data. - Confirm battery voltage meets OEM minimum (≥12.5 V) using a battery support unit before starting any programming operation. - Create a stable DoIP/Ethernet or CAN programming connection and initiate the VIN-writing and variant coding routine. - Enter variant coding parameters (camera type, lane width variant, market region) per the OEM coding guide without interrupting the programming cycle. - Run the FVC intrinsic calibration routine using the OEM camera target board. - Confirm image quality metrics (sharpness index, distortion coefficient) are within OEM acceptance limits from the intrinsic calibration output. - Position the OEM extrinsic calibration target at the specified longitudinal distance and lateral offset per the calibration procedure. - Run the pitch/roll/yaw extrinsic calibration and confirm the coordinate transform values are accepted and stored in non-volatile memory. - Conduct an LDW/LKA practical check confirming the FVC detects lane boundaries at OEM-specified distances and triggers outputs within the required latency. - Apply the OEM diagnostic flowchart to a simulated FVC fault (coding mismatch, image quality error, or bus dropout) and identify the root cause. - Document the corrective action on the fault analysis form and justify the diagnosis verbally to the assessor. |
| Troubleshoot Ultrasonic Park Assist Sensors | - Retrieve all active and historical park assist system DTCs using the OEM diagnostic platform - Locate the implicated sensor position(s) on the OEM wiring diagram sensor map. - Identify each fault as an open circuit, short circuit, internal sensor failure, or configuration error before commencing physical inspection. - Inspect sensor housings, wiring connectors, and mounting grommets for impact damage, moisture ingress, corroded pins, and correct seating depth. - Measure supply voltage and ground continuity at each suspect sensor connector using a calibrated DMM. - Activate individual sensor transmits pulses via the diagnostic platform and capture the echo return waveform for each suspect sensor. - Compare captured wave form frequency, amplitude, and echo delay against OEM acoustic performance specification values. - Install the correct OEM replacement sensor flush to the bumper skin within the specified protrusion tolerance with the connector fully locked and grommet fully sealed. - Run a post-replacement park assist system self-test using the diagnostic platform and verify the replaced sensor reports within the OEM-specified signal quality range. - Place a flat-face test target at OEM-defined detection zone distances and confirm correct audible and visual alert activation at each zone. - Separate a multi-sensor simultaneous fault scenario between individual sensor failures and a systemic cause (ECU supply, CAN error, or shared ground) using structured diagnostic reasoning. |
| Synchronize LiDAR and Camera Data Streams | - Confirm both the LiDAR and FVC are outputting PPS/PTP synchronization pulses at OEM-specified frequencies using a dual-channel oscilloscope. - Record the measured phase offset between the two sensor synchronization pulses. - Enter the OEM LiDAR-to-camera extrinsic transform parameters (rotation matrix and translation vector) into the sensor fusion ECU via the diagnostic platform. - confirm correct storage of the entered extrinsic transform values with a readback confirmation from the module. - Place a retroreflective calibration target at a known, measured position in front of the vehicle. - Capture simultaneous LiDAR point-cloud and camera image frames of the target. - Evaluate the target centroid spatial overlap in both data streams using OEM validation software and confirm co-registration error is within OEM tolerance. - Monitor live fused sensor output during a controlled low-speed pass (≤10 km/h) past a static obstacle using the OEM engineering tool. - Confirm LiDAR and camera detections are temporally aligned within OEM latency tolerance with no ghost objects or missed detections. - Analyze sensor logs to identify the specific parameter or hardware issue causing a synchronization fault (PTP drift, dropped LiDAR frames, or camera frame-rate mismatch). - Document the root cause and corrective action recommendation on the prescribed fault report. - Perform a final ADAS integration scan confirming ACC, AEB, and all fusion-dependent functions are within OEM operational specification. Operate connector lock/unlock mechanism to verify change in HVIL circuit status. - Compare measured results with OEM specifications and determine Pass/Fail status. - Record test results (connector ID, readings, operator details), capture evidence if required, and store records in a traceable system |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Occupational Health & Safety (OHS) precautions for ADAS sensor work and HV-adjacent systems.

- Principles of radio frequency (RF) radar operation: frequency bands (24 GHz / 77 GHz), beam angle, detection range, and azimuth/elevation offset concepts.

- OEM LRR calibration target specifications

- CAN bus and Ethernet/DoIP communication protocols used by ADAS ECUs and diagnostic platforms.

- LRR diagnostic trouble codes (DTCs): misalignment flags, radar blockage codes, reduced-performance mode indicators.

- Dynamic radar verification principles: detection zone geometry, target tracking logic, and data recording procedures.

- Operating principles of microwave/radar-based BSM sensors: detection zone geometry, frequency, sensitivity, and symmetry requirements.

- BSM system architecture and BSM fault classification: sensor placement at rear corners, control module interface, CAN communication, and warning indicator activation logic.

- Calibration errors, alignment faults, blockage codes, and hardware failures.

- FVC module architecture: image sensor type, field of view, focal length, mounting position, and interface to ADAS domain controller.

- Intrinsic camera calibration concepts: focal length, principal point, lens distortion coefficients, sharpness index, and OEM image quality acceptance metrics.

- Extrinsic calibration concepts: camera-to-vehicle coordinate frame, pitch, roll, yaw transform, and non-volatile memory storage verification.

- Lane Departure Warning (LDW) and Lane Keeping Assist (LKA) function operation: lane boundary detection distance, output latency, and road marking requirements.

- Principles of ultrasonic sensing: transducer operation, transmit pulse frequency (typically 40–48 kHz), echo return timing, and detection distance calculation.

- Park assists system architecture: sensor positions (front/rear), ECU signal processing, alert zone segmentation, and audible/visual output logic.

- Sensor wiring circuit topology: supply voltage, ground return, signal line, and connector pinout conventions in OEM wiring diagrams.

- LiDAR operating principles: time-of-flight (ToF) measurement, point-cloud generation, scan rate, field of view, and range specification.

- Camera-LiDAR sensor fusion architecture: ADAS domain controller, data stream management, and the role of synchronization in object detection accuracy.

- Hardware time synchronization methods: Pulse Per Second (PPS), GNSS-derived timing, and IEEE 1588 Precision Time Protocol (PTP) over Ethernet.

- Extrinsic calibration concepts: rotation matrix, translation vector, coordinate frame transformation between LiDAR and camera, and non-volatile storage.

- Co-registration test methodology, Temporal alignment and latency, Synchronization fault diagnosis.

- ADAS integration verification: ACC, AEB, and fusion-dependent system status confirmation, DTC clearance, and commissioning documentation.

- ISO 26262 Functional Safety and ISO/SAE 21434 Cybersecurity awareness as applied to ADAS sensor programming and fusion system commissioning.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Follow OHS and ADAS safety procedures and use correct PPE throughout all practical tasks.

- Set up OEM calibration targets at the correct position and verify alignment before initiating any calibration routine.

- Connect the OEM diagnostic platform, confirm all pre-calibration conditions, and execute static calibration sequences for LRR, BSM, and FVC to OEM acceptance thresholds.

- Perform FVC variant coding and confirm programmed parameters are correctly stored in non-volatile memory.

- Conduct intrinsic and extrinsic camera calibration and confirm image quality metrics and coordinate transform values meet OEM specifications.

- Inspect, test, and replace ultrasonic park assist sensors and verify correct system operation using a post-replacement self-test and obstacle detection check.

- Measure and confirm LiDAR–camera synchronization pulse alignment, enter extrinsic transform parameters, and verify co-registration error and temporal latency are within OEM tolerance.

- Diagnose ADAS sensor faults using OEM diagnostic flowcharts and present root cause and corrective action to the assessor.

- Clear all DTCs, complete job cards with calibration parameter values, and obtain assessor sign-off before returning the vehicle to service.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | Safety helmet | 25 |
| 2 | Safety goggles / face shield | 25 |
| 3 | Safety shoes (anti-static) | 25 |
| 4 | High-voltage insulated gloves (Class 0, IEC 60903) | 25 pairs |
| 5 | Arc-flash rated protective clothing / coveralls | 25 |
| 6 | Ear protection plugs | 25 pairs |
| 7 | Fire extinguisher (Class C – electrical) | 5 |
| 8 | Smoke / CO detecting alarm | 3 |
| 9 | First aid box (fully stocked) | 3 |
| 10 | Lock-out / Tag-out (LOTO) kit | 13 |
| 11 | Health and Safety Manual (ADAS / HV systems) | 5 |
| 12 | OEM LRR (Forward Radar) calibration target board | 5 |
| 13 | OEM FVC (Front Camera) intrinsic calibration target board | 5 |
| 14 | OEM FVC extrinsic calibration target with mounting fixture | 5 |
| 15 | Retroreflective LiDAR co-registration calibration target | 5 |
| 16 | Flat-face ultrasonic sensor test target (foam board) | 13 |
| 17 | Calibration target height-adjustment stand / tripod | 5 |
| 18 | Calibration floor marking tape / reference grid mat | 5 rolls |
| 19 | Laser measuring tape / distance reference tool (calibrated) | 5 |
| 20 | Precision levelling gauge / digital inclinometer | 5 |
| 21 | Tyre pressure gauge (digital, calibrated) | 13 |
| 22 | Digital multimeter (CAT III/IV, True RMS) | 13 |
| 23 | Clamp meter (AC/DC) | 5 |
| 24 | Dual-channel oscilloscope (≥200 MHz bandwidth) | 5 |
| 25 | IEEE 1588 PTP network analyzer / protocol tester | 3 |
| 26 | Calibrated torque wrench set (insulated, 5–50 Nm) | 5 |
| 27 | Depth gauge / flush-measurement tool (protrusion check) | 5 |
| 28 | Thermal imaging camera | 2 |
| 29 | Fire Extinguisher (Li-ion Compatible) | 5 |
| 30 | First Aid Kit | 5 |
| 31 | HV Warning Signs / Barricades | 5 |
| 32 | Emergency Rescue Hook | 5 |
| 33 | OEM variant coding & programming software license (network) | 5 seats |
| 34 | Battery support unit / intelligent charger (13.5–14.4 V stable output) | 5 |
| 35 | OEM sensor fusion / LiDAR validation software (PC-based) | 5 seats |
| 36 | Insulated screwdriver set (flat & Phillips, VDE rated) | 13 |
| 37 | Insulated spanner / wrench set (metric) | 13 |
| 38 | Insulated socket set with ratchet | 5 |
| 39 | Insulated pliers set (needle nose, combination, cutting) | 13 |
| 40 | Trim removal / plastic pry tool set | 13 |
| 41 | Connector pick tool set (for locking tab release) | 13 |
| 42 | Anti-static wrist strap and mat | 13 |
| 43 | Lint-free cloths / wipes | 5 packs |
| 44 | Electrical contact cleaner spray | 5 |
| 45 | Vehicle wheel chocks | 10 pairs |
| 46 | Vehicle lift / hoist (2-post or scissor) | 5 |
| 47 | LiDAR unit (demonstration / training unit with point-cloud output) | 3 |
| 48 | Front view camera module (training / replacement unit) | 5 |
| 49 | Ultrasonic Park assist sensor set (OEM replacement, various positions) | 13 sets |
| 50 | BSM sensor set (left & right, training / replacement unit) | 5 pairs |
| 51 | ADAS wiring harness simulator / breakout box | 3 |
| 52 | LRR radar module (training / replacement unit) | 3 |
| 53 | OEM service manuals / calibration procedure guides (ADAS) | 5 |
| 54 | OEM DTC reference guide (ADAS systems) | 5 |
| 55 | Job card / inspection report templates (printed sets) | 50 sets |
| 56 | Fault analysis form templates (printed sets) | 50 sets |
| 57 | Calibration record log book / data collection sheets | 25 |
| 58 | CBT assessment checklist booklets | 25 |
| 59 | OEM wiring diagram sets (ADAS sensor circuits) | 5 |
| 60 | Training vehicle (EV/HEV with ADAS suite: LRR, BSM, FVC, park assist, LiDAR) | 5 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | HV Cables (Training/Practice) | Bulk |
| 2 | Terminal Lugs (Various Sizes) | Assorted |
| 3 | Electrical contact cleaner spray | Assorted |
| 4 | Insulation Tape (HV Rated) | 5 rolls |
| 5 | Cable Marking Labels/Tags | Bulk |
| 6 | Cleaning Cloths/Wipes | Bulk |
| 7 | Calibration floor marking tape | 2 bottles |
| 8 | Printed job cards | 3 cans |
| 9 | Wire Brushes (Soft) | 5 |
| 10 | Inspection/Assessment Forms | As required |
| 11 | Clipboards / Tablets | 5 |
| 12 | Fault analysis forms | 3 |
| 13 | Anti-static bags | 20 |
| 14 | Lint-Free Cloth | Bulk |

## Perform HV Safety Isolation & Lockout-Tagout (LOTO)

Overview: This competency standard covers the skills and knowledge required to Establish a High-Voltage Exclusion Zone, Remove Switch by Manual Service Disconnect (MSD), Implement Lockout-Tagout (LOTO) Techniques, Verify Absence of High Voltage (Live-Dead-Live Test) and Discharge Residual High-Voltage Capacitors

| Competency Units | Performance Criteria |
| --- | --- |
| CU1 Establish High-Voltage Exclusion Zone | - Allocate designated EV bays and ensure clear marking as per handling of High Voltage Standard Operating Procedures (SOP) during training sessions. - Confirm safety fences are in place and secure before starting work on the EV vehicle to isolate the area and prevent unauthorized access. - Confirm HV-rated gloves are available and in good condition before starting work. - Confirm HV-rated non-conductive footwear are available and in good condition before starting work. - Remove any metal jewellery or accessories - Confirm HV system is de-energized - Confirm voltage meter with a High-Voltage probe (like Fluke HV probe) is available to verify circuit de-energization. - Confirm availability of HV warning signs or labels - Clear workspace of any flammable materials - Confirm good ventilation - Confirm availability of non-conductive work mat. - Installed Lockout/Tagout (LTO) Procedure protocols SOP - Confirm availability of HV-specific tools and equipment - Display a detailed buddy system protocol in the working bay, outlining specific guidelines for HV work, including communication procedures and emergency response steps, to ensure safe working practices. - Confirm Safety hook is readily available in case of emergency. |
| CU2 Remove Switch by Manual Service Disconnect (MSD) | - Confirm vehicle is secured by engaging the parking brake and shifting into park (automatic) or neutral (manual) gear before commencing work. - Perform de-energization of the HV system. - Confirm HV system de-energization and confirm absence of voltage (EOV) before commencing work. - Locate the Manual Service Disconnect (MSD) switch, typically situated adjacent to the high-voltage battery pack or HV connector assembly. - Remove the MSD in accordance with OEM specified procedures and guidelines. - Re-confirm HV system de-energization via voltage detection post-work. - Inspect HV system components for damage or wear post-work. - Document the MSD removal procedure and note any observations in the vehicle service record. |
| CU3 Implement Lockout-Tagout (LOTO) Techniques | - Check specific manufacturer's service manual for the EV model. - Put on insulating gloves (rated for 1000V, Class 0 or higher) and leather protectors. - Put cones, warning tape, and signs to designate a high-voltage work area. - Remove watches, rings, and jewelry to prevent short circuits - Turn off the ignition, turn off the HV system, and remove the key/proximity fob (store it at least 15 feet away). - Disconnect the negative terminal of the auxiliary 12V battery to prevent accidental HV relay activation. - Locate the HV Service Disconnect (Refer service manual). - Unlock and remove the orange plug as per service manual instructions. - Secure the removed Service Plug inside a lockbox or with a specialized LOTO cover over the plug socket. - Apply your personal padlock and tag to the lockbox, ensuring it cannot be re-energized by others - Wait for the required period as per given instructions in service manual (typically 5–10 minutes) for inverter capacitors to discharge. - Perform a voltage verification test using a voltmeter: first, test the meter on a known live source to confirm functionality; then, measure voltage at the HV battery terminals. - Measure voltage between HV positive and ground, and HV negative and ground |
| CU4 Verify Absence of High Voltage (Live-Dead-Live Test) | - Test your meter on a known live source (like the 12V auxiliary battery or a dedicated proving unit) to confirm the meter and leads are functioning correctly. - Test the target HV terminals where you intend to work. A reading of 0V confirms the system is powered down. - Perform a post-test verification on the known live source to validate meter functionality. - Keep one hand in your pocket when working with high voltage components to minimize risk of cardiac shock via a hand-to-hand path. - Inspect test leads for damage (cracks, burns, exposed wire). - Perform a continuity check by touching leads together on the Ω setting; expected reading is approximately 0 Ω, an OL (open loop) indication signifies faulty leads. - Perform an air-test on insulated gloves prior to use: trap air inside, roll cuff, and inspect for leaks. |
| CU5 Discharge Residual High-Voltage Capacitors | - Check the repair manual to identify capacitor bank and discharge points - Connect discharge tool to capacitor terminals - Monitor voltage drops; wait until <60V or manufacturer's threshold. - Confirm 0 V with multimeter before servicing |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Introduction to HV Safety

- LOTO Procedure

- HV System Components

- Personal Protective Equipment (PPE)

- Multimeter Safety

- Emergency Response

- EV Battery Safety

- Charging System Safety

- Insulation Resistance Testing

- Safety in Workshops

- Risk Assessment

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- HV Safety Standards.

- Types of Hazardous Energy

- LOTO Principles

- Emergency Procedures.

- Isolation Checklist/Plan

- Risk Assessment.

- Identification of Energy Sources:

- Proper Use of LOTO Devices

- Isolation & Testing

- Adherence to Procedures

- Removal Procedure

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | OEM Diagnostic Scanner / OBD Tool | 5 |
|  | Laptop/Tablet with Software | 5 |
|  | Data Logging Tool | 3 |
|  | Digital Multimeter (HV Rated) | 5 |
|  | Oscilloscope | 2 |
|  | Insulation Tester (Megohmmeter) | 3 |
|  | HV Test Leads/Probes | 5 sets |
|  | DC Clamp Meter | 5 |
|  | Non-contact Voltage Tester | 5 |
|  | Battery Load Tester | 2 |
|  | Insulated Hand Tools (1000V Rated) | 5 sets |
|  | Torque Wrench (Calibrated) | 3 |
|  | Socket & Ratchet Set | 5 sets |
|  | Cable Cutter/Crimping Tool | 3 |
|  | Trim Removal Tools | 3 sets |
|  | Battery Lift/Hoist | 2 |
|  | Insulated Rubber Gloves | 10 pairs |
|  | Leather Protector Gloves | 10 pairs |
|  | Safety Helmet with Face Shield | 5 |
|  | Safety Goggles / Arc Flash Visor | 5 |
|  | Flame-Resistant Clothing | 5 sets |
|  | Lock-Out/Tag-Out (LOTO) Kit | 3 sets |
|  | Insulated Safety Mat | 5 |
|  | Fire Extinguisher (Li-ion Compatible) | 5 |
|  | Spill Kit | 2 |
|  | First Aid Kit | 3 |
|  | Warning Signs / Barricades | 5 sets |
|  | HV Rescue Hook | 5 |
|  | Infrared Thermometer / Thermal Camera | 2 |
|  | Temperature & Humidity Meter | 2 |
|  | Tire Pressure Gauge | 1 |
|  | Inspection Forms / Checklists | As required |
|  | Clipboard / Digital Tablet | 5 |
|  | Camera / Mobile Device | 3 |
|  | Printer/Scanner | 1 |
|  | Workshop Software / PLM Access | 1 system |
|  | Secure Internet Connection | 1 |
|  | Data Backup Devices | 2 |
|  | 22KW wall mounted Charger along with Charging Kit | 1 |
|  | Work Benches | 10 |
|  | Trays for Tools and Parts | 10 |
|  | Seat and Steering Covers | 5 Set |
|  | Fender and Bumper Covers | 5 Set |
|  | Safety Shoes (Insulated) | 25 pairs |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | heat shrink Tubes | 3 dozen |
| 2 | Insulated Tap | 3 dozen |
| 3 | Thermal paste (for cooling systems) | 10 |
| 4 | WD-40 | 12 |
| 5 | Contact cleaners | 12 |
| 6 | Label makers and labels (for wiring and components) | 12 |
| 7 | Cable ties | 4 dozen |
| 8 | Wire connectors (various sizes) | As required |
| 9 | Fuses for different rating (Blade fuses, Bolt-down fuses) | As required |

- Repair On-Board Charging System (OBC)

Overview: This competency standard covers the skills and knowledge required to Analyse AC to DC conversion failures, Replace defective On-Board Charger (OBC) unit, Test Charging Connector proximity and pilot signals, Inspect charging port high-voltage pins for arcing and Configure Charging Control Module (CCM) software

| Competency Units | Performance Criteria |
| --- | --- |
| CU1 Analyse AC to DC conversion failures. | - Analyze charging complaints and identify possible causes. - Allocate designated EV bays and ensure clear marking as per handling of High Voltage Standard Operating Procedures (SOP) during training sessions. - Confirm safety fences are in place and secure before starting work on the EV vehicle to isolate the area and prevent unauthorized access. - Analyse the Diagnostic Trouble Codes (DTCs). - Confirm and check fuses, relays, and proper connections of Power, Neutral, and Earth (L, N & Ground). - Confirm and analyze the AC input supply. - Confirm and analyze the DC output parameters. - Assess thermal behavior and inspect power electronics, including the AC/DC converter circuit (rectifier circuit, capacitors, and PCB-circuit board). - Confirm the AC/DC integrated charging socket wiring harness assembly and check for interlock or short circuit failures. - Compare readings under different environmental conditions - Compare measured values with OEM specifications and confirm the root cause |
| CU2 Replace defective On-Board Charger (OBC) unit | - Retrieve and analyze Diagnostic Trouble Codes (DTCs) related to the OBC system - Apply High Voltage (HV) safety measures, including use of PPE and proper isolation. - Power off the vehicle and disconnect the 12V auxiliary battery - Disconnect High Voltage system and wait for the specified capacitor discharge time. - Inspect OBC wiring harness, connectors, and cooling lines for any damage or leakage. - Remove safely the defective OBC unit as per OEM service manual. - Install the new OBC unit and ensure proper mounting and torque specifications. - Reconnect all electrical connectors and cooling pipes securely. - Refill coolant (if applicable) and ensure no leakage in the system. - Clear all fault codes with a diagnostic scanner and perform system reset. - Perform practical verification by testing AC charging operation and confirming normal system performance. |
| CU3 Test Charging Connector proximity and pilot signals | - Analyze Possible Causes of Charging Failure and component identification. - Apply High Voltage (HV) safety measures and confirm proper PPE usage. - Inspect charging connector physically for damage, dirt, or bent pins. - Confirm proper connection between the charging gun and vehicle inlet. - Check Proximity (PP) signal to confirm correct cable connection and locking status. - Measure and analyze the Pilot (CP) signal waveform and voltage levels. - Confirm communication between EV and charger through pilot signal response. - Inspect wiring harness and connector pins for continuity, short circuit, or open circuit. - Compare measured PP and CP values with OEM specifications. - Identify faults such as signal interruption, incorrect voltage, or communication failure. - Confirm proper charging initiation and termination through signal validation. |
| CU4 Inspect charging port high-voltage pins for arcing | - Confirm the vehicle is powered off and the HV system is isolated before inspection. - Inspect charging port HV pins for signs of arcing, such as burn marks, discoloration, or pitting. - Check melted insulation, carbon deposits, or damaged pin surfaces. - Inspect connector alignment and ensure pins are not bent, loose, or recessed. - Confirm cleanliness of the charging port (no dust, moisture, or foreign particles). - Check overheating signs around the connector housing. - Inspect mating surface of charging gun/connector for similar damage. - Compare findings with OEM standards and allowable limits. - Identify root cause (poor contact, loose connection, contamination) |
| CU5 Configure Charging Control Module (CCM) software | - Confirm requirement for CCM configuration (software update, replacement, or fault rectification). - Connect approved diagnostic tool and ensure stable communication with the vehicle. - Confirm vehicle battery voltage (12V & HV) is within specified charging range before programming. - Confirm stable power supply (support charger) to avoid interruption during software configuration. - Retrieve and record existing software version and configuration data. - Download and select the correct OEM-approved CCM software version. - Perform CCM software configuration/programming as per OEM procedure. - Monitor programming process and confirm no interruption or communication loss. - Clear all Diagnostic Trouble Codes (DTCs) after programming, - Perform system reset and reinitialization of the CCM. - Confirm proper communication between CCM and other vehicle systems. - Perform practical test (AC/DC charging) to confirm correct operation. - Compare system parameters with OEM specifications. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- EV Charging System Fundamentals

- Working principle of AC to DC conversion

- Function and operation of On-Board Charger (OBC)

- Charging Control Module (CCM) role and communication

- Basics of AC and DC power systems voltage, current, resistance, and power calculations

- Rectifier circuits, capacitors, and power electronics (IGBT/MOSFET)

- Electrical & Electronic Concepts

- AC & DC charging system fundamentals

- Proximity Pilot (PP) and Control Pilot (CP) signal operation

- Communication between EV and charging station

- Charging Standards & Signals

- Diagnostic Trouble Codes (DTCs) interpretation

- Diagnostic tools and live data analysis

- Root cause analysis techniques

- Diagnostic & Fault Analysis

- High Voltage Safety Procedures

- HV system isolation procedures

- PPE (insulated gloves, tools)

- Capacitor discharge time and safety precautions

- Inspection & Testing Techniques

- Inspection of HV connectors and terminals (arcing, corrosion, damage)

- Measurement of AC input and DC output

- Insulation resistance testing and interpretation

- Thermal Management

- Heat generation in power electronics

- Cooling systems (air/liquid cooling)

- Overheating causes and protection mechanisms

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform AC to DC conversion fault diagnosis on an EV.

- Replace a defective On-Board Charger (OBC) following OEM procedure

- Test Proximity (PP) and Pilot (CP) signals using appropriate tools

- Inspect charging port HV pins for arcing or damage

- Configure CCM software using a diagnostic tool

- Confirm normal charging operation (AC/DC)

- Confirm no fault codes present.

- Confirm system performance after repair.

- Correctly identify faults (OBC failure, wiring issue, signal fault, arcing, etc.)

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
|  | OEM Diagnostic Scanner / OBD Tool | 1 |
|  | Extension cables (rated for EV use) | 1 |
|  | Digital Multimeter (HV Rated) | 3 |
|  | Oscilloscope (for Pilot & Proximity signal analysis) | 2 |
|  | Insulation Tester (Megohmmeter) | 3 |
|  | HV Test Leads/Probes | 5 sets |
|  | AC/DC Clamp Meter | 5 |
|  | AC Charging Gun (Type 2) | 2 |
|  | Insulated Hand Tools (1000V Rated) | 5 sets |
|  | Torque Wrench (Calibrated) | 3 |
|  | Socket & Ratchet Set | 5 sets |
|  | Cable Cutter/Crimping Tool | 3 |
|  | Trim Removal Tools | 3 sets |
|  | Coolant Filling Funnel | 2 |
|  | Insulated Rubber Gloves | 10 pairs |
|  | Leather Protector Gloves | 10 pairs |
|  | Safety Helmet with Face Shield | 5 |
|  | Safety Goggles / Arc Flash Visor | 5 |
|  | Flame-Resistant Clothing | 5 sets |
|  | Lock-Out/Tag-Out (LOTO) Kit | 2 sets |
|  | Insulated Safety Mat | 5 |
|  | Fire Extinguisher (Li-ion Compatible) | 2 |
|  | Spill Kit | 2 |
|  | First Aid Kit | 3 |
|  | Coolant Drain Tray | 2 |
|  | HV Rescue Hook | 5 |
|  | Infrared Thermometer / Thermal Camera | 2 |
|  | Temperature & Humidity Meter | 2 |
|  | Differential oil seal tool | As required |
|  | Inspection Forms / Checklists | As required |
|  | Rubber hammer | 3 |
|  | Allen key set | 2 |
|  | Workshop Software / PLM Access | 1 system |
|  | Wall mounted Charger along with Charging Kit | 1 |
|  | Work Benches | 5 |
|  | Trays for Tools and Parts | 10 |
|  | Seat and Steering Covers | 5 Set |
|  | Fender and Bumper Covers | 5 Set |
|  | Safety Shoes (Insulated) | 25 pairs |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Electrical contact cleaner | 3 dozen |
| 2 | Coolant (as per OEM specification) | As required |
| 3 | Thermal paste (for cooling systems) | 10 |
| 4 | WD-40 | 12 |
| 5 | Contact cleaners | 12 |
| 6 | Insulation tape | As required |
| 7 | Cable ties | 4 dozen |
| 8 | E-drive assembly lubricating oil | As required |

## Perform Sensor and actuator Calibration

Overview: This competency standard covers the skills and knowledge required to Flash Vehicle Control Unit (VCU) with updated firmware, Calibrate Accelerator Pedal Position Sensor (APPS), Perform zero-point calibration of steering angle sensor (SAS), Perform BMS State-of-Health (SOH) recalibration and Synchronize motor rotor position sensor (Resolver)

| Competency Units | Performance Criteria |
| --- | --- |
| Flash Vehicle Control Unit (VCU) with updated firmware | - Collect vehicle details, VCU specifications, VIN, and OEM firmware update requirements. - Review technical documentation to confirm correct firmware version, compatibility, and update procedure. - Detect availability and calibration/status of diagnostic tools, flashing device, and stable power supply. - Connect an external power source or battery support unit to maintain stable voltage during flashing. - Power down the vehicle as required and prepare the system for firmware update mode. - Apply safety precautions and wear appropriate PPE as per workshop standards. - Connect the diagnostic/flashing tool to the vehicle communication interface (OBD/VCU port). - Establish secure communication between the tool and the Vehicle Control Unit (VCU). - Backup existing firmware and record current version details before initiating the update. - Select and load the correct firmware file from OEM-approved source. - Initiate the flashing process following OEM sequence and monitor progress continuously. - Maintain stable connection and power supply throughout the flashing process to avoid interruption. - Confirm successful installation to update firmware version in the system. - Perform post-flash diagnostics, clear DTCs, and conduct functional tests to validate system operation. - Record updated details (firmware version, date/time, tool used, operator details), capture logs, and store records in a traceable system. |
| Calibrate Accelerator Pedal Position Sensor (APPS) | - Collect vehicle details, VIN, APPS specifications, and OEM calibration procedures. - Review technical documentation to identify calibration parameters, voltage ranges, and acceptance limits. - Confirm availability and calibration status of diagnostic scanner and measurement tools. - Power down the vehicle if required and prepare the system for calibration mode. - Apply safety precautions and wear appropriate PPE as per workshop standards. - Connect the diagnostic scanner to the vehicle and establish communication with the control unit. - Inspect the accelerator pedal assembly and sensor for physical damage, loose connections, or contamination. - Turn ignition ON (without starting the vehicle) as per OEM procedure. - Access APPS calibration or relearn function through the diagnostic tool. - Perform initial (zero/throttle closed) position calibration following OEM instructions. - Depress the accelerator pedal gradually to full position and perform full-scale calibration. - Monitor real-time sensor signals (voltage/percentage) for smooth and linear response. - Compare measured values with OEM specifications and confirm calibration within tolerance. - Perform functional test by operating the pedal and verifying correct system response. - Record calibration data (values, tool used, operator details), capture diagnostic logs if required, and store records in a traceable system. |
| Perform zero-point calibration of steering angle sensor (SAS) | - Collect vehicle details, VIN, steering system specifications, and OEM SAS calibration procedures. - Review technical documentation to identify zero-point calibration requirements and acceptance criteria. - Confirm availability and calibration status of diagnostic scanner and required tools. - Position the vehicle on a level surface with wheels aligned straight ahead. - Power down the system if required and prepare the vehicle for calibration mode. - Apply safety precautions and wear appropriate PPE as per workshop standards. - Connect the diagnostic scanner to the vehicle and establish communication with the control module. - Inspect steering components and sensor connections for damage, misalignment, or faults. - Turn ignition ON (engine OFF) as per OEM procedure. - Access steering angle sensor (SAS) calibration or zero-point reset function via the diagnostic tool. - Execute zero-point calibration while maintaining the steering wheel in the straight-ahead position. - Monitor calibration process and confirm successful completion through the diagnostic tool. - Confirm sensor output values (angle reading near zero) and compare with OEM specifications. - Perform practical checks by turning the steering wheel left and right to confirm accurate response. - Record calibration results (values, tool used, operator details), capture diagnostic logs if required, and store records in a traceable system. |
| Perform BMS State-of-Health (SOH) recalibration | - Collect vehicle details, VIN, battery specifications, and OEM BMS SOH recalibration procedures. - Review technical documentation to identify recalibration method, required conditions, and acceptance criteria. - Confirm availability and calibration status of diagnostic tools, battery cycler and data logging equipment. - Connect the vehicle to a stable power supply or charging system as specified by OEM. - Apply safety precautions and wear appropriate PPE for high-voltage systems. - Connect the diagnostic scanner to the vehicle and establish communication with the Battery Management System (BMS). - Retrieve and record initial battery data including SOH, SoC, voltage, current, temperature, and DTCs. - Inspect battery system status and confirm no active faults that may affect recalibration. - Initiate BMS SOH recalibration function or guided routine through the diagnostic tool. - Perform controlled charge and/or discharge cycle as required by OEM procedure. - Monitor real-time battery parameters during the recalibration process for stability and safety. - Allow the BMS to relearn capacity and update SOH values based on measured data. - Confirm updated SOH value and compare with expected range or OEM specifications. - Perform post-recalibration checks, clear DTCs if necessary, and confirm normal system operation. - Record recalibration details (initial/final SOH, parameters, tool used, operator details), capture logs, and store records in a traceable system. |
| Synchronize motor rotor position sensor (Resolver) | - Collect vehicle details, VIN, motor specifications, and OEM resolver synchronization procedures. - Review technical documentation to identify resolver calibration method, reference angles, and acceptance criteria. - Confirm availability and calibration status of diagnostic tools, oscilloscope (if required), and interface equipment. - Connect the vehicle to a stable power supply or auxiliary support unit as specified by OEM. - Power down the system where required and prepare the vehicle for safe calibration mode. - Apply safety precautions and wear appropriate PPE for high-voltage systems. - Connect the diagnostic tool to the vehicle and establish communication with the motor control unit (MCU/inverter). - Inspect resolver wiring, connectors, and shielding for damage, looseness, or interference. - Check for existing fault codes related to resolver or motor position and record them. - Clear relevant DTCs if required and confirm system readiness for synchronization. - Access resolver calibration or synchronization function through the diagnostic tool. - Align the motor rotor to the reference position as specified by OEM procedure. - Initiate resolver offset learning or synchronization routine via the control unit. - Monitor resolver signals (sine/cosine outputs) and verify correct phase alignment and signal integrity. - Confirm calculated rotor position matches expected reference within OEM tolerance limits. - Perform functional test by rotating the motor (or simulation) to verify accurate position tracking. - Confirm system performance, confirm no fault codes are present, and verify stable operation. - Record synchronization data (offset values, tool used, operator details), capture diagnostic logs, and store records in a traceable system. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Electric motor fundamentals, including rotor position sensing and control strategies.

- Resolver operation, including sine/cosine signal generation and phase relationship.

- Motor control systems, including Motor Control Unit (MCU) or inverter functionality.

- Calibration principles, including offset learning and zero/reference alignment.

- Diagnostic tools, communication protocols, and data interpretation.

- Electrical signal analysis using tools such as multimeters and oscilloscopes.

- Techniques of HV safety practices, including PPE usage and system isolation procedures.

- Fault diagnosis, including identification and interpretation of DTCs.

- System tolerances, accuracy requirements, and performance validation methods.

- Functional testing Techniques for motor operation and response verification.

- Documentation practices, traceability requirements, and compliance standards.

- Data logging, reporting systems, and audit requirements.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Completed calibration/assessment forms with resolver synchronization details.

- Recorded diagnostic data including initial and final resolver values and offsets.

- Evidence of correct use of diagnostic tools and test equipment.

- Diagnostic logs showing communication with the motor control unit and calibration execution.

- Recorded sine/cosine signal observations or oscilloscope readings

- DTC identification, recording, and clearance.

- Practical test results confirming accurate motor position tracking.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | OEM Diagnostic Scanner | 5 |
| 2 | Advanced Scan Tool (Multi-brand) | 2 |
| 3 | VCU Flashing/Programming Tool | 3 |
| 4 | Laptop/Tablet with OEM Software | 5 |
| 5 | OBD-II Interface Cables | 5 |
| 6 | Data Logging Tool/Software | 3 |
| 7 | Digital Multimeter (HV Rated) | 5 |
| 8 | Oscilloscope (2/4 Channel) | 2 |
| 9 | Clamp Meter (DC/AC) | 3 |
| 10 | Insulation Resistance Tester (Megger) | 2 |
| 11 | Test Leads & Probes (Insulated) | 5 sets |
| 12 | Battery Support Unit (Stable Power Supply) | 3 |
| 13 | EV Charger / Charging Interface | 2 |
| 14 | DC Power Supply (Programmable) | 2 |
| 15 | Extension Cables (Industrial Grade) | 5 |
| 16 | Battery Cycler (Charge/Discharge Unit) | 2 |
| 17 | Thermal Monitoring Device (IR Thermometer) | 2 |
| 18 | Temperature Sensors/Probes | 5 |
| 19 | HV Battery Interface Kit | 2 |
| 20 | Resolver Interface/Simulator | 1 |
| 21 | Signal Analyzer (if separate) | 1 |
| 22 | Motor Test Bench / Training Setup | 1 |
| 23 | Insulated Tool Kit (Spanners, Screwdrivers) | 5 sets |
| 24 | Torque Wrench (Insulated) | 3 |
| 25 | Wire Strippers & Cutters | 5 |
| 26 | Connector Removal Tools | 3 sets |
| 27 | HV Insulated Gloves (Class Rated) | 10 pairs |
| 28 | Leather Protector Gloves | 10 pairs |
| 29 | Safety Goggles / Face Shields | 5 |
| 30 | Flame-Resistant Clothing | 5 sets |
| 31 | Safety Shoes (Insulated) | 5 pairs |
| 32 | Arc Flash Protection Kit | 2 sets |
| 33 | Lock-Out/Tag-Out (LOTO) Kit | 3 sets |
| 34 | Insulated Safety Mats | 5 |
| 35 | Fire Extinguisher (Class ABC + Li-ion compatible) | 3 |
| 36 | First Aid Kit | 2 |
| 37 | Emergency Disconnect Tool | 2 |
| 38 | HV Warning Signs & Barricades | 5 sets |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Electrical Contact Cleaner | 3 cans |
| 2 | Isopropyl Alcohol | 2 bottles |
| 3 | Dielectric Grease | 2 tubes |
| 4 | Cleaning Wipes / Lint-Free Cloth | Bulk |
| 5 | Cable Ties & Harness Clips | Bulk |
| 6 | Clipboards / Tablets | 5 |
| 7 | Camera / Mobile Device | 3 |
| 8 | Printer/Scanner | 1 |
| 9 | Tool Calibration Certificates | All tools |
| 10 | Software License & Update Access | Valid |
| 11 | Firmware Repository Access (OEM) | 1 |
| 12 | Digital Record/PLM System Access | 1 |

## Perform HV Component Sealing & Ingress Protection (IP) Rating Validation

Overview: This competency standard covers the skills and knowledge required to Perform vacuum decay test on battery enclosure, Perform vacuum decay test on e-drive system, Inspect high-voltage connector O-rings for degradation and Apply specialized mastic sealant to battery tray

| Competency Units | Performance Criteria |
| --- | --- |
| Perform vacuum decay test on battery enclosure | - Collect vehicle details, battery enclosure specifications, and OEM vacuum decay test procedures. - Review technical documentation to identify test pressure limits, decay thresholds, duration, and acceptance criteria. - Confirm availability and calibration status of vacuum decay tester, pressure sensors, and data acquisition system. - Inspect test setup components including hoses, fittings, seals, and adapters for integrity and compatibility. - Power down the vehicle, isolate the HV battery system, and secure the work area. - Apply safety precautions and wear appropriate PPE for high-voltage and pressure testing. - Clean the battery enclosure surface and sealing interfaces to remove contaminants. - Inspect enclosure, gaskets, and sealing areas for visible damage, deformation, or leakage paths. - Seal all openings and connect the vacuum decay test equipment to the designated test port. - Evacuate air from the enclosure to reach the specified vacuum level as per OEM requirements. - Stabilize the system for the defined settling time to eliminate transient pressure variations. - Isolate the enclosure from the vacuum source and initiate the decay measurement phase. - Monitor and record pressure change over the specified test duration using calibrated sensors. - Calculate vacuum decay rate and compare with defined leak rate or pressure loss limits. - Identify potential leakage if pressure decay exceeds acceptable thresholds. - Perform secondary inspection or leak localization (if required) using approved methods. - Regulate Pass/Fail status based on OEM specifications and document any abnormalities. - Record complete test data (initial/final pressure, decay rate, duration, equipment used, operator details), capture logs/graphs, and store records in a traceable system |
| Perform vacuum decay test on e-drive system | - Collect vehicle details, e-drive system specifications, and OEM vacuum decay test procedures. - Review technical documentation to identify test pressure limits, decay thresholds, duration, and acceptance criteria. - Confirm availability and calibration status of vacuum decay tester, pressure sensors, and data logging system. - Inspect test equipment, hoses, fittings, and adapters for integrity and compatibility with the e-drive system. - Power down the vehicle, isolate the HV system, and secure the work area. - Apply safety precautions and wear appropriate PPE for high-voltage and pressure testing. - Clean external surfaces and sealing interfaces of the e-drive system to remove dirt, oil, or contaminants. - Inspect housing, seals, gaskets, and connectors for visible damage, wear, or potential leakage points. - Seal all openings and connect the vacuum decay tester to the designated test port of the e-drive system. - Evacuate air from the system to achieve the specified vacuum level as per OEM requirements. - Stabilize the system for the defined settling time to eliminate transient pressure fluctuations. - Isolate the system from the vacuum source and initiate the decay measurement phase. - Monitor and record pressure variation over the specified test duration using calibrated instruments. - Calculate vacuum decay rate and compare results with OEM-defined leakage limits. - Identify and document any excessive pressure loss indicating potential leakage. - Perform secondary inspection or leak localization procedures if leakage is detected. - Regulate Pass/Fail status based on comparison with OEM specifications and standards. - Record complete test data (initial/final pressure, decay rate, duration, equipment used, operator details), capture logs/graphs, and store records in a traceable system. |
| Inspect high-voltage connector O-rings for degradation | - Identify the high-voltage connector location and verify system isolation before inspection. - Disconnect power sources and confirm zero voltage using appropriate testing equipment. - Wear required personal protective equipment (PPE) such as insulated gloves and safety goggles. - Access the connector housing carefully to avoid damaging surrounding components. - Remove the connector following manufacturer-recommended procedures. - Locate the O-rings within the connector assembly. - Clean the O-rings and surrounding surfaces using approved, non-reactive cleaning agents. - Inspect the O-rings for cracks, cuts, or surface irregularities. - Check for signs of hardening, brittleness, or loss of elasticity. - Inspect the O-rings for swelling, deformation, or compression set. - Inspect for chemical degradation such as discoloration or residue buildup. - Measure O-ring dimensions and compare them with manufacturer specifications. - Confirm proper seating and alignment of O-rings within their grooves. - Assess environmental factors such as heat exposure, moisture, or contamination. - Adjust whether O-rings meet operational standards or require replacement. - Replace degraded O-rings with approved, specification-compliant components if necessary. - Reassemble the connector and ensure proper sealing and secure fitment. - Document findings, actions taken, and final condition in the assessment report. |
| Apply specialized mastic sealant to battery tray | - Review manufacturer specifications and safety guidelines for applying mastic sealant to the battery tray. - Isolate the battery system and disconnect all power sources before starting the task. - Wear personal protective equipment (PPE) including gloves, goggles, and protective clothing. - Inspect the battery tray for dirt, corrosion, moisture, or existing sealant residue. - Clean the battery tray surface using approved cleaning agents to ensure proper adhesion. - Remove old or degraded sealant completely using suitable tools without damaging the tray. - Dry the battery tray thoroughly to eliminate any moisture. - Inspect the tray for cracks, deformation, or structural damage prior to sealant application. - Select the correct specialized mastic sealant as per manufacturer recommendations. - Prepare the sealant according to product instructions (mixing or conditioning if required). - Load the sealant into an appropriate application tool such as a caulking gun. - Apply the mastic sealant evenly along designated seams and joints of the battery tray. - Confirm continuous and uniform bead formation to avoid gaps or air pockets. - Smooth and press the sealant into place to enhance adhesion and sealing effectiveness. - Inspect the applied sealant for consistency, coverage, and proper thickness. - Allow the sealant to cure as per specified curing time and environmental conditions. - Reassemble any removed components and reconnect the battery system safely. - Document the application process, materials used, curing time, and final inspection results in the assessment report. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

  - Principles of vacuum decay testing and its application in leak detection for battery enclosures and e-drive systems.

  - High-voltage (HV) system architecture, components, and associated hazards.

  - OEM technical documentation, specifications, and test procedures accurately.

  - Test parameters including pressure limits, decay thresholds, duration, and acceptance criteria.

  - Function, operation, and calibration requirements of vacuum decay testers, pressure sensors, and data acquisition systems.

  - Inspection techniques for connectors, O-rings, gaskets, seals, and enclosure housings.

  - Common failure modes such as leakage paths, material degradation, deformation, and contamination.

  - Properties and application techniques of specialized mastic sealants used in battery systems.

  - Surface preparation requirements for sealing and adhesion effectiveness.

  - Personal protective equipment (PPE) requirements for HV and pressure testing environments.

  - Safety measures for isolating HV systems and securing the work area.

  - Pressure variation data and calculate vacuum decay rates.

  - Environmental and operational factors affecting sealing performance (temperature, moisture, vibration).

  - Leak detection and localization techniques used in secondary inspections.

  - Criteria for determining pass/fail results based on OEM standards.

  - Proper handling, storage, and replacement of O-rings and sealing components.

  - Curing processes and conditions for sealants and their impact on performance.

  - Documentation practices, traceability, and reporting requirements in testing and inspection processes.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Completed vacuum decay test reports including initial and final pressure readings, decay rates, and test duration.

- Documented inspection records of battery enclosures, e-drive systems, connectors, and O-rings.

- Evidence of correct setup and calibration verification of testing equipment.

- Recorded data logs and graphical outputs from the data acquisition system.

- Reports identifying detected leaks and actions taken for rectification.

- Documentation of sealant application process including materials used and curing conditions.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | Vacuum Decay Tester | 5 |
| 2 | Calibrated Pressure Sensors | 10 |
| 3 | Data Acquisition / Logging System | 5 |
| 4 | Digital Multimeter (HV Rated) | 5 |
| 5 | Insulation Resistance Tester (Megger) | 5 |
| 6 | Vacuum Pump | 5 |
| 7 | Pressure Gauge (Digital/Analog) | 5 |
| 8 | Leak Detection Kit (Soap/Spray Type) | 5 |
| 9 | Insulated Tool Kit (Spanners, Screwdrivers) | 5 sets |
| 10 | Torque Wrench (Insulated) | 5 |
| 11 | O-ring Removal Tools (Plastic Picks) | 5 sets |
| 12 | Sealant Application Gun (Caulking Gun) | 5 |
| 13 | Cleaning Brushes (Soft & Hard) | 10 |
| 14 | Scraper/Sealant Removal Tool | 5 |
| 15 | Measuring Vernier Caliper | 5 |
| 16 | Flashlight/Inspection Lamp | 5 |
| 17 | Vacuum Hoses (HV Compatible) | 10 |
| 18 | Quick Connect Fittings & Adapters | 5 sets |
| 19 | Sealing Plugs/Caps for Ports | 5 sets |
| 20 | Test Port Connectors | 5 sets |
| 21 | Workstation Table with Insulated Surface | 5 |
| 22 | Component Holding Fixture | 5 |
| 23 | Insulated Gloves (Class Rated) | 10 pairs |
| 24 | Safety Goggles | 10 |
| 25 | Face Shield | 5 |
| 26 | Flame-Resistant Clothing / Lab Coat | 10 |
| 27 | Safety Shoes (ESD/Insulated) | 10 |
| 28 | Rubber Insulation Mats | 5 |
| 29 | Ear Protection (if required) | 5 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Specialized Mastic Sealant | 10 tubes |
| 2 | Replacement O-Rings (Assorted Kit) | 5 sets |
| 3 | Cleaning Solvent (Non-reactive) | 5 |
| 4 | Isopropyl Alcohol Wipes | 5 |
| 5 | Lint-Free Cloths | 50 pcs |
| 6 | Sealant Remover Chemical | 5 |
| 7 | Leak Detection Spray | 5 |
| 8 | Marking Chalk / Marker | 5 |
| 9 | Fire Extinguisher (Class C/Electrical) | 5 |
| 10 | First Aid Kit | 5 |
| 11 | Emergency Stop Switch Access | 5 |
| 12 | Warning Signage (HV Area) | 10 |
| 13 | Lockout/Tagout (LOTO) Kit | 5 |
| 14 | Spill Kit (Chemical/Sealant) | 5 |
| 15 | Test Report Sheets / Logbooks | 5 sets |
| 16 | Calibration Certificates (Copies) | 5 sets |
| 17 | OEM Technical Manuals | 5 |
| 18 | Clipboard & Pen | 5 |
| 19 | Computer/Laptop for Data Logging | 5 |
| 20 | Waste Disposal Bins (Hazardous/Non-Hazardous) | 10 |
| 21 | Cable Management Ties | 5 |
| 22 | Labels & Tags | 5 |
| 23 | Timer/Stopwatch | 5 |

## Maintain Regenerative Braking System

Overview: This competency standard develops the knowledge and skills required to Calibrate Electronic Brake Pedal Position Sensor (BPPS), Evaluate Blending Efficiency Between Friction and Regen, Test Motor-Generator Torque Response During Coasting, Inspect Regenerative Braking Cooling requirements and Update ABS/ESC Module for Optimized Energy Recovery

| Competency Units | Performance Criteria |
| --- | --- |
| Calibrate Electronic Brake Pedal Position Sensor (BPPS) | - Connect the OEM diagnostic platform to the vehicle CAN interface and navigate to the BPPS calibration function. - Check brake pedal is at the fully released rest position and confirm zero-point reading matches OEM specification before initializing calibration. - Apply a series of pedal inputs at OEM-defined incremental positions and record the corresponding sensor output voltage or signal values. - Compare recorded BPPS output values against the OEM calibration curve and confirm all data points fall within the specified tolerance band. - Perform a full-stroke pedal test and verify the BPPS signal transitions smoothly from zero to maximum without dropout, noise, or non-linearity. - Clear all BPPS-related DTCs and confirm zero active codes on a final diagnostic scan. - Document calibration parameter values and sensor ID on the job card and obtain assessor sign-off. |
| Evaluate Blending Efficiency Between Friction and Regen | - Retrieve and record all active and stored DTCs related to the brake blending control module using the OEM diagnostic platform. - Access the live data stream and monitor regen torque demand, friction brake pressure, and vehicle deceleration simultaneously during a controlled low-speed stop. - Analyze the regen-to-friction brake contribution ratio from the captured data and compare it against the OEM-specified blending efficiency target. - Conduct a graduated brake application test at three speed thresholds defined by OEM and verify the blend transition point occurs within the OEM-specified speed window. - Conduct a graduated brake application test at three speed thresholds defined by OEM and verify the blend transition point occurs within the OEM-specified speed window. - Identify any blending inefficiency from the test data and document the root cause and corrective action on the fault analysis form. - Clear resolved DTCs and record blending efficiency test results on the completed job card. |
| Test Motor-Generator Torque Response During Coasting | - Configure the OEM diagnostic platform to monitor motor-generator torque output, rotational speed (RPM), and DC bus voltage simultaneously in real time. - Conduct a coasting test by releasing the accelerator at OEM-specified vehicle speed and record the initial torque response of the motor-generator. - Measure the torque rise time from zero to OEM-specified peak regen torque and confirm it is within the OEM-defined response time tolerance. - Confirm DC bus voltage rises within the OEM-acceptable range during the coasting regen event without triggering overvoltage protection. - Repeat coasting test at two additional speed thresholds specified by OEM and record torque response data for each. - Compare all recorded torque response data against OEM performance benchmarks and identify any deviation outside tolerance. - Document test findings and any identified deviations on the job card and present results to the assessor. |
| Inspect Regenerative Braking Cooling requirements | - Retrieve all active DTCs related to the motor-generator and power electronics cooling circuit using the OEM diagnostic platform. - Inspect cooling circuit hoses, fittings, radiator, and pump for leakage, damage, or blockage. - Check coolant level and condition against OEM specification and record the result - Monitor motor-generator coolant inlet and outlet temperatures using the diagnostic platform during a simulated regen load event and confirm the delta-T is within OEM limits. - Check coolant pump operation by confirming OEM-specified flow rate or pump duty cycle is achieved at regen load conditions. - Inspect power electronics (inverter and motor controller) heatsink for contamination, corrosion, or fin damage. - Document all cooling system inspection findings and recommended maintenance actions on the job card. |
| Update ABS/ESC Module for Optimized Energy Recovery | - Confirm ABS/ESC module hardware version and software version compatibility with the target calibration file using OEM fitment data. - Connect a battery support unit and verify stable supply voltage (≥12.5 V) before initiating any module programming or calibration write operation. - Create a DoIP or CAN programming connection to the ABS/ESC module and initiate the OEM-specified software update sequence without interrupting the cycle. - Perform a wheel speed sensor plausibility check by rolling the vehicle at low speed and confirming all four-wheel speed signals are present and within tolerance on the diagnostic platform. - Conduct a controlled ABS stop and verify regen is correctly cut off at the OEM-defined wheel slip threshold, with friction brakes assuming full authority. - Clear all ABS/ESC-related DTCs, confirm zero active codes on a final scan, and document all programmed parameter values on the job card before assessor sign-off. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- OHS precautions and high-voltage safety procedures for brake and HV-adjacent systems, including correct PPE selection and use.

- Principles of regenerative braking: motor-generator operation in generator mode, torque production, back-EMF, and energy recovery to the HV battery.

- BPPS operating principles (Hall-effect, resistive, potentiometer types), signal characteristics (voltage range, linearity), and CAN bus communication with the brake blending module and motor controller.

- OEM calibration, zero-point setting, incremental pedal position mapping, calibration curve interpretation, and full-stroke linearity verification.

- Brake blending architecture: blending control module, hydraulic actuator (e-booster or vacuum servo), regen-to-friction contribution ratio, and OEM blending maps.

- Live data monitoring parameters: regen torque demand, wheel brake cylinder pressure, vehicle deceleration, motor-generator RPM, and DC bus voltage.

- DC bus voltage behavior during regens: acceptable rise range, overvoltage protection thresholds, and BMS interaction.

- Cooling circuit architecture: coolant pump, radiator/heat exchanger, motor-generator loop, delta-T measurement, and power electronics heatsink inspection.

- ABS/ESC system architecture in EVs: wheel speed sensors, hydraulic control unit, regen torque limit maps, wheel slip thresholds, and ESC regen cut-off strategies.

- Module programming interfaces: DoIP/Ethernet and CAN, session management, battery voltage requirements, calibration parameter writing, and readback verification.

- DTC interpretation: fault codes for BPPS, brake blending, cooling, and ABS/ESC systems; root cause analysis methodology.

- ISO 26262 functional safety awareness for safety-critical braking software updates; OEM service manuals, DTC reference guides, and job card completion.

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be assessed as competent in this competency standard:

- Follow OHS and high-voltage safety procedures and use correct PPE throughout all practical tasks.

- Calibrate the BPPS, confirm signal linearity across the full pedal stroke, and verify zero active codes on a final scan.

- Capture live regen and friction brake blending data, calculate the contribution ratio, and confirm blend transition against OEM specification.

- Record motor-generator torque response during coasting at OEM-defined speeds and compare results against OEM performance benchmarks.

- Inspect the regen braking cooling circuit, measure coolant delta-T under regen load, and document findings on the job card.

- Program the ABS/ESC module with OEM regen recovery parameters, validate wheel speed sensor plausibility, and confirm regen cut-off during an ABS event.

- Clear all DTCs, complete job cards with calibration and test parameter values, and obtain assessor sign-off before returning the vehicle to service.

List of Tools and Equipment

| S.# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | Safety helmet | 25 |
| 2 | Safety goggles / face shield | 25 |
| 3 | Safety shoes (anti-static) | 25 |
| 4 | High-voltage insulated gloves (Class 0, IEC 60903) | 25 pairs |
| 5 | Arc-flash rated protective clothing / coveralls | 25 |
| 6 | Ear protection plugs | 25 pairs |
| 7 | Fire extinguisher (Class C – electrical) | 5 |
| 8 | Smoke / CO detecting alarm | 3 |
| 9 | First aid box (fully stocked) | 3 |
| 10 | Lock-out / Tag-out (LOTO) kit | 13 |
| 11 | Health and Safety Manual (EV / HV braking systems) | 5 |
| 12 | OEM-approved EV/HEV diagnostic platform (e.g. AUTEL MaxiSYS or equivalent) | 5 |
| 13 | Laptop with OEM calibration / programming software installed | 5 |
| 14 | DoIP / Ethernet vehicle interface adapter | 5 |
| 15 | CAN bus interface / analyzer (e.g. PCAN-USB or equivalent) | 5 |
| 16 | Battery support unit / intelligent charger (13.5–14.4 V stable output) | 5 |
| 17 | OEM ABS/ESC programming software license (network seat) | 5 seats |
| 18 | Digital multimeter – CAT III/IV, True RMS | 13 |
| 19 | Clamp meter (AC/DC current) | 5 |
| 20 | Dual-channel oscilloscope (≥100 MHz bandwidth) | 5 |
| 21 | High-voltage probe set (for safe HV DC bus measurement) | 5 |
| 22 | Calibrated brake pedal force gauge / pedal effort meter | 5 |
| 23 | Brake pressure gauge / hydraulic test adapter | 5 |
| 24 | Coolant refractometer / condition test kit | 5 |
| 25 | Calibrated thermometer / temperature probe (for delta-T measurement) | 5 |
| 26 | Thermal imaging camera | 2 |
| 27 | Wheel speed sensor breakout / test adapter set | 5 |
| 28 | Insulated screwdriver set (flat & Phillips, VDE rated) | 13 |
| 29 | Insulated spanner / wrench set (metric) | 13 |
| 30 | Calibrated insulated torque wrench set (5–50 Nm) | 5 |
| 31 | Insulated pliers set (needle nose, combination, cutting) | 13 |
| 32 | Brake bleeding kit (vacuum / pressure type) | 5 |
| 33 | Coolant extractor / vacuum filler kit | 5 |
| 34 | Connector pick / pin removal tool set | 13 |
| 35 | Anti-static wrist strap and mat | 13 |
| 36 | Vehicle wheel chocks | 10 pairs |
| 37 | Vehicle lift / hoist (2-post or scissor) | 5 |
| 38 | Brake Pedal Position Sensor (BPPS) – OEM replacement unit | 5 |
| 39 | ABS/ESC hydraulic control unit (HCU) – training unit | 3 |
| 40 | Motor-generator training module / bench simulator | 3 |
| 41 | Regen braking wiring harness simulator / breakout box | 5 |
| 42 | EV/HEV training vehicle with regen braking system (ABS, ESC, regen blending) | 5 |
| 43 | OEM service manual – regenerative braking system | 5 |
| 44 | OEM DTC reference guide (ABS / ESC / regen braking) | 5 |
| 45 | OEM wiring diagram set (regen braking circuits) | 5 |
| 46 | Job card / inspection report templates (printed sets) | 50 sets |
| 47 | Fault analysis form templates (printed sets) | 50 sets |
| 48 | Calibration and test data log sheets | 25 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Brake fluid (DOT 4) | 5 liters |
| 2 | Coolant | 6 liters |
| 3 | Electrical contact cleaner spray | Assorted |
| 4 | Insulation Tape (HV Rated) | 5 rolls |
| 5 | Cable Marking Labels/Tags | Bulk |
| 6 | Cleaning Cloths/Wipes | Bulk |
| 8 | Printed job cards | 3 cans |
| 9 | Wire Brushes (Soft) | 5 |
| 10 | Inspection/Assessment Forms | As required |
| 12 | Fault analysis forms | 3 |
| 13 | Connector cleaning pins / pin removal kit | 20 |
| 14 | Lint-Free Cloth | Bulk |

## Test Insulation Resistance (Megohmmeter)

Overview: This competency standard covers the skills and knowledge required to Calibrate Insulation Tester for High-Voltage Diagnostics, Test internal insulation of HV battery pack, Measure Motor Stator Winding-To-Case Resistance, Conduct Chassis-To-HV-Component Isolation Check and Evaluate High-Voltage Cable Shielding Integrity

| Competency Units | Performance Criteria |
| --- | --- |
| Calibrate Insulation Tester for High-Voltage Diagnostics | - Select appropriate insulation tester for EV high-voltage system - Inspect insulation tester for physical and functional condition - Set voltage range on megohmmeter according to system requirement - Confirm calibration status of insulation tester before use - Record calibration checks results in test log |
| Test internal insulation of HV battery pack | - Isolate HV battery pack from vehicle system - Connect insulation tester to battery terminals correctly - Apply test voltage to measure internal insulation resistance - Record insulation resistance values from battery pack - Compare results with acceptable safety limits |
| Measure Motor Stator Winding-To-Case Resistance | - Isolate electric traction motor from power source - Connect megohmmeter between stator windings and motor casing - Confirm specified test voltage for insulation measurement - Measure and record winding-to-case resistance values - Evaluate insulation condition based on measured results |
| Conduct Chassis-To-HV-Component Isolation Check | - Disconnect HV system from vehicle chassis ground - Connect insulation tester between chassis and HV components - Apply test voltage to detect leakage paths - Measure insulation resistance between chassis and HV system - Record and assess isolation integrity results |
| Evaluate High-Voltage Cable Shielding Integrity | - Inspect HV cables for physical damage and wear - Connect insulation tester between cable shield and conductor - Apply test voltage for shielding integrity check - Measure insulation resistance of cable shielding system - Record results and determine cable safety condition |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Purpose of insulation resistance testing in high-voltage EV systems

- Working principle of a megohmmeter in HV diagnostics

- Safety precautions for insulation resistance testing on EV systems

- Calibration and functional checking of insulation testers before use

- Selection of test voltage for different EV high-voltage components

- Testing procedures for HV battery pack insulation resistance

- Measurement of motor stator winding-to-case insulation resistance

- Chassis-to-HV system isolation testing procedure

- Evaluation of HV cable shielding integrity

- Acceptable insulation resistance values for EV components

- Effects of moisture, contamination, and insulation damage on readings

- Safe isolation procedures prior to conducting HV tests

- Documentation requirements for insulation resistance test results

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Calibrate insulation tester before high-voltage diagnostics

- Verify insulation tester functionality prior to use

- Select appropriate test voltage for EV high-voltage components

- Isolate high-voltage battery pack safely from vehicle system

- Measure insulation resistance of HV battery pack accurately

- Measure motor stator winding-to-case insulation resistance

- Conduct chassis-to-HV system isolation test correctly

- Evaluate high-voltage cable shielding integrity using insulation tester

- Record insulation resistance values for all tested components

- Compare measured values with acceptable safety limits

- Identify insulation faults based on test results

- Apply safety procedures during all insulation resistance tests

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | Insulation resistance tester (Megohmmeter) | 5 units |
| 2 | High-voltage insulated gloves | 25 pairs |
| 3 | Safety goggles and face shield | 25 sets |
| 4 | Lockout Tagout (LOTO) kit | 5 sets |
| 5 | HV warning signs and barricade tape | 10 sets |
| 6 | Digital multimeter | 10 units |
| 7 | HV battery isolation tools set | 5 sets |
| 8 | Insulated hand tools set | 5 sets |
| 9 | High-voltage disconnect/service plugs | 5 sets |
| 10 | Torque wrench set | 5 sets |
| 11 | HV test probes and leads | 5 sets |
| 12 | Chassis grounding strap | 5 units |
| 13 | HV battery lifting trolley | 2 units |
| 14 | Emergency stop switch tester | 2 units |
| 15 | First aid kit (electrical shock response) | 2 units |
| 16 | Fire extinguisher (CO₂ / Class D) | 4 units |
| 17 | Insulation mat for EV work area | 5 units |
| 18 | Workshop service manual & wiring diagrams | 10 sets |
| 19 | Workshop communication system (walkie-talkie/intercom) | 5 units |
| 20 | HV test adapter cables set | 5 sets |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | High-voltage insulating tape | 20 rolls |
| 2 | Cable ties (assorted sizes) | 5 packs |
| 3 | Cleaning wipes for terminals and connectors | 10 packs |
| 4 | Isopropyl alcohol | 10 liters |
| 5 | Disposable nitrile gloves | 5 boxes |
| 6 | Marker pens for labeling | 25 pieces |
| 7 | HV warning labels / isolation tags | 100 pieces |
| 8 | Terminal protective caps and covers | 25 sets |
| 9 | Absorbent pads for moisture control | 10 packs |
| 10 | Printed test record sheets and log forms | 200 sheets |

## Maintain Technical Documentation & Compliance Reporting

Overview: This competency standard covers the skills and knowledge required to Generate a comprehensive High-Voltage (HV) Battery health report, Record isolation test results for safety compliance, Document firmware version history and update logs, Prepare a Root Cause Analysis (RCA) for HV component failures, Maintain workshop hazardous material (Battery) disposal log and Maintain record on repair order

| Competency Units | Performance Criteria |
| --- | --- |
| Generate a comprehensive High-Voltage (HV) Battery health report | - Collect vehicle details, previous maintenance records, and OEM specifications. - Dress Personal Protective Equipment (PPE) - Calibrate diagnostic tools - Inspect HV battery components - Record the inspection observations in assessment/ inspection form. - Capture photographic/Video evidence of any damage or abnormalities. - Connect diagnostic scanner to the vehicle for communication. - Retrieve and record all Diagnostic Trouble Codes (DTCs). - Record real-time battery parameters (SoC, SoH, voltage, current). - Conduct charge and discharge performance tests. - Record system behavior, responses, and any deviations during testing. - Test safety systems (HVIL, insulation, emergency shutdown). - Record measured values and indicate Pass/Fail status. - Compare results with OEM specifications and applicable standards. - Identify deviations, severity, and recommend corrective/preventive actions. - Review findings, complete overall assessment, sign/date the report, and store/share in digital compliance system. |
| Record isolation test results for safety compliance | - Check vehicle is powered down, ignition OFF, key removed, and secured. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear Personal Protective Equipment (PPE) during high-voltage operations. - Verify insulation tester (megohmmeter) is calibrated and certified for use. - Record vehicle and battery identification details (VIN, model, battery type). - Document ambient conditions (temperature, humidity) and test setup parameters. - Identify and record test equipment details (device name, model, calibration date, test voltage level). - Disconnect the HV battery system as per OEM Techniques for safe isolation. - Measure insulation resistance between, HV positive (+) and chassis ground & HV negative (–) and chassis ground - Apply specified test voltage and maintain for OEM-defined duration. - Record all measured values immediately and accurately. - Evaluate results against OEM specifications and valid safety standards; mark Pass/Fail. - Identify and document abnormalities (low resistance, fluctuating readings) and affected components. - Assign severity level and recommend corrective actions; attach supporting evidence (logs, photos, previous records). - Review compliance status, include inspector details and approvals, and store records in a controlled digital system with audit traceability. |
| Document firmware version history and update logs | - Collect system identification details (vehicle model, ECU/module name, VIN, software platform). - Retrieve current firmware version from the system using approved diagnostic tools. - Verify firmware authenticity and compatibility with OEM specifications. - Access historical firmware records from maintenance logs or digital systems. - Record previous firmware versions with corresponding release dates. - Document resolution of each firmware update (bug fix, performance improvement, security patch, feature update). - Record update source (OEM release, authorized service center, OTA update). - Document update method used (wired flashing, OTA, diagnostic tool-based update). - Record pre-update system condition (fault codes, system status, battery level). - Document update execution details (date, time, duration, technician/authority). - Record post-update firmware version and confirm successful installation. - Perform practical validation tests and record system performance after update. - Log any errors, failures, or rollback actions during or after update. - Compare firmware versions with OEM latest release and identify gaps or non-compliance. - Review update history, include approvals (signature/date), and store records in a controlled digital system with traceability. |
| Prepare a Root Cause Analysis (RCA) for HV component failures | - Collect system and vehicle details (vehicle model, VIN, HV component name, operating conditions). - Follow safety precautions (PPE, HV isolation, Lock-Out/Tag-Out procedures) before inspection. - Identify and document the problem statement clearly (failure type, symptoms, occurrence conditions). - Gather initial evidence including fault codes (DTCs), warning indicators, and system logs. - Review historical data (maintenance records, previous failures, firmware updates, usage patterns). - inspect HV components (battery, cables, connectors, insulation, cooling system). - Conduct diagnostic tests (voltage, current, insulation resistance, thermal readings). - Record all observations, measured values, and deviations from OEM specifications. - Identify causes with designed Techniques (Like 5 Whys, Fishbone/ Ishikawa analysis). - Analyze contributing factors (environmental, electrical, mechanical, software-related issues). - Isolate the root cause by validating suspected causes through testing or evidence correlation. - Assess severity and impact of the failure (safety risk, performance loss, system reliability). - Recommend corrective actions (repair, replacement, recalibration, software update). - Propose preventive actions to avoid recurrence (design improvement, maintenance changes, monitoring). - Document RCA findings, include supporting evidence (photos, logs), obtain approvals, and store report in a traceable system. |
| Maintain workshop hazardous material (Battery) disposal log | - Identify and categorize the battery as hazardous material (like lithium-ion HV battery, damaged/defective unit). - Collect and record battery details (type, capacity, serial number, vehicle VIN, condition). - Follow safety precautions (PPE, insulated tools, fire protection measures) are in place. - Isolate and store the battery in a designated hazardous material area as per safety guidelines. - Inspect the battery for damage, leakage, swelling, or thermal risk and document condition. - Label the battery clearly with hazard warnings and identification tags. - Record date of removal and reason for disposal (end-of-life, damage, failure, recall). - Document temporary storage conditions (location, temperature control, containment measures). - Select an authorized hazardous waste disposal/recycling vendor as per regulations. - Prepare required documentation (waste manifest, transport permits, compliance forms). - Record transportation details (date, transporter name, method, tracking reference). - Follow safe packaging and handling during dispatch (insulated, sealed, compliant with transport standards). - Obtain disposal/recycling confirmation certificate from authorized facility. - Record final disposal method (recycling, treatment, destruction) and compliance status. - Maintain complete disposal log with signatures, approvals, and store records in a traceable system for audits. |
| Maintain record on repair order | - Receive and register the vehicle with basic details (customer name, vehicle model, VIN, registration number). - Record customer complaint and service request clearly and accurately. - Assign a unique Repair Order (RO) number for traceability. - Document initial vehicle condition (mileage, battery level, visible damages). - Follow safety checks before work begins (HV isolation if required, PPE compliance). - Allocate job to technician and record technician details. - Perform diagnostic inspection and record findings (fault codes, test results). - Prepare detailed job description including required repairs, parts, and labor. - Obtain customer approval for estimated cost, scope of work, and timeline. - Record all repair activities carried out (step-by-step work performed). - Document parts used (part number, quantity, source, warranty details). - Conduct post-repair testing and quality checks; record results. - Update final job status, total cost, and completion time. - Obtain final customer acknowledgment/signature upon vehicle handover. - Store repair order record in digital/physical system with proper documentation for future reference and audits. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Principles of HV system, battery chemistry, and electrical hazards.

- Diagnostic methods, DTC interpretation, and testing procedures.

- Safety standards (LOTO, PPE, insulation resistance, fire protection).

- OEM specifications and international EV standards.

- Firmware versioning, update protocols, and cybersecurity considerations.

- RCA methodology, failure analysis, and preventive measures.

- Hazardous material handling, storage, and regulatory disposal procedures.

- Workshop operations, repair order management, and documentation practices.

- Environmental, regulatory, and compliance requirements for EV service.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Insulation test reports showing HV system compliance.

- Diagnostic scan results, including DTC retrieval and real-time parameter logs.

- Firmware version history and update logs with validation results.

- Root Cause Analysis (RCA) reports with supporting evidence, severity classification, and corrective actions.

- Hazardous battery disposal logs, waste manifests, and disposal certificates.

- Fully documented repair orders (ROs) including diagnostics, repairs, parts, and customer approvals.

- Demonstration of safe work practices including LOTO and proper PPE usage.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | OEM Diagnostic Scanner / OBD Tool | 5 |
| 2 | Laptop/Tablet with Software | 5 |
| 3 | Data Logging Tool | 3 |
| 4 | Firmware Programming Tool | 2 |
| 5 | Digital Multimeter (HV Rated) | 5 |
| 6 | Oscilloscope | 2 |
| 7 | Insulation Tester (Megohmmeter) | 3 |
| 8 | HV Test Leads/Probes | 5 sets |
| 9 | DC Clamp Meter | 5 |
| 10 | Non-contact Voltage Tester | 5 |
| 11 | Battery Load Tester | 2 |
| 12 | Insulated Hand Tools (1000V Rated) | 5 sets |
| 13 | Torque Wrench (Calibrated) | 3 |
| 14 | Socket & Ratchet Set | 5 sets |
| 15 | Cable Cutter/Crimping Tool | 3 |
| 16 | Trim Removal Tools | 3 sets |
| 17 | Battery Lift/Hoist | 2 |
| 18 | Insulated Rubber Gloves | 10 pairs |
| 19 | Leather Protector Gloves | 10 pairs |
| 20 | Safety Helmet with Face Shield | 5 |
| 21 | Safety Goggles / Arc Flash Visor | 5 |
| 22 | Flame-Resistant Clothing | 5 sets |
| 23 | Lock-Out/Tag-Out (LOTO) Kit | 3 sets |
| 24 | Insulated Safety Mat | 5 |
| 25 | Fire Extinguisher (Li-ion Compatible) | 3 |
| 26 | Spill Kit | 2 |
| 27 | First Aid Kit | 2 |
| 28 | Warning Signs / Barricades | 5 sets |
| 29 | HV Rescue Hook | 1 |
| 30 | Infrared Thermometer / Thermal Camera | 2 |
| 31 | Temperature & Humidity Meter | 2 |
| 32 | Pressure Gauge | 1 |
| 33 | Inspection Forms / Checklists | As required |
| 34 | Clipboard / Digital Tablet | 5 |
| 35 | Camera / Mobile Device | 3 |
| 36 | Printer/Scanner | 1 |
| 37 | Workshop Software / PLM Access | 1 system |
| 38 | OEM Firmware Access | 1 system |
| 39 | Secure Internet Connection | 1 |
| 40 | Data Backup Devices | 2 |
| 41 | Cybersecurity Tools | 1 |
| 42 | Safety Shoes (Insulated) | 5 pairs |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Cleaning Cloths / Wipes | Bulk |
| 2 | Contact Cleaner | 3 cans |
| 3 | Isopropyl Alcohol | 2 bottles |
| 4 | Thermal Paste | 2 tubes |
| 5 | Cable Ties | Bulk |
| 6 | Insulation Tape (HV Rated) | 5 rolls |
| 7 | Labels & Tags | Bulk |
| 8 | Fasteners (Nuts/Bolts) | Assorted kit |
| 9 | Battery Storage Containers | 2 |
| 10 | Insulated Packaging Material | Bulk |
| 11 | Absorbent Pads | 2 packs |
| 12 | Waste Manifest Forms | As required |
| 13 | Transport Documents | As required |
| 14 | Pallets / Containment Boxes | 2 |
| 15 | Straps / Securing Materials | 4 sets |
| 16 | Hazard Labels (UN Markings) | Bulk |
| 17 | Anti-static Bags | 10 |

## Perform HV Cabling & Connector Specialized Crimping

Overview: This competency standard covers the skills and knowledge required to Strip high-voltage orange insulation layers, Perform high-pressure hexagonal crimp, Perform terminal pull-test for mechanical strength, Apply adhesive-lined heat shrink tubing and Perform continuity Test of HV connector interlock (HVIL)

| Competency Units | Performance Criteria |
| --- | --- |
| Strip high-voltage orange insulation layers | - Collect vehicle details, HV cable specifications, and OEM stripping guidelines. - Review previous maintenance records and identify HV circuits to be worked on. - Verify availability and calibration of all required tools, PPE, and consumables. - Power down the vehicle, switch off ignition, and remove the key. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear appropriate PPE for HV systems (insulated gloves, sleeves, face shield, protective clothing). - Isolate the HV battery or system as per OEM procedures. - Prepare an insulated work surface and remove conductive objects from the area. - Inspect the HV cable for external damage or prior tampering. - Mark the cable section according to OEM stripping length specifications. - Strip the orange insulation layer using approved HV stripping tools without damaging the conductor. - Verify the exposed conductor length and remove any insulation residues. - Inspect the stripped cable for nicks, cuts, or uneven surfaces. - Record observations, cable ID, vehicle VIN, and operator details in the inspection form. - Capture photographs of the stripped cable sections as supporting evidence. |
| Perform high-pressure hexagonal crimp | - Collect vehicle details, cable specifications, terminal type, and OEM crimping requirements. - Review technical manuals to confirm correct crimping dimensions and standards. - Verify availability and calibration status of crimping tools, dies, and measuring instruments. - Power down the system, isolate the HV circuit, and remove ignition key. - Apply Lock-Out/Tag-Out (LOTO) procedures to prevent accidental energization. - Wear appropriate PPE including insulated gloves, eye protection, and protective clothing. - Inspect the cable and terminal lug for damage, corrosion, or contamination. - Strip the cable insulation to the specified length using approved stripping tools. - Clean the conductor strands and remove any debris or oxidation. - Insert the conductor fully into the hexagonal terminal lug as per specification. - Apply correct die size and position the terminal in the crimping tool. - Execute the high-pressure hexagonal crimp using the approved crimping tool. - Inspect the crimped joint for proper shape, uniform compression, and absence of defects. - Measure crimp dimensions and perform pull test (if required) to verify mechanical strength. - Record crimping details (cable ID, tool used, measurements, operator details), capture photographic evidence, and store records in a traceable system. |
| Perform terminal pull-test for mechanical strength | - Collect vehicle details, cable/terminal specifications, and OEM pull-test requirements. - Review technical standards to determine required pull force and acceptance criteria. - Verify availability and calibration status of pull-test equipment (tensile tester/force gauge). - Power down the system, isolate the HV circuit, and remove the ignition key. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear appropriate PPE including insulated gloves, eye protection, and protective clothing. - Inspect the crimped terminal and cable assembly for visible defects or damage. - Mount the cable and terminal securely in the pull-test fixture according to test setup guidelines. - Align the cable and terminal along the axis of the pulling force to avoid side loading. - Apply tensile force gradually using the pull-test equipment. - Increase the force up to the specified limit or until failure occurs as per test requirement. - Detect and record the maximum force applied and the mode of failure (if any). - Compare measured force with OEM specifications and determine Pass/Fail status. - Document any abnormalities such as slippage, conductor breakage, or improper crimping. - Record test results (force values, cable ID, tool used, operator details), capture photographic evidence, and store records in a traceable system. |
| Apply adhesive-lined heat shrink tubing | - Identify vehicle details, cable specifications, tubing size, and OEM requirements for heat shrink application. - Review technical guidelines to confirm correct tubing type (adhesive-lined), shrink ratio, and temperature range. - Arrange availability and condition of heat gun, temperature control device, and required consumables. - Power down the system, isolate the HV circuit, and remove the ignition key. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear appropriate PPE including heat-resistant gloves, eye protection, and protective clothing. - Inspect the cable and connection area for cleanliness, damage, or contamination. - Clean the cable surface using approved cleaning agents to remove dirt, oil, or moisture. - Select the correct size adhesive-lined heat shrink tubing based on cable diameter and specification. - Position the heat shrink tubing over the joint or exposed area with proper coverage length. - Apply controlled heat evenly using a heat gun while rotating the cable for uniform shrinkage. - Detect tubing contraction and adhesive flow to form a tight seal around the cable. - Allow the tubing to cool naturally without disturbance. - Inspect the finished application for uniform shrinkage, proper sealing, and absence of gaps or overheating marks. - Record application details (cable ID, tubing type, temperature used, operator details), capture photographic evidence, and store records in a traceable system. |
| Perform continuity Test of HV connector interlock (HVIL) | - Collect vehicle details, HVIL circuit specifications, and OEM testing requirements. - Review technical documentation to identify HVIL circuit path, connectors, and expected continuity values. - Arrange availability and calibration status of diagnostic tools and multimeter (continuity tester). - Power down the vehicle, switch off ignition, and remove the key. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear appropriate PPE including insulated gloves, eye protection, and protective clothing. - Isolate the HV system as per OEM Techniques before accessing connectors. - Locate HV connectors and identify HVIL pins using wiring diagrams. - Inspect connectors and wiring for damage, corrosion, or loose connections. - Set the multimeter to continuity or resistance measurement mode. - Connect test probes to HVIL circuit terminals across the connector. - Measure continuity and record resistance value or audible indication. - Operate connector lock/unlock mechanism to verify change in HVIL circuit status. - Compare measured results with OEM specifications and determine Pass/Fail status. - Record test results (connector ID, readings, operator details), capture evidence if required, and store records in a traceable system |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- HV system hazards and application of safe isolation practices including LOTO.

- HV cable construction, insulation, connectors, and HVIL systems.

- Knowledge of OEM specifications for cable stripping, tolerances, and conductor integrity.

- Crimping principles, tool/die selection, and quality requirements.

- Pull-test Techniques, force limits, and failure mode interpretation.

- Heat shrinks tubing types, application methods, and sealing requirements.

- Electrical testing principles including continuity and resistance measurement.

- Inspection techniques and application of Pass/Fail criteria.

- Documentation practices, traceability requirements, and reporting systems.

- Safety compliance including PPE usage and hazard control.

- Data recording, storage, and audit requirements.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Proper PPE usage and application of LOTO Techniques.

- Apply calibrated tools with identifiable tool records.

- Record defects, abnormalities, and corrective actions.

- Pass/Fail evaluations based on OEM specifications and standards.

List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | HV Cable Stripping Tool (Adjustable) | 5 |
| 2 | Precision Wire Strippers | 5 |
| 3 | Cable Cutters (Insulated) | 5 |
| 4 | Utility Knife (Insulated) | 5 |
| 5 | Measuring Scale / Vernier Caliper | 5 |
| 6 | Marking Tools (Marker/Pen) | 5 |
| 7 | Hydraulic/Manual Crimping Tool | 5 |
| 8 | Hexagonal Crimping Dies (Various Sizes) | 5 sets |
| 9 | Lug Holding Fixtures | 3 |
| 10 | Crimp Height Gauge / Measuring Tool | 3 |
| 11 | Pull-Test Machine / Tensile Tester | 2 |
| 12 | Digital Force Gauge | 3 |
| 13 | Cable Clamping Fixtures | 3 sets |
| 14 | Heat Gun (Temperature Controlled) | 5 |
| 15 | Infrared Thermometer | 2 |
| 16 | Heat-Resistant Work Mats | 5 |
| 17 | Digital Multimeter (HV Rated) | 5 |
| 18 | Continuity Tester | 5 |
| 19 | Test Leads & Probes (Insulated) | 5 sets |
| 20 | Diagnostic Scanner (if required) | 2 |
| 21 | Insulated Rubber Gloves (HV Rated) | 10 pairs |
| 22 | Leather Protector Gloves | 10 pairs |
| 23 | Safety Goggles / Face Shield | 5 |
| 24 | Flame-Resistant Clothing | 5 sets |
| 25 | Safety Shoes (Insulated) | 5 pairs |
| 26 | Heat-Resistant Gloves | 5 pairs |
| 27 | Lock-Out/Tag-Out (LOTO) Kit | 3 sets |
| 28 | Insulated Safety Mats | 5 |
| 29 | Fire Extinguisher (Li-ion Compatible) | 3 |
| 30 | First Aid Kit | 2 |
| 31 | HV Warning Signs / Barricades | 5 sets |
| 32 | Emergency Rescue Hook | 1 |

List of Consumable

The consumables required for this competency standard are provided below:

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | HV Cables (Training/Practice) | Bulk |
| 2 | Terminal Lugs (Various Sizes) | Assorted |
| 3 | Adhesive-lined Heat Shrink Tubing | Assorted |
| 4 | Insulation Tape (HV Rated) | 5 rolls |
| 5 | Cable Marking Labels/Tags | Bulk |
| 6 | Cleaning Cloths/Wipes | Bulk |
| 7 | Isopropyl Alcohol | 2 bottles |
| 8 | Contact Cleaner Spray | 3 cans |
| 9 | Wire Brushes (Soft) | 5 |
| 10 | Inspection/Assessment Forms | As required |
| 11 | Clipboards / Tablets | 5 |
| 12 | Camera / Mobile Device | 3 |
| 13 | Lint-Free Cloth | Bulk |

# List of Machines, Tools and Equipment

List of Machines, Tools and Equipment for the entire qualification of 25 Trainees.

Electric Vehicle (EV) Specifications

- 2-Wheeler EV	1 No.

- 3-Wheeler EV	1 No.

- 4-Wheeler EV	1 No.

| Specification | 2-Wheeler EV | 3-Wheeler EV | 4-Wheeler EV |
| --- | --- | --- | --- |
| Vehicle Type | Electric Motorcycle/ Scooter | Electric Rickshaw/ Cargo Loader | Electric Car/SUV/Bus |
| Number of Wheels | 2 | 3 | 4 |
| Seating Capacity | 1–2 | 3–6 | 4–7 (cars) |
| Motor Type | BLDC Hub or Mid-drive | BLDC/PMSM | PMSM or Induction Motor |
| Motor Power | 0.5–8 kW | 1–12 kW | 30–250+ kW |
| Maximum Speed | 25–120 km/h | 25–60 km/h | 100–220 km/h |
| Driving Range | 50–180 km | 70–180 km | 200–700 km |
| Charging Time | 3–6 hours | 4–8 hours | 30 minutes (DC Fast) to 12 hours (AC) |
| Transmission | Single Speed | Single Speed | Single Speed Reduction Gear |
| Regenerative Braking | Optional | Optional | Standard |
| Cooling System | Air cooled | Air/Liquid cooled | Liquid cooled |

| Name of Trade |  | 0716MSA21. Electric Vehicle (EV) Supervisor Level-4 |  |
| --- | --- | --- | --- |
| Duration of the Course |  | 1 Year |  |
| S. No | Tools and Equipment |  | Quantity |
| 1 | Basic hand tools set |  | 5Sets |
| 2 | Ball Rubber Peen Hammer |  | 5 |
| 3 | Digital multimeter low range |  | 5 |
| 4 | Digital multimeter High range |  | 5 |
| 5 | Insulated gloves Class 0 |  | 5Sets |
| 6 | Safety goggles aur face shield |  | 100 |
| 7 | Trim removal tools |  | 5 |
| 8 | OBD scanner (basic) |  | 5 |
| 9 | Insulated mat |  | 5Sets |
| 10 | Lockout Tagout kit (LOTO) |  | 5 |
| 11 | Warning signs and cones |  | 5Sets |
| 12 | Advanced EV diagnostic scanner |  | 5 |
| 13 | Insulated tool kit (1000V rated) |  | 5 |
| 14 | Insulation tester (Megger) |  | 5 |
| 15 | Clamp meter (DC high current) |  | 5 |
| 16 | Battery lifting tools |  | 5 |
| 17 | Battery Tester Heavy Load |  | 3 |
| 18 | HV disconnect tools |  | 5 |
| 19 | Thermal camera |  | 5 |
| 20 | Continuity tester |  | 5 |
| 21 | PPE kit |  | 5Sets |
| 22 | EV safety barriers and chains |  | 5 |
| 23 | Complete EV insulated master tool kit |  | 5 Kits |
| 24 | High voltage insulation tester (CAT III / IV) |  | 5 |
| 25 | Battery module tester |  | 5 |
| 26 | Cell balancing equipment |  | 5 |
| 27 | Oscilloscope (signal analysis) |  | 2 |
| 28 | Advanced thermal imaging camera |  | 5 |
| 29 | HV rescue hook |  | 5 |
| 30 | Arc flash suit (full body PPE) |  | 5Sets |
| 31 | Battery pack lifting table |  | 5 |
| 32 | Diagnostic software with OEM access |  | 3 |
| 33 | Data logging tools |  | 5 |
| 34 | Electrical insulation tape |  | 10 |
| 35 | Allen key set, hex keys |  | 5 Sets |
| 36 | Wire stripper |  | 10 |
| 37 | Crimping tool |  | 5 |
| 38 | Test light |  | 5 |
| 39 | Fuse puller |  | 5 |
| 40 | Trim removal tool Kits |  | 5 |
| 41 | Inspection mirror |  | 5 |
| 42 | Magnetic pickup tool |  | 5 |
| 43 | Flashlight, rechargeable |  | 5 |
| 44 | Creeper seat |  | 5Sets |
| 45 | Parts tray, magnetic |  | 5 |
| 46 | Insulated screwdrivers |  | 5 Sets |
| 47 | Insulation resistance tester |  | 5 |
| 48 | Torque wrench |  | 5 |
| 49 | Clamp meter (DC) |  | As Per Uses |
| 50 | Test leads replacement |  | 2 Kits |
| 51 | Absorbent pads, spill kit |  | 2 Kits |
| 52 | Thermal paste |  | 2 |
| 53 | Cell insulation sheets |  | 5 |
| 54 | Bus bar insulation covers |  | As per Uses |
| 55 | HV cables and lugs |  | 5 Sets |
| 56 | Coolant fluid specific type |  | 4 Liters |
| 57 | Adhesive foam seals |  | 3 Kits |
| 58 | Fire resistant blankets |  | 2 |
| 59 | Hazardous waste bags |  | 4 |
| 60 | Electrolyte neutralizer |  | 4 |
| 61 | Vehicle lift, 2 post or 4 post |  | 2 |
| 62 | Tool storage cabinet/Trolley |  | 5 |
| 63 | Battery charging station 12 Volts |  | 2 Station |
| 64 | Battery charging station, HV systems |  | 2 Station |
| 65 | Parts washer machine |  | 1 |
| 66 | Tire inflator pressure unit |  | 1 |
| 67 | Oil drain unit |  | 1 |
| 68 | Panel clip pliers |  | 5 |
| 69 | Lighting system |  | 1 |
| 70 | Fire extinguisher, Class ABC |  | 4 |
| 71 | HV isolation station (lockout tagout panel) |  | 2 Station |
| 72 | Safety barrier system (cones, chains, retractable barriers) |  | 2 |
| 73 | Insulated rubber flooring |  | 4 |
| 74 | Battery lifting table (controlled height adjustment) |  | 3 |
| 75 | Thermal monitoring station (overheating detection) |  | 2 Station |
| 76 | Battery pack lifting machine |  | 1 |
| 77 | HV battery test bench |  | 2 |
| 78 | Cell balancing station |  | 2 Station |
| 79 | Battery charge discharge machine |  | 2 |
| 80 | Cleaning spray, contact cleaner |  | 5 |
| 81 | Safety Helmet |  | 25 |
| 82 | Voltage detector pen |  | 5 |
| 83 | Insulated pliers |  | 5 Sets |
| 84 | Stethoscope |  | 5 |
| 85 | Cable cutter |  | 5 |
| 86 | Fire suppression system (lithium battery specific) |  | 2 |
| 87 | Explosion proof storage cabinet (damaged battery) |  | 1 |
| 88 | HV rescue equipment station |  | 2 Station |
| 89 | Data logging and monitoring system |  | 1 System |
| 90 | Clean room (sealed battery room setup) |  | 1 Room |
| 91 | HV voltage detector |  | 5 |
| 92 | Ventilation system |  | 5 |
| 93 | AVC Gas Recycling Machine |  | 1 |
| 94 | Soldering Iron Kit |  | 3 Kits |
| 95 | Heat Gun |  | 3 |
| 96 | Battery Pack Analyzer |  | 5 |
| 97 | Laptop with Diagnostic Software |  | 2 |
| 98 | Thermal Imaging camera |  | 2 |
| 99 | Battery Cycler Machine |  | 2 |
| 100 | Measuring Taps |  | 5 |

# List of Consumables

List of consumables for the entire qualification of 25 Trainees

Battery Specifications

- 2-Wheeler EV	1 No.

- 3-Wheeler EV	1 No.

- 4-Wheeler EV	1 No.

| Parameter | Typical Specification |  |  |
| --- | --- | --- | --- |
|  | 2-Wheeler EV | 3-Wheeler EV | 4-Wheeler EV |
| Battery Chemistry | Lithium-ion (LFP/NMC) | Lithium-ion or LFP | Lithium-ion (NMC/LFP) |
| Battery Voltage | 48V, 60V, 72V | 48V, 60V, 72V, 96V | 200V–800V |
| Battery Capacity | 20–80 Ah | 100–250 Ah | 100–250 Ah |
| Energy Capacity | 1–5 kWh | 5–15 kWh | 20–120 kWh |
| Battery Type | Mostly Low Voltage | Low Voltage (48–72V) | High Voltage (200–800V |
| BMS | Yes | Yes | Advanced BMS |
| Charger | 48V/60V/72V Smart Charger | AC Charger | DC CCS/NACS (Fast Charging) |
| Cooling | Air cooling | Air cooling | Liquid Cooling |

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical Contact Cleaner | 15 cans |
|  | Cable Ties | 25 pcs |
|  | Marker Pens / Labels | 25 pcs |
|  | Cleaning Cloths | 40 pcs |
|  | Probe Tips Replacement | 20 sets |
|  | Fuse Assortment (Practice) | 25 pcs |
|  | OEM Silicone Sealant | 20 tubes |
|  | Replacement Fuses | 50 pcs |
|  | Replacement Relays | 20 pcs |
|  | Gaskets / Sealing Kits | 20 sets |
|  | Battery Coolant | 20 liters |
|  | Abrasive Pads | 20 pcs |
|  | Cleaning Brushes | 20 pcs |
|  | EV-Specific Coolant | 20 Liters |
|  | Dielectric Grease | 10 Tubes (3oz) |
|  | Orange Shielded Cable | 10 Meters |
|  | High-Temp Loom/Tape | 10 Rolls |
|  | Terminal Pins/Sockets | 200 Pieces |
|  | Cotton Swabs/Lint-free Wipes | 2 Bulk Packs |
|  | Glove Dusting Powder | 3 Bottles |
|  | Distilled Water | 2 Gallons |
|  | Nylon Cable Ties | 500 Pieces |
|  | Heat shrink Tubes | 3 dozen |
|  | Insulated Tap | 3 dozen |
|  | Thermal paste (for cooling systems) | 10 |
|  | WD-40 | 12 |
|  | E-drive assembly lubricating oil | As required |
|  | Label makers and labels (for wiring and components) | 12 |
|  | Wire connectors (various sizes) | As required |
|  | Fuses for different rating (Blade fuses, Bolt-down fuses) | As required |
|  | HV Cables (Training/Practice) | Bulk |
|  | Terminal Lugs (Various Sizes) | Assorted |
|  | Insulation Tape (HV Rated) | 5 rolls |
|  | Cable Marking Labels/Tags | Bulk |
|  | Calibration floor marking tape | 2 bottles |
|  | Printed job cards | 3 cans |
|  | Wire Brushes (Soft) | 5 |
|  | Inspection/Assessment Forms | As required |
|  | Clipboards / Tablets | 5 |
|  | Fault analysis forms | 3 |
|  | Anti-static bags | 20 |
|  | Lint-Free Cloth | Bulk |
|  | Isopropyl Alcohol | 2 bottles |
|  | Cable Ties & Harness Clips | Bulk |
|  | Camera / Mobile Device | 3 |
|  | Firmware Repository Access (OEM) | 1 |
|  | Digital Record/PLM System Access | 1 |
|  | Specialized Mastic Sealant | 10 tubes |
|  | Replacement O-Rings (Assorted Kit) | 5 sets |
|  | Sealant Remover Chemical | 5 |
|  | Leak Detection Spray | 5 |
|  | Marking Chalk / Marker | 5 |
|  | First Aid Kit | 5 |
|  | Emergency Stop Switch Access | 5 |
|  | Warning Signage (HV Area) | 10 |
|  | Spill Kit (Chemical/Sealant) | 5 |
|  | Test Report Sheets / Logbooks | 5 sets |
|  | Calibration Certificates (Copies) | 5 sets |
|  | OEM Technical Manuals | 5 |
|  | Clipboard & Pen | 5 |
|  | Waste Disposal Bins (Hazardous/Non-Hazardous) | 10 |
|  | Cable Management Ties | 5 |
|  | Labels & Tags | 5 |
|  | Brake fluid (DOT 4) | 5 liters |
|  | Coolant | 6 liters |
|  | Connector cleaning pins / pin removal kit | 20 |
|  | High-voltage insulating tape | 20 rolls |
|  | Cable ties (assorted sizes) | 5 packs |
|  | Cleaning wipes for terminals and connectors | 10 packs |
|  | Disposable nitrile gloves | 5 boxes |
|  | HV warning labels / isolation tags | 100 pieces |
|  | Terminal protective caps and covers | 25 sets |
|  | Absorbent pads for moisture control | 10 packs |
|  | Printed test record sheets and log forms | 200 sheets |
|  | Thermal Paste | 2 tubes |
|  | Fasteners (Nuts/Bolts) | Assorted kit |
|  | Battery Storage Containers | 2 |
|  | Insulated Packaging Material | Bulk |
|  | Waste Manifest Forms | As required |
|  | Transport Documents | As required |
|  | Pallets / Containment Boxes | 2 |
|  | Straps / Securing Materials | 4 sets |
|  | Hazard Labels (UN Markings) | Bulk |
|  | Adhesive-lined Heat Shrink Tubing | Assorted |
|  | Contact Cleaner Spray | 3 cans |
|  | Cleaning wipes for tools and PPE | 10 packs |
|  | Battery terminal covers and caps | 25 sets |
|  | Warning labels and HV safety stickers | 100 pieces |
|  | Absorbent pads for electrolyte leakage | 10 packs |
|  | Cleaning solvent for workshop equipment | 10 liters |
|  | Permanent marker pens for labeling | 25 pieces |
|  | Printed safety briefing sheets and log forms | 200 sheets |
|  | Coolant (as per OEM specification) | As required |
|  | Contact cleaners | 12 |
|  | Refrigerant gas (R134a / R1234yf) | 10 cylinders |
|  | Compressor oil (PAG / POE oil) | 5 liters |
|  | O-rings and sealing gaskets | 100 pcs |
|  | Refrigerant filter drier | 25 pcs |
|  | Nitrogen gas cylinder (leak testing) | 3 cylinders |
|  | Condenser cleaning solvent | 10 liters |
|  | Cable ties (nylon) | 500 pcs |
|  | Insulation tape (HVAC grade) | 50 rolls |
|  | UV leak detection dye | 10 bottles |

# Members of Qualification Development Committee

| S. # | Name |  | Designation |  | Organization |
| --- | --- | --- | --- | --- | --- |
|  | Mrs. Farhat Hafeez |  | DG SS & C, |  | NAVTTC |
|  | Mr. Shahzad Ahmed |  | Director, SS & C, |  | NAVTTC |
|  | Mr. Abdul Hafeez Abbasi |  | Director Operation, |  | S-TEVTA |
|  | Engr. Muhammad Iqbal |  | DD. |  | KP-TEVTA |
|  | Engr. Tehseen Khalid |  | Lecturer, |  | KP-TEVTA |
|  | Engr. Muzammal Hussain |  | Assistant Director Technical |  | P-TEVTA |
|  | Engr. Muhammad Arsalan |  | EV Specialist |  | MCML |
|  | Mr. Muhammad Usama |  | EV Specialist, |  | Hyundai Nishat Motor |
|  | Mr. S. Umar Azhar Bukhari |  | EV Technician, |  | Youtong EV Buses |
|  | Mr. Muhammad Yousif Baloch |  | Director Academics & Training |  | STEVTA |
|  | Engr. Zohaib Afzal |  | Head of R & D, |  | Pakzon |
|  | Engr. Saqib Abdullah |  | GM Plant, |  | JOLTA. |
|  | Mr. Hafiz Waqas Hashmi |  | Assistant Director Curriculum, |  | PTEVTA |
|  | Mr. Farhan Ahmad Durrani |  | Head of CED |  | L.MC |
|  | Engr. Muhammad Muddasir Iqbal |  | Manager/ Academic |  | E. DB |
|  | Mr. Muhammad Farhat |  | Advisor, |  | E. DB |
|  | Mr. Hamza Nadeem |  | Assistant Director |  | NAVTTC |
|  | Mr. Aftab Hussain Somroo |  | Program Coordinator |  | TEVT- RDF |
|  | Mr. Muhammad Fahad |  | Advisor, |  | Industrial Engg. |
|  | Mr. Arshad Ali Danwer |  | Associate Director |  | Q & C- RDF |
|  | Mr. Muhammad Ishaq |  | Dy. Director (Convener) |  | NAVTTC |
|  | Engr. Inayat ur Rehman, |  | DACUM Facilitator |  | Islamabad |
| # Qualification Review Committee |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |