Government of Pakistan

National Vocational and Technical Training Commission

(NAVTTC)

"Prime Minister Youth Skill Development Program"

Course Contents / Lesson Plan

Course Title: BLDC PCB Repairing in Fan Manufacturing

Course Code: _______________

Duration: 3 Months

| Author Name | Abu Bakar, Humayyun Razzaq, Engr. Bilal Ahmad, Engr. Muhammad Ali |
| --- | --- |
| Date of Development | June, 12-16, 2026 |
| Course Title | BLDC PCB Repairing in Fan Manufacturing |
| Training Objective & Learning Outcomes | The objective of this three-month BLDC PCB Repairing course is to develop the technical knowledge and practical skills required for diagnosing, testing, maintaining, and repairing BLDC controller PCBs used in fan manufacturing. Trainees will gain an understanding of BLDC fan motor operation, electronic components, circuit diagrams, PCB assembly processes, soldering and disordering techniques, and the use of diagnostic instruments such as multi meters, oscilloscopes, and power supplies. The course will emphasize identifying common faults in BLDC fan control boards, including power supply issues, driver circuit failures, sensor faults, and component-level defects. By the end of the training, participants will be able to perform systematic troubleshooting, component replacement, functional testing, and quality verification of BLDC fan PCBs in accordance with manufacturing and maintenance standards. The course aims to enhance problem-solving abilities, improve repair efficiency, reduce production downtime and rejection rates, and prepare trainees for technical roles in fan manufacturing, quality control, after-sales service, and electronics repair industry. After successful completion of this 3-month course, Trainee will be able to: - Understand the fundamentals of BLDC (Brushless DC) motor technology used in ceiling, pedestal, and exhaust fans. - Identify and explain the functions of various components used on BLDC fan PCBs. - Read and interpret basic PCB layouts, circuit diagrams, and schematics. - Use electronic testing instruments such as multi meters, LCR meters, and power supplies safely and effectively. - Diagnose common faults in BLDC fan control boards and motor driver circuits. - Test and troubleshoot power supply sections, rectifiers, capacitors, resistors, MOSFETs, ICs, and protection circuits. - Perform PCB component-level repair, replacement, and soldering/desoldering techniques. - Identify and repair issues related to motor starting, speed control, noise, vibration, and overheating. - Analyze communication and control circuits used in remote-controlled BLDC fans. - Verify motor winding connections and perform basic motor electrical testing. - Apply ESD (Electrostatic Discharge) and electrical safety practices during repair operations. - Conduct functional testing and quality checks after PCB repair. - Maintain repair records, fault reports, and service documentation. - Understand manufacturing quality standards and repair procedures used in fan production facilities. - Improve repair turnaround time while maintaining product reliability and quality. - Perform preventive maintenance and root-cause analysis to reduce recurring PCB failures. - Work independently on BLDC fan PCB troubleshooting and repair tasks in a manufacturing or service environment. - Demonstrate employability skills including teamwork, communication, problem-solving, and workplace safety awareness. |
| Entry-level of trainees | Intermediate or Matric with2 Year field Experience |
| Minimum Qualification of Teachers | D.A.E in Electrical/ Electronic with 4-5 year Experience in relevant Experience |
| Scheme of Studies | Course Title Sr. No; Modules; Theory Hrs.; Practical Hrs.; Total Hrs.. Introduction to BLDC Fan Technology; 5; 20; 25. Fundamentals of Electricity with effects; 5; 20; 25. Fundamentals of Electronic with effects; 5; 20; 25. 3.; DC Power supply, working principle; 5; 20; 25. 4; BLDC Driver working principle; 5; 20; 25. 5; Identification of Electronic Components and Testing; 5; 20; 25. 6; identification of SMD & DIP components with Troubleshooting; 5; 20; 25. 7; Power supply and BLDC Drive Troubleshooting; 5; 20; 25. 8; SMD soldering and DIP Soldering Rework; 5; 20; 25. 9; BLDC Motor Fundamentals; 5; 20; 25. 10; BLDC Fan Manufacturing Process and Performance checking; 5; 20; 25. 11; Testing and Handling of Motor, Circuits and remote; 5; 20; 25. 12; Introduction to BLDC Fan Technology; 5; 20; 25. Total; 60; 240; 300. |
| Course Execution Plan | Duration of the course: 3 months (12 Weeks) Theory: 20% Practical: 80% Weekly hours: 25 hours per week Total contact hours: Maximum 300 hours |
| Companies offering jobs in the respective trade | 1.Rafiq Engineering Industries Ltd. 2.General Fan Company Ltd. 3.Starco Electrical Industries Pvt Ltd. 4.Wahid Industries Ltd. 5.SGS Electrical Industries 6.Indus Home Appliances 7.Super Asia MDS 8. Tamoor Fan Company 9. Facebook, linkdin, Websites and webfirms |
| No of Students | 25 |
| Learning Place | Classroom / Workshop |
| Instructional Resources | https://navttc.gov.pk - Youtube etc |

DETAIL OF COURSE CONTENTS

| Module Title | Learning Units | Learning elements | Task/Practical |
| --- | --- | --- | --- |
| Week 1: Introduction to BLDC Fan technology | Day 1 - Identify basic fan parts. - Demonstrate fan working principle. - Demonstrate fan assembly. - Inspect fan defects. - Apply safety rules in workshop. | - Recognize the basic parts of an electric fan. - Explain the working principle of an electric fan. - Describe the fan assembly process. - Identify common fan defects and their causes. - Discuss workshop safety rules and procedures. | Task: A BLDC fan is not working properly in final inspection. You are required to identify its components, explain working principles, compare BLDC with DC motor, check PCB function, detect faults, and follow safety procedures while troubleshooting. |
|  | Day 2 - Identify BLDC motor parts. - Identify BLDC working principle. - Differentiate BLDC and DC motor. - Demonstrate BLDC operation. - Apply safety in handling BLDC motor. | - Recognize the main parts of a BLDC motor. - Explain the working principle of a BLDC motor. - Differentiate between BLDC and conventional DC motors. - Describe the operation of a BLDC motor. - Discuss safety requirements for handling BLDC motors. | Task: A BLDC motor is not functioning in a fan assembly. You are required to identify its parts, explain its working principle, differentiate it from a DC motor, demonstrate its operation, and follow proper safety procedures during handling and testing. |
|  | Day 3 - Identify BLDC fan system components. - Understand role of PCB in fan operation. - Describe motor, sensor, and controller interaction. - Demonstrate basic BLDC fan working. - Apply correct assembly and safety practices. | - Recognize the main components of a BLDC fan system. - Explain the role of the PCB in fan operation. - Describe the interaction between motor, sensor, and controller. - Illustrate the working principle of a BLDC fan. - Discuss assembly requirements and safety practices. | Task: A BLDC fan is not operating correctly during final testing. You are required to identify system components, explain the PCB role, describe motor–sensor–controller interaction, demonstrate fan operation, and follow correct assembly and safety procedures. |
|  | Day 4 - Identify BLDC fan PCB components. - Explain PCB control function in fan speed. - Describe MOSFET and driver circuit role. - Demonstrate basic PCB operation testing. - Apply safe handling during PCB work. | - Recognize the major components of a BLDC fan PCB. - Discuss the function of the PCB in fan operation. - Explain the role of MOSFETs and driver circuits. - Interpret basic PCB testing results. - Describe safety requirements for PCB handling. | Task: A BLDC fan PCB is not functioning during testing. You are required to identify PCB components, explain speed control function, describe MOSFET and driver operation, demonstrate PCB testing, and follow safety procedures while handling the board. |
|  | Day 5 - Identify advantages of BLDC fans. - Explain energy saving and efficiency benefits. - Describe industrial uses of BLDC fans. - Demonstrate comparison with AC fans. - Apply BLDC knowledge in production planning. | - Recall key benefits of BLDC fans. - Explain how BLDC improves energy efficiency. - Outline industrial uses of BLDC fans. - Differentiate BLDC and AC fan performance. - Use BLDC knowledge in production planning | Task: A BLDC fan PCB is not functioning during production testing. You are required to identify advantages of BLDC fans, explain energy efficiency benefits, describe industrial applications, compare BLDC with AC fans, and apply this knowledge in production planning and decision-making. |
| Week 1 Fundamentals of Electricity with effects | Day 1 - Identify voltage, current, resistance, power - Calculate basic circuit values using formulas - Compare AC vs DC characteristics - Use multi meter for voltage and current measurement | - Explain electrical quantities and their relationships - Define Ohm’s Law in DC circuits - Elaborate AC and DC systems - Define electrical parameters using instruments | Task: Measure DC voltage of different batteries using a multi meter and record all readings in a table. Compare voltage values with rated specifications mentioned on the battery label. Identify correct multi meter range selection before testing. Ensure proper probe placement for accurate measurement. |
|  | Day 2 - Observe heat generation in resistive load - Create simple electromagnetic coil - Check battery chemical reaction - Apply basic electrical safety rules | - Demonstrate heating effect of current - Elaborate magnetic effect of electricity - Illustrate chemical effect in batteries - Define safety hazards of electricity | Task: Build a simple circuit to demonstrate heating and magnetic effect using a coil and battery. Observe coil heating and note how current affects temperature rise. Test magnetic field strength using small metal objects or compass movement. Ensure safe handling of wires during testing. |
|  | Day 3 - Draw AC waveform - Identify AC/DC sources in lab - Test adapter and battery outputs - Compare waveform stability | - Elaborate AC waveform behavior - Define AC and DC sources - Define power supply types | Task: Connect AC adapter and DC battery separately to a simple load and measure output voltage. Observe waveform difference conceptually and note voltage stability. Identify which source gives constant output. Record observations in a comparison sheet. |
|  | Day 4 - Assemble series resistor circuit - Assemble parallel circuit - Measure current in both circuits - Detect open/short faults | - Elaborate series circuits - Elaborate parallel circuits - Define current flow behavior - Elaborate circuit faults | Task: Construct series and parallel circuits using resistors and bulbs on a breadboard. Measure voltage and current in both configurations. Observe brightness difference in bulbs. Identify correct wiring mistakes if any malfunction occurs. |
|  | Day 5 - Apply safe voltage and current handling practices - Relate electrical effects to real-world devices - Adopt safe laboratory procedures | - Elaborate electrical hazards in heating and current - Define electrical effects to real-world devices - Demonstrate safe laboratory procedures | Task: Perform handling of low-voltage DC and AC sources in a controlled lab environment while strictly following safety procedures. Use a multi meter to verify voltage and current before connecting any load to ensure safe operating conditions. Observe heating and magnetic effects in a simple circuit and relate these effects to real devices such as fans and motors. Maintain proper insulation, tool handling, and workspace safety throughout the activity and record all observations in a lab log sheet. |
| Week 2: Fundamentals of Electronic with effects | Day 1: - Recognize electronic signals in simple circuits - Identify resistors, capacitors, diodes, transistors, ICs - Compare analog vs digital signal behavior - Observe electronics in real-life devices | - Define fundamentals of electronic systems and signal types - Define basic electronic components used in circuits - Differentiate analog and digital electronic signals - Elaborate electronics to control systems (fans, drivers, PCB) | Task: Perform identification of basic electronic components such as resistors, capacitors, diodes, transistors, and ICs on a PCB. Observe their physical appearance and locate them according to circuit layout. Classify components into active and passive types based on their function. Prepare a component identification sheet with proper labeling. |
|  | Day 2 - Measure resistance values using multi meter - Observe capacitor charging/discharging behavior - Identify inductive components in circuits - Compare component behavior under load | - Explain behavior of passive components in circuits - Define effects of resistors in current control - Describe capacitor charging and discharging effects - Define inductor magnetic energy storage | Task: Measure resistance values of different resistors using a multi meter and compare them with rated values. Observe capacitor charging and discharging behavior in a simple test circuit. Identify inductors and understand their role in energy storage. Record the behavior of each component under test conditions. |
|  | Day 3 - Test diode forward and reverse bias - Demonstrate transistor ON/OFF switching - Observe LED control through transistor circuit - Identify faulty switching components | - Define operation of active electronic components - Define diode rectification effect - Describe transistor switching operation - Define amplification effect in circuits | Task: Test diodes in forward and reverse bias using a multi meter to observe conduction behavior. Build a simple transistor switching circuit to control an LED. Observe ON/OFF switching response and identify correct terminal connections. Replace a faulty component if incorrect behavior is found. |
|  | Day 4 - Trace signal path on PCB - Measure voltage at different circuit points - Observe signal distortion or noise - Compare correct vs faulty circuit output | - Analyze input-output behavior of circuits - Define signal amplification and loss effects - Elaborate circuit performance under load | Task: Trace signal flow in a simple electronic circuit using PCB tracks. Measure voltage at different points of the circuit using a multi meter. Observe how signal changes from input to output stages. Identify any signal loss, distortion, or interruption in the circuit. |
|  | Day 5 - Assemble simple electronic control circuit - Test circuit operation under real conditions - Identify control signals in BLDC driver system - Verify stable electronic operation in load conditions | - Define application of electronics in control systems - Analyze electronic control in BLDC and fan systems - Elaborate PCB-based control circuits - Demonstrate basic electronic circuit functionality | Task: Assemble a simple control circuit using basic components to simulate electronic control behavior. Test circuit response under load conditions and observe stability. Identify how electronic components control switching and signal flow in real systems like fans or drivers. Document complete circuit behavior and performance results |
| Week 3 DC Power supply, working principle | Day 1 - Construct bridge rectifier circuit - Test AC input and DC output - Draw waveform conversion - Check polarity output | - Elaborate AC to DC conversion - Demonstrate rectifier circuits - Define AC and DC waveforms | Task: Identify rectifier circuit on PCB and trace AC to DC conversion path. Measure AC input and DC output using multi meter. Check diode bridge condition for faults. Record voltage conversion results |
|  | Day 2 - Install filter capacitor - Measure ripple effect - Replace faulty capacitor - Observe output smoothing | - Explain filter circuit operation - Define capacitor role - Define ripple voltage - Evaluate output quality | Task: Identify rectifier circuit on PCB and trace AC to DC conversion path. Measure AC input and DC output using multi meter. Check diode bridge condition for faults. Record voltage conversion results |
|  | Day 3 - Checking of filtering - Observe Smoothing of pulsating DC | - Elaborate Capacitor filters Voltage regulators Zennor diode and IC regulators | Task: Identify rectifier circuit on PCB and trace AC to DC conversion path. Measure AC input and DC output using multi meter. Check diode bridge condition for faults. Record voltage conversion results |
|  | Day 4 - Identify regulator IC pins - Measure regulated output - Compare input/output voltage - Check load response | - Explain regulator function - Elaborate Stabilize DC output - Define regulator IC - Analyze voltage variation | Task: Diagnose a faulty power supply with no output condition. Trace circuit step by step from input to output. Identify faulty diode, capacitor, or fuse. Repair and verify stable DC output. |
|  | Day 5 - Check fuse and rectifier - Trace circuit faults - Replace damaged parts - Test final output | - Define Diagnose power failures - Define short circuits - Define Repair faulty components - Define Verify restored output | Task: Assemble a complete DC power supply circuit on test board. Measure output stability under different loads. Check voltage consistency for different input conditions. Ensure safe insulation and wiring. |
| Week 4 BLDC Driver working principle | Day 1 - identify driver role in BLDC operation - Locate power, control, and output stages - Interpret block diagrams - Observe motor response behavior | - Explain BLDC driver architecture and function - Define driver internal circuit blocks - Describe signal flow in BLDC system - Elaborate controller-motor interface | Task: Identify all sections of BLDC driver board including power, control, and output stages. Trace signal flow from input to motor output. Inspect components for damage. Label each section clearly. |
|  | Day 2 - Trace A, B, C phase sequence - Observe timing signals - Test voltage output of phases - Compare correct vs faulty signals | - Explain 3-phase switching mechanism - Analyze commutation sequence operation - Define switching devices | Task: Measure phase outputs (U, V, W) of BLDC driver. Check waveform consistency during operation. Identify missing or weak phase signals. Record results systematically. |
|  | Day 3 - Generate PWM concept understanding - Adjust speed levels - Measure RPM changes - Verify control response accuracy | - Explain PWM speed control principle - Analyze speed variation mechanism - Define control input signals - Evaluate motor speed response | Task: Test speed control by varying input signal or knob. Observe motor RPM changes at different levels. Identify smooth and jerky speed transitions. Document response behavior. |
|  | Day 4 - Locate protection components - Simulate overload condition - Observe shutdown response - Reset driver safely | - Explain protection circuit operation - Define fault detection systems - Analyze overcurrent protection - Define shutdown functionality | Task: Simulate overload condition in BLDC driver system. Observe protection shutdown behavior. Reset system after fault condition. Verify system recovery. |
|  | Day 5 - Detect missing phase output - Check power input failure - Analyze abnormal waveform - Classify fault type | - Elaborate BLDC driver fault categories - Elaborate signal failure conditions - Define hardware vs signal faults | Task: Diagnose faulty BLDC driver with no output condition. Check power input, MOSFETs, and control signals. Identify failed section step-by-step. Repair and test system. |
| Week5 Identification of Electronic Components and Testing | Day 1 - Recognize resistors, capacitors, ICs - Identify component types - Identify Map PCB layout | - Define electronic components - Classify active/passive devices - Elaborate schematic symbols - Define components on board | Task: Identify components on damaged PCB board. Separate functional and non-functional parts visually. Use multimeter to confirm findings. Document component status. |
|  | Day 2 - Use multi meter correctly - Check polarity and continuity - Compare readings (good/faulty) - Record test values | - Define resistors and capacitors - Define diodes and transistors - Define IC voltage supply - Define component condition | Task: Test resistors, capacitors, and diodes using multimeter. Compare readings with standard values. Replace defective components. Verify circuit continuity. |
|  | Day 3 - Follow circuit path - Locate faulty components - Test suspect parts - Identify failure points | - Diagnose electronic faults - Analyze circuit failure - Define open/short circuits - Elaborate Trace PCB signal flow | Task: Test transistor switching circuits. Check input-output response using LED indicator. Identify faulty switching behavior. Replace transistor if required. |
|  | Day 4 - De solder components safely - Install replacements - Inspect solder joints - Validate output | - Describe PCB parts - Define repair quality - Define circuit function | Task: Trace signal flow on PCB and locate fault points. Identify open circuits using continuity test. Mark damaged tracks. Prepare repair plan. |
|  | Day 5 - Test full circuit operation - Measure output signals - Compare before/after repair | - Define PCB circuit performance - Define repaired systems - Evaluate component behavior - Elaborate system stability | Task: Repair faulty PCB by replacing components. Resolder weak joints carefully. Test complete circuit after repair. Confirm proper operation |
| Week 6 identification of SMD & DIP components with Troubleshooting | Day 1 - Identify SMD vs DIP - Recognize physical structure - Read codes and values - Identify Map PCB positions | - Differentiate SMD and DIP components - Define packaging types - Elaborate markings | Task: Identify SMD and DIP components on a mixed PCB board. Separate components based on size and mounting type. Use magnification tools for small SMD parts. Document component classification. |
|  | Day 2 - Use hot air station - Remove components properly - Check solder joints - Verify connection - Test circuit continuity | - Define SMD components - Remove SMD parts - Inspect solder quality | Task: Desolder and replace a faulty SMD resistor or capacitor. Use hot air station carefully to avoid PCB damage. Install new component with proper alignment. Test circuit continuity after replacement. |
|  | Day 3 - Identify Heat joints correctly - Remove components safely - Identify Insert correctly - Check continuity - Align leads properly | - Elaborate Solder DIP components - Elaborate de solder faulty parts - Define solder joints | Task: Perform DIP component soldering on a test PCB. Insert components correctly and solder each pin properly. Inspect solder joints for cold or weak connections. Verify electrical continuity. |
|  | Day 4 - Inspect visually - Test continuity - Identify track damage - Identify Confirm repair | - Elaborate PCB faults - Elaborate damaged tracks - Elaborate output | Task: Inspect PCB for damaged tracks and broken solder joints. Use continuity test to trace connections. Repair broken PCB tracks using jumper wires. Confirm circuit restoration after repair |
|  | Day 5 - Inspect wear and tear - Check thermal stress - Test vibration resistance - Validate stability | - Evaluate component reliability - Elaborate solder strength - Define heat impact - Elaborate durability | Task: Test a repaired PCB containing both SMD and DIP components. Power the circuit and observe output behavior. Identify any remaining faults in components or connections. Ensure stable operation. |
| Week 7 Power supply and BLDC Drive Troubleshooting | Day 1 - Identify AC to DC conversion path - Measure input and output voltage levels - Detect open circuit and short circuit faults - Inspect rectifier diodes and filter capacitors | - Elaborate DC power supply working stages - Elaborate common power supply faults - Define rectifier and filter circuits - Evaluate regulated output performance | Task: Inspect a faulty DC power supply and identify no-output conditions. Trace the circuit from AC input to DC output using a multi meter. Check rectifier diodes, filter capacitors, and regulator IC for faults. Record all findings and repair defective components. |
|  | Day 2 - Check regulator IC input/output pins - Test voltage drop under load - Compare stable vs unstable output - Identify overheating components | - Explain voltage regulation function - Elaborate regulator IC faults - Elaborate output voltage stability - Define load response behavior | Task: Test voltage regulator performance under different load conditions. Measure output voltage stability with and without load. Identify overheating or voltage drop issues. Replace faulty regulator if unstable output is detected. |
|  | Day 3 - Measure DC supply to driver circuit - Inspect MOSFET/driver IC condition - Trace missing phase output signals - Verify grounding and power continuity | - Define BLDC driver power section faults - Elaborate damaged switching components - Elaborate phase output failures - Elaborate driver power input stage | Task: Check BLDC driver power input section and verify correct DC supply. Test MOSFETs or switching components for short or open conditions. Trace missing voltage in driver circuit. Restore proper power flow after repair |
|  | Day 4 - Measure Hall sensor output signals - Check PWM waveform using meter/scope - Detect signal interruption or noise - Compare normal vs faulty control behavior | - Define control signal failure in BLDC driver - Describe Hall sensor signal issues - Elaborate PWM signal generation - Evaluate controller response behavior | Task: Test Hall sensor signals in BLDC system using multi meter. Rotate motor manually and observe signal changes. Identify missing or weak sensor output. Replace or correct faulty sensor connections. |
|  | Day 5 - Trace complete power flow from supply to motor - Identify multi-stage faults (power + driver) - Test system under real load conditions - Confirm stable motor operation after repair | - Define power supply with BLDC driver system - Define combined system failures - Define intermittent faults in operation - Elaborate full system recovery after repair | Task: Diagnose complete BLDC system with power and driver integration faults. Check wiring between power supply, driver, and motor. Identify system-level failures affecting rotation. Repair and verify stable motor operation. |
| Week 8 SMD soldering and DIP Soldering Rework | Day 1 - Classify SMD vs DIP parts - Recognize physical structure - Identify Read component codes - Identify Map placement on PCB | - Define SMD and DIP components - Define package types - Elaborate PCB mounting positions - Elaborate component markings | Task: Identify SMD and DIP components on a mixed PCB board. Separate components based on size and mounting type. Use magnification tools for small SMD parts. Document component classification |
|  | Day 2 - Use hot air station - Align small components - Check solder quality - Verify connections | - Elaborate SMD components - Define SMD parts - Elaborate solder joints - Elaborate circuit continuity | Task: Desolder and replace a faulty SMD resistor or capacitor. Use hot air station carefully to avoid PCB damage. Install new component with proper alignment. Test circuit continuity after replacement |
|  | Day 3 - Identify Heat solder joints properly - Remove damaged parts - Insert components correctly - Inspect solder joints | - Define DIP components - Define faulty parts - Elaborate Verify PCB connections | Task: Perform DIP component soldering on a test PCB. Insert components correctly and solder each pin properly. Inspect solder joints for cold or weak connections. Verify electrical continuity. |
|  | Day 4 - Inspect PCB visually - Test continuity - Identify broken tracks - Identify repair success | - Elaborate SMD/DIP faults - Elaborate PCB defects - Describe damaged tracks - Elaborate circuit operation | Task: Inspect PCB for damaged tracks and broken solder joints. Use continuity test to trace connections. Repair broken PCB tracks using jumper wires. Confirm circuit restoration after repair |
|  | Day 5 - Inspect long-term wear - Check thermal stress - Test vibration impact - Validate stability | - Evaluate component reliability - Elaborate solder joint strength - Define heat damage effects - Define PCB durability | Task: Test a repaired PCB containing both SMD and DIP components. Power the circuit and observe output behavior. Identify any remaining faults in components or connections. Ensure stable operation. |
| Week 9 BLDC Motor Fundamentals | Day 1 - Identify BLDC motor function - Locate rotor and stator - Identify motor types - Identify applications | - Explain BLDC working principle - Elaborate motor components - Explain BLDC vs DC motor - Elaborate applications | Task: Open a BLDC motor carefully and identify all internal parts including stator, rotor, windings, and Hall sensors. Observe the physical arrangement of magnets and coil structure. Inspect any dust, damage, or wear on bearings and shaft. Record all identified components in a structured format. |
|  | Day 2 - Open motor housing - Observe winding structure - Check magnet placement - Reassemble motor parts | - Elaborate Disassemble BLDC motor - Elaborate stator winding - Define rotor magnets - Elaborate Assemble motor structure | Task: Use a multi meter to measure resistance of all three motor phases (U, V, W). Compare all readings to ensure balance between phases. Identify any open circuit or short circuit condition in windings. Document readings and classify motor condition as good or faulty. |
|  | Day 3 - Identify switching sequence - Observe driver output - Interpret signal behavior - Draw circuit flow | - Explain commutation process - Elaborate phase switching - Define controller signals - Define current flow | Task: Rotate the motor shaft manually and observe smoothness of movement. Check for friction, noise, or uneven rotation. Inspect bearing condition and alignment of rotor magnets. Identify mechanical faults affecting motor performance |
|  | Day 4 - Locate sensor position - Measure signal voltage - Verify feedback response - Check sensor alignment | - Define Hall sensors - Define sensor output - Elaborate position feedback - Define sensor faults | Task: Connect multi meter to Hall sensor output terminals and rotate rotor slowly. Observe voltage changes corresponding to rotor position. Identify missing or incorrect sensor signals. Verify correct placement and working condition of all sensors |
|  | Day 5 - Apply load to motor shaft - Observe change in speed and torque | - Elaborate Load Test - Elaborate Safety Practice | Task: Connect BLDC motor with driver circuit and power supply. Observe starting behavior and rotation direction. Test motor under no-load condition for smooth operation. Identify faults such as jerking, overheating, or non-start condition and record findings |
| Week 10 BLDC Fan Manufacturing Process and Performance checking | Day 1 - Assemble BLDC fan motor with housing and blades - Fit rotor, stator, and shaft components correctly - Secure mechanical parts using proper fastening methods - Align fan structure for smooth rotation | - Elaborate BLDC fan components (motor, blades, driver, housing) - Elaborate Assemble motor with fan blades and rotor housing - Explain Connect BLDC driver with motor phases correctly - Define mechanical alignment and shaft fitting - Define fastening, screws, and mechanical stability | Task: Assemble BLDC fan motor with housing and blades. Ensure correct alignment of rotor and stator. Tighten all mechanical fittings properly. Test smooth rotation manually. |
|  | Day 2 - Connect BLDC motor to driver circuit - Wire phase terminals correctly (U, V, W) - Integrate control circuit with power supply - Ensure safe routing of electrical connections | - Elaborate wiring terminals of BLDC motor and driver - Elaborate phase wires (U, V, W) correctly - Elaborate Route wiring safely to avoid short circuits - Elaborate continuity of wiring connections - Define polarity and secure electrical joints | Task: Connect BLDC motor with driver circuit wiring. Ensure correct phase connections (U, V, W). Secure all electrical joints properly. Test continuity of wiring. |
|  | Day 3 - Operate BLDC fan using control system - Test speed variation and control response - Verify rotation stability and smooth operation - Check direction and control functionality | - Elaborate fan ON/OFF operation through driver circuit - Elaborate speed control response at different levels - Elaborate smooth rotation without jerks or noise - Describe direction control functionality (if applicable) - Elaborate abnormal heating or vibration | Task: Power the assembled fan and test ON/OFF operation. Observe smooth starting and running behavior. Check speed control response. Identify any abnormal noise or vibration |
|  | Day 4 - Measure RPM and speed levels - Evaluate airflow and cooling efficiency - Analyze power consumption under load - Compare rated and actual performance | - Elaborate fan RPM using tachometer or sensor - Evaluate airflow performance and output efficiency - Elaborate power consumption under different loads - Define rated vs actual performance values - Elaborate performance test results for quality control | Task: Test fan performance under different speed levels. Measure RPM changes if instrument available. Observe airflow and motor stability. Record performance behavior. |
|  | Day 5 - Inspect mechanical and electrical assembly quality - Detect faults in wiring, motor, or driver - Validate safety and insulation standards - Identify final product readiness for use | - Elaborate overall assembly quality and finishing - Elaborate mechanical or electrical defects - Describe safety standards and insulation integrity - Explain long-term operational stability - Elaborate final inspection report | Task: Perform final assembly inspection of BLDC fan. Check mechanical strength and electrical safety. Verify wiring insulation and fitting quality. Ensure product is ready for testing. |
| Week 11 Testing and Handling of Motor, Circuits and remote | Day 1 - Inspect motor housing, shaft, and bearings carefully - Measure winding resistance using multi meter - Handle rotor carefully to avoid magnet damage - Identify motor as good, weak, or faulty | - Explain BLDC motor testing procedures in fan industry - Define motor terminals and winding structure - Demonstrate safe handling of BLDC motors - Evaluate motor condition through basic tests | Task: Inspect BLDC motor carefully before testing and ensure proper handling to avoid damage to rotor magnets and bearings. Measure winding resistance of all three phases using a multi meter and compare readings. Handle motor with dry hands and proper support to avoid mechanical shock. Classify motor condition as good, weak, or faulty based on test results. |
|  | Day 2 - Test motor speed using RPM measurement tools - Observe vibration, noise, and imbalance - Check bearing smoothness by manual rotation - Ensure safe mounting before testing | - Define motor performance under load conditions - Define RPM and vibration levels - Explain mechanical safety in fan assembly - Define abnormal motor behavior | Task: Run BLDC motor under no-load condition and observe RPM, vibration, and noise level. Check smooth rotation by manually rotating shaft before powering the system. Identify abnormal heating or imbalance during operation. Ensure motor is properly mounted before testing to avoid accidents. |
|  | Day 3 - Identify PCB with anti-static precautions - Measure voltage at driver input/output stages - Trace signal flow in control circuit - Identify Avoid short circuits during testing readings | - Elaborate BLDC driver circuit functionality - Define PCB components and signal flow - Demonstrate safe PCB handling practices - Define circuit faults in driver system | Task: Inspect BLDC driver PCB carefully and ensure anti-static handling before testing. Measure input and output voltages at different stages of the circuit. Trace signal flow and identify any missing or weak output phases. Avoid short circuits while probing live PCB. |
|  | Day 4 - Check remote battery condition - Test ON/OFF and speed control functions - Inspect receiver module connections - Handle remote PCB carefully to avoid damage | - Explain remote control operation in BLDC fans - Demonstrate IR/RF signal transmission and reception - Elaborate remote control faults - Demonstrate safe handling of control systems | Task: Check remote control battery condition and replace if weak. Test all functions such as ON/OFF, speed control, and mode switching. Verify signal reception on BLDC receiver module properly. Handle remote PCB carefully to avoid damage to small components. |
|  | Day 5 - Connect motor-driver-remote system correctly - Test complete fan operation under load - Follow safety procedures during live testing - Identify faults in integrated system | - Integrate motor, driver, and remote system - Evaluate full fan system performance - Elaborate industrial safety in handling BLDC systems - Explain Troubleshoot system-level faults | Task: Connect BLDC motor, driver PCB, and remote system for full fan operation testing. Observe complete system response under different speed levels and load conditions. Follow strict electrical safety while working on live system. Identify and correct any system-level fault in operation. |

LIST OF MACHINERY / EQUIPMENT

For the Class of 25 Students (3-Month Course)

| S.No | Name of Items | Specifications (If Required) | Unit |
| --- | --- | --- | --- |
| 1 | BLDC Fan Control PCB Training Boards. | BLDC driver PCB with test points and fault simulation facility | 02 Nos. |
| 2 | BLDC Ceiling Fans | 30W–60W BLDC ceiling fan, 220V AC, remote operated | 02 Nos. |
| 3 | Faulty BLDC PCBs | Assorted faulty PCBs for troubleshooting and repair practice | 10 Nos. |
| 4 | Digital Multi meter | True RMS, Auto Range, AC/DC Voltage, Current, Resistance Measurement | 05 Nos. |
| 5 | Clamp Meter | AC/DC Current Measurement up to 600A | 05 Nos. |
| 6 | Bench Power Supply | 0–30V DC, 0–5A, Digital Display | 01 Nos. |
| 7 | Digital Storage Oscilloscope | 100 MHz, 2 Channel, USB Interface | 01 Nos. |
| 8 | Function Generator | 1Hz–20MHz, Sine, Square, Triangle Wave Output | 01 Nos. |
| 9 | LCR Meter | Inductance, Capacitance and Resistance Measurement. | 01 Nos. |
| 10 | Insulation Resistance Tester (Megger) | 250V/500V/1000V Test Range | 01 Nos. |
| 11 | PCB Continuity Tester | Audible Continuity Indication. | 01 Nos. |
| 12 | Soldering Station | 60–90W, Temperature Controlled, 200–480°C. | 02 Nos. |
| 13 | De soldering Station | Vacuum Pump Type, Adjustable Temperature | 02 Nos. |
| 14 | Hot Air Rework Station | 100–500°C Temperature Range | 01 Nos. |
| 15 | PCB Preheating Station | Adjustable Temperature up to 350°C. | 01 Nos. |
| 16 | Smoke Extractor | Activated Carbon Filter Type | 01 Nos. |
| 17 | ESD Workbench | Anti-static Surface with Grounding | 01 Nos. |
| 18 | ESD Wrist Strap | Adjustable Anti-static Wrist Strap | 01 Nos. |
| 19 | Anti-static Mat | Heat Resistant ESD Safe Mat | 01 Nos. |
| 20 | PCB Holder Clamp | Adjustable PCB Holding Fixture | 05 Nos. |
| 21 | Component Tester | MOSFET, BJT, IGBT, Diode and Capacitor Testing. | 01 Nos. |
| 22 | IC Tester | Digital and Analog IC Testing Capability. | 01 Nos. |
| 23 | LED Tester | 0–300V LED Testing Range. | 01 Nos. |
| 24 | Digital Tachometer | Non-contact RPM Measurement. | 01 Nos. |
| 25 | Infrared Thermometer | -50°C to 550°C Temperature Range | 01 Nos. |
| 26 | Power Analyzer | Voltage, Current, Power Factor and Energy Measurement. | 01 Nos. |
| 27 | Watt Meter | 0–3000W Measurement Range | 01 Nos. |
| 28 | Isolation Transformer | 220V AC, 1kVA Output | 01 Nos. |
| 29 | Precision Screwdriver Set | Electronics Maintenance Type | 02 Nos. |
| 30 | Long Nose Pliers | Insulated Handle, 6-inch. | 05 Nos. |
| 31 | Side Cutter | Electronic Component Cutting Type | 05 Nos. |
| 32 | Wire Stripper | 0.2–6 mm² Wire Range | 02 Nos. |
| 33 | Tweezers Set | ESD Safe Stainless Steel Set | 05 Nos. |
| 34 | Crimping Tool | Connector and Terminal Crimping Type. | 02 Nos. |
| 35 | Magnifying Lamp | LED Illuminated, 3X–5X Magnification | 01 Nos. |
| 36 | Electronic Tool Kit | Complete Repair Tool Set | 02 Nos. |
| 37 | BLDC Driver IC Kit | Common BLDC Fan Driver ICs | 01 Lot |
| 38 | MOSFET Assortment Kit | N-Channel and P-Channel Types. | 01 Lot |
| 39 | Diode Assortment Kit | Rectifier, Zener and Fast Recovery Diodes | 01 Lot |
| 40 | Capacitor Assortment Kit | Electrolytic, Ceramic and Film Capacitors. | 01 Lot |
| 41 | Resistor Assortment Kit | ¼ Watt Assorted Values. | 01 Lot |
| 42 | Voltage Regulator Kit | Linear and Switching Regulators | 01 Lot |
| 43 | Hall Sensor Kit | BLDC Motor Hall Effect Sensors | 01 Lot |
| 44 | Connectors and Terminals Kit | Kit. | 01 Lot |
| 45 | PCB Cleaning Chemicals | IPA Cleaner and Flux Remover | 01 Lot |
| 46 | Solder Wire | Lead-Free, 0.8 mm Diameter | 01 Lot |
| 47 | Flux Paste | No-Clean Electronic Grade Flux | 01 Lot |
| 48 | De soldering Wick | Copper Braid Type | 01 Lot |
| 49 | Heat Shrink Tubes | Assorted Sizes | 01 Lot |
| 50 | Desktop Computer | Core i5, 8GB RAM, 256GB SSD | 02 Nos. |
| 51 | Multimedia Projector | Full HD, HDMI Compatible | 01 Nos. |
| 52 | PCB Design Software | Ki Cad/Easy EDA or Equivalent | 01 Nos. |
| 53 | Fire Extinguisher | CO₂ Type, 5kg Capacity | 01 Nos. |
| 54 | First Aid Box | Industrial Standard First Aid Kit | 01 Nos. |
| 55 | Safety Goggles | Industrial Grade Eye Protection | 05 Nos. |
| 56 | Electrical Safety Gloves | 1000V Insulated Gloves | 01 Pair |
| 57 | Emergency Power Cut-Off Switch | Workshop Main Power Shutoff | 01 Nos. |
| 58 | Storage Cabinet | Metal Tool and Component Cabinet | 01 Nos. |
| 59 | Training Tables and Chairs | Electronics Lab Furniture.. | 25 Set |
| 60 | White Board with Marker Set | 4 ft × 6 ft Writing Surface | 01 Set |

LIST OF CONSUMABLE MATERIAL

For the Class of 25 Students (3-Month Course)

| S.No | Name of Items | Specifications (If Required) | Unit |
| --- | --- | --- | --- |
| 1 | BLDC Driver ICs |  | Assorted |
| 2 | MOSFETs |  | Assorted |
| 3 | Diodes |  | Assorted |
| 4 | Capacitors |  | Assorted |
| 5 | Resistor |  | Assorted |
| 6 | Voltage Regulators |  | Assorted |
| 7 | Hall Sensors |  | Assorted |
| 8 | Connectors and Terminals |  | Assorted |
| 9 | PCB Cleaning Chemicals |  | As Required |
| 10 | Solder Wire |  | As Required |
| 11 | Flux |  | As Required |
| 12 | Desoldering Wick |  | As Required |
| 13 | Heat Shrink Tubes |  | As Required |

Trade Registration Form

- Title of the Qualification.

BLDC PCB Repairing in Fan Manufacturing

- List of Competencies/Modules

| Sr.No. | Competency Standards/ Modules |
| --- | --- |
| 1 | Introduction to BLDC Fan Technology |
| 2 | Fundamentals of Electricity with effects |
| 3 | Fundamentals of Electronic with effects |
| 4 | DC Power supply, working principle |
| 5 | BLDC Driver working principle |
| 6 | Identification of Electronic Components and Testing |
| 7 | identification of SMD & DIP components with Troubleshooting |
| 8 | Power supply and BLDC Drive Troubleshooting |
| 9 | SMD soldering and DIP Soldering Rework |
| 10 | BLDC Motor Fundamentals |
| 11 | BLDC Fan Manufacturing Process and Performance checking |
| 12 | Testing and Handling of Motor, Circuits and remote |

- Class / Workshop Size/ Dimension (25 Students Capacity)

|  | Requirement | Size/Dimensions(feet) |
| --- | --- | --- |
| Classroom size | Yes | 25x30 |
| Workshop/Laboratory size | Yes | 40x40 |
| Any other Requirement | NO | No |

- Student Teacher Ratio

25:1

- Teacher Qualification:

D.A.E in Electrical/ Electronic with 4-5 year Experience in relevant Experience

- Trainee Eligibility:

Intermediate or Matric with2 Year field Experience

- Duration / Credit Hours / Contact Hours

| Contact Hours | Duration |
| --- | --- |
| 300 | 3 Month |

- Any other co-curricular activities

| Activity | Requirement |
| --- | --- |
| Industry Linkage | Yes |
| Teacher Training | Yes |
| Workshops and Seminars | Yes |