|  | Assessment Evidence Guide For “Auto & Farm Machinery Technician” Level-4 (One Year Program) Summative Assessment |
| --- | --- |
|  | Date 04-09 November 2024 Lahore |
|  | National Vocational & Technical Training Commission, Islamabad |

Instruction Sheet for the Candidate

| Qualification | Auto & Farm Machinery Technician |
| --- | --- |
| Competency Standard | 0716MSA1801 Comply Health and Safety Guidelines. 0716MSA1802 Follow the Communication Policy/Procedure at the Workplace. 0716MSA1803 Interpret Drawings. 0716MSA1804 Perform Bench Work. 0716MSA1805 Perform Repair/Maintenance of Petrol Engines. 0716MSA1806 Perform Repair/Maintenance of Diesel Engines. 0716MSA1807 Perform Repair/Maintenance of Suspension, Steering & Braking System. 0716MSA1808 Perform Repair/Maintenance of Transmission System. 0716MSA1809 Perform Repair/Maintenance of Farm Machinery. 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements. 0716MSA1811 Perform Repair/Maintenance of Electric Implements. 0716MSA1812 Adopt Green Practices at Workplace. |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment) by following all safety measures: Task: Perform replacement of cylinder liner from the engine of a tractor. |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Wear Personal Protective Equipment’s. - Arrange tools & equipment’s to replace the cylinder liner from the engine of a tractor. - Remove intake manifold. - Remove Exhaust manifold - Remove oil Sump. - Remove cylinder head from engine. - Remove pistons of engine. - Replace cylinder liner. - Install pistons as per given standards. - Assemble the cylinder block - Install cylinder head to the engine. - Install intake manifold to cylinder head. - Install exhaust manifold to cylinder head. - Set Engine timing. - Start engine for checking up engine performance. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Auto & Farm Machinery Technician |
| Competency Standard | 0716MSA1801 Comply Health and Safety Guidelines. 0716MSA1802 Follow the Communication Policy/Procedure at the Workplace. 0716MSA1803 Interpret Drawings. 0716MSA1804 Perform Bench Work. 0716MSA1805 Perform Repair/Maintenance of Petrol Engines. 0716MSA1806 Perform Repair/Maintenance of Diesel Engines. 0716MSA1807 Perform Repair/Maintenance of Suspension, Steering & Braking System. 0716MSA1808 Perform Repair/Maintenance of Transmission System. 0716MSA1809 Perform Repair/Maintenance of Farm Machinery. 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements. 0716MSA1811 Perform Repair/Maintenance of Electric Implements. 0716MSA1812 Adopt Green Practices at Workplace. |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Perform replacement of cylinder liner from the engine of a tractor. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Wear Personal Protective Equipment’s. |  |  |
| - Arrange tools & equipment’s to replace the cylinder liner from the engine of a tractor. |  |  |
| - Remove intake manifold. |  |  |
| - Remove Exhaust manifold |  |  |
| - Remove oil Sump. |  |  |
| - Remove cylinder head from engine. |  |  |
| - Remove pistons of engine. |  |  |
| - Replace cylinder liner. |  |  |
| - Install pistons as per given standards. |  |  |
| - Assemble the cylinder block |  |  |
| - Install cylinder head to the engine. |  |  |
| - Install intake manifold to cylinder head. |  |  |
| - Install exhaust manifold to cylinder head. |  |  |
| - Set Engine timing. |  |  |
| - Start engine for checking up engine performance. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: ______________________________

Assessors Judgment Guide

| Qualification | Auto & Farm Machinery Technician |
| --- | --- |
| Competency Standard | 0716MSA1801 Comply Health and Safety Guidelines. 0716MSA1802 Follow the Communication Policy/Procedure at the Workplace. 0716MSA1803 Interpret Drawings. 0716MSA1804 Perform Bench Work. 0716MSA1805 Perform Repair/Maintenance of Petrol Engines. 0716MSA1806 Perform Repair/Maintenance of Diesel Engines. 0716MSA1807 Perform Repair/Maintenance of Suspension, Steering & Braking System. 0716MSA1808 Perform Repair/Maintenance of Transmission System. 0716MSA1809 Perform Repair/Maintenance of Farm Machinery. 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements. 0716MSA1811 Perform Repair/Maintenance of Electric Implements. 0716MSA1812 Adopt Green Practices at Workplace. |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: _____________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Perform replacement of cylinder liner from the engine of a tractor. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Wore Personal Protective Equipment’s. |  |  |  |  |  |
|  | Arranged tools & equipment’s to replace the cylinder liner from the engine of a tractor. |  |  |  |  |  |
|  | Removed intake manifold. |  |  |  |  |  |
|  | Removed Exhaust manifold |  |  |  |  |  |
|  | Removed oil Sump. |  |  |  |  |  |
|  | Removed cylinder head from engine. |  |  |  |  |  |
|  | Removed pistons of engine. |  |  |  |  |  |
|  | Replaced cylinder liner. |  |  |  |  |  |
|  | Installed pistons as per given standards. |  |  |  |  |  |
|  | Assembled the cylinder block |  |  |  |  |  |
|  | Installed cylinder head to the engine. |  |  |  |  |  |
|  | Installed intake manifold to cylinder head. |  |  |  |  |  |
|  | Installed exhaust manifold to cylinder head. |  |  |  |  |  |
|  | Set up the Engine timing. |  |  |  |  |  |
|  | Started the engine and checked up the engine performance. |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Auto & Farm Machinery Technician |
| --- | --- |
| Competency Standard | 0716MSA1801 Comply Health and Safety Guidelines. 0716MSA1802 Follow the Communication Policy/Procedure at the Workplace. 0716MSA1803 Interpret Drawings. 0716MSA1804 Perform Bench Work. 0716MSA1805 Perform Repair/Maintenance of Petrol Engines. 0716MSA1806 Perform Repair/Maintenance of Diesel Engines. 0716MSA1807 Perform Repair/Maintenance of Suspension, Steering & Braking System. 0716MSA1808 Perform Repair/Maintenance of Transmission System. 0716MSA1809 Perform Repair/Maintenance of Farm Machinery. 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements. 0716MSA1811 Perform Repair/Maintenance of Electric Implements. 0716MSA1812 Adopt Green Practices at Workplace. |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe about personal safety? |  |  |
|  | Define First Aid treatment methods? |  |  |
|  | Describe Conflict resolution strategies? |  |  |
|  | Define Types of lines and dimensions? |  |  |
|  | Explain Different types of hacksaw blades and its use? |  |  |
|  | Explain the Engine Cycle? |  |  |
|  | What is Electronic Fuel Injection (EFI) System? |  |  |
| - 8 | Describe the Constant Velocity (CV) joint? |  |  |
|  | Describe the gear ratio and noise? |  |  |
|  | Describe how to maintain the Combine harvester? |  |  |
|  | Explain the functions of laser land leveler? |  |  |
|  | Describe the Techniques for cleaning and calibrating the equipment? |  |  |
|  | Elaborate the Importance of waste reduction strategies? |  |  |
|  | Describe gear mechanism? |  |  |
| Feedback to the Candidate |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature _________________ |  |  |  |  |