|  | Assessment Evidence Guide For Auto & Farm Machinery Technician (One Year Program) Summative Assessment |
| --- | --- |
|  | Date 4-9 November 2024 Lahore |
|  | National Vocational & Technical Training Commission, Islamabad |

Instruction Sheet for the Candidate

| Qualification | Auto & Farm Machinery Technician |
| --- | --- |
| Competency Standard | 0716MSA1801 Comply Health and Safety Guidelines 0716MSA1802 Follow Communication Policy/Procedure at the Workplace 0716MSA1803 Interpret Drawings 0716MSA1804 Perform Bench Work 0716MSA1805 Perform Repair/Maintenance of Petrol Engines 0716MSA1806 Perform Repair/Maintenance of Diesel Engines 0716MSA1807 Perform Repair/Maintenance of suspension, steering and braking system 0716MSA1808 Perform Repair/Maintenance of Transmission system 0716MSA1809 Perform Repair/Maintenance of Farm Machinery 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements 0716MSA1811 Perform Repair/Maintenance of Electric Components 0716MSA1812 Adopt Green Practices at Workplace |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): A) Perform calibration of laser leveler B) Replace chain of reaper |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Use Personal Protective Equipment. - Select tools & equipment to perform said task - Perform calibration of laser leveler - Replace chain of reaper - Perform post inspection of implements |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Auto & Farm Machinery Technician |
| Competency Standard | 0716MSA1801 Comply Health and Safety Guidelines 0716MSA1802 Follow Communication Policy/Procedure at the Workplace 0716MSA1803 Interpret Drawings 0716MSA1804 Perform Bench Work 0716MSA1805 Perform Repair/Maintenance of Petrol Engines 0716MSA1806 Perform Repair/Maintenance of Diesel Engines 0716MSA1807 Perform Repair/Maintenance of suspension, steering and braking system 0716MSA1808 Perform Repair/Maintenance of Transmission system 0716MSA1809 Perform Repair/Maintenance of Farm Machinery 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements 0716MSA1811 Perform Repair/Maintenance of Electric Components 0716MSA1812 Adopt Green Practices at Workplace |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | A) Perform calibration of laser leveler B) Replace chain of reaper |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| Task A |  |  |
| - Collect laser leveler equipment according to Sops |  |  |
| - Use personal protective equipment |  |  |
| - Place tripod according to instruction |  |  |
| - Attach transmitter according to SOPs |  |  |
| - Attach laser receiver with leveler blade |  |  |
| - Adjust receiver height according to given instructions |  |  |
| - Connect laser receiver to control box |  |  |
| - Set receiver sensitivity for required precision |  |  |
| - Establish reference point at desired elevation |  |  |
| - Perform test run |  |  |
| - Perform post check operations |  |  |
| Task B |  |  |
| - Locate chain cover and remove guards or shields to access chain. |  |  |
| - Loosen chain tensioner to release tension on the chain. |  |  |
| - Slide tensioner away from the chain if needed. |  |  |
| - Remove master link of chain |  |  |
| - Remove old chain from the sprockets. |  |  |
| - Inspect sprockets for wear or damage |  |  |
| - Clean sprockets to remove any dirt, debris, or old lubricant. |  |  |
| - Place new chain around the sprockets |  |  |
| - Join chain ends using the master link |  |  |
| - Ensure chain has slight slack but is not too loose. |  |  |
| - Apply lubricant to the chain for smooth operation |  |  |
| - Reinstall any guards or shields |  |  |
| - Perform test run |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: ______________________________

Assessors Judgment Guide

| Qualification | Auto & Farm Machinery Technician |
| --- | --- |
| Competency Standard | 0716MSA1801 Comply Health and Safety Guidelines 0716MSA1802 Follow Communication Policy/Procedure at the Workplace 0716MSA1803 Interpret Drawings 0716MSA1804 Perform Bench Work 0716MSA1805 Perform Repair/Maintenance of Petrol Engines 0716MSA1806 Perform Repair/Maintenance of Diesel Engines 0716MSA1807 Perform Repair/Maintenance of suspension, steering and braking system 0716MSA1808 Perform Repair/Maintenance of Transmission system 0716MSA1809 Perform Repair/Maintenance of Farm Machinery 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements 0716MSA1811 Perform Repair/Maintenance of Electric Components 0716MSA1812 Adopt Green Practices at Workplace |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: _____________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | A) Perform calibration of laser leveler B) Replace chain of reaper |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| Task A |  |  |  |  |  |  |
|  | Collect laser leveler equipment according to Sops |  |  |  |  |  |
|  | Use personal protective equipment |  |  |  |  |  |
|  | Place the tripod according to instruction |  |  |  |  |  |
|  | Attach transmitter according to Sops |  |  |  |  |  |
|  | Attach laser receiver with leveler blade |  |  |  |  |  |
|  | Adjust receiver height according to given instructions |  |  |  |  |  |
|  | Connect laser receiver to control box |  |  |  |  |  |
|  | Set receiver sensitivity for required precision |  |  |  |  |  |
|  | Establish reference point at desired elevation |  |  |  |  |  |
|  | Perform test run |  |  |  |  |  |
|  | Perform post check operations |  |  |  |  |  |
| Task B |  |  |  |  |  |  |
|  | Locate chain cover and remove guards or shields to access chain. |  |  |  |  |  |
|  | Loosen chain tensioner to release tension on the chain. |  |  |  |  |  |
|  | Slide tensioner away from the chain if needed. |  |  |  |  |  |
|  | Remove master link of chain |  |  |  |  |  |
|  | Remove old chain from the sprockets. |  |  |  |  |  |
|  | Inspect sprockets for wear or damage |  |  |  |  |  |
|  | Clean sprockets to remove any dirt, debris, or old lubricant. |  |  |  |  |  |
|  | Place new chain around the sprockets |  |  |  |  |  |
|  | Join chain ends using the master link |  |  |  |  |  |
|  | Ensure chain has slight slack but is not too loose. |  |  |  |  |  |
|  | Apply lubricant to the chain for smooth operation |  |  |  |  |  |
|  | Reinstall any guards or shields |  |  |  |  |  |
|  | Perform test run |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Auto & Farm Machinery Technician |
| --- | --- |
| Competency Standard | 0716MSA1801 Comply Health and Safety Guidelines 0716MSA1802 Follow Communication Policy/Procedure at the Workplace 0716MSA1803 Interpret Drawings 0716MSA1804 Perform Bench Work 0716MSA1805 Perform Repair/Maintenance of Petrol Engines 0716MSA1806 Perform Repair/Maintenance of Diesel Engines 0716MSA1807 Perform Repair/Maintenance of suspension, steering and braking system 0716MSA1808 Perform Repair/Maintenance of Transmission system 0716MSA1809 Perform Repair/Maintenance of Farm Machinery 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements 0716MSA1811 Perform Repair/Maintenance of Electric Components 0716MSA1812 Adopt Green Practices at Workplace |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the purpose of wearing personal protective equipment (PPE) during maintenance tasks? |  |  |
|  | Name two green practices you should follow when working on agricultural machinery. |  |  |
|  | During bench work, what is one common issue you should look for when inspecting engine components? |  |  |
|  | What does an unusual sound, like knocking, from an engine typically indicate? |  |  |
|  | How do you check for proper torque when reassembling engine parts? |  |  |
|  | Why is it important to calibrate agricultural implements like sprayers or seed drills? |  |  |
|  | Which part of a suspension system typically needs regular inspection? |  |  |
|  | How can you identify if a brake pad needs replacement? |  |  |
|  | What does a clogged air filter in an engine affect? |  |  |
|  | Why is it important to document each maintenance task? |  |  |
|  | What is the purpose of a Power Take-Off (PTO) shaft on farm machinery? |  |  |
|  | When should you replace the seals and hoses in a sprayer? |  |  |
|  | How does regular lubrication of moving parts affect machinery? |  |  |
|  | What is a common sign that a machine’s transmission system needs maintenance? |  |  |
|  | What should you do if you observe excess smoke from the exhaust after engine repair? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________