|  | Perform Measurement, Calculation and Checking Techniques |
| --- | --- |

Learning Outcomes:

After Completion of this learning module, trainee will be able to:

- Perform basic unit calculations

- Perform measurement using tools

- Solve formulas of cutting speed, feed of machines

- Perform checking using gauges

|  | Perform Basic Unit Calculations |
| --- | --- |

Learning Outcomes:

After Completion of this learning module, trainee will be able to:

Trainee will be able to:

Perform arithmetic operations addition, subtraction, multiplication, and division.

Perform inter conversion of measuring units.

Document the results as per requirement.

Solution of Numerical problems of surface area, volume, density and cost of material

Fundamental units & Conversion of different units

In engineering, fundamental units (also called base units) are the standard units of measurement from which other derived units are derived. These units are part of the International System of Units (SI). Understanding how to convert between units and apply them in calculations is essential for various fields, such as engineering, and everyday life.

Fundamental Units in the SI System:

The SI system defines seven fundamental units from which all other units can be derived. These include:

Remember

Do you know about SI system

Length (meter)

- Symbol: m

- The basic unit of distance or length.

Mass (kilogram)

- Symbol: kg

- The basic unit of mass.

Time (second)

- Symbol: s

- The basic unit of time.

Electric Current (ampere)

- Symbol: A

- The basic unit of electric current.

Temperature (kelvin)

- Symbol: K

- The basic unit of temperature.

Amount of Substance (mole)

- Symbol: mol

- The basic unit for the quantity of substance.

Luminous Intensity (candela)

- Symbol: cd

- The basic unit for luminous intensity.

- These fundamental units can be used to derive many other units, such as speed (meter per second), force (newton), and energy (joule).

Unit Conversion:

Converting between different units is essential for solving practical problems. The conversion factor is used to multiply or divide a measurement to convert it to another unit.

Basic Conversion Method:

To convert from one unit to another, multiply by the appropriate conversion factor or divide when converting to a larger unit.

For example:

1 meter = 100 centimeters: To convert 5 meters to centimeters, multiply by 100:

5 m=5×100=500 cm

1 kilometer = 1,000 meters: To convert 2.5 kilometers to meters, multiply by 1,000:

2.5 km=2.5×1,000=2,500

1 kilogram = 1,000 grams:

To convert 3 kilograms to grams, multiply by 1,000:

3 kg=3×1,000=3,000 g

Common Unit Conversion Factors:

Length:

- 1 kilometer (km) = 1,000 meters (m)

- 1 meter (m) = 100 centimeters (cm)

- 1 centimeter (cm) = 10 millimeters (mm)

- 1 inch (in) = 2.54 centimeters (cm)

- 1 foot (ft) = 12 inches (in)

- 1 mile (mi) = 1,609 meters (m)

Mass:

- 1 kilogram (kg) = 1,000 grams (g)

- 1 gram (g) = 1,000 milligrams (mg)

- 1 ton (US) = 907.18474 kilograms (kg)

- 1 pound (lb) = 0.45359237 kilograms (kg)

Time:

- 1 minute (min) = 60 seconds (s)

- 1 hour (h) = 60 minutes = 3,600 seconds (s)

- 1 day (d) = 24 hours

- 1 year (yr) = 365.25 days (including leap year adjustment)

Speed:

- 1 kilometer per hour (km/h) = 1,000 meters per hour (m/h)

- 1 meter per second (m/s) = 3.6 kilometers per hour (km/h)

- 1 mile per hour (mph) = 1.609 kilometers per hour (km/h)

Area:

- 1 square kilometer (km²) = 1,000,000 square meters (m²)

- 1 hectare (ha) = 10,000 square meters (m²)

- 1 square meter (m²) = 10,000 square centimeters (cm²)

- 1 acre = 4,840 square yards (yd²) = 4,047 square meters (m²)

Volume:

- 1 liter (L) = 1,000 milliliters (mL)

- 1 liter (L) = 1 cubic decimeter (dm³)

- 1 cubic meter (m³) = 1,000 liters (L)

- 1 gallon (US) = 3.78541 liters (L)

Force:

- 1 newton (N) = 1 kilogram meter per second squared (kg·m/s²)

- 1 pound-force (lbf) = 4.44822 newtons (N)

Understanding fundamental units and how to convert between them is essential for working with measurements in everyday tasks, engineering, and scientific research. Mastery of unit conversions ensures accurate calculations and consistency when working with different types of data.

3.1.2 Arithmetic operations

Arithmetic operations in basic unit calculations are essential for performing measurements and conversions in science, engineering, and various fields. These operations can involve simple addition, subtraction, multiplication, division, and applying the rules of significant figures and dimensional analysis when working with units.

Here’s a guide on how to perform arithmetic operations with basic units:

Addition and Subtraction of Units

When adding or subtracting measurements, units must be the same. If the units are different, you must first convert them into the same unit before performing the operation.

Example 1: Addition (Same Units)

- Suppose you have two lengths:

- L1=5 m

- L2=3 m

- To add these, you simply add the values:

- L1+L2=5 m+3 m=8 m

Example 2: Subtraction (Same Units)

- Suppose you have two lengths:

- L1=10 m

- L2=4 m

- To subtract the lengths:

- L1−L2=10 m−4 m=6

Example 3: Addition (Different Units)

- Suppose you have a length of 3 m and 50 cm and you want to add them.

- Convert 50 cm

- 50 cm=0.5 m

- Now add:

- 3 m+0.5 m=3.5 m

Multiplication of Units

When multiplying units, multiply the numerical values and apply the rule of exponents to the units. The resulting unit will be the combination of the two units.

Example 1: Multiplication (Length × Time)

- If you multiply length and time, you get speed (distance per time).

- L=5 m

- T=3s

- Multiply the numerical values and combine the units:

- Speed=L×T=5 m×3 s=15 ms

Division of Units

When dividing units, divide the numerical values and apply the rule of exponents to the units. The resulting unit will have the denominator and numerator units in the corresponding relationship.

Example 1: Division (Distance ÷ Time)

- If you divide distance by time, you get speed.

- D=100 m

- T=20 s

- Divide the numerical values and combine the units:

- Speed=DT=100 m20 s=5 m/s

Handling Unit Prefixes

Unit prefixes (such as kilometer, centimeter, millimeter, etc.) modify the base unit by powers of 10. These are important for converting between smaller or larger units.

Common Unit Prefixes:

- kilometer- (k): 1,000 (e.g., 1 km = 1,000 m)

- centimeter- (c): 1/100 (e.g., 1 cm = 0.01 m)

- millimeter- (m): 1/1,000 (e.g., 1 mm = 0.001 m)

- micro- (µ): 1/1,000,000 (e.g., 1 µm = 0.000001 m)

Example: Converting Units Using Prefixes

- To convert 3.5 km

- 3.5 km=3.5×1,000=3,500

Conclusion

- Performing arithmetic operations with basic units involves:

- Ensuring units are consistent (convert if necessary).

- Using the appropriate operation for each type of calculation (addition, subtraction, multiplication, division).

- Applying dimensional analysis to ensure units match on both sides of equations.

- Understanding and using unit prefixes for large and small unit conversions.

- Mastering these operations ensures accuracy in scientific calculations and real-world problem-solving.

Important principles of measuring

When performing basic unit calculations, it's important to understand the following principles of measurement:

Accuracy and Precision:

- Accuracy refers to how close a measurement is to the true or accepted value.

- Precision refers to how consistently you can obtain the same measurement under the same conditions.

- In unit calculations, accurate measurements lead to reliable results, while precise measurements ensure repeatability.

Units and Conversions:

- Base Units: Start with the standard base units in the International System of Units (SI), like meters (m) for length, kilograms (kg) for mass, and seconds (s) for time.

- Derived Units: These are combinations of base units, like meters per second (m/s) for velocity or newton’s (N) for force.

- Unit Conversions: Be sure to convert between different units when performing calculations. This involves multiplying or dividing by conversion factors, e.g., 1 kilometer (km) = 1,000 meters (m).

Significant Figures:

- When performing calculations, significant figures reflect the precision of a measurement.

- The number of significant figures in your final result depends on the numbers you started with.

- Multiplication and Division: The result should have as many significant figures as the number with the least significant figures.

- Addition and Subtraction: The result should have as many decimal places as the number with the least decimal places.

Dimensional Analysis:

- This method involves checking the units at each step of a calculation to ensure that they are consistent and that the calculation makes sense.

- Dimensional analysis can help convert units, verify equations, and avoid mistakes.

Order of Operations:

- Follow the correct sequence for calculations: parentheses first, then exponents, multiplication/division (from left to right), and finally addition/subtraction (from left to right). This ensures that you are handling complex unit conversions correctly.

Error Analysis:

- Always consider potential sources of error in measurement, such as instrument limitations or environmental factors.

- Calculate and account for errors, particularly when working with scientific data.

Consistency of Units:

- Keep all units consistent when performing calculations. For example, if you're working with distances in meters, use meters for all other related measurements (speed, velocity, etc.), unless converting.

Example Calculation:

- If you're calculating the speed of a car that travels 100 kilometers in 2 hours, you'd proceed like this:

- Convert kilometers to meters: 100 km = 100,000 meters.

- Convert hours to seconds: 2 hours = 7,200 seconds.

- Use significant figures to round the final result if necessary.

- By understanding these principles, you ensure that your unit calculations are both accurate and reliable.

Calculation of Numerical problems.

Fraction

A fraction is any number that is less than 1 and is written as the ratio of two numbers using a slash (/).

Decimal System:

- A number system where the base (denominator) is 10 is called the decimal system.

- The denominator of a decimal fraction is 10, 100, 1000, etc.

Addition:

Addition of Fractions

- Find the least common denominator (LCD) of all the fractions.

- Divide the least common denominator by the denominator of the first fraction.

- If necessary, simplify the fraction.

Addition of Decimal numbers

- The addition of decimal numbers is the same as the addition of whole numbers; the only thing to determine is the placement of the decimal point.

- Arrange the numbers in such a way that the decimal points are aligned under each other.

- Add each number just like you would add whole numbers.

- Place the decimal point exactly in the same position below.

Subtraction

Subtraction of Fractions

- Make sure that the denominators of the numbers being subtracted are the same.

- Subtract the numerators and place the result over the common denominator.

- If necessary, simplify the fraction.

==

Subtraction of Decimals

- The subtraction of decimal numbers is the same as the subtraction of whole numbers; the only thing to determine is the placement of the decimal point.

- Subtract the smaller number from the larger number.

- Solve: 62.1251 - 24.102

- Arrange the numbers in such a way that the decimal points are aligned under each other.

- Subtract each column just like you would with whole numbers.

- Place the decimal point exactly below.

Multiplication

Multiplication of Fractions:

- First, multiply the numerators.

- Then, multiply the denominators.

- Finally, simplify the fraction.

= =

Multiplication of Decimal Numbers:

- Multiply the decimal numbers as if they were whole numbers, without considering the decimal point.

- Count the number of digits to the right of the decimal point in both numbers.

- Place the decimal point in the answer after counting the same number of digits from the right.

Division

Division of Fractions:

The division of fractions is done by taking the denominators to the top and multiplying.

Exercises 1: If the distance between the centers of all the circles in the following figure is equal, find B, A, and C.

Exercise 2:

The following figure shows steel bars of different lengths but the same width.

- What will be the total length after bars A and C are joined together?

- What will be the total length after all the bars are joined together?

- If bar C is divided into three equal parts, what will be the length of each part?

- If the length of D is equivalent to 15 bars joined together, what will be their total length?

- How much of bar D needs to be cut to make it the same length as bar B?

Exercise 3:

Find the values of X and Y in the following figure.

Exercise 4:

Find the areas of the different sectors of the circle given in the figure.

Exercise 5:

Find the complete angle in the given drawing.

Trigonometry

Introduction

Trigonometry represents the relationship between the angles and sides of a triangle.

In the given right-angled triangle, based on Pythagoras' theorem:

Hypotenuse2 = Base2 + Perpendicular2

Trigonometric Ratios

Tan A=PerpendicularBase

Sin A=PerpendicularHypotenuse

Cos A=BaseHypotenuse

Exercise number 1:

Find B, A, and C in the given diagram.

Exercise number 2:

Find B, A, and C from both sides

Exercise number 3:

- Solve the following questions according to the shape given below:

- Find the length of the chord L if the angle A = 30° and the diameter D = 25 mm.

- Find the length of the chord L if the angle A = 45° and the diameter D = 25 mm.

- Find the diameter D if the angle A = 30° and the length of the chord L = 20 mm.

- Find the diameter D if the angle A = 45° and the length of the chord L = 35 mm.

- Find the diameter D if the angle A = 60° and the length of the chord L = 40 mm.

Practical Activity:

- Calculate conversion of units.

- Material required:

- White Board

- Marker

- Papper

Learning place:

- Class room

|  | Perform Measurement using tools |
| --- | --- |

Learning Outcomes:

After completion of this Learning Unit, trainee will be able to:

- Select appropriate measuring tool.

- Inspect the tool

- Conduct measurement using various tools. (Vernier Caliper, Micrometer etc)

- Record the measurements.

Types of measuring instruments linear measuring instrument Angular measuring instrument

Types of Measuring Instruments

Linear Measuring Instruments

### Remember

Correct use of measuring tool is key of success

These instruments are used to measure the straight-line distances or lengths. They include:

- Ruler or Scale: A basic tool for measuring short lengths (in mm, cm, inches).

- Vernier Caliper: Used for measuring internal and external dimensions with high precision.

- Micrometer: A precise instrument for measuring small objects or thicknesses (with higher accuracy than a caliper).

- Measuring Tape: Used for measuring larger distances (in meters, feet, or inches).

- Height Gauge: Used for measuring the height of objects or distances relative to a reference surface.

- Laser Distance Meter: A digital tool for measuring longer distances using laser technology.

Angular Measuring Instruments

These instruments are used to measure angles or angular displacement. They include:

- Protractor: A simple tool for measuring angles in degrees.

- Angle Gauge: A precision instrument used for setting and measuring angles in mechanical work.

Technique & observation of checking Zero Error

Technique & Observation of Checking Zero Error in Measuring Tools

- Zero error is the error that occurs when a measuring instrument doesn’t read zero when the measured object is absent or at the reference point. This error can arise due to wear, misalignment, or imperfections in the instrument. Checking for zero error is crucial for obtaining accurate measurements.

Technique for Checking Zero Error:

For Vernier Calipers:

Procedure:

- Close the jaws of the Vernier caliper completely without any object in between them.

- Look at the reading on the main scale and the Vernier scale.

- If the reading is exactly zero, there is no zero error.

- If there is any offset (positive or negative), that’s your zero error.

Observation:

If the caliper reads a small value (positive or negative), this means there is a zero error that needs to be accounted for when measuring objects.

For Micrometers:

Procedure:

- Close the spindle of the micrometer by gently rotating the ratchet until the anvil and spindle meet.

- Check the micrometer reading at the zero position.

- If the reading is not exactly zero, there is a zero error.

Observation:

- Any deviation from the zero reading indicates a zero error, which could be positive (too much measurement) or negative (not enough measurement).

- Some micrometers have a “ratchet stop” feature, which should click when the correct pressure is applied.

For Measuring Tapes:

Procedure:

- Stretch the measuring tape along a straight line and ensure that the tape’s start is at the zero mark.

- Check the zero point to ensure the tape is not damaged or stretched, leading to an incorrect zero.

- If the start of the tape is not at the zero, adjust for the error.

Observation:

- Any stretching or misalignment of the starting point will cause a zero error.

For Theodolites:

Procedure:

- Set up the theodolite on a level surface.

- Ensure that the instrument is calibrated correctly, with no horizontal or vertical angle.

- Observe the reading to confirm that it reads exactly zero for both horizontal and vertical scales when not measuring an angle.

Observation:

- A reading that does not indicate zero suggests the need for calibration or a zero error adjustment.

For Protractors:

Procedure:

- Align the arms of the protractor with the zero line.

- Ensure the pointer aligns exactly at zero.

- If there is a gap or deviation, note the error.

Observation:

- A protractor should read zero when not measuring any angle. A misalignment could indicate a zero error.

Types of Zero Errors:

Positive Zero Error:

- The instrument reads a value greater than zero when no object is measured. This requires subtracting the error from all subsequent measurements.

Negative Zero Error:

- The instrument reads a value less than zero when no object is measured. This requires adding the error to all subsequent measurements.

How to Correct Zero Error:

For Vernier Calipers & Micrometers:

- Adjust the reading of the instrument by subtracting or adding the zero error before taking measurements.

- If the error is consistently the same, it can be factored into calculations.

For Digital Instruments:

- Many digital measuring tools have a reset or calibration function to correct any zero error. Simply reset the device to zero before taking measurements.

For Manual Tools (e.g., Protractors, Tapes):

- Always ensure the tool is properly aligned and calibrated before measurement.

- If the tool has a known error, adjust calculations to compensate.

Important Considerations:

- Regular Calibration: Regularly calibrate measuring instruments to minimize zero errors.

- Environmental Factors: Temperature changes and humidity can affect measuring tools and cause zero errors, especially in precision instruments.

- Maintenance: Ensure that tools are clean, undamaged, and stored properly to avoid zero errors.

- By routinely checking and correcting for zero errors, you can ensure the accuracy and reliability of your measurements.

Proper Use & Holding Techniques during the Performance

Using measuring tools correctly and holding them in the right manner ensures accurate measurements, reduces errors, and extends the life of the tools. Here are the proper techniques for using and holding common measuring tools:

### Remember

Correct use of measuring tool is key of success

Vernier Calipers

Use Technique:

Before Measurement:

- Ensure the instrument is clean and free of dust or dirt that can affect accuracy.

- Verify that the caliper is free from any zero error (refer to the previous section for checking zero error).

- Ensure the scale is calibrated and the jaws are not damaged.

During Measurement:

- Open the caliper jaws slightly and insert the object being measured.

- Close the jaws gently around the object, ensuring they touch lightly and do not apply excessive pressure.

- Check the reading from the main scale and the Vernier scale.

- Take care to read the scales at eye level to avoid parallax error.

Holding Technique:

- Hold the calipers by the frame to avoid putting excessive pressure on the sliding components.

- Never force the jaws of the calipers onto the object; light contact is sufficient.

- Keep the calipers steady and avoid tilting them to ensure the scales are read accurately.

Micrometer

Use Technique:

Before Measurement:

- Ensure the micrometer is clean and free from any foreign objects.

- Check for zero error by closing the spindle and noting the reading.

- Verify that the micrometer is calibrated.

During Measurement:

- Place the object (e.g., a shaft or wire) between the spindle and anvil.

- Turn the thimble or ratchet to move the spindle gently until the object is lightly clamped.

- Use the ratchet mechanism to apply a consistent force, ensuring accurate measurements.

- Read the measurement on the thimble and sleeve scale.

Holding Technique:

- Hold the micrometer with both hands: one on the frame and the other to rotate the thimble.

- Always use the ratchet or micrometer's built-in torque control to apply consistent pressure, preventing over-tightening.

- When measuring small objects, avoid touching the micrometer’s measuring surfaces with your fingers to avoid transferring heat and affecting accuracy.

Measuring Tape

Use Technique:

Before Measurement:

- Ensure the tape is clean, free from wrinkles, and not damaged.

- Verify the starting point (zero mark) is correct.

During Measurement:

- Unroll the tape slowly, ensuring it remains straight and taut along the length to be measured.

- Avoid twisting the tape to maintain accuracy.

- Mark the point where the measurement ends.

- Take the reading at eye level to avoid parallax error.

Holding Technique:

- Hold the metal hook at the end of the tape firmly against the reference point (e.g., a corner or edge of an object).

- When extending the tape, use both hands to keep it straight and level.

- Avoid letting the tape slide or slip during measurement, especially when measuring long distances.

Theodolite

Use Technique:

Before Measurement:

- Set up the theodolite on a stable tripod and ensure it is level.

- Verify the zero reading for both horizontal and vertical angles.

During Measurement:

- Focus the theodolite’s telescope on the target to get a clear line of sight.

- Adjust the horizontal and vertical controls to aim the telescope precisely at the point of measurement.

- Take the reading after ensuring the instrument is stable.

Holding Technique:

- Hold the theodolite steady while adjusting the angle to prevent vibrations that could affect accuracy.

- Do not touch the telescope directly while taking measurements; adjust using the fine focus and angle controls.

- Ensure the tripod is firm and does not shift during use.

Protractor

Use Technique:

Before Measurement:

- Ensure the protractor is free from scratches or dust that could distort measurements.

- Confirm the protractor is placed on a flat surface.

During Measurement:

- Place the center of the protractor (the hole) at the vertex of the angle.

- Align one arm of the angle with the zero line of the protractor.

- Measure the angle by reading where the other arm intersects the scale.

Holding Technique:

- Hold the protractor flat against the surface where the angle is measured.

- Keep the protractor steady and aligned with the angle to avoid errors in reading.

- Avoid tilting or shifting the protractor while taking the measurement.

Level

Use Technique:

Before Measurement:

- Check that the bubble inside the level is centered when the tool is placed on a flat surface.

- Ensure the level is clean and free of debris that may affect the reading.

During Measurement:

- Place the level on the object or surface being checked.

- Adjust the position of the level until the bubble is centered in the vial.

- For more accurate readings, use the level in multiple positions (horizontal and vertical).

Holding Technique:

- Hold the level firmly against the object or surface to ensure proper alignment.

- Avoid any tilting or shifting of the level during measurement, as this can cause inaccuracies.

General Guidelines for Proper Use and Holding of Measuring Tools:

- Consistency: Always use consistent pressure and handling methods to avoid errors due to variation in force applied or alignment.

- Alignment: Ensure the measuring tool is properly aligned with the object being measured. Misalignment can lead to incorrect readings.

- Avoiding External Interference: Hold the tool securely to avoid accidental shifts, which can result in incorrect measurements. Additionally, avoid touching the measurement surfaces with your fingers, as this can introduce errors due to the transfer of heat or pressure.

- Check for Calibration Regularly: Tools should be calibrated regularly to maintain accuracy.

- Use the Right Tool for the Job: Ensure you're using the appropriate tool for the measurement, whether it's a caliper, micrometer, tape measure, or level, depending on the required precision.

By following these techniques, you will ensure that your measurements are accurate, reliable, and consistent, leading to successful outcomes in your work.

3.2.4 Practical Activity:

Perform measurements using different measuring instruments

Material required:

- Steel Rule

- Vernier Calliper

- (Dial, Digital)

- Micrometer

- (Inside & Outside, Depth, Thread)

- Plug Gauges of different size(Go Not Go)

- Snap Gauges of different size(Go Not Go)

- Bore Gauges of different limit.

- Telescope gauges

- Mechanical Comparators

- Height Gauges

Learning place:

- Class room and workshop

|  | Solve formulas of cutting speed, feed of machines |
| --- | --- |

Learning Outcomes:

- After completion of this Learning Unit, trainee will be able to:

- Identify material type, tool and operations.

- Apply speed and feed formula for CNC Lathe.

- Apply speed and feed formula for CNC Milling

Factor affecting on cutting speed, feed & depth of cut

Factors Affecting Cutting Speed, Feed, and Depth of Cut

In machining operations, cutting speed, feed, and depth of cut are key parameters that determine the efficiency, surface finish, and overall quality of the process. These parameters are influenced by various factors related to the material being machined, the tool used, and the machine's characteristics. Here’s an overview of the main factors that affect cutting speed, feed, and depth of cut:

### Remember

Do you know Machine setting mean machine RPM Cutting speed and feed rate

Cutting Speed (V)

Cutting speed refers to the rate at which the cutting tool engages the workpiece material. It is usually measured in meters per minute (m/min) or feet per minute (ft/min).

Factors Affecting Cutting Speed:

Material of the Work piece:

- Softer materials (e.g., aluminum, plastics) can generally be cut at higher speeds, while harder materials (e.g., steel, titanium) require lower speeds to prevent tool wear or failure.

Material of the Cutting Tool:

- Harder and tougher tool materials, such as carbide or high-speed steel (HSS), can withstand higher cutting speeds.

- Tool coatings (e.g., TiN, TiAlN) also improve the cutting speed by reducing friction and increasing tool life.

Tool Geometry:

- The rake angle, clearance angle, and cutting edge radius affect how efficiently the tool can cut, thereby influencing the maximum cutting speed.

Coolant/Lubrication:

- The presence of coolant or lubrication reduces friction and heat generation, allowing higher cutting speeds.

Machine Power:

- Machines with higher horsepower or torque can maintain higher cutting speeds without stalling or damaging the tool.

Surface Finish Requirements:

- For a smoother surface finish, lower cutting speeds are often used, while rough cuts can handle higher speeds.

Feed (f)

- Feed refers to the rate at which the cutting tool advances into the material during the machining process. It is usually measured in millimeters per revolution (mm/rev) or inches per revolution (in/rev).

Factors Affecting Feed:

Material of the Workpiece:

- Softer materials allow for higher feed rates, while harder materials require slower feed rates to avoid tool damage or excessive tool wear.

Cutting Tool Material and Geometry:

- The material and design of the cutting tool (e.g., the number of cutting edges) influence how much material can be fed into the cutting zone. Tools with multiple edges can handle higher feeds.

Depth of Cut:

- A higher feed rate can be used with a smaller depth of cut, as the tool is not removing as much material per pass. Conversely, with a larger depth of cut, a lower feed is often needed to reduce tool load.

Tool Wear:

- As the tool wears, its cutting efficiency decreases, requiring a reduction in feed to maintain quality and prevent damage.

Cutting Speed:

- Cutting speed and feed rate often have an inverse relationship. Higher cutting speeds typically require lower feed rates and vice versa.

Machine Rigidity:

- The stiffness and rigidity of the machine and tool setup influence the maximum allowable feed rate. If the machine is too flexible, excessive feed may lead to vibrations and poor surface finish.

Depth of Cut (d)

- Depth of cut refers to the thickness of the material being removed in a single pass. It is typically measured in millimeters (mm) or inches (in).

Factors Affecting Depth of Cut:

Material of the Workpiece:

- Softer materials allow for deeper cuts, while harder materials require shallower depths to prevent tool damage and excessive cutting forces.

Cutting Tool Material:

- Tool material affects how deep the cut can be taken without causing excessive wear. Harder tool materials like carbide can withstand deeper cuts.

Tool Geometry:

- The tool's shape, especially its cutting edge angle, can limit the maximum depth of cut. Steeper angles or stronger tools allow for deeper cuts.

Machine Power and Rigidity:

- Machines with more horsepower and greater rigidity can handle deeper cuts, while weaker machines might not be able to support heavy cutting without risking vibration or tool deflection.

Chip Removal:

- Higher depths of cut lead to thicker chips, which can increase cutting forces and heat. This may affect the choice of depth, especially when the chip removal is not efficient.

Workpiece Size:

- The size and shape of the workpiece may restrict the depth of cut, especially in smaller or irregularly shaped workpieces that require smaller cuts for safety and precision.

Tool Life and Wear:

- With prolonged cutting, tool wear can reduce the ability to take deep cuts. A worn tool requires a reduced depth of cut to avoid poor surface finish and tool breakage.

Other Influencing Factors

Cutting Fluid:

- The application of cutting fluids can significantly affect all three parameters (cutting speed, feed, and depth of cut). Coolants reduce heat and friction, allowing higher speeds, feeds, and depths of cut without damaging the tool or workpiece.

Cutting Environment:

- The machining environment, including temperature and humidity, can impact the cutting performance. High temperatures can cause thermal expansion, altering cutting tool geometry and material properties.

Workpiece Surface Finish:

- To achieve a smoother surface finish, lower cutting speeds, feeds, and depths of cut are often used. Higher material removal rates (larger depth of cut and higher feed) typically result in a rougher surface finish.

Tool Vibration:

- Excessive tool vibration (often caused by incorrect settings or machine instability) can limit the cutting speed, feed rate, and depth of cut, as it reduces machining accuracy and tool life.

Machine Condition:

- The general condition of the machine (e.g., wear on the spindle bearings, lack of proper alignment) influences how effectively and safely cutting parameters can be applied.

Summary of Relationships:

- Cutting Speed & Feed Rate: Higher cutting speeds generally require lower feed rates to maintain tool life and surface quality, while slower speeds allow for higher feed rates.

- Feed Rate & Depth of Cut: Larger depths of cut generally necessitate slower feed rates to manage cutting forces, while smaller depths can tolerate higher feed rates.

- Cutting Speed & Depth of Cut: Higher cutting speeds may allow for shallower cuts, as heat buildup can cause material deformation, while slower cutting speeds may enable deeper cuts.

By understanding these factors, operators can optimize cutting conditions to maximize efficiency, tool life, and the quality of the work piece.

Feed Rate for Milling Machine

Feed rate in milling refers to the speed at which the work piece is moved relative to the cutting tool. It is typically expressed in units of inches per minute (IPM) or millimeters per minute (mm/min), and it depends on several factors, including the cutting tool's material, the work piece material, cutting conditions, and the type of milling operation.

Formula for Feed Rate in Milling:

- The feed rate (F) can be calculated using the following formula:

- F=N×f×Z

Where:

- F = Feed rate (in/min or mm/min)

- N = Spindle speed (in RPM)

- f = Feed per tooth (in or mm)

- Z = Number of teeth on the cutting tool

Factors Affecting Feed Rate:

Spindle Speed (RPM):

- The speed of the milling machine spindle determines how quickly the cutting tool rotates. The relationship between the spindle speed and the feed rate is crucial, as the feed rate increases with the spindle speed (for a given feed per tooth and tool).

Feed per Tooth (f):

This refers to the amount of material the cutter advances during one revolution of the tool. It depends on:

- Tool geometry: A tool with more teeth will typically have a smaller feed per tooth, while tools with fewer teeth may use a larger feed.

- Material being cut: Softer materials generally allow for a higher feed rate, while harder materials require slower feed rates.

- Cutting conditions: The feed rate should be optimized for the depth of cut, cutting speed, and tool wear.

Number of Teeth on the Cutter (Z):

- Tools with more teeth can handle a higher feed rate as the material is distributed over multiple cutting edges, reducing the load on each individual tooth.

Tool Material and Coating:

- High-speed steel (HSS) tools may have different feed rates compared to carbide or coated tools. Carbide tools, being harder and more wear-resistant, can typically operate at higher feed rates.

Material of the Workpiece:

- Softer materials (like aluminum) allow for higher feed rates, while harder materials (such as steel or titanium) usually require slower feed rates to avoid tool wear or breakage.

Cutting Conditions:

- Depth of cut: A deeper cut typically requires a lower feed rate to prevent excessive cutting forces, while shallow cuts may allow for higher feed rates.

- Type of operation: Slotting or full-profile cuts may require slower feed rates compared to light milling or finishing operations.

Machine Capability:

- The rigidity, horsepower, and stability of the milling machine affect the maximum feed rate. High-powered, rigid machines can handle higher feed rates without sacrificing accuracy or tool life.

General Guidelines for Selecting Feed Rate:

- Roughing: For rough cuts, you typically use higher feed rates to quickly remove material, but with shallower cuts and lower spindle speeds.

- Finishing: For finishing cuts, the feed rate is usually lower to achieve a finer surface finish and higher accuracy.

- Deep Cuts: For deeper cuts, you need to reduce the feed rate to avoid excessive tool wear and maintain precision.

Typical Feed Rates (General Guideline):

| Material | Feed per Tooth (f) (mm/tooth) | Feed Rate (mm/min) |
| --- | --- | --- |
| Aluminum | 0.15 – 0.35 | 500 – 1000 |
| Brass | 0.15 – 0.30 | 400 – 900 |
| Steel (Mild) | 0.05 – 0.15 | 150 – 300 |
| Stainless Steel | 0.03 – 0.08 | 100 – 200 |
| Cast Iron | 0.10 – 0.20 | 250 – 600 |
| Titanium | 0.02 – 0.05 | 80 – 150 |

Important Notes:

- Too high of a feed rate can result in poor surface finish, tool wear, and excessive cutting forces that could cause the tool or workpiece to deform.

- Too low of a feed rate can lead to unnecessary heat buildup, excessive tool wear, and inefficient material removal.

- Coolants and lubrication can help increase feed rates by reducing heat and friction, allowing for higher speeds and feeds without damaging the tool or workpiece.

In summary, the feed rate in milling operations is a critical parameter for achieving efficient cutting and maintaining tool life and part quality. Proper calculation and adjustment based on the factors mentioned above help optimize milling processes.

Formulas for Cutting Speed, Feed, and Turning Time in a Specific Job

In machining, especially in turning operations, cutting speed, feed rate, and turning time are important factors that affect the overall performance, tool life, and efficiency of the machining process. Below are the key formulas used to calculate these parameters?

Cutting Speed (V)

Cutting Speed is the speed at which the cutting tool moves along the circumference of the workpiece. It is usually expressed in meters per minute (m/min) or feet per minute (ft/min).

Formula:

V=π×D×N

Where:

- V = Cutting speed (m/min or ft/min)

- D = Diameter of the workpiece (m or ft)

- N = Spindle speed (RPM)

Feed Rate (F)

Feed Rate is the distance the tool advances per revolution of the workpiece. It is usually expressed in millimeters per minute (mm/min) or inches per minute (in/min).

Formula:

F=f×N×Z

Where:

- F = Feed rate (mm/min or in/min)

- f = Feed per tooth (mm or in)

- N = Spindle speed (RPM)

- Z = Number of cutting edges or teeth on the tool

Example:

If the feed per tooth is 0.05 mm, the spindle speed is 1200 RPM, and the tool has 4 teeth, the feed rate would be:

- F=0.05×1200×4=240 mm/min

Turning Time (T)

Turning Time is the time it takes to complete one revolution of the workpiece during a turning operation. It is calculated based on the length of the cut and the feed rate.

Formula:

T=LFT

Where:

- T = Turning time (min or hours)

- L = Length of the workpiece (mm or inches)

- F = Feed rate (mm/min or in/min)

Alternatively, if the workpiece is rotating, the time for one full revolution can be calculated based on the spindle speed and the length of the workpiece's cut.

Additional Formulas and Considerations

Time for One Revolution (T_rev)

If you want to calculate the time for one complete revolution of the workpiece, you can use:

Trev=1NT

Where:

- T_rev = Time for one revolution (min)

- N = Spindle speed (RPM)

Material Removal Rate (MRR)

Material Removal Rate is a measure of the volume of material removed per unit time. It is important for determining the efficiency of a cutting operation.

MRR=π×D2×f×NMRR = \pi \times D^2 \times f \times N

Where:

- MRR = Material removal rate (mm³/min or in³/min)

- D = Diameter of the workpiece (mm or in)

- f = Feed per tooth (mm or in)

- N = Spindle speed (RPM)

Example Calculation for Turning Job

Let’s say we are turning a job with the following parameters:

- Diameter (D) = 100 mm

- Spindle Speed (N) = 1500 RPM

- Feed per tooth (f) = 0.05 mm/tooth

- Number of teeth (Z) = 4

- Length of workpiece (L) = 300 mm

Calculate Cutting Speed (V):

- V=π×D×N=π×100×1500=471238.9 mm/min

Calculate Feed Rate (F):

- F=f×N×Z=0.05×1500×4=300 mm/min

Calculate Turning Time (T):

- T=LF=300300=1 minute

- Thus, the time to turn the workpiece would be 1 minute.

Key Points to Consider:

- Cutting Speed (V) influences tool life and cutting efficiency. Higher cutting speeds usually provide a better surface finish, but can reduce tool life.

- Feed Rate (F) affects the material removal rate (MRR) and surface finish. High feed rates increase MRR but might result in a rough surface.

- Turning Time (T) is critical for production planning and calculating machine efficiency. It is directly influenced by feed rate and the length of the workpiece.

By using these formulas, you can optimize cutting parameters to improve machining efficiency, reduce tool wear, and ensure high-quality results in turning operations.

Practical Activity:

Calculate cutting speed, feed and rpm as per job requirement.

Material required:

White Board

Marker

Paper

Learning place:

Class room

|  | Perform checking using gauges |
| --- | --- |

Learning Outcomes:

After completion of this Learning Unit, trainee will be able to:

- Identify various types of gauges.

- Use appropriate gauge used in machining.

- Apply suitable gauge for quality control.

- Record for final inspection.

Introduction to Linear Measuring Instruments and Angular Measuring Instruments

Gauges are specialized tools used in both linear and angular measurement to ensure that parts or components conform to specific dimensions or angles. While general measuring instruments like rulers, calipers, and protractors measure a wide range of dimensions, gauges are often used to provide quick, accurate, and repeatable measurement or to check the conformity of a part to a standard or tolerance.

1. Gauges in Linear Measuring Instruments

Linear gauges are used to measure the length, width, height, or depth of an object. They provide a way to verify whether a workpiece meets the required dimensional specifications, often with higher precision than general measuring tools.

Common Types of Linear Measuring Gauges:

- Plug Gauge (Go/No-Go Gauge):

- Purpose: Used for checking the size of holes or internal diameters, confirming if the workpiece is within tolerance.

- Operation: There are two types: Go and No-Go. The Go gauge fits into the hole, confirming it is within tolerance, while the No-Go gauge should not fit if the hole is correctly sized.

- Application: Common in manufacturing, particularly for checking the internal dimensions of cylindrical parts like holes or bores.

Ring Gauge:

- Purpose: Similar to a plug gauge, but used to measure external dimensions, such as the outer diameter of shafts or rods.

- Operation: It provides a quick check to see if the external diameter of a part is within specification.

- Application: Used in quality control to measure external dimensions of cylindrical parts like shafts, rods, or bearings.

Height Gauge:

- Purpose: Measures the height of objects from a reference surface, often used in conjunction with a surface plate.

- Operation: The gauge slides along the surface plate and can be adjusted to measure the height of a part or surface.

- Application: Used to check the height of workpieces or to mark the position of cuts or features on a part during machining.

Thread Gauge:

- Purpose: Used to measure the pitch and dimensions of threads.

- Operation: Thread gauges come in different forms (e.g., pitch gauges for measuring thread pitch and plug gauges for checking the diameter and depth of threads).

- Application: Widely used in mechanical engineering and manufacturing to ensure that threaded components conform to standards (e.g., screws, bolts, nuts).

Depth Gauge:

- Purpose: Measures the depth of holes, slots, or cavities.

- Operation: A depth gauge typically consists of a probe that touches the bottom of the hole or cavity, and the measurement is read from a scale or digital readout.

- Application: Used in machining and quality control to ensure that holes or slots are drilled to the correct depth.

Caliper Gauge:

- Purpose: Used for precise measurements of internal and external dimensions of parts, as well as depths and steps.

- Operation: Calipers measure the distance between opposite sides of an object.

- Application: Used in a wide variety of applications, including machining, metalworking, and construction.

Gauges in Angular Measuring Instruments

Angular gauges are used to measure or set specific angles and are essential for checking the orientation or alignment of parts. These gauges ensure that components are positioned or manufactured with high precision when specific angles are required.

Common Types of Angular Measuring Gauges:

Bevel Gauge (Protractor Gauge):

- Purpose: Used to measure or set specific angles.

- Operation: A bevel gauge consists of a movable blade that can be adjusted to match the desired angle. It is typically used for measuring angles in machining or woodworking.

- Application: Commonly used in machining or for setting specific angles on tools or workpieces.

Sine Bar:

- Purpose: Measures angles by using the sine trigonometric function.

- Operation: A sine bar has a known length and is set to a specific height using gauge blocks. The angle is then calculated based on the ratio of the height to the length of the bar.

- Application: Used for high-precision angle measurements, such as setting angles on a surface plate during the machining of components.

Angle Gauge Block:

- Purpose: Used for setting or checking angles on a part or tool.

- Operation: This is a set of gauges that contains fixed angle blocks. The gauge blocks are stacked together or placed under a workpiece to set or check a specific angle.

- Application: Primarily used in precision machining or calibration tasks to ensure that tools and workpieces are at the correct angles.

Protractor (Precision Type):

- Purpose: A tool used for measuring or setting angles.

- Operation: The precision protractor may have a digital or mechanical scale that allows the operator to measure angles with high accuracy.

- Application: Used in precision engineering, where specific angle measurements are required.

Theodolite:

- Purpose: Measures both horizontal and vertical angles with very high precision.

- Operation: A theodolite uses optical components (such as telescopes) to sight distant points and measure angles precisely. It can be used for both angular measurements and distance measurements.

- Application: Used in surveying and civil engineering to measure angles between reference points on the ground.

Optical Comparator:

- Purpose: Measures angles and profiles using magnification and light.

- Operation: The optical comparator projects a magnified image of the workpiece onto a screen, allowing the operator to check angular and dimensional conformity against a template or standard.

- Application: Common in quality control environments for checking the angular accuracy of precision parts.

Inclinometer (Tilt Gauge):

- Purpose: Measures the angle of inclination or tilt of a surface relative to the horizontal or vertical.

- Operation: Inclinometers can be analog or digital, and they measure the angle of tilt, usually displayed in degrees.

- Application: Used to check the alignment of machinery, ensure level installation of equipment, or verify angles in construction and mechanical systems.

Conclusion

- Linear Gauges are specialized tools used to check the dimensional accuracy of Workpiece, ensuring that their lengths, diameters, depths, and thread pitches meet the required specifications. Plug gauges, ring gauges, height gauges, and caliper gauges are typical examples used in manufacturing, quality control, and machining processes.

- Angular Gauges are used to measure or set the angle between surfaces, ensuring parts are correctly aligned or oriented. Examples include bevel gauges, sine bars, protractors, and theodolites. These gauges are essential in industries where precise angles are critical, such as in machining, construction, and surveying.

- By using gauges, engineers and machinists can ensure that parts are manufactured to exact specifications, improving product quality and maintaining consistency across production processes.

Technique & observation of checking Zero Error

Zero Error in Gauges: Technique and Observation

Zero error refers to the discrepancy or deviation observed in the measurement reading when the instrument or gauge is not subjected to the measurement of the object or substance being measured. It is important to identify and correct zero errors in gauges to ensure accuracy in measurements.

Techniques for Checking Zero Error in Gauges:

Visual Inspection:

- Before using a gauge (e.g., micrometer, Vernier caliper, or pressure gauge), check if the reading on the instrument shows zero when no object is placed in the measuring area.

- If the gauge is digital, ensure that the display reads 0.000 or the equivalent zero reading.

Direct Observation of Scale:

- For analog gauges, visually inspect the scale. For micrometers, Vernier calipers, or dial indicators, ensure that the scale pointer aligns with the zero mark.

- If there is any offset (i.e., the scale reads a positive or negative value when it should show zero), note the error value.

Use of Calibration Standards:

- Compare the gauge reading with a known standard or reference object that should give a zero reading. For example, use a reference piece that is dimensionally exact to test for any deviation.

- If a standard is not available, using a flat, smooth surface to check the zero reading can help identify discrepancies.

Check for Backlash or Movement:

- Some mechanical gauges (e.g., micrometers) may have backlash or mechanical movement that can affect the zero setting. Gently test for any slop or unnecessary movement before checking the zero reading.

Cross-checking with Another Gauge:

- If available, use another gauge or tool of the same type to check if the zero reading aligns. This is especially useful for ensuring consistency across similar measuring instruments.

Using Calibration Tools:

- Calibration tools, such as gauge blocks or certified calibration standards, can be used to adjust and verify the zero error. These tools provide a precise reference against which the gauge can be tested.

Observations to Make While Checking for Zero Error:

Sign of Zero Error:

- If the gauge shows a reading higher than zero, it has a positive zero error.

- If the gauge shows a reading lower than zero, it has a negative zero error.

Magnitude of Error:

- Measure the deviation from zero carefully. For example, if a Vernier scale shows a reading of +0.05 mm when it should be zero, then the zero error is +0.05 mm.

- For micrometers, the zero error can often be observed by measuring a known object, such as a calibration standard, and comparing it to the expected value.

Consistency:

- Ensure that the zero error is consistent across repeated measurements. If the reading fluctuates, this may indicate a mechanical issue with the gauge or improper handling.

Environmental Factors:

- Temperature, humidity, and handling of the instrument can affect zero readings. Always check zero error in a controlled environment or after the gauge has acclimatized to the testing conditions.

Manufacturer Guidelines:

- Always refer to the manufacturer’s guidelines for the specific procedure to check for zero error, as different types of gauges may have specific calibration or zero-setting methods.

Correcting Zero Error:

Adjusting the Gauge:

- Some instruments allow you to adjust the zero point manually (e.g., micrometers often have a zero screw for fine-tuning).

Compensation:

- If the gauge cannot be adjusted directly, take the observed zero error into account when using the tool and make necessary compensations to readings during measurements.

- Importance of Checking Zero Error:

- Ensuring that zero error is identified and corrected is critical for accuracy in measurements. Even a small zero error can lead to large discrepancies in the final results, especially when precise measurements are required in fields like manufacturing, quality control, or scientific experiments.

Holding technique & observation technique.

The holding technique and observation technique for gauges are essential for ensuring accurate, repeatable measurements while minimizing errors. Below is a detailed breakdown of each technique:

1. Holding Technique for Gauges:

Proper handling of gauges is crucial to ensure accuracy, consistency, and prevent damage. The holding technique will vary depending on the type of gauge being used (e.g., micrometers, calipers, pressure gauges, etc.). However, certain principles can be applied across most types.

General Principles for Holding Gauges:

Keep Clean and Dry Hands:

- Always handle gauges with clean, dry hands. Dirt, oils, or moisture can affect the accuracy and result in measurement errors.

- Use gloves if necessary, especially when dealing with sensitive or high-precision instruments.

Grip the Gauge Properly:

- For tools like micrometers and calipers, use the main body or handle to hold the tool. Avoid holding the measuring surfaces (anvils, measuring faces) directly to prevent contamination and to ensure the instrument remains aligned properly.

- For dial indicators, hold the body firmly and avoid touching the dial or pointer to ensure an accurate reading.

Ensure Correct Alignment:

- Always ensure that the measuring faces or jaws of the gauge are perpendicular to the surface of the object being measured. Misalignment can cause inaccurate readings.

- Use a stable base for handheld gauges, especially for dial indicators or other precision tools, to prevent unnecessary movement.

Apply Consistent Force:

- For tools that require a certain amount of force (e.g., micrometers, spring calipers), apply the correct amount of pressure consistently. Excessive force can deform the object or the measuring faces, while insufficient force can lead to inaccurate measurements.

- Some micrometers and calipers have a friction thimble or a ratchet mechanism to ensure consistent pressure is applied.

Ensure Stability:

- When using gauges for precise measurements (e.g., calipers, micrometers), place the object to be measured on a stable surface or in a fixed position.

- Use a measurement stand or clamping tool if necessary to avoid shifting the part during measurement.

Specific Holding Techniques for Common Gauges:

Vernier Calipers:

- Hold the calipers with one hand while sliding the jaws along the object being measured with the other. Ensure the object is fully enclosed between the jaws for accurate readings.

Micrometers:

- Hold the micrometer body with one hand, while using the thimble or ratchet with the other hand to close the measuring faces gently onto the object. Always use the ratchet to ensure uniform measuring pressure.

Dial Indicators:

- Hold the dial indicator securely and ensure it is properly mounted on a stand. The stylus should be touching the object being measured, and you should avoid pressing too hard or misaligning the stylus.

Observation Technique for Gauges:

- Proper observation of gauge readings is equally important to ensure accuracy. Accurate observations require attention to detail and the correct interpretation of the gauge's output.

General Principles for Observing Gauge Readings:

Ensure Proper Viewing Angle:

- Always observe the reading from directly in front of the scale to avoid parallax error (an error that occurs due to the angle at which you view the scale).

- For analog scales, ensure that the pointer or needle is clearly aligned with the graduations on the scale.

Use the Correct Units:

- Ensure that you are reading the correct units from the gauge. Many instruments, especially digital ones, can display measurements in multiple units (e.g., millimeters, inches, microns). Double-check the unit displayed or the scale being used.

Avoid Handling During Observation:

- When taking a reading, avoid unnecessary movement or vibration that might affect the reading. For micrometers, for example, ensure the object is not moving or shifting before reading the measurement.

- For dial indicators, ensure the dial is not being disturbed while observing.

Specific Observation Techniques for Common Gauges:

Vernier Calipers:

- Read the main scale and vernier scale carefully. Align the zero point of the vernier scale with the main scale, and note the reading. If it's a digital caliper, simply read the digital display, but ensure it’s set to the correct units.

Micrometers:

- For outside micrometers, observe the reading on the thimble and sleeve. Pay attention to the ratchet mechanism to ensure consistent pressure. Observe the last digit carefully when reading a micrometer, and be aware of the fractional scale if applicable.

Dial Indicators:

- Observe the position of the dial pointer and record the total reading, taking into account any pre-set values. If using a dial indicator with a graduated scale, you may need to multiply readings based on the scale's increment.

Digital Gauges:

- For digital gauges, simply observe the reading on the display, but ensure the gauge is zeroed before measurement and ensure it’s set to the correct units.

Bore Gauges:

- Ensure the gauge is properly centered in the bore before reading the measurement. For dial bore gauges, observe the dial and note the dial’s deflection. For digital bore gauges, ensure the readings are stable before recording them.

Important Observations and Techniques to Minimize Errors:

Check for Zero Error:

- Always check the zero reading of the gauge before taking a measurement, especially for mechanical gauges like calipers, micrometers, and dial indicators. A zero error can introduce significant discrepancies in readings.

Calibration Verification:

- Regularly verify the calibration of your gauges to ensure accurate readings. Use standard calibration blocks or reference standards to check the accuracy of the instrument.

Record Consistently:

- Always record measurements immediately after taking them. This reduces the likelihood of misinterpreting or forgetting the correct reading.

Temperature Considerations:

- Temperature changes can cause expansion or contraction of the gauge or the object being measured. Always perform measurements at a consistent temperature, and, if necessary, apply temperature compensation.

By following the right holding techniques and observation practices, you can ensure that your gauge readings are accurate, reliable, and free from errors.

Practical Activity:

Measure the job using gauge as per drawing

Material required:

- Steel Rule

- Vernier Caliper

- (Dial, Digital)

- Micrometer

- (Inside & Outside, Depth, Thread)

- Plug Gauges of different size (Go Not Go)

- Snap Gauges of different size (Go Not Go)

- Bore Gauges of different limit.

- Telescope gauges

Learning place:

- Class room and workshop

Summery

Module 3: Perform Measurement, Calculation, and Checking Techniques

This module covers essential skills for performing basic unit calculations, including arithmetic operations and unit conversions using the SI system. It explains how to measure using tools like Vernier calipers, micrometers, and measuring tapes while ensuring accuracy by checking for zero errors. Additionally, it includes the application of cutting speed and feed formulas for CNC machines and understanding factors affecting machining operations. The use of various gauges for linear and angular measurements is also emphasized to ensure quality control. Practical activities help reinforce accurate measurement, calculation, and inspection techniques.

Here are some short questions and answers based on Module 3: Perform Measurement, Calculation, and Checking Techniques:

Section 1: Basic Unit Calculations

1. 	What are the seven fundamental SI units?

Length (meter), Mass (kilogram), Time (second), Electric Current (ampere), Temperature (kelvin), Amount of Substance (mole), Luminous Intensity (candela).

2.	How can you convert meters to centimeters?

Multiply by 100 (1 meter = 100 centimeters).

3.	What is the formula to convert kilometers to meters?

1 kilometer = 1,000 meters.

4.	Why is unit conversion important?

It ensures accuracy in calculations across different measurement systems.

5.	What is the least common denominator (LCD) used for?

To add or subtract fractions with different denominators.

Section 2: Performing Measurements Using Tools

6.	Name two types of linear measuring instruments.

Vernier Caliper and Micrometer.

7.	What is a micrometer used for?

Measuring small dimensions with high precision.

8.	What is the purpose of a height gauge?

To measure vertical dimensions from a reference surface.

9.	How can zero error be identified in a vernier caliper?

By closing the jaws and checking if the reading is exactly zero.

10.	What is the function of a laser distance meter?

To measure long distances using laser technology.

Section 3: Solving Formulas for Cutting Speed and Feed

11.	What is the formula to calculate cutting speed (V)?

V=π×D×NV = \pi \times D \times N

12.	What does N represent in the cutting speed formula?

Spindle Speed (in RPM).

13.	What is feed rate in milling operations?

The speed at which the workpiece moves relative to the cutting tool.

14.	Name a factor that affects the cutting speed.

Material of the workpiece.

15.	Why is coolant used during machining?

To reduce heat and friction, allowing for higher cutting speeds.

Section 4: Performing Checking Using Gauges

16.	What is a plug gauge used for?

To check the internal diameter of a hole.

17.	What is a go/no-go gauge?

A tool used to check if a part is within tolerance.

18.	How does a sine bar measure angles?

By using a height measurement and applying the sine function.

19.	What is zero error in measuring tools?

It occurs when a tool reads a non-zero value when no measurement is taken.

20.	How can zero error be corrected?

By adjusting the instrument or compensating for the error in the readings.

Here are 25 multiple-choice questions (MCQs) based on Module 3: Perform Measurement, Calculation, and Checking Techniques:

Multiple Choice Questions (MCQs)

1.	What is the SI unit of length?

A) Kilogram	B) Meter	C) Ampere	D) Second

2.	How many centimeters are in 1 meter?

A) 10	B) 100	C) 1,000	D) 1,000,000

3.	Which instrument is used for measuring small objects with high precision?

A) Ruler	B) Vernier Caliper	C) Measuring Tape	D) Compass

4.	What does a micrometer measure?

A) Large distances	B) Angles	C) Small thicknesses	D) Pressure

5.	Which formula calculates cutting speed (V) in machining?

A) V=π×D×N	B) V=D+N	C) V=D×N/π	D) V=N/D

6.	What is the unit of cutting speed?

A) Meters per second (m/s)	B) Kilograms (kg)

C) Joules (J)		D) Newtons (N)

7.	Which of the following is an angular measuring instrument?

A) Vernier Caliper	B) Micrometer	C) Protractor	D) Ruler

8.	How many millimeters are in 1 centimeter?

A) 5	B) 10	C) 50	D) 100

9.	What is a go/no-go gauge used for?

A) Measuring voltage	B) Checking tolerance limits

C) Measuring liquid volume	D) Measuring temperature

10.	What is the purpose of using cutting fluid during machining?

A) Improve appearance	B) Reduce heat and friction

C) Increase speed		D) Prevent tool movement

11.	Which unit measures mass in the SI system?

A) Kelvin	B) Mole	C) Kilogram	D) Ampere

12.	What should be done before using a vernier caliper?

A) Shake it to check functionality	B) Check for zero error

C) Apply lubricant		D) Heat it to expand the jaws

13.	What does the zero error in a measuring instrument indicate?

A) Incorrect calibration	B) Accurate measurement

C) Positive weight		D) Smooth operation

14.	Which is a derived unit in the SI system?

A) Meter	B) Kilogram	C) Newton	D) Second

15.	How is spindle speed calculated?

A) Using cutting speed and diameter	B) Using mass and time

C) Using force and distance	D) Using voltage and current

16.	What is the function of a height gauge?

A) Measure width		B) Measure height from a reference point

C) Check weight		D) Measure air pressure

17.	What is the correct unit for feed rate in milling operations?

A) Meters per minute (m/min)	B) Kilograms (kg)

C) Seconds (s)		D) Liters (L)

18.	What is a sine bar used for?

A) Cutting metal		B) Measuring angles accurately

C) Measuring weight	D) Cleaning tools

19.	What is the least common denominator (LCD) used for?

A) Measuring mass		B) Adding fractions with different denominators

C) Calculating time		D) Measuring pressure

20.	Which of the following affects the cutting speed?

A) Tool material	B) Workpiece material	C) Coolant usage	D) All of the above

21.	What is a typical use of a laser distance meter?

A) Calculate angles		B) Measure long distances accurately

C) Calculate weight		D) Check temperature

22.	How often should measuring instruments be calibrated?

A) Daily		B) Annually

C) When damaged		D) As per manufacturer’s recommendation

23.	Which measuring instrument is used for checking internal thread size?

A) Protractor	B) Ring Gauge	C) Plug Gauge	D) Height Gauge

24.	What is a common unit for force measurement?

A) Pascal (Pa)	B) Newton (N)	C) Volt (V)	D) Ampere (A)

25.	How is turning time calculated in a machining operation?

A) Using spindle speed and feed rate	B) Using mass and distance

C) Using pressure and area	D) Using time and temperature

True Answer

| 1. | b | 2. | b | 3. | b | 4. | c |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5. | a | 6. | a | 7. | c | 8. | b |
| 9. | b | 10. | b | 11. | c | 12. | b |
| 13. | a | 14. | c | 15. | a | 16. | b |
| 17. | a | 18. | b | 19. | b | 20. | d |
| 21. | b | 22. | d | 23. | c | 24. | b |
| 25. | a |  |  |  |  |  |  |