|  | Interpret and Develop Drawing and Design |
| --- | --- |

Learning Outcomes:

After Completion of this learning module, trainee will be able to:

- Develop Fundamentals of Engineering Drawing

- Interpret and Develop 2D Drawing/Sketch

- Develop 3D Models

|  | Develop Fundamentals of Engineering Drawing |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Select proper drawing units

- Identify coordinate system

- Practice different types of commands (Line, Poly line, Arc, Chamfer, Fillet etc)

- Practice to draw different types of angles.

- Practice to draw different types of Geometrical shapes. (Circle, Rectangle, Polygon, Ellipse, Triangle)

- Apply different Edit/Modify Commands.

Engineering Drawing

Introduction

Engineering drawing is a two-dimensional representation of three-dimensional objects. In general, it provides necessary information about the shape, size, surface quality, material, manufacturing process of the object.

Engineering drawing conveys detailed information about dimensions, materials, tolerances, and manufacturing processes, serving as a universal language between engineers, designers, and manufacturers. Engineering drawings follow specific conventions, such as orthographic projections, sectional views, and annotations, to ensure clarity and accuracy in the design and production process.

Importance of drawing

Engineering drawing plays a crucial role in the design, manufacturing, and construction industries by providing a clear and standardized method of communication among engineers, designers, and manufacturers.

Coordinate system

The coordinate system in engineering drawing is used to define the location of points, lines, and objects in a 2D or 3D space. It provides a reference framework for measurements and positioning in technical drawings.

Drawing tools

Drawing board

A drawing board is a flat, smooth surface used as a stable base for creating technical drawings. It is an essential tool for drafting, especially when using traditional hand-drawing methods. The board ensures accuracy, proper alignment, and ease of working with drawing instruments like T-squares, set squares, compasses, and scales. Typically, it is made of wood.

T- Square

T-squares are specifically used for horizontal lines on drafting tables.

Ruler

Remember

- Use a light touch - Working lairs - Use different sides - Clean regularly - Avoid over-blending

For precise lines and measurements Scales are made of wood, steel, celluloid or plastic or card board. Stainless-steel scales are more durable. Scales may be flat or of triangular cross-section. 15 cm long and 2 cm wide or 30 cm long and 3 cm wide flat scales are in common use.

Erasers

Essential for correcting mistakes and refining sketches; types include rubber and kneaded erasers.

Compasses and Protractors

Essential for drawing circles, arcs, and measuring angles, commonly used in geometric and technical drawings.

Remember

- Keep the needle sharp for stability and accuracy - A clean protector makes alignment easier

Set Square

A tool used to draw and measure precise angles, particularly right angles and other specific angles like 30°, 60°, and 45°.

Remember

- Always align it properly for precise measurements - Held firmly to prevent slipping - Metal or wooden T-Square are more durable

Pencils

Available in various grades from hard (H) to soft (B), pencils are fundamental for sketching and detailed work.

Remember

- Wex-based pencils are softer and easier to blend while oil-based are firmer and more durable - Artist grade pencils richer pigments and batter lairing than student grade pencils

Remember

Thick Lines attract more attention while thinker lines appear more delicate and suitable

Drawing Material

Drawing materials vary based on the medium and style you prefer. Here are some common materials:

Sheet Layout

The arrangement of a drawing sheet, which involves choosing its size, allocating space for margin, title block, parts list, revision panel, folding marks, and determining an appropriate scale, is referred to as the layout of the drawing sheet.

The layout ensures that all necessary information for understanding and producing a part or assembly is included and easily accessible.

Sheet Size

Remember

- Seal block contains crucial details like project name, drawing tiles, drawing no. revision history and date - Must be clear and readable typically placed in the bottom right corner

Standardized sizes (e.g., A0, A1, A2, A3, A4) are used to maintain consistency and facilitate storage and handling.

Margins

Designated areas around the edges of the sheet for binding and handling.

Title Block

A standardized area containing essential information about the drawing, such as title, drawing number, scale, revision history, and company information.

Paper

Drawing papers are available in many varieties. For ordinary pencil-drawings, the paper selected should be tough and strong. It should be uniform in thickness and as white as possible.

Remember

- A5-A6 good for quick sketches - A4-A3 ideal for detailed drawings - A2-A1-A0 best for posters, Technical Drawings or large-scale art

Method of Drawing

Engineering drawing is a technical discipline that involves creating detailed and precise representations of objects, structures, and systems. There are several methods used in engineering drawing, depending on the purpose and industry. Here are the key methods:

1. Manual Drawing Methods

These methods involve traditional drawing tools such as pencils, rulers, compasses, and protractors. They include:

Freehand Sketching

Quick, rough sketches without precise instruments, used for conceptualizing ideas.

Instrument Drawing

Uses tools like T-squares, set squares, and drafting machines for accuracy.

Perspective Drawing

Represents objects as they appear to the human eye (1-point, 2-point, or 3-point perspective).

2. Computer-Aided Design (CAD)

2D CAD Drawing

Software like AutoCAD is used for creating detailed 2D technical drawings.

3D CAD Modeling

Software like Auto cad, SolidWorks, CATIA, or Fusion 360 is used for designing three-dimensional objects with accurate measurements.

Geometrical shapes

Geometric shapes are mathematical shapes. They are perfect and regular. They are characterized by straight lines, angles and points. An exception to this would be a perfect circle as it has no straight lines or points. Other geometric shapes are rectangles, triangles, polygon, ellipse etc.

Rectangle

A polygon with four sides and four right angles is called a rectangle. It has two parallel sets of sides, and the lengths of the opposite sides.

Steps of construction:

- Prepare the Sheet Fix the drawing sheet on the board using tape or clips.

- Draw the Base Line Use a T-square to draw a horizontal base line where the rectangle will start.

- Mark the Length Using a ruler, measure and mark two points along the base line for the required length of the rectangle.

- Draw Vertical Lines Use a set square aligned with the T-square to draw perpendicular vertical lines from the two points.

- Mark the Height Measure the required height from the base line and mark two points on the vertical lines.

- Draw the Top Edge Connect the upper two points using the T-square to complete the rectangle.

- Ensure the lines are straight, properly aligned, and dimension the rectangle as per standards.

Circle

A circle is a two-dimensional shape with a closed boundary. It has a round shape without any corners or lines. Examples of circles are coins.

Steps of construction:

- Select the Center Point Mark the center of the circle on the drawing sheet using a sharp pencil.

- Set the Compass Radius Adjust the compass to the required radius using a scale.

- Place the Compass Needle Position the compass needle precisely on the center point.

- Rotate the Compass Keep the needle firm and rotate the pencil end smoothly in a circular motion to complete the circle.

Remember

- Radius (r) – Distance from the center to any point on the circle. - Diameter (d) – The longest chord, passing through the center. - Circumference (C) – The perimeter of the circle. - Area (A) – The space inside the circle

- Go over the circle lightly with a pencil to finalize it.

Triangle

Just like a circle, the triangle is also a closed two-dimensional figure. However, it contains 3 sides. Examples of triangles are a triangular.

Steps of construction:

- Use a scale (ruler) to draw a straight horizontal baseline of the required length.

- Mark both ends as points A and B.

- Set the compass to the length of the side.

- Place the compass at A and draw an arc.

- Without changing the compass width, place it at B and draw another arc to intersect the first arc.

- Mark the intersection points as C.

- Join A to C and B to C.

There are some types of triangles listed below.

Remember

- Regular Polygon: All sides and interior angles are equal (e.g., equilateral triangle, square). - Irregular Polygon: Sides and/or interior angles are not all equal.

Polygons

A polygon is any 2-dimensional shape formed with straight lines. Triangles, quadrilaterals, pentagons, and hexagons are all examples of polygons. The name tells you how many sides the shape has. For example, a triangle has three sides, and a quadrilateral has four sides.

Steps of construction

1. Using the Inscribed Method (Given a Circumscribed Circle)

- Draw a circle with the given radius.

- Mark the center and draw two perpendicular centerlines (horizontal and vertical).

- Divide the circle into equal segments according to the number of sides (e.g., for a hexagon, divide it into 6 equal parts).

- Label the intersection points on the circle.

- Connect the points in sequence to form the polygon.

2. Using the Circumscribed Method (Given an Inscribed Circle)

- Draw the inscribed circle with the given radius.

- Draw the required number of tangents around the circle at equal angles.

- Connect the tangent points to form the polygon.

Ellipse

An ellipse is a geometric shape, an oval, defined by two points called foci, where the sum of the distances from any point on the ellipse to the two foci is constant.

Step of construction

- Draw Two Perpendicular Axes

- Mark the major axis (longest diameter) and minor axis (shortest diameter).

- Label the center as O.

- Draw Two Circles

- With O as the center, draw a circle with a radius equal to half the major axis.

- Draw another circle with a radius equal to half the minor axis.

- Divide the Circles

- Divide both circles into 12 or 8 equal parts.

- Project Intersection Points

- From each division on the larger circle, draw vertical lines down.

- From each division on the smaller circle, draw horizontal lines across.

- The intersections of these lines give points on the ellipse.

- Join the Points Smoothly.

Types of Drawing

Technical drawings can be categorized as:

Part drawings (showing individual components)

Assembly drawings (showing how parts fit together)

Part Drawing

These drawings focus on individual components, detailing their dimensions, tolerances, and other manufacturing requirements.

Examples include drawings for brackets, extrusions, tubes, or bent metal sheets.

Assembly Drawings

These drawings illustrate how different parts are assembled together, showing their relationships and how they fit.

They are essential for understanding the construction and functionality of a product.

Assembly drawings are commonly used in engineering, manufacturing, and construction industries.

Projection of Drawing

Projection is a method used in engineering drawing to represent a three-dimensional object on a two-dimensional plane. It helps engineers, designers, and manufacturers visualize and interpret complex objects accurately.

Types of Projection

Projection methods are broadly classified into two main types

- Orthographic Projection

- Isometric Projection

Orthographic Projection

Orthographic projection is a technique used to represent a 3D object on a 2D plane using multiple views. It is an essential method in engineering drawing to accurately convey the shape, size, and features of an object.

Principle of Orthographic Projection

- The object is imagined to be enclosed in a transparent box.

- Views are projected onto different planes (front, top, side, etc.) using parallel projection lines.

- Each view represents a different perspective of the object.

Types of Orthographic Projection

First Angle Projection

- The object is placed in front of the projection plane.

- Views are arranged opposite to their actual positions (e.g., the top view appears below the front view).

- Denoted by a first-angle projection symbol (⭤ ).

Third Angle Projection

- The object is placed behind the projection plane.

- Views are arranged in their actual positions (e.g., the top view appears above the front view).

Denoted by a third-angle projection symbol (⭧).

Isometric Projection

Isometric projection is a method of visually representing three-dimensional objects in two dimensions. It is a type of axonometric projection where the three principal axes (X, Y, and Z) are equally inclined at an angle of 120° to each other. This technique allows engineers to depict objects in a way that maintains proportionality without perspective distortion.

Unit setting

In engineering drawings, unit settings define the measurement system used for dimensions, tolerances, and annotations. The two primary unit systems are:

Metric System (SI Units)

Uses millimeters (mm) as the standard unit unless otherwise specified. Sometimes meters (m) or centimeters (cm) are used for larger dimensions.

Imperial System (Inch-Pound Units)

Uses inches (in) or feet (ft) as the primary units

Scale

In engineering drawings, a title block is an essential element that provides critical information about the drawing and the part or assembly it represents. Typically located in the lower right corner of the drawing sheet, the title block includes details such as:

A scale is defined as the ratio of the linear dimensions of element of the object as represented in a drawing to the actual dimensions of the same element of the object itself.

The scales can be expressed in the following three ways:

Engineer’s scale

In this case, the relation between the dimension on the drawing and the actual dimension of the object is mentioned numerically in the style as 10 mm = 5 m etc.

Graphical scale

The scale is drawn on the drawing itself. As the drawing becomes old, the engineer's scale may shrink and may not give accurate results. However, such is not the case with graphical scale because if the drawing shrinks, the scale will also shrink. Hence, the graphical scale.is commonly used in survey maps.

Representative fraction:

The ratio of the length of the object represented on drawing to the actual length of the object represented is called the Representative Fraction (i.e. R.F.).

R.F. = Length of the drawing / Actual length of object

When a 1 cm long line in a drawing represents 1 meter length of the object, the R.F. is equal to 1 cm/1 cm = 1 /1x 100 cm = 1/100 and the scale of the drawing will be 1: 100 or 1/100 full size. The R.F. of a drawing is greater than unity when it is drawn on an enlarging scale. For example, when a 2 mm long edge of an object is shown in a drawing by a line 1 cm long, the R.F. 1 cm /10 mm =10mm/2mm =5 such a drawing is said to be drawn on scale 5:1 or five time full-size.

Types of Scales in Engineering Drawing

Scales in engineering drawing are categorized into different types based on their purpose and usage.

1. Full Scale (1:1)

The drawing size is exactly the same as the actual object.

Used when the object is small enough to be represented at its actual size.

2. Enlarged Scale (>1:1)

The drawing is larger than the actual object.

Used for small objects to show intricate details.

Example: 2:1, 5:1, 10:1

3. Reduced Scale (<1:1)

The drawing is smaller than the actual object.

Used for large objects like buildings, bridges, and machines.

Example: 1:2, 1:10, 1:100

Line

line is a fundamental element used to represent the shape, dimensions, and features of an object. Different types of lines convey different meanings and are standardized in technical drawings.

Importance of Lines

- Lines are the foundation of engineering drawings, playing a crucial role in communicating design ideas, manufacturing requirements, and construction details. Their importance includes:

- Different types of lines help convey the exact shape, size, and features of a component or structure.

- Each type of line (object, hidden, center, dimension, etc.) provides specific information.

- Engineering drawings require high precision, and correctly used lines help achieve exact dimensions.

Basic type of a drawing

Object Lines (Visible Lines)

- Represent the visible edges and outlines of an object.

- Appearance: Thick, continuous lines.

Hidden Lines

- Indicate edges or surfaces that are not directly visible in the current view.

- Appearance: Medium weight, short dashes evenly spaced along the line.

Center Lines

- Indicate the center of a circle, arc, or symmetrical feature.

- Appearance: Thin, alternating long and short dashes.

Dimension Lines and Extension Lines

- Show the extent and direction of a dimension (length, width, height, etc.).

- Extend from a feature to a dimension line, indicating the point where the dimension is taken.

- Appearance: Thin, solid lines with arrowheads at the ends.

Leader Lines

- Connect a note or symbol to a specific feature on the drawing.

- Appearance: Thin, solid lines with an arrowhead at the end.

Phantom Lines

- Indicate alternate positions of parts, repeated details, or the location of absent parts.

- Appearance: Thin lines alternating one long and two evenly spaced, short dashes, with a long dash at each end.

Cutting Plane Lines:

- Show the location and path of an imaginary cut made to reveal internal details.

- Appearance: Thick, alternating long lines separated by two short dashes, with arrowheads at each end showing the viewing direction.

Section Lines:

Indicate the surface of a cut section, typically shown in a sectional view.

Appearance: Thin, evenly spaced, short lines drawn at an angle.

Break line:

A break line is a line used to indicate that a portion of an object has been removed to fit the drawing within a limited space or to expose internal features for better clarity.

Short Break Line:

A thick, wavy, freehand line used to show small breaks in objects, such as to remove part of a shaft or pipe.

Long Break Line:

A straight, thin line with zigzag sections, typically used for larger objects to shorten their representation without altering essential dimensions.

Dimension

A dimension is a numerical value assigned to a feature in an engineering drawing, specifying its size, location, or geometric characteristics. It ensures accuracy in manufacturing, assembly, and inspection of the component.

Parts of Dimension

Types of Dimensioning

Linear Dimension

Specifies the length, width, or height of an object.

Examples: Distance between two points, edge lengths.

Parallel dimensioning

It is a method of dimensioning where multiple dimensions are measured from a common reference point or baseline and placed parallel to each other.

Angular Dimension

Defines the angle between two lines or surfaces.

Example: Angle between two intersecting edges.

Aligned dimensioning

It is a method where the dimension values are placed in line with the dimension lines, making them readable from the direction they represent. This ensures clarity and ease of interpretation in engineering drawings.

Radial and Diameter Dimension

Used for circular features such as holes, arcs, and fillets.

Examples: Radius (R) and Diameter (Ø).

Location Dimension

- Specifies the position of a feature relative to a reference point or datum.

- Example: Hole center distances from edges.

Datum Dimension

- Uses a reference (datum) to define other dimensions for precise manufacturing.

- Example: A feature’s distance from a fixed baseline.

Tolerance:

Tolerance is the allowable variation in a physical dimension or measurement. It defines the acceptable limits within which a part or product can deviate from the specified value while still functioning correctly. Tolerance is crucial in manufacturing and engineering to ensure parts fit and function properly without excessive cost or difficulty in production.

Important Tolerance

- Ensures interchangeability of parts.

- Improves manufacturing efficiency and cost control.

- Maintains functionality and performance of assemblies.

- Reduces rework and material wastage.

Types of Tolerance

Dimensional Tolerance

It specifies the permissible variation in the dimensions (length, width, height, diameter, etc.) of a part.

Unilateral Tolerance

Variation is allowed in only one direction (either positive or negative).

Bilateral Tolerance

Variation is allowed in both positive and negative directions.

Limit Tolerance

Defined by upper and lower limits instead of a nominal value with a tolerance range.

Geometric Dimensioning and Tolerancing

It follows GD&T (Geometric Dimensioning and Tolerancing) standards and includes:

Form Tolerance

Controls the shape of a part (e.g., straightness, flatness, circularity, cylindricity).

1. Straightness (⏤)

- Straightness controls how much a feature (line or axis) can deviate from a perfectly straight line.

- Application: Used for shafts, guideways, and edges.

2. Flatness (⏥)

- Flatness ensures that a surface remains within a specified tolerance zone without waviness or bending.

- Application: Used for mating surfaces in assemblies (e.g., machine beds, flanges).

3. Circularity (○)

- Circularity (or roundness) ensures that all points on a circular surface lie within a tolerance zone.

- Application: Used for round components like shafts, cylinders, and gears.

4. Cylindricity (◎)

- Cylindricity controls the roundness and straightness of a cylinder along its entire length.

- Application: Used for precision cylindrical parts like pistons, hydraulic rams, and rollers.

Orientation Tolerance:

Ensures correct angles or orientations (e.g., parallelism, perpendicularity, angularity).

1. Parallelism (∥)

- Ensures that a surface or axis remains equidistant from a reference datum.

- Application: Used in assemblies where two surfaces must remain consistently apart.

2. Perpendicularity (⊥)

- Ensures that a feature is at exactly 90° to a reference datum.

- Application: Used for mating parts, holes, and supports.

3. Angularity (∠)

- Ensures that a surface or axis is at a specific angle (other than 90°) relative to a reference datum.

- Application: Used in components with inclined surfaces.

Location Tolerance:

Specifies the accurate placement of features (e.g., position, concentricity, symmetry).

1. Position

- Ensures that a feature (hole, slot, pin, etc.) is within a specified tolerance zone relative to a datum.

- Application: Used for holes, pins, and slots to ensure proper alignment in assemblies..

2. Concentricity (◎)

- Ensures that the central axes of two cylindrical features (such as a shaft and a hole) are aligned within a tolerance zone.

- Application: Used for rotating parts like shafts, bearings, and gears.

3. Symmetry (⌭)

- Ensures that a feature is evenly balanced around a center plane.

- Application: Used for symmetrical parts like brackets and gears.

Runout Tolerance:

Ensures rotational consistency (e.g., circular runout, total runout).

Circular Runout

- Controls the variation of a surface as it rotates around an axis, measuring at a single cross-section.

- Application: Used for controlling roundness and alignment in rotating components.

Total Runout

- Controls the variation of an entire surface as it rotates around an axis, measuring along the entire length.

- Application: Used for ensuring complete surface accuracy over the length of a rotating part.

Fit Tolerance

Defines the allowance between mating parts to ensure proper assembly. It is classified into:

Clearance Fit

- Parts have a gap and move freely (e.g., shaft inside a bearing).

Interference Fit:

- Parts are tightly fitted, requiring force to assemble or disassemble.

Surface Tolerance

- Specifies the allowable roughness or finish of a surface to ensure proper function and appearance.

Sectional View

Sectional drawings are a type of technical drawing used in engineering, architecture, and manufacturing to show the internal features of an object or structure. These drawings are created by cutting through an object or building along a specific plane to reveal hidden details that cannot be seen in regular views. Sectional views help in understanding the internal construction, material distribution, and assembly of components.

Types of Sectional Drawings

1. Full Section

- A full section is obtained by cutting through the entire object along a single plane.

- It provides a complete internal view, eliminating all hidden lines.

- Used in mechanical and architectural drawings to show internal components clearly.

2. Half Section

- A half section is created by cutting only half of the object along the centerline.

- One half shows the internal details, while the other half retains the external shape.

- Commonly used for symmetrical objects like shafts, bearings, and housings.

3. Offset Section

- In an offset section, the cutting plane is bent or shifted to pass through important features that are not aligned in a straight line.

- Used when multiple internal features need to be displayed in a single view.

4. Broken-Out Section

- This section shows only a portion of an object’s interior by removing a small part of the drawing.

- No cutting plane is drawn, and it is used when only a small area needs internal detailing.

5. Revolved Section

- In this type, a sectional view of a specific feature is rotated 90° directly onto the same drawing.

- Commonly used for cylindrical parts like rods and shafts.

6. Removed Section

- A removed section is placed outside the main drawing to provide a detailed sectional view.

- It is used when there is not enough space in the main view to display internal details clearly.

Views

In engineering drawing, a view refers to a representation of an object from a specific to direction its shape, dimensions, and features clearly. Views are essential for understanding the complete geometry of an object.

Types of views

Types of Views in Technical Drawing

In engineering drawings, different types of views are used to represent objects accurately. These views help in understanding the shape, size, and features of an object from different perspectives.

Principal Views

Front View (Elevation):

- Shows the height and width.

- The most descriptive view of an object.

Top View (Plan):

- Shows the width and depth.

- Viewed from directly above.

Side View (Profile View):

- Shows the height and depth.

- Can be left or right side view.

Auxiliary Views

- Used to show inclined or slanted surfaces more accurately.

- Often used when principal views don’t provide enough detail.

Sectional Views

Used to show internal features by cutting through an object.

- Full Section: Cuts through the entire object.

- Half Section: Cuts through half of the object.

- Offset Section: Used for complex objects with multiple internal features.

Pictorial Views

- Represent objects in 3D for better visualization.

Isometric View

- Uses 30° angles to represent the object.

- Dimensions are proportional but not distorted.

Perspective View

- Mimics how the human eye sees objects.

- Used in architecture and product design.

- One-Point Perspective: All lines converge at one vanishing point.

- Two-Point Perspective: Two vanishing points are used.

- Three-Point Perspective: Used for very realistic 3D effects.

Material Required

| - Compter Lab | - Computer 20 |
| --- | --- |
| - Each 20 Trainee | - Multimedia |
| - Preintinger with Scanner (Color) | - White Board |
| - White Board Marker | - Paper Rim |
| - Software |  |

Practical Activity: Develop drawing by using basic drawing and editing tools

Activity 1:  Practice to draw coordinate system.

Activity 2:  Practice to draw manual drawing method using instrument draw different type of commands

| - Line | - Polyline |
| --- | --- |
| - Ark | - Chamfer |
| - Fillet |  |

Activity 3: Practice to draw orthographic projection and isometric projection of different shapes.

Isometric Projection

Orthoghrapic Projection

Activity 4:  Practice to draw representative fraction scale.

Activity 5:  Practice to draw following type of lines.

- Visible line

- Hidden line

- Center line

- Dimension line and extension line

- Cutting plane line

- Section line

- Break line

- Short break line

- Long break line

Activity 8:  Practice to draw following type of sectional view.

- Full section

- Half section

- Break out section

- Revolve section

- Removed section Top of Form

|  | Interpret and Develop 2D Drawing/Sketch |
| --- | --- |

Learning Outcomes:

After completion of this learning module, you will be able to:

- Interpret drawing requirement for the product or component.

- Use appropriate computer software for developing drawing of required product or component.

- Select and use appropriate user interface and apply relevant commands for developing mechanical drawing.

- Produce drawing according to other required dimensions by use of various drawing standards to meet job requirements.

- Modify/Edit the drawing as per requirement.

Learning Unit 2.1: Develop 2D Objects

Basic Drawings concepts

Types of Drawing

- 2-Dimensional drawing

- 3-Dimensional drawing

2-Dimensional drawing:

A physical object in which length and width are real attributes. Any object can be drawn with respect to x and y coordinates.

3-Dimensional drawing:

A physical object in which length, width, and depth are real attributes—a model, with X, Y, and Z-axes that can be rotated for viewing from different angles

AutoCAD interface introduction

AutoCAD is Automatic Computer-Aided Drafting and design (CAD), developed and marketed by Autodesk. AutoCAD was first released in December 1982 as a desktop app running on micro computers with internal graphic controllers. Since 2010, AutoCAD is released as a mobile-and web app as well, marketed as AutoCAD 360.

AutoCAD is used in the civil, mechanical, architecture, electrical, electronics and graphic designers.

Introduction of AutoCAD workspaces (screen):

The collection of tools displayed after creating or opening a drawing is called a workspace.

Some useful toolbars are discussed here.

Title bar

Title bar is shown on the top of the screen and also include the file name.

Quick Access toolbar

It is located to the right of application menu and provides immediate access to some of most frequently used commands to complete tasks such as creating, opening, saving ad plotting drawings. The Quick Access toolbar displays options to undo and redo changes to your file, save, print, the most use option can be added in access toolbar.

Menu Bar or Application Menu

Horizontal bar, typically located at the top of the screen below the title bar. It contains drop- down menus, including File, Edit, Insert, Format, Tools, Draw, Dimension, Modify, Parametric, Window, Help and Express.

Ribbon

Ribbon includes different tabs and groups. It is located immediately below the title bar and serves the primary launch pad for nearly every command used to compose drawings in AutoCAD.

Command bar or Command prompt

Command line is located in the bottom left corner of CAD interface. AutoCAD Command line is like a combined control panel and index of all AutoCAD commands you run.

Status Bar

Remember

Templates: is a pre- define layout available in AutoCAD Software which assist the learner.

Status bar is located below the command line and is shown on the bottom of screen. Status bar has different elements, coordinate values, drawings tool, quick properties, models, layouts, quick view tools, navigation tools, annotation tools workspace and clear screen.

Create or Open New File

Create or open new file means to get a new drawing in AutoCAD or learner can also use a template already available in CAD templates library.

How to Open templates in AutoCAD

Create New Template: Learner can open or create a template by clicking

File New  Select Template as shown in the figure.

Short keys of AutoCAD software

Function Keys

The Following functions keys are labeled F1 through F12 only keys F1 through F11 will be discussed here.

| F1 | Windows Online Help |
| --- | --- |
| F2 | Toggle Between Graphic Screen / Text Screen. |
| F3 | Toggle OSNAP setting dialogue box. |
| F4 | Toggle tablet mode on or off. |
| F5 | 3 Iso-plane modes to construct isometric drawing. |
| F6 | Toggle the coordinates. On/off. |
| F7 | Toggle the display of Grid on or off. |
| F8 | Toggle to Ortho mode. |
| F9 | Toggle Snap mode on or off. The SNAP command sets the current snap value. |
| F10 | Toggle Polar tracking on or off. |
| F11 | Object snap tracking on or off. Speakers |

Here are some commonly used AutoCAD shortcut keys to speed up your workflow:

| Ctrl + N → New Drawing | ARC → Arc | SC → Scale |
| --- | --- | --- |
| Ctrl + O → Open Drawing | SPL → Spline | MI → Mirror |
| Ctrl + S → Save Drawing | M → Move | F → Fillet |
| Ctrl + P → Print/Plot | CO (or CP) → Copy | CH → Chamfer |
| L → Line | O → Offset | STRETCH → Stretch |
| C → Circle | TR → Trim | Z → Zoom |
| REC → Rectangle | EX → Extend | P → Pan |
| PL → Polyline | RO → Rotate | LA → Layer Manager |
| LT → Line type Manager | LE → Leader | Ctrl + Y → Redo |
| MA → Match Properties | B → Create Block | Ctrl + C → Copy |
| T → Single-line Text | I → Insert Block | Ctrl + V → Paste |
| MT → Multiline Text | X → Explode | Ctrl + X → Cut |
| DIM → Dimension | Ctrl + Z → Undo | ESC → Cancel Command |

Dimensions, Text and Symbols

Dimension in AutoCAD?

A dimension in AutoCAD is a measurement annotation that shows the size, length, angle, or other geometric properties of an object in a drawing. Dimensions help to communicate the exact specifications of a design for manufacturing, construction, or drafting purposes.

Linear Dimension (DIMLINEAR)

Use to measure horizontal or vertical distances.

Procedure:

- Type DIM or DIMLINEAR in the command bar and press Enter.

- Click the first point (start of the dimension).

- Click the second point (end of the dimension).

- Move the mouse to place the dimension line and click to set it.

Aligned Dimension (DIMALIGNED)

Use for slanted or diagonal measurements.

Procedure:

- Type DIMALIGNED and press Enter.

- Click the first point of the dimension.

- Click the second point (aligned along the object).

- Move the cursor to position the dimension and click to place it.

Angular Dimension (DIMANGULAR)

Use to measure angles between two lines.

Procedure:

- Type DIMANGULAR and press Enter.

- Select the first line of the angle.

- Select the second line forming the angle.

- Move the cursor to adjust the angle’s position and click to place the dimension.

Radial Dimension (DIMRADIUS)

Use to measure the radius of circles or arcs.

Procedure:

- Type DIMRADIUS and press Enter.

- Click on the arc or circle.

- Move the cursor outward to adjust the dimension.

- Click to place the dimension.

Diameter Dimension (DIMDIAMETER)

Use to measure the diameter of circles.

Procedure:

- Type DIMDIAMETER and press Enter.

- Click on the circle or arc.

- Move the cursor outward and click to place the dimension.

Arc Length Dimension (DIMARC)

Use to measure the actual length of an arc (not just radius or diameter).

Procedure:

- Type DIMARC and press Enter.

- Click on the arc.

- Move the cursor outward and click to place the arc length dimension.

Ordinate Dimension (DIMORDINATE)

Use to display X and Y coordinates of a point.

Procedure:

- Type DIMORDINATE and press Enter.

- Click the point you want to measure.

- Move the cursor to place the X or Y coordinate and click.

Continuous Dimension (DIMCONTINUE)

Use to create a chain of dimensions.

Procedure:

- First, create a Linear Dimension.

- Type DIMCONTINUE and press Enter.

- Click on the previous dimension’s second point.

- Click additional points to continue the dimension chain.

Leader Dimension (MLEADER / QLEADER)

Use to create an annotation with a leader line.

Procedure:

- Type MLEADER (or QLEADER for older versions) and press Enter.

- Click the starting point of the leader line.

- Move the cursor and click to place the arrow.

- Type the annotation text and press Enter.

DIMEDIT Command in AutoCAD

The DIMEDIT command in AutoCAD is used to modify, rotate, move, or center existing dimensions. It is helpful for adjusting dimension text placement and appearance without deleting and redrawing dimensions.

Usage of DIMEDIT Command

The DIMEDIT command allows you to perform the following actions:

- Home – Moves the dimension text to its default position.

- New – Allows you to edit and replace the dimension text.

- Rotate – Rotates the dimension text to a specified angle.

- Oblique – Tilts the extension lines of a dimension to match an angled surface.

How to Use the DIMEDIT Command

- Move Dimension Text to Default Position (Home)

- Restores dimension text to its original position.

Procedure:

- Type DIMEDIT and press Enter.

- Type Home and press Enter.

- Select the dimension you want to reset.

- The text will return to its default position.

Edit Dimension Text (New)

Replaces the existing dimension text with new custom text.

Procedure:

- Type DIMEDIT and press Enter.

- Type New and press Enter.

- Select the dimension to modify.

- Type the new text and press Enter.

Example: Instead of "50", you can replace it with "50 mm".

Rotate Dimension Text

Changes the angle of dimension text.

Procedure:

- Type DIMEDIT and press Enter.

- Type Rotate and press Enter.

- Select the dimension to modify.

- Enter the rotation angle (e.g., 45°) and press Enter.

Oblique Dimension Lines

Tilts the dimension’s extension lines to match an inclined surface.

Procedure:

- Type DIMEDIT and press Enter.

- Type Oblique and press Enter.

- Select the dimension to modify.

- Enter the obliquing angle (e.g., 30°) and press Enter.

Text

Text in AutoCAD is essential for adding annotations, labels, dimensions, and descriptions to a drawing. AutoCAD provides two primary text types: Single-Line Text (DTEXT) and Multi-Line Text (MTEXT). These text types allow users to communicate design intentions clearly.

Single-Line Text (DTEXT)

Single-line text is a basic text entity where each line is independent. It is useful for simple annotations like labels, names, and dimensions.

How to Create Single-Line Text

- Type TEXT or DTEXT in the command line and press Enter.

- Specify the insertion point by clicking anywhere in the drawing.

- Enter the text height (or press Enter to accept the default).

- Enter the rotation angle (or press Enter for default 0°).

- Type the text and press Enter to complete it.

- Press Enter again to exit the command.

Properties of Single-Line Text

- Each line is treated as an individual object.

- No formatting options (like bold, italic, or paragraph spacing).

- Easily movable and editable.

- Best for short text such as room names, part numbers, or simple notes.

Editing Single-Line Text

- Double-click on the text to edit it directly.

- Use the DDEDIT or ED command to modify text.

- Change properties like font, height, and rotation using Properties Palette (Ctrl + 1).

Multi-Line Text (MTEXT)

Multi-line text is more advanced and supports multiple lines in a single text box. It provides formatting options, making it useful for longer annotations, such as project descriptions, instructions, or notes.

How to Create Multi-Line Text

- Type MTEXT and press Enter.

- Click and drag a rectangle to define the text box.

- The Text Editor opens automatically.

- Type the required text inside the text box.

- Format the text using the ribbon (bold, italic, font changes, alignment, etc.).

- Click outside the text box or press Ctrl + Enter to finish.

Properties of Multi-Line Text

- Supports multiple lines, bullet points, and paragraph formatting.

- Allows different font styles and colors within the same text box.

- Can include symbols, fractions, and special characters (%%D for °, %%C for ⌀).

- More control over alignment, spacing, and text width.

Editing Multi-Line Text

- Double-click on the text to open the M Text Editor.

- Use the PROPERTIES palette (Ctrl + 1) to adjust text height, width, and style.

Text Styles (STYLE Command)

Text styles allow users to standardize text across a drawing by defining font, height, width, and alignment.

How to Create and Modify Text Styles

- Type STYLE and press Enter.

- The Text Style Manager opens.

- Click New to create a new style or modify to change an existing one.

- Choose a Font, set Height, adjust Width Factor, and set Oblique Angle (for slanted text).

- Click Apply and then Close.

- Use TEXT or MTEXT and select the style from the Properties panel.

Common Text Style Settings

- Standard: Default style in AutoCAD.

- Annotative Text: Adjusts size automatically based on drawing scale.

- Bold or Italic Text: Can be achieved by choosing appropriate fonts in the Style Manager.

Converting Text

AutoCAD provides tools to convert text from one form to another.

Converting Single-Line Text to Multi-Line (TXT2MTXT)

- Type TXT2MTXT and press Enter.

- Select the single-line text objects to convert.

- Press Enter to combine them into a multi-line text block.

Exploding Multi-Line Text to Single-Line (EXPLODE)

- Type EXPLODE and press Enter.

- Select an MTEXT object.

- Press Enter to finish.

Symbols

AutoCAD uses various symbols to represent different geometric, annotation, and engineering elements. These symbols help in technical drawings, schematics, and architectural plans.

Common AutoCAD Symbols

A. Geometric Symbols

| Symbol | Name | Description |
| --- | --- | --- |
| ⌀ | Diameter | Represents the diameter of a circle. Used in dimensions. |
| R | Radius | Shows the radius of an arc or circle. |
| ∠ | Angle | Used to indicate angular dimensions. |
| ∅ | Round | Used to specify circular or rounded features. |
| ∆ | Triangle | Used for datum reference or tolerance zones. |

B. Dimension & Annotation Symbols

| Symbol | Name | Description |
| --- | --- | --- |
| ± | Tolerance | Represents plus/minus tolerance in dimensions. |
| ‡ | Reference | Denotes a reference dimension (not an actual measurement). |
| ⊥ | Perpendicular | Indicates a 90-degree relationship between two objects. |
| // | Parallel | Shows that two lines or surfaces are parallel. |
| ≠ | Not Equal | Used in constraints to specify inequality. |

C. Architectural & Structural Symbols

| Symbol | Name | Description |
| --- | --- | --- |
| ▭ | Door | Represents a door in floor plans. |
| └──┘ | Window | Symbolizes a window in building drawings. |
| ⌂ | Roof | Used in elevation drawings to indicate a roof. |

D. Electrical Symbols

| Symbol | Name | Description |
| --- | --- | --- |
| ⏚ | Ground | Represents electrical grounding. |
| ⏛ | Transformer | Symbolizes an electrical transformer. |
| ⌁ | Alternating Current (AC) | Used to indicate AC power supply. |
| ⎓ | Direct Current (DC) | Represents DC power source. |

E. Welding Symbols

| Symbol | Name | Description |
| --- | --- | --- |
| △ | Fillet Weld | Specifies a fillet weld. |
| ○ | Spot Weld | Indicates a spot weld location. |
| ─┴─ | Seam Weld | Represents a continuous weld seam. |

Scale and Paper Sizes in AutoCAD

Understanding Scale in AutoCAD

In AutoCAD, drawings are created in real-world dimensions using Model Space, while they are presented and printed at different scales in Layout Space.

Types of Scale in AutoCAD:

- Model Space Scale: Always 1:1 (real-world dimensions).

- Viewport Scale (Layout Space): Controls how the model is displayed in paper space.

Common Drawing Scales:

| Scale | Ratio | Used for |
| --- | --- | --- |
| 1:1 | Full Scale | Manufacturing drawings |
| 1:2 | Half Scale | Mechanical parts |
| 1:10 | 10 times smaller | Architectural floor plans |
| 1:50 | 50 times smaller | Site plans, large buildings |
| 1:100 | 100 times smaller | City maps, large-scale drawings |

How to Set Scale in Viewport (Layout Space)

- Switch to Layout and activate a Viewport.

- Select the Viewport, then choose a scale from the Properties Panel (e.g., 1:50).

- Lock the Viewport to prevent accidental changes.

Paper Sizes

AutoCAD supports different paper sizes based on ISO (A-series) and ANSI standards.

| Paper Size | Dimensions (mm) | Dimensions (inches) |
| --- | --- | --- |
| A0 | 841 × 1189 | 33.1 × 46.8 |
| A1 | 594 × 841 | 23.4 × 33.1 |
| A2 | 420 × 594 | 16.5 × 23.4 |
| A3 | 297 × 420 | 11.7 × 16.5 |
| A4 | 210 × 297 | × 11.7 |

ISO (International Standard) A-Series Paper Sizes

Drawing Limits and Units

Drawing Limits

Drawing limits define the drawing boundaries, usually matching the sheet size. In AutoCAD, setting limits correctly helps display the grid over the working area. To show all grid limits, enable the Grid element from the status bar.

Drawing Limits define drawing sheet with horizontal and vertical coordinates as shown below.

Procedure

Step-1: Type limits command in command bar, you will get the following instructions.

Step-2: Define the lower Left corner as 0, 0

Step-3: Define the Upper Right corner as 100-feet, 100-feet.

Step-4: Grid Display: Now you can see the defined limits to On Grid elements in status bar

Unit Control

The dialogue box is separated into four main sections.

Length type: Which refers to linear units: Like Decimal, Architectural, Engineering etc.

Angles: Referring to angular units

Precision: Precision is used to help a user to specify the precision value is to be needed for the drawing. Ideally precision value varies in terms of number of zeros after the decimal or fraction values. More Zero after decimal more precision will occur in drawing.

Insertion Scale: Selection of different measurement as per length type, like feet or inches for Architectural drawing.

AutoCAD Commands and Tools

AutoCAD provides a wide range of commands and tools for creating, modifying, and annotating drawings efficiently. Below is a categorized list of essential AutoCAD commands and their uses.

A. Basic Drawing Commands

| Command | Description |
| --- | --- |
| LINE | Draws a line between two points |
| POLYLINE | Creates a connected series of line segments |
| CIRCLE | Draws a circle |
| ARC | Creates an arc |
| RECTANGLE | Draws a rectangle |
| ELLIPSE | Draws an ellipse |

B. Modify Commands

| Command | Description |
| --- | --- |
| MOVE | Moves objects from one location to another |
| COPY | Creates a duplicate of selected objects |
| ROTATE | Rotates selected objects |
| SCALE | Resizes an object proportionally |
| MIRROR | Creates a mirrored copy of an object |
| OFFSET | Creates parallel copies of objects |
| TRIM | Cuts off parts of an object using another object as a boundary |
| EXTEND | Extends a line to meet another object |
| FILLET | Creates a rounded corner between two lines |
| CHAMFER | Creates a beveled edge between two lines |
| STRETCH | Stretches a portion of an object |

C. Annotation Commands

| Command | Description |
| --- | --- |
| TEXT | Creates a single-line text |
| MTEXT | Creates multi-line text |
| DIMLINEAR | Adds a linear dimension |
| DIMALIGNED | Creates a dimension aligned with an object |
| DIMRADIUS | Adds a radius dimension |
| DIMDIAMETER | Adds a diameter dimension |
| LEADER | Draws an annotation leader line |

D. Layers & Properties Commands

| Command | Description |
| --- | --- |
| LAYER | Opens the layer manager |
| LAYISO | Isolates a specific layer |
| LAYOFF | Turns off selected layers |
| MATCHPROP | Copies properties from one object to another |

E. Blocks & Hatching Commands

| Command | Description |
| --- | --- |
| BLOCK | Creates a block from selected objects |
| INSERT | Inserts a saved block |
| HATCH | Fills a closed shape with a pattern |
| ARRAY | Creates multiple copies of objects in a pattern |

F. View & Navigation Commands

| Command | Description |
| --- | --- |
| ZOOM | Zooms in or out |
| PAN | Moves the view without changing zoom |
| REGEN | Regenerates the drawing to update display |

G. Selection & Inquiry Commands

| Command | Description |
| --- | --- |
| DIST | Measures the distance between two points |
| ID | Displays the coordinates of a selected point |
| LIST | Displays properties of selected objects |
| SELECT | Selects objects in the drawing |

H. Advanced & Utility Commands

| Command | Description |
| --- | --- |
| XREF | Attaches external references |
| PURGE | Removes unused objects from the drawing |
| AUDIT | Checks for errors in the drawing |
| RECOVER | Recovers a corrupt drawing file |

I. File Management Commands

| Command | Description |
| --- | --- |
| NEW | Creates a new drawing |
| OPEN | Opens an existing drawing |
| SAVE | Saves the current drawing |
| PLOT | Prints the drawing |
| EXPORT | Saves the drawing in another format |

Functions of Creating, Editing and modifying tools

Draw toolbar

Draw Menu bar

Remember

There are six types of lines using drafting AutoCAD

Line:

Line Command is used to draw straight lines between two defined points.

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Line |  | Line/L |

How to draw a line

Step-1: Type L in command Bar

Step-2: Line Specify First Point: 0, 0 and Press Enter

Step-3: Line Specify Next Point: 5, 10 and Press Enter

Draw polyline

A polyline is a single object composed of line and arc segments.

Methods to Draw a Polyline:

Menu Bar: Draw → Polyline → Click on the icon

Shortcut Key: PL → Enter

Command: PL POLYLINE

- Specify start point: Select the point on the graphic window

e.g. 2,2 enter key

- Following message will be displayed in command line

- Current line-width is 0'-0"

- Specify next point or [Arc/Halfwidth/Length/Undo/Width]:

- To respond the message, give your options as per requirements as under.

- To draw the Arc type A and press enter Halfwidth:

- To draw the halfwidth type H and press Length:

- To draw the Arc type L and press enter Undo:

- To draw the Arc type U and press enter Width:

- To draw the Arc type W and press enter

Draw spline

Creates a smooth curve that passes through or near a set of fit points, or that is defined by the vertices in a control frame.

Methods to Draw a SPLINE

Draw menu bar  Spline  click on icon Or

Short key

SPL	Press Enter

Command: SPL    SPLINE

Command Line Messages for Polyline:

- Specify first point: Select a point in the graphics window (e.g., 2,6 → Enter).

- Current settings: Method=Fit Knots=Chord

- Specify first point or [Method/Knots/Object]:

- Method: Defines whether the spline uses fit points or control vertices.

- Knots: Determines how curve segments blend between fit points (controlled by SPLKNOTS system variable).

- Object: Converts 2D/3D spline-fit polylines into splines.

Draw Construction Line

- Creates a construction line of infinite length.

- Draw menu bar → construction Line → click on icon Or short key

- XL → Enter

Command: XL or XLINE

Following message will be displayed in command line

- Specify start point: Select the point on the graphic window e.g. 2 enter key

- Specify a point or [Hor/Ver/Ang/Bisect/Offset]:

- Hor (Horizontal): Draw the horizontal line

- Ver (Vertical): Draw the vertical line

- Ang (Angle): Draw the angle line

- Bisect: Bisect an angle define by a vertex and two end points.

- Offset: Draw construction line in offset.

Draw Ray line

A Ray is a linear object that starts at a point and extends infinitely in one direction.

Methods to Draw a Ray:

- Menu Bar: Draw → Ray → Click on the icon

- Shortcut Key: RAY → Enter

Command: Ray

Following message will be displayed in command line

- RAY Specify start point: Select the point on the graphic window e.g. 6 enter key

- Specify through point: To respond the message, give your options as per requirements

Draw Multi line

Multiline are composed of parallel lines, called elements. When you draw a multiline, you can use the STANDARD style, which has two elements

Draw menu bar → multi Line → click on icon Or

short key ML→ Enter

Multiline (ML)

- Command: ML → Enter

- Current Settings:

- Justification = Top

- Scale = 1.00

- Style = STANDARD

Command Line Options:

- Justification: Defines multiline position (Top, Bottom, Center).

- Scale: Sets the spacing between multiline components.

- Style: Defines the multiline style (e.g., STANDARD, custom styles).

Example:

- Start point: 10,10 → Enter

- Next points: 22,10 → 22,22 → 10,22 → C (Close)

Justification Option

The justification determines how a multiline is drawn between the specified points. Three justifications are available for the MLINE command. They are Top, Zero, and Bottom as shown in following figure.

Justification in multiline

Top

This justification provides with the points produces a multiline in which the top line coincides with the points you selected on the screen.

Zero

This option will produce a multiline in which the zero offset position o the multiline coincides with the selected points.

Bottom

This option will produce a multiline in which the bottom line (the line with the least offset distance) coincides with the selected point when the line is drawn from the left to right.

Figure: Different multiline scales

Scale Option

The scale option is used to change the scale of a multiline. For example, if the scale factor is 0.5, the distance between the lines (offset distance) will be reduced to half, as shown in following figure. Therefore, the width of the multiline will be half of what was defined of multiline style.

Line Type

Line Types in AutoCAD

Line types can be continuous or patterned (dashes, dots, text, symbols).

The current line type setting controls the default appearance of new objects.

Ways to Check/Change Line Type:

- Ribbon: Home tab → Properties panel (when no objects are selected).

Command Line:

- Type: LT → Enter

- AutoCAD Window: Displays line type settings and options.

Line Type Scale

Line type scale controls the size and spacing of repetitions of the line type pattern for each drawing unit. There are a number of scaling options that can affect in line by followings steps.

- Type LTS enter and type scale factor.

Circle Command and its parts

A circle is a simple closed shape

- The figure formed by the group of points that are an equal distance from a point, or center. The angles of a circle add up to 360 degrees.

- Circumference: Length of circle is called circumference.

- Radius: A straight line from the center to the circumference of a circle

- Diameter: A straight line passing from side to side through the center of a body or figure, esp. a circle or sphere.

- Chord: Any Straight Line which connects two points of circle is called chord, where as it does not pass from center of circle.

- Tangent: Any Straight Line which touches one point of circle is called Tangent.

- Arc: Part of a circle between any two points is called arc.

- Sector: Any area which is constructed by two Radiuses and an Arc is called Sector.

- Segment: Any area which is constructed by chord and an arc is called segment.

Procedure: How to create circle

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Circle |  | Circle/C |

Step-1: Type C, Circle in command Bar and Press Enter

Step-2: Specify any Point for Centre I.e. 5, 5 and Press Enter

Step-3: Specify radius of Circle or Diameter: 5 and Press Enter.

Tan Tan Radius (TTR)

Procedure: How to create circle with TTR

Step-1: Type C and press ENTER (You See: 3P/2P/TTr)

Step-2: Type T for (Tan Tan Radius (TTR))

Step-3: Specify point on object for first tangent of circle: Click on the First Tangent Element (Where the circle will touch)

Step-4: Specify point on object for second tangent of circle: Click on the Second Tangent Element (Where the circle will touch)

Step-5: Specify radius of circle <0'-0>: 2

3-Points Circle:

Procedure: How to create circle with 3-P

Step-1: Type C Command (see methods above)

Step-2: 3P/2P/TTR/<Center point>: Type 3p and then press.

Step-3: Specify first point on circle: Click on First Point.

Step-4: Specify second point on circle: Click on second Point.

Step-5: Specify third point on circle: Click on third Point

2-Points Circle

Procedure: How to create circle with 2-P

Step-1: Type C Command (see methods above)

Step-2: 3P/2P/TTR/<Center point>: Type 2P and then press.

Step-3: Specify first end point of circle's diameter: Click on First Point

Step-4: Specify second end point of circle's diameter: Click on second point

Tan Tan Tan (TTT)

Another sub command of Circle you can activate from the Draw Manu:

Draw →circle→ TTT

Procedure: How to create circle with TTT

Step-1: Click on Draw Manu: Draw→ Circle→ TTT

Step-2: 3p Specify first point on circle: _tan to. First Point

Step-3: 3p Specify second point on circle: _tan to. Second Point

Step-4: 3p Specify third point on circle: _tan to. Third Point

Arc Command:

Arc is used to draw an arc between two lines from one point to the next.

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Arc |  | Arc/A |

Procedure

How to draw ARC

Command entry: arc

Specify start point of arc or [Center]: Specify a point, enter c, or press ENTER to start tangent to last line, arc, or polyline

To create an arc, you can specify combinations of center, endpoint, start point, radius, angle, chord length, and direction values.

Start Point

Specifies the starting point of the arc.

Note: If you press ENTER without specifying a point, the endpoint of the last drawn line or arc is used and you are immediately prompted to specify the endpoint of the new arc. This creates an arc tangent to the last drawn line, arc, or polyline.

Specify second point of arc or [Center /End]:

Second Point

Draws an arc using three specified points on the arc's circumference. The first point is the start point (1). The third point is the endpoint (3). The second point (2) is a point on the circumference of the arc.

Specify end point of arc: Specify a point (3)

You can specify a three-point arc either clockwise or counter clockwise.

Center

- Specifies the center of the circle of which the arc is a part. Specify center point of arc:

- Specify end point of arc or [Angle/chord Length]:

End Point

Using the center point (2), draws an arc counter clockwise from the start point (1) to an endpoint that falls on an imaginary ray drawn from the center point through the third point (3).

The arc does not necessarily pass through this third point, as shown in the illustration.

Angle

Draws an arc counter clockwise from the start point (1) using a center point (2) with a specified included angle. If the angle is negative, a clockwise arc is drawn.

Specify included angle: Specify an angle

Chord Length

Draws either a minor or a major arc based on the distance of a straight line between the start point and endpoint.

If the chord length is positive, the minor arc is drawn counter clockwise from the start point. If the chord length is negative, the major arc is drawn counter clockwise.

Specify length of chord: Specify a length

End

Specifies the endpoint of the arc. Specify end point of arc:

Specify center point of arc or [Angle/Direction/Radius]:

Center Point

Draws an arc counter clockwise from the start point (1) to an endpoint that falls on an imaginary ray drawn from the center point (3) through the second point specified (2).

Angle

Draws an arc counter clockwise from the start point (1) to an endpoint (2), with a specified included angle. If the angle is negative, a clockwise arc is drawn.

Specify included angle: Enter an angle in degrees or specify an angle by moving the pointing device counter clockwise.

Direction

Begins the arc tangent to a specified direction. It creates any arc, major or minor, clockwise or counter clockwise, beginning with the start point (1), and ending at an endpoint (2). The direction is determined from the start point.

Specify tangent direction for the start point of arc:

Radius

Draws the minor arc counter clockwise from the start point (1) to the endpoint (2). If the radius is negative, the major arc is drawn. Specify radius of arc:

Center

Specifies the center of the circle of which the arc is a part. Specify center point of arc:

Specify start point of arc:

Specify end point of arc or [Angle/chord Length]:

End Point

Draws an arc counter clockwise from the start point (2) to an endpoint that falls on an imaginary ray drawn from the center point (1) through a specified point (3).

Angle

Draws an arc counter clockwise from the start point (2) using a center point (1) with a specified included angle. If the angle is negative, a clockwise arc is drawn.

Specify included angle:

Chord Length

Draws either a minor or a major arc based on the distance of a straight line between the start point and endpoint.

If the chord length is positive, the minor arc is drawn counter clockwise from the start point. If the chord length is negative, the major arc is drawn counter clockwise.

Specify length of chord:

Tangent to Last Line, Arc, or Polyline

Draws an arc tangent to the last line, arc, or polyline drawn when you press ENTER at the first prompt.

Specify end point of arc: Specify a point (1)

Rectangle

Creates a rectangular poly line from the specified the rectangle parameters (length, width, rotation) and type of corners (fillet, chamfer, or square).

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Rectangle |  | rec |

There are basic three types of rectangle, area, dimension and rotation.

Procedure

How to draw rectangular

Click Home tab  Draw panel  Rectangle.  Specify the first corner of the rectangle. Specify the other corner of the rectangle.

OR

Short key REC Enter

Area

Creates a rectangle using the area and either a length or a width. If the Chamfer or Fillet option is active, the area includes the effect of the chamfers or fillets on the corners of the rectangle.

Dimensions

Creates a rectangle using length and width values.

Rotation

Creates a rectangle at a specified rotation angle.

Chamfer

Sets the chamfer distances for the rectangle.

Elevation

Specifies the elevation of the rectangle.

Fillet

Specifies the fillet radius of the rectangle

Thickness

Specifies the thickness of the rectangle.

Width

Specifies the polyline width of the rectangle to be drawn.

Polygon

Polygons can be used to draw equilateral triangles, squares, pentagons, hexagons, and other multi-sided figures.

The following illustrations show polygons created using three methods: inscribed, circumscribed, and edge. In each case, two points are specifying.

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Polygon |  | POL |

Command: polygon

Enter number of sides <current>: Enter a value between 3 and 1024 or press ENTER Specify center of polygon or [Edge]: Specify a point (1) or enter

Center of Polygon

Defines the center of the polygon.

Enter an option [Inscribed in circle/Circumscribed about circle] <current>: Enter i or c or press ENTER

Inscribed in Circle

Specifies the radius of a circle on which all vertices of the polygon lie. Specify radius of circle: Specify a point (2) or enter a value

Specifying the radius with your pointing device determines the rotation and size of the polygon. Specifying the radius with a value draws the bottom edge of the polygon at the current snap rotation angle.

Circumscribed about Circle

Specifies the distance from the center of the polygon to the midpoints of the edges of the polygon.

Specify radius of circle: Specify a distance

Specifying the radius with your pointing device determines the rotation and size of the polygon. Specifying the radius with a value draws the bottom edge of the polygon at the current snap rotation angle.

Ellipse

When you draw an ellipse, its shape is determined by two axes that define its length and width: the major (longer) axis and the minor (shorter) axis.

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Ellipse |  | EL |

Axis Endpoint

Defines the first axis by its two endpoints. The angle of the first axis determines the angle of the ellipse. The first axis can define either the major or the minor axis of the ellipse.

Specify other endpoint of axis: Specify a point (2)

Specify distance to other axis or [Rotation]: Specify a distance by entering a value or locating a point (3), or enter r

Distance to second Axis

Defines the second axis using the distance from the midpoint of the first axis to the endpoint of the second axis (3).

Rotation

Creates the ellipse by appearing to rotate a circle about the first axis.

Specify rotation around major axis: Specify a point (3), or enter an positive angle value less than 90

Move the crosshairs around the center of the ellipse and click. If you enter a value, the higher the value, the greater the eccentricity of the ellipse. Entering 0 defines a circular ellipse.

Arc

Creates an elliptical arc.

The first two points of the elliptical arc determine the location and length of the first axis. The third point determines the distance between the center of the elliptical arc and the endpoint of the second axis. The fourth and fifth points are the start and end angles.

The angle of the first axis determines the angle of the elliptical arc. The first axis can define either the major or the minor axis depending on its size.

Specify axis endpoint of elliptical arc or [Center]: Specify a point or enter c

Hatch

Fills an enclosed area or selected objects with a hatch pattern, solid fill, or gradient fill. In hatch apply different material as required and justify the scale

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Hatch |  | H |

Command: Hatch

The Hatch and Gradient dialog box is displayed. If you enter -hatch at the Command prompt, options are displayed.

Note: By default, the program will not create a hatch pattern that consists of over 10,000 hatch segments. The limit is set by the Max Hatch setting in the registry. To reset the limit to 50,000, for example, enter (setenv "Max Hatch" "50000") at the Command prompt. The limit can be reset to any value between 100 and 10,000,000. You can choose from several methods to specify the boundaries of a hatch.

- Specify a point in an area that is enclosed by objects.

- Select objects that enclose an area.

- Drag a hatch pattern into an enclosed area from a tool palette or Design Center.

Donut

Creates a filled circle or a wide ring or Donut is special type of circle that describes inside outside circle in a same time.

A donut consists of two arc polylines that are joined end-to-end to create a circular shape. The width of the polylines is determined by the specified inside and outside diameters. If you specify an inside diameter of 0, the donut is a filled circle.

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Donut |  | DO |

Specifies the inside diameter of the donut as shown.

Command: donut

A donut consists of two arc polylines that are joined end-to-end to create a circular shape. The width of the polylines is determined by the specified inside and outside diameters. To create solid-filled circles, specify an inside diameter of zero.

Specify inside diameter of donut <current>: Specify a distance or press ENTER If you specify an inside diameter of 0, the donut is a filled circle.

Specify outside diameter of donut <current>: Specify a distance or press ENTER Specify center of donut or <exit>: Specify a point (1) or press ENTER to end the command.

The location of the donut is set based on the center point. After you specify the diameters, you are prompted for the locations at which to draw donuts. A donut is drawn at each point specified (2). How the interior of a donut is filled depends on the current setting of the FILL command.

Point

To set point style and size

POINT Creates a point object.

| Draw Manu | Draw Toolbar | Command Entry |
| --- | --- | --- |
| Point |  | point |

Command entry: point

Specify a point:

Points can act as nodes to which you can snap objects. You can specify a full three- dimensional location for a point. The current elevation is assumed if you omit the Z coordinate value.

The PDMODE and PDSIZE system variables control the appearance of point objects. PDMODE values 0, 2, 3, and 4 specify a figure to draw through the point. A value of 1 specifies that nothing is displayed.

Specifying the value 32, 64, or 96 selects a shape to draw around the point, in addition to the figure drawn through it:

Block

A block is simply a collection of objects (it could be one object) that has a name. Blocks have several advantages:

You can insert them again and again, saving time. In fact, you can insert them into other drawings as well.

A block uses less electronic space than individual objects, so your drawing file is smaller.

By updating a block’s definition, you can update all the blocks in the drawing.

How do you create a block?

To create a block, follow these steps:

- Draw the objects that you want in the block.

- Choose Home tab> Block panel> Create to start the BLOCK command. The Block Definition dialog box opens.

- Type a name in the Name text box. The name can have spaces.

- You need to specify a base point. That’s the point at which you’ll insert the block. In the Base Point section, click Pick Point. Be sure to use an object snap for accuracy! You’ll immediately be returned to the dialog box.

- In the Objects section, click the Select Objects button. Select the objects and press Enter to return to the dialog box. Tip: You can select the objects before using the command and they’ll show up in the dialog box.

- Just below, choose Retain, Convert to Block, or Delete. These options control what happens after you create the block.

- In the Behavior section, you can make a block Annotative (more info here), force it to scale uniformly and choose whether to allow exploding.

- In the Settings area, choose the block unit. You can choose Unitless but if you choose a unit, AutoCAD will try to scale the block appropriately when you insert it into another drawing. You can also add a hyperlink if you want.

- Finally, you can add a description in the Description box. A description is helpful in the Design Center, when you want to insert the block from another drawing.

- Click OK to complete the box. If you chose Delete, the objects disappear. You can use the OOPS command to bring them back.

Types of file extensions for saving files

Plotting for Save drawing in different format (e.g. dwg, PDF, JPG. etc.).

In this unit learner able to print out the Drawing in different formats like

- .PDF

- .JPG etc

Method of Saving Drawing in PDF

Step 1:

Applying command print in AutoCAD then show the dialog box of print / plot like (show in picture) Then click the PDF option.

Step 2:

Select the Drawing with Window option lick (showing in the picture) and click the OK option

Step 3:

Then give the path where you want save drawing lick (showing in the pictures)

Method of Saving Drawing in JPG.

Step 1:

Applying command print in AutoCAD then show the dialog box of print / plot like (show in the picture)

Then click the JPG option

Step 2:

Select the Drawing with Window option lick (showing in the picture) And click the OK option

Step 3:

Then give the path where you want save drawing lick (showing in the pictures)

Material Required

- Computer lab

- Computer 20

- Each 20 Trainee

- Multimedia

- Printer with scanner(color)

- White board

- White board marker

- Papers rim

- soft wear

Practical Activities Interpret and develop 2D drawing

Activity 1

Activity 2

Prepare the following object with its defined dimension.

Activity 3

|  | Develop 3D Models |
| --- | --- |

Learning Outcomes:

After Completion of this learning Units, you will be able to:

- Practice different types of 3D commands (Extrude, Extrude Cut, Revolve, Sweep, Blend, Loft etc.)

- Check and inspect design requirements for the productive component to be produced.

- Use appropriate computer software for developing 3Dmodeloftheproduct, component to reassembly.

- Use appropriate user interface and apply relevant commands for developing 3D model or component.

- Modify 3D Model in line with the requirements.

3D and Its concepts

What is 3D?

3D refers to objects or spaces that have three dimensions length, width, and depth.

Unlike 2D (two-dimensional) objects, which only have length and width, 3D objects have depth, making them appear more realistic and lifelike. 3D Space Defined by three axes X (horizontal), Y (vertical), and Z (depth). Coordinates: Points in 3D space are represented as (X, Y, Z).

Functions of 3D tools and commands

The functions of 3D tools and commands in software like AutoCAD, Blender, SolidWorks, or other 3D modeling programs are to create, modify, analyze, and visualize three-dimensional objects. These tools and commands enable users to design complex models, simulate real-world conditions, and produce accurate technical drawings or renderings. Below is a detailed breakdown of their functions.

Types of 3D shapes

Basic Geometric Primitives

- These are simple, fundamental 3D shapes that serve as building blocks for more complex models.

Cube

- A six-faced shape with all sides equal (e.g., a dice).

- Properties: 6 faces, 12 edges, 8 vertices.

Sphere

- A perfectly round shape with all points on its surface equidistant from the center (e.g., a ball).

- Properties: No edges or vertices, perfectly symmetrical.

Cylinder (CYL)

- A shape with two parallel circular bases connected by a curved surface (e.g., a can).

- Properties: 2 circular faces, 1 curved face, 2 edges.

Cone

- A shape with a circular base and a single vertex (apex) (e.g., an ice cream cone).

- Properties: 1 circular face, 1 curved face, 1 vertex.

Pyramid

- A shape with a polygonal base and triangular faces meeting at a common vertex (e.g., the Great Pyramid of Giza).

- Properties: Base can be triangular, square, pentagonal, etc.

Rectangular Prism (Cuboid)

- A six-faced shape with rectangular faces (e.g., a box).

- Properties: 6 faces, 12 edges, 8 vertices (like a cube, but sides are not equal).

Torus

- A doughnut-shaped object with a hole in the center.

- Properties: Circular cross-section, no edges or vertices.

Polyhedral (3D shapes with flat polygonal faces)

- Tetrahedron – A polyhedron with four triangular faces.

- Octahedron – A polyhedron with eight triangular faces.

- Dodecahedron – A polyhedron with twelve pentagonal faces.

- Icosahedron – A polyhedron with twenty triangular faces.

Common commands and Tool used

The following common commands used in 3D modeling.

Extrude Surface

The extrude command in AutoCAD creates a 3D surface by extending a 2D object along a specified direction. Unlike extruded solids, extruded surfaces have no thickness unless later converted into solids.

Steps to Create an Extruded Surface:

- Draw a 2D Shape (e.g., line, circle, spline, or polyline).

- Type EXTRUDE in the command line and press Enter or select extrude surface from ribbon

- Select the 2D shape. Specify the extrusion height or direction.

Loft Command

The LOFT command in AutoCAD creates a 3D surface by smoothly connecting multiple cross-sections (profiles). It is commonly used to generate complex and free form surfaces in mechanical, and product design.

- Create open or closed curves (splines, arcs, polylines, etc.) in different planes.

- Type LOFT in the command line and press Enter. or select from menu

- Click the profiles in the desired order.

- Choose from different lofting options: Normal AutoCAD generates a smooth transition between profiles. Railed Uses guide rails for more control over the shape. Point-Based Controls lofting using a point to influence the shape.

- Press Enter to Create the Surface. the lofted surface is generated.

Sweep Command

The SWEEP command in AutoCAD is used to create 3D surfaces by extruding a 2D profile along a specified path. This is useful for modeling pipes, tubes, and other complex curved surfaces.

- Draw a 2D shape (e.g., circle, rectangle, or custom profile).

- Draw a path (line, arc, spline, or polyline) along which the profile will be swept.

- Activate the Sweep Command:

- Type SWEEP in the command line and press Enter. or select from menu

- Select the profile first -entre, then select the path.

Revolve Surface

The REVOLVE command in AutoCAD creates a 3D surface by rotating a 2D profile around an axis. This is commonly used for designing cylindrical, spherical, and symmetrical objects such as bottles, wheels, and machine components.

- Create an open or closed shape using lines, arcs, splines, or polylines.

- Ensure the profile is positioned correctly relative to the axis of rotation.

- Type REVOLVE in the command line and press Enter.

- Select the profile to be revolved.

- Select the axis (two points defining the rotation axis).

- Enter a value (e.g., 360° for a full revolution, or a custom angle).

- Press Enter to generate the revolved surface.

Surface Blend

The Surface Blend tool in AutoCAD creates a smooth transition between two existing surfaces.

- The surfaces should have an open edge where the blend will be applied.

- Type BLENDSRF in the command line and press Enter.

- Click on the open edges of the two surfaces that need to be blended.

- Position (G0) – Basic connection between the surfaces.

- Tangency (G1) – Ensures a smooth transition with no sharp edges.

- Curvature (G2) – Creates an even smoother blend with curvature continuity

Offsetting Surfaces

Offsetting surfaces in AutoCAD allows you to create a parallel surface at a specified distance from an existing one. This is a very useful function for creating things like the thickness of a part.

- Navigate to the "Surface" tab in the AutoCAD ribbon.

- Locate the "Create" panel and click on "Offset." You can also type "OFFSET" in the command line.

- In the drawing area, select the surface you want to offset.

- Press Enter to confirm your selection.

- Enter the desired distance for the offset.

- Pay attention to the arrows that appear, indicating the offset direction. You may have the option to "Flip Direction" if needed.

- Some versions of autocad will give you the option to create a solid object from the offset. If this is desired, select the solid option.

Surface Fillet

When working with surfaces in AutoCAD, "filleting" refers to creating a smooth, rounded transition between two intersecting surfaces.

- Ensure the surfaces you want to fillet intersect or have edges that can be smoothly connected.

- It may be necessary to trim or extend surfaces to achieve a proper intersection.

- the object should not be a single entity

- activate filet, select first object then second object.

Surface Trim

Surface Trim in AutoCAD removes portions of a surface by using another surface or a curve as a cutting edge.

- Type "SURFTRIM" in the command line and press Enter.

- or find it within the surface portion of the ribbon.

- Select the surface you want to trim.

- Press Enter to confirm.

- Select the surface or curve that will act as the cutting edge.

- Press Enter to confirm.

- AutoCAD will highlight the portions of the surface that will be removed.

- Click on the side of the surface that you want to keep.

- The unwanted portion of the surface will be removed

Surface Extend

"Surface Extend" command is used to lengthen or expand the boundaries of a surface.

- Type SURFEXTEND (or EXTENDSURF) in the command line and press Enter.

- Or locate the Surface Extend command within the Surface tab on the ribbon.

- Select the surface you want to extend or select edge.

- Press Enter to confirm your selection.

- Select the edge(s) of the surface that you want to extend.

- Press Enter to confirm your selection.

- Enter the distance you want to extend the surface.

- You might be able to visually drag the edge to extend

Converting a 2D sketch into a 3D model

Converting a 2D sketch into a 3D model in AutoCAD involves several steps, including creating the 2D drawing, extruding, revolving, and modifying it to form a 3D solid.

Follow this step-by-step guide:

Step 1: Prepare the 2D Sketch:

- Open AutoCAD and start a new drawing.

- Use the Line (L), Circle (C), Arc (A), and Rectangle (REC) commands to draw your 2D shape.

- Ensure your sketch is fully closed if you plan to extrude it into a solid.

Step 2: Switch to 3D Modeling Workspace:

- Go to the Workspace menu in the top-right corner.

- Select 3D Modeling to enable 3D tools.

- Change the visual style to Shaded or Realistic for better visualization.

Step 3: Convert 2D Sketch to 3D Using Various Methods:

Method 1: Extrusion (For Linear Depth Objects):

- Select the 2D shape.

- Type "EXTRUDE" (Shortcut: EXT) and press Enter.

- Specify the height or depth for extrusion and press Enter.

- Your 2D shape is now a 3D solid.

Method 2: Revolve (For Circular or Rotational Objects)

- Select the 2D profile of a shape (e.g., a half-profile of a cylinder or cone).

- Type "REVOLVE" (REV) and press Enter.

- Select the axis of rotation (e.g., the centerline of a cylinder).

- Enter the angle of revolution (default is 360° for a full solid).

Method 3: Lofting (For Complex Shapes Between Profiles)

- Draw multiple 2D profiles at different positions.

- Type "LOFT" and press Enter.

- Select the profiles in order and press Enter.

- AutoCAD will create a smooth transition between them.

Method 4: Press pull (For Modifying Enclosed Areas)

- Type "PRESSPULL" (PUSH/PULL) and press Enter.

- Click inside a closed 2D shape.

- Drag to give it depth or height and press Enter.

Step 4: Modify the 3D Model

- Use Fillet (FILLETEDGE) to round edges.

- Use Chamfer (CHAMFEREDGE) for beveled edges.

- Use Boolean operations like Union, Subtract, and Intersect to combine or cut solids.

Step 5: Save and export the 3D Model

- Save the file in .DWG format.

- Export to STL or STEP if needed for 3D printing or further processing.

3.3 Introduction to Model Tree of 3D Modeling

Model Tree (similar to the feature tree in other CAD software). However, AutoCAD does not have a dedicated "Model Tree" like SolidWorks or CATIA. Instead, AutoCAD's 3D modeling structure relies on layers, blocks, groups, and the Properties Palette for managing 3D objects.

AutoCAD 3D Modeling Interface (Model Tree Equivalent)

A possible breakdown of the Model Tree in AutoCAD includes:

Layers

- Helps organize objects by category (e.g., construction, surfaces, solids).

Blocks & Groups

- Blocks: Reusable 3D objects or assemblies.

- Groups: Collections of objects treated as one unit.

Solid History (For Parametric Modeling)

- Shows the parent-child relationship of extrusions, lofts, sweeps, and other operations.

Properties Palette

- Displays the properties of selected objects, including material, dimensions, and transformation details.

View Cube & Navigation Bar

- Provides easy manipulation of 3D views.

Command Line & Ribbon Tools

- Allows access to modeling commands such as EXTRUDE, REVOLVE, SWEEP, LOFT, UNION, SUBTRACT, INTERSECT.

Introduction to Editing Tool of 3D Model

Editing Tools in 3D Modeling in AutoCAD

AutoCAD provides several editing tools for modifying 3D models. These tools allow users to transform, reshape, and refine their models efficiently. Some key 3D editing tools include:

Move

- Moves objects in 3D space.

Rotate 3D

- Rotates objects around a specified axis.

Scale

- Scales objects proportionally in all directions.

Mirror 3D

- Creates a mirrored copy of objects along a plane.

Align

- Aligns objects with other objects in 3D space.

3D Array

- Creates multiple copies of objects in a pattern.

Fillet and Chamfer

- Rounds or bevels edges of 3D objects.

Shell

- Creates a hollow version of a solid.

Extrude

- Extends 2D profiles into 3D solids.

Press pulls

- Extrudes or offsets a face dynamically.

Sweep

- Moves a 2D profile along a specified path.

Loft

- Creates a smooth transition between two or more 2D profiles.

Slice

- Cuts a 3D object into separate pieces.

Union

- Combines two or more solids into one.

Subtract

- Removes one solid from another.

Intersect

- Creates a new solid from overlapping volumes.

Offset Edge

- Offsets edges of a 3D solid.

Convert to Surface/Solid

- Converts objects between surface and solid types.

STL Out

- Exports models for 3D printing.

File Management in Computer Systems for AutoCAD

File management in AutoCAD refers to the process of organizing, saving, retrieving, and maintaining drawing files efficiently. Proper file management ensures smooth workflow, prevents data loss, and improves collaboration in engineering and design projects.

Key Aspects of File Management in AutoCAD

File Formats

- DWG: The primary file format for AutoCAD drawings.

- DXF (Drawing Exchange Format): Used for interoperability with other CAD software.

- DWT (Drawing Template): Used for predefined settings and standardization.

- BAK (Backup File): Auto-generated backup file for recovering unsaved work.

- SV$ (Autosave File): Temporary file created for crash recovery.

Saving and Backup

- Regularly save files using CTRL + S to avoid data loss.

- Enable AutoSave (Options > Open and Save > AutoSave every X minutes).

- Maintain multiple backup copies and use cloud storage solutions like OneDrive or Google Drive.

File Organization

- Store project files in structured folders (e.g., /Projects/Client Name/Project Name/Drawings).

- Use consistent naming conventions (e.g., Project Name/Version/Date.dwg).

- Use X refs (External References) for managing multiple drawings efficiently.

Version Control

- Keep track of different versions by appending dates or revision numbers.

- Use AutoCAD's Drawing History (available in AutoCAD 2021 and later) to track changes.

File Compression & Cleanup

- Use the PURGE command to remove unused elements and reduce file size.

- Use the AUDIT command to check and fix file corruption.

Sharing & Collaboration

- Use e Transmit to package all related files (X refs, fonts, etc.) for sharing.

- Utilize AutoCAD Web App for cloud-based editing and sharing.

- Work in Sheet Set Manager to manage multi-sheet projects efficiently.

File Security & Protection

- Use password protection for sensitive files (Save As > Tools > Security Options).

- Set user permissions when working in a networked environment.

Materials Required

| - Computer lab | - Computer 20 |
| --- | --- |
| - Each 20 Trainee | - Multimedia |
| - Printer with scanner(color) | - White board |
| - White board marker | - Papers rim |
| - Soft Wear |  |

Practical Activities Draw a 3D model from 2D sketch

Activity -1 Practice to Draw 2D Drawing and into 3D model using extrude and extrude cut command.

Activity -2 Draw and practice 3D model using revolve and sweep command.

Revolve

Sweep

Activity -3 Draw and practice 3D model using loft command.

Activity 4 Draw a 3D model from 2D sketch

Summary

- This model to understanding and developing technical drawings using CAD software.

- It covers essential drawing concepts, including drawing settings, unit selection, and coordinate systems.

- Common CAD tools and commands for creating and modifying 2D and 3D models.

- Learner’s practice drawing geometrical shapes, angles, and applying edit/modify commands. The transition from 2D sketches to 3D models is introduced, including model tree structures and editing tools.

- File management strategies ensure efficient storage and retrieval of designs.

- Advanced 3D commands such as Extrude, Revolve, Sweep, and Loft aid in complex modeling.  By mastering these skills, users can develop and refine mechanical drawings and 3D models efficiently.

FAQs

Q1: Write any four types of geometrical shapes?

Ans: Circle, Square, Rectangle, Triangle.

Q2: Define orthographic projection?

Ans: A method of representing 3D objects in 2D, showing top, front, and side views.

Q3: What is isometric view?

Ans: A 3D representation where all axes are equally foreshortened and angles are 120°.

Q4: Write any five tools used in freehand sketch drawing?

Ans: Pencils, Erasers, Rulers, Compass, Protractor.

Q5: Write types of triangles?

Ans: Equilateral, Isosceles, Scalene, Right-Angle, Acute, Obtuse.

Q6: What is the purpose of creating a seal block in technical sheets?

Ans: It validates the drawing by showing approval or certification.

Q7: Name the types of polygons.

Ans: Triangle, Quadrilateral, Pentagon, Hexagon, Octagon, etc.

Q8: How many types of perspectives are in use?

Ans: One-point, Two-point, Three-point perspective.

Q9: What is AutoCAD?

Ans: AutoCAD is a CAD (Computer-Aided Design) software used for creating engineering drawings and designs.

Q10: What is the shortcut for saving a file in AutoCAD?

Ans: Use CTRL + S or type SAVE command.

Q11: What is the default file format of AutoCAD?

Ans: The default file format is .DWG (Drawing).

Q12: What is the shortcut for the Line command?

Ans: Type L and press Enter.

Q13: Which command is used to draw a circle?

Ans: The C command is used.

Q14: What is the shortcut for the Offset command?

Ans: The O command is used.

Q15: What is the difference between Trim and Extend?

Ans: TRIM removes extra parts of a line, while EXTEND lengthens a line.

Q16: What is the shortcut for the Mirror command?

Ans: The MI command is used.

Q17: What is the shortcut for the Copy command?

Ans: The CO or CP command is used.

Q18: What is the shortcut for the Rotate command?

Ans: The RO command is used.

Q19: What is the function of the Scale command?

Ans: The SC command is used to increase or decrease the size of an object.

Q20: Why are layers important in AutoCAD?

Ans: Layers help organize different objects by categorizing them.

Q21: How to open layer properties?

Ans: Use the LA command.

Q22: What is the function of Grid and Snap?

Ans: The grid provides a reference, and Snap helps in placing objects at exact points.

Q23: Which command is used to write text?

Ans: DT (Single Line Text) and MT (Multi-Line Text) commands are used.

Q24: Which command is used to add dimensions?

Ans: The DIM command is used.

Q25: What is the function of the Hatch command?

Ans: The Hatch command is used to apply patterns or shading to an object.

Q26: What does the Leader command do?

Ans: The LE command attaches text with an arrow.

Q27: What is the difference between Polyline and Line?

Ans: A polyline is a connected segment, whereas a simple line consists of separate segments.

Q28: What is the shortcut for the Pan command?

Ans: The P command is used.

MCQs

1. 	Which sketching technique uses light, quick strokes to outline the shape before 	adding details?

a. Cross-hatching	b. Contour drawing	Gesture drawing	Blending

2.	What tool is commonly used for blending in sketching?

a. Eraser		b. Tortillon (blending stump)

c. Ruler		d. Ink pen

3.	Stippling is a technique that involves:

a. Using parallel lines for shading	b. Creating tones with tiny dots

c. Smudging graphite for smooth shading	d. Drawing with continuous strokes

4.	Which type of shading technique creates soft and smooth gradients?

a. Hatching	b. Cross-hatching	c. Stippling	d. Blending

5.	What does the letter "H" on a drawing pencil indicate?

a. Hardness	b. Softness	c. Darkness	d. Thickness

6.	Which pencil is the softest among the following?

a. 2H	b. HB	c. 4B	d. 6H

7.	What does the letter "B" in pencil grading stand for?

a. Bold	b. Black	c. Blending	d. Basic

8.	Which pencil is commonly used for general writing and sketching?

a. 6B	b. 4H	c. HB	d. 8B

9.	What is the primary characteristic of charcoal pencils compared to graphite pencils?

a. They are harder and lighter	b. They produce darker and richer blacks

c. They are more precise and fine-tipped	d. They are waterproof

10.	Which of the following is NOT a type of charcoal pencil?

a. Soft	b. Medium	c. Hard	d. Mechanical

11.	What is the primary use of a T-Square?

a. Measuring angles	b. Drawing straight horizontal lines

c. Cutting paper		d. Mixing colors

12.	In which profession is a T-Square most commonly used?

a. Architecture and drafting	b. Medicine

c. Cooking		d. Sculpture

13.	What is a seal block primarily used for?

a. Cutting materials		b. Creating stamped impressions

c. Measuring angles		d. Sharpening pencils

14.	Which of the following is a plane geometrical figure?

a. Sphere	b. Cube	c. Triangle	d. Cylinder

15.	What is the sum of the interior angles of a quadrilateral?

a. 180°	b. 270°	c. 360°	d. 540°

16.	Which of these figures has four equal sides and four right angles?

a. Rectangle	b. Trapezium	c. Square	d. Rhombus

17.	What is the primary purpose of orthographic projection?

a. To create a realistic 3D image of an object	b. To represent a 3D object accurately in 2D views

c. To add artistic effects to a drawing	d. To simplify an object into a single view

18.	Which field commonly uses orthographic projection?\

a. Medicine		b. Engineering and architecture

c. Music production		d. Cooking

Answer Keys

| 1. | c | 2. | b | 3. | b | 4. | d |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5. | a | 6. | c | 7. | b | 8. | c |
| 9. | b | 10. | d | 11. | b | 12. | a |
| 13. | b | 14. | c | 15. | c | 16. | c |
| 17. | b | 18. | b |  |  |  |  |

MCQs LU# 2

1. 	Which command convert discrete 2D objects in polyline?

a. Union	b. Subtract	c. Join	d. Polyline

2.	When drawing in 2D, what axis do you NOT work with?

a. X	b. Y	c. Z	d. WCS

3.	To print the selected project, you will choose to regulate what to plot?

a. Display	b. Extents	c. Limits	d. Window

4.	What is the difference between the Scale command from the command Zoom?

a. Scale for single object, while the Zoom whole plan

b. No difference

c. A Scale can grow / shrink a shape up 10 times, while the Zoom has no limits

d. A Scale changes the size of objects, while the Zoom changes the visibility of the project

5.	What is the function of the LINE command in AutoCAD?

a. To draw a polygon.	b. To draw a straight line between two points

c. To draw an arc		d. To create a 3D model

6.	What does the OFFSET command do?

a. Moves an object from one place to another

b. Creates a parallel copy of an object at a specified distance

c. Rotates an object

d. Extends an object

7.	Which command is used to trim excess parts of an object?

a. Extend	b. Trim	c. Erase	d. Move

8.	How can you draw a circle with a specific radius?

a. Use the CIRCLE command and specify the center and radius

b. Use the ARC command

c. Use the ELLIPSE command

d. Use the RECTANGLE command

9.	What is the purpose of using Layers in AutoCAD?

a. To divide the drawing into sections for better organization

b. To make a 3D model

c. To apply colors only

d. To increase the file size

10.	Which command is used to add dimensions in AutoCAD?

a. Measure	b. Dim	c. Length	d. Scale

11.	What is the function of the TEXT command?

a. To add textual information in a drawing	b. To create a table

c. To label blocks		d. To delete text

12.	Which command is used to make a copy of an object in AutoCAD?

a. Copy	b. Mirror	c. Duplicate	d. Move

13.	What is the minimum allowable number of layers in a drawing?

a. 0	b. 5	c. 1	d. 2

14.	How many SNAP points does an object have?

a. 1	b. 4	c. 5	d. Depend on object

15.	Which one is AutoCAD Extension.?

a. Dwg	b. Dxf	c. Dot	d. dws

Answer Keys

| 1. | c | 2. | c | 3. | d | 4. | d |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5. | b | 6. | b | 7. | b | 8. | a |
| 9. | a | 10. | b | 11. | a | 12. | a |
| 13. | c | 14. | d | 15. | a |  |  |