|  | Perform CNC Lathe Operations |
| --- | --- |

Learning Outcomes:

After Completion of this learning module, trainee will be able to:

- Understand and Operate the control panel

- Calculate coordinates of job

- Mount and align the job (Tool and work offset)

- Prepare Manual Program

- Prepare canned cycle program

- Perform Simulation (e.g. Swansoft/ CIMCO etc.) Simulators

- Feed the Program in machine for production

Introduction to CNC Lathe

Typical lathe machine controlled by a CNC system is known as CNC turning center or CNC lathe.

Remember

Use proper PPEs before Operating CNC Machine

Parts of CNC Lathe Machine

Head stock

Head stock of CNC machine consist on the main motor and Gear mechanism which drive the spindle of machine.

Bed

Turret and tail stock travel over the CNC bed. CNC bed is especially hardened so any kind of machining can’t affect it.

Chuck

Chuck of CNC Turning, clamp the work pieces which are to be machined. There are three types of chucks.

- Three Jaw Chuck

- Four Jaw Chuck

- Six Jaw Chuck

Tail stock

- Tail stock is used to hold the work piece center to center.

- It’s also provide an extra griping force on the other end of long work piece.

- Live center and dead center are also clamp in quill of tail stock.

- Different sizes of Drill and Reamers are also clamp in this quill.

Remember

Never Forget to remove chuck key before operating machine

Foot switches and pedals

A CNC machine has two pedal switches, through these pedals CNC machinists open and close the machine chuck to grip the components the same way tail stock quill is taken to forward position and reversed through these pedals.

Tool turret

(Slant bed tool turret)			(Front mount tool turret)

The cutting tools are mounted on the tool turret in CNC machine. We chose the suitable tools for desire operation from tool turret

Types

There are two main types of CNC turning center.

- Horizontal Turning Center

- Vertical Turning Center

- Horizontal turning center will be covered in this course.

Horizontal Turning Center

“Turning center with horizontal bed is called horizontal turning center. There are two types of horizontal turning center.

- Front mounted turret or flat bed

- Rear mounted turret or slant bed

Safety precautions

- When working on the lathe, always stand straight.

- Never lean on the lathe or reach over the chuck when it is in operation.

- If the workpiece vibrates, this is an indication that the speed is too fast. Reduce the speed until the workpiece rotates steadily, without vibration.

- If working with wood, beginners should avoid material that has splits, knots, or heavy bark.

- Before sanding or polishing, remove the tool rest.

- Metal chips can be razor-sharp and should not be removed with hands. Instead, use pliers for long chips and a brush or vacuum for short chips.

- Compressed air should not be used to remove chips from the lathe, since it can blow the chips back into the users face.

- The lathe motor should always be turned off before adjusting the tool rest or any other parts, or before cleaning, lubricating, or measuring. Likewise, turn off the power and wait until the lathe comes to a full stop when mounting or removing accessories.

- Never leave a running lathe unattended. Power it off and wait until it has come to a full stop before leaving the work area.

- Users should avoid operating the lathe when tired or under the influence of alcohol or drugs.

|  | Understand and Operate the control panel |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Use emergency stop button to stop the machine.

- Perform machine zero positioning.

- Move the turret in X and Z-axis by using Jog/Manual/MPG keys.

- Check position of axis by using Position key.

- Use program editing keys like Program, Edit, MDI, Insert, Alter, EOB, Cancel and Delete.

- Hold the Feed by Feed hold/ Cycle stop keys.

- Change Feed and Spindle Speed using Override Keys.

Interface of CNC Control Panel

USB port

Orientation keys

Percentage keys

Display keys

Axis direction keys

Operational keys

Feed ride keys

Function keys

Indicator bar

Address and numeric keys

Curser keys

Display screen

Editing keys

Display screen

The main screen of the machine that display the important information of the operation. Any active program can be viewed including current tool position, various offsets, parameters even a graphic representation of a tool path.

Address and numeric keys.	‘

Address keys consists on all the alpha alphabetic these keys allows an operator to enter the alphabetic in a program. Example X, Z, Y, P, Q etc.

Numeric keys allow an operator to enter numbers, a decimal point, minus (-) or plus (+) sign equal (=) in the program.

Page up and down keys

The up and down arrow keys allow an operator to rapidly move from one page to another at a time.

Editing keys

There are some editing keys on the control panel of the machine that’s we use while typing or editing of a program. These are following keys.

Alter:

If we require to change or replace a command or code in a program then we use the alter key.

Insert:

If we require to insert a block or a code in existing block then we use the insert key. This key will replace the existing code or block with new code or block.

Input:

Some time we wants to addition in a saved or existing block or want to enter some offset value then we use input key.

End of block: we commonly use this key to complete and starting a new block in a program.

Curser keys

These keys are commonly use to move the cursor in a specific direction like Up and Down, Right and Left.

Reset key

This key is located on the top left corner of the control panel that stops all machine motion and places the program cursor at the top of the current program.

Display Keyes Position:

This key displays the position screen that shows axis locations. When we press this button the display screen

- show us axis position in four functions.

- Relative or incremental

- Absolute position

- All positions (Relative ,absolute and machine position)

- Position and program ( in which positions will be shown along the program)

Program:

- When we press the program button it will show us four functions of program mode.

- It will displays the program screen and blocks of the current part program.

- We can also edit a program by using Edit and MDI mode.

- We can see all the save programs in machine memory.

- We can receive the program in control panel from USB. And also can send the programs from control panel to USB.

Offset:

A function key located on the MDI keypad that displays tool offsets and settings. There are two types of offsets.

- Geometric off sets

- Wear offsets

Remember

Machine Zero Positioning sets a reference point before machining.

Graph: we can watch the graphically movement of the tool by using this button.

Alarm: Alarm button notified us that what’s kind of alarm is occurred and what’s its reason

Axis direction keys Axis direction keys are used for selection of an axis. It can also use for tool movement manually in some axis.

Function keys

F1 F2 F3 F4 F5 F6 are function keys we can use them for different purposes. These keys are also use for selection of different modes.

Operational keys

Allow changes in a program stored in CNC memory. This mode also use for type a new program on CNC panel.

Auto: allows automatic operations.

MDI: That’s manual data input. Also use for type a program but this program will volatile.

Machine zero: Use to send the tool to its home position.

MPG: Manual pulse generate use for manually control the movement of axis.

Manual: Allows manual operations during setup.

Program zero:

MPG trial cut: we can take a trial cut for testing of a program with the help of MPG.

Single block

We can run the separately one by one each block of a program with this button. After the execution of a block the axis will stop and we will again press the cycle start button for run the next block.

Skip

For ignore a block or stop the execution of a block we put a / in start of block and turn on the skip button in start of a program. We will again press the cycle start button for execution of remaining program.

Machine lock

The axis of machine will lock. When we on the machine lock button the axis of machine will never move in any direction

M S T lock

When this button will ON, the M codes and spindle will never run. Butt axis will remain in moving position.

DRY

Enables program testing at fast feed rate without clamping a work piece.

Optional stop

This button use for temporarily stop the execution of a program. M01 code putt in start of the block and keep this button on.

Practical Activity: Use all keys one by one and operate the control panel.

Machinery/Tool and equipment:

CNC Lathe Machine with MPG Controller

Learning Place:

Class Room and workshop

Keep in mind the safety precautions before operating the CNC Lathe Machine.

Safety Precautions before Operating the CNC Lathe Control Panel

- Always wear safety glasses, gloves, and appropriate clothing.

- Check for loose parts, damaged wires, or leaks.

- Ensure the emergency stop button is functional.

- Remove unnecessary tools or objects near the machine.

- Ensure the machine starts from the correct reference point.

- Maintain proper levels before operation.

- Confirm correct program and toolpath before running.

- Never leave the control panel unattended while the machine is running.

- Always operate according to standard procedures for safety.

|  | Calculate coordinates of job |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Mark points on part drawing to identify position of job.

- Calculate coordinates of turning job as per drawing.

Coordinate Calculations

On a manual machine, the operator knows and sees where to take the cutting tool to perform the cutting operations. But a CNC machine does not have any eyes like us human to see and control the movement of tool.

So how does it know exactly where to take the tool to perform machining? The answer lies in finding the

coordinates of the position of different points where the tool has to be moved with respect to a reference point.

In turning, finding coordinates is a little different than milling

Finding Coordinates in CNC Lathe Machine

- First, axis are different than milling(X and Z axis).

- Second, the depth of cut taken in X axis results in reduction of diameter of workpiece by two times. For example if turning operation is performed taking 1mm depth of cut in X-axis, the diameter will be reduced by 2mm. So whenever finding X-coordinate on turning, we consider it double than the distance from the reference point.

- This does not apply to Z coordinates because 1mm depth of cut in Z-axis will reduce the length by 1mm only. So the distance from the reference point taken in Z-axis will be considered as such to be its coordinate.

Examples of Coordinate Calculation

In the figure shown, with reference at O, the coordinates of O and A-F points in absolute system are:

Remember

Use incremental or absolute coordinates as convenience. .

O (0, 0)

A (12, 0)

B (12, -9)

C (28, -9)

D (28, -18)

E (44, -18)

F (44, -53)

In the figure shown, with reference at O, find the coordinates of O and A-F points in absolute system:

Exercise: 2

In the figure shown, with reference at O, find the coordinates of O and A-J points in absolute system.

Practical Activity: Calculate the Coordinate of given drawing

In the figure shown, with reference at O, find the coordinates of O and A-J points in absolute system

Tool and equipment:

Vernier Caliper

Micrometer

Paper and pen

Learning Place:

Class Room

|  | Mount and align the job (Tool and work offset) |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Clamp the job in the chuck of CNC Lathe Machine.

- Align and straighten the work piece using dial indicator.

- Set and align tool properly in the Chuck

- Perform work reference setting for different tools. (Tool offsetting)

Setup on CNC Lathe

Proper setup is required when working on a CNC lathe.

Since this is a risk of tooling and machine damage, a proper setup is the best way to successfully turn and make parts for production. Following a thorough, proper setup procedure, you can reduce the risk of tool damage and wasted raw material.

Adjust chuck jaws to accommodate raw material.

Machine and workpiece Zero Point

An unchangeable reference point established by the machine manufacturer. Proceeding from this point the entire machine is measured.

Work piece zero point

Starting point for the dimensions in the part program. Can be freely established by the programmer and moved as desired within the part program.

The part program is based on work piece zero point. In order to set part zero:

- Manually move the tool (using Jog, Manual or MPG mode) to touch the face of the work piece from side.

- Note the Z-coordinate from the display screen.

- Move the tool to the top surface of the work piece and touch it.

- Note the X-coordinate and add the value of diameter of the work piece in it.

Remember

Machine Zero and Workpiece zero are not same.

When writing program, use these coordinates with G50 to set the work reference (explained later).

Practical Activity: Mount and align the given job on machine.

Tool and Equipment Required :

- 3-Jaw or 4-Jaw Chuck

- Raw Material

- Tool Turret

- Dial Indicator

Remember

Use a dial indicator to align the job.

Vernier Caliper

- Micrometer

Learning Place:

Class Room and workshop

|  | Prepare Manual Program |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Demonstrate functions of G-codes and M-codes for CNC Lathe machine.

- Develop facing program as per drawing.

- Develop turning/step turning program as per drawing.

- Develop Threading program as per drawing.

- Develop drilling/boring program as per drawing.Change Feed and Spindle Speed using Override Keys.

G Codes are known as Geometric Codes. These are commonly used for the positioning of cutting tools.

Interpolation Functions

| G code | Description | Purpose |
| --- | --- | --- |
| G00 | Rapid positioning | The G00 execute a non-cutting movement at a rapid feed rate to a specific position in the working range. The block for this purpose will written as: G00 X20 Z20; The tool will rapidly travel straight to the position (20,20) |
| G01 | Linear interpolation | The G01 executes a cutting movement in straight line at a set feed rate. The block for this purpose will written as :G01 X20 Z60; The tool will linearly travel in straight line to the position (20,60) |
| G02 | Circular interpolation clock-wise. | The G02 executes a clockwise circular cutting movement, at a set feed rate. The block will written as :G02 X30 Z65 R5; The tool will develop a clock-wise radius on the work piece. |
| G03 | Circular interpolation anti clock-wise | The G03 code execute an anti-clock-wise cutting movement, at a set feed rate. This code will act the same way like G02. The block will written as :G03 X30 Z65 R5: |

Dwell Function

| G04 | Dwell | The G04 code is used to enter the set time delay into the program. Example block: G04P5000 At this block, the tool feed will stop for 5 seconds. |
| --- | --- | --- |

Remember

Modal commands remain active in next blocks

Unit Functions

| G20 | Imperial (inches) | The G20 code is used for input the program values in inches. |
| --- | --- | --- |
| G21 | Metric millimeters | The G21 code is used for input the program values in millimeters The unit function should be put at the start of the program. |

Return to Machine Reference

| G28 | Tool return to machine home position | The reference point is a fixed position on the machine to which tool can be moved. A G28 code is used to move the tool automatically to the reference point. The block will written as G28 U0.0 W0.0: |
| --- | --- | --- |

Setting work reference G50

| G50 | Work piece reference | The G50 code is use to set the work piece reference. The cutting tool follows this reference point to perform operations. The block will written as: example G50 X 395 Z 260; The machine will set the reference on this point of work piece. |
| --- | --- | --- |

Canned Cycles

| G70 | Finishing cut | The G70 code used for input the value of finishing profile for finishing cut. The block will written as : G70 P10 Q20; P10 And Q20 are the first and last block numbers of canned cycle. |
| --- | --- | --- |
| G71 | Rough turning canned cycle | G71 use for rough turning canned cycle in which we perform tapers curves and straight cutting. Like turning, tapper turning and contour turning. It consist on two blocks explained in 7.9.4 |
| G72 | Facing canned cycle | G72 code is use for simple facing canned cycle. This code also consists of two blocks explained in 7.9.5 |
| G73 | Pattern repeat cycle | A canned cycle simplify the program by replacing complex machining sequence, programed by several blocks of information with just one or two blocks. |
| G74 | Drilling canned cycle | G74 code used for perform drilling in peck drilling explained in 7.9.8 |
| G75 | Grooving canned cycle | G75 code use for perform grooving in pecks explained in 7.9.9 |
| G76 | Threading canned cycle | G76 is threading canned cycle. In this code we convey the instructions in two blocks for operation explained in 7.9.11 |

Straight Cutting Cycles

| G90 | Straight turning cycle | G90 code is specifically use for straight turning. |
| --- | --- | --- |
| G94 | Straight facing cycle | G94 code is specifically use for straight facing. |

Straight Cutting Cycles

| G90 | Straight turning cycle | G90 code is specifically use for straight turning. |
| --- | --- | --- |
| G94 | Straight facing cycle | G94 code is specifically use for straight facing. |

Cutting Speed Setting

| G96 | Constant cutting speed on | G96 is used for setting constant cutting speed - Spindle speed increases as the work piece diameter decreases thus keeping cutting speed constant |
| --- | --- | --- |
| G97 | Constant cutting speed off or variable cutting speed on | G97 is used for setting variable cutting speed - Spindle speed remains constant as the workpiece diameter decreases thus resulting in change in cutting speed |

Feed Unit Setting

| G98 | Feed rate per minute | This code is used with G20 or G21: If G20G98; feed in inches/min If G21G98; feed in mm/min |
| --- | --- | --- |
| G99 | Feed rate per revolution | This code is used with G20 or G21: If G20G98; feed in inches/rev If G21G98; feed in mm/rev |

Common M Codes

Compulsory Stop (M00)

| M00 | Compulsory stop | When the machine controller read the code G00 within a block it pause the program. The cycle start key must be pressed to allow the program to continue. |
| --- | --- | --- |

Optional stop (M01)

| M01 | Optional stop | This code will also halt the program if optional (STOP) input key is activated. |
| --- | --- | --- |

Program end (M02)

| M02 | Program reset | This code indicates the end of a program and perform a general reset function on the machine controller. |
| --- | --- | --- |

Spindle functions (M03-M05)

| M03 | Spindle start forward | The clock wise direction of the spindle is determined by viewed from the back of the machine headstock, along the x axis towards the tail stock. |
| --- | --- | --- |
| M04 | Spindle reverse (anti clock wise) | M04 code use to rotate the spindle anti clock-wise. |
| M05 | Spindle stop | M05 code use to stop the spindle rotating. We use it usually at the end of the program. |

Coolant function (M07-M09)

| M07 | Mist coolant ON |  |
| --- | --- | --- |
| M08 | Coolant ON | This code switches the coolant ON. |
| M09 | Coolant OFF | This code switches the coolant OFF. |

Program End and Reset

| M30 | Program end and reset | This program stop the program running ,i.e. It signals the end of the program |
| --- | --- | --- |

Program Structure

Character:

A single alphanumeric character value or the “+” and “-“ sign.

Word: a series of characters defining a single function such as, “X” displacement or an “F” feedrate. A letter is the first character of a word for each of the different functions.

Block:

Each line of a CNC program is called a block.

Signs:

If the value following an address letter command such as A, B, C, I, J, K, R, U, V, W, X, Y, Z, is positive, the plus sign need not be programmed in. If it has a minus (-) value it must be programmed in with a minus (-).

Leading zero’s:

If the digits proceeding a significant number are zero, they need not be programmed. EXAMPLE: G0 for G00 and M1 for M01, Trailing zeros must be programmed: M30 not M3, G70 not G7.

Modal commands:

Codes that are active for more than the line in which they are issued are called MODAL commands. Rapid traverse, feed rate moves, and canned cycles are all examples of modal commands. A NONMODAL command which, once called, are effective only in the calling block, and are then immediately forgotten by the control.

Preparatory functions:

“G” codes use the information contained on the line to make the machine tool do specific operations.

Miscellaneous functions:

“M” codes are effective or cause an action to occur at the end of the block and only one M code is allowed in each block.

Sequence numbers:

Sequence numbers are codes N1 through N9999 that are only used to locate and identify a block or line and its relative position within a CNC program

A CNC program contain following parts:

| Block No. | Program | Description |
| --- | --- | --- |
|  | O1234 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0W0.0; | Return tool to machine home position |
| N20 | T0101; | Tool change |
| N25 | G50X_ Z S2000; | Work reference setting, maximum spindle speed |
| N30 | G96M03S1500; | Spindle start clockwise, constant cutting speed |
| N35 | G00X52.0Z2.0; | Bring the tool to safe point |
| N40 | Co-ordinates with G,M codes for operations | Point to Point Cutting (Tool path) |
| N45 |  |  |
| N50 |  |  |
| N55 | G00X52.0Z-18.0; | Rapid return to safe position |
| N60 | G28U0.0; | Return tool to machine home position in x-axis |
| N65 | G28W0.0; | Return tool to machine home position in z-axis |
| N70 | M05; | Spindle stop |
| N75 | M30; | Program end and reset to the start |

Manual Programming

Manual facing programing

A simple facing operation by using a manual program on CNC turning center according to diagram.

Parameters

| Spindle speed | 1400RPM | Feed rate | 0.15F |
| --- | --- | --- | --- |
| Depth of cut | 5mm | Note: The usual depth of cut is 0.5 -1.0 mm |  |

| Block No. | Program | Description |
| --- | --- | --- |
|  | O1234 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0W0.0; | Return tool to machine home position |
| N20 | T0101; | Tool change |
| N25 | G50X_ Z ; | Work reference setting, maximum spindle speed |
| N30 | G97M03S1400; | Spindle start clockwise, variable cutting speed |
| N35 | G00X52.0Z2.0; | Bring the tool to safe point |
| N40 | G00X52.0Z-5.0; | Depth of cut in x-axis |
| N45 | G01X-1.0Z-5F0.15; | Cutting with feed in z-axis |
| N50 | G00X-1.0Z-3.0; | Rapid retract |
| N55 | G00X52.0Z-3.0; | Rapid return to safe position |
| N60 | G00X52.0Z-10.0; | Depth of cut in x-axis |
| N65 | G01X-1.0Z-10.0; | Cutting with feed in z-axis |
| N70 | G00X-1.0Z-8.0; | Rapid retract |
| N75 | G00X52.0Z-8.0; | Rapid return to safe position |
| N80 | G00X52.0Z-15.0; | Depth of cut in x-axis |
| N85 | G01X-1.0Z-15.0; | Cutting with feed in z-axis |
| N90 | G00X-1.0Z-13.0; | Rapid retract |
| N95 | G00X52.0Z-13.0; | Rapid return to safe position |
| N100 | G00X52.0Z-20.0; | Depth of cut in x-axis |
| N105 | G01X-1.0Z-20.0; | Cutting with feed in z-axis |
| N110 | G00X-1.0Z-18.0; | Rapid retract |
| N115 | G00X52.0Z-18.0; | Rapid return to safe position |
| N120 | G28U0.0; | Return tool to machine home position in x-axis |
| N125 | G28W0.0; | Return tool to machine home position in z-axis |
| N130 | M05; | Spindle stop |
| N135 | M30; | Program end and reset to the start |

Manual step turning program

Perform a turning operation by using a manual program on CNC lathe according to diagram.

Parameters

| Spindle speed | 1400RPM | Feed rate | 0.15F |
| --- | --- | --- | --- |
| Depth of cut | 6mm | Note: The usual depth of cut is 0.5 -1.0 mm |  |

| Block No. | Program | Description |
| --- | --- | --- |
| N5 | O1235 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0W0.0; | Return tool to machine home position |
| N20 | T0101; | Tool change |
| N25 | G50X Z S1400; | Work reference setting, maximum spindle speed |
| N30 | G96M03S150; | Spindle start clockwise,constant cutting speed |
| N35 | G00X52.0Z2.0; | Bring the tool to safe point |
| N40 | G00X44.0Z2.0; | Depth of cut in x-axis |
| N45 | G01X44.0Z-30.0F0.15; | Cutting with feed in z-axis |
| N50 | G00X46.0Z-29.0; | 1mm rapid retract at 45 degree |
| N55 | G00X46.0Z2.0; | Rapid return to safe position |
| N60 | G00X38.0Z2.0; | Depth of cut in x-axis |
| N65 | G01X38.0Z-30.0; | Cutting with feed in z-axis |
| N70 | G00X40.0Z-29.0; | 1mm rapid retract at 45 degree |
| N75 | G00X40.0Z2.0; | Rapid return to safe position |
| N80 | G00X32.0Z2.0; | Depth of cut in x-axis |
| N85 | G01X32.0Z-30.0; | Cutting with feed in z-axis |
| N90 | G00X34.0Z-29.0; | 1mm rapid retract at 45 degree |
| N95 | G00X34.0Z2.0; | Rapid return to safe position |
| N100 | G00X30.0Z2.0; | Depth of cut in x-axis |
| N105 | G01X30.0Z-30.0; | Cutting with feed in z-axis |
| N110 | G00X32.0Z-29.0; | 1mm rapid retract at 45 degree |
| N115 | G00X32.0Z2.0; | Rapid return to safe position |
| N120 | G00X24.0Z2.0; | Depth of cut in x-axis |
| N125 | G01X24.0Z-20.0; | Cutting with feed in z-axis |
| N130 | G00X26.0Z-19.0; | 1mm rapid retract at 45 degree |
| N135 | G00X26.0Z2.0; | Rapid return to safe position |
| N140 | G00X20.0Z2.0; | Depth of cut in x-axis |
| N145 | G01X20.0Z-20.0; | Cutting with feed in z-axis |
| N150 | G00X22.0Z-19.0; | 1mm rapid retract at 45 degree |
| N155 | G00X22.0Z2.0; | Rapid return to safe position |
| N160 | G28U0.0; | Return tool to machine home position in x-axis |
| N165 | G28W0.0; | Return tool to machine home position in z-axis |
| N170 | M05; | Spindle stop |
| N175 | M30; | Program end and reset to the start |

Manual grooving program

Perform a grooving operation by using manual program on CNC turning center.

Parameters

| Spindle speed | 300RPM | Feed rate | 0.10F |
| --- | --- | --- | --- |
| Depth of cut | 3mm | Tip/insert width | 4mm |

| Sir No. | Program | Description |
| --- | --- | --- |
|  | O1236 | Program name |
| N5 | G21G99; | Units |
| N10 | G28U0.0W0.0; | Return tool to machine home position |
| N15 | T0101; | Tool change |
| N20 | G50X Z ; | Reference setting of work piece, maximum spindle speed |
| N25 | G97M03S300; | Spindle start clockwise, variable cutting speed |
| N30 | G00X52.0Z-22.0; | Bring the tool to safe point |
| N35 | G01X44.0Z-22.0F0.10; | Cutting with feed in x-axis |
| N40 | G00X46.0Z-22.0; | Rapid retract |
| N45 | G01X40.0Z-22.0; | Cutting with feed in x-axis |
| N50 | G00X52.0Z-22.0; | Rapid retract |
| N55 | G00X52.0Z-25.0; | Rapidly move to next point z axis |
| N60 | G01X44.0Z-25.0; | Cutting with feed in x-axis |
| N65 | G00X46.0Z-25.0; | Rapid retract |
| N70 | G01X40.0Z-25.0; | Cutting with feed in x-axis |
| N75 | G00X52.0Z-25.0; | Rapid retract |
| N80 | G00X52.0Z-39.0; | Rapidly move to next point z axis |
| N85 | G01X44.0Z-39.0; | Cutting with feed in x-axis |
| N90 | G00X46.0Z-39.0; | Rapid retract |
| N95 | G01X39.0Z-39.0; | Cutting with feed in x-axis |
| N100 | G00X52.0Z-39.0; | Rapid retract |
| N105 | G00X52.0Z-42.0; | Rapidly move to next point z axis |
| N110 | G01X44.0Z-42.0; | Cutting with feed in x-axis |
| N115 | G00X46.0Z-42.0; | Rapid retract |
| N120 | G01X39.0Z-42.0; | Cutting with feed in x-axis |
| N125 | G00X52.0Z-42.0; | Rapid retract |
| N130 | G00X52.0Z-43.0; | Rapidly move to next point z axis |
| N135 | G01X44.0Z-43.0; | Cutting with feed in x-axis |
| N140 | G00X46.0Z-43.0; | Rapid retract |
| N145 | G01X39.0Z-43.0; | Cutting with feed in x-axis |
| N150 | G00X52.0Z-43.0; | Rapid return to safe position |
| N155 | G28U0.0; | return tool to machine home position in x-axis |
| N160 | G28W0.0; | return tool to machine home position in z-axis |
| N165 | M05; | Spindle stop |
| N170 | M30; | program end and reset to the start |

Manual drilling program

Perform a drilling operation by using a manual program on CNC turning center.

Parameters

| Spindle speed | 300RPM | Feed rate | 0.10F |
| --- | --- | --- | --- |
| Depth of cut | 5mm | Drill diameter | 10mm |

| Block No. | Program | Description |
| --- | --- | --- |
|  | O1236 | Program Name |
| N5 | G21G99; | Units |
| N10 | G28U0.0W0.0; | Return tool to machine home position |
| N15 | T0101; | Tool change |
| N20 | G50X Z | Reference setting of work piece. |
| N25 | G97M03S300; | Spindle start clockwise, variable cutting speed |
| N30 | G00X0.0Z2.0; | Bring the tool to safe point |
| N35 | G01X0.0Z-5.0F0.10; | Cutting with feed in z-axis |
| N40 | G00X0.0Z-2.0; | Rapid retract |
| N45 | G01X0.0Z-10.0; | Cutting with feed in z-axis |
| N50 | G00X0.0Z-7.0; | Rapid retract |
| N55 | G01X0.0Z-15.0; | Cutting with feed in z-axis |
| N60 | G00X0.0Z-12.0; | Rapid retract |
| N65 | G01X0.0Z-20.0; | Cutting with feed in z-axis |
| N70 | G00X0.0Z-17.0; | Rapid retract |
| N75 | G01X0.0Z-25.0; | Cutting with feed in z-axis |
| N80 | G00X0.0Z-22.0; | Rapid retract |
| N85 | G01X0.0Z-30.0; | Cutting with feed in z-axis |
| N90 | G00X0.0Z-27.0; | Rapid retract |
| N95 | G01X0.0Z-35.0; | Cutting with feed in z-axis |
| N100 | G00X0.0Z-32.0; | Rapid retract |
| N105 | G01X0.0Z-40.0; | Cutting with feed in z-axis |
| N110 | G01X0.0Z2.0; | Rapid retract) |
| N115 | G28U0.0; | Return tool to machine home position in x-axis |
| N120 | G28W0.0; | Return tool to machine home position in z- axis |
| N125 | M05; | Spindle stop |
| N130 | M30; | Program end and reset to the start |

Manual boring program

Perform a boring operation by using a manual program on CNC turning center.

Parameters

| Spindle speed | 1400RPM | Feed rate | 0.15F |
| --- | --- | --- | --- |
| Depth of cut | 2mm |  |  |

| Block No. | Program | Description |
| --- | --- | --- |
| N5 | O1237 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0W0.0; | Return tool to machine home position |
| N20 | T0101; | Tool change |
| N25 | G50X Z S1400; | Work reference setting, maximum spindle speed |
| N30 | G96M03S150; | Spindle start clockwise, constant cutting speed |
| N35 | G00X8.0Z2.0; | Bring the tool to safe point |
| N40 | G00X14.0Z2.0; | Depth of cut in x-axis |
| N45 | G01X14.0Z-20.0; | Cutting with feed in z-axis |
| N50 | G00X12.0Z-19.0; | 1mm rapid retract at 45 degree |
| N55 | G00X12.0Z2.0; | Rapid return to safe position |
| N60 | G00X15.0Z2.0; | Depth of cut in x-axis |
| N65 | G01X15.0Z-20.0; | Cutting with feed in z-axis |
| N70 | G00X13.0Z-19.0; | 1mm rapid retract at 45 degree |
| N75 | G00X13.0Z2.0; | Rapid return to safe position |
| N80 | G00X19.0Z2.0; | Depth of cut in x-axis |
| N85 | G01X19.0Z-10.0; | Cutting with feed in z-axis |
| N90 | G00X17.0Z-9.0; | 1mm rapid retract at 45 degree |
| N95 | G00X17.0Z2.0; | Rapid return to safe position |
| N100 | G00X23.0Z2.0 | Depth of cut in x-axis |
| N105 | G01X23.0Z-10.0; | Cutting with feed in z-axis |
| N110 | G00X21.0Z-9.0; | 1mm rapid retract at 45 degree |
| N115 | G00X21.0Z2.0; | Rapid return to safe position |
| N120 | G00X27.0Z2.0; | Depth of cut in x-axis |
| N125 | G01X27.0Z-10.0; | Cutting with feed in z-axis |
| N130 | G00X25.0Z-9.0; | 1mm rapid retract at 45 degree |
| N135 | G00X25.0Z2.0; | Rapid return to safe position |
| N140 | G00X30.0Z2.0; | Depth of cut in x-axis |
| N145 | G01X30.0Z-10.0; | Cutting with feed in z-axis |
| N150 | G00X28.0Z-9.0; | 1mm rapid retract at 45 degree |
| N155 | G00X28.0Z2.0; | Rapid return to safe position |
| N160 | G28U0.0; | Return tool to machine home position in x-axis |
| N165 | G28W0.0; | Return tool to machine home position in z-axis |
| N170 | M05; | Spindle stop |
| N175 | M30; | Program end and reset to the start |

Practical Activity: Write Manual Program of Given Drawing(s):

| Sr.No. | Step | Tools/ equipment | Descriptions |
| --- | --- | --- | --- |
| 1 | Pre-requisite |  | Ensure that all information required for drawing are available like Blank size, tip radius, tools, required dimensions etc. |
| 2 | Programming |  | Prepare a program as per drawing with the help of G and M codes. |
| 3 | Test programming | Swan soft | Write the Programme in Swan soft and test the Programme. ۔ |
| 4 | Clamping | 3 –jaws chuck | Clamp work piece in 3 jaws chuck of CNC machine and set the reference of work piece. |
| 5 | Step Turning | Single point cutting tool | Feed Program into CNC control panel and perform turning operation. |

Practical Activity 2: Write Manual Progrom of give drawing:

Tool and Equipment :

- Paper and pen

- Part Drawing

Learning Place:

Class Room

| Sr.No. | Step | Tools/equipment | Descriptions |
| --- | --- | --- | --- |
| 1 | Pre-requisite | Grooving Tool 3mm Width And Center Drill | Ensure that all information required for planning are available like laser marking machine |
| 2 | Clamping and center Drill |  | Clamp the workpiece on Chuck and make center drill on both side of the work piece according to drawing |
| 3 | Tailstock support |  | Use tailstock to support the work piece from other sides. |
| 4 | Grooving |  | Use 3mm grooving tool and mount the tool on tool post, set safe distance make manual program and make grooving according to drawing |

|  | Prepare canned cycle program |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Develop Simple Facing Canned Cycle program as per drawing

- Develop Simple Turning Canned Cycle program as per drawing

- Develop Finishing Canned Cycle as per Drawing

- Develop drilling/boring canned cycle as per drawing

- Develop threading canned cycle as per drawing

Introduction to Canned Cycles

Canned cycle is a method of programing in which we provide all the information about operation just in few blocks. A canned cycle is a way of conveniently performing repetitive CNC machine operations. This cycle is more accurate and time saving cycle.

There are different Canned Cycles.

- Canned Cycle for Facing

- Canned cycle for OD and ID Turning

Remember

Canned Cycle simplifies complex task.

Canned Cycle for Grooving

- Canned Cycle for Threading

- Canned Cycle for Peck Drilling

Advantages of canned cycle

- It reduces the number of blocks.

- The storage space occupied by the program is less than the manual program.

- We can develop the program very easily and quickly at the machine.

- We can save the program for repeatability of same instructions.

- There are less probabilities of errors in canned cycle program.

- We can easily detect and solve the errors in a short program.

Different Canned Cycles

Straight Turning Cycle (G90)

G90 code is specifically use for straight turning purposes.

Note:	Before going to further a reminder. Don’t confuse G90 for lathe with G90 for machining center. In turning G 90 is a lathe cycle but G90 is the absolute mode in machining center.

G90 X------Z-------F	;

| Block Elements | Description |
| --- | --- |
| G90 | Straight Turning cycle |
| X | Depth of cut in X-axis |
| Z | Total length of turning |
| F | Cutting feed rate |

Parameters

| Spindle speed | 1500RPM | Feed rate | 0.15F |
| --- | --- | --- | --- |
| Depth of cut | 0.5mm |  |  |

G90 straight cycle:

| Block No. | Program | Description |
| --- | --- | --- |
|  | O1245 | Program Name |
| N5 | G21G99; | Units |
| N10 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N15 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N20 | T0101; | Tool Change |
| N25 | G50X----Z ; | Reference Setting of work piece, |
| N30 | G97M03S1500; | Spindle start clockwise, constant cutting speed |
| N35 | M08; | Coolant on |
| N40 | G00X52.0Z2.0; | Bring the Tool to Safe point |
| N45 | G90X49.0Z-30.0F0.15; |  |
| N50 | X48.0; | Depth of cuts in X-axis (Only X-values are given. G90 cycle will remember the Z-value and safe point value as they would not change. So there will be no repetitions like in manual programming) |
| N55 | X47.0; |  |
| N60 | X46.0; |  |
| N65 | X45.0; |  |
| N70 | X44.0; |  |
| N75 | X43.0; |  |
| N80 | X42.0; |  |
| N85 | X41.0; |  |
| N90 | X40.0 |  |
| N95 | M09; | Coolant off |
| N100 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N105 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N110 | M05; | Spindle stop |
| N115 | M30; | Program end |

Straight Facing Cycle (G94)

G94 X------Z------F	;

| Parameters | Description |
| --- | --- |
| G94 | Facing semi canned cycle |
| X | Cutting feed |
| Z | Depth of cut in z-axis |
| F | Cutting feed rate |

Parameters

| Spindle speed | 1500RPM | Feed rate | 0.15F |
| --- | --- | --- | --- |
| Depth of cut | 1mm |  |  |

Straight Facing Cycle (G94)

| Block No. | Program | Description |
| --- | --- | --- |
|  | O1245 | Program Name |
| N5 | G21G99; | Units |
| N10 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N15 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N20 | T0101; | Tool Change |
| N25 | G50X----Z ; | Reference Setting of work piece, |

| N30 | G97M03S1500; | Spindle start clockwise, constant cutting speed |
| --- | --- | --- |
| N35 | M08; | Coolant on |
| N40 | G00X52.0 Z2.0; | Bring the Tool to Safe point |
| N45 | G94X0.0 Z-1.0 F0.15; |  |
| N50 | Z-2.0; | Depth of cut in Z-axis |
| N55 | Z-3.0; | Depth of cut in Z-axis |
| N60 | Z-4.0; | Depth of cut in Z-axis |
| N65 | Z-5.0; | Depth of cut in Z-axis |
| N70 | Z-6.0; | Depth of cut in Z-axis |
| N75 | Z-7.0; | Depth of cut in Z-axis |
| N80 | Z-8.0; | Depth of cut in Z-axis |
| N85 | Z-9.0; | Depth of cut in Z-axis |
| N90 | Z-10; | Depth of cut in Z-axis |
| N95 | M09; | Coolant off |
| N100 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N105 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N110 | M05; | Spindle stop |
| N115 | M30; | Program end |

Rough Turning Canned Cycle (G71)

G71 code is used for rough turning cycle, rough-material removal from workpiece. It consist on two blocks.

G71 U--------R	;

G71 P---Q-----U------W-------F	;

| Parameters | Description | Parameter | Description |
| --- | --- | --- | --- |
| G71 | Code for Rough turning canned cycle | Q | Last block number of the finishing profile. |
| U | Incremental Depth of cut in X axis | U | Material left for finishing on the X axis. |
| R | Amount of retract from each cut | W | Material left for finishing on the Z axis. |
| P | First block number of the finishing profile. | F | Cutting feed rate |

Parameters

| Spindle speed | 1500RPM | Feed rate |  | 0.15mm/rev |  |
| --- | --- | --- | --- | --- | --- |
| Depth of cut | 0.5mm |  |  |  |  |

| Block No. | Program | Description |
| --- | --- | --- |
| N5 | O1239 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0; | Return tool to machine home position in x-axis |
| N20 | G28W0.0; | Return tool to machine home position in z-axis |
| N25 | T0101; | Tool change |
| N30 | G50X----Z S1500; | Reference setting, maximum spindle speed |
| N35 | G96M03S150; | Spindle start clockwise, constant cutting speed |
| N40 | M08; | Coolant on |
| N45 | G00X52.0Z2.0; | Bring the tool to safe point |
| N50 | G71U0.5R1.0; |  |
| N55 | G71P20Q30U0.1W0.2F0.15; |  |
| N60 | N20G01X20.0Z0.0; | 1st point of finishing profile for turning |
| N65 | G01X20.0Z-20.0; | 2nd point of finishing profile for turning |
| N70 | G01X30.0Z-20.0; | 3rd point of finishing profile |
| N75 | G01X30.0Z-30.0; | 4th point of finishing profile |
| N80 | G01X50.0Z-30.0; | 5th point of finishing profile |
| N81 | N30G01X52.0Z2.0; |  |
| N82 | G70P20Q30F0.1; |  |
| N85 | M09; | Coolant off |
| N90 | G28U0.0; | Return tool to machine home position in x-axis |
| N95 | G28W0.0; | Return tool to machine home position in z-axis |
| N100 | M05; | Spindle stop |
| N105 | M30; | Program end and reset to the start |

Rough Facing Canned Cycle (G72)

G72 Facing Cycle is very useful for performing facing operations.

G72 W--------R	;

G72 P---Q-----U------W-------F	;

| Parameters | Description | Parameters | Description |
| --- | --- | --- | --- |
| G72 | Code for Rough turning canned cycle | Q | Last block number of the finishing profile. |
| W | Incremental Depth of cut in z axis | U | Material left for finishing on the X axis. |
| R | Amount of retract after each cut | W | Material left for finishing on the Z axis. |
| P | First block number of the finishing profile. | F | Cutting feed rate |

Parameters

| Spindle speed | 1500RPM | Feed rate | 0.15mm/rev |
| --- | --- | --- | --- |
| Depth of cut | 0.5mm |  |  |

| Block No. | Program | Description |
| --- | --- | --- |
| N5 | O1238 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0; | Return tool to machine home position in x-axis |
| N20 | G28W0.0; | Return tool to machine home position in z-axis |
| N25 | T0101; | Tool change |
| N30 | G50X----Z---; | Reference setting, maximum spindle speed |
| N35 | G97M03S1500; | Spindle start clockwise with variable cutting speed |
| N40 | M08; | Coolant on |
| N45 | G00X52.0Z2.0; | Bring the tool to safe point |
| N50 | G72W0.5R1.0; |  |
| N55 | G72P55Q70U0.0W0.0F0.15; |  |
| N60 | N55G01X52.0Z-20.0; | 1st point of finishing profile of facing |
| N65 | G01X-1.0Z-20.0; | 2nd point of finishing profile of facing |
| N70 | G01X-1.0Z2.0; | 3rd point of finishing profile of facing |
| N71 | N70X52.0Z2.0; |  |
| N75 | M09; | Coolant off |
| N80 | G28U0.0; | Return tool to machine home position in x-axis |
| N85 | G28W0.0; | Return tool to machine home position in z-axis |
| N90 | M05; | Spindle stop |
| N95 | M30; | Program end and reset to the start |

Canned cycle for contour profile Parameters

| Spindle speed | 1500RPM | Feed rate | 0.15mm/rev |
| --- | --- | --- | --- |
| Depth of cut | 0.5mm |  |  |

| Block No. | Program | Description |
| --- | --- | --- |
|  | O1243 | Program name |
| N5 | G21G99; | Units |
| N10 | G28U0.0; | Return tool to machine home position in x-axis |
| N15 | G28W0.0; | Return tool to machine home position in z-axis |
| N20 | T0101; | Tool change |
| N25 | G50X----Z S2000; | Reference setting of work piece, maximum spindle speed |
| N30 | G96M03S150; | Spindle start clockwise, constant cutting speed |
| N35 | M08; | Coolant on |
| N40 | G00X52.0Z2.0; | Bring the tool to safe point |
| N45 | G71U0.5R1.0; |  |
| N50 | G71P10Q20U0.1W0.05F0.15; |  |
| N55 | N10G01X17.0Z0.0; | 1st point of finishing profile |
| N60 | G01X20.0Z-1.5; | 2nd point of finishing profile for chamfer |
| N65 | G01X20.0Z-20.0; | 3rd point of finishing profile for turning |
| N70 | G03X30.0Z-25.0R5.0; | 4th point of finishing profile c.c wise curve |
| N75 | G02X40.0Z-30.0R5.0; | 5th point of finishing profile for clockwise curve |
| N80 | G01X40.0Z-33.0; | 6th point of finishing profile |
| N85 | G01X48.0Z-38.0; | 7th point of finishing profile for taper turning |
| N90 | G01X48.0Z-80.0; | 8th point of finishing profile |
| N95 | G01X50.0Z-80.0; | 9th point of finishing profile |
| N96 | N20G01X52.0Z2.0 |  |
| N100 | G70P10Q20F0.10; | Final finishing cut |
| N105 | M09; | Coolant off |
| N110 | G28U0.0; | Return tool to machine home position in x-axis |
| N115 | G28W0.0; | Return tool to machine home position in z-axis |
| N120 | M05; | Spindle stop |
| N125 | M30; | Program end and reset to the start |

Pattern Repeat Cycle (G73)

G73 code is used for pattern Repeat Cycle. This cycle mostly used for casting and forged work pieces in industry. Forged work pieces required less material for removing on surface so we use this Pattern repeat cycle.

It consist on two blocks.

G73 U--------W------R	;

G73 P---Q-----U------W-------F	;

| Parameters | Description | Parameter | Description |
| --- | --- | --- | --- |
| G73 | Code for Rough turning canned cycle | Q | Last block number of the finishing profile. |
| U | Amount of material removing in X axis | U | Material left for finishing on the X axis. |
| W | Amount of material removing in Z axis | W | Material left for finishing on the Z axis. |
| R | Number of cuts | F | Cutting feed rate |
| P | First block number of the finishing profile. |  |  |

## Parameters

| Spindle speed | 1500RPM | Feed rate | 0.15mm/rev |
| --- | --- | --- | --- |
| Depth of cut | 0.5mm |  |  |

G73 Pattern Repeat cycle

| Block No. | Program | Description |
| --- | --- | --- |
|  | O1246 | Program Name |
| N5 | G21G99; | Units |
| N10 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N15 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N20 | T0101; | Tool Change |
| N25 | G50X----Z S1500; | Reference Setting of work piece, |
| N30 | G96M03S150; | Spindle start clockwise, constant cutting speed |
| N35 | M08; | Coolant on |
| N40 | G00X52.0 Z2.0; | Bring the Tool to Safe point |
| N45 | G73U2.0W2.0R5.0; | Pattern Repeat cycle |
| N50 | G73P10Q20U0.1W0.2F0.15; | Pattern Repeat cycle |
| N55 | N10G01X20.0Z0.0; | Cutting with feed in X-axis |
| N60 | X20.0Z-20.0; | Cutting with feed in Z-axis |
| N65 | X30.0Z-20.0; | Cutting with feed in X-axis |
| N70 | X30.0Z-30.0; | Cutting with feed in Z-axis |
| N75 | X50.0Z-30.0; | Cutting with feed in X-axis |
| N76 | N20X52.0Z2.0; |  |
| N80 | M09; | Coolant off |
| N85 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N90 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N95 | M05; | Spindle stop |
| N100 | M30; | Program end |

Finishing Canned Cycles (G70)

G70 code is specifically use for finishing cut. We have already use this code in above canned cycles programs i.e. G71 G73 G71 for contour profiles.

Drilling Canned Cycle (G74)

G74 is peck drilling canned cycle.

G74 R	;

G74 X------- Z--------Q---------F	;

| Parameters | Description | Param. | Description |
| --- | --- | --- | --- |
| G74 | Code for peck drilling canned cycle | Z | Total length of drilling |
| R | Amount of retract from each peck. | Q | Pecking value in z axis |
| X | Value of x axis(should be zero) | F | Feed rate |

Parameters

| Spindle speed | 300RPM | Feed rate | 0.10mm/rev |
| --- | --- | --- | --- |
| Peck value | 1000mic. |  |  |

| Block No. | Program | Description |
| --- | --- | --- |
| N5 | O1241 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0; | Return tool to machine home position in x-axis |
| N20 | G28W0.0; | Return tool to machine home position in z-axis |
| N25 | T0101; | Tool change |
| N30 | G50X----Z ; | Reference setting , maximum spindle speed |
| N35 | G97M03S300; | Spindle start clockwise, variable cutting speed |
| N40 | M08; | Coolant on |
| N45 | G00X0.0Z2.0; | Bring the tool to safe point |
| N50 | G74 R2.0; |  |
| N55 | G74X0.0Z-20.0Q1000F0.10; |  |
| N60 | G00X0.0Z5.0; | Bring the tool to safe point |
| N65 | M09; | Coolant off |
| N70 | G28U0.0; | Return tool to machine home position in x-axis |
| N75 | G28W0.0; | Return tool to machine home position in z-axis |
| N80 | M05; | Spindle stop |
| N85 | M30; | Program end and reset to the start |

Grooving canned cycle (G75)

G75 is a peck grooving canned cycle.by using this canned cycle we can perform the grooving operation several time on a work piece. It’s also consist on two blocks.

G75 R	;

G75 X----Z------P-------Q-------F	;

| Parameters | Description | Param. | Description |
| --- | --- | --- | --- |
| G75 | Code for peck grooving canned cycle | Z | Width of grove |
| R | Amount of retract from each peck. | Q | Tool travelling in z axis |
| X | Total depth of groove. | P | Grooving pecks in x axis |
| F | Feed rate |  |  |

Parameters

| Spindle speed | 400RPM | Feed rate | 0.10mm/rev |
| --- | --- | --- | --- |
| Peck value | 1000mic. |  |  |

Grooving canned cycle Program

| Block No. | Program | Description |
| --- | --- | --- |
| N5 | O1240 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0; | Return tool to machine home position in x-axis |
| N20 | G28W0.0; | Return tool to machine home position in z-axis |
| N25 | T0101; | Tool change |
| N30 | G50X----Z ; | Reference setting, maximum spindle speed |
| N35 | G97M03S400; | Spindle start clockwise, variable cutting speed |
| N40 | M08; | Coolant on |
| N45 | G00X52.0Z-22.0; | Bring the tool to safe point |
| N50 | G75R3.0; |  |
| N55 | G75X40.0Z-25P1000Q1000F0.10; |  |
| N60 | G00X52.0Z-25.0; | Bring the tool to safe point |
| N65 | G00X52.0Z-39.0; | Bring the tool to safe point |
| N70 | G75R3.0; |  |
| N75 | G75X39.0Z - 43.0P1000Q1000F0.10; |  |
| N80 | G00X52.0Z-43.0; | Bring the tool to safe point |
| N85 | M09; | Coolant off |
| N90 | G28U0.0; | Return tool to machine home position in x-axis |
| N95 | G28W0.0; | Return tool to machine home position in z-axis |
| N100 | M05; | Spindle stop |
| N105 | M30; | Program end and reset to the start |

Threading Canned Cycles (G92, G76)

In order to understand G92 or G76 cycles, we have to first understand thread terminologies and simple calculations involved for its programs.

Major Diameter: The largest diameter of a screw thread measured at the crest of a male thread and at the root of a female thread.

Minor Diameter: The minor diameter is the lower extreme diameter of the thread. Major diameter minus minor diameter, divided by two, equals the height of the thread. The minor diameter of a nut is it’s inside diameter.

Pitch Diameter: The diameter of the imaginary co-axial cylinder

Root: The flanks of a thread are the straight sides that connect the crest and the root.

Thread Angle: The angle of a threads the angle between the flanks, measured in an axial plane section.

Pitch: The pitch of a thread is the distance, measured parallel to its axis, between corresponding points on adjacent surfaces

There are some important calculations of threading that’s we require in canned cycle for threading.

Height of thread

H =  0.649 P

0.649 Is a standard value for metric V threads P = pitch of thread i.e. 1.5

H = 0.649 (1.5) = 0.9735mm or 973 microns

Diameter of threads

d = D – 2 h d is miner diameter of thread.

D is major diameter of thread

2 h here we put the value of height “H that we have already find on above calculation d = 20 – 2 (0.973) = 18.054mm

d = 18.054 mm

Threading Cycle G92

This canned cycle is also called semi canned cycle. Its cycle contains one main line: G92 X

Z	F	;

| Parameters | Description |
| --- | --- |
| G92 | Threading semi canned cycle |
| X | Major dia |
| Z | Length of thread in z-axis |
| F | Pitch of thread |

| Block No. | Program | Description |
| --- | --- | --- |
| N5 | O1244 | Program Name |
| N10 | G21G99; | Units |
| N15 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N20 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N25 | T0101; | Tool Change |
| N30 | G50X----Z ; | Reference Setting of work piece, |
| N35 | G97M03S250; | Spindle start clockwise, variable cutting speed |
| N40 | M08; | Coolant on |
| N45 | G00X22.0Z2.0; | Bring the Tool to Safe point |
| N50 | G92X19.4Z-25.0F1.5; | Threading Semi canned cycle |
| N55 | X19.000; | Tool Path |
| N60 | X18.800; |  |
| N65 | X18.600; |  |
| N70 | X18.400; |  |
| N75 | X18.200; |  |
| N80 | X18.100; |  |
| N85 | X18.080; |  |
| N90 | X18.060; |  |
| N95 | X18.053; |  |
| N100 | G00X22.0; | Bring the Tool to Safe point |
| N105 | G00Z2.0; | Bring the Tool to Safe point |
| N110 | M09; | Coolant off |
| N115 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N120 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N125 | M05; | Spindle stop |
| N130 | M30; | Program end |

G76 Treading canned cycle

Before applying threading canned cycle G76 we should must clear our concept about threading. To use G76 Threading Canned Cycle, following description should be cleared.

G76 P-----Q------R	;

G76 X----Z------P-------Q-------F	;

| Parameters | Description | Parameters | Description |
| --- | --- | --- | --- |
| G76 | Code for threading canned cycle | X | Minor dia. Of thread |
| P | Number of finishing cut with thread angle | Z | Length of thread |
| Q | Minimum depth of cut during threading | P | Height of thread |
| R | Material left for finishing cut. | Q | Maximum depth of cut |
|  |  | F | Pitch of thread. |

Threading Canned Cycle Program

| Block No. | Program | Description |
| --- | --- | --- |
|  | O1240 | Program Name |
| N5 | G21G99; | Units |
| N10 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N15 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N20 | T0101; | Tool Change |
| N25 | G50X----Z ; | Reference Setting of work piece, Maximum Spindle Speed |
| N30 | G97M03S300; | Spindle start clockwise, variable cutting speed |
| N35 | M08; | Coolant on |
| N40 | G00X22.0Z2.0; | Bring the Tool to Safe point |
| N45 | G76P06006Q200R0.05; | Main blocks of canned cycle |
| N50 | G76X18.053Z-25.0P973Q300F1.5; |  |
| N55 | G00X22.0; | Bring the Tool to Safe point |
| N60 | G00Z2.0; | Bring the Tool to Safe point |
| N65 | M09; | Coolant off |
| N70 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N75 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N80 | M05; | Spindle stop |
| N85 | M30; | Program end |

Boring or internal turning (G71)

G71 is also internal turning cycle is used to perform a boring operation on turning center. It also consist on two blocks.

G71 U--------R	;

G71 P---Q-----U------W-------F	;

| Parameters | Description | Param. | Description |
| --- | --- | --- | --- |
| G71 | Code for Rough turning canned cycle | Q | Last block number of the finishing profile. |
| U | Incremental Depth of cut in X axis | U | Material left for finishing on the X axis. |
| R | Amount of retract from each cut | W | Material left for finishing on the Z axis. |
| P | First block number of the finishing profile. | F | Cutting feed rate |

‘

| Block No. | Program | Description |
| --- | --- | --- |
| N5 | O1242 | Program name |
| N10 | G21G99; | Units |
| N15 | G28U0.0; | Return tool to machine home position in x-axis |
| N20 | G28W0.0; | Return tool to machine home position in z-axis |
| N25 | T0101; | Tool change |
| N30 | G50X----Z S2000; | Reference setting of work piece, maximum spindle speed |
| N35 | G96M03S150; | Spindle start clockwise, constant cutting speed |
| N40 | M08; | Coolant on |
| N45 | G00X8.0Z2.0; | Bring the tool to safe point |
| N50 | G71U0.8R1.0; |  |
| N55 | G71P40Q50U-0.1W0.05F0.10; |  |
| N60 | N40G01X30.0Z0.0; | 1st point of finishing profile |
| N65 | G01X30.0Z-10.0; | 2nd point of finishing profile |
| N70 | G01X15.0Z-10.0; | 3rd point of finishing profile |
| N75 | G01X15.0Z-20.0; | 4th point of finishing profile |
| N80 | G01X10.0Z-20.0; | 5th point of finishing profile |
| N85 | N50G01X10.0Z2.0; | 6th point of finishing profile |
| N90 | G70P40Q50; | final finishing cut |
| N95 | M09; | Coolant off |
| N100 | G28U0.0; | Return tool to machine home position in x-axis |
| N105 | G28W0.0; | Return tool to machine home position in z-axis |
| N110 | M05; | Spindle stop |
| N115 | M30; | Program end and reset to the start |

Exercises of Canned Cycles

|  | O1242 | Program Name |
| --- | --- | --- |
| N5 | G21G99; | Units |
| N10 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N15 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N20 | T0101; | Tool Change |
| N25 | G50X----Z ; | Reference Setting of work piece, Max Spindle Speed |
| N30 | G97M03S2000; | Spindle start clockwise, variable cutting speed |
| N35 | M08; | Coolant on |
| N40 | G00X52.0Z2.0; | Bring the Tool to Safe point |
| N45 | G72W0.5R1.0; | Using facing caned cycle |
| N50 | G72P10Q20U0.0W0.0F0.15; |  |
| N55 | N10G01X52.0Z-20.0; | Cutting with feed in Z-axis |
| N60 | G01X-1.0Z-20.0; | Cutting with feed in X-axis |
| N65 | N20G01X-1.0Z2.0; | Rapid retract |
| N70 | M09; | Coolant off |
| N75 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N80 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N85 | G50X----Z S2000; | Reference Setting of work piece |
| N86 | G96M03S250; |  |
| N90 | G00X52.0Z2.0; | Bring the Tool to Safe point |
| N95 | G71U0.5R1.0; | Rough turning canned cycle |
| N100 | G71P30Q40U0.1W0.05F0.15; |  |
| N105 | N30G01X17.0Z0.0; | Cutting with feed in X-axis |
| N110 | G01X20.0Z-1.5; | Chamfering (1.5x45dgree) |
| N115 | G01X20.0Z-20.0; | Cutting with feed in Z-axis |
| N120 | G03X30.0Z-25.0 R5.0; | anti- Clockwise radius |
| N125 | G02X40.0Z-30.0 R5.0; | Clockwise radius |
| N130 | G01X40.0Z-33.0; | Cutting with feed in Z-axis |
| N135 | G01X48.0Z-38.0; | Taper turning |
| N140 | G01X48.0Z-50.0; | Cutting with feed in Z-axis |
| N145 | G01X50.0Z-50.0; | Cutting with feed in X-axis |
| N146 | N40G01X52.0Z2.0; |  |
| N150 | G70P10Q20F0.10; | Finishing cut |
| N155 | M09; | Coolant off |
| N160 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N165 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N170 | T0202; | Tool Change |
| N175 | G50X----Z ; | Reference Setting of work piece |
| N180 | G97S350; | Spindle constant RPM or variable cutting speed |
| N185 | G00X22.0Z-17.0; | Bring the Tool to Safe point |
| N190 | G75R3.0; | Using Grooving canned cycle |
| N195 | G75X16.0Z- 18.0P1000Q1000F0.10; |  |
| N200 | G00X52.0; | Bring the Tool to Safe point |
| N205 | G00Z-45.0; | Bring the Tool to Safe point |
| N210 | G75R3.0; | Using Grooving canned cycle |
| N215 | G75X43.0Z- 46.0P1000Q1000F0.10; |  |
| N220 | G00X52.0; | Bring the Tool to Safe point |
| N225 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N230 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N235 | T0303; | Tool Change |
| N240 | G50X----Z ; | Reference Setting of work piece |
| N245 | G97S300; | Spindle constant cutting speed |
| N250 | G00X0.0Z2.0; | Bring the Tool to Safe point |
| N255 | G74R4.0; | Using Drilling Canned Cycle |
| N260 | G74X0.0Z-30Q1000F0.10; |  |
| N265 | G00Z2.0; | Bring the Tool to Safe point |
| N270 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N275 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N280 | T0404; | Tool Change |
| N285 | G50X----Z ; | Reference Setting of work piece |
| N290 | G97S300; | Spindle Variable cutting speed |
| N295 | G00X22.0Z2.0; | Bring the Tool to Safe point |
| N300 | G76P060060Q200R0.05; | Using Threading Canned Cycle |
| N305 | G76X18.053Z- 14.0P973Q300F1.5; |  |
| N310 | G00X22.0; | Bring the Tool to Safe point |
| N315 | G28U0.0; | Return Tool Machine Home position in x-axis |
| N320 | G28W0.0; | Return Tool Machine Home position in z-axis |
| N325 | M05; | Spindle stop |
| N330 | M30; | Program end and reset |

Practical Activity:

Write canned cycle program of given drawing.

Material Required:

- Paper and pen

- Part drawing

Learning Place:

Class Room

| Sr.No. | Step | Tools/equipment | Descriptions |
| --- | --- | --- | --- |
| 1 | Pre-requisite |  | Ensure that all information required for planning are available like blank size, tip, tools, etc. |
| 2 | Programming |  | Prepare a program with the help of G and M codes. |
| 3 | Test programming | Swan Soft | Write the Programme in Swan soft and test it. |
| 4 | Clamping | 3 –jaws chuck | Clamp work piece in 3 jaws chuck of CNC machine and set the reference of work piece. |
| 5 | Circular interpolation & chamfering | Single point cutting tool | Feed program into CNC control panel and perform circular interpolation and chamfering carefully. |

|  | Perform Simulation (e.g. Swansoft/ CIMCO etc.) Simulators |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Set job and tool in simulation software as per drawing.

- Insert part program into simulation software.

- Run simulation of program to identify and correct potential errors in part program. (if any)

- Obtain practical experience in operating virtual CNC machines, which prepares learners for real-world machine handling.

Introduction to CNC Simulator

SSCNC (SwanSoft Computer Numerical Control) is a CNC software by which students can learn and understand CNC programming and operations in short time and safely.

Display of CNC Simulator

Students can program by hand or import CAM NC program then simulate in SSCNC. By using the software in PC, students can lean operation of all kinds of CNC lathe, NC milling and machining center in short time.

Remember

Always test your program on simulation before actual machining

It consists of

- FANUC,

- GSK

- Simens

- Mitsubishi

- HEIDENHAIN

and various other CNC system to be worked on.

Practical Activity: Perform simulation of program as per given drawing

Material Required:

- Computer

- Simulator Software

- Part Drawing

Learning Place:

Computer Lab

| Sr.No. | Step | Tools/ equipment | Descriptions |
| --- | --- | --- | --- |
| 1 | Pre-requisite |  | Ensure that all information required for drawing are available like Blank size, tip radius, tools, required dimensions etc. |
| 2 | Programming |  | Prepare a program as per drawing with the help of G and M codes. |
| 3 | Test programming | Swan soft | Write the Programme in Swan soft and test the Programme. |
| 4 | Clamping | 3 –jaws chuck | Clamp work piece in 3 jaws chuck of CNC machine and set the reference of work piece. |
| 5 | Step Turning | Single point cutting tool | Feed Program into CNC control panel and perform turning operation. |

|  | Feed the Program in machine for production |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Use simple G-Codes and M-codes in MDI mode to check working of chuck, tool and coolant.

- Input program into CNC machine using PROGRAM, EDIT, INSERT, ALTER and EOB keys.

- Apply simulation of tool movement (dry run) to check the program for errors. (if any)

- Run it in single block mode.

- Run the program for production.

Input Methods

CNC machines allow program input through different modes:

- MDI Mode (Manual Data Input): Used for entering simple G-code commands manually.

- DNC Mode (Direct Numerical Control): The program is transferred from an external computer.

- Memory Mode: A saved program is retrieved from the machine’s memory.

Steps for inputting a program

- witch the machine to EDIT mode.

- Use the PROGRAM key to open the program window.

- Enter the program manually using the INSERT or ALTER key.

- Use the EOB (End of Block) key to confirm each command line.

- Save the program and switch to MEMORY mode to run it.

Importance of Optional Stop and Compulsory Stop

- Optional Stop (M01): Allows the operator to pause the program execution if required.

- Compulsory Stop (M00): Forces the machine to stop at a specific point until the operator resumes.

Importance of Single Block Mode

- Single block mode executes the program one line at a time, allowing the operator to verify each movement before proceeding.

- Helps in debugging and error prevention.

Dry Run and Program Simulation

- Dry Run: A test execution of the program without engaging the tool or workpiece to detect errors.

- Simulation Mode: Displays the tool path on the CNC screen before actual machining.

Steps for Simulation & Dry Run:

- Load the program into the CNC machine.

- Switch to Dry Run Mode.

- Execute the program without material to check tool movement.

- Make necessary modifications if errors are found.

Properties of different materials and machining finishing strategies

- Material Properties: Different materials (e.g., aluminum, steel, brass) require different cutting speeds, feed rates, and tool types.

- Finishing Strategies:

- Roughing: Removes excess material quickly with a high feed rate.

- Finishing: Uses lower feed rates for smooth surface quality.

- Peck Drilling: Used for deep holes to prevent tool breakage.

Running the program in single block mode

- Switch to Single Block Mode.

- Press Cycle Start to execute one line at a time.

- Verify tool movement after each block.

- Continue until the full program runs successfully.

Practical Activity: Feed and run program on the machine.

Tool and Equipment:

- CNC Lathe Machine

Remember

Always match parameters of simulator with actual machine

Tooling

- Jaw or 4 Jaw

- Raw Material Chuck

- PPEs

Learning Place:

Workshop

SUMMERY

- A CNC lathe machine is a computer-controlled tool used for precise turning and machining operations.

- The main components of a CNC lathe include the chuck, headstock, tailstock, and turret.

- The control panel consists of buttons for machine zero positioning, feed rate adjustment, and emergency stop.

- Operators use manual data input (MDI) mode for entering simple commands and direct numerical control (DNC) for automated operation.

- G-codes control the tool movements, while M-codes manage auxiliary functions such as coolant and spindle operation.

- CNC programming involves coordinate calculations in X and Z axes for precise cutting operations.

- Workpieces are mounted and aligned in the chuck using dial indicators for accuracy.

- Tool offsets are set to define the cutting tool’s position relative to the workpiece.

- Canned cycles like G71 and G72 automate rough and finish turning operations.

- Simulations using software like Swansoft and CIMCO help identify programming errors before execution.

- Programs are fed into the machine via memory mode or USB, and execution can be tested in single block mode.

- Safety precautions include wearing protective gear, keeping the work area clean, and never leaving the machine unattended.

- Dry run mode allows operators to verify tool paths without engaging the material.

- The tailstock supports long workpieces, preventing deflection during machining.

- The home position is the reference point where all machining operations start and end.

FAQs:

- What is a CNC lathe machine?

Ans: 	A CNC lathe is a computer-controlled machine used for precision turning operations.

- What is the purpose of the headstock in a CNC lathe?

Ans: 	It houses the spindle and motor, which drive the workpiece.

- How does the turret function in a CNC lathe?

Ans: 	The turret holds multiple tools and rotates to change tools during machining.

- What is the difference between G-codes and M-codes?

Ans: 	G-codes control tool movement, while M-codes control auxiliary functions like coolant and spindle.

- How does G71 differ from G72?

Ans: 	G71 is used for rough turning, while G72 is for rough facing.

- What does the G00 code do?

Ans: 	It moves the tool at rapid speed without cutting.

- What are the advantages of using canned cycles?

Ans: 	They simplify programming and reduce errors.

- What is single block mode in CNC?

Ans: 	It executes the program one line at a time for verification.

- How is tool offset set in a CNC lathe?

Ans: 	By setting reference points for tool positioning.

- What is the purpose of the emergency stop button?

Ans: 	It immediately stops all machine functions for safety.

- What is dry run mode and why is it important?

Ans: 	It simulates the toolpath without cutting to check for errors.

- What is the function of M00 and M01?

Ans: 	M00 is a compulsory stop, while M01 is an optional stop.

- How does the feed rate affect machining?

Ans: 	It determines the cutting speed and surface finish.

- What does G50 do in CNC programming?

Ans: 	It sets the maximum spindle speed limit.

- What is the difference between absolute and incremental coordinates?

Ans: 	Absolute coordinates use a fixed zero point, while incremental coordinates are based on the previous position.

- Why is machine zero positioning necessary?

Ans: 	It ensures the tool starts from a known reference point.

- What does a coolant system do in CNC machining?

Ans: 	It cools the tool and workpiece to prevent overheating.

- What is the purpose of peck drilling in CNC?

Ans: 	It drills deep holes in multiple steps to prevent tool breakage.

- How does a CNC lathe ensure precision?

Ans: 	By using accurate positioning systems and programmed toolpaths.

- What are the safety precautions while operating a CNC lathe?

Ans: 	Wear safety gear, avoid loose clothing, and never leave the machine unattended.

- What is the purpose of G76 in CNC programming?

Ans: 	G76 is used for threading cycles.

- How is spindle speed controlled in CNC lathes?

Ans: 	Spindle speed is set using G96 for constant speed and G97 for variable speed.

- What does the term ‘homing’ mean in CNC machines?

Ans: 	It refers to moving the machine axes to their reference starting position.

- Why is tool wear compensation important?

Ans: 	It adjusts for gradual tool wear, maintaining machining accuracy.

- What is backlash in CNC machines?

Ans: 	It is the small movement loss in mechanical components that can affect precision.

- How can CNC programs be transferred between machines?

Ans: 	Using USB, DNC mode, or network-based transfer systems.

- What is the role of M30 in CNC programs?

Ans: 	It ends the program and resets the machine.

- What does G90 and G91 represent in CNC programming?

Ans: 	G90 is absolute positioning, and G91 is incremental positioning.

- What is the significance of the tool nose radius in CNC turning?

Ans: 	It affects surface finish and cutting accuracy.

- Why is lubrication necessary in CNC machines?

Ans: 	It reduces friction, prevents wear, and enhances machine lifespan.

MCQs:

| Question | Option A | Option B | Option C | Option D |
| --- | --- | --- | --- | --- |
| What is the primary function of a CNC lathe machine? | Drilling | Grinding | Turning | Welding |
| Which component holds the workpiece in a CNC lathe? | Chuck | Tailstock | Turret | Spindle |
| What does the G00 code signify? | Linear interpolation | Rapid positioning | Circular interpolation | Threading cycle |
| Which mode allows manual entry of CNC commands? | DNC Mode | MDI Mode | Auto Mode | Edit Mode |
| What is the purpose of G50 in CNC programming? | Coolant control | Speed limitation | Drilling cycle | Tool offset |
| What does M03 do in CNC machining? | Stops the spindle | Starts the spindle clockwise | Turns on coolant | Ends the program |
| Which axis controls the depth of cut in turning operations? | X-axis | Y-axis | Z-axis | None of the above |
| What is the function of G71? | Threading cycle | Rough turning cycle | Finishing cycle | Facing cycle |
| What is the typical unit for feed rate in CNC lathes? | RPM | mm/min | kW | degrees |
| What does G76 represent in CNC programming? | Drilling cycle | Threading cycle | Facing cycle | Tapping cycle |
| What is the function of G94 in CNC lathe programming? | Facing cycle | Turning cycle | Threading cycle | Drilling cycle |
| Which of the following is used to stop the CNC lathe spindle? | M03 | M04 | M05 | M30 |
| What does G21 do in a CNC program? | Sets units to inches | Sets units to millimeters | Sets feed rate | Sets spindle speed |
| What is the purpose of G28? | Moves tool to home position | Enables threading cycle | Starts spindle | Cancels previous command |
| Which mode is used for direct numerical control of a CNC machine? | MDI Mode | Auto Mode | DNC Mode | Manual Mode |
| What is the function of M08? | Turns coolant ON | Turns coolant OFF | Stops spindle | Ends the program |
| What does G73 code represent in CNC programming? | Rough turning cycle | Pattern repeating cycle | Facing cycle | Peck drilling cycle |
| What does G50 limit in CNC programming? | Feed rate | Spindle speed | Cutting depth | Coolant flow |
| Which component moves the cutting tool along different axes? | Spindle | Turret | Chuck | Bed |
| What does the G96 command do? | Constant cutting speed ON | Constant cutting speed OFF | Tool change | Coolant control |
| What is the function of M30 in CNC programming? | Ends the program and resets | Starts spindle | Moves tool to home position | Engages coolant |
| What is the role of the tailstock in a CNC lathe? | Holds and rotates the workpiece | Supports long workpieces | Changes cutting tools | Moves the turret |
| What does the term ‘Jog Mode’ refer to in CNC? | Rapid positioning | Manual tool movement | Automatic machining | Stopping the machine |
| What is the typical feed rate unit in CNC turning? | Inches per minute | Millimeters per revolution | Rotations per second | Degrees per second |
| What is the function of the ‘Home Position’ in CNC lathe? | Starting point for machining | Workpiece clamping location | Coolant application point | Tool changing point |

Answer Keys

| Question Number | Answer | Question Number | Answer | Question Number | Answer |
| --- | --- | --- | --- | --- | --- |
| 1 | C | 2 | A | 3 | B |
| 4 | B | 5 | B | 6 | B |
| 7 | A | 8 | B | 9 | B |
| 10 | B | 11 | A | 12 | C |
| 13 | B | 14 | A | 15 | C |
| 16 | A | 17 | B | 18 | B |
| 19 | B | 20 | A | 21 | A |
| 22 | B | 23 | B | 24 | B |
| 25 | A | ... | ... | ... | ... |