|  | Perform CNC Milling Operations |
| --- | --- |

Learning Outcomes:

After Completion of this learning module, trainee will be able to:

- Understand and Operate the control panel

- Calculate coordinates of job

- Mount and align the job (Tool and work offset)

- Prepare Manual Program

- Prepare canned cycle program

- Perform Simulation (Swansoft / CIMCO Simulators)

|  | Understand and Operate the control panel |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Use emergency stop button to stop the machine.

- Perform machine zero positioning.

- Move the working table in X, Y and Z-axis by using Jog/Manual/MPG keys.

- Check position of axis by using Position key on MCU (Main control Unit).

- Use program editing keys like Program, Edit, MDI, Insert, Alter, EOB, Cancel and Delete.

- Hold the Feed by Feed hold/ Cycle stop keys.

- Change Feed and Spindle Speed using Override Keys.

Introduction to CNC Machines

A CNC (Computer Numerical Control) machine is a manufacturing tool that uses pre-programmed computer software to control machining operations. These machines automate cutting, drilling, milling, and turning processes with high precision and efficiency, reducing the need for manual intervention.

Advantages of CNC Machine

Remember

CNC stand for Computer Numerical Control CAD/CAM stand for Computer Aided Design and Computer Aided Manufacturing

- Greater operator efficiency

- Reduction of scrap

- Reduced lead time for

- Reduction of human error

- High degree of Complex machining operations

- Lower tooling costs

- Increased productivity

- Reduced parts inventory

- Less inspection required

CNC Application

- Automobile Industries: Manufacturing engine parts, gears, and chassis components

- Aerospace Industries : Precision parts for aircraft and spacecraft

- Medical Industries : Production of surgical instruments and implants

- Woodworking & Furniture : Creating complex wood designs

NC Machines

Numerical control has been used in industry for more than 50 years. Simply put Numerical control is a method of automatic operating a manufacturing machine based on code of letters, numbers and characteristics. A complete set of coded instruction for executing an operation is called a Program. The program is translated into corresponding electrical signals for input to motors that run the machine. Numerical control machine can be program manually.

If a computer used to create a program, the process is known as Computer Aided Programming. The approach taken in this text will be in the form of manual programming. Traditionally, numerical control systems have been composed of the following components:

Tape Punch:

Converts written instructions into a corresponding whole pattern. The hole is punched into a tape, which pass through this device. Much older units used a type writer device called a flexowriter. Later devices included a microcomputer coupled with a tape punch unit.

Tape Reader:

Read the whole pattern on the tape and converts the pattern to a corresponding electrical signal code.

Controller:

Receives the electrical signals from the tape reader and subsequently causes the NC machine to respond.

NC Machine:

Responded the programmed signals from the controller. Accordingly, the machine executes the required motions to manufacturing a part (spindle rotation on/off, table and or spindle movement along programmed axis direction, etc.)

Difference between CNC and NC Machines

CNC (Computer Numerical Control) and NC (Numerical Control) machines are both used for automated machining, but they differ in how they operate and their level of automation

| Features | NC (Numerical Control ) | CNC (Computer Numerical Control) |
| --- | --- | --- |
| Control System | Operated using punched cards or tapes with hardcoded instructions. | Uses a computer and software to control machining operations. |
| Programming | Fixed programs that require manual modification for changes. | Flexible programming with easy modifications using CAD/CAM software. |
| Automation level | Partially automated, requiring more human supervision. | Fully automated, reducing human intervention. |
| Flexibility | Limited to specific tasks and difficult to modify. | Highly flexible and can handle various complex designs. |
| Accuracy & Precision | Lower precision due to manual adjustments. | High precision with automated feedback and corrections. |
| Memory & Storage | No internal memory; each program must be manually loaded. | Stores multiple programs in memory for easy recall and reuse. |
| Complexity of Operation | Can perform only simple machining operations. | Capable of handling multi-axis machining and complex geometries |
| Speed & Productivity | Slower due to manual data input and setup | Faster with continuous automatic operation and tool changing. |
| Cost | Lower initial cost but higher labor costs over time. | Higher initial cost but lower long-term labor costs. |

Types Of CNC Machines

There are many difference types of CNC Machine tools used in industry, Such as:

1- CNC Milling & Machining Centers

2- CNC Lathes and Turning Centers

3- CNC Drilling Machines

4- CNC EDM Machines

5- CNC Flame and Laser-Cutting Machines

6- CNC Water Jet Profilers

CNC Machining Center

Machining Center is the latest development in CNC Technology. These systems come equipped with automatic tool changer capability of changing 90 or more tools. Many are also fitted with movable rectangular worktables called pallets. The pallets are used to automatically load and unload work pieces. At a single setup, machining centers can perform such operations as milling, drilling, tapping, boring, and counter boring, and so on.  Additionally, by utilizing indexing heads, some centers are capable of executing these tasks on many different faces of a part and at specified angles. Machining Centers save production time and cost by reducing the need for moving a part from one machine to another.

Key Features:

Multi-Axis Capabilities: Available in 3-axis, 4-axis, and 5-axis configurations for intricate part production.
Automatic Tool Changer (ATC): Allows for seamless switching between tools without manual intervention.

High-Speed Spindle: Provides rapid cutting with high accuracy.

Computer Control Panel: Uses G-code and CAD/CAM software for automation and precision

CNC Turning Center

A CNC Turning Center is a type of CNC machine designed for high-precision turning operations, primarily used for shaping cylindrical parts. Unlike conventional lathes, CNC turning centers offer multi-axis movement and automatic tool changes, enabling complex part production with minimal human intervention.

Key Features:

Multi-Axis Control: Can operate on 2-axis (X and Z) or more (3, 4, and 5-axis) for complex geometries. Automatic Tool Changer (ATC): Allows for multiple operations without manual tool switching.

High Precision & Speed: Ideal for mass production of components with tight tolerances. Live Tooling Option: Some models include milling, drilling, and tapping functions.

CNC Drilling Machines

A CNC Drilling Machine is a precision tool designed to create holes in various materials, such as metal, plastic, wood, and composites. It uses computer-controlled automation to ensure accurate and consistent drilling, making it ideal for mass production and high-precision applications.

Key Features:

- Automated Hole Drilling: Ensures precise hole placement and depth.

- Multi-Spindle Options: Can drill multiple holes at once.

- High-Speed Drilling: Capable of rapid production cycles.

- CAD/CAM Integration: Uses G-code programming for automated operations.

- Rotary and Multi-Axis Movement: Some models allow drilling at multiple angles.

CN EDM Machines

A CNC EDM Machine is a specialized CNC machine that uses electrical discharges (sparks) to cut and shape hard materials with high precision. It is widely used in tool and die-making, aerospace, and medical industries to manufacture complex and delicate components.

Types of CNC EDM

- Wire EDM (Wire Electrical Discharge Machining) – Uses a thin wire as an electrode to cut intricate shapes.

- Sinker EDM (Die-Sinking EDM) – Uses an electrode submerged in dielectric fluid to create deep cavities.

- Hole-Drilling EDM – Designed for drilling tiny, precise holes in hard metals.

Key Features:

- Non-Contact Cutting: Ideal for hard metals like titanium and tungsten.

- High Precision & Fine Detailing: Used for intricate designs and molds.

- No Mechanical Stress: Produces smooth finishes without tool wear.

- Works with Hard & Conductive Materials: Effective for cutting carbide, hardened steel, and super alloys.

CNC Laser Cutting

A CNC Laser Cutting Machine is a high-precision machine that uses a focused laser beam to cut, engrave, or mark materials like metal, wood, plastic, glass, and composites. It is widely used in automotive, aerospace, electronics, and signage industries due to its speed, accuracy, and ability to cut intricate designs.

Key Features:

- High Precision Cutting: Achieves fine details with minimal material wastage.

- Non-Contact Cutting: Reduces wear and tear on tools.

- Works on Various Materials: Effective on metals, acrylic, leather, and more.

- Computer-Controlled Operation: Uses CAD/CAM software for automation.

Different Laser Types:

- CO₂ Laser: Best for wood, plastic, and organic materials.

- Fiber Laser: Ideal for metals like stainless steel and aluminum.

CNC Water Jet Cutting Machine

A CNC Water Jet Cutting Machine is a high-precision cutting tool that uses a high-pressure stream of water (or water mixed with abrasive particles) to cut through various materials. It is widely used in industries such as aerospace, automotive, metal fabrication, and construction due to its ability to cut complex shapes without generating heat.

Key Features:

- Cold Cutting Process: No heat-affected zones (HAZ), preventing material distortion.

- Works on Various Materials: Cuts metal, glass, stone, wood, ceramics, composites, and rubber.

- High Precision & Smooth Finish: No need for additional finishing in many cases.

- Eco-Friendly: Uses water and abrasive materials instead of toxic chemicals.

- Multi-Axis Capabilities: Available in 3-axis, 4-axis, and 5-axis configurations for intricate designs.

CNC Machining Center Components

Machining Centers can be classified into two main types:

1- Vertical Spindle Machining Center

2- Horizontal Spindle Machining Center

Vertical Spindle Type

A Vertical Machining Center (VMC) is a type of CNC milling machine where the spindle is vertically oriented. It is one of the most commonly used CNC machines for precision milling, drilling, tapping, and contouring operations

Horizontal Spindle Type

A Horizontal Machining Center (HMC) is a type of CNC milling machine where the spindle is horizontally oriented. It is designed for high-efficiency and heavy-duty machining, making it ideal for automotive, aerospace, and industrial applications where complex and multi-sided machining is required.

Component of CNC Machining Center

Vertical Machining Center

Main Control Panel

ATC & Lubrication Unit

Pneumatic Power Unit

a. Main spindle

It is where milling cutters are installed. Machining centers cut work pieces with tool installed on the main spindle and rotates together with it. Rotation of the main spindle is regulated to the required speed according to commands from the NC unit by means of gears or belts.

b. Spindle head

The spindle head supports the main spindle through bearings and transmits the rotation of the spindle motor to the main spindle. The spindle head is move up and down by the servomotor, and forms the Z-axis in the vertical machining centre.

c. Table

The table is for setting work pieces with jigs and fixtures. Tee slots and tap holes are provided on the table for fixing Workpiece.

The saddle supports the table and makes front and rear movement on the bed.  It forms the Y-axis in the vertical machining centre, while it forms the Z-axis in the horizontal machining centre.

e. Column

The column holds the spindle head.  There are two types of column, i.e. one is a fixed type column which is fixed on the bed and the other one is a travelling type column which can travel on the bed.

f. Bed

The bed supports the column and the saddle and acts as the foundation of the entire machine.

g. Automatic tool changer (ATC)

ATC is a device use to change automatically a tool already set on the main spindle with a tool to be used for machining in the next stage. It comprises the ATC arm to grasp a tool and the ATC magazine to accommodate various tools.

h. Feed mechanism

The feed mechanism comprises a servomotor, ball screw, ball nut, etc.  It is a mechanism use to position and feed the table, saddle and spindle head by the command from the NC unit. The rotation of the servomotor is transmitted to the ball screw by means of gears, belts or couplings and causes linear motions of the table, saddle and spindle head connected directly to the ball nut.  For the servomotor, an electro hydraulic pulse motor was adopted in an early stage, then a DC servomotor and recently an AC servomotor is in use.

i. Coolant supplying device

The coolant supply device is use to supply coolant to tools or work pieces and recover it through pumps. Generally water-soluble coolant or water-insoluble coolant is used as coolant to lubricate and cool the cutting surfaces of the Work piece and the tool.

Methods of supplying coolant

1)  Nozzle coolant method

A method of supplying coolant from nozzles fitted close to the spindle nose.

2)  Through coolant method

A method of supplying coolant through the main spindle and the tool.

3)  Shower coolant method

A method of supplying coolant to the entire tool and Workpiece.

4)  Mist coolant method

A method of supplying coolant mixed with air in the form of mist.

j. Main spindle temperature adjusting unit

The main spindle temperature adjusting unit is a device use to lubricate and cool the main spindle, bearings and gears which are running high speed, and to prevent the degradation of machining accuracy cause by the thermal distortion of the machine.

k. Hydraulic power unit

The hydraulic unit is use as a power source for changing tools in the ATC and pallets

l. Pneumatic power unit

The pneumatic power unit is use as a pneumatic power source for the protection of the machine from chips and coolant, the cleaning of the main spindle hole and tool shank at the time of tool change, and mist coolant mixing cutting oil and air.

m. Sliding surface lubricating device

The sliding surface lubricating device is a device use to supply lubricating oil to the sliding surfaces of the spindle head, table and saddle.

CNC Control Panel

The CNC Control Panel is the brain of a CNC Machining Center. It allows the operator to input commands, monitor machining processes, and control machine movements. The control panel consists of multiple sections, each with a specific function.

Types of CNC Controllers

There are different types of controllers used for CNC machines .

- Siemens Controller (Germany)

- Fanuc Controller (USA)

- GSK Controller (China)

- Heidenhain Controller (Germany)

- Mazak  Controller  (Japan)

- Hass automation (USA)

- Okuma Controller (Japan)

There are two section of Machine Control Unit

- PPU (Panel Processing Unit)

- MCP (Machine Control Panel)

Element on the PPU

Elements on the MCP

Activity: Operate the control panel of CNC Vertical Milling Machine

- Wear appropriate safety gear

- Ensure the machine area is clean and free of obstructions.

- Engage the Emergency Stop (E-Stop) before powering on

- Turn on the main power supply and release the emergency stop

- Reference (home) the machine to set all axes to their zero position

- Moves the machine axes manually

- Adjust spindle speed and activate coolant

- Starts and pauses program execution

- Switches between manual, jog, and automatic operation

- Secure the work piece on the machine table using a vise or clamps

- Install and tighten the cutting tool in the spindle chuck or tool holder

- Set up tool length offset using a tool setter or touch probe

- Define the work coordinate system (WCS) by setting the work piece zero point

- Load and verify the tool magazine

- Insert the USB drive or input the program manually

- Load the program into the machine’s memory

- Edit and save the program

- Run the program in dry run mode (without cutting) to check tool movements

- Observe tool movements and check for vibrations or unusual noises

- Adjust feed rate and spindle speed

- Monitor the coolant flow to prevent overheating

- Return the machine to home position

- Power down the machine properly

|  | Calculate coordinates of job |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Mark points on part drawing to identify position of job.

- Calculate coordinates of milling job as per drawing.

Coordinate System

Coordinate system is a method or techniques used to determine the position of a point or location on a plane (2D) or in space (3D). CNC machine cannot be operated without understanding the coordinate system. Coordinate system is used to locate the position of cutter or position of any point on the work piece or on the table of CNC machine.

Types of Coordinate System

- Cartesian Coordinate System (Rectangular )

- Polar Coordinate System

- Cylindrical Coordinate System

- Spherical Coordinate System

- Top of Form

- Bottom of Form

Cartesian (Rectangular) Coordinate System

The most commonly used system in CNC machining is the Cartesian coordinate system, also called the rectangular coordinate system. It consists of three axes:

- X-axis: Moves the tool or workpiece left and right.

- Y-axis: Moves the tool or workpiece forward and backward.

- Z-axis: Moves the tool or workpiece up and down

The origin, or Datum, is where the three axes cross each other. The labels, orientations, and directions of the Cartesian coordinate system.

Machine Coordinate System (MCS)

The Machine Coordinate System is the default reference system set by the CNC machine manufacturer.

- It is based on the home position, where the machine resets after being powered on.

- The machine zero point (MCS origin) is fixed and cannot be changed.

- It is used to reference all movements and tool positions.

The Origin point for the machine coordinate system is called Machine Home. This is the position of the centerface of the machine spindle when the Zaxis is fully retracted and the table is moved to its limits near the back left corner.

Work Coordinate System (WCS)

The   WCS   is   a   point   selected   by   the   CNC   programmer   on   the   part,   stock   or fixture.   While   the   WCS   can   be   the   same as the part origin in CAD, while it can be located anywhere in the machine envelope. Its   selection   requires   careful   consideration.

- The WCS location must be able to be found by mechanical means such as an edge finder, coaxial indicator.

- It must be located with high precision

- It must be repeatable: parts must be placed in exactly the same position every time.

- It should take into account how the part will be rotated and moved as different sides of the part are machined.

WCS

Origin

WCS

WCS

Absolute & Incremental Coordinate System

Absolute Coordinate System

Absolute Coordinate system is that system in which each point or location on the table or workpiece is measured from a fixed origin. All the dimensions are mention from the origin point. This point may be MCS or WCS.

Remember

Incremental coordinate system is also called relative coordinate system.

Incremental Coordinate System

The incremental (or relative) coordinate system is a method where each point’s position is given relative to the previous point rather than an absolute reference (such as the origin in an absolute coordinate system).

Points Value:

- A(2,3)

- B(-3,1)

- C(3,-6)

- D(-5,-1)

- E(3,3)

Cutting Tool Geometry

A CNC milling cutter is designed to remove material efficiently while maintaining tool strength and accuracy. The geometry of a milling cutter plays a crucial role in its performance.

Cutting Flutes

- These are the sharp edges that cut into the material.

- More flutes mean a smoother finish but slower material removal.

- Fewer flutes allow for better chip evacuation and are better for softer materials.

Helix Angle

- The angle of the flutes relative to the tool’s axis.

- A high helix angle (e.g., 45°) produces smoother cuts and reduces cutting forces.

- A low helix angle (e.g., 30°) is better for hard materials

Rack angle

- The angle between the rake face (cutting edge) and the workpiece surface.

- A positive rake angle results in a sharper edge and reduces cutting forces.

- A negative rake angle makes the tool stronger but increases cutting resistance.

Relief Angle

- Ensures that only the cutting edge touches the material, reducing friction and wear.

- Primary relief angle (5°-15°) provides clearance for cutting.

- Secondary relief angle (higher than primary) extends tool life.

End Cutting Edges

- The cutting edge on the bottom of an end mill.

- Important for plunge cutting and slotting operations.

Core diameter

- The central portion of the cutter that provides structural strength.

- A larger core diameter makes the tool more rigid.

Tool coating

- Titanium Nitride (TiN), Titanium Carbonitride (TiCN), and Aluminum Titanium Nitride (AlTiN) coatings improve tool life and reduce friction.

Activities: Calculate the coordinates of given drawing

|  | Absolute | Incremental |
| --- | --- | --- |
| From Point (1) |  |  |
| To Point (2) |  |  |
| To Point (3) |  |  |
| To Point (4) |  |  |
| To Point (5) |  |  |
| To Point (6) |  |  |
| To Point (7) |  |  |
| To Point (8) |  |  |
| To Point (9) |  |  |
| To Point (10) |  |  |
| To Point (1) |  |  |

|  | Mount and align the job (Tool and work offset) |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Mount the job on the table of CNC machine.

- Align and straighten the work piece using dial indicator

- Set and align tool properly in the tool holder

- Perform work reference setting for different tools

Milling Cutter & Types

Milling cutters are essential tools used in CNC machining centers to remove material from a workpiece. They come in various shapes and sizes, each designed for specific milling operations like face milling, end milling, slotting, and profiling.

Number of Flutes

Milling tools usually have either two or four cutting flutes. Two flute cutters provide more chip clearance.

when milling in close areas. Four flute mills are more rigid, can be fed faster, and are preferred when greater chip clearance is not required such as when milling an outside contour.

Center-Cutting End Mills

Milling tools are either center cutting or noncenter cutting. Center cutting mills can plunge straight down into material. While non-center cutting tools cannot End Mill Cutter.

- Flat Nose: Flat nose mills are used for milling 2D contours and pockets.

- Ball Nose: Ball nose mills are used for 3D milling.

- Bull Nose: Bull nose end mills have a radius corner. They are used to create a fillet on the bottom of a wall.

- Chamfer: Chamfer mills have an angled nose used to create a chamfer or to de-burr parts.

Face Mill

A face mill has cutting inserts that are replaced when worn. They are rigid may have up to eight or more cutting edges, and can remove material quickly. They are often used for the first machining operation to quickly create a flat finished face on the part.

Corner Radius tool

Corner radios (also called Corner Round) tools are used to place a fillet on the outside corner of a part.

Slot Mill/Slotting Saw

Slot mills include side milling cutters, slitting saws, and Woodruff keyset cutters. Slitting saws and side milling cutters are installed on a special arbor. Woodruff cutters are single piece tools used for C rating slots and undercuts that can be held in a standard tool holder.

Spot Center Drill

- Center (spotting) drills are short and very rigid drills used to create a conic on the face of the part.

- Countersink drills are used to create the conical face for a machine screw.

- Combined spotting countersinks are used to create a screw clearance hole and countersink in one operation.

Twist Drill

A twist drill is a commonly used cutting tool designed for drilling holes in various materials like metal, wood, and plastic.

Twist drills are available in many diameters and lengths. Usually made of high speed steel carbide, or cobalt. They may also be coated with titanium nitride (TiN) for longer life. The tip angle of most twist drills is 118 degree.

Tape

- Cutting taps from threads by shearing material away. From taps (roll taps) from the thread by forming the metal to shape. From taps produce no chips and are used for soft materials including aluminum, copper, brass and plastics.

- Bottoming taps are used to tap blind Holes.

- Spiral point taps push the chip ahead and out the bottom of a through hole.

Reamer

A reamer is a rotary cutting tool used for precision finishing of drilled holes to achieve accurate size, smooth surface finish, and tight tolerances.

Couterbore

It is used to spot face holes, and the pilot ensures the spot face is centered on the hole.

Work offset

A work offset is the process of defining the zero point (datum) of a workpiece relative to the CNC machine’s coordinate system. This ensures the machine accurately positions the cutting tool for machining operations. There are different methods to set a work offset based on the tools and precision required.

- Edge Finder method

- Dial indicator method

- Touch probe method

- Paper or feeler gauge method

- Tool setter (Presenting Z work offset)

Edge Finder Method

This method is the most common used in the workshop for finding the work offset or work zero of the work piece. An edge finder is used, the edge finder may be mechanical or electrical. The step by step procedure is given below:

- Load the Edge Finder in the Spindle

- Select the Correct Work Coordinate System (WCS)

- Move the Spindle Close to the Workpiece Edge

- Slowly Touch the Edge with the Edge Finder

- Note the Machine Position & Apply Correction

- Enter the Offset Value into the CNC Control

- Repeat for the Other Axis

- Set the Z-Axis Work Offset

- Verify the Work Offset & Begin Machining

Dial Indicator method

The dial indicator method is used to precisely align workpiece or fixtures and set work offsets in CNC   machining. This method is particularly useful for ensuring a workpiece is parallel to the machine axes before machining.

- Mount the Dial Indicator in the Spindle

- Position the Indicator Near the Workpiece or Fixture

- Sweep the Indicator Along the Workpiece Edge

- Adjust the Workpiece or Fixture

- Zero the Work Offset in the CNC Control

- Repeat for the Other Axis

- Set the Z-Axis Offset

- Verify the Work Offset & Begin Machining

Touch probe method

The touch probe method is an automated way to set work offsets with high precision. This method uses a CNC probe to detect the exact position of a workpiece and automatically enter the offset values into the machine’s control system.

- Load the Touch Probe in the CNC Spindle

- Select the Work Offset (G54, G55, etc.)

- Position the Probe Close to the Workpiece

- Run the Automatic Probe Cycle

- Repeat for the Other Axis

- Set the Z-Axis Offset

- Verify the Work Offset & Begin Machining

Paper or feeler gauge method

The paper or feeler gauge method is a simple and effective way to set the Z-axis work offset in a CNC machine. It involves placing a thin material (like a piece of paper or a feeler gauge) between the tool and the workpiece to precisely determine the tool’s position.

- Load the Cutting Tool into the CNC Spindle

- Select the Work Offset (G54, G55, etc.)

- Move the Spindle Close to the Workpiece Surface

- Place the Paper or Feeler Gauge on the Workpiece

- Lower the Tool Until It Touches the Paper or Gauge

- Note the Machine Position & Apply Correction

- Enter the Z-Offset Value into the CNC Control

- Verify the Work Offset & Begin Machining

Tool setter

The tool setter method is a highly accurate and automated way to set tool offsets in CNC machining. It ensures that tools are precisely measured, reducing errors and improving machining efficiency. Tool setters can be manual (mechanical) or automatic.

- Install the Tool Setter on the CNC Table

- Load the Cutting Tool into the CNC Spindle

- Select the Tool Offset Measurement Mode

- Move the Tool Close to the Tool Setter

- Lower the Tool Until It Touches the Tool Setter

- Record the Tool Length Offset

- Store the Tool Offset in the CNC Control

- Repeat for All Tools

- Verify the Work Offset & Begin Machining

Tool offset

Tool offset in CNC machining refers to the adjustment values used by the machine to account for differences in tool length, diameter, and wear. It ensures that the cutting tool accurately follows the programmed tool path, leading to precise machining results.

Tool length offset

Tool Length Offset (TLO) is a correction value that compensates for differences in tool length in CNC machining. It ensures that each tool reaches the correct cutting depth relative to the workpiece, preventing incorrect cuts or crashes.

Tool diameter offset

Tool Diameter Offset (D-Offset) is a correction value used to compensate for the cutter’s radius or diameter in CNC machining. It ensures the tool follows the correct toolpath by adjusting for the actual tool size, especially in contour milling and pocketing operations.

Tool Wear offset

Tool Wear Offset is a correction value applied to compensate for the gradual wear of cutting tools during machining. As a tool wears down, its length and diameter change, which can affect part accuracy. Instead of modifying the CNC program, machinists update the wear offset to maintain precision.

Activity Mount & align the job and set work reference on CNC Milling machine table

Work Offset

Select the Work Reference Point

- Choose a reference point on the workpiece (corner, center, or top surface).

- This will be used as the work zero (G54, G55, etc.).

Find the X and Y Work Offsets

- Use an edge finder, dial indicator, or probe to locate the edges of the workpiece.

- Move the tool to the reference point and set the X and Y coordinates to zero in the CNC controller.

- Save the values under G54 (or G55, G56, etc.).

Find the Z Work Offset

- Lower the cutting tool until it lightly touches the workpiece surface.

- Set the Z-axis to zero at this point and save the value in the CNC control.

Tool Offset

Mount the Cutting Tool

- Insert the tool in the tool holder and secure it in the spindle.

- Ensure the tool is properly tightened.

Measure the Tool Length Offset

- Use a tool presetter, probe, or touch-off method to find the tool length.

- Store the tool length value in the tool offset table (H1, H2, etc.).

- Apply tool length compensation in the program using G43 H1 (for Tool 1).

Set Tool Diameter Compensation

- For milling operations, input the tool diameter in the offset table.

- Use G41 (left compensation) or G42 (right compensation) in the program.

|  | Prepare Manual Program |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Demonstrate functions of G-codes and M-codes for CNC milling machine.

- Develop drilling program as per drawing.

- Develop facing program as per drawing.

- Develop pocket milling program as per drawing.

- Develop slot milling program as per drawing.

- Write drilling program containing multiple holes patterns by applying sub programming.

Introduction to CNC Programming

CNC (Computer Numerical Control) programming is the process of writing coded instructions for a CNC machine to control the movement of cutting tools. It uses G-codes and M-codes to automate machining processes like milling, turning, and drilling.

There two method for CNC Programming.

- Manual or NC Programming

- CAM Programming

Before starting CNC programing basic knowledge of the following are needed:

- Co-ordinate System

- Units, Absolute or Incremental Positioning

- Co-ordinate, X,Y,Z

- Feed rate and Spindle speed

- Coolant On/Off, Flood, Mist

- Tool control: Tool and Tool Parameter

Programming consist of a series of instructions in the form of letter codes

PREPARATORY CODES:

- G codes - Initial machine setup and establishing operating condition

- N codes - Specify program line number to executed by MCU

- Axis code: xyz- Used to specify motion of the sliding along X,Y,Z directions

- Feed and Speed code- F and S specify Feed and Spindle Speed

- Tool code: T- Specify tool number

- Miscellaneous code- M codes for coolant control and other activates

Programming Key letters

- – Program number ( used for program identification)

- N – Sequence number ( used for line identification)

- G - Preparatory function

- X- X axis designation

- Y- Y axis designation

- Z- Z axis designation

- R-Radius designation

- F- Feed rate designation

- S-Spindle speed designation

- H-Tool length offset designation

- D-Tool radius offset designation

- T-Tool designation

- I,J,K Interpolation Parameter/Intermediate Point

- M-Miscellaneous function

Programme Structure:

Each line of instruction is referred to as a ―block‖ and a collection of blocks compose a Program. One block can be divided as follows:

X 50.0	Y 100.0	;	 One block

7.4.2   G-Codes & M-Codes

List of G-Codes Use in Programming

| G-code | Group | Function |
| --- | --- | --- |
| 00 | 01 | Positioning |
| 01 | 01 | Linear interpolation |
| 02 | 01 | Circular interpolation CW |
| 03 | 01 | Circular interpolation CCW |
| 04 |  | Dwell |
| 09 | 00 | Exact-stop check |
| 10 | 00 | Program parameter input |
| 17 | 02 | Plane selection X-Y |
| 18 | 02 | Plane selection Z-X |
| 19 | 02 | Plane selection Y-Z |
| 20 | 06 | inch command |
| 21 | 06 | mm command |
| 27 | 00 | Reference point check |
| 28 | 00 | Reference point return |
| 29 | 00 | Starting point return |
| 37 | 00 | Automatic tool length measurement |
| 39 | 00 | Tool diameter offset corner arc |
| 40 | 07 | Tool diameter offset cancel |
| 41 | 07 | Tool diameter offset to the left |
| 42 | 07 | Tool diameter offset to the right |
| 43 | 08 | Tool length offset (+) |
| 44 | 08 | Tool length offset (-) |
| 49 | 08 | Tool length offset cancel |
| 50 | 11 | Scaling cancel |
| 51 | 11 | Scaling |
| 50.1 | 19 | Mirror image cancel |
| 51.1 | 19 | Mirror image |
| 52 | 00 | Local coordinate system setting |
| 53 | 00 | Machine coordinate system selection |
| 54 to 59 | 12 | Workpiece coordinate system |
| 61 | 13 | Exact-stop check mode |
| 62 | 13 | Automatic corner override |
| 63 | 13 | Tapping mode |
| 64 | 13 | Cutting mode |
| 68 | 16 | Programmed coordinates rotation |
| 69 | 16 | Programmed coordinates rotation cancel |
| 71.1 | 09 | Fixed cycle (chamfering cutter 1) |
| 72.1 | 09 | Fixed cycle (chamfering cutter 2) |
| 73 | 09 | Fixed cycle (high speed deep-hole drilling) |
| 74 | 09 | Fixed cycle (reverse tapping) |
| 75 | 09 | Fixed cycle (boring) |
| 76 | 09 | Fixed cycle (boring) |
| 77 | 09 | Fixed cycle (back spot facing) |
| 78 | 09 | Fixed cycle (boring) |
| 79 | 09 | Fixed cycle (boring) |
| 80 | 09 | Fixed cycle cancel |
| 81 | 09 | Fixed cycle (spot drilling) |
| 82 | 09 | Fixed cycle (counter boring) |
| 83 | 09 | Fixed cycle (deep-hole drilling) |
| 84 | 09 | Fixed cycle (tapping) |
| 84.2 | 09 | Fixed cycle (synchr. tapping) |
| 84.3 | 09 | Fixed cycle (synchr. reverse tapping) |
| 85 | 09 | Fixed cycle (reaming) |
| 86 | 09 | Fixed cycle (boring) |
| 87 | 09 | Fixed cycle (back boring) |
| 88 | 09 | Fixed cycle (boring) |
| 89 | 09 | Fixed cycle (boring) |
| 90 | 03 | Absolute data input |
| 91 | 03 | Incremental data input |
| 92 | 00 | Coordinate system setting |
| 93 | 05 | Inverse time feed |
| 94 | 05 | Asynchronous feed (feed per minute) |
| 95 | 05 | Synchronous feed (feed per revolution) |
| 98 | 10 | Initial point level return in fixed cycle |
| 99 | 10 | R-point level return in fixed cycle |

3.7- List of M-codes

| M-code | Function |
| --- | --- |
| 00 | Program stop |
| 01 | Optional stop |
| 02 | End of program |
| 03 | Spindle rotation (normal) |
| 04 | Spindle rotation (reverse) |
| 05 | Spindle stop |
| 06 | Tool change |
| 07 | Mist coolant |
| 08 | Liquid coolant |
| 09 | Coolant off |
| 30 | End of program |
| 98 | Call-up subprogram |
| 99 | End of subprogram |

G-Codes:

Sub Program

The programmer will often encounter cases where a sequence of machining instructions is to be repeated. An example of this is a part having an identical geometric pattern appearing at different locations within its body. The programmer could code the instructions to machine each pattern individually for the part shown in Fig.

This would result in a program that has the same machining sequences appearing three times. Thus, the program would be lengthy, the chances for error would be increased, and the program would occupy more memory in the MCU*. A better approach is to create a separate program called a subprogram that contains the instructions for machining one pattern. A main program is also created that calls the subprogram as many times as the machining sequence is repeated. In this case, the subprogram would be called three times to machine pattern A. The subprogram can also be used by other main programs any time pattern A is required. In fact, a library of subprograms can be created for machining specific patterns.

It should be noted that both the main program and each subprogram are separate programs that must be stored in the MCU‘s memory prior to execution.

How to use subprograms

It is possible to call and run a program during the execution of another program. The called program is referred to as a subprogram. Subprogram is sometimes referred to as asubpro‘or ‗sub‘for short.

Format:

M 9 8 P 	 L 	 ;	: P is the number of the calling program.

: L is the repetition count.

Enter “end of subprogram” in the called program beforehand. (Change ‘M30’ to ‘M99’.) Example 1

| - <Main Program> | - <Subprogram> |
| --- | --- |
| - O0010 - G90G17 - G00G54X0Y0 - G43Z100.H01S10000M03 - M98P1001 (Call subprogram Pattern A) - G90G00Z100. - X-40. - M98P1001 - G90G00Z100. - X40. - M98P1001 - G91G00G28 Z0 - M30 ​ | - O1001 (Pattern A) - G91 - G00X10. - Z-95. - G01Z-10.F50 - G42X5.F100D60 - Y-12. - G02X-3.Y-3.I-3. - G01X-7. - G02X-10.I-5. - G01X-7. - G02X-3.Y3.J3. - G01Y22. - G02X5.Y5.I5. - G01X2. - G02X3.Y-3.J-3. - G01Y-2. - G03X10.I5. - G01Y2. - G02X3.Y3.I3. - G01X2. - G02X5.Y-5.J-5. |

Activity: Prepare manual program as per drawing

|  | Prepare canned cycle program |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Develop Simple Drilling Cycle (G81) program as per drawing

- Develop Spot Facing Cycle (G82) program as per drawing

- Develop Peck Drilling Cycle (G83) program as per drawing

- Develop Tapping Cycle (G84) program as per drawing

- Develop Boring Cycle (G85) program as per drawing

- Develop Boring, Rapid Retract (G86) program as per drawing

Canned Cycle

A canned cycle is a pre-programmed sequence of operations used to simplify repetitive machining tasks like drilling, tapping, boring, and pocketing. It reduces the number of G-code lines needed, making CNC programs shorter and more efficient.

| Code | Application | Compatible tools |
| --- | --- | --- |
| G73 | Rapid deep hole drilling | Drill |
| G74 | Counter clockwise tapping | Left-handed tap |
| G76 | Fine boring | Boring bar |
| G80 | Cancelling |  |
| G81 | Spot drilling | Drill, end mill |
| G82 | Counter boring | Drill, end mill |
| G83 | Deep hole drilling | Drill, end mill, reamer |
| G84 | Tapping | Tap |
| G85 | Reaming | Boring bar |
| G86 | Boring | Boring bar |
| G87 | Boring | Boring bar |
| G88 | Boring | Boring bar |
| G89 | Boring | Boring bar |

Format of fixed cycles

| G90 G91 | G98 G99 | G81 G82 G83 | X _ _ Y _ _ Z _ _ R _ _ Q _ _ P _ _ F _ _ L _ _M _ _ ; |
| --- | --- | --- | --- |

| G90, G91 | : | Data format of X, Y, Z and R |
| --- | --- | --- |
| G98, G99 | : | Return point level |
| G81,G82,G83 |  | Type cycle |
| X _ _ Y _ _ | : | Coordinates of drilling location |
| Z _ _ | : | Depth of drilling |
| R _ _ | : | Start height of drilling (reference point, R point) |
| Q _ _ | : | Depth of one cut (required when G73 and G83 are used) Shift amount (required when G76 and G87 are used) |
| P _ _ | : | Dwell time at bottom of hole (the same meaning as G04) |
| F _ _ | : | Feed rate ( F one-digit statement is possible) |
| L _ _ | : | Repetition count (if L0 is specified, data is memorized, but no machining is performed) |
| M _ _ | : | M codes may specified together (for example, M08) |

7

1

4

6

3

5

8

2

| - O0010 - G90G17 - G00G54X0Y0 - G43Z100.0H01S1200M03 - G98G81X60.0Y35.0R5.0 - Z-5.0F100 - ​X60.0Y-35.0 - X40.0Y0 - X0Y20.0 - X0Y-20.0 - X-40.0Y0 - X-60.0Y35.0 - X-60.0Y-35.0 - G80X0Y0M05 - G91G28Z0M30 - Return point level | - Initial point - Fixed cycle G81, Moves from the initial point tothe R point by rapid traverse. - Drills a hole from the R point to the Z point at the cutting feed rate. - Hole2 Drilling - Hole3 Drilling - Hole4 Drilling - Hole5 Drilling - Hole6 Drilling - Hole7 Drilling - Hole8 Drilling - Fixed cycle cancel |
| --- | --- |

Specifies the height when returning to second and subsequent holes.

| G98: | Initial point level | Height just before specification of fixed cycle |
| --- | --- | --- |
| G99: | R point level | Height of R point during execution of fixed cycle |

G98

R point

Z 0

Z point

G99

G85 Boring Cycle

G86 Boing Cycle

Activity: Prepare manual program by using canned cycle

|  | Perform Simulation (Swansoft / CIMCO Simulators) |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Set job and tool in simulation software as per drawing.

- Insert part program into simulation software

- Run simulation of program to identify and correct potential errors in part program

- Obtain practical experience in operating virtual CNC machines, which prepares learners for real-world machine handling.

CNC Simulation

CNC (Computer Numerical Control) simulation is the process of virtually modeling and testing CNC machining operations before running them on a real machine. It uses specialized software to visualize tool movements, material removal, and machine behavior, ensuring accuracy and efficiency in manufacturing.

CNC Simulation Software

CNC (Computer Numerical Control) simulation software is a tool used to virtually test and verify CNC machining programs before executing them on actual machines. It allows users to simulate tool movements, material removal, and machine operations in a virtual environment. This helps in detecting errors, optimizing tool paths, and improving overall machining efficiency.

CIMCO EDIT Simulation Software

SwanSoft Simulation Softwar

Importance:

- Error Detection – Helps identify programming mistakes, such as collisions, overcuts, and incorrect tool paths before actual machining.

- Cost Reduction – Minimizes material waste and machine downtime by preventing incorrect cuts.

- Machine Safety – Prevents potential damage to expensive CNC machines due to incorrect G-code execution.

- Process Optimization – Enhances efficiency by refining tool paths, reducing cycle times, and improving workflow.

- Training and Learning – Provides a risk-free platform for CNC operators and programmers to learn and practice without real-world consequences.

- Quality Assurance – Ensures the final product meets design specifications by verifying machining parameters in advance.

Benefits:

- Reduces Trial & Error

- Saves Time

- Enhances Productivity

- Supports Multiple Machines

- Improves Workflow

- Shop Safety

- Faster Shifting between machines

- Train at PC instead of machine

Simulation Software:

- Fusion 360 with Machining Extension – Includes powerful CNC simulation tools integrated with CAD/CAM.

- Mastercam Simulator – Provides detailed toolpath simulation and verification.

- SolidCAM iMachining – Offers real-time simulation and optimization.

- GibbsCAM – Focuses on high-performance machining simulation.

- VERICUT – Industry-leading software for simulating, verifying, and optimizing CNC programs.

- NC Viewer – A free online G-code simulator for quick validation.

- CIMCO Edit – Includes backplotting and simulation for G-code verification.

- CAMWorks Virtual Machine – Provides machine-level simulation for CNC operations.

- SprutCAM – Features 3D visualization for multi-axis machining.

- Predator Virtual CNC – Used for real-time machine simulation.

- Swansoft Simulator

Steps in performing Simulation

- Load the CNC Program (G-Code)

- Define Machine Setup

- Tool path Visualization

- Collision Detection & Error Checking

- Optimize Cutting Parameters

- Material Removal Simulation

- Final Verification & Code Optimization

- Export & Run on CNC Machine

Program Debugging Techniques

Debugging is the process of identifying and fixing errors in a program to ensure it runs correctly. In CNC programming, debugging helps prevent crashes, improve efficiency, and optimize machining processes. Below are some effective debugging techniques.

- Syntax Checking

- Dry Run

- CNC Simulator

- Check Work offset & Tool offset

- Steps through Execution (Single Block)

- Verifiy Speed & Feed Rate

- Check Toolpath & Machine Limits

- Use Error Message and Diagnostics

- Compare with good Known program

- Final Test with a scrap workpiece

Activity: Check & verify the prepared program by using simulation software for the drawing

|  | Feed the Program in machine |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Set job and tool in simulation software as per drawing.

- Use simple G-Codes and M-codes in MDI mode.

- Input program into CNC machine using PROGRAM, EDIT, INSERT, ALTER and EOB keys.

- Apply simulation of tool movement to check the program for errors.

- Modify program if required and run it in single block mode.

Input Methods

CNC machines allow program input through different modes:

- MDI Mode (Manual Data Input): Used for entering simple G-code commands manually.

- DNC Mode (Direct Numerical Control): The program is transferred from an external computer.

- Memory Mode: A saved program is retrieved from the machine’s memory.

Steps for inputting a program

- Switch the machine to EDIT mode.

- Use the PROGRAM key to open the program window.

- Enter the program manually using the INSERT or ALTER key.

- Use the EOB (End of Block) key to confirm each command line.

- Save the program and switch to MEMORY mode to run it.

Importance of Optional Stop and Compulsory Stop

- Optional Stop (M01): Allows the operator to pause the program execution if required.

- Compulsory Stop (M00): Forces the machine to stop at a specific point until the operator resumes.

Importance of Single Block Mode

- Single block mode executes the program one line at a time, allowing the operator to verify each movement before proceeding.

- Helps in debugging and error prevention.

Dry Run and Program Simulation

- Dry Run: A test execution of the program without engaging the tool or workpiece to detect errors.

- Simulation Mode: Displays the tool path on the CNC screen before actual machining.

Steps for Simulation & Dry Run:

- Load the program into the CNC machine.

- Switch to Dry Run Mode.

- Execute the program without material to check tool movement.

- Make necessary modifications if errors are found.

Properties of different materials and machining finishing strategies

- Material Properties: Different materials (e.g., aluminum, steel, brass) require different cutting speeds, feed rates, and tool types.

Finishing Strategies:

- Roughing: Removes excess material quickly with a high feed rate.

- Finishing: Uses lower feed rates for smooth surface quality.

- Peck Drilling: Used for deep holes to prevent tool breakage.

Running the program in single block mode

- Switch to Single Block Mode.

- Press Cycle Start to execute one line at a time.

- Verify tool movement after each block.

- Continue until the full program runs successfully.

Activity: Enter the program in machine & check its validity

|  | Operate the machine |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:.

- Run the program key in single block mode using cycle start.

Properties of material

CNC (Computer Numerical Control) machining centers are used in various industries, including aerospace, automotive, medical, and manufacturing. The choice of material plays a crucial role in determining the efficiency, durability, and quality of the final product. Understanding the properties of materials used in CNC machining helps in selecting the right material for specific applications.

Properties of Material

1.  Mechanical Properties

These properties determine how a material responds to mechanical forces such as tension, compression, and impact.

- Hardness – Resistance to surface indentation and wear (e.g., steel, titanium).

- Toughness – Ability to absorb energy and resist fracture (e.g., stainless steel).

- Ductility – Ability to stretch without breaking, important for forming processes (e.g., aluminum, copper).

- Tensile Strength – Maximum stress a material can withstand while being stretched (e.g., carbon steel).

2.  Machinability

Machinability refers to how easily a material can be cut, shaped, and finished with CNC tools.

- Good Machinability – Produces smooth surfaces with minimal tool wear (e.g., aluminum, brass).

- Poor Machinability – Requires special tooling and cooling systems (e.g., titanium, hardened steel).

3. Thermal Properties

These properties affect how materials behave under heat generated during machining.

- Melting Point – Determines temperature resistance (e.g., titanium has a high melting point).

- Thermal Conductivity – Affects heat dissipation and machining efficiency (e.g., copper has high thermal conductivity).

- Thermal Expansion – Impacts dimensional stability (e.g., aluminum expands more than steel when heated).

4. Corrosion Resistance

- Important for materials used in marine, medical, and aerospace applications.

- Stainless steel and titanium resist oxidation and rust, unlike carbon steel, which requires coatings or treatments.

5.   Cost & Availability

- Materials like aluminum and mild steel are cost-effective and widely available.

- High-performance materials like titanium and Inconel are expensive but necessary for specialized applications.

| Common Material Used in CNC Machining |  |  |  |
| --- | --- | --- | --- |
| Aluminum (6061, 7075, etc.) | Lightweight, corrosion-resistant, and highly machinable. Usd in aerospace, automotive, and consumer electronics. |  |  |
| Steel (Mild Steel, Stainless Steel, Tool Steel) | High strength and durability. Stainless steel resists corrosion and is used in medical and food industries. Tool steel is hardened for wear resistance, used in molds and cutting tools. |  |  |
| Brass & Copper | Excellent machinability and conductivity. Used in electrical components and decorative applications. |  |  |
| Titanium | High strength-to-weight ratio and excellent corrosion resistance. Used in aerospace, medical implants, and high-performance industries. |  |  |
| Nylon, PEEK, PTFE (Teflon), Acrylic | Lightweight and corrosion-resistant. Used in medical, aerospace, and electrical applications. |  |  |
| Carbon Fiber & Fiberglass Composites | High strength with low weight. Used in automotive, aerospace, and sports equipment. |  |  |
| Material Selection based on CNC machining Operation |  |  |  |
| Operation |  | Recommended Materials | Why? |
| High-Speed Machining |  | Aluminum, Brass | Soft and easy to cut. |
| Precision Machining |  | Stainless Steel, Titanium | Stable and dimensionally accurate. |
| Heavy-Duty Cutting |  | Tool Steel, Inconel | High wear resistance. |
| Heat-Resistant Parts |  | Titanium, Inconel | High-temperature stability. |
| Corrosion-Resistant Components |  | Stainless Steel, Plastics | Protection against oxidation and chemicals. |

Impact of Material Properties on Machining

The properties of a material directly influence the machining process, affecting factors such as tool selection, cutting speed, feed rate, surface finish, tool wear, and overall efficiency. Understanding these impacts is crucial for optimizing CNC machining operations.

Material Properties and Machining Parameters

| Material Type | Hardness (HRC) | Machinability | Typical Cutting Speed (m/min) | Feed Rate (mm/rev) | Depth of Cut (mm) | Tool Material |
| --- | --- | --- | --- | --- | --- | --- |
| Aluminum (6061, 7075) | 15-40 | Excellent | 250 - 500 | 0.05 - 0.3 | 1 - 6 | HSS, Carbide |
| Mild Steel (A36, 1018) | 10-20 | Good | 120 - 300 | 0.05 - 0.3 | 1 - 5 | HSS, Carbide |
| Stainless Steel (304, 316) | 40-50 | Moderate | 60 - 150 | 0.02 - 0.2 | 0.5 - 3 | Carbide, Cermet |
| Tool Steel (D2, H13) | 50-65 | Poor | 40 - 100 | 0.02 - 0.1 | 0.5 - 2 | Carbide, CBN |
| Titanium (Ti-6Al-4V) | 35-45 | Difficult | 30 - 80 | 0.02 - 0.1 | 0.5 - 3 | Carbide, CBN |
| Inconel (Nickel Alloys) | 40-55 | Very Difficult | 20 - 60 | 0.01 - 0.1 | 0.5 - 2 | Carbide, CBN, Ceramic |
| Brass (C36000) | 20-30 | Excellent | 200 - 400 | 0.05 - 0.3 | 1 - 5 | HSS, Carbide |
| Plastics (Nylon, PEEK, PTFE) | <10 | Very High | 300 - 800 | 0.1 - 0.5 | 1 - 6 | HSS, Carbide |

Run Program in Single Block

Single Block Mode in CNC machining allows the machine to execute one block (line) of the program at a time. After executing a single command, the machine stops and waits for the operator to press the cycle start button to continue. This mode is mainly used for debugging, testing, and precision machining.

Steps for Running a Single Block Program in CNC

Load the CNC Program

- Ensure the correct part program is loaded into the CNC controller.

- Verify the program syntax and tool offsets.

Select ‘Single Block’ Mode

- On the CNC control panel, press the Single Block mode button.

- This mode allows step-by-step execution of the G-code.

Start Execution

- Press the Cycle Start button to execute the first block of code.

- The machine moves according to that block’s instruction and then pauses.

Check Position & Motion

- Observe the machine’s movement and toolpath.

- Verify dimensions and cutting actions before proceeding.

Continue Step-by-Step Execution

- Press Cycle Start again to move to the next line of the program.

- Repeat until the entire program is executed.

Exit Single Block Mode

- Once debugging or step-by-step checking is complete, switch to Auto Mode for continuous execution.

Activity: Run the program in single block mode and check step by step sizes of manufactured job

1. Prepare the Machine

- Ensure the CNC machine is powered ON and in ready state.

- Load the correct CNC program into the controller.

- Verify the tool offsets (G43), work offsets (G54, G55, etc.), and cutting parameters.

2. Activate Single Block Mode

- On the Siemens/Fanuc controller, press the “Single Block” button.

- This mode will allow step-by-step execution of each program line.

3. Run the Program Step-by-Step

- Press Cycle Start to execute only one block (line) of the program.

- Observe the tool movement and verify if the cutting is proceeding as expected.

- Continue pressing Cycle Start after checking each step.

4. Measure the Job Dimensions at Each Step

- Pause execution at critical machining points (e.g., roughing, finishing passes).

- Use calipers, micrometers, or gauges to measure machined features.

- Compare measured values with programmed dimensions.

- Adjust tool offsets or cutting parameters if required.

5. Verify Final Dimensions

- After the final machining step, measure all critical dimensions of the part.

- Check tolerances against the engineering drawing specifications.

- If dimensions are incorrect, modify offsets or re-run necessary operations.

6. Exit Single Block Mode & Proceed to Full Run

- If all measurements are accurate, switch the machine to Auto Mode.

- Run the program continuously for full production

SUMMERY

This module teaches learners the basic skills to use CNC milling machines. They will learn how to understand and operate the CNC control panel. Learners will also practice calculating job coordinates and setting up work and tool offsets for accurate machining. The module includes writing simple CNC programs by hand and using canned cycles for automatic machining. Learners will also use SwanSoft or CIMCO simulators to test their programs before running them on a real machine. Hands-on exercises will help them learn how to mount, align, and fix problems with jobs. By the end of the module, learners will be able to perform CNC milling operations correctly and efficiently.

FAQs

Q1. What is CNC Milling?

Ans. CNC milling is a machining process where a rotating tool removes material from a stationary workpiece using computer-controlled movements.

Q2. What are the main axes in CNC milling?

Ans. CNC milling typically operates on X, Y, and Z axes, with additional rotary axes in advanced machines.

Q3. What is the purpose of G-code in CNC milling?

Ans. G-code controls the tool movements, speed, feed, and machining operations in a CNC machine.

Q4. What is an M-code in CNC programming?

Ans. M-code commands control auxiliary functions like spindle on/off, coolant, and tool changes.

Q5. What is the difference between roughing and finishing?

Ans. Roughing removes large material quickly, while finishing provides high accuracy and surface quality

Q6. How do you load a CNC program into a Siemens controller?

Ans. Programs can be manually entered, loaded from USB, or transferred via network/DNC.

Q7. What is a work offset (G54, G55, etc.)?

Ans. A work offset sets the reference zero point for the workpiece in the CNC machine.

Q8. What is a tool offset (G43, G44)?

Ans. A tool offset compensates for tool length and diameter variations in machining.

Q9. How do you verify a CNC program before running?

Ans. Use simulation, dry run, and check tool paths for errors.

Q10. What is the function of the tool wear compensation?

Ans. It adjusts tool offsets to compensate for tool wear and maintain accuracy.

Q11. What is Single Block Mode in CNC milling?

Ans. Single Block Mode executes one line of code at a time for step-by-step verification.

Q12. What is Dry Run Mode?

Ans. Dry Run Mode simulates the program without cutting to check tool movements.

Q13. How do you set a tool offset in Siemens CNC?

Ans. Use a tool presetter or touch-off method and enter values in the tool table.

Q14. What is feed rate override?

Ans. It allows real-time manual adjustment of the feed rate during operation.

Q15. How do you check the accuracy of a machined part?

Ans. Measure using Vernier calipers, micrometers, dial gauges, or CMM.

Q16. How do you determine the correct cutting speed and feed rate?

Ans. Based on material type, tool material, and manufacturer recommendations.

Q17. What is depth of cut in CNC milling?

Ans. The thickness of material removed in a single pass of the cutter.

Q18. What is the function of coolant in CNC milling?

Ans. Coolant reduces heat, improves surface finish, and extends tool life.

Q19. What are common CNC milling operations?

Ans. Face milling, end milling, slotting, drilling, and pocket milling.

Q20. What tool materials are used in CNC milling?

Ans. HSS, carbide, ceramic, and CBN depending on the material being cut.

Q21. How do you prevent tool breakage in CNC milling?

Ans. Use the correct cutting parameters, proper coolant, and gradual depth of cut.

Q22. What are common errors in CNC milling?

Ans. Overcutting, tool breakage, chatter, incorrect offsets, and dimensional inaccuracy.

Q23. How do you avoid tool collision?

Ans. Always simulate, verify offsets, and check tool paths before running.

Q24. What are essential safety precautions in CNC milling?

Ans. Wear PPE, secure workpieces properly, and never reach inside a running machine.

Q25. How do you maintain a CNC milling machine?

Ans. Regularly clean chips, check lubrication, inspect tool holders, and calibrate offsets.

MCQ’s

1.	What is the primary function of the CNC control panel?

a. Displays tool wear	b. Controls machining operations

c. Stores workpieces	d. Measures workpiece dimensions

2.	Which coordinate system is commonly used in CNC milling?

a. Polar	b. Cartesian	c. Cylindrical	d. Spherical

3.	What is the purpose of work offset (G54, G55, etc.)?

a. Defines the tool length	b. Sets the workpiece zero point

c. Stops the spindle rotation	d. Increases feed rate

4.	How is the tool offset set in CNC milling?

a. By measuring the tool manually	b. Automatically by the machine

c. By setting G54		d. Using M-codes

5.	What does G90 represent in CNC programming?

a. Incremental positioning	b. Absolute positioning

c. Feed rate adjustment	d. Tool change

6.	What is a canned cycle in CNC milling?

a. A loop in the program	b. A predefined machining operation

c. A special tool holder	d. A type of material

7.	Which G-code is used for drilling canned cycles?

a. G73	b. G81	c. G76	d. G92

8.	Which software can be used for CNC simulation?

a. AutoCAD	b. Swansoft	c. Photoshop	d. Excel

9.	What is the function of M06 in CNC programming?

a. Spindle start	b. Tool change	c. Coolant on	d. Program end

10.	What does G01 represent in CNC milling?

a. Rapid movement		b. Linear interpolation

c. Circular interpolation	d. Machine stop

11.	Which tool is commonly used for face milling?

a. End mill	b. Ball nose cutter	c. Drill bit	d. Tap

12.	In manual CNC programming, what does “F” specify?

a. Feed rate	b. Spindle speed	c. Tool number	d. Coolant function

13.	What does G17 specify in CNC milling?

a. XY plane selection	b. XZ plane selection

c. YZ plane selection	d. Work offset

14.	How do you verify a CNC program before actual machining?

a. Direct execution		b. Dry run or simulation

c. Random tool selection	d. Increasing spindle speed

15.	What is the primary function of the CIMCO simulator?

a. Image editing		b. CNC program simulation

c. Electrical circuit design	d. Workpiece measurement

16.	Which command is used for rapid tool movement?

a. G01	b. G02	c. G00	d. G90

17.	What is the significance of G43 in CNC milling?

a. Tool length compensation	b. Circular interpolation

c. Work coordinate setting	d. Spindle speed control

18.	What does an M30 command do in a CNC program?

a. Stops the spindle		b. Ends and resets the program

c. Changes the tool		d. Turns coolant off

19.	Which of the following is a method for job alignment in CNC milling?

a. Using a dial indicator	b. Random placement

c. Manual hand-fitting	d. Press-fit method

20.	What is the main advantage of simulation before machining?

a. Reduces tool wear and errors	b. Increases machine vibration

c. Eliminates tool offsets	d. Speeds up machine start-up

Answer Keys

| 1. | b | 2. | b | 3. | b | 4. | a |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5. | b | 6. | b | 7. | b | 8. | b |
| 9. | b | 10. | b | 11. | a | 12. | a |
| 13. | a | 14. | b | 15. | b | 16. | c |
| 17. | b | A | a | 19. | a | 20. | a |