Learning Outcomes

Trainee will be able to:

- Rewind EDM Wire for CNC operations.

- Perform routine maintenance (Check Dielectric fluid, Adjust Fixture/Holding device etc.) to prepare the machine for operations.

- Draw the sketch as per drawing.

- Make tool path on CNC wire cut controller.

- Check simulation of the tool path and modify tool path for errors.

- Select Machining parameters that may include wire offset, wire speed and current settings etc.

- Set up and align the job on the holding device at workstation.

- Set up the machine to machine zero position according to the job requirement.

- Set Wire and feed roller according to required sequence of operations.

- Start the machine, adjust parameters material, finish and accuracy desired.

- Run the program

- Check the cutting path, finish and accuracy after completion.

- Finish and remove the job after completion from holding device.

- Retract the program for machine zero.

|  | Prepare CNC Wire Cut for operations |
| --- | --- |

Trainee will be able to

- Rewind EDM Wire for CNC operations. Perform routine maintenance (Check Dielectric fluid, Adjust Fixture/Holding device etc.) to prepare the machine for operations.

PHENOMENON OF EDM

Introduction

Electrical discharge machining, die sinking or wire cut has proved valuable in the machining of super tough electrically conductive materials such as the new space-age alloys. These metals would have been difficult to machine by conventional methods. EDM has made it relatively simple to machine intricate shapes that would be impossible to produce with conventional cutting tools. This process is continually finding more applications in the metal-cutting industry. It is used extensively in the plastics industry to produce cavities of almost any shape in the molds.

HISTORY OF EDM

The circuit for making metallic powder in the reference published in Germany in 1919 is indicated. (1)

Discharge gap

Metal sticks

The metal of both ends becomes powder and precipitate

Existing electrical discharge machines were designed based on the research conducted by Lazarenko, a Russian scientist in 1943. Lazarenko’s achievement increased the application of capacitor charge/discharge circuits in processing metallic materials, into products desired shapes. It was not until Lazarenko’s days that the perforating of hard metallic substances – with small holes and holes of different shapes - become possible by manufacturing electrode with soft copper and brass. However, applications were limited to specific areas, which were blamed on extremely low machining rate and high electrode wear ratio.

Electrical discharge processing began to score remarkable progress after a transistorized switching element was installed in the current circuit. Transistorized circuits proved very effective not only to increase machining speed, but also to secure electrical conditions featuring the minimal electrode wear ratio.

Servo mechanism

Gap controller

Electrode

Workpiece

THE MAIN PARTS OF WIRE EDM  MACHINE

<<Machine control device>>

<<Body>>

<<Dielectric fluid unit>>

U axis

V axis

X axis

Y axis

Z axis

Filter

ion exchange resin

Resistively control

Power supply unit

CNC unit

CPU

The structure of the X and Y axis of wire electrical discharge machine is almost the same as that of electrical discharge machine (EDM sinker). Since a wire electrical discharge machine uses wire as an electrode, the following equipment for wire control is installed on the machine-column.

- Wire feed unit: Controls wire feed rate and wire tension

- Wire discharge unit: Collects used wire and controls wire speed.

- Wire guide: Holds wire passes and flushes machining fluid.

- Taper cutting unit: Auxiliary axis (U and V) unit for taper machining.

Machining tank

Electrical discharge machines also use machining fluid in order to ensure insulation resistance gap. When pure water is used as machining fluid, it is normally supplied into the gap only. In such cases, the work tank only serves as a cover.

On the contrary, when oil is used as machining fluid, the immersion machining method whereby discharge is performed in the fluid is used to prevent fire. In such cases, a work tank with the same structure as for an electrical discharge machine is used.

Today, the immersion machining method is also used in water machining in order to improve the machining precision and passively the machining of complex shapes. Accordingly, more and more wire electrical discharge machines are incorporating a work tank.

Note: In the case of electrical oil discharge machining, oil that is low in volatility and flammability and high in specific heat capacity is used. The heat is absorbed into the fluid even when discharged has occurred in the fluid and its temperature does not reach the inflammation level. However, temperature per volume including vaporized oil around the fluid level is apt to rise. Therefore, such oil easily catches fire. Fire extinguishing equipment is installed for emergencies.

The inflammation point of electrical discharge machining oil is generally around 100°C.

Dielectric tank

This is a reserve tank from which machining fluid is supplied. Its basic structure including a fluid feed pump and a filter is the same as that of the dielectric tank for an electrical discharge machine. As a result of an increasing demand for high-precision machining, more and more wire electrical discharge machines are using a machining fluid cooling system to prevent increase in fluid temperature.

CONTROL OF WIRE ELECTRODE

Control of the wire electrode is so important for wire electrical discharge machines as to affect machining quality.

Wire tension

Tension must be applied to the wire before use. If wire, used as tool, for wire electrical discharge machine is slack, neither precision machining nor control of the discharge gap is possible. Wire constantly receives delicate vibration under discharge pressure during machining while being supported by upper and lower wire guides. For this reason, the wire that is the lower in tension has the larger amplitude, thereby affecting precision. As a result, a draw down occurs in the vertical center of the workpiece during machining. This phenomenon is called “drum”.

On the contrary, if tension is too high, drum hardly occurs. However, wire is most likely to break when current flows due to changes in the discharge mode. Some high-class models incorporate a tension servo-mechanism to stabilize tension since fluctuations in tension affect machining precision.

Wire speed (transporting speed)

As wire is very thin that is normally 0.2mm in diameter, it will wear out and break if discharge occurs several times in the same position. In order to avoid this problem, new wire must be supplied continuously to the surface being machined by feeding wire at a fixed speed.

If wire feed speed is too slow, wire may break due to high machining current during roughing or precision. Also it may deteriorate the wire during finishing. On the contrary, if wire feed speed is to fast, it results in waste, because wire is disposed of after use. Thereby making long-time machining impossible depending on the volume of wire bobbin.

Wire guides

These are the guides which are above and below the workpiece supports wire. These guides are of so important for wire electrical discharge machines as to directly affect the precision. Although wire is made of soft brass, it travels continuously. Therefore, sapphires or diamonds are used for guides supporting the wire due to their hardness and long life. Guide types can be classified according to their profile.

Guides having a V-shaped profile

V-shape is advantageous in term of production cost. However, if machining is performed toward the closed edge of V, wire receives discharge pressure from the machined surface, and the guide contact pressure of the wire lowers, thereby deteriorating machining precision.

2) Guides having a round profile

In the case of a round guide, since the contact surface of both the guide and the wire is circular, contact pressure is low and stable in all direction, permitting machining in all directions. However, these guides make the production of dies difficult, and attention must be paid to the clearance between the wire and the die hole during routine maintenance.

CNC Wire Cut Holding Devices:

The holding devices in a CNC wire cutting machine are designed to securely hold the workpiece in place while it's being cut. Some common holding devices include:

- Workpiece Fixtures: These are specialized fixtures or clamps used to hold the material securely during the cutting process. They ensure that the material does not shift, which could cause inaccuracies in the cut. Fixtures are custom-designed based on the part geometry.

- Electrode Holders: In wire EDM, the wire is the cutting tool. The electrode holder keeps the wire in place and ensures the correct tension, alignment, and distance from the workpiece.

- Workpiece Clamps: These are adjustable clamps that hold the workpiece securely on the machine's table. Clamps can either be mechanical or pneumatic, depending on the size and material of the workpiece.

- Magnetic Chuck: For ferromagnetic materials, a magnetic chuck is sometimes used to hold the workpiece in place. The magnetic chuck provides strong, even holding force without mechanical clamping.

- Vacuum Holders: Some machines may use vacuum systems for holding workpieces, especially for delicate or thin materials. These systems use suction to hold the part steady during the machining process.

- Custom Jigs: In cases where a workpiece has an unusual shape, custom jigs or holders are created to ensure the part is held securely.

Routine Maintenance for CNC Wire Cut Machines:

Regular maintenance is essential to keep the wire cut EDM machine running smoothly, extend its lifespan, and maintain high precision in cuts. Some important routine maintenance checks include:

Wire Tension:

Check the tension of the wire periodically to ensure it's neither too loose nor too tight, as this can affect the accuracy of the cut.

Wire Spool and Replacement:

Monitor the condition of the wire spool. Replace the wire when it is worn, frayed, or damaged to ensure proper cutting performance.

Tank and Fluid Levels:

The dielectric fluid (usually deionized water) helps to flush away debris and provides insulation during the cutting process. Check the fluid level and the quality of the fluid regularly, replacing it when necessary.

The fluid filter should also be cleaned or replaced to prevent contamination from affecting the cutting process.

Electrode and Nozzle Condition:

Inspect the electrode and nozzle regularly for wear and tear. Replace them if they show signs of damage, as they play a critical role in maintaining the quality of the cut.

Guide and Rollers:

The wire passes through guides and rollers that can become worn out over time. Clean and inspect them regularly to ensure that the wire runs smoothly and remains aligned.

Cooling System:

The cooling system (if present) should be checked for leaks, coolant levels, and efficiency. Clean or replace filters as necessary to keep the system in good working condition.

Electrical Connections:

Check electrical connections for signs of wear, corrosion, or damage. Ensure that all cables and components are securely attached to prevent power loss or inconsistent operation.

Lubrication:

Regularly lubricate moving parts, such as the linear guides and ball screws, to ensure smooth motion and prevent excessive wear.

Machine Calibration:

Perform periodic calibration checks to ensure that the machine’s axes are aligned and that the cutting process remains accurate.

Air Supply (if applicable):

If the machine uses an air supply system for cooling or cleaning purposes, check the air compressor and filters to ensure proper airflow and pressure.

Inspection of the Power Supply and Transformer:

Check the power supply for stability, and ensure that the transformer is functioning correctly to avoid electrical issues.

Routine maintenance and the proper use of holding devices will ensure that the CNC wire cut EDM machine continues to produce high-quality parts with minimal downtime.

Material Required

- CNC Wire cut Machine

- EDM wire Role

- Conducting Material for Cutting

Learning Place

- Class room

- Workshop

|  | Generate the program |
| --- | --- |

Trainee will be able to:

- Draw the sketch as per drawing.

- Make tool path on CNC wire cut controller.

- Check simulation of the tool path and modify tool path for errors.

- Select Machining parameters that may include wire offset, wire speed and current settings etc.

Creating an AutoCAD sketch for a CNC wire cut machine (EDM) and entering it into the MCU (Main Control Unit) requires both CAD design and CNC programming knowledge.Here’s a step-by-step breakdown of the process:

Step 1: Drawing the Part in AutoCAD

First, you need to create a sketch of the part that will be cut by the CNC wire EDM. Here’s how to do that:

Open AutoCAD and create a new drawing file.

Sketch the geometry of the part you want to cut using AutoCAD’s drawing tools (such as lines, arcs, and circles). Make sure the geometry is to scale.

Use line and arc tools for precise geometry.

Define the correct dimensions for the part according to the specifications.

Ensure that the geometry is closed if you are cutting along a perimeter. The sketch should be a continuous loop.

Add reference points or key positions to help define the cutting path, especially for the wire EDM machine.

Step 2: Save the AutoCAD Drawing

Once your part is drawn in AutoCAD:

Save the drawing in a format compatible with your CNC machine’s controller (e.g., DXF or DWG).

Step 3: Enter the AutoCAD Drawing into the MCU (Main Control Unit)

The MCU is where the CNC wirecut machine receives and interprets the tool path information.

Convert the AutoCAD drawing into a format readable by the CNC wire cut machine. This is typically done by saving the file as a DXF file.

In AutoCAD: File > Save As > DXF to save the file in a suitable format.

Transfer the file to the CNC controller (MCU):

Some machines accept files via USB, Ethernet, or through a direct connection

Check your wire EDM machine's user manual to confirm how to transfer files to the MCU.

Load the DXF file into the MCU using the CNC machine’s interface. Usually, this is done through a software program or directly via the controller's interface.

Step 4: Define the Start Point and Tool Path

Now, you’ll need to set up the start point and define the cutting path in the MCU. Here's how this typically works:

Set the machine's starting point in the software or through the controller’s interface.

This is often a reference point such as a corner or the center of the part, which will serve as the origin for the cutting process.

Define the cutting path:

The wire cut EDM follows a programmed tool path based on the geometry of the part. This is determined by the DXF file you imported.

In the CNC controller, you’ll specify how the wire moves along the geometry (usually a series of straight lines or arcs).

Step 5: Generate G-Codes and M-Codes

To generate the necessary G-codes and M-codes, you need to either use CAM software to post-process the tool path or manually program them. Some CNC systems have built-in software that can generate the codes from the DXF file.

Using CAM software (e.g., Mastercam, SolidWorks CAM, or other CNC software):

Open the DXF file and set up the toolpath for the wire EDM machine.

The software will automatically generate the G-codes and M-codes that are required for the cutting process.

Material Required

- White board

- Marker

- Part Drawing

Learning Place

- Class room

- Workshop

|  | Mount the job / Work Offset |
| --- | --- |

Trainee must be able to:

Set up and align the job on the holding device at workstation.

Set up the machine to machine zero position according to the job requirement.

Set Wire and feed roller according to required sequence of operations

Set Up and Align the Job on the Holding Device at Workstation:

The first step is to ensure the workpiece is properly aligned and securely fastened to the holding device to prevent movement during the cutting process.

Inspect the Workpiece:

Ensure the workpiece is clean, free from any debris or contaminants that could interfere with the cutting process.

Measure the part to ensure it conforms to the specifications.

Prepare the Holding Device:

Use the appropriate holding device (clamps, fixtures, magnetic chuck, or vacuum chuck) based on the material and shape of the workpiece.

Check for proper alignment of the holding device with the machine's worktable. The workpiece should be placed precisely in the area where the cutting will occur.

Position the Workpiece:

Place the workpiece carefully onto the fixture or holding device on the machine's table.

Use precision instruments like dial indicators or height gauges to ensure the part is aligned to the machine's axes (X, Y, and Z).

Secure the Workpiece:

Ensure that the workpiece is securely fastened, with minimal chances of shifting during machining.

Use clamps, screws, or magnetic holds to ensure the workpiece is stable.

Set Up the Machine to Machine Zero Position According to the Job Requirement:

The machine zero (home position) is where the machine’s reference point is established. This position is critical to ensure that the cutting path is correct and the dimensions are accurate.

Steps:

Determine the Machine Zero:

Identify the starting point (machine zero) for the job. This is typically the corner or center of the workpiece, depending on how the part is designed and how the program has been written.

This zero position could be set from the machine’s fixture (e.g., the corner of the fixture) or from a known feature on the workpiece.

Move the Wire to the Zero Position:

Manually or automatically move the wire to the reference point. This can be done by jogging the machine in the X, Y, and Z directions, using the machine’s controls.

Use the machine's control panel to move the wire until it is exactly at the zero position for the workpiece.

Align the Wire Electrode:

Align the wire with the starting point of the cutting path on the workpiece. You can use the machine’s visual system (e.g., crosshairs or laser alignment) or manually move the wire to the point.

Set the Machine Zero on the Controller:

On the machine's control unit, press the "Set Zero" or "Zero Point" button to establish this as the starting reference position.

Ensure the machine registers this position as the zero for the upcoming toolpath.

Verify the Zero Position:

Double-check the machine zero position by moving the wire away and then returning to the set zero position.

If needed, fine-tune the position to ensure accuracy before proceeding with cutting.

Conducting Materials

CNC wire EDM (Electrical Discharge Machining) machines are designed to cut conductive materials by using a wire electrode and an electric discharge process. The wire electrode is typically made of brass, copper, or other conductive materials, while the material being cut must also be conductive for the process to work effectively. Below is an overview of common conductive materials used in CNC wire cutting, as well as some material considerations for the cutting process.

Titanium

- Copper and alloys

- Aluminum and aluminum alloys

- Brass

- Tungsten etc

|  | Run the program |
| --- | --- |

Trainee must be able to

Start the machine, adjust parameters material, finish and accuracy desired.

Run the program

Check the cutting path, finish and accuracy after completion.

Finish and remove the job after completion from holding device.

Retract the program for machine zero.

Program Setup:

Input the desired cutting path and parameters into the machine's control system, often using CAD/CAM software

Start Cutting: Initiate the cutting process. The machine will automatically follow the programmed path, using electrical discharges to cut the material. ​

Monitoring and Inspection:

Observe the Process: Monitor the cutting operation to ensure its proceeding correctly. ​

Check Accuracy and Finish: After completion, inspect the cut for dimensional accuracy and surface quality. ​

Retrieve the Work piece: Once cutting is complete, carefully remove the work piece from the machine. ​

Retract the Program: Reset the machine to its home position or machine zero, preparing it for the next operation. ​

Material Required

- CNC wire cut machine

- Tooling

- Blank

- PPEs

Learning Place

Workshop

Safety Precautions:

- Always wear appropriate personal protective equipment (PPE) during operation. ​

- Ensure the work area is clean and free from obstructions. ​

- Regularly maintain the machine to keep it in optimal working condition.