|  | Maintian Machines and Tools |
| --- | --- |

Learning Outcomes:

After Completion of this learning module, trainee will be able to:

Trainee will be able to:

- Prepare checklist for daily cleanliness of the workplace and housekeeping.

- Select appropriate methods and techniques for cleanliness and housekeeping from available facilities.

- Apply appropriate cleaning and housekeeping process

- Maintain daily checklist of cleanliness and housekeeping.

- Place all the tools & material in proper place to ensure safe work

- Prepare daily, weekly and monthly preventive maintenance and checklist schedule for oiling and greasing.

- Identify sounds of bearings, oil level, oil leakage, faulty/damaged/worn machine tool parts.

- Perform maintenance of machines & tools.

- Observe problems and carry out routine maintenance   as per given instructions and schedules

- Check condition and level of coolant, lubricant and cutting oil

- Report faults and problems of the machines, if not controllable, to the person concerned (daily, weekly and monthly basis)

|  | Perform General Cleanliness and Housekeeping |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Prepair checklist for daily cealnliness of the workplace and housekeeping.

- Select appropriate methods and techniques for cleanliness and housekeeping from available facilities.

- Apply appropriate cleaning and housekeeping process.

- Maitain daily checklist of cleanliness and housekeeping.

- Place all the tools & material in proper place to ensure safe work.

Introduction to Maintenance and Its Techniques

Introduction  Definition

Taking care of a production plant or machinery in such a way that it does not break down is called "maintenance". This work includes timely replacement of worn parts and routine oiling or greasing. This process is repeated at intervals during the working life of the machine or plant. So that the machine continues to work in proper condition

Objective of Maintenance

- Extending the life of the machine

- Maintain the quality of Product

- Maintain the performance of products

The purpose of machine tool or plant maintenance is to keep machine tools or plant faults to a minimum level so that they continue to operate in good working condition at minimum cost.

The primary duty of the maintenance department of every organization is to ensure that the organization's machines, resources, plants, buildings, and other facilities are in working condition so that the organization can be profitable.

The maintenance department also has the duty to maintain and maintain the working condition of safety devices that help in the event of any emergency. It is its duty to keep fire extinguishers, stairs, and other safety devices in good condition at all times.

Difference b/w Maintenance and Repair

Maintenance

Maintenance is the act of repairing mechanical or electrical parts before they become damaged or break. For example, checking the oil level of a machine daily, greasing machine parts, etc.

Repair

If a part of a machine gets damaged, broken, or is not working properly, and it can be repaired or replaced, this process is called repair. For example, a bearing getting damaged, a part of the machine breaking during an accident, or a wire getting torn, etc.

Types of Maintenance Techniques

- Autonomous Maintenance

- Corrective Maintenance

- Preventive Maintenance

- Predictive Maintenance

Autonomous Maintenance

Autonomous maintenance refers to the process where an operator checks and maintains all the parts of a machine on a daily basis. Examples of this include:

- Opening and cleaning the machine's parts

- Checking the oil level of the machine

- Greasing the machine parts

- Tightening the nuts and bolts of the machine, etc.

Corrective Maintenance:

Corrective maintenance is the process of identifying and fixing the problem in a malfunctioning machine, and restoring it to its proper working condition. This is referred to as Corrective Maintenance.

In industries, when planning or preventive maintenance is carried out, certain specific parts of machines start failing repeatedly. These parts are often overlooked during preventive maintenance or while performing routine maintenance. As a result, the machine becomes prone to failures at any time. To fix this issue, the maintenance department is called upon to correct the problem. This process is known as Corrective Maintenance.

Preventive Maintenance:

Preventive Maintenance is a system of maintenance where the aim of routine planned maintenance or preventive maintenance is to minimize sudden breakdowns. This method is carried out according to a set schedule. In this method, weak points of machines or plants, such as bearings, sliding surfaces, or mechanical components that endure vibrations or shocks, are regularly monitored. This approach helps to prevent major breakdowns by addressing small repairs in time. The core concept of preventive maintenance is to better than treatment.

Objectives of Preventive Maintenance:

- By maintaining machinery or other resources at regular intervals, protect the setup from repairs and wear and tear that could affect production.

- Use preventive maintenance to eliminate major faults in machines or other resources at an early stage.

- Preventive maintenance is adopted in about 75% of industries, tailored to each company's specific operational methods, to keep machinery in optimal condition at minimal cost.

- Keep the plant machinery in a functional condition at all times through preventive maintenance.

- Maintain the production from machinery or plants at the highest possible level.

- Keep the plant’s performance and health at a good level.

- Ensure that the plant or machinery is always in an operational condition.

- Minimize the damage caused by emergency repairs.

- Ensure the safety and well-being of labor and workers.

Predictive Maintenance:

Predictive maintenance is a relatively new maintenance approach in which the user attempts to detect faults based on sensory input. For example, if a specific part of the machine gets unusually hot, it indicates a potential problem, which can then be identified and corrected. This process is referred to as predictive maintenance.

Various methods are used to detect the cause of a machine or plant’s condition, sound, temperature, or production issues, such as:

- Sound-sensitive instruments

- Vibration sensors

- Amplified meters

- Gauges measuring pressure, temperature, resistance, and degradation.

These tools help predict potential faults before they cause significant damage.

Daily Cleanliness & Housekeeping Checklist

1. General Workplace Areas

- Floors: Sweep, mop, or vacuum all common areas and corridors.

- Workstations: Clean desks, chairs, and computer surfaces. Wipe down keyboards, mice, and other equipment.

- Windows: Clean glass surfaces and windows, inside and outside if applicable.

- Trash Disposal: Empty trash bins and recycling bins; replace liners.

- Restrooms: Ensure toilets are clean and stocked with necessary supplies (toilet paper, soap, etc.).

- Hallways & Stairs: Ensure clear pathways and remove any debris or hazards.

Office Equipment & Furniture

- Desks and Tables: Wipe down all flat surfaces.

- Computers and Screens: Clean monitor screens with appropriate cleaner, dust keyboards, and other office devices.

- Lighting: Ensure all lights are working and fixtures are clean.

Kitchen Area (If Applicable)

- Counters and Surfaces: Wipe down counters, sinks, and tables.

- Appliances: Clean coffee machines, microwave, fridge, etc.

- Utensils & Dishes: Ensure that dishes are washed and put away, and utensils are clean.

Safety Checks

- Exit Routes: Ensure all fire exits and routes are clear of obstructions.

- Electrical Appliances: Check that electrical equipment is turned off and unplugged when not in use, ensuring safety.

General Housekeeping

- Tools & Equipment: Store tools and materials in their proper places.

- Organization: Maintain an organized storage area for materials, equipment, and supplies.

- Personal Items: Remind staff to keep their personal items organized and avoid clutter.

2. Methods and Techniques for Cleanliness and Housekeeping

Sweep, Mop, and Vacuum Floors:

- Use appropriate floor cleaners for specific surfaces (e.g., hardwood, tiles, carpets).

- Consider using a microfiber mop for better dust retention.

Dusting:

- Use microfiber cloths to trap dust on furniture and electronics.

- Pay attention to high-touch surfaces like doorknobs, light switches, and handrails.

Sanitizing:

- Use appropriate disinfectants, especially in high-contact areas (like doorknobs, shared workspaces, and kitchens).

- Ensure that the products used are safe and effective on surfaces.

Trash & Recycling:

- Implement clear labeling for waste and recycling bins.

- Ensure that bins are emptied regularly to avoid odor buildup.

Restroom Cleaning:

- Use disinfectant cleaners for toilets, sinks, and other bathroom fixtures.

- Replenish soap and paper towels regularly to ensure availability.

Organizing Tools & Equipment:

- Use toolboxes, shelves, or cabinets for proper storage.

- Label shelves or areas to ensure easy identification and organization.

3. Applying Appropriate Cleaning & Housekeeping Processes

Daily Review of the Checklist:

- Ensure that the housekeeping and cleanliness tasks are checked off daily.

- Assign specific tasks to employees or housekeeping staff.

Frequent Inspections:

- Conduct quick daily inspections to ensure cleanliness standards are being maintained.

- Spot-check workstations, kitchen areas, and restrooms.

Adhere to Safety Guidelines:

- Always use the correct cleaning agents and personal protective equipment (PPE).

- Properly store cleaning chemicals to prevent accidents.

4. Maintaining the Daily Checklist

Record the Completed Tasks:

- Create a checklist document (physical or digital) and tick off tasks as they are completed.

- Assign a designated person or team responsible for daily cleanliness.

Periodic Review:

- At the end of the week, review the checklist to identify any patterns or areas requiring extra attention.

- Address recurring issues to ensure continuous improvement.

5. Placement of Tools & Materials for Safe Work

Tools Storage:

- Store cleaning equipment such as mops, brooms, dusters, and cleaning solutions in a designated, easily accessible area.

- Keep all cleaning agents labeled and stored properly to avoid accidents.

Labeling:

- Clearly label all cleaning products and tools to ensure everyone knows where they belong.

- Ensure hazardous materials are stored separately and in compliance with safety standards.

Safety Measures:

- Keep aisles, hallways, and workspaces clear of clutter to prevent accidents.

- Regularly inspect safety equipment (e.g., fire extinguishers, first-aid kits).

- By following these steps and maintaining a consistent daily checklist, you can ensure that the workplace remains clean, organized, and safe for everyone.

9.1.5 Material Required

- CNC Lathe

- CNC Milling

- CNC EDM Wire cut

- Maintenance Box with following tools (Flat nose plier, wire cutter, screw driver, straight and Philips head tips, screw driver kit of different size bits, Grip plier, WD-40, Oil cane, Greece gun,

- Greece, Emery paper.

- Steel wire brush

- Hacksaw Blade,

- Electrical pvc tapes

- Plastic Scrapper,

- Cotton rags,

- Truncated, cutting oil,

- Lubricating oil

- Learning Place

- Classroom

- Workshop

|  | Perform Preventive Maintenance |
| --- | --- |

Learning Outcomes

After completion of this Learning Unit, trainee will be able to:

- Prepare daily, weekly and monthly preventive maintenance and checklist schedule for oiling and greasing.

- Identify sounds of bearings, oil level, oil leakage, faulty/damaged/worn machine tool parts.

- Perform maintenance of machines & tools.

- Observe problems and carry out routine maintenance   as per given instructions and schedules

- Check condition and level of coolant, lubricant and cutting oil

- Report faults and problems of the machines, if not controllable, to the person concerned (daily, weekly and monthly basis.

Introduction

Condition Monitoring is a process in which the parameters of a machine, such as vibration, temperature, current, oil, etc., are monitored. The purpose of this process is to assess the machine's condition and predict potential failures so that appropriate actions can be taken in a timely manner.

MCM Techniques

There are various techniques for Condition Monitoring (MCM), including:

Periodic Monitoring:

- This method involves collecting and reviewing data related to machine maintenance after a specific time interval. The data is used to assess the machine's condition and take necessary actions.

Examples:

- Checking the machine's various parameters periodically.

- Regularly checking the oil level of the machine.

Continuous Monitoring:

- In this method, data related to the machine is collected and reviewed continuously in real-time. This allows for immediate detection of any failures, enabling quick corrective actions. The data collected through continuous monitoring can be used for corrective or planned maintenance.

This text explains two important methods of Condition Monitoring for machines. If you need more clarification or additional information, feel free to ask!

Lubricants and Lubrications

Definition

Lubrication:

Machines consist of numerous moving parts, and their movement generates friction and heat. To reduce this heat, a lubrication system is necessary. This system forms an oil film between the moving parts, preventing direct metal-to-metal contact and minimizing both heat and friction. Parts that do not receive adequate lubrication can overheat and melt, causing them to seize.

Benefits of Lubrication:

Lubrication provides the following six benefits:

- Reduction in Friction: When two solid bodies move against each other in opposite directions, friction and heat are generated because the raised surfaces of both bodies act against each other. Even though the parts may appear smooth to the naked eye, if we observe their surfaces under a microscope, we may see raised surfaces. Even with smooth surfaces, friction occurs when solid parts move together. Therefore, to reduce this friction, it is essential to create an oil film between the surfaces.

- Cooling: In machines, the movement of parts and cutting generates a significant amount of heat. Lubricating oil absorbs this heat and returns to the sump to cool down. However, in some machines, oil coolers are also used to further cool the oil.

- Sealing: An example of this is the clearance between the piston and the cylinder, which is referred to as piston clearance. A thin layer of oil fills this clearance between the piston and the cylinder, preventing the escape of compression pressure and burned gases. The sealing properties of lubricating oil are very important.

- Rust Prevention: Since the thin layer of oil covers the parts, it protects them from dirt, water, carbon, etc., preventing rust. Additionally, anti-rust additives are included in the oil to protect machine parts from chemical effects caused by carbon dioxide, water, carbon monoxide, and nitrogen oxides.

- Cleaning: In machines, oil is fed under pressure through a pump, and as it circulates between the moving parts, it carries metal particles, dust, oxides, hydrocarbons, and other debris with it, keeping the friction surfaces clean. Contaminants remain in the oil filter, and when the oil is recirculated, it is completely clean. To improve the cleaning process, detergent additives are included in the oil.

9.2.4 Use of Chemicals:

Generally, the following chemicals are added to oil:

- Anti-Rust: To prevent rust.

- Anti-Corrosion: To maintain the internal structure of the chemicals.

- Anti-Foaming: To minimize the formation of foam.

- Detergent: To clean carbon deposits.

- Viscosity Improver: To maintain the viscosity of the oil.

- Anti-Dispersant: To keep carbon and other necessary particles suspended.

Various Terms Related to Oil Usage:

- Viscosity: Viscosity refers to the resistance to flow in any liquid.

- Viscosity Index: The changes in the viscosity of an oil due to temperature variations are called the viscosity index.

Oil Viscosity and Its Impact:

- Thicker oil creates resistance to flow, while thinner oil flows more easily. Therefore, it is better to use thinner oil in cold weather, as it flows more easily and efficiently lubricates the moving parts. On the other hand, in hot weather, using thicker oil is more beneficial because it ensures that the oil flows properly through the moving parts without thinning out too quickly.

- The viscosity of oil is tested and graded by organizations like the Society of Automotive Engineers (SAE), which assigns a number to the oil based on its thickness. The higher the number, the thicker the oil. The American Petroleum Institute (API) is also an institution involved in the grading of oil based on its viscosity.

Types of Lubricants Oil:

Types:

- Motor Oil

- 3-in-1 Oil

- Sewing Machine Oil

- Bar and Chain Oil

Uses:

- Used in joints

- Used in bearings

- For tool protection

- To keep blades sharp

Grease:

Types:

- White Lithium Grease

- Marine Grease

- Silicon Grease

Fault Diagnosis and Repair

Fault Diagnosis:

- Oil Leakage Problem

- Spool Jams

- Coil Burns Out

Repair:

- Change the oil ring

- Service the hydraulic valve

- Replace the coil

Daily Preventive Maintenance Checklist:

- General Machine Maintenance

Sound Check:

- Listen for unusual sounds from bearings (e.g., grinding or squealing noises).

- Check for vibrations that might indicate misalignment or wear in the machine.

Oil and Lubricant Levels:

- Oil Levels: Ensure the oil levels in the machines are adequate.

- Coolant: Verify coolant levels and refill as needed.

- Lubricants: Ensure sufficient lubrication on moving parts (gears, bearings).

- Cutting Oil: Check the level and condition of the cutting oil. Refill if necessary.

Inspect for Leaks:

- Check for any oil or coolant leaks around the machine.

- Inspect hoses and seals for damage.

Condition of Machine Parts:

- Check for any visible signs of wear or damage to critical machine components (e.g., tools, gears, belts).

- Inspect tool parts for damage or excessive wear.

- Ensure the oil filter is clean and properly installed.

General Cleaning:

- Clean the machine surfaces to remove debris, dust, and excess grease/oil.

- Ensure that coolant and oil collection systems are free from blockages.

Reporting and Documentation

- Log Issues: Record any issues like low oil, leaks, or damaged parts in the maintenance log.

- Notify: Report any major faults or issues that cannot be fixed immediately to the supervisor or maintenance team.

Weekly Preventive Maintenance Checklist:

Machine Inspection and Cleaning:

Sound and Vibration Check:

- Perform a thorough inspection of the machine’s bearings, motors, and shafts. Listen for any abnormal sounds, such as grinding, squeaking, or excessive noise.

- Check vibration levels and ensure that no misalignment is causing stress on any parts.

Oil and Lubricant System:

- Check and top off oil levels for all machines.

- Inspect oil filters for cleanliness and change if necessary.

- Ensure proper distribution of lubrication to all moving parts (especially bearings).

Check for Leaks:

- Inspect all hoses, seals, and gaskets for oil or coolant leaks.

- Tighten any loose connections, and replace worn seals.

Condition of Moving Parts:

- Inspect gears, shafts, and bearings for wear and tear. Replace any damaged or excessively worn parts.

- Check the tension of belts and chains, and adjust them as needed.

Coolant and Cutting Oil:

- Check coolant levels and concentration (use appropriate test strips for coolant mix).

- Check the cleanliness of the coolant; replace if necessary to prevent contamination.

- Check cutting oil levels and cleanliness. Refill and replace if it has become thick or contaminated.

Report & Log:

- Log maintenance activities and any findings into the maintenance report.

- Report any issues requiring immediate attention to the supervisor.

Monthly Preventive Maintenance Checklist:

Detailed Machine Inspection:

Machine Disassembly and Cleaning:

- Perform a deep cleaning of the machine, including parts that are difficult to access on a daily or weekly basis.

- Clean the oil reservoir, replace oil filters, and inspect for any sludge buildup.

- Check all electrical connections and wiring for wear, damage, or fraying.

Full Lubrication System Check:

- Ensure the proper functioning of all grease guns and oil dispensers.

- Check grease levels in the bearings, gears, and lubrication points.

- Apply grease to the lubrication points, including guideways, sliding surfaces, and other friction zones.

Check for Faulty/Damaged Parts:

- Inspect the machine for any damaged or excessively worn tool parts (e.g., cutting tools, jigs).

- Examine the condition of bearings, bushings, and seals; replace any worn components.

Electrical and Hydraulic System Check:

- Inspect the hydraulic system for fluid levels, pressure, and leaks.

- Check electrical components for signs of wear or damage, including switches and sensors.

Coolant and Cutting Oil:

- Test the condition of the coolant and cutting oil. Clean or replace as necessary.

- Ensure coolant filtration systems are functioning correctly and are free of contaminants.

Calibrate Machines:

- Perform any necessary machine calibration to ensure they are running within the required tolerances.

Report & Log:

- Complete a full report of the maintenance conducted.

- Log any findings, including items needing follow-up or repair.

- Inform the supervisor about any non-routine issues or parts that need replacing soon.

Identifying Sounds, Oil Levels, and Faulty Parts:

Sounds to Listen For:

- Bearings: Grinding, screeching, or whining noises can indicate worn or dry bearings.

- Motors: A high-pitched or irregular humming sound may suggest motor misalignment or failure.

- General Machine Noise: Clanking or knocking noises may signal loose or damaged components.

Oil Level & Leakage:

- Low Oil Level: If the oil levels are low, topping up immediately will prevent damage from friction.

- Oil Leaks: Inspect around hoses, seals, and connections for leaks that could lead to unsafe operating conditions.

Faulty/Damaged/Worn Machine Tool Parts:

- Worn Parts: Check for visible wear, such as rounding edges on cutting tools or worn-out bearings.

- Damaged Tools: Look for chips, cracks, or discoloration on tools and replace them when necessary.

- Broken Components: Check parts like belts, pulleys, or gears for signs of cracks or fractures.

Reporting Faults and Problems:

- Daily: Any minor issues like low oil or noise should be logged and reported to the relevant team members for action.

- Weekly: Report more significant issues or recurring problems that need additional investigation.

- Monthly: For major faults or if a part requires ordering or extended downtime, inform the concerned person (e.g., supervisor or maintenance manager) immediately.

- By following this daily, weekly, and monthly maintenance schedule, you will ensure the machines and tools are always in optimal working condition, minimizing the risk of unexpected breakdowns and maximizing efficiency.

Material Required

- CNC Lathe

- CNC EDM Wirecut

- CNC Milling

Learning Place

- Workshop