National Competency Standards Level 3

Assistant Robotics Technician

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Level 3 Assistant Robotics Technician. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and universities work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards need periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	3

1.1	Date of development:	4

1.2	Date of review:	4

1.3	Entry requirement of trainee:	4

1.4	Trainer Qualification Requirement	4

1.5	Codes of qualification (ISCED)	4

2.	Summary of competencies	4

3.	Qualification Leveling and Packaging	5

4.	Details of Competency Standards	6

0031PSD01: Perform basic communication	6

0714ART01: Apply workplace safety procedures	7

0714ART02: Assist in Computer, Electrical and Digital System Operations	10

0714ART03: Install and configure robotic system	14

0714ART04: Maintain robotic equipment	18

0714ART05: Assist in diagnosis and troubleshooting of robotic system problems	20

0714ART06: Apply programming and logic control concepts to industrial robotics tasks	23

0714ART07: Implement robotic operations	25

0714ART08: Perform Documentation and reports	27

0714ART09: Identify Battery Specification	29

5.	Tools, equipment and non-consumables for the qualification	32

6.	The consumables for this Qualification are provided below:	37

7.	Members of Qualification Development & Review Committee	38

# Introduction

An Assistant Robotics Technician is a trained professional responsible for maintaining robotic systems used in industrial and automation environments. The technician performs preventive and corrective maintenance tasks under supervision, including the inspection of mechanical joints, electrical wiring, actuators, and sensors. Tasks include cleaning, lubrication, diagnostics, and calibration to ensure the robot's performance and reliability. This course provides the foundational skills required to safely carry out routine maintenance procedures using standard tools and work protocols. The technician contributes to reducing downtime and extending the operational life of robotic equipment in industrial and service sectors.

| # Date of development: | January, 2026 |
| --- | --- |
| # Date of review: | December, 2028 |
| # Entry requirement of trainee: | SSC (Science) and relevant vocational or technical qualification recognized by the national authority at Level 2 or above, with foundational knowledge in Computer, electrical or mechanical systems. |
| # Trainer Qualification Requirement | Bachelor’s degree (16 years) in Robotics/Electronics/Mechatronics/Mechanical/Computer Engineering or Equivalent with at least 2 years of relevant industry experience. |
| # Codes of qualification (ISCED) | Code: 0714ART Description: National Vocational Certificate Level-3 in Assistant Robotics Technician |

# Summary of competencies

| Sr No | Competency Standards | NVQF Level | Category | Estimated Contact hours |  |  | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. | Total |  |
| 1 | Perform basic communication | 3 | Generic | 4 | 12 | 16 | 1.0 |
| 2 | Apply workplace safety procedures | 3 | Generic | 5 | 30 | 35 | 2.0 |
| 3 | Assist in Computer, Electrical and Digital System Operations | 3 | Technical | 9 | 42 | 51 | 3.0 |
| 4 | Install and configure robotics systems | 3 | Technical | 5 | 30 | 35 | 2.0 |
| 5 | Maintain robotic equipment | 3 | Technical | 6 | 18 | 24 | 1.5 |
| 6 | Assist in diagnosis and troubleshooting of robotic system problems | 3 | Technical | 4 | 42 | 46 | 2.5 |
| 7 | Apply programming and logic control concepts to industrial robotics tasks | 3 | Technical | 5 | 30 | 35 | 2.0 |
| 8 | Implement robotic operations | 3 | Technical | 5 | 30 | 35 | 2.0 |
| 9 | Perform documents and reports | 3 | Technical | 5 | 30 | 35 | 2.0 |
| 10 | Identify Battery Specifications | 3 | Technical | 5 | 30 | 35 | 2.0 |
|  | Total (%) |  |  | 53 | 294 | 347 | 20 |

# Qualification Leveling and Packaging

| Sr. No | Occupations / Short Course | Modules/CS Codes | Level | Occupation Credit Hours | Training duration |
| --- | --- | --- | --- | --- | --- |
| 1 | Electrical & Maintenance Technician (Robotics) | 0714ART03 & 0714ART05 | 3 | 4.0 | 64 Hrs |
| 2 | Robotics Installation & Operations Technician | 0714ART04 & 0714ART08 | 3 | 4.0 | 70 hrs |
| 3 | Robotics Diagnostics & PLC Programmer | 0714ART06 & 0714ART07 | 3 | 5.0 | 80 hrs |

# Details of Competency Standards

## 0031PSD01: Perform basic communication

Overview:

After this Competency Standard, the trainee will be able to describe the skills and knowledge required to assist in the development of communication competency at the workplace by providing information regarding different forms of workplace communication and their appropriate use.

| Competency Units | Performance Criteria |
| --- | --- |
| Follow workplace instructions | - Execute workplace instructions. - Ask questions to extract relevant information. - Carry out instructions based on received information. - Provide feedback based on received written instructions. - Compose workplace notes and memos. |
| Develop effective workplace relationships | - Perform duties to improve team cooperation. - Consult team members when challenges arise. - Respect differences in personal values and beliefs. - Report observed workplace harassment to relevant authority. |
| Communicate and work with team members | - Identify individual roles and responsibilities within the team. - Recognize team members responsibilities with respect. - Identify the person for reporting inside and outside the team. - Participate in team meetings for discussions. - Communicate with team members through interaction tools and equipment. - Provide support to team members as needed during tasks. - Provide and respond to feedback |
| Respond effectively to customer requests | - Listen to customer requests effectively. - Speak to customers using polite words. - Deliver information to the relevant person in written or verbal form. |

Knowledge & Understanding

The candidate must be able to demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Mode of communication (e.g. written and nonverbal)

- Communication technology relevant to the workplace

- Effective communication (body language)

- Questioning techniques

- Introduction to process of basic workplace documentation

- Introduction to the rules on how to write memos, notes, messages

- Introduction to workplace harassment

- Understand feedback Process

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate the ability to clarify workplace messages and apply verbal and written communication skills accurately.

- Respond to feedback from clients and colleagues.

- Provide support to team members in a group task

Tools and equipment required

| SR. NO | NAME | QTY. REQUIRED |
| --- | --- | --- |
| 01 | Notebook | 25 |
| 02 | Pen | 25 |
| 03 | Board Marker | 5 |
| 04 | White Board | 1 |
| 05 | Projector or Multimedia | 1 |

## 0714ART01: Apply workplace safety procedures

Overview:

This unit develops competencies to apply workplace safety procedures and ensure safe operation of robotic systems by following approved protocols, using PPE, and conducting risk assessments. Learners will manage hazards, verify compliance, and maintain safety documentation in industrial or training environments.

| Competency Units | Performance Criteria |
| --- | --- |
| Prepare for safe work practices | - Identify safety requirements of job. - Identify emergency procedures and evacuation steps for workplace incidents. - Wear personal protective equipment - Follow safety instructions and signs during work. - Identify workplace hazards - Report hazards, incidents, and injuries using workplace procedures. - Participate in safety drills and toolbox talks - Demonstrate safe behavior during task execution |
| Perform Safe handling of tools and equipment | - Inspect tools and equipment for defects - Follow safety guidelines while using tools and equipment. - Clean and store tools and equipment after use - Report damaged tools and equipment to supervisor - Follow SOPs for tool usage and maintenance. |
| Maintain Fire Extinguisher | - Identify fire extinguisher according to type of fire - Check expiry of fire extinguisher - Operate fire extinguisher - Refill expired fire extinguisher |
| Perform Basic First Aid | - Ensure availability of first aid box - Check first aid box for requisite emergency medicines - Check expiry date of medicines - Perform basic first aid for minor injuries |
| Respond to Emergencies | - Assess risk and determine course of action - Follow emergency plan - Implement evacuation procedures as per SOP - Operate emergency equipment and supplies |
| Conduct Safety Assessments | P1. Observe workplace to detect potential hazards. P2. Use predefined checklist to locate risk-prone areas. P3. Categorize hazards using a risk matrix. P4. Record hazards in a standardized risk assessment form. P5. Report findings using SOP-based formats. P6. Mark high-risk zones with appropriate safety signage. P7. Verify functioning of warning systems before maintenance. |
| Implement Lockout/Tagout (LOTO) Procedures | P1. Locate energy sources using system diagrams. P2. Isolate energy sources following documented protocol. P3. Apply LOTO devices as per SOP. P4. Label LOTO devices with technician name, date, and job. P5. Test system to confirm zero energy state. P6. Document LOTO completion in the checklist. P7. Demonstrate lock verification before maintenance. P8. Remove LOTO devices only after confirming safe restart conditions. |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Safety rules and regulations of organization

- Safety signs and symbols

- Causes of workplace accidents relevant to the work environment

- Hierarchy of risk at workplace

- Techniques to handle emergency situations in different forms and contexts

- Workplace response to emergencies

- Standard safety practices, hazard types (mechanical, electrical, thermal, chemical, ergonomic), and risk management including emergency controls (E-stop, evacuation, fire).

- Lockout/Tagout (LOTO) devices, procedures, verification, and use of Personal Protective Equipment (PPE) including classification, selection, adjustment, removal, and disposal.

- Safety guards, light curtains, interlock functions, pre-operation inspections, and risk assessment using checklists.

- Standard Operating Procedures (SOPs) and manuals: interpretation, application, documentation, and continuous improvement.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Use fire extinguisher and personal protective equipment correctly.

- Submit a report containing identified risk and hazard at workplace to concerned

- Complete and verify risk assessments, LOTO procedures and pre-operation safety inspections in real or simulated environments.

Tools and Equipment Required

| SR. NO | NAME | QTY. REQUIRED |
| --- | --- | --- |
| 01 | Fire Extinguisher | 02 |
| 02 | First Aid Kit | 01 |
| 03 | Multimedia | 01 |
| 04 | PPE set (Gloves, Goggles, Helmet, Shoes) | 12 |
| 05 | Voltage tester | 25 |
| 06 | Multimeter | 02 |
| 07 | Lockout/Tagout kit | 12 |
| 08 | Safety Signs & Labels | 08 |
| 09 | Emergency Stop Button (Demo Unit) | 08 |

## 0714ART02: Assist in Computer, Electrical and Digital System Operations

Overview: This unit develops skills and knowledge in setting up, operating, and troubleshooting basic electrical and digital systems in robotics and automation. Learners will use standard office tools, electronic instruments, and safe work practices under supervision, equipping learners for further development in robotics design, control, and diagnostics.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Install computer hardware and operating system | P1. Assemble hardware components using appropriate tools. P2. Install operating systems using standard procedures. P3. Adjust basic BIOS/UEFI settings before OS installation. P4. Check installation by booting into the OS desktop. P5. Attach peripheral devices. P6. Correct basic driver and boot issues. |
| CU2. Operate office application tools. | P1. Operate office applications in a configured computer environment. P2. Perform Type and formatting text in MS Word. P3. Perform basic calculations in spreadsheets. P4. Make a presentation using suitable software. P5. Transfer office application files via USB or local network. |
| CU3. Construct basic databases for maintenance logs or inventory tracking. | P1. Operate database software and Construct a new database. P2. Organize fields for tables. P3. Enter sample maintenance or inventory records into the table. P4. Execute queries to filter specific entries. P5. Produce reports in the table and printable format. |
| CU4. Operate measurement tools for Mechanical Components | P1. Select appropriate measuring tools. P2. Arrange the object securely before taking measurement. P3. Adjust tool markings properly to ensure accurate readings. P4. Record measurements in standard units with tolerances. P5. Verify values to technical drawings or specs. |
| CU5. Operate measuring instruments to test fundamental electrical laws. | P1. Assemble circuit on a breadboard according to the schematic diagram. P2. Measure electrical parameters using a multi-meter. P3. Demonstrate application of Ohm’s Law to calculate values. P4. Verify measured values against theoretical results. P5. Record deviations. |
| CU6. Identify components and construct basic electronics circuits. | P1. Identify basic electronics components. P2. Interpret the given circuit diagram. P3. Verify correct component placement, polarity, and grounding during circuit assembly. P4. Perform circuit construction using soldering P5. Perform circuit construction using jumper wires P6. Check voltage and current using appropriate tools. P7. Test device functions using datasheets and basic diagnostic tools. P8. Simulate the same circuit using PCB design software P9. Transfer Gerber files. P10. Demonstrate basic understanding of the PCB fabrication workflow P11. Assemble and test basic DC power supply circuits including voltage regulators. P12. Demonstrate correct heat sink installation for regulators. P13. Test half-wave and full-wave rectifier outputs P14. Implement and test filtering circuits to observe ripple reduction. |
| CU7. Verify logic gates and construct combinational circuits. | P1. Construct truth tables of basic logic gates. P2. Assemble digital ICs on a breadboard as per the logic diagram. P3. Operate switches to provide logic inputs and Observe LED outputs. P4. Verify output behavior against the truth table. P5. Document circuit diagram and output values. |
| CU8. Assemble and operate motor circuits. | P1.Differentiate different types of motors. P2. Select appropriate motor drivers and control interfaces. P3. Wire motors using standard practices. P4. Demonstrate motor circuits. P5. Test motor connections using tools and devices. P6. Document motor circuit setups with diagrams and symbols. |
| CU9. Assemble basic mechanical systems. | P1. Assemble mechanical components. P2. Operate motorized systems to verify parameters. P3. Demonstrate safety procedures while handling devices. P4. Assemble a simple mechanical drive system for motion transmission. P5. Document the setup of mechanical systems using diagrams. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes to:

- Understand computer hardware components, operating system installation, and configuration procedures.

- Understand the functions, features, and file management of Office applications for documentation and data processing.

- Understand database structures, data entry, querying, and basic reporting techniques.

- Understand types, uses, and accuracy requirements of mechanical measuring instruments. basic electrical laws, measurement tools, and safe testing procedures.

- Understand electronics components, rectifier circuit principles, filters, power supply, regulator and assembly/testing methods.

- Understand digital logic concepts, truth tables, circuit construction, and verification methods.

- Understand motor types, drivers, control interfaces, wiring practices, and testing methods.

- Understand mechanical components, motion transmission, assembly techniques, and documentation.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate operating system installation and configuration after identifying hardware components accurately.

- Perform electrical measurements using instruments and apply Ohm’s law safely to analyze circuit behavior.

- Construct basic electronics circuits, logic circuits, verify outputs against truth tables, and document results systematically.

Tools and Equipment Required

| Sr. No. | Name | QTY. REQUIRED |
| --- | --- | --- |
| 1 | Computer System (8th Generation,8GB RAM,512 Hardware, Software (Compatible for C++) | 10 |
| 1 | Computer System (8th Generation,8GB RAM,512 Hardware, Software(Compatible for C++) | 10 |
| 2 | DC Motors (3–12V, 100–300 RPM, geared) | 10 |
| 3 | Stepper Motors (NEMA 17 or similar, 1.8° step angle) | 10 |
| 4 | Servo Motors (SG90 or MG996R, PWM-controlled) | 10 |
| 5 | Motor Driver Modules (L298N, L293D for DC/stepper motors) | 10 |
| 6 | Microcontroller Boards (Arduino Uno or compatible) | 10 |
| 7 | Breadboards (830-point, solderless) | 25 |
| 8 | Veroboards / Stripboards (100×160 mm, copper tracks) | 15 |
| 9 | Basic PCB Design Software (e.g., KiCad, Eagle) – open source/institutional license | 15 |
| 10 | PCB Milling Machine (Benchtop CNC, suitable for 2-layer FR4, 0.2 mm trace width) | 2 |
| 11 | Soldering Stations (Temperature controlled with fine tips) | 6 |
| 12 | PCB Drills (Mini drill press with 0.8–1.0 mm bits) | 2 |
| 13 | Multimeters (Digital, with continuity and DC current modes) | 5 |
| 14 | Power Supplies (DC Bench, 0–30V, dual channel, 3A) | 5 |
| 15 | Jumper Wire Sets (Male-male, male-female, assorted lengths) | 25 |
| 16 | Battery Packs (4×AA or Li-ion with holder and switch) | 20 |
| 17 | Mechanical Kits (Spur and Bevel Gears, pulleys, shafts, brackets, couplings, belts, shaft, bearings, couplings) | 5 |
| 18 | Metal CNC Milling Machine (Benchtop, 3-axis, for aluminum parts) | 1 |
| 19 | CNC Tooling Kit (Cutters, clamps, wrenches, ER collets) | 1 |
| 20 | Chassis Frames or Mounting Boards (for assembling motor systems) | 5 |
| 21 | Tachometers (Laser/contact type for motor RPM measurement) | 3 |
| 22 | Basic Tools (Screwdrivers, pliers, wire strippers, Allen keys) | 15 |
| 23 | Safety Goggles and Gloves (Lab-grade, reusable) | 15 |
| 24 | Anti-static Wrist Straps & ESD Mats (For PCB assembly safety) | 5 |
| 25 | Instructional Worksheets / Circuit Diagrams (Laminated or digital) | 5 |
| 26 | Switches, contactors, or push buttons | 20 |
| 27 | Protection devices (fuses, circuit breakers, overload relays) | 15 |

## 0714ART03: Install and configure robotic system

Overview: Demonstrate the skills and knowledge to interpret technical schematics, assemble robotic hardware, integrate electrical, pneumatic, and communication systems, program robotic controllers, and prepare the system for commissioning and testing.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Interpret technical drawings and schematics | P1. Trace schematics and CAD drawings. P2. Classify robotic systems and components in the schematics. P3. Distinguish actuators and end effectors of the robotic system. P4. Pinpoint coordinates of robotic systems and components in the schematics. P5. Demonstrate kinematic movements of the manipulator and end-effector. P6. Connect different robotic components and systems. P7. Verify power supply connections for each actuator. |
| CU2. Connect different mechanical parts | P1. Trace parts of the manipulator including end effectors, actuators, power supply, and main body. P2. Identify links and joints of the manipulator. P3. Assemble links, joints, and end effectors of the manipulator. P4. Classify hydraulic, pneumatic, and other actuators. P5. Adjust for any non-homogeneity between the manipulator assembly and the designated lab/floor area. P6. Install the robotic assembly at the designated location in the lab/floor. P7. Demonstrate the kinematics (movement) of the end effector. |
| CU3. Connect electrical and communication systems | P1. Distinguish electrical actuators. P2. Trace electrical connectivity between actuators, manipulators, and end effectors. P3. Locate the grounding point within the lab/floor. P4. Connect electrical cords to establish links between actuators, manipulators, and end effectors. P5. Install the power supply for the actuators. P6. Identify the communication port and select the required communication link/cord. P7. Attach the communication port to the controlling unit using the appropriate communication link/cord. |
| CU4. Configure robot OS | P1. Install the robot operating system. P2. Operate the robot’s user interface using the teach pendant or PC interface. P3. Load existing programs into the robot controller for initial use. P4. Set the robot’s home position and reference frames. P5. Configure and test safety interlocks to ensure safe movement of the manipulator and end effector during execution. P6. Upgrade the robot firmware. |
| CU5. Prepare the system for commissioning and testing | P1. Verify installation of all robotic subsystems (mechanical, electrical, pneumatic) against design specifications. P2. Inspect electrical and pneumatic connections to confirm they are secure, correctly routed, and grounded. P3. Connect power supply units while keeping them powered off pending safety checks. P4. Check communication links between the controller and subsystems using correct ports and cables, and visually detect potential faults P5. Power up the system and run basic control programs or diagnostics to confirm readiness of actuators, sensors, and safety interlocks without live motion. P6. Execute a pre-commissioning checklist, including environmental safety checks, emergency stop functionality, and safety zone verification. P7. Record preparation steps, system status, identified issues, and corrective measures for review prior to commissioning. |
| CU6. Connect 3rd party sensors | P1. Classify different types of sensors. P2. Determine electrical requirements for sensor operation. P3. Connect sensors compatible with robotic equipment. P4. Configure sensor parameters through programming. P5. Conduct a full-scale test of the sensors. |
| CU7. Perform remote monitoring | P1. Set necessary network parameters. P2. Connect the communication port. P3. Test network connectivity between the remote system and the robotic equipment. P4. Configure the remote software. P5. Monitor the robotic device during operation. |

Knowledge & Understanding

To perform the tasks outlined in the performance criteria from A-1 to A-6, the candidate must demonstrate integrated knowledge and understanding of the following:

- Explain the technical drawings, schematics, and CAD diagrams, including symbols and component layout for robotic systems.

- Describe assembly procedures and techniques for robotic hardware, including mechanical, electrical, pneumatic, and hydraulic systems.

- Distinguish between actuator, manipulator, end effector, joints etc

- Define electrical safety practices, power supply requirements, and grounding procedures in lab environments.

- Express programming of robot controllers using teach pendants and PC interfaces, including setting home positions, reference frames, and safety interlocks

- Identify sensors (ultrasonic, ir, magnetic, etc)

- Identify sensors communication type (USB, Serial, I2C)

- Identify robot communication ports

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Demonstrated ability to perform system installation, programming, and commissioning of robotic systems under guided and independent conditions.

- Evidence of correctly connecting electrical, pneumatic, and hydraulic systems as per schematic diagrams and operational guidelines.

Tools and Equipment Required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Screwdrivers (flathead and Phillips | 15 |
| 2 | Allen/hex key sets | 15 |
| 3 | Spanners and wrenches | 10 |
| 4 | Pliers (needle-nose, combination) | 15 |
| 5 | Wire cutters and strippers | 15 |
| 6 | Crimping tools | 10 |
| 7 | Multimeters | 10 |
| 8 | Soldering iron kits | 10 |
| 9 | Electrical connectors and terminal blocks: Assorted kits | 5 |
| 10 | Adjustable power supply units | 5 |
| 11 | USB, serial, and Ethernet cables | 25 |
| 12 | Tubing cutter and hose clamp tools | 5 |
| 13 | Pressure gauges | 5 |
| 14 | Air compressor | 2 |
| 15 | Basic hydraulic test equipment | 2 |
| 16 | Robot teach pendants | 5 |
| 17 | PCs/laptops with robot interface software | 10 |
| 18 | Calipers or micrometers | 10 |
| 19 | Measuring tapes and rulers | 15 |
| 20 | Angle gauges/alignment tools | 5 |
| 21 | Industrial or educational-grade robotic manipulators | 5 |
| 22 | End effectors (Parallel jaw grippers,Vacuum suction,Magnetic grippers) | 5 |
| 23 | Actuators (servo motors, stepper motors, linear actuators) | 15 |
| 24 | Robotic base platforms or stands | 5 |
| 25 | Mobile robotic platform with robotic arm and gripper | 10 |

## 0714ART04: Maintain robotic equipment

Overview This unit develops the competencies required to perform scheduled maintenance on robotic systems, focusing on joints, actuators, wiring, and sensors. It includes diagnostic checks, cleaning, lubrication, and basic calibration, with strict adherence to safety protocols, tool standards, and supervisory procedures.

| Competency Units | Performance Criteria |
| --- | --- |
| Perform preventive maintenance as per schedules | P1. Interpret schedules & SOPs to identify preventive tasks. P2. Select tools, materials, PPE per safety standards. P3. Apply lockout/tagout before maintenance. P4. Clean components with proper agents & tools. P5. Lubricate moving parts per guidelines. P6. Inspect robotic parts for wear, damage or leaks. P7. Tighten connections to specification. P8. Restore workspace to clean, functional order. P9. Record maintenance actions, findings, corrections. |
| Check mechanical components and joints | P1. Identify assemblies and joints via diagrams/specs P2. Examine mechanical links using visual and manual techniques P3. Evaluate joint alignment and motion while powered off P4. Detect misalignment, friction, or abnormal vibration P5. Communicate issues or follow-up needs to supervisor |
| Examine electrical circuits and connections | P1. Identify components & wiring via diagrams P2. Inspect wiring and connectors for damage/corrosion P3. Test continuity & voltage with multimeter P4. Ensure proper insulation & grounding P5. Document findings & report faults P6. Complete and submit inspection log with values |
| CU4. Test electrical and mechanical components and connections | P1. Select tools/software per component and test need P2. Conduct diagnostic tests on motors, sensors, actuators P3. Compare Analyze results with manufacturer specs P4. Identify faults and log findings |
| CU5. Calibrate sensors and feedback devices | P1. Identify sensors needing calibration via docs/indicators P2. Calibrate sensor outputs using calibration tools/software P3. Verify calibration against reference values P4. Document calibration results (values, technician) P5. Apply safety procedures when handling electronics during calibration |

Knowledge & Understanding:

The candidate must demonstrate basic understanding of the following concepts to perform maintenance tasks on robotic equipment:

- Explain the importance of preventive maintenance in robotic systems and the ability to follow standard operating procedures and manufacturer guidelines.

- Express identification process of common mechanical and electrical components such as joints, actuators, wires, and connectors,

- Describe basic inspection and testing methods using appropriate diagnostic tools.

- Define application of safety practices including PPE and lockout/tagout procedures, and recognize the value of documenting maintenance tasks for future reference and system reliability.

Critical Evidence(s) Required:

To be deemed competent in this unit, the candidate must successfully demonstrate the ability to:

- Carry out preventive maintenance on mechanical joints.

- Calibrate feedback devices for accurate system response.

- Record test results in structured maintenance logs.

Tools and equipment required for CS:

The tools and equipment required for this competency standard are provided below:

| Sr. No | Items | Quantity |
| --- | --- | --- |
|  | Mechanical Tool Kit | 5 |
|  | Screwdriver Set | 5 |
|  | Safety Tolls | 5 |
|  | Electrical Tool Kit | 5 |
|  | Cleaning Brushes & Cloths | 5 |
|  | Lubricants & Cleaners | 2 |
|  | Grease Gun | 2 |
|  | Digital Multimeter | 5 |
|  | Continuity Tester | 5 |
|  | Calibration Toolkit | 5 |
|  | Infrared Thermometer | 5 |
|  | Vibration Tester | 2 |
|  | Sample Motors (DC/Servo) | 5 |

## 0714ART05: Assist in diagnosis and troubleshooting of robotic system problems

Overview:

This unit prepares entry-level robotics technicians to assist in diagnosing and troubleshooting robotic systems under supervision. Learners develop skills in identifying, isolating, and resolving mechanical, electrical, and software faults. Tasks include interpreting error codes, using diagnostic tools, replacing faulty components, performing system resets, verifying system functionality, and restoring operational readiness of robot. All activities follow safety protocols, systematic procedures, and relevant OEM (Original Equipment Manufacturer) ISO/industrial standards.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Detect abnormal conditions | P1. Read error codes from the HMI/control panel or indicators. P2. Retrieve history or diagnostic data. P3. Record error details in maintenance log. P4. Match error codes with fault list. P5. Classify fault type P6. Prioritize fault for action P7. Communicate findings to the supervisor. |
| CU2. Perform fault diagnostic | P1. Identify the appropriate diagnostic tool P2. Verify tool calibration and settings P3. Configure the diagnostic tool P4. Apply tool to obtain electrical readings P5. Apply tool to obtain mechanical readings P6. Record measurement results P7. Follow safety protocols P8. Clean diagnostic tools after use P9. Store diagnostic tools in designated locations |
| CU3. Perform isolation | P1. Set the robot to a safe diagnostic state. P2. Confirm the fault on the diagnostic interface. P3. Reproduce the fault with a controlled test. P4. Apply the half-split method to localize the fault path. P5. Reseat connectors in the suspected path. P6. Inspect harnesses for damage. P7. Measure supply voltage at the subsystem input. P8. Measure signal continuity along the suspected path. P9. Swap a suspected component with a known-good unit. P10. Bypass the suspected element using approved test plugs. P11. Run a confirmatory test to pinpoint the fault source. P12. Log each isolation step in the maintenance record. P13. Record the final fault source in the maintenance record. |
| CU4. Perform fault correction | P1. Identify corrective action as repair, replace or recalibrate. P2. Ensure spare part or component availability. P3. Perform component replacement or repair per guidelines. P4. Ensure that no adjacent components are damaged P5. Update software or firmware as per guidelines. P6. Apply all reassembly standards during fault correction. P7. Restore protective covers, guards, and safety interlocks. P8. Record corrective action performed. |
| CU5. Reset software errors | P1.Understand system mechanism for reset. P2.Apply relevant error log to follow reset menu. P3.Perform reset under supervision. P4.Check indicators return to normal. P5.Analyze test protocol with supervision. P6.Run operation test. P7. Compare actual results with predefined standards. P8. Log reset action, time, response. |
| CU6: Verify system functionality | P1. Confirm the robot powers up and connected. P2. Confirm the robot initializations. P3. Run standard test cycles P4. Run basic routines P5. Check safety interlocks. P6. Execute diagnostic routines as per OEM guidelines. P7.Compare actual and response against predefined baselines. P8. Check system indicators P9. Check system alarms P10. Check recovery functions. P11. Record all outcomes in the maintenance log. P12. Communicate deviations to supervisor |
| CU7: Restore robotic system to operational readiness | P1.Restore robot operating mode to normal production or standby mode. P2.Reconnect peripheral devices disconnected during troubleshooting. P3.Secure panels, enclosures, and access doors to original condition. P4. Check cable routing and strain relief after intervention. P5. Verify end-effector mounting and fastening integrity. P6. Ensure work area around the robot is cleared of tools and materials. P7.Confirm the robotic system is ready for handover to operations. |

Knowledge & Understanding:

- Explain robotic error codes, alarms, and status indicators.

- Describe diagnostic tool selection, setup, and safe use.

- Outline fault isolation steps for mechanical, electrical, and software issues.

- Identify affected subsystems and fault types.

- Summarize software reset procedures, system recovery verification and operational restoration.

- Explain recording, reporting, and handover processes.

Critical Evidence:

- Calibrate sensors for accurate signal response.

- Record troubleshooting steps in maintenance logs.

Tools and equipment required for CS:

| Sr. No | Items | Quantity |
| --- | --- | --- |
| 1 | Digital Multimeter (Basic) | 25 |
| 2 | HMI or Teach Pendant | 5 |
| 3 | Screwdriver Set (Insulated) | 6 |
| 4 | Torque Wrench (Small) | 6 |
| 5 | Insulation Tape / Heat Shrink Kit | 12 |
| 6 | Component Cleaning Kit | 12 |
| 7 | Basic Diagnostic Cables (Continuity Tester / Cable Tester) | 12 |
| 8 | Robotic System Test Rigs / Simulators | 4 |

## 0714ART06: Apply programming and logic control concepts to industrial robotics tasks

Overview:

This unit equips learners to apply C++ and PLC logic for robotic control tasks. It includes configuring tools, writing code, simulating operations, and integrating logic in a supervised setting.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Configure C++ IDE | P1. Install a C++ Integrated Development Environment (IDE). P2. Set compiler options in the IDE. P3. Run a sample program to verify compiler operation. P4. Configure debugging tools in the IDE. P5. Create a project workspace for robotic tasks. P6. Save IDE configuration settings. P7. Restore default IDE settings when required. P8. Record setup steps in a configuration log. |
| CU2. Execute basic C++ programs | P1. Develop a C++ program that generates text output. P2. Execute a C++ program in the IDE. P3. Insert arithmetic operators in program statements. P4. Insert relational operators in program statements. P5. Declare variables for data storage. P6. Enter input functions for user data entry. P7. Display output using program statements. P8. Debug syntax errors using IDE tools. P9. Save source code with proper naming conventions. |
| CU3. Implement robotic functions in C++ | P1. Write loop structures for repetitive robotic actions. P2. Write conditional statements for robotic control. P3. Insert delay functions to control motor speed. P4. Simulate sensor input in program logic. P5. Adjust motor parameters in program code. P6. Annotate program with inline comments. P7. Compile and run the program on a test setup. P8. Debug logical errors in robotic functions. P9. Record program test results in a logbook. |
| CU4. Operate PLC ladder logic simulation | P1. Install PLC simulation software. P2. Configure project settings for PLC simulation. P3. Create ladder logic for start operations. P4. Create ladder logic for stop operations. P5. Insert timers into ladder diagrams. P6. Insert counters into ladder diagrams. P7. Run simulation of actuator response. P8. Run simulation of indicator response. P9. Debug errors identified in simulation. P10. Save ladder program with version control. |
| CU5. Convert logic from C++ to PLC | P1. Inspect C++ program to locate logical conditions. P2. Inspect PLC ladder diagram to locate logical conditions. P3. Translate C++ conditions into PLC ladder rungs. P4. Translate C++ loops into PLC ladder rungs. P5. Execute both programs to compare logic flow. P6. Record discrepancies in program behavior. P7. Revise mismatched logic in PLC or C++ code. P8. Validate converted logic with test runs. P9. Document translation process in a log. |
| CU6. Integrate C++ with PLC project | P1. Develop a C++ program for a robotic task. P2. Develop a PLC program for the same task. P3. Connect PLC with programming device. P4. Simulate data exchange between C++ and PLC. P5. Test synchronization of outputs across platforms. P6. Correct integration errors in the system. P7. Optimize execution time of integrated project. P8. Validate system against project requirements. P9. Compile technical report for project work. P10. Demonstrate project outcomes in front of assessors. |

Knowledge & Understanding:

- Basics of C++ programming: syntax, loops, conditions, I/O functions.

- Use of C++ IDEs (e.g., Code::Blocks, Visual Studio).

- Introduction to PLC ladder logic and simulation tools.

- Timers, counters, and their role in automation.

- Comparison between sequential programming and ladder logic.

- Documentation practices for programming and logic tasks.

Critical Evidence(s) Required:

- Simulate ladder logic diagrams using PLC software.

- Submit documented and commented code files for review.

Tools and equipment required for CS:

| Sr. No | Items | Quantity |
| --- | --- | --- |
| 1 | Computer (8th Generation,8GB RAM,512 Hardware, Software(Compatible for C++) | 25 |
| 2 | PLC Simulation Software | 8 |
| 3 | Robot Simulator (Desktop) | 8 |
| 4 | Digital I/O Trainer Board | 8 |
| 5 | C++ to PLC Integration Kit (Optional) | 8 |
| 6 | Basic Networking Kit | 12 |
| 7 | Serial Monitor or Debugging Tools | 4 |

## 0714ART07: Implement robotic operations

Overview: This unit equips learners with the practical skills to configure, fine-tune, and test robotic systems in real-world settings. It focuses on setting operational parameters, adjusting and optimizing them based on performance, documenting changes, and verifying through systematic testing.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Set parameters for robotic operations | P1. Locate essential operational parameters from the robot's technical manual. P2. Demonstrate function of each parameter when provided with real-world robotic application examples P3. Input default parameter values into a robot programming interface to configure a simple pick-and-place task. P4. Simulate two different parameter sets by simulating the same task and noting differences in execution time and precision. P5. Configure appropriate parameter presets based on a workpiece's size and weight to ensure the robot operates within safe load limits. |
| CU2. Adjust pre-set parameters | P1. Interpret task requirements from a production brief to determine which parameters need adjustment. P2. Adjust speed and acceleration values on a robot control interface to reduce task time by 10% while maintaining accuracy. P3. Test adjusted parameters using robot simulation software and log the outcome. P4. Inspect the impact of parameter changes on performance logs and recommend improvements. P5. Recalibrate robotic joint limits in a safety simulation scenario to avoid collisions in workspace. |
| CU3. Log modified parameters | P1. Document the key elements that must be included in a parameter modification log. P2. Log parameter changes after each lab session using a standard logging template. P3. Tag log entries with appropriate tags or version numbers to ensure traceability. P4. Generate a visual report using a spreadsheet tool to show before-and-after parameter comparisons. P5. Compile key parameter modifications and their outcomes in a technical report submitted weekly. |
| CU4. Test modified parameters | P1. Construct a basic test routine in a robot simulator that incorporates adjusted parameters. P2. Perform a robot motion test after modifying key parameters and document the results. P3. Compare test outcomes against expected performance values to assess effectiveness. P4. Troubleshoot faults or irregular behavior during robot testing using parameter feedback data. P5. Present parameter adjustments through test evidence and safety considerations during group presentation. |

Knowledge & Understanding:

By the end of this module, the candidate will be able to:

- Explain the principles of robot parameter configuration and optimization for safety and efficiency.

- Describe trajectory planning, motion constraints, and the influence of payload and cycle time on parameter settings.

- Explain data logging methods (manual, digital, version control) and their role in traceability.

- Define testing methods and tools for validating parameter changes under various operational conditions.

- Identify safety requirements and implications of parameter modification.

Critical Evidence(s) Required:

To demonstrate mastery of the module, the candidate must:

- Produce robotic program with pick-and-place routine.

- Maintain change log for modified parameters.

- Present validation report on system performance.

Tools and equipment required for CS:

| Sr. No | Name | Quantity |
| --- | --- | --- |
| 1 | Entry level Robot arms /Workstation ( DOBOT, Fanu LR Mate, UR3e/ FANUC, ABB, Yaskawa, KUKA) | 4 |
| 2 | Teach Pendant Simulators | 8 |
| 3 | Robot Programming Software (e.g., RoboDK, RobotStudio, TIA Portal) | 8 |
| 4 | Basic I/O Panels for Triggering Events | 8 |
| 5 | Path Visualization Tools (trajectory overlays, time stamps) | 8 |
| 6 | Stopwatch or cycle time measurement utilities | 12 |
| 7 | Basic Version Control System or Excel/Google Sheets | 4 |
| 8 | Word processors, templates for change control records, PDF editors | 8 |
| 9 | Digital logbooks, Excel-based change trackers, or simplified SCADA software | 8 |

## 0714ART08: Perform Documentation and reports

Overview: This unit develops the ability to accurately document maintenance activities, report faults, request components, and prepare routine shift reports in industrial robotic environments. Candidates maintain logs, compile structured reports, and communicate technical information clearly, following organizational documentation protocols.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Maintain service logs | P1. Record completed tasks in the service log immediately. P2. Record task date, time, component ID, and technician name P3. Apply standard technical terminology consistently. P4. Update logbook or digital system without gaps. P5. Organize log entries sequentially without duplication. P6. Submit completed service logs to the supervisor. P7. Store logs according to access and retention policy. P8. Correct inaccurate entries only with authorized approval. |
| CU2. Manage fault | P1. Identify faults. P2. Classify faults as mechanical, electrical, or control system. P3. Describe fault symptoms using technical language. P4. Document isolation procedures following LOTO standards. P5. Enter isolation data accurately into maintenance forms P6. Report critical faults immediately to the supervisor. P7. Attach supporting evidence to fault reports. P8. Mark the isolated system as out of service. P9. Record all updates for fault resolution. |
| CU3. Request component | P1. Locate missing, defective, or insufficient components. P2. Record part number, description, and quantity on the form. P3. Attach supporting evidence referencing logs or failures. P4. Categorize component needs as urgent or routine. P5. Submit requests through approved channels. P6. Track request status via inventory systems or follow-up. P7. Correct inaccurate requests promptly. P8. Apply strategies to avoid component delivery delays. |
| CU4. Develop routine reports | P1. Log completed maintenance tasks for the shift. P2. Organize the report into logs, issues, and recommendations. P3. Include timestamps, machine codes, and identifiers. P4. Attach supporting evidence to the report. P5. Apply terminology per manuals or SOPs. P6. Complete the report before shift closure. P7. Evaluate the report for accuracy, clarity, and completeness. P8. Submit a finalized report to the system or supervisor. P9. Store reports according to policy. P10. Incorporate supervisor feedback in stored reports. |

Knowledge & Understanding:

By the end of this module, the candidate will be able to:

- Purpose, structure, and required fields of service logs, fault reports, component requests, and routine reports.

- Standard technical terminology for maintenance documentation and communication.

- Methods to identify, classify, and describe faults, including inspections, system errors, and performance monitoring.

- Lockout/Tagout (LOTO) procedures and system isolation standards for safe handling and accurate documentation.

- Component requisition processes: identifying missing or defective parts, recording details, and submitting requests through approved channels.

- Structuring routine reports: logging tasks, organizing findings, including traceable data, and attaching supporting evidence.

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Verified portfolio of completed service logs, maintenance logs, fault reports, component requests, and routine reports.

- Demonstration (practical or simulated) of reporting, documentation, and fault-handling procedures

- Component requisition case study with supporting evidence, categorization of urgency and justification for request.

Tools and equipment required for CS:

| Sr. No | Name | Quantity |
| --- | --- | --- |
| 1 | Computer system (8th Generation,8GB RAM,512 Hardware ,Software( Compatible for C++) or workstation (with relevant maintenance software) | 25 |
| 2 | Digital logbook or service management system | 8 |
| 3 | Manual logbooks / maintenance forms | 12 |
| 4 | Barcode scanner or RFID reader (for equipment ID logging) | 12 |
| 5 | Label printer and tags (for system isolation labeling) | 4 |
| 6 | Camera or mobile device (for capturing error codes, faults, and evidence) | 8 |
| 7 | Internet-connected device (for submitting reports and requisitions) | 8 |
| 8 | Digital inventory/requisition system | 4 |
| 9 | Lockout/Tagout kit (for isolation procedures) | 8 |
| 10 | Diagnostic tools (e.g., multimeter, signal testers – if used during fault detection) | 8 |
| 11 | Reference manuals and SOP documentation | 4 |
| 12 | Stationery items (pens, markers, highlighters for manual entries) | 12 |
| 13 | Digital logbook or service management system | 8 |

## 0714ART09: Identify Battery Specification

Overview:

This unit equipped learners to select the right battery, understand its voltage, capacity, and type, connect it safely, and provide correct power to actuators. Learners will be able to test battery backup, measure performance, and handle batteries safely while documenting results efficiently.

| Competency Units | Performance Criteria |
| --- | --- |
| Interpret Primary and Secondary Batteries | - Distinguish between primary and secondary batteries. correctly - Identify voltage, capacity and discharge of the battery. - Select the suitable battery type for a given component (robot, sensor, controller). - Demonstrate understanding of advantages and limitations of each battery type |
| Connect Batteries in Series and Parallel | - Connect batteries in series to increase voltage. - Connect batteries in parallel to increase capacity (Ah). - Ensure proper polarity to avoid short circuits or damage. - Verify the total voltage and current capacity after connection using a multimeter. - Demonstrate safety measures while connecting multiple batteries. |
| Configure Power According to Actuator | - Identify the voltage and current requirement of each actuator (DC motor, servomotor, stepper motor, sensors). - Select and configure battery or battery pack to meet actuator requirements. - Use regulators or converters to provide stabilized voltage to sensitive components. - Ensure the actuators operate smoothly without voltage drops. - Demonstrate proper wiring and connections for power distribution |
| Test the Batteries Backup According to Load | - Measure battery voltage before and after application of load using a multimeter. - Calculate total load current of all connected actuators. - Compute expected backup/run time based on battery capacity and load. - Conduct practical testing to verify actual backup time under working load. - Ensure battery voltage does not fall below safe limits - Record observations and report battery performance clearly |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Battery Types: Classification into primary and secondary batteries.

- Primary Batteries: Alkaline, zinccarbon; singleuse, suitable for lowpower devices.

- Secondary Batteries: Liion, LiPo, NiMH, leadacid; rechargeable and widely used in robotics.

- Battery Specifications: Voltage, capacity (Ah/mAh), discharge rate, and chemistry.

- Series Connection: Increases voltage, capacity remains the same.

- Parallel Connection: Increases capacity, voltage remains constant.

- Power Configuration: Matching battery output to actuator requirements using regulators.

- Identify the voltage and current requirements of different actuators such as DC motors, servo motors, stepper motors, and sensors.

- Select and configure a suitable battery or battery pack according to actuator power requirements.

- Use voltage regulators or DC-DC converters to provide stable voltage to sensitive components.

- Ensure smooth operation of actuators without voltage drop during operation.

- Demonstrate correct wiring and safe power distribution connections in a robotic system.

- Measuring Battery Backup: Checking voltage under noload and load conditions.

- Backup Time Calculation: Backup Time (hours) = Battery Capacity (Ah) ÷ Load Current (A)

- Testing and Safety: Ensuring voltage remains within safe limits and results are properly documented.

- It should be recorded for maintenance and performance analysis.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Test battery voltage, current, and runtime in practice

- Calculate the expected operating runtime based on battery capacity and motor current consumption.

Tools and Equipment Required for CS:

| SR. NO | NAME | QTY. REQUIRED |
| --- | --- | --- |
| 01 | Primary Batteries (Non-Rechargeable) Zinc-Carbon Battery – 1.5V Alkaline Battery – 1.5V Lithium Primary Battery (Coin Cell – CR2032) – 3V | 15 |
| 02 | Secondary batteries Lead-Acid Battery – 6V / 12V Nickel-Cadmium (Ni-Cd) Battery – 1.2V Nickel-Metal Hydride (NiMH) Battery – 1.2V Lithium-Ion (Li-ion) Battery (18650) – 3.7V | 15 |
| 03 | Digital multimeter | 01 |
| 04 | Clamp meter (optional) | 12 |
| 05 | Battery tester | 13 |
| 06 | Battery holders | 15 |
| 07 | Connecting wires (red and black) | 50M |
| 08 | Alligator clips | 50 |
| 09 | Soldering kit (iron, solder, flux) | 13 |
| 10 | Voltage regulators / buck converters | 13 |
| 11 | Actuators: DC motors, servo motors, stepper motors | 4*13 |
| 12 | Microcontroller (Arduino kit, ESP32, etc.) | 15 |
| 13 | Insulating tape and heat shrink tubes | 15 |
| 14 | Safety gloves | 15 |
| 15 | Fireproof tray (for Li-ion/Li-Po batteries) | 15 |
| 16 | Resistors or dummy loads (for testing) | 15 |
| 17 | Stopwatch or timer | 15 |
| 18 | Notebook or log sheet | 15 |
| 19 | Screwdrivers | 15 |
| 20 | Wire strippers or scissors | 15 |
| 21 | Camera or phone (optional, for documentation) | 15 |

# Tools, equipment and non-consumables for the qualification

The tools and equipment for this Qualification are below:

| Sr. No | Name | Quantity |
| --- | --- | --- |
| 1 | CAD Workstations (min. i5/i7, 16GB RAM, GPU, SSD) | 12 |
| 2 | Computer System (8th Generation,8GB RAM,512 Hardware, Software (Compatible for C++) | 12 |
| 3 | Digital Calipers (0.01 mm resolution) | 12 |
| 4 | Tablet or Mobile Device | 8 |
| 5 | 3D Printers – FDM Typ | 4 |
| 6 | Benchtop CNC Milling Machines – 3-Axis | 4 |
| 7 | Metal CNC Milling Machine (Benchtop, 3-axis, for aluminum parts) | 4 |
| 8 | PCB Milling Machine (Benchtop CNC, suitable for 2-layer FR4, 0.2 mm trace width) | 4 |
| 9 | Laser Scanner (≤ 0.1 mm accuracy) | 4 |
| 10 | CNC Tooling Kits (Cutters, clamps, wrenches, ER collets) | 4 |
| 11 | Micrometers (0–25 mm, 0.001 mm resolution) | 12 |
| 12 | Dial Indicators with Magnetic Base | 12 |
| 13 | General Hand Tool Kits | 25 |
| 14 | Mechanical Tool Kit | 12 |
| 15 | Electrical Tool Kit Screwdriver Sets (Flathead, Phillips, Insulated) | 12 |
| 16 | Allen/Hex Key Sets | 12 |
| 17 | Spanners and Wrenches | 12 |
| 18 | Pliers (Needle-nose, Combination) | 12 |
| 19 | Wire Cutters and Strippers | 12 |
| 20 | Crimping Tools | 12 |
| 21 | Tubing Cutter and Hose Clamp Tools | 8 |
| 22 | Deburring and Finishing Tools | 8 |
| 23 | Torque Wrench (2–10 Nm, Small) | 8 |
| 24 | Workbenches with Vices | 12 |
| 25 | Fume Extraction Units or Ventilated Enclosures | 8 |
| 26 | First Aid Kits | 8 |
| 27 | Fire Extinguishers | 8 |
| 28 | Safety Signs & Labels | 4 |
| 29 | Emergency Stop Button (Demo Unit) | 8 |
| 30 | Emergency Stop Switches | 8 |
| 31 | Safety Relay Modules | 12 |
| 32 | Networking Equipment (RJ-45 CAT-6 cables) | 30 m |
| 33 | USB, Serial, and Ethernet Cables | 30 m |
| 34 | Basic Networking tool Kit | 8 |
| 35 | USB to RS232/RS485 Converter | 4 |
| 36 | Multimeters (True RMS, Digital, with Continuity and DC Current Modes) | 8 |
| 37 | Oscilloscopes (≥ 50 MHz, 2–4 channel) | 8 |
| 38 | Ethernet Switch (Managed/Unmanaged) 24 port | 4 |
| 39 | Phase Sequence Indicators | 8 |
| 40 | Power Analyzers | 8 |
| 41 | Voltage Tester / Continuity Tester | 8 |
| 42 | Breadboards (830-point, Solderless) | 26 |
| 43 | Veroboards / Stripboards (100×160 mm, copper tracks) | 26 |
| 44 | Microcontroller Boards (Arduino Uno or compatible, ESP32) | 8 |
| 45 | Soldering Stations (Temperature-controlled with fine tips, ESD protection) | 12 |
| 46 | PCB Drills (Mini drill press with 0.8–1.0 mm bits) | 8 |
| 47 | Logic Analyzers | 8 |
| 48 | Signal Generators | 8 |
| 49 | DC Motors (3–12V, 100–300 RPM, geared | 12 |
| 50 | Stepper Motors (NEMA 17 or similar, 1.8° step angle | 12 |
| 51 | Servo Motors (SG90 or MG996R, PWM-controlled) | 12 |
| 52 | Motor Driver Modules (L298N, L293D for DC/stepper motors) | 12 |
| 53 | Motor Drivers / Controllers | 12 |
| 54 | Variable Power Supplies (DC Bench, 0–30V, dual channel, ≥ 5A) | 12 |
| 55 | Battery Packs (4×AA or Li-ion with holder and switch) | 12 |
| 56 | Mechanical Kits (Gears, pulleys, shafts, brackets, couplings) | 12 |
| 57 | Chassis Frames or Mounting Boards (for assembling motor systems) | 12 |
| 58 | Tachometers (Laser/contact type for motor RPM measurement | 8 |
| 59 | Torque Sensors | 8 |
| 60 | Load Resistors / Mechanical Loads | 8 |
| 61 | Printer/Scanner | 8 |
| 62 | Robotic Workstations (5 sets + 1 spare, industrial-grade robotic arm, 4- or 6-DOF, controller with teach pendant, mounted base, safety enclosure) | 8 |
| 63 | Entry-level Robot Arms/Workstation (DOBOT, Fanuc LR Mate, UR3e, FANUC, ABB, Yaskawa, KUKA) | 8 |
| 64 | Mobile Robot Platforms with Arms and Grippers (TurtleBots) | 8 |
| 65 | Robotic Base Platforms or Stands | 8 |
| 66 | End Effectors (Parallel jaw grippers, Vacuum suction, Magnetic grippers) | 8 |
| 67 | Actuators (Servo motors, Stepper motors, Linear actuators) | 8 |
| 68 | Robot Teach Pendants | 8 |
| 69 | Teach Pendant Simulators | 8 |
| 70 | HMI or Teach Pendant | 8 |
| 71 | System Control Panels or HMI Terminals | 8 |
| 72 | Sensor Kits (Proximity, Force/Torque, Vision, Temperature, Light, Depth cameras, LiDAR, Ultrasonic) | 8 |
| 72 | I/O and Communication Interface Modules | 8 |
| 73 | Basic I/O Panels for Triggering Events | 8 |
| 74 | PLC Units with Communication Support | 8 |
| 75 | Industrial PLC Kit | 8 |
| 76 | Industrial HMI Panel | 8 |
| 77 | Embedded PC or Panel PC | 8 |
| 78 | Vision Camera | 8 |
| 79 | Relays/Solid State Relays | 8 |
| 80 | Debugging Cable Set | 8 |
| 81 | Diagnostic Tools (e.g., Signal Testers) | 8 |
| 82 | Basic Diagnostic Cables | 8 |
| 83 | Heat Gun | 8 |
| 84 | Pressure Gauges | 8 |
| 85 | Air Compressor | 8 |
| 86 | Basic Hydraulic Test Equipment | 8 |
| 87 | Measuring Tapes and Rulers | 8 |
| 89 | Angle Gauges/Alignment Tools | 8 |
| 90 | Laser Alignment and Calibration Tools | 8 |
| 91 | Calibration Benches with Reference Standards | 8 |
| 92 | Calibration Toolkit | 8 |
| 93 | Sensor Calibration Toolkit | 8 |
| 94 | Actuator Test Setup | 8 |
| 95 | Signal Conditioning Module | 8 |
| 96 | Vibration Meter | 8 |
| 97 | Vibration Tester | 8 |
| 98 | Thermal Imaging Camera | 8 |
| 99 | Infrared Thermometer | 8 |
| 100 | Clamp Meter (AC/DC, up to 1000 A, true RMS, inrush current detection) | 12 |
| 101 | Data Logger | 8 |
| 102 | Monitoring Dashboard | 4 |
| 103 | Signal Tracer / Logic Analyzer | 4 |
| 104 | Communication Protocol Sniffer (Modbus RTU/TCP, Ethernet/IP, real-time packet inspection) | 4 |
| 105 | Tool Carts or Storage Cabinets | 8 |
| 106 | Whiteboards and Monitors for Instruction | 2 |
| 107 | Projectors or Smart Displays | 2 |
| 108 | Robotic Training Kits or Simulation Platforms (Arduino-based or PLC-integrated) | 4 |
| 109 | Emergency System Tester | 4 |
| 110 | Safety Simulation Kits | 4 |
| 111 | Communication Tools (Two-way Radio) | 4 |
| 112 | Filing Cabinet or Digital Archiving System | 4 |
| 113 | Barcode Scanner or RFID Reader (for equipment ID logging) | 4 |
| 114 | Label Printer and Tags (for system isolation labeling) | 4 |
| 115 | Camera or Mobile Device (for capturing error codes, faults, and evidence) | 4 |
| 116 | Anti-Static Mats | 8 |
| 117 | Anti-Static Wrist Straps & ESD Mats | 8 |
| 118 | Cable Management Solutions (cable trays, ties, and organizers) | 8 |
| 119 | Backup Power Supplies (UPS) | 8 |
| 120 | ESD-safe Storage Bins* (for organizing electronic components) | 8 |
| 121 | Calibration Weights* (for precise equipment calibration) | 8 |
| 122 | CNC Controller Software Licenses | 4 |
| 123 | 3D Slicing Software (e.g., Cura) | 4 |
| 124 | Basic PCB Design Software (e.g., KiCad, Eagle, open source/institutional license) | 4 |
| 125 | OS Bootable Media / USB Drives | 4 |
| 126 | Internet Access + Wi-Fi | 4 |
| 127 | ChatGPT/Gemini/LLM Tool Access | 4 |
| 128 | Multi-Robot Workflow Software (e.g., RoboDK, V-REP/CoppeliaSim, ROS tools) | 4 |
| 129 | PLC Programming Software | 4 |
| 130 | PLC Simulation Software | 4 |
| 131 | Robot Programming Software (e.g., RoboDK, RobotStudio, TIA Portal, ROS) | 4 |
| 132 | Robot Simulator (Desktop) | 4 |
| 133 | Document Editing and Spreadsheet Software (e.g., MS Word, Excel, Google Sheets) | 8 |
| 134 | PDF Annotation Tools (e.g., Adobe Acrobat, PDF editors) | 8 |
| 135 | CMMS or ERP System | 8 |
| 136 | Version Control System (e.g., Git, SharePoint, Excel-based change trackers) | 8 |
| 137 | Text Editor or Programming IDE (e.g., VS Code, C++ IDE) | 8 |
| 138 | Screenshot and Annotation Tools (e.g., Snagit, Lightshot) | 8 |
| 139 | Flowchart & Diagram Tools | 8 |
| 140 | Documentation Software (e.g., MS Word, Google Docs, Adobe Acrobat) | 8 |
| 141 | Protocol Sniffer Software | 4 |
| 142 | Serial Monitor or Debugging Tools | 4 |
| 143 | C++ to PLC Integration Kit | 4 |
| 144 | OpenCV Library (C++) | 4 |
| 145 | Digital Compliance Tracker / App | 4 |
| 146 | Diagnostic Software (e.g., RobotStudio, TIA Portal Viewer) | 4 |
| 147 | Training Presentation Kit | 4 |
| 148 | Instructional Manuals and Maintenance SOPs | 8 |
| 149 | Instructional Worksheets / Circuit Diagrams (Laminated or digital) | 8 |
| 150 | Technical Documentation and Reference Manuals (robot specs, SOPs, user guides) | 8 |
| 151 | Service Logbooks and Job Order Templates | 8 |
| 152 | Digital Logbook or Service Management System | 8 |
| 153 | Technical Catalogs and Equipment Manuals | 8 |
| 154 | Checklists and Assessment Forms | 8 |
| 155 | PPE Audit Checklist | 8 |
| 156 | Industrial Safety Standard Book | 8 |
| 157 | Inspection Checklist Sheets | 8 |
| 158 | Inspection and Audit Templates | 8 |
| 159 | PPE Inventory Management Log | 8 |
| 160 | Safety Drill Log Book / Register | 8 |
| 161 | Risk Assessment Templates / Forms | 8 |
| 162 | Digital Inventory/Requisition System | 8 |
| 163 | Communication and Feedback Platforms (e.g., Email, MS Teams, internal ticketing system) | 8 |
| 164 | Path Visualization Tools (trajectory overlays, time stamps) | 8 |
| 165 | Stopwatch or Cycle Time Measurement Utilities | 8 |
| 166 | Digital I/O Trainer Board | 8 |
| 167 | Word Processors, Templates for Change Control Records | 8 |
| 168 | Fireproof tray (for Li-ion/Li-Po batteries) | 15 |
| 169 | Simplified SCADA Software | 4 |

# The consumables for this Qualification are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | 3D Printing Filament (PLA/PETG, 1kg spools) | 5 |
| 2 | Secondary batteries (Lead-Acid, Ni-Cd, NiMH, Li-ion 18650) | 12 |
| 3 | Primary batteries (Zinc-Carbon, Alkaline, CR2032) | 12 |
| 4 | Connecting wires (red & black) | As required |
| 5 | Alligator clips | As required |
| 6 | Jumper Wire Sets (Male-male, male-female, assorted lengths) | As required |
| 8 | Resistors | As required |
| 9 | IC Kits | 10 |
| 10 | Electrical Connectors and Terminal Blocks (Assorted kits) | 10 |
| 11 | Connectors, Cables, Terminals | As required |
| 12 | Writing Notebook | 12 |
| 13 | Pen/Pencil | 12 |
| 14 | Marker Pens* (for labeling and documentation) | 5 |
| 15 | Paper Rim (A4 Size) | 1 Pack |
| 16 | Grease Gun (Grease) | 5 |
| 17 | Lubricants & Cleaners | 5 |
| 18 | Cleaning Brushes & Cloths | 5 |
| 19 | Component Cleaning Kit | 5 |
| 20 | Insulation Tape / Heat Shrink Kit | 5 |
| 21 | Labeling and Tagging Materials | 5 |
| 22 | PPE Kits (Gloves, Goggles, Helmet, Shoes, Safety Glasses, Anti-static wristbands) | 25 |
| 23 | Lockout/Tagout Kits | 5 |
| 24 | Soldering Wire | 5 |
| 25 | Cleaning Supplies (Isopropyl alcohol, wipes, and brushes) | 5 |
| 26 | Adhesive Tapes (Electrical tape, double-sided tape) | 5 |

# Members of Qualification Development & Review Committee

The following members participated in the qualification development and review process:

| Qualification Development Committee |  |  |
| --- | --- | --- |
| S. No. | Name | Organization |
|  | Dr. Muhammad Ayub Tareen | Associate Professor, Department of Electronic Engineering, BUITEMS, Quetta |
|  | Dr. Humera Tariq | Assistant Professor, Computer Science Department (UBIT), University of Karachi |
|  | Dr Muhammad Hamza Akhlaq | Assistant Professor, AL-EYS, Islamabad |
|  | Dr. Kashif Ishaque | Professor & Director ORIC, Mohammad Ali Jinnah University |
|  | Dr. Khalid Rasheed Shaikh | Assistant Professor and Program Manager MSCS, SZABIST university Karachi |
|  | Dr. Abdul Rehman Abbasi | Manager (Academics), KINPOE -College of PIEAS, Karachi |
|  | Dr Anam Abid | Assistant Professor, University of Engineering and Technology Peshawar |
|  | Mr. Yassen Jaffar | CEO, ICON Solutions, Quetta |
|  | Mr. Muhmmad Aasim | Deputy Director, NAVTTC HQ |
|  | Kanwal Iqbal | Lecturer, Computer Science Department ,Mohammad Ali Jinnah University Karachi |
|  | Jamshaid Dastur | CEO Smartronix |
|  | Engr Summaya Imran | Assistant Professor, CUSIT, Peshawar |
| 13. | Engr. Mashaal Jawad | CEO STEMverse |
| 14. | Ms. Amber Lal | Assistant Professor, City University Peshawar |
| 15. | Engr. Javera Siraj | Research Assistant, NCAI, University of Engineering and Technology Peshawar |
| 16. | Engr. Aaisha Abid | Research Assistant, NCAI University of Engineering and Technology Peshawar |
| 17. | Mr. Ayaz Khan | Lecturer Technical Training Center Peshawar |
| 18. | Mr Ateeq Ur Rehman | Assistant Professor, GCT Peshawar |
| 19. | Engr. Kashif Mehmood | IoT and Network solutions, Islamabad |
| 20. | Mr Adeel Ahmad | IT In charge Islamabad |
| 21. | Abdul Basit | DACUM Facilitator |