Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 2 in Automation & Process Control Basic Electrical Engineering (Level 2) |
| --- | --- |
| Competency Standard | - Operate Measuring Instruments - Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits. - Verify Law of Combination of Capacitor - Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence - Implement Electromagnet to See Various Effects & Verify Faradays Law |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Make parallel circuit and measure the power absorbed by circuit and verify KCL. - Measure frequency of AC signal with oscilloscope - Implement a series circuit of capacitors with two different scheme (A) Circuit with three same value of capacitances (B) Circuit with three different value of capacitances. |
| Time: 04Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | •Make parallel circuit and measure the power absorbed by circuit and verify KCL. 1. Draw the circuit diagram of parallel connected resistor of different values with suitable DC power source .(9v battery) 2. Find the values of resistor using color coding and record and calculate equivalent resistance. 3. Measure the values of resistor and verify the recorded values of resistance. 4. Implement circuit according to diagram on bread board and connect to power supply. 5. Check the continuity of connections 6. Measure the total voltage of parallel network. 7. Measure the total current of network and separate branch current and record the values. 8. Verify Kirchhoff’s Current law and Ohm’s law by finding the equivalent resistance. 9. Calculate the power absorb by the circuit. • Measure frequency of AC signal with oscilloscope 1. Select a proper time/division and give AC voltage to oscilloscope. 2. Calculate number of divisions between the one AC cycles of the AC signal. 3. Apply the formula to calculate the actual frequency of AC voltage. 4. Apply formula to calculate the period of signal. • Implement a series circuit of capacitors with two different scheme (A) Circuit with three same value of capacitances (B) Circuit with three different value of capacitances 1. Assemble a series circuit of capacitors with 3 same capacitance values. 2. Connect the battery and apply 10 voltages to the combination of capacitors 3. Measure the voltage across each capacitor connected in series 4. Add the voltage of each capacitor and verify is it equal to applied voltage. 5. Use voltage of each capacitor and its value to find the charge on each capacitor. 6. Verify the amount of charge store must be equal. 7. Assemble a series circuit of capacitors with 3 different values. 8. Connect the battery and apply 10 voltages to the combination of capacitors 9. Measure the voltage across each capacitor connected in series 10. Add the voltage of each capacitor and verify is it equal to applied voltage. 11. Use voltage of each capacitor and its value to find the charge on each capacitor. 12. Verify the amount of charge store must be equal. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 2 in Automation & Process Control Basic Electrical Engineering (Level 2) |
| Competency Standard | - Operate Measuring Instruments - Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits. - Verify Law of Combination of Capacitor - Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence - Implement Electromagnet to See Various Effects & Verify Faradays Law |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - Make parallel circuit and measure the power absorbed by circuit and verify KCL.. - Implement a series circuit of capacitors with two different scheme (A) Circuit with three same value of capacitances (B) Circuit with three different value of capacitances. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - 1.Draw the circuit diagram of parallel connected resistor of different values with suitable DC power source (9v battery) |  |  |
| - 2. Find the values of resistor using color coding and record and calculate equivalent resistance. |  |  |
| - 3. Measure the values of resistor and verify the recorded values of resistance. |  |  |
| - Implement circuit according to diagram on bread board and connect to power supply. |  |  |
| - 5.Check the continuity of connections |  |  |
| - 6 .Measure the total voltage of parallel network. |  |  |
| - 7. Measure the total current of network and separate branch current and record the values. |  |  |
| - 8. Verify Kirchhoff’s Current law and Ohm’s law by finding the equivalent resistance. |  |  |
| - 9. Calculate the power absorb by the circuit. |  |  |
| - 10. Select a proper time/division and give AC voltage to oscilloscope. |  |  |
| - 11. Calculate number of divisions between the one AC cycles of the AC signal. |  |  |
| - 12. Apply the formula to calculate the actual frequency of AC voltage. |  |  |
| - 13. Apply formula to calculate the period of signal. |  |  |
| - Assemble a series circuit of capacitors with 3 same capacitance values. |  |  |
| - Connect the battery and apply 10 voltages to the combination of capacitors |  |  |
| - Measure the voltage across each capacitor connected in series |  |  |
| - Add the voltage of each capacitor and verify is it equal to applied voltage. |  |  |
| - Use voltage of each capacitor and its value to find the charge on each capacitor. |  |  |
| - Verify the amount of charge store must be equal. |  |  |
| - Assemble a series circuit of capacitors with 3 different values. |  |  |
| - Connect the battery and apply 10 voltages to the combination of capacitors |  |  |
| - Measure the voltage across each capacitor connected in series |  |  |
| - Add the voltage of each capacitor and verify is it equal to applied voltage. |  |  |
| - Use voltage of each capacitor and its value to find the charge on each capacitor. |  |  |
| - Connect phase line of AC supply to the energy meter. |  |  |
| - Connect AC load to the output terminals of energy meter. |  |  |
| - Power up the supply and take reading of energy in terms of unit after few minutes form display. |  |  |
| - Connect voltmeter & ampere meter with inductive load (Motor) |  |  |
| - Connect watt meter and measure. |  |  |
| - Connect AC supply and take the readings of volt, ampere and watt meter. |  |  |
| - Utilize the above readings and calculate power factor using power formula P=VI Cos Ø |  |  |
| - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |
| - Connect Galvanometer with coil. |  |  |
| - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. |  |  |
| - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. |  |  |
| - Verify the amount of charge store must be equal. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 2 in Automation & Process Control Basic Electrical Engineering (Level 2) |
| --- | --- |
| Competency Standard | - Operate Measuring Instruments - Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits. - Verify Law of Combination of Capacitor - Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence - Implement Electromagnet to See Various Effects & Verify Faradays Law |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Make parallel circuit and measure the power absorbed by circuit and verify KCL. - Implement a series circuit of capacitors with two different scheme (A) Circuit with three capacitors of same values of capacitances (B) Circuit with three capacitor of different values of capacitances |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  |  | Yes |  | No |  | Remarks |  |
|  | Drew the circuit diagram of parallel connected resistor of different values with suitable DC power source .(9v battery) |  |  |  |  |  |  |  |  |
|  | Found the values of resistor using color coding and record and calculate equivalent resistance. |  |  |  |  |  |  |  |  |
|  | Measured the values of resistor and verify the recorded values of resistance. |  |  |  |  |  |  |  |  |
|  | Implemented circuit according to diagram on bread board and connect to power supply. |  |  |  |  |  |  |  |  |
|  | Checked the continuity of connections |  |  |  |  |  |  |  |  |
|  | Measured the total voltage of parallel network. |  |  |  |  |  |  |  |  |
|  | Measured the total current of network and separate branch current and record the values. |  |  |  |  |  |  |  |  |
|  | Verified Kirchhoff’s Current law and Ohm’s law by finding the equivalent resistance. |  |  |  |  |  |  |  |  |
|  | Calculated the power absorb by the circuit. |  |  |  |  |  |  |  |  |
|  | Selected a proper time/division and give AC voltage to oscilloscope. |  |  |  |  |  |  |  |  |
|  | Calculated number of divisions between the one AC cycles of the AC signal. |  |  |  |  |  |  |  |  |
|  | Applied the formula to calculate the actual frequency of AC voltage. |  |  |  |  |  |  |  |  |
|  | Applied formula to calculate the period of signal. |  |  |  |  |  |  |  |  |
|  | Assembled a series circuit of capacitors with 3 same capacitance values. |  |  |  |  |  |  |  |  |
|  | Connected the battery and apply 10 voltages to the combination of capacitors |  |  |  |  |  |  |  |  |
|  | Measured the voltage across each capacitor connected in series |  |  |  |  |  |  |  |  |
|  | Added the voltage of each capacitor and verify is it equal to applied voltage. |  |  |  |  |  |  |  |  |
|  | Used voltage of each capacitor and its value to find the charge on each capacitor. |  |  |  |  |  |  |  |  |
|  | Verified the amount of charge store must be equal. |  |  |  |  |  |  |  |  |
|  | Assembled a series circuit of capacitors with 3 different values. |  |  |  |  |  |  |  |  |
|  | Connected the battery and apply 10 voltages to the combination of capacitors |  |  |  |  |  |  |  |  |
|  | Measured the voltage across each capacitor connected in series |  |  |  |  |  |  |  |  |
|  | Added the voltage of each capacitor and verify is it equal to applied voltage. |  |  |  |  |  |  |  |  |
|  | Used voltage of each capacitor and its value to find the charge on each capacitor. |  |  |  |  |  |  |  |  |
|  | Connected phase line of AC supply to the energy meter. |  |  |  |  |  |  |  |  |
|  | Connected AC load to the output terminals of energy meter. |  |  |  |  |  |  |  |  |
|  | Powered up the supply and took reading of energy in terms of unit after few minutes form display. |  |  |  |  |  |  |  |  |
|  | Connected voltmeter & ampere meter with inductive load (Motor) |  |  |  |  |  |  |  |  |
|  | Connected watt meter and measure. |  |  |  |  |  |  |  |  |
|  | Connected AC supply and take the readings of volt, ampere and watt meter. |  |  |  |  |  |  |  |  |
|  | Utilized the above readings and calculated power factor using power formula P=VI Cos Ø |  |  |  |  |  |  |  |  |
|  | Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |  |  |  |  |  |  |
|  | Connected Galvanometer with coil. |  |  |  |  |  |  |  |  |
|  | Moved permanent magnet inside the coil fast and slow and recorded the effect on reading of Galvanometer. |  |  |  |  |  |  |  |  |
|  | Hold the magnet inside the coil and did not move, now record the effect on reading of Galvanometer. |  |  |  |  |  |  |  |  |
|  | Verified the amount of charge store must be equal. |  |  |  |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |  |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 2 in Automation & Process Control Basic Electrical Engineering (Level 2) |
| --- | --- |
| Competency Standard | - Operate Measuring Instruments - Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits. - Verify Law of Combination of Capacitor - Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence - Implement Electromagnet to See Various Effects & Verify Faradays Law |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  |  |  | Satisfactory |  | Not Satisfactory |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | The unit of current is ________ |  |  |  |  |
|  |  |  | - Ampere - volt - ohm - farad |  |  |  |  |
|  |  |  | Unit of capacitance ……………… |  |  |  |  |
|  |  |  | - Ohm - Farad - ampere - henry |  |  |  |  |
|  |  |  | Volt meter is used in series with load to measure voltage |  |  |  |  |
|  |  |  | - TRUE - FALSE |  |  |  |  |
|  |  |  | Ohm meter is used to measure |  |  |  |  |
|  |  |  | a) voltage b) current c)resistance d) capacitance |  |  |  |  |
|  |  |  | Describe the formula of electric current |  |  |  |  |
|  |  |  | Describe the formula of power absorbed by resistor |  |  |  |  |
|  |  |  | State Ohm’s Law and express mathematical formula |  |  |  |  |
|  |  |  | Kirchhoff’s voltage law is applicable on open Circuit. |  |  |  |  |
|  |  |  | - TRUE - FALSE |  |  |  |  |
|  |  |  | Current is divided in series circuit in multiple paths |  |  |  |  |
|  |  |  | - TRUE - FALSE |  |  |  |  |
|  | The unit of power is ________ |  |  |  |  |  |  |
|  | - Ampere - volt - ohm - Watt |  |  |  |  |  |  |
|  | Unit of Power factor ……………… |  |  |  |  |  |  |
|  | - moh - Farad - Volt- Ampere - no units |  |  |  |  |  |  |
|  | Wattmeter is used in measure Electrical power drawn by load |  |  |  |  |  |  |
|  | - TRUE - FALSE |  |  |  |  |  |  |
|  | Ohm meter is used to measure |  |  |  |  |  |  |
|  | a) voltage b) current c) resistance d) capacitance |  |  |  |  |  |  |
|  | Describe the formula of electric power consumed by load from AC source. |  |  |  |  |  |  |
|  | Describe the formula of Electrical Energy dissipation in resistor |  |  |  |  |  |  |
|  | State Faradays’ Law of electromagnetic Induction |  |  |  |  |  |  |
|  | Electric Motors and generators works on principal of faradays law |  |  |  |  |  |  |
|  | - TRUE - FALSE |  |  |  |  |  |  |
|  | KVA is unit used for Real power consumed by load |  |  |  |  |  |  |
|  | - TRUE - FALSE |  |  |  |  |  |  |
|  | Galvanometer is used to detect the current. |  |  |  |  |  |  |
|  | - TRUE - FALSE |  |  |  |  |  |  |
| 20. |  |  | Home appliances are connected in parallel combinations |  |  |  |  |
|  |  |  | - TRUE - FALSE |  |  |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________