Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 3 in Automation & Process Control Electronics (Analog) (Level 3) |
| --- | --- |
| Competency Standard | - Identify Basic Electronics Components - Carry out Diode Application - Implement Bipolar Junction Transistor (BJT) in Different Applications - Implement Field Effect Transistor (FET) in Different Applications - Implement Thyristor Family in Various Application - Design a Rectifier using Diode - Applications of Operation Amplifier |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Implement Transistor as an amplifier using CE Configuration - Design the circuit of Common Gate (CS) amplifier - Construct the fan speed regulator circuit using Diac & Triac as shown in figure - Design Full Wave Bridge Rectifier - Construct an Inverting amplifier using operational amplifier - Construct a Comparator circuit using operational amplifier |
| Time: 04Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Implement Transistor as an amplifier using CE Configuration. - Draw the Circuit of CE configuration of transistor - Select the components for CE configurations. - Place the components for CE amplifier - Calculate the gain of transistor in CE modes. - Draw VI characteristics curve for CE Design the circuit of Common Source (CS) amplifier - Identify the FET and their terminal (gate, drain and Sources) whit the Help of Datasheet - Select the components for Common Source (CS) amplifier - Implement the circuit of Common Source (CS) amplifier - Analyze the different parameter of Common Source (CS) amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Common Source (CS) amplifier Construct the fan speed regulator circuit using Diac & Triac as shown in figure - Keep the variable resistor or potentiometer in maximum resistance position so that no triggering is applied to TRIAC and hence the TRIAC will be in cutoff mode. - Turn ON the power supply of the circuit and observe whether the fan is in standstill condition or not. Vary the potentiometer position slowly so that the capacitor starts charging at the time constant determined by the values of R1 and R2. - Once the voltage across the capacitor is more than the break over voltage of the DIAC, DIAC starts conducting. Thus, the capacitor starts discharging towards the gate terminal of TRIAC through DIAC. - Therefore, TRIAC starts conducting and hence the main current starts flowing into the fan through the closed path formed by TRIAC. - By varying the potentiometer R2, the rate at which capacitor is going to be charged get varied this means that if the resistance is less, the capacitor will charge at a faster rate so the earlier will be the conduction of TRIAC. - As the potentiometer resistance gradually increases, the conduction angle of TRIAC will be reduced. Hence the average power across the load will be varied. - Due to the bidirectional control capability of both TRIAC and DIAC, it is possible to control the firing angle of the TRIAC in both positive and negative peaks of the input. Design Full Wave Bridge Rectifier - Identify Full Wave Rectifier components - Draw circuit Diagram of Full Wave Rectifier - Observe the INPUT and Output wave form on oscilloscope - Calculate the ripple Factor - Calculate output voltage Construct an Inverting amplifier using operational amplifier - Identify the Operational amplifier terminals (Inverting, Non-inverting Inputs & Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of Inverting Op-Amp. - Select the components for Inverting Op-Amp. - Implement Inverting Op-Amp circuit. - Perform the operations of Inverting Op-Amp circuit. - Measure the output voltage & gain - Generate the Output report Construct a Comparator circuit using operational amplifier - Draw the Schematic diagram of Comparator circuit using Op-Amp. - Select the components for Comparator circuit. - Implement Comparator circuit. - Perform the operations of Comparator circuit. - Check the output for different input conditions |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 3 in Automation & Process Control Electronics (Analog) (Level 3) |
| Competency Standard | - Identify Basic Electronics Components - Carry out Diode Application - Implement Bipolar Junction Transistor (BJT) in Different Applications - Implement Field Effect Transistor (FET) in Different Applications - Implement Thyristor Family in Various Application - Design a Rectifier using Diode - Applications of Operation Amplifier |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - Implement Transistor as an amplifier using CE Configuration - Design the circuit of Common Gate (CS) amplifier - Construct the fan speed regulator circuit using Diac & Triac as shown in figure - Design Full Wave Bridge Rectifier - Construct an Inverting amplifier using operational amplifier - Construct a Comparator circuit using operational amplifier |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Draw the Circuit of CE configuration of transistor |  |  |
| - Select the components for CE configurations. |  |  |
| - Place the components for CE amplifier |  |  |
| - Calculate the gain of transistor in CE modes. |  |  |
| - Draw VI characteristics curve for CE |  |  |
| - Identify the FET and their terminal (gate, drain and Sources) whit the Help of Datasheet |  |  |
| - Select the components for Common Source (CS) amplifier |  |  |
| - Implement the circuit of Common Source (CS) amplifier |  |  |
| - Analyze the different parameter of Common Source (CS) amplifier |  |  |
| - Monitor the Output waveform on oscilloscope |  |  |
| - Draw the characteristic curves of Common Source (CS) amplifier |  |  |
| - Keep the variable resistor or potentiometer in maximum resistance position so that no triggering is applied to TRIAC and hence the TRIAC will be in cutoff mode. |  |  |
| - Turn ON the power supply of the circuit and observe whether the fan is in standstill condition or not. Vary the potentiometer position slowly so that the capacitor starts charging at the time constant determined by the values of R1 and R2. |  |  |
| - Once the voltage across the capacitor is more than the break over voltage of the DIAC, DIAC starts conducting. Thus, the capacitor starts discharging towards the gate terminal of TRIAC through DIAC. |  |  |
| - Therefore, TRIAC starts conducting and hence the main current starts flowing into the fan through the closed path formed by TRIAC. |  |  |
| - By varying the potentiometer R2, the rate at which capacitor is going to be charged get varied this means that if the resistance is less, the capacitor will charge at a faster rate so the earlier will be the conduction of TRIAC. |  |  |
| - As the potentiometer resistance gradually increases, the conduction angle of TRIAC will be reduced. Hence the average power across the load will be varied. |  |  |
| - Identify Full Wave Rectifier components |  |  |
| - Draw circuit Diagram of Full Wave Rectifier |  |  |
| - Observe the INPUT and Output wave form on oscilloscope |  |  |
| - Calculate the ripple Factor |  |  |
| - Calculate output voltage |  |  |
| - Identify the Operational amplifier terminals (Inverting, Non-inverting Inputs & Outputs) with the Help of Datasheet |  |  |
| - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. |  |  |
| - Draw the Schematic diagram of Inverting Op-Amp. |  |  |
| - Select the components for Inverting Op-Amp. |  |  |
| - Implement Inverting Op-Amp circuit. |  |  |
| - Perform the operations of Inverting Op-Amp circuit. |  |  |
| - Measure the output voltage & gain |  |  |
| - Generate the Output report |  |  |
| - Draw the Schematic diagram of Comparator circuit using Op-Amp. |  |  |
| - Select the components for Comparator circuit. |  |  |
| - Implement Comparator circuit. |  |  |
| - Perform the operations of Comparator circuit. |  |  |
| - Check the output for different input conditions |  |  |
| - Due to the bidirectional control capability of both TRIAC and DIAC, it is possible to control the firing angle of the TRIAC in both positive and negative peaks of the input. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 3 in Automation & Process Control Electronics (Analog) (Level 3) |
| --- | --- |
| Competency Standard | - Identify Basic Electronics Components - Carry out Diode Application - Implement Bipolar Junction Transistor (BJT) in Different Applications - Implement Field Effect Transistor (FET) in Different Applications - Implement Thyristor Family in Various Application - Design a Rectifier using Diode - Applications of Operation Amplifier |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Implement Transistor as an amplifier using CE Configuration - Design the circuit of Common Gate (CS) amplifier - Construct the fan speed regulator circuit using Diac & Triac as shown in figure - Design Full Wave Bridge Rectifier - Construct an Inverting amplifier using operational amplifier - Construct a Comparator circuit using operational amplifier |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes |  | No |  | Remarks |  |
|  | Drew the Circuit of CE configuration of transistor |  |  |  |  |  |  |  |  |  |
|  | Selected the components for CE configurations. |  |  |  |  |  |  |  |  |  |
|  | Placed the components for CE amplifier |  |  |  |  |  |  |  |  |  |
|  | Calculated the gain of transistor in CE modes. |  |  |  |  |  |  |  |  |  |
|  | Drew VI characteristics curve for CE |  |  |  |  |  |  |  |  |  |
|  | Identified the FET and their terminal (gate, drain and Sources) whit the Help of Datasheet |  |  |  |  |  |  |  |  |  |
|  | Selected the components for Common Source (CS) amplifier |  |  |  |  |  |  |  |  |  |
|  | Implemented the circuit of Common Source (CS) amplifier |  |  |  |  |  |  |  |  |  |
|  | Analyzed the different parameter of Common Source (CS) amplifier |  |  |  |  |  |  |  |  |  |
|  | Monitored the Output waveform on oscilloscope |  |  |  |  |  |  |  |  |  |
|  | Drew the characteristic curves of Common Source (CS) amplifier |  |  |  |  |  |  |  |  |  |
|  | Kept the variable resistor or potentiometer in maximum resistance position so that no triggering is applied to TRIAC and hence the TRIAC will be in cutoff mode. |  |  |  |  |  |  |  |  |  |
|  | Turned ON the power supply of the circuit and observe. Varied the potentiometer position slowly so that the capacitor starts charging at the time constant determined by the values of R1 and R2. |  |  |  |  |  |  |  |  |  |
|  | Once the voltage across the capacitor is more than the break over voltage of the DIAC, DIAC starts conducting. Thus, the capacitor starts discharging towards the gate terminal of TRIAC through DIAC. |  |  |  |  |  |  |  |  |  |
|  | Therefore, TRIAC starts conducting and hence the main current starts flowing into the fan through the closed path formed by TRIAC. |  |  |  |  |  |  |  |  |  |
|  | By varying the potentiometer R2, the rate at which capacitor is going to be charged get varied this means that if the resistance is less, the capacitor will charge at a faster rate so the earlier will be the conduction of TRIAC. |  |  |  |  |  |  |  |  |  |
|  | As the potentiometer resistance gradually increases, the conduction angle of TRIAC will be reduced. Hence the average power across the load will be varied. |  |  |  |  |  |  |  |  |  |
|  | Identify Full Wave Rectifier components |  |  |  |  |  |  |  |  |  |
|  | Draw circuit Diagram of Full Wave Rectifier |  |  |  |  |  |  |  |  |  |
|  | Observe the INPUT and Output wave form on oscilloscope |  |  |  |  |  |  |  |  |  |
|  | Calculate the ripple Factor |  |  |  |  |  |  |  |  |  |
|  | Calculate output voltage |  |  |  |  |  |  |  |  |  |
|  | Identify the Operational amplifier terminals (Inverting, Non-inverting Inputs & Outputs) with the Help of Datasheet |  |  |  |  |  |  |  |  |  |
|  | Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. |  |  |  |  |  |  |  |  |  |
|  | Draw the Schematic diagram of Inverting Op-Amp. |  |  |  |  |  |  |  |  |  |
|  | Select the components for Inverting Op-Amp. |  |  |  |  |  |  |  |  |  |
|  | Implement Inverting Op-Amp circuit. |  |  |  |  |  |  |  |  |  |
|  | Perform the operations of Inverting Op-Amp circuit. |  |  |  |  |  |  |  |  |  |
|  | Measure the output voltage & gain |  |  |  |  |  |  |  |  |  |
|  | Generate the Output report |  |  |  |  |  |  |  |  |  |
|  | Draw the Schematic diagram of Comparator circuit using Op-Amp. |  |  |  |  |  |  |  |  |  |
|  | Select the components for Comparator circuit. |  |  |  |  |  |  |  |  |  |
|  | Implement Comparator circuit. |  |  |  |  |  |  |  |  |  |
|  | Perform the operations of Comparator circuit. |  |  |  |  |  |  |  |  |  |
|  | Check the output for different input conditions |  |  |  |  |  |  |  |  |  |
|  | Due to the bidirectional control capability of both TRIAC and DIAC, it is possible to control the firing angle of the TRIAC in both positive and negative peaks of the input. |  |  |  |  |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 3 in Automation & Process Control Electronics (Analog) (Level 3) |
| --- | --- |
| Competency Standard | - Identify Basic Electronics Components - Carry out Diode Application - Implement Bipolar Junction Transistor (BJT) in Different Applications - Implement Field Effect Transistor (FET) in Different Applications - Implement Thyristor Family in Various Application - Design a Rectifier using Diode - Applications of Operation Amplifier |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Diode is two terminal device. |  |  |
|  | - TRUE - FALSE |  |  |
|  | How many junction/s do a diode consist? |  |  |
|  | - 0 - 1 - 2 - 3 |  |  |
|  | A diode (P-N junction) has two terminals. The P-terminal is known as |  |  |
|  | - Anode - Cathode - Either anode or cathode - None of these |  |  |
|  | The relation between α and β is _________ |  |  |
|  | - β=α/(1-α) - α= β/(1+β) - β=α/(1+α) - α= β/(1- β) |  |  |
|  | The current amplification factor β in CE Configuration is given by_________ |  |  |
|  | - IC/IB - IB/IC - IE/IB - IB/IE |  |  |
|  | In order to operate NPN transistor as an CE amplifier, BE junction ___________ biased and CE junction ___________ biased. |  |  |
|  | - Reversed, Forward - Forward, Forward - Forward, Reversed - Reversed, Reversed |  |  |
|  | Which of the following terminals does not belong to the MOSFET? |  |  |
|  | - Drain - Gate - Base - Source |  |  |
|  | Choose the correct statement |  |  |
|  | - MOSFET is a uncontrolled device - MOSFET is a voltage controlled device - MOSFET is a current controlled device - MOSFET is a temperature controlled device |  |  |
|  | The controlling parameter in MOSFET is |  |  |
|  | - Vds - Ig - Vgs - Is |  |  |
|  | DIAC is ___________ layer ____________ terminal device |  |  |
|  | - 4 layer, 2 terminal - 4 layer, 3 terminal - 3 layer, 2 terminal - 3 layer, 3 terminal |  |  |
|  | Diode is also called as: |  |  |
|  | - Amplifier - Rectifier - Resistance - Reactance |  |  |
|  | During positive half cycle of total secondary voltage of bridge rectifier, number of diodes forward biased are |  |  |
|  | - one - two - three - four |  |  |
|  | Average Value of fullwave rectified voltage with a peak Value of 60 V is equals to_________ |  |  |
|  | - 23 V - 32.14 V - 38.12 V - 12.45 V |  |  |
|  | Number of diodes used in a full wave- bridge rectifier is_________ |  |  |
|  | - 1 - 2 - 3 - 4 |  |  |
|  | The circuits of an inverting amplifying comprises of ___________number of resistors. |  |  |
|  | - 1 - 2 - 3 - 4 |  |  |
|  | In inverting amplifier, phase difference between input and output voltages must be |  |  |
|  | - 30° - 45° - 90° - 180° |  |  |
|  | Differentiation amplifier produces |  |  |
|  | - Output waveform as integration of input waveform - Input waveform as integration of output waveform - Output waveform as derivative of input waveform - Input waveform as derivative of output waveform |  |  |
|  | The input applied to an Inverting amplifier is ______________ |  |  |
|  | - Equal to output - Equal to Inverted output - Not equal to output - Output is equal to input |  |  |
|  | Depending on the value of input and reference voltage a comparator can be named as |  |  |
|  | - Voltage follower - Digital to analog converter - Schmitt trigger - Voltage level detector |  |  |
|  | An OP-AMP comparator is a circuit that compares the signal voltage on one of its inputs with a _ |  |  |
|  | - Non-inverting voltage at output - Reference voltage on the other - Virtual input - Output |  |  |
|  | TRIAC is ___________ layer ____________ terminal device |  |  |
|  | - 4 layer, 2 terminal - 4 layer, 3 terminal - 3 layer, 2 terminal - 3 layer, 3 terminal |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________