National Certificate (Level-3) in Mechanical Technology with Specialization in Construction Machinery

Level-3 Equivalent to G-II

Competency Standards

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

Contents

1.	Introduction	5

2.	Purpose of the qualification	6

3.	Summary of Competency Standards	6

4.	Date of validation	7

5.	Codes of Qualifications	7

6.	Qualifications Development Committee	8

7.	Qualifications Validation Committee	9

8.	Entry Requirements	9

9.	Packaging of Qualifications	9

10.	Detail of Competency Standards	11

National Certificate Level-3 in Construction Machinery……………………………….11

10.4.	Internal Combustion Engine……………………………………………………..11

10.4.1	Identify Engine Types, their Component and Specifications	11

10.4.2	Disassemble and assemble Basic Engine Parts	13

10.4.3	Maintain Engine Cooling System Components and Their Relationship	15

10.4.4	Inspect Engine Lubrication System and working of its Components	17

10.4.5	Locate Diesel Engine’s Fuel System Components	19

10.4.6	Maintain the Engine intake and Exhaust System and its Components	21

10.4.7	Perform Overhauling of Four Stroke Engine	23

10.5.	Automotive Electrics and Electronics	26

10.5.1	Perform Basic Electricity Measurements	26

10.5.2	Identify Electromagnetism and its application.	27

10.5.3	Perform Battery Service of Vehicle	30

10.5.4	Repair Engine Starting System	31

10.5.5	Repair Battery Charging System	33

10.5.6	Repair Spark Ignition System of Internal Combustion Engine	34

10.5.7	Adjust Ignition Timing of Spark Ignition Engine	35

10.5.8	Repair Distributer-Less Ignition System	36

10.5.9	Repair Lightning System of Vehicle	37

10.5.10	Repair Cooling System and Vehicle Accessories	38

10.5.11	Recognize Electrical system of Vehicle	39

10.6.	Generic	42

10.6.1	Participate in Workplace Communication	42

10.6.2.	Practice Occupational Health and Safety Procedures	44

10.6.3.	Perform Work in Team Environment	46

11.	List of Machinery, Tools & Equipment.	47

11.1. Internal Combustion Engine…………………………………………………………47

11.2. Automotive Electrics and Electronics……………………………………………...48

# Introduction

Heavy Machine Operators and Technicians are in demand across the country and abroad. Their services are required for everything from road and bridge construction, back hoe operation, bulldozing, loading and grading, dredging and heavy paving, to excavating and much, much more. This is a good career opportunity for a reliable and responsible individual with a strong work ethic. Heavy Machine Operators and Technicians not only work on regular construction building jobs, but also on infrastructure projects (roads, bridges, Dams and ports, otherwise called non-building construction), and in mining and timber operations.

In order to build the capacity of technical and vocational training institutes in Pakistan through provision of demand driven competencies-based trainings in construction sector the NAVTTC, and TEVT Sector Support Program (TSSP) have joined hands together to develop qualifications for construction sector. These qualifications will not only build the capacity of existing workers of this sector but also support the youth to acquire skills best fit for this sector. The benefits and impact of development of these qualifications will be on both demand and supply side.

Based upon this demand of industry these competency‐based qualifications for Heavy Construction Machinery technology are developed under National Vocational Qualification Framework (NVQF) (Level 3).  The qualifications mainly cover competencies along with related knowledge and professional skills which are essential for getting a job or self-employed.

The National Vocational & Technical Training Commission (NAVTTC) has approved the Qualification Development Committee (QDC). The QDC consists experts from the relevant industries from different geographical locations across Pakistan and academicians who were consulted during the development process to ensure input and ownership of all the stakeholders. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

Heavy Machine Operator and Technician curriculum will prepare students to efficiently operate heavy machines such as Dozers, Loaders, Excavator and Graders and to perform basic preventive maintenance on most types of heavy equipment. Coursework includes Safety, Heavy Machines operations, grades, Legal & environmental concerns & equipment maintenance. Graduates of this program may find employment with state and local government agencies and private contractors engaged in highway or other construction activities.

In fact, no sphere of life may be identified that does not include the contribution of construction machinery. Thus, the importance of construction machinery may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechanical technology with Specialization in Construction Machinery under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# Purpose of the qualification

The purpose of these qualifications is to set high professional standards for Construction Machinery. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for Construction Machinery.

- Enable the existing workforce to capacitate themselves in new technologies and method so that train persons in this field can maintain construction machinery for its longer life.

# Summary of Competency Standards

The Construction Technology qualifications of level-3 consists 20% Theory and 80% Practical. The Core competencies of the qualification are as follows:

Level-3

Internal Combustion Engine

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-24 | Identify Engine Types Their Component and its Specifications | 3 | 6 | 24 | 30 | 3 | Technical |
| CS-25 | Disassemble and assemble Engine Parts | 3 | 6 | 24 | 30 | 3 | Technical |
| CS-26 | Maintain Engine Cooling System Components and Their Relationship | 3 | 6 | 24 | 30 | 3 | Technical |
| CS-27 | Inspect Engine Lubrication System and working of its Components | 3 | 6 | 24 | 30 | 3 | Technical |
| CS-28 | Locate Diesel Engine’s Fuel System Components | 3 | 6 | 24 | 30 | 3 | Technical |
| CS-29 | Maintain the Engine intake and Exhaust System Components | 3 | 6 | 24 | 30 | 3 | Technical |
| CS-30 | Perform Overhauling of Four Stroke Engine | 3 | 10 | 40 | 50 | 5 | Technical |
|  | Total |  | 47 | 171 | 218 | 21.8 |  |

Automotive Electrics and Electronics

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-31 | Perform Basic Electricity Measurements | 3 | 7 | 30 | 37 | 3.7 | Technical |
| CS-32 | Identify The Electromagnetism and Its Application | 3 | 6 | 30 | 36 | 3.6 | Technical |
| CS-33 | Perform Battery Service of Vehicle | 3 | 4 | 18 | 22 | 2.2 | Technical |
| CS-34 | Repair Engine Starting System | 3 | 6 | 18 | 24 | 2.4 | Technical |
| CS-35 | Repair Battery Charging System | 3 | 6 | 18 | 24 | 2.4 | Technical |
| CS-36 | Repair Spark Ignition System of Internal Combustion Engine | 3 | 6 | 27 | 33 | 3.3 | Technical |
| CS-37 | Adjust Ignition Timing of Spark Ignition Engine | 3 | 6 | 27 | 33 | 3.3 | Technical |
| CS-38 | Repair Distributer-Less Ignition System | 3 | 6 | 27 | 33 | 3.3 | Technical |
| CS-39 | Repair Lightning System of Vehicle | 3 | 6 | 21 | 27 | 2.7 | Technical |
| CS-40 | Repair Cooling System and Vehicle Accessories | 3 | 8 | 36 | 44 | 4.4 | Technical |
| CS-41 | Recognize Electrical System of vehicle | 3 | 9 | 36 | 45 | 4.5 | Technical |
|  | Total |  | 70 | 288 | 358 | 35.8 |  |

Generic

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-42 | Participate In Workplace Communication | 3 | 2 | 6 | 8 | 1 | Generic |
| CS-43 | Practice Occupational Health and Safety Procedures | 3 | 2 | 6 | 8 | 1 | Generic |
| CS-44 | Perform Work In Team Environment | 3 | 2 | 6 | 8 | 1 | Generic |
|  | Total |  | 6 | 18 | 24 | 3 |  |
|  | Total |  | 123 | 477 | 600 | 60 |  |

# Date of validation

The level 3 of National qualification on Construction Technology has been validated by the Qualifications Development Committee (QDC) members on ………… and will remain in currency until …………………………

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Construction Technology level-5 |  |
| --- | --- |
| Code | Description |
| 000000000 | National Certificate of level-2, in “Construction Machinery” |
| 000000000 | National Certificate of level-3, in “Construction Machinery” |
| 000000000 | National Certificate of level-4, in “Construction Machinery” |
| 000000000 | National Certificate of level-5, in “Construction Machinery” |

# Qualifications Development Committee

The following members participated in the qualification’s development workshop 18th March 2019

To 22nd March 2019 at Grand Palm Hotel, Lahore:

| S.No. | Name & Designation |  | Organization |
| --- | --- | --- | --- |
|  | Muhammad Ishaq, Dy Director (TE) | NAVTTC, HQ |  |
|  | Muzammal Hussain, Instructor (Auto & Diesel) | GCT, Rawalpindi, TEVTA |  |
|  | Muhammad Ejaz, HOD, Mechanical Department | C.T.T.I, Islamabad |  |
|  | Hassan Raza, Jr. Instructor | C.T.T.I, Islamabad |  |
|  | Umer Farooq, Instructor, Mechanical | G.S.P.C.T Gujrat |  |
|  | Muhammad Rizwan Sarwar, Sr. Instructor (Auto Mechanic) | G.T.T.I, Shiekhupura |  |
|  | Hafiz Ghulam Mohyudin, Asst. Manager Services | Dynamic Equipment & Control, Lahore |  |
|  | Mehr Ali Qureshi, Sr. Service Engineer | Murshid Builders (PVT) Limited, Lahore |  |
|  | Syed Hussein-el-Edroos, Sr. Service Engineer | Independent Consultant |  |
|  | Waqar Ahmed, Service Manager | Jaffer Brothers (PVT) Limited, Lahore |  |
|  | Ghulam Dastgir, Incharge Projects | Ex-DESCON Engineering, Lahore |  |
|  | Zain-ul-Abideen, Instructor | G.T.T.I, Mughalpura, Lahore |  |
|  | Atif Waheed Abbasi, Heavy Machine Operator | National Logistic Cell Mandra, Islamabad |  |
|  | Sikander Mehmood Jr. Instructor | C.T.T.I, Islamabad |  |
|  | Engr. Mohsin Jahanzeb Instructor | GSTC,Faisalabad |  |
|  | Muhammad Amjad Iqbal Service Engineer | Greaves Pakistan Ltd,Lahore |  |
|  | Sayed Fayyaz Mustafa Naqvi J. Instructor Civil | GSTC,Faisalabad |  |
|  | Saba Sadiq, MS Mechanical Engineer/ DACUM Facilitator, | University of Lahore, Lahore |  |

# Qualifications Validation Committee

The following members participated in the qualification development and validation workshop on:

| S.No. | Name & Designation | Organization |
| --- | --- | --- |
|  | Mr. Muhammad Ejaz | HOD, CTTI, Islamabad |
|  | Mr. Hassan Raza | Instructor, CTTI, Islamabad |
|  | Mr. Muzammal Hussain | Instructor (Auto & Diesel) , GCT, Rawalpindi |
|  | Mr. Sikandar Masood | Director, NAVTTC, Islamabad |
|  | Mr. Abdur Rehman | Representative from PBTE, Lahore |
|  | Mr. Zafar Iqbal | Representative from KP-TEVTA |
|  | Mr. Rafique Ahmed Shaikh | Representative from Singh-TEVTA |
|  | Mr. Naveed Ahmed Ajmal | Representative from P-TEVTA |
|  | Mr. Irfan Khalil | Representative from Sector Skill Council/ ABAD Karachi |
|  | Saba Sadiq | DACUM, Facilitator, Lahore (UOL) |

# Entry Requirements

The entry for National Certificate level-3, in Construction Technology are

- A person having National Vocational Certificate level-2, in Construction Technology.

# Packaging of Qualifications

The National Vocational Qualifications have been packaged as detailed below:

| Level | Name of Occupation/Nomenclature | Generic Competencies | Allied competencies require | Core /Technical Competencies required |
| --- | --- | --- | --- | --- |
| - Diesel Engine Technician |  |  |  |  |
| Level-2 | Pro Diesel Mechanic | 150,151,152,153,154 |  | 1,2,3,4,5,6,7 |
| Level-3 | Engine Rebuilder | 150,151,152,153,154 |  | 44,45,46,47,48,49,50,51,52 |
| Level-3 | Generator Technician | 150,151,152,153,154 |  | 61,62,63,64,65,66 |
| Level-3 | Engine Technician | 149,150,151,152,153,154 |  | 53,54,55,56,57,58,59,60 |
| Level-3 | Pro Technician Diesel Pump | 149,150,151,152,153,154 |  | 67,68,69,70,71,72 |
| Level-4 | Diesel Pump Technician | 140,141,142,143,144,145,146,147,148, 149,150,151,152,153,154 |  | 128,129,130,131,132,133 |
| Level-4 | CNC Machine operator | 150,151,152,153,154 |  | 15,16,17,18,19,20,21,22,23,24,25,26,27 |
| Level-4 | Thermo Man | 150,151,152,153,154 |  | 39,40,41,42,43 |
| - Electrician |  |  |  |  |
| Level-2 | Auto Electrician | 150,151,152,153,154 |  | 28,29,30,31,32,33,34,35,36,37,38 |
| - Engineering Drawing |  |  |  |  |
| Level-3 | Draft Man | 152,153,154 |  | 8,9,10,11,12,13,14 |
| - Heavy Machine Operator |  |  |  |  |
| Level-3 | Excavator Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,78 |
| Level-3 | Bulldozer Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,80 |
| Level-3 | Dump truck Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,79 |
| Level-3 | Wheel Loader Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,81 |
| Level-3 | Wheel Mounted Loading Crane Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,83 |
| Level-4 | Master operator (Excavator, Bulldozer, Dump truck, Wheel Loader, Grader, Crane and asphalt paver) | 149,150,151,152,153,154,155 |  | 73,74,75,76,77,78,79,80,81,82,83,84,85 |
| - Heavy Machinery Mechanic |  |  |  |  |
| Level-5 | Construction Machinery Transmission Technician | 149, 150,151,152,153,154 |  | 86,87,88,89,90,91,92,93 |
| Level-5 | Steering and Brake Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 94,95,96,97,98,99,100,101,102,103 |
| level-5 | Under Carriage Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 104,105,106,107,108,109,110 |
| Level-5 | Hydraulic Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 111,112,113,114, 115,116,117,118 |
| Level-5 | Hydraulic Excavator Mechanic of Construction Machinery | 140,141,142,143,144,145,146,147,148,149, 150,151,152,153,154 |  | 119,120,121,122,123,124,125,126,127 |
| Level-5 | Construction field Manager | 140,141,142,143,144,145,146,147,148,149, 150,151,152,153,154 |  | 134,135,136,137,138,139 |

# Detail of Competency Standards

## National Certificate Level-3 in Construction Machinery

## Internal Combustion Engine

- Identify Engine Types, their Component and Specifications

Overview: This competency standards will cover to Recognize Petrol Engine, Recognize Diesel Engine, Analyze the functioning of Diesel Engine and Analyse the CI engine Ignition system.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Recognize Petrol Engine | - Locate spark plugs and it functions - Point out distributor and its function - Identify ignition coil and its functions - Identify plug leads and its functions - Identify carburetor and its function - Locate EFI system and its main components |
| - Recognize Diesel Engine | - Identify the typical diesel engines and function of their main components. - Locate fuel injectors and its function - Point out high pressure fuel pump and its functions - Identify heater plug and its function - Identify the Common rail fuel system. - Identify HEUI fuel system. |
| - Point out the main components of Diesel Engine | - Locate the cylinder, cylinder head and its basic function - Identify the function of piston. - Identify the function of connecting rod, crank shaft and valves. - Locate the flywheel, camshaft, timing gears and its function. |
| - Analyse the diesel engine Ignition system. | P1. Inject oil into the combustion chamber P2. Atomize the oil jet droplets P3. Evaporate the droplets. P4. Mix the vapors with hot air and form combustible mixture. |

Knowledge & Understanding

- Describe the vehicle engines in a variety of forms and locations as following:

  - Types (Petrol four stroke, diesel four stroke, and rotary/Wankel).

  - Aspirated (Natural, turbocharged, supercharged)

  - Cylinder layout (single, multi, in-line, V shape, horizontally opposed).

  - Location of engine (front, rear, transverse, longitudinal)

Critical Evidences Required

- Capable to identify petrol/diesel engine.

- Capable to differentiate petrol EFI and carburetor engine.

- Capable to explain different parts in petrol/diesel engine.

List of Tools and Equipment

- Measuring tools (Vernier caliper, micrometer, Cylinder bore gauge)

- Hand tools trolley

- Special tools (ring compressor, belt tensioner, torque wrench)

- Engine Petrol/ diesel

- Vehicle

- Disassemble and assemble Basic Engine Parts

Overview: These competency standards designed to provide skills and knowledge for engines their understanding the working principles, construction, types, components and their relationship between different components, system of vehicle by heavy machinery, in accordance with the manufacturer’s Manual.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Disassemble Engine block | - Select appropriate tools. - Perform disassembly of engine according to manufacturing shop manual. - Inspect the disassemble components |
| - Assemble Engine block | - Select appropriate tools and perform crankshaft assembly in the engine block with all related components. - Select appropriate tools and perform piston ring assembly in the block under the specified procedure. - Assembly of the all engine components as specified procedure in the manual. |
| - Disassemble Engine head | - Select appropriate tools. - Remove Head Gasket. - Carry out disassembly of engine camshaft according to specified procedure in the manual. - Carry out disassembly of engine head valve train according to specified procedure in the manual. - Dissemble Retainer Plate and Rotter coil - Inspect the disassemble components |
| - Assemble Engine head | - Assemble Retainer Plate and Rotter coil - Assembly of engine head valve train according to specified procedure in the manual. - Assembly of engine camshaft according to specified procedure in the manual. - Place the Head Gasket. |

Knowledge & Understanding

- Describe the engine terms as following:

Top dead Centre (TDC), bottom dead Centre (BDC), bore, stroke, engine capacity, clearance volume, swept volume, compression ratio, single overhead cam, double overhead cam, overhead valve,

- Describe the main engine components and functions:

Components: cylinder heads, cylinder blocks, liners(wet/dry), manifolds, valve operating mechanisms, timing, gears, camshafts, pistons, pushrods, connecting rods, crankshafts, flywheels, machined faces, securing devices, journals/ bearings, seals.

- Describes the faults in engine as following:

White smoke, black smoke, engine oil level decreasing, incorrect adjustment of tappets, incorrect adjustment of ignition timing, incorrect adjustment of engine timing.

Critical Evidences Required

- Capable to assemble the engine components

- Capable to adjust engine timing and ignition timing

- Capable to service air filter and spark plugs

List of Tools and Equipment

- Measuring tools (Vernier caliper, micrometer, Cylinder bore gauge)

- Hand tools trolley

- Special tools (ring compressor, belt tensioner, torque wrench)

- Engine Petrol/ diesel

- Vehicle

Consumable Items:

- Engine Oil, Gasket Set, Piston Rings, Oil Seals, Valve Seats, Silicon, Emery Paste and Stick, Cotton Waste.

- Maintain Engine Cooling System Components and Their Relationship

Overview: This Learning Module is designed to provide skills and knowledge for understanding the working principles, construction, types, components and their relationship between different components to service and repair air- and water-cooled engine cooling systems and to diagnose fault/s and other maintenance issues by Auto Mechanic, in accordance with the manufacturer’s Manual.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Identify Cooling system in the vehicle. | - Demonstrate main components of cooling system and their functions. - Locate the water pump and understand the function of its components. - Point out the thermostat Valve. - Identify the radiator, its types and understand the function of the components. - Demonstrate the radiator fan and understand its drive types including fan motor and its pump. |
| - Demonstrate Antifreeze/Inhibiters | - Identify the antifreeze its compositions and types. - Identify the rust corrosion in the engine and preventions. |
| - Perform Flushing of Cooling system | - Remove the Coolant from engine. - Add the solution of water and Cleaning agents (Freshers) - Start the engine for specified hours. - Remove the solution - Add new coolant. |

Knowledge & Understanding

- Describe the operating principles and terminologies of cooling system as following:

Air cooled engine, liquid cooled engine, operating temperature, thermal efficiency, pressurized system, heat exchange method (conduction, radiation, and convection), corrosion, and inhibitors/antifreeze)

- Describe the functions and purpose of engine cooling system components as:

Electrical motor operated fan with thermo switch, belt operated fan, temperature controlled/viscous coupling fan, pressure cap, water body/pump, drive belt, thermostat, radiator, core plug, hoses/clamps, and gasket/ seals.

Critical Evidence(s) Required

- Capable of diagnose, adjust and rectify the cooling system faults.

List of Tools and Equipment

- Hand tools trolley

- Radiator leak tester

- Vehicle/ simulator with a cooling system

- Drive belt tensioner gauge

Consumable Items:

- Coolants, Sealant/gasket, Hose Pipe, Thermostat Valve

- Inspect Engine Lubrication System and working of its Components

Overview: This Learning Module is designed to provide skills and knowledge for understanding the working principles, construction, types, components and their relationship between different components to repair different types of engine lubricating systems by Auto Mechanic, in accordance with the manufacturer’s Manual.

| Units of Competency | Performance Criteria |
| --- | --- |
| - Identify the Lubrication system | - Identify the main components of engine lubrication system. - Identify types of oil pumps. - Identify the main relief valve. - Identify the Temperature, pressure and level sensor of oil. - Identify Oil filter. - Identify the Oil cooler - Identify oil coolers for the high-performance engine (air cooled/ Water cooled type) |
| - Analyze different Lubricants in the Engine. | - Identify the types of engine oil used in the engine with respect to viscosity. - Identify the function of oil filter and its components. - Identify the oil sump and its components. |
| - Sketch Lubrication system diagram | - Draw The lubrication system diagram with EGR - Draw The lubrication system diagram without EGR |

Knowledge & Understanding

- The functions/operating principle and main components of engine lubrication system as following.

- Wet sump, dry sump, total loss, Oil pumps and its types, pressure relief valves, oil filter, oil cooler, ventilation/ PCV Valve, hoses, and oil level indicators.

- Lubricant (oils viscosity, viscosity index, oil classifications, Reduction of frictional forces, cooling effect, cleaning effect, corrosion resistance, and noise reduction).

- The (Law of friction, static/dynamic friction (difference), coefficient of friction).

- Explain flow of oil in Lubrication system diagram with EGR.

- Explain flow of oil in Lubrication system diagram without EGR.

- Define Micron rating of Oil Filters.

- Define Oil Grades and multi grade oil.

Critical Evidence(s) Required

- Capable of explain the lubrication system components.

- Capable to know the engine oil and filter.

List of Tools and Equipment

- Hand tools trolley

- Car Lift

- Vehicle/ simulator with a cooling system

- Sealant/gasket

- Locate Diesel Engine’s Fuel System Components

Overview: These competency standards designed to enable the work on vehicle fuel system to understand the faults in diesel engines.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Identify PT (pressure time) fuel system | - Locate the diesel tank. - Demonstrate the primary fuel filters. - Identify the PT pump. - Identify the priming pump. - Identify the secondary fuel filters. - Identify the PT injectors. - Identify the pressure regulator valve. - Identify the return line. - Draw the block diagram of PT pump fuel system. |
| - Locate Inline fuel injection (BOSCH) system. | - Identify the diesel tank. - Identify the primary fuel filters. - Identify the transfer pump. - Identify the priming pump. - Identify the secondary fuel filters - Identify the BOSCH pump. - Identify the nozzle - Identify the return line. - Draw the block diagram of BOSCH pump fuel system. |
| - Inspect Common rail injection (CRI) fuel system. | - Identify the ECM. - Identify the Transfer pump. - Identify the primary pump. - Identify the secondary fuel filters - Identify the main pump. - Identify the Common Rail. - Identify the injectors. - Identify the return line. - Draw the block diagram of CRI system. |
| - Demonstrate Hydraulic electronic unit injection (HEUI) system | - Identify the transfer pump. - Identify the secondary fuel filters - Identify the injectors. - Identify the pressure regulator valve. - Identify the oil pan - Identify the oil pump. - Identify the oil cooler. - Identify the oil filter - Identify the unit injection pump - Identify the injector |
| - Locate Mechanical Electronic Unit Injection (MEUI)system | - Identify the ECM (main pump). - Identify the MEUI injector. - Identify the MEUI regulator valve - Identify the MEUI return line. - Draw the block diagram of PT pump fuel system. |

Knowledge & Understanding

- Describe the features of main components of engine fuel system as following.

Fuel tanks, filler cap/ security, venting, fuel level sender, lines/ filters, carburetor, and fuel lift pumps (mechanical/ electrical), fuel (octane rating, Octane number, cetane Number, cetane rating, leaded, unleaded).

- Describe the Diesel injection systems main components as following.

Single/ multi-hole injectors, inline fuel injection pump, rotary fuel injection pump, fuel injection pressure, bleed points, fuel cutoff switch, governor operation, in Combustion chamber types, main features of pressure/ volume diagrams/ effects of advanced/ retarded/ no fuel injection

- Describe the as following statements.

Diesel fuel composition, effects of low temperature, viscosity, volatility, calorific value, cetane rating.

Critical Evidence(s) Required

- Capable understanding the diesel engine fuel system

- Capable understanding the diesel EFI engine fuel system

List of Tools and Equipment

- Toll trolley (Complete set)

- Diesel engine vehicle or simulator

- Safety wears

- Maintain the Engine intake and Exhaust System and its Components

Overview: These competency standards designed to enable the work on vehicle fuel system to diagnose and rectify the faults in petrol and diesel engines.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Maintain Intake system components | - Inspect the performance of main components of Intake manifold - Remove and assemble intake manifold from engine - Remove the pre cleaner and air filter - Identify the impeller of turbo charger. - Identify After cooler. - Identify inter-cooler. |
| - Maintain Exhaust system and its components | - Inspect the performance of main components of exhaust manifold - Remove and assemble exhaust manifold from engine - Replace the muffler and EGR under the specified procedure. - Replace the gasket of muffler under the specified procedure. - Remove and assemble turbocharger. - Inspect the performance of main components of Turbocharger. |
| - Identify Emission control system | - Identify EGR and its function. - Identify the positive crankcase ventilation (PCV). - Identify the exhaust gas recirculation system. - Identify the main components of Evaporative control system of fuel system. |

Knowledge & Understanding

- Describe the purpose of exhaust system and its functions.

- Describe the muffler assembly operation.

- Describe the exhaust gas regulator (EGR).

- Describe the positive crank case ventilation PCV system and it operation

- Describe the exhaust gas recirculation and its operation

- Describe function of inter-cooler and after-cooler.

- Describe the operation of turbocharger.

Critical Evidence(s) Required

- Capable of understanding the exhaust system

- Capable of understanding EGR.

- Capable of understanding Catalytic converter.

List of Tools and Equipment

- Vehicle

- Tools trolley (Complete set of hand tools)

- Gas analyzer

- Vacuum pump

- Perform Overhauling of Four Stroke Engine

Overview: This competency standard is designed to provide skills and knowledge to Overhauling, in accordance with the Manufacturer’s Manual.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Perform Overhauling of Four stroke engine cylinder head. | - Arrange proper tools, equipment and shop manual. - Disconnect cables, wires, and muffler, Air filter from cylinder head for dismantling cylinder head. - Remove cylinder head assembly. - Clean cylinder head and its components to identify the damage parts. - Install Repaired cylinder head and its attachments according to shop manual. - Test engine performance to verify servicing of cylinder head. |
| - Service the cylinder block, Piston and Piston Rings | - Arrange proper tools and equipment to service cylinder block, piston and piston rings. - Remove cylinder head assembly from the cylinder block to service cylinder block piston and piston Rings as per shop manual. - Clean cylinder block, piston and its components to remove dust. - Remove cylinder block, piston and inspect its components to identify the damage parts as per Shop manual. - Install Cylinder Block, piston, piston rings, and cylinder head and connect all the wires, cables to the engine as per shop manual. - Test engine performance to verify servicing of cylinder block, piston and piston rings as per shop manual. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard.

This includes the knowledge of:

- Knowledge about PPE (personal protection equipment).

- Different types of fire extinguishers

- Safe work environment

- Safety measures and precautions

- First Aid

- Proper use and handling of equipment

- Knowledge about securing measuring tools

- Types of engines

- Schedule of Motorcycle Maintenance

- Different parts of Motorcycle

- Working principles and comparison of 2 and 4 Stroke Engine

- Types and procedure of Oiling & greasing

- Inspections and Measurements

- Cleaning process of different Motorcycle parts / Components.

- Replacement of Parts / Components

- Adjustment of parts /Components

- Working principles of engine and its terminologies

- Different systems of engine

- Parts of engine

- Diagnostic procedure of Engine

- Engine dismantling procedure

- Engine inspection and repair procedure

- Engine assembling procedure

- Engine oil circuits

- Engine Timing

- Manufacturers and workshop manual of given vehicle

- Working principle Fuel system

- Exhaust system

- Euro standards

- Driving tips and safety

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Remove and install single cylinder engine.

List of Tools and Equipment

- Tools trolley (complete set of tools)

- Compression tester

- Measuring tools (Complete set for engine overhauling)

- Fuel pressure Tester

- Special Service Tools

Consumable Items:

Piston ring, Engine oil, Silicon, Oil sump Jain, oil filter, head Gasket, grease, cotton waste.

# Automotive Electrics and Electronics

- Perform Basic Electricity Measurements

Overview: These competency standards designed to enable the learner to perform electrical related calculations and measurements, as well as conceptualize different automotive functions related to the electrical operations.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Basic Electricity | - Prepare series circuit on work bench using appropriate tools. - Prepare parallel circuit on work bench using appropriate tools. - Prepare series parallel circuit on work bench using appropriate tools. |
| - Basic Electricity Measurement | - Measure/ record voltage by using Digital Multi METER (DMM) - Measure/ record current by using Digital Multi Meter (DMM) - Measure/ record resistance by using Digital Multi Meter. - Measure/ record continuity by using Digital Multi Meter. |

Knowledge & Understanding

- Describe the following terms:

Atom and its particles (electron, neutrons & protons), EMF, current, voltage, resistance, magnetism, electro magnetism, flux, inductance, mutual inductance, capacitance, and diode.

- Calculation of ohm law, (V= IR) calculation of power law (P= VI).

Critical Evidence(s) Required

- Calculation involving the series, parallel, and series parallel circuits

- Correct use of Digital multi meter (DMM)

- Point out electromagnets in the vehicle.

List of Tools and Equipment

1	Digital Multi meter (DMM).

2	Load (resistors)

3	Wires

4	Test benches (Complete accessories)

5	Lamps

Consumable Items:

- Wires, Bulb and Bulb Holder, Thimble (Male & Female), Circuit Board, Wire Clumps, Battery

- Identify Electromagnetism and its application.

Overview: This competency standard covers the skills and knowledge required to implement the electromagnet, effect on current carrying conductor in magnetic field, effect on conductor by varying the current with the help of rheostat, Plot magnetic lines of forces of bar magnet, observe its various characteristics and verify the Faraday’s law and emf through induction.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Construct Electromagnet | - Prepare series circuit on work bench using appropriate tools. - Prepare parallel circuit on work bench using appropriate tools. - Prepare series parallel circuit on work bench using appropriate tools. |
| - Implement circuit to determine the effect on current carrying conductor in magnetic field. | - Measure/ record voltage by using Digital Multi Meter - Measure/ record current by using Digital Multi Meter - Measure/ record resistance by using Digital Multi Meter. - Measure/ record continuity by using Digital Multi Meter |
| - Determine the effect on conductor by varying the current with the help of rheostat. | - Reduce the rheostat resistance - Record the effect on copper rod. - Increase the rheostat resistance - Record the effect on copper rod. |
| Plot magnetic lines of forces of bar magnet | - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet. - Mark points on paper where the compass needle stop. - Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4 |
| Verify Faradays law by moving magnet inside the coil. | - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvano meter with coil. - Move permanent magnet inside the coil, vary its speed and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvano meter. |
| Verify Faradays law by moving coil near the magnet field. | - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvano meter with coil. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvano meter. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvano meter. |
| Verify emf through induction. | - Take step down transformer and connect its secondary with a Galvano meter and primary winding with a DC battery through a rheostat. - Continuously variate the rheostat and observe the reading on the Galvano meter |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Magnet, magnetism & electromagnet

- The functions of iron core

- Strong magnet

- The movement of current carrying conductor which is placed in magnetic field

- Fleming’s left-hand rule

- Magnetic lines of force.

- Magnetic field.

- Travel of magnetic lines of force with respect to each other.

- The first & 2nd law of michael faraday

- The role of magnetic strength in faraday’s law

- The role of conductor’s length or turns in faraday’s law

- Lenz’s law

- Induction

- Faraday’s first law of electro-magnetic induction

- Coupled coils

- The induction of emf in 2nd coil, when voltage is applied to 1st coil

- Mutual induction core

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

Iron nail as core

Thin coated copper wire

Current carrying conductor

Horse shoe magnet

Power Supply

Compass needle

Bar Magnet

Paper

Lead pencil

Coil

Galvanometer

Magnet

Connecting leads

Transformer

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Construct an electromagnet and verify its operation.

- Plot magnetic field of around the magnet using compass.

- Verify faradays law using magnet and coil.

- Implement a circuit to verify emf through induction

- Perform Battery Service of Vehicle

Overview: These competency standards designed to enable the learner to diagnose the problem related to the battery and starting system and enable the learner to troubleshoot the system if not working

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Service of battery | - Perform the battery test by use of hydrometer, Terminal clean, tight, and insulated, secure with battery holder as per given specification. - Inspect and change the Ground (Earth) cable from the vehicle body. - Test battery by using the load tester/battery tester and assess the battery performance. |
| - Battery replacement | - Perform Safe removal and installation of Battery from the vehicle Inspect/service the battery and assess safety fitting and correctness under specified procedure. - Carry out safe procedure to charge the battery using an external charger. |

Knowledge & Understanding

- Lead Acid battery and its internal components:

Material, acid and water ratio, plates, cells, ampere hour rating, voltage drop, corroded/ lose terminals, maintenance free batteries.

- Describe the chemical activities in Lead Acid Battery while charging and discharging.

Critical Evidence(s) Required

- Troubleshoot the problems related to the vehicle battery.

List of Tools and Equipment

1	Vehicle or simulator.

2	Tool box

3	Safety goggles

4	Gloves

5	Battery External chargers

6	Battery load tester

7	DMM

8	Wires

9	Hydrometer

- Repair Engine Starting System

Overview: These competency standards designed to enable the learner to diagnose the problem related to the battery and starting system and enable the learner to troubleshoot the system if not working

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Repair starting system | - Inspect the vehicle starting system and troubleshoot the following: Low cranking speed/ not cranking, Ignition switch operation malfunctioning. - Inspect the vehicle starting system and troubleshoot the following: Unusual noise during operation, corrosive battery terminals, safety neutral switch, incorrect pinion engagement/ disengagement, blown fuse/ relay, lose/ corroded main starting wire. |
| - Service Starter Motor. | - Carryout safe procedure to replace the starter motor from the vehicle. - Service/ replace components of starter motor and perform bench testing. |

Knowledge & Understanding

- Short circuits and battery related hazards.

Wrong battery installation, touch the positive terminal with the body, un-insulated wire touches with the vehicle body, battery size not matches with the maker, battery acid injury (human eye/ body), disposal of faulty batteries.

- Describe the Wiring diagram associated with the electrical/electronic symbols.

Circuit protection device (Fuse, fusible link, circuit breaker) Circuit switching device (manual switches, electromagnet switch/relay normal open and normal close, pressure switch, temperature switch) Resister (Rating, identification method, effect of change of temperature) Electronic devices (Diode, Zener diode, bipolar transistor, Thyristor, thermistor, heat sink, LED)

- Distinguish between increase/decrease cranking current consumption, voltage drop in a system, electrical load, and reason of high ampere draw from a system/overload.

- Describe the Lenz’s law of motor principle, single phase, three phase motor, AC Motor, DC Motor.

Critical Evidence(s) Required

- Troubleshoot the problems related to the vehicle battery.

- Capable to describe the fault related to the battery and take appropriate decisions.

- Capable to troubleshoot the vehicle starting system.

- Service the starter motor.

List of Tools and Equipment

1	Vehicle or simulator.

2	Toolbox

3	Safety goggles

4	Gloves

5	Battery External chargers

6	Battery load testers

7	DMM

8	Wires

9	Hydrometer

- Repair Battery Charging System

Overview: These competency standards designed to enable the learner to troubleshoot battery charging related problems and conceptualize electronics used in the system.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Repair charging system | - Inspect vehicle charging system and diagnose the following. Battery warning light indication on instrument panel, test output voltage of alternator, drive belt condition, alternator bearing noise, wire harness, fuse, and proper insulated wires. - Dismantle/ service of alternator as following; Repair connector, replace carbon brushes, Replace Rectifier Bridge, replace voltage regulator and replace stator winding. |
| - Replace alternator | - Perform safe working practice to remove the alternator from the vehicle and refit under the specified procedure. - Perform test bench of alternator to confirm the operation. |

Knowledge & Understanding

- Describe the Purpose and function of Alternator’s following parts.

Rotor, stator windings, Voltage regulator, Rectifier Bridge, Internal cooling fan, Bearings, carbon brush, mounting/ adjustment bolts), drive belt and its types, ratio alternator/ crank pulleys, RPM, Alternator input/ output voltages.

- Describe the functions and uses of the following.

Semiconductor and its types, Rectification (Half wave/ full wave), transformer and its types, capacitor and its types.

Critical Evidence(s) Required

- Capable to troubleshoot the vehicle charging system

- Capable to describe the fault related to the alternator and take appropriate decisions.

- Capable to Service the alternator

List of Tools and Equipment

1.		Vehicle/simulator

2.		Test lamp

3.		DMM

4.		Tool kit

5.		Ampere meter

6.		Fuses

7.		Relay

- Repair Spark Ignition System of Internal Combustion Engine

Overview: These competency standards designed to diagnose and troubleshoot conventional spark ignition system as well as conceptualize the ignition system mechanism in the engine.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Repair Ignition System | - Inspect vehicle Ignition system and diagnose the following: Crank engine and test presence of spark in a system, identify faulty fuse in fuse box (Note: follow safe working procedure). - Inspect vehicle Ignition system and diagnose following: Test performance of plug cable (high tension leads) and plugs, inspect distributor cap/rotor for performance, assess correct ignition timing, inspect vacuum advancer, assess C.B. Point condition/gap and test condenser. (Note: follow safe working procedure). |
| - Repair Ignition System (CDI System) | - Inspect the vehicle Ignition system and diagnose the ignition coil performance, (Note: follow safe working procedure). - Inspect the vehicle Ignition system and diagnose the CDI of distributor. (Note: follow safe working procedure). |

Knowledge & Understanding

- Describe the Purpose and function of ignition system’s following parts in Spark ignition system contact breaker type, spark ignition system breaker less type. Ignition switch, Ignition coil, resister, plug cable, distributer cap, rotor arm, vacuum advancer, centrifugal advancer, contact breaker assembly, condenser, high tension leads/ connectors, suppression, spark plug (construction, heat range type, size), Magnetic Reflector, Hall Effect trigger/ amplifier, differentiate the performance of contact breaker and contact breaker less type.

Critical Evidence(s) Required

- Troubleshot the vehicle ignition system (Non EFI engines)

- Capable to adjust the ignition timing

- Capable to service the ignition system.

List of Tools and Equipment

1.		Vehicle/ Simulator

2.		Digital Multi meter (DMM).

3.		Tools box

4.		Test Lamp

5.		Timing gun

- Adjust Ignition Timing of Spark Ignition Engine

Overview: These competency standards designed to diagnose and troubleshoot conventional spark ignition system as well as conceptualize the ignition system mechanism in the engine

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Adjust Ignition Timing | - Installation of distributer assembly under specified procedure and start the engine, adjust ignition timing with the help of timing gun. - Inspect/ Replace ignition switch, inspect/ replace ignition coil, and Repair/ replace electrical connectors under specified procedure. |
| - Repair Ignition System | - Inspection of crank position sensor under specified procedure and starts the engine. - Inspection/replace of ignition fuse under specified procedure and starts the engine. |

Knowledge & Understanding

- Describe primary circuit, secondary circuit of ignition system, identifies change in the spark timing in accordance to the load and speed of the engine.

- Explain Faradays law of mutual induction and calculate the primary and secondary voltages and current.

Critical Evidence(s) Required

- Troubleshot the vehicle ignition system (Non EFI engines)

- Capable to adjust the ignition timing

- Capable to service the ignition system.

List of Tools and Equipment

1.		Vehicle/ Simulator

2.		Digital Multi meter (DMM).

3.		Tools Box

4.		Test Lamp

5.		Timing Gun

- Repair Distributer-Less Ignition System

Overview: These competency standards designed to enable the learner to troubleshoot the electronic ignition related problems in the vehicle.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Troubleshoot Distributer Less Systems | - Use of scanner and find out the fault related to the ignition coil and reset the trouble code by use of scanner. - Use of scanner and find out the fault related to the crank sensor and reset the trouble code by use of scanner. - Use of scanner and find out the fault related to the crank sensor and reset the trouble code by use of scanner. |
| - Repair Distributor Less Systems | - Use test lamp and identify the faulty fuse in the fuse box under the specified procedure. - Replace the faulty fuse with the same color and start the engine under the specified procedure. - Replace the faulty ignition coil and faulty sensor under the specified procedure. |

Knowledge & Understanding

- Ignition advance mechanism in Distributor less system, Dual coil system, Coil on plug system, crank sensor signals,

Critical Evidence(s) Required

- Capable to diagnose and troubleshoot the electronic ignition system.

- Capable to replace faulty parts of electronic ignition system.

List of Tools and Equipment

1	Tools trolley (Complete set of hand tools)

2	Scanner

3	Test lamp

4	DMM (Multi meter)

- Repair Lightning System of Vehicle

Overview: These competency standards designed to enable the learner to rectify electrical related troubles IN the vehicle and understand the Wiring of the vehicle.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Troubleshoot The Lightening System | - Inspect vehicle lightening system and diagnose following; under specified testing instructions, Blown fuse finding from interior or exterior fuse box. (Note: follow safe working procedure). - Inspect vehicle lightening system and diagnose following; under specified testing instructions, faulty relay and faulty flasher. (Note: follow safe working procedure). |
| - Repair Lights | - Inspect vehicle lightening system and diagnose following under specified testing instructions. Inspect/ Replace head lamp, parking bulb and indicator bulb (Note: follow safe working procedure). - Inspect vehicle lightening system and diagnose following under specified testing instructions. Back light bulb, reverse light bulb. (Note: follow safe working procedure). |

Knowledge & Understanding

- Identify the terms used and calculations associated with the electricity. Definitions (magnetism, self/ mutual induction). Laws (amperes rule, Fleming’s rule) Switch (manual, magnet, pressure, temperature) sender (fuel level, temperature)

Critical Evidence(s) Required

- Capable to troubleshoot the vehicle lightening system

List of Tools and Equipment

1	Tools trolley (Complete set of hand tools)

2	Scanner

3	Test lamp

4	DMM (Multi meter)

- Repair Cooling System and Vehicle Accessories

Overview: These competency standards designed to enable the learner to rectify electrical related troubles IN the vehicle and understand the Wiring of the vehicle.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Repair Cooling System and Vehicle Accessories | - Inspects the Radiator fan operation for correct operation under specified testing instructions. - Inspects the thermal switch for correct operation under specified testing instructions. - Inspects the temperature sender for correct operation under specified testing instructions. |
| - Repair Electrical Accessories | - Inspects the oil warning switch operation for correct operation under specified testing instructions. - Inspects fuse, relay operation for correct operation under specified testing instructions. - Inspects the fuel sender operation for correct operation under specified testing instructions. - Inspects the wiper motor, washer motor for correct operation under specified testing instructions. - Inspects heating Elements for correct operation under specified testing instructions |

Knowledge & Understanding

- Identify terms used and calculations associated with the electricity. Definitions (magnetism, self/mutual induction). Laws (amperes rule, Fleming’s rule)

- Describe the use of following materials in vehicle with example. Conductor (Good/ bad), Insulator, semiconductors, electrical Cable (Color coding, ampere rating, wire harness, cross sectional area, cable identification method).

- Differentiates each in following. Switch (manual, magnet, pressure, temperature)sender (fuel level, temperature)

Critical Evidence(s) Required

- Capable to troubleshoot the vehicle lightening system

- Capable to describe the fault related to the electrical lights and cooling system, and take appropriate decisions.

List of Tools and Equipment

1	Vehicle/ simulator,2	Digital Multi Meter (DMM).

3	Tools box, 4	test lamp

- Recognize Electrical system of Vehicle

Overview: This Competency Standard identifies the competencies required to identify machine electrical system, Inspect machine electrical system,

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify machine electrical system | - Identify the wiring - Identify the sensors - Identify the solenoids - Identify the switches - Identify the fuse - Identify the relay - Identify the alternator - Identify the starting motor - Identify the battery - Identify the motors - Identify the resistors, diodes - Identify lights, horn |
| - Inspect machine electrical system | - Inspect the wiring - Inspect the sensors - Inspect the solenoids - Inspect the switches - Inspect the fuse - Inspect the relay - Inspect the alternator - Inspect the starting motor - Inspect the battery - Inspect the motors - Inspect the resistors, diodes - Inspect lights, horn |
| - Troubleshoot of machine electrical system | - Troubleshoot the wiring - Troubleshoot the sensors - Troubleshoot the solenoids - Troubleshoot the switches - Troubleshoot the fuse - Troubleshoot the relay - Troubleshoot the alternator/ - Troubleshoot the starting motor - Troubleshoot the battery - Troubleshoot the motors - Troubleshoot the resistors, diodes. - Troubleshoot the lights, horn |
| - Repairing of machine electrical system | - Repair the wiring - Repair the alternator - Repair the starting motor - Repair the motors |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe Electrical wiring

- Describe the sensors

- Describe the solenoids

- Describe the switches

- Describe the fuse

- Describe the relay

- Describe the alternator

- Describe the starting motor

- Describe the battery

- Describe the motors

- Describe the resistors, diodes

- Describe the lights, horn

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to repair Electrical wiring

- Capable to check the sensors

- Capable to check the solenoids

- Capable to check switches

- Capable to check fuse

- Capable to check relay

- Capable to check and repair alternator

- Capable to check and repair starting motor

- Capable to check battery

- Capable to check motors

- Capable to check resistors, diodes

- Capable to check lights, horn

List of Tools and Equipment’s

1	Tools trolley (Complete set of Hand Tools)

2	Special Service tools

# Generic

### 10.6.1	Participate in Workplace Communication

Overview: This unit covers the knowledge, skills and attitudes required to gather, interpret and convey information in response to workplace requirements

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Obtain and convey Workplace information | - Assess the Specific and relevant information from the appropriate sources - Convey and gather the information of Effective questioning, active listening and speaking skills. - Transfer the information using the appropriate medium and ideas. - Use appropriate non- verbal communication. - Identify and follow appropriate lines of communication with supervisors and colleagues. - Use the defined workplace procedures for the location and storage of information. - Carry out Personal interaction clearly and concisely. |
| - Participate in workplace meetings and discussions | - Attend the team meetings on time - Clearly express your own opinions and listen other’s point of view without interruption. - Established the protocols in the courteous manner of meeting inputs with the meeting purpose and Workplace interactions. - Questions about simple routine workplace procedures and matters concerning working conditions of employment are asked and responded to Meetings outcomes are interpreted and implemented |
| - Complete relevant work related documents | - Range of forms relating to conditions of employment are completed accurately and legibly - Workplace data is recorded on standard workplace forms and documents - Perform the basic mathematical processes that are used for routine calculations - Identify the errors in recording information on forms/ documents and properly acted upon - Reporting requirements to supervisor are completed according to organizational guidelines |

##### Knowledge & Understanding

Describe the following.

- Appropriate sources: Team members, Suppliers, Trade personnel, Local government, Industry bodies

- Medium: Memorandum, Circular, Notice, Information discussion, Follow-up or verbal, instructions, Face to face communication.

- Storage: Manual filing system, Computer-based filing system.

- Forms: Personnel forms, telephone message forms, safety reports.

- Workplace interactions: Face to face, Telephone, Electronic and two way radio, Written including electronic, memos, instruction, and forms, non-verbal including gestures, signals, signs and diagrams

- Protocols: Observing meeting, Compliance with meeting decisions, Obeying meeting instructions

### 10.6.2.	Practice Occupational Health and Safety Procedures

Overview; This unit covers the outcomes required to comply with regulatory and organizational requirements for occupational health and safety

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Identify hazards and risks | - Identify the safety regulations and workplace safety and hazard control practices and procedures are clarified and explained based on organization procedures - Identify the hazards/risks in the workplace and their corresponding indicators to minimize or eliminate risk to co-workers, workplace and environment in accordance with organization procedures. - Contingency measures during workplace accidents, fire and other emergencies are recognized and established in accordance with organization procedures |
| - Evaluate hazards and risks | - Identify the terms of maximum tolerable limits which when exceeded will result in harm or damage are identified based on threshold limit values (TLV) - Identify the effects of the hazards. - Identify the OHS issues and/or concerns and safety hazards are reported to designated personnel in accordance with workplace requirements and relevant workplace OHS legislation. |
| - Control hazards and risks | - Identify the occupational Health and Safety (OHS) procedures for controlling hazards/risks in workplace are consistently followed - Procedures for dealing with workplace accidents, fire and emergencies are followed in accordance with organization OHS policies - Use of Personal protective equipment (PPE) in accordance with organization OHS procedures and practices. - Provide the appropriate assistance in the event of a workplace emergency in accordance with established organization protocol. |
| - Maintain OHS awareness | - Practice the emergency-related drills and trainings as per established organization guidelines and procedures. - Update and complete the OHS personal records in accordance with workplace requirements. |

##### Knowledge & Understanding

- Safety regulations: May include but are not limited to: Clean Air Act, Building code, National Electrical and Fire Safety Codes, Waste management statutes and rules, Philippine Occupational Safety and Health Standards, DOLE regulations on safety legal requirements, ECC regulations.

- Hazards/Risks: May include but are not limited to: Physical hazards – impact, illumination, pressure, noise, vibration, temperature, radiation, Biological hazards- bacteria, viruses, plants, parasites, mites, molds, fungi, insects, Chemical hazards – dusts, fibers, mists, fumes, smoke, gasses, vapors Ergonomics, Psychological factors – over exertion/ excessive force, awkward/static positions, fatigue, direct pressure, varying metabolic cycles Physiological factors – monotony, personal, relationship, work out cycle.

- Contingency measures: May include but are not limited to: Evacuation, Isolation, Decontamination, (Calling designed) emergency personnel.

- PPE: May include but are not limited to: Mask, Gloves, Goggles, Hair Net/cap/bonnet, Face mask/shield, Ear muffs, Apron/Gown/coverall/jump suit, Anti-static suits.

- Emergency-related drills and training: Fire drill, Earthquake drill, Basic life support/CPR, First aid, Spillage control, Decontamination of chemical and toxic, Disaster, preparedness/management.

- OHS personal records: Medical/Health records, Incident reports, Accident reports, OHS-related training completed.

### Perform Work in Team Environment

Overview: This unit covers the skills, knowledge and attitudes to identify role and responsibility as a member of a team

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Describe team role and scope | - Identify the role and objective of the team from available sources of information. - Identify team parameters, reporting relationships and responsibilities from team discussions and appropriate external sources. |
| - Identify own role and responsibility within team | - Identify the individual role and responsibilities within the team environment. - Identify and recognize the roles and responsibility of other team members. - Reporting relationships within team and external to team are identified |
| - Work as a team member | - Effective and appropriate forms of communications used and interactions undertaken with team members who contribute to known team activities and objectives - Effective and appropriate contributions made to complement team activities and objectives, based on individual skills and competencies and workplace context - Observed protocols in reporting using standard operating procedures - Contribute to the development of team work plans based on an understanding of team’s role and objectives and |

Knowledge & Understanding:

- Role and objective of team: Work activities in a team environment with enterprise or specific sector, Limited discretion, initiative and judgment maybe demonstrated on the job, either individually or in a team environment.

- Sources of information: Standard operating and/or other workplace procedures, Job procedures, Machine/equipment manufacturer’s specifications and instructions, Organizational or external personnel, Client/supplier instructions, Quality standards, OHS and environmental standards.

- Workplace context: Work procedures and practices, Conditions of work environments, Legislation and industrial agreements, Standard work practice including the storage, safe handling and disposal of chemicals, Safety, environmental, housekeeping and quality guidelines.

- List of Machinery, Tools & Equipment.

(FOR A CLASS OF 25 STUDENTS)

## 11.1. Internal Combustion Engine

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
|  | Car Lift | 01 No |  | Emery Paste and Stick, | 25 Nos |
|  | Compression tester | 05 Nos |  | Hose Pipe, | 25 Nos |
|  | Diesel engine vehicle or simulator | 01 No |  | Oil Seals, | 25 Nos |
|  | Drive belt tensioner gauge | 05 Nos |  | Coolants, | 15 Liters |
|  | Engine Petrol/ diesel | 01 No each |  | cotton waste. | 10 kg |
|  | Gas analyzer | 01 No |  | Engine Oil, | 10 Liters |
|  | Hand tools trolley | 05 Nos |  | Gasket Set, | 25 Nos |
|  | Measuring tools (Complete set for engine overhauling) | 05 Nos |  | grease, | 2 kg |
|  | Measuring tools (Vernier caliper, micrometer, Cylinder bore gauge) | 25 Nos |  | head Gasket, | 25 Nos |
|  | Pressure Tester | 05 Nos |  | oil filter, | 25 Nos |
|  | Radiator leak tester | 05 Nos |  | Oil sump Jain, | 25 Nos |
|  | Safety wears | 25 Nos |  | Piston ring, | 25 Nos |
|  | Sealant/gasket | 25 Nos |  | Sealant/gasket, | 25 Nos |
|  | Special tools (ring compressor, belt tensioner, torque wrench) | 25 Nos |  | Silicon, | 5 Packet’s |
|  | Tools trolley (Complete set of hand tools) | 25 Nos |  | Thermostat Valve | 25 Nos |
|  | Vacuum pump | 05 Nos |  | Valve Seats, | 25 Nos |
|  | Vehicle | 02 Nos |  |  |  |
|  | Vehicle/ simulator with a cooling system | 01 No |  |  |  |

## 11.2. Automotive Electrics and Electronics

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
|  | Ampere meter | 25 Nos |  | Battery | 05 Nos |
|  | Bar Magnet | 25 Nos |  | Bulb and Bulb Holder, | 25 Nos |
|  | Battery External charger | 25 Nos |  | Circuit Board, | 25 Nos |
|  | Battery load tester | 25 Nos |  | Thimble (Male & Female), | 25 Nos |
|  | Coil | 25 Nos |  | Wire Clumps, | 25 Nos |
|  | Compass needle | 25 Nos |  | Wires, | 05 Nos |
|  | Connecting leads | 25 Nos |  |  |  |
|  | Current carrying conductor | 25 Nos |  |  |  |
|  | Digital Multi Meter (DMM). | 25 Nos |  |  |  |
|  | Fuses | 25 Nos |  |  |  |
|  | Galvanometer | 05 Nos |  |  |  |
|  | Gloves | 25 Nos |  |  |  |
|  | Horse shoe magnet | 25 Nos |  |  |  |
|  | Hydrometer | 25 Nos |  |  |  |
|  | Iron nail as core | 25 Nos |  |  |  |
|  | Lamps | 25 Nos |  |  |  |
|  | Load (resistors) | 25 Nos |  |  |  |
|  | Magnet | 25 Nos |  |  |  |
|  | Power Supply | 05 Nos |  |  |  |
|  | Relay | 25 Nos |  |  |  |
|  | Safety goggles | 25 Nos |  |  |  |
|  | Scanner | 02 Nos |  |  |  |
|  | Special Service tools | 25 Nos |  |  |  |
|  | Test benches (Complete accessories) | 25 Nos |  |  |  |
|  | Test lamp | 25 Nos |  |  |  |
|  | Thin coated copper wire | 25 Nos |  |  |  |
|  | Timing gun | 05 Nos |  |  |  |
|  | Tool box | 25 Nos |  |  |  |
|  | Tools trolley (Complete set of Hand Tools) | 05 Nos |  |  |  |
|  | Transformer | 05 Nos |  |  |  |
|  | Vehicle or simulator. | 01 No |  |  |  |
|  | Wires | 05 Nos |  |  |  |