National Vocational Diploma Level -5 in “Electronics Technology” (Associate Engineer /Electronics Supervisor)

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

# ACKNOWLEDGEMENTS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to many representatives of business, industry, academia, government agencies, Provincial TEVTAs, Sector Skill Councils and trade associations who speared their time and expertise to the development and validation of these National Vocational Qualifications (Competency Standards, Curricula, Assessments Packs and related material). This work would not have been possible without the financial and technical support of the TVET Sector Support Programme co-funded by European Union, Germans and Norwegian and German Governments implemented by GIZ Pakistan. NAVTTC is especially indebted to Dr. Muqeem ul Islam, who lead the project from the front. The core team was comprised on:

- Dr. Muqeem ul Islam, Director General (Skills Standards and Curricula) NAVTTC

- Mr. Muhammad Naeem Akhtar, Senior Technical Advisor TSSP-GIZ,

- Mr. Muhammad Yasir, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Ishaq, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Fayaz Soomro, Deputy Director (SS&C Wing) NAVTTC

NAVTTC team under the leadership of Dr. Muqeem ul Islam initiated development of CBT & A based qualifications of diploma level-5 as a reform project of TVET sector in November 2018 and completed 27 NVQF diplomas of Level-5 in September, 2019. It seems worth highlighting that during this endeavor apart from developing competency standards/curricula in conventional trades new dimensions containing high-tech trades in TVET sector in the context of generation IR 4.0 trades have also been developed which inter alia includes Robotics, Mechatronics, artificial intelligence, industrial automation, instrumentation and process control. Moreover, trades like entrepreneurship, green/environmental skills and variety of soft/digital skill have also been developed to equipped the Pakistani youth with skills set as per requirement of the global trends. These skills have been made integral part of all the 27 diplomas.

Nobody has been more important in the pursuit of this project than Dr. Nasir Khan, Executive Director, NAVTTC, whose patronage and support remain there throughout the development process and lastly to thanks specially to Syed Javed Hassan Chairman NAVTTC and Raja Saad Khan, Deputy Team Lead TSSP-GIZ who made it happened in this challenging time.

CONTENTS

Contents

ACKNOWLEDGEMENTS	2

1.	INTRODUCTION	8

2.	PURPOSE OF THE QUALIFICATION	9

3.	COMMON COURSES WITH RESPECTIVE LEVELS	9

4.	MAPPING OF THE QUALIFICATION	11

5.	DATE OF VALIDATION	12

6.	DATE OF REVIEW	12

7.	CODE OF QUALIFICATION	12

8.	QUALIFICATIONS DEVELOPMENT COMMITTEE	13

9.	QUALIFICATION VALIDATION COMMITTEE	14

10.	ENTRY REQUIREMENTS	15

11.	OCCUPATION PROFILE LEVEL PACKAGING CHART	15

12.	PACKAGING OF QUALIFICATION	16

Soft Skills………………...	19

Develop workplace policy and procedures for sustainability	19

Maintain professionalism in the workplace	21

Manage personal work priorities and Professional development	22

Manage workforce planning	24

Undertake project work	27

Prepare and implement negotiation	30

Manage meetings	………………………………………………………………………………..33

Organize schedules	35

Identify and Communicate Trends in Career Development	37

Apply Specialist Interpersonal and Counselling Interview Skills	40

0714E&A35	Digital Electronics	44

Manipulate the Number System	44

Verify Truth tables of Digital Logic Gates	46

Construct & Verify the Combinational Logic Circuit	48

Construct Shift Registers and Counters Using Flip Flops	51

Use 555 Timer IC as Multi-Vibrator	53

Communication systems	54

Identify the parts of Analog& digital telephone set & verify their functions	54

Construct the AM / FM Transmitter & Receiver & verify their functionality	56

Demonstrate modulation, demodulation, multiplexing & de-multiplexing	58

Install Chord less telephone, PABX, Fax machine & satellite dish system	61

Apply different concepts of propagation of waves	62

Power Electronics	63

Design Electronic Control circuits using Power Diodes	63

Design Single Phase and Three Phase Converters	66

Design the Inverters	69

Industrial Electronics	71

Carry Out System Automation Using Sensors and Transducers	71

Carry out Industrial Automation Using PLC	75

Carry out System Automation using Linear Control system	80

Microcontroller Programming and Applications	81

Carryout Microcontroller Applications.	81

Apply Microcontroller to build Control System.	83

Equipment Maintenance and Servicing	85

Repair/service different Home Appliance	85

Entrepreneurial Skills	90

Develop a Marketing Plan	90

Develop and Review a Business Plan.	94

Manage Human Resources	97

Negotiate for Resolving Business Issues	100

Manage Personal Finances	102

Lead Small Teams….	104

# INTRODUCTION

The Technical and Vocational is a profession that is increasingly getting attention in Pakistan, not only among the youth seeking to enter the industry but also among adults who wish to polish their skills to develop a career out of it. Scope and demand of Electronics engineers, Supervisors, Operators and Technicians is now increasing day by day in the industry. This qualification is designed so as to inculcate the competencies of basic design and construction of electronic circuits, identification and implementation of certain electronic components, linkage between the components and their industrial workability. The fundamental concepts of digital systems, the practical aspects of digital system design will be covered using hardware which is widely used in industry. Students will gain hands on experience in designing, Servicing and implementing a number of digital systems.

Electronics Diploma is further sub-divided into many other fields including electronics, computer engineering, power engineering, telecommunications, control systems, and microelectronics. Many of these sub disciplines overlap with other engineering branches, spanning a huge number of specializations such as hardware engineering, power electronics, avionics, mechatronics and nanotechnology.

The National Skills Strategy (NSS) aims at achieving a paradigm shift from time-bound   and supply led to competency-based and demand driven training in Pakistan. Competency-based training approach focuses on the demonstration of actual skills required in the workplace/industry. To achieve this strategy, NAVTTC in collaboration with GIZ is involved in the development process of CBT program to ensure competent skilled labor in demand driven vocational trades.

- National Vocational Qualification Framework (NVQF) as designed in consultation with the stakeholders including academia, researchers, industry, chambers and TEVTAs. Its aim is to identify knowledge & skills required for a qualification, determine equivalence, provide guidelines for Recognition of Prior Learning (RPL) and assure quality of training. NVQF designed for the Vertical and horizontal progression for learners, recognition of qualification, meeting with the national and international standards, facilitate conversion of informal training to formal training through RPL, improvement in quality of training and increased options for learners for selecting training programs in different trades. Competency Standards are performance specification that identify the knowledge and competencies an individual need to succeed in the workplace.

# PURPOSE OF THE QUALIFICATION

This qualification will equip the students with knowledge of how the industry works, as well as giving them the technological knowledge and technical skills which are needed to design, assess and improve electrical and electronic systems.

This qualification will help candidate to design and construct the electronic circuits, identify and implement certain electronic components, elaborate linkage between the components and their industrial workability & maintenance of various electronic devices.

# COMMON COURSES WITH RESPECTIVE LEVELS

LEVEL 2

LEVEL 3

LEVEL 4

LEVEL 5

# MAPPING OF THE QUALIFICATION

Power Electronics…..

Analog Electronics

Digital Electronics

Wiring

AC DC Machines…

Process Variable Measurements….

Microcontrollers

Analog Electronics

Digital Electronics

Soft Skills

Digital Skills

Entrepreneurship

# DATE OF VALIDATION

These national qualifications have been validated by the Qualification Development Committee (QVC) on 27-29 May, 2019 in Lahore and will remain valid till 29-May-2029.

# DATE OF REVIEW

These national qualifications shall be reviewed by the validation committee on 30-May-2022.

# CODE OF QUALIFICATION

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Electrical Technology level 5 |  |
| --- | --- |
| Code | Description |
| 0714E&A(1) | 1st Level D.A. E National Certificate of level-5, in “ Electronics Technology” |
| 0714E&A(2) | 2nd Level D.A. E National Certificate of level-5, in “Electronics Technology” |
| 0714E&A(3) | 3rd Level D.A. E National Certificate of level-5, in “Electronics Technology” |
| 0714E&A(4) | 4th Level D.A. E National Certificate of level-5, in “Electronics Technology” |
| 0714E&A(5) | 5th Level D.A. E National Certificate of level-5, in “Electronics Technology” |

# QUALIFICATIONS DEVELOPMENT COMMITTEE

The Qualifications Development Committee consisted of following members

| Name & Designation | Organization |
| --- | --- |
| Mr. Muhammad Yasir, AD NAVTTC | NAVTTC |
| Mr.Ayub Nasir | DACCUM Facilitator |
| Mr. Raheel Ahmed, Lecturer | COMSATS University Islamabad |
| Mr. Muhammad Aleem, Researcher | PAEC, DG Khan |
| Mr. Noman Khan, Senior Engr | COMCEPT Electronics |
| Mr. Adeel Iqbal, Lecturer | COMSATS University Islamabad |
| Mr. Tahir Muhammad Khan, Design Engineer | CARE , Islamabad |
| Mr. Hamza Ikhlaq, Lecturer | ISRA University, Islamabad |
| Mr. Imran Ahmed, Instructor | GCT Rawalpindi |
| Mr. Mudassar Ali, Instructor | Swedish Group of Colleges |
| Mr. SardarHussain, Instructor PLC | PITAC, Peshawar |
| Mr. Abdul BasitTaj, Instructor | NIE Islamabad |
| Mr. ShehzadRaza, Lab Designer | NSETS Pvt. LTD |
| Ms.Siqa,Instructor | P-TEVTA |
| Mr.AliAnwer Shah, assistant Professor | S-TEVTA |

# QUALIFICATION VALIDATION COMMITTEE

The following members participated in the qualification’s development workshop 27-29 May 2019 in Lahore.

| Name & Designation | Organization |
| --- | --- |
| Ms. Bushra Asghar DACCUM Facilitator | P-TEVTA |
| M. Hamza Akhlaq, Lecturer | Isra University, Islamabad. |
| Eng. Syed Ali Anwar Shah, Assistant Professor | S-TEVTA |
| Mst.Saiqa Ijaz, Instructor (GCT) | P-TEVTA |
| F.M.Kayfi, Chairman | PBTE |
| Fazal-E-Elahi,Asst.prof. | KP-TEVTA |
| Amir Amin, HOD Electrical. | City Poly Tech. Inst. |
| Aamir Ali Khan, Asst. Director, NAVTTC | NAVTTC |
| Muhammad Yasir, Asst. Director, NAVTTC | NAVTTC |

# ENTRY REQUIREMENTS

Entry requirements of this qualification are Matric Science or level 4 or matric equivalent

# OCCUPATION PROFILE LEVEL PACKAGING CHART

# PACKAGING OF QUALIFICATION

The national vocational qualifications are packaged as per following:

| Competency Standards | NVQF Level | Category | Estimated Contact Hours |  |  | Credit Hours |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Theory | Practical | Total |  |
| National Vocational Certificate Level 5 in Electronics Technology (Associate Engineer/Supervisor) |  |  |  |  |  |  |
| Develop Workplace Policy and Procedures for Sustainability | Level 5 | Generic | 6 | 12 | 18 | 1.8 |
| Maintain Professional Development and Career Professionalism | Level 5 | Generic | 6 | 24 | 30 | 3 |
| Manage Personal Work Priorities and Professional Development | Level 5 | Generic | 6 | 15 | 21 | 2.1 |
| Manage Workforce Planning | Level 5 | Generic | 6 | 12 | 18 | 1.8 |
| Perform Undertaking of Project Work | Level 5 | Generic | 5 | 18 | 23 | 2.3 |
| Prepare and Implement Negotiation | Level 5 | Generic | 6 | 12 | 18 | 1.8 |
| Manage Meetings | Level 5 | Generic | 6 | 12 | 18 | 1.8 |
| Organise Schedule | Level 6 | Generic | 6 | 12 | 18 | 1.8 |
| Identify and Communicate Trends in Career Development | Level 5 | Generic | 6 | 15 | 21 | 2.1 |
| Apply Specialist Interpersonal and Counselling Interview Skills | Level 6 | Generic | 6 | 15 | 21 | 2.1 |
| Manipulate The Number Systems | Level 5 | Technical | 6 | 24 | 30 | 3 |
| Verify the Truth Tables for Logic Gates | Level 5 | Technical | 9 | 24 | 33 | 3.3 |
| Construct & Verify the combinational Logic Circuits | Level 5 | Technical | 9 | 24 | 33 | 3.3 |
| Construct Shift Registers and Counters using Flip Flops | Level 5 | Technical | 9 | 24 | 33 | 3.3 |
| Use 555 Timer IC as a Multi-Vibrator | Level 5 | Technical | 9 | 24 | 33 | 3.3 |
| Identify the parts of Analog & digital telephone set & verify their functions | Level 5 | Technical | 8 | 18 | 26 | 2.6 |
| Construct the AM / FM Transmitter & Receiver & verify their functionality | Level 5 | Technical | 8 | 24 | 32 | 3.2 |
| Demonstrate modulation, demodulation, multiplexing & de-multiplexing | Level 5 | Technical | 8 | 24 | 32 | 3.2 |
| Install Chord less telephone, PABX, Fax machine & satellite dish system | Level 5 | Technical | 8 | 24 | 32 | 3.2 |
| Apply different concepts of propagation of waves | Level 5 | Technical | 8 | 24 | 32 | 3.2 |
| Design the Electronic Control circuits using Power Diodes | Level 5 | Technical | 10 | 30 | 40 | 4 |
| Design Single Phase & Three Phase Converters. | Level 5 | Technical | 8 | 36 | 44 | 4.4 |
| Design the Inverters | Level 5 | Technical | 9 | 36 | 45 | 4.5 |
| Carryout System Automation using Sensors & Transducers | Level 5 | Technical | 12 | 36 | 48 | 4.8 |
| Carryout Industrial Automation Using PLC | Level 5 | Technical | 14 | 36 | 50 | 5 |
| Carryout System Automation using Linear Controls | Level 5 | Technical | 12 | 36 | 48 | 4.8 |
| Carryout various Microcontroller Applications. | Level 5 | Technical | 9 | 36 | 45 | 4.5 |
| Apply Microcontroller to build Control System. | Level 5 | Technical | 10 | 36 | 46 | 4.6 |
| Repair/service different Electronics Home Appliance | Level 5 | Technical | 12 | 36 | 48 | 4.8 |
| Develop a Marketing Plan | Level 5 | Enterpreneureship | 6 | 36 | 42 | 4.2 |
| Develop and Review a Business Plan. | Level 5 | Enterpreneureship | 6 | 36 | 42 | 4.2 |
| Manage Human Resources | Level 5 | Enterpreneureship | 9 | 30 | 39 | 3.9 |
| Negotiate for Resolving Business Issues | Level 5 | Enterpreneureship | 9 | 30 | 39 | 3.9 |
| Manage Personal Finances | Level 5 | Enterpreneureship | 6 | 45 | 51 | 5.1 |
| Lead Small Teams | Level 5 | Enterpreneureship | 6 | 45 | 51 | 5.1 |
| Total |  |  | 279 | 921 | 1200 | 120 |

# Soft Skills

### Develop workplace policy and procedures for sustainability

Overview:

This unit describes the skills and knowledge required to develop and implement a workplace sustainability policy and to modify the policy to suit changed circumstances. It applies to individuals with managerial responsibilities who undertake work developing approaches to create, monitor and improve strategies and policies within workplaces and engage with a range of relevant stakeholders and specialists.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Develop workplace sustainability policy | Define scope of sustainability policy Gather information from a range of sources to plan and develop policy Identify and consult stakeholders as a key component of the policy development process Include appropriate strategies in policy at all stages of work for minimizing resource use, reducing toxic material and hazardous chemical use and employing life cycle management approaches Make recommendations for policy options based on likely effectiveness, timeframes and cost Develop policy that reflects the organization s commitment to sustainability as an integral part of business planning and as a business opportunity Agree to appropriate methods of implementation, outcomes and performance indicators |
| Communicate workplace sustainability policy | P1 Promote workplace sustainability policy, including its expected outcome, to key stakeholders P2 Inform those involved in implementing the policy about expected outcomes, activities to be undertaken and assigned responsibilities |
| Implement workplace sustainability policy | P1 Develop and communicate procedures to help implement workplace sustainability policy P2 Implement strategies for continuous improvement in resource efficiency P3 Establish and assign responsibility for recording systems to track continuous improvements in sustainability approaches |
| Review workplace sustainability policy implementation | P1 Review workplace sustainability policy implementation P2 Investigate successes or otherwise of policy P3 Monitor records to identify trends that may require remedial action and use to promote continuous improvement of performance P4 Modify policy and or procedures as required to ensure improvements are made |

Knowledge and Understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Outline the environmental or sustainability legislation, regulations and codes of practice applicable to the organization identify internal and external sources of information and explain how they can be used to plan and develop the organization s sustainability policy

- Explain policy development processes and practices

- Outline organizational systems and procedures that relate to sustainability

- Outline typical barriers to implementing policies and procedures in an organization and possible strategies to address them.

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to develop workplace policy and procedures for sustainability. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- scope and develop organizational policies and procedures that comply with legislative requirements and support the organization s sustainability goals covering at a minimum:

  - minimizing resource use

  - resource efficiency

  - reducing toxic material and hazardous chemical use

  - employing life cycle management approaches

  - continuous improvement

- plan and implement sustainability policy and procedures including:

  - agreed outcomes

  - performance indicators

  - activities to be undertaken

  - assigned responsibilities

  - record keeping, review and improvement processes

- consult and communicate with relevant stakeholders to generate engagement with sustainability policy development, implementation and continuous improvement

- Review and improve sustainability policies.

### Maintain professionalism in the workplace

Overview:

This unit of competency describes the outcomes required maintain a professional image in the workplace, including behaving ethically, demonstrating motivation, respecting timeframes and maintaining personal appearance.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Respect work timeframes | P1 Demonstrate punctuality in meeting, set working hours and times. P2 Utilize working hours only for working and follow company regulations. P3 Complete work tasks within deadlines according to order of priority P4 Supervisors are informed of any potential delays in work times or projects. |
| 2. Maintain personal appearance and hygiene | P1 Clean hair, body and nails regularly. P2 Wear suitable cloths for the workplace, and respect local and cultural contexts P3 Meet specific company dress code requirements |
| 3. Maintain adequate distance with colleagues and clients | P1 Respect personal space of colleagues and clients with reference to local customs and cultural contexts. P2 Keep sufficient distance from others P3 Avoid cross transmission of infections (especially through respiration). |
| 4. Work in an ethical manner | P1 Follow company values/ethics codes of ethics and/or conduct, policies and guidelines. P2 Use company resources in accordance with company ethical standards. P3 Conduct personal behavior and relationships in accord with ethical standards and company policies. P4 Undertake work practices in compliance with company ethical standards, organizational policy and guidelines. P5 Instruct co-workers on ethical, lawful and reasonable directives. P6 Share company values/practices with co-workers using appropriate behavior and language. P7 Report work incidents/situations and/or resolved in accordance with company protocol/guidelines. |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain application of good manners and right conduct

- Explain basic practices for oral and personal hygiene

- Describe common products used for oral and personal hygiene

- Outline the company code of conduct/values

- Outline the Company regulations, performance and ethical standards

- Explain work responsibilities/job functions

- Describe communication skills

- State workplace hygiene standards

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to maintain professionalism in the workplace .The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- clarify and affirm work values/ethics/concepts consistently in the workplace;

- comply with required working times;

- conduct work practices satisfactorily and consistently, in compliance with work ethical standards, organizational policy and guidelines;

- Develop suitable hygiene

- Keep adequate distance while interacting with colleagues and clients.

### Manage personal work priorities and Professional development

Overview:

This unit describes the skills and knowledge required to create systems and process to organize information and prioritize tasks. It applies to individuals working in managerial positions who have excellent organizational skills. The work ethic of individuals in this role has a significant impact on the work culture and patterns of behavior of others as managers at this level are role models in their work environment.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Establish personal work goals | Serve as a positive role model in the workplace through personal work planning Ensure personal work goals, plans and activities reflect the organization s plans, and own responsibilities and accountabilities Measure and maintain personal performance in varying work conditions, work contexts and when contingencies occur |
| 2. Set and meet own work priorities | Take initiative to prioritize and facilitate competing demands to achieve personal, team and organizational goals and objectives Use technology efficiently and effectively to manage work priorities and commitments Maintain appropriate work-life balance, and ensure stress is effectively managed and health is attended to |
| 3. Develop and maintain professional competence | Assess personal knowledge and skills against competency standards to determine development needs, priorities and plans Seek feedback from employees, clients and colleagues and use this feedback to identify and develop ways to improve competence Identify, evaluate, select and use development opportunities suitable to personal learning style/s to develop competence Participate in networks to enhance personal knowledge, skills and work relationships Identify and develop new skills to achieve and maintain a competitive edge |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain principles and techniques involved in the management and organization of:

- performance measurement

- personal behavior, self-awareness and personality traits identification

- a personal development plan

- personal goal setting

- time

- Discuss management development opportunities and options for self

- Describe methods for achieving a healthy work-life balance

- Outline organization s policies, plans and procedures

- Explain types of learning style/s and how they relate to the individual

- Describe types of work methods and practices that can improve personal performance.

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage personal work priorities and professional development. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- use business technology to create and use systems and processes to organise and priorities tasks and commitments

- measure and maintain personal work performance including assessing competency against competency standards and seeking feedback

- maintain an appropriate work-life balance to manage personal health and stress

- participate in networks

- develop a personal development plan which includes career objectives and an action plan

- Develop new skills.

### Manage workforce planning

Overview:

This unit describes the skills and knowledge required to manage planning in relation to an organization’s workforce including researching requirements, developing objectives and strategies, implementing initiatives and monitoring and evaluating trends. It applies to individuals who are human resource managers or staff members with a role in a policy or planning unit that focuses on workforce planning.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Research workforce requirements | Review current data on staff turnover and demographics Assess factors that may affect workforce supply Establish the organization’s requirements for a skilled and diverse workforce |
| 2. Develop workforce objectives and strategies | Review organizational strategy and establish aligned objectives for modification or retention of the workforce Consider strategies to address unacceptable staff turnover, if required Define objectives to retain required skilled labor Define objectives for workforce diversity and cross-cultural management Define strategies to source skilled labor Communicate objectives and rationale to relevant stakeholders Obtain agreement and endorsement for objectives and establish targets Develop contingency plans to cope with extreme situations |
| 3. Implement initiatives to support workforce planning objectives | Implement action to support agreed objectives for recruitment, training, redeployment and redundancy Develop and implement strategies to assist workforce to deal with organizational change Develop and implement strategies to assist in meeting the organization’s workforce diversity goals Implement succession planning system to ensure desirable workers are developed and retained Implement programs to ensure workplace is an employer of choice |
| 4. Monitor and evaluate workforce trends | Review workforce plan against patterns in exiting employee and workforce changes Monitor labor supply trends for areas of over- or under-supply in the external environment Monitor effects of labor trends on demand for labor Survey organizational climate to gauge worker satisfaction Refine objectives and strategies in response to internal and external changes and make recommendations in response to global trends and incidents Regularly review government policy on labor demand and supply Evaluate effectiveness of change processes against agreed objectives |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain current information about external labor supply relevant to the specific industry or skill requirements of the organization

- Outline industrial relations relevant to the specific industry

- Describe labor force analysis and forecasting techniques

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage workforce planning. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- review and interpret information from a range of internal and external sources to identify:

  - current staff turnover and demographics

  - labor supply trends factors that may affect workforce supply

  - organization’s workforce requirements objectives and strategies

- manage workforce planning including developing, implementing, monitoring and reviewing strategies to meet workforce needs

- review relevant trends and supply and demand factors that will impact on an organization’s workforce

- Develop a workforce plan that includes relevant research and specific strategies to ensure access to a skilled and diverse workforce.

### Undertake project work

Overview:

This unit describes the skills and knowledge required to undertake a straightforward project or a section of a larger project. It covers developing a project plan, administering and monitoring the project, finalizing the project and reviewing the project to identify lessons learned for application to future projects. This unit applies to individuals who play a significant role in ensuring a project meets timelines, quality standards, budgetary limits and other requirements set for the project.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Define project | Access project scope and other relevant documentation Define project stakeholders Seek clarification from delegating authority of issues related to project and project parameters Identify limits of own responsibility and reporting requirements Clarify relationship of project to other projects and to the organization’s objectives Determine and access available resources to undertake project |
| 2. Develop project plan | Develop project plan in line with the project parameters Identify and access appropriate project management tools Formulate risk management plan for project, including Work Health and Safety (WHS) Develop and approve project budget Consult team members and take their views into account in planning the project Finalize project plan and gain necessary approvals to commence project according to documented plan |
| 3. Administer and monitor project | Take action to ensure project team members are clear about their responsibilities and the project requirements Provide support for project team members, especially with regard to specific needs, to ensure that the quality of the expected outcomes of the project and documented time lines are met Establish and maintain required recordkeeping systems throughout the project Implement and monitor plans for managing project finances, resources and quality Complete and forward project reports as required to stakeholders Undertake risk management as required to ensure project outcomes are met Achieve project deliverables |
| 4. Finalize project | Complete financial recordkeeping associated with project and check for accuracy Ensure transition of staff involved in project to new roles or reassignment to previous roles Complete project documentation and obtain necessary sign-offs for concluding project |

Knowledge and Understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Give examples of project management tools and how they contribute to a project

- Outline types of documents and other sources of information commonly used in defining the parameters of a project

- Explain processes for identifying and managing risk in a project

- Outline the organization’s mission, goals, objectives and operations and how the project relates to them

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to undertake project work. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance Requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

Define the parameters of the project including:

- project scope

- project stakeholders, including own responsibilities

- relationship of project to organizational objectives and other projects

- reporting requirements

- resource requirements

- Use project management tools to develop and implement a project plan including:

- deliverables

- work breakdown

- budget and allocation of resources

- timelines

- risk management

- recordkeeping and reporting

Consult and communicate with relevant stakeholders to generate input and engagement in planning, implementing and reviewing the project

Provide support to team members to enable them to achieve deliverables and to transition them as appropriate at completion of the project

Finalize the project including documentation, sign-offs and reporting

Review and document the project outcomes.

### Prepare and implement negotiation

Overview:

This unit covers the skills, knowledge and attitudes required to prepare for and participate in a process of negotiation.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Prepare for the negotiation | - Identify objectives and preferred outcome of the negotiation and determine minimum acceptable outcome - Understand in relation to what can be offered and what is needed from the other party - Gather information regarding the other party – objectives,needs, preferences, resources, what they want to achieve – inorder to determine best negotiating points - List and rank the issues to consider concessions that may be made. - Find examples and refine negotiation argument. - Check information to ensure it is correct and up-to-date. - Develop a negotiation plan that includes information about the other party and its interests and a set of responses and strategies to the anticipated tactics. - Prepare an agenda in advance, which includes discussion topics, participants, location and schedule |
| Participate in negotiations | - Analyze all aspects of the incident for degree of hazard, priorities, optional outcomes and appropriate strategies - Analyze and determine strategies and priorities on the incident sought from a range of sources - Assess long term objectives against resources and priorities - Apply a range of communication techniques to make and maintain contact with the key people - Provide clear and factual information to enable an honest and realistic assessment of the interests of the key people and their positions - Resolve the conflict and express their likely consequences clearly and do an analysis of the benefits - Reassess points of disagreements for common positive positions |
| Coordinate support Services | - Assess the need for support services in terms of the determined strategies and priorities - Negotiate the resources of support services according to established procedures and availability - Provide information on strategies to support services and maintain the communication - Delegate roles and responsibilities according to expertise and resources |
| Restore order | - Assess the incidents for degree of risk and take appropriate action to reduce and remove the impact of the incident and restore order - Take action designed to minimize risk and the preserve the safety and security of all involved - Take action to prevent the escalation of the incident appropriate to the circumstances and agreed procedures. - Carry out the use of force for the restoration of control and the maintenance of security in the least restrictive manner. - Complete reports accurately and clearly provided to the appropriate authority promptly - Review, evaluate and analyze the incident and the organizational response to it and report it promptly and accurately. |
| Provide leadership. direction and guidance to the work group | - Link between the function of the group and the - goals of the organization - Participate in decision making routinely to develop, implement and review work of the group and to allocate responsibilities where appropriate - Give opportunities and encouragement to others to develop new and innovative work practices and strategies - Identify conflict and resolve with minimum disruption to work group function - Provide staff with the support and supervision necessary to perform work safely and without risk to health - Allocate tasks within the competence of staff and support with appropriate authority, autonomy and training - Supervise appropriately the changing priorities and situations and takes into account the different needs of individuals and the requirements of the task |

Knowledge and Understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain organization’s policies, guidelines and procedures related to control and surveillance, safety and preventing and responding to incidents and breaches of orders covered in the range of variables.

- Explain organization’s management and accountability systems

- Describe teamwork principles and strategies

- Outline the principles of effective communication

- Outline the guidelines for use of equipment and technology

- Explain code of conduct

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to solve problems which jeopardize safety and security. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- Evidence of effective communication strategies including negotiation, counseling, mediation, advocacy demonstrated under pressure working effectively in a team environment

- Evidence of knowledge and application of organizations policies, procedures andguidelines for critical incidents

- Evidence of accurate and safe use of all emergency equipment

- Evidence of managing effective outcomes using strategic planning, team

- Leadership and situational analysis

### Manage meetings

Overview:

This unit describes the skills and knowledge required to manage a range of meetings including overseeing the meeting preparation processes, chairing meetings, organizing the minutes and reporting meeting outcomes. It applies to individuals employed in a range of work environments who are required to organize and manage meetings within their workplace, including conducting or managing administrative tasks in providing agendas and meeting material. They may work as senior administrative staff or may be individuals with responsibility for conducting and chairing meetings in the workplace.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Prepare for meetings | Develop agenda in line with stated meeting purpose Ensure style and structure of meeting are appropriate to its purpose Identify meeting participants and notify them in accordance with organizational procedures Confirm meeting arrangements in accordance with requirements of meeting Dispatch meeting papers to participants within designated timelines |
| 2. Conduct meetings | Chair meetings in accordance with organizational requirements, agreed conventions for type of meeting and legal and ethical requirements Conduct meetings to ensure they are focused, time efficient and achieve the required outcomes Ensure meeting facilitation enables participation, discussion, problem-solving and resolution of issues Brief minute-taker on method for recording meeting notes in accordance with organizational requirements and conventions for type of meeting |
| 3. Follow up meetings | Check transcribed meeting notes to ensure they reflect a true and accurate record of the meeting and are formatted in accordance with organizational procedures and meeting conventions Distribute and store minutes and other follow-up documentation within designated timelines, and according to organizational requirements Report outcomes of meetings as required, within designated timelines |

Knowledge and Understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Outline meeting terminology, structures, arrangements

- Outline responsibilities of the chairperson and explain group dynamics in relation to managing meetings

- Describe options for meetings including face-to-face, teleconferencing, web-conferencing and using webcams

- Identify the relevant organizational procedures and policies regarding meetings, chairing and minutes including identifying organizational formats for minutes and agendas.

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage meetings. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

Apply conventions and procedures for formal and informal meetings including:

- developing and distributing agendas and papers

- identifying and inviting meeting participants

- organizing and confirming meeting arrangements

- running the meeting and following up

- Organize, take part in and chair a meeting

- Record and store meeting documentation

- Follow organizational policies and procedures.

### Organize schedules

Overview:

This unit describes the skills and knowledge required to manage appointments and diaries for personnel within an organization, using manual and electronic diaries, schedules and other appointment systems. It applies to individuals employed in a range of work environments who provide administrative support to teams and individuals.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Establish schedule requirements | Identify organizational requirements and protocols for diaries and staff planning tools Identify organizational procedures for different types of appointments Determine personal requirements for diary and schedule items for individual personnel Establish appointment priorities and clarify in discussion with individual personnel |
| 2. Manage schedules | Identify recurring appointments and deadlines, and schedule these in accordance with individual and organizational requirements Establish availability of attendees, and schedule new appointments in accordance with required timelines and diary commitments Negotiate alternative arrangements and confirm when established appointments are changed Record appointments and manage schedules in accordance with organizational policy and procedures |

Knowledge and Understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Identify the key provisions of relevant legislation, standards and codes that affect aspects of business operations or the achievement of team goals

- Describe organizational requirements for managing appointments for personnel within the organization

- Summarize the range of appointment systems that could be used

- Outline important considerations when managing the schedules of others.

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to organize schedules. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Appropriately manage the schedules of various individuals through a process of careful planning and negotiation.

### Identify and Communicate Trends in Career Development

Overview:

This unit describes the skills and knowledge required to research and confirm career trends, Assess and confirm ongoing career development needs of target group and Maintain quality of career development services and professional practice conduct research to identify and communicate career trends. It establishes the need to interact professionally with others in assessing career needs, to effectively assist clients identify competencies they require for a career and employability in a given context. It also examines how to maintain quality of career development services and professional practice. It applies to individuals seeking to identify and communicate trends in career development.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Research and confirm career trends | - Apply knowledge of changing organizational structures, lifespan of careers and methods of conducting work search, recruitment and selection processes - Analyze changing worker and employer issues, rights and responsibilities in context of changing work practices - Examine importance of quality careers development services - Maintain all research, documentation, sources and references (electronic or physical) to a high degree of currency and relevance - Analyze implications of relevant policy, legislation, professional codes of practice and national standards relating to worker and employer issues - Investigate the changes and trends in theory of career development counseling and practice - Confirm clusters, levels and combinations of transferable employability skills and preferences that may open employment options spanning more than one occupation or career pathway |
| Assess and confirm ongoing career development needs of target group | - Analyze history and records in assessing needs of target group - Assess success of previous career development services and techniques used for individual or target group - Deploy other means to investigate appropriate care and counseling approaches as required - Maintain privacy and security of all data, research and personal records according to relevant policy, legislation, professional codes of practice and national standards - Establish existing work-life balance requirements, issues and needs |
| Maintain quality of career development services and professional practice | - Analyze and review relevance of career theories, models, frameworks and research for target group - Incorporate into career development services and professional practice, major changes and trends influencing workplace and career-related options and choices - Comply with all relevant policy, legislation, professional codes of practice and national standards that influence delivery of career development services |

Knowledge and Understanding:

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain client care and counseling techniques and processes in the context of career development services

- Describe diversity and its potential effects on career choices

- Outline human psychological development and needs in relation to careers development

- Outline relevant policy, legislation, codes of practice and standards relevant to career development

- Explain recruitment and selection processes in the context of career development services

- Describe a range of data gathering and research techniques

- Explain techniques used to analyze trends.

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to identify and communicate trends in career development. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance Requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints.

Demonstrated evidence is required of the ability to:

- Research and analyze current economic, labor market, employment, career and vocational, educational and training trends

- Identify choices and career development needs for individuals and target groups within a given context

- Report and document management of research and career development materials

- Comply with all relevant local, state/territory and national legislation, policies and practices.

### Apply Specialist Interpersonal and Counselling Interview Skills

Overview:

This unit describes the skills and knowledge required to communicate effectively, Use specialized counseling interviewing skills and use advanced and specialized communication skills in the client-counselor relationship. This unit applies to individuals whose job role involves working with clients on personal and psychological issues within established policies, procedures and guidelines.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1.Communicate effectively | - Identify communication barriers and use strategies to overcome these barriers in the client-counselor relationship - Facilitate the client-counselor relationship through selection and use of micro skills - Integrate the principles of effective communication into work practices - Observe and respond to non-verbal communication cues - Consider and respond to the impacts of different communication techniques on the client-counselor relationship in the context of individual clients - Integrate case note taking with minimum distraction |
| Use specialized counseling interviewing skills | - Select and use communication skills according to the sequence of a counseling interview - Identify points at which specialized counseling interviewing skills are appropriate for inclusion - Use specialized counseling communication techniques based on their impacts and potential to enhance client development and growth - Identify and respond appropriately to strong client emotional reactions |
| Evaluate own communication | - Reflect on and evaluate own communication with clients - Recognize the effect of own values and beliefs on communication with clients - Identify and respond to the need for development of own skills and knowledge |

Knowledge and understanding:

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Legal and ethical considerations for communication in counseling practice, and how these are applied in individual practice:

- codes of conduct/practice

- discrimination

- duty of care

- human rights

- practitioner/client boundaries

- privacy, confidentiality and disclosure

- rights and responsibilities of workers, employers and clients

- work role boundaries responsibilities and limitations of the counselor role

- work health and safety

- Principles of person-centered practice

- key objectives of counseling interviewing

- stages of a counseling interview

- potential impacts of using different communication skills and techniques in counseling contexts

- communication techniques and micro-skills including:

- attending behaviors active listening, reflection of content feeling, summarizing

- questioning skills open, closed, simple and compound questions

- client observation skills

- noting and reflecting skills

- providing client feedback

- Specialized counseling communication techniques, and how they are used, including:

- challenging

- reframing

- focusing

- Components of the communication process including:

- encoder

- decoder

- Primary factors that impact on the communication process including:

- context

- participants

- rules

- messages

- channels

- noise

- feedback

- Communication barriers and resolution strategies, including:

- Environmental

- Physical

- Individual perceptions

- Cultural issues

- Language

- Age issues

- Disability

- Observational techniques including:

- Facial expressions

- Non-verbal behavior

- Posture

- Silence

- Ways in which different people absorb information, including:

- visual

- auditory

- kinesthetic

- Obstacles to the counseling process

- Impacts of trauma and stress on the communication process, including on:

- concentration and attention

- memory

- use of verbal and written language

- use of body language

- challenging within the counseling session

- Self-evaluation practices, including:

- how to recognize own biases

Impact of own values on the counseling relationship

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to apply specialist interpersonal and counseling interview skills. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance Requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints.  Demonstrated evidence is required of the ability to:

- interviewed at least 3 different clients using specialized interpersonal communication and counseling interviewing skills, including:

- micro-skills and communication techniques, including:

  - attending behaviors active listening,

  - reflection of content, summarizing

  - questioning skills open, closed, simple and compound questions

  - client observation skills

  - noting and reflecting skills

  - providing client feedback

- specialized counseling interviewing skills, including:

  - challenging

  - reframing

  - focusing

- integrated clear case note taking into the interview process

- Completed a structured process of self-reflection and evaluation of own communication used during the 3 interviews.

# Digital Electronics

### Manipulate the Number System

Overview:

This competency standard identifies the competencies required to Manipulate Number Systems, apply the fundamental concepts of arithmetic logic function like number systems, their conversion and Boolean algebra.

| Competency Units | Performance Criteria |
| --- | --- |
| 1. Convert Decimal Number to Octal, Binary , Hexadecimal Numbers | Convert Decimal numbers into Binary numbers. Convert Decimal numbers into Octal numbers Convert Decimal numbers into Hexadecimal numbers. |
| Convert Octal Number to Decimal, Binary, Hexadecimal Numbers | Convert Octal numbers into Binary numbers. Convert Octal numbers into Decimal numbers Convert Octal numbers into Hexadecimal numbers. |
| Convert Binary Number to Decimal, Octal , Hexadecimal Numbers | Convert Binary numbers into Octal numbers Convert Binary numbers into Decimal numbers. Convert Binary numbers into Hexadecimal numbers. |
| Apply arithmetic operations on number systems | Performed Addition on Binary, Decimal, Octal, and Hexadecimal numbers. Performed Subtraction on Binary, Decimal, Octal, and Hexadecimal numbers. Performed Multiplication on Binary, decimal, Octal , Hexadecimal numbers Compute One’s and two’s compliment of Binary numbers |

Knowledge and Understanding

- Describe essential elements of the binary, octal and hexadecimal numbering system

- Describe Binary, Octal and Hexadecimal Counting

- Define compliment of a number

- Explain Binary Addition, Subtraction & Multiplication

- Learn Number-Base Conversions

- Explain Signed Binary Numbers

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this standard:

Evidence of the following is essential:

- Compute One’s & two’s compliment of Binary numbers

- Convert different type of codes and number systems which are used in digital communication and computer systems

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Stationary |

### Verify Truth tables of Digital Logic Gates

Overview :

This competency standard covers the skills and knowledge required to Verify the truth table of AND gate, Verify the truth table of OR gate, Verify the truth table of NOT gate, Verify the truth table of NAND gate, Verify the truth table of NOR gate, Verify the truth table of X-OR gate and Verify the truth table of X-NOR gate Verify the truth table of AND gate, OR gate, NOT gate, NAND gate, NOR gate, X-OR gate and X-NOR gate

.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Verify the truth table of AND gate | - Identify the symbol of logic gate, IC & logic function. - Place (AND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU2. Verify the truth table of OR gate | - Identify the symbol of logic gate, IC & logic function. - Place (OR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU3. Verify the truth table of NOT gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOT gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU4. Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU5. Verify the truth table of NOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU6. Verify the truth table of X-OR gate | - Identify the symbol of logic gate, IC & logic function. - Place (X-OR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU7. Verify the truth table of X-NOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (X-NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |

Knowledge & Understanding

- Study logic gates Logic gates. AND, OR, NAND, NOR, NOT, X-OR and X-NOR.

- Study the Boolean expression of AND, OR, NAND, NOR, NOT, X-OR and X-NOR, gate and its equivalent electrical circuit

- Define Universal gate and enlist its types.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | AND gate (7408 2-input Quad) |
| 2 | OR gate (7432 2-input Quad) |
| 3 | NOT gate (7404 Hex) |
| 4 | NAND gate (7400 2-input Quad) |
| 5 | NOR gate (7402 2-input Quad) |
| 6 | X-OR gate (7486 2-input Quad) |
| 7 | X-NOR gate (74266 2-input Quad) |
| 8 | Bread board |
| 9 | DC supply (5 V) |
| 10 | LED |
| 11 | Connecting leads |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Verify the truth table of X-NOR gate

### Construct & Verify the Combinational Logic Circuit

Overview:

This competency standard covers the skills and knowledge required to Construct & verify the truth table of Half adder, full adder and Half Subtractor.

| Competency Units | Performance Criteria |
| --- | --- |
| Apply Karnaugh mapping & Boolean algebra to simplify logic expressions | Identify the SOP & POS Apply Boolean algebra &Karnaugh mapping to simplify SOP & POS. Construct logic circuits with simplified SOP & POS. |
| Construct & verify the truth table of Half adder | Place (AND gate IC) & (X-OR gate IC) on bread board. Identify the input, output, Vcc and ground pin. Connect LED to the output pin of IC and apply different logics at input pins. Record & verify the output result against each given input Design, Construct, and test a half-adder circuit using one X-OR gate and two NAND gates. |
| Construct & verify the truth table of Full adder | Place (AND gate IC) & (X-OR gate IC) on bread board. Identify the input, output, Vcc and ground pin. Connect LED to the output pin of IC and apply different logics at input pins. Record & verify the output result against each given input Design, Construct, and test a full-adder circuit using two ICS &7486 and &7400. |
| Construct & verify the truth table of Full Subtractor | Place (AND, NOT&X-OR gate IC) on bread board. Identify the input, output, Vcc and ground pin. Connect LED to the output pin of IC and apply different logics at input pins. Record & verify the output result against each given input. |
| Verify the Decoder | Place (Decoder IC) on bread board. Identify the input, output, Vcc and ground pin. Connect LED to the output pin of IC and apply different logics at input pins. Record & verify the output result against each given input. |
| Operate seven segment displays with seven segment decoder. | Insert (7 segment decoder IC) and 7 segment display on bread board. Identify the input, output, Vcc and ground pin. Connect segment display with seven segment decoder input output pins. Record & verify the output result against each given input. |
| Verify the Encoder | Place (Encoder IC) on bread board. Identify the input, output, Vcc and ground pin. Connect LED to the output pin of IC and apply different logics at input pins. Record & verify the output result against each given input. |
| Verify multiplexer and DE- multiplexer | Implement following function with multiplexer F(ABC)=∑(0,2,3,4,5,6): Implement 4-to-1 mux and one 2-to-1 mux. Implement 1-to-4 dmux using 1-to-2 dmux. |

Knowledge and understanding

- Describe the laws and rules of Boolean algebra.

- Understanding of commutative and distributive expression. That is, A • (B + C) = (A • B) + (A • C) and A + (B • C) = (A + B) • (A + C).

- Study the combinational logic circuit.( Half adder, Full adder, Half subtractor, Full subtractor, Binary Multiplier , Magnitude Comparator )

- Study the Product-of-Sums & Sum-of-product simplification

- Knowledge of Don't-Care Conditions

- Understanding of Karnaugh Map of four variables.

- Understanding of Decoders & Encoders

- Knowledge of Pin configuration of ICs

- Explain pin 7 segment display and common cathode 7 segment display.

- Define limiting resistor.

- Understand implementation functions using multiplexers.

- Explain the Multiplexer & De-Multiplexer

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- Verify multiplexer and DE- multiplexer

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | X-OR gate (7486 2-input Quad) |
| 2 | AND gate (7408 2-input Quad) |
| 3 | OR gate (7432 2-input Quad) |
| 4 | NOT gate (7404 Hex not gate) |
| 5 | Bread board |
| 6 | LED |
| 7 | DC supply (5 V) |
| 8 | Connecting leads |
| 9 | 74LS139 IC |
| 10 | Seven segment display |
| 11 | Resistances (1K ohm) |
| 12 | Connecting leads |
| 13 | Mux KL-33006 block e |
| 14 | Mux KL-33006 block f |

### Construct Shift Registers and Counters Using Flip Flops

Overview:

After the completion of this competency standard the candidate will be able to construct a variety of registers to be used for industrial purposes.

| Competency Units | Performance Criteria |
| --- | --- |
| Construct a 4 bit shift register by using Flip Flops | Make connection of D-Flip Flop as per diagram to construct 4 bit shift register. Apply data at the input of register and give clock pulse Observed and recode the output according to the input. |
| Construct a 4 bit binary counter using Flip Flops | Make connection of JK-Flip Flop as per diagram to construct 4 bit binary counter. Connect LEDs to the outputs pins. Apply the clock pulse and record the output. |
| Repair &Troubleshoot combinational logic circuits | Identify principles and operations of types of logic gates Locate drawing and diagrams. Check the grounding system in electronic equipment. Locate external and internal digital IC faults |

Knowledge and understanding

- Describe the Symbols for Combinational Elements (Symbols for Registers, Counters, RAM)

- Understand the function of registers and counters in digital circuits

- Explain the basic computer memories and its types.

- Explain precautions when handling components.

- Demonstrate proper use of ESD equipment.

- Explain how to distinguish hardware issues from software issues.

- Show how pulsars are used for digital signal tracing and how logic probes are used to verify states in digital equipment.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- Construct a 4 bit binary counter using Flip Flops

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Software Simulator and IDE’s. |
| 2 | DLD trainer |
| 3 | IC’s and Components |
| 4 | Multi-meter |
| 5 | Logic tester |
| 6 | DC Supply |
| 7 | Connecting wires |

### Use 555 Timer IC as Multi-Vibrator

Overview:

This competency standard covers the skills and knowledge required to construct 555 Timer IC as A-stable, Mono-stable & Bi-stable Multi-vibrator and observe their outputs.

| Competency Units | Performance Criteria |
| --- | --- |
| Construct 555 Timer IC as A-stable Multi-vibrator | Place 555 Timer IC on bread board. Make connection as per diagram. Apply voltage to circuit and observe the output with the help of oscilloscope. |
| Construct 555 Timer IC as Mono-stable Multi-vibrator | Place 555 Timer IC on bread board. Make connection as per diagram. Apply voltage to circuit and observe the output with the help of oscilloscope. |
| Construct 555 Timer IC as Bi-stable Multi-vibrator and verify its set and reset conduction | Place 555 Timer IC on bread board. Make connection as per diagram. Change the position of switch and verify set and reset conditions from output pin with the help of oscilloscope. |

Knowledge and understanding

- Describe basic elements of 555 timer IC.

- Explain pin configuration of 555 Timer IC and its specification.

- Explain function of voltage control input.

- Describe the A-stable Multi-vibrator.

- What is mono-stable Multi-vibrator

- What is bi-stable Multi-vibrator

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- Construct 555 Timer IC as Bi-stable Multi-vibrator and verify its set and reset conduction

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | 555 Timer IC |
| 2 | Dual trace Oscilloscope 0-20MHZ |
| 3 | Resistors 10 KΩ |
| 4 | Capacitor 0.01µF |
| 5 | Bread board |
| 6 | DC supply (5 V) |
| 7 | Connecting leads |

# Communication systems

### Identify the parts of Analog& digital telephone set & verify their functions

Overview:

This competency standard covers the skills and knowledge required to identify the parts of analogue & digital telephone sets and their functions

| Competency Units | Performance Criteria |
| --- | --- |
| Dismantle and identify the parts of automatic telephone set | Select the automatic telephone set and establish a method for disassembling activity as per SOP. Use standard tools described in user manual Apply disassembling techniques Organize the tag-identification of the parts/components/wires of telephone set. Identify the parts (magneto bell, dial, antiside tone circuit, transmitter, receiver, cradle switch etc.) Check for the proper operation/ functionality Apply assembling techniques. |
| Dismantle and identify the parts of digital telephone set | Select the digital telephone set and establish a method for disassembling activity as per SOP. Use standard tools described in user manual Apply disassembling techniques Organize the tag-identification of the parts/components/wires of telephone set. Identify the parts (telephone buzzer, dial, Dial IC, speech IC transmitter, receiver, bridge rectifier, cradle switch etc.) Check for the proper operation/ functionality Apply assembling techniques. |
| Demonstrate the operation of Telephone buzzer | Connect the telephone set to telephone line. Dial the telephone No. which is connected to line, form any mobile/telephone. Hear the ringing tone in mobile/telephone and note what happen in telephone set connected to line. |
| Perform the operation of Transmitter & receiver | Connect the telephone set to telephone line. Dial the telephone No. which is connected to line, form any mobile/telephone. On hearing ringing bell, pic up hand set and speak on the lower portion of hand set (transmitter). On replying form other side note the effect from upper portion of hand set (receiver). |
| Identify the parts of rotary dial & verify its function. | Select automatic telephone set. Identify the dial plate of dialer. Identify the dial hole on dial plate. Identify the finger stop. Dial any number from rotary dial Note the sound of pulsed on receiver and count them and compare them to dial number. |
| Identify the parts of digital dial & verify its function. | Select digital telephone set. Identify Key pad dialer on the telephone set. Press different numbers from key pad dialer. Note and compare the difference of sound effect in receiver against each dial number. |

Knowledge & Understanding

- Define telephone set.

- Define telephone transmitter and receiver.

- Define antiside tone circuit.

- Define function of telephone buzzer.

- Describe the function of balancing circuit.

- Explain the function of cradle switch

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Enlist the parts of telephone set.

- Enlist the parts of rotary dial.

- Differentiate between rotary and digital dial.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
|  | Electrical test bench |
|  | Multimeter |
|  | Test probes |
|  | Power source (AC/DC) |
|  | Screw drivers |
|  | Twizer |
|  | Wire cutter |
|  | Nose plier |
|  | Screw drivers |
|  | Twizer |
|  | Wire cutter |
|  | Nose plier |

### Construct the AM / FM Transmitter & Receiver & verify their functionality

Overview:

This competency standard covers the skills and knowledge required to construct the FM/AM transmitters and receivers to be used for industrial purposes.

| Competency Units | Performance Criteria |
| --- | --- |
| Construct AM Transmitter | Place the components on a Vero-board as per circuit diagram. Solder the components together. Solder/connect a wire to function as an antenna. Power on the circuit. Use a spectrum analyzer to detect the transmission frequency. Adjust the transmission frequency by varying the capacitance of the variable capacitor connected to the tank circuit alongside the antenna if the transmission frequency detected by the analyzer is not in range of 540 KHz to 1600 KHz. Play a sound into the Mic of the transmitter. Play AM Radio and Tune it to the transmission frequency of your circuit. Hear the transmitted voice playing on the AM Radio. |
| Construct AM Receiver | Place the components on a Vero-board as per circuit diagram. Solder the components together. Solder/connect a wire to function as an antenna. Power on the circuit. Transmit a voice using your FM Transmitter circuit that you made previously. Tune your AM Receiver circuit to the frequency of the transmitter. Hear the transmitted voice playing on your AM Receiver. |
| Construct FM Transmitter | Place the components on a Vero-board as per circuit diagram. Solder the components together. Solder/connect a wire to function as an antenna. Power on the circuit. Use a spectrum analyzer to detect the transmission frequency. Adjust the transmission frequency by varying the capacitance of the variable capacitor connected to the tank circuit alongside the antenna if the transmission frequency detected by the analyzer is not in range of 88 MHz to 108MHz. Play a sound into the Mic of the transmitter. Open FM Radio on your phone and Tune it to the transmission frequency of your circuit. Hear the transmitted voice playing on the FM Radio of your phone. |
| Construct FM Receiver | Place the components on a Vero-board as per circuit diagram. Solder the components together. Solder/connect a wire to function as an antenna. Power on the circuit. Transmit a voice using your FM Transmitter circuit that you made previously. Tune your FM Receiver circuit to the frequency of the transmitter. Hear the transmitted voice playing on your FM Receiver. |

Knowledge & Understanding

- Identification of basic electronic components like resistors, capacitors, inductors and transistors.

- Circuit construction

- Wire wound inductors

- Transmitting signals using basic FM Transmitter circuit.

- Transmitting signals using basic FM Transmitter circuit.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Draw the block diagram of AM transmitter.

- Draw the block diagram of AM Receiver

- Draw the block diagram of FM transmitter.

- Draw the block diagram of FM Receiver

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
| 1. | Circuit diagram |
| 2. | Components as per circuit diagram |
| 3. | Vero board |
| 4. | Soldering station |
| 5. | Power supply |
| 6. | Spectrum analyzer |
| 7. | FM transmitter |

### Demonstrate modulation, demodulation, multiplexing & de-multiplexing

Overview:

This competency standard covers the skills and knowledge required to perform the modulation, demodulation, multiplexing and de-multiplexing operations on certain signals.

| Competency Units | Performance Criteria |
| --- | --- |
| Perform Amplitude Modulation of Information Signal | Calibrate CRO. Generate Low Frequency Message signal and observe on channel 1 of CRO. Apply the Message and Carrier signals to the AM Modulator Section of the AM Trainer. Connect the output of the modulator to channel 2 of CRO and observe the signal. |
| Perform Demodulation of Modulated AM Signal | Calibrate CRO. Generate AM Modulated signal and observe on channel 1 of CRO. Apply the Modulated and Carrier signals to the AM De-Modulator Section of the AM De-Modulation Trainer. Connect the output of the de-modulator to channel 2 of CRO and observe the signal. Compare the Message signal to the Demodulated signal. |
| Calculate band width and modulation index | Calibrate CRO. Measure the frequency and voltage of Low frequency and carrier frequency signal with oscilloscope. Calculate modulation by formula Calculate bandwidth by formula |
| Demonstrate the function of pre-emphasis circuit. | Give the audio signal to spectrum analyzer and note the relative amplitude of audio frequency signals. Apply audio frequency signal to pre-emphasis circuit. Give the output of pre-emphasis to spectrum analyzer and note the relative amplitude of audio frequency signals. Form above readings compare the normal audio signal and pre-emphasized signal and write down the difference |
| Demonstrate the function of de-emphasis circuit. | Take the audio signal form FM detector of FM radio receiver and give it to spectrum analyzer and note the relative amplitude of audio frequency signals. Take the audio signal form de-emphasis circuit of FM radio receiver and apply it to spectrum analyzer and note the relative amplitude of audio frequency signals. Form above readings compare the detected audio signal and de-emphasized signal and write down the difference |
| Perform Frequency Modulation of Information Signal | Calibrate CRO. Generate Low Frequency Message signal and observe on channel 1 of CRO. Apply the Message and Carrier signals to the FM Modulator Section of the FM Trainer. Connect the output of the modulator to channel 2 of CRO and observe the signal. |
| Perform Demodulation of Modulated FM Signal | Calibrate CRO. Generate FM Modulated signal and observe on channel 1 of CRO. Apply the Modulated and Carrier signals to the FM De-Modulator Section of the FM De-Modulation Trainer. Connect the output of the de-modulator to channel 2 of CRO and observe the signal. Compare the Message signal to the Demodulated signal. |
| Perform Pulse Code Modulation (PCM) of an analog signal | Generate Sine wave signal and observe on channel 1 of CRO. Apply the generated signal and Clock signal to the PCM Modulator using jumpers. Observe the output on channel 2 of CRO and also note the pattern of blinking of output LEDs. |
| Perform Time Division Multiplexing (TDM) and De-multiplexing | Generate multiple low frequency signals. Apply the signals to TDM Multiplexer. Observe the multiplexed output on channel 1 of CRO. Apply to multiplexed output to the De-multiplexer. Observe output for first message signal on channel 1 of CRO and compare the output signal with the relative Message signal by observing it on channel 2 of CRO. Repeat the step 5 for all the de-multiplexed signals one by one. |
| Perform Frequency Division Multiplexing and De-multiplexing | Generate multiple message signals. Generate multiple carrier signals of different frequencies. Apply each of the message and carrier signal pairs to their respective Balanced Modulators. Combine the modulated signals using Adder and observe this FDM Multiplexed output on Channel 1 of CRO. Apply the multiplexed signal to respective Band-Pass filters for each De-multiplexer section (or user). Apply the output of each Filter to their respective Demodulators. Observe the demodulated and hence FDM De-multiplexed signals for each user and compare them to their respective Transmitted signals by viewing both on CRO at the same time. |

Knowledge & Understanding

- Calibration of CRO.

- Working Principle of AM Modulator

- Working Principle of AM Demodulator.

- Define low frequency or information signal

- Define high frequency or carrier signal.

- Define modulation index

- Define bandwidth.

- Define USB “Upper side band”

- Define LSB “Lower side band”

- Define pre-emphasis

- Define de-emphasis

- Define the use of spectrum analyzer

- Working Principle of FM Modulator

- Working Principle of FM Demodulator

- Sine wave and its different parameters.

- Working principle of PCM.

- Working principle of TDM.

- Define the Calibrating and Using CRO.

- Explain Working principle of CRO.

- Explain the AM DSB and SSB

- Define AM demodulation

- Describe the Filtering

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Draw the wave shape of Amplitude modulated signal.

- Draw the circuit diagram AM demodulator.

- Calculate %modulation, modulation index form amplitude modulated signal.

- Draw the wave shape of Frequency modulated signal.

- Draw pre-emphasis circuit diagram.

- Draw De-emphasis circuit diagram.

- Enlist types multiplexing.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
| 1. | Oscilloscope with probes |
| 2. | AM trainer jumpers |
| 3. | AM demodulation trainer |
| 4. | Signal generator |
| 5. | FM radio receiver |
| 6. | Connecting leads |
| 7. | Spectrum analyzer |
| 8. | FM demodulation trainer |
| 9. | Jumpers |
| 10. | PCM trainer |
| 11. | TDM trainer |
| 12. | Multiplexing trainer |

### Install Chord less telephone, PABX, Fax machine & satellite dish system

Overview:

This competency standard covers the skills and knowledge required to install chord less telephones, fax machines, PABX and satellite dish systems.

| Competency Units | Performance Criteria |
| --- | --- |
| Install cord less telephone system. | Chose proper location to install the base station Connect the phone line to the base the base station of cord less telephone. Connect the supply to the base station. Pic-up the Mobile unit from cordless telephone and make a call to ensure the working functionality. |
| Install and Configure of Private Automatic Branch Exchange(PABX) | Lay 2-Pair Telephone drop wire around the premises. Plug the telephone cord into your PBX console in an input that says, "Telephone Line." Connect the other end into a wall jack. This allows your PBX system to send and receive phone calls. Insert the PBX plug into the console. Plug the other end into the wall outlet. Wait for your PBX system to light up. This lets you know that you have properly installed the system. Pick up the receiver to place a call. Dial the extension or phone number you would like to reach. Wait for the caller to answer on the other end. Wait for the system to ring and light up, which means you have an incoming call. Pick up the receiver and answer the call. You can also press "Intercom" or "Hands Free" to answer the call |
| Install Home satellite dish system. | Perform dish assembly as per SOP Perform dish installation on roof top as per SOP. Route coaxial cable and connect dish receiver to dish antenna. Connect dish receiver output to TV receiver input. Turn on the supply of both receivers, enter the frequency of required channel. Adjust the antenna with the help of satellite locator chart till required channel is observed on TV screen. |
| Install and configure FAX Machine | Remove all of the components of the fax machine from the box. Shake gently the ink toner cartridge to evenly distribute the toner. Insert the toner cartridge into the Fax machine. Install RJ-11 connector on both ends of 2-Pair telephone drop wire. Connect one end of the wire to Fax machine and the other to the Rosset Box connected to the telephone line. Load paper into the machine. Power on the Fax machine Run a test sheet through the machine. |

Knowledge & Understanding

- Function and operating procedure of cordless telephone

- Installing RJ-11 Connectors on cable.

- Function and operating procedure of PABX

- Explain the Function & operating procedure of satellite dish system

- Installing Rosset box on Telephone line

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Enlist the steps to install the cord less telephone.

- Draw the block diagram of PABX

- Install and configure Fax machine.

- Make connections and configure satellite dish system.

### Apply different concepts of propagation of waves

Overview:

After studying the subject the students will be able to: Understand a vector-calculus based description of static electric fields in cases of fixed charges, conductors, and dielectrics. Student will be able to describe the moving charges (for the case of steady electric currents) and resulting static magnetic fields are also presented and student will be able capable of understanding the Maxwell equations and the classical description of electromagnetic fields. Problem solving makes frequent use of symmetry and invariance.

| Competency Units | Performance Criteria |
| --- | --- |
| Identify Transmission Lines | Categorize Transmission frequencies Differentiate various transition lines according to their band width and material |
| Classify Waveguides | Identify different types of waveguides according to their shape . Identify different modes of waveguides. |
| Categorized Antennas | Identify various types of Dipole Antenna. Categorize the Characteristics of Antennas |

Knowledge & Understanding

???

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify Transmission Lines

- Classify Waveguides

- Classify Waveguides

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Multi-meter |
| 2 | Power supply |
| 3 | Trainer |
| 4 | Resistor |
| 5 | Capacitor |
| 6 | Inductor |
| 7 | ICs |
| 8 | Source of data sheets |

# Power Electronics

### Design Electronic Control circuits using Power Diodes

Overview:

After completion of this competency standard the candidate will be able to design and install various electronic circuits using power diodes

| Competency Units | Performance Criteria |
| --- | --- |
| Control the load by using Power Diode | Identify the Power Diodes Draw the V-I Characteristic of Power Diodes Operate Power Diodes in Series circuit. Operate Power Diodes in Parallel Circuits. Use Resistive, loads with Half wave rectifier and measure the outputs Use Capacitive loads with Half wave rectifier and measure the outputs Use Inductive loads with Half wave rectifier and measure the outputs Generate the lab report |
| Design Single Phase diode bridge rectifier | Draw the Circuit of Single Phase diode bridge rectifier. Select the components for Single Phase diode bridge rectifier Place the components for Single Phase diode bridge rectifier. Perform the operation of diode bridge rectifier with Resistive load. Perform the operation of diode bridge rectifier with capacitive load. Perform the operation of diode bridge rectifier with inductive load. Verify the output voltage of single phase diode bridge rectifier. Generate the lab report |
| Design three phase diode bridge rectifier | Draw the Circuit of three Phase diode bridge rectifier. Select the components for three Phase diode bridge rectifier Place the components for three Phase diode bridge rectifier. Perform the operation of diode bridge rectifier with Resistive load. Perform the operation of diode bridge rectifier with capacitive load. Perform the operation of diode bridge rectifier with inductive load. Verify the output voltage of three phase diode bridge rectifier. Generate the lab report |

Knowledge & Understanding

- Explain simple diode, power diode & their uses

- Explain structure properties and working principles of power diode

- Explain Series and Parallel Circuits

- Interpret the Operation of Power diodes in series and Parallel circuits

- Describe Half and Full wave Rectifiers

- Explain Resistive, Capacitive and Inductive Load

- Explain behavior of half wave rectifier with Resistive, capacitive and inductive load configuration.

- Learn basic knowledge of diode and diode bridge, there working principle for single phase and three phase

- Explain working principles of rectifier circuit

- Explain working principles of three phase diode bridge circuit

- Describe three phase diode bridge, resistive, capacitive and inductive loads

- Explain the calculation of output voltage using mathematical expressions

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Implement parallel operation of power diodes.

- Implement Series operation of power diodes.

- Draw the half wave rectifier using power diode.

- Implement power in diodes in bridge circuit for rectification.

- Draw the circuit diagram to show the free-wheeling action of power diode.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Power diodes |
| 2 | Multi-meters |
| 3 | Transistors |
| 4 | Oscilloscope |
| 5 | Resistor |
| 6 | Capacitor |
| 7 | Inductor |
| 8 | Power supply |
| 9 | Soldering station |
| 10 | Function generator |
| 11 | Nose pliers and wire cutter |
| 12 | Screw driver set |
| 13 | Vero board |
| 14 | Breadboard |
| 15 | Push buttons |
| 16 | SCR |
| 17 | MOSFETS |
| 18 | IGBTs |
| 19 | Clamp meter |

### Design Single Phase and Three Phase Converters

Overview:

After completion of this competency standard the candidate will be able to implement single phase & three phase converters in multiple situations

| Competency Units | Performance Criteria |
| --- | --- |
| Design the Power Control circuit using SCR in Series | Draw the schematic for Power SCR in Series Circuits. Select the components for Power SCR. Place the components for Power SCR Draw the V-I characteristic of power SCR in Series Control different load by Using SCR in Series. Generate the lab report. |
| Design the Power Control circuit using SCR in Parallel | Draw the schematic for Power SCR in Parallel Circuits. Select the components for Power SCR. Place the components for Power SCR in Parallel circuit Draw the V-I characteristic of power SCR in Parallel Circuits. Control different load (AC Motor) by Using SCR in Parallel Circuits. Generate the lab report. |
| Design a single phase Converter | Draw the schematic for a single phase Semi and full Converter Select the components for single phase Semi and full Converter Place the components for single phase Semi and full Converter Use RL load with Semi & Full converter Inspect the Output wave form of Semi converter with load Inspect the Output wave form of Semi Full converter with load Generate the output report |
| Design a three phase Full Converter | Draw the schematic for a three phase Semi and full Converter Select the components for three phase Semi and full Converter Place the components for three phase Semi and full Converter Use RL load with Semi & Full converter Inspect the Output wave form of Semi converter with load Inspect the Output wave form of Full converter with load Generate the output report |
| Design the DC-DC Buck converter | P1. Draw the schematic for a Buck converter Select the components for Buck converter Place the components for Buck converter Measure the input and output voltage Generate the output lab report |
| Design the DC-DC Boost converter | Draw the schematic for a Boost converter Select the components for Boost converter Place the components for Boost converter Measure the input and output voltage Generate the output report |
| Design the DC-DC Buck-Boost converter | Draw the schematic for a Buck-Boost converter Select the components for Buck-Boost converter Place the components for Buck-Boost converter Measure the input and output voltage Generate the output report |
| Implement DC-DC Flyback Converter | Draw the schematic for a DC-DC (flyback) converter Select the components for DC-DC (flyback) converter Place the components for DC-DC (flyback) converter Operate the flyback converter in CCM and DCM mode Measure the input and output voltage Design Switch Mode Power Supply using converter Generate the output report |
| Implement DC-DC Forward Converter | Draw the schematic for a DC-DC (Forward) converter Select the components for DC-DC (Forward) converter Place the components for DC-DC (Forward) converter Operate the Forward converter in CCM and DCM mode Measure the input and output voltage Design Switch Mode Power Supply using converter Generate the output report |

Knowledge &Understanding

- Basic knowledge of SCR and working principle of SCR

- Study data sheet of SCR

- Recognize the power ratings of SCR

- Comprehend the V-I characteristic curve

- Calculate the gate recovery time using formulae

- Explain various types of single &Three phase converters & their operations in different configurations.

- Explain triggering angle and displacement factor

- Explain Symmetrical and asymmetrical single phase semi converters

- Explain working principle of buck, boost and buck-boost converter in CCM and DCM mode and its operation

- Describe uses of buck, boost and buck-boost converter

- Describe the working principle of flyback, DC-DC converter operation

- Explain uses of flyback and DC-DC converter & forward DC-DC  converter

- Explain working principle of full bridge DC-DC converter

- Describe applications of flyback& Full bridge converter

- Describe the working principle of forward, DC-DC converter operation

- Explain uses of forward  DC-DC converter

- Describe applications of forward converter

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Implement SCR in phase control rectifier circuit.

- Draw the VI curve of SCR.

- Implement 3 phase full converter.

- Differentiate between buck and boost converter.

- Differentiate between buck-boost and flyback converter.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Power diodes |
| 2 | Multi-meters |
| 3 | Transistors |
| 4 | Oscilloscope |
| 5 | Resistor |
| 6 | Capacitor |
| 7 | Inductor |
| 8 | Power supply |
| 9 | Soldering station |
| 10 | Function generators |
| 11 | Nose pliers and wire cutter |
| 12 | Screw driver set |
| 13 | Vero board |
| 14 | Breadboard |
| 15 | Push buttons |
| 16 | SCR |
| 17 | MOSFETS |
| 18 | IGBTs |
| 19 | Clamp meter |

### Design the Inverters

Overview:

After the completion of this competency standard the student will be able to design a variety of inverters.

| Competency Units | Performance Criteria |
| --- | --- |
| Design the Single phase inverter using push pull configuration (Iron core transformer) | Draw the schematic for a inverter using push pull configuration (Iron core transformer) Select the components for inverter using push pull configuration (Iron core transformer) Place the components for inverter using push pull configuration (Iron core transformer) Measure the input and output voltage Generate the output report |
| Design the Single phase inverter using Ferrite Core Transformer (H-Bridge) | Draw the schematic for a inverter using Ferrite Core Transformer (H-Bridge) Select the components for inverter using Ferrite Core Transformer (H-Bridge) Place the components for inverter using Ferrite Core Transformer (H-Bridge) Measure the input and output voltage Generate the output report |
| Design the Three Phase inverter | Identify the voltage source and current source inverter Draw the schematic for a inverter using Iron Core Transformer (H-Bridge) Select the components for inverter using Iron Core Transformer (H-Bridge) Place the components for inverter using Iron Core Transformer (H-Bridge) Measure the input and output voltage Generate the output report |

Knowledge & Understanding

- Describe square wave, sine wave and step square wave inverters

- Explain iron core and ferrite core transformer, H-Bridge, MOSFETS and IGBTs

- Describe square wave, sine wave and step square wave inverters

- Explain iron core and ferrite core transformer, H-Bridge, MOSFETS and IGBTs

- Explain VSI (voltage source inverter)and CSI (current source inverter)

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Implement single phase inverter using push pull configuration.

- Draw the circuit diagram of three phase inverter.

Tools and Equipment required

The tools and equipment required for this competency standard are givenbelow:

| Sr.No. | Name |
| --- | --- |
| 1 | Power diodes |
| 2 | Multi-meters |
| 3 | Transistors |
| 4 | Oscilloscope |
| 5 | Resistor |
| 6 | Capacitor |
| 7 | Inductor |
| 8 | Power supply |
| 9 | Soldering station |
| 10 | Function generator |
| 11 | Nose plier and wire cutter |
| 12 | Screw driver set |
| 13 | Vero board |
| 14 | Breadboard |
| 15 | Push buttons |
| 16 | SCR |
| 17 | MOSFETS |
| 18 | IGBTs |
| 19 | Clamp meter |

# Industrial Electronics

### Carry Out System Automation Using Sensors and Transducers

Overview:

After the completion of this competency standard the candidate will be able to design an application for industrial, domestic and commercial automation using sensors and transducers.

| Competency Unit | Performance Criteria |
| --- | --- |
| Construct electronics switch to operate load (Relay) using Transistor | - Draw the circuit of transistor as switch - Select the components for transistor as switch. - Place the components for transistor as switch. - Perform the operation of transistor as switch - Measure the output voltage - Generate the lab report |
| Install photo transistor (Position Sensors) to detect the position | - Draw the circuit of photo transistor for position detecting - Select the components of photo transistor for position detecting - Place the components of photo transistor for position detecting - Calculate the Common mode rejection ratio of Photo transistor for position detecting - Measure the output voltage. - Generate the lab report |
| Install photo diode (smoke detector) to detect the smoke | - Draw the circuit of photo diode for smoke detection - Select the components of photo diode circuit for smoke detection - Place the components of photo diode circuit for smoke detection. - Measure the output voltage. - Generate the lab report |
| Construct isolation circuit (High and Low voltage) using opto-couplers | - Draw the circuit of isolation circuit using opto- couplers - Select the components of isolation circuit using opto- couplers - Place the components of isolation circuit using opto- couplers - Measure the output voltage - Generate the lab report |
| Design a circuit of street light using LDR | - Draw the circuit of street light using LDR - Select the components of street light using LDR - Place the components of street light using LDR - Measure the output voltage - Calculate the common mode rejection ratio - Generate the lab report |
| Design the potentiometer transducer | - Identify the types of transducer - Draw the circuit of potentiometer transducer - Select the components of potentiometer transducer - Place the components of potentiometer transducer - Measure the output voltage - Generate the lab report |
| Measure the displacement using LVDT( Linear Variable Differential Transformer) | - Identify the working principle of LVDT - Draw the circuit of LVDT ( Linear Variable Differential Transformer) - Select the components of LVDT - Place the components of LVDT - Calculate the null position, sensitivity in LVDT - Measure the LVDT displacement (mm to cm) - Measure the output voltage on the voltmeter - Generate the lab report |
| Measure the temperature in form of voltage using thermocouple | - Analyze the working principle of thermocouple - Draw the circuit for temperature measurement (in form of voltage) using thermocouple, - Select the components for temperature measurement using thermocouple. - Place the components for temperature measurement using thermocouple - Measure the voltage variation on volt meter by applying temperature variation. - Generate the lab report |
| Perform working operation of piezo-electric transducer in electronic circuit. | - Analyze the working principle of piezo-electric transducer. - Draw the circuit of piezo-electric transducer. - Select the components for piezo-electric transducer. - Place the components for piezo-electric transducer. - Measure the voltage variation on volt meter by applying Pressure. - Generate the lab report |
| Perform working operation of optical transducer in electronic circuit. | - Analyze the working principle of optical transducer. - Draw the circuit of optical transducer. - Select the components optical transducer. - Place the components optical transducer. - Measure the voltage variation on volt meter by applying light on optical sensor. - Generate the lab report |
| Assemble a speed controller for Servo System | - Identify different types of servo motors - Assemble different section for control servo mechanism - Tune the control for specific speeds. - Record & Observe the results in report |
| Construct a closed loop servo control system of a voltage stabilizer | - Analyze the working principle of digital servo controlled voltage stabilizer - Draw the circuit of digital servo controlled voltage stabilizer. - Select the components of digital servo controlled voltage stabilizer. - Place the components of digital servo controlled voltage stabilizer - Measure the output voltage on the voltmeter - Generate the lab report |

Knowledge & Understanding

- Explain the Darlington pair configuration

- Describe power rating of transistor

- Explain  inductive load

- Describe freewheeling diode(Protective diode)

- Explain the use of transistor as switch

- Explain forward blocking mode and Reverse blocking mode

- Explain the Rms voltage , current and peak voltage

- Compute the power dissipation of the transistor

- Describe LED& LASCR wavelength

- Explain seven segment display

- Explain types of photo devices

- Explain characteristic of photo transistor

- Describe intrusion detection system

- Explain characteristic of photodarlington opto-coupler

- Differentiate between sensor and transducer

- Explain the applications of transducer

- Explain the LVDT and its applications.

- Understand the principle of Weight , force pressure measurement using LVDT

- Describe null position and sensitivity

- Describe RTD and thermocouple

- Explain the types and range of resistance thermometer and its working principle.

- Describe piezo electric transducer and its purpose.

- Explain parameters of piezo electric transducer, Seven segment display and counters

- Explain optical transducer and its applications

- Explain Armature and field controller

- Describe Characteristics of Servo System.

- Explain Automatic voltage stabilizer (servo type)

- Understand the working principle of servo control system

- Explain the difference between closed loop and open loop control system.

- Describe main components of feedback control system.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Perform on/off operation of relay with the help of transistor.

- Implement smoke detector circuit with photo diode.

- Implement isolation circuit using optocoupler.

- Implement transistor as a switch.

- Use LVDT for displacement measurement.

- Draw block diagram of servo-based voltage stabilizer.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Transistors |
| 2 | Power Supply |
| 3 | Diodes |
| 4 | Resistors |
| 5 | Capacitor |
| 6 | Inductor |
| 7 | Multi-meter |
| 8 | Bread Board |
| 9 | Vero Board |
| 10 | Micro-Controllers |
| 11 | UJT’s |
| 12 | SCR |
| 13 | MOSFETS |
| 14 | Temperature Sensors |
| 15 | Thyristors |
| 16 | LED |
| 17 | LASCR |
| 18 | Battery |
| 19 | 1N5400 (3Amp) |
| 20 | Transformer |
| 21 | Seven segment display |
| 22 | Photo transistor |
| 23 | Photo diode |
| 24 | Opto-coupler |

### Carry out Industrial Automation Using PLC

Overview:

After this competency the candidate will be able to design, develop, install and maintain the automation and control system.

| Competency Unit | Performance Criteria |
| --- | --- |
| Analyze user requirements and specifications | - Draw the general value chain of the end user industry - Highlight the set of activities that a firm operating in a specific industry performs in the value chain drawing - Enlist the equipment/gauges/sensors/actuators/transducers used in different stages of the process - Identify critical stages in the process - Identify the safety aspect required in the critical stages of the process - Analyze the possible automation in the existing processes and global trends in automation - Analyze the client requirement at broad level from the proposal - Generate a report of various industrial processes involved in industry - Collect the required specification of the equipment (if already prepared by the user) and clarify the technical specification. |
| Prepare work plan | - Suggest globally practiced and accepted automation systems if the user is not aware of the technical specifications - List down the sub systems that are involved in the process - List down sensors and actuators requirement. - Collect information on process logic - Collect information for operator station screens - Decide on whether the system can be developed as per the user requirement - Support the project manager in calculating the time required for each stage to ensure completion of project - Assist in preparing the work plan with deliverables and timelines - Explain the expected output to the user - Calculate the number of days needed for commissioning of the panel at site - Summarize the user requirement. |
| Design and program PLC | - Develop PLC application as per user requirement by following the standard operating procedure (SOP) of the organization - Apply approved engineering concepts, processes and principles in developing the PLC application - Install organization approved software (system and application software) to develop the system - Identify the requirement of indications, switchgears and accessories - Develop the control circuit drawing - Prepare wiring plans - Integrate the main process system with the sub-systems as per the user requirement (e.g., using communication protocol) - Ensure that safety aspect of the process is captured in the design plan - Program PLC as per FDF - Program SCADA Application - PLC-SCADA Communication - Create backup copies of all designs developed for control panel and store in a secure location - Prepare a product manual and store them for future references. |
| Test the PLC | - Locate field devices and their interface to PLC - Test the system in off line mode using simulator - Test the gauges independently - for integration of main system with the sub-systems (if applicable) - Verify that the system conforms with all the user specifications during testing - Rework if there are any issues found and fix them - Send the test report for review to the customer - Perform Factory Acceptance Test (FAT) - Perform site acceptance test plan |
| Ensure quality and productivity standards | - Ensure timely delivery of the control panel design as per agreed timeline - Ensure that total cost and man hours spent is as per the budget planned - Ensure compliance with relevant regulations, standards and codes of practices - Ensure compliance of the application with manufacturing requirements and process capabilities analysis of the organization - Ensure that the design conforms with normal safety standards - Develop reliable panels so that the system does not fail during the usage |
| Install and Commission the PLC | - Check availability of panel and tools required for installation - Check the internal panel wiring and ensure that it is in accordance with the design drawing - Carry out insulation check of internal panel wiring and devices within the panel - Check if batteries and chargers have been assembled in accordance with the manufacturers recommended procedures - Identify the conductors size and capacity for installation - Ensure that the panel is positioned as prescribed, following safety norms - Make connections to socket outlets, switches and protective conductors - Perform settings as per customer requirements on the equipment in each of the panels - Test all control system interlocks - Check each digital control point by comparing the command at the control panel and status of the device that it controls - Ensure that fuses, switches and other protective devices are labelled correctly - Follow the grounding and earthling procedures while commissioning - Put danger and warning notices,(f necessary) - Test continuity, insulation resistance, functions of all devices, etc., after completion of installation |
| Carry out maintenance of PLC | - Select and use required tools and equipment as per the job requirement - Identify the faults as per the LED indications on module - Identify different signal modules (DI, DO, AI, AO) and take necessary action as per job requirement - Identify & Check the terminals inside IO panels and take necessary action as per the job requirement - Check & test the condition of input power supply to IO panel, input/output power of SMPS and take necessary action as per job requirement - Check the conditions of breakers and fuses and take necessary action as per the job requirement |

Knowledge & Understanding

- Define the term ‘value chain’ of industry.

- Define human machine interface (HMI)

- Define the Standard operating procedure (SOP) of the organization for control panel development process.

- Define Piping and instrumentation diagram/drawing (P&ID)

- How to prepare wiring diagram

- Define IEC standards

- Define formula for Electrical load calculations

- Describe the operations of relays, contactors, circuit breaker, solenoid, actuators, controllers, sensors, transducers etc. their types and applications

- Understand Working Principle of PLC and construction of PLC

- Understand different type of  PLC

- Describe the advantages of the PLC

- Describe the applications of PLC.

- Describe the types of input and output signals.

- List down the PLC, DCS programming software.

- List down the SCADA, HMI development software.

- Describe the various tools used during the installation process

- How to troubleshoot Frequently occurring errors, causes and preventive measures

- Describe the procedure followed for repairing/replacement of various components in a PLC

- What are safety precautions while working in an electronic assembly unit

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Implement electrical circuit using PLC software.

- Program PLC to run a specific control operation.

- Install and commission the PLC for a specific task

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Reference Material |
| 2 | Clipboard |
| 3 | Calculation Sheet |
| 4 | Calculator |
| 5 | Psychometric Charts and Tables |
| 6 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc |
| 7 | Site Visit |
| 9 | Clipboard |
| 10 | PLC Module |
| 11 | PLC Software |
| 12 | Connecting Cables |
| 13 | Simulating software(Emulator) |
| 14 | Power Supply |
| 15 | Sensors/Transducers |
| 16 | Anemometer |
| 17 | Tachometer |
| 18 | Data loggers |
| 19 | Multi-meter |
| 20 | Electronic Hygrometer |
| 21 | Electronic Pressure Gauge |
| 22 | Connecting Cables |
| 23 | Tool box |
| 24 | Protective gear |
| 25 | International Safety Standards |
| 26 | Budget Sheet |

### Carry out System Automation using Linear Control system

Overview:

After completion of this competency standard the candidate will be able to implement various types of linear controllers in industry

| Competency Unit | Performance Criteria |
| --- | --- |
| Analyze. working principle of PI Controller in closed loop control system | - Identify PI control temperature control device. - Analyze the working operation of PI controller - Draw the block diagram of PI controller |
| Analyze working principle of PD Controller in closed loop control system | - Identify PD control in temperature control device. - Analyze the working operation of PD controller - Draw the block diagram of PD controller |
| Analyze working principle of PID Controller in closed loop control system | - Identify an on/off control in a system - Identify PID control in temperature control device. - Analyze the working operation of PD controller - Draw the block diagram of PD controller |

Knowledge & Understanding

- Describe linear control system

- Describe Controller Fundamentals and applications of PI,   PD and PID Controllers.

- Control loop components

- Open and closed loop control

- Control loop block diagram

- Direct acting and reverse acting controllers

- Study data sheet of controller(P,PI,PD,PID)

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Enlist different types of controllers.

- Differentiate between PI, PD and PID controller.

- Draw the schematic diagram of PI controller using operational amplifier.

- Draw the schematic diagram of PD controller using operational amplifier.

- Draw the schematic diagram of PID controller using operational amplifier.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Different Automated systems for demonstration |

# Microcontroller Programming and Applications

### Carryout Microcontroller Applications.

Overview:

After completion of this competency standard the student will be able to perform a variety of tasks on a microcontroller and will be able to integrate the programming knowledge into the microcontroller.

| Competency Units | Performance Criteria |
| --- | --- |
| Perform BCD addition and subtraction using Microcontroller | Draw the schematic diagram of microcontroller for BCD addition and subtraction operation Select the components of microcontroller for BCD addition and subtraction operation Implement the circuit of microcontroller for BCD addition and subtraction operation Program the microcontroller for BCD addition Functions Apply the Inputs and verify the output, Program the microcontroller for subtraction Functions Apply the Inputs and verify the output. Generate Lab report |
| Perform multiplication and division using Microcontroller | Draw the schematic diagram of microcontroller for multiplication and division operation Select the components of microcontroller for multiplication and division operation Implement the circuit of microcontroller for multiplication and division operation Program the microcontroller for multiplication Functions Apply the Inputs and verify the output, Program the microcontroller division Apply the Inputs and verify the output,. Generate Lab report |
| Perform the operation to convert Centigrade in to Fahrenheit using Microcontroller | Draw the schematic diagram of microcontroller for the conversion of Centigrade in to Fahrenheit Select the components of microcontroller for the conversion of Centigrade in to Fahrenheit Implement the circuit of microcontroller for the conversion Program the microcontroller for the conversion of Centigrade in to Fahrenheit Apply the Inputs and verify the output Generate Lab report |
| Interface LCD with microcontroller | Identify the pins of LCD and its function Draw the schematic diagram of LCD interfacing with microcontroller Select the components of LCD interfacing with microcontroller Implement the circuit of LCD interfacing with microcontroller Write a code(Program) of LCD interfacing with microcontroller Generate the hex file and load in microcontroller Monitor the output Generate Lab report |

Knowledge and understanding

- Basic knowledge of addition subtraction and BCD conversion BCD

- Explain the Block diagram, Registers; General purpose registers, Stack Pointer, Program counter, Special Function Registers (SFR), Program Status word, Data Pointer (DPTR), Timer Registers, Ports, Control Registers

- Explain  the RAM organization of Microcontroller

- Explain clock circuit and ports of Microcontroller

- Explain pin description, connections, Parallel I/O ports, RAM organization

- Basic knowledge of the conversion of Centigrade in to Fahrenheit

- Basic knowledge of LCD

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Control a AC motor load using microcontroller

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
|  | Micro controllers with different specifications |
|  | Crystal |
|  | Power supply |
|  | LED |
|  | Microcontroller programmer |
|  | Microcontroller trainer |
|  | Microcontrollers with different specifications |
|  | Application tool for Microcontroller |
|  | Capacitor |
|  | Data sheet |
|  | DIP switches |
|  | Terminal block |
|  | PC |

### Apply Microcontroller to build Control System.

Overview:

After completion of this competency standard the student will be able to construct a control system using microcontrollers and will be able to demonstrate programming skills for the controls system.

| Competency Units | Performance Criteria |
| --- | --- |
| Control Industrial systems using microcontroller | Select the AC load (light, motor) Draw a schematic diagram to control AC loads using microcontroller Implement the control circuit of AC load using microcontroller Create a code (PWM Technique) to control an AC Load through Relay Switching using microcontroller Monitor the output pin (duty cycle ) using scope. Generate a lab report |
| Control AC load using Arduino | Operate motor in forward and reverse direction using Arduino Design a traffic light controller system using Arduino Design a temperature controller system using Arduino |
| Control AC load using ATMEL | Operate motor in forward and reverse direction using ATMEL Design a traffic light controller system using ATMEL Design a temperature controller system using ATMEL |

Knowledge and understanding

- Explain the Block diagram, Registers; General purpose registers, Stack Pointer, Program counter, Special Function Registers (SFR), Program Status word, Data Pointer (DPTR), Timer Registers, Ports, Control Registers

- pin description, connections, Parallel I/O ports, RAM organization

- Explain  the RAM organization of Microcontroller

- Explain clock circuit and ports of Microcontroller

- Basic knowledge Of PWM techniques

- Basic Knowledge of Arduino

- Understand the devices , interfacing with Arduino

- Knowledge of programming of controllers

- Basic Knowledge of Arduino

- Understand the devices , interfacing with Arduino

- Knowledge of programming of controllers

- Basic Knowledge of ATMEL

- Understand the devices, interfacing with ATMEL

- Learn basic Knowledge of programming of controllers

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Identify various gates and there data sheet

- Design a circuit by using Multiplexer and De-multiplexer

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
| 1. | Micro controllers with different specifications |
| 2. | Crystal |
| 3. | Power supply |
| 4. | LED |
| 5. | Microcontroller programmer |
| 6. | Microcontroller trainer |
| 7. | Microcontrollers with different specifications |
| 8. | Application tool for Microcontroller |
| 9. | Capacitor |
| 10. | Data sheet |
| 11. | DIP switches |
| 12. | Terminal block |
| 13. | AC load (fan, bulb etc) |
| 14. | Arduino with different specifications |
| 15. | DC load |
| 16. | ATMEL with different specifications |
| 17 | PC |

# Equipment Maintenance and Servicing

### Repair/service different Home Appliance

Overview:

This Competency Standard identifies the competencies required to carry out repair and maintenance of home appliances in accordance with the manufacturer’s instructions. The underpinning knowledge regarding repair and maintenance of transformer will be sufficient to provide the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| Repair the electric iron | - Apply Occupational, Health and Safety regulations, codes and practices in the workplace - Dismantle the electric Iron for internal tests/servicing/repairs according to manufacturer’s instructions - Check the continuity of wire/switch/protective device by using specified test - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and temperatures. - Check the thermostats by using specified test instruments to detect defects.. - Diagnose the faulty area - Clean the parts of the iron with specified cleaning material - Repair/replace the faulty parts of iron as per diagnosed fault. - Re assembled the iron and check iron in test bench as per standard. |
| Repair & Service the Pedestal & Ceiling Fan. | - Apply Occupational, Health and Safety regulations, codes and practices in the workplace - Dismantle the fan for internal tests/servicing/repairs according to manufacturer’s instructions - Check the continuity of wire/switch/protective device by using specified test - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and temperatures. - Check the speed and capacitor - Check the winding by using specified test instruments to detect defects. - Diagnose the faulty parts - Clean the parts of the fan with specified cleaning material - Repair/replace the faulty parts of fan as per diagnosed fault. - Rewind the winding - Assembled and test fan as per standard. |
| Repair the juicer machine, kitchen instruments | - Apply Occupational, Health and Safety regulations, codes and practices in the workplace - Dismantle the juicer/grinder for internal tests/servicing/repairs according to manufacturer’s instructions - Check the continuity of wire/switch/protective device by using specified test - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and temperatures. - Check the winding by using specified test instruments to detect defects. - Diagnose the faulty components Clean the parts of the fan with specified cleaning material - Repair/replace the faulty parts as per diagnosed fault. - Rewind the winding if wind is burnt - Re assembled the juicer/grinder and check juicer in test bench as per standard. |
| Repair & service the Refrigerator /Air conditioner | - Apply Occupational, Health and Safety regulations, codes and practices in the workplace - Turn off the Refrigerator /Air conditioner - Dismantle the Refrigerator /Air conditioner for internal tests/servicing/repairs according to manufacturer’s instructions. - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and Check the cooling system of air conditioner. - Check the thermostat glitches - check the temperature/heat or condenser coil and identify the problems - Check the Helium gas pressure with pressure gauge - check the leaking or dirty ducts - Check the drainage of water - replace dirty filters - check ducts and thermostat with specified test equipments - Diagnose the faulty components Clean the parts of the fan with specified cleaning material - Repair/replace the faulty parts as per diagnosed fault. - Refill the gas up to required standard (10 to 20 psi) - Before refilling gas create vacuum in compressor or zero gas pressure - Assembled Refrigerator /Air conditioner and performed test bench as per standard. |
| Repair and service the CCTV setup | - Apply Occupational, Health and Safety regulations, codes and practices in the workplace Inspect physical appearance of system. - Test power and cables of the system. - Test system software and update - Identify the Service and maintenance manual. - Select proper tools. - Perform de-assembling procedure - Identify major sections of the System. - Perform fault trace in different sections of system. - Replace the faulty board/part. - Apply service procedure to system. - Assemble the system. - Perform test run. |
| Repair and service the Microwave Oven | - Apply Occupational, Health and Safety regulations, codes and practices in the workplace - Dismantle the Microwave Oven for internal tests/servicing/repairs according to manufacturer’s instructions - Check the continuity of wire/switch/protective device by using specified test - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and temperatures. - Solve problems in D.C circuits. - Diagnose the faulty components Clean the parts of the Microwave Oven with specified cleaning material - Use drawings, diagrams, schedules, standards, codes and specifications. - Repair/replace the faulty parts as per diagnosed fault. - Re assembled the Microwave Oven and check Microwave Oven in test bench as per standard. |
| Repair and service the Washing/Dryer Machine | - Apply Occupational, Health and Safety regulations, codes and practices in the workplace - Dismantle the Washing Machine for internal tests/servicing/repairs according to manufacturer’s instructions - Check the continuity of wire/switch/protective device by using specified test - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and temperatures. - Diagnose the faulty components Clean the parts of the Washing Machine with specified cleaning material. - Use drawings, diagrams, schedules, standards, codes and specifications. - Repair/replace the faulty parts as per diagnosed fault. - Re assembled the Washing Machine and check Washing Machine in test bench as per standard. |
| Repair and service the Cellular Phone | - Apply Occupational, Health and Safety regulations, codes and practices in the workplace - Inspect physical appearance of system. - Identify the Service and maintenance manual. - Select proper tools to Perform de-assembling procedure - Identify major sections in block diagrams of System. - Perform fault trace in different sections of system. - Apply power to the system. - Test system software and update - Replace the broken\faulty board/part. - Apply service procedure to system. - Assemble the system. - Perform test run on system. |

Knowledge & Understanding

- Understand Type, Construction and working principle of iron

- Types of electrical measuring instruments used in testing electric iron

- Working principle of thermostat Fundamentals of transformer.

- Understand Type, Construction and working principle of fan

- Explain the working principle of capacitor

- Understand  Construction and working principle of Refrigerator /Air conditioner

- Explain the working principle of electrical measuring instruments used in testing Refrigerator /Air conditioner.

- Study the working principle of condenser and compressor

- Explain the basics of helium gas

- Study about pressure gauges

- Explain the safety measures.

- Understand the function (Software and Hardware) on system

- Understand the service manual.

- Learn assembling and dissembling procedures.

- Study system function.

- Understand system part.

- Understand servicing procedure

- Understand the service manual.

- Identification of different tools.

- Knowledge of component testing.

- Knowledge of soldering.

- Knowledge of data sheets.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- Repair Electric iron

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
|  | Electric iron |
|  | Ammeter |
|  | Battery charger |
|  | Battery cleaning kit |
|  | Bearing puller |
|  | Bench vice |
|  | Cable/wire gauge |
|  | Cable cutter |
|  | Cable tester |
|  | Cable knife |
|  | Chisel |
|  | Cells tester |
|  | Circuit board |
|  | Combination plier set |
|  | Clamp meter |
|  | Disk Grinder |
|  | Duct rod |
|  | Earth tester |
|  | Files set |
|  | Filer gauge |
|  | Flux |
|  | Gloves |
|  | Grinder |
|  | Hydrometer |
|  | IR temperature gun |
|  | L Scale |
|  | Lux meter |
|  | Magnetic conductor |
|  | Lugs Punching Machine (Hydraulic and Manual) |
|  | Meggar (Insulation Tester) |
|  | Micron meter |
|  | Multimeter |
|  | Phase sequence meter |
|  | Overall combination |
|  | OTDR Meter |
|  | Ring spanner set |
|  | RPM meter |
|  | Philips Screw drivers Set |
|  | Set of nose pliers |
|  | Set of screw drivers |
|  | All related Safety tools |
|  | Soldering Lead |
|  | Soldering Iron |
|  | Specific Gravity Chart |
|  | Splicing Machine |
|  | Test lamp |
|  | Tachometer |
|  | Spring tension checking meter |
|  | Thermometer |
|  | Torque wrench |
|  | Thimble Press Pliers |
|  | Transformer |
|  | Vernier Caliper |
|  | Pressure gauge |
|  | Wire Striper |
|  | Gas cylinder |
|  | Voltmeter |

# Entrepreneurial Skills

### Develop a Marketing Plan

Overview :

This competency describes the performance outcomes, skills and knowledge required to research, develop and present a marketing plan for an entrepreneurship business

| Competency Unit | Performance Criteria |
| --- | --- |
| - Devise marketing strategies | P1.Evaluate marketing opportunity options that address organisational objectives, and evaluate their risks and returns in the selection process P2.Develop marketing strategies that address strengths and opportunities within the organisation's projected capabilities and resources P3. Develop strategies which increase resources or organisational expertise where gaps exist between current capability and marketing objectives P4.Develop feasible marketing strategies and communicate reasons that justifies their selection P5. Ensure strategies align with organisation's strategic direction P6.Develop a marketing performance review strategy, incorporating appropriate marketing metrics to review of organisational performance against marketing objectives |
| - Plan marketing tactics | P1. Detail tactics to implement each marketing strategy in terms of scheduling, costing, accountabilities and persons responsible P2. Identify coordination and monitoring mechanisms for scheduled activities P3.Ensure tactics are achievable within organisation's projected capabilities and budget P4. Ensure tactics meeting legal and ethical requirements P5.Ensure tactics provide for ongoing review of performance against objectives and budgets, and allow marketing targets to be adjusted if necessary |
| - Prepare and present a marketing plan | P1.Ensure marketing plan meets organisational, as well as marketing, objectives and incorporates marketing approaches and a strategic marketing mix P2. Ensure marketing plan contains a rationale for objectives and information that supports the choice of strategies and tactics P3. Present marketing plan for approval in the required format and timeframe P4. Adjust marketing plan in response to feedback from key stakeholders and disseminate for implementation within the required timeframe |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of: Culturally appropriate communication skills to relate to people from diverse backgrounds and people with diverse abilities

- Describe the market & marketing

- Differentiate between sellers and buyers’ market

- Describe the five ‘w’ of market

- Explain the procedure for assessing the market size and demand

- Explain the major factors to be considered when selecting a location for a business

- Describe the basic types of business ownership and the limitation of each

- Explain the computation of initial and working capital needed to start an enterprise

- Identify the advantages and disadvantages of using various sources of capital to start an enterprise

- Explain the component of cost of product

- Explain the breakeven analysis for a new business

- Calculate the breakeven point for various new business

Tools and Equipment

| S No. | Tools |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Papers and Pencil |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

Devising, documenting and presenting a marketing plan

Detailing approaches and

The marketing mix to achieve organisational marketing objectives.

### Develop and Review a Business Plan.

Overview :

This competency standard covers the process of developing and reviewing business for a small business enterprise. It requires the application of knowledge and skills to determine the scope of the business plan, prepare a business plan, determine goals, trial systems, and document, monitor and review the business plan.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Determine scope of business | - Determine scope of the business plan and associated systems is determined in consultation with specialist personnel. - Access accurate information for inform business plan development - Account for and incorporate trends and seasonal variations into the business plan. - Account for strategic goals, targets and directions of the enterprise in the development of the business plan - Comply Legal obligations in developing the business plan. |
| - Prepare business plan | - Develop operational goals and targets to meet the enterprise strategic plan. - Identify and incorporate supply chains into the business plan. - Identify risk management needs are within the business plan. - Incorporate trial systems in order to test budgetary impact and operational potential prior to full implementation of the business plan. - Set clear and measureable indicators of operational performance to allow for realistic analysis of performance. |
| - Document and review business plan | - Include fiscal and operational systems that enhance performance management and suit enterprise requirements. - Incorporate resource considerations the business plan. - Document accurately and clearly communicate business Plan to all relevant parties. - Monitor to identify strengths, weaknesses and areas for improvement performance against the business plan - Make recommendations to improve the business plan and associated systems as required. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Appreciate the importance of business plan

- Explain the process of writing a business plan

- Develop feasibility for a business idea

- Realize the problem that may be encountered when starting a small business/Enterprise

- Develop a business plan for a small business on the standard format

- Evaluate the business plan in a real market satiation

- The knowledge requirements for this competency standard are listed below:

- budgeting

- forecasting

- operational systems

- relevant industrial awards and agreements

- communication techniques

- logical and analytic methods

- profit and loss and cash flow systems

- working knowledge of environmental, OHS, industrial relations, taxation, corporate and industry legislation as they relate to the enterprise

Tools and Equipment

| S No. | Tools |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Rule,tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock andcrowbar andpinchbarfor given tasks. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard.

The skills and knowledge required to develop and review a business plan must be transferable to a different work environment.  For example, if competence is demonstrated in developing a business plan for a small enterprise, it must also be evident in reviewing a business plan in medium or large enterprise environment.

### Manage Human Resources

Overview :

This competency standard covers the skills and knowledge required to manage human resources and to manage and develop human resources to achieve organization’s operational objectives

| Competency Unit | Performance Criteria |
| --- | --- |
| - Lead and motivate people | P1.Establish goals for people and teams to optimise achievement in work tasks. P2.Take into account the capabilities of people and teams. P3. Provide advice and support sensitive to the individual's needs to people in the performance of their duties. P4. Undertake activities to achieve commitment to common goals. P5. Recognise and encourage initiative and innovation P6. Recognise and communicate achievements within the organisation. |
| - Undertake human resource planning | P1.Determine human resource needs within the anticipated operational needs and allocated budget. P2. Analyse alternatives to staffing levels which clearly demonstrate returns to the organisation. P3. Develop contingency plans for staffing which meet key provisions of the human resources plan. P4. Compare existing competencies of staff with the needs of the work group. P5. Plan staffing levels and negotiate with stakeholders within the organisational framework to achieve maximum efficiency of operations. |
| - Develop and facilitate performance | P1.Negotiate Performance Criteria individuals, teams and work groups. P2. Review Performance Criteria as circumstances change. P3. Conduct performance appraisal based on clearly established and agreed Performance Criteria. P4.Identify and propose the total performance development system strategies to rectify performance shortfalls and recognise success. P5. Address performance problems confidentially and in a constructive and timely manner, in line with relevant organisational procedures. P6. Make selections, transfers and promotions in accordance with organisation policies and supported with documented information. P7. Develop and implement mechanisms for the identification of human resource development needs within the work group taking account of the strategic plan for the organisation. |
| - Facilitate training, education and development opportunities | P1.Make information on planned training events widely available throughout the organisation. P2. Include training, education and development plans as part of individual/team performance plans. P3. Facilitate individual/team access to, and participation in, training, education and development opportunities. P4.Contribute coaching and mentoring effectively to the training, education and development of personnel in an environment of change. P5.Enhance training, education and development opportunities of individual, team and organisational performance. P6. Create workplace environment is which facilitates training, education and development |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe the hiring method/Procedures

- Describe the term & conditions of services and job description for various employments

- Describe the characteristics of successful sales personals

- communication principles

- conflict resolution principles and practice

- Equal Employment Opportunity

- grievance procedures

- interpersonal relations

- leadership theory and principles

- management principles and practice

- Occupational Health and Safety

- training and education principles

- training need analysis

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

It is essential that competence be demonstrated in the application of human resource management in a wide range of contexts in achieving the organisation's objectives.

- Evidence should be gathered over a period of time in a range of actual or simulated workplace environments.

- Evidence of competent performance should be obtained by observing an individual in a management role within the workplace or exercise or operational environment. Knowledge may be assessed through written assignments, project reports, debriefings and action learning projects.

### Negotiate for Resolving Business Issues

Overview:

This competency standard covers the skills and knowledge required to negotiate for resolving business issues

| Competency Unit | Performance Criteria |
| --- | --- |
| - Develop and implement issue resolution and grievance procedures | P1. Establish problem solving/issue resolution procedures within legislative requirements and organisation's guidelines. P2. Manage grievances and complaints are in a timely and caring way to optimise likelihood of a favourable outcome for all parties and in line with organisational objectives and procedures. P3.Document and communicate individual's rights and obligations under industrial awards/agreements and legislation are in a clear and concise manner and in appropriate language. P4. Conduct hearings, interviews and meetings within the principles of industrial democracy and participative, consultative processes. P5. Identify and invite all relevant industrial parties to participate in the resolution process. |
| - Manage disciplinary matters | P1.Implement disciplinary matters in accordance with organisation's procedures. P2. Conduct investigations in a caring and confidential manner to maintain performance and morale. P3. Provide feedback promptly. P4.Institute appeals processes in accordance with organisational procedures. P5. Recognize and recommend deficiencies in procedures for changes made |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Communication skills

- Forecasting

- Organizational procedures

- Business communication

### Manage Personal Finances

Overview :

This unit of competency describes the outcomes required to develop, implement and monitor a personal budget in order to plan regular savings and manage debt effectively.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Develop a personal budget | - Calculate current living expenses using available information to prepare a personal budget. - Keep a record of all income and expenses for a short period of time to help estimate ongoing expenses. - Subtract total expenses from total income to determine a surplus or deficit budget for the specified period. - Find reasons for a deficit budget and ways to reduce expenditure identified. - Identify ways to increase income, if possible |
| - Develop longer term personal budget | - Analyze income and expenditure and set longer term personal, work and financial goals. - Develop a longer-term budget based on the outcomes of short-term budgeting, and adjust to meet living, work and future career requirements. - Identify obstacles that might affect finances such as job loss, sickness or unexpected expenses contingency savings - Formulate a regular savings plan based on budget, using secure savings products and services. - Monitor expenditure against budget and identify areas of possible expenditure saving |
| - Identify ways to maximize future finances | - Determine sources and ways to maximize personal income, including from work, investments or available government payments/allowances. - Get further education or training to maintain or improve future income. - Identify the need for debt to finance living and other expenses, and determine the appropriate levels of debt and repayment. - Consolidate existing debt, where possible, to minimize interest costs and fees. - Seek professional money management services, where available, to ensure financial plans are effective and achievable. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- .Explain the abilities to plan and organize to keep records and monitor a personal budget

- .Describe abilities to set and review goals

- Explain basic financial management and record keeping to enable development and management of a personal budget

- Describe benefits of financial goal setting and personal budgeting to enable effective management of personal finances

- Outline numeracy skills to compare income and expenditure

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage personal finances. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- develop a personal budget based on analysis of expenditure and income;

- formulate goals and identify financial contingency plans; and

- Monitor expenditure for a period of up to 2 weeks

### Lead Small Teams

Overview :

This unit describes the outcomes required to lead small teams including setting and maintaining team and individual performance standards

.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Facilitate team Development | - Identify work requirements, standards and purpose to team members. - Assist team to develop objectives, targets and key performance indicators relevant its purpose and workplace goals. - Allocate duties regard to the skills required to properly - undertake the assigned task and according to company policy - Identify roles, responsibilities and expectations of each team member - Disseminate and discuss performance expectations to individual team members. |
| - Motivate and build the Team | - Develop positive and constructive relationships with and between team members - Facilitate team communication processes - Involve team members in the process of examining risks and options and making decisions, to ensure acceptance and support. - Encourage individual and team efforts and contributions - Strengths and weaknesses of team members are determined and sharing of work tasks is promoted to up skill team members. - Recognize team members’ queries and discuss and deal with it. |
| - Facilitate and monitor team effectiveness | - Monitor the implementation of work plan and team and individual performance against agreed strategies, targets and standards, according to workplace policies and procedures. - Monitor performance against defined Performance Criteria and/or assignment instructions and corrective action taken if required. - Support team in identifying and resolving problems that may impede performance and to suggest improvements in team Performance. - Consult team members in any review and revision of team objectives and goals. - Address performance issues which cannot be rectified within the team to appropriate personnel according to employer policy. - Refer concerns of a team and individual are referred to next level of management or appropriate specialist and conduct negotiations on their behalf. - Keep team members inform of any changes in the priority allocated to assignments, or tasks which might impact on client/customer needs and satisfaction. - Monitor team operations to ensure that internal or external employer/client needs and requirements are met. - Provide follow-up communication on all issues affecting the team - Conduct team meetings to review work operations and address issues according to workplace policies and procedures. - Support team in identifying and resolving problems that may impede performance and to suggest improvements in team performance. - Consult team members in any review and revision of team objectives and goals. - Raise any inappropriate values and standards exhibited in the workplace with the person concerned. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain conflict resolution techniques

- Explain management styles

- Describe methods of monitoring performance

- Outline the relevant legal requirements

- Outline strategies for dealing effectively with team member complaints or grievances and

- Explain team dynamics and facilitation processes

- Describe communication skills

- State workplace policies and procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

A person who demonstrates competency in this unit must be able to provide evidence of the ability to lead small teams. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- demonstrate the ability to build positive team spirit and effectively manage overall team

- performance within a workplace context;

- demonstrate the ability to coordinate a work team in a range of contexts or occasions; and

- Show knowledge of leadership, motivation and teamwork principles.

# Islamabad 31st May, 2019

# NOTIFICATION

# No. F. 5(13)/2018-DD (TE):     In pursuance of sub-section (d) of section-6" Functions of the Commission" National Vocational & Technical Training Commission (NAVTTC) Act-2011, NAVTTC is pleased  to approve and notify following qualifications in twenty (20) trades for Level 1-5 under National Vocational Qualification Framework (NVQF), which have been developed in compatibility with latest global trends in the fields and fulfilling requirements  of competency based training and assessment (CBT&A) system. The qualifications have been developed and validated in collaboration with TEVTAs, QABs, industry and other relevant stakeholders: -

| # S# | # National Vocational Qualifications |
| --- | --- |
|  | # National Qualification Level-5 diploma in Automobile Technology |
|  | National Qualification Level-5 diploma in Civil Technology |
|  | National Qualification Level-5 diploma in Construction Technology |
|  | National Qualification Level-5 diploma in Information & Commutation Technology (ICT) |
|  | National Qualification Level-5 diploma in Garment Manufacturing Technology |
|  | National Qualification Level-5 diploma in Electrical Technology |
|  | National Qualification Level-5 diploma in Electronics Technology |
|  | National Qualification Level-5 diploma in Instrumentation Technology |
|  | National Qualification Level-5 diploma in Computer Aided Design & Manufacturing (CAD /CAM) |
|  | National Qualification Level-5 diploma in Mechanical Technology |
|  | National Qualification Level-5 diploma in Graphics Designing |
|  | National Qualification Level-5 diploma in Heating, Ventilation, Air-conditioning & Refrigeration (HVACR) Technology |
|  | National Qualification Level-5 diploma in Media Production |
|  | National Qualification Level-5 diploma in Hotel Management |
|  | National Qualification Level-5 diploma in Professional Chef |
|  | National Qualification Level-5 diploma in Tourism Management |
|  | National Qualification Level-5 diploma in Hair & Beauty Services |
|  | National Qualification Level-5 diploma in Fashion Designing |
|  | National Qualification Level-5 diploma in Ceramics Technology |
|  | # National Qualification Level-5 diploma in Telecom Technology |

# 2.	All the TVET related institutions / organizations are required to implement aforementioned qualifications so that a uniform and standardized TVET qualification system is established in Pakistan and efforts are made for international equivalence / recognition of these qualifications.

# 3.	Competency Standards of the above enlisted qualifications can be accessed at NAVTTC's website (www.navttc.org).

# (Muqeem Islam)

# Director General (Skill Standards & Curricula)

# Phone: 051-9215385

# Distribution:

- Federal Secretary, Ministry of Federal Education & Professional Training, Govt of Pakistan

- Federal Secretary, Ministry of Overseas Pakistanis and Human Resource Development, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Industry and Production, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Textile Industry, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Commerce, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Railway, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Climate Change, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Religious Affairs, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Communication, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Aviation Division, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Science & Technology, Govt of Pakistan, Islamabad

- Chairperson, Punjab Technical Education and Vocational Training Authority (P-TEVTA), Lahore

- Managing Director,  Khyber Pakhtunkhwa Technical Education and Vocational Training Authority (KP-TEVTA),

- Managing Director,  Sindh Technical Education and Vocational Training Authority (S-TEVTA), Karachi

- Chairman, Azad Jammu & Kashmir, Technical Education and Vocational Training Authority (AJ&K TEVTA), Muzafarabad

- Director TVET Cell, Gilgit Baltistan, Gilgit

- Director General, Punjab Vocational Training Council (PVTC), Punjab

- Managing Director,  Technology Upgradation and Skill Development Company (TUSDEC) Lahore

- Project Director, Punjab Skill Development Program (PSDP) Lahore

- CEO, Punjab Skill Development Fund, Lahore

- Rector, UNTECH University Islamabad

- National Deputy Leader, GIZ Islamabad

- PS to Minister of Federal Education & Professional Training, Govt of Pakistan

- PS to Special Adviser to the Prime Minister on Youth Affairs, Prime Minister's Office, Islamabad

- Chairperson, Federal of Pakistan Chamber of Commerce and Industry (FPCCI), Karachi

- Conveyor, Sector Skills Council (Textile/ Construction/ Renewable Energy/ Hospitality and Tourism)

- Director Technical Education and Vocational Training Authorities (TEVTA), Balochistan

- Chairman, Pakistan Tourism Development Corporation, Lahore

- Chairman, PCSIR Headquarters, Islamabad

- Director General, Pakistan Forest Institute, Peshawar

- Chairman, Wafaq ul Madaris, Multan

- Director General, Staff Welfare, Islamabad

- Director General, NISTE Capital Administration and Development Division, Islamabad

- Director General, National Training Bureau, Islamabad

- Chairmen, Provincial Technical Education Boards

- Chairmen, Provincial Trade Testing Boards

- Secretary, IBCC, Islamabad: with the request that National qualifications of Level 5 diploma in the aforementioned trades may be considered equivalent to Diploma of Associate Engineer/HSSC after inclusion of compulsory courses in the light of IBCC general requirement.

# Copy for information to: -

- DG (P&D)/(A&F)/ (A&C) (S&C) NAVTTC

- Director General(s), NAVTTC Regional Office(s).

- Sr. Technical Advisor, TSSP-GIZ

- Staff Officer to Chairman, NAVTTC

- PS to Executive Director, NAVTTC Islamabad

- Concerned File/ Office Copy