Instruction Sheet for the Candidate

| Qualification | Level-5 in Mechanical Technology (CAD/CAM) |
| --- | --- |
| Competency Standard | Perform CAM Operations |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Manufacture an object by using Step Turning Technique on CNC Lathe shown in (Annexure-A) - Manufacture an object by using Drilling Operation Technique on CNC Milling Machine shown in (Annexure-A) - Knowledge Assessment |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Perform CAM Operations on CNC Milling & Lathe Machines by using drawings given in Annexure-A as per given dimension and specification “demonstrating the following criteria: |
| Minimum Evidence Required | - For Drilling on CNC Milling Machine, Start new part document in Feature CAM - Select Milling Setup - Select Block stock and give dimensions as per standard - Select stock material as per standard - Select axis of milling as per drawing - Select part program Zero location on stock - Select off-set if required as per job - Select Feature in Tool box menu - Select Pocket - Give dimensions of Pocket in dialogue box - Identify the Distance of Pocket on stock from corners - Select finish to complete operationSelect Assembly tab in dialogue box appears - For Turning Operation, Start new part document in Feature CAM - Select Turning Setup - Select Round stock and give dimensions as per standard - Select stock material as per standard - Select axis of milling as per drawing - Draw geometry/profile on stock as per drawing - Select finish - Select Feature in Tool box menu - Select Turn - Select the geometry/profile drawn - Set cutting parameters as per standard - Select finish to complete operation - Run simulation and check for errors |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Level-5 in Mechanical Technology (CAD/CAM) |
| Competency Standard | Perform CAM Operations |
| Assessment Task | - Manufacture an object by using Step Turning Technique on CNC Lathe shown in (Annexure-A) - Manufacture an object by using Drilling Operation Technique on CNC Milling Machine shown in (Annexure-A) |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - For Drilling on CNC Milling Machine, Start new part document in Feature CAM |  |  |
| - Select Milling Setup |  |  |
| - Select Block stock and give dimensions as per standard |  |  |
| - Select stock material as per standard |  |  |
| - Select axis of milling as per drawing |  |  |
| - Select part program Zero location on stock |  |  |
| - Select off-set if required as per job |  |  |
| - Select Feature in Tool box menu |  |  |
| - Select Pocket |  |  |
| - Give dimensions of Pocket in dialogue box |  |  |
| - Identify the Distance of Pocket on stock from corners |  |  |
| - Select finish to complete operation and Select Assembly tab in dialogue box appears |  |  |
| - For Turning Operation, Start new part document in Feature CAM |  |  |
| - Select Turning Setup |  |  |
| - Select Round stock and give dimensions as per standard |  |  |
| - Select stock material as per standard |  |  |
| - Select axis of milling as per drawing |  |  |
| - Draw geometry/profile on stock as per drawing |  |  |
| - Select finish |  |  |
| - Select Feature in Tool box menu |  |  |
| - Select Turn |  |  |
| - Select the geometry/profile drawn |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

STEP TURNING

ALL DIMENSIONS ARE IN „mm‟

DRILLING

Assessors Judgment Guide

| Qualification | Level-5 in Mechanical Technology (CAD/CAM) |
| --- | --- |
| Competency Standard | Perform CAM Operations |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ___________________ Signature: ________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Manufacture an object by using Step Turning Technique on CNC Lathe shown in (Annexure-A) - Manufacture an object by using Drilling Operation Technique on CNC Milling Machine shown in (Annexure-A) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | For Drilling on CNC Milling Machine, Start new part document in Feature CAM |  |  |  |  |  |
|  | Select Milling Setup |  |  |  |  |  |
|  | Select Block stock and give dimensions as per standard |  |  |  |  |  |
|  | Select stock material as per standard |  |  |  |  |  |
|  | Select axis of milling as per drawing |  |  |  |  |  |
|  | Select part program Zero location on stock |  |  |  |  |  |
|  | Select off-set if required as per job |  |  |  |  |  |
|  | Select Feature in Tool box menu |  |  |  |  |  |
|  | Select Pocket |  |  |  |  |  |
|  | Give dimensions of Pocket in dialogue box |  |  |  |  |  |
|  | Identify the Distance of Pocket on stock from corners |  |  |  |  |  |
|  | Select finish to complete operationSelect Assembly tab in dialogue box appears |  |  |  |  |  |
|  | For Turning Operation, Start new part document in Feature CAM |  |  |  |  |  |
|  | Select Turning Setup |  |  |  |  |  |
|  | Select Round stock and give dimensions as per standard |  |  |  |  |  |
|  | Select stock material as per standard |  |  |  |  |  |
|  | Select axis of milling as per drawing |  |  |  |  |  |
|  | Draw geometry/profile on stock as per drawing |  |  |  |  |  |
|  | Select finish |  |  |  |  |  |
|  | Select Feature in Tool box menu |  |  |  |  |  |
|  | Select Turn |  |  |  |  |  |
|  | Select the geometry/profile drawn |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Level-5 in Mechanical Technology (CAD/CAM) |
| --- | --- |
| Competency Standard | Perform CAM Operations |
| Candidate Details | Name: _____________________________________________________________ Registration Number: __________________ Candidate Signature: _____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor: _______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is an Abbreviation of CNC? |  |  |
|  | What is the main difference in between CNC Milling and Lathe? |  |  |
|  | Assembly part modeling technique is conventional or not?? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________