Instruction Sheet for the Candidate

| Qualification | Heat Treatment and Material Testing |
| --- | --- |
| Competency Standard | Preform Pattern Hardness tests for different metallic materials |
| Purpose of Assessment | Formative Assessment – Hardness tests for different metallic materials |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Measure hardness of specimen on brinnel hardness tester - Measure hardness of specimen on Rockwell hardness tester B or C scale |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Measure hardness of specimen on brinnel hardness tester and Rockwell hardness tester B or C scale“ demonstrating the following criteria: |
| Minimum Evidence Required | - Prepare the sample according to standard requirements. - Flat and clean the specimen surface with file or emery paper - Color/Etch the indent ball to make the indentation diameter visible - Apply the load /pressure for specific time according to material requirement. - Note the load /pressure. - Measure the diameter of indent with microscope and calculate the average diameter. - Calculate the hardness with the help of Brinnel Hardness Number formula and record the results. - Install the required indenter according to B or C-scale. - Set the anvil of the Rockwell tester. - Prepare the sample by performing filling, cleaning and drying operations. - Place the sample on anvil. - Apply the minor load then major load. - Note the reading from B or C-scale of the dial indicated by black or red color. - Record the results. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Heat Treatment and Material Testing |
| Competency Standards | Preform Pattern Hardness tests for different metallic materials |
| Purpose of Assessment | Formative Assessment – Hardness tests for different metallic materials |
| Assessment Task | - Measure hardness of specimen on brinnel hardness tester - Measure hardness of specimen on Rockwell hardness tester B or C scale |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare the sample according to standard requirements. |  |  |
| - Flat and clean the specimen surface with file or emery paper |  |  |
| - Color/Etch the indent ball to make the indentation diameter visible |  |  |
| - Apply the load /pressure for specific time according to material requirement. |  |  |
| - Note the load /pressure. |  |  |
| - Measure the diameter of indent with microscope and calculate the average diameter. |  |  |
| - Calculate the hardness with the help of Brinnel Hardness Number formula and record the results. |  |  |
| - Install the required indenter according to B or C-scale. |  |  |
| - Set the anvil of the Rockwell tester. |  |  |
| - Prepare the sample by performing filling, cleaning and drying operations. |  |  |
| - Place the sample on anvil. |  |  |
| - Apply the minor load then major load. |  |  |
| - Note the reading from B or C-scale of the dial indicated by black or red color. |  |  |
| - Record the results. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | Heat Treatment and Material Testing |
| --- | --- |
| Competency Standards | Preform Pattern Hardness tests for different metallic materials |
| Purpose of Assessment | Formative Assessment – Hardness tests for different metallic materials |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1.Measure hardness of specimen on brinnel hardness tester 2.Measure hardness of specimen on Rockwell hardness tester B or C scale |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepare the sample according to standard requirements. |  |  |  |  |  |
|  | Flat and clean the specimen surface with file or emery paper |  |  |  |  |  |
|  | Color/Etch the indent ball to make the indentation diameter visible |  |  |  |  |  |
|  | Apply the load /pressure for specific time according to material requirement. |  |  |  |  |  |
|  | Note the load /pressure. |  |  |  |  |  |
|  | Measure the diameter of indent with microscope and calculate the average diameter. |  |  |  |  |  |
|  | Calculate the hardness with the help of Brinnel Hardness Number formula and record the results. |  |  |  |  |  |
|  | Install the required indenter according to B or C-scale. |  |  |  |  |  |
|  | Set the anvil of the Rockwell tester. |  |  |  |  |  |
|  | Prepare the sample by performing filling, cleaning and drying operations. |  |  |  |  |  |
|  | Place the sample on anvil. |  |  |  |  |  |
|  | Apply the minor load then major load. |  |  |  |  |  |
|  | Note the reading from B or C-scale of the dial indicated by black or red color. |  |  |  |  |  |
|  | Record the results. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

Knowledge Assessment

| Qualification | Heat Treatment and Material Testing |
| --- | --- |
| Competency Standards | Preform Pattern Hardness tests for different metallic materials |
| Purpose of Assessment | Formative Assessment – Hardness tests for different metallic materials |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Question Define hardness testing? |  |  |
|  | Candidate’s response |  |  |
|  | Question Describe Brinnel and Rockwell testing machines? |  |  |
|  | Candidate’s response |  |  |
|  | Question Indenters required for Scale B&C? |  |  |
|  | Candidate’s response |  |  |
|  | Question Importance of hardness testing?. |  |  |
|  | Candidate’s response |  |  |
|  | Question What is the ASTM standard used for hardness testing techniques? |  |  |
|  | Candidate’s response |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________