National Competency Standard Level - 4 in Mechanical Technology

Level-4 Equivalent to G-I

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

ACKNOWLEDGEMENTS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to many representatives of business, industry, academia, government agencies, Provincial TEVTAs, Sector Skill Councils and trade associations who speared their time and expertise to the development and validation of these National Vocational Qualifications (Competency Standards, Curricula, Assessments Packs and related material). This work would not have been possible without the financial and technical support of the TVET Sector Support Programme co-funded by European Union, Norwegian and German Governments implemented by GIZ Pakistan. NAVTTC is especially indebted to Dr. Muqeem ul Islam, who lead the project from the front. The core team was comprised on:

- Dr. Muqeem ul Islam, Director General (Skills,Standards and Curricula) NAVTTC

- Mr. Muhammad Naeem Akhtar, Senior Technical Advisor TSSP-GIZ,

- Mr. Muhammad Yasir, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Ishaq, Deputy Director (SS&C Wing) NAVTTC

- Mr. Fayaz A. Soomro, Deputy Director (SS&C Wing) NAVTTC

NAVTTC team under the leadership of Dr. Muqeem ul Islam initiated development of CBT & A based qualifications of diploma level-5 as a reform project of TVET sector in November 2018 and completed 27 NVQF diplomas of Level-5 in September, 2019. It seems worth highlighting that during this endeavor apart from developing competency standards/curricula in conventional trades new dimensions containing high-tech trades in TVET sector in the context of generation IR 4.0 trades have also been developed which inter alia includes Robotics, Mechatronics, artificial intelligence, industrial automation, instrumentation and process control. Moreover, trades like entrepreneurship, green/environmental skills and variety of soft/digital skill have also been developed to equip the Pakistani youth with skills set as per requirement of the global trends. These skills have been made integral part of all the 27 diplomas.

Nobody has been more important in the pursuit of this project than Dr. Nasir Khan, Executive Director, NAVTTC, whose patronage and support remain there throughout the development process and lastly to thanks specially to Syed Javed Hassan, Chairman NAVTTC and Raja Saad Khan, Deputy Team Lead TSSP-GIZ who made it happened in this challenging time.

OCCUPATION PROFILE LEVEL PACKAGING CHART

LEVEL 2

Grinding

Operator

Power Plant Operator

Basic Machine Shop-I

Industrial Management

Hydraulic & Hydraulics Machines-II

Hydraulic & Hydraulics Machines-I

Entrepreneurship

Millwright

Boiler Operator

Calibration of Machines

Heat Treatment

Plastic Processing Machine

Design of Machine Elements

Digital Skills

CAD/CAM

Tool Design

Welding-II

Metrology

Basic Machine Shop-II

IT Skills

Engineering Drawing

LEVEL 5

LEVEL 4

LEVEL 3

Design                      &          Manufacturing

Welding-I

Advance Machining

Foundry

Thermal/Fluid

Table of Contents

1.	Introduction	7

2.	Purpose Of The Qualification	7

3.	Duration Of Core Competencies.	8

4.	Description And Structure Of The Course	8

5.	Date Of Validation	11

6.	Codes Of Qualifications	12

7.	Members Of Qualifications Development Committee	12

8.	Qualifications Validation Committee	13

9.	Entry Requirements	13

10.	Scheme Of Studies	14

11.	Detail Of Qualifications, Occupations And Its Competency Standards	15

(0715-M&Mt-3) National Qualification Certificate Of Level-4, Part-1 In “Mechanical Technology”	15

0715.11. Industrial Management	15

0715.11.1. Perform Store Operation In Industry	15

0715.11.2. Carry Out Maintenance Activities	15

0715.11.3paraphrase Industrial Planning	17

0715.11.4. Carry Out Site Selection For An Industry	18

0715.11.5. Develop Plant Layout	19

0715.11.6. Demonstrate Production Method	21

0715.11.7. Carry Out Job Analysis	22

0715.11.8. Carryout Production Planning And Control	23

0715.11.9. Analyse Quality Control And Quality Assurance	25

0715.11.10. Carry Out Cost Determination And Control	27

0715.12. Calibration	29

0715.12.1. Calibrate Mechanical Measuring Instruments/Tools	29

0715.12.2. Calibrate Mechanical Machines	32

0715.12.3. Calibrate Mechanical Equipment	36

0715.13. Mechanical Engineering Drawing	37

0715.13.1. Perform Basic Manual Drawing	37

0715.13.2. Construct Different Engineering Curves.	40

0715.13.3. Construct Multi-View Drawings	44

0715.13.4. Manage Display In Autocad Software	47

0715.13.5. Develop 2d Drawings	48

0715.13.6. Develop 3d Drawing	49

0715.14. Hydraulics And Hydraulic Machinery	51

0715.14.1. Operate Hydraulic Bench And Its Functions	51

0715.14.2. Calibrate Bourdon Tube And Diaphragm Pressure Gauge	53

0715.14.3. Operate Hydraulic Press	55

0715.14.4. Verify Bernoulli’s Equation	56

0715.14.5. Analyse The Performance Of Turbines	58

0715.14.6. Analyse The Performance Of Pumps	60

0715.14.7. Actuate Double Acting Hydraulic Cylinder	62

0715.14.8. Setup A Pressure Device And Hold Specific Load On A Double Acting Cylinder	63

0715.14.9. Construct Circuit Of Double Acting Hydraulic Cylinder For Mechanical Interlocking	64

0715.14.10. Set Speed And Direction Of Hydraulic Motor	67

0715.14.11. Perform Pressure Measuring Instruments	68

0715.14.12. Operate Flow Regulating Devices	71

0715.15. Applied Thermodynamics	73

0715.15.1. Solve Problems Related To Fundamentals Of Thermodynamics	73

0715.15.2. Solve Problems Of Gas Laws And Properties Of Perfect Gases	75

0715.15.3. Derive Thermodynamics Processes And Cycles	76

(0715-M&Mt-3) National Qualification Certificate Of Level-4, Part-2 In “Mechanical Technology”	78

0715.16. Hvac	78

0715.16.1	Install Residential Air Conditioner	78

0715.16.2	Repair/Service Residential Air-Conditioner	81

0715.16.3	Repair / Service Residential Refrigeration	85

0715.16.4	Carry Out Calculations And Prepare Estimates	87

0715.16.5	Install Commercial Refrigeration Units	89

0715.16.6	Repair/Service Commercial Refrigeration	92

0715.16.7	Carry Out Fabrication For Hvac Work	95

0715.16.8	Install Central Air Conditioning Systems	99

0715.16.9	Repair/Service Central Air Conditioning	104

0715.16.10	Perform Preventive Maintenance	108

0715.17. Mill Wright/ Industrial Mechanic	112

0715.17.1	Perform Routine Trade Activity	112

0715.17.2	Perform Measuring And Layout Of Work Piece	113

0715.17.3	Perform Cutting And Welding Operation	115

0715.17.4	Perform Rigging, Hoisting / Lifting And Moving	116

0715.17.5	Service General Mechanical Equipment	118

0715.18. Tool Design And Tool Making	122

0715.18.1. Design And Draw The Cam Profile With Knife Edge Follower For Uniform Velocity	122

0715.18.2. Draw Standard Parts Of Jigs & Fixtures (Manual Drawing)	124

0715.18.3. Draw Different Types Of Clamps	127

0715.18.4. Draw Locators (Manual)	129

0715.18.5. Draw Template Jig And Plate Type Drill Jig	131

0715.18.6. Draw Swinging Leaf Jig	133

0715.18.7. Draw An Indexing Jig	136

0715.18.8. Draw An Angle Milling Fixture	139

0715.18.9. Draw Lathe Fixture For Turning And Boring Operation	142

0715.18.10. Draw Simple Blanking Die For Making A Blank	144

0715.18.11. Draw A Progressive Die For Making A Washer Of Inner Diameter 20 And Outer 40 Mm	146

0715.18.12. Draw A Simple Drawing Die For A Cup	148

0715.19. Boiler Operator	150

0715.19.1	Maintain Boiler House Record	150

0715.19.2	Set And Adjust Burners	151

0715.19.3	Perform Boiler Operations	152

0715.19.4	Operate Boiler Auxiliaries	155

0715.19.5	Perform Maintenance Of Boiler	159

3.	List Of Machinery, Tools & Equipment.	161

Notification	168

168

# Introduction

Mechanical engineering is the discipline that applies engineering, physics, engineering mathematics, and materials science principles to design, analyze, manufacture, and maintain mechanical systems. It is one of the oldest and broadest of the engineering disciplines.

Mechanical Technology Courses, classes and training Programs. Mechanical technology courses are available through undergraduate and advanced degree programs. Curricula typically include courses in mechanical principles, processes and materials, welding, safety and quality control.

Mechanical engineering technicians help mechanical engineers manufacture industrial machinery and other equipment. Most employers prefer to hire candidates with associate's degrees or other postsecondary training in mechanical engineering technology.

The importance of mechanical engineering may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being conscious of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechanical Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Mechanical industries, Engineering Universities and Technical Education of the country.

# Purpose of the Qualification

The purpose of this qualification is to set high professional standards for Mechanical technology sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for Mechanical Technology industry

- Enable the existing workforce to capacitate themselves in new technologies and methods

# Duration of Core competencies.

The entire Competency Standards for Level-4 are composed of 66 competencies that will be covered in 1276 Learning hours. Duration of the course is to be One year. The total weightage for technical competencies is 1276 hours. A total of 324 hours has been reserved for allied subjects that are Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical Competencies are given as under:

Level - 2 = 6 months.

Level - 3 = 6 months.

Level - 4 = 1 Year. (Part-I & Part-II)

Level - 5 = 1 Year. (Part-I & Part-II)

The distribution of contact hours and credit hours in level-4 is given below:

Level - 4 (Part-I & Part-II) = 1 Year (16 weeks + 16 Weeks)

| Level-4 (Part-1) | Level-4 (Part-2) |
| --- | --- |
| Total. 662 hours. & 62.2 Credits Theory. 161 hours (24.32 %) Practice. 501 hours (75.68 %) | Total. 614 hours. & 61.4 Credits Theory. 116 hours (18.89 %) Practice. 498 hours (81.11 %) |

Level-4 (Part-1 + Part-2)

Total. 1276 hours. & 127.6 Credits Theory. 277 hours (21.71 %) Practice. 999 hours (78.29 %)

# Description and structure of the course

Following is the structure of the course:

(0715-M&MT-3) National Qualification Certificate of level-4, Part-1 in “Mechanical Technology”

0715.11. Industrial Management

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Perform Store Operation in Industry | 4 | Functional | 3 | 6 | 9 | 0.9 |
| 2 | Perform Maintenance Activities | 4 | Functional | 3 | 9 | 12 | 1.2 |
| 3 | Paraphrase Industrial Planning | 4 | Functional | 1 | 3 | 4 | 0.4 |
| 4 | Carry out Site Selection for an Industry | 4 | Functional | 2 | 3 | 5 | 0.5 |
| 5 | Develop Plant Layout | 4 | Functional | 6 | 9 | 15 | 1.5 |
| 6 | Demonstrate Production Method | 4 | Functional | 2 | 3 | 5 | 0.5 |
| 7 | Carry out Job Analysis | 4 | Functional | 2 | 3 | 5 | 0.5 |
| 8 | Carryout Production Planning and Control | 4 | Functional | 5 | 9 | 14 | 1.4 |
| 9 | Perform Quality control and Quality Assurance | 4 | Functional | 2 | 3 | 5 | 0.5 |
| 10 | Carry out Cost Determination and Control | 4 | Functional | 6 | 0 | 6 | 0.6 |
|  | Total |  |  | 32 | 48 | 80 | 8 |

0715.12. Calibration

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Calibrate Mechanical Measuring Instruments/tools | 4 | Technical | 7 | 21 | 28 | 2.8 |
| 2 | Calibrate Mechanical Machines | 4 | Technical | 12 | 36 | 48 | 4.8 |
| 3 | Calibrate Mechanical Equipment | 4 | Technical | 6 | 18 | 24 | 2.4 |
|  | Total |  |  | 25 | 75 | 100 | 10 |

0715.13. Mechanical Engineering Drawing

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Perform Basic Manual Drawing | 4 | Technical | 5 | 21 | 26 | 2.6 |
| 2 | Construct different Engineering Curves | 4 | Technical | 8 | 36 | 44 | 4.4 |
| 3 | Construct multi-view drawings | 4 | Technical | 7 | 33 | 40 | 4 |
| 4 | Manage display in AutoCAD software | 4 | Technical | 5 | 18 | 23 | 2.3 |
| 5 | Develop 2D drawings | 4 | Technical | 5 | 30 | 35 | 3.5 |
| 6 | Develop 3D drawing | 4 | Technical | 6 | 48 | 54 | 5.4 |
|  | Total |  |  | 36 | 186 | 222 | 22.2 |

0715.14. Hydraulics and Hydraulic Machinery

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Operate Hydraulic Bench and Its Functions | 4 | Technical | 4 | 9 | 13 | 1.3 |
| 2 | Calibrate Bourdon Tube and Diaphragm Pressure Gauge | 4 | Technical | 2 | 6 | 8 | 0.8 |
| 3 | Operate Hydraulic Press | 4 | Technical | 2 | 6 | 8 | 0.8 |
| 4 | Verify Bernoulli’s Equation | 4 | Technical | 3 | 9 | 12 | 1.2 |
| 5 | Analyse the performance of Turbines | 4 | Technical | 4 | 12 | 16 | 1.6 |
| 6 | Analyse the performance of Pumps | 4 | Technical | 4 | 12 | 16 | 1.6 |
| 7 | Actuate Double Acting Hydraulic Cylinder | 4 | Technical | 4 | 12 | 16 | 1.6 |
| 8 | Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder | 4 | Technical | 2 | 6 | 8 | 0.8 |
| 9 | Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking | 4 | Technical | 4 | 12 | 16 | 1.6 |
| 10 | Set Speed and Direction of Hydraulic Motor | 4 | Technical | 2 | 6 | 8 | 0.8 |
| 11 | Perform Pressure Measuring Instruments | 4 | Technical | 7 | 21 | 28 | 2.8 |
| 12 | Operate Flow Regulating Devices | 4 | Technical | 6 | 18 | 24 | 2.4 |
|  | Total |  |  | 44 | 129 | 173 | 17.3 |

0715.15. Applied Thermodynamics

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Solve problems related to fundamentals of thermodynamics | 4 | Technical | 7 | 15 | 22 | 2.2 |
| 2 | Solve problems of Laws and properties of perfect gases | 4 | Technical | 4 | 12 | 16 | 1.6 |
| 3 | Derive Thermodynamics Processes and Cycles | 4 | Technical | 13 | 36 | 49 | 4.9 |
|  | Total |  |  | 24 | 63 | 87 | 8.7 |

(0715-M&MT-3) National Qualification Certificate of level-4, Part-2 in “Mechanical Technology”

0715.16. HVAC

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Install Residential Air Conditioner | 4 | Technical | 2 | 18 | 20 | 2 |
| 2 | Repair/Service Residential Air-Conditioner | 4 | Technical | 2 | 24 | 26 | 2.6 |
| 3 | Repair / Service Residential Refrigeration Units | 4 | Technical | 2 | 18 | 20 | 2 |
| 4 | Carry out Calculations and Prepare Estimates | 4 | Technical | 6 | 0 | 6 | 0.6 |
| 5 | Install Commercial Refrigeration Units | 4 | Technical | 4 | 24 | 28 | 2.8 |
| 6 | Repair/Service Commercial Refrigeration Units | 4 | Technical | 4 | 24 | 28 | 2.8 |
| 7 | Carry out Fabrication for HVAC Work | 4 | Technical | 4 | 24 | 28 | 2.8 |
| 8 | Install Central Air Conditioning Systems | 4 | Technical | 6 | 36 | 42 | 4.2 |
| 9 | Repair/Service Central Air Conditioning System | 4 | Technical | 2 | 18 | 20 | 2 |
| 10 | Perform Preventive Maintenance | 4 | Technical | 4 | 12 | 16 | 1.6 |
|  | Total |  |  | 36 | 198 | 234 | 23.4 |

0715.17. Mill Wright/ Industrial Mechanic

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Perform Routine Trade Activity | 4 | Technical | 2 | 6 | 8 | 0.8 |
| 2 | Perform Measuring and Layout of Workpiece | 4 | Technical | 2 | 9 | 11 | 1.1 |
| 3 | Perform Cutting and Welding Operation | 4 | Technical | 2 | 12 | 14 | 1.4 |
| 4 | Perform Rigging, Hoisting/Lifting & Moving | 4 | Technical | 4 | 12 | 16 | 1.6 |
| 5 | Service general mechanical equipment | 4 | Technical | 9 | 42 | 51 | 5.1 |
|  | Total |  |  | 19 | 81 | 100 | 10 |

0715.18. Tool design and tool making

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Design and draw the CAM profile with knife edge follower for uniform velocity | 4 | Technical | 2 | 9 | 11 | 1.1 |
| 2 | Draw standard parts of jigs & fixtures (manual drawing) | 4 | Technical | 6 | 18 | 24 | 2.4 |
| 3 | Draw different types of clamps | 4 | Technical | 6 | 18 | 24 | 2.4 |
| 4 | Draw locators (Manual) | 4 | Technical | 1 | 6 | 7 | 0.7 |
| 5 | Draw template jig and plate type drill Jig | 4 | Technical | 2 | 9 | 11 | 1.1 |
| 6 | Draw swinging leaf jig | 4 | Technical | 2 | 15 | 17 | 1.7 |
| 7 | Draw an indexing jig | 4 | Technical | 2 | 18 | 20 | 2 |
| 8 | Draw an angle milling fixture | 4 | Technical | 2 | 21 | 23 | 2.3 |
| 9 | Draw lathe fixture for turning and boring operation | 4 | Technical | 5 | 12 | 17 | 1.7 |
| 10 | Draw simple blanking die for making a blank dia 40 mm | 4 | Technical | 5 | 12 | 17 | 1.7 |
| 11 | Draw a progressive die for making a washer of inner diameter 20 and outer 40 mm | 4 | Technical | 5 | 12 | 17 | 1.7 |
| 12 | Draw a simple drawing die for a cup for inner dia 40 mm, depth 30 mm and sheet thickness 1mm | 4 | Technical | 3 | 9 | 12 | 1.2 |
|  | Total |  |  | 41 | 159 | 200 | 20 |

0715.19. Boiler Operator

| S # | Competency standards | Level | Category | Contact Hour |  | Total | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. |  |  |
| 1 | Maintain Boiler house Record | 4 | Technical | 2 | 6 | 8 | 0.8 |
| 2 | Set and adjust burners | 4 | Technical | 2 | 12 | 14 | 1.4 |
| 3 | Perform Boiler Operations | 4 | Technical | 6 | 24 | 30 | 3 |
| 4 | Operate boiler auxiliaries | 4 | Technical | 6 | 12 | 18 | 1.8 |
| 5 | Perform Maintenance of Boiler | 4 | Technical | 4 | 6 | 10 | 1 |
|  | Total |  |  | 20 | 60 | 80 | 8 |

# Date of Validation

National qualification of Level-4 on Mechanical Technology has been validated by the Qualifications Validation Committee (QVC) members on 20-22 May, 2019 and will remain in currency until 19 May, 2029

*Shall be reviewed after 3 years

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for DAE Mechanical Technology level-5 |  |
| --- | --- |
| Code | Description |
| 0715-M&MT-1 | 2nd Level National Certificate of level-5, in “Mechanical Technology” |
| 0715-M&MT-2 | 3rd Level National Certificate of level-5, in “Mechanical Technology” |
| 0715-M&MT-3 | 4th Level National Certificate of level-5, in “Mechanical Technology” |
| 0715-M&MT-4 | 5th Level National Certificate of level-5, in “Mechanical Technology” |

# Members of Qualifications Development Committee

The following members participated in the qualification’s development and validation of these qualifications:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Rizwan Zafar | Assistant Professor | University of Lahore |
| 2 | Ali Raza | Principal | Quaid-e-Azam College of Engineering & Technology Okara |
| 3 | Muhammad Azhar | Senior Instructor Mech. | Descon Training Institute Lahore |
| 4 | Saba Sadiq | Lecturer | University of Lahore |
| 5 | Faizan | Installation Engineer | Buhler Pakistan pvt Ltd |
| 6 | Engr. Farooq Iftikhar | Metallurgy engineer | PCSIR Lahore |
| 7 | Engr Haris Iqram | Foundry Engineer | PCSIR Lahore |
| 8 | zain-ul-abideen | Instructor (Machinist) | GTTI , Mughal Pura Lahore |
| 9 | Shakeel Ahmed | Master Trainer, | Govt Staff Training College Lahore, |
| 10 | Waqas Iqbal | Instructor (Machinist) | GTTI, Mughal Poora Lahore |
| 11 | Khawar Abbas | Machinist (Mechanical) | GSTC Gujrat |
| 12 | Muhammad Arsalan Khan | Lecturer | University Of Lahore |
| 13 | Engr. Inayat Ur Rehman | DACUM Facilitator, | Ex. KPK TVETA |
| 14 | Muhammad Yasir Khan | Asstt. Director | NAVTTC Islamabad |

# Qualifications Validation Committee

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Jamal Akbar | Associate Professor | GPI, Peshawar KP-TEVTA |
| 2 | Engr. Muahmmad Akram Sarfraz | Instructor (Mechanical) | GCT Railway Road,Lahore |
| 3 | Engr. M Shahzad Javed | Research Officer (Technical) | PBTE, Lahore |
| 4 | Engr. Farooq Iftikhar | Metallurgy Engineer | PCSIR Lahore |
| 5 | Muhammad Arsalan Khan | Lecturer | University of Lahore |
| 6 | Ali Raza | HOD Mechanical | Quaid E Azam College Sahiwal |
| 7 | Saba Sadiq | DACUM CO-Facilitator, Lecturer | University of Lahore |
| 8 | Engr. Inayat Ur Rehman | DACUM Facilitator, | Ex. KPK TEVTA |
| 9 | Muhammad Yasir Khan | Asstt. Director | NAVTTC Islamabad |

# Entry Requirements

The entry for National Certificate level 5, in Mechanical Technology are

- A person having National Vocational Certificate level 3, in Mechanical Technology.

# Scheme of Studies

(0715-M&MT-3) National Qualification Certificate of level-4, Part-1 in “Mechanical Technology”

| Code |  | Name of Subjects | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  |
| 0715.11 | Industrial Management |  | Technical | 32 | 48 | 80 | 8 |
| 0715.12 | Calibration |  | Technical | 25 | 75 | 100 | 10 |
| 0715.13 | Mechanical Engineering. Drawing |  | Technical | 36 | 186 | 222 | 22.2 |
| 0715.14 | Hydraulics and Hydraulic Machinery |  | Technical | 44 | 129 | 173 | 17.3 |
| 0715.15 | Applied Thermodynamics |  | Technical | 24 | 63 | 87 | 8.7 |
|  |  |  | Allied |  |  |  |  |
|  |  |  | Allied |  |  |  |  |
| Total |  |  |  | 161 | 501 | 662 | 66.2 |

(0715-M&MT-3) National Qualification Certificate of level-4, Part-2 in “Mechanical Technology”

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  |
| 0715.16 | HVAC | Technical | 36 | 198 | 234 | 23.4 |
| 0715.17 | Millwright | Technical | 19 | 81 | 100 | 10 |
| 0715.18 | Tool design and tool making | Technical | 41 | 159 | 200 | 20 |
| 0715.19 | Boiler Operator | Technical | 20 | 60 | 80 | 8 |
|  |  | Allied |  |  |  |  |
|  |  | Allied |  |  |  |  |
| Total |  |  | 116 | 498 | 614 | 61.4 |

# Detail of Qualifications, Occupations and its Competency Standards

# (0715-M&MT-3) National Qualification Certificate of level-4, Part-1 in “Mechanical Technology”

## 0715.11. Industrial Management

### 0715.11.1. Perform Store Operation in Industry

Overview: This competency standard covers the skills and knowledge required to explain procedure of receipt items, describe forms in store operation and issuance procedure of store items.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Explain procedure adopted by the store on receipt of store items | P1. Ensure Materials Report at Central Receiving section P2. Unload the goods P3. Verify the consignment P4. Record on goods receive note P5. Use gauges to inspect the material |
| CU-2: Describe forms used in store operation | P1. Check the Stores in-ward Receipt P2. Read the Inspection Report P3. Inspect the Rejection Memo |
| CU-3: Explain the procedure of issuance of store items | P1. Check the form and identify date, requisition number, name, department, and job number, material specifications, and quality, place of delivery, unit cost, total cost, and signature of the person making requisition. P2. Issue the Materials against the materials requisition P3. Keep Records |

Knowledge & Understanding

- Describe store keeping in an industry

- Describe store keepers’ duties and operations

- Describe the importance of record keeping

- Describe various methods of store keeping

- Describe various methods of material Handling in a store

### 0715.11.2. Carry out Maintenance Activities

Overview: This competency standard covers the skills and knowledge required to explain duties of maintenance department, types of maintenance and replacement studies.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Identify Duties of Maintenance Department | P1. Carry out Repair of equipment in shortest possible time P2. Ensure Minimum breakdown equipment P3. Ensure Existing equipment maintenance P4. Carry out Equipment inspection, lubrication and cleaning P5. Perform Alteration of Existing equipment P6. Ensure Protection of Equipment waste, disposal, overhaul P7. Develop Repair/maintenance Schedule P8. Perform Preventive maintenance P9. Develop Facilities of erection and maintenance of machinery |
| CU-2: Identify Types of Maintenance | P1. Demonstrate Breakdown Maintenance P2. Demonstrate Preventive Maintenance P3. Demonstrate Predictive Maintenance P4. Demonstrate Corrective Maintenance |
| CU-3: Demonstrate Replacement Studies | P1. Locate the trouble P2. Stop the operation P3. Carry out trouble shooting P4. Carry out trade-off between the need of replacement or repair P5. Perform replacement if necessary |

Knowledge & Understanding

- Define maintenance

- Highlight benefits of maintenance

- Describe and explain various types of maintenance.

- Explain trade-off procedure

### 0715.11.3Paraphrase Industrial Planning

Overview: This competency standard covers the skills and knowledge required to define and explain need of Industrial Planning and explaining Phases of Industrial Planning.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Identify the Need of Industrial Planning | P1. Enlist Planning Principles P2. Mention Industrial Planning Requirements P3. Identify the resources and data required for industrial planning P4. Draw a flow chart diagram for industrial planning process |
| CU-2: Identify Phases of Industrial Planning | P1. Identify Assets and Liabilities in Financial Planning P2. Prepare reports based on financial planning P3. Analyse the reports and mention the feasibility P4. Identify the Product and Raw Materials in Product Planning Phase P5. Draw Product Life Cycle P6. Identify all stages of the product life cycle P7. Report marketing analysis of the product P8. Estimate the unit cost of manufacturing in Process Planning P9. Select the right equipment required for production P10. Select the manufacturing process P11. Estimate the quantity envisaged |

Knowledge & Understanding

- Define Planning

- Describe Industry

- Describe Industrial Planning

- Distinguish between simple planning and industrial planning

- Explain various phases of Industrial Planning

- Describe the importance of Report writing

- Explain Life Cycle

- Describe market Analysis importance

### 0715.11.4. Carry out Site Selection for an Industry

Overview: This competency standard covers the skills and knowledge required to explain Economic and Technical factors of Site Selection.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Highlight Economic Factors of Site selection | P1. Highlight the importance of site selection P2. Compare the Land cost of optional areas P3. Create surveys based on appropriate geography, cost of utilities, road network, community facilities etc. P4. Prepare a selection criteria P5. Analyse the results of surveys P6. Choose the site based on the selection criteria |
| CU-2: Highlight Technical Factors of Site selection | P1. Create product focused surveys based on Skilled man power, Raw material availability, finished product transportation, spares and servicing P2. Prepare a selection criteria P3. Analyse the results of surveys P4. Choose the site based on the selection criteria |

Knowledge & Understanding

- Explain the potential factors important for an industrial site

- Describe the methods required to analyses a potential site

- Describe difference between economic and technical factors of site selection

- Explain results presentation method

### 0715.11.5. Develop Plant Layout

Overview: This competency standard covers the skills and knowledge required to define plant layout, objectives of layout, types of layouts, criteria of good layout, advantages of good layout and factors in preparation of layout.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Paraphrase Plant Layout Importance | P1. Highlight the relation between operational cost and material handling P2. Identify the importance of access to equipment for quick maintenance and reduction of downtime P3. Identify the importance of flexibility of Layout P4. Identify the safety requirements for smooth operation |
| CU-2: Mention Objectives of Layout | P1. Identify material handling, material control, and good house keeping P2. Identify effective utilization of people, equipment, space and energy P3. Ensure minimum capital investment P4. Ensure route flexible P5. Promote ease of maintenance P6. Provide employee safety and job satisfaction |
| CU-3: Mention Types of Layouts | P1. Enlist details of fixed position layout P2. Enlist details of Process Layout P3. Enlist details of Product layout P4. Enlist details of Mixed Layout |
| CU-4: Identify the Criteria of a good layout | P1. Highlight the arrangement of machines, equipment and work areas that material is moved smoothly P2. Elimination of all possible delays P3. Unique flow pattern of each product P4. Optimum utilisation of cubic space P5. Safe working conditions P6. Reducing Investment P7. Allow flexibility |
| CU-5: Identify the Advantages of a good layout | P1. Highlight the Labour Cost factor P2. Highlight the Production Control benefits P3. Highlight the benefits of good layout in Supervision P4. Mark other manufacturing Costs P5. Paraphrase the advantages to the Worker |
| CU-6: Identify the Factors in Preparation of a Layout | P1. Paraphrase the Material factor P2. Paraphrase the Machine factor P3. Paraphrase the Man factor P4. Paraphrase the Movement Factor P5. Paraphrase the Waiting Factor P6. Paraphrase the Service Factor P7. Paraphrase the Building Factor P8. Paraphrase the Change Factor |

Knowledge & Understanding

- Describe plant Layout

- Describe the elements that must be considered while designing a plant layout

- Describe a floor Layout

- Describe the important attributes of good layout

- Compare between various types of layouts

- Give examples specific to each layout type

- Differentiate a good layout from a bad layout

- Differentiate flexible layout from fixed layout

- Describe the indirect advantages of a good layout

- Describe indirect cost saving benefits

- Describe time and motion study

- Describe synchronization

### 0715.11.6. Demonstrate Production Method

Overview: This competency standard covers the skills and knowledge required to define production and explain types of production.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Paraphrase Production | P1. Enlist important activities involved in production P2. Highlight Need for Production |
| CU-2: Explain Types of Production | P1. Enlist all types of production P2. Paraphrase the Intermittent or Batch Production P3. Paraphrase the Continuous production P4. Paraphrase the Group Technology P5. Paraphrase the Mass Production P6. Paraphrase the Just-in-time P7. Paraphrase the applications of each production methods |

Knowledge & Understanding

- Define production

- Describe importance of production

- Describe production types

- Differentiate between production and service industry

### 0715.11.7. Carry out Job Analysis

Overview: This competency standard covers the skills and knowledge required to explain motion study and explain time study.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Demonstrate Motion Study | P1. Identify the task to be observed P2. Prepare Process chart with columns for to and from destinations P3. Perform activity and note all the movements during the process P4. Mark time required for each movement P5. Analyse the important and non-important activities and the activities that took maximum P6. Devise method to improve activity completion time and movement reduction P7. Perform any activity according to the principles of motion study |
| CU-2: Demonstrate Time Study | P1. Identify the task to be analysed P2. Ensure equipment required to carry out the Time and Motion study e.g. Time Study Form, Timing Device, Pen/Pencil, Safety equipment, Clipboard P3. Prepare Process charts P3. Perform activity and note all the movements during the process P4. Mark time required for each movement P5. Analyse the important and non-important activities and the activities that took maximum time P6. Devise method to improve activity completion time and movements reduction P7. Perform any activity according to the principles of Time study |

Knowledge & Understanding

- Explain the time and motion study process

- Explain the importance of time and motion study

- Identify an area in your knowledge where this study would be relevant

### 0715.11.8. Carryout Production Planning and Control

Overview: This competency standard covers the skills and knowledge required to define production planning and control (PPC), describe objectives of PPC, explain functions of production control, explain routing, scheduling and loading, explain packaging and dispatching.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Paraphrase Production Planning and Control | P1. Highlight the defining points of Production Planning and Control P2. Highlight the activities important for Production Planning and Control P3. Highlight the data required for a good Production Planning and Control process completion P4. Develop a Complete Manufacturing plan for any identified product |
| CU-2: Enlist Objectives of Production Planning and Control | P1. Highlight the sequence of operation P2. Highlight the issue coordinated work schedule P3. Highlight the plan plant facility P4. Highlight and maintain sufficient inventories P5. Highlight and maintain production and employment level |
| CU-3: Enlist and explain Functions of Production Control | P1. Identify Pre-Planning Functions including Product Design, Process Design, Flow Design and Forecast Demand P2. Identify Planning Functions including Planning Resources, Process Planning, Scheduling and Estimation P3. Identify Control Function including Dispatching, Inspection, Expediting and Evaluation |
| CU-4: Explain Routing, Scheduling and Loading | P1. Paraphrase Routing P2. Identify the specification of flow or sequence of operation and the process to be followed in producing a particular manufacturing lot. P3. Enlist the Factors of routing P4. Paraphrase Scheduling P5. Prepare Time-table of production functioning P6. Enlist the Objectives of scheduling P7. Enlist factors affecting the scheduling P8. Paraphrase Loading |
| CU-5: Explain Packaging and Dispatching | P1. Paraphrase Packaging P2. Enlist activities involved in Packaging P3. Identify the release of resources along with the necessary instruction for the production function |

Knowledge & Understanding

- Describe Production Planning and Control

- Describe Objectives of Production Planning and Control

- Describe functions of Production Planning and Control

- Describe routing, scheduling and loading

- Differentiate between routing, scheduling and loading

- Describe Packaging process

- Describe Dispatching process

### 0715.11.9. Analyse Quality control and Quality Assurance

Overview: This competency standard covers the skills and knowledge required to explain inspection and its types as well as quality control and assurance.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Explain Inspection and its Types | P1. Carryout Pre-production inspection P2. Carryout During production inspection P3. Carryout Final random inspection P4. Carryout Container loading inspection |
| CU-2: Explain Quality Control and Assurance | P1. Highlight important aspects of quality P2. Enlist quality traits of a product P3. Enlist the techniques involved in quality control P4. Enlist the tools and techniques involved in quality assurance |

Knowledge & Understanding

- Describe the importance of inspection

- Describe various inspection methods

- Identify various national and international standards of Quality

- Differentiate between quality control and quality assurance

### 0715.11.10. Carry out Cost Determination and Control

Overview: This competency standard covers the skills and knowledge required to explain cost calculation of industrial products and describe cost control.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Demonstrate Cost Calculation of Industrial Products | P1. Procure data from Production Planning Department P2. Identify Material Cost P3. Identify Standard Time P4. Identify Labour Cost P5. Identify Prime Cost P6. Identify Factory Overheads P7. Identify Administrative overheads P8. Identify Packaging and delivery charges P9. Determine Total cost P10. Determine Sale Price P11. Identify Discount Allowed P12. Decide Delivery Time |
| CU-2: paraphrase Cost Control | P1. Analyse the external factors like market supply and demand competition P2. Demonstrate that Cost is internal to the organisation |

Knowledge & Understanding

- Describe cost calculation procedure and benefits

- Describe supply-demand relationship

## 0715.12. Calibration

### 0715.12.1. Calibrate Mechanical Measuring Instruments/Tools

Overview This competency standard covers the skills and knowledge required to calibrate the measuring devices like Vernier calipers, micrometer screw gauge, dial gauge indicator, calibrate pressure and temperature gauges, calibrate pressure and temperature sensors, and calibrate tachometer.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1: Calibrate Vernier Caliper | P1. First of all, close the both jaws of Vernier Caliper. P2. Check the zero error P1. If there is any zero error then open up the jaws and precisely match the both jaws and then assemble the device P4. Again, close the jaws and check the zero error, if both the zeros of Vernier scale and main scales are matching in same location then the caliper is calibrated. |
| CU2: Calibrate Micrometer Screw Gauge | P1. First of all, close the both jaws of Micrometer Screw Gauge P2. Check the zero error P3. If there is any zero error then open up the jaws and precisely match the both jaws and then assemble the device P4. Again, close the jaws and check the zero error, if both the zeros of Vernier scale and main scales are matching in same location then the micrometer is calibrated. |
| CU3: Calibrate Dial Gauge | P1. First of all, place the gauge on a flat surface and check its correctness P2. Check the zero error P3. If there is any zero error then check the push needle, either its loose or tight? If lose then open the spring and replace it. P4. If pin is working properly then, there are two possibilities either it would be the surface base plate fault or the gauge’s needle’s fault. P5. Check the surface flatness base plate P6. If everything is working properly then disassemble the gauge and start fixing its needle by checking its position. P7. After fixing the inside needle, close the gauge and check its zero error again, if it is giving exact value then the gauge is calibrated. |
| CU4: Calibrate Gauges | P1. First of all, verify the type of gauge P2. Check the zero error P3. If there is any zero error then disassemble the gauge very carefully by pouring the fluid in a clean bowl P4. After dismantling each component of the gauge now check the needle’s base either its loose or tight, if loose the fix the problem P5. Assemble the gauge and check its zero error, if needle is on exactly upon zero then the gauge is calibrated |
| CU5: Calibrate Sensors | P1. Make it sure which type of sensor is to be calibrated, first P2. Trace the fault P3. After knowing the fault, fix it |
| CU6: Calibrate Tachometer | P1. Check the speed of any revolving part by using the tachometer P2. If rpm is not directly proportional to the regulatory switch, like by increasing or decreasing it had some fluctuations, then it needed to be fix P3. Carefully, dismantle the device and find the fault, either it is mechanical issue or some electronic issue |

Knowledge & Understanding

- What is Vernier caliper?

- What are the main components of Vernier Caliper?

- What is main scale and Vernier Scale?

- How to open/close the VC

- What is least count of VC

- What is Micrometer Screw gauge?

- What are the main components of Micrometer Screw Gauge?

- How to open/close the micrometer

- What is least count of micrometer?

- What is Dial Gauge?

- What is gauge?

- What are the different types of gauges?

- How to open/close the gauge

- How to fix the needle

- Knowledge of all types of sensors used in mechanical field

- Knowledge of working of all types of sensors used in mechanical field

- Pre-knowledge of all types of rotating devices.

- Knowledge of measuring devices used to measure speed

- Terminologies involved in speed and rpm

Tool & Equipment

- Vernier Caliper

- Plier

- Screw Driver

- Scale

- Oil

- Micrometer Screw Gauge

- Gauge

- Plier

- Sensors

- Sensor calibrator

- Tachometer

- Tachometer tools

- Nose plier

- Soldering iron

- Soldering wire

### 0715.12.2. Calibrate Mechanical Machines

Overview This competency standard covers the skills and knowledge required to Calibrate Power/Circular Heck Saw, Calibrate Lathe Machine, Calibrate Drilling Machine, Calibrate Shaper/Planer Machine, Calibrate CNC Machine, Calibrate Slotter Machine, Calibrate Milling Machine, Calibrate Hobbing Machine, Calibration of Grinding Machine, Calibrate Arc Welding Plant (Digital)

| Competency Units | Performance Criteria |
| --- | --- |
| CU1: Calibrate Power/Circular Heck Saw | P1. Check the teeth of hacksaw P2. Check the bottom and top dead centers of the hacksaw P3. Check the speed/velocity of one cycle of the hacksaw |
| CU2: Calibrate Lathe Machine | P1. Perform operation on lathe machine and analyze the chuck, tool post, tail stock, barrel, motor, gear, tools, face plate and check thoroughly to trace the fault P2. Identify the faults by rotating each circular chuck and handles of the lathe machine P3. Dismantle the machine part and fix the problem and assemble the part again and then check the correctness |
| CU3: Calibrate Drilling Machine | P1. Determine the desired seeding rate using the current lot of seed P2. Set the drill to the seeding rate desired P3. Jack the driving wheel of the drill up, turn 20 revolutions, catching the seed from 1 run of the seed drill in the bag or sack. Weigh sack to determine pounds of seed collected. P4. Determine strip length P5. determine the current seeding rate of the drill: |
| CU4: Calibrate Shaper/Planer Machine | P1. Perform operations on shaper and planar machines to diagnose the faults, normally experienced faults are the speed of the quick return mechanisms so it needs to be working on replacing or maintenance |
| CU5: Calibrate CNC Machine | P1. Identifying Positional Errors P2. Straightness error P3. Pitch and yaw errors P4. Roll Errors P5. Squareness errors |
| CU6: Calibrate Slotter Machine | P1. Check the slotter machine tool and fixture, and diagnose the fault if any P2. After diagnosing the fault, it needs to be repair |
| CU7: Calibrate Milling Machine | P1. Operate the milling machine and observe the operation like spindle movement, bed movement and if there is any fault in bed movement note it down. P2. Check the alignment of bed by measuring the surface-by-surface gauge. |
| CU8: Calibrate Hobbing Machine | P1. Check the operation of hobbing gear cutting machine, common type of faults are its tool and chuck alignment, carefully repair it. |
| CU9: Calibration of Grinding Machine | P1. Check the drill chuck (spindle), is it loose of tight. P2. Vary the spindle speed P3. Change the bush if required |
| CU10: Calibrate Arc Welding Plant | P1. Connect the cable clamp electrode to the positive terminal of the welding machine to be tested P2. Connect the ground lead plate test and the negative terminal of the welding machine P3. Connect the welding machine; P4. Set the lever in the desired position amp P5. Engage with the volt-ammeter clamp the cable clamp electrode P6. Carry out the reading of the clamp volt-ampere ammeter, comparing it with the values shown on the recorder panel welding machine. P6. If there is some problem in working then check the electronic card if it needed to be replaced then replace it and then start the plant operation. |

Knowledge & Understanding

- Complete knowledge of circular and power hacksaws

- Had a complete knowledge of all types of lathe machines, turret lathe, capstan lathe, copy lathe,

- Using the current seed lot, determine the pounds per acre of bulk seed that is needed.

- Bulk seed of current lot needed (lbs/ac) = Desired rate of seed in PLS (lbs/ac) % PLS of current seed lot

- Circumference is the measurement around the outside of the tire in feet. This is easily accomplished by measuring the circumference of the wheel with a tape measure.

- Strip length (in shop) = 1.1(No. of revolutions X wheel circumference (ft)

- Seeding rate in bulk seed (lbs/ac) = 43560 ft2 /ac X lbs seed collected Drill width (ft) X Strip Length (ft)

- The job is rigidly fixed on the machine table. The single point cutting tool held properly in the tool post is mounted on a reciprocating ram. The reciprocating motion of the ram is obtained by a quick return motion mechanism. As the ram reciprocates, the tool cuts the material during its forward stroke. During return, there is no cutting action and this stroke is called the idle stroke. The forward and return strokes constitute one operating cycle of the shaper

- Laser interferometer, Step gauge, and Gauge blocks, end bars, ball arrays.

- Straightness interferometer, Mechanical and optical straight edge,

- Alignment telescope with target, Alignment laser. Displacement indicator or sensors, taut wire

- Differential interferometer, Mechanical and Electronic level, Autocollimator, Measurement of positional error along lines with different Abbe’s offset. Angular laser interferometer.

- Electronic levels, Reference Flat, Measurement of straightness errors of two parallel lines

- Optical and Mechanical Squareness standard, Length standard inclined under defined angles. Diagonal measurements.

- Cutting grooves, keyways and slots of various shapes.

- Used for making regular and irregular surfaces both internal and external.

- For handling large and awkward workpiece.

- For cutting internal or external gears and many other operations which cannot be easily machined in any other machine tool described before.

- Knowledge of milling cutters, gang milling, surface finishing.

- Knowledge of horizontal milling machine and vertical milling machine.

- Knowledge of vertical and horizontal gear hobbing machine

- Hand-cranked knife-sharpening stones (grindstones)

- Handheld power tools such as angle grinders and die grinders.

- Various kinds of expensive industrial machine tools called grinding machines.

- Bench grinders

- Complete knowledge of arc welding, safety equipment etc.

Tool &Equipment

- Mechanical Tool Kit

- Lathe Machines

- Drilling machines

- Shaper Machine

- Planar Machine

- CNC Milling Machine

- CNC Lathe Machine

- Tool Kit Electrical

- Laser

- Tachometer

- Slotter Machine

- Vertical Milling

- Horizontal Milling

- Vertical Gear Hobbing machine

- Horizontal Gear Hobbing machine

- Bench grinders

- Hand grinders

- Grinding tools

- Arc Welding Plant

### 0715.12.3. Calibrate Mechanical Equipment

Overview: This competency standard covers the skills and knowledge required to calibrate universal testing machine, for compression strength, bend strength, shear strength of material using universal testing machine, impact Testing Machine Equipment using Charpy tester and Calibrate hardness testing machine (Rockwell hardness tester).

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Calibrate universal testing machine up to standards | P1. Check all the dimensions/positions of the chucks of Tensile, Compressive and bending as per standards. P2. If there is any error, diagnose it and read carefully the manual and P3. Prepare the machine for testing. P4. Check the machine as per default readings. |
| CU2- Calibrate impact Testing Machine Equipment using Charpy tester | P1. Calibrate the impact testing machine according to standards and defaults using manufacturer manuals and data. P2. Check the machine after calibration |
| CU3- Calibrate hardness testing machine (Rockwell hardness tester) | P1. Calibrate the hardness testing machine according to defaults manufacturing. P2. Read the manuals and dismantle the machine. P3. Assemble the machine and check the readings according to manuals. |

Knowledge & Understanding

- Material properties of common engineering materials/machines.

- Operation and maintenance of Universal testing machine.

- ASTM A 370

- ASTM E-23 18 Standard test methods for notched bar impact testing of metallic materials

- Basic knowledge of Hardness testing machine:

- Rockwell hardness tester

Tools & Equipment

- Universal Testing Machine

- Vernier Calliper

- Steel rule

- Charpy Impact Tester

- Tongs

- Holder for sample cantering

- Rockwell Hardness Tester.

- Spirit Leveller

## 0715.13. Mechanical Engineering Drawing

### 0715.13.1. Perform Basic Manual Drawing

Overview: This competency standard covers the skills and knowledge required to Draw single stroke capital vertical lettering, Draw single stroke capital inclined lettering, Draw horizontal, vertical and inclined lines, Use of compass, circles, half circles, radius, Drawing Center   lines, centers, curves, and crossing of lines, Construction of parallel-lines, perpendicular, bisects line, angles and equal division of lines, Draw round corners, circles elements, quadrilaterals inside and outside circle and Construction of angles and triangles.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw single stroke capital vertical lettering. | - Prepare Drawing sheet. - Select the tools. - Use Proper pencil for lettering with holding techniques. - Draw Boundaries lines as per standards. - Make title bar - Draw upper and lower lines for lettering according to standards. - Start Writing Vertical Lettering with different style like Gothic, Roman and free hand lettering. |
| - Draw single stroke capital inclined lettering. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Draw upper and lower lines for lettering according to standards. - Start Writing inclined Lettering with different style like Gothic, Roman and free hand lettering. |
| - Draw horizontal, vertical and inclined lines. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw lines at 30, 45, 60,90and 120 angles. |
| - Draw circles, half circles, radius with compass, | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia meters circles and half circles. |
| - Draw Lines | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Center lines, - Draw parallel-lines, - Draw perpendicular & bisects line, - Draw equal division of lines - Make different angle curves. - Draw crossing line |
| - Draw round corners, circles elements, quadrilaterals inside and outside circle. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia circles. - Make inside and outside different types of diagrams that touch the circles at the tangent points. |
| - Construct angles and triangles | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Equilateral Triangle, Isosceles triangle, Scalene Triangle, Right Triangle, Obtuse Triangle, Acute Triangle. |

Knowledge & Understanding

- Importance of Technical Drawing.

- Language of engineering terminology.

- Uses of Technical Drawing

- Type of Drawing

- Application of Technical drawing

- Drawing Pencil, their grading, sharpening and using techniques.

- Style of letters.

- General rules for lettering

- Basic lines

- Importance of lines

- Common Types of lines and correct line weightage.

- Application of lines.

- Introduction to geometry

- Introduction to sketching techniques.

- Introduction to geometry

- Introduction to sketching techniques.

- Techniques of sketching straight lines in different directions.

- Define Triangles, Quadrilateral, and Polygons.

Tool & Equipment

- Graph and drawing sheet.

- Drawing Board/Table.

- T-Square

- Set Square.

- Templets.

- Geometry Box.

- Tee-square and compass.

### 0715.13.2. Construct different Engineering Curves.

Overview: This competency standard covers the skills and knowledge required to Construct inscribe and circumscribe figures, Construct a pentagon, Hexagon and Octagon by circumscribe method, Construct a pentagon, Hexagon and Octagon by inscribe method, Construct a Tangents of circles (Inside & Outside) When the centre of the given circle is known and when the circle of centre is not known, Construct an Ellipse by Concentric Circle Method, Rectangle Method, Oblong Method, Arcs of Circle Method, Rhombus Method and Basic Locus Method, Construct a parabola curve by Rectangle Method, Method of Tangents( Triangle Method) and Basic Locus Method, Construct a hyperbola curve, Construct a Archimedean Spiral curve, Construct a involutes curve of square rectangle hexagon and circle and Construct of cycloid, epicycloids, and hypocycloid.

| Competency Units | Performance Criteria |
| --- | --- |
| - Construct inscribe and circumscribe figures. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw square, triangle and hexagon according to dimension. |
| - Construct a pentagon, Hexagon and Octagon by circumscribe method. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon . |
| - Construct pentagon, Hexagon and Octagon by inscribe method | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon. |
| - Construct Tangents of circles (Inside & Outside) | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Tangents Inside of a circle When the centre of the circle is known. - Draw Tangents Inside of a circle When the centre of the circle is unknown - Draw Tangents outside of a circle When the centre of the circle is known - Draw Tangents outside of a circle When the centre of the circle is unknown |
| - Construct Ellipse | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw an Ellipse by Concentric Circle. - Draw an Ellipse by Rectangle Method - Draw an Ellipse by Oblong Method - Draw an Ellipse by Arcs of Circle Method - Draw an Ellipse by Rhombus Method. - Draw an Ellipse by Basic Locus Method |
| - Construct a parabola curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a parabola curve by Rectangle - Draw a parabola curve by Method of Tangents (Triangle Method) - Draw a parabola curve by Basic Locus Method |
| - Construct a hyperbola curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a hyperbola curve. |
| - Construct a Archimedean Spiral curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw spiral curve. |
| - Construct involute curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw involute curve by square - Draw involute curve by rectangle - Draw involute curve by hexagon - Draw involute curve by circle. |
| - Construct of cycloid, epicycloid, and hypocycloid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw the generating circle and the base line equal to the circumference of the generating circle - Divide the circle and the base line in to equal number of parts - Complete the cycloid, epicycloids, and hypocycloid. |

Knowledge & Understanding

- Techniques of sketching straight lines in different directions

- Define Triangles, Quadrilateral and Polygons

- Describe circular arc using different line method

- Describe circular arc

- Types of Geometric Shape

- Two-dimensional shapes

- Three-dimensional shapes

- Types of Geometric Shape

- Regular Polyhedrons

- Methods of drawing Tangents & Normal

- Describe ellipse

- Describe different methods of sketching ellipse

- Describe parabola

- Describe different methods of parabola

- Describe hyperbola curve

- Describe different methods of hyperbola curve.

- Describe spiral curve

- Describe involute curve

- Describe cycloid

- Describe epicycloids

- Describe hypocycloid

Tool and Equipment

- Graph and drawing sheet

- Drawing Board/Table

- Tea-Square

- Set Square

- Templets

- Geometry Box

### 0715.13.3. Construct multi-view drawings

Overview: This competency standard covers the skills and knowledge required to Sketch Orthographic projection 1st angle, Sketch Orthographic projection 3rd angle, Sketch Oblique Drawing, Construct multi view drawing of Simple Bearing, Construct multi view drawing of Open Bearing, Sketch prism, Sketch cone and Draw pyramid.

| Competency Units | Performance Criteria |
| --- | --- |
| - Sketch Orthographic projection in 1st angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view |
| - Sketch Orthographic projection 3rd angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view |
| - Sketch Oblique Drawing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts - Draw the front or side view of the object. |
| - Construct multi view drawing of Simple Bearing. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of simple bearing - Draw front view of simple bearing - Draw side view of simple bearing |
| - Construct multi view drawing of Open Bearing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of open bearing - Draw front view of open bearing - Draw side view of open bearing |
| - Sketch prism | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch prism |
| - Sketch cone | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Start with a horizontal oval - draw the two sides of a triangle which meets at a common point |
| - Draw pyramid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch pyramid |

Knowledge & Understanding

K1.	Explain Orthographic projection 1st angle.

K2.	Explain Orthographic projection 3rd angle.

K3.	Explain Oblique Drawing.

K4.	Explain Multi view drawing of Simple Bearing.

K5.	Explain Prism, Cone and pyramid

Tool and Equipment

- Graph and drawing sheet.

- Drawing Board/Table.

- Tea-Square

- Set Square.

- Templets.

- Geometry Box.

### 0715.13.4. Manage display in AutoCAD software

Overview: This competency standard covers the skills and knowledge required to install software and create new file and create basic drawing.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Install software and Create New File | - Install latest software version - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits |
| CU2. Create Basic Drawings | - Select Coordinate System as per requirements - Draw a rectangle using line command - Draw an ARC - Draw a circle with given requirements - Draw a circle with 3-P touching outer corner of Equilateral Triangle - Use the Erase Command |

Knowledge & Understanding: This competency standard will provide knowledge related to

K1.	Basic Drawing Settings

K2.	Unit setting

K3.	Limit’s setting

K4.	User coordinate system Workspace setting

K5.	Object Snap Settings

K6.	Basic Commands and Concepts Angles and lines in AutoCAD.

K7.	Differentiate between absolute, relative and polar system

K8.	DIMSTYLE and MTEXT commands

K9.	HATCHING concepts in AutoCAD

K10.	Differentiate between CHAMFER and FILLET command

K11.	Types of Arrays

K12.	OFFSET, CIRCLE and ROTATE short commands

K13.	Zooming options

K14.	Tool’s palettes window

K15.	Design centre

K16.	Scale and paper sizes

K17.	Modify dimension style and text size according to paper size

K18.	Backup file

Tool & Equipment

- Computer with all accessories, AutoCAD software disk and Models

### 0715.13.5. Develop 2D drawings

Overview: This competency standard covers the skills and knowledge required to Develop and prepare 2D objects

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Develop 2D Objects | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Save AutoCAD drawing files in different file formats (DWG, PDF, and JPG). - Create 2D Objects with given measurements - Edit 2D Objects to meet set standards |
| CU2. Prepare Final Set of 2D Drawings | - Use appropriate command and tools to develop 2D Drawing - Develop 2D Drawing with given project specifications and measurements - Create title block layout as required - Plot drawing on scale according to required size and orientation |

Knowledge & Understanding

K1.	Basic Drawing Settings

K2.	Unit setting

K3.	Limit’s setting

K4.	User coordinate system Workspace setting

K5.	Object Snap Settings

K6.	Basic Commands and Concepts Angles and lines in AutoCAD.

K7.	Differentiate between absolute, relative and polar system

K8.	DIMSTYLE and MTEXT commands

K9.	HATCHING concepts in AutoCAD

K10.	Differentiate between CHAMFER and FILLET command

K11.	Types of Array

K12.	OFFSET, CIRCLE and ROTATE short commands

K13.	Zooming options

K14.	Tools palettes window

K15.	Design center

K16.	Scale and paper sizes

K17.	Modify dimension style and text size according to paper size

K18.	Backup file

Tool and Equipment

- Computer with all accessories

- AutoCAD software disk

- Models

### 0715.13.6. Develop 3D drawing

Overview: This competency standard covers the skills and knowledge required to develop 3D objects, manipulate and Edit 3D objects and render 3D objects.

| Competency Units | Performance Criteria |
| --- | --- |
| C1. Develop 3D Objects | P1. Setup & save 3D drawing interface for required specifications. P2. Setup 3D user interface settings for required specifications. P3. Create 3D objects with given measurements. |
| C2. Manipulate 3D objects using 3D Editing Tools | P1. Modify 3D objects in line with the requirements. P2. Make customized 3D models according to the requirement of given job. P3. Convert 3D Face objects into a single mesh object. |
| C3. Render 3D Model | P1. Apply material to required 3D Model as per given specification P2. Apply lights to get the requisite scene of required 3D model P3. Assign cameras to execute different views of required 3D Model. P4. Render and print the 3D model according to required size & orientation. P5. Apply texture to 3D model as per given specification. |

Knowledge & Understanding

K1. 3D modelling in AutoCAD

- 3D solids,

- surfaces,

- meshes, and

- Wireframe objects.

- Differentiate between Surface Modelling and Solid Modelling.

K2. 3D face and Edges

- Boolean operation concepts

- Subtraction

- Intersection

- Union

K3. 3D Navigate control

- Functions of different camera settings.

- Importance of scene creation

- Pre-set views such as isometric, top, bottom, front, left, etc.

- Perspective projection and parallel projection

- Walk

- Constrained Orbit

K4. Material and light control

K5. Planner mapping

K6. Texture map

K7. Opacity control

K8. Render context

K9. Render sampling

Tool and Equipment

- Computer with all accessories

- AutoCAD software disk

- Models

## 0715.14. Hydraulics and Hydraulic Machinery

### 0715.14.1. Operate Hydraulic Bench and Its Functions

Overview: This competency standard covers the skills and knowledge required to provide knowledge for observing Hydraulic Bench, identify various standard parts of Hydraulic Bench and its main functions.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1: Identify parts of hydraulic bench | P-1. Enlist all parts of a Hydraulic Bench P-2. Identify Control valve, Drain valve and Dump valve. P-3. Identify Sump Tank and Volumetric Tank. P-4. Identify Open Channel and Vertical Pipe. P-5. Identify Centrifugal Pump P-6. Identify Actuators |
| CU-2: Identify/perform Various Functions of/on Hydraulic Bench | P-1. Ensure that pump is switched off. P-2. Set up the device to be tested. P-3. Attach a water supply for open channel flow or a close conduct device to the water inlet. P-4. Ensure that all connections are secure. P-5. Ensure that supply valve is closed, and dump valve is open. P-6. Turn the pump on. P-7. Open slowly the supply valve to allow water to circulate the bench. P-8. Make appropriate measurement in the device being tested for a given setting of supply valve e.g. pressure reading, water depths etc. P-9. Closedown the supply valve upon the completion of test. P-10. Turn off the valve. P-11. Disconnect the device used for testing. |

Knowledge and Understanding

- Describe the purpose of a Hydraulic Bench

- Explain the working of a hydraulic bench with help of circuit diagram.

- Describe the importance of Centrifugal pump in a Hydraulic Bench.

- Describe the function of Main Tank and of Sump Tank.

- Explain the purpose and working of Control valve, Drain Valve and Dump Valve.

- Describe centrifugal pump

- Differentiate between sump tank and volumetric tank

- Differentiate control valve, drain valve and dump valve

- Differentiate main channel and side channels

- Describe water level indicator

- Describe stilling baffle

- Differentiate between overflow and starter

- Read a Schematic Diagram of Hydraulic Bench

Apparatus

Hydraulic bench

•	Control Valve

•	Centrifugal pump

•	Sump tank

•	Vertical pipe

•	Channel

•	Drain valve

•	Volumetric tank

•	Water Level Indicator

•	Dump valve

•	Actuators

•	Side Channel

### 0715.14.2. Calibrate Bourdon Tube and Diaphragm Pressure Gauge

Overview: This competency standard covers the skills and knowledge required to record the working principle of mechanical Pressure gauges and incorporate the Calibration Methods for Mechanical Gauges.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1: Caliberate Bourdon Tube using dead wight Pressure gauge and master gauge Caliberator | P-1. Connect Bourdon Tube with the dead weight pressure gauge. P-2. Fill the cylinder of dead weight pressure gauge with water. P-3. Place a known weight on the plunger of the dead weight pressure gauge. P-4. Note the pressure reading of the gauge to be calibrated. P-5. Add more weights of known values and note corresponding pressure readings. P-6. Compare these readings with the values in the table. P-7. Also compare dead weight gauge with the master gauge. |
| CU-2: Caliberate Diaphragm Pressure Gauge using dead wight pressure gauge and master gauge Caliberator | P-1. Connect the Diaphragm Pressure Gauge with the dead weight pressure gauge. P-2. Fill the cylinder of dead weight pressure gauge with water. P-3. Place a known weight on the plunger of the dead weight pressure gauge. P-4. Note the pressure reading of the gauge to be calibrated. P-5. Add more weights of known values and note corresponding pressure readings. P-6. Compare these readings with the values in the table. P-7. Also compare dead weight gauge with the master gauge. |

Knowledge & Understanding

- Define a pressure gauge

- Describe basic types of pressure gauges

- Describe Bourdon tube gauge

- Differentiate between measuring gauge and calibration gauge

- Describe dead weight pressure gauge

- Describe Pascal’s Law

- Describe Diaphragm pressure gauge.

- Describe master gauge calibrator

- Describe calibration Method and Frequency

- Describe Pascal’s Law

Apparatus

- Deadweight gauge Tester

- Weights

- Master Gauge Calibrator

- Bourdon Tube

- Deadweight gauge Tester

- Weights

- Master Gauge Calibrator

- Diaphragm Pressure Gauge

### 0715.14.3. Operate Hydraulic Press

Overview: This competency standard covers the skills and knowledge required to operate a hydraulic press and observe power required to drive it.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1: Operate hydraulic press | P-1. Prepare the work piece as per size requirement. P-2. Inspect the press for oil level and other necessary elements. P-3. Turn on the hydraulic press and select the load according to the thickness of the material. P-4. Place the work piece between the die and bench. P-5. Exert a force on the work piece with the help of punch. This would prepare the work piece to fit in the die. P-6. Turn off the hydraulic press and note all observations and calculate power required to drive the hydraulic press. |
| CU-2: Obseve the power requirement to drive a Hydraulic press | P-1. Prepare the work piece as per size requirement. P-2. Inspect the press for oil level and other necessary elements. P-3. Turn on the hydraulic press and select the load according to the thickness of the material. P-4. Place the work piece between the die and bench. P-5. Exert a force on the work piece with the using the punch of the press. This would prepare the work piece to fit in the die. P-6. Turn off the hydraulic press and note all observations. P-7. Enter values of Area of Ram, Area of Plunger, length of the stroke of the Plunger, loaded arm and weight lifted by the arm in the Formula to calculate the power required. |

Knowledge and Understanding

- Define a hydraulic press

- Describe construction of a hydraulic press

- Describe working principle of hydraulic press.

- Describe Pascal’s Law.

- Enlist all the parameters involved in Formula.

- Explain Mechanical Advantage of a machine.

Apparatus

- Plunger

- Ram

- Chamber and Weight

### 0715.14.4. Verify Bernoulli’s Equation

Overview: This competency standard covers the skills and knowledge required to verify Velocity Head, Pressure head and Datum Head as well as Law of Conservation of Energy by measuring pressure and velocity at various points.

| Competency Unit | Performance Measure |
| --- | --- |
| CU-1: Verification of Velocity Head, Pressure Head and Datum Head | P-1. Open inlet valve of hydraulic bench supply tank to allow the water to flow out of the tank. P-2. Adjust the valves to obtain continuous flow at inlet and outlet valve. P-3. Ensure continuous pressure head in the piezometer tube and note the reading. P-4. Determine the water quantity collected in the measuring tank during the time noted by stop watch. P-5. Determine the cross-sectional area of piezometer tube. P-6. Calculate the velocity head and pressure head. P-7. Change input and output supply and note the readings. |
| CU-2: Observe the performance of friction pipe apparatus | P-1. Select a pipe of suitable size as per available equipment for the experiment. P-2. Start the flow of water in the pipe with the help of hydraulic bench. P-3. Observe the change in mercury level of U-tube manometer due to water flow in the pipe. P-4. Note h1 and h2. P-5. Note the time from stop watch to fill the water in the tank up to the height of 5 cm. P-6. Change the flow and repeat the procedure. P-7. Calculate coefficient of discharge. |
| CU-3: Examine loss of head due to sudden enlargement, Contraction and entrance in a pipe | P-1. Note the length and diameter of the pipe present between hydraulic bench and pressure tank. P-2. Keep the pressure tank, end of the, pipe open and the other end closed. P3. Turn on the power supply of the hydraulic bench. P-4. Start the flow water in the circuit. P-5. Adjust the flow up to maximum level. P-6. Determine the total discharge in the circuit with the help of left and right limbs of U-tube manometer. P-7. Keep the pressure tank, end of the, pipe close and the other end opened. P-8. Repeat the process. |

Knowledge and Understanding

- Describe Bernoulli’s theorem

- Describe piezometer tube

- Define velocity head

- Define pressure head

- Define Datum head

- Describe friction pipe apparatus

- Describe effects of friction on fluid flow

- Describe Differential type U-tube Manometer

- Define coefficient of discharge

- Explain Darey’s friction factor

- Describe loss of head

- Describe co-efficient of loss

- Describe G.I. Pipes

Apparatus

- Hydraulic Bench

- Tapered inclined pipe fitted with number of piezometer tube points

- Measuring tank

- Scale

- Stop watch

- Pipe provided with inlet and outlet and pressure tapping

- Differential U-tube manometer

- Piezometer tube

- Flow circuit of G.I. Pipes of different fittings via large bend, small bend

- Sudden enlargement from 25 mm to 50 mm diameter

- Sudden contraction from 50 mm to 25 mm diameter.

### 0715.14.5. Analyse the performance of Turbines

Overview: This competency standard covers the skills and knowledge required to observe and analyse impulse turbine as well as Reaction turbine.

| Competency Units | Performance measures |
| --- | --- |
| CU-1: Analyze the performance of impulse turbine/ Pelton Wheel | P-1. Prime the pump with water P-2. Keep the nozzle-opening to the required position. P-3. Open the gate valve 1 or 2 rotations. P-4. Start the motor P-5. Allow the water into the turbine and the turbine will start rotating. P-6. Fix the weigh hanger to the rope of the brake drum with no load on weight hanger. P-7. By varying the gate vale, keep the head constant using the pressure gauge to the required head in case of experiment on constant head or keep the speed constant using the tachometer to the required speed in case of experiment on constant speed. P-8. Note down the following readings: - Pressure gauge reading G - Vacuum gauge reading V - Speed of the turbine N - Manometer readings h1, and h2 - Load on weight hanger T1 - Spring balance reading indicating the frictional loss between the brake drum and rope T2 P-9. Repeat last step for different load conditions by varying the load on the weight hanger either to constant head or for constant speed P-10. Note down the above readings G, V, N, h1 and h2, T, T2 P-11. Take at least 5 sets of readings by varying the load P-12. Calculate the efficiency of the turbine |
| CU-2: Analyze the performance of Reaction Turbine/ Francis Turbine | P-1. Prime the pump with water P-2. Keep the nozzle-opening to the required position. P-3. Open the gate valve 1 or 2 rotations. P-4. Start the motor P-5. Allow the water into the turbine and the turbine will start rotating. P-6. Fix the weigh hanger to the rope of the brake drum with no load on weight hanger. P-7. By varying the gate vale, keep the head constant using the pressure gauge to the required head in case of experiment on constant head or keep the speed constant using the tachometer to the required speed in case of experiment on constant speed. P-8. Note down the following readings: - Pressure gauge reading G - Vacuum gauge reading V - Speed of the turbine N - Manometer reading h1, and h2 - Load on weight hanger T1 - Spring balance reading indicating the frictional loss between the brake drum and rope T2 P-9. Repeat last step for different load conditions by varying the load on the weight hanger either to constant head or for constant speed P-10. Note down the above readings G, V, N, h1 and h2, T, T2 P-11. Take at least 5 sets of readings by varying the load P-12. Calculate the efficiency of the turbine |

Knowledge and Understanding

- Describe working principle of Impulse Turbine

- Describe Pelton wheel construction

- Describe breaking jet

- Describe nozzle function

- Describe runner and bucket

- Describe working principle of Reaction Turbine

- Describe spiral casing purpose.

- Describe function of guide mechanism.

- Describe draft tube

Apparatus

- Hydraulic Bench

- Impulse Turbine (Pelton Wheel)

- Electric Circuit Supply

- Reaction Turbine (Francis Turbine)

- Generator assembly and Electric Circuit

### 0715.14.6. Analyse the performance of Pumps

Overview: This competency standard covers the skills and knowledge required to observe and analyse centrifugal pump as well as reciprocating pump.

| Competency Units | Performance |
| --- | --- |
| CU-1: Analyze the performance of Centrifugal Pump | P-1. Prime the pump P-2. Open the gate valve 1 or 2 rotations P-3. Start the motor and set the vacuum gauge reading to the required head P-4. Note down the following readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x P-5. Set the vacuum gauge reading to the other heads P-6. Note down the above readings G, V, T, and t P-7. Take at least 5 sets of readings by varying the head through delivery valve and note down the readings |
| CU-2: Analyse the performance of Reciprocating Pump | P-1. Prime the pump P-2. Open the gate valve 1 or 2 rotations P-3. Start the motor and set the vacuum gauge reading to the required head P-4. Note down the following readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x P-5. And then set the vacuum gauge reading to the other heads P-6. Note down the above readings G, V, T, and t P-7. Take at least 5 sets of readings by varying the head through delivery valve and note down the readings. |

Knowledge and Understanding

- Describe centrifugal pump working principle

- Describe pump casing

- Describe impeller

- Describe suction pipe and delivery pipe

- Describe reciprocating pump

- Describe suction valve and delivery valve

- Describe crank and connecting rod mechanism

- Describe suction and delivery pipe

- Describe function of Piston

- Differentiate between centrifugal and reciprocating pump.

Apparatus

- Centrifugal Pump

- Test Bench

- Pipe Hoses

- Voltmeter

- Ammeter

- Reciprocating Pump

- Tachometer

- Stop watch

- Pressure gauge

- Energy meter

### 0715.14.7. Actuate Double Acting Hydraulic Cylinder

Overview: This competency standard covers the skills and knowledge required to actuate a double acting hydraulic cylinder by a switch, speed regulation through throttle valve and flow control valves. It also explains actuation of double acting hydraulic cylinder by one-way throttle valve.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1: Actuate double acting hydraulic cylinder by switch and draw a circuit diagram | P-1. Complete all connections of hydraulic circuit as per drawing P-2. Turn on the push button switch to start the supply of fluid with which double acting cylinder will start moving P-3. Move the piston to initiate the supply from forward stroke P-4. Turn on the second push button switch after the piston reaches top dead center. This will start supply in the opposite direction P-5. Observe the functions from control valve by varying its speed after the completion of supply in the circuit P-6. Note all observations. |
| CU-2: Actuate double acting hydraulic cylinder by oneway throttle valve | P-1. Prepare the hydraulic circuit as per drawing P-2. Turn on the circuit power supply and ensure that all accessories are working properly P-3. Ensure throttle valve on return line of the circuit works properly. P-4. Open the valve and ensure extra supply of oil to observe quick return of cylinder. P-5. Observe cylinder speed at various positions of the valve. P-6. Note all the observations |

Knowledge and Understanding

- Describe types of hydraulic cylinder

- Describe Double acting hydraulic circuit

- Describe direction control valve

- Describe function of push button

- Describe throttle valve function

- Describe one way throttle valve

- Explain role of cylinder speed

Apparatus

- Hydraulic bench

- Double acting cylinder

- Pipe hoses and fittings

- Pressure gauges

- Flow control valve

- Direction control valve

- Push button

### 0715.14.8. Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder

Overview: This competency standard covers the skills and knowledge required to use pressure reducing valve on double acting cylinder and holding load on double acting cylinder.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1: Set-up pressure device on a double acting cylinder using pressure reducing valve | P-1. Complete the hydraulic circuit as drawing given P-2. Turn on the circuit supply and check if all devices are working properly P-3. Ensure Fluid starts flowing and the initially adjusted pressure is passed on to the circuit while passing through the relief valve. P-4. Observe and note the pressure reading at various pressure gauges. P-5. Change direction with the help of DCV and ensure control of double acting cylinder according to this direction. P-6. Re-observe and re-note the readings on all the pressure gauges. |
| CU-2: Hold specific load using double acting cylinder | P-1. Complete the hydraulic circuit as per drawings. P-2. Turn on the circuit supply and check if all devices are working properly P-3. Adjust pressure according to desired/required load. P-4. Ensure fluid flow from port A. This would uplift the load. P-5. Now change the position of DCV and ensure fluid entrance from port B. P-6. Observe the downward motion of loaded ram. P-7. Take the pressure reading and observe the cylinder return. |

Knowledge and Understanding

- Describe pressure reducing valve

- Describe purpose of double acting cylinder as a pressure device

- Describe applications of double acting cylinder with the pressure pipe

- Describe function of pilot operated check valve

- Describe DCV

- Describe the role of RAM

Apparatus

- Hydraulic Bench

- Double ACTING Cylinder

- Pressure reading valve

- DCV pipe hoses

- Pilot operated check valve

- Direction control valve

- Various loads

- Pipe hoses

- Pressure gauges

### 0715.14.9. Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking

Overview: This competency standard covers the skills and knowledge required to construct a hydraulic cylinder for mechanical interlocking and use of push buttons, limit switch and stored energy in double acting hydraulic cylinder

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1: Construct a hydraulic circuit for double acting hydraulic cyclinder for mechanical interlocking with switch | P-1. Complete the hydraulic circuit as per drawings. P-2. Turn on the circuit supply and check if all devices are working properly P-3. Operate hydraulic circuit P-4. Try lift some weight P-5. Drop the pressure using pressure gauge before the piston starts moving P-6. Construct Hydraulic Circuit Diagram P-7. Construct electric circuit diagram |
| CU-2: Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch | P-1. Prepare simulation circuit of double acting cylinder as per drawing P-2. Fix the push button and limit switch at their designated place as per drawing P-3. Turn on the circuit power supply and ensure proper working of all equipment. P-4. Press the push button to start fluid supply and will enable the cylinder for forward stroke supply. P-5. Observe that as the push rod touches limit switch the supply of coil would disconnect and the piston would stop. P-6. Press the second push button to complete backward stroke of piston, this will enable the supply of side coil and the backward stroke of piston would be completed and as it enters the surroundings of limit switches the supply would be disconnected. P-7. Change 5/2 DCV and complete the opposite circuit. P-8. Observe this changed circuit and note down all the observations. |
| CU-3: Construct hydraulic circuit using Accumulator stored Energy by DAC | P-1. Complete the hydraulic circuit as per drawing. P-2. Turn on the motor power supply and ensure proper working of all devises and equipment. P-3. Use flow control valve to ensure fluid flow. P-4. Observe the movement of piston rod. P-5. When the circuit is burnt/completed the turn off the power supply. P-6. Observe that due to accumulator the double acting cylinder completes its stroke and doesn’t stop immediately after the burning of circuit. |
| CU4: Measure pressure at various connections in hydraulic circuits | P-1. prepare the hydraulic circuit as per drawing P-2. install pressure gauges on the points where pressure is to be noted P-3. Turn on the hydraulic circuit P-4. Ensure fluid flow through the whole circuit P-5. Complete the circuit and note the readings of all gauges P-6. Enter all readings in observation table |

Knowledge and Understanding

- Describe mechanical interlocking function

- Describe the function of hydraulic circuit

- Describe double acting hydraulic cylinder

- Describe 3/2 push buttons

- Describe 5/2 double acting side pilot operated DCV

- Describe function of connecting tubes

- Describe function   of limit switches

- Describe the role of accumulator

- Describe mechanism of energy storage in accumulator

- Describe working of hydraulic accumulator

- Describe fluid port

- Describe construction of hydraulic actuator

- Describe function of actuators

- Describe coupling

- Describe functioning of chillers

- Describe proximity switch or limit switch

- Describe purpose of hydraulic circuit

- Draw analogy between hydraulic and electrical circuit

- Describe function of pressure control valve

- Describe flow control valve

Apparatus

- Hydraulic bench

- Mechanical interlocking

- Double acting cylinder

- Fluid control valve

- DC pressure Gauges

- Double acting cylinder

- 3/2 push button

- 5/2 double acting side pilot operated DCV

- Contacting tubes

- Limit switch

- Pressure gauges

- Accumulator Flow Control

- DCV

- Pipe Hoses

- Pressure control valve

- Flow control valve

- Hydraulic pump hoses

### 0715.14.10. Set Speed and Direction of Hydraulic Motor

Overview: This competency standard covers the skills and knowledge required to set-up a hydraulic motor using flow control valve and setting-up direction using direction control valve.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1: Set hydraulic motor R.P.M using flow control valve | P-1. Complete the hydraulic circuit as per drawing. P-2. Turn on the motor power supply and ensure proper working of all devises and equipment. P-3. Fix the tachometer with the rotor to note motor RPM P-4. Use flow control valve to control the flow, this would also control the motor RPM. P-5. Observe the speed during this process. P-6. Note RPM at different flow rates and analyze them. |
| CU-2: Set hydraulic motor direction by using direction control valve | P-1. Complete the hydraulic circuit as per drawing. P-2. Turn on the motor power supply and ensure proper working of all devises and equipment. P-3. Use DCV to rotor the motor Clockwise and Anti Clockwise. P-4. Observe the direction during this process. P-5. Repeat the process. |

Knowledge and Understanding

- Describe hydraulic motor

- Describe operating principle of hydraulic motor

- Describe tachometer

- Describe flow control valve

- Describe DCV

Apparatus

- Hydraulic bench

- Hydraulic motors

- Tachometer

- Flow control valve

- Direction control valve

- Pipe hoses

- Pressure gauges

### 0715.14.11. Perform Pressure Measuring Instruments

Overview: This competency standard covers the skills and knowledge required to identify, and explain the construction and working of various pressure measuring devices.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1: U Tube Manometer | P-1. Set up the equipment. Connect one leg of manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. P-2. Observe the difference in head between the two legs of the manometer and record. P-3. Compute the pressure difference between the pipe inlet and outlet. P-4. Repeat the experiment for various flow rates of fluid through the test pipe. |
| CU-2: Inclined manometer | P-5. Set up the equipment. Connect one leg of inclined manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. P-6. Observe the difference in head between the two legs of the manometer and record. P-7. Compute the pressure difference between the pipe inlet and outlet. P-8. Repeat the experiment for various flow rates of fluid through the test pipe. |
| Cu-3: Micro Manometer | P-1. Observe the enlarged ends of both tubes of the manometer P-2. Adjust the instrument initially as p1 = p2. P-3. Note the reading of the micrometer P-4. Apply the unknown pressure difference P-5. Observe the meniscus to move off the hairline, which can be restored to the initial position by raising or lowering the well (mercury sump). P-6. Note the difference in the initial and final micrometer readings P-7. Note the height of the mercury column and hence the pressure. |
| CU-4: Pressure measurement in The Mercury Barometer | P-1. Take a reservoir and fill with any fluid P-2. Put a glass tube in the reservoir in inverted state P-3. Make the top portion of glass tube as air tight while the internal surface as complete vacuum P-4. When the glass tube is dipped in the tank, an improper balanced pressure is created P-5. Observe water from the tank rises in the tube P-6. Note that the water would rise to particular height/ limit P-7. Note that point where the water would stop lifting up P-8. This pressure noted directly from the scale of the tube would give us the exact atmospheric/ air pressure |
| CU-5: Borden Tube | P-1. Observe the device consisting of a metallic tube of elliptical section closed at one end A P-2. Note the other end B being fitted to the gauge point where the pressure is to be measured. P-3. Enter the fluid in the tube P-4. Observe the tube tending to straighten. P-5. Observe a pinion-sector arrangement pivoted with the pointer needle and already calibrated markings P-6. Note the small elastic deformation of the tube is communicated to a pointer over a graduated dial. P-7. Calibrate the device by subjecting it to various known pressures. |
| CU-6: Piezo Metric Tube | P-1. Close all the valves of the tubes manifold. P-2. Fill the tank until the lower part of the piezo metric tube is in contact with water. P-3. Connect the pump to the upper plug of the tank with the non-return valve in the correct direction and pressurize the tank. P-4. Observe if the water head in the piezo metric tube rises. P-5. Observe the level is kept constant when we stop to pressurize with the pump. |
| CU-7: Elastic Diaphragms | P-1. Observe the diaphragm fixed in a tubular member. P-2. Observe the initial pressure on a diaphragm P-3. Apply the unknown pressure from one side. P-4. Observe a direct connection between diaphragm and pinion-sector P-5. Observe a pinion-sector arrangement pivoted with the pointer needle and already graduated markings on the dial P-6. Note the small elastic deformation of the diaphragm is communicated to a pointer over the graduated dial. |

Knowledge and Understanding

- Differentiate between absolute pressure and gauge pressure

- Differentiate between u tube inclined and differential manometer

- Pressures as low as 0.025mm water column can be measured.

- What is a barometer

- What is the basic function of a barometer?

- Explain the selection process of a pressure gauge

- Write the types of Bourdon tubes

- Explain the purpose of different Bourdon tubes

- Explain the formula for calculating a static head

- Convert a pressure gauge into a level gauge

- mention the units of Pressure

- Explain the necessity of putting the plug in the tank

- Explain natural frequency

Apparatus

- U Tube Manometer, Hydraulic Bench (if working fluid is water), Air Compressor (if working fluid is air), Scale

- Inclined Manometer, Hydraulic Bench (if working fluid is water), Air Compressor (if working fluid is air), Scale

### 0715.14.12. Operate Flow Regulating Devices

Overview: This competency standard covers the skills and knowledge required to identify, evaluate and explain different types of valves.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1: Ball Valve | P-1. Identify various parts of the ball valve including Handle, shaft, Ball, Seats and valve body P-1. Observe a ball with a hole drilled through the center swivel mounted within the valve body P-2. Ensure the hole in the ball is orientated in the same direction as the pipe to allow flow P-3. Observe the hole in the ball is oriented away from the direction of the pipe, the flowrate will be restricted and finally cut off completely when the hole is oriented at 90 degrees to the pipe direction Note that the hole in the ball is a lesser diameter than the nominal bore of the pipe |
| CU-2: Butterfly Valve | P-1. Identify the gear box, handle, butterfly and the body of the butterfly valve P-1. Observe a circular disc in the middle of fluid pipe. P-2. Ensure the butterfly is oriented in the same direction as the pipe to allow full flow. P-3. Oriented the butterfly away from the direction of the pipe, the flowrate will be restricted by the increased area of obstruction to the fluid P-4. Keep the butterfly orienting away from the fluid until 90 degrees to completely restrict the flow |
| CU-3: Gate valve | P-1. Identify gate and body of the gate valve P-1. Identify the gate in the valve P-2. Identify the shape of the gate with respect to the pipe diameter P-3. Identify the direction of movement of the gate P-4. Ensure the complete upward motion of the gate for full flow of the fluid P-5. Ensure compete downward motion of the gate until the opposite end for complete restriction to the fluid flow. |
| CU-4: Diaphragm Valve | P-1. Identify bonnet, diaphragm/ flexible sheet, seat and main body of the valve P-2. Identify whether the valve is manual or pneumatic actuated P-3. observe the fluid flow rates from the pipe as the diaphragm is pushed towards the seat for complete obstruction to the flow |
| CU-5: Non-Return Valve | P-1. Identify the following parts in a non-return/ check valve: body, cover plate, disc, seats, hinge, hinge pin, nut, cotter pin, stud bolt, gasket, washer P-2. observe the fluid flow as the disk is released P-3. observe the disk being fixed as the flow direction is reversed |
| CU-6: Pressure Relief Valve | P-1. Identify the following parts in pressure relief valve: set pressure adjusting screw, spring, disk holder, seat disk, nozzle, bonnet, bonnet plug and body P-2. identify various types of pressure relief valve P-3. observe system pressure and spring pressure P-4. observe the valve opening as the system pressure increases then spring pressure |

Knowledge and Understanding

- Define a valve

- Identify applications of Ball Valve

- Explain the safety precautions of ball valve

- explain the terminology of ‘butterfly valve’

- identify its industrial applications

- Explain the safety precautions of Butterfly Valve

- describe Gate/ Knife gate valve

- explain the isolation valves

- explain the precaution for knife gate valve

- Identify its Industrial Applications

- Explain the safety precautions of Gate valve

- Compare diaphragm and gate valve

- Explain the industrial applications of diaphragm valve

- Explain the safety precautions of Diaphragm Valve

- Mention and explain various types of non-return valve

- Explain unidirectional valve working mechanism

- Identify various industrial applications of non-return valve

- Explain the safety precautions of Non-Return Valve

- Explain the safety aspects of pressure relief valve

- Describe industrial applications of Pressure relief valve

- Explain the safety precautions of Pressure Relief Valve

Apparatus

- Ball Valve

- Butterfly Valve

- Gate Valve

- Diaphragm Valve

- Non-Return Valve

- Pressure Relief Valve

## 0715.15. Applied Thermodynamics

### 0715.15.1. Solve problems related to fundamentals of thermodynamics

Overview: The competency standard covers knowledge and skills to solve problems of fundamentals of thermodynamics, heat, mass & weight, force, work done and power. Develop skill, mathematical attitudes and logical perception in the use of mathematical instruments as required in the mechanical field.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1 Perform conversion of Temperature scales | - Measure temperature with appropriate tool - Convert the given temperature to required scale. - Solve given problems with suitable formula. |
| CU-2 Covert the vacuum and pressure gauge into each other | P1. Measure the vacuum with vacuum gauge on engine. P2. Measure the pressure with the pressure gauge in different scales. P3. Perform the conversation of vacuum and pressure in different scales. P4. Solve the problems of mathematical attitudes and logical perception in the use of mathematical formulas |
| CU-3 Solve the problems of Different energies and thermodynamics laws | P1. Solve the problems of following energies: - Potential Energy - Kinetic Energy - Thermal Energy - Chemical Energy P2. Solve problems related to law of thermodynamics - Zeroth law of thermodynamics - 1st law of thermodynamics - 2nd law of thermodynamics |
| CU-4 Derive the basic gas laws of thermodynamics | P1. Drive the following laws: - Boyles Law - Charles Law - Joules Law P2. Solve the mathematical problems applying the laws |

Knowledge & Understanding

- Basic Concept of thermodynamics

- Laws of thermodynamics

- Different types of energies

- Gas laws of thermodynamics

Tool & Equipment

Stationary Items

Measuring tools and instruments for temperature

Measuring tools and instruments for vacuum and pressure gauges

### 0715.15.2. Solve problems of gas Laws and properties of perfect gases

Overview: The competency standard covers knowledge and skills to solve problems related to fundamentals of thermodynamics, laws and properties of gases.

| Competency Units | Performance Criteria |
| --- | --- |
| - Apply Gas Laws in problems | P1. Derive the following mathematical relations: - General gas equation - Characteristic Gas equation - Universal Gas equation P2. Identify the specific heats of a gas and derive its mathematical relations. |
| - Derive the mathematical equation of enthalpy and entropy | - Derive the mathematical equation of Enthalpy of a Gas. - Derive the mathematical equation of Entropy of a Gas. |

Understanding and Knowledge:

- Perfect gas and its property

- Derive the mathematical relations.

- Boyle’s law.

- Charles’s law

- Joule’s law

- Derive the mathematical relations.

- General gas equation

- Characteristic Gas equation

- Universal Gas equation

- Describe the following.

- The two specific heats of a gas and derive its mathematical relations.

- Entropy of a gas

- Enthalpy of a Gas

Tool & Equipment

- Stationary Items

### 0715.15.3. Derive Thermodynamics Processes and Cycles

Overview: The competency standard covers knowledge and skills to prepare the properties of steam, steam boilers and their performance, steam and Gas turbines, I.C. Engines, Air compressors and their problems.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1 Draw the different Cycles of thermodynamics | - Sketch CARNOT CYCLE with the help of required diagram - Derive mathematical relations of Carnot cycle for Air Standard Efficiency during the cycle of operation. - Sketch OTTO CYCLE with the help of required diagram - Derive mathematical relations of Otto cycle for Air Standard Efficiency during the cycle of operation - Sketch diesel cycle with the help of required diagram - Derive mathematical relations of diesel cycle for Air Standard Efficiency during the cycle of operation. - Sketch dual combustion cycle with the help of required diagram - Derive its mathematical relations for Air Standard Efficiency during the cycle of operation. |
| CU-2 Derive and solve the mathematically problems of I.C engines | - Solve problems of following - Torque, and its unit in SI system. - Mean effective pressure. - Indicated power and its formula. - Brake Horse power and its formula. - Measurement of Brake Horse power - Solve problems of following - Friction Horse power. - Mechanical efficiency. - Thermal efficiency. - Volumetric Efficiency |
| CU-3 Evaluate the working of Rankine cycle | - Sketch cycle with the help of required diagram - Derive mathematical relations Rankine cycle for Air Standard Efficiency during the cycle of operation. - Solve problems related to Rankine cycle |
| CU-4 Evaluate the working of Brayton cycle | - Sketch cycle with the help of required diagram - Derive mathematical relations Brayton cycle for Air Standard Efficiency during the cycle of operation. - Solve problems related to Brayton cycle |

Knowledge and Understanding

K.1 Explain CARNOT CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation.

K.2 Explain OTTO CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation.

K.3 Explain DIESEL CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation.

K.4 Explain DUAL COMBUSTION CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation.

K.5 Evaluate the working of Rankine cycle

K.6 Evaluate the working of Brayton cycle

Tools and Equipment

- Stationery items

# (0715-M&MT-3) National Qualification Certificate of level-4, Part-2 in “Mechanical Technology”

## 0715.16. HVAC

### Install Residential Air Conditioner

Overview:  This Competency Standard identifies the competencies required to install different types of Window type / Split type residential Air conditioners skills at workplace in accordance with the organization’s / client’s guidelines. This unit covers the knowledge regarding safety rules, Personal Protective Equipment (PPE), and international standards for installing Residential Air conditioners to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU 1: Install Window Air-Conditioner | P1. Select tools, equipment and related accessories according to job requirements P2. Mark the location on the wall where Window Air Conditioner to be installed according to Unit specifications and client requirements P3. Make opening at the marked area on the wall P4. Fix Iron / wooden frame in the opening firmly and insert in it the Air Conditioner cover according to the instructional manual and standards P5. Install the Air conditioner in the framed opening with standard slope so that condensate water drops outside P6. Cover / Seal side air gaps of opening with insulation material P7. Fix the fancy wooden border around the Air conditioner grill as per client’s requirement P8. Fix the Air Conditioner condensate drain pipe and put it into main sewerage line P9. Arrange power supply with circuit breaker near the Air Conditioner P10. Make sure that all packing materials - Cardboard, Styrofoam, Tape and Plastic Film, have been removed from the site after installation P11. Switch on the Air Conditioner and check Air Conditioner performance as per capacity and specifications |
| CU 2: Install Split Air Conditioner | P1. Select tools, equipment and related accessories according to job requirements P2. Select and mark the area on the wall where Indoor and Outdoor units are to be installed according to P3. Perform physical inspection of indoor and outdoor unit according to unit specifications P4. Make opening for the refrigerant pipes, condensate pipe and control wires to pass through P5. Mount the Indoor unit wall mounting plate according to manufacturer specifications and install the Indoor unit on it P6. Prepare base for the fixing of Outdoor unit according to manufacturer specifications and fix the Outdoor unit there P7. Connect the refrigerant pipes amongst both indoor and outdoor units, supply and control wires according to manufacturer manual P8. Add additional refrigerant for additional piping according to manufacturer recommendations P9. Make oil trap in copper pipe as per site requirement P10. Perform leak test, evacuation procedure, charge refrigerant and open the service valves P11. Insulate the joints and refrigerant pipes according to standards and manufacturer installation manual P12. Make sure that all packing materials - Cardboard, Styrofoam, Tape and Plastic film, have been removed P13. Switch on the Air Conditioner and check performance as per capacity and specifications |

Knowledge & Understanding

- Basic load calculation for cooling / heating

- Fundamental knowledge of HVACR, Electric and Electronics

- Techniques for installation of Window / Split (DC Inverter)

- Technical Operations of all types of Split Air Conditioners

- Electrical / HVAC layout plans/wiring diagrams

- Type of Electrical wires and cables including underground cables, their ratings and applications

- Methods of Copper Tube Cutting / Reaming / Bending / Swaging / Flaring / Brazing / Jointing / Fixing

- Basic masonry and carpentry applications

- Gas welding (Soldering and Brazing)

- Types of insulation and their applications

- Compressor types and applications

- Methods of Pressurizing / Evacuations / Purging / Refrigerant Charging

- Types of Refrigerants, its properties, recycling, recovery and reclaiming

Tools and Equipment

- Windows Air-conditioning Unit to be installed.

- Air-conditioning Kit

- Gas Cylinder

- Safety kit

- Split Air-conditioning Unit to be installed.

- Air-conditioning Kit

- Gas Cylinder

- Safety kit

- Copper Pipe

- Hammers

- Chisel

- Teflon Tape

- Tachometer

- Multi meter

- Drill Machine

- Wire Tape

- Heavy Wire

### Repair/Service Residential Air-Conditioner

Overview: This Competency Standard covers the competencies required to diagnose / repair / service residential Air Conditioners at workplace in accordance with the manufacturer specifications / guidelines. This unit covers the knowledge regarding safety rules, Personal Protective Equipment, and international standards for repairing / servicing of Residential Air Conditioners to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1: Diagnose Faults In Residential Air Conditioner | P1. Check for obvious problem to determine which component or system is causing the problem P2. Select tools, equipment and related accessories according to requirements and standards P3. Check power supply, electric wiring, electric / electronic components and refrigerant pressure to determine the exact problem by using AVO meter / Gauge manifold / flow chart as recommended by manufacturer, and record the results P4. Eliminate the causes of the problem according to the manufacturer manual and standards P5. Isolate and recheck the causes of the problem and rectify the fault P6. Start the Air Conditioner and recheck the unit as specified in the manufacturer manual and record the results |
| CU 2: Repair Window/Split Air Conditioner | P1. Select tools, equipment and related accessories according to job requirements P2. Disconnect the Air Conditioner from electric supply and follow the manual instructions for rectification P3. Rectify the diagnosed faults; repair / replace the components, as necessary P4. Switch on the Air Conditioner to check the performance of electrical/ electronic and mechanical components as specified in the manufacturer manual and record the results |
| CU 3: Service Window Air Conditioner | P1. Select tools, equipment and related accessories according to job requirements P2. Start the Air Conditioner, check and record performance by using specified test instruments P3. Disconnect the Air Conditioner from electric supply |
|  | and remove Air Conditioner from the cover P4. Secure the electric / electronic components with polythene sheet P5. Clean / wash all mechanical parts of Window Air conditioner with specified cleaning agents / detergent by using pressure pump P6. Fix the Air conditioner in the cover, connect with electric supply, check and record performance |
| CU 4: Service Split Air Conditioner | P1. Select tools, equipment and related accessories according to job requirements P2. Start the Air Conditioner, check and record performance by using specified test instruments P3. Pump down the split Air Conditioner and dismantle both indoor unit and condensing unit P4. Clean the components of Air conditioner with specified cleaning agents/tools and material P5. Re-Install the indoor and outdoor unit; connect with refrigerant pipes, control wires and evacuation; open the service valves and leak testing P6. Switch on the Air Conditioner, check and record performance |

Knowledge & Understanding

- Knowledge of American Society of Heating, Refrigeration, And Air Conditioning Engineers (ASHRAE)

- Knowledge of Ozone Protection and Synthetic Greenhouse Gas Management Act 1989

- Basic load calculation for cooling /heating

- Fundamental knowledge of HVACR, Electric and Electronics

- Techniques for repairing of Window /Split Air Conditioners

- Technical Operations of Split Air Conditioners

- Electrical / copper piping layout plans/wiring diagrams

- Type of Electrical wires and cables including underground cables, their ratings and applications

- Techniques of diagnosing and troubleshooting of Residential Air Conditioners

- Familiarity with Residential Air Conditioner error codes and solution

- Types of motors used in Residential Air Conditioners

- Types of lubricants and their properties

- Compressor types / Specifications and applications

- Methods of Copper Tube cutting / Bending/Swaging / Flaring /Brazing / Jointing /fixing

- Gas welding (Soldering and Brazing)

- Types of Insulation and their applications

- Methods of Pressurizing / Evacuation /Purging / Refrigerant Charging

- Types of Refrigerants, its properties, recovery and reclaiming

- Read all the relevant data regarding the unit very carefully and then start servicing.

Tools and Equipment

- Personal Protective Equipment

- Allen Key Set

- Bench Vice

- Center Punch

- File Set

- Hand Hacksaw Frame with Blade

- Metal Drill Bit Set

- Hammer Set

- Mallet Set

- Diagonal Side Cutter

- Combination Plier

- Nose Plier Set

- Locking Plier

- Rivet Gun / Plier

- Adjustable Screw Wrench

- Adjustable Pipe Wrench

- Ratchet Wrench

- Box Spanner Set

- Socket Set

- Open Ended Spanner Set

- Box Spanner Screw Drivers

- Steel Ruler

- Scissors

- Scriber

- Tap and Die Set

- Try Square

- Chisel Set

- Pulley Wheel Puller

- Gas Welding Set with All Accessories

- Nitrogen Gas Cylinder with Hose Pipe, Regulator and Back Arrestor

- Pipe Cutter

- Pipe Vice

- Manual Screw Driver Set

- Electric Screw Driver Set

- Electric Hand Grinder

- Soldering Iron

- Crimping Tool

- Insulation Remover

- Wire Stripper

- Digital Multi Meter

- Digital Clamp-On Ampere Meter

- Electric Hand Drills

- Hot Air Gun

- Digital Capacitor Analyzer

- Hand Electric Blower

- Digital Humidity Meter

- Digital Psychrometric (Hygrometer)

- Tube Cutter

- Digital Optical Tachometer

- Micron Pressure Gauge

- Digital Pressure Gauges Set (High &Combine)

- Pinch-Off Plier

- Flaring and Swaging Tool Kit

- Vacuum Pump 2-Stage, 6cfm

- Tube Benders (Spring Type and Pulley Bender Type)

- ohm meter (0 - 1000 Volts)

- Laser Temperature Measuring Device

- Electronic Leak Detector

- Water Pressure Gun for Service

- Digital Air Flow / Velocity Meter

- Portable Refrigerant Charging Station

### Repair / Service Residential Refrigeration

Overview: This Competency Standard covers the competencies required to diagnose / repair / service residential refrigeration units at workplace in accordance with the manufacturer specifications / guidelines. This unit covers the knowledge regarding safety rules, Personal Protective Equipment, and international standards for repairing / servicing of residential refrigeration units to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU 1: Diagnose Faults in Residential Refrigeration Units | P1. Check for obvious problem to determine which component or system is causing the problem P2. Select tools, equipment and related accessories according to requirements and standards P3. Check power supply, electric wiring, electric / electronic components and refrigerant pressure to determine the exact problem by using AVO Meter / Gauge manifold as recommended by manufacturer and record the results P4. Eliminate the causes of the problem according to the manufacturer manual and standards P5. Isolate and recheck the causes of the problem and rectify the fault P6. Start the refrigeration unit and recheck as specified in the manufacturer manual and record the results |
| CU 2: Repair / Service Residential Refrigerator / Freezer | P1. Select tools, equipment and related accessories according to job requirements P2. Disconnect the Refrigerator / Freezer from electric supply and follow the instructions in manufacture manual for rectification P3. Rectify the diagnosed faults; repair / replace the components, as necessary P4. Check, wash and restore to the actual condition Refrigerator / Freezer Body / Cabinets P5. Check, service, and replace, if necessary, the proper functioning of Thermostat / Door Gasket / Heaters P6. Switch on the Refrigerator / Freezer to check the performance of electrical/ electronic and mechanical components as specified in the manufacturer manual and record the results |
| CU 3: Repair/ Service Residential Electric Water Cooler / Water Dispenser | P1. Select tools, equipment and related accessories according to job requirements P2. Disconnect the water cooler / dispenser from electric supply and follow the manual instructions for rectification P3. Rectify the diagnosed faults; repair / replace the components, as necessary P4. Check, wash and restore to actual condition Water Cooler / Dispenser Body / Mounts P5. Switch on water cooler / dispenser to check the performance of electrical/ electronic and mechanical components as specified in the manufacturer manual and record the results |

Knowledge & Understanding

- Knowledge of Ozone Protection and Synthetic Greenhouse Gas Management Act 1989

- Fundamental knowledge of HVACR, Electric and Electronics

- Techniques for repairing of Refrigerators /Freezers / Water Coolers and Water Dispensers

- Technical Operations of Non-Frost refrigerators

- Types of electrical wires and cables, their ratings and applications

- Techniques for Diagnosing and Troubleshooting of Residential Refrigerators /Freezers / Water Coolers and Water Dispensers

- Types of Lubricants and their properties

- Capable to replace PCB Card

- Compressor types/ Specifications and applications

- Methods of Copper Tube Cutting /Bending /Swaging / Flaring / Brazing /Jointing / Fixing

- Gas welding (Soldering and Brazing)

- Methods of Pressurizing/ Evacuation /Purging / Refrigerant Charging

- Types of Refrigerant, its properties, recovery and reclaiming

- Record keeping and reporting

- Capable to replace PCB Card

Tools & Equipment

Air-Conditioning Kit

### Carry out Calculations and Prepare Estimates

Overview:  This competency standard identifies the competencies required to prepare estimates/cooling load calculations in accordance with client’s guidelines. You will be expected to estimate, ensuring cost effectiveness, conforming to standards and regulations. Your underpinning knowledge regarding estimation skills will be sufficient to provide the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU 1: Develop Basic Drawing/Cooling Load Calculation | P1. Calculate the cooling load for conditioned space P2. Calculate the electric load for circuit distribution P3. Develop drawing according to requirement P3. Negotiate any alteration to the job specifications with clients |
| CU 2: Perform Estimation of Materials | P1. Take measurements as per drawing P2. Quantify the material as per drawing P3. Calculate cost of material and accessories in accordance with current market prices P4. Make the cost estimate of material P5. Incorporate the agreed alteration in Job specifications for estimation |
| CU 3: Prepare Costing for the Work | P1. Calculate man-hours for work as per drawing P2. Make the cost estimate of labor as per requirement |

Knowledge & Understanding

- Cooling load calculations terminologies and formulas along with charts & Tables

- Basic electrical terminologies

- Electrical Circuit Characteristics

- Basic electric formulas & numeracy

- Norms in interacting & negotiating with customers/clients

- Interpretation of layout plans/wiring diagrams, service manuals and manufacturer specifications, technical Sketches, graphic symbols etc.

- Types of electrical control and protective switchgear and accessories used in electrical circuits

- Types of electrical wires and cables and their ratings

- Types of electrical conduits, casing & capping etc.; their applications and their cutting/jointing/fixing methods

- Types of insulation material used in electrical installations

- Types of earth electrodes and their applications

- Method of calculating labor Cost/overheads/profit margin etc.

- Norms and standard formats of preparing estimates/BOQ

- Make Rate analysis according to market Rates.

- Record keeping and reporting

### Install Commercial Refrigeration Units

Overview: This Competency Standard covers the competencies required to install different types and sizes of walk-in coolers / freezers / ice making machines / chilled water tanks at work place in accordance with the organization guidelines. This unit covers the knowledge regarding safety rules, personal protective equipment, and international standards for installing Commercial refrigeration units to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU 1: Install Walk in Cooler / Freezer | P1. Select tools, equipment and related accessories according to job requirements P2. Prepare insulated room for preserving the food on lowest temperature as per drawing and requirements P3. Prepare steel / concrete foundation / frame for installation of condensing unit following manufacturer specifications P4. Prepare and level the place to fix the evaporator and condensing unit firmly, according to manufacturer specifications P5. Layout piping and control wiring from indoor to outdoor unit according to instructional manual P6. Perform leak test, evacuation and charge the refrigerant according to unit specifications and standards P7. Connect the electric supply and operate the unit to check the performance according to unit specifications |
| CU 2: Install Ice Making Machine | P1. Select tools, equipment and related accessories according to job requirements P2. Fix the machine on potable water supply by following manufacturer specifications P3. Measure the clearance on each side to be sure it meets the standards set by the manufacturer P4. Make water drain connections in order to empty purged and melt water as per manual instructions and client requirements P5. Install shut of valve on water supply near the machine according to unit specifications P6. Make electric supply switched on and check performance according to machine specification by using specific instruments |
| CU 3: Install Chilled Water Tank | P1. Select tools, equipment and related accessories according to job requirements P2. Fix the cooler / tank on potable water supply by following manufacturer specifications and client requirements P3. Measure the clearance on each side to be sure it meets the standards set by the manufacturer P4. Make water drain connections adjacent to water supply as per instructions manual P5. Install shut off valve and no return valve at water supply line P6. Fix minimum water level protection and interlocking with refrigeration unit to prevent empty freezing P7. Make power supply as per manual instructions |

Knowledge & Understanding

K1. Knowledge of Sheet Metal and Air Conditioning Contractors National Association (SMACNA)

K2. Ozone Protection and Synthetic Greenhouse Gas Management Act 1989

K3. Methods of Pipe Cutting /Welding / Jointing / Fixing /Sealing

K4. Preparation of hanging supportive structures for piping

K5. Types of insulation and their application

K6. Techniques for installation of Pipes by using specified test instruments

K7. Types of electrical wires and cables, including underground cables, their ratings and applications

K8. Techniques for installation of refrigeration units

Tools and Equipment

- Personal Protective Equipment

- Allen Key Set

- Bench Vice

- Center Punch

- Pedestal Drill Machine

- Digital Vernier Caliper

- Metal Drill Bit Set

- Masonry Drill Set

- File Set

- Hand Hacksaw Frame

- Hammer Set

- Mallet Set

- Diagonal Side Cutter

- Combination Plier

- Nose Plier Set

- Locking Plier

- Measuring Tape

- Rivet Gun / Plier

- Adjustable Screw Wrench

- Adjustable Pipe Wrench

- Nitrogen Gas Cylinder with Hose Pipe, Regulator and Back Arrestor

- Pipe Cutter

- Pipe Vice

- Bearing Puller

- Manual Screw Driver Set

- Electric Screw Driver Set

- Spirit Level

- Electric Hand Grinder

- Wire Stripper

- Digital Multi Meter

- Digital Clamp-On Ampere Meter

- Electric Hand Drills

- Hot Air Gun

- HILTI Drill Machine (Piston Type)

- Swaging Tool Kit

- Vacuum Pump 2-Stage, 6cfm

- Tube Benders (Spring Type and Pulley Bender Type)

- Laser Distance Measuring Device

- Electronic Leak Detector

- Manometer

- Digital Air Flow / Velocity Meter

- Portable Refrigerant Charging Station

- Fins Straightening Comb Set

### Repair/Service Commercial Refrigeration

Overview: This Competency Standard covers the competencies required to diagnose / repair / service residential refrigeration units at workplace in accordance with the manufacturer specifications / guidelines. This unit covers the knowledge regarding safety rules, Personal Protective Equipment, and international standards for repairing / servicing of refrigeration units to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU 1: Diagnose Faults in Commercial Refrigeration Units | P1. Check for obvious problem to determine which component or system is causing problem P2. Select tools, equipment and related accessories according to requirements and standards P3. Check the power supply, electric wiring, electric / electronic components and refrigerant pressure to determine the exact problem by using flow chart as recommended by manufacturer and record the results P4. Eliminate the causes of the problem according to the manufacturer manual and standards P5. Isolate and recheck the causes of the problem and correct the fault P6. Start the refrigeration unit and recheck the unit as specified in the manufacturer manual and record the results |
| CU 2: Repair Commercial Refrigeration Units | P1. Select tools, equipment and related accessories according to requirements and standards P2. Disconnect the unit from electric supply and follow the manual instructions for rectification P3. Rectify the diagnosed fault by repairing / replacing the components P4. Switch on the unit to check the performance of electrical/ electronic and mechanical components as specified in the manufacturer manual and record the results |

Knowledge & Understanding

- Techniques for Diagnosing and Troubleshooting of Residential Refrigeration Units

- Familiarity with Refrigeration Units error codes and solution

- Types of Lubricants and their properties

- Types of Refrigerants and its properties

- Methods of Pressurizing/ Evacuation / Purging / Refrigerant Charging

- Repair leak in refrigeration unit

- Replace the compressor

- Replace the thermostat

- Repair replace PCB Card

Tools and Equipment

- Digital Vernier Caliper

- Metal Drill Bit Set

- Masonry Drill Set

- File Set

- Hand Hacksaw Frame

- Hammer Set

- Mallet Set

- Digital Micro Meter

- Diagonal Side Cutter

- Combination Plier

- Nose Plier Set

- Locking Plier

- Measuring Tape

- Rivet Gun / Plier

- Adjustable Screw Wrench

- Adjustable Pipe Wrench

- Ratchet Wrench

- Box Spanner Set

- Socket Set

- Open Ended Spanner Set

- Box Spanner Screw Drivers

- Steel Ruler

- Scissors

- Scriber

- Tap and Die Set

- Try Square

- Chisel Set

- Pulley Wheel Puller

- Gas Welding Set with All Accessories

- Nitrogen Gas Cylinder with Hose Pipe, Regulator and Back Arrestor

- Pipe Cutter

- Pipe Vice

- Bearing Puller

- Manual Screw Driver Set

- Electric Screw Driver Set

- Spirit Level

- Electric Hand Grinder

- De-Soldering Tool

- Lcr Meter

- Soldering Iron

- Crimping Tool

- Insulation Remover

- Wire Stripper

- Digital Multi Meter

- Digital Clamp-On Ampere Meter

- Electric Hand Drills

- Hot Air Gun

- Digital Optical Tachometer

- Meg ohmmeter (0 - 1000 Volts)

- Water Pressure Gun for Service

- Digital Capacitor Analyzer

- Hand Electric Blower

- Tube Cutter

- Micron Pressure Gauge

- Digital Pressure Gauges Set (High & Combine)

- Pinch-Off Plier

- Flaring and Swaging Tool Kit

- Vacuum Pump 2-Stage, 6cfm

- Tube Benders (Spring Type and Pulley Bender Type)

- Laser Temperature Measuring Device

- Electronic Leak Detector

- Manometer

- Digital Air Flow / Velocity Meter

- Portable Refrigerant Charging Station

- Fins Straightening Comb Set

### Carry out Fabrication for HVAC Work

Overview: This Competency Standard identifies the competencies required to fabricate different types of Ducts and Pipes at workplace in accordance with the international HVAC standards. Your underpinning knowledge regarding safety rules, Personal Protective Equipment, and international standards for fabricating and installing of Duct and Piping will be sufficient to provide the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Fabricate Ducts | P1. Select tools, machines, equipment, sheet materials and related accessories according to requirements and standards P2. Take measurements, mark proper location and select Duct fittings as specified in the layout plan/ diagram P3. Prepare different types of Edges / Joints / Seams / Bends and Notches used to join Sheet Metal work P4. Prepare flexible connections between Duct sections and unit fittings (Air Handling Units / Fan Coil Units) P5. Prepare duct in required size, shape and pieces and connect with each other according to Duct route drawing P6. Seal Duct joints with good mastic material, to avoid air leakage, according to SMACNA /ASHRAE standards P7. Fabricate cladding to protect insulation |
| CU2. Fabricate Pipes | P1. Select tools, machines, equipment, pipe materials and related accessories according to requirements and standards P2. Prepare joints (Elbow / Reducer / Tees / Offset / Couplings / Flanges etc.) for jointing of Pipes as per layout plan / requirements P3. Prepare and clean Pipe joints for smooth Welding / Threading work according to layout plan / diagram P4. Carry out Jointing using Welding or Threading methods to ensure proper fitting according to international standards |
| CU3. Install Ducts and Piping | P1. Follow International standards of ASHRAE or SMACNA while installing ducts and piping P2. Select tools, equipment, Pipe materials and related accessories according to requirements and standards P3. Mark location for Duct and Pipes installations according to drawing specifications P4. Mount Hangers, Angle rails, threaded rods / Studs and Straps according to standards P5. Install the Ducts and Pipes on angle rails and ensure the straps are fabricated into one unit according to layout plan/ diagram P6. Seal all the joints with good mastic material to avoid leakage according to standards P7. Install Grills/Diffusers/Dampers and Flexible Duct according to drawing / layout plan P8. Install the valves in pipes as per drawing P9. Perform leak testing of piping and duct system as per standards |
| CU4. Insulate Piping Ducts | P1. Select tools, equipment, pipe materials and related accessories according to requirements and standards P2. Prepare insulation material / Sound liner according to manufacturer specifications P3. Insulate water / steam pipes with specified insulation according to ASHRAE or SMACNA standards P4. Apply vapor barrier paper, cotton cloth wrapping and sheet metal cladding firmly on the ducts / pipes according to manufacturer specifications |

Knowledge & Understanding

- Knowledge of Sheet Metal& Air Conditioning Contractors’ National Association (SMACNA)

- Layout plans, drawing and manufacturer specifications, and HVAC symbols etc.

- Working at heights and platforms

- Use and transport of Ladders / Scaffolds safely

- Type of Ducts, Duct fittings and their applications

- Type of Pipes, Pipe fittings and their applications

- Preparation of hanging supportive structures for Ducts and Piping

- Fabricate Duct accessories

- Fabricate Pipe accessories

- Perform welding / Brazing on Pipe joints

- Insulate the Ducts / Pipes

- Perform Leakage test of Ducts and Pipes in accordance with standard HVAC procedures

- Types of Insulation and their application

- Techniques for installation of Ducts and Pipes by using specified test instruments

- Procedures of pressure testing of Pipes by using specified test instruments

- Procedures of leakage testing in Ducts by using light source

- Record keeping and reporting

Tools and Equipment

- Personal protective equipment

- Working tables

- Sheet metal tools

- Bench vice

- Pedestal drill machine

- Digital Vernier caliper

- Allen key set

- Center punch

- Metal Drill bit set

- Masonry drill set

- File set

- Hand Hacksaw frame

- Electric hand grinder

- Hammer set

- Mallet set

- Diagonal side cutter

- Combination plier

- Nose plier set

- Locking plier

- Measuring tape

- Rivet gun / plier

- Adjustable screw wrench

- Adjustable pipe wrench

- Ratchet wrench

- Box spanner set

- Socket set

- Open ended spanner set

- Box spanner screw drivers

- Steel ruler

- Scriber

- Tap and Die set

- Try square

- Chisel set

- Tin Cutter

- Gas welding set with all accessories

- Scissors

- Pipe cutter

- Pipe vice

- Electric hand drills

- Manual Screw driver set

- Electric Screw driver set

- Spirit level

- HILTI drill machine (Piston type)

- Laser distance measuring device

- Tube cutter

### Install Central Air Conditioning Systems

Overview: This Competency Standard identifies the competencies required to install Package type unit / VRF / VRV / Absorption Chiller System at workplace in accordance with the organization / client guidelines under the supervision of HVAC Engineer. This unit covers the knowledge regarding safety rules, Personal Protective Equipment, and international standards for the installation of central air conditioning system to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Install Package Unit | P1. Select tools, equipment and related accessories according to job requirements P2. Mark the location and area according to layout plan and manufacturer specifications P3. Prepare foundation as per drawing, place, adjust and level the Package unit by giving attention to safety precautions P4. Connect Package unit with supply, return and fresh air duct through flexible connection as per drawing / unit specification P5. Insulate duct where required P6. Remove all packaging material from work place P7. Connect the power supply and control wire P8. Switch on the supply and check the performance according to manufacturer instructions and standards |
| CU2. Install Variable Refrigerant Flow (VRF) / Variable Refrigerant Volume (VRV) System | P1. Select tools, equipment and related accessories according to job requirements P2. Prepare foundation as per drawing, place, adjust and level the outdoor unit by giving attention to safety precautions P3. Prepare piping, weld / braze them according to unit specifications and layout drawings P4. Install indoor units according to layout diagrams, client requirements and manufacturer instruction manual P5. Fix Shut-off valves with service ports on every indoor unit P6. Check leaks by applying pressure method before the joint’s insulation; repair leaks, if any P7. Insulate the copper pipe joints according to manufacturer instructions and standards P8. Evacuate the system and charge the refrigerant as per requirement P9. Connect the power supply and control wires attached with central control / Building Management (BMS) system P10. Switch on the system and check performance |
| CU3. Install Water Chiller System | P1. Select tools, equipment and related accessories according to job requirements P2. Mark the location and area according to layout plan P3. Prepare foundation as per drawing, place, adjust and level the chiller by giving attention to safety precautions P4. Install Air Handling Units (AHU) / Fan Coil Unit (FCU) at different locations inside the building according to drawing P5. Install high pressure MS Pipe from chiller to Air Handling Units/ Fan Coil Unit inside the building P6. Fabricate and install G.I sheet ducting inside the building as per drawing P7. Install Cooling Tower and water pumps (Only for water cooling condenser) P8. Establish the electric power supply system for the chiller and air handling units and cooling tower as per Requirements and Standards |
| CU4. Carry out Commissioning | P1. Select tools, equipment and related accessories according to job requirements P2. Start the condenser and chilled water pump for water circulation in the system P3. Start the chiller and take reading of all parameters regarding temperature / pressure and electric power, check unusual vibrations / noises etc., if any P4. Perform Air Balancing and water balancing to check the air / water distribution according to design requirements P5. Check the system performance as per design criteria |

Knowledge & Understanding

- Techniques for installation of central Air conditioners

- Fundamental and Technical Operations of VRF / VRV system

- Electrical / HVAC layout plans/wiring diagrams

- Types of electrical control wires, cables, including underground cables, their ratings and its applications

- Methods of Copper Tube Cutting / Bending /Swaging / Flaring / Brazing / Y-Jointing / Fixing

- Basic Masonry and Carpentry applications

- Types of Insulation and their applications

- Types of AHU, FCU, Chillers and Cooling Towers, Compressor, Condenser and pump types and their applications

- Types of Refrigerants, its properties, recovery and reclaiming

- Procedure regarding commissioning the central HVAC systems

- Record keeping and reporting

- Mark location according to specifications and standards

- Perform electric connections to joint indoor unit and outdoor unit

- Install the indoor / outdoor unit according to HVAC standards

- Balancing the system (Air / Water)

Tools and Equipment

- Personal Protective Equipment

- Allen Key Set

- Bench Vice

- Center Punch

- Pedestal Drill Machine

- Digital Vernier Caliper

- Metal Drill Bit Set

- Masonry Drill Set

- File Set

- Hand Hacksaw Frame

- Hammer Set

- Mallet Set

- Digital Micro Meter

- Diagonal Side Cutter

- Combination Plier

- Nose Plier Set

- Locking Plier

- Measuring Tape

- Rivet Gun / Plier

- Adjustable Screw Wrench

- Adjustable Pipe Wrench

- Ratchet Wrench

- Box Spanner Set

- Socket Set

- Open Ended Spanner Set

- Box Spanner Screw Drivers

- Steel Ruler

- Scissors

- Scriber

- Tap and Die Set

- Try Square

- Chisel Set

- Pulley Wheel Puller

- Gas Welding Set with All Accessories

- Nitrogen Gas Cylinder with Hose Pipe, Regulator and Back Arrestor

- Pipe Cutter

- Pipe Vice

- Manual Screw Driver Set

- Electric Screw Driver Set

- Spirit Level

- Electric Hand Grinder

- Soldering Iron

- Crimping Tool

- Insulation Remover

- De-Soldering Tool

- LCR Meter

- Wire Stripper

- Digital Multi Meter

- Digital Clamp-On Ampere Meter

- Electric Hand Drills

- Hot Air Gun

- HILTI Drill Machine (Piston Type)

- Digital Optical Tachometer

- Meg ohmmeter (0 - 1000 Volts)

- Digital Capacitor Analyzer

- Hand Electric Blower

- Tube Cutter

- Micron Pressure Gauge

- Digital Pressure Gauges Set (High &Combine)

- Pinch-Off Plier

- Flaring and Swaging Tool Kit

- Vacuum Pump 2-Stage, 6cfm

- Tube Benders (Spring Type and Pulley Bender Type)

- Laser Distance Measuring Device

- Laser Temperature Measuring Device

- Electronic Leak Detector

- Manometer

- Digital Air Flow / Velocity Meter

- Portable Refrigerant Charging Station

- Fins Straightening Comb Set

- Digital Humidity Meter

- Digital Psychrometric (Hygrometer)

### Repair/Service Central Air Conditioning

Overview: This Competency Standard covers the competencies required to diagnose / repair / service central air conditioning system at workplace in accordance with the manufacturer specifications / guidelines. This unit covers the knowledge regarding safety rules, Personal Protective Equipment, and international standards for repairing / servicing of central air conditioning units to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Diagnose Fault in Central Air Conditioning System | P1. Check for obvious problem to determine which component or system is causing the problem P2. Select tools, equipment and related accessories according to requirements and standards P3. Arrange drawings and manuals of the equipment to be attended for diagnosing the fault P4. Check the power supply, electric wiring, electric / electronic components and refrigerant pressure to determine the exact problem and record the results P5. Start the air conditioning unit and recheck as specified in the manufacturer manual and record the results |
| CU2. Repairing of Chillers / Air handling Unit (AHU) / Fan Coil Unit (FCU) | P1. Select tools, equipment and related accessories according to job requirements P2. System shut down and follow the instructions manual for rectification P3. Rectify the diagnosed faults; repair / replace the components, as necessary P4. Switch on the system to check the performance of electrical / mechanical components as specified in the manufacturer manual and record the results |
| CU3. Repair Pumps and Cooling Tower | P1. Select tools, equipment and related accessories according to job requirements P2. System shut down and follow the instructions manual for rectification P3. Check water level of cooling tower P4. Check the fan assembly, to check the bearings and motor abnormalities according to manufacturer specifications and HVAC standards P5. Check the float valves and strainers to maintain water level P6. Perform chemical treatment to prevent the sludge / scaling problems according to manufacturer specifications and HVAC standards P7. Condenser / chilled water pumps start and check their performance according to their specifications; service / repair, if necessary P8. Clean the sprinkler assembly water pan from leaves, mud, and scale, if any P9. Rectify the fault and restart the cooling tower and pumps |
| CU4. Repair Control System | P1. Select tools, equipment and related accessories according to job requirements P2. System shut down and follow the instructions manual for rectification P3. Arrange drawings and manuals of the equipment to be attended for diagnosing the fault P4. Check the details of fault / errors on computer screen to rectify the same P5. Eliminate the causes of the problem according to the manufacturer manual and standards P6. Isolate and recheck the causes of the problem and correct the fault P7. Start the air conditioning unit and recheck as specified in the manufacturer manual and record the results |

Knowledge & Understanding

- Technical Operations of Chillers, Cooling Tower, Pumps, AHUs & FCUs

- Techniques for Diagnosing and Troubleshooting of central Air conditioning systems

- Capability to replace the PCB Cards and controls

- Familiarity with central Air conditioners error codes and solution

- Types of Motors used in central Air conditioners

- Types of controls and their functions used in central air conditioning systems

- Capability to use and handle the micron gauge for deep vacuum measuring

- Compressor, Condenser, Evaporator types and applications

- Types of Refrigerants, its properties, recovery and reclaiming

- Repair refrigerant leaks o Replace the Condenser motor o Replace the control

- Replace water pump

- Replace motorized actuators

Tool and Equipment

	Personal Protective Equipment

	Allen Key Set

	Bench Vice

	Center Punch

	Pedestal Drill Machine

	Digital Vernier Caliper

	Metal Drill Bit Set

	Masonry Drill Set

	File Set

	Hand Hacksaw Frame

	Hand Saw

	Hammer Set

	Mallet Set

	Digital Micro Meter

	Diagonal Side Cutter

	Combination Plier

	Nose Plier Set

	Locking Plier

	Measuring Tape

	Rivet Gun / Plier

	Adjustable Screw Wrench

	Adjustable Pipe Wrench

	Ratchet Wrench

	Box Spanner Set

	Socket Set

	Open Ended Spanner Set

	Box Spanner Screw Drivers

	Steel Ruler

	Scissors

	Scriber

	Tap and Die Set

	Try Square

	Chisel Set

	Pulley Wheel Puller

	Gas Welding Set with Table and All Accessories

	Nitrogen Gas Cylinder with Hose Pipe, Regulator and Back Arrestor

o	Pipe Cutter

	Pipe Vice

	Bearing Puller

	Manual Screw Driver Set

	Electric Screw Driver Set

	Spirit Level

	Electric Hand Grinder

	Soldering Iron

	Crimping Tool

	Insulation Remover

	De-Soldering Tool

	LCR Meter

	Wire Stripper

	Digital Multi Meter

	Digital Clamp-On Ampere Meter

	Electric Hand Drills

	Hot Air Gun

	HILTI Drill Machine (Piston Type)

	Digital Optical Tachometer

	Meg ohmmeter (0 - 1000 Volts)

	Digital Capacitor Analyzer

	Hand Electric Blower

	Tube Cutter

	Micron Pressure Gauge

	Digital Pressure Gauges Set (High &Combine)

	Flaring and Swaging Tool Kit

	Vacuum Pump 2-Stage, 6cfm

	Tube Benders (Spring Type and Pulley Bender Type)

	Laser Distance Measuring Device

	Laser Temperature Measuring Device

	Electronic Leak Detector

	Manometer

	Digital Air Flow / Velocity Meter

	Portable Refrigerant Charging Station

	Fins Straightening Comb Set

	Digital Humidity Meter

	Digital Psychrometer (Hygrometer)

### Perform Preventive Maintenance

Overview: This Competency Standard covers the competencies required to calibrate and carry out maintenance of refrigeration / air conditioning system skills at workplace in accordance with the manufacturer specifications / guidelines. This unit covers the knowledge regarding safety rules, Personal Protective Equipment, and international standards for calibrating / maintenance of refrigeration / air conditioning units to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Calibrate / Replace Measuring Instruments | P1. Shut down the system if necessary and dismantle measuring instruments P2. Select and arrange tools, equipment and related accessories according to job requirements / sequence of operation P3. Calibrate / replace the instruments following the manufacturer instructions P4. Reinstall measuring instruments and start the system to check the performance of calibrated instrument as specified by the manufacturer manual P5. Record the output result of measuring instrument for future reference P6. Follow HVACR standards to complete the job |
| CU2. Carry out Maintenance | P1. Pump down / Shut down the system if necessary P2. Select and arrange tools, equipment and material according to job requirements / sequence of operation P3. Check different machines with measuring instruments for temperature, vibration and noise, etc. so as to operate the plant at desired efficiency P4. Carry out weekly / monthly / annual maintenance according to schedule P5. Check and record the performance of system after maintenance P6. Follow HVACR standards to complete the job |

Knowledge & Understanding

- Technical Operations of Control

- Purpose / Advantages of preventive maintenance

- Techniques for Diagnosing and Troubleshooting of controls

- Controls error codes and solution

- Types of controls and their functions used in Central Air Conditioning Systems

- Procedure to replace PCB cards and controls

- Methods of calibration of different measurement instruments

- Record keeping and reporting

- Calibrate air temperature pressure gauge control device o Measure airflow / water flow difference

- Provide system log sheet and recommend repairs or replacement as necessary

Tool and Equipment

- Personal Protective Equipment

- Calibrating Tool Set

- Allen Key Set

- Bench Vice

- Centre Punch

- Pedestal Drill Machine

- Digital Vernier Calliper

- Metal Drill Bit Set

- Masonry Drill Set

- File Set

- Hand Hacksaw Frame

- Hammer Set

- Mallet Set

- Digital Micro Meter

- Diagonal Side Cutter

- Combination Plier

- Nose Plier Set

- Locking Plier

- Measuring Tape

- Rivet Gun / Plier

- Adjustable Screw Wrench

- Adjustable Pipe Wrench

- Ratchet Wrench

- Box Spanner Set

- Socket Set

- Open Ended Spanner Set

- Box Spanner Screw Drivers

- Steel Ruler

- Scissors

- Scriber

- Tap and Die Set

- Try Square

- Chisel Set

- Pulley Wheel Puller

- Gas Welding Set with All Accessories

- Nitrogen Gas Cylinder with Hose Pipe, Regulator and Back Arrestor

- Pipe Cutter

- Tube Cutter

- Pipe Vice

- Bearing Puller

- Manual Screw Driver Set

- Electric Screw Driver Set

- Spirit Level

- Electric Hand Grinder

- Soldering Iron

- De-Soldering Tool

- LCR Meter

- Crimping Tool

- Insulation Remover

- Wire Stripper

- Digital Multi Meter

- Digital Clamp-On Ampere Meter

- Electric Hand Drills

- Hot Air Gun

- HILTI Drill Machine (Piston Type)

- Digital Optical Tachometer

- Meg ohmmeter (0 - 1000 Volts)

- Digital Capacitor Analyser

- Hand Electric Blower

- Micron Pressure Gauge

- Digital Pressure Gauges Set (High &Combine)

- Pinch-Off Plier

- Flaring and Swaging Tool Kit

- Vacuum Pump 2-Stage, 6cfm

- Tube Benders (Spring Type and Pulley Bender Type)

- Laser Distance Measuring Device

- Laser Temperature Measuring Device

- Electronic Leak Detector

- Manometer

- Digital Air Flow / Velocity Meter

- Water Pressure Gun for Service

- Digital Humidity Meter

- Digital Psychrometer (Hygrometer)

- Portable Refrigerant Charging Station

- Fins Straightening Comb Set

## 0715.17. Mill Wright/ Industrial Mechanic

### Perform Routine Trade Activity

Overview: This competency standard covers the skills and knowledge required to lubricate system and equipment, Perform levelling of components and system, Maintain fastening and retaining devices, Interpret mechanical drawing and specifications and Perform heat treatment of metal.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Lubricate system and equipment | - Identify attachments for lubrication process - Clean the surface of equipment - Select the correct lubricant for specific applications. - Maintain the safe use, storage and handling of lubricants. |
| CU2. Perform leveling of components and system | - Select and safely use fixed shop machines and equipment. - Maintain and repair fixed shop machines and equipment |
| CU3. Maintain fastening and retaining devices | - Identify, select and use fasteners and locking devices - Identify and use tools required for installation - set torque values - calculate strengths of fasteners |

Knowledge & Understanding

- Describe types of lubrication.

- Describe various lubrication system

- Describe the properties and characteristics of lubricants.

- Describe layout of equipment

### Perform Measuring and Layout of Work piece

Overview: This competency standard covers the skills and knowledge required to Prepare work area, tools and equipment, Layout and fabricate work piece and Maintains precision measuring of tools

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare work area, tools and equipment | - Identify tools and equipment as per job requirement - Select and use millwright’s hand tools. - Clean and maintain millwright’s hand tools - Select and safely use portable power tools Maintain and repair portable power tools |
| CU2. Layout and fabricate work piece | - Identify and use measuring tools and instruments. - Describe maintenance and storage procedures for measuring tools and instruments. - Identify and use layout tools and instruments - Maintain and store procedures for layout tools and instruments |
| CU3. Maintains precision measuring of tools | - Identify and describe tolerances, fits and fitting methods. - Apply fitting techniques and procedures associated with the assembly of equipment |

Knowledge & Understanding

K1.	Define terminology associated with measuring and layout

K2.	Identify types of precision measuring tools and describe their applications and procedures for use.

  - Micrometers

  - Calipers

  - Dial indicators

  - Protractors

  - Height gauges

  - Feeler gauges

  - Plug, ring and snap gauges

  - Gauge blocks

K3.	Identify types of layout tools and describe their applications and procedures for use.

  - Straight edges

  - Solid squares

  - Combination sets

  - Surface plate

  - Scribers

  - calipers

  - Dividers

  - Trammels

  - Prick and center punches

  - Angle plates

  - Surface gauges

  - n. Layout dye

Tool and Equipment

Micrometers

Calipers

Dial indicators

Protractors

Height gauges

Feeler gauges

Plug, ring and snap gauges

Gauge blocks

### Perform Cutting and Welding Operation

Overview: This competency standard covers the skills and knowledge required to perform cutting operations on material as per requirements and Perform welding operations.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform cutting operations on material as per requirements | P1. Cut piece with manual cutting operations P2. Cut material with oxy-fuel and plasma arc cutting equipment |
| CU2. Perform welding operations | - Join material using oxy-fuel welding equipment - Weld material using shielded metal arc welding (SMAW) equipment - Weld material with gas metal arc welding (GMAW) equipment - Weld material with gas tungsten arc welding (GTAW) equipment - Maintain welding equipment |

Knowledge & Understanding

K1. Describe cutting operations

K2. Describe different types of welding

### Perform Rigging, Hoisting / Lifting and Moving

Overview: This competency standard covers the skills and knowledge required to Design a rigging plan, Select and use hoisting and lifting equipment, Set and use sling and rigging attachment and Service chain and belt drive system.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Design a rigging plan | - Determines load - Selects rigging equipment - Selects hoisting/lifting and moving equipment - Secures area |
| CU2. Selects and use hoisting and lifting equipment | - Identify and describe the use of mobile access equipment. - Perform routine inspection of lifting equipment - Use ladders and scaffolding. |
| CU3. Set and use sling and rigging attachment | - Identify wire and fiber rope. - Identify the types and use rigging attachments. - Identify and use hand rigging equipment and devices. - Select, inspect and use wire rope - Select, inspect and use slings - Select, inspect and use rigging attachments - Spreader bars - Eyebolts - Hoist rings - Shackles - Hooks - End terminations - Turnbuckles - Blocks and pulleys - Maintenance and Storage - Select, inspect and use hand rigging equipment |
| CU4. Service chain and belt drive system | - Identify and select gear system - Install, repair and maintain gear system |

Knowledge & Understanding

- Describe the use of mobile access equipment.

- Describe the applicable Work Safe BC regulations pertaining to rigging equipment and procedures

- Describe the types and use wire and fiber rope

Tool and Equipment

  - Spreader bars

  - Eyebolts

  - Hoist rings

  - Shackles

  - Hooks

  - End terminations

  - Turnbuckles

  - Blocks and pulleys

  - Ladders and scaffolding.

### Service general mechanical equipment

Overview: This competency standard covers the skills and knowledge required to Service couplings, clutches and brake system, Service shaft, bearing seals and their alignment procedures ,Service gear systems , Service Pumps, Service fan and blowers, Service compressors, Service piping, tank and containers, Service hydraulic system, Service pneumatic and vacuum system, Service conveying system,  Service prime movers, Perform preventive and predictive maintenance , Perform commissioning and decommissioning of equipment and Service mechanical robot and automated equipment

| Competency Units | Performance Criteria |
| --- | --- |
| - Service couplings, clutches and brake system | - Select, install and maintain coupling system - Select, install and maintain clutch system - Select, install and maintain brake system |
| - Service shaft, bearing seals and their alignment procedures | - Select, install and maintain shaft system - Select, install and maintain bearing system - Select, install and maintain seal system - Perform rough alignment procedures - Perform dial alignment procedure - Perform laser alignment procedure - Fit, mount and dismount friction and anti-friction bearings. - Inspect and maintain bearings and diagnose causes of bearing failures |
| - Service gear systems | - Identify and select gear system - Install, repair and maintain gear system |
| CU4. Service Pumps | - Identify and select positive displacement pump - Install, repair and maintain positive displacement pump - Identify and select non-positive displacement pump - Install, repair and maintain non-positive displacement pump |
| CU5. Service fan and blowers | - Identify and select fan system - Install, repair and maintain fan system - Identify and select blower system - Install, repair and maintain blower system |
| CU6. Service compressors | - Identify and select compressor - Install, maintain and repair compressor |
| CU7. Service piping, tank and containers | - Select, install and repair tank and container system - Select, install and repair piping system |
| CU8. Service hydraulic system | - Identify hydraulic system components - Assemble hydraulic system components - Repair and maintain hydraulic system components - Test the hydraulic system |
| CU9. Service pneumatic and vacuum system | - Identify pneumatic and vacuum components - Assemble pneumatic and vacuum components - Maintain and repair pneumatic and vacuum components |
| CU10. Service conveying system | - Identify the conveying system components - Assemble conveying components as per manufacturer manual - Maintain and repair conveying system |
| CU11. Service prime movers | - Service electric motor - Service internal combustion engine - Service turbines |
| CU12.Perfrom preventive and predictive maintenance | - Identify the related maintenance type as per requirement - Perform vibration analysis procedures - Perform balancing procedures - Perform non-destructive evaluation procedures |
| CU13. Perform commissioning and decommissioning of equipment | - Identify reports and other data required for commissioning equipment - Commission the system and equipment - Recommend commissioning improvements where appropriate - Review test and trial schedules - Check equipment and systems - Fuel - Lube oil - Freedom of movement - Electricity - Test equipment - Perform test and trials - Specific performance standards - Comparison to specifications - Safety/guards/emergency stops - Maintenance arrangements - Nose levels - Defects - Identify and document required changes - Sumps - Drainage - Vibration - Pipe hangers - Insulation - Couplings - Report warranty and defects - Test results examination - Defects - Deficiencies - Decommission the system and equipment |
| CU14. Service mechanical robot and automated equipment | - Identify robotic and automated equipment - Identify defect and issue in system - Inspect the fault - Repair/replace the defective parts - Test the replacement of part in system - Perform finishing operations |

Knowledge & Understanding

- Describe couplings, clutches and brake system

- Describe shaft, bearing seals and their alignment procedures

- Describe types of gear system.

- Describe pump system

- Describe fan and blowers

- Describe compressors

- Describe piping, tank and containers

- Describe hydraulic system

- Describe pneumatic and vacuum system

- Describe conveying system

- Describe prime movers

- Describe types of maintenance

- Describe commissioning and decommissioning of equipment

- Describe mechanical robot and automated equipment

- Describe various types of friction and load conditions associated with bearings.

Tool and Equipment

- couplings, clutches and brake system

- shaft, bearing and seals

- gear systems

- positive displacement pump

- non-positive displacement pump fan and blowers

- compressors

## 0715.18. Tool design and tool making

### 0715.18.1. Design and draw the CAM profile with knife edge follower for uniform velocity

Overview: This competency standard covers the skills and knowledge required to Movement of knife edge follower, Practice to sketch Displacement, velocity and acceleration diagram when knife edge Follower, Movement of follower with Simple Harmonic Motion (S.H.M), Practice to sketch Displacement, velocity and acceleration diagram when knife edge follower moves with uniform velocity and Practice to draw CAM profile as given data

| Competency Units | Performance Criteria |
| --- | --- |
| CU1: Movement of knife edge follower | P1. Differentiate between the cam and follower P2. Choose required type of follower P3. Identify the modulus of the follower P4. Measure the dimensions of the follower |
| CU2: Practice to sketch Displacement, velocity and acceleration diagram when knife edge Follower. | P1. Differentiate between the cam and follower P2. Choose required type of follower P3. Identify the modulus of the follower P4. Measure the dimensions of the follower P5. Measure the displacement of the knife edge follower P6. Calculate the velocity of the knife edge follower P7. Calculate the acceleration of the knife edge follower |
| CU3: Movement of follower with Simple Harmonic Motion (S.H.M) | P1. Measure the dimensions of the follower P2. Calculate the displacement of the knife edge follower P3. Calculate the velocity of the knife edge follower P4. Calculate the acceleration of the knife edge follower P5. Observe the movement of the follower and compare its motion with simple harmonic motion |
| CU4: Practice to sketch Displacement, velocity and acceleration diagram when knife edge follower moves with uniform velocity | P1. Differentiate between the cam and follower P2. Choose required type of follower P3. Identify the modulus of the follower P4. Measure the dimensions of the follower P5. Calculate the displacement of the knife edge follower with uniform velocity P6. Calculate the velocity of the knife edge follower with uniform velocity P7. Calculate the acceleration of the knife edge follower with uniform velocity |
| CU5. Practice to draw CAM profile as given data | P1. Measure the dimensions of the follower P2. Calculate the displacement of the knife edge follower P3. Calculate the velocity of the knife edge follower P4. Calculate the acceleration of the knife edge follower P5. Observe the movement of the follower and draw cam profile as per given requirement |

Knowledge and Understanding

K1. Knowing follower and cam theory

K2. Understanding of the knife edge follower

K3. Understanding about the follower types

K4. Knowledge about the modulus of the follower

K5. Understanding the K.E and P.E concepts

K6. Understanding about the follower displacement behavior

K7. Calculate the velocity and acceleration behavior

Tools and Equipment

- Cam and follower

- Various types of followers

- Measuring instruments

- Complete mechanism with displacement finder

- Blank graph paper

### 0715.18.2. Draw standard parts of jigs & fixtures (manual drawing)

Overview: This competency standard covers the skills and knowledge required to Draw hex head nut, Draw hex head bolt, Draw wing nut, Draw knurling nut, Draw counter sinking head screw, Draw cap screw, Draw machine screw, Draw socket head screw, Draw plain washer, Draw spring washer and Draw c-type washer

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw hex head nut | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw hex head nut as per requirement |
| CU2. Draw hex head bolt | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a bolt P7. Draw a bolt head P8. Draw an end view of the head P9. Draw hex head bolt |
| CU3. Draw wing nut | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of nut P9. Draw wing nut according to the requirements |
| CU4. Draw knurling nut | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw knurling nut |
| CU5. Draw counter sinking head screw | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw counter sinking head screw |
| CU6. Draw cap screw | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw cap screw |
| CU7. Draw machine screw | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw machine screw |
| CU8. Draw socket head screw | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw socket head screw |
| CU9. Draw plain washer | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw plain washer |
| CU10. Draw spring washer | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw spring washer |
| Cu11. Draw c-type washer | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw c-type washer |

Knowledge & Understanding

K1. Knowledge about drawing sheets

K2. Knowledge about drawing instruments

K3. Understanding about the lines and sketch phenomena

K4. Understanding about nut terminology

K5. Understanding about nut types

Tools and Equipment

  - Drawing instruments and drawing sheet

  - Drawing board

  - Drawing sheet

  - Drawing sheet holder

  - Set squares

  - Scale

  - Divider

  - T square

  - Drawing pencils- H, 2H, HB

  - Paper tape

  - Pencil Sharpeners Eraser

  - Protractor

  - Drawing instruments and drawing sheet

  - Drawing board

  - Drawing sheet holder

  - Set squares

### 0715.18.3. Draw different types of clamps

Overview: This competency standard covers the skills and knowledge required to Draw hook clamp, Draw latch clamp, Draw cam clamp, Draw eye bolt, Draw strap clamps and Draw plain wedge clamp

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw hook clamp | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a clamp P7. Draw an end view of the clamp P8. Draw hook clamp |
| CU2. Draw latch clamp | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a clamp P7. Draw a latch clamp |
| CU3. Draw cam clamp | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a clamp P7. Draw cam clamp |
| CU4. Draw eye bolt | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a bolt P7. Draw eye bolt |
| CU5. Draw strap clamps | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a clamp P7. Draw an end view of the clamp P8. Draw strap clamp |
| CU6. Draw plain wedge clamp | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a clamp P7. Draw an end view of the clamp P8. Draw wedge clamp |

Knowledge & Understanding

K1. Knowledge about drawing sheets

K2. Knowledge about drawing instruments

K3. Understanding about the lines and sketch phenomena

K4. Understanding about washer terminology

K5. Understanding about hook clamp

Tools and Equipment

  - Drawing instruments and drawing sheet

  - Drawing board

  - Drawing sheet holder

  - Set squares

  - Scale

  - Divider

  - T square

  - Drawing pencils H, 2H, HB

  - Paper tape

  - Pencil Sharpeners Eraser

  - Protractor

  - Drawing instruments and drawing sheet

  - Drawing board

  - Drawing sheet holder

  - Set squares

### 0715.18.4. Draw locators (Manual)

Overview: This competency standard covers the skills and knowledge required to Draw pin and button location, Draw rest pad and plates, Draw nest or cavity location and Draw jig feet (rest button, hex rest button, jig leg, flat jig feet)

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw pin and button location | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a pin P7. Draw an end view of the button P8. Draw pin and button location |
| CU2. Draw rest pad and plates | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a pad P7. Draw an end view of the plates P8. Draw rest pad and plates |
| CU3. Draw nest or cavity location | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of nest P7. Draw an end view of the cavity P8. Draw nest or cavity location |
| CU4. Draw jig feet (rest button, hex rest button, jig leg, flat jig feet) | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of a jig feet P7. Draw an end view of the rest parts |

Knowledge & Understanding

K1. Knowledge about drawing sheets

K2. Knowledge about drawing instruments

K3. Understanding about the lines and sketch phenomena

K4. Understanding about nut terminology

K5. Understanding about pin & button location

Tools and Equipment

  - Drawing instruments and drawing sheet

  - Drawing board

  - Drawing sheet holder

  - Set squares

  - Scale

  - Divider

  - T square

  - Drawing pencils- H, 2H, HB

  - Paper tape

  - Pencil Sharpeners Eraser

  - Protractor

### 0715.18.5. Draw template jig and plate type drill Jig

Overview: This competency standard covers the skills and knowledge required to Draw the work piece to be drilled, Draw the body of jig as per given job and Draw the template and plate drill jig as per given job

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw the work piece to be drilled | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the work piece P7. Draw an end view of the drill P8. Draw the main drill hole |
| CU2. Draw the body of jig as per given job | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the jig P7. Draw an end view of the body of jig |
| CU3. Draw the template and plate drill jig as per given job | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the jig P7. Draw an end view of the body of jig P8. Draw the main plate drill jig |

Knowledge & Understanding

K1. Knowledge about drawing sheets

K2. Knowledge about drawing instruments

K3. Understanding about the lines and sketch phenomena

K4. Understanding about drilling terminology

K5. Understanding about work piece

K4. Understanding about jig terminology

K5. Understanding about body of the jig

K5. Understanding about templates and jigs

Tools and Equipment

- Drawing instruments and drawing sheet

- Drawing board

- Drawing sheet holder

- Set squares

- scale

- Divider

- T square

- Drawing pencils- H, 2H, HB

- Paper tape

- Pencil Sharpeners Eraser

- Protractor

### 0715.18.6. Draw swinging leaf jig

Overview: This competency standard covers the skills and knowledge required to Draw the work piece to be drill, Draw the body of jig as per given work piece, Draw clamps for accurate and easy clamping, Draw locators for accurate location, Draw proper drill bushings for guidance of twist drill and Draw suitable jig feet(flat jig feet)

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw the work piece to be drill | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the work piece P7. Draw an end view of the drill P8. Draw the main drill hole |
| CU2. Draw the body of jig as per given work piece | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the jig P7. Draw an end view of the body of jig |
| CU3. Draw clamps for accurate and easy clamping | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the clamping place P7. Draw an end view of the clamp P8. Draw the main and accurate clamps for easy clamping |
| CU4. Draw locators for accurate location | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body locator P7. Draw an end view of the locator P8. Attach locator for accurate location |
| CU5. Draw proper drill bushings for guidance of twist drill | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the bush P7. Draw an end view of the drill P8. Draw proper drill bushings for guidance of twist drill |
| CU6. Draw suitable jig feet (flat jig feet) | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the jig P7. Draw an end view of the body of jig P8. Draw the main plate drill jig/ flat jig |

Knowledge & Understanding

- Knowledge about Drawing sheets

- Knowledge about Drawing instruments

- Understanding about the lines and sketch phenomena

- Understanding about drilling terminology

- Understanding about jig terminology

- Understanding about body of the jig

- Understanding about clamps terminology

- Understanding about clamps

- Understanding about locator’s terminology

- Understanding about locators

- Understanding about drill bushings terminology

- Understanding about twist drill

- Understanding about flat jig terminology

- Understanding about suitable jig feet

Tools and Equipment

•	Drawing instruments and drawing sheet

•	Drawing board

•	Drawing sheet holder

•	Set squares

•	Scale

•	Divider

•	T square

•	Drawing pencils- H, 2H, HB

•	Paper tape

•	Pencil Sharpeners Eraser

•	Protractor

### 0715.18.7. Draw an indexing jig

Overview: This competency standard covers the skills and knowledge required to Draw the work piece to be drilled, Draw the body of work piece as per given w / p, Draw locators for accurate location, Draw clamps for accurate clamping, Draw drill bushing for guidance of twist drill and Draw indexing mechanism

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw the work piece to be drilled | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the work piece P7. Draw an end view of the drill P8. Draw the main drill hole |
| CU2. Draw the body of work piece as per given w / p | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the work piece P7. Draw an end view of the drill P8. Draw the main drill hole as per given w/p |
| CU3. Draw locators for accurate location | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the locators P7. Draw an end view of the locators P8. Draw the main and accurate locators for accurate location |
| CU4. Draw clamps for accurate clamping | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body clamps P7. Draw an end view of the clamps P8. Attach locator for accurate clamping |
| CU5. Draw drill bushing for guidance of twist drill | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the bush P7. Draw an end view of the drill bush P8. Draw proper drill bushings for guidance of twist drill |
| CU5. Draw indexing mechanism | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the remaining parts P7. Draw an end view of the body of indexing mechanism P8. Draw the main indexing jig |

Knowledge & Understanding

- Knowledge about drawing sheets

- Knowledge about drawing instruments

- Understanding about the lines and sketch phenomena

- Understanding about the work piece to be drilled

- Understanding about work piece terminology

- Understanding about w/p

- Understanding about locator terminology

- Understanding about accurate location

- Understanding about clamps terminology

- Understanding about accurate clamping

- Understanding about drill bushing terminology

- Understanding about twist drilling

- Understanding about indexing terminology

- Understanding about indexing mechanism

Tools and Equipment

•	Drawing instruments and drawing sheet

•	Drawing board

•	Drawing sheet holder

•	Set squares

•	Scale

•	Divider

•	T square

•	Drawing pencils- H, 2H, HB

•	Paper tape

•	Pencil Sharpeners Eraser

•	Protractor

### 0715.18.8. Draw an angle milling fixture

Overview: This competency standard covers the skills and knowledge required to Draw the work piece to be milled, Draw the main parts, the base, Draw Clamps, Draw Rest blocks or nest, Draw locating points, Draw gauging surfaces and Draw the assembled single piece milling fixture

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw the work piece to be milled | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the work piece to be milled |
| CU2. Draw the main parts, the base | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the work piece P7. Draw an end view of the base |
| CU3. Draw Clamps | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the clamp |
| CU4. Draw Rest blocks or nest, | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the blocks/rest as per requirement |
| CU5. Draw locating points, | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the locating points |
| CU 6. Draw gauging surfaces | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the gauge surfaces |
| CU7. Draw the assembled single piece milling fixture | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the single piece milling fixture |

Knowledge & Understanding

- Knowledge about drawing sheets

- Knowledge about drawing instruments

- Understanding about the lines and sketch phenomena

- Understanding about milling terminology

- Understanding about work piece to be milled

- Understanding about the base terminology

- Understanding about main parts

- Understanding about clamps terminology

- Understanding about rest blocks terminology

- Understanding about nest

- Understanding about locating point’s terminology

- Understanding about gauging terminology

- Understanding about surfaces

- Understanding about milling fixtures terminology

- Understanding about single piece assembly

Tools and Equipment

•	Drawing instruments and drawing sheet

•	Drawing board

•	Drawing sheet holder

•	Set squares

•	Scale

•	Divider

•	T square

•	Drawing pencils- H, 2H, HB

•	Paper tape

•	Pencil Sharpeners Eraser

•	Protractor

### 0715.18.9. Draw lathe fixture for turning and boring operation

Overview: This competency standard covers the skills and knowledge required to draw the work piece to be turned or bored, Draw the turning fixture and Draw holding mechanism of fixture on lathe spindle

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw the work piece to be turned or bored | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the work piece P7. Draw an end view of the drill P8. Draw the main drill hole |
| CU2. Draw the turning fixture | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw the turning fixture |
| CU3. Draw holding mechanism of fixture on lathe spindle | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw holding mechanism of fixture on lathe spindle |

Knowledge & Understanding

- Knowledge about drawing sheets

- Knowledge about drawing instruments

- Understanding about the lines and sketch phenomena

- Understanding about turning terminology

- Understanding about turning and boring

- Understanding about turning fixture terminology

- Understanding about fixture

- Understanding about holding mechanism terminology

- Understanding about fixture on lathe spindle

Tools and Equipment

- Drawing instruments and drawing sheet

- Drawing board

- Drawing sheet holder

- Set squares

- scale

- Divider

- T square

- Drawing pencils- H, 2H, HB

- Paper tape

- Pencil Sharpeners Eraser

- Protractor

### 0715.18.10. Draw simple blanking die for making a blank

Overview: This competency standard covers the skills and knowledge required to Draw the blank position in the strip, Calculate the size of punch considering the elastic recovery of material, Calculate the size of die considering the elastic recovery of material, Draw the piercing die and punch, Draw the piercing punch and Draw the different types of punches to reduce the cutting forces

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw the blank position in the strip | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the strips P7. Draw an end view of the blank position P8. Draw the blank position in the strip |
| CU2. Calculate the size of punch considering the elastic recovery of material | P1. Consider elastic recovery of material P2. Calculate the size of punch |
| CU3. Calculate the size of die considering the elastic recovery of material | P1. Select measuring tools P2. Consider elastic recovery of the material P3. Analyze the tolerance of the material |
| CU4. Draw the piercing die and punch | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the piercing die and punching |
| CU5. Draw the piercing punch | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the piercing punch |
| CU6. Draw the different types of punches to reduce the cutting forces | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the punches which reduces the cutting forces |

Knowledge & Understanding

- Knowledge about drawing sheets

- Knowledge about drawing instruments

- Understanding about the lines and sketch phenomena

- Understanding about strips terminology

- Understanding about blank position in strips

- Understanding about punching terminology

- Understanding about elasticity

- Understanding about dies terminology

- Understanding about elasticity

- Understanding about piercing terminology

- Understanding about punching

- Understanding about reduction terminology

- Understanding about reduction of cutting forces in punching

Tools and Equipment

•	Drawing instruments and drawing sheet

•	Drawing board

•	Drawing sheet holder

•	Set squares

•	Scale

•	Divider

•	T square

•	Drawing pencils- H, 2H, HB

•	Paper tape

•	Pencil Sharpeners Eraser

•	Protractor

### 0715.18.11. Draw a progressive die for making a washer of inner diameter 20 and outer 40 mm

Overview: This competency standard covers the skills and knowledge required to Draw the piercing position in the strip inner dia 20 and outer dia 40mm, Draw the blanking position in the strip inner dia 20 and outer dia 40mm, Draw the die block and calculate the size of punch after considering the elastic recovery of Material, Draw the die block and calculate the size of die after considering the elastic recovery of Material and Draw the pilot punch used in progressive die

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw the piercing position in the strip inner dia 20 and outer dia 40mm | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the piercing position as per given requirements |
| CU2. Draw the blanking position in the strip inner dia 20 and outer dia 40mm | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the blanking position as per given requirements |
| CU3. Draw the die block and calculate the size of punch after considering the elastic recovery of Material | P1. Select measuring tools P2. Consider elastic recovery of the material P3. Analyze the tolerance of the material P4. Select drawing sheet P5. Select drawing instruments P6. Set drawing sheet on drawing table/board P7. Set arm location P8. Choose starting position P9. Draw body of the die block |
| CU4. Draw the die block and calculate the size of die after considering the elastic recovery of Material | P1. Select measuring tools P2. Consider elastic recovery of the material P3. Analyze the tolerance of the material P4. Select drawing sheet P5. Select drawing instruments P6. Set drawing sheet on drawing table/board P7. Set arm location P8. Choose starting position P9. Draw body of the die block |
| CU5. Draw the pilot punch used in progressive die | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the pilot punch |

Knowledge & Understanding

- Knowledge about drawing sheets

- Knowledge about drawing instruments

- Understanding about the lines and sketch phenomena

- Understanding about measurements of the piercing terminology

- Understanding about dimensional analysis

- Understanding about die block terminology

- Understanding about punching

- Understanding about elasticity

- Understanding about pilot punch terminology

- Understanding about progressive die

Tools and Equipment

•	Drawing instruments and drawing sheet

•	Drawing board

•	Drawing sheet holder

•	Set squares

•	Scale

•	Divider

•	T square

•	Drawing pencils- H, 2H, HB

•	Paper tape

•	Pencil Sharpeners Eraser

•	Protractor

### 0715.18.12. Draw a simple drawing die for a cup

Overview: This competency standard covers the skills and knowledge required to Draw the work piece to be draw, Draw the single action draw die with spring loaded knockout, Calculate the percent reduction and depth of draws, Calculate blank size for a cup of 40mm inner dia., 30mm depth and sheet thickness 1mm and Calculate the drawing forces

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Draw the work piece to be draw | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the base lines P7. Draw an end view of the work piece |
| CU2. Draw the single action draw die with spring loaded knockout | P1. Select drawing sheet P2. Select drawing instruments P3. Set drawing sheet on drawing table/board P4. Set arm location P5. Choose starting position P6. Draw body of the spring P7. Draw an end view of the single action draw die |
| CU3. Calculate the percent reduction and depth of draws | P1. Select measuring tools P2. Calculate change I length P3. Develop relationship to get % reduction P4. Follow same procedure to calculate the depth of draws |
| CU4. Calculate blank size for a cup of 40mm inner dia., 30mm depth and sheet thickness 1mm | P1. Select measuring tools P2. Calculate change I length P3. Develop relationship to get % reduction as per given condition P4. Follow same procedure to calculate the depth of draws as per condition |
| CU5. Calculate the drawing forces | P1. Select measuring tools P2. Calculate change I length P3. Develop relationship to get % reduction as per given condition P4. Follow same procedure to calculate the depth of draws as per condition |

Knowledge & Understanding

K1. Knowledge about drawing sheets

K2. Knowledge about drawing instruments

K3. Understanding about the lines and sketch phenomena

K4. Understanding about work piece terminology

K5. Understanding about drawing

K4. Understanding about single action drawing terminology

K5. Understanding about spring loading

K4. Understanding about % reduction terminology

K5. Understanding about % depth of draws

K4. Understanding about cup terminology

K5. Understanding about thickness

K5. Understanding about drawing forces

Tools & Equipment

- Drawing instruments and drawing sheet

- Drawing board

- Drawing sheet holder

- Set squares

- scale

- Divider

- T square

- Drawing pencils- H, 2H, HB

- Paper tape

- Pencil Sharpeners Eraser

- Protractor

## 0715.19. Boiler Operator

### Maintain Boiler house Record

Overview: This competency standard covers the skills and knowledge required to maintain record book & log book and maintain boiler tools
& emergency spares.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Maintain record book & log book | - Entries made regularly in check lists records & log books. - Arrangements made for statutory annual inspection of boiler as per factories ordinance. |
| CU2. Maintain boiler tools & emergency spares | - Necessary tools and essential spares for the boiler stocked in a suitable storage facility. - Appropriate firefighting equipment correctly located and maintained regularly. - Maintain Cleanliness and tidiness of the boiler house by following good housekeeping practices. |

Knowledge & Understanding

- Principles of boiler operation

- Statutory requirements regarding boiler operation

- English Technical Terms

- Basic stock management

- Fire safety and firefighting equipment

- Safety procedures to be followed

Tool and Equipment

- General mechanic’s tools

- Fire extinguishers

- Sand buckets

- Cleaning tools

- Record books

- Commonly used boiler spares

- Personal protective equipment

### Set and adjust burners

Overview: This competency standard covers the skills and knowledge required to Carry out basic startup of burner and maintain burner system.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Carry out basic startup of burner | - Check Electrode setting and adjust /replace if necessary. - Check Nozzle tips and clean / replace if necessary. - Check Photocell / UV scanner / flame rod and replace if necessary. - Check Heater thermostats and set / replace if necessary. |
| CU2. Maintain burner system | - Check Nozzle closure and replace if necessary. - Check Nozzle head and adjust if necessary. - Air dampers position checked and adjusted if necessary. - Check/ adjust Atomizer (air / fuel) checked - Check Oxygen & carbon dioxide content and temperature of flue gas, if gas analyzer available and air/fuel ratio adjust if necessary. - Check Solenoid valve checked / adjusted / replaced as required. |

Knowledge & Understanding

- Principles of combustion.

- Types of burners and their mode of operation

- Methods of analyzing flue gases

- Methods of adjusting air/fuel ratio

- Types of fuels and their characteristics

- Basic principles of electricity and electronics

- Safety rules and regulations

- Relevant ISO/ ASME/ BS codes

- Fire safety

- Safety procedures to be followed

- Factories ordinance

- Testing instruments

- Technical terms in English language

- Ability to understand engineering drawings and flow charts

Tool and Equipment

- General mechanic’s tools

- Gauge glass, rubber cones

  - Set of nozzles and cleaning kit

  - Pair of electrodes

  - Sets of gaskets

  - Photo cell

  - Gland packings

  - Personal protective equipment

  - Smoke analyzing test kit

  - Flame rod

  - UV Scanner

  - Solenoid valve

  - Thermostats

### Perform Boiler Operations

Overview: This competency standard covers the skills and knowledge required to inspect boiler & accessories for safe operation, Check and operate boiler mountings & burner, Operate feed water pumps / Injector, Check fuel system, Fire the burner, Check control panels and Inspect openings and Shutdown burner.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Inspect boiler & accessories for safe operation | - Inspect openings & refractory linings - Inspect furnace and tube nest - Check Boiler utilities - Check exhaust system |
| CU2. Check and operate boiler mountings & burner | - Check water gauge column, water level and steam pressure controls - Check pressure gauges - Check safety valves - Check main valve - Check feed water and stop valves - Operate blow down valves - Operate air vent valve |
| CU3. Operate feed water pumps / Injector | - Maintain all supply and delivery valves in proper open position. - Check Feed water pumps to ensure that they are in proper operating condition, after powering on. - Check Feed water injector to ensure that they are in proper operating conditions. - Clean Strainers cleaned if necessary |
| CU4. Check fuel system | - Maintain all valves in fuel supply line in open position. - Check Fuel pump to ensure that it is building up proper pressure. - Check fuel pump and fuel filter checked to ensure proper fuel supply. - Maintain Temperature and final pressure in fuel oil system/s. |
| CU5. Fire the burner | - Control Furnace purged to prevent possible backfire following recommended procedure. - Open Fuel and air lines for pilot burner prior to operation. - Inspect Sight glass and magic eye and clean to observe boiler operations. - Inspect Burner flame through sight glass to ensure proper operation. - Check Chimney outlet checked for abnormal/excessive smoke. |
| CU6. Check control panels and Inspect openings | - Check Indicator lamps to ensure that all are in proper working order. - Check Alarms to ensure that they are in working order. - Inspect Manholes, hand holes and joints - inspect for leaks and result entered correctly in the check list |
| CU7. Shutdown burner | - Switch Burner switched off as instruction given by the authorized person - Close Inlets & outlets valves closed appropriately - Cool down the boiler to atmospheric condition ensured as specified - Open Air vent |

Knowledge & Understanding

- Principles of boiler operation

- Preventive & corrective maintenance

- Reading and understanding instructions

- Safety rules and regulations

- Safety procedures to be followed.

- Factories Ordinance

- Proper operation of valves and pumps

- Material Safety Data Sheet (MSDS)

- Technical terms in English language

- Ability to understand engineering drawings and flow charts

Tool and Equipment

- Personal protective equipment

- Packing and gaskets

- General mechanic’s tools and electricians’ basic tools

- Tube brush

- General mechanic’s tools • Gauge glass, rubber cones

  - Set of gaskets

  - Gland packing

  - Dial thermometer 0-600° C • Oil pressure gauge 0-600 psi

  - Personal protective equipment • Pyrometer

### Operate boiler auxiliaries

Overview: This competency standard covers the skills and knowledge required to check water softener, Check chemical dosing system, Check make-up water tank and Check day oil tank and Inspect main header and oil storage tank.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Check water softener | - Monitor operation of water softener system. - Check level of brine tank to ensure adequate supply. |
| CU2. Check chemical dosing system | - Check Chemical storage tank levels to ensure adequate chemical supply. - Check Chemical dosing pump for proper functioning |
| CU3. Check make-up water tank and Check Day oil tank | - Check Make-up water tank to ensure that sufficient water level is maintained. - Check Feed and outlet valves to ensure proper supply of water. - Ensure required quality of make-up water by testing Check Day oil tank to ensure that oil level is sufficient. - Drain Water and sludge from the oil tank as and when necessary. - Check Temperature of day oil tank, record and report as required. |
| CU4. Inspect main header and oil storage tank | - Check Safety valve, pressure gauge and steam trap c for proper operation. - Check Steam valves checked for leaks. - Check Fuel oil level, fuel transfer pump, fuel oil filter/s and valves to ensure proper operation. - Drain Water/sludge drained when required. - Monitor Fuel stock and inform to superior if necessary for required action. |

Knowledge and Understanding

- Understanding water hardness and its adverse effects on boiler performance

- Understanding chemical dosing phenomenon

- Proper operation of valves and pumps

- Understanding safety rules & regulations

Tool and Equipment

- General mechanic’s tools

- Gauge glass, rubber cones

- Chemicals (for testing & softening)

- Bucket

- Sets of gaskets

- Cotton waste

- Gland packing

- Personal protective equipment

- Cleaning agents

- Boiler water testing kits

### Perform Maintenance of Boiler

Overview: This competency standard covers the skills and knowledge required to Carry out routine maintenance and Carry out basic maintenance operations.

| Competency Units | Performance Criteria |
| --- | --- |
| - Carry out routine maintenance | - Regenerate water softener plant - Clean / Replace air filters - Clean / Replace oil filters - Clean / Replace ignition electrodes - Clean diffuser plates - Clean/replace burner and sight glass - Clean carbon deposits in smoke box & chimney |
| - Carry out basic maintenance operations | - Clean atomizer and damper - Clean/replace burner nozzle - Clean fire tubes - Clean chamber of the boiler |

Knowledge & Understanding

K1.	Types of maintenance

Tool and Equipment

- oil filters

- air filters

- water softener plant

- ignition electrodes

- diffuser plates

- burner and sight glass

- List of Machinery, Tools & Equipment.

(FOR A CLASS OF 25 STUDENTS)

3. National Qualification Certificate level-4, Part-1 in “Mechanical Technology”

      - Calibration

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Vernier Caliper | 25 Nos | 1 | Cutting Oil | 10 liters |
| 2 | Plier | 25 Nos | 2 | Soldering wire | 02 lbs. |
| 3 | Screw Driver | 25 Nos | 3 | Grease | 10 lbs. |
| 4 | Scale | 25 Nos | 4 | Cotton waste | 5 kg |
| 5 | Micrometer Screw Gauge | 25 Nos |  |  |  |
| 6 | Gauge | 25 Nos |  |  |  |
| 7 | Sensors | 25 Nos |  |  |  |
| 8 | Sensor calibrator | 25 Nos |  |  |  |
| 9 | Tachometer | 25 Nos |  |  |  |
| 10 | Nose plier | 25 Nos |  |  |  |
| 11 | Soldering iron | 25 Nos |  |  |  |
| 12 | Mechanical Tool Kit | 25 Nos |  |  |  |
| 13 | Lathe Machines | 10 Nos |  |  |  |
| 14 | Drilling machines | 02 Nos |  |  |  |
| 15 | Shaper Machine | 02 Nos |  |  |  |
| 16 | Planar Machine | 01 No |  |  |  |
| 17 | CNC Milling Machine | 01 No |  |  |  |
| 18 | CNC Lathe Machine | 01 N0 |  |  |  |
| 19 | Tool Kit Electrical | 25 Nos |  |  |  |
| 20 | Laser | 01No |  |  |  |
| 21 | Slotter Machine | 01No |  |  |  |
| 22 | Vertical Milling | 01No |  |  |  |
| 23 | Horizontal Milling | 01No |  |  |  |
| 24 | Vertical Gear Hobbing machine | 01No |  |  |  |
| 25 | Horizontal Gear Hobbing machine | 01No |  |  |  |
| 26 | Bench grinders | 05 Nos |  |  |  |
| 27 | Hand grinders | 05 Nos |  |  |  |
| 28 | Grinding tools | 01No |  |  |  |
| 29 | Arc Welding Plant | 01No |  |  |  |
| 30 | Universal Testing Machine | 01No |  |  |  |
| 31 | Steel rule | 25 Nos |  |  |  |
| 32 | Charpy Impact Tester | 01No |  |  |  |
| 33 | Tongs | 25 Nos |  |  |  |
| 34 | Holder for sample cantering | 01No |  |  |  |
| 35 | Rockwell Hardness Tester. | 01No |  |  |  |
| 36 | Spirit Leveller | 25 Nos |  |  |  |

- Mechanical Engineering Drawing

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Drawing Board/Table. | 25 Nos | 1 | Graph and drawing sheet. | 100 Nos |
| 2 | T-Square | 25 Nos | 2 | Templets. | 25 Nos |
| 3 | Set Square. | 25 Nos |  |  |  |
| 4 | Geometry Box. | 25 Nos |  |  |  |
| 5 | Compass. | 25 Nos |  |  |  |
| 6 | Computer with all accessories | 25 Nos |  |  |  |
| 7 | AutoCAD software disk | 01 No |  |  |  |
| 8 | Models | 1 No each |  |  |  |

- Hydraulics and Hydraulic Machinery

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Hydraulic bench | 05 Nos | 1 |  |  |
| 2 | 3/2 push button | 05 Nos | 2 |  |  |
| 3 | 5/2 double acting side pilot operated DCV | 05 Nos | 3 |  |  |
| 4 | Accumulator Flow Control | 05 Nos | 4 |  |  |
| 5 | Actuators | 05 Nos | 5 |  |  |
| 6 | Ammeter | 05 Nos | 6 |  |  |
| 7 | Ball Valve | 05 Nos | 7 |  |  |
| 8 | Bourdon Tube | 05 Nos | 8 |  |  |
| 9 | Butterfly Valve | 05 Nos | 9 |  |  |
| 10 | Centrifugal pump | 05 Nos | 10 |  |  |
| 11 | Chamber | 05 Nos | 11 |  |  |
| 12 | Channel | 05 Nos | 12 |  |  |
| 13 | Contacting tubes | 05 Nos | 13 |  |  |
| 14 | Control Valve | 05 Nos | 14 |  |  |
| 15 | DC pressure Gauges | 05 Nos | 15 |  |  |
| 16 | DCV & DCV pipe hoses | 05 Nos | 16 |  |  |
| 17 | Deadweight gauge Tester | 05 Nos | 17 |  |  |
| 18 | Diaphragm Pressure Gauge | 05 Nos | 18 |  |  |
| 19 | Diaphragm Valve | 05 Nos | 19 |  |  |
| 20 | Differential U-tube manometer | 05 Nos | 20 |  |  |
| 21 | Direction control valve | 05 Nos | 21 |  |  |
| 22 | Double acting cylinder | 05 Nos | 22 |  |  |
| 23 | Drain valve | 05 Nos | 23 |  |  |
| 24 | Dump valve | 05 Nos | 24 |  |  |
| 25 | Electric Circuit Supply | 05 Nos | 25 |  |  |
| 26 | Energy meter | 05 Nos | 26 |  |  |
| 27 | Flow circuit of G.I. Pipes of different fittings via Large bend, Small bend | 05 Nos | 27 |  |  |
| 28 | Flow control valve | 05 Nos | 28 |  |  |
| 29 | Gate Valve | 05 Nos | 29 |  |  |
| 30 | Generator assembly and Electric Circuit | 05 Nos | 30 |  |  |
| 31 | Hydraulic pump hoses | 05 Nos | 31 |  |  |
| 32 | Impulse Turbine (Pelton Wheel) | 05 Nos | 32 |  |  |
| 33 | Inclined Manometer | 05 Nos | 33 |  |  |
| 34 | Limit switch | 05 Nos | 34 |  |  |
| 35 | Master Gauge Calibrator | 05 Nos | 35 |  |  |
| 36 | Measuring tank | 05 Nos | 36 |  |  |
| 37 | Mechanical interlocking | 05 Nos | 37 |  |  |
| 38 | Non-Return Valve | 05 Nos | 38 |  |  |
| 39 | Piezometer tube | 05 Nos |  |  |  |
| 40 | Pilot operated check valve | 05 Nos |  |  |  |
| 41 | Pipe hoses and fittings | 05 Nos |  |  |  |
| 42 | Plunger | 05 Nos |  |  |  |
| 43 | Pressure control valve | 05 Nos |  |  |  |
| 44 | Pressure gauges | 05 Nos |  |  |  |
| 45 | Pressure reading valve | 05 Nos |  |  |  |
| 46 | Ram | 05 Nos |  |  |  |
| 47 | Reaction Turbine (Francis Turbine) | 05 Nos |  |  |  |
| 48 | Reciprocating Pump | 05 Nos |  |  |  |
| 49 | Scale | 05 Nos |  |  |  |
| 50 | Side Channel | 05 Nos |  |  |  |
| 51 | Stop watch | 05 Nos |  |  |  |
| 52 | Sudden contraction from 50 mm to 25 mm diameter. | 05 Nos |  |  |  |
| 53 | Sudden enlargement from 25 mm to 50 mm diameter | 05 Nos |  |  |  |
| 54 | Sump tank | 05 Nos |  |  |  |
| 55 | Tachometer | 05 Nos |  |  |  |
| 56 | Tapered inclined pipe fitted with number of piezometer tube points | 05 Nos |  |  |  |
| 57 | Test Bench | 05 Nos |  |  |  |
| 58 | U Tube Manometer | 05 Nos |  |  |  |
| 59 | Vertical pipe | 05 Nos |  |  |  |
| 60 | Voltmeter | 05 Nos |  |  |  |
| 61 | Volumetric tank | 05 Nos |  |  |  |
| 62 | Water Level Indicator | 05 Nos |  |  |  |
| 63 | Weights | 05 Nos |  |  |  |

- Applied Thermodynamics

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Measuring tools and instruments for temperature | 25 Nos | 1 | Stationary Items | 25 Sets |
| 2 | Measuring tools and instruments for vacuum and pressure gauges | 25 Nos |  |  |  |

- National Qualification Certificate level-4, Part-2 in “Mechanical Technology”

- HVACR

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Adjustable Pipe Wrench | 25 Nos | 1 | Teflon Tape | 25 Nos |
| 2 | Adjustable Screw Wrench | 25 Nos | 2 | User Manual | 25 Nos |
| 3 | Air-conditioning Kit | 25 Nos | 3 | Wire Tape | 25 Nos |
| 4 | Allen Key Set | 25 Nos | 4 | Copper Pipe | 10 kg |
| 5 | Bearing Puller | 25 Nos | 5 | Gas Cylinder | 05 Nos |
| 6 | Bench Vice | 25 Nos | 6 | Health & Safety Manuals | 25 Nos |
| 7 | Box Spanner Screw Drivers | 25 Nos | 7 | Heavy Wire | 02 Roll |
| 8 | Box Spanner Set | 25 Nos |  |  |  |
| 9 | Center Punch | 25 Nos |  |  |  |
| 10 | Chisel Set | 25 Nos |  |  |  |
| 11 | Combination Plier | 25 Nos |  |  |  |
| 12 | Crimping Tool | 25 Nos |  |  |  |
| 13 | De-Soldering Tool | 25 Nos |  |  |  |
| 14 | Diagonal Side Cutter | 25 Nos |  |  |  |
| 15 | Digital Air Flow / Velocity Meter | 25 Nos |  |  |  |
| 16 | Digital Capacitor Analyzer | 25 Nos |  |  |  |
| 17 | Digital Clamp-On Ampere Meter | 25 Nos |  |  |  |
| 18 | Digital Humidity Meter | 25 Nos |  |  |  |
| 19 | Digital Micro Meter | 25 Nos |  |  |  |
| 20 | Digital Multi Meter | 25 Nos |  |  |  |
| 21 | Digital Optical Tachometer | 25 Nos |  |  |  |
| 22 | Digital Pressure Gauges Set (High & Combine) | 25 Nos |  |  |  |
| 23 | Digital Psychrometric (Hygrometer) | 25 Nos |  |  |  |
| 24 | Digital Vernier Caliper | 25 Nos |  |  |  |
| 25 | Electric Hand Drills | 05 Nos |  |  |  |
| 26 | Electric Hand Grinder | 05 Nos |  |  |  |
| 27 | Electric Screw Driver Set | 25 Nos |  |  |  |
| 28 | Electronic Leak Detector | 25 Nos |  |  |  |
| 29 | File set | 25 Nos |  |  |  |
| 30 | Fins Straightening Comb Set | 25 Nos |  |  |  |
| 31 | Fire safety equipment | 25 Nos |  |  |  |
| 32 | Flaring and Swaging Tool Kit | 25 Nos |  |  |  |
| 33 | Gas Welding Set with All Accessories | 05 Nos |  |  |  |
| 34 | Hammer Set | 25 Nos |  |  |  |
| 35 | Hand Electric Blower | 25 Nos |  |  |  |
| 36 | Hand Hacksaw Frame with Blade | 25 Nos |  |  |  |
| 37 | HILTI Drill Machine (Piston Type) | 02 Nos |  |  |  |
| 38 | Hot Air Gun | 25 Nos |  |  |  |
| 39 | Insulation Remover | 25 Nos |  |  |  |
| 40 | Laser Distance Measuring Device | 25 Nos |  |  |  |
| 41 | Laser Temperature Measuring Device | 25 Nos |  |  |  |
| 42 | Lcr Meter | 25 Nos |  |  |  |
| 43 | Locking Plier | 25 Nos |  |  |  |
| 44 | Mallet Set | 25 Nos |  |  |  |
| 45 | Manometer | 25 Nos |  |  |  |
| 46 | Manual Screw Driver Set | 25 Nos |  |  |  |
| 47 | Masonry Drill Set | 25 Nos |  |  |  |
| 48 | Measuring Tape | 25 Nos |  |  |  |
| 49 | Meg ohmmeter (0 - 1000 Volts) | 25 Nos |  |  |  |
| 50 | Metal Drill Bit Set | 25 Nos |  |  |  |
| 51 | Micron Pressure Gauge | 25 Nos |  |  |  |
| 52 | Multi meter | 25 Nos |  |  |  |
| 53 | Nitrogen Gas Cylinder with Hose Pipe, Regulator and Back Arrestor | 05 Nos |  |  |  |
| 54 | Nose Plier Set | 25 Nos |  |  |  |
| 55 | Ohm meter (0 - 1000 Volts) | 25 Nos |  |  |  |
| 56 | Open Ended Spanner Set | 25 Nos |  |  |  |
| 57 | Pedestal Drill Machine | 02 Nos |  |  |  |
| 58 | Personal Protective Equipment | 25 Nos |  |  |  |
| 59 | Pinch-Off Plier | 25 Nos |  |  |  |
| 60 | Pipe Cutter | 25 Nos |  |  |  |
| 61 | Pipe Vice | 25 Nos |  |  |  |
| 62 | Portable Refrigerant Charging Station | 25 Nos |  |  |  |
| 63 | Pulley Wheel Puller | 25 Nos |  |  |  |
| 64 | Ratchet Wrench | 25 Nos |  |  |  |
| 65 | Rivet Gun / Plier | 25 Nos |  |  |  |
| 66 | Safety Kit | 25 Nos |  |  |  |
| 67 | Scissors | 25 Nos |  |  |  |
| 68 | Scriber | 25 Nos |  |  |  |
| 69 | Sheet metal tools | 25 Nos |  |  |  |
| 70 | Socket Set | 25 Nos |  |  |  |
| 71 | Soldering Iron | 25 Nos |  |  |  |
| 72 | Spirit Level | 25 Nos |  |  |  |
| 73 | Split Air-conditioning Unit to be installed. | 05 Nos |  |  |  |
| 74 | Steel Ruler | 25 Nos |  |  |  |
| 75 | Swaging Tool Kit | 25 Nos |  |  |  |
| 76 | Tachometer | 25 Nos |  |  |  |
| 77 | Tap and Die Set | 25 Nos |  |  |  |
| 78 | Tin Cutter | 25 Nos |  |  |  |
| 79 | Try Square | 25 Nos |  |  |  |
| 80 | Tube Benders (Spring Type and Pulley Bender Type) | 25 Nos |  |  |  |
| 81 | Tube cutter | 25 Nos |  |  |  |
| 82 | Vacuum Pump 2-Stage, 6cfm | 05 Nos |  |  |  |
| 83 | Water Pressure Gun for Service | 05 Nos |  |  |  |
| 84 | Windows Air-conditioning Unit to be installed. | 05 Nos |  |  |  |
| 85 | Wire Stripper | 25 Nos |  |  |  |
| 86 | Working tables | 05 Nos |  |  |  |

- Mill Wright/ Industrial Mechanic

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Micrometers | 25 Nos | 1. | Kerosene oil | 10 liters |
| 2 | Blocks and pulleys | 25 Nos | 2. | Cotton Waste | 05 kg |
| 3 | Calipers | 25 Nos | 3. | Grease | 10 lbs. |
| 4 | compressors | 05 Nos |  |  |  |
| 5 | couplings, clutches and brake system | 5 nos. each |  |  |  |
| 6 | Dial indicators | 25 Nos |  |  |  |
| 7 | End terminations | 25 Nos |  |  |  |
| 8 | Eyebolts | 25 Nos |  |  |  |
| 9 | Feeler gauges | 25 Nos |  |  |  |
| 10 | Gauge blocks | 25 Nos |  |  |  |
| 11 | gear systems | 25 Nos |  |  |  |
| 12 | Height gauges | 25 Nos |  |  |  |
| 13 | Hoist rings | 25 Nos |  |  |  |
| 14 | Hooks | 25 Nos |  |  |  |
| 15 | Ladders and scaffolding. | 05 Nos |  |  |  |
| 16 | non-positive displacement pump fan and blowers | 25 Nos |  |  |  |
| 17 | Plug, ring and snap gauges | 25 Nos |  |  |  |
| 18 | positive displacement pump | 25 Nos |  |  |  |
| 19 | Protractors | 25 Nos |  |  |  |
| 20 | Shackles | 25 Nos |  |  |  |
| 21 | shaft, bearing and seals | 25 Nos |  |  |  |
| 22 | Spreader bars | 25 Nos |  |  |  |
| 23 | Turnbuckles | 25 Nos |  |  |  |

- Tool design and tool making

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Cam and follower | 25 Nos | 1 | Blank graph paper | 100 Nos |
| 2 | Complete mechanism with displacement finder | 25 Nos | 2 | Drawing pencils H, 2H, HB | 5 dozen each |
| 3 | Divider | 25 Nos | 3 | Paper tape | 5 dozen each |
| 4 | Drawing board | 25 Nos | 4 | Pencil Sharpeners Eraser | 5 dozen each |
| 5 | Drawing instruments and drawing sheet | 25 Nos |  |  |  |
| 6 | Drawing sheet holder | 25 Nos |  |  |  |
| 7 | Measuring instruments | 25 Nos |  |  |  |
| 8 | Protractor | 25 Nos |  |  |  |
| 9 | Scale | 25 Nos |  |  |  |
| 10 | Set squares | 25 Nos |  |  |  |
| 11 | T square | 25 Nos |  |  |  |
| 12 | Various types of followers | 25 Nos |  |  |  |

- Boiler Operator

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Boiler water testing kits | 05 Nos |  | air filters | 25 Nos |
| 2 | Bucket | 05 Nos |  | Cleaning agents | 02 kg |
| 3 | burner and sight glass | 05 Nos |  | Cotton waste | 05 kg |
| 4 | Cleaning tools | 05 Nos |  | Gland packing | 25 Nos |
| 5 | Commonly used boiler spares | 05 Nos |  | oil filters | 25 Nos |
| 6 | Dial thermometer 0-600° C • Oil pressure gauge 0-600 psi | 05 Nos |  | Packing and gaskets | 25 Nos |
| 7 | diffuser plates | 05 Nos |  | Photo cell | 25 Nos |
| 8 | Fire extinguishers | 05 Nos |  | Record books | 25 Nos |
| 9 | Flame rod | 05 Nos |  | Sand buckets | 25 Nos |
| 10 | Gauge glass, rubber cones | 05 Nos |  | Set of gaskets | 25 Nos |
| 11 | General mechanic’s tools | 25 Nos |  | Chemicals (for testing & softening) | 10 lg |
| 12 | electricians’ basic tools | 25 Nos |  |  |  |
| 13 | ignition electrodes | 25 Nos |  |  |  |
| 14 | Pair of electrodes | 25 Nos |  |  |  |
| 15 | Personal protective equipment | 25 Nos |  |  |  |
| 16 | Pyrometer | 25 Nos |  |  |  |
| 17 | Set of nozzles and cleaning kit | 25 Nos |  |  |  |
| 18 | Smoke analyzing test kit | 05 Nos |  |  |  |
| 19 | Solenoid valve | 25 Nos |  |  |  |
| 20 | Thermostats | 25 Nos |  |  |  |
| 21 | Tube brush | 25 Nos |  |  |  |
| 22 | UV Scanner | 05 Nos |  |  |  |
| 23 | water softener plant | 01 No |  |  |  |

# NOTIFICATION

No. F. 5(13)/2018-DD (TE):     In pursuance of sub-section (d) of section-6" Functions of the Commission" National Vocational & Technical Training Commission (NAVTTC) Act-2011, NAVTTC is pleased  to approve and notify following qualifications in twenty (20) trades for Level 1-5 under National Vocational Qualification Framework (NVQF), which have been developed in compatibility with latest global trends in the fields and fulfilling requirements  of competency based training and assessment (CBT&A) system. The qualifications have been developed and validated in collaboration with TEVTAs, QABs, industry and other relevant stakeholders: -

S#	National Vocational Qualifications

- National Qualification Level-5 diploma in Automobile Technology

- National Qualification Level-5 diploma in Mechanical Technology

- National Qualification Level-5 diploma in Construction Technology

- National Qualification Level-5 diploma in Information & Commutation Technology (ICT)

- National Qualification Level-5 diploma in Garment Manufacturing Technology

- National Qualification Level-5 diploma in Electrical Technology

- National Qualification Level-5 diploma in Electronics Technology

- National Qualification Level-5 diploma in Instrumentation Technology

- National Qualification Level-5 diploma in Computer Aided Design & Manufacturing (CAD /CAM)

- National Qualification Level-5 diploma in Mechanical Technology

- National Qualification Level-5 diploma in Graphics Designing

- National Qualification Level-5 diploma in Heating, Ventilation, Air-conditioning & Refrigeration (HVACR) Technology

- National Qualification Level-5 diploma in Media Production

- National Qualification Level-5 diploma in Hotel Management

- National Qualification Level-5 diploma in Professional Chef

- National Qualification Level-5 diploma in Tourism Management

- National Qualification Level-5 diploma in Hair & Beauty Services

- National Qualification Level-5 diploma in Fashion Designing

- National Qualification Level-5 diploma in Ceramics Technology

- National Qualification Level-5 diploma in Telecom Technology

2.	All the TVET related institutions / organizations are required to implement aforementioned qualifications so that a uniform and standardized TVET qualification system is established in Pakistan and efforts are made for international equivalence / recognition of these qualifications.

3.	Competency Standards of the above enlisted qualifications can be accessed at NAVTTC's website (www.navttc.org).

(Muqeem Islam)

Director General (Skill Standards & Curricula)

Phone: 051-9215385

Distribution:

- Federal Secretary, Ministry of Federal Education & Professional Training, Govt of Pakistan

- Federal Secretary, Ministry of Overseas Pakistanis and Human Resource Development, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Industry and Production, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Textile Industry, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Commerce, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Railway, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Climate Change, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Religious Affairs, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Communication, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Aviation Division, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Science & Technology, Govt of Pakistan, Islamabad

- Chairperson, Punjab Technical Education and Vocational Training Authority (P-TEVTA), Lahore

- Managing Director, Khyber Pakhtunkhwa Technical Education and Vocational Training Authority (KP-TEVTA),

- Managing Director, Sindh Technical Education and Vocational Training Authority (S-TEVTA), Karachi

- Chairman, Azad Jammu & Kashmir, Technical Education and Vocational Training Authority (AJ&K TEVTA), Muzafarabad

- Director TVET Cell, Gilgit Baltistan, Gilgit

- Director General, Punjab Vocational Training Council (PVTC), Punjab

- Managing Director,  Technology Upgradation and Skill Development Company (TUSDEC) Lahore

- Project Director, Punjab Skill Development Program (PSDP) Lahore

- CEO, Punjab Skill Development Fund, Lahore

- Rector, UNTECH University Islamabad

- National Deputy Leader, GIZ Islamabad

- PS to Minister of Federal Education & Professional Training, Govt of Pakistan

- PS to Special Adviser to the Prime Minister on Youth Affairs, Prime Minister's Office, Islamabad

- Chairperson, Federal of Pakistan Chamber of Commerce and Industry (FPCCI), Karachi

- Conveyor, Sector Skills Council (Textile/ Construction/ Renewable Energy/ Hospitality and Tourism)

- Director Technical Education and Vocational Training Authorities (TEVTA), Balochistan

- Chairman, Pakistan Tourism Development Corporation, Lahore

- Chairman, PCSIR Headquarters, Islamabad

- Director General, Pakistan Forest Institute, Peshawar

- Chairman, Wafaq ul Madaris, Multan

- Director General, Staff Welfare, Islamabad

- Director General, NISTE Capital Administration and Development Division, Islamabad

- Director General, National Training Bureau, Islamabad

- Chairmen, Provincial Technical Education Boards

- Chairmen, Provincial Trade Testing Boards

- Secretary, IBCC, Islamabad: with the request that National qualifications of Level 5 diploma in the aforementioned trades may be considered equivalent to Diploma of Associate Engineer/HSSC after inclusion of compulsory courses in the light of IBCC general requirement.

Copy for information to: -

- DG (P&D)/(A&F)/ (A&C) (S&C) NAVTTC

- Director General(s), NAVTTC Regional Office(s).

- Sr. Technical Advisor, TSSP-GIZ