Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 3 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Prepare and Install Distribution Boards |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Prepare estimate for wiring material - Prepare Distribution Board - Install Distribution Board and wiring |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Prepare estimate for wiring material - Develop Basic Drawing - Perform Estimation of Materials - Calculate Labor Cost Prepare Distribution Board - Select Distribution board w.r.t. size, current rating, voltage, No. of C.Bs and phases. - Select Main Incoming Residual Current Circuit Breaker (RCCB) having minimum sensitivity. - Select outgoing Miniature Circuit Breaker (MCB) according to load. - Provide space for future MCB. - Select Voltmeter, ampere meter and indicator according to load and phases. - Select wire according to load and for wiring in DB. - Make neutral common for all load. - Connect accessories according to the circuit Install Distribution Board and wiring - Fix the distribution board. - Take wire from DB to load without joint. - Distribute load equally on all phases. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 3 in Mechatronics Technology |
| Competency Standard | Prepare and Install Distribution Boards |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare estimate for wiring material - Prepare Distribution Board - Install Distribution Board and wiring |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Develop Basic Drawing |  |  |
| - Perform Estimation of Materials |  |  |
| - Calculate Labor Cost |  |  |
| - Select Distribution board w.r.t. size, current rating, voltage, No. of C.Bs and phases. |  |  |
| - Select Main Incoming Residual Current Circuit Breaker (RCCB) having minimum sensitivity. |  |  |
| - Select outgoing Miniature Circuit Breaker (MCB) according to load. |  |  |
| - Provide space for future MCB. |  |  |
| - Select Voltmeter, ampere meter and indicator according to load and phases. |  |  |
| - Select wire according to load and for wiring in DB. |  |  |
| - Make neutral common for all load |  |  |
| - Connect accessories according to the circuit |  |  |
| - Fix the distribution board. |  |  |
| - Take wire from DB to load without joint |  |  |
| - Distribute load equally on all phases |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 3 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Prepare and Install Distribution Boards |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare estimate for wiring material - Prepare Distribution Board - Install Distribution Board and wiring |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Develop Basic Drawing |  |  |  |  |  |
|  | Perform Estimation of Materials |  |  |  |  |  |
|  | Calculate Labor Cost |  |  |  |  |  |
|  | Select Distribution board w.r.t. size, current rating, voltage, No. of C.Bs and phases. |  |  |  |  |  |
|  | Select Main Incoming Residual Current Circuit Breaker (RCCB) having minimum sensitivity. |  |  |  |  |  |
|  | Select outgoing Miniature Circuit Breaker (MCB) according to load. |  |  |  |  |  |
|  | Provide space for future MCB. |  |  |  |  |  |
|  | Select Voltmeter, ampere meter and indicator according to load and phases. |  |  |  |  |  |
|  | Select wire according to load and for wiring in DB. |  |  |  |  |  |
|  | Make neutral common for all load. |  |  |  |  |  |
|  | Connect accessories according to the circuit |  |  |  |  |  |
|  | Fix the distribution board. |  |  |  |  |  |
|  | Fix the distribution board. |  |  |  |  |  |
|  | Take wire from DB to load without joint. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 3 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Prepare and Install Distribution Boards |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________