Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Mechatronics Technology. |
| --- | --- |
| Competency Standard | Analyze DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Calculate different terms used in armature winding of DC machines - Develop Single Layer Lap Winding - Develop Double Layer Wave Winding - Measure the resistance of field winding, armature winding and terminal polarity - Identify DC shunt generator - Identify DC series generator - Identify DC compound generator |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Calculate different terms used in armature winding of DC machines - Count the number of slots (S) of armature. - Calculate pole pitch (Y). - Calculate Coil span/pitch. - Calculate Back pitch (Yb) - Calculate Front pitch (Yf) - Calculate Resultant Pitch (YR) - Calculate commentator pitch (Yc) - Develop Single Layer Lap Winding - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Subtract the value of back pitch from the added value. - Connect added value from front side to subtracted value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. - Develop Double Layer Wave Winding - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Add the value of front pitch in previous added value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. - Measure the resistance of field winding, armature winding and terminal polarity - Open the terminal box. - Connect the ohm meter with the terminal of both winding. - Read down ohm meter reading. - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. - Rotate the generator slowly by hand. - Touch the probe of galvanometer with each winding. - Identify armature with the deflection of galvanometer pointer. - Mark the polarity of terminals with the deflection of galvanometer. - Identify DC shunt generator - Measure the terminal resistance of generator with ohm meter. - Identify DC shunt generator from measured value of resistance. - Disconnect both windings. - Identify the shunt field winding of shunt generator. - Identify armature of shunt generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. - Identify DC series generator - Measure the terminal resistance of generator with ohm meter. - Identify DC series generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series generator. - Identify armature of series generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. - Identify DC compound generator - Measure the terminal resistance of generator with ohm meter. - Identify DC Compound generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of generator. - Identify the shunt field winding of generator. - Identify armature of shunt generator. - Measure and record the resistance of both winding separately. - Measure and Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 4 in Mechatronics Technology |
| Competency Standard | Analyze DC Generators |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Calculate different terms used in armature winding of DC machines - Develop Single Layer Lap Winding - Develop Double Layer Wave Winding - Measure the resistance of field winding, armature winding and terminal polarity - Identify DC shunt generator - Identify DC series generator - Identify DC compound generator |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Count the number of slots (S) of armature. |  |  |
| - Calculate pole pitch (Y). |  |  |
| - Calculate Coil span/pitch. |  |  |
| - Calculate Back pitch (Yb) |  |  |
| - Calculate Front pitch (Yf) |  |  |
| - Calculate Resultant Pitch (YR) |  |  |
| - Calculate commentator pitch (Yc) |  |  |
| - Count the number of slot in armature core and numbering on it. |  |  |
| - Calculate total number of conductors (Z) |  |  |
| - Calculate average pitch, back pitch and commentator pitch. |  |  |
| - Start winding from slot no 1. |  |  |
| - Add the value of back pitch in conductor. |  |  |
| - Connect conductor 1 from back side to the added value. |  |  |
| - Subtract the value of back pitch from the added value. |  |  |
| - Connect added value from front side to subtracted value. |  |  |
| - Continue this process till completion of all conductors. |  |  |
| - Connect armature segment with armature conductor. |  |  |
| - Insert brushes in proper position. |  |  |
| - Count the number of slot in armature core and numbering on it. |  |  |
| - Calculate total number of conductors (Z) |  |  |
| - Calculate average pitch, back pitch and commentator pitch. |  |  |
| - Start winding from slot no 1. |  |  |
| - Add the value of back pitch in conductor. |  |  |
| - Connect conductor 1 from back side to the added value. |  |  |
| - Add the value of front pitch in previous added value. |  |  |
| - Continue this process till completion of all conductors. |  |  |
| - Connect armature segment with armature conductor. |  |  |
| - Insert brushes in proper position. |  |  |
| - Open the terminal box. |  |  |
| - Connect the ohm meter with the terminal of both winding. |  |  |
| - Read down ohm meter reading. |  |  |
| - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. |  |  |
| - Rotate the generator slowly by hand. |  |  |
| - Touch the probe of galvanometer with each winding. |  |  |
| - Identify armature with the deflection of galvanometer pointer. |  |  |
| - Mark the polarity of terminals with the deflection of galvanometer. |  |  |
| - Measure the terminal resistance of generator with ohm meter. |  |  |
| - Identify DC shunt generator from measured value of resistance. |  |  |
| - Disconnect both windings. |  |  |
| - Identify the shunt field winding of shunt generator. |  |  |
| - Identify armature of shunt generator. |  |  |
| - Measure the resistance of both winding separately. |  |  |
| - Record the resistance of armature winding and field winding. |  |  |
| - Compare the resistance of armature winding and field winding to identify the difference between them. |  |  |
| - Measure the terminal resistance of generator with ohm meter. |  |  |
| - Identify DC series generator from measured value of resistance. |  |  |
| - Disconnect both windings. |  |  |
| - Identify the series field winding of series generator. |  |  |
| - Identify armature of series generator. |  |  |
| - Measure the resistance of both winding separately. |  |  |
| - Record the resistance of armature winding and field winding. |  |  |
| - Compare the resistance of armature winding and field winding to identify the difference between them. |  |  |
| - Measure the terminal resistance of generator with ohm meter. |  |  |
| - Identify DC Compound generator from measured value of resistance. |  |  |
| - Disconnect both windings. |  |  |
| - Identify the series field winding of generator. |  |  |
| - Identify the shunt field winding of generator. |  |  |
| - Identify armature of shunt generator. |  |  |
| - Measure and record the resistance of both winding separately. |  |  |
| - Measure and Record the resistance of armature winding. |  |  |
| - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Analyze DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Calculate different terms used in armature winding of DC machines - Develop Single Layer Lap Winding - Develop Double Layer Wave Winding - Measure the resistance of field winding, armature winding and terminal polarity Identify DC shunt generator - Identify DC series generator - identify DC compound generator |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Counted the number of slots (S) of armature. |  |  |  |  |  |
|  | Calculated pole pitch (Y). |  |  |  |  |  |
|  | Calculated Coil span/pitch. |  |  |  |  |  |
|  | Calculated Back pitch (Yb) |  |  |  |  |  |
|  | Calculated Front pitch (Yf) |  |  |  |  |  |
|  | Calculated Resultant Pitch (YR) |  |  |  |  |  |
|  | Calculated commentator pitch (Yc) |  |  |  |  |  |
|  | Counted the number of slot in armature core and numbering on it. |  |  |  |  |  |
|  | Calculated total number of conductors (Z) |  |  |  |  |  |
|  | Calculated average pitch, back pitch and commentator pitch. |  |  |  |  |  |
|  | Started winding from slot no 1. |  |  |  |  |  |
|  | Added the value of back pitch in conductor. |  |  |  |  |  |
|  | Connected conductor 1 from back side to the added value. |  |  |  |  |  |
|  | Subtracted the value of back pitch from the added value. |  |  |  |  |  |
|  | Connected added value from front side to subtracted value. |  |  |  |  |  |
|  | Continued this process till completion of all conductors. |  |  |  |  |  |
|  | Connected armature segment with armature conductor. |  |  |  |  |  |
|  | Inserted brushes in proper position. |  |  |  |  |  |
|  | Counted the number of slot in armature core and numbering on it. |  |  |  |  |  |
|  | Calculated total number of conductors (Z) |  |  |  |  |  |
|  | Calculated average pitch, back pitch and commentator pitch. |  |  |  |  |  |
|  | Started winding from slot no 1. |  |  |  |  |  |
|  | Added the value of back pitch in conductor. |  |  |  |  |  |
|  | Connected conductor 1 from back side to the added value. |  |  |  |  |  |
|  | Added the value of front pitch in previous added value. |  |  |  |  |  |
|  | Continued this process till completion of all conductors. |  |  |  |  |  |
|  | Connected armature segment with armature conductor. |  |  |  |  |  |
|  | Inserted brushes in proper position. |  |  |  |  |  |
|  | Opened the terminal box. |  |  |  |  |  |
|  | Connected the ohm meter with the terminal of both winding. |  |  |  |  |  |
|  | Read down ohm meter reading. |  |  |  |  |  |
|  | Connected the probe of galvanometer with any terminal and touch other probe to other terminals one by one. |  |  |  |  |  |
|  | Rotated the generator slowly by hand. |  |  |  |  |  |
|  | Touched the probe of galvanometer with each winding. |  |  |  |  |  |
|  | Identified armature with the deflection of galvanometer pointer. |  |  |  |  |  |
|  | Marked the polarity of terminals with the deflection of galvanometer. |  |  |  |  |  |
|  | Measured the terminal resistance of generator with ohm meter. |  |  |  |  |  |
|  | Identified DC shunt generator from measured value of resistance. |  |  |  |  |  |
|  | Disconnected both windings. |  |  |  |  |  |
|  | Identified the shunt field winding of shunt generator. |  |  |  |  |  |
|  | Identified armature of shunt generator. |  |  |  |  |  |
|  | Measured the resistance of both winding separately. |  |  |  |  |  |
|  | Recorded the resistance of armature winding and field winding. |  |  |  |  |  |
|  | Compared the resistance of armature winding and field winding to identify the difference between them. |  |  |  |  |  |
|  | Measured the terminal resistance of generator with ohm meter. |  |  |  |  |  |
|  | Identified DC series generator from measured value of resistance. |  |  |  |  |  |
|  | Disconnected both windings. |  |  |  |  |  |
|  | Identified the series field winding of series generator. |  |  |  |  |  |
|  | Identified armature of series generator. |  |  |  |  |  |
|  | Measured the resistance of both winding separately. |  |  |  |  |  |
|  | Recorded the resistance of armature winding and field winding. |  |  |  |  |  |
|  | Compared the resistance of armature winding and field winding to identify the difference between them. |  |  |  |  |  |
|  | Measured the terminal resistance of generator with ohm meter. |  |  |  |  |  |
|  | Identified DC Compound generator from measured value of resistance. |  |  |  |  |  |
|  | Disconnected both windings. |  |  |  |  |  |
|  | Identified the series field winding of generator. |  |  |  |  |  |
|  | Identified the shunt field winding of generator. |  |  |  |  |  |
|  | Identified armature of shunt generator. |  |  |  |  |  |
|  | Measured and record the resistance of both winding separately. |  |  |  |  |  |
|  | Measured and Record the resistance of armature winding. |  |  |  |  |  |
|  | Compared the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Analyze DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is pole pitch (Y)? |  |  |
|  | How many parallel paths in lap winding? |  |  |
|  | What is mean by double layer winding? |  |  |
|  | What is different between field winding and armature winding? |  |  |
|  | Explain the technique to identify the terminal polarity |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________