Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Mechatronics Technology. |
| --- | --- |
| Competency Standard | Perform Tests On DC Motors |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Plot the load characteristics of D.C shunt motor - Plot the load characteristics of D.C series motor. - Plot the load characteristics of D.C compound motor - Control the speed of DC series motor. - Control the speed of DC shunt motor. - Start DC shunt motor with 3-point starter. - Identify the effect of back EMF in DC motor. - Calculate Brake Horse Power (BHP) of motor by Brake test. - Calculate efficiency of the motor by Swinburne Test - Perform Regenerative / Hopkinson/ Back to back Test |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Plot the load characteristics of D.C shunt motor - Couple electro dynamo meter to the DC motor with belt - Connect shunt motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC shunt motor - Plot the load characteristics of D.C series motor - Select DC series motor and achieve connection as per diagram. - Turn on DC power supply and increase the load to full value. - Record the value of speed and current on each load. - Draw the graph between load current and speed. - Plot the load characteristics of D.C compound motor - Couple electro dynamo meter to the DC motor with belt - Connect compound motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC compound motor - Control the speed of DC series motor - Make the connection as per circuit diagram. - Insert external resistances in series to armature and field coil. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different supply voltages. - Make graph between speed and applied voltage. - Control the speed of DC shunt motor - Make the connection as per circuit diagram. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different field current. - Make graph between speed and field current - Fixed the field current and very armature current - Record the speed at different armature current. - Make graph between speed and armature current - Start DC shunt motor with 3 point starter - Make the connection as per circuit diagram. - Switch on the supply and move starter handle from off position to position 1. - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. - Identify the effect of back EMF in DC motor - Select DC series motor and make connection as per diagram. - Turn on DC power supply and increase the load 0 to full value. - Record the value of speed and current on each load. - Calculate back EMF by using the recoded values and appropriate formula. - Make graph between speed and back EMF and interpret the result. - Calculate Brake Horse Power (BHP) of motor by Brake test - Connect the spring balance with one end of the rope. - Connect other end of rope with weight (W1). - Measure the radius of the pulley. - Hang the weight on the pulley of the motor and fix the spring balance end with earth. - Run the motor. - Measured the RPM of the motor. - Adjust the weight to pass full load current from motor. - Calculate spring balance weight and hanged weight (W1) - Calculate shaft torque. - Calculate BHP of motor by shaft torque. - Calculate efficiency of the motor by Swinburne Test - Perform DC test to find the value armature resistance and field resistance - Run the motor at rated voltage without load. - Adjust the value of shunt regulator to attain rated speed. - Measure Field current and No load current with ampere meter. - Calculate total copper losses and iron losses. - Calculate input and output power. - Calculate efficiency from calculated power. - Perform Regenerative / Hopkinson/ Back to back Test - Identify two shunt machines. - Couple the shafts of both machines. - Connect the machines electrically in such a way that one machine work as a motor and other as a generator. - Connect external power supply to the machines to overcome the machine losses. - Run the both machine at no load. - Reduce the field excitation of one machine which work as a motor. - Increase the field excitation of one machine which work as a generator. - Adjust the field winding so that voltage of generator becomes equal to the applied voltage which result the machines runs in parallel - Calculate the efficiency by using reading of voltmeter and ammeter. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 4 in Mechatronics Technology |
| Competency Standard | Perform Tests On DC Motors |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Plot the load characteristics of D.C shunt motor - Plot the load characteristics of D.C series motor. - Plot the load characteristics of D.C compound motor - Control the speed of DC series motor. - Control the speed of DC shunt motor. - Start DC shunt motor with 3 point starter. - Identify the effect of back EMF in DC motor. - Calculate Brake Horse Power (BHP) of motor by Brake test. - Calculate efficiency of the motor by Swinburne Test - Perform Regenerative / Hopkinson/ Back to back Test |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Couple electro dynamo meter to the DC motor with belt |  |  |
| - Connect shunt motor according to the diagram. |  |  |
| - Set the dynamo meter control knob in such position to produce a minimum starting load. |  |  |
| - Turn on the power supply |  |  |
| - Run the motor at rated RPM. |  |  |
| - Apply a load to DC motor by varying the dynamo meter control knob. |  |  |
| - Measure and calculate the current, speed and torque. |  |  |
| - Plot the load characteristics on DC shunt motor |  |  |
| - Select DC series motor and achieve connection as per diagram. |  |  |
| - Turn on DC power supply and increase the load to full value. |  |  |
| - Record the value of speed and current on each load. |  |  |
| - Draw the graph between load current and speed |  |  |
| - Couple electro dynamo meter to the DC motor with belt |  |  |
| - Connect compound motor according to the diagram. |  |  |
| - Set the dynamo meter control knob in such position to produce a minimum starting load. |  |  |
| - Turn on the power supply |  |  |
| - Run the motor at rated RPM. |  |  |
| - Apply a load to DC motor by varying the dynamo meter control knob. |  |  |
| - Measure and calculate the current, speed and torque. |  |  |
| - Plot the load characteristics on DC compound motor |  |  |
| - Make the connection as per circuit diagram. |  |  |
| - Insert external resistances in series to armature and field coil. |  |  |
| - Switch on the supply and increase the voltage gradually to its rated voltage. |  |  |
| - Record speed at different supply voltages. |  |  |
| - Make graph between speed and applied voltage. |  |  |
| - Make the connection as per circuit diagram. |  |  |
| - Switch on the supply and increase the voltage gradually to its rated voltage. |  |  |
| - Record speed at different field current. |  |  |
| - Make graph between speed and field current |  |  |
| - Fixed the field current and very armature current |  |  |
| - Record the speed at different armature current. |  |  |
| - Make graph between speed and armature current |  |  |
| - Make the connection as per circuit diagram. |  |  |
| - Switch on the supply and move starter handle from off position to position 1. |  |  |
| - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. |  |  |
| - Select DC series motor and make connection as per diagram. |  |  |
| - Turn on DC power supply and increase the load 0 to full value. |  |  |
| - Record the value of speed and current on each load. |  |  |
| - Calculate back EMF by using the recoded values and appropriate formula. |  |  |
| - Make graph between speed and back EMF and interpret the result. |  |  |
| - Connect the spring balance with one end of the rope. |  |  |
| - Connect other end of rope with weight (W1). |  |  |
| - Measure the radius of the pulley. |  |  |
| - Hang the weight on the pulley of the motor and fix the spring balance end with earth. |  |  |
| - Run the motor. |  |  |
| - Measured the RPM of the motor. |  |  |
| - Adjust the weight to pass full load current from motor. |  |  |
| - Calculate spring balance weight and hanged weight (W1) |  |  |
| - Calculate shaft torque. |  |  |
| - Calculate BHP of motor by shaft torque. |  |  |
| - Perform DC test to find the value armature resistance and field resistance |  |  |
| - Run the motor at rated voltage without load. |  |  |
| - Adjust the value of shunt regulator to attain rated speed. |  |  |
| - Measure Field current and No load current with ampere meter. |  |  |
| - Calculate total copper losses and iron losses. |  |  |
| - Calculate input and output power. |  |  |
| - Calculate efficiency from calculated power. |  |  |
| - Identify two shunt machines. |  |  |
| - Couple the shafts of both machines. |  |  |
| - Connect the machines electrically in such a way that one machine work as a motor and other as a generator. |  |  |
| - Connect external power supply to the machines to overcome the machine losses. |  |  |
| - Run the both machine at no load. |  |  |
| - Reduce the field excitation of one machine which work as a motor. |  |  |
| - Increase the field excitation of one machine which work as a generator. |  |  |
| - Adjust the field winding so that voltage of generator becomes equal to the applied voltage which result the machines runs in parallel |  |  |
| - Calculate the efficiency by using reading of voltmeter and ammeter. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Perform Tests On DC Motors |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Plot the load characteristics of D.C shunt motor - Plot the load characteristics of D.C series motor. - Plot the load characteristics of D.C compound motor - Control the speed of DC series motor. - Control the speed of DC shunt motor - .Start DC shunt motor with 3 point starter. - Identify the effect of back EMF in DC motor. - Calculate Brake Horse Power (BHP) of motor by Brake test. - Calculate efficiency of the motor by Swinburne Test - Perform Regenerative / Hopkinson/ Back to back Test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Coupled electro dynamo meter to the DC motor with belt |  |  |  |  |  |
|  | Connected shunt motor according to the diagram. |  |  |  |  |  |
|  | Set the dynamo meter control knob in such position to produce a minimum starting load. |  |  |  |  |  |
|  | Turned on the power supply |  |  |  |  |  |
|  | Ran the motor at rated RPM. |  |  |  |  |  |
|  | Applied a load to DC motor by varying the dynamo meter control knob. |  |  |  |  |  |
|  | Measured and calculated the current, speed and torque. |  |  |  |  |  |
|  | Plotted the load characteristics on DC shunt motor |  |  |  |  |  |
|  | Selected DC series motor and achieve connection as per diagram. |  |  |  |  |  |
|  | Turned on DC power supply and increase the load to full value. |  |  |  |  |  |
|  | Recorded the value of speed and current on each load. |  |  |  |  |  |
|  | Draw the graph between load current and speed |  |  |  |  |  |
|  | Coupled electro dynamo meter to the DC motor with belt |  |  |  |  |  |
|  | Connected compound motor according to the diagram. |  |  |  |  |  |
|  | Set the dynamo meter control knob in such position to produce a minimum starting load. |  |  |  |  |  |
|  | Turned on the power supply |  |  |  |  |  |
|  | Ran the motor at rated RPM. |  |  |  |  |  |
|  | Applied a load to DC motor by varying the dynamo meter control knob. |  |  |  |  |  |
|  | Measured and calculated the current, speed and torque. |  |  |  |  |  |
|  | Plotted the load characteristics on DC compound motor |  |  |  |  |  |
|  | Made the connection as per circuit diagram. |  |  |  |  |  |
|  | Inserted external resistances in series to armature and field coil. |  |  |  |  |  |
|  | Switched on the supply and increase the voltage gradually to its rated voltage. |  |  |  |  |  |
|  | Recorded speed at different supply voltages. |  |  |  |  |  |
|  | Made graph between speed and applied voltage. |  |  |  |  |  |
|  | Made the connection as per circuit diagram. |  |  |  |  |  |
|  | Switched on the supply and increased the voltage gradually to its rated voltage. |  |  |  |  |  |
|  | Recorded speed at different field current. |  |  |  |  |  |
|  | Made graph between speed and field current |  |  |  |  |  |
|  | Fixed the field current and changed armature current |  |  |  |  |  |
|  | Recorded the speed at different armature current. |  |  |  |  |  |
|  | Made graph between speed and armature current |  |  |  |  |  |
|  | Made the connection as per circuit diagram. |  |  |  |  |  |
|  | Switched on the supply and moved starter handle from off position to position 1. |  |  |  |  |  |
|  | Moved the starter arm from position 1 to position 2 and continued this process till starter arm reach at its extreme position. |  |  |  |  |  |
|  | Selected DC series motor and make connection as per diagram. |  |  |  |  |  |
|  | Turn on DC power supply and increased the load 0 to full value. |  |  |  |  |  |
|  | Recorded the value of speed and current on each load. |  |  |  |  |  |
|  | Calculated back EMF by using the recoded values and appropriate formula. |  |  |  |  |  |
|  | Made graph between speed and back EMF and interpreted the result. |  |  |  |  |  |
|  | Connected the spring balance with one end of the rope. |  |  |  |  |  |
|  | Connected other end of rope with weight (W1). |  |  |  |  |  |
|  | Measured the radius of the pulley. |  |  |  |  |  |
|  | Hanged the weight on the pulley of the motor and fixed the spring balance end with earth. |  |  |  |  |  |
|  | Ran the motor. |  |  |  |  |  |
|  | Measured the RPM of the motor. |  |  |  |  |  |
|  | Adjusted the weight to pass full load current from motor. |  |  |  |  |  |
|  | Calculated spring balance weight and hanged weight (W1) |  |  |  |  |  |
|  | Calculated shaft torque. |  |  |  |  |  |
|  | Calculated BHP of motor by shaft torque. |  |  |  |  |  |
|  | Performed DC test to find the value armature resistance and field resistance |  |  |  |  |  |
|  | Ran the motor at rated voltage without load. |  |  |  |  |  |
|  | Adjusted the value of shunt regulator to attain rated speed. |  |  |  |  |  |
|  | Measured Field current and No load current with ampere meter. |  |  |  |  |  |
|  | Calculated total copper losses and iron losses. |  |  |  |  |  |
|  | Calculated input and output power. |  |  |  |  |  |
|  | Calculated efficiency from calculated power. |  |  |  |  |  |
|  | Identified two shunt machines. |  |  |  |  |  |
|  | Coupled the shafts of both machines. |  |  |  |  |  |
|  | Connected the machines electrically in such a way that one machine work as a motor and other as a generator. |  |  |  |  |  |
|  | Connected external power supply to the machines to overcome the machine losses. |  |  |  |  |  |
|  | Ran the both machine at no load. |  |  |  |  |  |
|  | Reduced the field excitation of one machine which work as a motor. |  |  |  |  |  |
|  | Increased the field excitation of one machine which work as a generator. |  |  |  |  |  |
|  | Adjusted the field winding so that voltage of generator becomes equal to the applied voltage which result the machines runs in parallel |  |  |  |  |  |
|  | Calculated the efficiency by using reading of voltmeter and ammeter. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Mechatronics Technology |
| --- | --- |
| Competency Standard | Perform Tests On DC Motors |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | Why we never start DC series motor without load. |  |  |  |
|  | Explain the effect of field current on motor speed |  |  |  |
|  | What is the different between shunt, series and compound motor load characteristics? |  |  |  |
|  | How many techniques to control the speed of DC shunt motor? |  |  |  |
|  | What is 3-point starter? |  |  |  |
|  | How to calculate BHP from shaft torque? |  |  |  |
|  | Why we find copper loss at full load. |  |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________