Instruction Sheet for the Candidate

| Candidate Details | Name________________________________________________________ Registration/Roll Number______________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): CU-1: Caliberate Bourdon Tube using dead wight Pressure gauge and master gauge Caliberator CU-2: Caliberate Diaphragm Pressure Gauge using dead wight pressure gauge and master gauge Caliberator |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Connect Bourdon Tube with the dead weight pressure gauge. - Fill the cylinder of dead weight pressure gauge with water. - Place a known weight on the plunger of the dead weight pressure gauge. - Note the pressure reading of the gauge to be calibrated. - Add more weights of known values and note corresponding pressure readings. - Compare these readings with the values in the table. - Also compare dead weight gauge with the master gauge. - Connect the Diaphragm Pressure Gauge with the dead weight pressure gauge. - Fill the cylinder of dead weight pressure gauge with water. - Place a known weight on the plunger of the dead weight pressure gauge. - Note the pressure reading of the gauge to be calibrated. - Add more weights of known values and note corresponding pressure readings. - Compare these readings with the values in the table. - Also compare dead weight gauge with the master gauge. |
| Minimum Evidence Required |  |

| Qualification | National Certificate Level -5 in Mechatronics Technology (Hydraulics and Pneumatics) |
| --- | --- |
| Competency Standard(s) | Calibrate Bourdon Tube and Diaphragm Pressure Gauge |
| Purpose of Assessment | Formative Assessment |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Certificate Level - 5 in Mechatronics Technology National Certificate Level - 5 in Mechatronics Technology |
| Competency Standards | Calibrate Bourdon Tube and Diaphragm Pressure Gauge |
| Assessment Task | Caliberate Bourdon Tube using dead wight Pressure gauge and master gauge Caliberator Caliberate Diaphragm Pressure Gauge using dead wight pressure gauge and master gauge Caliberator |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Connect Bourdon Tube with the dead weight pressure gauge. |  |  |
| - Fill the cylinder of dead weight pressure gauge with water. |  |  |
| - Place a known weight on the plunger of the dead weight pressure gauge. |  |  |
| - Note the pressure reading of the gauge to be calibrated. |  |  |
| - Add more weights of known values and note corresponding pressure readings. |  |  |
| - Compare these readings with the values in the table. |  |  |
| - Also compare dead weight gauge with the master gauge. |  |  |
| - Connect the Diaphragm Pressure Gauge with the dead weight pressure gauge. |  |  |
| - Fill the cylinder of dead weight pressure gauge with water. |  |  |
| - Place a known weight on the plunger of the dead weight pressure gauge. |  |  |
| - Note the pressure reading of the gauge to be calibrated. |  |  |
| - Add more weights of known values and note corresponding pressure readings. |  |  |
| - Compare these readings with the values in the table. |  |  |
| - Also compare dead weight gauge with the master gauge. |  |  |
| - Connect Bourdon Tube with the dead weight pressure gauge. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Calibrate Bourdon Tube and Diaphragm Pressure Gauge |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________Signature: __________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Connected Bourdon Tube with the dead weight pressure gauge. |  |  |  |  |  |
|  | Filled the cylinder of dead weight pressure gauge with water. |  |  |  |  |  |
|  | Connected Bourdon Tube with the dead weight pressure gauge. |  |  |  |  |  |
|  | Placed a known weight on the plunger of the dead weight pressure gauge. |  |  |  |  |  |
|  | Noted the pressure reading of the gauge to be calibrated. |  |  |  |  |  |
|  | Added more weights of known values and note corresponding pressure readings. |  |  |  |  |  |
|  | Compared these readings with the values in the table. |  |  |  |  |  |
|  | Also compared dead weight gauge with the master gauge. |  |  |  |  |  |
|  | Connected the Diaphragm Pressure Gauge with the dead weight pressure gauge. |  |  |  |  |  |
|  | Filled the cylinder of dead weight pressure gauge with water. |  |  |  |  |  |
|  | Placed a known weight on the plunger of the dead weight pressure gauge. |  |  |  |  |  |
|  | Noted the pressure reading of the gauge to be calibrated. |  |  |  |  |  |
|  | Added more weights of known values and noted corresponding pressure readings. |  |  |  |  |  |
|  | Compared the readings with the values in the table. |  |  |  |  |  |
|  | Also compared dead weight gauge with the master gauge |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Calibrate Bourdon Tube and Diaphragm Pressure Gauge |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define a pressure gauge |  |  |
|  | Describe basic types of pressure gauges |  |  |
|  | Describe Bourdon tube gauge |  |  |
|  | Differentiate between measuring gauge and calibration gauge |  |  |
|  | Describe dead weight pressure gauge |  |  |
|  | Describe Pascal’s Law |  |  |
|  | Describe Diaphragm pressure gauge |  |  |
|  | Describe master gauge calibrator |  |  |
|  | Describe calibration Method and Frequency |  |  |
|  | Describe Pascal’s Law |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________