Instruction Sheet for the Candidate

| Qualification | National Certificate Level - 5 in Mechatronics Technology ( Hydraulics and Pneumatics Level-) |
| --- | --- |
| Competency Standard(s) | Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number______________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): CU-1: Construct a hydraulic circuit for double acting hydraulic cyclinder for mechanical interlocking with switch CU-2: Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch CU-3: Construct hydraulic circuit using Accumulator stored Energy by DAC CU4: Measure pressure at various connections in hydraulic circuits |
| Time: 2 hours | During a practical assessment, under observation by an assessor, you are required to - Complete the hydraulic circuit as per drawings. - Turn on the circuit supply and check if all devices are working properly - Operate hydraulic circuit - Try lift some weight - Drop the pressure using pressure gauge before the piston starts moving - Construct Hydraulic Circuit Diagram - Construct electric circuit diagram - Prepare simulation circuit of double acting cylinder as per drawing - Fix the push button and limit switch at their designated place as per drawing - Turn on the circuit power supply and ensure proper working of all equipment. - Press the push button to start fluid supply and will enable the cylinder for forward stroke supply. - Observe that as the push rod touches limit switch the supply of coil would disconnect and the piston would stop. - Press the second push button to complete backward stroke of piston, this will enable the supply the supply would be disconnected. - Change 5/2 DCV and complete the opposite circuit. - Observe this changed circuit and note down all the observations. - Complete the hydraulic circuit as per drawing. - Turn on the motor power supply and ensure proper working of all devises and equipment. - Use flow control valve to ensure fluid flow. - Observe the movement of piston rod. - When the circuit is burnt/completed the turn off the power supply. - Observe that due to accumulator the double acting cylinder completes its stroke and doesn’t stop immediately after the burning of circuit. - Prepare the hydraulic circuit as per drawing - Install pressure gauges on the points where pressure is to be noted - Turn on the hydraulic circuit - Ensure fluid flow through the whole circuit - Complete the circuit and note the readings of all gauges - Enter all readings in observation table |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Certificate Level - 5 in Mechatronics Technology National Certificate Level - 5 in Mechatronics Technology |
| Competency Standards | Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking |
| Assessment Task | - Construct a hydraulic circuit for double acting hydraulic cyclinder for mechanical interlocking with switch - Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch - Construct hydraulic circuit using Accumulator stored Energy by DAC - Measure pressure at various connections in hydraulic circuits |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Complete the hydraulic circuit as per drawings. |  |  |
| - Turn on the circuit supply and check if all devices are working properly |  |  |
| - Operate hydraulic circuit |  |  |
| - Try lift some weight |  |  |
| - Drop the pressure using pressure gauge before the piston starts moving |  |  |
| - Construct Hydraulic Circuit Diagram |  |  |
| - Construct electric circuit diagram |  |  |
| - Prepare simulation circuit of double acting cylinder as per drawing |  |  |
| - Fix the push button and limit switch at their designated place as per drawing |  |  |
| - Turn on the circuit power supply and ensure proper working of all equipment. |  |  |
| - Press the push button to start fluid supply and will enable the cylinder for forward stroke supply. |  |  |
| - Observe that as the push rod touches limit switch the supply of coil would disconnect and the piston would stop. |  |  |
| - Press the second push button to complete backward stroke of piston, this will enable the supply the supply would be disconnected. |  |  |
| - Change 5/2 DCV and complete the opposite circuit. |  |  |
| - Observe this changed circuit and note down all the observations. |  |  |
| - Complete the hydraulic circuit as per drawing. |  |  |
| - Turn on the motor power supply and ensure proper working of all devises and equipment. |  |  |
| - Use flow control valve to ensure fluid flow. |  |  |
| - Observe the movement of piston rod. |  |  |
| - When the circuit is burnt/completed the turn off the power supply. |  |  |
| - Observe that due to accumulator the double acting cylinder completes its stroke and doesn’t stop immediately after the burning of circuit. |  |  |
| - Prepare the hydraulic circuit as per drawing |  |  |
| - Install pressure gauges on the points where pressure is to be noted |  |  |
| - Turn on the hydraulic circuit |  |  |
| - Ensure fluid flow through the whole circuit |  |  |
| - Complete the circuit and note the readings of all gauges |  |  |
| - Enter all readings in observation table |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Certificate Level - 5 in Mechatronics Technology National Certificate Level - 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________ Signature: __________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Construct a hydraulic circuit for double acting hydraulic cyclinder for mechanical interlocking with switch - Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch - Construct hydraulic circuit using Accumulator stored Energy by DAC - Measure pressure at various connections in hydraulic circuits |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Complete the hydraulic circuit as per drawings. |  |  |  |  |  |
|  | Turn on the circuit supply and check if all devices are working properly |  |  |  |  |  |
|  | Operate hydraulic circuit |  |  |  |  |  |
|  | Try lift some weight |  |  |  |  |  |
|  | Drop the pressure using pressure gauge before the piston starts moving |  |  |  |  |  |
|  | Construct Hydraulic Circuit Diagram |  |  |  |  |  |
|  | Construct electric circuit diagram |  |  |  |  |  |
|  | Prepare simulation circuit of double acting cylinder as per drawing |  |  |  |  |  |
|  | Fix the push button and limit switch at their designated place as per drawing |  |  |  |  |  |
|  | Turn on the circuit power supply and ensure proper working of all equipment. |  |  |  |  |  |
|  | Press the push button to start fluid supply and will enable the cylinder for forward stroke supply. |  |  |  |  |  |
|  | Observe that as the push rod touches limit switch the supply of coil would disconnect and the piston would stop. |  |  |  |  |  |
|  | Press the second push button to complete backward stroke of piston, this will enable the supply the supply would be disconnected. |  |  |  |  |  |
|  | Change 5/2 DCV and complete the opposite circuit. |  |  |  |  |  |
|  | Observe this changed circuit and note down all the observations. |  |  |  |  |  |
|  | Complete the hydraulic circuit as per drawing. |  |  |  |  |  |
|  | Turn on the motor power supply and ensure proper working of all devises and equipment. |  |  |  |  |  |
|  | Use flow control valve to ensure fluid flow. |  |  |  |  |  |
|  | Observe the movement of piston rod. |  |  |  |  |  |
|  | When the circuit is burnt/completed the turn off the power supply. |  |  |  |  |  |
|  | Observe that due to accumulator the double acting cylinder completes its stroke and doesn’t stop immediately after the burning of circuit. |  |  |  |  |  |
|  | Prepare the hydraulic circuit as per drawing |  |  |  |  |  |
|  | Install pressure gauges on the points where pressure is to be noted |  |  |  |  |  |
|  | Turn on the hydraulic circuit |  |  |  |  |  |
|  | Ensure fluid flow through the whole circuit |  |  |  |  |  |
|  | Turn on the motor power supply and ensure proper working of all devises and equipment. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Certificate Level - 5 in Mechatronics Technology National Certificate Level - 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | Describe mechanical interlocking function |  |  |  |
|  | Describe the function of hydraulic circuit |  |  |  |
|  | Describe double acting hydraulic cylinder |  |  |  |
|  | Describe 3/2 push buttons |  |  |  |
|  | Describe 5/2 double acting side pilot operated DCV |  |  |  |
|  | Describe function of connecting tubes |  |  |  |
|  | Describe function of limit switches |  |  |  |
|  | Describe the role of accumulator |  |  |  |
|  | Describe mechanism of energy storage in accumulator |  |  |  |
|  | Describe working of hydraulic accumulator |  |  |  |
|  | Describe fluid port |  |  |  |
|  | Describe construction of hydraulic actuator |  |  |  |
|  | Describe function of actuators |  |  |  |
|  | Describe coupling |  |  |  |
|  | Describe proximity switch or limit switch |  |  |  |
| Feedback to the Candidate |  |  |  |
| Candidate’s Signature________________________ Assessor’s Signature _________________________ |  |  |  |