Instruction Sheet for the Candidate

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Construct circuit for Double Acting Hydraulic Cylinder |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number______________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): CU-1: Actuate double acting hydraulic cylinder by switch and draw a circuit diagram CU-2: Actuate double acting hydraulic cylinder by oneway throttle valve |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Complete all connections of hydraulic circuit as per drawing - Turn on the push button switch to start the supply of fluid with which double acting cylinder will start moving - Move the piston to initiate the supply from forward stroke - Turn on the second push button switch after the piston reaches top dead center. This will start supply in the opposite direction - Observe the functions from control valve by varying its speed after the completion of supply in the circuit - Note all observations. - Prepare the hydraulic circuit as per drawing - Turn on the circuit power supply and ensure that all accessories are working properly - Ensure throttle valve on return line of the circuit works properly. - Open the valve and ensure extra supply of oil to observe quick return of cylinder. - Observe cylinder speed at various positions of the valve. - Note all the observations |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Certificate Level - 5 in Mechatronics Technology National Certificate Level - 5 in Mechatronics Technology |
| Competency Standards | Construct circuit for Double Acting Hydraulic Cylinder |
| Assessment Task | - Actuate double acting hydraulic cylinder by switch and draw a circuit diagram - Actuate double acting hydraulic cylinder by oneway throttle valve |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Complete all connections of hydraulic circuit as per drawing |  |  |
| - Turn on the push button switch to start the supply of fluid with which double acting cylinder will start moving |  |  |
| - Move the piston to initiate the supply from forward stroke |  |  |
| - Turn on the second push button switch after the piston reaches top dead center. This will start supply in the opposite direction |  |  |
| - Observe the functions from control valve by varying its speed after the completion of supply in the circuit |  |  |
| - Note all observations. |  |  |
| - Prepare the hydraulic circuit as per drawing |  |  |
| - Turn on the circuit power supply and ensure that all accessories are working properly |  |  |
| - Ensure throttle valve on return line of the circuit works properly. |  |  |
| - Open the valve and ensure extra supply of oil to observe quick return of cylinder. |  |  |
| - Observe cylinder speed at various positions of the valve. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Construct circuit for Double Acting Hydraulic Cylinder |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________Signature: __________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1: Actuate double acting hydraulic cylinder by switch and draw a circuit diagram 2: Actuate double acting hydraulic cylinder by oneway throttle valve |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Completed all connections of hydraulic circuit as per the drawing |  |  |  |  |  |
|  | Turned on the push button switch to start the fluid supply for double acting cylinder, which was started moving. |  |  |  |  |  |
|  | Moved the piston to initiate the supply from forward stroke |  |  |  |  |  |
|  | Turned on the second push button switch after the piston reached top dead center. This action started supply in the opposite direction. |  |  |  |  |  |
|  | Observed the functions from control valve by varying its speed after the completion of supply in the circuit. |  |  |  |  |  |
|  | Noted all observations. |  |  |  |  |  |
|  | Prepared the hydraulic circuit as per drawing |  |  |  |  |  |
|  | Turned on the circuit power supply and ensure that all accessories are properly. |  |  |  |  |  |
|  | Ensured throttle valve worked properlyon return line of the circuit. |  |  |  |  |  |
|  | Opened the valve and ensure extra supply of oil returnedto cylinderquickly. |  |  |  |  |  |
|  | Observed cylinder speed at the valvevarious positions. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Construct circuit for Double Acting Hydraulic Cylinder |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe types of hydraulic cylinder |  |  |
|  | Explain Double acting hydraulic circuit |  |  |
|  | Describe DCV (direction control valve) |  |  |
|  | Describe function of push button |  |  |
|  | Explain throttle valve function |  |  |
|  | Describe one way throttle valve |  |  |
|  | Explain role of cylinder speed |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________