Instruction Sheet for the Candidate

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Operate Flow Regulating Device |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number______________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): - Butterfly Valve - Gate valve - Diaphragm Valve - Non Return Valve - Pressure Relief Valve |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Identify various parts of the ball valve including Handle, shaft, Ball, Seats and valve body - Observe a ball with a hole drilled through the center swivel mounted within the valve body - Ensure the hole in the ball is orientated in the same direction as the pipe to allow flow - Observe the hole in the ball is oriented away from the direction of the pipe, the flowrate will be restricted and finally cut off completely when the hole is oriented at 90 degrees to the pipe direction - Note that the hole in the ball is a lesser diameter than the nominal bore of the pipe - Identify the gear box, handle, butterfly and the body of the butterfly valve - Observe a circular disc in the middle of fluid pipe. - Ensure the butterfly is oriented in the same direction as the pipe to allow full flow. - Oriented the butterfly away from the direction of the pipe, the flowrate will be restricted by the increased area of obstruction to the fluid - Keep the butterfly orienting away from the fluid until 90 degrees to completely restrict the flow - Identify gate and body of the gate valve - Identify the gate in the valve - Identify the shape of the gate with respect to the pipe diameter - Identify the direction of movement of the gate - Ensure the complete upward motion of the gate for full flow of the fluid - Ensure compete downward motion of the gate until the opposite end for complete restriction to the fluid flow. - Identify bonnet, diaphragm/ flexible sheet, seat and main body of the valve - Identify whether the valve is manual or pneumatic actuated - observe the fluid flow rates from the pipe as the diaphragm is pushed towards the seat for complete obstruction to the flow - Identify the following parts in a non-return/ check valve: body, cover plate, disc, seats, hinge, hinge pin, nut, cotter pin, stud bolt, gasket, washer - observe the fluid flow as the disk is released - observe the disk being fixed as the flow direction is reversed - Identify the following parts in pressure relief valve: set pressure adjusting screw, spring, disk holder, seat disk, nozzle, bonnet, bonnet plug and body - identify various types of pressure relief valve - observe system pressure and spring pressure - observe the valve opening as the system pressure increases then spring pressure |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| Competency Standards | Operate Flow Regulating Device |
| Assessment Task |  |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify various parts of the ball valve including Handle, shaft, Ball, Seats and valve body |  |  |
| - Observe a ball with a hole drilled through the center swivel mounted within the valve body |  |  |
| - Ensure the hole in the ball is orientated in the same direction as the pipe to allow flow |  |  |
| - Observe the hole in the ball is oriented away from the direction of the pipe, the flowrate will be restricted and finally cut off completely when the hole is oriented at 90 degrees to the pipe direction |  |  |
| - Note that the hole in the ball is a lesser diameter than the nominal bore of the pipe |  |  |
| - Identify the gear box, handle, butterfly and the body of the butterfly valve |  |  |
| - Observe a circular disc in the middle of fluid pipe. |  |  |
| - Ensure the butterfly is oriented in the same direction as the pipe to allow full flow. |  |  |
| - Oriented the butterfly away from the direction of the pipe, the flowrate will be restricted by the increased area of obstruction to the fluid |  |  |
| - Keep the butterfly orienting away from the fluid until 90 degrees to completely restrict the flow |  |  |
| - Identify gate and body of the gate valve |  |  |
| - Identify the gate in the valve |  |  |
| - Identify the shape of the gate with respect to the pipe diameter |  |  |
| - Identify the direction of movement of the gate |  |  |
| - Ensure the complete upward motion of the gate for full flow of the fluid |  |  |
| - Ensure compete downward motion of the gate until the opposite end for complete restriction to the fluid flow. |  |  |
| - Identify bonnet, diaphragm/ flexible sheet, seat and main body of the valve |  |  |
| - Identify whether the valve is manual or pneumatic actuated |  |  |
| - observe the fluid flow rates from the pipe as the diaphragm is pushed towards the seat for complete obstruction to the flow |  |  |
| - Identify the following parts in a non-return/ check valve: body, cover plate, disc, seats, hinge, hinge pin, nut, cotter pin, stud bolt, gasket, washer |  |  |
| - observe the fluid flow as the disk is released |  |  |
| - observe the disk being fixed as the flow direction is reversed |  |  |
| - Identify the following parts in pressure relief valve: set pressure adjusting screw, spring, disk holder, seat disk, nozzle, bonnet, bonnet plug and body |  |  |
| - identify various types of pressure relief valve |  |  |
| - observe system pressure and spring pressure |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Operate Flow Regulating Device |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________Signature: __________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identify various parts of the ball valve including Handle, shaft, Ball, Seats and valve body |  |  |  |  |  |
|  | Observe a ball with a hole drilled through the center swivel mounted within the valve body |  |  |  |  |  |
|  | Ensure the hole in the ball is orientated in the same direction as the pipe to allow flow |  |  |  |  |  |
|  | Observe the hole in the ball is oriented away from the direction of the pipe, the flowrate will be restricted and finally cut off completely when the hole is oriented at 90 degrees to the pipe direction |  |  |  |  |  |
|  | Note that the hole in the ball is a lesser diameter than the nominal bore of the pipe |  |  |  |  |  |
|  | Identify the gear box, handle, butterfly and the body of the butterfly valve |  |  |  |  |  |
|  | Observe a circular disc in the middle of fluid pipe. |  |  |  |  |  |
|  | Ensure the butterfly is oriented in the same direction as the pipe to allow full flow. |  |  |  |  |  |
|  | Oriented the butterfly away from the direction of the pipe, the flowrate will be restricted by the increased area of obstruction to the fluid |  |  |  |  |  |
|  | Keep the butterfly orienting away from the fluid until 90 degrees to completely restrict the flow |  |  |  |  |  |
|  | Identify gate and body of the gate valve |  |  |  |  |  |
|  | Identify the gate in the valve |  |  |  |  |  |
|  | Identify the shape of the gate with respect to the pipe diameter |  |  |  |  |  |
|  | Identify the direction of movement of the gate |  |  |  |  |  |
|  | Ensure the complete upward motion of the gate for full flow of the fluid |  |  |  |  |  |
|  | Ensure compete downward motion of the gate until the opposite end for complete restriction to the fluid flow. |  |  |  |  |  |
|  | Identify bonnet, diaphragm/ flexible sheet, seat and main body of the valve |  |  |  |  |  |
|  | Identify whether the valve is manual or pneumatic actuated |  |  |  |  |  |
|  | observe the fluid flow rates from the pipe as the diaphragm is pushed towards the seat for complete obstruction to the flow |  |  |  |  |  |
|  | Identify the following parts in a non-return/ check valve: body, cover plate, disc, seats, hinge, hinge pin, nut, cotter pin, stud bolt, gasket, washer |  |  |  |  |  |
|  | observe the fluid flow as the disk is released |  |  |  |  |  |
|  | observe the disk being fixed as the flow direction is reversed |  |  |  |  |  |
|  | Identify the following parts in pressure relief valve: set pressure adjusting screw, spring, disk holder, seat disk, nozzle, bonnet, bonnet plug and body |  |  |  |  |  |
|  | identify various types of pressure relief valve |  |  |  |  |  |
|  | observe system pressure and spring pressure |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Operate Flow Regulating Device |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define a valve |  |  |
|  | Identify applications of Ball Valve |  |  |
|  | Explain the safety precautions of ball valve |  |  |
|  | explain the terminology of ‘butterfly valve’ |  |  |
|  | identify its industrial applications |  |  |
|  | Explain the safety precautions of Butterfly Valve |  |  |
|  | describe Gate/ Knife gate valve |  |  |
|  | explain the isolation valves |  |  |
|  | explain the precaution for knife gate valve |  |  |
|  | Explain the safety precautions of Gate valve |  |  |
|  | Compare diaphragm and gate valve |  |  |
|  | Explain the industrial applications of diaphragm valve |  |  |
|  | Explain the safety precautions of Diaphragm Valve |  |  |
|  | Mention and explain various types of non-return valve |  |  |
|  | Explain unidirectional valve working mechanism |  |  |
|  | Identify various industrial applications of Non-return valve |  |  |
|  | Explain the safety precautions of Non Return Valve |  |  |
|  | Explain the safety aspects of pressure relief valve |  |  |
|  | Describe industrial applications of Pressure relief valve |  |  |
|  | Explain the safety precautions of Pressure Relief Valve |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________