Instruction Sheet for the Candidate

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Operate Pressure Measuring Instruments |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number______________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): CU-1: U Tube Manometer CU-2: Inclined manometer Cu-3: Micro Manometer CU-4: Pressure measurement in The Mercury Barometer CU-5: Borden Tube CU-6: Piezo Metric Tube |
| Time: 4 hours | During a practical assessment, under observation by an assessor, you are required to - Set up the equipment. Connect one leg of manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. - . Observe the difference in head between the two legs of the manometer and record. - Compute the pressure difference between the pipe inlet and outlet. - Repeat the experiment for various flow rates of fluid through the test pipe. - Set up the equipment. Connect one leg of inclined manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. - Observe the difference in head between the two legs of the manometer and record. - Compute the pressure difference between the pipe inlet and outlet. - Repeat the experiment for various flow rates of fluid through the test pipe. - Observe the enlarged ends of both tubes of the manometer - Adjust the instrument initially as p1 = p2. - Note the reading of the micrometer - Apply the unknown pressure difference - Observe the meniscus to move off the hairline, which can be restored to the initial position by raising or lowering the well (mercury sump). - Note the difference in the initial and final micrometer readings - Note the height of the mercury column and hence the pressure. - Take a reservoir and fill with any fluid - Put a glass tube in the reservoir in inverted state - Make the top portion of glass tube as air tight while the internal surface as complete vacuum - When the glass tube is dipped in the tank, an improper balanced pressure is created - Observe water from the tank rises in the tube - Note that the water would rise to particular height/ limit - Note that point where the water would stop lifting up - This pressure noted directly from the scale of the tube would give us the exact atmospheric/ air pressure - Observe the device consisting of a metallic tube of elliptical section closed at one end A - Note the other end B being fitted to the gauge point where the pressure is to be measured. - Enter the fluid in the tube - Observe the tube tending to straighten. - Observe a pinion-sector arrangement pivoted with the pointer needle and already calibrated markings - Note the small elastic deformation of the tube is communicated to a pointer over a graduated dial. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| Competency Standards | Operate Pressure Measuring Instruments |
| Assessment Task |  |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Set up the equipment. Connect one leg of manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. |  |  |
| - . Observe the difference in head between the two legs of the manometer and record. |  |  |
| - Compute the pressure difference between the pipe inlet and outlet. |  |  |
| - Repeat the experiment for various flow rates of fluid through the test pipe. |  |  |
| - Set up the equipment. Connect one leg of inclined manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. |  |  |
| - Observe the difference in head between the two legs of the manometer and record. |  |  |
| - Compute the pressure difference between the pipe inlet and outlet. |  |  |
| - Repeat the experiment for various flow rates of fluid through the test pipe. |  |  |
| - Observe the enlarged ends of both tubes of the manometer |  |  |
| - Adjust the instrument initially as p1 = p2. |  |  |
| - Note the reading of the micrometer |  |  |
| - Apply the unknown pressure difference |  |  |
| - Observe the meniscus to move off the hairline, which can be restored to the initial position by raising or lowering the well (mercury sump). |  |  |
| - Note the difference in the initial and final micrometer readings |  |  |
| - Note the height of the mercury column and hence the pressure. |  |  |
| - Take a reservoir and fill with any fluid |  |  |
| - Put a glass tube in the reservoir in inverted state |  |  |
| - Make the top portion of glass tube as air tight while the internal surface as complete vacuum |  |  |
| - When the glass tube is dipped in the tank, an improper balanced pressure is created |  |  |
| - Observe water from the tank rises in the tube |  |  |
| - Note that the water would rise to particular height/ limit |  |  |
| - Note that point where the water would stop lifting up |  |  |
| - This pressure noted directly from the scale of the tube would give us the exact atmospheric/ air pressure |  |  |
| - Observe the device consisting of a metallic tube of elliptical section closed at one end A |  |  |
| - Note the other end B being fitted to the gauge point where the pressure is to be measured. |  |  |
| - Enter the fluid in the tube |  |  |
| - Observe the tube tending to straighten. |  |  |
| - Observe a pinion-sector arrangement pivoted with the pointer needle and already calibrated markings |  |  |
| - Note the small elastic deformation of the tube is communicated to a pointer over a graduated dial. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Operate Pressure Measuring Instruments |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________Signature: __________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Set up the equipment. Connect one leg of manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. |  |  |  |  |  |
|  | . Observe the difference in head between the two legs of the manometer and record. |  |  |  |  |  |
|  | Compute the pressure difference between the pipe inlet and outlet. |  |  |  |  |  |
|  | Repeat the experiment for various flow rates of fluid through the test pipe. |  |  |  |  |  |
|  | Set up the equipment. Connect one leg of inclined manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. |  |  |  |  |  |
|  | Observe the difference in head between the two legs of the manometer and record. |  |  |  |  |  |
|  | Compute the pressure difference between the pipe inlet and outlet. |  |  |  |  |  |
|  | Repeat the experiment for various flow rates of fluid through the test pipe. |  |  |  |  |  |
|  | Observe the enlarged ends of both tubes of the manometer |  |  |  |  |  |
|  | Adjust the instrument initially as p1 = p2. |  |  |  |  |  |
|  | Note the reading of the micrometer |  |  |  |  |  |
|  | Apply the unknown pressure difference |  |  |  |  |  |
|  | Observe the meniscus to move off the hairline, which can be restored to the initial position by raising or lowering the well (mercury sump). |  |  |  |  |  |
|  | Note the difference in the initial and final micrometer readings |  |  |  |  |  |
|  | Note the height of the mercury column and hence the pressure. |  |  |  |  |  |
|  | Take a reservoir and fill with any fluid |  |  |  |  |  |
|  | Put a glass tube in the reservoir in inverted state |  |  |  |  |  |
|  | Make the top portion of glass tube as air tight while the internal surface as complete vacuum |  |  |  |  |  |
|  | When the glass tube is dipped in the tank, an improper balanced pressure is created |  |  |  |  |  |
|  | Observe water from the tank rises in the tube |  |  |  |  |  |
|  | Note that the water would rise to particular height/ limit |  |  |  |  |  |
|  | Note that point where the water would stop lifting up |  |  |  |  |  |
|  | This pressure noted directly from the scale of the tube would give us the exact atmospheric/ air pressure |  |  |  |  |  |
|  | Observe the device consisting of a metallic tube of elliptical section closed at one end A |  |  |  |  |  |
|  | Note the other end B being fitted to the gauge point where the pressure is to be measured. |  |  |  |  |  |
|  | Enter the fluid in the tube |  |  |  |  |  |
|  | Observe the tube tending to straighten. |  |  |  |  |  |
|  | Observe a pinion-sector arrangement pivoted with the pointer needle and already calibrated markings |  |  |  |  |  |
|  | Note the small elastic deformation of the tube is communicated to a pointer over a graduated dial. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Operate Pressure Measuring Instruments |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Differentiate between absolute pressure and gauge pressure |  |  |
|  | Differentiate between u tube inclined and differential manometer |  |  |
|  | Pressures as low as 0.025mm water column can be measured. |  |  |
|  | What is a barometer |  |  |
|  | What is the basic function of a barometer |  |  |
|  | Explain the selection process of a pressure gauge |  |  |
|  | Write the types of Bourdon tubes |  |  |
|  | Explain the purpose of different Bourdon tubes |  |  |
|  | Explain the formula for calculating a static head |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________