Instruction Sheet for the Candidate

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number______________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): CU-1: Set-up pressure device on a double acting cylinder using pressure reducing valve CU-2: Hold specific load using double acting cylinder |
| Time: 1 hour | During a practical assessment, under observation by an assessor, you are required to - Complete the hydraulic circuit as par drawing given - Turn on the circuit supply and check if all devices are working properly - Ensure Fluid starts flowing and the initially adjusted pressure is passed on to the circuit while passing through the relief valve. - Observe and note the pressure reading at various pressure gauges. - Change direction with the help of DCV and ensure control of double acting cylinder according to this direction. - Re-observe and re-note the readings on all the pressure gauges. - Adjust pressure according to desired/required load. - Ensure fluid flow from port A. This would uplift the load. - Now change the position of DCV and ensure fluid entrance from port B. - Observe the downward motion of loaded ram. - Take the pressure reading and observe the cylinder return |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| Competency Standards | Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder |
| Assessment Task | - Set-up pressure device on a double acting cylinder using pressure reducing valve - Hold specific load using double acting cylinder |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Complete the hydraulic circuit as drawing given |  |  |
| - Turn on the circuit supply and check if all devices are working properly |  |  |
| - Ensure Fluid starts flowing and the initially adjusted pressure is passed on to the circuit while passing through the relief valve. |  |  |
| - Observe and note the pressure reading at various pressure gauges. |  |  |
| - Change direction with the help of DCV and ensure control of double acting cylinder according to this direction. |  |  |
| - Re-observe and re-note the readings on all the pressure gauges. |  |  |
| - Adjust pressure according to desired/required load. |  |  |
| - Ensure fluid flow from port A. This would uplift the load. |  |  |
| - Now change the position of DCV and ensure fluid entrance from port B. |  |  |
| - Observe the downward motion of loaded ram. |  |  |
| - Take the pressure reading and observe the cylinder return |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________Signature: __________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Set-up pressure device on a double acting cylinder using pressure reducing valve - Hold specific load using double acting cylinder |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Completed the hydraulic circuit as par drawing given |  |  |  |  |  |
|  | Turned on the circuit supply and checkedfor all devices were working properly |  |  |  |  |  |
|  | Ensured Fluid starts flowing and the initially adjusted pressure is passed on to the circuit while passing through the relief valve. |  |  |  |  |  |
|  | Observed and noted the pressure reading at various pressure gauges. |  |  |  |  |  |
|  | Changed direction with the help of DCV and ensured control of double acting cylinder according to desired direction. |  |  |  |  |  |
|  | Re-observe and noted that the readings on all the pressure gauges were accordingly. |  |  |  |  |  |
|  | Observed and notedthat the pressure reading at various pressure gauges. |  |  |  |  |  |
|  | Changed direction with the help of DCV and ensured controlled of double acting cylinder according to this direction. |  |  |  |  |  |
|  | Adjust pressure according to desired/required load. |  |  |  |  |  |
|  | Ensured fluid flow from port A. This was uplifted the load. |  |  |  |  |  |
|  | Now changed the position of DCV and ensuredthat fluid entrance from port B. |  |  |  |  |  |
|  | Observed the downward motion of loaded ram. |  |  |  |  |  |
|  | Taken the pressure reading and observed the cylinder returned action. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain pressure reducing valve |  |  |
|  | Describe the purpose of double acting cylinder as a pressure device |  |  |
|  | Explainthe applications of double acting cylinder |  |  |
|  | Describe function of pilot operated check valve |  |  |
|  | What is DCV, describe two important points |  |  |
|  | Describe the role of RAM |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________