Instruction Sheet for the Candidate

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Verify Bernoulli’s Equation |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number______________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): - Verification of Velocity Head, Pressure Head and Datum Head - Observe the performance of friction pipe apparatus - Examine loss of head due to sudden enlargement, Contraction and entrance in a pipe |
| Time: 2 | During a practical assessment, under observation by an assessor, you are required to - Open inlet valve of hydraulic bench supply tank to allow the water to flow out of the tank. - Adjust the valves to obtain continuous flow at inlet and outlet valve. - Ensure continuous pressure head in the piezometer tube and note the reading. - Determine the water quantity collected in the measuring tank during the time noted by stop watch. - Determine the cross-sectional area of piezometer tube. - Calculate the velocity head and pressure head. - Change input and output supply and note the readings. - Select a pipe of suitable size as per available equipment for the experiment. - Start the flow of water in the pipe with the help of hydraulic bench. - Observe the change in mercury level of U-tube manometer due to water flow in the pipe. - Note h1 and h2. - Note the time from stop watch to fill the water in the tank up to the height of 5 cm. - Change the flow and repeat the procedure. - Calculate coefficient of discharge. - Note the length and diameter of the pipe present between hydraulic bench and pressure tank. - Keep the pressure tank, end of the, pipe open and the other end closed. - Turn on the power supply of the hydraulic bench. - Start the flow water in the circuit. - Adjust the flow up to maximum level. - Determine the total discharge in the circuit with the help of left and right limbs of U-tube manometer. - Keep the pressure tank, end of the, pipe close and the other end opened. Repeat the process |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| Competency Standards | Verify Bernoulli’s Equation |
| Assessment Task | - Verification of Velocity Head, Pressure Head and Datum Head - Observe the performance of friction pipe apparatus - Examine loss of head due to sudden enlargement, Contraction and entrance in a pipe |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Open inlet valve of hydraulic bench supply tank to allow the water to flow out of the tank. |  |  |
| - Adjust the valves to obtain continuous flow at inlet and outlet valve. |  |  |
| - Ensure continuous pressure head in the piezometer tube and note the reading. |  |  |
| - Determine the water quantity collected in the measuring tank during the time noted by stop watch. |  |  |
| - Determine the cross-sectional area of piezometer tube. |  |  |
| - Calculate the velocity head and pressure head. |  |  |
| - Change input and output supply and note the readings. |  |  |
| - Select a pipe of suitable size as per available equipment for the experiment. |  |  |
| - Start the flow of water in the pipe with the help of hydraulic bench. |  |  |
| - Observe the change in mercury level of U-tube manometer due to water flow in the pipe. |  |  |
| - Note h1 and h2. |  |  |
| - Note the time from stop watch to fill the water in the tank up to the height of 5 cm. |  |  |
| - Change the flow and repeat the procedure. |  |  |
| - Calculate coefficient of discharge. |  |  |
| - Note the length and diameter of the pipe present between hydraulic bench and pressure tank. |  |  |
| - Keep the pressure tank, end of the, pipe open and the other end closed. |  |  |
| - Turn on the power supply of the hydraulic bench. |  |  |
| - Start the flow water in the circuit. |  |  |
| - Adjust the flow up to maximum level. |  |  |
| - Determine the total discharge in the circuit with the help of left and right limbs of U-tube manometer. |  |  |
| - Keep the pressure tank, end of the, pipe close and the other end opened. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocational Diploma Level 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Verify Bernoulli’s Equation |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________Signature: __________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Verification of Velocity Head, Pressure Head and Datum Head - Observe the performance of friction pipe apparatus - Examine loss of head due to sudden enlargement, Contraction and entrance in a pipe |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Open inlet valve of hydraulic bench supply tank to allow the water to flow out of the tank. |  |  |  |  |  |
|  | Adjust the valves to obtain continuous flow at inlet and outlet valve. |  |  |  |  |  |
|  | Ensure continuous pressure head in the piezometer tube and note the reading. |  |  |  |  |  |
|  | Determine the water quantity collected in the measuring tank during the time noted by stop watch. |  |  |  |  |  |
|  | Determine the cross-sectional area of piezometer tube. |  |  |  |  |  |
|  | Calculate the velocity head and pressure head. |  |  |  |  |  |
|  | Change input and output supply and note the readings. |  |  |  |  |  |
|  | Select a pipe of suitable size as per available equipment for the experiment. |  |  |  |  |  |
|  | Start the flow of water in the pipe with the help of hydraulic bench. |  |  |  |  |  |
|  | Observe the change in mercury level of U-tube manometer due to water flow in the pipe.Note h1 and h2 |  |  |  |  |  |
|  | Note the time from stop watch to fill the water in the tank up to the height of 5 cm. |  |  |  |  |  |
|  | Change the flow and repeat the procedure. |  |  |  |  |  |
|  | Calculate coefficient of discharge. |  |  |  |  |  |
|  | Note the length and diameter of the pipe present between hydraulic bench and pressure tank. |  |  |  |  |  |
|  | Keep the pressure tank, end of the, pipe open and the other end closed. |  |  |  |  |  |
|  | Turn on the power supply of the hydraulic bench. |  |  |  |  |  |
|  | Start the flow water in the circuit. |  |  |  |  |  |
|  | Adjust the flow up to maximum level. |  |  |  |  |  |
|  | Determine the total discharge in the circuit with the help of left and right limbs of U-tube manometer. |  |  |  |  |  |
|  | Keep the pressure tank, end of the, pipe close and the other end opened. |  |  |  |  |  |
|  | Repeat the process. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Certificate Level - 5 in Mechatronics Technology National Certificate Level - 5 in Mechatronics Technology |
| --- | --- |
| Competency Standard(s) | Verify Bernoulli’s Equation |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe Bernoulli’s theorem |  |  |
|  | Describe piezometer tube |  |  |
|  | Define velocity head |  |  |
|  | Define pressure head |  |  |
|  | Define Datum head |  |  |
|  | Describe friction pipe apparatus |  |  |
|  | Describe effects of friction on fluid flow |  |  |
|  | Describe Differential type U-tube Manometer |  |  |
|  | Define coefficient of discharge |  |  |
|  | Explain Darey’s friction factor |  |  |
|  | Describe loss of head |  |  |
|  | Describe co-efficient of loss |  |  |
|  | Describe G.I. Pipes |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________