National Vocational Certificate Level 4 in Mechatronics Technology

(Senior Mechatronics Technician)

Competency Standards

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

Table of Contents

1.	Introduction	7

2.	Purpose of the Qualification	8

3.	Core competencies of the Qualification	8

4.	Common Courses with respective levels	19

5.	Mapping of the Qualification	20

6.	Date of Validation	21

7.	Date of Review	21

8.	Codes of Qualifications	21

9.	Members of Qualification Development Committee	21

10.	Qualification Validation Committee	22

11.	Entry Requirements	22

12.	Categorization and Levelling of the Competency Standards	22

13.	Detail of Qualifications and its Competency Standards	33

Health and Safety	33

0714E&A1.	Maintain Health and Safety	33

0714E&A2.	Perform safety practices at work place	36

0714E&A3.	Develop Professionalism	39

Basic Machining	42

0714E&A4.	Perform Metal / Bench Work	42

0714E&A5	Perform general machining	45

0714E&A6	Perform cutting on Metal Circular/Power Heck Saw	47

0714E&A7	Perform tool sharpening	49

0714E&A8	Perform Basic Lathe Machine Operations	51

0714E&A9	Perform Drilling Machine Operations	56

0714E&A10	Perform Shaper, Planar and Slotter Machining Operations	58

0714E&A11	Perform Milling Operations	61

Electrical Essentials & Networks	66

0714E&A12	Operate Measuring Instruments	66

0714E&A13	Verify Ohm’s & Kirchhoff’s Laws by Implementing Series/Parallel Circuits.	68

0714E&A14	Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence	70

0714E&A15	Construct Electromagnet to See Various Effects & Verify Faradays Law.	73

0714E&A16      Verify Law of Combination of Capacitor & Determine Break Down Voltage of Capacitor.	76

0714E&A17	Maintain Tools & Equipment	78

0714E&A18	Make Cable/Wire Joints	80

0714E&A19	Prepare and Install Distribution Boards.	82

0714E&A20	Carryout Basic Electrical Installations	84

0714E&A21	Install Single Phase Electrical Wiring	86

0714E&A22	Perform Testing of Electrical Wiring	90

0714E&A23	Operate Single Phase Transformer	94

0714E&A24	Perform Tests on Single Phase Transformer	97

Digital Skills	100

0714E&A25	Install computer operating systems and hardware	100

0714E&A26	Operate digital media technology	102

0714E&A27	Perform computer operations	104

0714E&A28	Use computer applications	106

0714E&A29	Create user documentation	109

0714E&A30	Create technical documentation	111

0714E&A31	Create basic databases	113

0714E&A32	Use social media tools for collaboration and engagement	115

0714E&A33	E-Commerce- SEO (Search Engine Optimization)	117

0714E&A34	E-Commerce- SCM (Supply Chain Management)	118

0714E&A35	E-Commerce- Social Media Marketing	119

0714E&A36	Use digital devices	120

0714E&A37	Operate word-processing applications	122

0714E&A38	Operate spreadsheet applications	124

0714E&A39	Operate presentation packages	126

0714E&A40	Perform writing and editing tasks	128

Metrology	129

0714E&A41	Take measurements with graduated tools	129

0714E&A42	Take measurements with combination set	132

0714E&A43	Perform Levelling	134

0714E&A44	Take measurements with gauges	136

0714E&A45	Perform measurements through Micrometer	137

0714E&A46	Measure dimensions with Vernier tools	139

0714E&A47	Measure angles with angle measuring Instruments.	141

0714E&A48	Measure Threads and Gears	143

Basic Engineering Drawings	143

0714E&A49	Perform Basic Manual Drawing	143

0714E&A50	Construct different Engineering Curves.	146

0714E&A51	Construct multi-view drawings	151

0714E&A52	Develop 2D CAD drawings	154

0714E&A53	Develop 3D CAD drawing	156

Welding	158

0714E&A54	Interpret Part and Assembly Drawing	158

0714E&A55	Evaluate the Use of Hand and Power Tools	160

0714E&A56	Perform Pre-Welding Operations	162

0714E&A57	Perform Oxy Acetylene Welding	165

0714E&A58	Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting)	167

0714E&A59	Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F)	170

0714E&A60	Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	172

0714E&A61	Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	176

0714E&A62	Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), H (2F, 2G), V (3F, 3G) and O.H (4F, 4G) Positions	180

0714E&A63	Flux Cored Arc Welding (FCAW)	184

0714E&A64	Perform Soldering and Brazing Operations	187

Microprocessor and Microcontroller	189

0714E&A65.     Design Microprocessor applications	189

0714E&A66	Identify Microcontroller Types and its Architecture	191

0714E&A67	Interface Microcontroller with system	194

0714E&A68	Carryout various Microcontroller Applications.	197

0714E&A69	Apply Microcontroller to build Control System.	199

Analogue Electronics	201

0714E&A70	Identify Basic Electronics Components	201

0714E&A71	Design a Rectifier Using Diode	203

0714E&A72	Carry Out Diode Application	205

0714E&A73	Implement Bipolar Junction Transistor (BJT) in Different Applications	207

0714E&A74	Implement Field Effect Transistors (FET) In Different Applications	209

0714E&A75	Implement (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in various application	212

0714E&A76	Design Operation Amplifier	214

Soft Skills	216

0714E&A77.	Develop workplace policy and procedures for sustainability	216

0714E&A78.	Manage meetings	219

0714E&A79.	Manage recruitment selection and induction processes	221

0714E&A80.	Manage personal work priorities and professional development	223

0714E&A81.	Manage workforce planning	225

0714E&A82.	Undertake project work	227

0714E&A83.	Identify and communicate trends in career development	230

0714E&A84.	Apply specialist interpersonal and counseling interview skills	232

0714E&A85.	Work safely in an office environment	235

0714E&A86.	Develop workplace documents	237

0714E&A87.	Prepare and implement negotiation	240

0714E&A88.	Maintain professionalism in the workplace	243

0714E&A89.	Organize schedules	245

Computer Programming	247

0714E&A90.	Installation of C++, IDE for Arduino and Python	247

0714E&A91.	Carryout Basic C++ Programming	249

0714E&A92.	Carryout Basic Programming using python	252

0714E&A93.	Perform basic programming using microcontrollers	256

Computer Integrated Manufacturing (CIM)	257

0714E&A94.	Perform CNC Machine Operations	257

0714E&A95.	Perform CNC Lathe Operations	261

0714E&A96.	Perform CNC Milling Operations	264

Machines and Mechanism	267

0714E&A97.	Demonstrate Simple Mechanisms	267

0714E&A98.	Perform Kinematic Analysis of Simple Mechanisms	269

0714E&A99.	Demonstrate Mechanical Power Transmission Elements.	270

0714E&A100.	Perform Kinematic Analysis of Mechanical Power Transmission Elements.	271

AC & DC Machines	272

0714E&A101.	Verify Basic Laws of Electrical Machines	272

0714E&A102.	Analyse an Alternator	275

0714E&A103.	Analyse Single Phase Motors.	278

0714E&A104.	Analyse Special Purpose Motors.	280

0714E&A105.	Analyse Three Phase Motors.	282

0714E&A106.	Analyze Dc Generators	287

0714E&A107.	Perform Tests on DC Generators	290

0714E&A108.	Analyze Dc Motors.	295

0714E&A109.	Perform Tests On DC Motors.	297

Digital Electronics	301

0714E&A110.	Verify Truth Tables of Digital Gates	301

0714E&A111.	Construct & Verify Combinational Logic Circuit	304

0714E&A112.	Construct and Verify Function of Flip Flops	307

0714E&A113.	Construct Multivibrator with 555 IC	309

0714E&A114.	Construct Shift Registers and Counters with the Help of Flip Flops	311

Sensors and Actuators	312

0714E&A115.	Test Sensors and Transducers	312

0714E&A116	Calibrate Sensors and Transducers	315

0714E&A117	Interface the Sensor & Transducers	320

0714E&A118	Test Actuators	323

0714E&A119	Interface and control Actuator with sensors	325

Programmable Logic Controllers (PLCS)	327

0714E&A120	Install PLC software and Simulator	327

0714E&A121	Interface PLC with system	330

0714E&A122	Program PLC using Ladder Logic	332

0714E&A123	Design and Test the PLC	334

0714E&A124	Carry out Industrial Automation on PLC	338

Designing of Mechanical Members	341

0714E&A125	Calculate Stresses in Machine Parts	341

0714E&A126	Calculate diameter of cylinder for hoop and longitudinal stresses	343

0714E&A127	Calculate thickness and diameter of spherical shell for circumferential stresses	344

0714E&A128	Design welded joints for transverse and parallel fillet under static and fatigue loading	345

0714E&A129	Calculate stresses due to initial tightening and external load on screws	348

0714E&A130	Design dimension of square and rectangular keys	350

0714E&A131	Design shafts subjected to twisting moment only	352

0714E&A132	Design flange coupling for specific torque	354

Industrial Process Control & Instrumentation	356

0714E&A133	Design & Implement a process control	356

0714E&A134	Install Transducer & Transmitters used in Industrial Applications	359

0714E&A135	Install the process Regulators	361

0714E&A136	Install the Variable Frequency Drive (VFD)	363

0714E&A137	Install Pneumatic & Hydraulic systems	364

Robotics	367

0714E&A138	Identify Various Parts of Robots	367

0714E&A139	Identify Various Parts of Robot Arm	369

0714E&A140	Develop Robot program	371

0714E&A141	Develop robotic arm structure	373

0714E&A142	Test the Robot	375

Mechatronic Projects	376

0714E&A143	Access Control System using RFID Reader RC522	376

0714E&A144	Finger print Sensor Based Biometric Attendance System	377

0714E&A145	Humidity and Temperature Monitoring System.	377

0714E&A146	Smart Stick for Blinds	378

0714E&A147	Perform Line Following Robot	378

0714E&A148	Perform Smart Phone Control Robot Using Bluetooth	379

0714E&A149	Perform IoT Controlled based home automation system	381

0714E&A150	Perform Smart Phone Control Door Lock System	382

0714E&A151	Perform Security System Using Motion Detection	382

0714E&A152	Perform Water-level Detection in Tank	383

0714E&A153	Perform Mini CNC 2D Plotter	384

0714E&A154	Perform Voice Controlled Robot	385

Hydraulic and Pneumatics	386

0714E&A155	Operate Hydraulic Bench and Its Functions	386

0714E&A156	Calibrate Bourdon Tube and Diaphragm Pressure Gauge	389

0714E&A157	Operate Hydraulic Press	391

0714E&A158	Verify Bernoulli’s Equation	393

0714E&A159	Analyse the performance of Pumps	395

0714E&A160	Construct circuit for Double Acting Hydraulic Cylinder	397

0714E&A161	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder	399

0714E&A162	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking	401

0714E&A163	Set Speed and Direction of Hydraulic Motor	404

0714E&A164	Operate Pressure Measuring Instruments	406

0714E&A165	Operate Flow Regulating Devices	409

Entrepreneurial Skill	412

0714E&A166	Develop Entrepreneurial Skills	412

0714E&A167	Apply project information management and communications techniques	414

0714E&A168	Apply project human resources management approaches	416

0714E&A169	Direct human resources management of a project program	418

0714E&A170	Develop a project management plan	421

0714E&A171	Maintain business resources	424

0714E&A172	Develop a sales plan	426

0714E&A173	Plan and implement business-to-business marketing	428

0714E&A174	Address customer needs	430

0714E&A175	Manage personal finances	433

0714E&A176	Solve problems which jeopardize safety and security	435

0714E&A177	Coordinate a work team	438

0714E&A178	Lead small teams	440

0714E&A179	Plan and organize work	443

0714E&A180	Develop teams and individuals	446

0714E&A181	Apply problem solving techniques in the workplace using critical thinking	448

0714E&A182	Manage human resource services	450

- Introduction

Mechatronics is an interdisciplinary field that combines elements of engineering, machining, information technology, robotics, and electronics. It synchronize these diverse fields for the purpose of designing, fabrication and field testing of the production lines and manufacturing processes in order to optimize their performance and efficiencies. In this field several mechanical systems such as hydraulic, pneumatic, camera,  electric motor and engines are interface, configure and combine in order to peform the tasks with increase efficiency and performance. Since several differenct mechanical, electrical and optical systems are inference, the continual operational maintenance, confirgation and upgradation is required which need trained main power in the field of mechantronics in order to run the industrial processes and machines with any system failure or breakdown.

Typically, a mechatronics technologist need to perform several task such as stand alone operation / testing of the mechanical and electronics systems in order to diagnose the faulty parts. These task may attribute to the checking, installation and configuration of electronics curcuits, motor drives, hydraulic / pneumatic moving system. The industry is lacking the expert technologist who has sound knowledge and command over all mechanical and electronics systems  especially when these are interface and working in the close loop environment.

Mechatronics technologists can work in several  fields, but they are  commonly found in the manufacturing and automation sector. Several industrial process and production lines can be optimize by implementing fine mechanical precision in environmental conditions that may be dangerous or inefficient  for human hands and can be perform optimally by adding mechanrtonic solutions.Mechatronics has had a measurable impact both on the speed with which new products can be assembled.

The professionals that are skilled in this area of mechantronics  are in high demand on production lines that range from projects as large as automotive vehicle manufacturing to the smallest microelectronic products. For instance, blood pressure monitoring device has pneumatic pump, a sensor to sense the blood pressure and electronics / microprocessor based circuit to measure the blood pressure. Similarly, several home appliances such as washing machines, ovens and refrigerators are now equipped with sophisticated mechatronics systems  in order to optimized their products in terms of performance and electricitical power consumption. Therfore, to repair and maintain such home appliances, the traditional technician are fail to work because of their training explicitly in the field of mechanical, electrical or electronics. Being conscious of these facts , National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechatronics Technology under National Vocational Qualifications Framework (NVQF). Followings  are the competency standards which are   developed by  the Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Mechatronics industries from public and private sector, Engineering Universities and Technical Education of the country.

- Purpose of the Qualification

The purpose of this qualification is to set the high professional standards for the Mechatronics technological sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of the skills attained through formal or informal pathways

- Improve the quality and effectiveness of the training and assessment for Mechatronics Technological sector

- Enabling / helping / facilitating the existing workforce to indulge themselves in new technologies and methods

- Core competencies of the Qualification

The Mechatronics Technology qualification of level 5 consists of the 40% of theory and 60% of the  practical. The Core competencies of the qualification are as follows:

| Competency Standards | Level |
| --- | --- |
| Health and Safety |  |
| Maintain Occupational Health and Safety | 2 |
| Adopt Safety Regulations, Labor Protection Laws, Environmental Protection Laws at Workplace | 2 |
| Total |  |
| Basic Machining |  |
| Perform Metal/Bench Work | 2 |
| Perform general machining | 2 |
| Perform cutting on Metal Circular/Power Heck Saw | 2 |
| Perform tool sharpening | 2 |
| Perform Basic Lathe Machine Operations | 3 |
| Perform Drilling Machine Operations | 2 |
| Perform Shaper, Planar and Slotter Machining Operations | 3 |
| Perform Milling Operations | 3 |
| Electrical Essentials & Networks |  |
| Operate Measuring Instruments | 3 |
| Verify Ohm’s & Kirchhoff’s Laws by Implementing Series/Parallel Circuits | 3 |
| Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence | 3 |
| Construct Electromagnet to See Various Effects & Verify Faradays Law | 3 |
| Verify Law of Combination of Capacitor & Determine Break Down Voltage of Capacitor | 3 |
| Maintain Tools & Equipment | 3 |
| Make Cable/Wire Joints | 2 |
| Prepare and Install Distribution Boards | 2 |
| Carryout Basic Electrical Installations | 2 |
| Install Single Phase Electrical Wiring | 2 |
| Perform Testing of Electrical Wiring | 2 |
| Operate Single Phase Transformer | 5 |
| Perform Tests on Single Phase Transformer | 5 |
| Digital Skills |  |
| Install Computer Operating Systems And Hardware | 3 |
| Operate Word-Processing Applications | 3 |
| Operate Spreadsheet Applications | 3 |
| Operate Presentation Packages | 3 |
| Perform Writing And Editing Tasks | 3 |
| Perform Computer Operations | 3 |
| Use Computer Applications | 3 |
| Create User Documentation | 3 |
| Create Technical Documentation | 3 |
| Create Basic Databases | 3 |
| Operate Digital Media Technology | 3 |
| Use Social Media Tools For Collaboration And Engagement | 3 |
| E-Commerce | 3 |
| Use Digital Devices | 3 |
| Metrology |  |
| Take measurements with graduated tools | 3 |
| Take measurements with combination set | 3 |
| Perform ling | 3 |
| Take measurements with gauges | 3 |
| Perform measurements through Micrometer | 3 |
| Measure dimensions with Vernier tools | 3 |
| Measure angles with angle measuring Instruments | 3 |
| Measure Threads and Gears | 3 |
| Basic Engineering Drawing |  |
| Perform Basic Manual Drawing | 3 |
| Construct different Engineering Curves | 3 |
| Construct multi-view drawings | 3 |
| Install CAD Software | 3 |
| Develop 2D CAD drawings | 3 |
| Develop 3D CAD drawings | 3 |
| Welding - I |  |
| Interpret Parts and Assembly Drawing | 2 |
| Evaluate the Use of Hand and Power Tools | 2 |
| Perform Pre-Welding Operations | 2 |
| Perform Oxy Acetylene Welding | 3 |
| Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting) | 3 |
| Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F) | 3 |
| Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 4 |
| Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 4 |
| Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 4 |
| Flux Cored Arc Welding (FCAW) | 4 |
| Perform Soldering and Brazing Operations | 3 |
| Microprocessor & Microcontrollers |  |
| Design Microprocessor applications | 4 |
| Identify Microcontroller Types and its Architecture | 4 |
| Interface Microcontroller with system | 4 |
| Carryout various Microcontroller Applications. | 4 |
| Apply Microcontroller to build Control System. | 4 |
| Total |  |
| Analogue Electronics |  |
| Identify the Semiconductor Devices | 4 |
| Construct The RF & AF Amplifiers | 4 |
| Design the Oscillators | 4 |
| Analyze operational Amplifier circuits | 4 |
| Design the Multi-Vibrators | 4 |
| Construct Ramp Generator circuits | 4 |
| Design Function Generator & Comparator circuits | 4 |
| Total |  |
| Soft Skills |  |
| Develop workplace policy and procedures for sustainability | 4 |
| Manage meetings | 4 |
| Manage recruitment selection and induction processes | 4 |
| Manage personal work priorities and professional development | 4 |
| Manage workforce planning | 4 |
| Undertake project work | 4 |
| Identify and communicate trends in career development | 4 |
| Apply specialist interpersonal and counseling interview skills | 4 |
| Work safely in an office environment | 4 |
| Develop workplace documents | 4 |
| Prepare and implement negotiation | 4 |
| Maintain professionalism in the workplace | 4 |
| Organize schedules | 4 |
| Computer Programming |  |
| Installation of C++, IDE for Arduino and Python | 4 |
| Carryout Basic C++ Programming | 4 |
| Carryout Basic Programming using python | 4 |
| Perform basic programming using microcontrollers | 4 |
| Total |  |
| Computer Integrated Manufacturing (CIM) |  |
| Perform CNC Machine Operations | 4 |
| Perform CNC Lathe Operations | 4 |
| Perform CNC Milling Operations | 4 |
| Machines and Mechanism |  |
| Demonstrate Simple Mechanisms | 4 |
| Perform Kinematic Analysis of Simple Mechanisms | 5 |
| Demonstrate Mechanical Power Transmission Elements | 4 |
| Perform Kinematic Analysis of Mechanical Power Transmission Elements | 5 |
| Total |  |
| Verify Basic Laws of Electrical Machines |  |
| Verify Basic Laws of Electrical Machines | 4 |
| Analyze an Alternator | 4 |
| Analyze Single Phase Motors | 4 |
| Analyze Special Purpose Motors | 4 |
| Analyze Three Phase Motors | 4 |
| Analyze Dc Generators | 4 |
| Perform Tests on DC Generators | 4 |
| Analyze Dc Motors. | 4 |
| Perform Tests On DC Motors. | 4 |
| Total |  |
| Digital Electronics |  |
| Verify Truth Tables of Digital Gates | 3 |
| Construct & Verify Combinational Logic Circuit | 4 |
| Construct and Verify Function of Flip Flops | 4 |
| Construct Multi vibrator with 555 IC | 4 |
| Construct Shift Registers and Counters with the Help of Flip Flops | 4 |
| Total |  |
| Sensors and Actuators |  |
| Test Sensors and Transducers | 4 |
| Calibrate Sensors and Transducers | 4 |
| Interface the Sensor & Transducers | 4 |
| Test Actuators | 4 |
| Interface and control Actuator with sensors | 4 |
| Total |  |
| Programmable Logic Controllers (PLC) (5) |  |
| Install PLC software and Simulator | 5 |
| Interface PLC with system | 5 |
| Program PLC using Ladder Logic | 5 |
| Design and Test the PLC | 5 |
| Carry out Industrial Automation on PLC | 5 |
| Design of Machine Members (5) |  |
| Calculate Stresses in Machine Parts | 5 |
| Calculator diameter of cylinder for hoop and longitudinal stresses | 5 |
| Calculate thickness and diameter of spherical shell for circumferential stresses | 5 |
| Design welded joints for transverse and parallel fillet under static and fatigue loading | 5 |
| Calculate stresses due to initial tightening and external load on screws | 5 |
| Design dimension of square and rectangular keys | 5 |
| Design shaft subjected to twisting moment only | 5 |
| Design flange coupling for specific torque | 5 |
| Total |  |
| Industrial Instrumentation (5) |  |
| Design & Implement a process control | 5 |
| Install Transducer & Transmitters in Industrial Applications | 5 |
| Install the process Regulators | 5 |
| Install the Variable Frequency Drive (VFD) | 5 |
| Install Pneumatic & Hydraulic systems | 5 |
| Total |  |
| Robotics(5) |  |
| Identify Various Parts of Robots | 3 |
| Identify Various Parts of Robot Arm | 3 |
| Develop Robot program | 5 |
| Develop robotic arm structure | 5 |
| Test the Robot | 5 |
| Total |  |
| Mechatronic Projects |  |
| Access Control System using RFID Reader RC522 | 5 |
| Finger print Sensor Based Biometric Attendance System | 5 |
| Humidity and Temperature Monitoring System. | 5 |
| Smart Stick for Blinds | 5 |
| Perform Line Following Robot | 5 |
| Perform Smart Phone Control Robot Using Bluetooth | 5 |
| Perform IoT Controlled based home automation system | 5 |
| Perform Smart Phone Control Door Lock System | 5 |
| Perform Security System Using Motion Detection | 5 |
| Perform Water- Detection in Tank | 5 |
| Perform Mini CNC 2D Plotter | 5 |
| Perform Voice Controlled Robot | 5 |
| Total |  |
| Hydraulics and Pneummatics 5 |  |
| Operate Hydraulic Bench and Its Functions | 5 |
| Calibrate Bourdon Tube and Diaphragm Pressure Gauge | 5 |
| Operate Hydraulic Press | 4 |
| Verify Bernoulli’s Equation | 5 |
| Analyse the performance of Pumps | 5 |
| Construct circuit for Double Acting Hydraulic Cylinder | 4 |
| Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder | 5 |
| Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking | 5 |
| Set Speed and Direction of Hydraulic Motor | 5 |
| Operate Pressure Measuring Instruments | 5 |
| Operate Flow Regulating Devices | 5 |
| Entrepreneurial Skill (5) |  |
| Develop Entrepreneurial Skills | 5 |
| Apply project information management and communications techniques | 5 |
| Apply project human resources management approaches | 5 |
| Direct human resources management of a project program | 5 |
| Develop a project management plan | 5 |
| Maintain business resources | 5 |
| Develop a sales plan | 5 |
| Plan and implement business-to-business marketing | 5 |
| Address customer needs | 5 |
| Manage personal finances | 5 |
| Solve problems which jeopardize safety and security | 5 |
| Coordinate a work team | 5 |
| Lead small teams | 5 |
| Plan and organize work | 5 |
| Develop teams and individuals | 5 |
| Apply problem solving techniques in the workplace using critical thinking | 5 |
| Manage human resource services | 5 |
| Total |  |

- Common Courses with respective levels

LEVEL 3

LEVEL 2

LEVEL 4

LEVEL 5

- Mapping of the Qualification

Mechatronics projects

Sensors and Actuators……

PLC Supervisor

Applications of Automation in daily life……..

Mapping of the Qualifications

Analog/Digital Electronics

PLC Supervisor

Safety Supervisor

Soft Skills

Digital Skills

Entrepreneurship

Metrology

Analog/Digital/Industrial Electronics

Robotics…..

Robotics System

Robotics Programming

Robotic Applications…………

Basic Electrical Engineering/ Wiring

Analog/Digital/Industrial Electronics

- Date of Validation

The level 5 of National DAE qualification on Mechanical Technology has been validated by the Qualifications Validation Committee (QVC) members on 22-24 August, 2019 and will remain valid till 24 Aug, 2029

- Date of Review

The level 5 of National DAE qualification on Mechanical Technology shall be reviewed after three years of the validation i.e. 25 Aug, 2022

- Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for DAE Mechanical Technology level-5 |  |
| --- | --- |
| Code | Description |
| 0714E&A(1) | 1st Level National Certificate of level-5, in “ Mechatronics Technology” |
| 0714E&A(2) | 2nd Level National Certificate of level-5, in “Mechatronics Technology” |
| 0714E&A(3) | 3rd Level National Certificate of level-5, in “Mechatronics Technology” |
| 0714E&A(4) | 4th Level National Certificate of level-5, in “Mechatronics Technology” |
| 0714E&A(5) | 5th Level National Certificate of level-5, in “Mechatronics Technology” |

- Members of Qualification Development Committee

The following members participated in the qualifications development and validation of this qualification:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Javed Rehman Larik | Assistant Professor | MUET Jmashoro |
| 2 | Dr. Saifullah Samo | Assistant Professor | MUET Jamshoro |
| 3 | Muhammad Sohaib | Assistant Manager | Lucky feed processing plant |
| 4 | Eng Sajid Ali | Instructor | Mehran university |
| 5 | Mr.Usman Ali | Assistant Manager(Lining) | Orient Automotive Industry (Pvt) Ltd |
| 6 | Mr.Ahmad Ali | Instructor | Aman Tech Karachi |
| 7 | Mr. Atif Rehman | Sr.Instructor | MITI, Karachi |
| 8 | Mr. Mehboob | Sr.Instructor | SOS, Karachi |
| 9 | Engr. Aijaz Ahmed Zia | D&A Engineer | INTECH Process Automation |
| 10 | Engr. Inayat Ur Rehman | DACUM Facilitator, | Ex. KPK TVETA |
| 11 | Muhammad Ishaq | Dy. Director | NAVTTC Islamabad |

- Qualification Validation Committee

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Javed Rehman Larik | Assistant Professor | MUET Jmashoro |
| 2 | Dr. Saifullah Samo | Assistant Professor | MUET Jamshoro |
| 3 | Engr. Fazal-ur-Rehman | Associate Professor | KP- TEVTA |
| 4 | Engr. Touseef Asghar | Instructor | P - TEVTA |
| 8 | Engr. Inayat Ur Rehman | DACUM Facilitator, | Ex. KPK TEVTA |
| 9 | Fayaz Ahmed Soomro | Dy. Director | NAVTTC Islamabad |

- Entry Requirements

The entry for D.A. E National Certificate level 5, in Mechatronics Technology are

- A person having National Vocational Certificate level 4, in Mechatronics Technology.

- A person having Matric certificate with Science subjects

- Categorization and Levelling of the Competency Standards

| Competency Standard |  | Category | Contact hr. |  |  | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th. | Pr. | Total |  |
| Health and Safety |  |  |  |  |  |  |
| Maintain Occupational Health and Safety | 2 | Generic | 10 | 30 | 40 | 4 |
| Adopt Safety Regulations, Labor Protection Laws, Environmental Protection Laws at Workplace | 2 | Generic | 10 | 30 | 40 | 4 |
| Total |  |  | 20 | 60 | 80 | 8 |
| Basic Machining |  |  |  |  |  |  |
| Perform Metal/Bench Work | 2 | Technical | 6 | 9 | 15 | 1.5 |
| Perform general machining | 2 | Technical | 3 | 5 | 8 | 0.8 |
| Perform cutting on Metal Circular/Power Heck Saw | 2 | Technical | 2 | 3 | 5 | 0.5 |
| Perform tool sharpening | 2 | Technical | 2 | 3 | 5 | 0.5 |
| Perform Basic Lathe Machine Operations | 3 | Technical | 9 | 14 | 23 | 2.3 |
| Perform Drilling Machine Operations | 2 | Technical | 2 | 3 | 5 | 0.5 |
| Perform Shaper, Planar and Slotter Machining Operations | 3 | Technical | 6 | 8 | 14 | 1.4 |
| Perform Milling Operations | 3 | Technical | 10 | 15 | 25 | 7 |
|  |  |  | 40 | 60 | 100 | 10 |
| Electrical Essentials & Networks |  |  |  |  |  |  |
| Operate Measuring Instruments | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Verify Ohm’s & Kirchhoff’s Laws by Implementing Series/Parallel Circuits | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Construct Electromagnet to See Various Effects & Verify Faradays Law | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Verify Law of Combination of Capacitor & Determine Break Down Voltage of Capacitor | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Maintain Tools & Equipment | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Make Cable/Wire Joints | 2 | Technical | 9 | 21 | 30 | 3 |
| Prepare and Install Distribution Boards | 2 | Technical | 9 | 21 | 30 | 3 |
| Carryout Basic Electrical Installations | 2 | Technical | 9 | 21 | 30 | 3 |
| Install Single Phase Electrical Wiring | 2 | Technical | 9 | 21 | 30 | 3 |
| Perform Testing of Electrical Wiring | 2 | Technical | 9 | 21 | 30 | 3 |
| Operate Single Phase Transformer | 5 | Technical | 3 | 6 | 9 | 0.9 |
| Perform Tests on Single Phase Transformer | 5 | Technical | 6 | 9 | 15 | 1.5 |
|  |  |  | 90 | 228 | 318 | 31.8 |
| Digital Skills |  |  |  |  |  |  |
| Install Computer Operating Systems And Hardware | 3 | Generic | 6 | 24 | 30 | 3 |
| Operate Word-Processing Applications | 3 | Generic | 6 | 24 | 30 | 3 |
| Operate Spreadsheet Applications | 3 | Generic | 6 | 24 | 30 | 3 |
| Operate Presentation Packages | 3 | Generic | 6 | 24 | 30 | 3 |
| Perform Writing And Editing Tasks | 3 | Generic | 6 | 24 | 30 | 3 |
| Perform Computer Operations | 3 | Generic | 6 | 24 | 30 | 3 |
| Use Computer Applications | 3 | Generic | 6 | 15 | 21 | 2.1 |
| Create User Documentation | 3 | Generic | 6 | 15 | 21 | 2.1 |
| Create Technical Documentation | 3 | Generic | 6 | 24 | 30 | 3 |
| Create Basic Databases | 3 | Generic | 6 | 24 | 30 | 3 |
| Operate Digital Media Technology | 3 | Generic | 6 | 24 | 30 | 3 |
| Use Social Media Tools For Collaboration And Engagement | 3 | Generic | 6 | 24 | 30 | 3 |
| E-Commerce | 3 | Generic | 6 | 24 | 30 | 3 |
| Use Digital Devices | 3 | Generic | 6 | 24 | 30 | 3 |
|  |  |  | 84 | 318 | 402 | 40.2 |
| Metrology |  |  |  |  |  |  |
| Take measurements with graduated tools | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Take measurements with combination set | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Perform ling | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Take measurements with gauges | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Perform measurements through Micrometer | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Measure dimensions with Vernier tools | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Measure angles with angle measuring Instruments | 3 | Technical | 6 | 18 | 24 | 2.4 |
| Measure Threads and Gears | 3 | Technical | 6 | 18 | 24 | 2.4 |
|  |  |  | 48 | 144 | 192 | 19.2 |
| Basic Engineering Drawing |  |  |  |  |  |  |
| Perform Basic Manual Drawing | 3 | Technical | 6 | 18 | 14 | 1.4 |
| Construct different Engineering Curves | 3 | Technical | 6 | 18 | 17 | 1.7 |
| Construct multi-view drawings | 3 | Technical | 9 | 27 | 25 | 2.5 |
| Install CAD Software | 3 | Technical | 6 | 18 | 17 | 1.7 |
| Develop 2D CAD drawings | 3 | Technical | 9 | 27 | 25 | 2.5 |
| Develop 3D CAD drawings | 3 | Technical | 9 | 27 | 19 | 1.9 |
|  |  |  | 45 | 135 | 117 | 11.7 |
| Welding - I |  |  |  |  |  |  |
| Interpret Parts and Assembly Drawing | 2 | Technical | 3 | 3 | 6 | 0.6 |
| Evaluate the Use of Hand and Power Tools | 2 | Technical | 2 | 3 | 5 | 0.5 |
| Perform Pre-Welding Operations | 2 | Technical | 6 | 10 | 16 | 1.6 |
| Perform Oxy Acetylene Welding | 3 | Technical | 12 | 24 | 15 | 1.5 |
| Cut Metal by Oxy-Acetylene / Oxy-Fuel Flame/Grinding (Manually and Auto Cutting) | 3 | Technical | 12 | 24 | 17 | 1.7 |
| Perform Gas Welding On Mild Steel Plates (1F, 2F, 3F) | 3 | Technical | 12 | 24 | 11 | 1.1 |
| Gas Metal Arc Welding (GMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 4 | Technical | 12 | 24 | 30 | 3 |
| Gas Tungsten Arc Welding (GTAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 4 | Technical | 12 | 24 | 30 | 3 |
| Shielded Metal Arc Welding (SMAW) in Flat (1F, 1G), Horizontal (2F, 2G), Vertical (3F, 3G) and Overhead (4F, 4G) Positions | 4 | Technical | 12 | 24 | 24 | 2.4 |
| Flux Cored Arc Welding (FCAW) | 4 | Technical | 12 | 24 | 30 | 3 |
| Perform Soldering and Brazing Operations | 3 | Technical | 9 | 21 | 16 | 1.6 |
|  |  |  | 104 | 205 | 200 | 20 |
| Microprocessors & Microcontrollers |  |  |  |  |  |  |
| Design Microprocessor applications | 4 | Technical | 4 | 12 | 16 | 1.6 |
| Identify Microcontroller Types and its Architecture | 4 | Technical | 8 | 15 | 23 | 2.3 |
| Interface Microcontroller with system | 4 | Technical | 5 | 21 | 26 | 2.6 |
| Carryout various Microcontroller Applications. | 4 | Technical | 4 | 20 | 24 | 2.4 |
| Apply Microcontroller to build Control System. | 4 | Technical | 3 | 24 | 27 | 2.7 |
| Total |  |  | 24 | 92 | 116 | 11.6 |
| Analogue Electronics |  |  |  |  |  |  |
| Identify the Semiconductor Devices | 4 | Technical | 9 | 21 | 30 | 3 |
| Construct The RF & AF Amplifiers | 4 | Technical | 9 | 21 | 30 | 3 |
| Design the Oscillators | 4 | Technical | 9 | 21 | 30 | 3 |
| Analyze operational Amplifier circuits | 4 | Technical | 9 | 21 | 30 | 3 |
| Design the Multi-Vibrators | 4 | Technical | 9 | 21 | 30 | 3 |
| Construct Ramp Generator circuits | 4 | Technical | 9 | 21 | 30 | 3 |
| Design Function Generator & Comparator circuits | 4 | Technical | 9 | 21 | 30 | 3 |
| Total |  |  | 63 | 147 | 210 | 21 |
| Soft Skills |  |  |  |  |  |  |
| Develop workplace policy and procedures for sustainability | 4 | Generic | 6 | 24 | 30 | 3 |
| Manage meetings | 4 | Generic | 6 | 24 | 30 | 3 |
| Manage recruitment selection and induction processes | 4 | Generic | 6 | 24 | 30 | 3 |
| Manage personal work priorities and professional development | 4 | Generic | 6 | 24 | 30 | 3 |
| Manage workforce planning | 4 | Generic | 6 | 24 | 30 | 3 |
| Undertake project work | 4 | Generic | 6 | 24 | 30 | 3 |
| Identify and communicate trends in career development | 4 | Generic | 6 | 24 | 30 | 3 |
| Apply specialist interpersonal and counseling interview skills | 4 | Generic | 6 | 24 | 30 | 3 |
| Work safely in an office environment | 4 | Generic | 6 | 24 | 30 | 3 |
| Develop workplace documents | 4 | Generic | 6 | 24 | 30 | 3 |
| Prepare and implement negotiation | 4 | Generic | 6 | 24 | 30 | 3 |
| Maintain professionalism in the workplace | 4 | Generic | 6 | 24 | 30 | 3 |
| Organize schedules | 4 | Generic | 6 | 24 | 30 | 3 |
|  |  |  | 78 | 312 | 390 | 39 |
| Computer Programming |  |  |  |  |  |  |
| Installation of C++, IDE for Arduino and Python | 4 | Functional | 2 | 9 | 11 | 1.1 |
| Carryout Basic C++ Programming | 4 | Functional | 5 | 11 | 15 | 1.5 |
| Carryout Basic Programming using python | 4 | Functional | 3 | 9 | 12 | 1.2 |
| Perform basic programming using microcontrollers | 4 | Functional | 1 | 11 | 12 | 1.2 |
| Total |  |  | 11 | 40 | 50 | 5 |
| Computer Integrated Manufacturing (CIM) |  |  |  |  |  |  |
| Perform CNC Machine Operations | 4 | Technical | 20 | 30 | 50 | 5 |
| Perform CNC Lathe Operations | 4 | Technical | 20 | 30 | 50 | 5 |
| Perform CNC Milling Operations | 4 | Technical | 20 | 30 | 50 | 5 |
|  |  |  | 60 | 90 | 150 | 15 |
| Machines and Mechanism |  |  |  |  |  |  |
| Demonstrate Simple Mechanisms | 4 | Technical | 11 | 12 | 23 | 2.3 |
| Perform Kinematic Analysis of Simple Mechanisms | 5 | Technical | 9 | 12 | 21 | 2.1 |
| Demonstrate Mechanical Power Transmission Elements | 4 | Technical | 11 | 18 | 29 | 2.9 |
| Perform Kinematic Analysis of Mechanical Power Transmission Elements | 5 | Technical | 9 | 18 | 27 | 2.7 |
| Total |  |  | 40 | 60 | 100 | 10 |
| Verify Basic Laws of Electrical Machines |  |  |  |  |  |  |
| Verify Basic Laws of Electrical Machines | 4 | Technical | 6 | 24 | 30 | 3 |
| Analyze an Alternator | 4 | Technical | 6 | 24 | 30 | 3 |
| Analyze Single Phase Motors | 4 | Technical | 6 | 24 | 30 | 3 |
| Analyze Special Purpose Motors | 4 | Technical | 6 | 24 | 30 | 3 |
| Analyze Three Phase Motors | 4 | Technical | 6 | 24 | 30 | 3 |
| Analyze Dc Generators | 4 | Technical | 6 | 24 | 30 | 3 |
| Perform Tests on DC Generators | 4 | Technical | 6 | 24 | 30 | 3 |
| Analyze Dc Motors. | 4 | Technical | 6 | 24 | 30 | 3 |
| Perform Tests On DC Motors. | 4 | Technical | 6 | 24 | 30 | 3 |
| Total |  |  | 54 | 216 | 270 | 27 |
| Digital Electronics |  |  |  |  |  |  |
| Verify Truth Tables of Digital Gates | 3 | Technical | 9 | 21 | 30 | 3 |
| Construct & Verify Combinational Logic Circuit | 4 | Technical | 9 | 21 | 30 | 3 |
| Construct and Verify Function of Flip Flops | 4 | Technical | 9 | 21 | 30 | 3 |
| Construct Multi vibrator with 555 IC | 4 | Technical | 9 | 21 | 30 | 3 |
| Construct Shift Registers and Counters with the Help of Flip Flops | 4 | Technical | 9 | 21 | 30 | 3 |
| Total |  |  | 45 | 105 | 150 | 15 |
| Sensors and Actuators |  |  |  |  |  |  |
| Test Sensors and Transducers | 4 | Technical | 12 | 18 | 30 | 3 |
| Calibrate Sensors and Transducers | 4 | Technical | 10 | 18 | 28 | 2.8 |
| Interface the Sensor & Transducers | 4 | Technical | 12 | 18 | 30 | 3 |
| Test Actuators | 4 | Technical | 12 | 18 | 30 | 3 |
| Interface and control Actuator with sensors | 4 | Technical | 11 | 21 | 32 | 3.2 |
| Total |  |  | 57 | 93 | 150 | 15 |
| Programmable Logic Controllers (PLC) (5) |  |  |  |  |  |  |
| Install PLC software and Simulator | 5 | Technical | 12 | 30 | 42 | 4.2 |
| Interface PLC with system | 5 | Technical | 12 | 30 | 42 | 4.2 |
| Program PLC using Ladder Logic | 5 | Technical | 12 | 30 | 42 | 4.2 |
| Design and Test the PLC | 5 | Technical | 12 | 30 | 42 | 4.2 |
| Carry out Industrial Automation on PLC | 5 | Technical | 12 | 30 | 42 | 4.2 |
|  |  |  | 60 | 150 | 210 | 21 |
| Design of Machine Members (5) |  |  |  |  |  |  |
| Calculate Stresses in Machine Parts | 5 | Technical | 10 | 12 | 22 | 2.2 |
| Calculator diameter of cylinder for hoop and longitudinal stresses | 5 | Technical | 8 | 12 | 20 | 2 |
| Calculate thickness and diameter of spherical shell for circumferential stresses | 5 | Technical | 8 | 9 | 17 | 1.7 |
| Design welded joints for transverse and parallel fillet under static and fatigue loading | 5 | Technical | 8 | 15 | 23 | 2.3 |
| Calculate stresses due to initial tightening and external load on screws | 5 | Technical | 8 | 12 | 20 | 2 |
| Design dimension of square and rectangular keys | 5 | Technical | 8 | 12 | 20 | 2 |
| Design shaft subjected to twisting moment only | 5 | Technical | 6 | 9 | 15 | 1.5 |
| Design flange coupling for specific torque | 5 | Technical | 4 | 9 | 13 | 1.3 |
| Total |  |  | 60 | 90 | 150 | 15 |
| Industrial Instrumentation (5) |  |  |  |  |  |  |
| Design & Implement a process control | 5 | Technical | 9 | 21 | 30 | 3 |
| Install Transducer & Transmitters in Industrial Applications | 5 | Technical | 9 | 21 | 30 | 3 |
| Install the process Regulators | 5 | Technical | 9 | 21 | 30 | 3 |
| Install the Variable Frequency Drive (VFD) | 5 | Technical | 9 | 21 | 30 | 3 |
| Install Pneumatic & Hydraulic systems | 5 | Technical | 9 | 21 | 30 | 3 |
| Total |  |  | 45 | 105 | 150 | 15 |
| Robotics(5) |  |  |  |  |  |  |
| Identify Various Parts of Robots | 3 | Technical | 8 | 18 | 26 | 2.6 |
| Identify Various Parts of Robot Arm | 3 | Technical | 6 | 24 | 30 | 3 |
| Develop Robot program | 5 | Technical | 9 | 15 | 24 | 2.4 |
| Develop robotic arm structure | 5 | Technical | 8 | 18 | 26 | 2.6 |
| Test the Robot | 5 | Technical | 8 | 15 | 23 | 2.3 |
| Total |  |  | 39 | 90 | 129 | 12.9 |
| Mechatronic Projects |  |  |  |  |  |  |
| Access Control System using RFID Reader RC522 | 5 | Technical | 9 | 30 | 30 | 3 |
| Finger print Sensor Based Biometric Attendance System | 5 | Technical | 9 | 30 | 30 | 3 |
| Humidity and Temperature Monitoring System. | 5 | Technical | 9 | 27 | 27 | 2.7 |
| Smart Stick for Blinds | 5 | Technical | 9 | 30 | 30 | 3 |
| Perform Line Following Robot | 5 | Technical | 9 | 33 | 33 | 3.3 |
| Perform Smart Phone Control Robot Using Bluetooth | 5 | Technical | 9 | 36 | 36 | 3.6 |
| Perform IoT Controlled based home automation system | 5 | Technical | 9 | 40 | 40 | 4 |
| Perform Smart Phone Control Door Lock System | 5 | Technical | 9 | 36 | 36 | 3.6 |
| Perform Security System Using Motion Detection | 5 | Technical | 9 | 36 | 36 | 3.6 |
| Perform Water- Detection in Tank | 5 | Technical | 9 | 33 | 33 | 3.3 |
| Perform Mini CNC 2D Plotter | 5 | Technical | 9 | 33 | 33 | 3.3 |
| Perform Voice Controlled Robot | 5 | Technical | 9 | 36 | 36 | 3.6 |
| Total |  |  | 108 | 400 | 400 | 40 |
| Hydraulics and Pneummatics 5 |  |  |  |  |  |  |
| Operate Hydraulic Bench and Its Functions | 5 | Technical | 3 | 3 | 6 | 0.6 |
| Calibrate Bourdon Tube and Diaphragm Pressure Gauge | 5 | Technical | 3 | 6 | 9 | 0.9 |
| Operate Hydraulic Press | 4 | Technical | 3 | 10 | 13 | 1.3 |
| Verify Bernoulli’s Equation | 5 | Technical | 5 | 9 | 14 | 1.4 |
| Analyse the performance of Pumps | 5 | Technical | 5 | 9 | 14 | 1.4 |
| Construct circuit for Double Acting Hydraulic Cylinder | 4 | Technical | 3 | 5 | 8 | 0.8 |
| Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder | 5 | Technical | 3 | 7 | 10 | 1 |
| Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking | 5 | Technical | 8 | 11 | 19 | 1.9 |
| Set Speed and Direction of Hydraulic Motor | 5 | Technical | 3 | 5 | 8 | 0.8 |
| Operate Pressure Measuring Instruments | 5 | Technical | 12 | 16 | 28 | 2.8 |
| Operate Flow Regulating Devices | 5 | Technical | 7 | 14 | 21 | 2.1 |
|  |  |  | 55 | 95 | 150 | 15 |
| Entrepreneurial Skill (5) |  |  |  |  |  |  |
| Develop Entrepreneurial Skills | 5 | Generic | 6 | 18 | 24 | 2.4 |
| Apply project information management and communications techniques | 5 | Generic | 9 | 15 | 24 | 2.4 |
| Apply project human resources management approaches | 5 | Generic | 9 | 15 | 24 | 2.4 |
| Direct human resources management of a project program | 5 | Generic | 9 | 15 | 24 | 2.4 |
| Develop a project management plan | 5 | Generic | 6 | 15 | 21 | 2.1 |
| Maintain business resources | 5 | Generic | 6 | 15 | 21 | 2.1 |
| Develop a sales plan | 5 | Generic | 6 | 24 | 30 | 3 |
| Plan and implement business-to-business marketing | 5 | Generic | 6 | 24 | 30 | 3 |
| Address customer needs | 5 | Generic | 9 | 21 | 30 | 3 |
| Manage personal finances | 5 | Generic | 9 | 15 | 24 | 2.4 |
| Solve problems which jeopardize safety and security | 5 | Generic | 6 | 24 | 30 | 3 |
| Coordinate a work team | 5 | Generic | 6 | 24 | 30 | 3 |
| Lead small teams | 5 | Generic | 6 | 24 | 30 | 3 |
| Plan and organize work | 5 | Generic | 6 | 24 | 30 | 3 |
| Develop teams and individuals | 5 | Generic | 6 | 24 | 30 | 3 |
| Apply problem solving techniques in the workplace using critical thinking | 5 | Generic | 6 | 24 | 30 | 3 |
| Manage human resource services | 5 | Generic | 6 | 24 | 30 | 3 |
| Total |  |  | 117 | 345 | 462 | 46.2 |

NOTE:- The semester project may please be allotted to the students by the concerned teacher in relevant technology, while the final semester project is already laid down in the curriculum.

- Detail of Qualifications and its Competency Standards

## Microprocessor and Microcontroller

### Design Microprocessor applications

Overview

After completion of this competency standard the student will be able to perform a variety of tasks on a microprocessor and will be able to integrate the programming knowledge into the microprocessor

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Identify microprocessors | - Describe the Microprocessor architecture - Compare and contrast the characteristics of Microprocessor & Microcontroller - Explain micro-processor selection criteria - Understand processor features and Pin-Configuration |
| - Identify function of microprocessors and their associated pins | - Describe memory functions and their allocation - Describe Arithmetic and Logic units - Explain the bus structure of microprocessor - Understand the IOs and their application |
| - Program a microprocessor using basic instruction set | - List Down Basic instruction set and explain any - Explain addressing mode - Explain data transfer via accumulator |
| - Program a microprocessor to perform basic arithmetic and logical operation | - List Down and explain arithmetic and logical operation. - Addition & Subtraction - Multiplication & Division - AND, OR, NOT operation - Jump operation |
| - Program a microprocessor to perform special operations | - List Down and explain special function operations - Loops - Input / Output - Subroutines and interrupt - Explain Debugging and its procedure |

Knowledge and understanding

- Learn to identify microprocessor IC & their applications in different circuits

- Learn to identify Functions of microprocessor & their applications in different circuits

- Learn to Understand Instruction Sets and how to read and implement them

- Learn to Understand Memory types, their allocation and addressing procedures

- Learn to Implement basic microprocessor operations including move, arithmetic and logical operations

- Learn to implement Loops, Conditions, IOs, Subroutines, Interrupt and other Special function registers

- Learn to troubleshoot the problems via debugging tool

Tools & Equipment

| SN |  |
| --- | --- |
|  | Microprocessor |
|  | Development Kit |
|  | Breadboard |
|  | IC remover tool |
|  | Voltmeter |
|  | Programming Computer |
|  | Power supply |
|  | Trainer |

Critical Evidence(s) Required

The student needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Identify various microprocessor and there data sheet

- Program a microprocessor to perform complex set of instructions

### Identify Microcontroller Types and its Architecture

Overview: This competency standard covers the skills and knowledge required to install, run, simulate and apply on field work.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Introduction to Microcontroller (Arduino, Raspberry Pi, etc.) | P1. Identify the various Microcontrollers and their models. P2. Identify RAM, SRAM, RAM, ROM, EEPROM, EPROM. P3. Identify Timer, Counters, Clock/processing speed of Microcontrollers P4. Identify busses and Processing Speed (4, 8, 16, 32, 64 bits) P5. Identify number of Analog to digital converters (ADCs) and Digital to analog converter (DAC), P6. Identify Analog Input and Output Pins. P7. Identify Digital Input and Output Pins. P8. Identify the PWM Pins. P9. Identify the UART Pins. P10. Identify the I2C pins. |
| CU2. Introduction to Arduino IDE | P1. Perform the Arduino IDE Installations. P2. Perform Arduino Sketch/Programming P2. Use Arduino supporting libraries and functions P3. Write down sketch for Arduino in IDE. P4. Identify the Shields, Accessories, and Sensors for Arduino |
| CU3. Program Arduino Microcontroller by using conditional and loop instructions. | P1. Use Boolean Expressions P2. Use If, If-Else, Nested if Statements P3. Identify Logical Operators P4. Identify Simple Recursion P5. Use While Loop P6. Use for loop |
| CU4. Perform programing using Timer, and Interrupt. | P1. Identify Timers in microcontroller P2. Identify Interrupts in microcontroller P3. Differentiate Internal and External interrupts P4. Identify Interrupt priority P5. Use While Loop P6. Use for loop |

Knowledge and understanding

- Describe PIC Microcontroller

- Explain Architecture of 8051 Microcontroller

- Describe microprocessors and microcontrollers

- Differentiate between micro-processor and micro controllers

- Define the components of a micro-processing system (Central Processing Unit, Input Output Interfaces and Memory)

- Explain the functions of various registers.

- Understand interrupt structure of 8051.

- Explain Analog to digital converters (ADCs) and Digital to analog converter (DAC),

- Understand serial data communication concepts

- Describe the types of buses (data bus, address bus, control bus)

- Describe the architecture of a microprocessor and its major components (arithmetic and logic unit (ALU), registers and its types, control units, clock generator)

- Describe the memory of a micro processing system for storage; understand its various types and functions of each type (ROM, PROM, EPROM, EEPROM, RAM, and Flash)

- Describe the input and output ports for external connectivity (peripheral devices, making connections, polling/interrupts, parallel and serial I/O).

- Describe 8-bit, 16-bit and 32-bit microcontrollers

- Basics of embedded control and software.

- Describe to AVR family IC

Equipment and Tools

- 8051 Microcontroller

- Arduino Microcontroller with development board

- Computer

- Programming software.

- Raspberry Pi Microcontroller with development board

- SD card

- Simulation software

- Communication cable.

- Power Adaptor

- Jumpers

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Writing program

- Execution of program

- Output & results from program

- Identify Microcontrollers.

- Installation of software for microcontroller

### Interface Microcontroller with system

Overview: This competency standard covers the skills and knowledge required to install, run, simulate and apply on field work.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Interface Analog Sensor with microcontroller. | P1. Identify Analog Sensors and pin configurations P2. Open Arduino IDE software. P2. Select and connect analog sensor to Analog Input Pin. P3. Make a source code/sketch. P4. Compile and Debug P5. Run Program. P6. Get results (Analog values) |
| CU2. Interface Digital Sensors with microcontroller. | P1. Identify Digital Sensors and pin configurations P2. Open Arduino IDE software. P2. Select and connect digital sensor to Analog Input Pin. P3. Make a source code/sketch. P4. Compile and Debug P5. Run Program. P6. Get results (Digital values 0 or 1) |
| CU3. Interface 7 segment display and LCD display with microcontroller. | P1. Identify pin configurations P2. Open Arduino IDE software. P2. Select and connect LCD display Module to Pin.? P3. Make a source code/sketch to display the number 3 on 7 segment display, and display text PAKISTAN on LCD display Module. P4. Compile and Debug P5. Run Program. P6. Get results (PAKISTAN, 3) |
| CU3. Interface Servomotor with microcontroller. | P1. Identify pin configurations P2. Open Arduino IDE software. P2. Select and connect Servo motor to desired Pin. P3. Make a source code/sketch to rotate about 45 degrees. P4. Compile and Debug P5. Run Program. P6. Get results (Rotation value 45o) |
| CU4. Interface Serial (UART) based communication | P1. Open Arduino IDE software. P2. Connect module serial out TX pin with Arduino board serial RX pin and connect module serial RX pin with Arduino board TX pin. P3. Make a source code and set baud rate. P4. Compiling and Debugging. P5. Run Program. |
| CU5. Interface I2C based communication | P1. Open Arduino IDE software. P2. Connect two wires of sensor with SCL and SDA pins of Arduino board. P3. Make a source code using two wire (I2C) libraries. P4. Compiling and Debugging. P5. Run Program. |

Knowledge and understanding

- Describe interfacing

- Describe five senses: (vision, hearing, smell, taste, and touch)

- Describe input and output of sensors and actuators.

- Describe Acceleration, Shock and Vibration Sensors

- Explain SCADA as interfacing techniques using ADC and DAC.

- Describe Sensor interfacing

- Describe Actuator/relay interfacing

- Describe Input/key/switch/Keypad interfacing

- Describe communication interfacing

- Describe I2C interfacing

- Describe SPI. (Serial Peripheral Interface)

- Describe RS232 interfacing

Equipment and Tools

- Arduino development board

- Computer

- Programming software.

- Simulation software

- Communication cable.

- Power Adaptor

- Servo Motor

- Analog Sensors

- Digital Sensors

- Display Module

- I2C module for LCD Display

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Identify components.

- Installation of Arduino IDE software.

- Run Arduino IDE and design source code.

- Interface sensors and actuators.

### Carryout various Microcontroller Applications.

Overview: This competency standard covers the skills and knowledge required to install, run, simulate and apply in the  field work.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Perform a task by connecting level/floating sensor and ON/OFF relay with microcontroller. | P1. Identify level sensors to detect the level of water. P2. Identify relay to ON/OFF the water pump. P3. Identify the Input and Output pins. P4. Identify the Digital, Analog and PMW pins. P5. Make a layout and connect components P6. Write, compile, debug and upload a program to microcontroller. P7. RUN and Get results (ON/OFF depends on level) |
| CU2. Perform a task by connecting light sensor and servomotor with microcontroller. | P1. Identify light sensors to detect the resistivity of light. P2.Identify servo motor to rotate/position the system. P3. Identify the Input and Output pins. P4. Identify the Digital, Analog and PMW pins. P5. Make a layout and connect components P6. Write, compile, debug and upload a program to microcontroller. P7. RUN and Get results (Position depends on resistivity) |
| CU3. Perform a task by connecting pressure sensor and pneumatic actuator with microcontroller. | P1. Identify pressure sensors . P2.Identify pneumatic actuator to apply force in system. P3. Identify the Input and Output pins. P4. Identify the Digital, Analog and PMW pins. P5. Make a layout and connect components P6. Write, compile, debug and upload a program to microcontroller. P7. RUN and Get results (Force depends on pressure) |
| CU4. Perform a task by connecting potentiometer and Servomotor with microcontroller. | P1. Identify potentiometer to detect the resistance/angle. P2.Identify servomotor to position the system. P3. Identify the Input and Output pins. P4. Identify the Digital, Analog and PMW pins. P5. Make a layout and connect components P6. Write, compile, debug and upload a program to microcontroller. P7. RUN and Get results (Angle depends on Potentiometer) |

Knowledge and understanding

- Describe microcontroller applications,

- Describe problem identification

- Define need to solve problems

- Describe the solution for problems

- Describe the importance of solution

- Describe application of

- Consumer Electronics Products: (Toys, Cameras, Robots, Washing Machine, Microwave Ovens etc.)

- Instrumentation and Process Control: (Oscilloscopes, Multi-meter, Leakage Current Tester, Data Acquisition and Control etc.)

- Medical Instruments: (ECG Machine, Accu-Check etc.)

- Communication: (Cell Phones, Telephone Sets, Answering Machines etc.)

- Office Equipment: (Fax, Printers etc.)

- Multimedia Application: (Mp3 Player, PDAs etc.)

- Automobile: (Speedometer, Auto-breaking system etc.)

Equipment and Tools

- Arduino development board

- Computer

- Programming software.

- Simulation software

- Communication cable.

- Power Adaptor

- Servo Motor

- Analog Sensors

- Digital Sensors

- Display Module

- I2C module for LCD Display

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Identify components.

- Installation of Arduino IDE software.

- Run Arduino IDE and design source code.

- Interface sensors and actuators.

### Apply Microcontroller to build Control System.

Overview: This competency standard covers the skills and knowledge required to install, run, simulate and apply on field work.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Build a Access Control System Using RFID Reader. | P1. Identify RFID (radio frequency identification devices). P2. Identify tag, read/write devices for RFID P3. Identify host system for data collection, processing, and transmission. P4. Identify the Digital, Analog and PMW pins. P5. Make a layout and connect components P6. Write, compile and debug a program for writing P7. Write, compile and debug a program for reading P8. upload a program to microcontroller for writing a valid key. P0714E&A upload a program to microcontroller for reading key and unlock the door if key is valid. |
| CU2. Build a System to Control Light Using Smart Phone. | P1. Identify light sensors to detect the resistivity ?? Intensity of light. P2. Identify servomotor to rotate/position the system. P3. Identify the Input and Output pins. P4. Identify the Digital, Analog and PMW pins. P5. Make a layout and connect components P6. Write, compile, debug and upload a program to microcontroller. P7. RUN and Get results (Position depends on resistivity) |

Knowledge and understanding

- Describe the Radio Frequencies

- Describe the Servomotor

- Describe the tag

- Describe the read and write devices.

- Describe the types of light sensors

- Define the types of Timers.

Equipment and Tools

- RFID tag

- RFID Device

- Computer/laptop

- Arduino development board

- USB/Data cable

- Light sensor

- Servomotor

- Bluetooth device

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Identify components.

- Installation of Arduino IDE software.

- Run Arduino IDE and design source code.

- Interface sensors and actuators.

## Analogue Electronics

### Identify Basic Electronics Components

Overview: This competency standard covers the skills and knowledge required to Identify Various Diodes, Identify Resistors in circuit, Identify Capacitor in circuit, identify Inductor in circuit and Identify IC’s Packages. After this competency standard the candidate will be able to identify variety of basic electronic components and their usage in industry.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Identify Various Diodes | - Identify the Diodes - Identify its types & polarities - Draw the Diode characteristics curves in forward and reverse Biased |
| CU2. Identify Resistors in circuit | - Identify the Resistor & its types - Recognize Coding &Color coding of resistor - Design series & Parallel circuit of Resistor - Use formulae for Series & parallel circuit of resistors |
| CU3. Identify Capacitor in circuit | - Identify Capacitor & its types - Recognize Coding &rating of Capacitor - Design Parallel and series circuit of Capacitor - Use formulae for Series & parallel circuit of Capacitor |
| CU4. Identify Inductor in circuit | - Identify an Inductor - Recognize Coding & Rating of Inductor - Use formulas for Series & Parallel circuit of Inductor - Analyze Circuit of Inductor |
| CU5. Identify IC’s Packages. | P1. Identify IC Packages & types. P2. Apply the appropriate ICs Packages in circuit |

Knowledge & Understanding:

- Describe the diodes, polarities & their applications in circuits

- Explain the  uses of Multimeter& power Supply

- Explain the  data sheets

- Explain Resistor & their applications in Parallel & Series circuits

- Describe Capacitor& their applications in circuits

- Describe the Inductor& their applications in circuits

- Describe the basics of IC Packages

- Understand the data sheets

Tools & Equipment:

- Multimeter

- Power supply

- Trainer

- Resistor

- Inductor

- ICs

- Source of data sheets

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard

Identification of resistor, capacitor, inductor, IC packages & Diode types.

### Design a Rectifier Using Diode

Overview: This competency standard covers the skills and knowledge required to. This Identify The parameter of Diode and Draw the characteristic curve of Diode, Design Full Wave Rectifier (two diode rectifier) and Design Full Wave Rectifier using Diode Bridge competency standard will help the candidate in construction of Diode rectifier and its uses in industry.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Identify The parameter of Diode and Draw the characteristic curve of Diode | - Identify the Diodes and their terminals (Anode and Cathode) whith the help of Datasheet - Describe various parameter (i.e. Current, Voltage, and power rating) of Diode using Diode Data sheets. Don’t capitalize the wors’s first letter !! - Implement the Diode in forward and Revers Configuration - Perform the forward and reveres biases operation - Monitor the Output waveform on oscilloscope - Draw the characteristic curves in forward and reverse Biased - Generate the Lab report |
| CU2. Design half wave and Full Wave Rectifier (two diode rectifier) | - Identify Full Wave and half wave Rectifier components Draw circuit Diagram of half wave Rectifier Don’t capitalize the wors’s first letter !! - Draw circuit Diagram of Full Wave Rectifier - Observe the INPUT and Output wave form on oscilloscope - Calculate the ripple Factor - Calculate output voltage using proper formulae - Generate output report |
| CU3. Design Full Wave Rectifier using Diode Bridge | - Identify Full Wave Rectifier components - Draw circuit Diagram of Full Wave Rectifier - Observe the INPUT and Output wave form on oscilloscope - Calculate the ripple Factor - Calculate output voltage |

Knowledge & Understanding:

- Study the basic of diodes, & their applications in circuits

- Knowledge of basic electronics

- Understand Multimeter& power Supply

- Understand the data sheets

- Explain basics of diodes, AC & DC voltages, transformers & Rectifiers.

- Explain the  uses of Multimeter

- Explain the  uses oscilloscope and Power Supply

Tools & Equipment:

- Multimeter

- Power supply

- Trainer

- Diodes

- Digital

- Oscilloscope

- Datasheets

- Oscilloscope

- Resistors

- Center taped Transformer

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard.

- Perform half- wave rectification.

- Perform full-wave rectification with diode.

- Perform full-wave rectification with bridge rectification.

### Carry Out Diode Application

Overview: This competency standard covers the skills and knowledge required to make voltage regulator using zener diode.Make Seven Segment Using Light Emitting Diode After completing this competency standard, the student will be able to design a voltage regulator circuit using a zener diode to maintain a constant DC output voltage across the load in spite of variations in the input voltage or changes in the load current.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Make voltage regulator using Zener diode | - the the voltage Regulator circuit - Select the components according to the drawing - Buit / for voltage regulator circuits on the breadboard / PCB / board. - Measure Input and outputs of the voltage regulator - Verify the output with respect to the required output and specifications - Generate the input / output report |
| CU2. Make Seven Segment Using LED(Light Emitting Diode) | - Draw the Seven Segment Display Circuit - Select required components according to the drawing Placed and assemble the circuit on the breadboard / PCB - Perform basic operations of Seven Segment Display and Verify it with specifications and requirements. Generate the report. |

Knowledge & Understanding:

- Learn basic knowledge of zener Diode& its applications

- Learn knowledge of components

- Learn to use oscilloscope &power Supply

- Understand the data sheets

- Learn to Solder the Components

- Learn adequate knowledge of hand tools

- Learn basic knowledge of LED & its applications

- Learn knowledge of components

- Learn to use power Supply

Tools & Equipment:

- Oscilloscope

- Zener diode

- Soldering iron

- Connecting wire

- Light Emitting diode

- Resistor,

- Variable DC power supply,

- Millimeter

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard.

- Regulated power supply using zener diode.

- Seven segment display using LED (light emitting diode.)

### Implement Bipolar Junction Transistor (BJT) in Different Applications

Overview: This competency standard covers the skills and knowledge required to Perform the Biasing of Transistors, Implement Transistor as an amplifier using CB Configuration, Implement Transistor as an amplifier using CC Configuration, Implement Transistor as an amplifier using CE Configuration, Design the circuit of Class A Power Amplifier and Implement BJT as a switch After completion of this competency standard the student will be able to regulate the current or voltage flow and implement a switch for electronic signals.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Perform the Biasing of Transistors | P1. Identify the Transistor& its types. P2. Identify the base, collector & Emitter of transistors P3. Perform the standard Biasing of PNP & NPN Transistor |
| CU2. Implement Transistor as an amplifier using CB Configuration. | - Draw the Circuit of CB configuration of transistor - Select the components for CB configurations. - Place the components for CB amplifier - Calculate the gain of transistor in CB modes. - Draw VI characteristics curve for CB |
| CU3. Implement Transistor as an amplifier using CC Configuration. | - Draw the Circuit of CC configuration of transistor - Select the components for CC configurations. - Place the components for CC amplifier - Calculate the gain of transistor in CC modes. - Draw VI characteristics curve for CC |
| CU4. Implement Transistor as an amplifier using CE Configuration. | Draw the Circuit of CE configuration of transistor - Select the components for CE configurations. - Place the components for CE amplifier - Calculate the gain of transistor in CE modes. - Draw VI characteristics curve for CE |
| CU5. Design the circuit of Class A Power amplifier | - Identify the Class a Power Amplifier - Select the component for Class a Power Amplifier - Implement the circuit of PNP OR NPN transistor in Class A Power Amplifier Configuration - Analyze the different parameter of Class A Power Amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Class A Power Amplifier - Calculate the Voltage gain and Power Gain of Class A Power Amplifier - Generate the Lab report |
| CU6. Implement BJT as a switch. | - Draw the Circuit of transistor in switching configuration. - Select the components for switching circuits - Place the components - Operate an LED using transistor as a switch - Measure the output and generate the report |

Knowledge & Understanding:

- Learn basic concepts of transistor & Biasing

- Study semiconductor theory

- Learn Doping Procedure.

- Study the datasheet of transistor

- Learn basic concepts & working principles of transistor

- Learn basics of Coupling Capacitor.

- Learn the V-I Characteristics

- Learn Doping Procedure.

- Study the basic of BJTs, & their applications in circuits

- Knowledge of basic electronics

- Understand Multimeter & Oscilloscope

- Power Supply

Tools & Equipment:

- Trainer

- Transistors

- Multimeter

- Capacitors

- Resistors

- Power Supplies

- Oscilloscope

- Datasheets

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard

- Testing of BJT.

- Gain of transistor.

- Transistor as a Switch.

- Transistor as an amplifier.

### Implement Field Effect Transistors (FET) In Different Applications

Overview: This competency standard covers the skills and knowledge required to Perform the Biasing of FET, Implement MOSFET as a switch, Draw the VI characteristics curves for FETs, Design the circuit of Common Drain (CD) Amplifier, Design the circuit of Common Gate (CG) Amplifier, design a switching Circuit Using MOSFET and Design a Low voltage transistor based regulated power supply. After completion of this competency standard the student will be able to implement the FET (Field Effect Transistor) to control the current flow through the device.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Perform the Biasing of FET | P1. Identify the FET and & its types. P2. Identify the Drain, Gate and & Source of FET P3. Perform the standard Biasing of MOSFET (N-channel, P-channel) P4. Measure the Gate-Source voltage (Veg’s) & Threshold Voltage (Vth) |
| CU2. Implement MOSFET as a switch. | - Draw the Circuit of MOSFET in switching configuration. - Select the components for switching circuits - Place the components - Operate an DC lamp using MOSFET as a switch - Measure the output and generate the report |
| CU3. Draw the VI characteristics curves for FETs | P1. Construct an amplifier circuit using FETs P2. Apply Vds & Vgs P2. Measure the drain current P4. Draw VI characteristic curves |
| CU4.Design the circuit of Common Drain (CD) Amplifier | - Identify the FET and there terminal (gate, drain and Sources) whit the Help of Datasheet - Select the components for Common Drain (CD) Amplifier - Implement the circuit of Common Drain (CD) Amplifier - Analyze the different parameter of Common Drain (CD) Amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Common Drain (CD) Amplifier - Generate the Lab report |
| CU5. Design the circuit of Common Gate (CG) Amplifier | - Identify the FET and there terminal (gate, drain and Sources) whit the Help of Datasheet - Select the components for Common Gate (CG) Amplifier - Implement the circuit of Common Gate (CG) Amplifier - Analyze the different parameter of Common Gate (CG) Amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Common Gate (CG) Amplifier - Generate the Lab report |
| CU6. Design a switching Circuit Using MOSFET | - Identify the MOSFET and there terminal (gate, drain and Sources) whith the Help of Datasheet - Select the components for the Switching Circuit - Implement Switching Circuit using MOSFET - Perform the operation of switching - Monitor the Output - Generate the Lab report |
| CU7. Design a Low voltage transistor based regulated power supply | - Draw the Schematic of power supply - Select the components for power supply - Implement the circuit of power supply - Perform individual operations on different sections of a power supply - Measure Output Voltage - Generate the Lab report |

Knowledge & Understanding:

- Learn the basics of FET

- Learn the concept of FET Biasing.

- Learn the power rating of FET

- Study The datasheet of FET

- Learn the Switching theory

- Learn the behavior of current and voltage in FET’s

- Learn biasing mechanism and basic formulae of FET’s

- Learn the vgs, vds, Idss&Rds as per datasheet.

- Study the basic of FETs, & their applications in circuits

- Knowledge of basic electronics

- Understand Multimeter & power Supply

- Understand the data sheets

- Study the basic of FETs, & their applications in circuits

- Study the basic of BJTs, ICs & their applications in circuits

- Knowledge of basic electronics and amplifier

- Understand Multimeter, Oscilloscope & power Supply

Tools & Equipment:

- Multimeter

- Capacitors

- Resistors

- Trainer

- Oscilloscope

- Simple FET

- MOSFET

- FET Trainer

- Power supply

- Datasheets

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard.

- Testing of transistor.

- Transistor as a switch

- Transistor as an amplifier.

- MOSFET as a power transistor.

### Implement (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in various application

Overview: This competency standard covers the skills and knowledge required to Implement the UJT in electronic circuits as switch, Implement the SCR in electronic circuits as switch and Construct the dimmer circuit using Diac &Triac. After the completion of this standard the candidate will be able to install Uni junction Transistor (UJT), Silicon-controlled rectifier (SCR) in power Control Application.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Implement the UJT in electronic circuits as switch | - Identify the UJT - Draw the circuit of switch using UJT. - Select the components for the relaxation oscillator circuits - Construct the relaxation oscillator circuits using UJT - Measure the input and output voltage - Generate the lab report |
| CU2. Implement the SCR in electronic circuits as switch | - Identify the SCR terminals - Draw the circuit of switch using SCR. - Select the components for SC switching circuits. - Construct the SC switching circuit. - Apply the trigger Pulse and Check out the desired outputs |
| CU3. Construct the dimmer circuit using Diac &Triac. | - Identify the Diacv and Triac - Draw the dimmer circuit using Diac and & Triac. - Select the components for the dimmer circuit. - Construct the dimmer circuits. - Control the load using dimmer Generate the lab report |

Knowledge & Understanding:

- Learn the basics of UJT

- Understand the data sheets

- Learn adequate knowledge of hand tools

- Learn basics of SCR

- Learn the uses of oscilloscope, power Supply& Multimeter

- Learn the basics of diac&triac

- Learn the uses of oscilloscope, power

Tools & Equipment:

  - Oscilloscope

  - UJT,

  - Resistor,

  - Variable DC power supply,

  - Multimeter

  - Soldering iron, wire

  - Breadboard or trainer

  - Trainer/Breadboard

  - SCR, Diodes, Resistors, Inductors, Capacitors & connecting wire

  - Power supply

  - Diac &Triac resistors

  - Diode/ Capacitor

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard

- Testing of transistor.

- Transistor as switch with SCR.

- Transistor as an AC regulator

### Design Operation Amplifier

Overview: This competency standard identifies the competencies required to Construct a Non-inverting amplifier using operational amplifier, construct an Inverting amplifier using and Construct a differentiator circuit using operational amplifier. Construct Operation Amplifier.

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Construct a Non-inverting amplifier using operational amplifier | - Identify the Operational amplifier and its terminals (Inverting, Non-inverting Inputs &Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of non-Inverting Op-Amp. - Select the components for Non-Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Non-Inverting Op-Amp circuit. - Measure the output frequency response & gain - Draw the characteristic curves of Op-Amp. - Generate the Output report |
| - Construct an Inverting amplifier using operational amplifier | - Identify the Operational amplifier and its terminals (Inverting, Non-inverting Inputs &Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of Inverting Op-Amp. - Select the components for Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Inverting Op-Amp circuit. - Measure the output frequency response & gain - Draw the characteristic curves of Op-Amp. - Generate the Output report |
| - Construct a differentiator circuit using operational amplifier | - Draw the Schematic diagram of differentiator circuit using Op-Amp. - Select the components for differentiator circuit. - Implement differentiator circuit. - Perform the operations of differentiator circuit. - Measure the output, frequency response & gain - Draw the characteristic curves of differentiator circuit. - Generate the Output report |

Knowledge & Understanding:

- Study the basics of Transistors, ICs, Capacitors, Resistors, Op-Amp & their applications in circuits

- Explain the  basics of electronics & its applications

- Understand Multimeter, Oscilloscope & power Supply & their applications

- Understand the data sheets

- Explain the  basics of electronics, Differentiator and their applications

Tool & Equipment:

  - Multimeter

  - Power supply

  - Trainer

  - Op-Amp

  - Resistors

  - Inductors

  - Capacitors

  - Digital Oscilloscope

  - Datasheets

  - Functions Generator

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard

Op-amp as inverting & non-inverting amplifier.

## Soft Skills

- Develop workplace policy and procedures for sustainability

Overview: This unit describes the skills and knowledge required to develop and implement a workplace sustainability policy and to modify the policy to suit changed circumstances. It applies to individuals with managerial responsibilities who undertake work developing approaches to create, monitor and improve strategies and policies within workplaces and engage with a range of relevant stakeholders and specialists.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Develop workplace sustainability policy | P1 Define scope of sustainability in the policies P2 Gather information from a range of sources to plan and develop policy P3 Identify and consult stakeholders as a key component of the policy development process P4 Include appropriate strategies in policy at all stages of work for minimizing resource use, reducing toxic material and hazardous chemical use and employing life cycle management approaches P5 Make recommendations for policy options based on likely effectiveness, timeframes and cost P6 Develop policy that reflects the organization s commitment to sustainability as an integral part of business planning and as a business opportunity P7 Agree to appropriate methods of implementation, outcomes and performance indicators |
| 2. Communicate workplace sustainability policy | P1 Promote workplace sustainability policy, including its expected outcome, to key stakeholders P2 Inform those involved in implementing the policy about expected outcomes, activities to be undertaken and assigned responsibilities |
| 3. Implement workplace sustainability policy | P1 Develop and communicate procedures to help implement workplace sustainability policy P2 Implement strategies for continuous improvement in resource efficiency P3 Establish and assign responsibility for recording systems to track continuous improvements in sustainability approaches |
| 4. Review workplace sustainability policy implementation | P1 Review workplace sustainability policy implementation P2 Investigate successes or otherwise of policy P3 Monitor records to identify trends that may require remedial action and use to promote continuous improvement of performance P4 Modify policy and or procedures as required to ensure improvements are made |

Knowledge & Understanding

K1: outline the environmental or sustainability legislation, regulations and codes of practice applicable to the organization identify internal and external sources of information and explain how they can be used to plan and develop the organization s sustainability policy

K2: explain policy development processes and practices

K3: outline organizational systems and procedures that relate to sustainability

K4: outline typical barriers to implementing policies and procedures in an organization and possible strategies to address them. Assessment Conditions

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to Develop workplace policy and procedures for sustainability. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- scope and develop organizational policies and procedures that comply with legislative requirements and support the organization s sustainability goals covering at a minimum:

  - minimizing resource use

  - resource efficiency

  - reducing toxic material and hazardous chemical use

  - employing life cycle management approaches

  - continuous improvement

- plan and implement sustainability policy and procedures including:

  - agreed outcomes

  - performance indicators

  - activities to be undertaken

  - assigned responsibilities

  - record keeping, review and improvement processes

- consult and communicate with relevant stakeholders to generate engagement with sustainability policy development, implementation and continuous improvement

- Review and improve sustainability policies.

Soft Skills

### Manage meetings

Overview: This unit describes the skills and knowledge required to manage a range of meetings including overseeing the meeting preparation processes, chairing meetings, organizing the minutes and reporting meeting outcomes. It applies to individuals employed in a range of work environments who are required to organize and manage meetings within their workplace, including conducting or managing administrative tasks in providing agendas and meeting material. They may work as senior administrative staff or may be individuals with responsibility for conducting and chairing meetings in the workplace.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Prepare for meetings | P1 Develop agenda in line with stated meeting purpose P2 Ensure style and structure of meeting are appropriate to its purpose P3 Identify meeting participants and notify them in accordance with organizational procedures P4 Confirm meeting arrangements in accordance with requirements of meeting P5 Dispatch meeting papers to participants within designated timelines |
| 2. Conduct meetings | P1 Chair meetings in accordance with organizational requirements, agreed conventions for type of meeting and legal and ethical requirements P2 Conduct meetings to ensure they are focused, time efficient and achieve the required outcomes P3 Ensure meeting facilitation enables participation, discussion, problem-solving and resolution of issues P4 Brief minute-taker on method for recording meeting notes in accordance with organizational requirements and conventions for type of meeting |
| 3. Follow up meetings | P1 Check transcribed meeting notes to ensure they reflect a true and accurate record of the meeting and are formatted in accordance with organizational procedures and meeting conventions P2 Distribute and store minutes and other follow-up documentation within designated timelines, and according to organizational requirements P3 Report outcomes of meetings as required, within designated timelines |

Knowledge & Understanding

K1: outline meeting terminology, structures, arrangements

K2: outline responsibilities of the chairperson and explain group dynamics in relation to managing meetings

K3: describe options for meetings including face-to-face, teleconferencing, web-conferencing and using webcams

K4: identify the relevant organizational procedures and policies regarding meetings, chairing and minutes including identifying organizational formats for minutes and agendas.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage meetings. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- apply conventions and procedures for formal and informal meetings including:

  - developing and distributing agendas and papers

  - identifying and inviting meeting participants

  - organizing and confirming meeting arrangements

  - running the meeting and following up

- organize, take part in and chair a meeting

- record and store meeting documentation

- Follow organizational policies and procedures.

### Manage recruitment selection and induction processes

Overview: This unit describes the skills and knowledge required to manage all aspects of recruitment selection and induction processes in accordance with organizational policies and procedures. It applies to individuals or human resource personnel who take responsibility for managing aspects of selecting new staff and orientating those staff in their new positions. It is not assumed that the individual will be directly involved in the selection processes themselves, although this may well be the case.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Develop recruitment, selection and induction policies and procedures | P1 Analyze strategic and operational plans and policies to identify relevant policies and objectives P2 Develop recruitment, selection and induction policies and procedures and supporting documents P3 Review options for technology to improve efficiency and effectiveness of recruitment and selection process P4 Obtain support for policies and procedures from senior managers P5 Trial forms and documents supporting policies and procedures and make necessary adjustments P6 Communicate policies and procedures to relevant staff and provide training if required |
| 2. Recruit and select staff | P1 Determine future human resource needs in collaboration with relevant managers and sections P2 Ensure current position descriptors and person specifications for vacancies are used by managers and others involved in recruitment, selection and induction processes P3 Provide access to training and other forms of support to all persons involved in recruitment and selection process P4 Ensure advertising of vacant positions complies with organizational policy and legal requirements P5 Utilize specialists where necessary P6 Ensure selection procedures are in accordance with organizational policy and legal requirements P7 Ensure processes for advising applicants of selection outcome are followed P8 Ensure job offers and contracts of employment are executed promptly, and new appointments are provided with advice about salary, terms and conditions |
| 3. Manage staff induction | P1 Provide access to training and ongoing support for all persons engaged in staff induction P2 Check induction processes are followed across the organization P3 Oversee management of probationary employees and provide them with feedback until their employment is confirmed or terminated P4 Obtain feedback from participants and relevant managers on extent induction process is meeting its objectives P5 Make refinements to induction policies and procedures |

Knowledge & Understanding

K1: describe recruitment and selection methods, including assessment centers

K2: explain the concept of outsourcing

K3: describe the purpose of employee contracts and industrial relations

K4: summarize relevant legislation, regulations, standards and codes of practice that may affect recruitment, selection and induction

K5: explain why terms and conditions of employment are an important aspect of recruitment

K6: explain the relevance of psychometric and skills testing programs to recruitment.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage recruitment selection and induction processes. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- develop or critically analyze a policy and procedures framework for recruitment, selection and induction

- identify the need for recruitment

- prepare and oversee appropriate documentation required for recruitment

- select and advise job applicants appropriately

- manage the induction process

- Comply with relevant legislation and organizational requirements.

### Manage personal work priorities and professional development

Overview: This unit describes the skills and knowledge required to create systems and process to organize information and prioritize tasks. It applies to individuals working in managerial positions who have excellent organizational skills. The work ethic of individuals in this role has a significant impact on the work culture and patterns of behavior of others as managers at this level are role models in their work environment.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Establish personal work goals | P1 Serve as a positive role model in the workplace through personal work planning P2 Ensure personal work goals, plans and activities reflect the organization s plans, and own responsibilities and accountabilities P3 Measure and maintain personal performance in varying work conditions, work contexts and when contingencies occur |
| 2. Set and meet own work priorities | P1 Take initiative to prioritize and facilitate competing demands to achieve personal, team and organizational goals and objectives P2 Use technology efficiently and effectively to manage work priorities and commitments P3 Maintain appropriate work-life balance, and ensure stress is effectively managed and health is attended to |
| 3. Develop and maintain professional competence | P1 Assess personal knowledge and skills against competency standards to determine development needs, priorities and plans P2 Seek feedback from employees, clients and colleagues and use this feedback to identify and develop ways to improve competence P3 Identify, evaluate, select and use development opportunities suitable to personal learning style/s to develop competence P4 Participate in networks to enhance personal knowledge, skills and work relationships P5 Identify and develop new skills to achieve and maintain a competitive edge |

Knowledge & Understanding

K1: explain principles and techniques involved in the management and organization of:

- performance measurement

- personal behavior, self-awareness and personality traits identification

- a personal development plan

- personal goal setting

- time

K2: discuss management development opportunities and options for self

K3: describe methods for achieving a healthy work-life balance

K4: outline organization s policies, plans and procedures

K5: explain types of learning style/s and how they relate to the individual

K6: describe types of work methods and practices that can improve personal performance.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage personal work priorities and professional development. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- use business technology to create and use systems and processes to organise and prioritise tasks and commitments

- measure and maintain personal work performance including assessing competency against competency standards and seeking feedback

- maintain an appropriate work-life balance to manage personal health and stress

- participate in networks

- develop a personal development plan which includes career objectives and an action plan

- Develop new skills.

### Manage workforce planning

Overview: This unit describes the skills and knowledge required to manage planning in relation to an organization’s workforce including researching requirements, developing objectives and strategies, implementing initiatives and monitoring and evaluating trends. It applies to individuals who are human resource managers or staff members with a role in a policy or planning unit that focuses on workforce planning.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Research workforce requirements | P1 Review current data on staff turnover and demographics P2 Assess factors that may affect workforce supply P3 Establish the organization’s requirements for a skilled and diverse workforce |
| 2. Develop workforce objectives and strategies | P1 Review organizational strategy and establish aligned objectives for modification or retention of the workforce P2 Consider strategies to address unacceptable staff turnover, if required P3 Define objectives to retain required skilled labor P4 Define objectives for workforce diversity and cross-cultural management P5 Define strategies to source skilled labor P6 Communicate objectives and rationale to relevant stakeholders P7 Obtain agreement and endorsement for objectives and establish targets P8 Develop contingency plans to cope with extreme situations |
| 3. Implement initiatives to support workforce planning objectives | P1 Implement action to support agreed objectives for recruitment, training, redeployment and redundancy P2 Develop and implement strategies to assist workforce to deal with organizational change P3 Develop and implement strategies to assist in meeting the organization’s workforce diversity goals P4 Implement succession planning system to ensure desirable workers are developed and retained P5 Implement programs to ensure workplace is an employer of choice |
| 4. Monitor and evaluate workforce trends | P1 Review workforce plan against patterns in exiting employee and workforce changes P2 Monitor labor supply trends for areas of over- or under-supply in the external environment P3 Monitor effects of labor trends on demand for labor P4 Survey organizational climate to gauge worker satisfaction P5 Refine objectives and strategies in response to internal and external changes and make recommendations in response to global trends and incidents P6 Regularly review government policy on labor demand and supply P7 Evaluate effectiveness of change processes against agreed objectives |

Knowledge & Understanding

K1: explain current information about external labor supply relevant to the specific industry or skill requirements of the organization

K2: outline industrial relations relevant to the specific industry

K3: describe labor force analysis and forecasting techniques

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage workforce planning. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- review and interpret information from a range of internal and external sources to identify:

  - current staff turnover and demographics

  - labor supply trends factors that may affect workforce supply

  - organization’s workforce requirements objectives and strategies

- manage workforce planning including developing, implementing, monitoring and reviewing strategies to meet workforce needs

- review relevant trends and supply and demand factors that will impact on an organization’s workforce

- Develop a workforce plan that includes relevant research and specific strategies to ensure access to a skilled and diverse workforce.

### Undertake project work

Overview: This unit describes the skills and knowledge required to undertake a straightforward project or a section of a larger project. It covers developing a project plan, administering and monitoring the project, finalizing the project and reviewing the project to identify lessons learned for application to future projects. This unit applies to individuals who play a significant role in ensuring a project meets timelines, quality standards, budgetary limits and other requirements set for the project.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Define project | P1 Access project scope and other relevant documentation P2 Define project stakeholders P3 Seek clarification from delegating authority of issues related to project and project parameters P4 Identify limits of own responsibility and reporting requirements P5 Clarify relationship of project to other projects and to the organization’s objectives P6 Determine and access available resources to undertake project |
| 2. Develop project plan | P1 Develop project plan in line with the project parameters P2 Identify and access appropriate project management tools P3 Formulate risk management plan for project, including Work Health and Safety (WHS) P4 Develop and approve project budget P5 Consult team members and take their views into account in planning the project P6 Finalize project plan and gain necessary approvals to commence project according to documented plan |
| 3. Administer and monitor project | P1 Take action to ensure project team members are clear about their responsibilities and the project requirements P2 Provide support for project team members, especially with regard to specific needs, to ensure that the quality of the expected outcomes of the project and documented time lines are met P3 Establish and maintain required recordkeeping systems throughout the project P4 Implement and monitor plans for managing project finances, resources and quality P5 Complete and forward project reports as required to stakeholders P6 Undertake risk management as required to ensure project outcomes are met P7 Achieve project deliverables |
| 4. Finalize project | P1 Complete financial recordkeeping associated with project and check for accuracy P2 Ensure transition of staff involved in project to new roles or reassignment to previous roles P3 Complete project documentation and obtain necessary sign-offs for concluding project |

Knowledge & Understanding

K1: give examples of project management tools and how they contribute to a project

K2: outline types of documents and other sources of information commonly used in defining the parameters of a project

K3: explain processes for identifying and managing risk in a project

K4: outline the organization’s mission, goals, objectives and operations and how the project relates to them

K5: explain the organization’s procedures and processes that are relevant to managing a project including:

- lines of authority and approvals

- quality assurance

- human resources

- budgets and finance

- recordkeeping

- reporting

Outline the legislative and regulatory context of the organization in relation to project work, including work health and safety (WHS) requirements.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to undertake project work. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- define the parameters of the project including:

  - project scope

  - project stakeholders, including own responsibilities

  - relationship of project to organizational objectives and other projects

  - reporting requirements

  - resource requirements

- use project management tools to develop and implement a project plan including:

  - deliverables

  - work breakdown

  - budget and allocation of resources

  - timelines

  - risk management

  - recordkeeping and reporting

- consult and communicate with relevant stakeholders to generate input and engagement in planning, implementing and reviewing the project

- provide support to team members to enable them to achieve deliverables and to transition them as appropriate at completion of the project

- finalize the project including documentation, sign-offs and reporting

Review and document the project outcomes.

### Identify and communicate trends in career development

Overview: This unit describes the skills and knowledge required to conduct research to identify and communicate career trends. It establishes the need to interact professionally with others in assessing career needs, to effectively assist clients identify competencies they require for a career and employability in a given context. It also examines how to maintain quality of career development services and professional practice. It applies to individuals seeking to identify and communicate trends in career development.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Research and confirm career trends | P1 Apply knowledge of changing organizational structures, lifespan of careers and methods of conducting work search, recruitment and selection processes P2 Analyze changing worker and employer issues, rights and responsibilities in context of changing work practices P3 Examine importance of quality careers development services P4 Maintain all research, documentation, sources and references (electronic or physical) to a high degree of currency and relevance P5 Analyze implications of relevant policy, legislation, professional codes of practice and national standards relating to worker and employer issues P6 Research changes and trends in theory of career development counseling and practice P7 Confirm clusters, levels and combinations of transferable employability skills and preferences that may open employment options spanning more than one occupation or career pathway |
| 2. Assess and confirm ongoing career development needs of target group | P1 Analyze history and records in assessing needs of target group P2 Assess success of previous career development services and techniques used for individual or target group P3 Deploy other means to investigate appropriate care and counseling approaches as required P4 Maintain privacy and security of all data, research and personal records according to relevant policy, legislation, professional codes of practice and national standards P5 Establish existing work-life balance requirements, issues and needs |
| 3. Maintain quality of career development services and professional practice | P1 Analyze and review relevance of career theories, models, frameworks and research for target group P2 Incorporate into career development services and professional practice, major changes and trends influencing workplace and career-related options and choices P3 Comply with all relevant policy, legislation, professional codes of practice and national standards that influence delivery of career development services |

Knowledge & Understanding

K1: explain client care and counseling techniques and processes in the context of career development services

K2: describe diversity and its potential effects on career choices

K3: outline human psychological development and needs in relation to careers development

K4: outline relevant policy, legislation, codes of practice and standards relevant to career development

K5: explain recruitment and selection processes in the context of career development services

K6: describe a range of data gathering and research techniques

K7: explain techniques used to analyze trends.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to identify and communicate trends in career development. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- research and analyze current economic, labor market, employment, career and vocational, educational and training trends

- identify choices and career development needs for individuals and target groups within a given context

- report and document management of research and career development materials

- Comply with all relevant local, state/territory and national legislation, policies and practices.

### Apply specialist interpersonal and counseling interview skills

Overview: This unit describes the skills and knowledge required to use advanced and specialized communication skills in the client-counselor relationship. This unit applies to individuals whose job role involves working with clients on personal and psychological issues within established policies, procedures and guidelines.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Communicate effectively | P1 Identify communication barriers and use strategies to overcome these barriers in the client-counselor relationship P2 Facilitate the client-counselor relationship through selection and use of micro skills P3 Integrate the principles of effective communication into work practices P4 Observe and respond to non-verbal communication cues P5 Consider and respond to the impacts of different communication techniques on the client-counselor relationship in the context of individual clients P6 Integrate case note taking with minimum distraction |
| 2. Use specialized counseling interviewing skills | P1 Select and use communication skills according to the sequence of a counseling interview P2 Identify points at which specialized counseling interviewing skills are appropriate for inclusion P3 Use specialized counseling communication techniques based on their impacts and potential to enhance client development and growth P4 Identify and respond appropriately to strong client emotional reactions |
| 3. Evaluate own communication | P1 Reflect on and evaluate own communication with clients P2 Recognize the effect of own values and beliefs on communication with clients P3 Identify and respond to the need for development of own skills and knowledge |

Knowledge & Understanding

K1: legal and ethical considerations for communication in counseling practice, and how these are applied in individual practice:

- codes of conduct/practice

- discrimination

- duty of care

- human rights

- practitioner/client boundaries

- privacy, confidentiality and disclosure

- rights and responsibilities of workers, employers and clients

- work role boundaries responsibilities and limitations of the counselor role

- work health and safety

K2: principles of person-centered practice

K3: key objectives of counseling interviewing

K4: stages of a counseling interview

K5: potential impacts of using different communication skills and techniques in counseling contexts

K6: communication techniques and micro-skills including:

- attending behaviors active listening, reflection of content feeling, summarizing

- questioning skills open, closed, simple and compound questions

- client observation skills

- noting and reflecting skills

- providing client feedback

K7: specialized counseling communication techniques, and how they are used, including:

- challenging

- reframing

- focusing

K8: components of the communication process including:

- encoder

- decoder

K9: primary factors that impact on the communication process including:

- context

- participants

- rules

- messages

- channels

- noise

- feedback

K10: communication barriers and resolution strategies, including:

- environmental

- physical

- individual perceptions

- cultural issues

- language

- age issues

- disability

K11: observational techniques including:

- facial expressions

- non-verbal behavior

- posture

- silence

K12: ways in which different people absorb information, including:

- visual

- auditory

- kinesthetic

K13: obstacles to the counseling process

K14: impacts of trauma and stress on the communication process, including on:

- concentration and attention

- memory

- use of verbal and written language

- use of body language

- challenging within the counseling session

K15: self-evaluation practices, including:

- how to recognize own biases

- Impact of own values on the counseling relationship.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to apply specialist interpersonal and counseling interview skills. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- interviewed at least 3 different clients using specialized interpersonal communication and counseling interviewing skills, including:

- micro-skills and communication techniques, including:

  - attending behaviors active listening,

  - reflection of content, summarizing

  - questioning skills open, closed, simple and compound questions

  - client observation skills

  - noting and reflecting skills

  - providing client feedback

- specialized counseling interviewing skills, including:

  - challenging

  - reframing

  - focusing

- integrated clear case note taking into the interview process

- Completed a structured process of self-reflection and evaluation of own communication used during the 3 interviews.

### Work safely in an office environment

Overview: This unit describes the performance outcomes, skills and knowledge required to participate in workplace occupational health and safety (OHS) processes to protect workers own health and safety, and that of others.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Work safely | P1 Follow established safety procedures when conducting work P2 Carry out pre-start systems and equipment checks in accordance with workplace procedures |
| 2. Implement workplace safety requirements | P1 Identify designated persons for reporting queries and concerns about safety in the workplace P2 Identify existing and potential hazards in the workplace, report them to designated persons and record them in accordance with workplace procedures P3 Identify and implement workplace procedures and work instructions for controlling risks P4 Report emergency incidents and injuries to designated persons |
| 3. Participate in OHS consultative processes | P1 Contribute to workplace meetings, inspections or other consultative activities P2 Raise OHS issues with designated persons in accordance with organizational procedures P3 Take actions to eliminate workplace hazards or to reduce risks |
| 4. Follow safety procedures | P1 Identify and report emergency incidents P2 Follow organizational procedures for responding to emergency incidents |

Knowledge & Understanding

K1: Explain responsibilities of employers and employees under relevant health and safety regulation

K2: describe emergency procedures including procedures for fires, accidents and evacuation

K3: outline commonly used hazard signs and safety symbols

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to participate in workplace OHS processes. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. The unit assessment must ensure the safety processes; hazards and risk are relevant to the area of work. Evidence of the following is essential:

- Accurately following all relevant safety procedures

- Identifying and reporting hazards to designated personnel

- Knowledge of relevant health and safety regulations

- Knowledge of relevant materials, equipment and work processes.

### Develop workplace documents

Overview: This unit covers interpreting and composing a range of workplace documents from a number of sources. It includes interpreting written information for workplace purposes as well as planning, drafting and reviewing a basic document before writing the final version. The focus is on the content and structure of written materials and not on the use of computer technology

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Interpret written information | P1 Read workplace materials to identify the subject and key information for using or reporting to others. P2 Read procedural manuals and codes of practice to locate specific information to carry out work functions in accordance with policy and standards. P3 Read a range of written materials to locate and select required information for summaries, short reports and responses to requests. P4 Identify the cultural context and prior knowledge required to interpret workplace information and obtain assistance when required. P5. Determine audience and purpose for the document P6 Seek assistance with interpretation of complex materials in accordance with organizational procedures. |
| 2. Develop written materials | P1 Identify and comply with established requirements for a range of written materials in accordance with organizational procedures and standard templates. P2. Determine format and structure P3. Establish key points for inclusion P4. Identify organizational requirements P5. Establish method of communication P6. Establish means of communication |
| 3. Draft document | P1 Develop draft document to communicate key points P2. Obtain and include any required additional information P3 Prepare written information in an accurate, concise and unambiguous manner that meets intended audience and organizational requirements. |
| 4. Review document | P1 Check draft for suitability of tone for audience, purpose, format and communication style P2. Check draft for readability, grammar, spelling, sentence and paragraph construction and correct any inaccuracies or gaps in content. P3. Check draft for sequencing and structure P4. Check draft to ensure it meets organizational requirements P5. Ensure draft is proofread, where appropriate, by supervisor or colleague |
| 5. Write final document | P1 Make and proofread necessary changes P2. Ensure document is sent to intended recipient within required time frames P3. File copy of document in accordance with organizational policies and procedures |

Knowledge & Understanding

K1: Explain the reading and writing procedures at a level to cope with a range of workplace materials

K2: Explain the integration of information from a number of sources in order to generate meaning

K3: Describe the ways to write and sequence paragraphs according to the required purpose of written material

K4: Outline the linking ideas in written material through selection and use of words, grammatical structures, headings and punctuation appropriate to the purpose

K5: Elaborate spelling, punctuation and grammar for workplace documents at an experienced level

K6: Explain the response to diversity, including gender and disability

K7: Explain the implementation of ergonomic requirements for office work

K8: Explain the environmental policies such as those relating to paper use/wastage/recycling

K9: Describe the preparation of general information and papers according to target audience

K10: Elaborate the ways of proofreading and editing documents to ensure clarity of meaning and conformity to organizational requirements

K11: Describe the problem-solving skills to determine document design and production processes

K12: Explain the usage of resources to assist in document production, such as dictionary, thesaurus, templates, style sheets

K13: Describe the ways to produce business letters, memos, job applications, resumes, meeting agendas and minutes

K14: Explain the ways to fold and insert letters into a standard and window faced envelope.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to interpret written information for workplace purposes and plan, draft and review a basic document before writing the final version. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- Producing a range of documents that accurately convey required information including single and multipage business letters, memos, job applications, resumes, meeting agendas and minutes.

- Using formatting suitable for intended audience

- Knowledge of organizational policies and procedures for document production

### Prepare and implement negotiation

Overview: This unit covers the skills, knowledge and attitudes required to prepare for and participate in a process of negotiation.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Prepare for the negotiation | P1 Identify objectives and preferred outcome of the negotiation and determine minimum acceptable outcome P2 Understand in relation to what can be offered and what is needed from the other party P3 Gather information regarding the other party objectives, needs, preferences, resources, what they want to achieve - in order to determine best negotiating points P4 List and rank the issues to consider concessions ?? that may be made. P5 Find examples and refine negotiation argument. P6 Check information to ensure it is correct and up-to-date. P7 Develop a negotiation plan that includes information about the other party and its interests and a set of responses and strategies to the anticipated tactics. P8 Prepare an agenda in advance, which includes discussion topics, participants, location and schedule |
| 2. Participate in negotiations | P1 Analyze all aspects of the incident for degree of hazard, priorities, optional outcomes and appropriate strategies P2 Analyze and determine strategies and priorities on the incident sought from a range of sources P3 Assess long term objectives against resources and priorities P4 Apply a range of communication techniques to make and maintain contact with the key people P5 Provide clear and factual information to enable an honest and realistic assessment of the interests of the key people and their positions P6 Resolve the conflict and express their likely consequences clearly and do an analysis of the benefits P7 Re assess points of disagreements for common positive positions |
| 3. Coordinate support services | P1 Assess the need for support services in terms of the determined strategies and priorities P2 Negotiate the resources of support services according to established procedures and availability P3 Provide information on strategies to support services and maintain the communication P4 Delegate roles and responsibilities according to expertise and resources |
| 4. Restore order | P1 Assess the incidents for degree of risk and take appropriate action to reduce and remove the impact of the incident and restore order P2 Take action designed to minimize risk and the preserve the safety and security of all involved P3 Take action to prevent the escalation of the incident appropriate to the circumstances and agreed procedures. P4 Carry out the use of force for the restoration of control and the maintenance of security in the least restrictive manner. P5 Complete reports accurately and clearly provided to the appropriate authority promptly P6 Review, evaluate and analyze the incident and the organizational response to it and report it promptly and accurately. |
| 5. Provide leadership. direction and guidance to the work group | P1 Link between the function of the group and the goals of the organization P2 Participate in decision making routinely to develop, implement and review work of the group and to allocate responsibilities where appropriate P3 Give opportunities and encouragement to others to develop new and innovative work practices and strategies P4 Identify conflict and resolve with minimum disruption to work group function P5 Provide staff with the support and supervision necessary to perform work safely and without risk to health P6 Allocate tasks within the competence of staff and support with appropriate authority, autonomy and training P7 Supervise appropriately the changing priorities and situations and takes into account the different needs of individuals and the requirements of the task |

Knowledge & Understanding

K1: Explain organization’s policies, guidelines and procedures related to control and surveillance, safety and preventing and responding to incidents and breaches of orders covered in the range of variables.

K2: Explain organization’s management and accountability systems

K3: Describe teamwork principles and strategies

K4: Outline the principles of effective communication

K5: Outline the guidelines for use of equipment and technology

K6: Explain code of conduct

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to solve problems which jeopardize safety and security. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- evidence of effective communication strategies including negotiation, counseling, mediation, advocacy demonstrated under pressure working effectively in a team environment

- evidence of knowledge and application of organizations policies, procedures and

- guidelines for critical incidents

- evidence of accurate and safe use of all emergency equipment

- evidence of managing effective outcomes using strategic planning, team leadership and situational analysis

### Maintain professionalism in the workplace

Overview: This unit of competency describes the outcomes required maintain a professional image in the workplace, including behaving ethically, demonstrating motivation, respecting timeframes and maintaining personal appearance.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Respect work timeframes | P1 Demonstrate punctuality in meeting, set working hours and times. P2 Utilize working hours only for ?? working and follow company regulations. P3 Complete work tasks within deadlines according to order of priority P4 Supervisors are informed of any potential delays in work times or projects. |
| 2. Maintain personal appearance and hygiene | P1 Clean hair, body and nails regularly. P2 Wear suitable cloths for the workplace, and respect local and cultural contexts P3 Meet specific company dress code requirements |
| 3. Maintain adequate distance with colleagues and clients | P1 Respect personal space of colleagues and clients with reference to local customs and cultural contexts. P2 Keep sufficient distance from others P3 Avoid cross transmission of infections (especially through respiration). |
| 4. Work in an ethical manner | P1 Follow company values/ethics codes of ethics and/or conduct, policies and guidelines. P2 Use company resources in accordance with company ethical standards. P3 Conduct personal behavior and relationships in accord with ethical standards and company policies. P4 Undertake work practices in compliance with company ethical standards, organizational policy and guidelines. P5 Instruct co-workers on ethical, lawful and reasonable directives. P6 Share company values/practices with co-workers using appropriate behavior and language. P7 Report work incidents/situations and/or resolved in accordance with company protocol/guidelines. |

Knowledge & Understanding

K1: Explain application of good manners and right conduct

K2: Explain basic practices for oral and personal hygiene

K3: Describe common products used for oral and personal hygiene

K4: Outline the company code of conduct/values

K5: Outline the Company regulations, performance and ethical standards

K6: Explain work responsibilities/job functions

K7: Describe communication skills

K8: State workplace hygiene standards

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to maintain professionalism in the workplace .The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- clarify and affirm work values/ethics/concepts consistently in the workplace;

- comply with required working times;

- conduct work practices satisfactorily and consistently, in compliance with work ethical standards, organizational policy and guidelines;

- Develop suitable hygiene

Keep adequate distance while interacting with colleagues and clients.

### Organize schedules

Overview: This unit describes the skills and knowledge required to manage appointments and diaries for personnel within an organization, using manual and electronic diaries, schedules and other appointment systems. It applies to individuals employed in a range of work environments who provide administrative support to teams and individuals.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Establish schedule requirements | P1 Identify organizational requirements and protocols for diaries and staff planning tools P2 Identify organizational procedures for different types of appointments P3 Determine personal requirements for diary and schedule items for individual personnel P4 Establish appointment priorities and clarify in discussion with individual personnel |
| 2. Manage schedules | P1 Identify recurring appointments and deadlines, and schedule these in accordance with individual and organizational requirements P2 Establish availability of attendees, and schedule new appointments in accordance with required timelines and diary commitments P3 Negotiate alternative arrangements and confirm when established appointments are changed P4 Record appointments and manage schedules in accordance with organizational policy and procedures |

Knowledge & Understanding

K1: identify the key provisions of relevant legislation, standards and codes that affect aspects of business operations or the achievement of team goals

K2: describe organizational requirements for managing appointments for personnel within the organization

K3: summarize the range of appointment systems that could be used

K4: outline important considerations when managing the schedules of others.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to organize schedules. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Appropriately manage the schedules of various individuals through a process of careful planning and negotiation.

## Computer Programming

### Installation of C++, IDE for Arduino and Python

Overview: This competency standard covers the skills and knowledge required to develop how to write a computer program. It will create the knowledge to write lists of instructions for a computer to follow and to develop the software programs, the scripts, or other sets of instructions for the computers to execute.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Install Turbo C++ on Windows | P1. Download Turbo C++ P2. Create turboc directory inside C drive and extract the tc3. zip inside c:\turboc P3. Double click on install.exe file P4. Click on the tc application file located inside c:\TC\BIN to write the C program. |
| CU2. Install Arduino IDE on Windows | P1. Download the Arduino Software (IDE) P2. RUN setup file and choose components to install. P3. Choose the installation directory and complete installation. P4. Proceed with board specific instructions (Uno, Mega, etc.) P5. Open software to view IDE. |
| CU3. Install Python IDE on Windows | P1. Download the Python 3 Installer P2. Run the installer P3. Download the python IDE P4. Identify Interpreter vs Script Mode P5. Enabling VNC Server P6. Enabling VNC Server graphically P7. Enabling VNC Server at the command line P8. Connecting to your Raspberry Pi with VNC Viewer P9. Establishing a direct connection P10. Establishing a cloud connection P11. Authenticating to VNC Server P12. Creating a virtual desktop |

Knowledge and understanding

- Describe the System Software

- Describe C++

- Describe Arduino microcontroller

- Describe Raspberry Pi microcontroller

- Define Interpreter and Script Mode

- Describe the Programming Software

- Describe the Application Software

- Describe the Program languages

- Describe VNC Server

- Describe the syntax and semantics.

- Differentiate Programs and Algorithms.

Equipment and Tools

        - Computer/Laptop System

        - Turbo C++ Software

        - Arduino Software

        - Python Software

        - Arduino Development Board

        - Raspberry Pi Development Board

        - SD Card

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

  - Installation Software

  - Running of Software

### Carryout Basic C++ Programming

Overview: This competency standard covers the skills and knowledge required to develop how to write a computer program. It will create the knowledge to write lists of instructions for a computer to follow and to develop the software programs, the scripts, or other sets of instructions for the computers to execute. This unit will cover the knowledge of programming language C which has variety of data types and powerful operators. Due to this, students will be able to write computer programs efficiently and easily.:

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Develop a computer program for mathematic functions | P1. Open turbo C++ software P2. Create new file P3. Write a program for addition / subtraction / multiplication / Division. P4. Save and run the program P5. Identify the error in compiler P6. Remove the error if required |
| CU2. Develop a computer program for trigonometric functions | P1. Open turbo C++ software P2. Create new file P3. Write a program for sin / cosine / tan P4. Save and run the program P5. Identify the error in compiler P6. Remove the error if required |
| CU3. Develop a computer simple program for given problem. | P1. Analyze the given problem P2. Open the IDE for coding P3. Code a simple program P4. Compile a code P5. Debug the code (in case of error) P6. Run a code |
| CU4. Develop a program based on control structures | P1. Identify the keywords and identifiers P2. Perform comments, indentation and statements P3. Recognize variables and data types P4. Check standard Input and Output P5. Use Operators P6. Use of IF statement P7. Use of IF ELSE statement P8. Use of SWITCH statement |
| CU5. Develop program using loop structuers | P1. Use of FOR loop P2. Use of WHILE loop P3. Use of DO-WHILE loop P4. Use of nested loop P5. Use Break and Continue |
| CU6. Develop a program using Arrays | P1. Use of Arrays P2. Use of one dimension array declaration. P3. Use of Array declaration. P4. Use of Array initialization. P5. Use of Array elements accessing. P6. Use of Two dimension array declaration. |

Knowledge and understanding

- Define Compiler

- Define C++ language

- Describe basic purpose of C++ language.

- Describe Data Type

- Describe operational procedure of C++.

- Describe of complex number and rectangular and polar form.

- Define a given problem

- Describe the basic programming concept

- Define IDE

- Define basics of C language

- Define Constants and Variables.

- Define basic input and output statements.

- Define debugging of a code

- Define IF statement

- Define IF ELSE statement

- Describe the use of SWITCH statement

- Define loop structure

- Define FOR loop

- Define WHILE loop

- Define DO-WHILE loop

- Define nested loop

- Define language translator

Equipment and Tools

- Computer/Laptop System

- Turbo C++ Software

- Arduino Software

- Python Software

- Arduino Development Board

- Raspberry Pi Development Board

- SD Card

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Develop program using loops

- Develop program using conditions

### Carryout Basic Programming using python

Overview: This competency standard covers the skills and knowledge required to develop how to write a computer program. It will create the knowledge to write lists of instructions for a computer to follow and to develop the software programs, the scripts, or other sets of instructions for the computers to execute. This unit will cover the knowledge of programming language C which has variety of data types and powerful operators. Due to this, students will be able to write computer programs efficiently and easily.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Introduction to LINUX commands | P1. Opening Linux Terminal P2. Perform Basics of Linux Terminal P3. Use Commands to display directory P4. Create file on the directory P5. Use Commands to run the file on LINUX |
| CU2. Develop a program based on control structures | P1. Identify Basic Data Types P2. Use Assignment Operators P3. Use print() function P4. Use Comments in Python P5. Use input() functions P6. Identify the Code Blocks and Indentation P7. Identify Reserved words in Python P8. Use Assignment Operators and Expressions P9. Use Arithmetic Operations P10. Identify Precedence and Associativity P11. Identify Docstring |
| CU3. Develop program Using Conditional Structures and Operators | P1. Use Boolean Expressions P2. Use If Statement P3. Use If-Else P4. Use Nested-If P5. Use If-Elif-Else P6. Identify Membership Operators P7. Identify Logical Operators P8. Use Bitwise Operators P0714E&A Identify Simple Recursion P10. Use Factorial with Recursion P11. Program to display Fibonacci sequence using recursion P12. Use Indirect Recursion |
| CU4. Develop program using loop | P1. Use While Loop P2. Use While-break P3. Use While-Continue P4. Use for-list P5. Use for-range P6. Use for-continue-break P7. Program to display prime numbers in an interval P8. Calculate factorial P0714E&A Compute Fibonacci Series |
| CU5. Develop a program using Functions | P1. Use of simple FUNCTIONS. P2. Use of Function Declaration. P3. Use of Function Prototype. P4. Use of Function Arguments P5. Use of Function definition. P6. Use of Function Call. P7. Use of passing parameters to Function. P8. Use of Function Arbitrary Arguments |
| CU6. Python file handling | P1. Create a Text File P2. Append Data to a File P3. Read a File P4. Read a File line by line P5. Identify File Modes in Python P6. Handling of CSV file P7. Handling of Excel File |
| CU7. Develop a program using Python Data Structrre | P1. Declare Strings P2. Create Lists in python P3. Apply Lists methods P4. Constructing Tuples in python P5. Apply Tuples methods P6. Create Sets P7. Use Operators on Sets P8. Create Dictionary P0714E&A Methods to query information from dictionaries P10. Create Strings P11. Formatters and Operator on String P12. Methods on String |

Knowledge and understanding

- Understand Installation of python

- Understand Raspbian environment

- Define VNC connection

- Understand the LINUX commands

- Define File handling of python

- Define Tuples and its methods

- Define Operators and sets

- Define Dictionary and mrthods

- Define Strings and its methods

- Define Relational operators

- Define Logical operators.

- Define conditional operator.

- Define Arrays.

- Explain one dimension array declaration.

- Explain the initialization of array.

- Explain the accessing of array elements.

- Define the declaration of two dimension array.

- Define Functions.

- Explain types of functions

- Explain function prototype

- Explain function definition.

- Explain the use of function call.

- Explain the passing parameters to Function.

- Define the pointer

Equipment and Tools

- Computer/Laptop System

- Turbo C++ Software

- Arduino Software

- Python Software

- Arduino Development Board

- Raspberry Pi Development Board

- SD Card

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Develop program using Structures

### Perform basic programming using microcontrollers

Overview: This competency standard covers the skills and knowledge required to develop how to write a computer program. It will create the knowledge to write lists of instructions for a computer to follow and to develop the software programs, the scripts, or other sets of instructions for the computers to execute. This unit will cover the knowledge of programming language C which has variety of data types and powerful operators. Due to this, students will be able to write computer programs efficiently and easily.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Introduction to Arduino IDE | P1. Perform the Arduino IDE Installations. P2. Perform Arduino Sketch / Programming P2. Use Arduino supporting libraries and functions P3. Write down the sketch for Arduino in IDE. P4. Identify the Shields, Accessories, and Sensors for Arduino |
| CU2. Program Arduino Microcontroller for mathematical function | P1. Open Arduino IDE software. P2. Connect Display with Arduino P3. Make a source code/sketch for addition / subtraction / multiplication / Division. P4. Compile and Debug P5. Run Program. P6. Display results |
| CU3. Program Arduino Microcontroller for Invers Trigonometric function | P1. Open Arduino IDE software. P2. Connect LCD Display with Arduino P3. Make a source code/sketch for inverse sine / cosine / tan P4. Compile and Debug P5. Run Program. P6. Display the results |
| CU2. Program Arduino Microcontroller by using conditional and loop instructions. | P1. Use Boolean Expressions P2. Use If, If-Else, Nested if Statements P3. Identify Logical Operators P4. Identify Simple Recursion P5. Use While Loop P6. Use for loop P7 Display values from 1 to 100 as per given instructions. |

Knowledge and understanding

- Define librarires

- Define Arduino shields

- Describe LCD Display

- Describe interfacing LCD Display

- Define conditional operator.

- Define Arrays.

- Explain one dimension array declaration.

- Explain the initialization of array.

- Explain the accessing of array elements.

- Define the declaration of two dimension array.

- Define Functions.

- Explain types of functions

- Explain function definition.

- Explain the use of function call.

- Explain the passing parameters to Function.

Equipment and Tools

- Computer/Laptop System

- Turbo C++ Software

- Arduino Software

- Python Software

- Arduino Development Board

- Raspberry Pi Development Board

- SD Card

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Uploading program to microcontroller

- Displaying Results on LCD Display

## Computer Integrated Manufacturing (CIM)

### Perform CNC Machine Operations

Overview: This competency standard covers the skills and knowledge required to Set CNC machine according to job requirements, Perform Milling Operations Using CNC Machine and Perform turning operations using CNC machine

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Comply with occupational health and safety and environmentally sustainable work practices | - Follow the safty rules and work place instruction according to the provided manuals. - Report and indentify the workplace hazards and accidents - Follow and indentify the emergency procedures - Identify the current resources in connection with environmental issues. - Comply with the environmental regulations. - Seek the opportunities to improve the environmental risks and resources in order to increase the efficiency. |
| - Set CNC machine according to job requirements | - Mount and set the cutting tool and work-piece according to procedures. - Set up and adjust the machine’s parameters according to the work specification. - Report the concerned person or incharge reguarly about the uncertainties / deviations /errors which were face / observe during the operation. . - Follow / Observe the safety and workplace instructions in order to avoid injuries and accidents. |
| - . Perform Milling Operations Using CNC Machine | - Match the work piece data with CAD data through software simulation. - Execute the program on a CNC milling to perform milling operations (e.g. surfacing, drilling, slotting, tapping, key ways, step cutting etc.) to achieve the work specifications. - Follow the correct specifications for the component to be produced. - Report the concerned person or incharge reguarly about the uncertainties / deviations /errors which were face / observe during the operation - Follow / Observe safety and workplace instructions to avoid injuries and accidents. |
| - Perform turning operations using CNC machine | - Match the work piece data with a CAD data through a software simulation. - Execute the program on a CNC Lathe to perform turning operations to the achieve work specifications. - Follow the correct specifications for the component to be produced. - Report the concerned person or incharge reguarly about the uncertainties / deviations /errors which were face / observe during the operation - Follow / Observe safety and workplace instructions to avoid injuries and accidents. |

Knowledge & Understanding

- Basic knowledge of CNC machine.

- Machine’s process standards and functions.

- Methods and techniques   for adjustment of the  operating parameters of machine.

- Interpreting the drawnings and work specifications.

- Techniques for checking quality of components produced.

- Basic knowledge of G-Code and M-Code.

- Basic computer operations.

- Procedure for reporting the uncertainties and deviations to the person concerned for timely action.

- X, Y, and Z axis.

- Safety precautions and guidelines.

- Use of control panel.

- Quality check points with standards.

- Basic knowledge of machine margins and alignments.

- Repeated as aboveRepeated as aboveUse of the coordinate system

- Functions of a CNC Lathe Machine and range of the turning operations which include facing, grooving, tapering, taper turning, step turning, form turning, threading, knurling, drilling, boring, reaming.

Critical Evidence

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Set CNC machine according to job requirements

- Match work piece data with CAD data through software simulation

- Execute CNC Milling program to perofrm milling operation

Tools & Equipment

- CNC machine

- CNC Manual

- CNC milling machine with all accessories

- Cutting Tools

- Tool Kit

- Gauges

- Measuring Instruments

- CNC Lathe machine with all accessories

- Cutting Tools

- Tool Kit

- Gauges

- Measuring Instruments

### Perform CNC Lathe Operations

Overview: This competency standard covers the skills and knowledge required to mount the Job, Generate the Program, Run Simulation, Feed the Program, Perform CNC Lathe Operations

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Mount the Job | - Mount the work-piece according to the working capacity of the machine along with the drawing / design of the job. job to the drawing / design - Select the appropriate work holding device(s) in order to achieve the dimensional accuracy Enter - Separate this Clamp the job firmly in the work holding device according to the standard procedure. - Attain the proper alignment of the tool / cutter and work-piece e.g. concentricity of rotating jobs as per set / standared practice - Follow / Keep the safety measures while mounting the work-piece so that the unwanted operation by machine may not be initiated |
| CU2. Generate the Program | - Interpret the job requirements, i.e. calculation of the offsets, errors and new reference point during the removal of materials, Define the absolute or incremental coordinates system, tool path strategies, machining features and tool compensation for generating the tool path as per standard procedure - Use the appropriate part programming credentials (Sequence, G-codes, M-codes, coordinates, feed, speed, tooling information etc.) according to the control parameters of the CNC machine - Keep the record of generated part program in soft / hard form in order to feed into a machine control unit as per standard procedures |
| CU3. Run Simulation | - Feed the program of the drawing for the generated part into an appropriate simulation software’s platform and run the simulation for checking the tool gouge according to safety measures - Run the simulation, observe the movements of tool / cutter and verify the measurement with the defined the sequences - Identify the occurrence of errors and modify the program as per defined procedures and sequences |
| CU4. Feed the Program | - Ensure the proper synchronization between the machine’s control unit with the program of the part’s drawing as per standard operating procedures - Switch the machine in the receiving mode and feed the program of the desired part drawing / file into the machine control unit for execution as per standard operating procedures - Select the program’s file for the desired part for execution as per standard operating procedures |
| CU5. Perform CNC Lathe Operations | - Ensure and check the safety control operation of the CNC machines as per standards before executing the program file. - Control the feeds, speeds and override of the machine before execution according to the prescribed procedures - Switch the machine to execution mode (single block or auto) and press cycle start to run the machining - Sequence as per prescribed method ??? - Compare the block-wise movements of the machining, - Not clear or repeatedComplete the job and measure / calculate the the accuries / errors according to the executed program of the drawing / design |

Knowledge & Understanding

- Work place safety and health considerations.

- Use of Personal Protective Equipment

- Reading Drawing

- Mechanism of working of CNC lathe machine

- Use of control panel and commands

- Program debugging techniques

- Use of Simulation software

- Use of portable devices for CNC lathe

- Use of Turret / Magazine and their sequence of tool mounting

- Possible accidents and their counteractions

- Coolant types along with benefits and uses

- Methods of calculating Coordinates techniques

- G codes and M codes

- Feed and speed concepts in Lathe Machine

- Lathe operations such as Facing, Turning, Drilling, Grooving, Threading, Knurling, Boring

- Use of Lathe tools and their types with respect to operations and materials

- Feed and speed concepts in Lathe Machine

Critical Evidence

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Use appropriate part programing credentials (seence, G-Code, M-code, Coordinates, feed, speed, and tooling information etc.) according to machine control units.

- Run simulation and verify movment of tools

- Perform CNC Lath operation

Tools & Equipment

- CNC Lathe Machine or Turning Centre along with Standard Accessories

- Lathe Tooling (Assorted Range)

- CNC Programming Manual

- CAM Software with Simulation Module

- Measuring Instruments (Vernier, Inside/Outside Calipers, Micrometer, Steel Rule,

- Tri-Square, Bevel Protractor etc.)

- Work Holding Devices

- Measuring Gauges

- Tooling Catalogue

- Complete Set of Computer System with Multimedia Projector

- Personal Protective Equipment (PPE)

- CNC Manual

### Perform CNC Milling Operations

Overview: This competency standard covers the skills and knowledge required to Mount  the  job  on  Milling Machine, Generate  the  Program  for CNC Milling, Run Simulation, Feed the Program into CNC Milling and Perform  CNC  Milling Operations

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Mount the job on Milling Machine | - Rewrite and edit all according to the previous - Mount the work-piece by considering the working capacity of machine as well as job requirement according to the drawing/design - Select appropriate work holding device(s) in order to achieve dimensional accuracy and clamp the job firmly as per standard practice - Attain proper alignment of tool/ cutter and work-piece e.g. co-axiality, concentricity of rotating jobs as per set practice - Keep safe measures while mounting the work-piece so that unwanted operation by machine may not be initiated as per safety precautions |
| CU2. Generate the Program for CNC Milling | - Select appropriate CAM software according to the machine control unit and import 3D model into it as per standard procedure Rewrite and edit all according to the previous - Define reference point and apply material/stock for machining to the design/model as per job requirements P3. Apply machining feature(s), toolpath strategies and leads/links as per prescribed procedure P4. Generate part program file against the applied machining sequence according to the post processor of CNC machine P5. Use appropriate part programming credentials (Sequence, G-codes, M-codes, Coordinates, Feed, Speed, Tooling Information etc.) according to the CNC machine control unit P6. Keep record of generated part program file in order to feed into machine control unit as per standard procedure |
| CU3. Run Simulation | P1. Feed the generated part program into appropriate simulation platform and run simulation for checking the tool gouge according to safety measures P2. Run simulation and verify movements of tool/cutter to get same results as per defined sequence P3. Identify occurrence of errors and modify the program as per defined procedure |
| CU4. Feed the Program into CNC Milling | P1. Ensure proper synchronization between machine control unit and part program file as per standard operating procedure P2. Switch machine to receiving mode and feed the desired part program file into machine control unit for further execution as per standard operating procedure - P3. Select the desired part program file for execution as per standard operating procedure Rewrite and edit all according to the previous |
| CU5. Perform CNC Milling Operations | P1. Ensure to control the safe operation of working on CNC machines before executing part program according to the safety measures P2. Control the feeds, speeds and override of machine before operating according to the prescribed procedure P3. Switch machine to execution mode (single block or auto) and press cycle start to run the machining sequence as per prescribed method P4. Compare the block-wise movements of machining sequence thoroughly during operating of machine according to the part program file P5. Complete the job and inspect its accuracy and precision according to the drawing/design |

- Rewrite and edit all according to the previous

Knowledge & Understanding

- Work place safety and health considerations

- Use of CAM and 3D models

- Mechanism of working of CNC Milling Machine

- Use of control panel and commands

- Program debugging techniques

- Use of Simulation Software

- Use of portable devices for CNC Milling

  - Use of turret / Magazine and their sequence of tool mounting

  - Possible accidents and their counteractions

  - Coolant types along with benefits and uses

  - Methods of calculating Coordinates techniques

  - G codes and M codes

  - Use of Clamping devices and their types

  - Feed and speed concepts in Milling machine

- Milling machine such as Facing, Drilling, Grooving, threading, Boring etc.

  - Use of Milling tools and their types with respect to operations and materials

Critical Evidence

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Use appropriate part programing credentials (seence, G-Code, M-code, Coordinates, feed, speed, and tooling information etc.) according to machine control units.

- Run simulation and verify movment of tools

- Perform CNC milling operation

Tools & Equipment

- CNC Milling Machine or Machining Centre along with Standard Accessories

- Lathe Tooling (Assorted Range)

- Zero Setter, Edge Finder and Dial Indicator

- Power Vice

- CNC Programming Manual

- CAM Software with Simulation Module

- Measuring Instruments (Vernier, Inside/Outside Calipers, Micrometer, Steel Rule,

- Tri-Square, Bevel Protractor etc.)

- Work Holding Devices

- Measuring Gauges

- Tooling Catalogue

- Complete Set of Computer System with Multimedia Projector

- CNC Manual

- Personal Protective Equipment (PPEs)

## . Machines and Mechanism

### Demonstrate Simple Mechanisms

Overview: This competency standard covers the knowledge and skills related to basic mechanisms. It includes primary and secondary joints, simple and complex links, four bar and quick return mechanisms. After completing this unit student should be competent to identify different types of simple mechanisms, Draw and interpret kinematic diagrams of simple mechanisms and perform kinematic inversion of simple mechanisms.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Identify Commonly Used Links and Joints | - Recognize the pin joints. - Recognize sliding joints. - Identify cam joints. - Identify gear joints. - Differentiate the simple and complex links. |
| - CU2. Identify simple mechanisms | - Identify the four bar mechanisms. - Identify Crank Rocker Mechanism - Identify Slider Crank Mechanism. - Identify Quick Return Mechanism. |
| - CU3. Analyze simple mechanisms | - Differentiate simple and complex mechanism Correct according to the previous suggestions - Draw kinematic diagram of simple mechanisms. - Interpret kinematic diagram of simple mechanisms - Identify degrees of freedom in mechanisms |
| - CU4. Perform kinematic inversion of mechanisms | - Perform Kinematic Inversion of four bar mechanisms. Correct according to the previous suggestions - Perform Kinematic Inversion of Crank Rocker Mechanism - Perform Kinematic Inversion of Slider Crank Mechanism. - Perform Kinematic Inversion of Quick Return Mechanism. |

Knowledge and Understanding:

- Kinematics

- Constrained Motion

- Machine Elements

- Simple mechanisms and their types

- Terminology of Mechanisms

- Kinematic Inversion

- Techniques of Mechanism Analysis

- Kinematic Diagrams

Critical Evidence(s) Required

- Recognise primary and secondary joints.

- Identify different types of simple mechanism

- Identify degrees of freedom in motion.

- Interpret kinematic diagrams of simple mechanisms.

Tools & Equipment:

- Pin Joints, Sliding Joints, Cams, Gears

- Four Bar Mechanism Model

- Slider Crank Mechanism Model

- Crank Rocker Mechanism Model

- Quick Return Mechanism Model

### Perform Kinematic Analysis of Simple Mechanisms

Overview:  This competency standard covers knowledge and skills required to kinematic analysis of simple mechanisms. It includes model building and simulation of simple mechanisms in working model software and calculation of liner and angular displacements and velocities.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Develop computer models of simple mechanisms | - Build Model of Slider Crank Mechanism Correct according to the previous suggestions - Simulate model of Slider Crank Mechanism - Build Model of Gear Mechanism - Simulate Gear Mechanism |
| - CU2. Perform Displacement Analysis | - Define a position of a point in a mechanism with a position vector. - Determine Liner Displacement. - Determine Angular Displacement |
| - CU3. Perform Velocity Analysis | - Determine Liner Velocity of a point on a link. - Determine Angular Velocity of a point on a link. - Relate Linear and Angular velocities of a point on a link. |

Correct according to the previous suggestions

Knowledge and Understanding:

- CAD Modelling

- Working Model Software

- Model Building in Working Model.

- Simulation of Mechanisms in Working Model

- Kinematic Analysis.

- Gear Terminology

- Techniques of Mechanism Analysis

Critical Evidence(s) Required

- Model Simple Mechanism in Working Model Software.

- Simulate Simple Mechanism in Working Model Software.

- Calculate liner and angular displacement of a point on a link.

Tools & Equipment:

- Computers

- CAD software

- Working Model Software

### Demonstrate Mechanical Power Transmission Elements.

Overview:  This competency standard covers the various methods of mechanical power transmission methods. Covering the knowledge related to belts, gears and chain drives, this unit will enable students to select appropriate power transmission method.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Belts Drives | - Identify the Belt Types. - Identify the Pulley Types - Understand the Belt Drive Geometry. |
| - CU2. Gears | - Identify the types - Understand Spur Gear Terminology. - Understand Spur Gear Geometry. |
| - CU3. Chain Drives | - Select chain types. - Understand Chain Derive Geometry |

Knowledge and Understanding:

- Mechanical power transmission fundamentals.

- Types of belt drives and pulleys

- Belt Drives Geometry

- Types of Gears

- Gear Geometry

- Types of chain Drives

- Chain Drive Geometry.

Critical Evidence(s) Required

- Recognise various types of gears, belts and chains.

- Describe the various terms of gear geometry.

Tools & Equipment:

- Belts and Pulleys of Various Sizes

- Various types of Gears.

- Various types of Chain Drives

### Perform Kinematic Analysis of Mechanical Power Transmission Elements.

Overview: This competency standard covers the knowledge and skills required for kinematic analysis of    mechanical power transmission elements. Covering the knowledge related to belts, gears and chain drives, this unit will enable students to perform the kinematic analysis of selected power transmission method

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Belts Drives | - Select the belts size - Determine the diameter of driver and driven pulleys. - Calculate velocity ratio for belts - Calculate the belt speed. |
| - CU2. Gears | - Determine the rotational velocities of driver and driven gears. - Calculate Velocity Ratios Gears. |
| - CU3. Chain Drives | - Determine the Velocity Ratio for Chains. - Calculate Chain Speed |

Knowledge and Understanding:

- Mechanical power transmission fundamentals.

- Belt Drives Geometry

- Belt Drives Kinematics

- Gear Geometry

- Gear Kinematics

- Chain Drive Geometry

- Chain Drive Kinematics

Critical Evidence(s) Required

- Calculate Velocity Ratios for Gears, Belt and Chain Drives

Tools & Equipment:

- Belts and Pulleys of Various Sizes

- Various types of Gears.

- Various types of Chain Drives

- Computers

- Working Model Software

## AC & DC Machines

### Verify Basic Laws of Electrical Machines

Overview: This competency standard covers the skills and knowledge required to verify Faraday’s law by moving permanent magnet inside the coil, verify Faraday’s law by moving coil near the magnet field, verify Faraday’s law using relative motion of coil and magnet, verify Faraday’s Law using simple loop generator, verify EMF through induction, verify Torque induce in a current carrying loop and verify Mutual induction.

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Verify Faraday’s law by moving permanent magnet inside the coil. | - Construct a coil. - Connect a Galvanometer with coil. - Move permanent magnet inside the coil fast and slow. - Record the effect of movement of magnet on reading of Galvanometer. - Hold the magnet inside the coil and keep it stationary Record the effect on reading of ?? Galvanometer again. |
| - Verify Faraday’s law by moving coil near the magnet field. | - Construct a coil. - Connect Galvanometer with coil. - Move the coil with various speeds (i.e from slow to the fast) inside / outside the permanent magnet which is set at the stationary position. - Record the effect coilon via Galvanometer. - Hold the coil near the magnetic field and keep them stationary . - Record the effect via Galvanometer. |
| - Verify Faraday’s law using relative motion of coil and magnet. | - Construct a coil. - Connect the Galvanometer with coil. - Make relative motion of coil and magnet. - Record the effect of movement on the coil via Galvanometer. |
| - Verify Faraday’s Law using simple loop generator. | - Identify the single loop generator and its parts. - Select field winding. - Connect Galvanometer with single loop coil. - Rotate the single loop coil of generator in the field - Measure the voltage induces in the loop generator. - Record the effect via Galvanometer. |
| - Verify Torque induce in a current carrying loop | - Construct a coil. - Apply DC voltage to the coil. - Place a current carrying loop in the coil. - Check the direction of force to verify the torque produce not clear ??. |
| - Verify Mutual induction | - Construct two coils on the two different legs of the similar core. - Apply AC voltage to either one of the coil (act as primary winding). - Check the volts induce in the another coil via voltmeter. |

Knowledge & Understanding:

- Explain first law of Faraday.

- Explain second law of Faraday.

- How can we find the direction of induced emf?

- Explain the  role of magnetic strength in Faraday’s Law?

- Explain the  role of conductor’s length or turns in Faraday’s Law?

- Explain Lenz’s Law.

- Explain torque.

- Why force produce on current carrying conductor.

- Explain core.

- Explain induction.

- Explain Faraday’s first law of Electro-Magnetic Induction.

- Explain meaning of coupled coils.

- Why emf induced in 2nd coil, when voltage is applied to 1st coil.

- Explain mutual induction.

- Explain core.

- Which machine work on the principle of mutual induction

Tools & Equipment:

- Galvanometer.

- Magnet.

- Connecting leads.

- Transformer

- Rheostat

- Loop

- Voltmeter

- Coil

- DC source

- Connecting leads.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Verify Faraday’s law using any method.

### Analyse an Alternator

Overview: This competency standard identifies the competencies required to measure the stator winding resistance with DC test, perform no load test, perform short circuit test, measure the efficiency of an alternator, measure the effect of field current on terminal voltage, measure the effect of speed changes on alternator frequency and active power and perform parallel operation of Alternators.

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Measure the stator winding resistance with DC test. | - Connect DC Ampere meter in series with armature. - Apply DC voltage to either one of the two phases. - Measure current and voltage via multimeter. - Measure the resistance per phase by ohm’s law. |
| - Perform no load test. | - Make connection according to diagram. - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Measure Iron losses from watt meter. |
| - Perform short circuit test. | - Make connection according to diagram. - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage 10 to 15 % to its rated value. - Measure the copper loss from the watt meter. |
| - Measure the efficiency of an alternator. | - Perform no load test to measure Iron losses. - Perform short circuit test to measure copper losses. - Apply formula to find efficiency. |
| - Measure the effect of field current on terminal voltage. | - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage to its rated value.not clear - Draw no load curve between field current and terminal voltage.not clear |
| - Measure the effect of speed changes on alternator frequency and active power. | - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Apply electrical load on alternator. - Measure the effects of load on alternator speed and frequency. - Adjust the prime mover speed to its rated value. - Measure the effect of this change on alternator speed and frequency. |
| - Perform parallel operation of Alternators. | - Run the incoming alternator to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Use synchronous ?? scope to synchronize alternator’s frequency and phase angle. - Check the phase sequence with the help of dark lamp method. - Check the voltage of bus bar and incoming machine via volt meter. - Switch on the breaker after completing parallel operation ??. |

Knowledge & Understanding:

- How to connect DC source with three phase stator?

- How to calculate the resistance of winding by using meter reading.

- Which type of losses is found with no load test.

- Explain the  effect of field current on terminal voltage.

- Explain iron loss.

- Explain copper loss.

- Which type of losses are found with short circuit test.

- Explain the  effect of rheostat resistance on terminal voltage.

- Explain core saturation.

- Why the graph between field current and armature voltage is not a straight line.

- How to change the RPM of the generator?

- Explain the  effects of load on alternator speed and frequency.

- Explain the  conditions of paralleling of alternators.

- Explain the connect alternators in parallel.

- Explain the  working of synchronous scope.

- Explain phase sequence.

- Explain phase angle.

- Explain dark lamp method.

Tools & Equipment:

- Alternator

- Ampere meter

- Voltmeter

- DC source

- Connecting wires

- Alternator

- Tachometer

- Ampere meter

- Voltmeter

- Wattmeter

- DC source

- Connecting wires

- Frequency meter

- Resistive load

- Connecting wires

- Synchronous scope

- Lamps

- Hz meter

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard

- Measure the efficiency of an alternator

- Inspect the pulley and bearing

### Analyse Single Phase Motors.

Overview: This competency standard identifies the competencies required to operate split phase single phase AC motor, operate capacitor start and Capacitor run single phase AC motor and operate shaded pole single phase motor.

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Operate split phase single phase AC motor. | - Make the connections according to circuit diagram. - Apply rated voltage to the stator. - Measure RPM and direction of rotation. - Disconnect the auxiliary winding from centrifugal switch. - Check the effect of this change. |
| - Operate capacitor start and Capacitor run single phase AC motor. | - Make connections according to circuit diagram. - Apply rated voltage to the stator. - Measure the RPM. - Remove the starting capacitor with centrifugal switch. - Measure the RPM again. - / Analyze / Observe the effect of starting capacitor on motor’s torque. |
| - Operate shaded pole single phase motor. | - Make connections according to circuit diagram. - Apply rated voltage to the stator. - Change the applied voltage to measure the effect on motor’s speed. |

Knowledge & Understanding

- Explain starting winding?

- Explain running winding?

- How to rotate magnetic field in single phase motor.

- Explain the connect capacitor with starting winding.

- Explain the  effect of capacitor on starting torque.

- Explain starting winding.

- Explain running winding.

- How to rotate magnetic field in single phase motor.

- Explain the  working of shaded pole motor.

- Which type of voltage applied to the stator of shaded pole motor.

Tools & Equipment:

- Single phase induction motor

- Tachometer

- Centrifugal switch

- Connecting wires

- Single phase induction motor

- Capacitor

- Shaded pole motor

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

- Operate single phase Motor and carry out measurements.

### Analyse Special Purpose Motors.

Overview: This competency standard identifies the competencies required to operate and speed control of AC series motor, operate miniature (reluctance and hysteresis) single phase AC motors, construct and operate stepper motor and construct and operate and develop control circuit with the help of servo motor.

| - Operate and speed control of AC series motor. | - Connect the armature and field winding in series. - Apply rated voltage to the motor. - Measure and analyze the effect on motor’s speed by change in applied voltage. |
| --- | --- |
| - Operate miniature (reluctance and hysteresis) single phase AC motors. | - Connect the stator of motor with single phase supply. - Apply mechanical load by hand - Measure the effect of load change on motor speed. - Draw torque speed curves. |
| - Construct and operate stepper motor. | - Connect the stator of the motor with control circuit. - Adjust the control voltage of control unit. - Measure the RPM. - Readjust the control voltage of control unit. - Measure the RPM again. - Analyze and distinguish the effect of Control voltage and RPM. |
| - Construct, Operate and develop control circuit of servo motor. | - Pair the servo motor with some encoder to provide position and speed feedback. - Compare the measured position to external input to the controller. - Check the effect of feedback on motor position. |

Knowledge & Understanding:

- Explain the working of AC series motor.

- Explain the effect of applied voltage on speed of the motor.

- Explain the working of reluctance motor.

- Explain the working of hysteresis motor.

- Explain the working of stepper motor.

- Explain the working of control unit for stepper motor.

- Explain the working of servo motor.

- Explain the working of control unit for servo motor.

Tools & Equipment:

- AC series motor

- Tachometer

- Connecting wires

- Reluctance motor

- Hysteresis motor

- Stepper motor

- Control unit

- Servo motor

- Control circuit

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard

- Construct and operate stepper motor

### Analyse Three Phase Motors.

Overview: This competency standard identifies the competencies required to identify and operate 3 phase Squirrel cage motor, verify rotating magnetic field , measure the starting and running current of three phase induction motor, draw torque slip curve of three phase induction motor, determine the slip of 3 phase induction motor by stroboscope, start 3 phase Induction motor with direct on line, start 3 phase Induction motor with Auto transformer starter, start 3 phase Induction motor with Star Delta starter, make connection to reverse the direction of induction motor, determine the efficiency of three phase induction motor, control the speed of three phase induction motor by changing primary voltage, control the speed of three phase induction motor by rotor resistance control method, operate two motors in cascade, start the synchronous motor using external prime mover and identify the effect of field current on three phase synchronous motor.

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Identify and operate 3 phase Squirrel cage motor. | - Identify and select three 3 phase squirrel cage motor. - Make connection as per diagram. - Connect the supply and interpret the result. |
| - Verify rotating magnetic field | - Connect the three terminals of stator winding with each other and with ground. - Apply three phase supply to the other terminals. - Place a compass between the stator and note the direction of rotating magnetic field. - Change the two phases with each other. - Observe / Note the direction of rotating magnetic field. |
| - Measure the starting and running current of three phase induction motor. | - Connect the stator winding in star. - Connect an ammeter in series with stator. - Note down the ammeter reading at instant of just starting. - Note down the reading of ammeter at running condition. |
| - Draw torque slip curve of three phase induction motor | - Connect three phase induction motor with electro dynamo meter with Belt. - Start the motor and run at rated speed - Measure the RPM. - Change the torque with electro dynamo meter and note the speed - Calculate slip with given values of speed. - Draw graph between slip and torque. |
| - Determine the slip of three 3 phase induction motor by stroboscope | - Connect the stator of three phase induction motor in star - Apply single phase AC supply to the stroboscope. - Mark dot on motor shaft with marker. - Apply three phase supply to the motor - Note the RPM of the motor with the help of stroboscope. - Calculate slip. |
| - Start three 3 phase Induction motor with direct on line. | - Draw line diagram of direct-on three 3-phase motor Starter. - Identify and select 3 phase Induction motor. - Make Connection of motor with DOL as per diagram. - Switch on the 3 phase breaker and interpret the result. |
| - Start 3 phase Induction motor with Auto transformer starter. | - Connect power circuit with the help of diagram. - Connect control circuit with the help of diagram. - Apply rated voltage to the motor. - Gradually reduce the taping of auto transformer with the help of control circuit. |
| - Start 3 phase Induction motor with Star Delta starter. | P1: Draw line diagram of a starter-delta starter. P2: Identify and select 3 phase Induction motor. P3: Make Connection of motor with star –delta- starter as per diagram. P4: Switch / Power on via three 3 phase breaker and interpret the result. |
| - Make connection to reverse the direction of induction motor. | P1: Draw line diagram to reverse the direction of induction motor P2: Identify and select 3 phase Induction motor. P3: Make Connection of motor through 3 pole 2 way switch as per diagram. P4: Move the handle of 3 Pole 2 Way switch from off position to position 1. P5: Move the handle of 3 Pole 2 Way switch from Position 1 to off position and wait until the motor is stopped P6: Move the handle of 3 Pole 2 way switch from off position to position 2 and interpret the result. |
| - Determine the efficiency of three phase induction motor | - Connect wattmeter, ammeter with the stator of three phase induction motor. - Run the motor at rated speed. - Measure the reading of wattmeter and voltmeter. - Stop the motor - Couple three phase induction motor and electro dynamo meter with belt. - Run the motor. - Supply single phase supply to the dynamo meter. - Adjust the supply of electro dynamo meter so that the maximum current passes through the motor. - Read the meters reading. - Calculate the efficiency of the motor by using these reading. |
| - Control the speed of three phase induction motor by changing primary voltage | - Connect the stator of the motor in star connection. - Insert rheostat in series with stator of the motor. - Set the rheostat resistance to its max value. - Run the motor - Change rheostat resistance and note the effect of speed changes. |
| - Control the speed of three phase induction motor by rotor resistance control method | - Select wound rotor induction motor. - Connect the stator of motor in star connection. - Connect a star connected starter with motor rotor. - Set rotor resistance to its max value. - Run the motor. - Change the rotor resistance and note the effect of speed change. |
| - Operate two motors in cascade | - Select two wound rotor induction motor. - Couple shaft of the both motors. - Connect the stator of motor 2 with the rotor of motor 1. - Connect the motor 2 rotor in star connection. - Apply three phase voltage to the stator of motor 1. - Control the speed of both motors by changing the rotor resistance of motor 2. |
| - Start the synchronous motor using external prime mover. | - Couple the shaft of the synchronous motor with external prime mover (DC Shunt motor) - Connect the stator winding of synchronous motor in star connection. - Apply rated voltage to the stator. - Run the prime mover, which results in locking of rotor with stator magnetic field. |
| - Identify the effect of field current on three phase synchronous motor. | - Apply rated voltage to stator. - Excite the field winding. - Change the field winding voltage and measure the effect on Armature current. - Draw the V curve between Armature Current and Field current. |

Knowledge & Understanding:

- How many types of three phase induction motor with respect to the rotor?

- Explain the term rotating magnetic field.

- How to change the direction of rotation of motor.

- Explain the initially start the motor in star connection.

- Why motor take large current at the time of starting.

- Explain back EMF.

- How to change the mechanical load on motor?

- How to measure the speed of DC motor?

- Explain motor slip.

- Explain the effect of torque on motor slip.

- Explain the working of stroboscope.

- Explain motor slip.

- How to calculate the slip from motor speed.

- What in magnetic contactor.

- Explain relay.

- Explain the working of DOL.

- Explain auto transformer.

- How to change the taping of auto transformer?

- Explain the never start motor without load.

- Which type of connection is use at the starting of motor.

- Which type of connection is use at the running of motor.

- What in magnetic contactor.

- Explain different between control circuit and power circuit.

- What in magnetic contactor.

- How to reverse the direction of motor.

- Explain the connection of wattmeter.

- Explain the working of electro dynamo meter.

- Explain iron loss.

- Explain copper loss.

- Explain the find copper loss at full load.

- Which loss having maximum value and why.

- How many techniques are used to control the speed of three phase induction motor.

- Explain rheostat.

- Explain the effect of voltage changes on speed of motor.

- Explain the connect the stator of motor in star.

- How to insert the resistance in rotor circuit.

- Explain the insert maximum resistance in rotor circuit at the time of starting.

- Explain back EMF.

- Explain mean by cascade.

- Write the name of starting method of synchronous motor.

- Why synchronous motor is not self-starting.

- Which type of supply voltage is applied to the synchronous motor rotor.

- How to change the field excitation of synchronous motor?

- Explain over and under excitation.

- Explain V curve of synchronous motor.

Tools & Equipment:

- Three phase squirrel cage induction motor

- Connecting wires

- Three phase stator of induction motor

- Compass

- Three phase induction motor

- Electro dynamo meter

- Belt

- Tachometer

- Stroboscope

- Marker

- Tachometer

- DOL starter

- Auto transformer

- Synchronous motor

- DC shunt motor

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard

- Determine phase sequence

### Analyze Dc Generators

Overview: This competency standard identifies the competencies required to calculate different terms used in armature winding of DC machines, develop Single Layer Lap Winding, develop Double Layer Wave Winding, measure the resistance of field winding, armature winding and terminal polarity, identify DC shunt generator, identify DC series generator and identify DC compound generator.

| Critical Evidence |  | Performance Criteria |
| --- | --- | --- |
| - Calculate different terms used in armature winding of DC machines | - Count the number of slots (S) of armature. - Calculate pole pitch (Y). - Calculate Coil span / pitch. - Calculate Back pitch (Yb) - Calculate Front pitch (Yf) - Calculate Resultant Pitch (YR) - Calculate commutator pitch (Yc) |  |
| - Develop Single Layer Lap Winding | - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commutator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Subtract the value of back pitch from the added value. - Connect added value from front side to subtracted value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. |  |
| - Develop Double Layer Wave Winding | - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commutator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Add the value of front pitch in previous added value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. |  |
| - Measure the resistance of field winding, armature winding and terminal polarity | - Open the terminal box. - Connect the ohm meter with the terminal of both winding. - Read down ohm meter reading. - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. - Rotate the generator slowly by hand. - Touch the probe of galvanometer with each winding. - Identify armature with the deflection of galvanometer pointer. - Mark the polarity of terminals with the deflection of galvanometer. |  |
| - Identify DC shunt generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC shunt generator from measured value of resistance. - Disconnect both windings. - Identify the shunt field winding of shunt generator. - Identify armature of shunt generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |  |
| - Identify DC series generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC series generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series generator. - Identify armature of series generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |  |
| - Identify DC compound generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC Compound generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of generator. - Identify the shunt field winding of generator. - Identify armature of shunt generator. - Measure and record the resistance of both winding separately. - Measure and Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |  |

Knowledge & Understanding:

- Explain the term armature slots.

- Explain pole pitch (Y).

- Explain coil span/pitch.

- Explain Back pitch (Yb)

- Explain Front pitch (Yf)

- Explain Resultant Pitch (YR)

- Explain commutator pitch (Yc)

- Explain lap winding.

- How many parallel paths in lap winding?

- Explain mean by single layer winding.

- Explain wave winding.

- How many parallel paths in wave winding.

- Explain mean by double layer winding.

- How to read the scale of galvanometer?

- Explain difference between field winding and armature winding?

- Which winding having low resistance and why?

- Explain the  technique to identify the terminal polarity.

- How many winding in shunt generator?

- Explain different between field winding and armature winding.

- Which winding having low resistance and why.

- Explain the  technique to identify the terminal polarity.

- How to disconnect both winding of shunt generator.

- How many winding in series generator?

- Explain different between field winding and armature winding.

- Which winding having low resistance and why.

- Explain the  technique to identify the terminal polarity.

- How to disconnect both winding of series generator.

- Which conduction you follow at the time of reconnection of both winding.

- Study of faraday’s law

- study of Motors and generators

- study of rules of generators

- Define the Type of DC  machines

- Explain the principle of simple loop generator.

Tools & Equipment:

- Stator frame

- Armature

- Winding machine

- Carbon brushes

- Insolating paper

- Sleeve

- Soldering iron

- Soldering wire

- DC machine

- Galvanometer

- Ohm meter

- Connecting wires

- DC machine

- Galvanometer

- Ohm meter

- Connecting wires

- DC machine

- Galvanometer

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard

- Identify DC Generator Type

- Measure the resistance of field winding, armature winding and terminal polarity

### Perform Tests on DC Generators

Overview This competency standard identifies the competencies required to Plot the Open circuit characteristics of a Separately excited D.C  generator, plot the Open circuit characteristics of a D.C shunt generator, plot the load characteristics of D.C shunt generator, plot the load characteristics of D.C series generator, plot the load characteristics of D.C compound generator for under, flat and over compounding, perform parallel operation of two DC shunt generators and determine copper, iron & friction losses of shunt generator by actual loading.

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Plot the Open circuit characteristics of a Separately excited D.C generator | - Connect DC supply to field coil through rheostat and ampere meter. - Rotate armature at constant speed. - Connect voltmeter across armature terminal. - Decrease the resistance of rheostat in steps and measure and record the relevant output voltage of armature and field current. - Repeat the P2 to P4 till voltage of armature reach at its saturation point. - Plot the graph between field current and armature voltage from the recorded values. |
| - Plot the Open circuit characteristics of a D.C shunt generator | - Connect DC supply to field coil through rheostat and ampere meter. - Rotate armature at constant speed. - Connect voltmeter across armature terminal. - Decrease the resistance of rheostat in steps and measure and record the relevant output voltage of armature and field current. - Repeat the P2 to P4 till voltage of armature reach at its saturation point. - Plot the graph between field current and armature voltage from the recorded values. |
| - Plot the load characteristics of D.C shunt generator. | - Select DC shunt generator and connect variable load. - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. |
| - Plot the load characteristics of D.C series generator. | - Select DC series generator and connect variable load. - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Increase the load in steps unless specific reduction in terminal voltage will observed. - Plot the graph between load current and terminal voltage from the recorded values. |
| - Plot the load characteristics of D.C compound generator for under, flat and over compounding | - Select DC compound generator and connect variable load. Correct as the previous - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Adjust field diverter on zero ohm. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. - Such off the load. - Adjust the field diverter at such position that the terminal voltages are equal to no load voltages. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. - Such off the load. - Adjust the field diverter at such position that the terminal voltages are greater than no load voltages. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. |
| - Perform parallel operation of two DC shunt generators. | - Connect the circuit according to the circuit diagram. - Place all switches in off position. - Run generator 1 at rated speed. - Reduce field resistance of generator 1 - Connect generator with bus bar after attaining rated voltage. - Run generator 2 with prime mover. - Reduce the field resistance of generator 2 to attain its rated voltage. - Close the switches for parallel operation. - Calculate and record the reading of voltage and current on different loads. |
| - Determine copper, iron & friction losses of shunt generator by actual loading. | - Connect ammeter in series with both winding. - Connect voltmeter in parallel with both winding. - Apply 10-20 DC volt. - Record the readings of voltmeter and ammeter. - Calculate Ra and Rsh. - Run the generator at rated speed with prime mover. - Turn on the load switch and record the meter reading. - Calculate input power and output power. - Calculate total losses (Copper loss, Friction loss, Iron loss) |

Knowledge & Understanding:

- Explain different between shunt and separately excited generator.

- Explain rheostat.

- How to use rheostat with field winding?

- Explain the  effect of rheostat resistance on terminal voltage.

- Explain core saturation.

- Why the graph between field current and armature voltage is not a straight line.

- Explain different between shunt and separately excited generator.

- Why the graph between field current and armature voltage is not a straight line.

- Explain armature reaction.

- Explain the reason of voltage reduce due to increase in load.

- How to compensate armature reaction?

- How to connect field and armature winding?

- How to connect field and armature winding in compound generator?

- Explain under compounding.

- Explain flat compounding.

- Explain over compounding.

- Explain the conditions of paralleling of DC generators.

- How to connect field and armature winding in shunt generator?

- Explain the connect DC generators in parallel.

- Explain iron loss.

- Explain copper loss.

- Explain the find copper loss at full load.

- Which loss having maximum value and why.

- Determine the condition of maximum efficiency of DC machine

Tools & Equipment:

- Connecting wires

- DC generators

- Ampere meters

- Voltmeters

- Tachometer

- Rheostats

- Switches

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard

### Analyze Dc Motors.

Overview This competency standard identifies the competencies required to identify DC shunt motor, identify DC series motor and identify DC compound motor.

| Critical Evidence |  | Performance Criteria |
| --- | --- | --- |
| - Identify DC shunt motor | - Measure the terminal resistance of motors with ohm meter. - Identify DC shunt motor from measured value of resistance. - Disconnect both windings. - Identify field winding of shunt motor. - Identify armature of shunt motor. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |  |
| - Identify DC series motor | - Measure the terminal resistance of DC motors with ohm meter. - Identify DC series motor from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series motor. - Identify armature of series motor. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |  |
| - Identify DC compound motor | - Measure the terminal resistance of DC motor with ohm meter. - Identify DC Compound motor from measured value of resistance. - Disconnect both windings. - Identify the series field winding of motor. - Identify the shunt field winding of motor. - Identify armature of compound motor. - Measure and record the resistance of both winding separately. - Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |  |

Knowledge & Understanding:

- How many winding in shunt motor?

- Explain different between field winding and armature winding.

- Which winding having low resistance and why.

- Explain the  technique to identify the terminal polarity.

- How to disconnect both winding of shunt motor.

- How many winding in series motor?

- Explain different between field winding and armature winding.

- Which winding having low resistance and why.

- Explain the technique to identify the terminal polarity.

- How to disconnect both winding of series generator.

- Which conduction you follow at the time of reconnection of both winding.

- Study of faraday’s law.

- Study of Motors and generators

- Study of rules of generators

- Define the Type of DC     machines

- Explain the principle of simple loop generator.

- Define Basic DC Systems.

- Describe types of DC Generators.

Tools & Equipment:

- DC shunt motor

- Ohm meter

- Connecting wires

- DC series motor

- Connecting wires

- DC compound motor

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard

- Identify DC Motor Type

### Perform Tests On DC Motors.

Overview This competency standard identifies the competencies required to Plot the load characteristics of D.C shunt motor, plot the load characteristics of D.C series motor, plot the load characteristics of D.C compound motor, control the speed of DC series motor, control the speed of DC shunt motor, start  DC shunt motor with 3 point starter, identify the effect of back EMF in DC motor, calculate BHP of motor by Brake test, calculate efficiency of the motor by Swinburne Test and perform Regenerative / Hopkinson/ Back to back Test.

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Plot the load characteristics of D.C shunt motor | - Couple electro dynamo meter to the DC motor with belt - Connect shunt motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter’s control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics for the DC shunt motor.. |
| - Plot the load characteristics of D.C series motor. | - Select DC series motor and achieve connection as per diagram. - Turn on DC power supply and increase the load to full value. - Record the value of speed and current on each load. - Draw the graph between load current and speed |
| - Plot the load characteristics of D.C compound motor* | - Couple electro dynamo meter to the DC motor with belt - Connect compound motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC compound motor.. |
| - Control the speed of DC series motor. | - Make the connection as per circuit diagram. - Insert external resistances in series to armature and field coil. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different supply voltages. - Make graph between speed and applied voltage. |
| - Control the speed of DC shunt motor. | - Make the connection as per circuit diagram. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different field current. - Make graph between speed and field current - Fixed the field current and very armature current - Record the speed at different armature current. - Make graph between speed and armature current |
| - Start DC shunt motor with 3 point starter. | - Make the connection as per circuit diagram. - Switch on the supply and move starter handle from off position to position 1. - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. |
| - Identify the effect of back EMF in DC motor. | - Select DC series motor and make connection as per diagram. - Turn on DC power supply and increase the load 0 to full value. - Record the value of speed and current on each load. - Calculate back EMF by using the recoded values and appropriate formula. - Make graph between speed and back EMF and interpret the result. |
| - Calculate Brake Horse Power (BHP) of motor by Brake test. | - Connect the spring balance with one end of the rope. - Connect other end of rope with weight (W1). - Measure the radius of the pulley. - Hang the weight on the pulley of the motor and fix the spring balance end with earth. - Run the motor. - Measured the RPM of the motor. - Adjust the weight to pass full load current from motor. - Calculate spring balance weight and hanged weight (W1) - Calculate shaft torque. - Calculate BHP of motor by shaft torque. |
| - Calculate efficiency of the motor by Swinburne Test | - Perform DC test to find the value armature resistance and field resistance - Run the motor at rated voltage without load. - Adjust the value of shunt regulator to attain rated speed. - Measure the Field and no load current via ampere meter. - Calculate total copper losses and iron losses. - Calculate input and output power. - Calculate efficiency from calculated power. |
| - Perform Regenerative / Hopkinson/ Back to back Test | - Identify two shunt machines. - Couple the shafts of both machines. - Connect the machines electrically in such a way that one machine work as a motor and other as a generator. - Connect external power supply to the machines to overcome the machine losses. - Run both the machines at no load. - Reduce the field excitation of one machine which work as a motor. - Increase the field excitation of one machine which work as a generator. - Adjust the field winding in a way that voltage of generator becomes equal to the applied voltage which result the machines runs in parallel - Calculate the efficiency by using voltmeter and ammeter. |

Knowledge & Understanding:

- How many winding in compound generator?

- Explain different between field winding and armature winding.

- Which winding having low resistance and why.

- Explain the  technique to identify the terminal polarity.

- How to disconnect both winding of series generator.

- Which conduction you follow at the time of reconnection of both field windings.

- Why series winding having low turns.

- Explain the  effect of field current on motor speed.

- Explain the  effect of load on motor speed.

- Explain thenever start DC series motor without load.

- Why series winding having low turns and shunt winding having high number of turns.

- Explain the  different between shunt, series and compound motor load characteristics.

- Explain speed control of DC motor with applied voltage.

- Explain the control the speed of DC motor.

- How many techniques to control the speed of DC shunt motor.

- Explain the effect of field current changes on motor speed.

- Explain 3-point starter.

- Explain thenever start DC shunt motor without starter.

- Explain back EMF.

- Explain generator action in motor.

- Explain the  effect of motor speed on back EMF.

- How to measure the RPM of the motor?

- Explain shaft torque.

- How to calculate BHP from shaft torque?

- Explain iron loss.

- Explain copper loss.

- Explain the find copper loss at full load.

- Which loss having maximum value and why.

- Explain the connect external source with motor generator set.

- Explain mean by field excitation.

- Explain the term efficiency.

Tools & Equipment:

- DC shunt motor

- Electro dynamo meter

- Belt

- Ohm meter

- Tachometer

- Ammeter

- Connecting wires

- DC series motor

- DC compound motor

- Resistor

- Voltmeter

- Rope

Critical Evidence(s) Required

- Perform Brake Test

- Perform Swinburne Test

- Perform back to back Test

## Digital Electronics

### Verify Truth Tables of Digital Gates

Overview: This competency standard covers the skills and knowledge required to Verify the truth table of AND gate, Verify the truth table of OR gate, Verify the truth table of NOT gate, Verify the truth table of NAND gate, Verify the truth table of NOR gate, Verify the truth table of XOR gate and Verify the truth table of XNOR gate Verify the truth table of AND gate, OR gate, NOT gate, NAND gate, NOR gate, XOR gate and XNOR gate.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Verify the truth table of AND gate | - Identify the symbol of logic gate, IC & logic function. - Place (AND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU2. Verify the truth table of OR gate | - Identify the symbol of logic gate, IC & logic function. - Place (OR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU3. Verify the truth table of NOT gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOT gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU4. Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU5. Verify the truth table of NOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU6. Verify the truth table of XOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU7. Verify the truth table of XNOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |

Knowledge & Understanding:

- Study logic gates Logic gates. AND, OR, NAND, NOR, NOT, XOR and XNOR.

- Study the Boolean expression of AND, OR, NAND, NOR, NOT, XOR and XNOR, gate and its equivalent electrical circuit

Tools & Equipment:

- Define Universal gate and enlist its types.

- AND gate (7408 2-input Quad)

- OR gate (7432 2-input Quad)

- NOT gate (7404 Hex)

- NAND gate (7400 2-input Quad)

- NOR gate (7402 2-input Quad)

- X-OR gate (7486 2-input Quad)

- X-NOR gate (74266 2-input Quad)

- Bread board

- LED

- DC supply (5 V)

- Connecting leads

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard

- Construct logic gate circuit.

### Construct & Verify Combinational Logic Circuit

Overview: This competency standard covers the skills and knowledge required to . Apply Karnaugh mapping & Boolean algebra to simplify logic expressions, Construct & verify the truth table of Half adder, Construct & verify the truth table of Full adder, Construct & verify the truth table of Half Subtraction,   Operate seven segment display with seven segment decoder, Construct & verify the truth table of Full Subtraction, Verify Encoder, Verify Encoder and Verify multiplexer and DE- multiplexer Construct & verify the truth table of Half adder, full adder and Half Subtraction.

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Apply Karnaugh mapping & Boolean algebra to simplify logic expressions | - Identify the SOP & POS - Apply Boolean algebra & Karnaugh mapping to simplify SOP & POS. - Construct logic circuits with simplified SOP & POS. |
| - Construct & verify the truth table of Half adder | - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a half-adder circuit using one XOR gate and two NAND gates. |
| - Construct & verify the truth table of Full adder | - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a full-adder circuit using two ICS 7486 and & ?? 7400. |
| - Construct & verify the truth table of Half Subtraction | - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input |
| - Construct & verify the truth table of Full Subtraction | - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| - Verify Decoder | - Place (Decoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| - Operate seven segment display with seven segment decoder. | - Insert (7 segment decoder IC) and 7 segment display on bread board. - Identify the input, output, Vcc and ground pin. - Connect segment display with seven segment decoder input output pins. - Record & verify the output result against each given input. |
| - Verify Encoder | - Place (Encoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| - Verify multiplexer and DE- multiplexer | - Implement following function with multiplexer F(ABC)=∑ (0,2,3,4,5,6): - Implement 4-to-1 mux and one 2-to-1 mux. - Implement 1-to-4 dmux using 1-to-2 dmux. |

Knowledge & Understanding:

- Describe the laws and rules of Boolean algebra.

- Understanding of commutative. • and distributive expiration. That is, A • (B + C) = (A • B) + (A • C) and A + (B • C) = (A + B) • (A + C).

- Study the combinational logic circuit. (Half adder, Full adder, Half subtraction, Full subtraction, Binary Multiplier, Magnitude Comparator)

- Study the Product-of-Sums& SOP Simplification

- Knowledge of Don't-Care Conditions

- Understanding of Karnaugh Map of four Variables.

- Understanding of Decoders & Encoders&   Multiplexers.

- Knowledge of Pin configuration of iCs

- Knowledge of7 segment display.

- Explain pin 7 segment display and common cathode 7 segment display.

- Define limiting resistor.

- Understanding how to implement functions using multiplexers.

- To study DE multiplexer

Tools & Equipment:

- X-OR gate (7486 2-input Quad)

- AND gate (7408 2-input Quad)

- OR gate (7432 2-input Quad)

- NOT gate (7404 Hex not gate)

- Bread board

- LED

- DC supply (5 V)

- Connecting leads

- 74LS139 IC

- 74LS47 IC

- Seven segment display

- Resistances (1K ohm)

- Mux KL-33006 block e

- Mux KL-33006 block f

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard.

- Verification of Boolean expression with help of logic gates and circuits.

- Construct adder & subtraction.

### Construct and Verify Function of Flip Flops

Overview: This competency standard covers the skill and knowledge required to Construct and verify the truth table of RS latch using NAND gate, Construct and verify the truth table of clocked RS latch using NAND gate, verify function of D flip flop, verify function of JK/T flip flop construct and verify the truth table of RS latch using NAND gate, clocked RS latch using NAND gate, D flip flop and JK flip flop.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Construct and verify the truth table of RS latch using NAND gate | - Place the NAND gate IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to the outputs pins. - Apply various logic inputs to Record & verify the output result against each given input. |
| CU2. Construct and verify the truth table of clocked RS latch using NAND gate | - Place the NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply various logic inputs to Record & verify the output result against each given input. |
| CU3. Verify function of D flip flop. | - Insert the D flip flop IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to the outputs pins. - Apply various logic inputs to Record & verify the output result against each given input. |
| CU4. Verify function of JK/T flip flop | - Insert 74112 (JK flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply various logic inputs to Record & verify the output result against each given input. |

Knowledge & Understanding:

- Define Latch

- How many inputs are given to SR Latch?

- Which IC is used for NAND and NOR gate?

- Explain clocked RS flip flop.

- Explain difference between Latch and flip flop.

- Symbols for Combinational Elements (Symbols for Flip-Flops.)

- Define D flip flop.

- Define clock pulse.

- Explain difference between synchronous & asynchronous input.

- Define JK/T flip flop.

- Draw the symbol of JK flip flop

- Which IC is used for JK flip flop?

Tools & Equipment:

- NOR gate (7402 IC)

- NAND gate (7400 IC)

- Digital clock

- Bread board

- LED

- DC supply (5 V)

- Connecting leads

- D Type Flip Flop (7474 Dual IC)

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard.

- Construction of JK Flip Flop with help of logic gate circuits.

- Construction of D Flip Flop with help of logic gate circuits.

### Construct Multivibrator with 555 IC

Overview: This competency standard covers the skills and knowledge required to.  Construct 555 IC as Actable Multivibrator, Construct 555 IC as Mono-stable Multivibrator, Construct 555 IC as Bi-stable Multivibrator and verify its set and reset conduction. Construct 555 IC as A-stable, mono-stable &bi-stable Multivibrator and observe their outputs.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Construct 555 IC as Actable Multivibrator | - Draw circuit diagram for Actable Multivibrator - Place 555 IC on bread board / trainer - Make connection as per diagram. - Apply voltage to circuit. - Recode the output signal wave shape from oscilloscope. |
| CU2. Construct 555 IC as Mono-stable Multivibrator | - Draw circuit diagram for Mono-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. |
| CU3. Construct 555 IC as Bi-stable Multivibrator and verify its set and reset conduction | - Draw circuit diagram for Bi-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. |

Knowledge & Understanding:

- Describe basic elements of 555 timer IC.

- Name pins of 555 timer IC.

- Explain function of voltage control input.

- Explain A stable Multivibrator.

- Explain mono-stable Multivibrator.

- Explain bi-stable Multivibrator.

Tools & Equipment:

- 555 Timer IC

- Dual trace Oscilloscope 0-20MHZ

- Resistors 10 KΩ

- Capacitor 0.1µF

- Capacitor 0.01µF

- Bread board

- DC supply (5 V)

- Connecting leads

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard.

- Construct of A-stable circuit.

- Construct a Mono-stable circuit.

- Construct a Bi-stable.

### Construct Shift Registers and Counters with the Help of Flip Flops

Overview: This competency standard covers the skills and knowledge required to Construct a 4-bit shift register by Using Flip Flops, Construct a 4-bit binary counter Using Flip Flops, Construct 4-bit synchronous Counter with D flip-Flops and  Repair & Troubleshoot combinational logic circuits

| Critical Evidence | Performance Criteria |
| --- | --- |
| - Construct a 4 bit shift register by Using Flip Flops | - Draw circuit diagram 4-bit register. - Make connection of D-Flip Flop as per diagram to construct 4-bit shift register. - Apply data at the input of register and give clock pulse - Recode the output according to the input. |
| - Construct a4-bit binary counter Using Flip Flops | - Draw circuit diagram counter. - Make connection of JK-Flip Flop as per diagram to construct 4-bit binary counter. - Connect LEDs to the outputs pins. - Apply the clock pulse and record the output. |
| - Construct 4-bitsynchronous Counter with D flip-Flops | P1. Draw circuit diagram synchronous counter. P2. Make connection of JK-Flip Flop as per diagram to construct 4-bit synchronous counter. P3. Connect LEDs to the output pins. P4. Apply the clock pulse and record the output. |
| - Repair & Troubleshoot combinational logic circuits | - Identify principles and operations of types of logic gates - Locate drawing and diagrams. - Check the grounding system in electronic equipment. - Locate external and internal digital IC faults |

Knowledge & Understanding:

- Symbols for Combinational Elements (Symbols for Registers, Counters, RAM.)

- Understand the function of registers, and counters in digital circuits

- Knowledge of basic computer memories and its types.

- Explain precautions when handling components.

- Demonstrate proper use of ESD equipment.

- Explain how to distinguish hardware from software issues

- Show how pulsars are used for digital signal tracing and how logic probes are used to verify states in digital equipment.

- Software Simulator and IDE’s.

Tools & Equipment:

- DLD trainer

- IC’s and Components

- Multimeter

- Logic tester

Critical Evidence(s) Required

The candidates need to produce following Critical Evidence(s) in order to be competent in this competency standard.

- Construct shift register.

- Construct binary counter.

## Sensors and Actuators

### Test Sensors and Transducers

Overview:This competency standard covers the skills and knowledge required to Identify Various types of Sensors, Identify Various types of Transducers, Identify Mode of Various Sensors and Transducers, Operate Oscilloscope for testing sensors and transducers, Test sensor / transducer with Oscilloscope, Test Temperature   Sensor and Test the IR sensor

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Identify Various types of Sensors | P1. Identify temperature sensors. P2. Identify sound sensors. P3. Identify proximity sensors. P4. Identify pressure sensors. P5. Identify light sensors. P6. Identify position sensors. P7. Identify voltage sensors. P8. Identify current sensors. P0714E&A Identify the vision sensors. P11. Identigy infrared (IR) sensors. P12. Identify power requirement for each sensor.shift down in last |
| CU2. Identify Various types of Transducers | P1. Identify Temperature transducers P2. Identify Displacement transducers P3. Identify Motion transducers (Speed, Velocity, Acceleration) P4. Identify Force/ Torque transducer P5. Identify Strain transducer P6. Identify Vibration transducer P7. Identify Sound transducer P8. Identify Flow rate transducer P0714E&A Identify Flow transducer P10. Identify Level transducer P11. Identify Pressure transducer P12. Identify Light transducer P13. Identify Humidity transducer P14. Identify Power requirement for each transducer |
| CU3. Identify Mode of Various Sensors and Transducers | P1. Identify energy mode (Active / Passive) P2. Identify Signal mode (Analog /Digital) P3. Identify contact / non-contact mode P4. Identify Null / deflection mode |
| CU4. Operate Oscilloscope for testing sensors and transducers | P1. Identify components of oscilloscope, i.e. Display, power supply, etc P2. Identify Basic controls and functions of oscilloscope P3. Measure voltage and frequency of signal. P4. Identify signals in oscilloscope P5. Measure time dependent electrical signals from sensors and transducers. |
| CU5. Test sensor / transducer with Oscilloscope | P1. Identify the pins (Power, Signal) of sensor/transducer P2. Apply required Power to transducer/sensor power pins. P3. Connect signal pin with oscilloscope. P4. Write down the obtained data. P5. Compare the data with transducer/sensor datasheet. |
| CU6. Test Temperature Sensor | P1. Open Arduino IDE. P2. Connect the Arduino board to PC using USB cable. P3. Program the Arduino board with given code P4. Connect the LM35 sensor to output pin of Arduino P5. Power ON the oscilloscope P6. Connect the Channel 1 probe of the oscilloscope to output pin of Arduino P7. Acquire the signal(s) from circuit on oscilloscope Measure the RMS voltage at different temperatures (25, 26, 27, 28, 29, 30, 31, 32). P8. Tabulate the Temperature versus measured RMS voltage. P0714E&A Plot to verify linear relationship between Temperature and RMS voltage. |
| CU7. Test the IR sensor | P1. Open Arduino IDE. P2. Connect the Arduino board to PC using USB cable. P3. Program the Arduino board with given code P4. Connect the IR sensor to output pin of Arduino P5. Power ON the oscilloscope P5. Connect the Channel 1 probe of the oscilloscope to output pin of Arduino P6. Acquire the signal(s) from circuit on oscilloscope P7. Keep a white body at a distance of 5 cm from the sensor. P8 Keep moving the object away from the sensor by 1cm and note the MAXIMUM measured voltage P0714E&A Vary the distance between object and IR sensor (5 to 40 cms with the step of 2 cms) and measure the voltage P10. Tabulate the distance versus measured MAXIMUM voltage. P11. Plot the voltage V/S distance graph and check its validation with the datasheet and sensor specification. |

Knowledge and understanding

- Describe the Transducer and its types

- Describe the Sensor and its types

- Describe the Range, Span, error, resolution, linearity of sensor

- Describe solenoids as actuator

- Describe torque of motors.

- Describe contactors, relays and timers.

- Describe phototransistor (converts light energy into electric energy)

- Describe signal conditioning method of phototransistor

- Describe RTD and Thermocouple

- Describe signal conditioning circuits of RTD and Thermocouple

- Describe LVDT transducer

- Describe Tacho-generator Transducer

- Describe Photoelectric Speed transducer

- Describe LM35 temperature sensors,

- Describe PIR Motion detector module

- Describe Proximity Sensor (Magnetic, Inductive, Capacitive, Optical)

- Describe the Program languages

- Describe the syntax and semantics.

- Differentiate Programs and Algorithms.

- Oscilloscope functions and control

Equipment and Tools

- Analog and Digital Oscilloscope

- Arduino Development board.

- Voltage probe (provided with oscilloscope) / BNC cables.

- Jumpers / wires.

- LM35 Temperature Sensor.

- Sharp IR proximity sensor

- Proximity Sensor (Inductive)

Critical Evidence(s) Required

- The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

  - Operation of oscilloscope

  - Testing of sensors and transducers

### Calibrate Sensors and Transducers

Overview: This competency standard covers the skills and knowledge required to Calibrate Capacitive Transducer for Displacement measurement, Calibrate Linear Variable Differential Transformer (LVDT) Transducer for Displacement measurement, Calibrate Resistance Temperature Detector (RTD) for Temperature measurement, Calibrate Thermocouple for Temperature Measurement, Calibrate thermistor for temperature measurements, Calibrate Pressure cell for pressure Measurements, Calibrate strain gauge for temperature measurement, Calibrate Stroboscope for Speed Measurement and Calibrate  rotometer for flow measurement

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1 Calibrate Capacitive Transducer for Displacement measurement | P1. Check connection made to the instrument P2. Allow the instrument in ON position for 10 minutes for initial warm up P3. Pull the top plate to Zero position P4. Adjust the ZERO potentiometer so that the display reads ‘000’ P5 Move the plate in step of 5 to 10 mm and note down the reading in the tabular column till 50mm. |
| CU2 Calibrate LVDT Transducer for Displacement measurement | P1. Connect the power supply chord at the rear panel to the 230V 50Hz supply. P2. Switch on the instrument by pressing down the toggle switch. The display glows to indicate the instrument is ON. P3. Allow the instrument in ON position for 10 minutes for initial warm-up. P4. Rotate the micrometer untill it shows “20.0”. P5. Adjust the CAL potentiometer at the front panel so that the display reads “10.0” P6. Rotate the core of micrometer untill the micrometer shows reads “10.0” and adjust the ZERO potentiometer till the display reads “00.0”. P7. Rotate back the micrometer core upto 20.0 and adjust once again CAL Potentiometer till the display read (Now the instrument is calibrated for +/- 10.0 mm range. As the core of LVDT moves the display reads the displacement in mm.) P8. Rotate the core of the micrometer in steps of 1 or 2 mm and tabulate the readings. The micrometer will show the exact displacement given to the LVDT core the display will read the displacement sensed by the LVDT. P0714E&A Tabulate the readings and Plot the graph Actual V/s indicator readings. |
| CU3. Calibrate RTD for Temperature measurement | P1. Check connection made and Switch ON the instrument by rocker switch at the front panel (The display glows to indicate the instrument is ON.) P2. Allow the instrument in ON Position for 10 minutes for initial warm-up. P3. Pore around 3/4th full of water to the kettle and place sensors and thermometer inside the kettle. P4. Note down the Initial water temperature from the thermometer. P5. Select the sensor on which the experiment to be conducted through selection switch on the front panel. P6. Adjust the Initial set Potentiometer in the front panel till the display shows / reads initial water temperature. P7. Switch the power and wait untill the water boils, note down the reading in the thermometer and set Final set potentiometer ?? till the display reads boiling water temperature. P8. Remove the sensor from the boiling water immerse in it I ?? the cold water. Set the cold water temperature using initial set potentiometer. Repeat the process till the display reads exact boiling water and cold water temperature. P0714E&A Change the water in the kettle with and re heat the water. Now the display starts showing exact temperature raise in the kettle. P10 Experiment can be repeated for all the three sensors. P11. Temperature in the thermometer and the indicator readings in steps of 10o C can ??be tabulated |
| CU4. Calibrate Thermocouple for Temperature Measurement | P1. Check connection made and Switch ON the instrument by rocker switch ?? at the front panel. P2. The display glows to indicate the instrument is ON. P3. Allow the instrument in ON Position for 10 minutes for initial warm-up. P4. Pore around 3/4th full of water to the kettle and place sensors and thermometer inside the kettle. P5. Note down the Initial water temperature from the thermometer. P6. Select the sensor on which the experiment to be conducted through selection switch on the front panel. P7. Adjust the Initial set Potentiometer in the front panel till the display reads initial water temperature. P8. Switch on the kept and wait till the water boils note down the reading in the thermometer and set Final set potentiometer till the display reads boiling water temperature. P0714E&A Remove the sensor from the boiling water immerse it ??I the cold water. Repeat the corrections as above P10. Set the cold-water temperature using initial set potentiometer. P11. Repeat the process till the display reads exact boiling water and cold-water temperature. P12. Change the water in the kettle with and re heat the water. Now the display starts showing exact temperature raise in the kettle. P13. Experiment can be repeated for all the three sensors. P14. Temperature in the thermometer and the indicator readings in steps of 100 C can be tabulated. |
| CU4. Calibrate thermistor for temperature measurements | P1. Check connection made and Switch ON the instrument by rocker switch at the front panel. Repeat the corrections as above P2. The display glows to indicate the instrument is ON. P3. Allow the instrument in ON Position for 10 minutes for initial warm-up. P4. Pore around 3/4th full of water to the kettle and place sensors and thermometer inside the kettle. P5. Note down the Initial water temperature from the thermometer. P6. Select the sensor on which the experiment to be conducted through selection switch on the front panel. P7. Adjust the Initial set Potentiometer in the front panel till the display reads initial water temperature. P8. Switch on the kept and wait till the water boils note down the reading in the thermometer and set Final set potentiometer till the display reads boiling water temperature. P0714E&A Remove the sensor from the boiling water immerse it I the cold water. Set the cold-water temperature using initial set potentiometer. P10. Repeat the process till the display reads exact boiling water and cold-water temperature. Change the water in the kettle with and re heat the water. Now the display starts showing exact temperature raise in the kettle. P11. Experiment can be repeated for all the three sensors. Temperature in the thermometer and the indicator readings in steps of 10o C can be tabulated. |
| CU5. Calibrate Pressure cell for pressure Measurements | P1. Check connection made and switch ON the instrument by rocker switch at the front panel. The display glows to indicate the instrument is ON. Repeat the corrections as above P2. Allow the instrument in ON Position for 10 minutes for initial warm-up. P3. Adjust the Potentiometer in the front panel till the display reads “000” P4. Apply pressure on the sensor using the loading arrangement provided. P5. The instrument reads the pressure coming on the sensor and display through LED. P6. Readings the data. P7. Tabulate the data and % error of the instrument, linearity can be calculated. |
| CU6. Calibrate strain gauge for temperature measurement. | P1. Check connection made and Switch ON the instrument by toggle switch at the back of the box. Repeat the corrections as above P2. The display glows to indicate the instrument is ON. P3. Allow the instrument in ON Position for 10 minutes for initial warm-up. P4. Adjust the ZERO Potentiometer on the panel till the display roads ‘ OOP’. P5. Apply load on the sensor using the loading arrangement provided in steps of 100g upto 1 Kg. P6. The instrument display exact micro-strain strained by the cantilever beam. P7. Note down the readings in the tabular column. Percentage error in the readings. P8. Hysteresic and Accuracy of the instrument can be calculated by comparing with the theoretical values. |
| CU7. Calibrate Stroboscope for Speed Measurement | P1. Check the connections of the equipment. Repeat the corrections as above P2. Select the mode of operation on Xenon flash lamp (Hi/lo). P3. Make a mark on the rotating wheel. P4. Now switch on the Xenon flash lamp and adjust that the light exactly projected on the rotating wheel. P5. Adjust the speed of the motor such that ONE STATIONARY mark is visible on the wheel. Note down the reading in the tabular column. P6. Now adjust the sped of the motor such that Two Stationary marks are visible and note the readings. P7. Similarly for Three and more marks and note the readings. |
| CU8. Calibrate rotometer for flow measurement | P1. Connect the turbine flow sensor with indicator marked as flow sensor input. Repeat the corrections as above P2. Connect the two pin of the motor to the instrument. P3. Vary the flow control potentiometer to any required set level. P4. Compare the Rotameter reading and digital reading with set reading. P5. Take reading for different set of flows rate. P6. Plot the graph of Rotameter Reading with Digital Indicator Reading. |

Knowledge and understanding

- Describe the calibration of Transducer and Sensor

- Describe the Calibration Curves.

- Describe validation of results

- Describe Noise.

- Describe Hysteresis.

- Describe Offset

- Describe Gain

- Describe Linearization

- Describe unit scaling

- Describe Errors in Sensor Measurement.

- Describe Calibration Process

- Describe Error due to Improper Zero Reference

- Describe Error due to Mechanical Wear or Damage

Equipment and Tools

        - Oscilloscope

        - Arduino Development board.

        - Voltage probe (provided with oscilloscope) / BNC cables.

        - Jumpers / wires.

        - LM35 Temperature Sensor.

        - Sharp IR proximity sensor

        - Proximity Sensor (Inductive)

        - Strain gauge

        - Stroboscope

        - Rotometer

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

  - Calibration of sensor and transducer

### Interface the Sensor & Transducers

Overview: This competency standard covers the skills and knowledge required to Interface LVDT with microcontroller for sensing linear displacement, Interface temperature transducer with microcontroller for sensing temperature, Interface rotary flow transducer with microcontroller for sensing flow rate, nterface Proximity sensor to sense the speed of DC Motor and Interface MQ135 Gas Sensor with Arduino

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Interface LVDT with microcontroller for sensing linear displacement | P1. Open Arduino IDE. P2. Connect the Arduino board to PC using USB cable. P3. Program the Arduino board with given code P4. Connect the LVDT to Arduino P5. Acquire the data (voltages) form LVDT due to change in resistance. P5. Convert the data (voltages) to linear displacement by equation. |
| CU2. Interface temperature transducer with microcontroller for sensing temperature. | P1. Open Arduino IDE. P2. Connect the Arduino board to PC using USB cable. P3. Program the Arduino board with given code P4. Connect the temperature transducer (LM35) to Arduino P5. Acquire the data (voltages) form LM35 due to change in resistance. P5. Convert the data (voltages) to temperature by equation. |
| CU3. Interface rotary flow transducer with microcontroller for sensing flow rate | P1. Open Arduino IDE. P2. Connect the Arduino board to PC using USB cable. P3. Program the Arduino board with given code P4. Connect the rotary flow transducer to Arduino P5. Acquire the data (rpm) form rotary flow transducer due to flow. P5. Convert the data (rpm) to linear displacement by equation. |
| CU4. Interface Proximity sensor to sense the speed of DC Motor | P1. Open Arduino IDE. P2. Connect the Arduino board to PC using USB cable. P3. Program the Arduino board with given code P4. Connect the DC Motor to Arduino P5. Attach a rotating (hollow) wheel to DC Motor shaft. P5. Connect the inductive proximity sensor to Arduino P5. Attach proximity sensor near rotating wheel of DC motor P6. Acquire the data form sensor |
| CU5. Interface MQ135 Gas Sensor with Arduino | P1. Identify pin configurations P2. Open Arduino IDE software. P3. Connect MQ135 gas sensor to desired Pin. P4. Make a source code / sketch to sense the gas. P5. Compile and Debug P6. Run Program. P7. Acquire the data form sensor P8. Assign a task on sensor data. |

Knowledge and understanding

- Describe the Transducer

- Describe the Sensor

- Describe the Range, Span, error, resolution, linearity  of sensor

- Describe solenoids as actuator

- Describe torque of motors.

- Describe contactors, ralays and timers.

- Describe phototransistor (converts light energy into electric energy)

- Describe signal conditioning method of phototransistor

- Describe RTD and Thermocouple

- Describe signal conditioning circuits of RTD and Thermocouple

- Describe LVDT transducer

- Describe Tacho-generator Transducer

- Describe Photoelectric Speed transducer

- Describe LM35 temperature sensors,

- Describe PIR Motion detector module

- Describe Proximity Sensor (Magnetic, Inductive, Capacitive, Optical)

- Describe the Program languages

- Describe the syntax and semantics.

- Differentiate Programs and Algorithms.

Equipment and Tools

- Oscilloscope

- Arduino Development board.

- Voltage probe (provided with oscilloscope) / BNC cables.

- Jumpers / wires.

- LM35 Temperature Sensor.

- Sharp IR proximity sensor

- Proximity Sensor (Inductive)

- LVDT

- MQ135 Gas Sensor

- Flow sensor

- DC Motor

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

  - Interfacing of sensor and transducer.

  - Data acquisition

### Test Actuators

Overview: This competency standard covers the skills and knowledge required to Identify various types of Actuators, Leak test for Pneumatic actuators/tubes, Leak test Hydraulic actuators/tubes, Force/Power test and Piezoelectric actuator test

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Identify various types of Actuators | P1. Identify Mechanical Actuators (Spring, Gear) P2. Identify Electrical Actuators P3. Identify Hydraulic Actuators P4. Identify Pneumatic Actuators. P5. Identify Electromechanical Actuators P6. Identify Electromagnetic Actuators P7. Identify Manual Actuators P8. Identify power sources of actuators |
| CU2. Leak test for Pneumatic actuators/tubes | P1. Select test type. (Bubble leak test, Chemical Reactivity test, Pressure drop) P2. Apply high pressure in actuator/tubes for bubble leak test. P3. Make soap water P4. Apply soap water on joints and seals. P5. Mark the places of bubbles. P6. Make a report and suggest maintenance or repairing. |
| CU3. Leak test Hydraulic actuators/tubes | P1. Select test type. (Bubble leak test, Chemical Reactivity test, Pressure drop) P2. Attach pressure gauges to actuator for pressure drop test. P3. Apply high pressure in actuator/tubes and seal it. P4. Note down the pressure and time. P5. Note down the pressure after each 5 minutes. P6. Make a report and suggest maintenance or repairing. |
| CU4. Force/Power test | P1. Select actuator type. (Pneumatic, Hydraulic) P2. Attach a load cell with actuator as given. P3. Apply controlled pressure in actuator. P4. Note down the reading on load cell. P5. Increase the pressure step by step of 5 psi and take data. P6. Tabulate the data. |
| CU5. Piezoelectric actuator test | P1. Select a piezoelectric actuator. P2. Identify power range for piezoelectric actuator. P3. Select displacement sensor. P4. Install displacement sensor. P5. Apply the voltage of step 0.1 volts from 0 to maximum range. P6. Take the results from displacement sensor at step of 0.1 volts P7. Tabulate the data. |

Knowledge and understanding

- Describe the Actuators types.

- Describe the Pneumatic actuators

- Describe the Hydraulic actuators

- Describe the types of leak tests (Bubble leak test, Chemical Reactivity test, Pressure drop)

- Describe solenoids as actuator

- Describe piezoelectric actuators.

- Describe torque of motors.

- Describe contactors, relays and timers.

- Describe the Pneumatic actuators components.

- Describe the Hydraulic actuators components.

Equipment and Tools

- Air Compressor

- Pneumatic Actuators.

- Pneumatic valves and accessories.

- Pressure gauges

- Hydraulic pump

- Hydraulic Actuators.

- Hydraulic valves and accessories.

- Load cell

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

  - Testing of pneumatic and hydraulic actuators and lines.

### Interface and control Actuator with sensors

Overview: This competency standard covers the skills and knowledge required to Identify motion of Actuators, Control extension length of pneumatic actuator, Control the room temperature with relay (actuator) and Control angle of Servomotor with potentiometer.

| Critical Evidence | Performance Criteria |
| --- | --- |
| CU1. Identify motion of Actuators | P1. Identify Linear Actuators P2. Identify Rotary Actuators P3. Identify Binary Actuators P4. Identify Continues Actuators |
| CU2. Control extension length of pneumatic actuator | P1. Select pneumatic actuator P2. Select displacement sensor P3. Select a solenoid valve with relay to actuate pneumatic cylinder P4. Select controller (Arduino) P5. Make a wiring diagram for connection of sensor, relay and controller. P6. Open Arduino IDE. P7. Connect the Arduino board to PC using USB cable. P8. Program the Arduino board to ON/OFF the relay for minimum and maximum extension of actuating rod. P0714E&A Perform reciprocating motion of rod. |
| CU3. Control the room temperature with relay (actuator) | P1. Select temperature sensor P2. Select heater or air conditioner P3. Select a relay (actuator) for operating heater or air conditioner P4. Select controller (Arduino). P5. Make a wiring diagram for connection of heater or air conditioner, sensor, relay and controller. P6. Open Arduino IDE. P7. Connect the Arduino board to PC using USB cable. P8. Program the Arduino board to ON/OFF the relay to maintain temperature. |
| CU4. Control angle of Servomotor with potentiometer. | P1. Identify potentiometer (manual actuator) to detect the resistance/angle. P2. Identify servomotor (rotary actuator) to position the system. P3. Identify the Input and Output pins. P4. Identify the Digital, Analog and PMW pins. P5. Make a layout and connect components P6. Write, compile, debug and upload a program to microcontroller. P7. RUN and Get results (Angle depends on Potentiometer) |

Knowledge and understanding

- Describe the types of actuator motion.

- Describe the Pressure force relationship

- Describe the Control of system

- Describe open loop systems

- Describe close loop systems.

- Describe potentiometer as manual rotary actuator.

- Describe relay as binary actuator.

- Describe DC motor as rotary actuator

- Describe Servo motor as rotary actuator

- Describe pneumatic actuator as linear actuator

Equipment and Tools

- Oscilloscope

- Arduino Development board.

- Voltage probe (provided with oscilloscope) / BNC cables.

- Jumpers / wires.

- LM35 Temperature Sensor.

- Sharp IR proximity sensor

- Proximity Sensor (Inductive)

- LVDT

- Servomotor

- Pneumatic actuator

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) in order to be competent in this competency standard:

  - Control of a system