National Vocational Diploma level – 5 in Robotics Technology

(Robototics Supervisor)

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

1.	Introduction	4

2.	Purpose of the Qualification	5

3.	Date of Validation	5

4.	Date of Review	5

5.	Codes of Qualifications	6

6.	Members of Qualification Development Committee	7

7.	Members of Qualification Validation Committee	8

8.	Entry Requirements	9

9.	Regulations for the Qualification and Schedule of Units	9

10.	Mapping of the Qualification	9

11.	Common Courses with respective levels	10

12.	Packaging of Qualifications (with respect to occupations)	10

13.	Summary of Competency Standards	12

14.	Detail of Competency Standards	15

A.	Robotic Maintenance Technician	15

0714E&A1.	Perform Robot Calibration	15

0714E&A2.	Inspect Industrial sensors and actuators	17

0714E&A3.	Test/Calibrate sensors, Stepper motor& Servo Motor	20

0714E&A4.	Ensure Product Quality as per standards	22

0714E&A5.	Inspect the hazardous area for installation	24

B.	Machine & Mechanism	26

0714E&A83.	Demonstrate Simple Mechanisms	26

0714E&A84.	Perform Kinematic Analysis	28

0714E&A85.	Demonstrate Mechanical Power Transmission Elements	30

0714E&A86.	Perform Kinematic Analysis of Mechanical power Transmission Elements	31

C.	Robotic Programing	32

0714E&A82: Advanced Programming-I	32

0714E&A82-A: Advanced Programming-II	35

D.	Robotic System Evaluation	38

0714E&A87.	Perform Robot Installation	38

0714E&A88.	Perform Diagnose precise robot movement	40

0714E&A89.	Perform Diagnose Coupling Parameters	42

0714E&A90.	Implement the performance characteristics identified in ISO9283 standards	44

E.	Robotic Application	46

0714E&A91.	Develop Line Following Robot	46

0714E&A92.	Develop Smart phone control robot using Bluetooth	48

0714E&A93.	Develop IoT based home automation	50

0714E&A94.	Develop Smart Phone Control Door Lock System	52

0714E&A95.	Develop Security System Using Motion Detector	54

0714E&A96.	Develop Water-level Detection in Tank	56

0714E&A97.	Develop Mini CNC 2D Plotter	58

0714E&A98.	Develop Voice Controlled Robot	60

F.	Industrial Electronics	62

0714E&A99.	Implement diode and Thyristor in power control application.	62

0714E&A100.	Construct single phase and three phase Inverter	65

0714E&A101.	Control speed of DC/AC Motor	67

G.	Hydraulic & Pneumatic Machinery	70

0714E&A102.	Operate Hydraulic Bench and Its Functions	70

0714E&A103.	Analyse the performance of Pumps	72

0714E&A104.	Working/operation directional control valves	74

0714E&A105.	Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder	77

0714E&A106.	Actuate Double Acting Hydraulic Cylinder	79

0714E&A107.	Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking	81

0714E&A108.	Set Speed and Direction of a Hydraulic Actuator	84

0714E&A109.	Perform Pressure measuring instruments operations	86

0714E&A110.	Operate Flow Regulating Devices (Valves)	89

0714E&A111.	Maintain Pneumatic System	92

H.	Develop Entrepreneur Skills	94

0714E&A112.	Investigate micro business opportunities	94

0714E&A113.	Develop and review a business plan.	97

0714E&A114.	Organise finances for the micro business	100

0714E&A115.	Manage human resources	102

0714E&A116.	Market products and services	105

0714E&A117.	Monitor and review business performance	109

0714E&A118.	Negotiate for resolving business issues	112

0714E&A119.	Lead small teams	114

# Introduction

This qualification gives an introduction to robotics so that the student will have the competence to supervise and implement robotic systems. This is done by presenting relevant topics from geometry, kinematics, dynamics, control systems, and Programing techniques. . Students will gain hands on experience in robotic assembling, synchronization with system, repair & maintenance along with the some fabrication and machining work.

The National Skills Strategy (NSS) aims at achieving a paradigm shift from time-bound and supply lead to competency-based and demand driven training in Pakistan. Competency-based training approach focuses on the demonstration of actual skills required in the workplace/industry. To achieve this strategy, NAVTTC in collaboration with GIZ is involved in the development process of CBT program to ensure competent skilled labor in demand driven vocational trades.

National Vocational Qualification Framework (NVQF) as designed in consultation with the stakeholders including academia, researchers, industry, chambers and TEVTAs. Its aim is to identify knowledge & skills required for a qualification, determine equivalence, provide guidelines for Recognition of Prior Learning (RPL) and assure quality of training. NVQF designed for the Vertical and horizontal progression for learners, recognition of qualification, meeting with the national and international standards, facilitate conversion of informal training to formal training through RPL, improvement in quality of training and increased options for learners for selecting training programs in different trades. Competency Standards are performance specification that identify the knowledge and competencies an individual need to succeed in the workplace.

The National Competency Standards could be used as a referral document for the development of curriculum to be used by training institutions.

# Purpose of the Qualification

This qualification will help student to supervise and implement robotic systems. The candidate will be able to perform fundamental operations of robot logic functions, systems, understand the concepts of various digital systems, their functions and designs, grasp concepts of various memory structures, register counter and their integration with various digital system and there troubleshooting. This qualification will help candidate in assembling, synchronization and construction of the robotic system, identify and implement certain robotic components, elaborate linkage between the components and their industrial workability & maintenance of various robotic devices.

This course enables the youth with greater employment opportunities and entrepreneurial skills.

# Date of Validation

The level 5 (Diploma) of National vocational qualification on “Robotics” has been validated by the Qualifications Development Committee (QDC) members on 22-24 August 2019 at Karachi and will remain valid for ten years i.e. 24 August,2029

# Date of Review

The level 5 (Diploma) of National vocational qualification on “Robotics” has been validated by the Qualifications Development Committee (QDC) members on 22-24 August 2019 at Karachi and shall be reviewed after three years from the date of validation i.e. 25 August,2022

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Robotics Technician level 5 |  |
| --- | --- |
| Code | Description |
| 0714E&A(5) | 5th Level D.A. E National Certificate of level-5, in “Robotics Technology” |

# Members of Qualification Development Committee

The following members participated in the Qualification development process.

| S# | Name | Organization |
| --- | --- | --- |
| 1 | Prof. Dr. Zeeshan Ali Memon (Chairman Qualification Development Committee) | Head of Industrial Engg. Deptt. Dawood University of Engg. & Tech. |
| 2 | Engr. Nimra Ghazal | Dawood University of Engg. & Tech. |
| 3 | Engr. Javed Larik | Mehran University of Engg. & Tech. |
| 4 | Engr. Muhammad Ismail | Dawood University of Engg. & Tech. |
| 5 | Dr. Abid Khan | APC&IC Lab, PCSIR |
| 6 | Mr. Muhammad Aasim | CASE School of Robotics. |
| 7 | Mr. Usama Khan | PCI, Automotive Pvt. Ltd. |
| 8 | Mr. Babar Sayeed | PSTC, PCSIR |
| 9 | Engr. Dr. Rana Javaid Masood | Usman Institute of Technology |
| 10 | Engr. Aurangzaib Ansari | Usman Institute of Technology |
| 11 | Engr. Talha Tariq | Dawood University of Engg. & Tech. |
| 12 | Dr. Arsalan Ansari | Head of Electronics Dawood University of Engg. & Tech. |
| 13 | Mr. Liaqat Ali Jamro | Director Academics, STEVTA |
| 14 | Mr. Shaikh Asim Qamar | Dacum Facilitator |
| 15 | Mr. Yasir | Assistant Director, NAVTTC |

# Members of Qualification Validation Committee

The following members participated in the Qualification development process.

| S# | Name | Organization |
| --- | --- | --- |
| 1 | Dr. Zeeshan Ali Memon (Chairman Qualification Development Committee) | Head of Industrial Engg. Deptt. Dawood University of Engg. & Tech. |
| 2 | Engr. Nimra Ghazal | Research Associate, Dawood University of Engg. & Tech. |
| 3 | Dr. Asif | Associate Professor, Ziauddin University Karachi |
| 4 | Mr. Liaqat Ali Jamro | Director Academics, STEVTA |
|  | Dr. Saifullah Samo | Assistant Professor, Mechatronics Department MUET |
| 5 | Mr. Ayaz Khan | Lecturer, Govt. Advance Technical Training Centre, KP(Tevta) |
| 6 | Mr. Syed Mansoor Ahmed | Assistant IT Manager, SBTE Karachi |
| 7 | Engr. Mustafa Mohiuddin | Lab Engineer, Dawood University of Engg. & Tech. |
| 8 | Mr. Muhammad Usman | Incharge & Instructor Mechatronics, GCT Taxila(P-TEVTA) |
| 9 | Engr. Aijaz Ahmed Zia | D&A Engineer/INTECH |
| 10 | Mr. Yasir | Assistant Director, NAVTTC/ DACUM Expert |

# Entry Requirements

The entry for National Vocational Certificate level 5, in “Robotics” is given below:

| Title | Entry Requirements |
| --- | --- |
| National Vocational Certificate level 5 | Entry into training institute for this qualification is candidate having Middle Level (Science) and or National Vocational Certificate level 4. |

# Regulations for the Qualification and Schedule of Units

Not Applicable.

# Mapping of the Qualification

# Common Courses with respective levels

LEVEL 5

# Packaging of Qualifications (with respect to occupations)

The National Vocational Qualifications have been packaged as detailed below:

Level 5

(Robotic system supervisor, Robot service technician, Associate Robot programmer)

Robotic system supervisor

- Assemble various robotic parts

- Perform Robot Calibration

- Inspect Industrial sensors and actuators

- Test/Calibrate sensors, Stepper motor& Servo Motor

- Perform Robot Installation and Maintenance Principles

- Perform Diagnose precise robot movement

- Perform Diagnose Coupling Parameters

- Robotics control Board

- Develop robotic arm structure

- Configure and test robotic arm

- Develop Object recognition system

- Ensure Product Quality as per standards

- Develo

- p Line Following Robot

- Develop smart phone control robot using Bluetooth

- Develop IoT Controlled i based home automation system

- Develop Smart Phone Control Door Lock System

- Develop Security System Using Motion Detection

- Develop  Water-level Detection in Tank

- Develop Mini CNC 2D Plotter

- Develop Voice Controlled Robot

- Implement the performance characteristics identified in ISO9283 standards

Robot service technician

- Assemble various robotic parts

- Perform Robot Calibration

- Inspect Industrial sensors and actuators

- Test/Calibrate sensors, Stepper motor& Servo Motor

- Perform Robot Installation and Maintenance Principles

- Perform Diagnose precisie robot movement

- Perform Diagnose Coupling Parameters

Associate Robot programmer

- Basic Programming skills

- Object Oriented Programming

- Advance Programming

- Develop Robot program

# Summary of Competency Standards

The National Vocational Qualifications have been packaged as detailed below:

| Level 5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Robotic Maintenance Technician | Level |  | Th. | Pr. | Total | Cr.Hrs |
| Perform Robot Calibration | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Inspect Industrial sensors and actuators | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Test/Calibrate sensors, Stepper motor& Servo Motor | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Ensure Product Quality as per standards | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Inspect the hazardous area for installation | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Machine & Mechanisms |  |  |  |  |  |  |
| Demonstrate Simple Mechanisms | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Perform Kinematic Analysis | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Demonstrate Mechanical Power Transmission Elements | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Perform Kinematic Analysis of Mechanical power Transmission Elements | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Robotic Programming |  |  |  |  |  |  |
| Advanced Programming-I | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Advanced Programming-II | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Robotic System Evaluation |  |  |  |  |  |  |
| Perform Robot Installation and Maintenance Principles | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Perform Diagnose precisie robot movement | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Perform Diagnose Coupling Parameters | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Implement the performance characteristics identified in ISO9283 standards | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Robotic Application |  |  |  |  |  |  |
| Develop Line Following Robot | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Develop smart phone control robot using Bluetooth | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Develop IoT Controlled i based home automation system | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Develop Smart Phone Control Door Lock System | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Develop Security System Using Motion Detection | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Develop Water-level Detection in Tank | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Develop Mini CNC 2D Plotter | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Develop Voice Controlled Robot | Level 5 | Technical | 10 | 16 | 26 | 2.6 |
| Industrial Electronics |  |  |  |  |  |  |
| Implement diode and Thyristor in power control application. | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Construct single phase and Three phase Inverter | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Control speed of DC/AC Motor | Level 5 | Technical | 10 | 15 | 25 | 2.5 |
| Hydraulic & Pneumatic Machinery |  |  |  |  |  |  |
| Operate Hydraulic Bench and Its Functions | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Analyze the performance of Pumps | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Working/operation directional control valves | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Actuate Double Acting Hydraulic Cylinder | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Construct Circuit for Double Acting Hydraulic Cylinder | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Set Speed and Direction of a Hydraulic Actuators | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Perform Pressure measuring instruments operations | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Operate Flow Regulating Devices (Valves) | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Maintain Pneumatic System | Level 5 | Technical | 10 | 22 | 32 | 3.2 |
| Develop Entrepreneur Skills |  |  |  |  |  |  |
| Investigate Microbusiness Opportunities | Level 5 | Generic | 8 | 18 | 26 | 2.6 |
| Develop And Review A Business Plan | Level 5 | Generic | 8 | 18 | 26 | 2.6 |
| Organize Finances For The Micro Business | Level 5 | Generic | 8 | 18 | 26 | 2.6 |
| Manage Human Resources | Level 5 | Generic | 8 | 18 | 26 | 2.6 |
| Market Products And Services | Level 5 | Generic | 8 | 18 | 26 | 2.6 |
| Monitor And Review Business Performance | Level 5 | Generic | 8 | 18 | 26 | 2.6 |
| Negotiate For Resolving Business Issues | Level 5 | Generic | 8 | 18 | 26 | 2.6 |
| Lead Small Teams | Level 5 | Generic | 8 | 18 | 26 | 2.6 |
| Level 5 total |  |  | 424 | 771 | 1195 | 120 |

# Detail of Competency Standards

### Robotic Maintenance Technician

# Perform Robot Calibration

Overview:

*Level-1 calibration only models differences between actual and reported joint displacement values, (also known as mastering). Level-2 calibration, also known as kinematic calibration, concerns the entire geometric robot calibration which includes angle offsets and joint lengths. Level-3 calibration, also called a non-kinematic calibration, models errors other than geometric defaults such as stiffness, joint compliance and friction

This competency standard covers the skills and knowledge related to Calibration in measurement technology and metrology is the comparison of measurement values delivered by a device under test with those of a calibration standard of known accuracy. The term calibration means just the act of comparison, and does not include any subsequent adjustment.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Perform level-I calibration(Mastering) | - Read manual of level one calibration - Perform zero error calibration of all parts - Inspect arm according to manual. - Calibrate arm according to instruction. |
| - CU2. Perform level-II calibration(Kinematics) | - Read manual of level two calibration - Perform linear/angular sensors calibration. - Inspect arm according to manual. - Calibrate arm according to instruction of kinematics as given. |
| - CU3. Perform level-III calibration(Non-Kinematics) | - Follow OHS standards. - Read manual of level three calibration - Inspect arm according to manual. - Calibrate arm according to non-kinematics instruction as given. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand the maintenance terminology

- Understand the calibration standards measuring devices

- Understand the operational principle of adjustment hardware components

- Understand and explain the standard operational procedure of maintenance

- Understand and explain the standard operational procedure of calibration

- Understand the functionality of mechanical parts of robots and tolerances

- Understand the understand the robot controller programming

- Understand the kinematics of Robot arm

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform Level-I/Level-II/Level-III calibration on various measuring instruments.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Optical CMM |
|  | Scanner. |
|  | Slip gauges. |
|  | Measuring tools |
|  | Coordinate calibration tools |

# Inspect Industrial sensors and actuators

Overview: This competency standard covers the skills and knowledge required to identify and interface sensors with controller.

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify various types of sensors for measuring physical quantities. | - Identify various types of pressure sensors. - Identify various types of temperature sensors. - Identify various types of optical sensors. |
| - Interface pressure sensor with controller board. | - Identify pressure sensor. - Connect jumper wires with controller board and sensor. - Design source code for measuring pressure from sensor. - Display pressure in quantifies form. |
| - Interface temperature sensor with controller board. | - Identify temperature sensor. - Connect jumper wires with controller board and sensor. - Design source code for measuring temperature from sensor. - Display temperature in quantifies form. |
| - Interface optical sensors with controller boards. | - Identify various types of optical sensors according to application. - Connect jumper wires with controller board and sensors. - Design source code for measuring optical based physical quantity. - Display optical based physical quantity in numeric form. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify various types sensors.

- Interface various types of sensors with controller board.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Controller board |
|  | Computer |
|  | Programming software. |
|  | Simulation software |
|  | Communication cable. |
|  | Power Adaptor |
|  | Pressure sensor |
|  | Temperature sensor |
|  | Optical sensors |

# Test/Calibrate sensors, Stepper motor& Servo Motor

Overview:

This competency standard covers the basic Tests related to sensors, Stepper motor& Servo Motor

| Competency Units | Performance Criteria |
| --- | --- |
| - Test/Calibrate sensors using Continuity method | - Check physical status of the optical sensors. - Disconnect the wires of sensor from the power source - Make connection as per standard/operational manual - Check the Continuity - Measure the resistance of Sensor as per SOP/operation manual - Verify the Resistance as per SOP/operational manual - Repeat these steps between the source and sensor to isolate the problem. |
| - Test/Calibrate sensors using Voltage method | - Connect the sensors with power source. - Disconnect the power wires at the sensor - Make Connection as per SOP/operational manual - Turn on the power source - Measure the DC Voltage - Verify the voltage as per SOP/operational manual |
| - Test / Calibrate Stepper motor of Actuator using four coil method | - Measure the resistance of both phases - Measuring the resistance across one wire from each of the two phases - Measure the internal resistance of similar wire - Verify the resistance as per SOP/operational manual - Perform connection of stepper motor accordingly and Check clockwise and anti clockwise motion of stepper motor - Verify the stepper motor as per SOP/operational manual |

Knowledge & Understanding

- Define voltage, resistance and its unit.

- Describe series and parallel circuit.

- Describe purpose of Multimeter

- Describe how Ohm and voltage meter is connected in the circuit.

- Define the Ohm law

- Define primary and secondary winding

- Define motor winding ratio

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Multimeter |
|  | Voltmeters |
|  | Ohmmeters |
|  | Resistors of different values. |
|  | DC Stepper Motor |
|  | Sensors (Pressure, Temperature, optical etc) |
|  | Multimeter |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Turn on the supply and note the reading of drop voltage against each resistor

- Define different type of sensors

- Define different type of actuators

- Differentiate the stepper and servo motors

# Ensure Product Quality as per standards

Overview: This competency standard covers the skills and knowledge related to product quality means to incorporate features that have a capacity to meet consumer needs (wants) and gives customer satisfaction by improving products (goods) and making them free from any deficiencies or defects.

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| - CU1. Perform periodic maintenance of robot. | - Remove Make schedule for periodic maintenance. - Perform scheduled maintenance. - Make report of every maintenance performed. |  |
| - CU2. Identify various tool set for maintenance. | - Identify tools required for specific maintenance. - Performed maintenance with suitable tools only. - Keep tools clean, arranged and ready to use. |  |
| - CU3. Perform quality control on product. | - Take random samples of manufactured product. - Conduct inspection of product. - Make report of inspection - Take suitable action to improve product quality further. |  |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define the term quality control

- Understand quality standards

- Understand inspection procedure

- Understanding of instrument for quality control

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform Quality test on product

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Maintenance schedule chart |
|  | Quality control standards document |
|  | Data managing software |
|  | Instrument for QC |
|  | Tools cleaning agents |

# Inspect the hazardous area for installation

Overview: This competency standard covers the skills and knowledge related to hazardous areas are those places where fire or explosion hazards may exist due to flammable gases, flammable liquid–produced vapours, combustible liquid–produced vapours, combustible dusts, or ignitable fibers / flying present in the air in quantities sufficient to produce explosive or ignitable mixtures. Electrical equipment that must be installed in such classified locations should be specially designed and tested to ensure it does not initiate an explosion, due to arcing contacts or high surface temperature of equipment.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify hazardous areas before robot installation | - Inspect hazards in specific environment. - Note down every type of hazards. - Classify hazards types. - Make suggestions to avoid hazards. |
| - CU2. Perform suitable actions to overcome hazards. | - Keep floor clean. - Stop unauthorized access to system. - Maintain power source with proper voltage and current. - Stop extra ordinary vibrations. - Perform proper fixing of joints. |
| - CU3. Emergency actions for unwanted situations | - Take Identify the brake release switch when a person is caught by robot - Identify the robot protective functions - Identify the message of residual risk that is warning and caution. - Classify the emergency response planning (ERP) for every possible incident or accident. - Highlight the safety measures for each activity. - Implementation of safety measures in place. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define the term safety and hazards

- Define Mechanical hazards

- Define Electrical  Hazards

- Define fire hazards

- Define Chemical Hazards

- Define environmental hazards

- Explain emergency plan for unwanted situations

- Memorize important emergency phone number

- Understand safety policy

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform the Diagnose Potential Malfunction in Robot.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Software |
|  | Complete design of the Robot |
|  | Measuring tool kit |
|  | Diagnostic tool kit |
|  | Robotic fastening tool kit |
|  | Trainer |
|  | Plant layout |
|  | Workplace design and layout |
|  | Log book |
|  | Recommended Safety instructions |
|  | Personal Protective Equipment (PPEs) |

- Machine & Mechanism

- Demonstrate Simple Mechanisms

Overview: This competency standard covers the skills and knowledge required to basic mechanisms. It includes primary and secondary joints, simple and complex links, four bar and quick return mechanisms. After completing this unit student should be competent to identify different types of simple mechanisms, Draw and interpret kinematic diagrams of simple mechanisms and perform kinematic inversion of simple mechanisms.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Identify Commonly Used Links and Joints | - Recognize pin joints. - Recognize sliding joints. - Identify cam joints. - Identify gear joints. - Differentiate simple and complex links. |
| CU2. Identify simple mechanisms | - Identify four bar mechanisms. - Identify Crank Rocker Mechanism - Identify Slider Crank Mechanism. - Identify Quick Return Mechanism. |
| CU3. Analyze simple mechanisms | - Differentiate simple and complex mechanism - Draw kinematic diagram of simple mechanisms. - Interpret kinematic diagram of simple mechanisms - Identify degrees of freedom in mechanisms |
| CU4. Perform kinematic inversion of mechanisms | - Perform Kinematic Inversion of four bar mechanisms. - Perform Kinematic Inversion of Crank Rocker Mechanism - Perform Kinematic Inversion of Slider Crank Mechanism. - Perform Kinematic Inversion of Quick Return Mechanism. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Kinematics

- Describe Constrained Motion

- Describe Machine Elements

- Define Simple mechanisms and their types

- Define Terminology of Mechanisms

- Define Kinematic Inversion

- Define Techniques of Mechanism Analysis

- Define Kinematic Diagrams

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Recognize primary and secondary joints.

- Identify different types of simple mechanism

- Identify degrees of freedom in motion.

- Interpret kinematic diagrams of simple mechanisms.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Pin Joints, Sliding Joints, Cams, Gears |
|  | Four Bar Mechanism Model |
|  | Slider Crank Mechanism Model |
|  | Crank Rocker Mechanism Model |
|  | Quick Return Mechanism Model |

- Perform Kinematic Analysis

Overview: This competency standard covers the skills and knowledge required to kinematic analysis of simple mechanisms. It includes model building and simulation of simple mechanisms in working model software and calculation of liner and angular displacements and velocities.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Develop computer models of simple mechanisms | - Build Model of Slider Crank Mechanism - Simulate model of Slider Crank Mechanism - Build Model of Gear Mechanism - Simulate Gear Mechanism |
| CU2. Perform Displacement Analysis | - Define a position of a point in a mechanism with a position vector. - Determine Liner Displacement. - Determine Angular Displacement |
| CU3. Perform Velocity Analysis | - Determine Liner Velocity of a point on a link. - Determine Angular Velocity of a point on a link. - Relate Linear and Angular velocities of a point on a link. |
| CU4. Perform kinematic inversion of mechanisms | - Perform Kinematic Inversion of four bar mechanisms. - Perform Kinematic Inversion of Crank Rocker Mechanism - Perform Kinematic Inversion of Slider Crank Mechanism. - Perform Kinematic Inversion of Quick Return Mechanism. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe CAD Modelling

- Describe Working Model Software

- Define Model Building in Working Model.

- Define Simulation of Mechanisms in Working Model

- Define Kinematic Analysis.

- Describe Gear Terminology

- Define Techniques of Mechanism Analysis

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform Model Simple Mechanism in Working Model Software.

- Perform Simulate Simple Mechanism in Working Model Software.

- Perform Calculation liner and angular displacement of a point on a link.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computers |
|  | CAD software |
|  | Working Model Software |

- Demonstrate Mechanical Power Transmission Elements

Overview: This competency standard covers the skills and knowledge covers the various methods of mechanical power transmission methods. Covering the knowledge related to belts, gears and chain drives, this unit will enable students to select appropriate power transmission method.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Belts Drives | - Identify Belt Types. - Identify Pulley Types - Understand Belt Drive Geometry. |
| CU2. Gears | - Identify gear types - Understand Spur Gear Terminology. - Understand Spur Gear Geometry. |
| CU3. Chain Drives | - Select chain types. - Understand Chain Derive Geometry |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe Mechanical power transmission fundamentals.

- Describe Types of belt drives and pulleys

- Define Belt Drives Geometry

- Describe Types of Gears

- Define Gear Geometry

- Describe Types of chain Drives

- Define Chain Drive Geometry.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify various types of gears, belts and chains.

- Explain the various terms of gear geometry.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Belts and Pulleys of Various Sizes |
|  | Various types of Gears. |
|  | Various types of Chain Drives |

- Perform Kinematic Analysis of Mechanical power Transmission Elements

Overview: This competency standard covers the skills and knowledge required for kinematic analysis of    mechanical power transmission elements. Covering the knowledge related to belts, gears and chain drives, this unit will enable students to perform the kinematic analysis of selected power transmission method

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Belts Drives | - Select belts size - Determine diameter of driver and driven pulleys. - Calculate velocity ratio for belts - Calculate belt speed. |
| CU2. Gears | - Determine rotational velocities of driver and driven gears. - Calculate Velocity Ratios Gears. |
| CU3. Chain Drives | - Determine Velocity Ratio for Chains. - Calculate Chain Speed |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe Mechanical power transmission fundamentals.

- Define Belt Drives Geometry

- Define Belt Drives Kinematics

- Define Gear Geometry

- Define Gear Kinematics

- Define Chain Drive Geometry

- Define Chain Drive Kinematics

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform calculate the Velocity Ratios for Gears, Belt and Chain Drives

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Belts and Pulleys of Various Sizes |
|  | Various types of Gears. |
|  | Various types of Chain Drives |
|  | Computers |
|  | Working Model Software |

- Robotic Programing

# 0714E&A82: Advanced Programming-I

Overview: This competency standard covers the skills and knowledge required to develop how to write a computer program. It will create the knowledge to write lists of instructions for a computer to follow and to develop the software programs, the scripts, or other sets of instructions for the computers to execute. This unit will cover the knowledge of programming language C which has variety of data types and powerful operators. Due to this, students will be able to write computer programs efficiently and easily.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Python Setup on Windows | - Download the Python 3 Installer - Run the installer - Download the python IDE - Identify Interpreter vs Script Mode |
| - CU2. Remote Connection to RPi with VNC | - Enabling VNC Server - Enabling VNC Server graphically - Enabling VNC Server at the command line - Connecting to your Raspberry Pi with VNC Viewer - Establishing a direct connection - Establishing a cloud connection - Authenticating to VNC Server - Creating a virtual desktop |
| - CU3. Raspberry Pi Raspbian OS Setup | - Download and Install raspbian and image writer - Setup the SD card formatter on windows - Write the image - Write the OS on the SD card - Setting up the pi - Run the Setup - Reboot the Raspberry pi - Update the firmware - Identify the RPi Models and GPIO - Python package installation using pip |
| - CU4. Introduction to LINUX commands | - Opening Linux Terminal - Perform Basics of Linux Terminal - Use Commands to display directory - Create file on the directory - Use Commands to run the file on LINUX |
| - CU5. Develop a program based on control structures | - Identify Basic Data Types - Use Assignment Operators - Use print() function - Use Comments in Python - Use input() functions - Identify Code Blocks and Indentation - Identify Reserved words in Python - Use Assignment Operators and Expressions - Use Arithmetic Operations - Identify Precedence and Associativity - Identify Docstring |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand Installation of python

- Understand Raspbian environment

- Define VNC connection

- Understand the LINUX commands

- Define File handling of python

- Define Tuples and its methods

- Define Operators and sets

- Define Dictionary and methods

- Define Strings and its methods

- Define Relational operators

- Define Logical operators.

- Define conditional operator.

- Define Arrays.

- Explain one dimension array declaration.

- Explain the initialization of array.

- Explain the accessing of array elements.

- Define the declaration of two dimension array.

- Define Functions.

- Explain types of functions

- Explain function prototype

- Explain function definition.

- Explain the use of function call.

- Explain the passing parameters to Function.

- Define the pointer

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Develop a computer system based on Raspbian board.

- Install and run python based simple code.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Python |
|  | Raspberry pi |
|  | Linux operating system ( Raspbian) |
|  | Memory card |
|  | RJ-45 cable |
|  | Adaptor |

# 0714E&A82-A: Advanced Programming-II

Overview: This competency standard covers the skills and knowledge required to develop how to write a computer program. It will create the knowledge to write lists of instructions for a computer to follow and to develop the software programs, the scripts, or other sets of instructions for the computers to execute. This unit will cover the knowledge of programming language C which has variety of data types and powerful operators. Due to this, students will be able to write computer programs efficiently and easily.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Develop program Using Conditional Structures and Operators | - Use Boolean Expressions - Use If Statement - Use If-Else - Use Nested-If - Use If-Elif-Else - Identify Membership Operators - Identify Logical Operators - Use Bitwise Operators - Identify Simple Recursion - Use Factorial with Recursion - Program to display Fibonacci sequence using recursion - Use Indirect Recursion |
| - CU2. Develop program using loop | - Use While Loop - Use While-break - Use While-Continue - Use for-list - Use for-range - Use for-continue-break - Program to display prime numbers in an interval - Calculate factorial - Compute Fibonacci Series |
| - CU3. Develop a program using Functions | - Use simple FUNCTIONS. - Use Function Declaration. - Use Function Prototype. - Use Function Arguments - Use Call Functions. - Pass parameters to Function. - Use Function Arbitrary Arguments |
| - CU4. Python file handling | - Create a Text File - Append Data to a File - Read a File - Read a File line by line - Identify File Modes in Python - Handling of CSV file - Handling of Excel File |
| - CU5. Develop a program using Python Data Structure | - Declare Strings - Create Lists in python - Apply Lists methods - Constructing Tuples in python - Apply Tuples methods - Create Sets - Use Operators on Sets - Create Dictionary - Methods to query information from dictionaries - Create Strings - Formatters and Operator on String - Methods on String |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand Installation of python

- Understand Raspbian environment

- Define VNC connection

- Understand the LINUX commands

- Define File handling of python

- Define Tuples and its methods

- Define Operators and sets

- Define Dictionary and methods

- Define Strings and its methods

- Define Relational operators

- Define Logical operators.

- Define conditional operator.

- Define Arrays.

- Explain one dimension array declaration.

- Explain the initialization of array.

- Explain the accessing of array elements.

- Define the declaration of two dimension array.

- Define Functions.

- Explain types of functions

- Explain function prototype

- Explain function definition.

- Explain the use of function call.

- Explain the passing parameters to Function.

- Define the pointer

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Develop a python program using various libraries.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Python |
|  | Raspberry pi |

### Robotic System Evaluation

# Perform Robot Installation

Overview: This competency standard covers the skills and knowledge related to installation of robotic arm, method of hardware installation at workplace and installation of robot operating software and its configuration. Set them to do. Like in factories all they have to do with making their products is make the robot.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Perform Hardware installation of robotic arm. | - Remove protective plastic from robot - Inspect for damages and missing parts - Move the robot to designated installation area - Bolt the robot in place - Connect cables and peripherals - Connect end effector. |
| - CU2. Perform Software installation of robotic arm. | - Install software suite for robotic arm. - Check connectivity cable - Configure settings with hardware specification. - Check interlocking. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

Learn and understand how to analyze industrial environment.

- Learn and understand types and working of bolts.

- Learn and understand what are firmware

- Learn and understand types of software use for operation of robotic arm.

- Learn how to read datasheet.

- Learn interlock.

- Learn the mobility of robots.

- Understand how to inspect damages and missing parts.

- Learn troubleshooting of software.

- Understanding about payload.

- Understanding about various types of cables.

- Understanding about various types of connectors.

- Understanding about interlocking features

- Understand to read installation manual

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Develop and maintain hardware structure of robot.

- Perform Install and configure software of robot.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer System |
|  | Connectors |
|  | Cables |
|  | Tools for installation |
|  | Bolts and nuts |
|  | End effectors |
|  | Robot operating software |
|  | Cleaning and lubricating chemicals |

# Perform Diagnose precise robot movement

Overview: This competency standard covers the skills and knowledge related to robot faults the detection of faults and detail knowledge of fault causes, how robot can be used in precision working jobs and tolerances and Limitation, speed of robot movement and precision control.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Perform fault detection | - Remove check behaviour of joints as per manual. - Check behaviour of end effector as per manual. - Check power wiring. - Check the behaviour of all sensors & switches. |
| - CU2. Perform repeatability. | - Give coordinate to perform task. - Check if the given task is completed successfully, again give the same task. - Check both outputs. |
| - CU3. Detect error. | - Give coordinate to perform task. - Measure the error between given coordinate and actual result - Calculate the error in percentage. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Learn Understanding of repeatability procedure.

- Understanding of analyzing precision of specific instrument.

- Understanding of analyzing accuracy of specific instrument.

- Understanding of robot functionality.

- Understanding of motion range of robot.

- Understand and learn different task to check the precision of robotic movement.

- Understanding of cable routing.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform fault detection.

- Perform repeatability test.

- Perform Accuracy and precision test.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Multimeter |
|  | Computer system |
|  | Data analyzing Software |
|  | Precision toolkit |
|  | Reference measuring instrument |

# Perform Diagnose Coupling Parameters

Overview: This competency standard covers the skills and knowledge related to choosing a specific robot, a lot of different configurations can be made regarding the end effector's fixation method. In fact, most of the time when buying a specific gripper and robotic arm, they won’t match with each other. This is why most companies use mechanical couplings or adapters to be able to fix the gripper to the robot arm.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Verify physical structure | - Check the broken parts - Ensure the accurate connectivity of all parts according to manual. - Tight parts (if required). - Verify smooth and normal operation of all parts. |
| - CU2. Diagnose Coupling Parameters | - Install Identify the method of determining aspects of movement quality. - Identify the kinematic couplings for precision fixtures and assembly. - Identify the precision robot calibration using kinematic placed inclinometers. - Identify the checking of stability and mechanical interchangeability. |
| - CU3. Apply diagnose strategy | - Give Diagnose coupling of end effector - Diagnose coupling of joints - Diagnose coupling with base |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand and learn the method of determining aspects of movement quality.

- Understand and learn the kinematic couplings for precision fixtures and assembly.

- Understand and learn the precision robot calibration using kinematic placed inclinometers

- Understand and learn the checking of stability and mechanical interchangeability.

- Understand and learn the tools for diagnosing the kinematic coupling.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform diagnostic procedures on Robot.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Position measuring tool |
|  | Fixtures and assembly. |
|  | Inclinometer |
|  | Mechanical strength measuring tools |
|  | Coordinate calibration tools |
|  | Tool Box |

# Implement the performance characteristics identified in ISO9283 standards

Overview: This competency standard covers the skills and knowledge related to ISO9283 intended to facilitate understanding between users and manufacturers of robots and robot systems. It defines the important performance characteristics, describes how they shall be specified and recommends how they should be tested.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify performance characteristics of ISO9283 | - Identify the proper tool according to standard. - Determine the error according to ISO Standard. - Identify the Coordinate calibration error. - Identify the measurement error. - Identify the robot initialization error - Identify the vibration and others error. - Identify the data selection error. |
| - CU2. Apply performance characteristics of ISO9283 | - Install Apply ISO9283 on robot manufacturing. - Apply ISO9283 on robot system |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand and learn to read and solve error messages according to ISO9283 Standard.

- Understand to resolve the Coordinate calibration error.

- Understand to resolve the measurement error

- Understand to resolve the robot initialization error

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform the Test of precision, accuracy and repeatability of robot.

- Perform Fault detection of robot.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Vibration sensors |
|  | Jumpers |
|  | Measuring tools |
|  | Error measuring tools |
|  | Coordinate calibration tools |

### Robotic Application

# Develop Line Following Robot

Overview: This competency standard covers the skills and knowledge required to develop one of the advanced mobile robots is the Line Follower Robot. It is basically a robot which follows a particular path or trajectory and decides its own course of action which interacts with obstacle. The path can be a black line on the white floor (visible) or a magnetic field (invisible). Its applications start from basic domestic uses to industrial uses, etc. The present condition in industry is they are carrying the parcels or materials one place to another place using the crane system. Sometimes lifting of big weights at that time may cause the breakage of lifting materials and will be cause damage to the parcels also. The line following robots is commonly used for carry children through shopping malls, homes, entertainment places, industries. The use of line following robotic vehicle is transport the materials from one place to another place in the industries. This robot movement completely depends on the track. The robot can do anything you set them to do. Like in factories all they have to do with making their products is make the robot.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify parts of Line following robot | - Open basic trainer kit of Line following robot. - Identify actuators - Identify sensors to detect path. - Arrange parts according to their utilization. |
| - CU2. Assembly of Line following robot | - Assembly of the parts of line following robot - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for controller and actuator. - Recognize the program algorithm for the line following robot |
| - CU3. Programming of controller | - Open programming software IDE on computer - Develop code for line following robot - Compile and upload code in controller. |
| - CU4. Perform robot movement test | - Preform the robot movement detection test - Check the line following robot feasibility and manipulability. - Check of line following robot operating conditions - Check the condition of each component and overall assembly of the line following robot. - Check all the connections of the components. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of numerous types of Line following robot

- Understanding of sensors used in line following robot.

- Understanding of numerous types of actuators

- Understanding and learn connection controller, sensors and actuators.

- Understanding of controller programming software

- Understanding and learn to develop code for line following robot

- Understanding and learn troubleshooting of code.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform integration of line following robot.

- Perform Uploading the code in controller of line following robot.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Turbo C, C++, Code block |
|  | Line following trainer kit |
|  | Screw driver |
|  | Jumper wires |
|  | Chassis for line following robot. |

# Develop Smart phone control robot using Bluetooth

Overview: This competency standard covers the skills and knowledge required to design and control the motion of robot using Bluetooth device of an Android phone. Many android apps available today which are helpful to achieve this task.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify parts of robot | - Open basic trainer kit of ground vehicle robot. - Identify mobile base. - Identify actuators - Identify Bluetooth modem to for communication. - Arrange parts according to their utilization. |
| - CU2. Assembly of robot | - Assemble the parts - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for controller and actuator. - Recognize the program algorithm for the Smart Phone Control Robot Using Bluetooth. |
| - CU3. Programming of controller | - Program the controller to change direction of robot according to the receive signal from smart phone android application via Bluetooth. - Download and Install the android application on smartphone. - Connect the Bluetooth with Bluetooth module on controller. |
| - CU4. Perform robot movement test | - Identification of movement of the Smart Phone Control Robot Using Bluetooth. - Preform the robot movement detection test - Check the robot feasibility and manipulability. - Check the robot all possible operating conditions. - Check the Bluetooth range and its connectivity. - Check the condition of each component and overall assembly of the robot. - Check all the connections of the components. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of numerous types of controlled robot

- Understanding of Bluetooth transmitter.

- Understanding of numerous types of actuators

- Understanding and learn connection controller, transmitter, receiver and actuators.

- Understanding of controller programming software

- Understanding and learn to develop Bluetooth controlled robot

- Understanding and learn troubleshooting of code.

- Understanding about android operating system

- Understanding about android application.

- Understanding the configuration of android application used in project.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform integration of Bluetooth controlled robot.

- Perform Uploading of code in controller of robot.

- Perform download and installation of smart phone application.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Turbo C, C++, Code block |
|  | Robot trainer kit |
|  | Screw driver |
|  | Jumper wires |
|  | Bluetooth transceiver. |
|  | Controller board. |
|  | Smart phone |

# Develop IoT based home automation

Overview: This competency standard covers the skills and knowledge required to Home automation system uses the portable devices as a user interface. They can communicate with home automation network through an Internet gateway, by means of low power communication protocols like Zigbee, Wi-Fi etc. This project aims at controlling home appliances via Smartphone using Wi-Fi as communication protocol.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify basic components of home automation | - Identify basic components of home automation - Identify the specification of home appliances and things to automate. - Select the components for each part of the automation system. - Identify the sensors to detect light, movement, temperature and other. |
| - CU2. Perform integration of parts | - Perform the WiFi/Ethernet module connection with controller. - Connect the LCD display with controller. - Design the model and overall assembly on single control unit. - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller. |
| - CU3. Configure system | - Recognize the power supply distribution for sensors, controller and actuator. - Recognize the program algorithm for each sensor. - Setup the cloud for getting the sensors data on cloud. - Program the controller to automate the door, turn light on/off from smart phone, control the Fan from smart phone and display the temperature and humidity on screen. |
| - CU4. Perform testing | - Download and Install the android application on smartphone. - Connect the Internet with Internet module on controller. - Control light, fans and home appliances with mobile using IoT. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of numerous types of controller

- Understanding of WIFI transceiver.

- Understanding of numerous types of actuators

- Understanding and learn connection controller, transmitter, receiver and actuators.

- Understanding of controller programming software

- Understanding and learn to develop WIFI connection.

- Understanding and learn troubleshooting of code.

- Understanding about android operating system

- Understanding about android application.

- Understanding the configuration of android application used in project.

- Understanding and learning of IoT protocol

- Understanding and learning of relay based module.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform integration of WIFI module with internet.

- Perform Uploading of code in controller.

- Perform download and installation of smart phone application.

- Perform automation of home appliances.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Turbo C, C++, Code block |
|  | Home automation kit |
|  | Screw driver |
|  | Jumper wires |
|  | WIFI transceiver. |
|  | Controller board. |
|  | Smart phone |
|  | Internet availability |

# Develop Smart Phone Control Door Lock System

Overview: This competency standard covers the skills and knowledge required for Android mobile controlled door security locking system has its main application in security systems. It can be used in house, shop, offices, industry.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify the components of remote control door locks | - Specify the types Of Smart Phone Control Door Lock System. - Identify the specification of door lock. - Select the components. - Identify the actuators. - Identify the module to detect the Bluetooth signals |
| - CU2. Assemble door locks with bluetooth | - Perform the Bluetooth module connection with controller. - Connect the LCD display with controller. - Design the model and overall assembly on single control unit. - Select the Controller - Draw the circuit diagram. |
| - CU3. Configure and Perform testing | - Download and Install the android application on smartphone. - Connect the Bluetooth with Bluetooth module on controller. - Identification of door activity according to pin code provide to unlock the door. - Preform the Door Lock System error detection test. - Check the robot feasibility and manipulability. - Check the Door Lock System all possible operating conditions. - Check the Bluetooth range and its connectivity. - Check the condition of each component. - Check all the connections and wiring of the components. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of numerous types of controller

- Understanding of Bluetooth transceiver.

- Understanding of numerous types of actuators

- Understanding and learn connection controller, transmitter, receiver and actuators.

- Understanding of controller programming software

- Understanding and learn to connect Bluetooth transceiver.

- Understanding and learn troubleshooting of code.

- Understanding about android operating system

- Understanding about android application.

- Understanding the configuration of android application used in project.

- Understanding and learning various types of motors

- Understanding and learning of relay based module.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform integration of WIFI module with internet.

- Perform Uploading of code in controller.

- Perform download and installation of smart phone application.

- Perform controlling of door lock.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Turbo C, C++, Code block |
|  | Home automation kit |
|  | Screw driver |
|  | Jumper wires |
|  | WIFI transceiver. |
|  | Controller board. |
|  | Smart phone |
|  | Internet availability |
|  | Door lock |

# Develop Security System Using Motion Detector

Overview: This competency standard covers the skills and knowledge required for motion sensor is device that detects when someone is in your home when they shouldn’t be. A motion sensor uses one or multiple technologies to detect movement in an area. If a sensor is tripped, a signal is sent to your security system’s control panel, which connects to your monitoring center, alerting you and the monitoring center to a potential threat in your home.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify parts of security system | - Specify the types Of Security System Using Motion Detector. - Identify the specification of Security System. - Select the components. - Identify the actuators. - Identify the sensor to detect the Motion. |
| - CU2. Perform Assembly and integrate | - Select the Controller. - Design the model and overall assembly on single control unit. - Draw the circuit diagram. - Electric wiring of sensors, actuator and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for sensors, controller and actuator. |
| - CU3. Perform Programming and configuration | - Write code for security system according to provided sensor and actuator. - Upload code into controller - Perform the Security System movement detection test. - Check the Security System in all possible operating conditions. - Check the buzzer range. - Check the condition of each component. - Check all the connections and wiring of the components. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of numerous types of motion sensor

- Understanding of numerous types of actuators

- Understanding and learn connection controller, motion sensor and actuators.

- Understanding of controller programming software

- Understanding and learn troubleshooting of code.

- Understanding and learning various types of alarm system

- Understanding and learning of relay based module.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform integration of motion sensor with controller.

- Perform Uploading of code in controller.

- Perform Installation of alarm.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Turbo C, C++, Code block |
|  | Alarm |
|  | Screw driver |
|  | Jumper wires |
|  | Motion sensor |
|  | Controller board. |
|  | Loud speaker |

# Develop Water-level Detection in Tank

Overview: This competency standard covers the skills and knowledge required for motion sensor is device that detects when someone is in your home when they shouldn’t be. A motion sensor uses one or multiple technologies to detect movement in an area. If a sensor is tripped, a signal is sent to your security system’s control panel, which connects to your monitoring center, alerting you and the monitoring center to a potential threat in your home.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify parts for water level detection | - Specify the types of Water-level Detection in Tank. - Identify the specification of Water-level Detection Tank. - Select the components. - Identify the actuators. - Identify the sensor to detect the Water-level. |
| - CU2. Perform assembly and integration of parts | - Select the Controller. - Design the model and overall assembly on single control unit. - Draw the circuit diagram. - Electric wiring of sensors, actuator and controller. - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for sensors, controller and actuator. |
| - CU3. Perform Programming and configuration | - Program using Algorithm to detect the Water-level in tank and display reading on the LCD. - Preform the Water-level Detection test on different ranges of sensor with varying the level of water. - Check the Water-level Detection System feasibility and manipulability. |
| - CU4. Perform Running and testing | - Check the Water-level Detection System all possible operating conditions. - Check the sensor range. - Check the condition of each component. - Check all the connections and wiring of the components. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of numerous types of water level sensing technique

- Understanding of numerous types of indicators

- Understanding and learn connection controller, level sensor and indicator.

- Understanding of controller programming software

- Understanding and learn troubleshooting of code.

- Understanding and learning various types of alarm system

- Understanding and learning of motor control system.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform integration of level sensor with controller

- Perform Uploading of code in controller.

- Perform Installation of alarm system

- Performing motor controlling

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Turbo C, C++, Code block |
|  | Alarm |
|  | Screw driver |
|  | Jumper wires |
|  | Level sensor |
|  | Controller board. |
|  | Motor |
|  | Computer system |

# Develop Mini CNC 2D Plotter

Overview: This competency standard covers the skills and knowledge required for use of system to draw computerize 2D plotting on paper. It is use t draw map and design.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify parts of CNC machine | - Specify the types 2D CNC Plotter. - Identify static base of 2D CNC Plotter. - Identify the actuators. - Select the components. - Identify the sensors. |
| - CU2. Perform Assembly of CNC parts | - Design the model and overall assembly of the CNC machine. - Assembly of the parts - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for controller and actuator. - Recognize the program algorithm for the 2D CNC |
| - CU3. Perform Programming of CNC system | - Program the controller to trace the input image on the page. - Apply translator to convert image into G&M Code - Upload code. - Identification of movement of the 2D CNC Plotter. |
| - CU4. Perform Running and testing | - Preform the robot movement detection test - Check the 2D CNC Plotter feasibility and manipulability. - Check of 2D CNC Plotter operating conditions. - Check the condition of each component and overall assembly of the 2D CNC Plotter. - Check all the connections of the components. - Perform debugging (If required) |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of numerous types of 2D imaging technique

- Understanding of numerous types of coordinates system

- Understanding and learn connection controller, stepper motor and indicator.

- Understanding of controller programming software

- Understanding and learn troubleshooting of code.

- Understanding and learning paper installation technique

- Understanding and learning of 2D motor control system.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform integration of parts of CNC machine

- Perform Uploading of code in controller.

- Perform Print 2D plot

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Turbo C, C++, Code block |
|  | Alarm |
|  | Screw driver |
|  | Jumper wires |
|  | Controller board. |
|  | Motors |
|  | Old CD ROMs |

# Develop Voice Controlled Robot

Overview: This competency standard covers the skills and knowledge required for Voice Controlled Robotic Vehicle helps to control robot through voice commands received via android application. The integration of control unit with Bluetooth device is done to capture and read the voice commands. The robotic vehicle then operates as per the command received via android application.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify parts of voice controlled robot | - Specify the types of Voice Controlled Robot. - Identify mobile base of robot. - Identify the actuators for of robot. - Select the components for robot. - Identify the sensors to recognize the voice instructions. |
| - CU2. Perform Assembly of Robot | - Design the model and overall assembly of the robot. - Assembly of the parts of line following robot - Select the Controller - Draw the circuit diagram - Electric wiring of sensors, actuator and controller - Design the algorithm flowchart to implement on controller. - Recognize the power supply distribution for controller and actuator. - Recognize the program algorithm for the Voice Controlled Robot. |
| - CU3. Perform Programming and configuration | - Program the controller to change direction of robot according to the voice command. - Upload the code into the controller. - Identification of movement of the Voice Controlled Robot. |
| - CU4. Perform Running and testing | - Preform the robot movement detection test. - Check the robot feasibility and manipulability. - Check of line robot operating conditions. - Check the condition of each component and overall assembly of the robot. - Check all the connections of the components. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of numerous types of voice controlled mechanism

- Understanding of numerous voice command sensor

- Understanding and learn connection controller, sensor and indicator.

- Understanding of controller programming software

- Understanding and learn troubleshooting of code.

- Understanding and learning Bluetooth connection and pairing

- Understanding and learning of UART based communication.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform integration of parts of voice controlled robot

- Perform Uploading of code in controller.

- Perform robot movement by using voice commands

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Computer system |
|  | Integrated Development Environment (IDE) |
|  | Turbo C, C++, Code block |
|  | Smart phone |
|  | Screw driver |
|  | Jumper wires |
|  | Controller board. |
|  | Motors |

### Industrial Electronics

# Implement diode and Thyristor in power control application.

Overview: This competency standard covers the skills and knowledge required to Build forward bias circuit of diode and observe its behaviour, Build reverse bias circuit of diode and observe its behaviour, Use SCR to build Phase control rectifier, Build force commutated circuit for SCR and Build full wave converter and observe natural commutation. Build forward bias and reverse bias circuit of diode and observe its behaviour.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Build forward bias circuit of diode and observe its behavior. | - Draw forward bias circuit diagram of diode. - Make connection as per diagram. - Connect volt meter parallel to diode and ampere meter in series - Turn on the supply and take readings of ampere meter as well as voltmeter. - Note the behaviour of diode |
| - CU2. Build reverse bias circuit of diode and observe its behavior. | - Draw reverse bias circuit diagram of diode. - Make connection as per diagram. - Connect volt meter parallel to diode and ampere meter in series - Turn on the supply and take readings of ampere meter as well as voltmeter. - Note the behaviour of diode. |
| - CU3. Use SCR to build Phase control rectifier | - Draw circuit diagram of phase control rectifier. - Make connection as per diagram. - Switch on the power supply. - Vary the value of potentiometer to set the trigger level for the SCR - Recode the output signal wave shape from oscilloscope. |
| - CU4. Build force commutated circuit for SCR | - Draw circuit diagram of force commutated circuit for SCR. - Make connection as per diagram. - Switch on the power supply and apply triggering pulse at the gate of SCR. - Connect oscilloscope across the load resistor and record the output wave shape |
| - CU5. Build full wave converter and observe natural commutation. | - Draw circuit diagram of full wave converter. - Make connection as per diagram. - Switch on the power supply and apply triggering pulse at the gate of SCR. - Connect oscilloscope across the load resistor and record the output wave shape the output wave shape |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define forward biasing

- Define reverse biasing

- Define Reverse breakdown voltage

- Define junction potential

- Define SCR

- Describe the operation of SCR

- Define phase control rectifier

- Define triggering of SCR

- Describe the use of SCR

- Define VAK

- Define commutation and its types

- Define natural commutation.

- Define force commutation

- Name the methods of force commutation.

- Describe the use of force commutation

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Build forward bias circuit of diode and observe its behavior

- Build reverse bias circuit of diode and observe its behavior

- Use SCR to build Phase control rectifier

- Build force commutated circuit for SCR

- Build full wave converter

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Power diode |
|  | Resistor of 1 K ohms |
|  | Volt meter |
|  | Ampere meter |
|  | DC supply |
|  | Connecting leads |
|  | Power supply |
|  | Centre tapped transformer |
|  | Bread Board |
|  | Oscilloscope |
|  | Resistor of 10 K ohms |
|  | Capacitor 10uF |
|  | SCR |
|  | Oscilloscope Dual |
|  | Potentiometer 500 o |
|  | Resistor of 10 K & 100 K ohms |
|  | Inductor 2mH |
|  | Diode |

# Construct single phase and three phase Inverter

Overview: This competency standard covers the skills and knowledge required to Construct single phase half bridge inverter and observe its output, construct single phase full bridge inverter and observe its output and Construct three phase bridge inverter and observe its output Construct single phase half bridge inverter, single phase full bridge, three phase bridge inverter and observe their outputs.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Construct single phase half bridge inverter and observe its output | - Draw circuit diagram of single-phase half bridge inverter. - Make connection as per diagram. - Apply triggering pulses to circuit with the help of signal generator. - Connect oscilloscope across the load resistor and record the output wave shape |
| - CU2. Construct single phase full bridge inverter and observe its output | - Draw circuit diagram of single-phase full bridge inverter. - Make connection as per diagram. - Apply triggering pulses to circuit with the help of signal generator. - Connect oscilloscope across the load resistor and record the output wave shape |
| - CU3. Construct three phase bridge inverter and observe its output | - Draw circuit diagram of three-phase bridge inverter. - Make connection as per diagram. - Apply output of controller IC to the gates of MOSFET’s for triggering pulses. - Connect oscilloscope across the load resistor and record the output wave shape |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Inverter

- Describe different types of inverter

- Define half bridge inverter

- Define Full bridge inverter

- Define 3 phase bridge inverter

- Describe the use of function generator.

- Define use of controller IC

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Construct single phase half bridge inverter and observe its output

- Construct single phase full bridge inverter and observe its output

- Construct three phase bridge inverter and observe its output

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Power MOSFET IRF 840 |
|  | Power Diode 1N4007 |
|  | DC Source 12V |
|  | Resistor 10K |
|  | Function Generator |
|  | IC 7404 |
|  | Controller IC |
|  | Oscilloscope |
|  | Connecting leads |

# Control speed of DC/AC Motor

Overview: This competency standard covers the skills and knowledge required to control speed of DC/AC motor.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Implement armature control method to control the speed of DC motor with 3 phase control bridge rectifier and Bridge rectifier. | - Draw circuit diagram of armature control method to control the speed of DC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now change the triggering of SCR’s and record the effect on speed of DC motor. |
| - CU2. Implement Field control method to control the speed of DC motor with 3 phase bridge rectifier and control Bridge rectifier. | - Draw circuit diagram of field control method to control the speed of DC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now changes the triggering of SCR’s and record the effect on speed of DC motor. |
| - CU3. Implement Field control method & armature control method to control the speed of DC motor with 3 phase bridge rectifier. | - Draw circuit diagram of field control & armature control method to control the speed of DC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now changes the triggering of SCR’s and record the effect on speed of DC motor. |
| - CU4. Implement AC to AC converter to control the speed of 3 phase AC motor. | - Draw circuit diagram of AC to AC converter to control the speed of AC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now change the triggering of SCR’s and record the effect on speed of AC motor. |
| - CU4. . Implement single phase cycle-converter with center tapped transformer and SCR’s and change the frequency of output AC signal. | - Draw circuit diagram of single phase cycle-converter to produce variable frequency AC signal. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Apply Triggering to SCR’s 1 & 3 for 1st 25 cycles to get positive half cycle and apply triggering for remaining 25 half cycle to SCR’s 2 & 4 to get the negative half cycle with controller IC. - Connect the oscilloscope across the load and record the output |
| - CU6. Implement synch’s to transmit torque | - Place synchrony transmitter and synchrony receiver at two different places. - With help of cord establish connection between corresponding terminal of TX& Rx i.e. S1 to S1, S2 to S2 and S3 to S3 respectively. - Give single phase AC supply to the rotor of both TX and Rx. - Rotate the rotor of TX in step to 30 degrees and observe the new position of rotor of Rx. - Enter the input angular position & output angular position in the table. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define DC motor

- Describe parts of DC motor

- Enlist the methods to control the speed of DC motor.

- Describe armature control method.

- Describe field control method.

- Describe the range of firing angle of SCR’s

- Define use of controller IC.

- Define AC to AC conversion

- Define cycle-converter.

- Describe types of cycle-converter

- Define single phase cycle-converter.

- Define three phase cycle-converter.

- Define synchs.

- Define synchrony generator or TX

- Define synchrony receiver or Rx

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Implement armature control method to control the speed of DC motor with 3 phase control bridge rectifier and Bridge rectifier

- Implement Field control method & armature control method to control the speed of DC motor with 3 phase bridge rectifier

- Implement AC to AC converter to control the speed of 3 phase AC motor

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Multimeter |
|  | Connecting leads |
|  | Synchrony Transmitter & Receiver |
|  | Supply |
|  | Power supply |
|  | Connecting leads |
|  | CRO |
|  | Firing Circuit |
|  | SCR’s |
|  | Power Electronics Trainer Kit |
|  | Power Diode 1N4007 |
|  | Controller IC |
|  | DC Motor |
|  | AC motor |
|  | Connecting leads |
|  | Three phase supply |

### Hydraulic & Pneumatic Machinery

# Operate Hydraulic Bench and Its Functions

Overview: This competency standard covers the skills and knowledge required to provide knowledge for observing Hydraulic Bench, identify various standard parts of Hydraulic Bench and its main functions.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Identify parts of hydraulic bench | - Enlist all parts of a Hydraulic Bench - Identify Control valve, Drain valve and Dump valve. - Identify Sump Tank and Volumetric Tank. - Identify Open Channel and Vertical Pipe. - Identify Centrifugal Pump - Identify Actuators |
| - CU2. Identify/perform Various Functions of/on Hydraulic Bench | - Ensure that pump is switched off. - Set up the device to be tested. - Attach a water supply for open channel flow or a close conduct device to the water inlet. - Ensure that all connections are secure. - Ensure that supply valve is closed, and dump valve is open. - Turn the pump on. - Open slowly the supply valve to allow water to circulate the bench. - Make appropriate measurement in the device being tested for a given setting of supply valve e.g. pressure reading, water depths etc. - Closedown the supply valve upon the completion of test. - Turn off the valve. - Disconnect the device used for testing. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe the purpose of a Hydraulic Bench

- Explain the working of a hydraulic bench with help of circuit diagram.

- Describe the importance of Centrifugal pump in a Hydraulic Bench.

- Describe the function of Main Tank and of Sump Tank.

- Explain the purpose and working of Control valve, Drain Valve and Dump Valve.

- Describe centrifugal pump

- Define Difference between sump tank and volumetric tank

- Define Difference control valve, drain valve and dump valve

- Define Difference main channel and side channels

- Describe water level indicator

- Describe stilling baffle

- Define Difference between overflow and starter

- Describe a Schematic Diagram of Hydraulic Bench

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify parts of hydraulic bench

- Identify Functions of/on Hydraulic Bench

- Perform Various Functions of/on Hydraulic Bench

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Hydraulic bench |
|  | Control Valve |
|  | Centrifugal pump |
|  | Sump tank |
|  | Vertical pipe |
|  | Channel |
|  | Drain valve |
|  | Volumetric tank |
|  | Water Level Indicator |
|  | Dump valve |
|  | Actuators |
|  | Side Channel |

# Analyse the performance of Pumps

Overview: This competency standard covers the skills and knowledge required to observe and analyse centrifugal pump as well as reciprocating pump.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Analyze the performance of Centrifugal Pump | - Prime the pump - Open the gate valve 1 or 2 rotations - Start the motor and set the vacuum gauge reading to the required head - Note down the following readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x - Set the vacuum gauge reading to the other heads - Note down the above readings G, V, T, and t - Take at least 5 sets of readings by varying the head through delivery valve and note down the readings |
| - CU2. Analyse the performance of Reciprocating Pump | - Prime the pump - Open the gate valve 1 or 2 rotations - Start the motor and set the vacuum gauge reading to the required head - Note down the following readings - Pressure gauge reading G - Vacuum Gauge Reading V - Time taken for 10 revolutions in the energy meter T - Time taken to fill up 200 cm rise in the collecting tank, t - The difference in the levels of the pressure and vacuum gauges, x - Set the vacuum gauge reading to the other heads - Note down the above readings G, V, T, and t - Take at least 5 sets of readings by varying the head through delivery valve and note down the readings |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe centrifugal pump working principle

- Describe pump casing

- Describe impeller

- Describe suction pipe and delivery pipe

- Describe reciprocating pump

- Describe suction valve and delivery valve

- Describe crank and connecting rod mechanism

- Describe suction and delivery pipe

- Describe function of Piston

- Define difference between centrifugal and reciprocating pump.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify the centrifugal pump gauges

- Operate the centrifugal pump

- Analyze the performance of centrifugal pump

- Identify the reciprocating pump gauges

- Operate the reciprocating pump

- Analyze the performance of reciprocating pump

- Identify difference between centrifugal and reciprocating pump.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Apparatus |
|  | Centrifugal Pump |
|  | Test Bench |
|  | Pipe Hoses |
|  | Voltmeter |
|  | Ammeter |
|  | Reciprocating Pump |
|  | Tachometer |
|  | Stop watch |
|  | Pressure gauge |
|  | Energy meter |

# Working/operation directional control valves

Overview: This competency standard covers the skills and knowledge required to operate gate valve, operate globe valve, operate check valve, operate plug valve, operate ball valve, operate butterfly valve, operate needle valve, operate pinch valve, Operate relief valve, pressure gauges, temperature gauges, Flow gauges and Different types of sensor.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Operate gate valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve - Perform the same procedure for remaining types of the valves |
| - CU2. Operate globe valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |
| - CU3. Operate check valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |
| - CU4. Operate plug valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |
| - CU5. Operate ball valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |
| - CU6. Operate butterfly valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |
| - CU7. Operate needle valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |
| - CU8. Operate pinch valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |
| - CU9. Operate relief valve | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |
| - CU10. Demonstrate Pressure gauges, temperature gauges, Flow gauges and Different types of sensor. | - Choose the type of valve - Create pressure according to the requirement - Calculate the pressure and mass flow rate of the water w.r.t. time - Calculate efficiency of the valve |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe  the motion of the valves

- Describe the measurements of pressure

- Define Calculate mass flow rate in both cases either fully opened or fully closed

- Describe to control air, fuel gas, feed water, steam, lube oil hydrocarbon and other services

- Define the efficiency calculation of the valves

- Define the efficiency calculation of the gauges w.r.t. sensor values

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform to operate gate valve, operate globe valve, operate check valve, operate plug valve, operate ball valve, operate butterfly valve, operate needle valve, operate pinch valve, operate relief valve, pressure gauges, temperature gauges, Flow gauges and Different types of sensor.

- Perform to control air, fuel gas, feed water, steam, lube oil hydrocarbon and other services

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Various types of valves |
|  | Water resources |
|  | Pipe lines |
|  | Valve test bench |
|  | Pressure gauges |
|  | Various types of pressures, temperatures And flow gauges |
|  | Heat resources |
|  | Sensors |

# Setup a Pressure Device and Hold Specific Load on a Double Acting Cylinder

Overview: This competency standard covers the skills and knowledge required to use pressure reducing valve on double acting cylinder and holding load on double acting cylinder.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Set-up pressure device on a double acting cylinder using pressure reducing valve | - Complete the hydraulic circuit as drawing given - Turn on the circuit supply and check if all devices are working properly - Ensure Fluid starts flowing and the initially adjusted pressure is passed on to the circuit while passing through the relief valve. - Observe and note the pressure reading at various pressure gauges. - Change direction with the help of DCV and ensure control of double acting cylinder according to this direction. - Re-observe and re-note the readings on all the pressure gauges. |
| - CU2. Hold specific load using double acting cylinder | - Complete the hydraulic circuit as per drawings. - Turn on the circuit supply and check if all devices are working properly - Adjust pressure according to desired/required load. - Ensure fluid flow from port A. This would uplift the load. - Now change the position of DCV and ensure fluid entrance from port B. - Observe the downward motion of loaded ram. - Take the pressure reading and observe the cylinder return. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe pressure reducing valve

- Describe purpose of double acting cylinder as a pressure device

- Describe applications of double acting cylinder with the pressure pipe

- Describe function of pilot operated check valve

- Describe DCV

- Describe the role of RAM

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform pressure adjustment according to the required load.

- Identify the graphic symbols for various types of hydraulic components.

- Explain methods use in holding load on double acting cylinder.

- Describe purpose of double acting cylinder as a pressure device

- Describe applications of double acting cylinder with the pressure pipe

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Apparatus |
|  | Hydraulic Bench |
|  | Double ACTING Cylinder |
|  | Pressure reading valve |
|  | DCV pipe hoses |
|  | Hydraulic bench |
|  | Double acting cylinder |
|  | Pilot operated check valve |
|  | Direction control valve |
|  | Various loads |
|  | Pipe hoses |
|  | Pressure gauges |

# Actuate Double Acting Hydraulic Cylinder

Overview: This competency standard covers the skills and knowledge required to actuate a double acting hydraulic cylinder by a switch, speed regulation through throttle valve and flow control valves. It also explains actuation of double acting hydraulic cylinder by one-way throttle valve.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Actuate double acting hydraulic cylinder by switch and draw a circuit diagram | - Complete all connections of hydraulic circuit as per drawing - Turn on the push button switch to start the supply of fluid with which double acting cylinder will start moving - Move the piston to initiate the supply from forward stroke - Turn on the second push button switch after the piston reaches top dead centre. This will start supply in the opposite direction - Observe the functions from control valve by varying its speed after the completion of supply in the circuit - Note all observations. |
| - CU2. Actuate double acting hydraulic cylinder by oneway throttle valve | - Prepare the hydraulic circuit as per drawing - Turn on the circuit power supply and ensure that all accessories are working properly - Ensure throttle valve on return line of the circuit works properly. - Open the valve and ensure extra supply of oil to observe quick return of cylinder. - Observe cylinder speed at various positions of the valve. - Note all the observations |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe types of hydraulic cylinder

- Describe Double acting hydraulic circuit

- Describe direction control valve

- Describe function of push button

- Describe throttle valve function

- Describe one-way throttle valve

- Explain role of cylinder speed

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Working of Actuate double acting hydraulic cylinder by switch

- Working of Actuate double acting hydraulic cylinder by one-way throttle valve

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Apparatus |
|  | Hydraulic bench |
|  | Double acting cylinder |
|  | Pipe hoses and fittings |
|  | Pressure gauges |
|  | Flow control valve |
|  | Direction control valve |
|  | Push button |
|  | Hydraulic bench |
|  | Double acting cylinder |
|  | Pipe hoses And fittings |
|  | Pressure gauges |
|  | One-way Throttle valve |
|  | Direction control valve |

# Construct Circuit of Double Acting Hydraulic Cylinder for Mechanical Interlocking

Overview: This competency standard covers the skills and knowledge required to construct a hydraulic cylinder for mechanical interlocking and use of push buttons, limit switch and stored energy in double acting hydraulic cylinder

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Construct a hydraulic circuit for double acting hydraulic cyclinder for mechanical interlocking with switch | - Complete the hydraulic circuit as per drawings. - Turn on the circuit supply and check if all devices are working properly - Operate hydraulic circuit - Try lift some weight - Drop the pressure using pressure gauge before the piston starts moving - Construct Hydraulic Circuit Diagram - Construct electric circuit diagram |
| - CU2. Construct circuit to control double acting Hydraulic Cylinder using 2 push buttons and canceling with limit switch | - Prepare simulation circuit of double acting cylinder as per drawing - Fix the push button and limit switch at their designated place as per drawing - Turn on the circuit power supply and ensure proper working of all equipment. - Press the push button to start fluid supply and will enable the cylinder for forward stroke supply. - Observe that as the push rod touches limit switch the supply of coil would disconnect and the piston would stop. - Press the second push button to complete backward stroke of piston, this will enable the supply of side coil and the backward stroke of piston would be completed and as it enters the surroundings of limit switches the supply would be disconnected. - Change 5/2 DCV and complete the opposite circuit. - Observe this changed circuit and note down all the observations. |
| - CU3. Construct hydraulic circuit using Accumulator stored Energy by DAC | - Turn on the motor power supply and ensure proper working of all devises and equipment. - Use flow control valve to ensure fluid flow. - Observe the movement of piston rod. - When the circuit is burnt/completed the turn off the power supply. - Observe that due to accumulator the double acting cylinder completes its stroke and doesn’t stop immediately after the burning of circuit. |
| - CU4. Measure pressure at various connections in hydraulic circuits | - prepare the hydraulic circuit as per drawing - Install pressure gauges on the points where pressure is to be noted - Turn on the hydraulic circuit - Ensure fluid flow through the whole circuit - Complete the circuit and note the readings of all gauges - Enter all readings in observation table |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe mechanical interlocking function

- Describe the function of hydraulic circuit

- Describe double acting hydraulic cylinder

- Describe 3/2 push buttons

- Describe 5/2 double acting side pilot operated DCV

- Describe function of connecting tubes

- Describe function   of limit switches

- Describe the role of accumulator

- Describe mechanism of energy storage in accumulator

- Describe working of hydraulic accumulator

- Describe fluid port

- Describe construction of hydraulic actuator

- Describe function of actuators

- Describe coupling

- Describe functioning of chillers

- Describe proximity switch or limit switch

- Describe purpose of hydraulic circuit

- Draw analogy between hydraulic and electrical circuit

- Describe function of pressure control valve

- Describe flow control valve

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Working of Actuate double acting hydraulic cylinder by switch

- Describe mechanical interlocking function

- Describe double acting hydraulic cylinder

- Describe function of connecting tubes

- Describe mechanism of energy storage in accumulator

- Describe working of hydraulic accumulator

- Describe construction of hydraulic actuator

- Describe function of actuators

- Describe purpose of hydraulic circuit

- Describe function of pressure control valve

- Describe flow control valve

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Apparatus |
|  | Hydraulic bench |
|  | Mechanical interlocking |
|  | Double acting cylinder |
|  | Fluid control valve |
|  | DC pressure Gauges |
|  | Double acting cylinder |
|  | 3/2 push button |
|  | 5/2 double acting side pilot operated DCV |
|  | Contacting tubes |
|  | Hydraulic bench |
|  | Push button |
|  | Limit switch |
|  | Pressure gauges |
|  | Hydraulic Bench |
|  | Double Acting Cylinder |
|  | Accumulator Flow Control |
|  | Valve |
|  | DCV |
|  | Pipe Hoses |
|  | Pressure Gauges |
|  | Hydraulic bench |
|  | Pressure control valve |
|  | Flow control valve |
|  | Hydraulic pump hoses |

# Set Speed and Direction of a Hydraulic Actuator

Overview: This competency standard covers the skills and knowledge required to set-up a hydraulic motor using flow control valve and setting-up direction using direction control valve.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Set hydraulic Actuator(Motor) R.P.M using flow control valve | - Complete the hydraulic circuit as per drawing. - Turn on the motor power supply and ensure proper working of all devises and equipment. - Fix the tachometer with the rotor to note motor RPM - Use flow control valve to control the flow, this would also control the motor RPM. - Observe the speed during this process. - Note RPM at different flow rates and analyze them. |
| - CU2. Set hydraulic Actuator (Motor) direction by using direction control valve | - Complete the hydraulic circuit as per drawing. - Turn on the motor power supply and ensure proper working of all devises and equipment. - Use DCV to rotor the motor Clockwise and Anti Clockwise. - Observe the direction during this process. - Repeat the process. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe hydraulic motor

- Describe operating principle of hydraulic motor

- Describe tachometer

- Describe flow control valve

- Describe DCV

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Describe operating principle of hydraulic motor

- Describe tachometer

- Describe operating principle of hydraulic motor

- Describe tachometer

- Describe flow control valve

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Apparatus |
|  | Hydraulic bench |
|  | Hydraulic motors |
|  | Tachometer |
|  | Flow control valve |
|  | Direction control valve |
|  | Pipe hoses |
|  | Pressure gauges |
|  | Hydraulic bench |
|  | Hydraulic motors |
|  | Tachometer |
|  | Flow control valve |
|  | Direction control valve |
|  | Pipe hoses |
|  | Pressure gauges |

# Perform Pressure measuring instruments operations

Overview: This competency standard covers the skills and knowledge required to identify, and explain the construction and working of various pressure measuring devices.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. U Tube Manometer | - Set up the equipment. Connect one leg of manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. - Observe the difference in head between the two legs of the manometer and record. - Compute the pressure difference between the pipe inlet and outlet. - Repeat the experiment for various flow rates of fluid through the test pipe. |
| - CU2. Inclined manometer | - Set up the equipment. Connect one leg of inclined manometer to the inlet port and the other leg to the outlet port. Initiate the flow of fluid through the test pipe. - Observe the difference in head between the two legs of the manometer and record. - Compute the pressure difference between the pipe inlet and outlet. - Repeat the experiment for various flow rates of fluid through the test pipe. |
| - CU3. Micro Manometer | - Observe the enlarged ends of both tubes of the manometer - Adjust the instrument initially as p1 = p2. - Note the reading of the micrometer - Apply the unknown pressure difference - Observe the meniscus to move off the hairline, which can be restored to the initial position by raising or lowering the well (mercury sump). - Note the difference in the initial and final micrometer readings - Note the height of the mercury column and hence the pressure. |
| - CU4. Pressure measurement in The Mercury Barometer | - Take a reservoir and fill with any fluid - Put a glass tube in the reservoir in inverted state - Make the top portion of glass tube as air tight while the internal surface as complete vacuum - When the glass tube is dipped in the tank, an improper balanced pressure is created - Observe water from the tank rises in the tube - Note that the water would rise to particular height/ limit - Note that point where the water would stop lifting up - This pressure noted directly from the scale of the tube would give us the exact atmospheric/ air pressure |
| - CU5. Borden Tube | - Observe the device consisting of a metallic tube of elliptical section closed at one end A - Note the other end B being fitted to the gauge point where the pressure is to be measured. - Enter the fluid in the tube - Observe the tube tending to straighten. - Observe a pinion-sector arrangement pivoted with the pointer needle and already calibrated markings - Note the small elastic deformation of the tube is communicated to a pointer over a graduated dial. - Calibrate the device by subjecting it to various known pressures. |
| - CU6. Piezo Metric Tube | - Close all the valves of the tubes manifold. - Fill the tank until the lower part of the piezo metric tube is in contact with water. - Connect the pump to the upper plug of the tank with the non-return valve in the correct direction and pressurize the tank. - Observe if the water head in the piezo metric tube rises. - Observe the level is kept constant when we stop to pressurize with the pump. |
| - CU7. Elastic Diaphragms | - Observe the diaphragm fixed in a tubular member. - Observe the initial pressure on a diaphragm - Apply the unknown pressure from one side. - Observe a direct connection between diaphragm and pinion-sector - Observe a pinion-sector arrangement pivoted with the pointer needle and already graduated markings on the dial - Note the small elastic deformation of the diaphragm is communicated to a pointer over the graduated dial. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Difference between absolute pressure and gauge pressure

- Define Difference between u tube inclined and differential manometer

- Define Pressures as low as 0.025mm water column can be measured.

- Define What is a barometer

- Define What is the basic function of a barometer

- Explain the selection process of a pressure gauge

- Describe the types of Bourdon tubes

- Explain the purpose of different Bourdon tubes

- Explain the formula for calculating a static head

- Define Conversion of pressure gauge into a level gauge

- Define the units of Pressure

- Explain the necessity of putting the plug in the tank

- Explain natural frequency

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Differentiate between absolute pressure and gauge pressure

- Differentiate between u tube inclined and differential manometer

- Explain the selection process of a pressure gauge

- Explain the purpose of different Bourdon tubes

- Explain the formula for calculating a static head

- Explain the necessity of putting the plug in the tank

- Explain natural frequency

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | U Tube Manometer |
|  | Hydraulic Bench (if working fluid is water) |
|  | Air Compressor (if working fluid is air), Scale |
|  | Inclined Manometer |
|  | Hydraulic Bench (if working fluid is water) |
|  | Air Compressor (if working fluid is air), Scale |

# Operate Flow Regulating Devices (Valves)

Overview: This competency standard covers the skills and knowledge required to identify, evaluate and explain different types of valves.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Ball Valve | - Identify various parts of the ball valve including Handle, shaft, Ball, Seats and valve body - Observe a ball with a hole drilled through the centre swivel mounted within the valve body - Ensure the hole in the ball is orientated in the same direction as the pipe to allow flow - Observe the hole in the ball is oriented away from the direction of the pipe, the flowrate will be restricted and finally cut off completely when the hole is oriented at 90 degrees to the pipe direction - Note: that the hole in the ball is a lesser diameter than the nominal bore of the pipe |
| - CU2. Butterfly Valve | - Identify the gear box, handle, butterfly and the body of the butterfly valve - Observe a circular disc in the middle of fluid pipe. - Ensure the butterfly is oriented in the same direction as the pipe to allow full flow. - Oriented the butterfly away from the direction of the pipe, the flowrate will be restricted by the increased area of obstruction to the fluid - Keep the butterfly orienting away from the fluid until 90 degrees to completely restrict the flow |
| - CU3. Gate valve | - Identify gate and body of the gate valve - Identify the gate in the valve - Identify the shape of the gate with respect to the pipe diameter - Identify the direction of movement of the gate - Ensure the complete upward motion of the gate for full flow of the fluid - Ensure compete downward motion of the gate until the opposite end for complete restriction to the fluid flow. |
| - CU4. Diaphragm Valve | - Identify bonnet, diaphragm/ flexible sheet, seat and main body of the valve - Identify whether the valve is manual or pneumatic actuated - Observe the fluid flow rates from the pipe as the diaphragm is pushed towards the seat for complete obstruction to the flow |
| - CU5. Non Return Valve | - Identify the following parts in a non-return/ check valve: body, cover plate, disc, seats, hinge, hinge pin, nut, cotter pin, stud bolt, gasket, washer - Observe the fluid flow as the disk is released - Observe the disk being fixed as the flow direction is reversed |
| - CU6. Pressure Relief Valve | - Identify the following parts in pressure relief valve: set pressure adjusting screw, spring, disk holder, seat disk, nozzle, bonnet, bonnet plug and body - Identify various types of pressure relief valve - Observe system pressure and spring pressure - Observe the valve opening as the system pressure increases then spring pressure |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define a valve

- Identify applications of Ball Valve

- Explain the safety precautions of ball valve

- Explain the terminology of ‘butterfly valve’

- Identify its industrial applications

- Explain the safety precautions of Butterfly Valve

- Describe Gate/ Knife gate valve

- Explain the isolation valves

- Explain the precaution for knife gate valve

- Identify its Industrial Applications

- Explain the safety precautions of Gate valve

- Compare diaphragm and gate valve

- Explain the industrial applications of diaphragm valve

- Explain the safety precautions of Diaphragm Valve

- Mention and explain various types of non-return valve

- Explain unidirectional valve working mechanism

- Identify various industrial applications of Non-return valve

- Explain the safety precautions of Non Return Valve

- Explain the safety aspects of pressure relief valve

- Describe industrial applications of Pressure relief valve

- Explain the safety precautions of Pressure Relief Valve

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify a different types of valves

- Explain the safety precautions of ball valve

- Explain the terminology of ‘butterfly valve’

- Explain the safety precautions of Gate valve

- Explain the industrial applications of diaphragm valve

- Explain the safety precautions of Diaphragm Valve

- Mention and explain various types of non-return valve

- Explain various industrial applications of Non-return valve

- Describe industrial applications of Pressure relief valve

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Apparatus |
|  | Ball Valve |
|  | Section view drawing of Ball Valve |
|  | Butterfly Valve |
|  | Section view drawing of Butterfly Valve |
|  | Gate Valve |
|  | Section view drawing of Gate Valve |
|  | Diaphragm Valve |
|  | Section view drawing of |
|  | Non Return Valve |
|  | Section view drawing of Non Return Valve |
|  | Pressure Relief Valve |
|  | Section view drawing of Pressure Relief Valve |

# Maintain Pneumatic System

Overview: This competency standard covers the skills and knowledge required to Pneumatic system maintenance in process industry. It also deals with basic Pneumatic concepts, Test performed and verification of system. Maintenance of temperature & Perform repairing in pneumatic system.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Perform pneumatic system tests | - Apply desired pneumatic pressure as per system requirement - Verify pneumatic cylinder performance for normal operation - Test filters performance for clogging - Verify performance of gauges performance for normal operation - Verify performance of pneumatic Pumps performance for normal operation |
| - CU2. Verify Pneumatic system parameter and temperature | - Verify performance of pneumatic system performance for normal operation - Verify physical condition of pneumatic system performance for normal operation - Test compressing equipment parameter implementing recommended testing methods - Adjust equipment controls for desired parameters |
| - CU3. Diagnose fault in Pneumatic system | - Determine air leakage in pneumatic system implementing leakage testing methods. - Verify Pneumatic equipment performance for normal operation |
| - CU4. Replace equipment | - Remove desired equipment from Pneumatic system - Mount equipment in Pneumatic system |
| - CU5. Repair pneumatic system | - Diagnose causes of leakage in the system - Replace faulty equipment in pneumatic system implementing recommended testing methods |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Interpret Pneumatic drawing / manual

- Define physical properties of air

- Describe Volume and pressure in pneumatic system

- Relate Pressure, Volume and temperature in Gases

- Describe working of air compressor

- Enlist equipment of a typical pneumatic system

- Describe working of pneumatic equipment

- Describe the method of compress air Preparation

- Describe the production method and application of compressed air and vacuum

- Describe the method of pneumatic system performance test

- Describe the method of parameter testing in pneumatic system

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Define basic Pneumatic concepts, Test performed and verification of system.

- Perform to Maintain temperature

- Perform repairing in pneumatic system.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Measuring tapes |
|  | Mechatronics simulator station |
|  | Number punch and alphabet punch |
|  | Oil separator |
|  | Old and damaged instruments for better inspiration |
|  | Pinching Pliers |
|  | Pipe wrench |
|  | Piping and Tubing Training System |
|  | Pliers set (combination, side cutter and nose) |
|  | Pneumatic filters |
|  | Pneumatic valves |
|  | Pressure calibrator |

### Develop Entrepreneur Skills

# Investigate micro business opportunities

Overview: This competency describes the performance outcomes, skills and knowledge required to develop business ideas, and to investigate market needs and factors affecting potential markets.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Describe business ideas | P1. Gather information for business ideas from appropriate sources P2. List details of business ideas and opportunities P3. Research alternative business ideas in light of the resources available P4. Specify and list products and services to match business ideas P5. .Identify and research potential customer information for business ideas P6. Identify and take into account financial, business and technical skills available when researching business opportunities |
| - Identify market needs | P1. Collect information regarding market size and potential from appropriate sources P2. Investigate market trends and developments to identify market needs relative to business ideas P3. Gather market information from primary and secondary sources to identify possible market needs in relation to business ideas P4. Identify ethical and cultural requirements of the market and their impact on business ideas P5. Identify new and emerging markets and document their features P6. Identify and organise information on expected market growth or decline and associated risk factors |
| - Investigate factors affecting the market | P1. Identify projected changes in population, economic activity and the labour force that may affect business ideas P2. Identify movements in prices and projected changes in availability of resources P3. Review trends and developments and identify their potential impact on business ideas |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Define entrepreneurship.

- Explain the concept of entrepreneurship

- Explain the various types of enterprise that exist in the community

- Identify and interpret the terms and elements involved in the concept of enterprise

- Appreciate that the advancement of individual and society in general when entrepreneurship is adopted

- Explain various motivational factors that entrepreneurs possess and utilize.

- Exhibit the skills needed to assess and evaluate a risk

- Describe the outline of small enterprise

- Describe the creativity and innovation

- Apply the techniques for developing creative abilities

- Explain the resources of business idea

- Explain the collective and creative thinking

- Explain how to generate a business idea

- Appreciate the importance of, and possess techniques for identifying and assessing business opportunities.

- Identify the various entrepreneurial characteristics

- Access personal potential for becoming future entrepreneurs.

- Identify leadership qualities which are essential to the success of entrepreneurs

- Identify self- management skills and how they are important to be enterprising

- Apply a rational approach to make personal and business decisions

- Explain the steps for decision making and rating of decision making skills

- Apply the rules of negotiation for resolving business issues

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) in order to be competent in this competencystandard:

Evidence of the following is essential:

Thorough investigation of business opportunities and ideas

Clearly identified products/services and customer information for each business idea

Thorough collection and analysis of market information and associated factors relating to business ideas

Knowledge of ethical and cultural requirements.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Calculator |
|  | Ruler |
|  | Papers and Pencil |

# Develop and review a business plan.

Overview: This competency standard covers the process of developing and reviewing business for a small business enterprise.

It requires the application of knowledge and skills to determine the scope of the business plan, prepare a business plan, determine goals, trial systems, and document, monitor and review the business plan.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Determine scope of business | - Determine scope of the business plan and associated systems is determined in consultation with specialist personnel. - Access accurate informationfor inform business plan development - Account for and incorporate trends and seasonal variationsinto the business plan. - Account for strategic goals, targets and directionsof the enterprise in the development of the business plan - Comply Legal obligationsin developing the business plan. - Develop operational goals and targets to meet the enterprise strategic plan. - Identify and incorporate supply chains into the business plan. - Identify risk management needsare within the business plan. - Incorporate trial systems in order to test budgetary impact and operational potential prior to full implementation of the business plan. - Set clear and measureable indicators of operational performanceto allow for realistic analysis of performance. |
| - Prepare business plan | P1. Develop operational goals and targets to meet the enterprise strategic plan. P2. Identify and incorporate supply chains into the business plan. P3. Identify risk management needs are within the business plan. P4. Incorporate trial systems in order to test budgetary impact and operational potential prior to full implementation of the business plan. P5. Set clear and measureable indicators of operational performance to allow for realistic analysis of performance. |
| - Document and review business plan | - Include fiscal and operational systemsthat enhance performance management and suit enterprise requirements. - Incorporate resource considerations the business plan. - Document accurately and clearly communicate business Plan to all relevant parties. - Monitorto identify strengths, weaknesses and areas for improvement performance against the business plan - Make recommendations to improve the business plan and associated systems as required. |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Appreciate the importance of business plan

- Explain the process of writing a business plan

- Develop feasibility for a business idea

- Realize the problem that may be encountered when starting a small business/Enterprise

- Develop a business plan for a small business on the standard format

- Evaluate the business plan in a real market satiation

- The knowledge requirements for this competency standard are listed below:

- budgeting

- forecasting

- operational systems

- relevant industrial awards and agreements

- communication techniques

- logical and analytic methods

- profit and loss and cash flow systems

- working knowledge of environmental, OHS, industrial relations, taxation, corporate and industry legislation as they relate to the enterprise

Critical Evidence(s) Required

The Student needs to produce following critical evidences in order to competent in this competency standard.

Competence in develop and review a business plan requires evidence that demonstrates ability to scope the business plan and determine key objectives and targets.  It also requires competence in specifying key performance targets, assessing the relevance of the business plan, and trial systems.  The business plan also needs to be documented and reviewed.

The skills and knowledge required to develop and review a business plan must be transferableto a different work environment.  For example, if competence is demonstrated in developing a business plan for a small enterprise, it must also be evident in reviewing a business plan in medium or large enterprise environment.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Construction Lab Tools |
|  | Rule,tape, square, hammer, hand saw, handplane, chisel,shovel, wheelbarrow, sledge hammer, pick. |
|  | Number punch and alphabet punch |

# Organise finances for the micro business

Overview: This competency standard describes the performance outcomes, skills and knowledge required to investigate the financial capacity to enter into a micro business, to determine the projected cash flow, to source finances and to monitor the profitability of the business.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Ascertain own financial position and the ability to provide capital/equity for the business | P1. Realistically detail personal, family or community financial situation in terms of funds available and commitments already incurred P2. Determine equity finance and assets available for micro business from personal, family or community sources |
| - Determine projected cash flow for the business | P1. Determine the level of forecast business activity over a year and the business mix P2. Estimate establishment costs for the business and repayment schedule for borrowings P3. Calculate the monthly variable and fixed costs needed to conduct business activity over a year P4. Estimate personal drawings needed to be taken from the business P5.Estimate the monthly income generated by the business for a year based on price per unit item or hourly charge rate for labour P6. Develop a cash flow budget for the first year of business operation P7. Seek professional advice to estimate goods and services tax and operating finance required for the business |
| - Source the required funds to establish the business | P1. Estimate required funding to establish and run the business based on expected sales and activity levels, available finances and commitments P2. Investigate methods of accessing alternative sources of finance P3. Identify strategies for meeting financial obligations P4. Implement plans to access available funds as required |
| - Monitor profitability of the business | P1. Maintain and review monthly expenditure and income records P2. Compare equity at beginning and end of a year to estimate business performance P3. Assess the financial viability of the business after a year of operation P4. Seek professional advice on depreciation, insurance and tax implications of the business |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Basic budgeting

- basic costing for the business

- financial commitments and requirements

- financial reports and terminology

- methods and relative costs of obtaining finance

- own financial position

- Sources of advice and assistance.

Critical Evidence(s) Required

The Student needs to produce following critical evidences in order to competent in this competency standard.

Evidence of the following is essential:

- investigation of own financial position and needs

- investigation of projected cash flow for the business

- estimation of the funding needed to establish and operate the business

- assessment of the financial viability of the business

- Knowledge of basic budgeting.

# Manage human resources

Overview: This competency standard covers the skills and knowledge required to manage human resources and to manage and develop human resources to achieve organization’s operational objectives

| Competency Unit | Performance Criteria |
| --- | --- |
| - Lead and motivate people | P1.Establish goals for people and teams to optimise achievement in work tasks. P2.Take into account the capabilities of people and teams. P3. Provide advice and support sensitive to the individual's needs to people in the performance of their duties. P4. Undertake activities to achieve commitment to common goals. P5. Recognise and encourage initiative and innovation P6. Recognise and communicate achievements within the organisation. |
| - Undertake human resource planning | P1.Determine human resource needs within the anticipated operational needs and allocated budget. P2. Analyse alternatives to staffing levels which clearly demonstrate returns to the organisation. P3. Develop contingency plans for staffing which meet key provisions of the human resources plan. P4. Compare existing competencies of staff with the needs of the work group. P5. Plan staffing levels and negotiate with stakeholderswithin the organisational framework to achieve maximum efficiency of operations. |
| - Develop and facilitate performance | P1.Negotiate performance criteria individuals, teams and work groups. P2. Review performance criteria as circumstances change. P3. Conduct performance appraisal based on clearly established and agreed performance criteria. P4.Identify and propose the total performance development system strategies to rectify performance shortfalls and recognise success. P5. Address performance problems confidentially and in a constructive and timely manner, in line with relevant organisational procedures. P6. Make selections, transfers and promotions in accordance with organisation policies and supported with documented information. P7. Develop and implement mechanisms for the identification of human resource development needs within the work group taking account of the strategic plan for the organisation. |
| - Facilitate training, education and development opportunities | P1.Make information on planned training events widely available throughout the organisation. P2. Include training, education and development plans as part of individual/team performance plans. P3. Facilitate individual/team access to, and participation in, training, education and development opportunities. P4.Contribute coaching and mentoring effectively to the training, education and development of personnel in an environment of change. P5.Enhance training, education and development opportunities of individual, team and organisational performance. P6. Create workplace environment is which facilitates training, education and development |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe the hiring method/Procedures

- Describe the term & conditions of services and job description for various employments

- Describe the characteristics of successful sales personals

- communication principles

- conflict resolution principles and practice

- Equal Employment Opportunity

- grievance procedures

- interpersonal relations

- leadership theory and principles

- management principles and practice

- Occupational Health and Safety

- training and education principles

- training need analysis

Critical Evidence(s) Required

The Student needs to produce following critical evidences in order to competent in this competency standard.

- It is essential that competence be demonstrated in the application of human resource management in a wide range of contexts in achieving the organisation's objectives.

- Evidence should be gathered over a period of time in a range of actual or simulated workplace environments.

- Evidence of competent performance should be obtained by observing an individual in a management role within the workplace or exercise or operational environment. Knowledge may be assessed through written assignments, project reports, debriefings and action learning projects.

# Market products and services

Overview: This competency standard covers the skills and knowledge required to market products and services

| Competency Unit | Performance Criteria |
| --- | --- |
| - Analyze market information | P1. Identify, research and analyse existing or new markets for existing or new products or services using techniques to ensure reliable data P2. Analyse past trends and developments to determine market variability and associated risks P3. Develop gross margin budgets to account for market variability P4.Identify and evaluate competing products to determine strengths and weaknesses of own products P5. Monitor market environment to ensure information is current and reliable P6. Identify the legal, ethical and environmental constraints of the markets and their effect on the enterprise P7. Identify product specifications that suit market requirements and price advantage at the time P8. Present clear and concise information to the enterprise management team. |
| - Identify and evaluate factors to include in a marketing plan | P1. Identify and evaluate production processes to ensure required product specifications are met P2. Identify and assess alternative selling strategies and techniques to identify marketing targets and methods P3. Identify and assess distribution channels and their role in your marketing strategies P4. Ensure the data used is reliable and the market environment and trends are substantiated P5. Evaluate the role of marketing professionals in providing advice |
| - Develop a marketing plan for your products and services | P1. Establish marketing objectives based on current and potential product specifications P2.Select appropriate production processes to ensure product specifications are met P3. Select selling strategies to ensure required prices are achieved P4. Select appropriate distribution channel options to ensure access to target markets is achieved efficiently and appropriately P5.Establish time-frames for production, distribution and selling activities P6.Develop a gross margin budget to demonstrate the cost effectiveness of the marketing plan P7.Develop partial gross margin budgets to account for market variability |
| - Determine promotional strategies | P1. Prepare and record detailed plans for promotional activities P2. Outline objectives, level of exposure and available markets P3. Ensure strategies take account of time management and scheduling issues, and resource constraints P4. Create promotional materials that enhance the product and commercial presentation P5.Record and communicate priorities, responsibilities, timelines and budgets for promotional activities. |
| - Implement marketing activities | P1. Schedule planned marketing activities within appropriate timeframes P2. Develop measurable performance targets that meet business plan objectives P3. Organise distribution channels and ensure product and service information is accurate and readily available to clients P4. Implement marketing activities within budgetary constraints to meet legal, ethical and enterprise requirements |
| - Evaluate marketing performance. | P1. Review the established marketing objectives to ensure they remain viable P2. Make an objective assessment of the marketing plan and its implementation by a comparison of valid and reliable data against the established objectives P3.Assess product, pricing and distribution policies in relation to market changes, marketing objectives and enterprise requirements P4. Identify areas of positive marketing performance and take corrective action to remedy poor marketing performance areas P5. Document and distribute information for continual analysis and effective planning management |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe the life cycle of product

- Identify the various ways of selecting suppliers,

- Explain the inventory management of stock, raw material and finished goods etc.

- Appreciate the importance of financial record keeping in a small business

- Explain techniques to keep cost as low as possible

- Develop balance sheet for a small enterprise

- Explain the operating cycle concept

- Explain the income tax computation procedure for a small business

- Explain the basic scheme of sales tax

- Explain the assessment procedure for returns and filling of returns.

Critical Evidence(s) Required

The evidence required to demonstrate competency in this unit must be relevant to workplace operations and satisfy holistically all of the requirements of the performance criteria and required skills and knowledge and include achievement of the following:

- identify the marketable features of the product and potential markets

- develop a range of marketing alternatives

- collect and analyses data to assess alternatives in a marketing plan

- evaluate performance targets and recommend modifications or improvements

- implement and evaluate a marketing plan

- Monitor and review business performance

Overview: This competency standard covers the skills and knowledge required tomonitor and review business performance

| Competency Unit | Performance Criteria |
| --- | --- |
| - Evaluate commercial performance | P1. Gather and analyse data relating to enterprise performance to identify historical and current performance. P2. Review and analyse operational structures to determine the suitability of organisational processes to enterprise objectives. P3. Evaluate enterprise strengths and weaknesses against market conditions to determine current and future capacities. P4. Evaluate enterprise objectives are to identify variations and scope for future development. |
| - Allocate and co-ordinate business resources | P1. Identify and communicate roles and responsibilities of personnel. P2. Identify resource requirements for enterprise and cost them using standard financial analysis techniques. P3. Calculate costs of ensuring sustainability of enterprise operations and factor into business planning for the enterprise. |
| - Identify performance requirements | - Develop realistic performance indicators within available timeframes and resources - Identify and minimize factors inhibiting performance against objectives. - Monitor and assess market conditions based on relevant data. - Prepare and incorporate strategies and programs to promote the sustainability of operations into enterprise procedures. |
| - Review business performance | - Review regularly enterprise operations to identify opportunities for improvements in performance. - Monitor and anticipate impact of natural conditions on enterprise to assess sustainability of resource use. - Compare costs and estimates with resource allocation. - Determine operational plans to determine schedule of activities |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- rates of return for products and/or services

- financial analysis techniques

- structure and operation of small businesses relevant State/Territory Occupational Health and Safety (OHS)

- legislative requirements

- environmental conditions, positive environmental practices and negative impact minimisation measures

- human resource requirements for the enterprise

- transport requirements for the enterprise

- Enterprise/property improvement requirements.

- market performance in commodities

- statutory marketing requirements

Critical Evidence(s) Required

The evidence required to demonstrate competency in this unit must be relevant to workplace operations and satisfy holistically all of the requirements of the performance criteria and required skills and knowledge and include achievement of the following:

- gather and analyse data relating to enterprise performance

- review operational structures to determine effectiveness

- identify available resources to assess capacity

- develop realistic performance indicators

- review enterprise operations against performance indicators

- Plan to improve business performance by addressing results of review.

# Negotiate for resolving business issues

Overview: This competency standard covers the skills and knowledge required to negotiate for resolving business issues

| Competency Unit | Performance Criteria |
| --- | --- |
| - Develop and implement issue resolution and grievance procedures | P1. Establish problem solving/issue resolution procedures within legislative requirements and organisation's guidelines. P2. Manage grievances and complaints are in a timely and caring way to optimise likelihood of a favourable outcome for all parties and in line with organisational objectives and procedures. P3.Document and communicate individual's rights and obligations under industrial awards/agreements and legislation are in a clear and concise manner and in appropriate language. P4. Conduct hearings, interviews and meetings within the principles of industrial democracy and participative, consultative processes. P5. Identify and invite all relevant industrial parties to participate in the resolution process. |
| - Manage disciplinary matters | P1.Implement disciplinary matters in accordance with organisation's procedures. P2. Conduct investigations in a caring and confidential manner to maintain performance and morale. P3. Provide feedback promptly. P4.Institute appeals processes in accordance with organisational procedures. P5. Recognize and recommend deficiencies in procedures for changes made |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe Concrete ingredients ratio

- Explain batching and finishingtechniques

- Identify concretematerials

- Explain methods of concreteplacement

- State curingtimes for concrete.

- Describe fresh concrete levellingtechniques

- Describe types, characteristics, usesandlimitations ofplant,toolsand equipment

Critical Evidence(s) Required

The Student needs to produce following critical evidences in order to competent in this competency standard.

- useahandtrowelandpowertroweltofinishaminimumof10 square metresofconcrete in foundation.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Construction Lab Tools |
|  | - Chutes, Linepumps - Measuring tapes and rules,mechanized dumpers - Rakes, Screed boards,shovels,stipple devices - Trowels troweling machines - Vibrators, Wheel barrows, kibble, pump orhopper. |
|  | Number punch and alphabet punch |

# Lead small teams

Overview:

This unit describes the outcomes required to lead small teams including setting and maintaining team and individual performance standards.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Facilitate team development | - Identify work requirements, standards and purpose to team members. - Assist team to develop objectives, targets and key performance indicators relevant its purpose and workplace goals. - Allocate duties regard to the skills required to properly - undertake the assigned task and according to company policy - Identify roles, responsibilities and expectations of each team member - Disseminate and discuss performance expectations to individual team members. |
| - Motivate and build the team | - Develop positive and constructive relationships with and between team members - Facilitate team communication processes - Involve team members in the process of examining risks and options and making decisions, to ensure acceptance and support. - Encourage individual and team efforts and contributions - Strengths and weaknesses of team members are determined and sharing of work tasks is promoted to up skill team members. - Recognize team members’ queries and discuss and deal with it. |
| - Facilitate and monitor team effectiveness | - Monitor the implementation of work plan and team and individual performance against agreed strategies, targets and standards, according to workplace policies and procedures. - Monitor performance against defined performance criteria and/or assignment instructions and corrective action taken if required. - Support team in identifying and resolving problems that may impede performance and to suggest improvements in teamPerformance. - Consult team members in any review and revision of team objectives and goals. - Address performance issues which cannot be rectified within the team to appropriate personnel according to employer policy. - Refer concerns of a team and individual are referred to next level of management or appropriate specialist and conduct negotiations on their behalf. - Keep team members inform of any changes in the priority allocated to assignments, or tasks which might impact on client/customer needs and satisfaction. - Monitor team operations to ensure that internal or external employer/client needs and requirements are met. - Provide follow-up communication on all issues affecting the team - Conduct team meetings to review work operations and address issues according to workplace policies and procedures. - Support team in identifying and resolving problems that may impede performance and to suggest improvements in team performance. - Consult team members in any review and revision of team objectives and goals. - Raise any inappropriate values and standards exhibited in the workplace with the person concerned. |

Knowledge and understanding

The Student must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- K1: Explain conflict resolution techniques

- K2: Explain management styles

- K3: Describe methods of monitoring performance

- K4: Outline the relevant legal requirements

- K5: Outline strategies for dealing effectively with team member complaints or grievances and

- K6: Explain team dynamics and facilitation processes

- K7: Describe communication skills

- K8: State workplace policies and procedures

Critical Evidence(s) Required

The Student needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to lead small teams. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.