National Competency Standards Level-4 of

Solid Waste Management (Supervisor)

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Level 4 in Solid Waste Management (Supervisor). This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts in Industry, Academia, and TVET experts of TEVTAs, BTEs, and universities work diligently to make this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility for all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that the development of Skill Standards is a dynamic and ongoing process, and the developed competency standards need periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan, will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	4

1.1	Date of development:	4

1.2	Date of Validation	4

1.3	Date of review:	4

1.4	Entry requirements of trainee:	4

1.5	Trainer Qualification Requirement	4

1.6	Codes of qualification (ISCED)	4

2.	Summary of competencies	4

3.	Qualification Leveling and Packaging	5

4.	Detail of Competency Standards	6

1022-001-GC: Manage Workplace Safety Systems	6

1021CSWMSWCS01: Manage Fleet Mobilization	10

1021CSWMSWCS02:  Supervise waste collection operations	12

1021CSWMSWCS03: Manage Material Recovery Facility (MRF)	15

1021CSWMSWCS04: Manage Temporary Collection Point (TCP) Operations	16

1021CSWMSWCS05: Manage Scientific Landfill Site Operations	18

1021CSWMSWCS06: Handle Community Service Issues	20

1021CSWMSWCS07: Manage Event-Based Waste Operational Activities	22

0611-001-GC Perform Basic IT Operations	24

1022-001-GC: Manage Documentations	26

On-Job Training	27

5.	Members of the Qualification Development & Review Committee	28

6.	Members of Qualification Validation Committee	29

# Introduction

This qualification is designed to develop competent, skilled, and industry-ready supervisors in Solid Waste Management through a Competency-Based Training and Assessment (CBT&A) approach aligned with the National Vocational Qualifications Framework (NVQF) of Pakistan. It focuses on practical and supervisory competencies required to effectively manage municipal and private sector waste management operations in real workplace environments.

The program develops competencies in workplace safety management, fleet mobilization, waste collection supervision, material recovery facility operations, temporary collection point (TCP) management, scientific landfill site operations, community service issue handling, event-based waste operational activities, documentation management, and basic IT operations. It emphasizes operational planning, workforce supervision, resource coordination, safety compliance, environmental sustainability, and service quality management.

In addition to technical and supervisory competencies, the qualification integrates essential employability skills including leadership, communication, professionalism, problem-solving, teamwork, reporting, and digital workplace practices. Graduates will be able to effectively supervise and manage waste management operations while contributing to improved public health, environmental protection, and efficient municipal service delivery.

| # Date of development: | April, 2026 |
| --- | --- |
| # Date of Validation | June 19 2026 |
| # Date of review: | April, 2028 |
| # Entry requirements of trainee: | NVC Level Level-3 in Solid Waste Management (Waste Collection Machine Operator) and Matric |
| # Trainer Qualification Requirement | Graduation with Minimum 1-year relevant experience OR Intermediate with 3-year relevant experience OR NVC Level-4 in Solid Waste Management (Supervisor) with 3-year experience |
| # Codes of qualification (ISCED) | Code: 1021CSWMSW003 Description: National Vocational Certificate Level 4 in Solid Waste Management (Supervisor) |

# Summary of competencies

| Sr No | Competency Standard | NVQF Level | Category | Estimated Contact hours |  |  | Credit Hours |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Theory | Practical | Total |  |
| 1 | 1022-001-GC Manage Workplace Safety Systems | 4 | Generic | 15 | 48 | 63 | 3.9 |
| 2 | 0011-001-GC Supervise Work Place Professional practices | 4 | Generic | 15 | 48 | 63 | 3.9 |
| 3 | 1021CSWMSWCS01 Manage Fleet Mobilization | 4 | Technical | 15 | 57 | 72 | 4.4 |
| 4 | 1021CSWMSWCS02 Supervise Waste Collection Operations | 4 | Technical | 15 | 57 | 72 | 4.4 |
| 5 | 1021CSWMSWCS03 Manage Material recovery facility | 4 | Technical | 15 | 57 | 72 | 4.4 |
| 6 | 1021CSWMSWCS04 Supervise Temporary Collection Point Operation | 4 | Technical | 16 | 57 | 73 | 4.5 |
| 7 | 1021CSWMSWCS05 Manage Scientific Landfill Site Operation | 4 | Technical | 15 | 57 | 72 | 4.4 |
| 8 | 1021CSWMSWCS06 Handle Community Service Issues | 4 | Technical | 15 | 48 | 63 | 3.9 |
| 9 | 1021CSWMSWCS07 Manage Event-Based Waste Operational Activities | 4 | Technical | 16 | 48 | 64 | 4.0 |
| 10 | 0611-001-GC Perform Basic IT Operations | 4 | Technical | 13 | 39 | 52 | 3.3 |
| 11 | 1022-001-GC Manage Documentation | 4 | Functional | 12 | 39 | 51 | 3.2 |
| 12 | On Job Training (OJT) | 4 | Technical | 0 | 240 | 240 | 6 |
|  | Total |  |  | 162 | 795 | 957 | 50.0 |
|  | % age |  |  | 17% | 83% |  |  |

# Qualification Leveling and Packaging

| 1 | Supervisor Waste Collection Operations | 0011-001-GC 1022-001-GC 1021CSWMSWCS01 1021CSWMSWCS02 |  | 15 | 3 months |
| --- | --- | --- | --- | --- | --- |
| 2 | Supervisor Scientific Landfill Site Operation | 1021CSWMSWCS03 1021CSWMSWCS04 1021CSWMSWCS05 |  | 15 | 3 Months |

# Detail of Competency Standards

## 1022-001-GC: Manage Workplace Safety Systems

Overview: This Competency Standard describes the skills, knowledge, and attitude required to manage workplace safety systems. It includes the ability to implement safety procedures, monitor PPE usage, conduct safety briefings, and ensure compliance with safety standards.

| Competency Units | Performance Criteria |
| --- | --- |
| Implement safety procedures | - Apply safety procedures at workplace - Ensure operational procedures adaptation - Conduct safety inspections - Update procedures when required |
| Monitor PPE usage | - Ensure PPE availability - Ensure correct usage by workers - Identify non-compliance - Take corrective measures |
| Conduct safety briefings | - Plan safety meetings - Deliver safety instructions - Explain hazards and controls Co-workers - Ensure worker’s understanding |
| Manage emergency responses | - Identify emergency situations - Activate response as per procedures - Coordinate in emergency situation - Report emergencies to authorities |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Safety management systems

- Risk assessment techniques

- Safety policies and compliance

- Emergency management at Waste storage/station/dump yard

- Emergency management on the road/ during waste transportation

- Need of regular safety inspections

- compliant and non-compliant practices

- Methods to monitor PPE usage and enforce compliance

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Implement safety procedures at workplace

- Monitor PPE usage within team

- Conduct safety briefings

- Ensure compliance with safety standards

Tools and equipment required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Personal Protective Equipment (PPE) Kit (Helmet, Safety Gloves, Safety Boots, Dust Mask, Reflective Jacket, Ear plugs) | 25 sets |
| 2 | First Aid Kit (complete box with basic medicines) | 02 |
| 3 | Fire Extinguisher (A,B,C Type) | 02 |
| 4 | Reflective Traffic Cones & Warning Triangles | 10 |
| 5 | Spill response kits | 02 |
| 6 | Emergency stretchers | 01 |
| 7 | Raincoat & Long Boots | 25 |
| 8 | Heavy-duty Reusable Gloves (for waste handling & segregation) | 10 |
| 9 | Whiteboard / smart board | 1 |
| 10 | Computer | 1 |
| 11 | Projector - Multimedia | 1 |
| 12 | Printer | 1 |
| 13 | Training Manuals | 25 |
| 14 | Emergency stretchers | 1 |

The consumables required for this competency standard are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Notebook | 25 |
| 2 | Pen/pencil | 25 |
| 3 | White Board Markers (Blue, Black, Green, Red) | 01 packet each |
| 4 | Disposable Gloves | 100Pairs |
| 5 | Masks | Boxes |
| 6 | Soap | 6 |
| 7 | Hand Sanitizer | 2 |

# 0011-001-GC: Supervise Workplace Professional Practices

# Overview: This competency standard outlines the skills, knowledge, and attitude required to supervise workplace professional practices, including monitoring workplace discipline, punctuality, professional behavior, ethical conduct, teamwork, and compliance with organizational rules and standards in accordance with workplace requirements.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Ensure workplace discipline implementation | - Communicate workplace rules and policies to team members - Verify understanding of workplace rules. - Monitor compliance with rules during operations - Check attendance and punctuality of workers - Identify absenteeism, late arrivals - Take corrective actions for the violations |
| CU2. Supervise professional practices | - Demonstrate professional workplace behavior - Guide workers on professional interaction - Identify ethical concerns at the workplace - Ensure compliance with ethical standards - Address unethical and unprofessional practices |
| CU3. Coordinate teamwork to achieve operational targets | - Plan daily team activities according to operational requirements - Assign roles and responsibilities according to tasks - Align team activities with operational targets - Monitor task progress and completion - Ensure availability of required resources |
| CU4. Support collaboration for task completion. | - Provide guidance to workers during tasks - Assist workers facing operational difficulties - Encourage teamwork and idea sharing - Facilitate joint problem-solving among staff - Support cooperative work practices for timely task completion |
| CU5. Resolve workplace conflicts | - Observe workplace conflicts - Identify causes of disputes among workers - Counsel workers according to organizational procedures - Mediate to resolve issues - Take appropriate action against inappropriate behavior. |
| CU6. Ensure continuous improvement | P1. Evaluate team productivity and efficiency P2. Identify performance gaps among workers P3. Provide constructive feedback for improvement P4. Recognize and acknowledge good performance P5. Encourage consistent performance. P6. Recommend improvement actions for better workplace practices |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Workplace professionalism, discipline, and ethical standards

- Workplace rules, policies, and methods of communication and enforcement

- Professional and non-professional behavior and workplace conduct

- Importance of punctuality, attendance, and compliance.

- Misconduct types and basic disciplinary procedures.

- Team roles, responsibilities, and basic team management concepts.

- Collaboration, communication, motivation, and teamwork effectiveness.

- Performance monitoring and identification of performance gaps.

- Conflict situations and basic handling and coordination methods

- Promotion of ethical, disciplined, and performance-based workplace culture

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Ensuring compliance with workplace rules and discipline

- Monitoring attendance, punctuality, and workplace discipline

- Guiding team on professional behavior and workplace conduct.

- Addressing misconduct and unprofessional behavior.

- Supervising and coordinating team activities and tasks.

- Supporting team members during task execution.

- Resolving conflicts within the team.

- Monitoring team performance and ensuring standards compliance

Tools and equipment required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Whiteboard / smart board | 1 |
| 2 | Computer | 1 |
| 3 | Projector - Multimedia | 1 |
| 4 | Printer | 1 |
| 5 | Training Manuals | 25 |
| 6 | Digital reporting devices | 2 |

The consumables required for this competency standard are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Notebook | 25 |
| 2 | Pen/pencil | 25 |
| 3 | White Board Markers (Blue, Black, Green, Red) | 01 packet each |
| 4 | Disposable Gloves | 100Pairs |
| 5 | Masks | Boxes |
| 6 | Soap | 6 |
| 7 | Hand Sanitizer | 2 |
| 8 | Logbook / Daily Record Register (for reporting, task completion & waste data) | 25 |

## 1021CSWMSWCS01: Manage Fleet Mobilization

Overview: This Competency Standard defines the skills, knowledge, and attitude required to supervise fleet mobilization from yard to GTS, including planning and monitoring fleet dispatch operations, ensuring vehicle, driver, and documentation readiness, and maintaining adherence to schedules and routes. It also covers coordinating communication between yard and GTS, managing on-road incidents and contingencies to minimize disruptions, and optimizing fleet utilization and operational efficiency through data-driven improvements.

| Competency Units | Performance Criteria |
| --- | --- |
| Supervise Fleet Dispatch Operations | - Plan dispatch of vehicles according to operational schedules and routes - Verify readiness of fleet, before mobilization - Monitor fleet movement and adherence to assigned routes and timelines - Implement corrective actions for dispatching issues |
| CU2. Monitor Fleet operations | - Oversee fleet movement schedules. - Verify vehicle allocation. - Track fuel consumption in logbooks. - Monitor driver performance - Report fleet operational issues |
| CU3. Coordinate with disposal sites staff | - Share waste delivery schedules with site staff. - Communicate vehicle unloading procedures. - Ensure compliance with environmental guidelines. - Resolve issues raised during disposal operations. |
| CU4 Manage On-Road incidents | - Assess on-road incidents involving waste collection vehicles - Minimize service disruption as per SOPs. - Communicate incidents to management |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Fleet dispatch planning and scheduling principles

- Vehicle readiness requirements including safety checks and documentation compliance

- Route planning, monitoring of fleet movement, and adherence to operational timelines

- Identification and resolution of dispatch delays and operational bottlenecks

- Communication systems and protocols between yard and GTS operations

- Effective coordination techniques for operational updates and instruction flow

- Documentation practices for communication records, operational issues, and reporting

- Identification and management of on-road incidents and emergency response procedures

- Safety requirements and risk mitigation measures during transport operations

- Incident reporting procedures and preventive action planning

- Fleet utilization concepts and performance monitoring indicators

- Analysis of operational data for route optimization and efficiency improvement

- Principles of fuel efficiency, turnaround time reduction, and resource optimization

- Methods for improving overall fleet productivity and operational effectiveness

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

  - Supervise fleet dispatch operations in accordance with schedules and routes

  - Coordinate communication between yard and GTS effectively

  - Manage on-road incidents and contingencies to ensure continuity of operations

  - Optimize fleet utilization and operational efficiency

Tools and equipment required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Computer / Laptop | 1 |
| 2 | Printer | 1 |
| 3 | GPS Tracking System / Fleet Monitoring Software | 1 |
| 4 | Communication Devices (Walkie-Talkie / Mobile Phones) | 6 |
| 5 | Vehicle Inspection Checklist Forms | 25 |
| 6 | Fleet Register / Logbooks | 25 |
| 7 | Dispatch Board / Whiteboard | 1 |
| 8 | CCTV Monitoring System (Yard/GTS) | 1 |
| 9 | Fuel Monitoring System / Meter | 1 |
| 10 | Basic Tool Kit for Vehicles | 5 |
| 11 | First Aid Kit | 2 |
| 12 | Fire Extinguishers | 2 |
| 13 | PPE Kit (Helmet, Safety Gloves, Safety Boots, Dust Mask, Reflective Jacket, Ear plugs) | 25 |
| 14 | Incident Reporting Forms | 25 |

The consumables required for this competency standard are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Notebooks / Log Sheets | 25 |
| 2 | Pens / Pencils | 100 |
| 3 | White Board Markers (Blue, Black, Green, Red) | 01 packet each |
| 4 | Disposable Gloves | 100 Pairs |
| 5 | Face Masks | As req. |
| 6 | Soap | 5 |
| 7 | Hand Sanitizer | 2 |
| 8 | Fuel (Diesel / Petrol) | As req. |
| 9 | Lubricants / Engine Oil | As req. |
| 10 | Cleaning Materials (Detergents/Disinfectants) | 5 |

## 1021CSWMSWCS02:  Supervise waste collection operations

Overview: This Competency Standard describes the skills, knowledge, and attitude required to manage waste collection operations. It includes the ability to plan waste collection workflow, supervise segregation process, ensure contamination control of waste, and manage waste storage, coordinate with waste collection staff & Monitor waste collection performance.

| Competency Units | Performance Criteria |
| --- | --- |
| Plan waste collection workflow | - Estimate waste quantity - Plan collection routes and sequence of activities. - Allocate resources for collection tasks. - Optimize workflow for timely waste removal |
| Supervise segregation process | - Monitor pre-segregation activities - Ensure classification of recyclable materials - Verify segregation standards |
| Manage waste storage | - Identify Storage point - Designate point for different waste categories - Ensure safe storage - Take measures against spillage. |
| Ensure contamination control of waste | - Inspect the quality of sorted materials - Enforces separation of hazardous, recyclable, and organic waste without cross-contamination. - Manage labeling process Implement corrective measures for quality improvement |
| Coordinate with waste collection staff | - Communicate with collection stakeholders - Arrange transportation of waste - Implement waste dispatch schedule - Maintain coordination records |
| Monitor waste collection performance | - Measure quantity of waste - Record type-wise waste output - Evaluate efficiency of waste collection operations - Report recycling performance to management |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Waste Management Fundamentals

- Resource & Workforce Management

- Operational Procedures of Waste Collection

- Waste collection equipment & vehicle

- Community & Stakeholder Engagement for waste collection

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) to be competent in this competency standard:

- Implement Waste collection systems under organizational guidelines.

- Monitor sorting processes according to checklist

- Manage waste storage according to organizational guidelines.

- Coordinate activities in the waste collection operation

Tools and equipment required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Whiteboard / smart board | 1 |
| 2 | Computer | 1 |
| 3 | Projector - Multimedia | 1 |
| 4 | Printer | 1 |
| 5 | Training Manuals | 25 |
| 6 | Weighing scale (basic platform scale) | 2 |
| 7 | Sorting Conveyor | 1 |
| 8 | Waste bin, labeled | 6 |
| 9 | Moisture meter | 6 |
| 10 | Contamination test kits | 6 |
| 11 | Storage containers | 6 |
| 12 | Pallets & racks | 6 |
| 13 | Barcode/labeling system | 02 |
| 14 | Performance dashboards | 02 |
| 15 | Record-keeping software | 02 |

The consumables required for this competency standard are provided below:

| S. No. | Items | Quantity |
| --- | --- | --- |
| 1 | Notebook | 26 |
| 2 | Pen/pencil | 26 |
| 3 | White Board Markers (Blue, Black, Green, Red) Pack of 12 | 6 |
| 4 | Disposable Gloves | 100 Pairs |
| 5 | Single-use masks | 200 |
| 6 | Soap | 6 |
| 7 | Hand Sanitizer | 3 |
| 8 | Sealed collection bags | 75 |
| 9 | Barcode or RFID tags | 50 |
| 10 | Labels and stickers | 50 |
| 11 | Spill kits | 15 |
| 12 | Twine | 05kg |
| 13 | zip ties (assorted size, pack of 100) | 08 |
| 14 | Tape | 24 |
| 15 | Batteries | 24 |
| 16 | Paper sheets | 2 rims |
| 17 | Cleaning supplies (assorted) | 10kg |
| 18 | Drinking water supply | 30 liter |

## 1021CSWMSWCS03: Manage Material Recovery Facility (MRF)

Overview: This Competency Standard describes the skills, knowledge, and attitude required to supervise Material Recovery Facility. It includes the ability to inspect generated waste, sort operational oversights, control baling process & coordinate dispatch process.

| Competency Units | Performance Criteria |
| --- | --- |
| Inspect generated waste | - Verify source segregation - Ensure weighbridge records are accurate - Detect contamination in incoming loads. - Direct corrective sorting measures |
| Sort operational oversight | - Monitor Manual sorting activity - Monitor mechanical sorting activity - Ensure equipment operating condition - Ensure equipment log updating |
| Control bailing process | - Oversee baling of recyclables - Ensure safe staking - Ensure labeling - Prevent cross-contamination between material categories. |
| Coordinate dispatch operation | - Provide disposal schedules - Ensure smooth dispatch operations - Resolve coordination issues |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in the competency standard of ‘’Supervise Material Recovery facility’’. This includes the knowledge of:

- Waste Stream Identification

- Waste storage

- Material handling

- Wasted Material Recovery Facility is patch process

Critical Evidence Required:

The candidate needs to produce the following critical pieces of evidence to be competent in this competency standard:

- Inspect generated waste for categorization;

- Sort operational oversight manually;

- Control bailing process according to organizational Policies;

- Coordinate dispatch operation according to planned schedule;

Tools and equipment required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Conveyor belts (manual & automated sorting lines) | 1 |
| 2 | Magnetic separator | 1 |
| 3 | Eddy current separator | 1 |
| 4 | Optical sorter | 1 |
| 5 | Air classifier/blower | 1 |
| 6 | Shredders and crushers | 1 |
| 7 | Baler | 02 |
| 8 | Torch | 15 |
| 9 | Weighing scale | 15 |
| 10 | Handheld communication device | 15 |
| 11 | Consumable storage container | 04 |

The consumables required for this competency standard are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Disposable gloves | 50 |
| 2 | Single-use face masks | 250 |
| 3 | Hand sanitizers | 2000ml |
| 4 | Cleaning agents (detergents, chlorine solution, lime powder) | 200kg |
| 5 | Waste collection bags (color-coded: organic, recyclable, hazardous) | 50 each |
| 6 | Labels, tags, and stickers for waste identification | 50 each |
| 7 | Markers | 12 |
| 8 | Spill absorbents (sand, sawdust, absorbent pads) | 25 |
| 9 | Batteries for portable devices (torches, weighing scales, walkie-talkies) | 48 |
| 10 | Pet water bottle 500ml (Basic welfare consumables) | 250 |
| 11 | Tissue (Paper) box/pack of 100 sheets | 20 |
| 12 | Insect repellents or pest control consumables (sprays, traps) | 15 |

## 1021CSWMSWCS04: Manage Temporary Collection Point (TCP) Operations

Overview: This Competency Standard describes the skills, knowledge, and attitude required to manage TCP operations. It includes the ability to set up a collection point, control TCP operations and restore TCP site for restoration.

| Competency Units | Performance Criteria |
| --- | --- |
| Set up temporary collection point | - Identify a suitable location for TCP. - Prepare TCP setting up plan - Obtain necessary permission from authorities - Arrange physical infrastructure - Install safety and hazard signage. - Ensure segregation facilities for different waste streams (organic, recyclable, hazardous). |
| Control TCP operations | - Conduct routine inspections for safety hazards. - Verify equipment functionality - Assign operational roles to staff. - Maintain records of incoming waste - Provide customer/community guidance on disposal. |
| Restore TCP site for restoration | - Remove temporary infrastructure. - Transport collected waste to designated disposal/recycling facilities. - Restore site to original condition (cleaning, disinfection, hazard removal). - Prepare closure report with data on waste handled and lessons learned. |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Waste Management Principles

- Operational Logistics of TCP

- Legal and Regulatory Compliance

- Closure and Restoration of TCP

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

  - Set up a temporary collection point, in compliance with organizational policy

  - Control TCP operations as per SOPs.

  - Closure TCP site for future operations.

Tools and equipment required

| S. No. | Items | Quantity |
| --- | --- | --- |
| 1 | Waste collection bins/containers | 10 |
| 2 | Temporary fencing or barriers | 100 |
| 3 | Shade/shelter (canopies, tents covering 100 sq yards) | 3 |
| 4 | Signage boards (entry/exit, segregation instructions, hazard warnings) | 20 |
| 5 | Lighting equipment (portable lamps, floodlights) | 25 |
| 6 | Weighing scale | 10 |
| 7 | Digital Record-keeping & communication device (Mobile phone, tablets, walkie-talkie) | 25 set |
| 8 | Labeling tools (tags, markers, stickers) | 10 each |
| 9 | Hand tools for handling waste (shovels, tongs, grabbers) | 26 set |
| 10 | Cleaning tools (brooms, disinfectants, sprayers) | 26 set |
| 11 | Face shields | 26 |
| 12 | Masks/respirators | 26 |
| 13 | Safety boot | 26 pairs |
| 14 | Portable barriers/cones for traffic control | 50 |
| 15 | Wheelbarrow | 05 |
| 16 | Forklift or cart | 03 |

The consumables required for this competency standard are provided below:

| S. No. | Items | Quantity |
| --- | --- | --- |
| 1 | Disposable gloves (Heavy duty) | 100 pair |
| 2 | High-visibility jacket | 26 |
| 3 | Spill kit for hazardous waste | 12 |
| 4 | Log Book, register, | 26 |
| 5 | Lime | 100kg |
| 6 | Phenyl | 3 Bottles |
| 7 | Sanitizers in 1000ml bottle | 12 |
| 8 | Documentation templates for closure reports | 50 |
| 9 | Incident reporting forms | 50 |
| 10 | Inspection checklist (printed or digital) | 50 |

# 1021CSWMSWCS05: Manage Scientific Landfill Site Operations

Overview: This Competency Standard describes the skills, knowledge, and attitude required to Manage Scientific Landfill Site Operations. It includes the ability to inspect and prepare landfill operational areas, manage waste placement and compaction processes, monitor environmental protection measures, coordinate vehicle movement and landfill activities, and maintain operational records of the landfill site.

| Competency Units | Performance Criteria |
| --- | --- |
| Prepare Landfill Operational Area | - Inspect landfill operational area before activities - Identify hazards and unsafe conditions at site - Prepare designated waste disposal area for operation |
| Manage Waste Placement with Compaction Activities | - Direct waste unloading at designated landfill cells - Spread waste in defined layers as per SOPs - Coordinate compaction activities safely - Prevent waste spillage and uncontrolled dumping |
| Monitor Environmental Protection Measures | - Monitor leachate and drainage control systems - Identify environmental hazards - Apply daily cover procedures after waste placement - Maintain environmental compliance at site |
| Supervise Vehicle Movement Operations | - Coordinate vehicle movement within landfill site - Direct vehicles to designated disposal areas - Maintain safe traffic flow during operations - Follow operational SOPs and communication procedures |
| Maintain Landfill Operational Records | - Record waste disposal activities in prescribed formats - Maintain daily operational logs and reports - Provide accurate operational information to supervisor - Follow documentation procedures and reporting requirements |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Principles and workflow of scientific landfill operations

- Landfill operational zones, traffic flow, and vehicle movement procedures

- Waste disposal, placement, and unloading requirements

- Documentation, reporting formats, and record keeping procedures

- Communication protocols and coordination methods at landfill site

- Workplace health, safety, and PPE requirements

- Hazard identification, risk prevention, and emergency response procedures

- SOPs and environmental compliance requirements related to landfill operations

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Safe waste handling operations

- Controlled vehicle movement coordination

- Hazard identification and reporting

- Accurate operational record keeping

- PPE use and safety compliance

Tools and equipment required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Whiteboard / smart board | 1 |
| 2 | Computer | 1 |
| 3 | Projector - Multimedia | 1 |
| 4 | Printer | 1 |
| 5 | Training Manuals | 25 |
| 6 | hand carts | 1 |
| 7 | Shovels, rakes, and hand tools | 1 |
| 8 | Seasonal Safety Gadgets | 5 |
| 9 | Traffic cones | 5 |
| 10 | barricade tapes | 5 |
| 11 | Signage boards (safety and directional) | 10 |
| 12 | Communication devices (walkie-talkies) | 5 |
| 13 | Site layout maps and operational charts | 5 |
| 14 | Inspection checklists, logbooks, and reporting forms | 5 |

The consumables required for this competency standard are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Notebook | 25 |
| 2 | Pen/pencil | 25 |
| 3 | White Board Markers (Blue, Black, Green, Red) | 10 |
| 4 | Hand Wash (1 liter) | 05 |
| 5 | Cleaning Towel | 05 |
| 6 | PPE items (gloves, masks – disposable types) | 25 |

## 1021CSWMSWCS06: Handle Community Service Issues

Overview: This competency standard outlines the skills, knowledge, and attitude required to receive and record community complaints, coordinate and support resolution of service issues, verify complaint closure and maintain accurate records, demonstrate professional conduct during community interactions, and apply safety practices during field and community activities.

| Competency Units | Performance Criteria |
| --- | --- |
| Record Community Complaints | - Receive community complaints through prescribed channels - Record complaint details accurately in prescribed formats - Identify complaint type and priority level - Forward complaint information to concerned personnel |
| Supervise Resolution of Service Issues | - Coordinate with operational staff for complaint resolution - Communicate service status to complainants - Follow complaint response timelines - Monitor corrective actions during operations |
| Verify Complaint Resolution | - Maintain completion status of assigned complaints - Update complaint records and resolution details - Prepare reports using prescribed formats - Escalate unresolved issues to management |
| Maintain Professional Conduct During Community Interaction | - Communicate professionally with community members - Maintain respectful behavior during complaint handling - Handle difficult situations using communication protocols - Maintain confidentiality of complaint information |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of community service procedures in solid waste management

- Knowledge of complaint receiving, recording, and response procedures

- Awareness of waste management services and operational workflow

- Understanding of communication protocols and public dealing practices

- Knowledge of documentation, reporting formats, and record keeping requirements

- Awareness of workplace safety procedures and PPE requirements during field activities

- Understanding of hazard identification and reporting procedures

- Knowledge of SOPs, service standards, and organizational compliance requirements

- Awareness of confidentiality and professional ethics during community interaction

- Understanding of coordination procedures with operational teams and management

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Accurate complaint recording

- Proper issue classification

- Timely complaint resolution

- Effective coordination and follow-up

- Professional community communication

- SOP and safety compliance

- Proper record maintenance

- Hazard identification and reporting

Tools and equipment required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Whiteboard / smart board | 1 |
| 2 | Computer | 1 |
| 3 | Projector - Multimedia | 1 |
| 4 | Printer | 1 |
| 5 | Training Manuals | 25 |
| 6 | Complaint App/Portal/register / logbook | 1 |
| 7 | Mobile phone / communication device | 1 |
| 8 | SOP manuals and guidelines | 1 |

The consumables required for this competency standard are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Notebook | 25 |
| 2 | Pen/pencil | 25 |
| 3 | White Board Markers (Blue, Black, Green, Red) | 10 |
| 4 | Complaint forms | 25 |
| 5 | Reporting sheets | 25 |
| 8 | Feedback forms | 25 |

## 1021CSWMSWCS07: Manage Event-Based Waste Operational Activities

Overview: This Competency Standard describes the skills, knowledge, and attitude required to manage event-based waste operational activities. It includes the ability to supervise scheduled event operations, manage emergency Response, prepare for Emergency and Special Event Operations,

| Competency Units | Performance Criteria |
| --- | --- |
| Supervise scheduled event operations | - Plan workforce deployment for scheduled events - Manage waste collection operations in crowd area - Ensure timely service delivery in high-density zones - Coordinate with stakeholders and local authorities - Maintain Operational discipline during event |
| Manage emergency Response | - Monitor emergency-affected areas - Deploy teams to priority locations - Ensure continuity of services - Safeguard human safety |
| Prepare for significant event operations | - Communicate operational plans to team members - Assign field responsibilities - Ensure availability of resources - Verify workforce readiness for deployment - Brief teams on significant event risks (flood, rain, epidemic, festivals, and gatherings etc.) |
| Coordinate with event-based operational ground staff | - Report incidents on time - Sustain communication with field team - Issue operational instructions during emergencies - Report operational activities - Escalate critical issues as per SOPs |
| Ensure Risk Control | - Identify hazards in emergency and event areas. - Implement safety measures for workers/public. - Ensure proper use of resources/supplies - Control risks in crowded and sensitive zones - Enforce compliance with safety procedures |
| Evaluate event-based operations for improvement | - Assess effectiveness of operations - Identify operational gaps - Collect feedback from stakeholders - Recommend corrective actions - Prepare operational performance reports |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in the competency standard of ‘’Supervise Material Recovery facility’’. This includes the knowledge of;

- Event Planning & Scheduling

- Staff Roles & Responsibilities in emergencies

- Awareness of Incident Management System

- Emergency Procedures & Protocols

- Coordinate with event-based operational ground staff

- Risk Controlling techniques

- Resource & Workforce Management

- Performance indicators for event-based cleaning & sanitation operations.

& ProcessesStorage & Material Handling

Critical Evidence Required:

The candidate needs to produce the following critical pieces of evidence to be competent in this competency standard:

- Demonstrates ability to plan, allocate resources, and oversee routine event logistics.

- Applies emergency protocols, coordinates staff, and ensures safety compliance during incidents.

- Develops comprehensive operational plans, risk assessments, and contingency measures.

- Maintains effective communication, role clarity, and teamwork with ground staff.

- Identifies hazards, implements mitigation strategies, and enforces compliance standards.

- Conducts post-event reviews, analyzes performance gaps, and recommends improvements.

Tools and equipment required

| S.No. | Tool / Equipment | Quantity |
| --- | --- | --- |
| 1 | Color-coded emergency waste bins | 1 |
| 2 | Portable skip containers | 1 |
| 3 | Roll-off bins | 1 |
| 4 | Wheelbarrows | 2 |
| 5 | Hand trolleys | 2 |
| 6 | Dollies | 2 |
| 7 | Mini waste collection vehicles | 1 |
| 8 | Compact waste transport trucks | 1 |
| 9 | Hydraulic lifters | 1 |
| 10 | Hydraulic loaders | 1 |
| 11 | GPS tracking devices | 2 |
| 12 | Digital tablets | 10 |
| 13 | Mobile reporting devices/apps | 5 |
| 14 | Semi-mobile transfer stations | 1 |
| 15 | Portable waste compactors | 2 |
| 16 | Signage boards | 2 |
| 17 | Public announcement systems | 2 |
| 18 | Drones | 2 |
| 19 | CCTV cameras | 3 |

The consumables required for this competency standard are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Disposable gloves | 50 |
| 2 | Single-use face masks | 250 |
| 3 | Hand sanitizers | 1000ml |
| 4 | Cleaning agents (detergents, chlorine solution, lime powder) | 20kg |
| 5 | Waste collection bags (color-coded: organic, recyclable, hazardous) | 50 each |
| 6 | Labels, tags, and stickers for waste identification | 50 each |
| 7 | Spill absorbents (sand, sawdust, absorbent pads) | 25 |
| 8 | Batteries for portable devices (torches, weighing scales, walkie-talkies) | 48 |
| 9 | Pet water bottle 500ml (Basic welfare consumables) | 250 |
| 10 | Tissue (Paper) box/pack of 100 sheets | 20 |
| 11 | Insect repellents or pest control consumables (sprays, traps) | 15 |
| 12 | Checklists and logbooks for compliance documentation | 25 |

## 0611-001-GC Perform Basic IT Operations

Overview: This module aims to equip individuals with the foundational skills necessary to effectively use a computer in a professional setting. This module covers essential computer literacy skills that are critical for a wide range of tasks in today's digital workplace. Key components of this module include:

| Competency Units | Performance Criteria |
| --- | --- |
| Configure Computer Systems | - Connect computer components and peripherals as per requirement. - Install Drivers and applications according to the software specification. - Troubleshoot Applications to trace and fix faults |
| Prepare a Document using MS WORD | - Compose a document as per requirement. - Format Word Document according to given requirements. - Print Word Documents both sides of paper according to requirements. |
| Prepare a Worksheet using MS EXCEL | - Develop a worksheet as per given data. - Format the worksheet according to given criteria. - Apply Formulas according to the requirement. - Generate Charts/Graphs according to the given data. |
| Prepare a Presentation using MS PowerPoint | - Insert Slides with different Layouts according to requirements of presentation. - Insert text, tables, images, etc. according to the requirement. - Apply a set of effects to animate the slide according to requirement. - Apply Slide Transitions on Slides according to requirement. - Apply Sound Effects on Objects/text/images according to requirement. - Present a presentation according to 7Cs of communication. |

Knowledge & Understanding

- Operating systems

- Hardware and Software

- Troubleshooting

- Internet and E-mailing

- Hyperlink and referencing

- Printing

- Formulas

- Short Keys

- WPM (Word Per Minute)

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Install MS Office Application correctly

- Prepare a formatted document using MS Word

- Enter data into the respective columns and rows as per given instructions

- Set page layouts and margins

- Apply any slide transition on entire presentation.

The tools and equipment required for this competency standard are given below

| Sr.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Computer System | 26 |
| 2 | MS Office Suite | 10 |
| 3 | Printer | 1 |
| 4 | Scanner | 1 |
| 5 | Printing Paper A4 Size | As Needed |

## 1022-001-GC: Manage Documentations

Overview: This competency standard outlines the skills, knowledge, and attitude required to manage documentation through hard copy and digital systems, including data entry and record updating, synchronization across platforms, document storage and retrieval, report generation and submission, and maintenance of data security, confidentiality, and compliance requirements.

| Competency Units | Performance Criteria |
| --- | --- |
| Record data in prescribed system | - Capture data through digital system - Enter data into prescribed formats - Ensure accuracy and completeness of entered information |
| Synchronize Records Across Platforms | - Update records in real time across systems - Synchronize data hard and soft copies - Correct discrepancies between systems - Maintain consistency of information |
| Store data | - File documents in soft and hard copies - Organize records using classification rules - Retrieve documents prescribed source - Maintain document security and accessibility - Submit report to management |

Knowledge & Understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding of documentation procedures in operations, HR, and fleet management

- Knowledge of manual and digital record keeping systems

- Awareness of mobile application and web portal functions for data entry and reporting

- Understanding of document classification, filing, and retrieval procedures

- Knowledge of reporting formats, logs, and operational registers

- Awareness of data accuracy, validation, and synchronization requirements

- Understanding of confidentiality, data protection, and access control procedures

- Knowledge of SOPs and organizational compliance requirements related to documentation

- Awareness of communication and reporting protocols within departments

- Understanding of error identification, correction, and audit requirements in documentation systems

Critical Evidence(s) Required:

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Accurate data entry practices

- Proper document filing procedures

- Timely report submission compliance

- Confidentiality and data security

- Error identification and correction

Tools and equipment required

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Whiteboard / smart board | 1 |
| 2 | Computer | 1 |
| 3 | Projector - Multimedia | 1 |
| 4 | Printer | 1 |
| 5 | Training Manuals | 25 |
| 6 | Record App/Portal/register/ logbook | 1 |
| 7 | SOP manuals and guidelines | 1 |

The consumables required for this competency standard are provided below:

| S.No. | Items | Quantity |
| --- | --- | --- |
| 1 | Notebook | 25 |
| 2 | Pen/pencil | 25 |
| 3 | White Board Markers (Blue, Black, Green, Red) | 10 |

### On-Job Training

Overview: The On-Job Training component of this qualification is integrated across Level 4 to enable learners to apply their skills, knowledge, and attitude in real workplace environments of solid waste management. Learners progressively perform basic, skilled, and supervisory roles in areas such as waste collection, transportation, segregation, recycling, disposal, safety, and reporting.

The training is conducted in municipal services, waste management companies, recycling facilities, and landfill sites, ensuring practical exposure in accordance with workplace procedures, environmental guidelines, and OHS standards.

# Members of the Qualification Development & Review Committee

The following members participated in the qualification development and review process:

| Qualification Development Committee |  |  |  |
| --- | --- | --- | --- |
| Sr.# | Name | Designation | Organization |
|  | Syed Ghous Mohiuddin | Deputy Director Operation Manager | DMC Central GANSUE/SSWMB, Sindh. |
|  | Waheed Ali Arbab | Project Manager | Livelihood Restoration Plan LRP. NDF Pakistan. |
|  | Muhammad Khalid | Advisor Environment | SWM Hangzhou Jinjiang Sanitation Company for SSWMB. |
|  | Asif John | Zonal Manger | Hangzhou Jinjiang group SSWMB |
|  | Engr. Arsalan Hameed Khan | Solid Waste Management Expert Manager (Projects) | Suthra Punjab Authority Lahore Head Quarter |
|  | Abdul Basit Mansoor | AEE Civil | Pak PWD Gujranwala |
|  | Dr. Nazir Ahmed Memon | Education Consultant & Curriculum Expert | SELD Govt. Of Sindh |
|  | Mohammad Adeel Khan | Assistant Professor | KP-TEVTA |
|  | Naeem Ahmed | Incharge NVQF Cell | Trade Testing Board Sindh. |
|  | Altaf Shaikh | Deputy Director (Academics & Training) | STEVTA HQ, Karachi. |
|  | Shahzad Ahmed | Director Skill Standard and Curricula | NAVTTC-Islamabad |
|  | Muhammad Ishaq | Deputy Director | NAVTTC-Islamabad |
|  | Samina Sharafat | Assistant Director | NAVTTC-Islamabad |
|  | Aftab Soomro | Project Manager | RDF |
|  | Imtiaz Hassan Sial | Assistant Director | NAVTTC-Islamabad |
|  | Furqan Aziz | DACUM Facilitator, Head of Administration, Project & Grants | Memon Industrial & Technical Institute (MITI), Karachi. |

# Members of Qualification Validation Committee

The Qualifications Validation Committee consisted of the following members:

| Qualification Development Committee |  |  |  |
| --- | --- | --- | --- |
| Sr.# | Name | Designation | Organization |
|  | Ms. Farhat Hafeez | DG (SS&C) | NAVTTC HQ |
|  | Salman Shah | Director Academics | PTEVTA, Lahore |
|  | Muhammad Yousuf Baloch | Director Academics | STEVTA, Karachi |
|  | Ms. Ghazal Nagi | Deputy Director | BTEVTA, Quetta |
|  | Muhammad Nadeem Akram | Manager Curriculum | PVTC, Lahore |
|  | Zahid Iqbal | Director | PSDA, Lahore |
|  | Muhammad Aleem | Deputy Director | NUTECH, Board |
|  | Amir Sarwar | Member | Punjab TTB |
|  | Muhammad Sohail | Member | KPK TTB |
|  | Alamgir Memon | Member | Sindh TTB |
|  | Zahoor Ahmed | Member | Baluchistan TTB |
|  | Abdul Hafeez Abbasi | Director Operation | STEVTA, Karachi |
|  | Muhammad Suleman Khan Dashti | Consultant Chief Minister | BBSHRRDB |
|  | Aijaz Ahmed Brohi | Secretary | TTB, Sindh |
|  | Muhammad Hamza | Assistant Director | NAVTTC-Islamabad |
|  | Akmal Khan | Head of Projects | WWH |
|  | Aftab Soomro | Program Coordinator TVET | RDF |
|  | Furqan Aziz | DACUM Facilitator, Head of Administration, Project & Grants | Memon Industrial & Technical Institute (MITI), Karachi. |