Curriculum for National Diploma Level 4 in Transformer Technician

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Level 4 Transformer Technician. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	1

2.	Overall objectives of training program	3

3.	Scheme of Studies	3

4.	Overview of the Curriculum for Transformer Technician	4

5.	Details of Modules	6

1)	0713E&E4301 Maintain Safe Working Environment of Transformer	6

2)	0713E&E4302 Perform Installation and Commissioning of Transformer	9

3)	0713E&E4303 Perform Preventive Maintenance of Transformer	12

4)	0713E&E4304 Perform Corrective Maintenance of Transformer	15

5)	0713E&E4305 Perform Transformer Winding Operation	18

6)	0713E&E4306 Fabricate & Weld Transformer Components	23

7)	0713E&E4307 Develop Entrepreneurship/Business	28

6.	List of Personal Protective Equipment (For A Class Of 25 Students)	31

7.	List of Tools and Equipment	31

8.	List of Consumables	33

9.	Members of Qualification Development Committee	35

# Introduction

This Transformer Technician Level–4 course is designed to address a vital skill gap in Pakistan’s power and electrical sector by producing certified Transformer Technicians, a qualification currently limited in availability across the country. The course aims to equip learners with the technical knowledge, practical skills, and professional attitude required to install, maintain, test, and repair power and distribution transformers efficiently and safely. As the demand for reliable power infrastructure grows and industries move toward modernization, the need for skilled transformer technicians has become increasingly important for ensuring system reliability, energy efficiency, and safety compliance. Throughout this course, participants will gain in-depth understanding of transformer construction, operation, testing, fault diagnosis, and preventive maintenance techniques. They will also learn to interpret electrical drawings, perform insulation resistance and winding tests, manage transformer oil testing, and ensure proper commissioning and decommissioning procedures. Through hands-on practical sessions and real-world case studies, learners will develop the ability to troubleshoot transformer faults, apply national and international standards, and follow best practices for transformer maintenance and safety. Additionally, the course integrates training in occupational safety, project management, and entrepreneurship, enabling learners to either work effectively within organizations or establish independent transformer maintenance services.

This course is ideal for individuals aspiring to build or enhance their careers in electrical power distribution, maintenance, and industrial sectors. By the end of this program, graduates will be well-prepared to ensure the reliable performance of transformers, contributing to the stability, efficiency, and sustainability of Pakistan’s power sector.

| 1.1 Date of Development | 29th September 2025 to 03rd October 2025 |
| --- | --- |
| 1.2 Date of Review | July 2027 |
| 1.3 Entry requirement | Industrial Electrician (Level-3), or Equivalent qualification |
| 1.4 Minimum qualification for teachers / instructors | BS/BSc/BE Engineering (Electrical/ Electronics) with 3 years of Experience OR B.Tech (Honour) / BS Technology (Electrical/ Electronics) with 3 years of Experience OR Diploma of Associate Engineers (DAE in Electrical/ Electronic) with 5 years’ experience |
| 1.5 Codes of Qualification (ISCED) | Code: 0713E&E43 Description: Transformer Technician |
| 1.6 Student Teacher Ratio | 1:25 |
| 1.7 Medium of Instruction | English |
| 1.8 Duration of the Course | Theory: 0 90 Hours Practical: 500 Hours On Job Training: 240 Hours Total: 830 Hours |

# Overall objectives of training program

# The Transformer Technician Level–4 training program aims to produce competent and skilled professionals capable of ensuring the reliable operation, maintenance, and safety of power and distribution transformers. The program is designed to develop both theoretical knowledge and practical expertise in transformer construction, operation principles, and maintenance procedures. Learners will be trained to perform installation, testing, troubleshooting, repair, and preventive maintenance tasks using modern tools and techniques. Emphasis is placed on promoting safety awareness and adherence to national and international electrical safety standards to prevent accidents and equipment failures. The course also focuses on enhancing analytical and problem-solving skills, enabling participants to identify and rectify transformer faults efficiently. In addition, it fosters professionalism, teamwork, and effective communication, which are essential qualities in industrial and power sector environments. The program encourages entrepreneurship by preparing learners to establish independent maintenance services or pursue employment in public and private utilities. Overall, the training supports national development goals by promoting energy efficiency, system reliability, and sustainable growth within Pakistan’s power distribution network.

# Scheme of Studies

| Code | Competency standard | NVQF Level | Category | Estimated contact hours |  |  | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Tot. |  |
| 0713E&E4301 | Maintain Safe Working Environment of Transformer | 4 | Functional | 10 | 40 | 50 | 3 |
| 0713E&E4302 | Perform Installation and Commissioning of Transformer | 4 | Technical | 10 | 80 | 90 | 5 |
| 0713E&E4303 | Perform Preventive Maintenance of Transformer | 4 | Technical | 20 | 80 | 100 | 6 |
| 0713E&E4304 | Perform Corrective Maintenance of Transformer | 4 | Technical | 10 | 60 | 70 | 4 |
| 0713E&E4305 | Perform Transformer Winding operation | 4 | Technical | 10 | 100 | 110 | 6 |
| 0713E&E4306 | Fabricate and weld transformer components | 4 | Technical | 20 | 80 | 100 | 6 |
| 0713E&E4307 | Develop Entrepreneurship/ Business | 4 | Generic | 10 | 60 | 70 | 4 |
| Total Cr. Hr. Institute |  |  |  | 90 | 500 | 590 | 34 |
| 0713E&E4308 | On job training (OJT) | 4 | Technical | - | - | 240 | 06 |
| G.Total (%) |  |  |  | 830 |  |  | 40 |

# Overview of the Curriculum for Transformer Technician

| Module Title | Learning Units | Theory hours | Workplace hours | Timeframe of modules |
| --- | --- | --- | --- | --- |
| 0713E&E4301 Maintain Safe Working Environment of Transformer | - Identify hazards at workplace - Categorize the potential risks - Mitigate the Potential Risks - Monitor the Safety Protocols - Maintain the safety signage - Document the reports | 10 | 40 | 50 |
| 0713E&E4302 Perform Installation and Commissioning of Transformer | - Interpret the transformer drawings and specifications - Install the transformer - Transformer commissioning - Connect the transformer - Energize the transformer - Document the reports | 10 | 80 | 90 |
| 0713E&E4303 Perform Preventive Maintenance of Transformer | - Perform visual inspection of transformer - Plan schedule for preventive maintenance. - Conduct the preventive maintenance - Analyze the preventive maintenance results - Document the reports | 20 | 80 | 100 |
| 0713E&E4304 Perform Corrective Maintenance of Transformer | - Isolate the transformer - Perform troubleshooting - Categorize the nature of faults - Carry Out Alternate Arrangements for Smooth Power Supply - Repair/Replace Faulty Component - Document the reports | 10 | 60 | 70 |
| 0713E&E4305 Perform Transformer Winding Operation | - Disassemble the transformer - Interpret transformer drawings & specifications - Perform winding operation - Perform reassembling of transformer - Perform post winding inspection and testing for smooth functioning - Reinstall and energize the transformer | 10 | 100 | 110 |
| 0713E&E4306 Fabricate & Weld Transformer Components | - Check Welding Equipment & Tools - Interpret fabrication & welding drawing - Perform welding operation - Perform fabrication operation - Perform post-welding processes | 20 | 80 | 100 |
| 0713E&E4307 Develop Entrepreneurship/ Business | - Assess market opportunities - Develop a business plan - Manage business finances - Implement risk management strategies - Communicate effectively with clients - Promote business services - Plan for business growth. | 10 | 60 | 70 |
| Total |  | 90 | 500 | 590 |

# Details of Modules

## 0713E&E4301 Maintain Safe Working Environment of Transformer

## Objective: This module pertains to establishing and upholding a safe environment within the operations of transformer technology. Specifically, it includes the evaluation and classification of workplace risks, the application of appropriate risk control measures, and the compliance with health and safety regulations. This responsibility also includes the enforcement of equipment health regulations, compliance with safety protocol inspections, and vigilant care of safety signs. In completing these activities, learners actively enhance and safeguard the personnel, equipment, and environmental risks within the workplace and advance safety culture

## Duration: 50 Hours	            		Theory: 10 Hours				Practice: 40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Identify hazards at work place | - Check workplace environment. - Assess unsafe conditions - Recognize workplace hazards | - Potential impacts due to different hazards - Detection methods of hazards - Reporting procedures and compliance Practical Activity: Candidate is required to conduct a workplace inspection to identify hazards as per checklist. | Theory-01Hrs Practice-08Hrs Total- 09Hrs | Consumable - Inspection checklist (printed) - Pens and markers - Coloured sticky notes for marking hazard locations | - Classroom - Workshop |
| LU2. Categorize the potential risks | - Assess potential risks - Categorize risks as immediate, high-priority, or tolerable. - Communicate categorization outcomes to management and workforce | - Types of risks and their potential impacts. - Risk assessment methods for categorization. Practical Activity: Candidate is required to conduct workplace assessment for risk categorization | Theory-01Hrs Practice-09Hrs Total- 11Hrs | Consumable - Risk assessment templates or forms (printed/digital) - Inspection checklist | - Classroom - Workshop |
| LU3. Mitigate the Potential Risks | - Apply hierarchy of controls - Safely isolate circuits using lock-out/tag-out procedures. - Install barriers, guards, and safety devices to minimize risk - Ensure effective use of PPE/SPs for all personnel. - Perform safety drills/ T & P prade | - Risk mitigation control and procedures - Types of PPEs, barriers and guards Practical Activity: Candidate is required to conduct a task where identified hazards are mitigated by applying control measures using appropriate PPEs. | Theory-02Hrs Practice-09Hrs Total- 11Hrs | Consumable - Lock-out/tag-out tags and labels - Safety barrier tape or cones - Risk mitigation checklist forms | - Classroom - Workshop |
| LU4. Monitor the Safety Protocols | - Verify safety protocol compliance with SOPs. - Record safety violations - Maintain compliance records for legal purposes | - Workplace safety protocols. - Methods and tools for monitoring. - Safety protocols documentation Practical Activity: Candidate is required to perform a workplace inspection to monitor adherence to safety protocols. | Theory-02Hrs Practice-07Hrs Total- 09Hrs | Consumable - Safety inspection documents - Colored sticky notes for marking non-compliance areas | - Classroom - Workshop |
| LU5. Maintain the safety signage | - Place safety signage at standardized visible locations. - Implement national/international signage, symbol, codes, and color | - Maintenance procedures for Safety signage. - Types of safety signages. Practical Activity: Candidate is required to conduct a workplace inspection to check the condition, visibility, and compliance of existing safety signage, record any issues, and replace or repair signs as needed according to SOPs. | Theory-01Hrs Practice-07Hrs Total- 08Hrs | Consumable Safety signage pictures | - Classroom - Workshop |

| LU6. Document the reports | - Prepare clear, accurate on hazards, risks, and monitoring reports. - Maintain detailed logs of inspections, accidents, and corrective actions. - Submit reports in prescribed formats within required timelines. - Ensure documentation complies with audit and regulatory standards. - Maintain secure and retrievable records for future reference | - Procedures for recording inspections, incidents, and corrective actions. - Report templates with checklists and other formats. - Awareness of safety legal regulation document requirements. Practical Activity: Candidate is required to conduct an activity at workplace to identify, assess, categorize, mitigate risks and document as per SOPs in the template. | Theory-02Hrs Practice06Hrs Total- 07Hrs | Consumables - Inspection checklist forms - Logbooks or recording sheets | - Classroom - Workshop |
| --- | --- | --- | --- | --- | --- |

## 0713E&E4302 Perform Installation and Commissioning of Transformer

Objective: This module covers the knowledge and  skills required to carry out mechanical and electrical installation, conducting tests, energizing safely, and maintaining documentation, all in compliance with safety and industry standards.

Theory: 10 Hours				Practice: 80 Hours 					Duration: 90 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Interpret the transformer drawings and specifications | - Identify various types of transformer drawings and symbols - Interpret electrical and mechanical specifications of transformers - Verify ratings, configurations, and installation requirements as per manufacturer’s documentation - Verify site conditions with transformer design parameters | - Different types of transformer installation drawings - Interpret transformer single line diagrams - Transformer specifications Practical Activity 1 Candidate is required to develop a site plan by interpreting transformer installation drawings and specifications | Theory-02Hrs Practical-15Hrs Total- 17Hrs | Consumables: - Writing Pads - Writing and Sketching Materials (Drawing Sheets, Pencils, Rulers) | - Classroom - Workshop |
| Install the transformer | - Prepare installation site and readiness. - Perform correct alignment and positioning of the transformer. - Secure mechanical fittings and structural supports. - Perform grounding of the transformer as per standard requirements. - Connect auxiliary components including cooling fans, protection relays. - Implement physical clearances and ventilation as per safety guidelines | - Types of transformer installation - Auxiliary Components of Transformer - Earthing and bonding of transformer Practical Activity Candidate is required to perform the installation, grounding (tank and neutral) and Install Auxiliary Components of transformer | Theory-02Hrs Practical-15Hrs Total- 17Hrs | Consumables: - Grounding Wire - Nut Bolts and Washers - Contact Cleaner - Degreaser (Mineral Spirit) - Dusters | - Classroom - Workshop |
| Commission the Transformer | - Conduct pre-commissioning checks including insulation, oil level, tap changer position, leaks, breather, gaskets - Conduct key mechanical functional tests, including the cooling system, Buchholz relay, temperature indicators, oil level indicators, and pressure relief valve - Conduct key electrical tests, including Insulation Resistance Test, Transformer Turns Ratio (TTR) Test, Capacitance and Dissipation Factor (C&DF) Test, Dissolved Gas Analysis (DGA), Open Circuit Test, and Short Circuit Test - Evaluate test results against manufacturer’s and industry standards - Comply with commissioning procedures and safety protocols | - Commissioning procedure and its compliance - Types of pre-commissioning tests and protocols Practical Activity Each group of trainees will perform the following transformer tests - Oil Leakage - Tap Changer - Breather check - Temperature Rise - Turns Ratio (TTR) Test - IR Test. - Capacitance and Dissipation Factor (C&DF) Test - Dissolved Gas Analysis (DGA) - Open Circuit Test, and Short Circuit Test | Theory-02Hrs Practical-15Hrs Total- 17Hrs | Consumables: - Gaskets and sealants - Grease - Silica Gel - Contact Cleaner and wipes - Transformer Oil - Sampling Container - Absorbent Pads | - Classroom - Workshop |
| LU4. Connect the transformer | - Identify primary and secondary connection based on the wiring diagram. - Perform electrical connection/terminations using appropriate tools and techniques. - Carryout continuity test. - Comply with phase sequence and polarity requirements. | - H.T./L.T. bushings - Types of termination - Continuity test, Phase Sequence Test and Polarity Test. Practical Activity: Candidate is required to carry out HT, LT & earthing connections and perform continuity, Phase Sequence & polarity Tests. | Theory02Hrs Practical-15Hrs Total-17Hrs | Consumables: - Connection Leads - Contact Cleaners - Identification Tags (Markers and Labels - Jumper Wires (Insulated Cables for Bridging connections during Polarity Test) | - Classroom - Workshop |
| LU5. Energize the transformer | - Perform final safety checks before energizing. - Coordinate with relevant departments for synchronization. - Energize the transformer following standard procedures. - Monitor operational parameters during energization. - Take corrective actions if abnormal conditions are observed. | - Steps of transformer energization. - Operational parameters for energization. Practical Activity Candidate is required to energize the transformer step by step and check the parameters as per checklist. | Theory-01Hrs Practical-15Hrs Total- 16Hrs | Consumables: - Insulated Tools - Logbook and Forms - Lint Free Wipes - Spill Cleanup kit - Touch up Paint | - Classroom - Workshop |
| LU6. Document the reports | - Maintain detailed records of installation and commissioning activities. - Record all testing and inspection data accurately. - Generate a commissioning report as per organizational standard. - Submit documentation for review and approval. - Archive reports for future reference and regulatory compliance. | - Templates/Documents for transformer installation & commissioning. Practical Activity Candidate is required to prepare a complete document by compiling installation and commissioning of transformer record. | Theory-01Hrs Practical-15Hrs Total- 16Hrs | Consumables: - Logbooks - Box Files - Google Sheets - Cloud Storage (Record Backup) | - Classroom - Workshop |

## 0713E&E4303 Perform Preventive Maintenance of Transformer

Objective: This module covers the skills and knowledge to perform preventive maintenance on transformers, including inspection, servicing, and safety checks. It focuses on identifying potential faults, ensuring reliability, and completing accurate maintenance records

Theory: 20 Hours				Practice: 80 Hours 					Duration: 100 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Perform visual Inspection of transformer | - Check tank, conservator and joints for oil leaks or rust - Verify oil level and gauges - Inspect bushing for cracks, damage and leakage - Examine cooling system for cleanliness/operation - Check breather and silica gel conditions - Verify grounding connections - Inspect for unusual noise, vibration, smell and hot spots | - Visual Defects and signs of faults Practical Activity Candidate is required to carry out a complete visual inspection of a transformer to record defects as per checklist. | Theory-04Hrs Practical-16Hrs Total- 20Hrs | Consumables - Cleaning cloths/wipes - Gloves - Inspection checklist/forms | - Classroom - Workshop |
| LU2. Plan schedule for the preventive maintenance | - Prepare plan for preventive maintenance and submit to concerned authority - Obtain final approval before execution - Coordinate timing with operations and maintenance team | - Preventive Maintenance plan - Communication procedures for maintenance planning Practical Activity Scenario Candidate is required to Schedule a preventive maintenance task for a transformer. | Theory-04Hrs Practical-16Hrs Total- 20Hrs | Consumable - Writing material (paper, pen) Non -Consumable - Maintenance guide | - Classroom - Workshop |
| LU3. Conduct the preventive Maintenance | - Get permit to work (PTW) - De-energize the transformer - Connect grounding for safety purpose - Perform tightening of joints, repair leaks, clean and repair rust - Perform refilling of oil, take sample of oil for testing and repair gauges - Clean, tighten replace bushings if cracked/damaged - Clean radiator, service/replace fans & pumps - Replace silica gel, clean/replace breather - Tight and clean earthing connections - Test and calibrate protection and monitoring devices - Perform basic electrical tests including IR, winding resistance, TTR, DGA, C&DF | - Permit to Work (PTW) procedure. - Shutdown and grounding procedures - Different types of transformer tests Practical Activity A group of trainees will carry out a full preventive maintenance procedure as per SOPs | Theory-04Hrs Practical-16Hrs Total- 20Hrs | Consumables - PTW forms - Oil sampling bottles - Transformer oil - Silica gel packs - Rust remover/anti-rust paint - Spare fans and pumps Non- Consumables - Tools (wrenches, screwdrivers, torque wrench) - Grounding cables/clamps - Testing instruments (multimeter, IR tester, TTR, DGA, C&DF meters) | - Classroom - Workshop |
| LU4. Analyze the Preventive Maintenance results | - Record preventive maintenance results. - Analyze results with standards/specifications. - Identify abnormal trends or deviations. - Follow Standard operating procedures (SOPs) | - Purpose of analyzing Preventive Maintenance data - Standard values for key electrical tests - Methods for interpreting diagnostic results Practical Activity Candidate is required to review recorded preventive maintenance data of a transformer, compare it with standard values to identify any abnormalities according to SOPs. | Theory-04Hrs Practical-16Hrs Total- 20Hrs | Consumable - Writing Materials (pens, markers) - Highlighters Non-Consumable - Maintenance log book | - Classroom - Workshop |
| LU.5 Document the Reports | - Compile all inspection and test results. - Use prescribed reporting format. - Highlight key findings and abnormal conditions. | - Preventive maintenance documents/formats. Practical Activity Candidate is required to prepare a checklist and Submit a Transformer Preventive Maintenance Report | Theory-04Hrs Practical-16Hrs Total- 20Hrs | Consumables - Papers Non- Consumables - Computer system with printer | - Classroom |

## 0713E&E4304 Perform Corrective Maintenance of Transformer

Objective: This module outlines the foundational skills and knowledge needed for a trainee to accurately perform corrective maintenance on the transformers. Upon mastering the competency standard, the student is expected to appropriately document maintenance procedures, isolate and troubleshoot faulty transformers, diagnose the underlying issues, perform necessary repairs, and confirm proper alternate power supply arrangements have been made, all within the safety protocols of the Competency-Based Training System.

Theory: 10 Hours				Practice:  60 Hours 					Duration:  70 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Isolate the transformer | - Wear the required PPEs. - Isolate transformers from supply safely. - Ensure grounding and discharge residual charge. - Place warning signage, lock-out/tag-out (LOTO) at isolation points. - Verify isolation using line tester. - Confirm with supervisor before proceeding. | - Safety Procedures for Isolation - Isolation steps for transformer - Lockout/Tagout (LOTO) method and tools Practical Activity Candidate is required to Isolate a transformer as per SOPs. | Theory-02Hrs Practical-10Hrs Total- 12Hrs | Consumables - Lockout tags and padlocks - Warning signs/posters - Isolation checklists or forms Non- Consumables - Transformer - Grounding rods and cables - Insulated tools | - Classroom - Workshop |
| LU2. Perform Troubleshooting | - Carryout visual inspection of the transformer - Identify common symptoms (noise, overheating, abnormal readings). - Apply diagnostic tools (multimeter, Megger, thermal scanner). - Perform tests to diagnose the faults. - Analyze test results to locate the fault. - Record preliminary fault observations. - Compare results with manufacturer’s standard values. | - Diagnostic procedures Practical Activity Candidate is required to perform troubleshooting of a faulty transformer. | Theory-02Hrs Practical-10Hrs Total- 12Hrs | Consumables - Insulating tape - Cotton waste / cleaning cloth - Report/log sheets - Marker pens /pencil - WD 40 spray Non- Consumables - Distribution transformer (lab/training unit, small rating. - High pot tester - Series test lamp | - Classroom - Workshop |
| LU3. Categorize the Nature of Faults | - Identify the nature of faults. - Assess the severity of the faults. - Evaluate cost and downtime implications of the fault. - Record the data. | - Transformer Faults and its nature Practical Activity Candidate is required to introduce to a transformer with any type of troubleshooting in which the student must ever to categorize the nature of the transformer. | Theory-2Hrs Practical-10Hrs Total- 12Hrs | Consumables - Writing Material - Paper - Checklists Non- Consumables - SOP Manual | - Classroom - Workshop |
| LU4. Carry Out Alternate Arrangements for Smooth Power Supply | - Provide temporary power backup (DG set or alternate transformer). - Coordinate with utility/grid for load transfer. - Verify alternate supply safety compliance. - Inform consumers/users about temporary arrangements. - Monitor performance of alternate supply during operation. | - Scenario of alternate power supply arrangements - Load transfer procedures - Ring main unit Practical Activity Candidate is required to carry out alternate power supply arrangements and connections. | Theory-01Hrs Practical-10Hrs Total- 11Hrs | Consumables - Insulating tape - H.T/L.T wires - Fuel (diesel for DG set) Non- Consumables - Diesel Generator (DG set)/ alternate transformer - Changeover switch (manual/automatic) - Distribution board / training panel | - Classroom - Workshop |
| LU.5 Repair/Replace Faulty Component | - Disassemble the transformer - Remove the faulty components - Repair/replace the faulty components - Apply proper torque and insulation techniques. - Conduct preliminary function test of replaced part. - Refill and test transformer oil (if applicable). - Reassemble and tighten all connections securely. - Perform post-repair safety and insulation resistance tests. - Perform tagging-in of the transformer after successful repair and testing. | - Repair/replace procedures of the faulty components of transformer Practical Activity Candidate is required to perform repair/replace of a faulty transformer. | Theory-01Hrs Practical-10Hrs Total- 11Hrs | Consumables - Insulating Tape - Basic Replacement Parts (e.g., fuse, terminal, bushing) etc. - Maintenance/Repair Record Sheet Non- Consumables - Transformer - Insulated tools - High pot tester | - Classroom - Workshop |
| L.U. 6 Document the Reports | - Record type and cause of fault. - Note corrective actions taken. - Compile maintenance log for future reference. - Submit report to supervisor/concerned authority - Attach photographs and test reports for verification. - Maintain digital/ERP entry of maintenance activity. - Provide recommendations to avoid similar faults in future. | - Corrective maintenance documents and templates Practical Activity Candidate is required to document a full report on a faulty transformer repaired in work shop. | Theory-2Hrs Practical-10Hrs Total- 12Hrs | Consumables - Writing material - Checklists Non- Consumables Computer with Printer | - Classroom - Workshop |

## 0713E&E4305 Perform Transformer Winding Operation

## Objective: This competency standard covers the knowledge and skills required for a student/trainee will be able to disassemble the transformer, understand winding details, perform winding work, reassemble the parts, test the transformer, and safely reinstall and energize it.

Theory: 10 Hours				          Practice:  100 Hours 					                                  Duration:  110 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Dis-assemble the transformer | - Assess the condition of transformer - Drain transformer oil - Dis-assemble the component of the transformer - Label and document parts for proper re-assembly and record keeping. | - Transformer structure, parts, and functions. - Tools, equipment, and proper techniques used for disassembly. - Documentation and labeling procedures for safe storage of parts. Practical Activity: Candidate is required to perform disassembly of a transformer by first isolating and securing the unit, then using proper tools to remove components step-by-step, labeling each part clearly, and recording details in a logbook for safe storage and reassembly. | Theory-01Hrs Practice16Hrs Total- 17Hrs | Consumable - Labeling tags and markers - Logbook - Cleaning cloths/wipes - Safety signage for isolation area - PPE items (gloves, safety goggles, face shield, helmets) | - Classroom - Workshop |
| LU2. Interpret Winding Drawings & Specifications | - Check transformer type, rating, and winding configuration from drawings. - Identify number of turns, wire gauge, layering, insulation type with clearances, tapping points as per specifications. - Interpret winding direction and coil arrangement accurately. - Check all data before starting winding to avoid errors during operation. | - Different types of transformer winding drawings Practical Activity: Candidate is required to develop a site plan by interpreting transformer winding drawings | Theory-01Hrs Practice-16Hrs Total- 17Hrs | Consumable Risk assessment templates Inspection checklist | - Classroom - Workshop |
| LU3. Perform winding operation | - Prepare and setup the transformer - Allocate requisite material for transformer - Set up winding machine or jig with correct tension, guides, and alignment. - Anchor conductor securely before starting winding - Perform winding operation while maintaining correct tension and alignment - Apply interlayer insulation and spacers where required. - Label winding clearly as per phase sequence. - Ensure number of turns and layers meet design specifications. - Avoid overlaps, loose turns, or damaged conductors during winding. - Tie, tape, or secure winding according to standard practices. - Install and insulate terminals or leads correctly. - Perform required test as per SOPs | - Transformer winding type and their applications. - Winding materials such as copper/aluminum conductors, insulation paper, and varnish. - Winding techniques and safety precautions during operation. Practical Activity: Candidate is required to perform winding operation on a practice coil includes preparing conductor material, setting up equipment, winding as per specifications with proper insulation and alignment, followed by inspection for defects and recording results. | Theory-02Hrs Practice17Hrs Total- 19Hrs | Consumable: - Copper/aluminum wire or strip conductors - Insulation paper/tape - Varnish - Marking tags/labels - Logbook or inspection sheets | - Classroom - Workshop |
| LU4. Perform reassembling of transformer | - Clean and inspect components before assembly. - Place windings core as per design. - Connect leads, taps, and bushings securely according to wiring diagram. - Assemble and tighten clamping structures, tank covers, and fasteners with proper torque. - Refill transformer oil as per SOPs | - Re-assembling steps and procedures for transformer re-assembling. Practical Activity: Reassembling a transformer involves placing windings, core, and bushings into the tank with proper insulation and fastening. | Theory-02Hrs Practice-17Hrs Total- 19Hrs | Consumable - Insulation paper/tape - Gaskets and sealing material - Fasteners (nuts, bolts, washers) - Marking tags/labels - Logbook or reassembly checklist sheets | - Classroom - Workshop |
| LU5. Perform Post Winding Inspection and Testing for smooth functioning | - Prepare transformer for post winding inspection test - Conduct quality inspection tests as per SOP - Document the records | - Application of safety procedures in winding. - Transformer winding faults and its inspection. Practical Activity: Candidate is required to perform post winding inspection, carry out required tests for smooth functioning and record results. | Theory-02Hrs Practice-17Hrs Total- 19Hrs | Consumable - Test log sheets or inspection forms - Marking labels/tags - Insulation paper/tape (for minor corrections) - Cleaning wipes/cloths - Pens and markers | - Classroom - Workshop |
| LU6. Reinstall and energize the transformer | - Position transformer at designated site using proper handling and lifting tools. - Connect HV/LV terminals, bushings, and earthling as per wiring diagram. - Energize transformer on no-load and monitor voltage, current, and abnormal noise. | - Transformer reinstallation steps and procedures. Practical Activity: Candidate is required to reinstall the transformer, test connections and oil, energize under supervision, record readings. | Theory-02Hrs Practice-17Hrs Total- 19Hrs | Consumables - Logbook or energization report sheets - Insulation tape and marking labels - Transformer oil (for topping up if required) - Cleaning cloths/wipes - Pens and markers | - Classroom - Workshop |

## 0713E&E4306 Fabricate & Weld Transformer Components

Objective: This Module covers the check, inspect, and verify the functionality, condition, and safety of welding equipment and tools prior to use, read, understand, and interpret fabrication and welding drawings, perform basic welding operations as part of transformer assembly, repair, or fabrication tasks, carry out basic fabrication operations relevant to transformer manufacturing and repair and perform essential post-welding processes on transformer components.

Theory: 20 Hours				Practice: 80 Hours 					Duration: 100 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Check Welding Equipment & Tools | - Ensure the work area is clean and free from flammable materials before inspection begins. - Check welding machine for physical damage, loose connections, or missing parts. - Verify input power and grounding connections are secure and safe. - Inspect welding cables, electrode holder, and ground clamp for cuts, burns, or loose fittings. - Test output voltage and amperage (if applicable) for proper function. - Check electrode holders, nozzles, and contact tips for cleanliness and wear. - Ensure that tools are clean, undamaged, and stored properly when not in use. - Identify and record any equipment or tool that is unsafe or not functioning correctly. | - Types and functions of welding equipment used in transformer fabrication - Basic operating principles of welding machines (SMAW & Oxy fuel) Practical Activity Candidate is required to perform pre-operational safety inspection of a welding station, identify any potential hazards and inspection of welding tools and Equipment as "Ready for Use," "Needs Maintenance," or "Unsafe." | Theory-05Hrs Practical-13Hrs Total- 18Hrs | - Inspection and Documentation: - Pre-operational safety checklist - Equipment to Inspect: - Welding machine (e.g., SMAW & Oxy fuel) - Welding cables (electrode and ground) - Electrode holder and ground clamp - Power supply cables and industrial plug. - Tools for Inspection: - Inspection mirror - Flashlight - Wire brush | - Classroom - Workshop |
| Interpret Fabrication & Welding Drawing | - Recognize title block information including drawing number, title, scale, and revision. - Interpret drawing lines, notations, and general layout for fabrication. - Interpret dimensional and geometric tolerances - Identify datum points and baseline references from the drawing. - Recognize standard welding symbols - Interpret weld type, size, location, and length from drawings. - Identify material specifications such as type, grade, thickness, and quantity. - Use drawing to plan layout, cutting, fitting, and welding sequence | - Basic types of engineering drawings used in transformer fabrication. - Basic math, geometry, and units of measurement in drawings. - Types of welds and joints (fillet, groove, lap, butt) - Types of material used in fabrication in welding of Transformer Practical Activity Candidates are required to translate a complete fabrication drawing into a step-by-step manufacturing plan, detailing each phase from material preparation to final welding | Theory-05Hrs Practical-13Hrs Total- 18Hrs | - Consumable: - Drawing Template | - Classroom - Workshop |
| Perform Welding Operation | - Select appropriate welding process (e.g., SMAW, MIG) based on material and job specification. - Inspect and set up welding equipment, cables, clamps, and accessories for operational readiness. - Identify and prepare welding materials (e.g., transformer tanks, brackets, enclosures). - Strike and maintain a stable arc during welding. - Perform welds using correct technique, electrode, and current settings as per job requirement. - Control heat input to avoid warping or damage to thin transformer sheets or oil tanks. - Maintain correct weld bead profile, overlap, and penetration in line with specifications - Clean weld joints using appropriate tools (wire brush, chipping hammer, grinder). | - Basic welding principles and arc characteristics. - Potential welding defects and their causes. Practical Activity Candidates are required to practice selecting the correct welding process (SMAW or oxy fuel) for a given material and to execute a quality weld by controlling all necessary parameters. | Theory-05Hrs Practical-18Hrs Total- 23Hrs | - Consumable for SMAW: - Electrode - Base Metal - Consumable for Oxy fuel: - Oxygen gas - Acetylene gas - Filler metal | - Classroom - Workshop |
| Perform fabrication Operation | - Select appropriate materials, tools, and equipment for the fabrication process. - Measure work pieces accurately using appropriate tools (e.g., tape measure, steel rule, vernier caliper). - Mark lines and reference points according to drawings or templates. - Verify layout accuracy before proceeding to cutting or drilling operations. - Bend or form materials using appropriate equipment (e.g., press brake, manual bender). - Smooth cut edges and remove burrs using grinding or filing tools. | - Basics of fabrication processes used in transformer manufacturing. - Measuring and marking tools - Common fabrication defects and prevention methods. Practical Activity Candidates are required to design, measure, lay out, cut, bend, and finish a small custom metal bracket according to a provided technical drawing. | Theory-05Hrs Practical-18Hrs Total- 23Hrs | - Consumables: - Sheet metal - Cutting discs - Grinding discs - Flap discs - Marking chalk or bluing - Primer or paint | - Classroom - Workshop |
| Perform Post-Welding Processes | - Remove slag, spatter, and oxidation using appropriate tools (e.g., wire brush, chipping hammer, grinder). - Clean weld areas thoroughly to ensure readiness for inspection or finishing. - Handle and dispose of waste material safely and in compliance with workplace standards. - Perform visual inspection of welds for common defects (e.g., cracks, porosity, undercut, incomplete fusion). - Measure weld size, alignment, and bead consistency as per drawing or standard. - Mark and report any defective areas requiring repair. - Prepare welded tanks or enclosures for testing (e.g., sealing openings). - Perform leak test using air pressure or vacuum (according to standard test procedures). - Grind or gouge out defective welds using suitable equipment. - Re-weld affected area following standard welding practices. - Clean and inspect reworked welds to confirm defect has been removed. - Apply anti-corrosion coatings (e.g., primer, rust- proof paint) using appropriate tools (brush, spray). | - Types of leakages in transformer. - Air pressure or vacuum leak testing procedures for transformer tanks - Disposal techniques of waste material. Practical Activity Candidates are required to practice identifying weld defects, performing necessary repairs, and verifying the quality of the repaired area. | Theory-05Hrs Practical-18Hrs Total- 23Hrs | - Consumable: - Base metal: - Welding Electrodes - Grinding discs - Filler metal - Cleaning solvents - Anti-corrosion coating - Leak Detection Solution. | - Classroom - Workshop |

## 0713E&E4306 Develop Entrepreneurship/Business

Objective: This Module covers the underpinning knowledge and skills required for a student/trainee to be competent in developing entrepreneurial skills and business operations within the energy sector. After completing this competency standard, the Trainee/student will be able to successfully perform all the tasks related to communication, business structuring, financial management, customer engagement, and regulatory compliance in accordance with the essence of the Competency-Based Training System. After completing the competency standard, the student will be able to perform all the tasks (LU1 to LU10) in accordance with the essence of the Competency-Based Training System.

Theory: 10 Hours				Practice: 60 Hours 					Duration: 35 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Assess market opportunities | - Identify market demand for transformer installation, maintenance, repair, and monitoring in a specific region or niche. - Analyze the competitive landscape, including established companies and independent technicians. - Research pricing models and customer service gaps in the current market. | - Transformer market supply and demand principles - Consumer behavior and decision-making (quality and price) Scenario: A group of trainees is required to conduct market survey for the feasibility to start transformer service and business. | Theory-01Hrs Practical-08Hrs Total- 09Hrs | - Feasibility plan template | - Classroom - Workshop |
| Develop a business plan | - Create a business plan that outlines the vision, mission, and objectives of the transformer service business. - Define the target market, service offerings, and unique value proposition. - Develop a marketing strategy to attract and retain clients. | - Procedures and steps to develop new business plan. Scenario: Candidate is required to develop a business plan as per market survey. | Theory-01Hrs Practical-08Hrs Total- 09Hrs | - Business Plan Template | - Classroom - Workshop |
| Manage business finances | - Prepare and manage a budget for business operations. - Maintain accurate financial records of income and expenses. - Understand financial statements like cash flow and profit-and-loss reports | - Basic business financial management Scenario: Candidate is required to generate daily consumable, non-consumable financial report. | Theory-01Hrs Practical-08Hrs Total- 09Hrs | - Budget Plan Template | - Classroom - Workshop |
| Implement risk management strategies | - Identify potential business risks, such as liability, client issues, and economic downturns. - Evaluate insurance needs for the business and personal liability. - Develop contingency plans to minimize business disruption. | - Basic risk management strategy Scenario: Candidate is required to evaluate contingency plan to navigate the risk issues. | Theory-01Hrs Practical-08Hrs Total- 09Hrs | - Risk Assessment Template | - Classroom - Workshop |
| Communicate effectively with clients | - Communicate clearly and confidently with clients to understand their needs. - Provide detailed quotes and transparent explanations of work to be performed. - Handle client inquiries, complaints, and negotiations professionally. | - Types of communication - Principles of communication Scenario: Candidate is required to roleplay as a businessman and client for different effective means of communications. | Theory-01Hrs Practical-09Hrs Total- 10Hrs | - BOQ Template | - Classroom - Workshop |
| Promote business services | - Create and manage a professional online presence, such as a website or social media profile. - Market services through various channels, including digital marketing, local directories, and industry partnerships. - Develop a strong customer service approach to generate repeat business and positive referrals. | - Effective means for promotion Scenario: Candidate is required to develop a brochure, template and social media account. | Theory-01Hrs Practical-09Hrs Total- 10Hrs | - Printer Pages. | - Classroom - Workshop |
| Plan for business growth. | - Assess opportunities for business expansion, such as hiring employees or expanding service lines. - Develop strategies for scaling operations while maintaining quality and efficiency. - Prepare for business succession or an eventual exit strategy. | - Business growth models and its strategies Scenario: Candidate is required to systematically evaluate expansion opportunities, create a detailed scaling plan, and formulate a long-term strategy for succession or a business exit. | Theory-01Hrs Practical-10Hrs Total- 11Hrs | - Printing Pages. | - Classroom - Workshop |

# List of Personal Protective Equipment (For A Class Of 25 Students)

| Name of Trade |  | Transformer Technician |  |
| --- | --- | --- | --- |
| Duration of Course |  | 1 Year |  |
| Sr. # | Description |  | Quantity |
|  | First AID Box |  | 02 Nos |
|  | Fire Extinguisher Cylinder |  | 02Nos |
|  | Fire Blanket |  | 06 Nos |
|  | Fire Bucket |  | 06 Nos |
|  | Safety googles |  | 25 Nos |

# List of Tools and Equipment

| S.No | Items | Quantity |
| --- | --- | --- |
| 1 | Personal Protective Equipment’s (PPEs) (Safety Gloves (upto rated 2kV&18kV), Safety Helmets, Safety Glasses, Arc Flash Suit, Safety Shoes, Full Body Harness, Safety Belt, Ear Plugs, Ear Moff, Apron, Safety Jacket) | 25 Sets |
| 2 | Fire Extinguishers (CO2, DCP) | 01 each |
| 3 | Fire Hydrants | 01 Nos |
| 4 | Fire Ball Extinguisher | 02 Nos |
| 5 | Fire Blankets | 10 Nos |
| 6 | Sand Buckets | 06 Nos |
| 7 | First Aid Kit | 02 Nos |
| 8 | Safety Mask N95 | 02 Nos |
| 9 | Portable Temporary Ground | 02 Nos |
| 10 | Grounding Rods & Cables | 02 Nos |
| 11 | Lockout/Tagout (LOTO) Kits | 02 Nos |
| 12 | Safety Barriers & Cones | 02 Nos |
| 13 | Smoke & Gas Detectors | 02 Nos |
| 14 | Emergency Alarm Systems | 02 Nos |
| 15 | Digital Multimeter | 02 Nos |
| 16 | DC/AC Clamp Meter | 02 Nos |
| 17 | Power Quality Analyzer | 02 Nos |
| 18 | Infrared Thermometer | 02 Nos |
| 19 | Motor Generator Set | 02 Nos |
| 20 | Fuel Flow Meter (For Diesel Generators) | 02 Nos |
| 22 | Solar Irradiance Meter (For Renewable Systems) | 02 Nos |
| 23 | Lockout/Tagout (LOTO) Kit | 06 Nos |
| 24 | Grounding Rods & Clamps | 06 Nos |
| 25 | Insulation Resistance Tester | 06 Nos |
| 26 | Circuit Breaker Analyzer | 06 Nos |
| 27 | Synchroscope | 06 Nos |
| 28 | Magnetic Contactor 220V | 06 Nos |
| 30 | Magnetic Contactor 400V | 06 Nos |
| 31 | On-Off Push Buttons | 06 Nos |
| 32 | Rev-Stop-For Push Button | 06 Nos |
| 33 | Thermal Overload | 06 Nos |
| 34 | Phase Sequence Relay | 06 Nos |
| 35 | Current Transformer (CTs) | 06 Nos |
| 36 | Potential Transformers (PTs) | 02 Nos |
| 37 | Galvanometer | 02 Nos |
| 38 | Exciters | 02 Nos |
| 40 | Automatic Voltage Regulator | 02 Nos |
| 41 | Frequency Meter | 02 Nos |
| 42 | Single Phase Wattmeter | 02 Nos |
| 43 | Three Phase Wattmeter | 02 Nos |
| 44 | DC Wattmeter (100W) | 02 Nos |
| 45 | Digital Lux Meter | 02 Nos |
| 46 | Analog Lux Meter | 02 Nos |
| 47 | Solar Panels (180W, 585W) | 02 Nos |
| 48 | Inverters (1000W, 2kW, 5kW) | 02 Nos |
| 49 | Controller Trainer | 04 Nos |
| 50 | Charge Controller PWM (20A, 30A) | 03 each |
| 51 | Charge Controller MPPT (40A) | 04 Nos |
| 52 | Battery 100Ah | 05 Nos |
| 53 | Battery 200Ah | 05 Nos |
| 54 | Single Phase Energy meters (Analog and Digital) | 05 each |
| 55 | Three Phase Energy meters (Analog and Digital) | 05 each |
| 56 | Testing Board | 25 Nos |
| 57 | Single Pole Circuit Breaker (C6, C13, C63) | 02 each |
| 58 | Two Pole Circuit Breaker (C6, C13, C63) | 02 each |
| 59 | Three Pole Circuit Breaker (C6, C13, C63) | 02 each |
| 60 | Power Factor meter | 05 Nos |
| 61 | kVAR meter | 02 Nos |
| 62 | Four Pole Circuit Breaker (C6, C13, C63) | 02 each |
| 63 | Barcode Scanners (For Equipment Tagging & Tracking) | 01 Nos |
| 64 | RFID Tags & Readers (For Asset Management) | 01 Nos |
| 65 | Handheld Label Printers (For Equipment & Component Labelling) | 01 Nos |
| 66 | Digital Weighing Scales (For Measuring Components & Materials) | 02 Nos |
| 67 | Microsoft Office 2024 | 01 Nos |
| 68 | Computer | 25 Nos |
| 69 | Frequency Telephone Set | 02 Nos |
| 70 | kVARs Meters | 05 Nos |

# List of Consumables

| S. No. | Items | Quantity |
| --- | --- | --- |
|  | Cable (various gauges: 1.5mm², 2.5mm², 4mm², etc.) | 90m per type |
|  | Log Books | 25 Nos |
|  | Microsoft Office 2024 | 01 Nos |
|  | Safety googles, helmets, high voltage gloves. | 25 each |
|  | Connectors & terminals (lug connectors, ring terminals) | 1000 pcs |
|  | Fuses (various ratings: 5A, 10A, 15A, 20A) | 100 each |
|  | Circuit breakers (MCB, MCCB, RCD) | 50 each |
|  | Batteries (for meters & analyzers, AA, AAA, 9V, lithium-ion) | 24 each |
|  | Insulation tape (red, black, yellow, green, blue) | 5 roll each |
|  | Heat shrinks tubing (assorted sizes) | 100m |
|  | Cable glands (assorted sizes) | 100 pcs |
|  | Cable lugs & ferrules | 1000 pcs |
|  | Safety gloves (electrical insulated, cut-resistant, nitrile) – | 50 pairs |
|  | Helmets | 50 pcs |
|  | Safety glasses | 50 pcs |
|  | Arc flash suits | 5 sets |
|  | Face masks (N95, surgical) | 300 pcs |
|  | Earplugs | 200 pairs |
|  | Face shields | 10 pcs |
|  | High-visibility vests | 50 pcs |
|  | Respirators | 10 pcs |
|  | Safety shoes (optional if required) | 50 pairs |
|  | Cable ties (various sizes) | 2000 pcs |
|  | Marking labels (self-adhesive, cable labels) | 500 pcs |
|  | Lockout/Tagout tags | 100 pcs |
|  | Industrial-grade lubricants & grease | 20 Liters |
|  | Contact cleaner spray | 10 cans |
|  | Rust removers | 5 Liters |
|  | Wire strippers & cutters (extra consumable blades) | 10 pcs |
|  | Calibration stickers & logbooks | 10 set |
|  | Ethernet cables (Cat 6, Cat 5e) | 1000m roll |
|  | RJ45 connectors | 500 pcs |
|  | Fiber optic cables | 500m |
|  | Fiber optic connectors (SC, LC, ST) | 100 pcs |
|  | Network switch replacement modules | 5 pcs |
|  | Power strips & surge protectors | 20 pcs |
|  | BNC connectors (for oscilloscopes) | 10 pcs |
|  | Fire extinguishers (CO₂, dry chemical, foam) | 2 pcs |
|  | Fire extinguisher refills | 5 sets |
|  | Fire blankets | 10 pcs |
|  | First aid kits | 10 kits |
|  | Emergency exit sign | 10 pcs |
|  | Emergency lights (batteries included) | 10 pcs |
|  | Eye wash station solution | 5 Liters |
|  | Safety cones & barriers | 20 sets |
|  | Multimeter probes & leads | 50 sets |
|  | Clamp meter accessories (flexible CT coils, extra clamps) | 10 sets |
|  | Oscilloscope probes | 20 sets |
|  | Pressure gauge calibration kits | 5 sets |
|  | Infrared thermometer lens cleaning wipes | 50 packs |
|  | Battery analyzer test leads | 20 sets |
|  | Earth tester auxiliary electrodes | 10 sets |
|  | Whiteboard Markers | 50 sets |
|  | Whiteboard Cleaner | 10 bottles |
|  | Projector bulbs | 5 pcs |
|  | Presentation boards & flip charts | 5 pcs |
|  | Printed training manuals & datasheets | 25 copies |
|  | USB flash drives (for software & reports) | 50 pcs |
|  | Ethernet patch cables (for network devices) | 100 pcs |
|  | Barcode labels (for scanner testing) | 1000 pcs |

# Members of Qualification Development Committee

The Qualifications Development Committee consisted of the following members:

| Sr. # | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1. | Engr. Dr. Mian Khuram Ahsan | Assistant Professor (Electrical) (DACUM) | GPI Mardan ( KP TEVTA) |
| 2. | Hafiz Muhammad Fahad Sultan | Instructor Electrical | GCT Jhang, PTEVTA |
| 3. | Khalid Mahmood | Instructor Electrical | GCT Sialkot, PTEVTA |
| 4. | Engr. Hassan Tahir | Assistant Manager | International Link pvt. Ltd. |
| 5. | Muhammad Shafiq | Sr. Instructor Electrical | VTI DG Khan (PVTC) |
| 6. | Altaf Solangi | Manager | K Electric (Karachi) |
| 7. | Engr. Muzammil Idrees | Service manager | Bestway Engineering Multan |
| 8. | Engr Waleed Akhter | Lecturer Electrical | GCT Kohat, K Tevta |
| 9. | M Zulqarnain | Service Manager | Morgan Technologies |
| 10. | Moazzam Munir | Instructor (Electrical) | GIST Sialkot, P.TEVTA |
| 11. | Saeed Hussain Shah | Assistant Professor (Electrical) | GCT Tangi, KP TEVTA |