Competency Standards

for

National Diploma Level 4

in

Water Quality Technician

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Water Quality Technician. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the industry experts and other private industries, Academic experts from different universities, experts of TEVTAs, PVTC, PCSIR, PSQCA, PCRWR, CDA, PHED diligently for making this qualification worthy and error-free for which all credit goes to them. However, NAVTTC accepts responsibility for all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards need periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of national and international industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	1

1.1	Date of development:	1

1.2	Date of review	1

1.3	Entry requirement of trainee	1

1.4	Trainer qualification requirement	1

1.5	Codes of qualification (ISCED)	1

2.	Summary of Competencies	2

3.	Qualification Levelling and Packaging	2

4.	Description of Qualification Package	2

5.	Competency Standards	3

5.1 0712EPT1101 Perform Sampling of Water	3

5.2 0712EPT1102 Perform Disposal of waste	4

5.3 0712EPT1103 Perform Advance Physical testing of Wastewater	6

5.4 0712EPT1104 Perform Advance Chemical Testing of Water	7

5.5 0712EPT1105 Perform Advance Chemical Testing of Wastewater	10

5.6 0712EPT1106 Perform Advance Microbiological Testing of water	13

5.7 0712EPT1107 Apply Quality Assurance Procedure in Water Testing Operations	15

5.8 0712EPT1108 Manage Data in Water Testing Operations	17

5.9 0712EPT1109: Apply Environmental Compliance in Water Testing	18

5.10 0712EPT1110: On-Job Training/ Project	20

6.	List of Consolidated Consumables, Tools, Machinery & Equipment	21

7.	Members of Qualifications Development & Review Committee	24

# Introduction

This comprehensive course is designed to equip learners with foundational competencies essential for entering the field of Water Quality Technology. The curriculum follows a hands-on, skill-based learning approach that fosters both technical proficiency and professional growth. Participants will begin by mastering advance water testing skills, including water sampling and advance physical, chemical, and microbiological testing of both water and wastewater, data management along with environmental regulations. Emphasis is placed on proper waste handling procedures and adherence to testing fundamentals and core laboratory protocols, ensuring learners build a strong foundation for advanced analytical practices.

To enhance employability and career readiness, the program integrates modules on professional portfolio development and the application of water testing competencies in real-world contexts. Learners will gain hands-on experience through On-Job Training (OJT), where they will apply classroom knowledge in practical settings under professional supervision. By the end of the program, graduates will be well-prepared to contribute effectively to maintaining and improving water quality across a range of industries and environmental sectors.

| ## Date of development: | 13th October 2025 – 17th October 2025 |
| --- | --- |
| ## Date of review | October 2027 |
| ## Entry requirement of trainee | National Vocational Qualification for Water Quality Assistant Level – 3 OR Secondary School Certificate in Science with at least 6 Month of relevant work experience certificate |
| ## Trainer qualification requirement | MSc in Chemistry/Microbiology/Environmental Sciences/ Bio-chemistry with at least 1 year experience in relevant field OR BSc in Chemistry/Microbiology/Environmental Sciences/Bio-chemistry with at least 2-year experience in relevant field OR DAE in Chemical Technology/Lab Technology with at least 4-year experience in relevant field |
| ## Codes of qualification (ISCED) | Code: 0712EPT11 Description: National Vocational Certificate Level 4 in Water Sector (Water Quality Technician) |

# Summary of Competencies

| CODE | Competency Standards | NVQF Level | Category | Estimated Contact Hours |  |  | Cr. Hr. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. | Pr. | Total |  |
| 0712EPT1101 | Perform Sampling of Water | 4 | Technical | 9 | 42 | 51 | 3 |
| 0712EPT1102 | Perform disposal of Waste | 4 | Technical | 9 | 42 | 51 | 3 |
| 0712EPT1103 | Conduct Advance Physical Testing of Wastewater | 4 | Technical | 8 | 54 | 62 | 3.5 |
| 0712EPT1104 | Conduct Advance Chemical Testing of Water | 4 | Technical | 19 | 72 | 91 | 5.5 |
| 0712EPT1105 | Conduct Advance Chemical Testing of Wastewater | 4 | Technical | 9 | 72 | 81 | 4.5 |
| 0712EPT1106 | Conduct Advance Microbiological Testing of Water | 4 | Technical | 15 | 90 | 105 | 6 |
| 0712EPT1107 | Implement Quality Assurance Procedure in Water Testing Operations | 4 | Technical | 6 | 48 | 54 | 3 |
| 0712EPT1108 | Manage Data in Water Testing Operations | 4 | Technical | 12 | 36 | 48 | 3 |
| 0712EPT1109 | Maintain Environmental Compliance in Water Testing | 4 | Technical | 10 | 30 | 40 | 2.5 |
| 0712EPT1110 | On-Job Training/ Project | 4 | Technical | - | - | 240 | 6 |
|  | Total |  |  | 97 | 486 | 823 | 40 |

- Qualification Levelling and Packaging

| S.No | Occupations / Short Course | Modules/CS Codes | Level | Occupation Credit Hours | Training duration |
| --- | --- | --- | --- | --- | --- |
|  | N/A | N/A | N/A | N/A | N/A |

- Description of Qualification Package

| Level 2 | Level 3 | Level 4 | Level 5 |
| --- | --- | --- | --- |
| N/A | Water Quality Assistant | Water Quality Technician | N/A |

- Competency Standards

### 5.1 0712EPT1101 Perform Sampling of Water

Overview:

This competency standard covers knowledge and skills required to understand water sampling. It is the process of collecting water from various sources to analyze physical, chemical, and microbiological parameters. Proper sampling procedure and field equipment’s are essential to avoid contamination and ensure reliable analytical results

| Competency Units | Performance Criteria |
| --- | --- |
| Prepare field sample kit | - Identify work plan required for sampling - Select field proformas required - Identify sample bottles required for sampling - Select preservatives for required sample - Calibrate EC meter required - Calibrate pH meter required - Calibrate DO meter required - Check functionality of thermometer - Check functionality of total and free chlorine kit required - Check ice box for sample storage |
| Collect water sample | - Identify instrument required for sampling - Identify source for water sample collection - Label sample bottles as per requirement - Collect water for microbiological sample in required bottle aseptically - Collect water sample as per required test - Perform sample preservation with required reagents |
| Collect wastewater sample | - Identify source point for wastewater sample - Select required glass bottle for wastewater sample collection - Clean sampling bottle three times with sample water - Take Sample from sufficient depth as per SOP - Preserve sample for COD with H2SO4 to lower pH |
| Perform field test analysis | - Select source for test analysis - Record temperature of water sample source - Record pH of source using pH meter - Record electrical conductivity using EC meter - Record dissolved oxygen (DO) of source using field DO meter - Record total and free chlorine using chlorine kit |
| Perform sample handling | - Identify sample bottle - Label required sample bottles as per SOP - Seal sample bottles with paper tape - Place samples inside the insulated ice boxes - Maintain required temperature during transportation - Transport samples to laboratory as per required SOP |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes Knowledge and understanding of:

- Understand proper field kit preparation and field instruments calibration, before sampling

- Classification of water sources

- Types of water contaminants and their hazards

- Knowledge of standard sampling procedure according to APHA

- Understand sample preservation and temperature control in field

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Submit calibration record of on-site field-testing equipment

- Submit properly filled field performa.

Consumables, Tools, Machinery & Equipment

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Electrical Conductivity Meter | 1 |
| 02 | TDS Meter | 1 |
| 03 | pH meter | 1 |
| 04 | Dissolved Oxygen Meter | 1 |
| 05 | Total and Free Chlorine Meter | 1 |
| 06 | Thermometer | 1 |
| 07 | GPS | 1 |

### 5.2 0712EPT1102 Perform Disposal of Waste

Overview:

After this Competency Standard, the trainee will be able to handle, transport and dispose off solid, chemical and microbiological waste in compliance with health, safety, and environmental guidelines. It emphasizes the use of PPEs, safe handling practices, and record keeping to maintain a clean and sustainable workplace.

| Competency Units | Performance Criteria |
| --- | --- |
| Perform disposal of solid waste | - Identify types of collected solid waste - Apply PPEs as per standards - Perform handling as per requirement - Arrange transport for dispose-off materials - Perform transport of solid waste at designated site - Perform waste disposal as per environmental guidelines - Maintain cleanliness of works place - Maintain record of solid waste |
| Perform disposal of chemical waste | - Identify types of collected chemical waste - Apply PPEs as per standards - Perform handling as per requirement - Store waste chemicals prior to dispose-off - Arrange transport for dispose-off materials - Perform transport of waste chemical at designated site - Perform chemical waste disposal as per environmental guidelines - Maintain cleanliness of works place - Maintain record of chemical waste |
| Perform disposal of microbiological waste | - Identify types of collected microbiological waste - Apply PPEs as per standards - Perform handling as per requirement - Perform disinfection of microbiological waste prior to dispose off - Arrange transport for dispose-off materials - Perform transport of microbiological waste at designated site - Perform microbiological waste disposal as per environmental guidelines - Maintain cleanliness of works place - Maintain record of microbiological waste |

Knowledge & Understanding

The candidate must be able to demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understanding personal protective equipment (PPE)

- Introduction to waste Handling and Transportation Procedures

- Introduction to treatment and disposal Methods

- Understanding emergency and Incident Management

- Record Keeping and Reporting

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate use of appropriate Personal Protective Equipment (PPE)

- Demonstrate operation of waste disposal equipment such as autoclaves or disinfection units following standard operating procedures (SOPs)

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Lab coat | 25 |
| 02 | Disposable gloves | 25 |
| 03 | Face mask | 25 |
| 04 | Safety goggles | 25 |
| 05 | Safety shoes | 25 |
| 06 | Head cap | 25 |
| 07 | Brushes, mops, and cleaning cloths | 03 |
| 08 | Wastewater disposal hoses | 02 |
| 09 | First aid kit | 01 |
| 10 | Absorbent pads | 02 |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Collection trolleys | 03 |
| 02 | Spill containment trays | 01 |
| 03 | Autoclave unit | 01 |
| 04 | Incinerator | 01 |
| 05 | Chemical disinfectant containers | 01 |
| 06 | Measuring cylinders | 03 |
| 07 | Shredder or crusher | 03 |
| 08 | Sodium hypochlorite or bleach solution | 05 |
| 09 | Detergents and disinfectants | 05 |

### 5.3 0712EPT1103 Conduct Advance Physical testing of Wastewater

Overview:

This competency standard will provide skills and knowledge related to Advance physical testing of water and wastewater. Trainee will be able to demonstrate his/her your skills in evaluation of oil and grease and total suspended solids.

| Competency Units | Performance Criteria |
| --- | --- |
| Perform oil & grease test | - Identify sample for test - Add hydrochloric acid in sample - Add n-hexane in acidified sample - Separate hexane layer by separating funnel - Evaporate hexane to dryness - Record residue weight - Calculate oil and grease concentration in mg/L by using formula |
| Perform total suspended solids test | - Identify sample for test - Pass sample through pre-weight filter paper - Dry filter paper with residue at 105ºC for 1 hour - Re-weight filter paper - Record TSS by using formula |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Lab testing procedure in APHA

- Procedure of Total suspended solids.

- Lab procedure for oil and grease testing.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Submit test data through laboratory worksheet

Consumables, Tools, Machinery & Equipment

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Drying oven 250ºC | 1 unit |
| 02 | Electronic analytical balance, capacity 220g | 1unit |
| 03 | Filter paper pack | 1pack |
| 04 | Funnels 100 mm, | 5 |
| 05 | Cylinders (50ml, 100ml) | 4 |
| 06 | Separating funnel, capacity 250 ml | 5 |
| 07 | Tripod stand | 5 |

### 5.4 0712EPT1104 Conduct Advance Chemical Testing of Water

Overview:

This competency standard will provide skills and knowledge related to perform advance chemical testing of water & wastewater. It involves testing mainly Na, K, COD, BOD, cyanides, ammonia and total nitrogen. These analyses evaluate overall water quality and treatment efficiency, ensuring compliance with environmental standards and safeguarding public health.

| Competency Units | Performance Criteria |
| --- | --- |
| Perform sodium test | - Identify required sample - Switch on flame photometer as per OEM - Select sodium filter - Calibrate equipment for measurement of required concentration - Perform sample filtration, if needed - Record sodium concentration in mg/L by using formula - Dilute sample, if required - Clean flame photometer after test as per SOP |
| Perform potassium test | - Identify required sample - Switch on flame photometer as per OEM - Select potassium filter - Calibrate equipment for measurement of required concentration - Perform sample filtration, if needed - Record potassium concentration in mg/L by using formula - Dilute sample, if required - Clean flame photometer after test as per SOP |
| Perform nitrite test | - Identify sample bottle - Prepare series of standards from stock solution - Pipette standards in beaker - Add reagent for complex color formation - Calibrate spectrophotometer by measuring absorbance of standards - Calculate nitrite concentration in mg/L by using calibration curve |
| Perform nitrate test | - Identify required sample - Prepare series of nitrate standards from stock solution - Pipette standards in beakers - Add required acid - Calibrate spectrophotometer by taking absorbance of sample in quartz sample cell - Calculate nitrate concentration in mg/L by using calibration curve |
| Perform iron test | - Collect acidified water sample in polyethylene bottle - Prepare iron calibration working standards from stock - Pipette standard in beaker - Add buffer solution to control pH - Add 1,10-phenanthroline reagent to each standard - Calibrate instrument by measuring absorbance - Record iron concentration in mg/L by using calibration curve |
| Perform fluoride test | - Identify sample for fluoride test - Prepare fluoride standard series from stock solution - Pipette standards in beakers - Add SPADNS reagent by mixing in sample - Calibrate equipment taking absorbance of standards in sample cell - Calculate sample concentration in mg/L by using calibration curve |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes Knowledge and understanding of :

- Testing procedures as American Public Health Association (APHA) standard method book.

- Understanding of the local and international regulatory guidelines for drinking and wastewater quality

- Well versant of the operation of the spectrophotometer as per OEM

- Aware of Good Laboratory Practices and instrument routine maintenance

- Knowledge of the hazardous nature of chemicals

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Submit analysis results of tested sample(s) in the data sheet.

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Sodium Chloride | 500 gm |
| 02 | Potassium Chloride | 500 gm |
| 03 | Potassium Nitrite | 500 gm |
| 04 | Potassium Nitrate | 500 gm |
| 05 | Ferrous ammonium sulfate | 500 gm |
| 06 | SPADNS Reagent for Fluoride | 500 mL |
| 07 | chloramine-T | 100 gm |
| 08 | Lead carbonate, PbCO3 | 250 gm |
| 09 | Sulfamic acid, NH2SO3H | 250 gm |
| 10 | Potassium Cyanide | 250 gm |
| 11 | Barbituric acid | 250 gm |
| 12 | Pyridine | 500 mL |
| 13 | sodium acetate trihydrate (NaC2H3O2 _ 3H2O) | 500 gm |
| 14 | Glacial Acetic Acid | 2.5 L |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Flame Photometer with accessories (LPG Cylinder, CRMs for Sodium & potassium) | 01 No |
| 02 | UV/VIS Spectrophotometer with variable sample holder and accessories | 01 Unit |
| 03 | Cyanide Distillation Assembly | 01 Unit |
| 04 | Refrigerator | 1 |

### 5.5 0712EPT1105 Conduct Advance Chemical Testing of Wastewater

Overview:

This competency standard will provide skills and knowledge related to perform advance chemical testing of wastewater mainly ammonia, phosphate, COD, BOD, Sulfate and total nitrogen. These tests help assess water quality and the effectiveness of treatment processes. Regular monitoring ensures compliance with environmental standards and protects public health.

| Competency Units | Performance Criteria |
| --- | --- |
| Perform sulfate test | - Take 100 ml sample in beaker - Adjust pH with acid - Add BaCl2 to heated sample until precipitate appear - Digest precipitate at required temperature - Filter precipitate with ashless filter paper - Ignite residue at 800°C in muffle furnace - Calculate Sulfate concentration in mg/L by using formula |
| Perform ammonia test | - Select sample for ammonia test - Add sample in distillation flask - Add NaOH solution in flask - Dispense 4% boric acid with mixed indicator in titration flask - Steam distill sample for collecting it in receiving flask - Run reagent blank using distilled water in the same manner - Titrate distillate against standard acid - Record titrant volumes for sample and blank - Calculate ammonia as nitrogen in mg/L by using formula |
| Perform phosphate test | - Select sample for required test - Prepare phosphate standard series from stock solution - Pipette standard in clean beaker - Add ammonium molybdate reagent - Add stannous chloride reagent - Calibrate spectrophotometer by measuring absorbance - Record phosphate concentration in mg/L by using calibration curve |
| Perform chemical oxygen demand test | - Identify sample for COD testing - Prepare series of COD standards from stock solution - Pipette required volume of standards in COD vials - Digest COD vials at 150 C for 2 hours - Cool digested COD vials in dark - Calibrate instrument by measuring absorbance - Measure sample COD in mg/L by using calibration curve |
| Perform biological oxygen demand test | - Identify sample for BOD test - Prepare dilution water - Add required dilution water in prepared BOD bottles - Add nitrification inhibitor in BOD bottles - Record initial Dissolved Oxygen using Dissolved Oxygen Meter - Incubate sample, blank, seed BOD bottles at 20±1°C - Record final dissolved oxygen using DO Meter after 5 days - Calculate corrected oxygen uptake by sample - Calculate BOD in mg/L by using formula |
| Perform hexavalent chromium test | - Select acidified preserved sample - Prepare standard Cr(VI) series from stock solution - Pipette standard in clean beaker - Adjust sample pH using dilute acid - Add 1,5-diphenylcarbazide reagent - Calibrate spectrophotometer by measuring absorbance - Record sample Chromium (VI) concentration in mg/L by using calibration curve |
| Perform total arsenic | - Rinse test bottle of field test kit with sample - Add sample in required bottle - Fix arsenic test strip in flask cap - Add required reagent-1 in sample - Add one spoon of reagent-2 in sample - Add reagent -3 in sample - Attach immediately cap with test strip - Remove test strip from the cap after required time - Compare developed color at strip with standard color chart - Record arsenic concentration in µg /L sample |
| Perform total nitrogen | - Select required sample into a Kjeldahl digestion flask - Add digestion reagents in sample - Perform digestion of sample - Add distilled water in cooled digested sample - Add 40% NaOH to make solution alkaline - Steam-distill released ammonia into 4% boric acid containing mixed indicator - Titrate the distillate against standard acid - Calculate total nitrogen in mg/L by using formula |

Knowledge & Understanding

The candidate must be able to demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Relevant standard testing procedures defined in American Public Health Association (APHA) for water and wastewater.

- Procedure of operation and use of spectrophotometer

- Preparation of reagent and its standardization

- Understanding of the local and international regulatory guidelines for drinking and wastewater quality

- Overview and testing procedure for advance chemical quality parameters in APHA

- Good Laboratory Practices and instrument routine maintenance

- Danger and hazardous nature of chemicals

- Basic chemical concentration units, formulas and terminologies

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Submit the analysis results at the laboratory calculation log sheet.

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Arsenic Test Kit | 01 Kit |
| 02 | Barium Chloride | 500 gm |
| 03 | Hydrochloric Acid | 2.5 L |
| 04 | Sodium Hydroxide | 500 gm |
| 05 | Boric Acid | 500 gm |
| 06 | Sulfuric Acid | 2.5 L |
| 07 | Methyl Orange Indicator | 100 gm |
| 08 | methyl red Indicator | 100 gm |
| 09 | methylene blue Indicator | 100 gm |
| 10 | Ammonium Molybdate | 100 gm |
| 11 | Stannous Chloride | 100 gm |
| 12 | ammonium metavanadate | 100 gm |
| 13 | di-Potassium hydrogen phosphate (K2HPO4) | 250 gm |
| 14 | Sodium Sulfate | 500 gm |
| 15 | Potassium hydrogen phthalate | 500 gm |
| 16 | Calcium chloride | 500 gm |
| 17 | Magnesium sulfate | 500 gm |
| 18 | Ferric chloride | 500 gm |
| 19 | potassium dihydrogen phosphate KH2PO4 | 250 gm |
| 20 | sodium phosphate dibasic heptahydrate Na2HPO4_7H2O | 250 gm |
| 21 | Ammonium Chloride (NH4Cl) | 500 gm |
| 22 | Magnesium Sulfate MgSO4_7H2O | 500 gm |
| 23 | Nitrification Inhibitor | 100 gm |
| 24 | Glucose | 250 gm |
| 25 | Glutamic Acid | 250 gm |
| 26 | Potassium Dichromate | 500 gm |
| 27 | 1,5-diphenylcarbazide | 100 gm |
| 28 | Nitric Acid | 2.5 L |
| 29 | Potassium Sulfate (K2SO4) | 500 gm |
| 30 | Copper Sulfate (CuSO4) | 500 gm |
| 31 | Sodium Thiosulfate (Na2S2O3 5H2O) | 500 gm |
| 32 | China Dish 100 cc | 10 Nos. |
| 33 | Whatmann Filter Papers 42 ash-less | 1 Pack |
| 34 | Kjeldhal Digestion and Distillation Apparatus 500mL | 01Set |
| 35 | Glass Beads | 500 gm |
| 36 | Glass Burette 50 mL | 02 Nos. |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Analytical Balance 0.1 to 220 gm (0.1 mg) | 1 Unit |
| 02 | Heating Mantle 500 mL 350 C | 1 Unit |
| 03 | Muffle Furnace 1100 C | 1 Unit |
| 04 | Drying Oven 240 C | 1 Unit |
| 05 | UV/VIS Spectrophotometer with variable sample holder and accessories | 01 Unit |
| 06 | COD Reactor 150 C, having 16 mm wells | 01 Unit |
| 07 | COD Vials 1500 mg/L | 25 Nos. |

### 5.6 0712EPT1106 Conduct Advance Microbiological Testing of water

Overview:

This competency standard will provide skills and knowledge related to microbiological examination of water and wastewater. You will be able to demonstrate your skills in understanding and performing quantitative analysis of major microbiological tests in laboratory for routine analysis/monitoring of water and wastewater

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform total coliform test | - Perform sterilization using laminar hood - Prepare Lauryl Tryptose Sulfate broth as required in tubes - Inoculate sample in tubes in prescribed sets with quantity of 10.0 ml, 1.0 ml and 0.1 ml - Incubate Lauryl Tryptose Sulfate tubes at 35°C for required time - Record gas positive tubes for Lauryl Tryptose Sulfate broth - Subculture positive tubes in Brilliant Green Bile broth - Incubate Brilliant Green Bile tubes at 35°C for required time - Record gas positive tubes for Brilliant Green Bile broth - Calculate Most Probable Number from Standard Table |
| CU2. Perform fecal coliform test | - Perform sterilization using laminar hood - Prepare Lauryl Tryptose Sulfate broth as required in tubes - Inoculate sample in tubes in prescribed sets with quantity of 10.0 ml, 1.0 ml and 0.1 ml - Incubate Lauryl Tryptose Sulfate tubes at 35oC for required time - Record gas positive tubes for Lauryl Tryptose Sulfate broth - Subculture positive tubes in Escherichia coli broth - Incubate Escherichia coli tubes at 44.5oC for required time - Record gas positive tubes for Escherichia coli broth - Calculate Most Probable Number from Standard Table |
| CU3. Perform total plate count test | - Perform sterilization using laminar hood - Select Plate Count Agar as required - Prepare 1:99 dilution of sample in phosphate buffer - Mix required solution on vortex mixer - Prepare serial dilution as required - Pour 1.0 ml of dilutions in petri plate with Plate count Agar - Incubate Plate count agar plates at 35oC for required time - Count number of colonies using colony counter - Report colony counting units per ml of the sample |
| CU4. Perform yeast & mold test | - Perform sterilization using laminar hood - Prepare Potato Dextrose Agar as required - Prepare serial dilution as required - Pass 100 ml on sample through required filter - Place filter paper on solidified plates of Potato Dextrose Agar - Incubate prepared plates at 25°C for 5 days - Count number of colonies using colony counter - Report colony counting units per ml of the sample |
| CU5. Perform E. coli test | - Perform sterilization using laminar hood - Prepare Lauryl Tryptose Sulfate broth as required - Inoculate sample in tubes in prescribed sets with quantity of 10.0 ml, 1.0 ml and 0.1 ml - Incubate Lauryl Tryptose Sulfate tubes at 35°C for required time - Subculture in Tryptone Broth for E.coli growth - Incubate required tubes at 44.5°C for 48h - Add Kovac’s reagent as required - Calculate Most Probable Number (MPN) |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes knowledge of:

- Types of microbiological contaminants

- Maintaining sterile environment inside the work area of laminar hood

- Significance of testing water and wastewater

- Qualitative estimation of microbial contamination in water and wastewater

- Quality status of water and wastewater

- Overview and testing procedure for advance microbiological quality parameters in APHA

- National and International guidelines/standards criteria for water and wastewater

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Submit a report containing calibration, and culture data

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Lauryl Tryptose Sulfate Agar | 1 bottle |
| 02 | Brilliant Green Bile Agar | 1 bottle |
| 03 | Escherichia coli Broth | 1 bottle |
| 04 | Plate Count Agar | 1 bottle |
| 05 | Butterfield’s Phosphate Buffer | 1 bottle |
| 06 | Potato Dextrose Agar | 1 bottle |
| 07 | Tryptone Broth | 1 bottle |
| 08 | Kovac’s reagent | 1 bottle |
| 09 | Micropipettes set and accessories | 2 |
| 10 | Glass tubes | 100 |
| 11 | Petri plates disposable | 500 |
| 12 | Glass beakers | 100 |
| 13 | Erlenmeyer flasks | 50 |
| 14 | Filter papers (0.4 µ) | 1 box |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Projector/Multimedia/White board | 1 |
| 02 | Laminar hood | 1 |
| 03 | Autoclave | 1 |
| 04 | Incubators | 3 |
| 05 | Analytical balance | 1 |
| 06 | Colony counter | 1 |
| 07 | Filtration Assembly | 1 |

### 5.7 0712EPT1107 Implement Quality Assurance Procedure in Water Testing Operations

Overview:

After this Competency Standard, the trainee will be able to ensure that all analytical activities in the laboratory are carried out in compliance with established quality assurance (QA) and quality control (QC) procedures to guarantee accuracy, precision, and reliability of results

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Calibrate analytical instruments | - Select calibration standards for each required instrument - Calibrate instruments according to OEM - Validate instrument performance using reference materials - Document calibration data in instrument logbooks - Troubleshoot malfunctioning of instrument, if any - Verify calibration status before analytical use |
| CU2. Apply internal quality procedure | - Identify non-conformances in analytical processes - Identify cause using analytical tools - Implement corrective and preventive actions to eliminate cause of malfunction - Verify effectiveness of corrective actions through re-analysis - Maintain records for audit reference - Communicate corrective actions to relevant personnel |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Principles of Quality Assurance (QA) and Quality Control (QC)

- Calibration and Validation Fundamentals

- Role of SOPs in ensuring consistent calibration, testing and documentation.

- Non-Conformance and Corrective Action (CAPA) Procedures

- ISO/IEC 17025 and Laboratory Accreditation Principles

- Professional Ethics and Data Integrity

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate calibration and validation of analytical instruments using correct standards and SOPs.

- Submit completed and maintained QA documentation, including calibration logs, QC charts and non-conformance records.

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | ISO/IEC 17025 Standard | As per requirement |
| 02 | NEQS/WHO water quality guidelines | As per requirement |
| 03 | SOP Manuals | As per requirement |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | pH meter | 01 |
| 02 | EC meter | 01 |
| 03 | DO meter | 05 |
| 04 | Spectrophotometer | 01 |
| 05 | turbidimeter | 01 |
| 06 | PPEs | 01 |

### 5.8 0712EPT1108 Manage Data in Water Testing Operations

Overview:

After this Competency Standard, the trainee will be able to manage analytical data and documentation in water testing laboratories. It includes recording, validating, and reporting data in compliance with ISO/IEC 17025 and environmental regulations. The technician ensures accuracy, traceability and confidentiality of all laboratory information.

| Competency Units | Performance Criteria |
| --- | --- |
| Maintain analytical data | - Record all analytical results in laboratory logs - Select standardized templates to capture analytical parameters - Verify completeness of recorded data - Back up data periodically as per requirement - Maintain confidentiality of analytical information |
| Maintain chain of custody documentation | - Assign unique identification codes to each sample - Record all sample transfers in chain-of-custody forms - Authenticate documentation with signatures and timestamps - Verify storage conditions during custody |
| Validate data integrity | - Check data entries for errors - Check analytical results with corresponding QC - Identify data anomalies - Maintain clear audit trail linking reports - Apply coding for data traceability - Report discrepancies to supervisor |
| Prepare analytical reports | - Compile verified analytical results into standard report templates - Add QC summaries & interpretation notes in reports - Review report contents for accuracy, completeness and compliance - Submit final reports to regulatory agencies - Archive all reports according to data retention policy |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Principles of Data Management in Laboratory Operations

- Structure and use of laboratory logbooks, worksheets and digital databases (LIMS or spreadsheets).

- Role of SOPs in ensuring consistent calibration, testing and documentation.

- Standardized formats, templates, units and nomenclature used in analytical documentation.

- Procedures for managing data revisions, backups and access control.

- Documentation and reporting requirements under ISO/IEC 17025, WHO, EPA and NEQS frameworks.

- Purpose of CoC, legal and procedural significance, and elements of a valid CoC record (unique codes, signatures, timestamps and transfer logs).

- Professional Ethics and Data Integrity

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Submit a complete analytical report that includes validated results, test methods and QC summaries.

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Laboratory Information Management System | As per requirement |
| 02 | ISO/IEC 17025 Standard | As per requirement |
| 03 | NEQS/WHO water quality guidelines | As per requirement |
| 04 | SOP Manuals | As per requirement |

### 5.9 0712EPT1109: Maintain Environmental Compliance in Water Testing

Overview:

After this Competency Standard, the trainee will be able to ensure that all analytical activities in the laboratory are carried out in compliance with established quality assurance (QA) and quality control (QC) procedures to guarantee accuracy, precision and reliability of results

| Competency Units | Performance Criteria |
| --- | --- |
| Identify environmental regulations | - Identify applicable environmental laws, acts and policies related to water quality - Report key parameters and permissible limits from required guidelines - Identify compliance parameters for drinking water, wastewater and industrial effluent - Perform classification of water samples based on regulatory scope |
| Verify compliance of analytical result | - Compare analytical results with required environmental standards - Check variations in test results against permissible limits - Calculate analytical uncertainty as per SOP - Record compliance calculation results |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Principles of Quality Assurance (QA) and Quality Control (QC)

- Calibration and Validation Fundamentals

- Role of SOPs in ensuring consistent calibration, testing and documentation.

- Non-Conformance and Corrective Action (CAPA) Procedures

- ISO/IEC 17025 and Laboratory Accreditation Principles

- Professional Ethics and Data Integrity

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate calibration and validation of analytical instruments using correct standards and SOPs.

- Submit completed and maintained QA documentation, including calibration logs, QC charts and non-conformance records.

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | ISO/IEC 17025 Standard | As per requirement |
| 02 | NEQS/WHO water quality guidelines | As per requirement |
| 03 | SOP Manuals | As per requirement |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | pH meter | 01 |
| 02 | EC meter | 01 |
| 03 | DO meter | 05 |
| 04 | Spectrophotometer | 01 |
| 05 | Turbidimeter | 01 |
| 06 | PPEs | 01 |
| 07 | pH meter | 01 |

### 5.10 0712EPT1110: On-Job Training/ Project

Overview

The On-Job Training (OJT) segment of this qualification enables learners to apply their theoretical knowledge and technical skills in real-world water testing and laboratory environments. During this phase, learners are placed in foundational or support roles such as Water Quality Assistant, Laboratory Technician, Water Sampling Technician, Wastewater Testing Technician, or Environmental Laboratory Support Staff. These positions are offered across diverse sectors including public health, environmental monitoring, municipal services, industrial quality control and water treatment facilities.

In case of unavailability of relevant industry for OJT, a project may be carried out for the fulfilment of OJT requirement.

- List of Consolidated Consumables, Tools, Machinery & Equipment

| 6A. List of Consolidated Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Sodium hypochlorite or bleach solution | 05 |
| 02 | Detergents and disinfectants | 05 |
| 03 | Sodium Chloride | 500 gm |
| 04 | Potassium Chloride | 500 gm |
| 05 | Sodium Nitrite | 500 gm |
| 06 | Potassium Nitrate | 500 gm |
| 07 | Ferrous ammonium sulfate | 500 gm |
| 08 | SPADNS Reagent for Fluoride | 500 mL |
| 09 | chloramine-T | 100 gm |
| 10 | Lead carbonate, PbCO3 | 250 gm |
| 11 | Sulfamic acid, NH2SO3H | 250 gm |
| 12 | Potassium Cyanide | 250 gm |
| 13 | Barbituric acid | 250 gm |
| 14 | Pyridine | 500 mL |
| 15 | sodium acetate trihydrate (NaC2H3O2 _ 3H2O) | 500 gm |
| 16 | Glacial Acetic Acid | 2.5 L |
| 17 | Barium Chloride | 500 gm |
| 18 | Hydrochloric Acid | 2.5 L |
| 19 | Sodium Hydroxide | 500 gm |
| 20 | Boric Acid | 500 gm |
| 21 | Sulfuric Acid | 2.5 L |
| 22 | Methyl Orange Indicator | 100 gm |
| 23 | methyl red Indicator | 100 gm |
| 24 | methylene blue Indicator | 100 gm |
| 25 | Ammonium Molybdate | 100 gm |
| 26 | Stannous Chloride | 100 gm |
| 27 | ammonium metavanadate | 100 gm |
| 28 | di-Potassium hydrogen phosphate (K2HPO4) | 250 gm |
| 29 | Sodium Sulfate | 500 gm |
| 30 | Potassium hydrogen phthalate | 500 gm |
| 31 | Calcium chloride | 500 gm |
| 32 | Magnesium sulfate | 500 gm |
| 33 | Ferric chloride | 500 gm |
| 34 | potassium dihydrogen phosphate KH2PO4 | 250 gm |
| 35 | sodium phosphate dibasic heptahydrate Na2HPO4_7H2O | 250 gm |
| 36 | Ammonium Chloride (NH4Cl) | 500 gm |
| 37 | Magnesium Sulfate MgSO4_7H2O | 500 gm |
| 38 | Nitrification Inhibitor | 100 gm |
| 39 | Glucose | 250 gm |
| 40 | Glutamic Acid | 250 gm |
| 41 | Potassium Dichromate | 500 gm |
| 42 | 1,5-diphenylcarbazide | 100 gm |
| 43 | Nitric Acid | 2.5 L |
| 44 | Potassium Sulfate (K2SO4) | 500 gm |
| 45 | Copper Sulfate (CuSO4) | 500 gm |
| 46 | Sodium Thiosulfate (Na2S2O3 5H2O) | 500 gm |
| 47 | Ethyl Alcohol | 3L |
| 48 | Lauryl Tryptose Sulfate Agar | 1 bottle |
| 49 | Brilliant Green Bile Agar | 1 bottle |
| 50 | Escherichia coli Broth | 1 bottle |
| 51 | Plate Count Agar | 1 bottle |
| 52 | Butterfield’s Phosphate Buffer | 1 bottle |
| 53 | Potato Dextrose Agar | 1 bottle |
| 54 | Tryptone Broth | 1 bottle |
| 55 | Kovac’s reagent | 1 bottle |
| 56 | Laboratory Information Management System | As per requirement |
| 57 | ISO/IEC 17025 Standard | As per requirement |
| 58 | NEQS/WHO water quality guidelines | As per requirement |
| 59 | SOP Manuals | As per requirement |

| 6B. Consolidated List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Electrical Conductivity Meter | 1 |
| 02 | TDS Meter | 1 |
| 03 | pH meter | 1 |
| 04 | Dissolved Oxygen Meter | 1 |
| 05 | Total and Free Chlorine kit | 1 |
| 06 | Thermometer | 1 |
| 07 | GPS | 1 |
| 08 | Lab coat | 25 |
| 09 | Disposable gloves | 25 |
| 10 | Face mask | 25 |
| 11 | Safety goggles | 25 |
| 12 | Safety shoes | 25 |
| 13 | Head cap | 25 |
| 14 | Collection trolleys or carts | 03 |
| 15 | Spill containment trays | 01 |
| 16 | Autoclave unit | 01 |
| 17 | Incinerator | 01 |
| 18 | Chemical disinfectant containers | 01 |
| 19 | Measuring cylinders | 03 |
| 20 | Shredder or crusher | 03 |
| 21 | Brushes, mops, and cleaning cloths | 03 |
| 22 | Wastewater disposal hoses | 02 |
| 23 | First aid kit | 01 |
| 24 | Absorbent pads | 02 |
| 25 | Drying oven 250ºC | 1 unit |
| 26 | Electronic analytical balance, capacity 220g | 1unit |
| 27 | Funnels 100 mm, | 5 |
| 28 | Cylinders (50ml, 100ml) | 4 |
| 29 | Separating funnel, capacity 250 ml | 5 |
| 30 | Tripod stand | 5 |
| 31 | Flame Photometer with accessories (LPG Cylinder, CRMs for Sodium & potassium) | 01 No |
| 32 | UV/VIS Spectrophotometer with variable sample holder and accessories | 01 Unit |
| 33 | Cyanide Distillation Assembly | 01 Unit |
| 34 | Heating Mantle 500 mL 350 C | 1 Unit |
| 35 | Muffle Furnace 1100 C | 1 Unit |
| 36 | COD Reactor 150 C, having 16 mm wells | 01 Unit |
| 37 | COD Vials 1500 mg/L | 25 Nos. |
| 38 | Arsenic Test Kit | 01 Kit |
| 39 | China Dish 100 cc | 10 Nos. |
| 40 | Whatmann Filter Papers 42 ash-less | 1 Pack |
| 41 | Kjeldhal Digestion and Distillation Apparatus 500mL | 01Set |
| 42 | Glass Beads | 500 gm |
| 43 | Glass Burette 50 mL0 | 02 Nos. |
| 44 | Laminar hood | 1 |
| 45 | Autoclave | 1 |
| 46 | Incubators | 3 |
| 47 | Colony counter | 1 |
| 48 | Filtration Assembly | 1 |
| 49 | Micropipettes set and accessories | 2 |
| 50 | Glass tubes | 100 |
| 51 | Petri plates disposable | 500 |
| 52 | Glass beakers | 100 |
| 53 | Erlenmeyer flasks | 50 |
| 54 | Filter papers (0.4 µ) | 1 box |
| 55 | Projector/Multimedia/White board | 1 |
| 56 | MS Office Suite | 1 |
| 57 | Turbidimeter | 01 |
| 58 | PPEs | 01 |

- Members of Qualifications Development & Review Committee

| Qualification Development Committee |  |  |  |
| --- | --- | --- | --- |
| Sr.# | Name | Designation | Organization |
|  | Dr. Asma Saeed | Chief Scientific Officer, Head of FBRC | PCSIR, Lahore |
|  | Dr. Asma Sheikh | Senior Scientific Officer, FBRC | PCSIR, Lahore |
|  | Dr. Muhammad Ziad | Manager Solid Waste Management, Water and Sanitation Services | Peshawar |
|  | Dr. Munawar Hussain | Assistant Director Technical, Quality Control Center | PSQCA, Lahore |
|  | Dr. Habib Ur Rehman | Research Officer | PCRWR Lahore |
|  | Dr. Kashif Ali | Assistant Professor, Department of Soil Science | University of Layyah |
|  | Dr. Adnan Zahid | Assistant Professor, Dept of Agriculture | Lahore |
|  | Mr. Faisal Shahzad | Instructor, PVTC | Lahore |
|  | Mr. Muhammad Karamat Shaheen | Chemist, CDA | Islamabad |
|  | Mr. Atta Ur Rahman | Health Safety and Environment Engineer | National Engineering Corporation |
|  | Mr. Nouman Zafar | Scientific Officer II, | Qarshi Research International, Lahore |
|  | Mr. Nadeem Ahmad | Junior Research Officer | Central Research Laboratory, PHED, Lahore |
|  | Miss Mahrukh Khan | Research Assistant | University of the Punjab |
|  | Mr. Yasir Akram Awan | Director | Skylark Engineering |
|  | Mr. Nasir Jadoon | Country Director | Skylark Engineering |
|  | Muhammad Abdul Moeez | DACUM Facilitator | RMCE |
|  | Muhammad Ishaq | Deputy Director | NAVTTC |