Curriculum

For

National Diploma Level 4 in

Water Treatment Technician

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Micro Hydro Power Plant Technician Level-4. This work was made possible with the technical support of the experts.

It may not be prudent to mention here that all the industry experts, and TVET experts of TEVTAs, BTEs, PVTC, and work diligently for making this qualification worthy and error-free for which all credit goes to them. However, NAVTTC accepts responsibility for all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	4

1.1.	Date of Development	4

1.2.	Date of Review and Validation	4

1.3.	Entry Requirement	4

1.4.	Minimum qualification for teachers	4

1.5.	Codes of qualification (ISCED)	4

1.6.	Medium of instruction	4

1.7.	Duration of Course	5

2.	Scheme of Studies	5

3.	Details of Modules	6

3.1 0712WTT07 Identify types of wastewater	6

3.2 0712WTT08 Monitor National & International Policy Rules & Regulation	10

3.3 0712WTT09- Perform waste water treatment	18

3.4 0712WTT10- Monitor Waste Water Treatment Techniques	24

3.5 0712WTT 11 – Monitor Stages of Waste Water Treatment Process	29

3.6 0712WTT 12 – Perform Operations & Maintenance	34

3.7 0712WTT 13 – Perform Customer Care Service 	38

3.8 0712 WTT 14 – Identify Risk Management in Water Treatment	41

3.9 0712 WTT 15 – Apply Data Management 	45

3.10 0712 WTT 16 – Perform Assets Management Services 	49

3.11 0712 WTT 17 – On Job Training	52

# Introduction

The National Vocational and Technical Training Commission (NAVTTC) acknowledge the vital role of vocational and technical education in equipping Pakistan's workforce with the skills and competencies necessary to thrive in emerging industries. To address the increasing demand for skilled professionals in dynamic and innovative sectors, NAVTTC has initiated the development of Competency Standards (CS) for the trade of Micro Hydro Power Plant technician in Pakistan. This initiative underscores NAVTTC’s commitment to workforce development, enhancing employability, and driving Pakistan’s digital transformation while aligning with global industry trends.

Micro Hydro Power Plant Technology is a rapidly growing sector with significant contributions to the global economy, spanning industries such as rural electrification, sustainable energy, agriculture, and industrial applications. This multidisciplinary field demands expertise in hydrodynamics, electrical engineering, turbine design, power generation, and system integration. With the accelerating adoption of renewable energy solutions, the micro hydro industry is witnessing a surge in demand for skilled professionals, creating substantial career opportunities both locally and internationally. By establishing these competency standards, NAVTTC ensures the development of a workforce prepared to meet industry requirements and contribute to Pakistan's economic advancement.

The qualification framework for Micro Hydro Power Plant Technician focuses on equipping learners with essential skills and knowledge, ranging from entrepreneurial skills, Advance welding, Fluid Mechanics & Hydraulics Machines, Quantity Survey/Estimation, Electrical Equipment, Electrical Fault in Generator, Transmission of Electrical Power, Distribution of Electrical Power, Protection of Distribution and Transmission System, Install Turbine, Install Generator, Control MHP plant shutdown for emergency standby electrical systems, Analyze faults in MHP plant operation, preventive maintenance procedures as part of MHPP operations.

| ## Date of Development | 17th – 21st Nov, 2025 – 17th October 2025 |
| --- | --- |
| ## Date of Review and Validation | 21st Nov, 2027 |
| ## Entry Requirement | National Vocational Qualification for Water treatment Assistant Level – 3 OR National Vocational Qualification for Water treatment Assistant Level – 3 OR Secondary School Certificate in Science with at least 6 Month of relevant experience certificate |
| ## Minimum qualification for teachers | MSc in Chemistry/Microbiology/Environmental Sciences/Bio-chemistry with at least 1 year experience in relevant field OR BSc in Chemistry/Microbiology/Environmental Sciences/Bio-chemistry with at least 2-year experience in relevant field OR DAE in Chemical Technology/Lab Technology with at least 4-year experience in relevant field |
| ## Codes of qualification (ISCED) | Code: 0712WTT00 Description: : National Vocational Certificate Level 4 in Water Sector (Water Treatment Technician) |
| ## Medium of instruction | English |
| ## Duration of Course | Total. 840 hours. Theory. 80 hours Practical. 520 hours On Job Training. 240 hours |

# Scheme of Studies

| Module code | Name of Competency |  | NVQF Level |  | Category |  | Estimated Contact Hours |  |  |  |  |  | Cr Hr |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Th |  | Pr |  | Total |  |  |  |
| 0712WTT07 | Identify types of wastewaters |  | 4 |  | Technical |  | 10 |  | 60 |  | 70 |  | 4 |  |
| 0712WTT08 | Monitor National & International Policy Rules & Regulations |  | 4 |  | Technical |  | 6 |  | 48 |  | 54 |  | 3 |  |
| 0712WTT09 | Perform Wastewater Treatment |  | 4 |  | Technical |  | 10 |  | 60 |  | 70 |  | 4 |  |
| 0712WTT010 | Monitor Wastewater Treatment Techniques |  | 4 |  | Technical |  | 10 |  | 60 |  | 70 |  | 4 |  |
| 0712WTT011 | Monitor Stages of Wastewater Treatment Process |  | 4 |  | Technical |  | 6 |  | 30 |  | 36 |  | 2 |  |
| 0712WTT012 | Perform Operations & Maintenance |  | 4 |  | Technical |  | 10 |  | 60 |  | 70 |  | 4 |  |
| 0712WTT013 | Perform Customer Care Services |  | 4 |  | Technical |  | 7 |  | 60 |  | 67 |  | 3.7 |  |
| 0712WTT014 | Identify Risk Management in Water Treatment |  | 4 |  | Technical |  | 8 |  | 54 |  | 62 |  | 3.5 |  |
| 0712WTT015 | Apply Data Management |  | 4 |  | Technical |  | 6 |  | 54 |  | 60 |  | 3.3 |  |
| 0712WTT016 | Perform Assets Management Services | 4 |  | Technical |  | 7 |  | 34 |  | 41 |  | 2.4 |  |
| 0712 WTT017 | On-Job-Training |  | 4 |  | Technical |  |  |  |  |  | 240 |  | 8 |  |
| Total |  |  |  |  |  |  | 80 |  | 520 |  | 840 |  | 40 |  |

# Daily Contact Hours

| Daily Contact Hours by Institute | Duration |
| --- | --- |
| 5 | 09 months |
| 4 | 12 months |
| 3 | 15 months |
| 2 | 18 months |
| Note: For Institute offering 6 days a week. The weekend institute may use formula: Credit hours / Daily hours = Months Duration |  |

# Details of Modules

## 3.1 0712WTT07 Identify types of wastewaters

Objective:

This module will provide the necessary skills and knowledge required for wastewater treatment involving removing contaminants from domestic, industrial, or storm water sources to protect public health and the environment. The main goal is to produce treated water that is safe for discharge or reuse and to minimize the impact on natural water bodies.

Duration: 70 Hours    	Theory: 10 Hours	          Practice: 60 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials (Tools & Equipment) Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Inspect of domestic wastewater | - Use PPEs before wastewater Sampling - Collect domestic wastewater Samples - Collect Location Wise Samples - Provide samples in lab - Collect results from the lab - Submit report to supervisor/manager | - Identify sources and types of domestic wastewater - Observe physical characteristics like color, odor, and turbidity - Measure basic parameters such as pH, temperature, and suspended solids - Recognize potential contaminants and their effects on health and environment - Record and report observations accurately Practical Activity Measure basic parameters like pH, temperature, and turbidity and record the results | Theory: 3Hr Practical: 16Hrs Total: 19Hrs | - Sample containers or bottles - pH meter or pH strips - Thermometer - Turbidity meter or Secchi tube - Gloves and safety goggles - Notebook and pen - Labels and markers | - Classroom - /workshop |
| LU2. Inspect industrial wastewater | - Use PPEs before wastewater Sampling - Collect domestic wastewater Samples - Collect Location Wise Samples - Ensure Sample Preservation - Ensure sample Shipment of in lab - Analyze Samples - Collect results from the lab - Submit report to supervisor/manager | - Identify types of industrial wastewater based on industry (textile, chemical, food processing, metal, etc.) - Observe physical characteristics such as color, odor, oil/grease presence, and suspended solids - Measure basic parameters like pH, temperature, turbidity, TDS, and COD/BOD - Recognize specific contaminants typical to the industry and their environmental impact - Record and report observations accurately Practical Activity Measure pH, temperature, turbidity, and industry-relevant chemical parameters, then record and compare results | Theory: 2Hr Practical: 14Hrs Total: 16Hrs | - containers or bottles - pH meter or pH strips - Thermometer - Turbidity meter - Chemical testing kits (COD, BOD, TDS, heavy metals) - Gloves, safety goggles, and apron - Notebook and pen - Labels and markers | - Classroom/workshop |
| LU3. Inspect grey water | - Use PPEs before wastewater Sampling - Collect domestic wastewater Samples - Collect Location Wise Samples - Provide samples in lab - Collect results from the lab - Submit report to supervisor/manager | - Identify sources and characteristics of greywater from households and commercial facilities - Observe physical properties such as color, odor, and turbidity - Measure basic parameters like pH, temperature, and suspended solids - Recognize common contaminants in greywater (soaps, detergents, food particles) - Record and report observations accurately Practical Activity Measure pH, temperature, and turbidity, then record results | Theory: 3Hr Practical: 16Hrs Total: 19Hrs | - Sample containers or bottles - pH meter or pH strips - Thermometer - Turbidity meter - Gloves and safety goggles - Notebook and pen - Labels and markers | - Classroom/workshop |
| LU4. Inspect produced water | - Collect information about the produced water from drilling site - Prepare contact list of relevant officials/ departments - Check first aid box - Report non availability of items in first aid box and lack of PPEs - Ensure PPEs, tools and equipment’s in the store - Update inventory list and report - Check all wiring, fire extinguishers etc. in place - Display SOPs on site/ in plant | - Identify sources and characteristics of produced water from industrial or oil and gas operations - Observe physical properties such as color, odor, and presence of oil, grease, or solids - Measure basic parameters like pH, temperature, salinity, and suspended solids - Recognize contaminants specific to produced water and their environmental impact - Record and report observations accurately Practical Activity Measure pH, temperature, salinity, and turbidity, then record the results | Theory: 2Hr Practical: 14Hrs Total 16Hrs | - Sample containers or bottles - pH meter or pH strips - Thermometer - Turbidity meter - Salinity or conductivity meter - Gloves, safety goggles, and apron - Notebook and pen - Labels and markers | - Classroom/workshop |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand types of wastewaters, domestic waste water (grey water/black water), industrial waste water, etc.

- City sewerage system

- Drainage system

- Storm water system

- Safety rules and regulations of organization

- Understanding the use of PPEs, first aid box etc.

- Causes of workplace accidents relevant to the work environment

- Understanding of emergency protocols. SOPs, departments relevant to emergencies

- Workplace response/SOPs to emergencies

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Prepare report on various types of waste water in the city

- Prepare presentation of Sewerage system of the city.

- Use fire extinguishers and personal protective equipment correctly.

- Submit a report containing identified risk and hazard at workplace and in the jurisdiction to concerned

- Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | First Aid Kit | 1 |
| 02 | Sample collecting Bottles | 75 |
| 03 |  |  |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Fire Extinguisher | 1 |
| 02 | Rubber shoes | 25 |
| 03 | Costume | 25 |
| 04 | Automated Samplers | 02 |

## 3.2 0712WTT08: National & International Policy Rules & Regulations

Objective:

The objective of this module is to apply the knowledge of national and provincial policies relevant to the sector, and regulatory authorities related to wastewater management and their mandate.

Duration: 54 Hours	 Theory: 6 Hours	Practice: 48 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials (Tools & Equipment) Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Monitor NEQs for wastewater parameters | - Identify and list on-Site Parameters for wastewater monitoring - Identify and list lab Parameters for wastewater monitoring - Prepare list of Permissible Limits for identified parameters - Develop reporting format for daily and monthly reporting | - Understand National Environmental Quality (NEQ) standards for wastewater parameters - Identify key wastewater parameters such as pH, BOD, COD, TSS, TDS, and heavy metals - Compare measured values with NEQ limits to assess compliance - Record and report findings accurately for regulatory purposes Practical Activities: - Collect wastewater samples and measure NEQ parameters like pH, BOD, COD, TSS, and TDS | Theory: 2Hr Practical: 13Hrs Total: 15Hrs | - Sample containers or bottles - pH meter or pH strips - Thermometer - Turbidity meter - Chemical testing kits (BOD, COD, TSS, TDS, heavy metals) - Gloves, safety goggles, and apron - Notebook and pen - Labels and markers | - Classroom/workshop |
| LU2. Monitory of regulatory & monitoring authority | - Develop presentation on the National Regulatory Monitoring Authority and their mandate - Develop presentation on the Provincial Regulatory Monitoring Authority and their mandate - Prepare the list of the applicable Policies for Compliance | - Identify key regulatory and monitoring authorities responsible for water and wastewater standards - Understand roles, responsibilities, and compliance requirements set by these authorities - Recognize reporting procedures and documentation needed for regulatory inspections Practical Activities: - Review regulatory guidelines and compare them with on-site monitoring practices | Theory: 2Hr Practical: 18Hrs Total: 20Hrs | - Regulatory guidelines or NEQ standards - Notebook and pen - Checklists or reporting templates - Laptop or printed documents | - Classroom/workshop |
| LU3. Monitory of zero discharge of hazardous chemicals policy | - Prepare the list of the zero discharge of hazardous chemicals Policies for Compliance - Develop SOPs for Compliance | - Understand the purpose and requirements of the Zero Discharge of Hazardous Chemicals (ZDHC) policy Identify hazardous chemicals commonly used in industries and their restricted limits - Monitor wastewater streams for compliance with ZDHC guidelines and reporting procedures Practical Activities: - Check industrial chemical inventories and compare them with ZDHC restricted substance lists | Theory: 2Hr Practical: 17Hrs Total: 19Hrs | - ZDHC guidelines and restricted substance lists - Sample containers or bottles - Chemical testing kits (COD, TDS, specific hazardous chemicals) - Gloves, safety goggles, and apron Notebook and pen | - Classroom/workshop |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Overview of the NEQS and its permissible limits

- Overview of Zero Discharge of Hazardous Chemicals Policy

- Understand National Regulatory Monitoring Authorities and their mandate

- Understand provincial Regulatory Monitoring Authority and their mandate

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Documentation of compliance reports, audits, and performance evaluations aligning with both national laws and international treaties.

Consumables, Tools, Machinery & Equipment

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Water and sanitation Policies | 02 |
| 02 | Water acts | 02 |

## 3.3 0712WTT09 Perform Wastewater Treatment

Objective:

This Module will provide skills and knowledge involving removing contaminants from domestic, industrial, or storm water sources to protect public health and the environment. The main goal is to produce treated water that is safe for discharge or reuse and to minimize the impact on natural water bodies.

Duration: 70 Hours		Theory: 10 Hours		Practice: 60 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials (Tools & Equipment) Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Perform aerobic wastewater treatment | - Identify the purpose of aerobic treatment. - Perform primary Treatment Sedimentation and Physical settling of solid contents - Assess Biological Treatment to maintain F/M in the reactor tank. - Maintain MLSS in specific range as per the designed capacity of the plant. - Maintain DO in the range of 1.5 to 2.5mg per liter. - Convey of Sludge from Primary clarifier as well as from secondary clarifier to the sludge thickener - Remove harmful bacteria, pathogen and viruses. | - Biological treatment principle - Aerobic vs. anaerobic - Role of microorganisms - Role of dissolved oxygen - Importance of MLSS - Maintenance of F/M ratio Practical Activity Measure COD and MLSS of the reactor basin and confirm F/M ratio | Theory: 1Hr Practical: 7Hrs Total: 8Hrs | - DO Meter - MLSS Meter - Aerator , diffuser and blower - Sludge Thickener | - Classroom-Lab |
| LU2. Perform anaerobic wastewater treatment | - Identify Anaerobic Process. - Identify the biodegradation Process during treatment. - Identify Acetogenesis. - Identify Bio gas generation - Control pH range. - Ensure oil and grease removal | - Anaerobic biology - End products - Energy recovery - Anaerobic vs. aerobic - Hydrolysis - Acetogenysis - Reactor Environment Maintenance Practical Activity Measure PH of the reactor tank and maintain it within the limits | Theory: 2Hr Practical: 10Hrs Total: 12Hrs | - PH Meter - Anaerobic Reactor - Flow meters (magnetic, ultrasonic) - Level sensors (ultrasonic, pressure) - Feed/ sludge pumps |  |
| LU3. Perform decentralized wastewater treatment | - Identify Waste Water Sources - Identify Waste Water Effluent - Select Suitable Waste Water treatment System - Understand Waste Water treatment system components - Assess Environmental Impact | - Becentralized system components - Definition of decentralized wastewater treatment - Types of wastewater generated - Basic principles of decentralized wastewater treatment - Advantages and Application - Reuse potentials Practical Activity Inspect the outlet drain pipe and maintain the water level of sedimentation tank at 0.3 m from the bottom | Theory: 1Hr Practical: 8Hrs Total: 9Hrs | - Screens - Settler - Primary treatment equipment’s - ABR - Anaerobic Filter - Constructed Wetland - Flow and Level controller - PH and DO meter |  |
| LU4. Perform wetland treatment | - Assess Influent volume - Identify Preliminary treatment - Examine Soil Characteristics - Assess Topography and Drainage - Confirm Wetland Indicators - Identify Wetland clogging - Maintain data log book | - Natural vs constructed wetlands - Types of constructed wetlands - Process Principles and mechanism - Components of a constructed wetland system - Design considerations - Selection of Vegetation - O&M of wetland - Appliocation Practical Activity Inspect Media of the wetland and take influnt and effluent samples. Measure TSS and DO | Theory: 2Hr Practical: 12Hrs Total: 14Hrs | - Ph meter - Do meter - TSS meter - Flow measuring devices - Excavators - HDPE/LDPRE Sheet Rolls - Graded gravels - Plants - Outlet/inlet devices |  |
| LU5. Perform bioremediation | - Identify Waste Type - Identify media/culture suitability - Identify Role of Suitable Microorganisms - Identify suitable plants - Monitor MoResults | - Understanding bioremediation - Microbial and biological fundamentals - Bioremediation techniques Practical Activity Inspect media gravel and measure the thickness of micro-flock film on its surface | Theory: 2Hr Practical: 10Hrs Total: 12Hrs | - Soil augers and sampling tools - Dissolved oxygen meters, pH meters, EC meters - Air blowers, compressors (for venting/sparing) - Tilling equipment, mixers - Bioreactor vessels, pumps, aeration systems - Nutrient dosing systems - Safety equipment (PPE, spill) kits, gas detectors) |  |
| LU6. Perform chemical treatment | - Assess the need of chemical treatment - Perform the Jar test - Assess the efficiency of coagulant and process - Assess the efficiency of flocculants and process - Monitor Results | - Introduction to chemical treatment - Types of chemical treatments - Chemical storage, handling and safety Activity: Take sample of raw water and determine its coagulant dose using Jar-test | Theory: 2Hr Practical: 13Hrs Total: 15Hrs | - Chemical handling storage tank - Dosing and injection equipment - Mixer - Flocculent - Monitoring and measuring tools |  |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand the importance of wastewater treatment in protecting public health and the environment.

- Know the sources of wastewater (domestic, industrial, storm water, and agricultural).

- Understand the objectives of treatment — removal of solids, organic matter, nutrients, and pathogens.

- Main stages of treatment.

- Understand the difference between aerobic and anaerobic treatment processes.

- Be aware of sludge handling and disposal methods.

- Understand factors affecting treatment efficiency, including flow rate, temperature, and pollutant load.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Demonstration of understanding the three main treatment stages (Primary, Secondary, and Tertiary).

- Ability to classify wastewater sources (domestic, industrial, storm water).

- Correct identification of treatment methods used in each stage

Consumables, Tools, Machinery & Equipment

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Alum (Aluminum Sulphate) | 20kg |
| 02 | Lime (Calcium Hydroxide) | 20kg |
| 03 | Chlorine (Liquid ) | 30 Litters |
| 04 | Seed sludge / activated sludge | 1 Bag |
| 05 | Anti-foaming agents | 1 Bag |
| 06 | Sand with vessels | 01 |
| 07 | Carbon with vessels | 01 |

## 3.4 0712WTT10 Monitor Wastewater Treatment Techniques

Objective:

This Module will provide skills and knowledge related to removing contaminants from domestic, industrial, and commercial water before it is released back into the environment. The main goal is to produce water that is safe for discharge or reuse, and to protect public health and the ecosystem. Proper identification and selection of wastewater treatment techniques ensure environmental sustainability, regulatory compliance, and resource recovery.

Duration: 70 Hours			Theory: 10 Hours				Practice: 60 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials (Tools & Equipment) Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Monitor activated sludge process | - Prepare for work activities - Identify and observe activated sludge process operation - Conduct basic process measurements - Interpret process performance - document and report the result - Ensure DO, FM ratio - Handle troubleshooting | - Monitor preliminary treatment process concept understanding - Process flow and operational steps - Safety protocols and compliance knowledge - Monitoring and troubleshooting techniques - Quality control and reporting requirements Practical activity: - Find out DO and TSS of the aeration chamber | Theory-3 Hrs Practical-16 Hrs Total-19 Hrs | - PPE (gloves, goggles, safety shoes) - Flow meters - Sampling bottles - Control panel access - Operational log sheets | - Classroom - Industry |
| LU2. Monitor sequence batch reactor techniques | - Observe SBR components and layout - Record SBR operational cycle - Measure operating parameters - document and report the result | - Understand SBR Process Fundamentals - Identify Components of an SBR System - Monitor SBR Operating Parameters - Monitor Biological Activity - Operate and Monitor SBR Automation / Control Systems - Inspect and Maintain SBR Equipment - Recordkeeping and Documentation Practical Activity: - Record duration, DO levels, and equipment running in each phase. | Theory-2 Hrs Practical-14 Hrs Total- 16 Hrs | - Pencil - Eraser - Sharpener - Measuring Tape (100ft) - Calculator - Ruler | - Class Room - workshop |
| LU3. Monitor moving bed bio-film reactor techniques | - Identify components and layout of MBBR system - Measure key operational parameters - Inspect biofilm media reactor condition - Document and report the result | - Understand Fundamentals of the MBBR Process - Identify Components of MBBR System - Monitor Operational Parameters of the MBBR Process - Inspect Biofilm Growth on Carriers - Monitor MBBR Performance through Sampling and Testing Practical activity: - Observe media movement in the reactor (should be free-floating and well mixed). | Theory-3 Hrs Practical-16Hrs Total- 19 Hrs | - Pencil - Eraser - Sharpener - Measuring Tape (100ft) - Calculator - Ruler | - Class Room - workshop |
| LU4. Monitor Oxidation Pond Technique | - Assess the need Analyses of oxidation technique - Assess aspect ratio of pond - Identify aerobic condition - Identify Anaerobic condition - Assess polishing of final pond - Document and report the results | - Understand Oxidation Pond Fundamentals - Identify Components and Layout of an Oxidation Pond - Monitor Key Operational Parameters - Monitor Algal and Microbial Activity - Inspect Structural and Environmental Conditions - Assess polishing of final pond - Monitor Sludge Accumulation and Pond Depth - Collect and Analyze Water Samples Practical Activity: - Confirm the depth of oxidation pond according to given specifications required - Collect sample and compare DOs of influent and effluent | Theory: 2Hr Practical: 14Hrs Total: 16Hrs |  |  |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Importance of wastewater treatment.

- Able to know the sources of waste water (Industrial, domestic and portable water).

- Able to understand the environmental impact of waste water, pollution on health and environment.

- Able to know the different stages of treatment process.

- Able to understand key role of different parameters (BOD, COD, DO, SS, pH).

- Interpretation and monitoring of data and troubleshooting.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Understands the sequence and function of each wastewater treatment stage.

- Can recognize and explain the components and operational principles of different waste water treatment techniques.

- Is capable of performing basic inspections, recording operational, analysis data, and reporting findings accurately.

Consumables, Tools, Machinery & Equipment

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | pH meter (Bench Type + Portable) | 02 |
| 02 | EC meter (Bench Type + Portable) | 02 |
| 03 | Turbidity meter (Bench Type) | 01 |
| 04 | Flow meter | 02 |
| 05 | DO meter (Portable) | 01 |
| 06 | TDS Meter (Bench Type + Portable) | 02 |
| 07 | COD Reactor (Bench Type) | 01 |
| 08 | BOD Apparatus (Bench Type) | 01 |
| 09 | Double Beam Spectrophotometer (Bench Type) | 01 |
| 10 | Flame photometer | 01 |
| 11 | Vacuum Filtration Assembly Apparatus | 01 |
| 12 | Auto titrate | 05 |
| 13 | Beakers Different Sizes (50, 100, 500 and 1000ml) | 06 Each |
| 14 | Conical Flask 250ml, Graduated Cylinder 50ml, Funnel Measuring Flask 250,500, 1000ml | 06 Each |

## 3.5 0712WTT11 – Monitor Stages of Wastewater Treatment Process

Objective:

This module is designed to develop the skills and knowledge needed to monitor the wastewater treatment process, a systematic approach to removing contaminants and making water safe for discharge or reuse. Treatment occurs in multiple stages preliminary, primary, secondary, and tertiary each targeting specific pollutants to ensure effective purification.

Duration: 36 Hours        	Theory: 6 Hours 	Practice:  30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials (Tools & Equipment) Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Monitor preliminary treatment process | - Operate screening - Operate pumping station - Operate grit removal units - Operate equalization - Observe flow control systems | - Identify preliminary treatment components. - Monitor operating conditions (flow, headloss, grit load). - Record process data and report faults. - Apply safety procedures during monitoring. Practical activity Measure and record flow rate and screen differential levels. | Theory: 1Hr Practical: 4Hrs Total: 05Hrs | - PPE (boots, gloves, hard hat, goggles) - Flow meter / reading device - Depth probe - Flashlight - Logbook or digital recorder | - Classroom - Workshop |

| LU2. Monitor primary treatment process | - Perform sedimentation process - Control process parameters - Transport sludge to thickener - Observe smooth operation of plant. | - Identify primary treatment components (primary clarifiers, sludge scrapers, scum removal systems). - Monitor clarifier performance (settling efficiency, sludge blanket depth, flow distribution). - Record operational data and report deviations. - Apply safety and operational procedures. Practical activity Measure and record sludge blanket depth, flow rate, and scum levels. | Theory: 1Hrs. Practical: 5Hrs. Total: 6Hrs. | - PPE (boots, gloves, hard hat, goggles) - Sludge blanket depth probe - Flow meter - Clarifier inspection tools (rod, scoop, viewing glass) - Logbook or digital recorder | - Classroom - Workshop |
| --- | --- | --- | --- | --- | --- |
| LU3. Monitor secondary treatment process | - Operate secondary treatment units - Monitor reactor tank parameters - Monitor level gauge of aeration process - Observe flow meter and take readings - Collect intermediate samples | - Identify secondary treatment units (aeration tanks, blowers, secondary clarifiers). - Monitor biological process conditions (DO levels, MLSS, aeration rate). - Record process data and report abnormal conditions. - Apply safety and process control procedures. Practical activity Inspect aeration tanks, diffusers, blowers, and secondary clarifiers. | Theory: 1Hrs. Practical: 7Hrs. Total: 8 Hrs. | - PPE - DO meter - MLSS sampling kit - Flow meter - Sludge judge / depth probe - Logbook | - Classroom - Workshop |
| LU4. Monitor tertiary treatment process | - Operate tertiary treatment systems - Monitor different parameters - Control different process parameters - Document and record results. | - Identify tertiary units (filters, disinfection systems, polishing steps). - Monitor filter performance, turbidity, and disinfectant dose. - Record water quality parameters. - Apply safety and operational guidelines. Practical activity Measure turbidity, residual disinfectant, and filter headloss. | Theory: 1Hrs. Practical: 6Hrs. Total:7 Hrs | - PPE - Turbidity meter - Residual chlorine tester / UV intensity meter - Flow meter - Filter inspection tools - Logbook | - Classroom - Workshop |
| LU5. Handle sludge treatment process | - SEnsure safe handling of sludge - Reserve effluent water for beneficial uses - Interpret data - Convey the compost to treatment area - Spread the compost on granular media - Ensure proper percolation for drying - Recycle the percolated water to pumping station | - Identify sludge handling units (thickener, digester, dewatering, storage). - Monitor sludge volume, solids content, and digestion process. - Record sludge treatment data and report abnormalities. - Apply safety and hygiene procedures. Practical activity Inspect sludge thickener, digester, and dewatering equipment.. | Theory: 2Hr Practical: 8Hrs Total: 10Hrs | - PPE (boots, gloves, goggles, apron) - Sludge sampling tools - TS/VS testing kit - Flow meter - Dewatering equipment monitoring tools - Logbook | - Classroom - Workshop |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Able to understand the objective of waste water treatment.

- Able to understand detailed operational processes and techniques of waste water treatment.

- Able to understand different water and waste water treatment analysis and its standards.

- Understand the usage of recycle water and compost.

- Know the importance of solid waste treatment and its impact on public health and environment.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- The candidate must demonstrate evidence of the following competencies through practical performance, observation, and documentation

Consumables, Tools, Machinery & Equipment

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | pH meter (Bench Type + Portable) | 02 |
| 02 | EC meter (Bench Type + Portable) | 02 |
| 03 | Turbidity meter (Bench Type) | 01 |
| 04 | Flow meter | 02 |
| 05 | DO meter (Portable) | 01 |
| 06 | TDS Meter (Bench Type + Portable) | 02 |
| 07 | COD Reactor (Bench Type) | 01 |
| 08 | BOD Apparatus (Bench Type) | 01 |
| 09 | Double Beam Spectrophotometer (Bench Type) | 01 |
| 10 | Flame photometer | 01 |
| 11 | Vacuum Filtration Assembly Apparatus | 01 |
| 12 | Auto titrate | 05 |
| 13 | Beakers Different Sizes (50, 100, 500 and 1000ml) | 06 Each |
| 14 | Conical Flask 250ml, Graduated Cylinder 50ml, Funnel Measuring Flask 250,500, 1000ml | 06 Each |

## 3.6 0712WTT12 – Perform Operations & Maintenance

Objective:

This module is designed to develop the skills and knowledge needed to monitor, operate, and maintain the wastewater treatment process, ensuring safe and efficient removal of contaminants. Treatment occurs in multiple stages preliminary, primary, secondary, and tertiary each targeting specific pollutants to achieve water quality standards.

Duration: 70		                    Theory: 10 Hours		Practice: 60 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop operation & maintenance plan | - Set a Maintenance Plan / Checklists - Apply the Lifecycle Concept to Plant Equipment - Assign Roles and Responsibilities of O&M Teams - Develop Lubrication and Inspection Plans - Create and Maintain Equipment Inventory Lists - Apply PPE Requirements for Operators and Maintenance | - Identify critical equipment and treatment units for maintenance. - Plan routine inspections, cleaning, lubrication, and calibration. - Schedule preventive maintenance and spare parts management. - Ensure safety, compliance, and proper documentation. Practical activity - Document and track maintenance activities, repairs, and equipment performance. | Theory-2 Hrs Practical-14 Hrs Total-16 Hrs | - O&M logbook / software - Maintenance tools (wrenches, lubricants, cleaning equipment) - Spare parts inventory - PPE | Classroom / Industry |
| LU2. Operate electro-mechanical equipment’s | - Describe Electro-Mechanical Systems (Valves, Motors, Panels) - Explain Basic Electricity Concepts: Power, Voltage, and Current - Identify Different Types of Pumps - Perform Preventive Maintenance Procedures - Apply Safety Procedures During Operation and Emergencies | - Identify automation components: sensors, PLCs, HMI, SCADA, and cloud platforms. - Monitor real-time process parameters remotely. - Analyze alarms, trends, and historical data for operational decision-making. - Apply cybersecurity and safety protocols. Practical activity Configure SCADA displays and dashboards for monitoring treatment units. | Theory: 3 Hrs. Practical: 16 Hrs. Total: 19 Hrs. | - PPE - Computer/Tablet with SCADA software - Sensors & PLCs - Cloud platform access (Internet connection) - Communication tools (LAN/Wi-Fi/Router) - Logbook for backup and manual records | - Classroom - Workshop |
| LU3. Develop operation & maintenance plan | - Apply the Lifecycle Concept to Plant Equipment - Define Roles and Responsibilities of O&M Teams - Prepare Maintenance Schedules - Develop Equipment Maintenance Checklists - Implement Lubrication and Inspection Plans - Create and Maintain Equipment Inventory Lists - Follow PPE Requirements for Operators and Maintenance Staff | - Identify pumps, blowers, mixers, valves, and motors. - Start, stop, and adjust equipment according to process requirements. - Monitor equipment performance (vibration, noise, temperature, load). - Apply safety procedures and lockout/tagout before maintenance. Practical activity Operate pumps, blowers, and mixers as per SOP. | Theory: 2 Hrs Practical: 14 Hrs Total: 16 Hrs | - PPE, - Wrenches and basic mechanical tools, Motor starters, - Vibration/temperature monitoring tools, - Logbook | - Classroom - Workshop |
| LU4. Use operational tools & equipment/ machinery | - Identification and Safe Handling of Tools - Operation and Inspection of Pumps and Valves - Develop skill in using key measuring instruments to monitor system performance - Lubrication and Preventive Maintenance Task - Develop SOPs for operations and Maintenance of plant | - Identify different types of tools, equipment, and machinery used in a specific operational setting. - Demonstrate proper handling and operation techniques for tools and machinery. - Follow safety procedures while using operational tools and equipment. - Perform basic maintenance and troubleshooting of tools and machinery. Practical activity Operate a selected piece of machinery or tool under supervision following safety guidelines. | Theory: 3 Hrs. Practical: 16 Hrs. Total: 19 Hrs. | - Batteries - PPE - Maintenance tools - Safety manuals | - Classroom - Workshop |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand the purpose and importance of operations and maintenance in plant efficiency and reliability

- Know the types of maintenance: preventive, corrective, and predictive

- Understand basic functions of plant equipment (pumps, valves, motors, control panels)

- Know standard operating procedures for equipment operation

- Understand maintenance schedules and how to prepare and follow them

- Know how to use maintenance checklists and logs effectively

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Complete scheduled maintenance tasks following standard procedures and safety protocols

- Correctly apply lockout/tag out (LOTO) and use personal protective equipment (PPE) during maintenance

- Accurately document maintenance activities, including faults and actions taken

Consumables, Tools, Machinery & Equipment

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | pH meter (Bench Type + Portable) | 02 |
| 02 | EC meter (Bench Type + Portable) | 02 |
| 03 | Turbidity meter (Bench Type) | 01 |
| 04 | Flow meter | 02 |
| 05 | DO meter (Portable) | 01 |

## 3.7 0712WTT13 Perform Customer Care Services

Objective:

This module covers the skills and knowledge required to deliver efficient customer care services in water operations. It includes responding to inquiries, addressing complaints, and ensuring clear communication to maintain customer satisfaction

Duration: 67 Hours                                    	Theory: 7 Hours           	                	Practice: 60 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Perform Community Awareness | - Identify community needs and issues - Develop and implement awareness plans - Deliver clear and appropriate information regarding health safety. - Collect and analyze community feedback | - Understanding community health and sanitation needs - Awareness of local environmental hazards - Methods of educating community on health safety - Techniques for garbage handling and hygiene promotion - Collecting and interpreting community feedback - Communication and presentation skills Practical Activity Conduct a small awareness session in a local community or school | Theory: 2Hrs. Practical: 18Hrs. Total: 20Hrs. | - Community awareness pamphlets and posters - Waste collection tools (gloves, bins, bags) - Sample health safety kits - Notepad and feedback forms - Projector or presentation tools | Classroom Community center Local schools or health posts |
| LU2. Interpret ICE material | - Identify key information and specifications from ICE material documents - Analyze technical data and drawings accurately - Apply interpretation of ICE material - Report findings and discrepancies to relevant personnel | - Understanding ICE documentation structure - Identifying key specifications - Reading technical drawings - Analyzing data for compliance - Reporting techniques and documentation standards - Problem-solving based on data interpretation Practical activity Interpret a sample ICE document and identify key technical details | Theory: 2Hrs. Practical: 20Hrs. Total: 22Hrs. | - ICE material documents - Technical drawing sheets - Notebooks and pens - Computer with MS Office for reporting - Highlighters and markers | Classroom Workshop Industry/Field office |
| LU3. Perform Complaint Management | - Document complaints promptly and accurately - Analyze complaints to identify root causes and categorize issues - Communicate effectively with complainants to provide updates and resolutions - Implement corrective actions and follow up to ensure complaint resolution | - Understanding types of complaints in water services - Techniques for complaint documentation - Root cause analysis methods - Categorization and prioritization of issues - Effective communication with complainants - Implementing and monitoring corrective actions Practical activity Test the operation of a control relay | Theory: 3Hrs. Practical: 22Hrs. Total: 25Hrs | - Complaint forms or software - Notebooks and pens - Computer for documentation - Case study examples - Communication tools (phone/email)s | Classroom Workshop Customer service center |

Knowledge & Understanding

The candidate must demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand basic customer care principles and importance

- Know common customer needs and behaviors

- Understand effective communication and listening skills

- Know company products, services, and policies

- Understand complaint handling and escalation procedures

- Know confidentiality and data protection requirements

- Understand use of customer service tools and systems

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

- Demonstrate effective communication and problem-solving with customers

- Accurately record and manage customer inquiries and complaints

- Apply company policies and procedures to resolve customer issues satisfactorily

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Community Survey Tools (e.g., Survey Monkey, Google Forms) | 1 Subscription |
| 02 | Social Media Management Tools (e.g., Hoot suite, Buffer) | 1 Subscription |
| 03 | Presentation Software (e.g., Microsoft PowerPoint, Google Slides) | 1 License |
| 04 | Feedback Collection Tools (e.g., Feedback kiosks, online forms) | 1 Set |
| 05 | Reporting and Data Analysis Software (e.g., Excel, Google Sheets) | 1 License |

## 3.8 0712WTT14 – Identify Risk Management in Water Treatment

Objective:

This module provides the skills and knowledge required to identify risks within water treatment processes. It includes recognizing potential hazards, assessing their impact, and supporting preventive measures to ensure safe and reliable water quality.

Duration: 62 Hours			Theory:8 Hours				Practice: 54 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials (Tools & Equipment) Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify risks | - Assess the treatment plant site - Identify hazards and its potential risks - Document the risks | - Understanding common risks in water treatment processes (biological, chemical, mechanical) - Identifying unsafe operational practices that can compromise water quality - Recognizing equipment-related hazards such as leaks, contamination, or malfunction - Assessing environmental and human health impacts caused by improper treatment Practical Activity Perform a risk-spotting walkthrough of a water treatment setup (real or simulated) and list all identified hazards | Theory: 2Hr Practical: 16Hrs Total: 18Hrs | - Risk assessment checklist - Safety PPE (gloves, goggles, mask) - Basic reference guide on water treatment risks - Flashlight for equipment inspection | - Well-equipped classroom - Workshop |
| LU2. Assess risks | - Evaluate risks into low medium and high risks - Document the high risks | - Evaluate the severity and likelihood of identified hazards - Prioritiz risks based on their potential impact on water quality and system performance - Appropriate control measures to reduce or eliminate risks Practical Activity Conduct a basic risk-assessment exercise by rating the severity and likelihood of hazards identified in a treatment process scenario | Theory: 3Hr Practical: 18Hrs Total: 21Hrs | - Risk assessment forms or templates - Hazard severity/likelihood rating chart - PPE for practical assessment (gloves, goggles, mask) | - Well-equipped classroom - Workshop |
| LU3. Manage risks | - Provided measures to control risks - Provide measures to transfer the risks - Document and report risks to the supervisor | - Implement control measures to minimize or eliminate identified risks - Monitor risk-control effectiveness during daily - Adjust operational procedures when new risks or system changes arise - Maintain proper documentation of risks, actions taken, and system safety status Practical Activity Perform coupling of hydro turbine with the generator as per given specification. | Theory: 3Hr Practical: 20Hrs Total: 23Hrs | - Standard operating procedures (SOPs) for treatment processes - PPE (gloves, mask, goggles) - Monitoring checklist for control measures | - Well-equipped classroom - Workshop |

Knowledge & Understanding

The candidate must be able to demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes knowledge of:

- Learn basic knowledge on Risks

- Gain Knowledge on Hazards and incidents

- Document risk and incident

- Report on risk to Supervisor

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Site Risk Assessment Report

- Hazard Identification Checklist

- Risk Evaluation Matrix (Low / Medium / High)

- Risk Control and Mitigation Plan

- Emergency Response Plan

Consumables, Tools, Machinery & Equipment

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Safety Helmet | 01 Per Person |
| 02 | Safety Gloves | 01 Per Person |
| 03 | Safety Goggles | 01 Per Person |

## 3.9 0712WTT15 – Apply Data Management

Objective:

This module covers the skills and knowledge required for effective data management in water and wastewater treatment. It involves collecting, organizing, and analyzing operational data such as flow rates, water quality, and equipment status. Proper data management supports decision making, reporting, and maintenance planning, ensuring efficient plant operations

Duration: 60 Hours			Theory: 8 Hours				Practice: 54 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required |  | Learning Place |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Develop tools for data collection | - Collect field data on daily basis - Maintain the register - Report data on weekly and monthly basis | - Understand the purpose of data-collection tools. - Identify the type of data needed (quantitative/qualitative) - Develop simple tools such as checklists, forms, or questionnaires. - Test and refine the tools for clarity and accuracy. Practical Activity Design a basic data-collection form for asset details. | Theory: 2Hr Practical: 23Hrs Total: 25Hrs |  | - Sample forms/checklists - Pen/notebook | - Classroom - Workshop |
| LU2. Use of GPS for special data collection | - Study and collect data on handheld GPS - Maintain data register | - Understand what GPS is and how it works. - Identify locations where spatial data is needed. - Collect coordinates using a GPS device or mobile app. - Record and store GPS data accurately for mapping or analysis. Practical Activity Use a GPS device/app to capture coordinates of selected points. | Practical: 4Hrs Total: Theory: 31Hr 35Hrs |  | - GPS device or mobile phone with GPS app - Notebook/pen | - Classroom - Workshop |

Knowledge & Understanding

The candidate must be able to demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes knowledge of:

- Basics of Handheld GPS

- Collection and recording of data on GPS

- Reporting of data to supervisor

- Basics of MS Word and MS Excel

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Daily Field Data Collection Sheet

- Weekly and Monthly Data Reports

- GPS-Based Spatial Data Record

- Data Register (Manual or Digital)

- Equipment Status and Flow Rate Logs

- Water Quality Test Record Sheet

- Digital Database Entry Record (Excel / System Export)

- Data Accuracy Verification Checklist

- Supervisor’s Data Submission Report

- Mapping or GIS Output from GPS Data

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Field Data Collection Form / Sheet | As Required |
| 02 | Register / Logbook | 01 Per Operator |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Computer / Laptop with MS Excel & Word | 01 Per Office |
| 02 | Handheld GPS Device | 01 Per Person |

## 3.10 0712WTT16 Perform Assets Management Services

Objective:

This module covers the skills required for asset management in water and wastewater treatment, focusing on tracking, maintaining, and optimizing physical assets. Proper asset management ensures efficient system performance, reduces downtime, and extends equipment life.

Duration: 41 Hours                                   	Theory: 7 Hours    	                                	Practice: 34 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop assets inventory | - Develop inventory formats in excel - Count all assets and document its quality - Data entry in excel - Maintain and update the inventory list | - Define what an asset inventory is. - Identify different types of assets. - Collect basic asset information. - Record and maintain an asset list. Practical activity Fill an asset inventory sheet with collected data. | Theory: 2Hr. Practical: 13 Hrs. Total: 15Hrs. | - Inventory sheet - Pen/notebook | - Classroom - Workshop |
| LU2. Develop Assets tagging | - Tag all assets with a unique number provided by the supervisor - Monitor all assets on quarterly basis and update the inventory - Report discrepancies to the supervisor | - Understand the purpose of asset tagging. - Identify assets that require tags. - Apply tagging methods (barcode, QR code, number labels). - Record tag numbers in the inventory. Practical activity Tag selected assets using sample labels.. | Theory: 5 Hr. Practical: 21Hrs. Total: 26 Hrs. | - Asset tags/labels - Marker or label printer - Inventory sheet | - Classroom - Workshop |

Knowledge & Understanding

The candidate must be able to demonstrate the underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes knowledge of:

- Development of inventory tool

- Knowledge on MS Word and Excel

- Basic knowledge of treatment Plants Assets

Critical Evidence(s) Required

The candidate needs to produce the following critical evidence(s) in order to be competent in this competency standard:

- Asset Inventory Format (Developed in Excel)

- Complete Asset Count Report

- Asset Condition Assessment Sheet

- Updated Asset Register / Database Record

- Asset Tagging Log (with Unique ID Numbers)

- Barcode or Label Tag Record Sheet

- Quarterly Asset Monitoring Report

- Asset Discrepancy / Variance Report

- Maintenance History Record Sheet

- Supervisor Verification / Approval Form

Consumables, Tools, Machinery & Equipment

| List of Consumables |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Barcode Labels or Metal Tags | As Required |
| 02 | Permanent Marker / Label Pen | 01 Per Operator |
| 03 | Asset Inventory Sheet / Register | 01 Per Operator |

| List of Tools, Machinery & Equipment |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
| 01 | Computer / Laptop with MS Excel & Word | 01 Per Operator |
| 02 | Asset Tagging Machine / Label Printer | 01 Unit |

## 3.11 0712WTT17 On-Job-Training

Objective:

The OJT component lets learners apply their technical skills in real water testing and laboratory environments. They gain experience in roles such as Water Quality Assistant, Laboratory Technician, and Sampling or Testing Assistant across various sectors. If industry placements are unavailable, an alternative project may be completed to fulfill OJT requirements.

Duration: 46 Hours			Theory: 4 Hours				Practice: 42 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Your OP Here | - Your Performance Criteria Here | - Learning Elements Here Practical activity Any Activity . | Theory: 1Hr. Practical: 9Hrs. Total: 10Hrs. | - List Here | - List Here |

# List of Consumables, Tools, Machinery & Equipment

| 4A. List of Consolidated Consumables |  |  |  |
| --- | --- | --- | --- |
| Sr. # | Consumable |  | Quantity |
| 01 |  |  | 25 No’s |
| 02 | Lab Notebooks / Logbooks |  | 75 Pieces |
| 03 | Ballpoint Pens |  | 50 Pieces |
| 04 | Highlighters (Mixed Colors) |  | 50 Pieces |
| 05 | Markers (Permanent & Whiteboard) |  | 10 No’s |
| 06 | A4 Paper Rims |  | 5 Rims |
| 07 | A3 Paper |  | 200 Sheets |
| 08 | Graph Papers |  | 25 Sets |
| 09 | Printed SOPs & Manuals |  | 80 No’s |
| 10 | Water Sample Bottles (Plastic) |  | 20 No’s |
| 11 | Water Sample Bottles (Glass) |  | 25 No’s |
| 12 | Grab Sampling Containers |  | 200 Pieces |
| 13 | Sample Bottle Labels |  | 20 Pieces |
| 14 | Waterproof Markers |  | 20 Liters |
| 15 | Distilled / Deionized Water |  | 5 Bottles |
| 16 | pH Buffer Solution (4.0) |  | 5 Bottles |
| 17 | pH Buffer Solution (7.0) |  | 5 Bottles |
| 18 | pH Buffer Solution (10.0) |  | 10 Kits |
| 19 | Chlorine Test Reagents (DPD) |  | 10 Kits |
| 20 | Turbidity Test Reagents |  | 10 Kits |
| 21 | Hardness Test Reagents |  | 10 Kits |
| 22 | Alkalinity Test Reagents |  | 10 Kits |
| 23 | Acidity Test Reagents |  | 5 Kits |
| 24 | Dissolved Oxygen Reagents |  | 5 Sets |
| 25 | COD Digestion Chemicals |  | 5 Sets |
| 26 | BOD Nutrient Buffer Pills |  | 25 Kg |
| 27 | Alum (Coagulant) |  | 20 Kg |
| 28 | Lime Powder |  | 10 Kg |
| 29 | Ferric Chloride |  | 5 Kg |
| 30 | Chlorine Tablets |  | 5 Kg |
| 31 | Chlorine Powder |  | 20 Liters |
| 32 | Sodium Hypochlorite |  | 5 Kg |
| 33 | Polymer (Flocculant) |  | 25 Kg |
| 34 | Activated Carbon (Granular) |  | 10 Kg |
| 35 | Activated Carbon (Powdered) |  | 10 Liters |
| 36 | Antiscalant Chemical |  | 50 Kg |
| 37 | Sand Filter Media |  | 50 Kg |
| 38 | Gravel Filter Media |  | 20 Pieces |
| 39 | Cartridge Filter Elements |  | 10 Pieces |
| 40 | RO Pre-filter Cartridges |  | 50 Pieces |
| 41 | Gaskets (Rubber) |  | 100 Pieces |
| 42 | O-Rings (Assorted Sizes) |  | 20 Rolls |
| 43 | Teflon Tape |  | 10 Bottles |
| 44 | Thread Sealant |  | 10 Tins |
| 45 | PVC Solvent Cement |  | 20 No’s |
| 46 | Cleaning Brushes |  | 5 Bottles |
| 47 | Lab Glassware Cleaning Liquid |  | 5 Liters |
| 48 | Isopropyl Alcohol |  | 5 Bottles |
| 49 | Detergent / Degreaser |  | 100 Pieces |
| 50 | Cotton Cloth / Rags |  | 20 Rolls |
| 51 | Tissue Paper Rolls |  | 10 Bottles |
| 52 | Hand Wash Liquid |  | 10 Bottles |
| 53 | Hand Sanitizer |  | 50 Pairs |
| 54 | Latex / Nitrile Gloves |  | 100 Pieces |
| 55 | Disposable Face Masks |  | 25 No’s |
| 56 | Safety Goggles (Basic) |  | 25 No’s |
| 57 | Aprons (Disposable) |  | 100 Pieces |
| 58 | Calibration Stickers |  | 100 Pieces |
| 59 | Warning Labels |  | 100 Pieces |
| 60 | Hazard Identification Tags |  | 25 Books |
| 61 | Log Sheets (Daily Operation) |  | 25 Sets |
| 62 | Maintenance Record Sheets |  | 25 Sets |
| 63 | Water Quality Report Formats |  | 25 Sets |
| 64 | Jar Test Recording Sheets |  | 50 Pieces |
| 65 | Pens for Lab Use |  | 50 Pieces |
| 66 | Pencils |  | 25 Pieces |
| 67 | Erasers |  | 25 Pieces |
| 68 | Rulers |  | 25 No’s |
| 69 | File Folders |  | 25 No’s |
| 70 | Document Covers |  | 25 No’s |
| 71 | Clip Boards |  | 50 Pads |
| 72 | Sticky Notes |  | 20 Rolls |
| 73 | Adhesive Tape |  | 10 Rolls |
| 74 | Electrical Insulation Tape |  | 100 Pieces |
| 75 | Indicator Labels |  | 5 Sets |
| 76 | Sample Preservation Chemicals |  | 2 Kg |
| 77 | Sodium Thiosulfate (Dechlorination) |  | 50 Packs |
| 78 | Silica Gel Packs |  | 5 Liters |
| 79 | RO Membrane Cleaning Chemical (Acid) |  | 5 Liters |
| 80 | RO Membrane Cleaning Chemical (Alkali) |  | 5 Liters |
| 81 | Anti-foaming Agent |  | 5 Liters |
| 82 | Lubricating Oil (Light) |  | 5 Kg |
| 83 | Grease (General Purpose) |  | As Required |
| 84 | PVC Pipe Cutting Waste (Practice) |  | 50 Pieces |
| 85 | Hose Clamps |  | 200 Pieces |
| 86 | Cable Ties |  | 20 Pieces |
| 87 | Sample Transport Ice Packs |  | 5 Sets |
| 88 | Neutralization Chemicals |  | 100 Pieces |
| 89 | Glassware Labels |  | 10 Pieces |
| 90 | Measuring Tape (Disposable Type) |  | 20 Pieces |
| 91 | Marker Chalk |  | 10 Bottles |
| 92 | Soap Solution |  | 10 Bottles |
| 93 | Disinfectant Solution |  | 5 No’s |
| 94 | Emergency Eye Wash Bottles |  | 25 No’s |
| 95 | Stickers for Fault Area Marking |  | 50 Pcs |
| 96 | Printouts of Fault Diagnosis Flowcharts |  | 25 Sets |
| 97 | Maintenance Checklist Sheets |  | 100 Sheets |

| 4B. Consolidated List of Tools, Machinery & Equipment |  |  |  |
| --- | --- | --- | --- |
| Sr. # | Tools and Equipment |  | Quantity |
| 01 | pH Meter (Digital) |  | 5 Units |
| 02 | Turbidity Meter |  | 3 Units |
| 03 | TDS / Conductivity Meter |  | 5 Units |
| 04 | Dissolved Oxygen Meter |  | 2 Units |
| 05 | Portable Water Testing Kit |  | 5 Kits |
| 06 | Jar Test Apparatus |  | 2 Units |
| 07 | Digital Thermometer |  | 10 Units |
| 08 | Laboratory Balance (Digital) |  | 2 Units |
| 09 | Magnetic Stirrer |  | 2 Units |
| 10 | Hot Plate |  | 2 Units |
| 11 | BOD Incubator |  | 1 Unit |
| 12 | COD Digestion Apparatus |  | 1 Unit |
| 13 | UV–Visible Spectrophotometer |  | 1 Unit |
| 14 | Distillation Unit (Lab Scale) |  | 1 Unit |
| 15 | Autoclave (Laboratory) |  | 1 Unit |
| 16 | Laboratory Work Benches |  | 10 Units |
| 17 | Laboratory Stools |  | 25 Units |
| 18 | Fume Hood / Exhaust Cabinet |  | 1 Unit |
| 19 | Water Sampling Pole |  | 5 Units |
| 20 | Ice Box / Sample Transport Cooler |  | 5 Units |
| 21 | Filtration Unit (Vacuum / Gravity) |  | 2 Units |
| 22 | Vacuum Pump |  | 1 Unit |
| 23 | RO Training Plant (Skid Mounted) |  | 1 Unit |
| 24 | Multimedia Sand Filter (Demo) |  | 1 Unit |
| 25 | Activated Carbon Filter (Demo) |  | 1 Unit |
| 26 | Cartridge Filter Housing |  | 5 Units |
| 27 | Chemical Dosing Pump |  | 3 Units |
| 28 | Chlorination Unit (Demo) |  | 1 Unit |
| 29 | UV Disinfection Unit |  | 1 Unit |
| 30 | Ozonation Unit (Training Model) |  | 1 Unit |
| 31 | Storage Tank (Plastic / SS) |  | 2 Units |
| 32 | Transfer Pump |  | 3 Units |
| 33 | Centrifugal Pump (Cut-Section) |  | 1 Unit |
| 34 | Pressure Gauges |  | 5 Units |
| 35 | Flow Meters (Rotameter / Digital) |  | 3 Units |
| 36 | Level Sensors |  | 5 Units |
| 37 | Control Valves (Demo Set) |  | 5 Units |
| 38 | Gate Valves |  | 5 Units |
| 39 | Ball Valves |  | 5 Units |
| 40 | Non-Return Valves (NRV) |  | 5 Units |
| 41 | PVC Pipe Training Racks |  | 5 Sets |
| 42 | Pipe Threading Tool |  | 2 Units |
| 43 | Pipe Cutting Machine |  | 1 Unit |
| 44 | Hand Pipe Cutter |  | 5 Units |
| 45 | Adjustable Wrenches |  | 10 Units |
| 46 | Spanner Set (Full) |  | 5 Sets |
| 47 | Screwdriver Set |  | 5 Sets |
| 48 | Pliers Set |  | 5 Sets |
| 49 | Measuring Tape (Steel) |  | 10 Units |
| 50 | Vernier Caliper |  | 3 Units |
| 51 | Spirit Level |  | 5 Units |
| 52 | Drill Machine |  | 2 Units |
| 53 | Bench Vice |  | 5 Units |
| 54 | Grinder Machine |  | 1 Unit |
| 55 | Electrical Multimeter |  | 10 Units |
| 56 | Insulation Tester (Megger) |  | 1 Unit |
| 57 | Clamp Meter |  | 3 Units |
| 58 | Control Panel Training Board |  | 1 Unit |
| 59 | PLC Training Kit (Basic) |  | 1 Unit |
| 60 | SCADA Demonstration System |  | 1 Unit |
| 61 | Electrical Wiring Trainer |  | 2 Units |
| 62 | Indicator Lamps & Switches Panel |  | 2 Units |
| 63 | Emergency Stop Switch |  | 5 Units |
| 64 | Earthing Rods |  | 5 Units |
| 65 | Safety Helmet |  | 25 Units |
| 66 | Safety Goggles |  | 25 Units |
| 67 | Face Shields |  | 10 Units |
| 68 | Safety Shoes |  | 25 Pairs |
| 69 | Chemical Resistant Aprons |  | 25 Units |
| 70 | Fire Extinguisher (CO₂ / DCP) |  | 2 Units |
| 71 | Emergency Eye Wash Station |  | 2 Units |
| 72 | First Aid Box |  | 2 Units |
| 73 | Spill Control Kit |  | 2 Units |
| 74 | Portable Gas Detector |  | 1 Unit |
| 75 | Noise Level Meter |  | 1 Unit |
| 76 | Lux Meter |  | 1 Unit |
| 77 | Weather Monitoring Kit |  | 1 Unit |
| 78 | GPS Device |  | 2 Units |
| 79 | Laptop / Desktop Computer |  | 5 Units |
| 80 | Printer / Scanner |  | 1 Unit |
| 81 | Projector |  | 1 Unit |
| 82 | Whiteboard |  | 2 Units |
| 83 | CCTV Camera (Training) |  | 2 Units |
| 84 | PA System / Speaker |  | 1 Unit |
| 85 | Stop Watch |  | 10 Units |
| 86 | Wall Clock |  | 5 Units |
| 87 | Storage Racks |  | 10 Units |
| 88 | Chemical Storage Cabinet |  | 2 Units |
| 89 | Tool Cabinet |  | 2 Units |
| 90 | File Cabinets |  | 2 Units |
| 91 | Sample Storage Refrigerator |  | 1 Unit |
| 92 | Water Heater / Geyser |  | 1 Unit |
| 93 | Generator (Backup) |  | 1 Unit |
| 94 | UPS System |  | 2 Units |
| 95 | Voltage Stabilizer |  | 2 Units |
| 96 | Training Models (Cut-Section Pipes & Valves) |  | 5 Sets |
| 97 | Flow Diagram Display Boards |  | 5 Units |
| 98 | Safety Sign Boards |  | 10 Units |
| 99 | Calibration Weight Set |  | 1 Set |
| 100 | Glassware Drying Rack |  | 2 Units |
| 101 | Sample Mixing Tank |  | 1 Unit |
| 102 | Sludge Dewatering Model |  | 1 Unit |
| 103 | Sludge Drying Bed (Demo) |  | 1 Unit |
| 104 | Effluent Treatment Plant Model |  | 1 Unit |
| 105 | Drinking Water Treatment Plant Model |  | 1 Unit |
| 106 | Wastewater Treatment Plant Model |  | 1 Unit |
| 107 | Training Display Charts & Posters |  | 10 Sets |
| 108 | Portable Crane / Chain Block |  | 2 Units |
| 109 | Inspection Flashlight and Mirror Set |  | 3 Sets |

# Members of Qualification Development Committee

The Qualifications Development Committee consisted of the following members:

| Sr. # | Name | Designation | Organization |
| --- | --- | --- | --- |
|  | Mr. Muhammad Musa | Assistant Director | CDA Islamabad |
|  | Mr. Dr. Aftab Ahmad | Assistant Professor | Kohsar University Murree |
|  | Mr. Zohaib ud Din | GM-Design & Operation | WR-Pakistan |
|  | Muhammad Usman | Instructor | PVTC |
|  | Shakeel Badshah | RO | PCRWR |
|  | Mr M.Yaqoob Gulzar | Instructor | PMDC Rawalpindi |
|  | Mr. Muhammad Ali | Water Management Officer | Bahria Town |
|  | Usama Tariq | Site Engineer | Soil Water Multan |
|  | Muhammad Asif | Sr.Manager | RCH |
|  | Mr. Kashif Mehmood | AD WASA | WASA |
|  | Mr. Eng. M.Wamiq Ayaz | DACUM Facilitator | Rawalpindi |
|  | Engr. Masood Ahmad | Asst. Director | PCRWR, Islamabad |