| DACUM Panel - Muhammad Touqir Ali - Maarib Ali Jadoon - Dr. Habib Ur Rehman Marwat - Muhammad Momin - Syed Jawad - Muhammad Shoaib DACUM Facilitator: Muhammad Nasir Khan | Occupational Profile Chart Of Gemstone Faceting & Polishing 6MonthsCourse (Gemstone Cutter/Lapidary Assistant) |
| --- | --- |
|  | Date 8th July, 2024 |
|  | NAVTTC HQ, Islamabad, Pakistan |
|  | National Vocational & Technical Training Commission |

“Course of Gemstone Faceting and Polishing”

| DUTIES and TASKS |  |  |  |
| --- | --- | --- | --- |
| - Comply Health and Safety Guidelines |  |  |  |
| A1. Identify Hazards at workplace | A2. Apply Personal Protective and safety equipment (PPE) | A3. Develop basic communication skills | A4. Mine First Aid |
| - Perform round cut: |  |  |  |
| B1. Prepare pre-form of stone | B2. Put main facet Angle (42) indexing 64-32 , 16 48, 8-40, 24-56 | B3. Star facet Angle 27, indexing 4-12-20-28-36-44-52-60, | B4. Put Girdle facet angle 44 to 51, (adjustable), indexing 8-2+-2, 16+-2, 24+-2, 32+-2, 40+-2, 48+-2, 56+-2, 64+-2, |
| B5. Angle 0, for table indexing 64 | B6. Marke with aluminum 64 index. | B7. Re-dop pavilion side | B8. Adjust main facet opposite of crown main facet |
| B9. Follow Angle 43, index 64-32 , 16-48, 8-40, 24-56 | B10. Girdle facet angle 44 to 51, (adjustable), indexing( 8 2+-2, 16+-2, 24+-2, 32+-2, 40+-2, 48+-2, 56+-2, 64+-2) |  |  |
| - Perform Step cut/Emerald cut: |  |  |  |
| C1. Cut girdle first angle 90 indexing 64-32 , 16-48 as Put main facet | C2. Put Angle 42 indexing 64-32 , 16-48, 8-40,24-56 | C3. Star facet angle 27 Indexing 64-32 , 16-48, 840, 24-56 | C4. Put Angle 0 Indexing (64-32 , 16-48, 8-40, 24-56) Four facet cut angle 90 indexing 8-40,24-56 |
| C5. Re-dop pavilion side | C6. Adjust main facet opposite of crown main facet | C7. Put Angle 63, index 64-32 ,16-48, 8-40, 24-56 | C8. Put Angle 53, index 64-32 ,16-48, 8-40, 24-56 |
| C9. Put Angle 43, index 64-32 ,16-48 |  |  |  |
| - Perform Polishing brilliant oval cut |  |  |  |
| D1. Arrange polishing material as per requirement | D2. Shape the stone as per requirement | D3. Put Angle 42 Indexing 64-32 | D4. Put Angle 35 and Indexing 16-48 |
| D5. Adjust Angle 41. Indexing (6-26, 38-58 ) | D6. Star Facet Angle 27 and Indexing 64-+3, 32-+3 (adjustable)Girdle facet | D7. Adjust Angle 45-52 and Indexing 64 -+2,-+4,32+2, -+4, | D8. Put Angle 41-42(adjustable) indexing 16-+2, 48-+2 |
| D9. Put Angle 45 and Indexing 8,24,40,56 | D10. Follow Angle 0, for table indexing 64 Marking with aluminum. | D11. Re-dop pavilion side | D12. Adjust main facet opposite of crown main |
| D13. Follow Angle 43 and Indexing 64-32 | D14. Put Angle37 and Indexing 16-48 | D15. Put Angle 42 and Indexing 6,26,38,58 | D16. Girdle facet and 4552(adjustable) Indexing 64-+2, -+4 32-+2, -+4, |
| D17. Adjust Angle 41-42 Indexing 16-+2, 48-+2 | D18. Adjust Angle 45-46 Indexing8,24,40,56 |  |  |
| - Perform Triangular Mixed Cut: |  |  |  |
| E1. GIRDLE:- Can be cut first or last using 1ndex 1-16-31-33 48-63-65-80-95, at 90°. Girdle w1ll be of 1rregular width. | E2. ORDER OF CUTTING – Crown | E3. GRIND Three facets at 42 indexíng 96-32-64. | E4. GRIND Three facets at 43 Indexing 16-48-80. |
| E5. GRIND Three facet at 32 indexing 96-32-64. | E6. GRIND Three facets at 24 indexing 16-48-80. | E7. GRIND Six facet8 at 25 indexlng 2-30 -34-62-66-94. | E8. GRIND Six facetg at 44 Indexing 15-17-47-49-79-81. |
| E9. GRIND Six facet 50 Indexing l-31-33-63-65-95. ORDER OF POLISHING using same angles and index as per given Above | E10. ORDER OF CUTTING - Pavilion | E11. GRIND Three facets at 57 indexing 96-32-64. | E12. GRIND Three facets at 61 Indexing 96-32- 64. |
| E13. GRIND Three facets at 55 Indexing 96-32-64. | E14. GRIND Three facets at 49Indexing 96-32-64. | E15. GRIND Three facets at 30 indexing 96-32-64 | E16. GRIND Three facets at 85indexing 16-48-80. |
| E17. ORDER OF POLISHING See polishing order on left, using same angles and index was used in cutting. |  |  |  |
| - Brilliant Cut Briolette: |  |  |  |
| F1. GRIND Eight facets at 80 indexing 64-8- 16, etc | F2. GRIND Sixteen facets at 85° indexing 2- 6-10, etc. | F3. GRIND Eight facets at 75 indexing 64-8- 16, etc. | F4. GRIND Eight facets at 69° indexing 4-12- 20, etc. |
| F5. GRIND Eight facets at 63 indexing 4-12- 20, etc. | F6. GRIND Eight facets at 58 indexing 64-8- 16, etc. | F7. GRIND Eight facets at 55 indexing 64-8- 16, etc. | F8. ORDER OF POLISHING Polish in the reverse order, first the 7 th GRIND, then the 6th, 5 th, 4th, 3rd, lst and 2nd. |
| F9. OPTIONAL: Girdle facets may be ground first, or last, at 90° using Index 2-6-10, etc, | F10. Polishing should be completed at this setting. Top point can be ground flat in 45° Angle Dop to pemit drilling, but extreme care 18 needed ORDER OF CUTTING - BOTTOM | F11. GRIND Eight facets at 65" indexing 64-8- 16, etc. | F12. GRIND Sixteen facets at 72° indexing 2- 6-10, etc. |
| F13. GRIND Eight facets at 57° indexing 64-8- 16, etc | F14. GRIND Eight facets at 47° indexing 4-12- 20, etc. | F15. GRIND Eight facets at 35 indexing 4-12- 20, etc. | F16. GRIND Eight facets at 25 indexing 64-8- 16, etc |
| F17. ORDER OF P0LISHING Polish in the reverse order, first the 6th GRIND, then the 5th, 4th, 3rd, First and last |  |  |  |
| - Reverse Cut: |  |  |  |
| G1. ORDER OF CUTTING - Crown GRIND Four facets at 42° indexing 64-16-32-48, | G2. GRIND Four facets at 34 indexing 63-1-31-33, | G3. GRIND Four facets at 36° indexing 49-15-17-47, | G4. ORDER OF POLISHING Al rays polish towards the glrdle, see pol ish ing order on left, u3- ing sNe an gles and index &g as used in cuttinge ORDER OF CUTTING-Pavili |
| G5. GRIND To facets at 43° indexing 64-32, | G6. GRIND Two facets at 39° indexing 16-48, | G7. GRIND Four facets at 3° indexing 62-2-30-34. | G8. GRIND Four facets at 32 indexing 50-14- 18 -46. |
| G9. GRIND To facets at 23 indexing 64-32, | G10. ORDER OF POLISH ING See order of polishing on left, using same angles and index as Was used in cutting. |  |  |
| - Brilliant Cross Cut (Edith B.Strout)-Index(96) |  |  |  |
| H1. CUT GIRDLE AND SHAPE STONE BEFORE STARTING ANGLE FACETS. | H2. GRIND EIGHT EQUAL SIZED CUTS AT 90 DEGS. INDEX-95-1-23-25-47-49-71-73 | H3. CROWN CUTTING ORDER: lst, FOUR FACETS AT 52 DEGS. INDEX--96-24-48-72 | H4. FOUR FACETS AT 38 DEGS. INDEX--96-24-48-72 |
| H5. FOUR FACETS AT 23 DEGS. INDEX--96-24-48-72 | H6. ElGHT FACETS AT 36 DEGS. INDEX--8-16-32-40-56-64-80-88 | H7. EIGHT FACETS AT 60 DEGS. (plus or minus) INDEX--95-1-23-25-47-49-71-73 | H8. PAVILION CUTTING ORDER: lst. FOUR FACETS AT 60 DEGS. INDEX--96-24-48-72 |
| H9. Put FOUR FACETS AT 55 DEGS. INDEX--96-24-48-72 | H10. Put FOUR FACETS AT 50 DEGS. INDEX--96-24-48-72 | H11. Put FOUR FACETS AT 44% DEGS. (plus-minus) INDEX--96-24-48-72 | H12. Put EIGHT FACETS AT 4l DEGS. INDEX--94-2-22-26-46-50-70-74 |
| H13. Put ElGHT FACETS AT 59 DEGS. (plus-minus) INDEX--93-3-2 1-27 -45-51-69-75 | H14. Put ElGHT FACETS AT 65% DEGS. (plus-mings) INDEX--95-1-23-25-47-49-71-73 |  |  |
| - Fairy Wand by(Edith Strout) |  |  |  |
| I1. CUT AND POLISH TAHCE EN 45 DEGS. ANGLE DOP OVER 60% OF THE WIODTH OF THE STONE. CUT GIRDLE AND SHAPE STONE ROUND BEFORE STARTING ANGLE FACETS, CUT AT 90 DEGS. | I2. Put EIGHT FACETS AT 42 DEGS. INDEX--64-8-16-24-32, etc. | I3. Put EIGHT FACETS AT 35 DEGS. INDEX--64--8–16-24-32, etc. | I4. Put SIXTEEN FACETS AT 28 DEGS. INDEXING 2-6-10-14-18, . |
| I5. Put EIGHT FACETS AT 26 DEGS. INDEX--4-12-20-28-36, etc. | I6. Put SIXTEEN FACETS AT 50 DEGS. INDEX--2-6-10-14-18, etc. | I7. Put EIGHT FACETS AT 53 DEGS. INDEX--4-12-20-28-36, etc. | I8. PAVILION CUTTING ORDER: lst. EIGHT FACETS AT S1 DEGS. INDEX--64-8--16-24-32, etc. |
| I9. Put ElGHT FACETS AT 46 DEGS. INDEX-64-8-16-24-32, etc. | I10. Put SIXTEEN FACETS AT 41 DEGS. INDEX--2-6-10-14-18, etc. | I11. Put SIXTEEN FACETS AT S5 DEGS. INDEX--2-6-10-14- 18, etc. | I12. Put EIGHT FACETS AT 56 DEGS. INDEX--4-12-20-28-36, etc. |
| I13. POLISH ORDER USE SAME ANGLES-ADJ UST TO FIT. |  |  |  |
| - High Twelve by(Gorald I.Homrich) |  |  |  |
| J1. CROWN CUTTING ORDER: lst. TWELVE FACETS AT 55 DEGS. INDEX--4-12–20–28-36, etc. | J2. Put TWELVE FACETS AT 45 DEGS. INDEX--96-8-16-24-32, etc. | J3. Put TWELVE FACETS AT 35 DEGS. INDEX-4-12-20-28-36, etc. | J4. Put TELVE FACETS AT 25 DEGS. INDEX--96-8-16-24-32, etc. |
| J5. POLISH ORDER | J6. PAVILION CỨTTING ORDER: lst. TWELVE FA CETS AT 75 DEGs. INDEX4-12-20-28-36, etc. | J7. Put TWELVE FACETS AT 70 DEGS. INDEX-96-8-16-24-32, etc. | J8. Put TWELVE FACETS AT 65 DEGS. INDEX--4-12-20-28-36, et |
| J9. Put TWELVE FACETS AT 60 DEGS. INDEX--96-8–16-24-3 2, etc. | J10. Put TWELVE FACETS AT 55 DEGS. INDEX--4-12-20-28-36, etc. | J11. Put TWELVE FACETS AT 50 DEGS. INDEX--96-8–16-24-32, etc. | J12. Put TWELVE FACETS AT 45 DEGS. INDEX--4-12-20-28-36, etc. |
| J13. Put TWELVE FACETS AT 41 DEGS. INDEX--96-8-16-24-32, etc. | J14. POLISH ORDER. USE SAME ANGLES |  |  |
| - Moonbeam Cut by(Clom V.Svoboda) |  |  |  |
| K1. CROWN CUTTING ANGLES: lst. EIGHT MAIN FACETS AT 35 DEGS. INDEX-64-8-16-24-32, etc | K2. Put ElGHT LOWER MAIN FACETS AT 40 DEGS. INDEX-64-8-16-24-32, etc. | K3. Bring the se up so they cut off or less from top mains. Put ElGHT BREAK FACETS AT 24 DEGS. (plus) INDEX-4-12-20-28-36, etc. | K4. Put SIXTEEN TOP GIRDLE FACETS AT 42 2/3 DEGS. INDEX--2-6-10–14-18, etc. 2% |
| K5. Put SIXTEEN LOWER GIRDLE FACETS AT 46 1/3 DEGS. INDEX--3-5-l1-13-19-2 1, etc. | K6. POLISHING ORDER USE SAME ANGLES-ADJUST TO FIT. | K7. PAVILION CUTTING ORDER: lst. SIXTEEN MAIN FACETS AT 44 DEGS. INDEX--64–4-8-12-16, etc. | K8. 2nd. SIXTEEN LOWER GIRDLE FACETS AT 54 DEGS. INDEX--2-6-10-14-18, etc. |
| K9. Put Sixteen upper GIRDLE FACETS AT 72 DEGS. INDEX-2-6-10-14-18, etc | K10. POLISHING ORDER USE SAME ANGLES- |  |  |
| - Pinwheel by Ed Hume. |  |  |  |
| L1. CROWN CUTTING ORDER: Ist. ElGHT FACETS AT 49 DEGSA INDEX--96-12-24-36, etc | L2. Put ElGHT FACETS ÁT 39 DEGS. INDEX--94-10--22-34, etc. | L3. 3rd. EIGHT FACETS AT 30 DEGS. % INDEX--92-8-20-32, etc. | L4. Put ElGHT FACETS AT 22 DEĞs. INDEX--90--6-18-30, etc. |
| L5. POLISH ORDER USE SAME ANGLES- | L6. PAVILION CUTTING ORDER: lst. ElGHT FACETS AT 53 DEGS. INDEX--96-12-24-36, etc. | L7. Put ElGHT FACETS AT 46 DEGS. INDEX--94-10-22-34, etc. | L8. Put EIGHT FA CETS AT 40 DEGS, JNDEX-92-8-20-32, etc. |
| L9. Put ElGHT FACETS AT 34 DEGS. INDEX--90-6-18-30, etc. | L10. Put ElGHT FACETS AT 28 DEGS. INDEX-88--4-16-28, etc. |  |  |
| - Octabril by(R.Thompson) |  |  |  |
| M1. SHAPE STONE AND CUT GIRDLE AT 90 DEGS. USING THE FOLLOWING INDEX--96-12-24– 36-48-60–72-84. CUT AND POLISH TABLE IN 45 DEG. ANG LE DOP. | M2. CROWN CUTTING ORDER: lst. EIGHT FACETS AT 49 DEGS. INDEX--96-12-24-36, etc. | M3. Put EIGHT FACETS AT 42 DEGS. INDEX--96–12-24-36, etc. | M4. Put EIGHT FACETS AT 30 DEGS. INDEX--96-12-24-36-etc. |
| M5. Put EIGHT FACETS AT 42 DEGS. INDEX--6-18-30-42, etc. | M6. POLISHING ORDER- USE SAME ANGLES | M7. PAVILION CUTTING ORDER: lst. ElGHT FACETS AT 48 DEGS. INDEX---96-12-24-36, etc. | M8. Put SIXTEEN FACETS AT 43 DEGS. INDEX-4-8-16-20-28-32, etc. |
| M9. Put SIXTEEN FACETS AT 43 DEGS. INDEX-4-8-16-20-28-32, etc. | M10. POLISHING ORDER USE SAME ANGLES |  |  |
| - Pasha With Twist by (Jack Hamm) |  |  |  |
| N1. CUT GIRDLE AND SHAPE STONE BE FORE CUTTING ANGLE FACETS. CUT AT 90 DEGS. FOLLOW GIRDLE INDEX--4-12-20, etc. CUT AND POLISH TABLE IN 45 DEG. ANGLE DOP MAKING IT ABOUT 75% THE DIAMETER OF THE STONE. | N2. CROWN CUTTING ORDER: lst. ElGHT FACETS AT 43 DEGS. INDEX--4-12-20-28-36, etc | N3. Put EIGHT FACETS AT35 DEGS, Adif Sle INDEX--8-16-24-32-40, etc. | N4. Put SlXTEEN FACETS AT 67 DEGS. INDEX-63-1-7-9-15-17, etc |
| N5. POLISHING ORDER USE SAME ANGLES | N6. ADJUST TO FIT. DO NOT POLISH GIRDLE FACETS | N7. PAVILION CUTTING ORDER: lst. ElGHT FACETS AT 42 DEGS. INDEX--64–8-16-24-32, etc. | N8. Put EIGHT FACETS AT 44 DEGS. INDEX--9-17-25-33-4l, etc. |
| N9. Put Eight FACETS AT 53 DEGS. INDEX-1l-19-27-35-43, etc. | N10. POLISH ORDER USE SAME ANGLES |  |  |
| - Adopt Soft Skills: Annex Attached |  |  |  |

Additional Information for Assistant Chemical Technician:

| Tools and Equipment | Entry Requirements | Duration of Training Required | Career Paths |
| --- | --- | --- | --- |
| Doping unit Dipsticks (round, V-Shaped and plate) Sprit Lamp and sprit Doping wax Master lap Faceting Unit Facetor (J-P-C) L-keys | Matric/Middle | The proposed duration 6 months | Gemstone Cutter/Lapidary Assistant |