## Course / Subject Name : NQ-Mech-127–Workshop Practice-I (A). Perform General Metal Work

Assessment Tasks (Summative Assessment)

Task-1: Prepare a name plate using MS Flat Strip

OR

Task-2: Cut external and internal Threads with the help of tap and die

OR

Task-3: Prepare a Dovetail joint using MS Flat Strip

Performance criteria:

Task-1: Prepare a name plate using MS Flat Strip

- Arrange marking tools as per job requirement

- Clamp sheet in vice.

- Cut sheet as per drawing

- Perform surface finishing with file

- Level surface with tri-square

- Mark plate as per name requirements

- Punch marked area

- Perform finishing with sand paper

Task-2: Cut external and internal Threads with the help of tap and die

- Collect tap according to hole in MS plate

- Collect Die according to MS rod

- Identify work piece clamping method

- Align tap and die

- Apply lubricants while threading

- Take safety measure to avoid unwanted engraving and slips

- Cut internal threads with Tap.

- Cut external threads with Die

Task-3: Prepare a Dovetail joint using MS Flat Strip

- Arrange marking tools as per job requirement

- Cut sheet as per drawing

- Perform surface finishing with file

- Level surface of both work pieces with tri-square

- Mark both work pieces according to drawing

- Create outer notch on work piece using flat file and hacksaw

- Create inner notch using hacksaw and chisel

- Assemble both the pieces by inserting outer notch into inner notch

- Perform finishing with sand paper.