Course / Subject Name: NQ-Mech-127–Workshop Practice-I. (B) Perform Wood Work and Pattern Making

Assessment Tasks (Summative Assessment)

Perform planing and squaring to precise dimensions, sharpen a wood chisel, and create a dado joint or a middle half-lap cross joint.

Performance Criteria:

- Measure and mark the wood accurately to specified dimensions.

- Use a hand plane or machine to achieve a smooth and flat surface.

- Ensure edges and faces are squared to precise angles.

- Check dimensions frequently using a square, ruler, or caliper.

- Select the correct sharpening stone (coarse and fine grit).

- Maintain the appropriate bevel angle while sharpening.

- Ensure the chisel edge is sharp, smooth, and free of burrs.

- Test the sharpness by cutting wood fibers cleanly.

- Measure and mark the dado dimensions on the wood.

- cut the groove to the specified depth and width with a saw or router.

- Clean the edges and bottom of the groove with a chisel for accuracy.

- Test the fit of the adjoining piece and ensure a snug, precise joint.

- Mark the cross-lap location and dimensions on both pieces of wood accurately.

- Cut half the thickness of each piece at the marked section with a saw or chisel.

- Ensure the mating surfaces are flat and even for a tight fit.

- Assemble and check the joint for alignment and strength.