Course / Subject Name: Mech-127. Workshop Practice-I (C). Perform Welding and Forging

- Assessment Tasks (Summative Assessment)

Task-01: Perform a "V" groove butt joint in the flat position using oxy-acetylene, create a bead on an MS plate,

OR

Task-02: Practice spot welding on a 05 mm MS sheet.

Performance Criteria:

Task-01

- Set up the oxy-acetylene equipment correctly, ensuring proper gas pressures and flame adjustment.

- Prepare the MS plates by cleaning and beveling edges to form the "V" groove.

- Align and securely clamp the plates in the flat position.

- Execute the weld with proper torch angle, speed, and filler material application to ensure complete fusion.

- Inspect the joint for uniformity, penetration, and absence of defects such as porosity or cracks.

Task-02

- Prepare the MS plate and sheet by cleaning and positioning appropriately.

- Set the welding machine parameters (current, voltage, and time) according to the material thickness.

- Perform a straight bead on the MS plate, ensuring consistent arc length, speed, and weld width.

- Conduct spot welding on the MS sheet with accurate electrode placement and pressure for proper joint formation.

- Inspect the bead for uniformity and the spot welds for strength and correct indentation.