Course / Subject Name:- NQ-Mech-127-Workshop Practice-I (D) Perform Basic Machining-I

Assessment Tasks (Summative Assessment)

- Perform machining operations (facing, turning, step turning, taper turning, and threading) as per attached drawing.

Note: All the dimensions are in mm. While the value for tolerance is as ±0.2mm.

Performance criteria (Part A)

- Interpret drawing

- Follow safety protocols

- Perform facing operation to achieve the length of 65 mm.

- Turn the job to a diameter 23 mm up to a length of 22 mm.

- Turn the first step turning diameter 11 mm upto 4 mm length.

- Perform taper turning to reduce diameter from 16 mm to 12 mm for a length of 21 mm.

- Turn a second step to achieve 8 mm diameter for a length of 4 mm.

- Cut M10 external V-thread having pitch 1.5 mm for a 14mm length.

- Remove the part from the machine.

- Perform necessary post machining operations like filing, or deburring.

- Turnoff the machine.

- Perform shaper operation as per drawing to shape a workpiece with a 6mm slot and 2 holes of M10

Note: All the dimensions are in mm. While the value for tolerance is as ±0.2mm.

Performance criteria (Part B)

- Set up the shaper machine mounting appropriate tool.

- Mount the workpiece in shaper machine vice.

- Shape the job size as 60mm x 48mm x 12mm.

- Align the tool at 21mm from the top edge for slot cutting.

- Cut a groove of 6mm width and 4mm depth.

- Drill two holes as per drawing.

- Clean the hole through reamer.

- Cut internal threads of M10 with a Tap.

- Deburr all edges of the workpiece

- Clean the work area

- Perform necessary post machining operations like filing, or deburring.

- Turn off the machine.