Course / Subject Name: NQ_Engineering Drawing and Graphics (NQ-Mech-173)

- Assessment Tasks (Summative Assessment)

- Draw the Half sectional,Front,Side and Top view of Gland Stuffing Box according to the given drawing in first angle of projection , draw a title block with drawing title, scale, date, and drawing number.

Performance Criteria

- Arrange the drawing instruments required to perform above mentioned task.

- Fix drawing sheet on drawing table

- Divide drawing in equal parts according to given drawing

- Draw a title block with drawing title, scale, date, and drawing number

- Make Drawing according to scale with accurate dimensions of the gland stuffing box, including the internal and external features such as flanges, bolts, sealing surfaces, and any other relevant components.

- Ensure that all dimensions are consistent and labeled clearly.

- Apply standard engineering scales and conventions for proper dimensioning (e.g., 1:1, 1:2, etc.)

- Draw Half sectional view to show the internal details of the gland stuffing box.

- Draw view along a plane that divides the box, exposing the internal components like the packing gland, shaft sleeve, and any seals or gaskets.

- Draw Front  view to show the gland stuffing box from the front (as seen from the shaft or pump side).

- Draw Front  view to show showing the gland stuffing box from above.

- Label all key components of the gland stuffing box clearly, such as the gland, packing, shaft sleeve, O-rings, bolts, nuts, and other sealing or mechanical elements.

- Draw sectional views to show  appropriate sectioning symbols to indicate the areas that have been cut.

- Indicate Direction of Sight with arrow.

- Draw section lines in bold to differentiate from other lines.

- Draw Section lines at an angle (usually 45°) and evenly spaced

- Label all relevant parts, and any other essential details.

- Ensure that the views are arranged logically and maintain a consistent alignment between the Half Sectional, Front, and Top views for easy comparison.

- Proper spacing between the views should be maintained to avoid overlap or confusion.

- Draw all the dimension lines.

- Write the all dimensions.