Course / Subject Name:   NQ-Elect-212. Applied Electricity and Electronics

Assessment Tasks (Summative Assessment)

Task-1: Design and construct a series-parallel circuit using resistors of different values on a breadboard. Measure the voltage drop across each resistor and the total current through the circuit. Additionally, calculate the total power consumed by the circuit. Include a clear schematic diagram to guide the construction

OR

Task-2: Construct a circuit using transistor as a switch at a base of zero and 5 volt to control working of LED for measuring voltage drop across LED

Performence Critiria

Task-1

- draw clear schematic diagram of the circuit on worksheet

- Collect three diferent values of resistors,

- Connect  one resistor in series with two connected in parallel on breadboard

- Connect breadboard with DC power supply

- Use the multimeter to measure the voltage drop across each resistor

- Set the multimeter to measure current and connect it in series with the circuit to measure total current

- Calculate the total resistance of the circuit based on the series-parallel configuration.

- Calculate total power consumed by putting total current and voltage value in power formulla

- Prepare a report including the schematic diagram, measured values, calculated results, and observations

- Follow safety protocol

- Adopt greening practice and sustainability

Task-2

- Collect the Transistor, LED, 5V DC power supply, resistors, wires, and breadboard.

- Connect the 5V DC power supply to the breadboard

- Connect the collector of the transistor to the positive terminal of the LED

- Connect the emitter of the transistor to the negative terminal of the power supply

- Connect the base of the transistor to the resistor

- Connect the other end of the  resistor to the 5V DC power supply

- Connect the  resistor in series with the LED to limit the current

- Measured and recorded the voltage drop across the LED when the base of the transistor is at 0V and 5V

- Observed the LED's on/off state based on the transistor's switching action

- Recorded and analyzed the results, comparing the measured voltage drop across the LED with the expected values

- Follow safety protocol

- Adopt greening practice and sustainability