Course / Subject Name NQ-Mech-246 Workshop Practice-II (A) Perform Basic Machining-II

Assessment Tasks (Summative Assessment)

Task-A: Cut Helical Gear of 19 teeth on Milling Machine.

OR

Task-B. Grind the given Blank and Cut Gear on Gear Hobbing Machine for 49 teeth gear.

Performance Criteria for Task-A:

- Perform calculation regarding 19 teeth Helical gear.

- Select an indexing plate as per calculation to machine for a helical gear on a Milling machine.

- Mount gear set or Gear train to engage lead screw and indexing head spindle

- Center indexing head with its tail stock.

- Fix indexing head and tail stock on milling table.

- Implement PPE.

- Apply Collant continuously

- Adjust speed

- Adjust feed

- Adjust direction of the cutter.

- Mount Gear blank on mandrel.

- Hold one side of mandrel in chuck of indexing head and other side in tail stock

- Start machine

- Carry out cutter at zero point vertically.

- Carry out cutter at zero point horizontally.

- Apply depth for rough cut

- Engage machine automatically in longitudinal direction

- Move table back at zero point.

- Apply full depth for final cut

- Engage machine automatically in forward direction.

- Repeated the process simultaneously until tooth is obtained.

Performance Criteria for Task-B:

- Calculate the require calculation regarding 49 teeth gear.

- Adjust optimal cutting speed for the grinding wheel.

- Set feed rate to balance material removal with surface finish quality.

- Adjust depth of cut to avoid overloading the wheel and to achieve desired material removal without compromising surface quality.

- Measure to verify surface finish of the workpiece.

- Check dimensions of workpiece to conform with specified tolerances.

- Prepare the work-piece by required machining (Grinding) and get it ready for turning the blank.

- Clamp gear blank and hob cutter into their holding devices as per standard practice.

- Adjust safe distance between gear blank and hub cutter as per prescribed method.

- Adjust parameters of speed and feed from control unit as per prescribed method.

- Perform simple/angled/differential indexing and divide the gear into required number of divisions.

- Implement PPE.

- Check hob cutter and gear blank that both are positioned properly and adjust them.

- Hold one side of mandrel on chuck of indexing head and other side in tail stock.

- Start machine and carry out cutter at zero point vertically.

- Apply depth for rough cut and engage machine automatically in longitudinal direction

- Move table back at zero point.

- Apply full depth for final cut and engage machine automatically in forward direction.

- Repeat process simultaneously until required teeth gear obtained.