Course / Subject Name NQ-Mech-246. Workshop Practice-II (C) Perform Advance Welding

- Assessment Tasks (Summative Assessment)

- Cut Two M.S plates size (1/8x1x4 inches) with Gas cutting Manually and on Auto machine. Perform GMAW, GTAW and GMAW welding in Flat(1F,1G) Horizontal (2F,2G) Vertical (3F,3G) positions

OR

- Weld two MS rod by Submerge and Friction Welding

OR

- Perform Non- Destructive and Destructive Welding Tests

Performance criteria

Task 1

- Collect material according to specification

- Mark the cut lines on the MS plate.

- Set up the oxy-fuel cutting torch with the appropriate settings.

- Manually cut along the marked lines, ensuring smooth and precise cuts.

- Inspect the cut edges for any irregularities or oxidation.

- Set up the Automated cutting machine with the MS plate and programming for the required cut.

- Adjust the cutting speed, oxygen pressure, and other settings for optimal cutting.

- Execute the automated cutting process, ensuring precise cuts and minimal material wastage.

- Inspect the cut edges for quality.

- Set up the GMAW equipment with the correct wire and shielding gas.

- Weld along the joint of the two pieces using a continuous motion and proper heat settings.

- Inspect the weld for smoothness, penetration, and minimal spatter.

- Set up the GTAW machine with the correct tungsten electrode and filler material.

- Clean the metal surfaces and perform the weld along the joint, ensuring a stable arc and minimal oxidation.

- Inspect the weld for smoothness, precision, and even bead formation.

Task 2

- Set up the SAW machine, ensuring the correct wire and flux for the welding process.

- Adjust the machine settings to ensure the right heat input for deep penetration.

- Perform the weld and inspect the slag removal process.

- Inspect the weld for consistency and deep penetration.

- Set up the Friction welding machine with the required parameters for the MS plate.

- Perform the friction welding process by rotating one piece of the material while applying pressure to create heat and join the pieces.

- Inspect the welded joint for consistency and strength.

Task 3

- Inspect the welded joints with the naked eye or magnifying tools for any visible defects such as cracks, porosity, undercuts, or incomplete fusion.

- Document the findings and mark any visible surface defects.

Tensile Test:

- Cut a sample from the welded joint and prepare it according to the dimensions required for the tensile test.

- Set up the tensile testing machine and calibrate it.

- Perform the tensile test by applying a gradual force to the specimen until failure.

- Record the load at failure and measure the elongation.

- Analyze the fracture surface to determine the failure mode (weld or base material).

Impact Test:

- Prepare the welded sample for an impact test (Charpy or Izod).

- Set up the impact testing machine.

- Perform the impact test by striking the specimen with a known force.

- Record the energy absorbed during fracture and analyze the toughness of the weld.