Course / Subject Name: NQ-Mech-352 Computer Aided Designing/Computer Aided Manufacturing

Assessment Tasks (Summative Assessment)

Create a 3D drawing of the given figure using CAD software. Then import the file into CAM and generate the CNC code using the software. Import the program and run the simulation

Performance Criteria:

- Open CAD software (like SolidWorks, AutoCAD, CATIA, or Fusion 360).

- Recreate the design by referencing the 2D dimensions provided in the image.

  - Use tools such as extrusion, revolve, or cut for various features.

  - Ensure that tolerances, dimensions, and hole placements are precisely modeled.

- After completing the 3D model, export the file in a format compatible with CAM software, such as:

  - STEP (*.step / *.stp)

  - IGES (*.iges / *.igs)

  - STL (*.stl)

- Open a CAM software like Mastercam, Fusion 360 (CAM workspace), or Autodesk Powermill.

- Import the 3D model file.

- Define the stock material dimensions (size of the raw material block).

- Choose the tooling (e.g., drills, end mills) based on the material specified in the drawing (e.g., AISI 321 Stainless Steel).

- Apply the toolpaths for:

  - Facing

  - Drilling

  - Pocketing

  - Contouring

- Optimize the tool paths to minimize machining time and improve accuracy.

- Set up the machine configuration (e.g., 3-axis or 5-axis CNC machine).

- Post-process the tool paths to generate CNC code (G-code or M-code).

- Save the CNC program as a *.nc file.

- Import the generated CNC code back into the CAM software or a CNC simulation tool.

- Run the simulation to ensure:

  - The toolpaths avoid collisions.

  - The final product matches the design.

  - Machining is efficient.