National Competency Standards

“Auto & Farm Machinery Technician” Level-4

Occupation (Auto & Farm Machinery Technician)

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Level 4 Auto & Farm Machinery Technician. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.1.	Date of development:	5

1.2.	Date of review & validation:	5

1.3.	Entry requirement:	5

1.4.	Codes of qualification	5

1.5.	Summary of competencies	5

0716MSA1801: Comply Health and Safety Guidelines	7

0716MSA1803: Interpret Drawings	12

0716MSA1804: Perform Bench Work	15

0716MSA1805: Perform Repair/Maintenance of Petrol Engines	19

0716MSA1806: Perform Repair/Maintenance of Diesel Engines	25

0716MSA1807: Perform Repair/Maintenance of Suspension, Steering and Braking System	31

0716MSA1808: Perform Repair/Maintenance of Transmission system	35

0716MSA1809: Perform Repair/Maintenance of Farm Machinery	39

0716MSA1810: Perform Repair/Maintenance of Agriculture Implements	43

0716MSA1811: Perform Repair/Maintenance of Electric Components	47

0716MSA1812: Adopt Green Practices at Workplace	51

0716MSA1813:.Develop Entrepreneurship Skills	54

1.6.	Members of Qualification Development & Review Committee	63

Introduction

The National Vocational and Technical Training Commission (NAVTTC) has developed competency standards (CS) for the "Auto & Farm Machinery Technician" trade in Pakistan, recognizing the importance of vocational and technical education in equipping the workforce with the necessary skills to meet industry needs. This field is essential to agriculture sector requiring a broad range of knowledge and skills for the maintenance, repair, and operation of agriculture and farm machinery. As technology advances and the focus on efficiency and sustainability grows, so does the demand for skilled professionals. The competency standards will provide a comprehensive framework that defines the essential knowledge, skills, and attitudes required for effective performance, serving as a benchmark for training providers, employers, and learners. Additionally, incorporating green skills into TVET is crucial for building a workforce that is environmentally conscious and capable of adopting sustainable practices, supporting the transition to a low-carbon economy. This qualification can significantly impact efficient farm management, maximize crop yields, and promote safety and environmental procedures, consequently contributing to Pakistan's economic growth.

| # Date of development: | April 2016 |
| --- | --- |
| # Date of review & validation: | 22 July, 2027 |
| # Entry requirement: | Middle Preferably Matric |

# Codes of qualification

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications is assigned as follows:

| ISCED Classification |  |
| --- | --- |
| Code | Description |
| 0716 A&FM | National Vocational Certificate Level 4 in Auto & Farm Machinery Technician |

# Summary of competencies

| Competency Standards | NVQF Level | Category | Estimated Contact Hours |  |  | Cr Hr |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  |
| 0716MSA1801 Comply Health and Safety Guidelines | L2 | Functional | 4 | 16 | 20 | 2 |
| 0716MSA1802 Follow the Communication Policy/Procedure at the Workplace | L2 | Generic | 4 | 16 | 20 | 2 |
| 0716MSA1803 Interpret Drawings | L3 | Functional | 10 | 30 | 40 | 4 |
| 0716MSA1804 Perform Bench work | L3 | Technical | 16 | 84 | 100 | 10 |
| 0716MSA1805 Perform Repair/Maintenance of Petrol Engines | L4 | Technical | 26 | 154 | 180 | 18 |
| 0716MSA1806 Perform Repair/Maintenance of Diesel Engines | L4 | Technical | 30 | 170 | 200 | 20 |
| 0716MSA1807 Perform Repair/Maintenance of suspension, steering and braking system | L4 | Technical | 12 | 48 | 60 | 6 |
| 0716MSA1808 Perform Repair/Maintenance of transmission system | L4 | Technical | 20 | 60 | 80 | 8 |
| 0716MSA1809 Perform Repair/Maintenance of Farm Machinery | L4 | Technical | 15 | 85 | 100 | 10 |
| 0716MSA1810 Perform Repair/Maintenance of Agriculture Implements | L4 | Technical | 48 | 132 | 180 | 18 |
| 0716MSA1811 Perform Repair/Maintenance of Electric Components | L4 | Technical | 48 | 132 | 180 | 18 |
| 0716MSA1812 Adopt Green Practices at Workplace | L3 | Generic | 9 | 10 | 19 | 2 |
| 0716MSA1813 Develop Entrepreneurship Skills | L3 | Generic | 4 | 16 | 20 | 2 |
| Adopt Soft Skills (Annexed) |  | Generic | 1 | 0 | 1 | 0.1 |
| Total |  |  | 247 | 953 | 1200 | 120 |

### 0716MSA1801: Comply Health and Safety Guidelines

Overview:

This competency standard is designed to develop skill and competency required to maintain Occupational Health and Safety and take remedial measures to deal with the emergencies in a professional manner, thus minimizing the losses and providing a safe and healthy working environment. The learner will also be able to practice green skills and become more environmentally friendly.

| Competency Units | Performance Criteria |
| --- | --- |
| - Analyze Hazards at Workplace | - Interpret relevant work processes and procedures to identify risk to Health, hygiene and safety at workplace. - Record processes, tools, equipment and consumable materials that have the potential to cause harm. - Check expiry of Fire extinguisher. - Operate fire extinguisher. - Prepare Report of the identified risk to Health, hygiene and safety. |
| - Use Personal Protective and Safety Equipment (PPE) as per requirement | - Ensure personal protective equipment is clean. - Select personal protective equipment in terms of type and quantity according to work orders. - Wear, adjust, and maintain personal protective equipment to ensure correct fit and optimum protection in compliance with company procedures. |
| - Dispose-off hazardous Waste materials | - Segregate hazardous waste/ drug materials which needs to be disposed of. - Collect hazardous or non-hazardous waste carefully from the designated area as per approved procedure. - Cary out disposal of hazardous waste hazardous waste as per SOP's. - Adopt necessary precautions such as putting masks and gloves while disposing hazardous waste/ materials as per standard operating procedure |

Knowledge & Understanding

- Safety rules and regulations of organization.

- List of personal protection and safety equipment.

- Meaning of safety signs and symbols.

- Safety related Standard Operating Procedure/guidelines.

- Waste disposal SOPs.

- Best practices relating to clean work environment.

- Best practices relating to safe work environment.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Interpret Health, hygiene and safety procedures/precautions.

- Interpret Health, hygiene and safety signs and symbols.

- Safety reporting procedures and documentation.

- Use of Personal Protective Equipment.

- First Aid treatment methods.

- Analyze possible hazards at workplace.

Tools and equipment Required

The tools and equipment required for this competency standard are provided below:

| S.No. | Items |
| --- | --- |
| 1. | Safety shoes, |
| 2. | Helmet, |
| 3. | Safety gloves |
| 4. | Overall |
| 5 | Safety glasses, |
| 6. | Fire extinguishers |
| 7. | First aid box kit |

0716MSA1802: Follow Communication Policy/Procedure at the Workplace

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skills and knowledge required to follow communication policy/procedure at workplace. It encompasses the communication techniques used at the workplace. This unit emphasizes the importance of communication/procedure at the workplace

| Competency Units | Performance Criteria |
| --- | --- |
| - Communicate in a team to achieve intended outcomes | - Treat team members with respect and maintain positive relationships to achieve common organizational goals - Listen supervisor's instructions carefully & comply - Provide work related information to team members and identify interrelated work activities to avoid confusion - Adopt communication skills, appropriate to work activities and organizational procedures - Analyze problems and resolve them through discussion and mutual agreement |
| - Follow Supervisor’s instructions | - Listen to the instructions of Supervisor - Follow instructions of the supervisor - Report to the supervisor as per SOPs |

Knowledge & Understanding

- Reporting techniques

- importance and application of work ethics

- Explain the importance of Good communication skills (7Cs of effective communication)

- Workplace dress code

- Describe the role of team members and functionality of the teams

- Describe team dynamics and stages of team development

- Describe Conflict resolution strategies

- Prepare relevant documents and reports

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Effective communication with colleagues, clients and supervisors

- Prepare office report as per instructions.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
| 1 | Communication Policies. |
| 2 | Standard Operating Procedures |

### 0716MSA1803: Interpret Drawings

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to perform calculations, interpret drawings and mark layouts. It encompasses a broad range of techniques that are essential to Perform calculations, interpret drawings and mark layouts.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform Basic Mathematical calculations | - Perform multiplication, subtraction, division and addition. - Calculate radius, area, perimeter circumference, volume of circles, square, triangle etc. - Find percentages and diameter |
| - Interpret Drawing | - Interpret symbols from the given drawing. - Interpret scale of the given drawing. - Interpret different types of abbreviations from the given drawing. - Extract dimensions from the given drawing. - Estimate material from the given job specification/ drawing. - Interpret different section views/ coordinated sections from the given drawing. - Interpret revisions in drawing. |
| - Mark Layout | - Mark points as per dimensions taken from drawing. - Mark lines between points. - Perform center punching in the layout as per job requirement. - Verify the marked layout as per given drawing before starting the operations. |

Knowledge & Understanding

- Understanding of addition, subtraction, division and multiplication.

- Understanding of area, perimeter, circumference, volume of circles, square triangle etc.

- Knowledge of finding radius, percentage and diameter.

- Knowledge of engineering drawings.

- Understand Types of lines and dimensions.

- Identification of tools & equipment/instruments of drawing.

- Identification of tools & equipment/ instruments for marking layouts.

- Describe types of drawings/specifications and their applications

- Explain symbols relating to drawing and their applications.

- Define line diagram.

- Describe abbreviations and scale of drawing.

- Interpret & extract information from drawing & specifications.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform addition, subtraction, multiplication and division.

- Interpret symbols from the given drawing.

- Extract measurements from the given drawing.

- Mark layout as per the instructions.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
| 1 | Tool board |
| 2 | Calculator |
| 3 | Drawing table |
| 4 | Drawing Board |
| 5 | Steel foot rule |
| 6 | Pencils. |
| 7 | Marking tools. |
| 8 | Geometry Box |

### 0716MSA1804: Perform Bench Work

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to perform suitable operations of cutting, sawing, filing, chiseling, straightening, riveting, hand threading and grinding. It encompasses a broad range of diagnostic techniques and procedures that are essential for ensuring the efficient and effective operation in both automotive and agricultural machinery workshops.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform cutting and sawing operation | - Select blade according to the material - Set the blade in hacksaw frame. - Mark layout of job as per drawing using marking tool. - Clamp work piece according to the instructions. - Perform sawing according to the instructions. - Check quality of the component at suitable intervals. - Verify the final job with the given drawing. |
| - Perform filling | - Select file type according to the material & profile. - Select marking tool and mark layout of job as per drawing. - Clamp work piece according to the drawing. - Perform filing as per SOP' - Check quality of the component at suitable intervals. - Verify the final job with the given drawing. |
| - Perform Chiseling | - Select chisel type and size according to the material & profile. - Select marking tool and mark layout as per drawing. - Perform chiseling as per SOP's - Check quality of the component at suitable intervals. - Verify the final job with the given drawing. |
| - Produce threads on work piece by tap & die | - Select tap/die according to the job specification. - Select marking tool and mark layout as per drawing. - Clamp work piece in clamping device. - Perform pre-drilling to produce hole according to tap size. - Perform taping as per job specification. - Verify the final job with given drawing |
| - Perform Straightening | - Select straightening tool/hammer according to the material & profile. - Clamp work piece in clamping device. - Perform hammering. - Check quality of the component at suitable intervals. - Verify the final job with the given drawing. |
| - Perform riveting | - Select rivet type and size according to the material & profile. - Select marking tool and mark layout as per drawing. - Clamp work piece in clamping device. - Perform riveting as per standard procedures. - Check quality of the component at suitable intervals. - Verify the final job with the given drawing. |
| - Perform grinding using portable grinding machine | - Select portable grinding machine according to the job specification. - Select marking tool and mark layout as per drawing. - Clamp work piece in clamping device. - Perform grinding to complete size. - Verify the final job with given drawing. |

Knowledge & Understanding

- Types & applications of clamping devices.

- Types and uses of basic measuring instruments required during different operations.

- Different types of hacksaw blades and its use.

- File types and uses.

- Types and application of hammers.

- Types and application of chisels.

- Methods of hand threading.

- Types of pedestal grinding machine and its uses.

- Types of portable grinding machine cutters.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Select measuring, marking & cutting tools & equipment according to the requirement.

- Produce component by performing marking, cutting/sawing, filing, chiseling, straightening, riveting, hand threads and grinding.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
|  | Pedestal Grinder with accessories and consumables. |
|  | Steel Rules. |
|  | Vernier Caliper. |
|  | Thread gauges. |
|  | Screw pitch gauges. |
|  | Feeler gauges. |
|  | Dial indicators with magnetic stand. |
|  | Micrometer. |
|  | Set of Adjustable Wrench. |
|  | Pipe wrench. |
|  | L-key sets. |
|  | Nose pliers. |
|  | Grip pliers. |
|  | Straight peen Hammer. |
|  | Ball peen Hammer. |
|  | Mallets Hammer. |
|  | Hammer. |
|  | Scriber. |
|  | Hand hacksaw with blades. |
|  | Safety gloves. |
|  | Safety goggles. |
|  | Disc grinder/cutter |
|  | Working table. |
|  | Sand papers of different grade. |
|  | File set. |
|  | Chisel Set |

### 0716MSA1805: Perform Repair/Maintenance of Petrol Engines

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to diagnose and repair faults in petrol engines. It encompasses a broad range of diagnostic techniques and repair procedures that are essential for ensuring the efficient and effective operation of petrol engines in both automotive and agricultural machinery.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange tools and equipment to diagnose fault in petrol engine | - Use PPE as per SOPs - Arrange tools and equipment required to diagnose fault in petrol engine. |
| - Dismantle the engine from the vehicle | - Select tools to dismantle the engine. - Remove external accessories from the engine. - Dismantle accessories from the engine. - Dismantle the engine from the vehicle. |
| - Disassemble the engine | - Remove cylinder head from engine. - Disassemble cylinder head assembly. - Inspect the condition of cam and cam bush. - Inspect the condition of cylinder head for crack, valve seat damage, valve guide damage. - Inspect and check the components of valve assembly. - Remove engine pistons and check the condition of pistons rings, counter shaft, crank Shaft, timing chain/belt, oil pump, bearing, cylinder sleeves. |
| - Diagnose fault in Electrical System | - Inspect electrical components: - Electric Control Unit (ECU). - Engine Wiring/devices. - EFI System/sensors. - Spark plugs and Ignition cables. - Detect fault in electric system as per SOPs. |
| - Repair/Replace faulty components in electrical system | - Remove the faulty components. - Repair/Replace the faulty components. - Install/assemble the replaced/repaired components. - Test repaired/replaced components as per SOPs. |
| - Diagnose fault in Fuel & ignition system | - Inspect fuel system (fuel tank, fuel line, fuel filters) and ignition system (battery, ignition switch, current distributor, cables) of the engine. - Detect fault in electric system as per SOPs. |
| - Repair/Replace faulty components in Fuel & ignition system | - Remove the faulty components. - Repair/Replace the faulty components. - Install/assemble the replaced/repaired components. - Test repaired/replaced components as per SOPs. |
| - Diagnose fault in cooling system | - Inspect cooling system (Radiator leakage, hose pipes, thermostat valve, water pump, radiator fan). - Detect fault in cooling system as per SOPs. |
| - Replace/repair faulty components in cooling system | - Remove the faulty components. - Repair/Replace the faulty components. - Install/assemble the replaced/repaired components. - Test repaired/replaced components as per SOPs. |
| - Diagnose fault in Lubrication system | - Inspect lubrication system (oil level, oil pump, blocking of oil filter, oil pressure, sensors). - Detect fault in lubrication system as per SOP. |
| - Replace/repair faulty components in lubrication system | - Remove the faulty components - Repair/Replace the faulty components - Install/assemble the replaced/repaired components - Test repaired/replaced components as per SOPs. |
| - Diagnose fault in Intake & Exhaust system. | - Inspect intake system (intake manifold, carburetor/EFI, MAF sensor, EGR, PCV valve, turbo and super charger, O2 sensor) - Inspect exhaust system (exhaust manifold, muffler, catalytic converter) - Detect fault in intake & exhaust system as per SOPs |
| - Replace/repair faulty components in intake & exhaust system | - Remove the faulty components - Repair/Replace the faulty components - Install/assemble the replaced/repaired components - Test repaired/replaced components as per SOPs |
| - Assemble the Engine | - Assemble engine pistons and piston rings. - Assemble cylinder head from engine - Assemble cylinder head assembly - Connect crankshaft with piston rod - Assemble the engine |
| - Interconnect external accessories of engine | - Clean external accessories - Re-connect external accessories to the engine as per SOPs |
| - Carry out work performance | - Check final inspection to ensure work is done according to expectation of workplace. - Set engine performance in accordance with the manufacturer’s manual. - Start engine. - Warm up engine to normal operating temperature. - Clean the work area and dispose of waste and non-recycle materials. - Check tools and equipment are stored according to SOP's. - Process the Workplace documents according to workplace procedures. |

Knowledge & Understanding

- Understanding of Personal Protective Equipment (PPE).

- Understanding use of various tools and equipment.

- Dismantling the engine.

- Disassembling the engine.

- Assembling the engine.

- Repair/replacement of faulty parts.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Uses of Personal Protective Equipments.

- Dismantle the engine.

- Disassemble and assemble the petrol engine.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
|  | Safety Cap |
|  | Safety Gloves |
|  | Work Overall |
|  | Safety Shoes |
|  | Hand Wash |
|  | First Aid Box |
|  | Multi meter |
|  | Diagnostic scanner |
|  | Piston Kit |
|  | Complete tool kit |
|  | Snap ring pliers |
|  | Seal extractor |
|  | Bearing puller |
|  | Gear Puller |
|  | Air Compressor |
|  | Creeper |
|  | Trolley jack |
|  | Ring extractor/compressor |
|  | Stethoscope |
|  | Torque wrench |
|  | Hammer/Rubber Hammer |
|  | Chain hoist 2 ton |
|  | Engine Stand |
|  | Valve Spring Compressor |
|  | Engine Timing Gun |
|  | Magnet Stick |
|  | Impact Wrench |
|  | Allen Key Set |
|  | Timing Gun |
|  | Techo meter |
|  | Test Lamp |
|  | Vacuum Gauge |
|  | Universal Strip Wrench/Oil Filter Spinner |
|  | Vernier Caliper |
|  | Micro meter |
|  | Tool Trolly (complete set) |
|  | Washing Tray |
|  | Wall Spring |

### 0716MSA1806: Perform Repair/Maintenance of Diesel Engines

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to diagnose and repair faults in diesel engines. It encompasses a broad range of diagnostic techniques and repair procedures that are essential for ensuring the efficient and effective operation of diesel engines in both automotive and agricultural machinery.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange tools and equipment to diagnose fault in Diesel engine | - Use PPE as per SOPs. - Arrange tools and equipment to diagnose fault in Diesel engine. |
| - Dismantle the engine from the vehicle | - Select tools to dismantle the engine. - Remove external accessories from the engine. - Dismantle the engine as per SOPs. |
| - Disassemble the engine | - Remove cylinder head from engine. - Disassemble cylinder head assembly. - Inspect the condition of cam and cam bush. - Inspect the condition of cylinder head for crack, valve seat damage, valve guide damage. - Inspect and check the components of valve assembly. - Remove engine pistons and check the condition of pistons rings, counter shaft, crank Shaft, timing chain/belt, oil pump, bearing, cylinder sleeves. |
| - Diagnose fault in Fuel System | - Inspect fuel system (fuel pump, fuel filter, fuel lines, fuel injectors, fuel feeding pump, fuel tank, ECU, Air intake Sensors). - Detect fault in fuel system as per SOPs. |
| - Repair/Replace faulty components in fuel system | - Remove the faulty components. - Repair/Replace the faulty components - Install/assemble the replaced/repaired components - Test repaired/replaced components as per SOPs |
| - Diagnose fault in cooling system | - Inspect cooling system (Radiator Leakage, hose pipes, thermostat valve, water pump, radiator fan) - Detect fault in cooling system as per SOPs. |
| - Replace/repair faulty components in cooling system | - Remove the faulty components. - Repair/Replace the faulty components. - Install/assemble the replaced/repaired components. - Test repaired/replaced components as per SOPs. |
| - Diagnose fault in Lubrication system | - Inspect lubrication system (oil level, oil pump, Blocking of oil filter, Oil Pressure Sensor). - Detect fault in lubrication system as per SOPs. |
| - Replace/repair faulty components in lubrication system | - Remove the faulty components. - Repair/Replace the faulty components. - Install/assemble the replaced/repaired components. - Test repaired/replaced components as per SOPs. |
| - Diagnose fault in Intake & Exhaust system. | - Inspect intake system (intake manifold, turbo/super charger, EGR Valve, MAF Sensor, Vacuum Leakage). - Inspect exhaust system (exhaust manifold, Temp Sensor, muffler). - Detect fault in intake & exhaust system as per SOPs. |
| - Replace/repair faulty components in intake & exhaust system | - Remove faulty components. - Repair/Replace faulty components. - Install/assemble the replaced/repaired components. - Test repaired/replaced components as per SOPs. |
| - Assemble the Engine | - Re-connect engine pistons and pistons rings. - Re-connect cylinder head from engine. - Re-assemble cylinder head assembly. |
| - Interconnect external accessories of engine | - Clean external accessories. - Re-connect external accessories to the engine as per SOPs. |
| - Carry out work performance | - Check final inspection to ensure work is done according to expectation of workplace. - Set engine performance in accordance with the manufacturer’s manual. - Start engine. - Warm up engine to normal operating temperature. - Clean the work area and dispose of waste and non-recycle materials. - Check tools and equipment are stored according to SOP's. - Process the Workplace documents according to workplace procedures. |

Knowledge & Understanding

- Understanding use of Personal Protective Equipment (PPE).

- Understand use of various tools and equipment.

- Dismantling the engine.

- Disassembling the engine.

- Assembling the engine.

- Understanding of Replacement/Removal of faulty parts.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Uses of Personal Protective Equipments

- Select tools & equipments to dismantle, disassemble and re-assemble the diesel engine.

- Disassemble and re-assemble the diesel engine

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| 12 | NAME |
| --- | --- |
|  | Safety Cap |
|  | Safety Gloves |
|  | Work Overall |
|  | Safety Shoes |
|  | Hand Wash/toilet soap |
|  | First Aid Box |
|  | Fuel injector tester |
|  | Compression tester |
|  | Piston Kit |
|  | Tool kit |
|  | Snap ring pliers |
|  | Seal extractor |
|  | Bearing puller |
|  | Gear Puller/fuel filter wrench |
|  | Air Compressor |
|  | Creeper |
|  | Trolley jack |
|  | Ring extractor/compressor |
|  | Stethoscope |
|  | Torque wrench |
|  | Hammer/Rubber Hammer |
|  | Chain hoist 2 ton |
|  | Engine Stand |
|  | Valve Spring Compressor |
|  | Engine Timing Gun |
|  | Magnet Stick |
|  | Diesel Fuel Pump Tester |
|  | Flaring Tools |
|  | Magnet Stick |
|  | Impact Wrench |
|  | Allen Key Set |
|  | Timing Gun |
|  | Techo meter |
|  | Test Lamp |
|  | Vacuum Gauge |
|  | Universal Strip Wrench/Oil Filter Spinner |
|  | Vernier Caliper |
|  | Micro meter |
|  | Tool Trolley (complete set) |
|  | Washing Tray |
|  | Wall Spring compressor |

### 0716MSA1807: Perform Repair/Maintenance of Suspension, Steering and Braking System

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to diagnose and repair faults in Suspension, Steering and Braking System. It encompasses a broad range of diagnostic techniques and repair procedures that are essential for ensuring the efficient and effective operation of Suspension, Steering and Braking System in both automotive and agricultural machinery.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange tools and equipment to diagnose fault in suspension, steering and braking system | - Use PPE as per SOPs. - Arrange tools and equipment used to diagnose fault in suspension, steering and braking system |
| - Diagnose fault in suspension system | - Perform road test. - Raise the vehicle on jack and inspect the following components. - Inspect lower control bush, leakage in shock absorber oil, broken spring, ball joint, control arm, stabilizer bar. - Detect fault in suspension system as per SOPs. |
| - Replace/repair faulty components in suspension system | - Remove the faulty components. - Repair/Replace the faulty components. - Assemble/connect the repaired/replaced component. - Test the Repair/Replace component as per SOPs. |
| - Diagnose fault in Steering System | - Perform road test. - Detect the faults in steering system, Rack & Pinion, Fluid of power steering as per SOPs. |
| - Repair/Replace faulty components in steering system | - Remove the faulty components. - Repair/Replace the faulty components. - Assemble/connect the repaired/replaced component. - Test the Repair/Replace component as per SOPs. |
| - Diagnose fault in braking system | - Perform road test. - Detect fault in braking system (Brake Shoe, Fluid, Brake lines) as per SOPs. |
| - Repair/Replace faulty components in braking system | - Remove the faulty components - Repair/Replace the faulty components - Assemble/connect the repaired/replaced component - Test the Repair/Replace component as per SOPs. |

Knowledge & Understanding

- Knowledge of Personnel Protective Equipment (PPE).

- Understand the Uses of various tools and equipment.

- Describe the suspension system.

- Describe steering system.

- Describe braking system.

- Disassembling the engine.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of Personal Protective Equipments.

- Select tools & equipments to diagnose fault in suspension, steering and braking system.

- Disassemble/assemble CV joint of the suspension system.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
|  | Safety Cap |
|  | Safety Gloves |
|  | Work Overall/Apron |
|  | Safety Shoes |
|  | Hand Wash/toilet soap |
|  | First Aid Box |
|  | Vehicle lifter hydraulic jack |
|  | Combination spanner set |
|  | Screw driver set |
|  | Hose plier |
|  | Air blow gun |
|  | Air breeding job |
|  | Micro meter |
|  | Vernier caliper |
|  | Hammer/Rubber Hammer |
|  | Steering Wheel Puller |
|  | Tie rod puller |
|  | Ball Joint Separator |
|  | Alignment Tools |
|  | Control Arm Tool Kit |

### 0716MSA1808: Perform Repair/Maintenance of Transmission system

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to diagnose and repair faults in transmission System. It encompasses a broad range of diagnostic techniques and repair procedures that are essential for ensuring the efficient and effective operation of transmission System in both automotive and agricultural machinery.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange tools and equipment to diagnose fault in transmission system | - Use PPE as per SOPs. - Arrange tools and equipment to diagnose fault in transmission system. |
| - Diagnose fault in manual transmission system | - Inspect manual gear system. - Inspect clutch mechanism (paddle, linkage). - Inspect hydraulic clutch fluid conditions and leakage, according to specified standards. - Inspect gear mechanism (linkage bushes and bearings, shafts of gears), oil level and leakage. - Detect fault in manual transmission system as per SOPs. |
| - Replace/repair faulty components in manual transmission system | - Remove the faulty components. - Repair/Replace the faulty components. - Assemble/connect the repaired/replaced component. - Test the Repair/Replace component as per SOPs. |
| - Diagnose fault in automatic transmission System | - Perform road test at different speeds for smooth operations of torque converter and gear shifting as per SOPs. - Inspect automatic transmission mounts for faults. - Inspect automatic transmission fluid (ATF) level according to SOPs and identify leaks. - Check automatic transmission solenoid by using automotive scanner. - Check electrical controls and Hydraulic pressure of automatic transmission. - Diagnose faults in automatic transmission system as per SOPs. |
| - Replace/repair faulty components in automatic transmission system | - Remove the faulty components. - Repair/Replace the faulty components. - Assemble/connect the repaired/replaced component. - Test the Repair/Replace component as per SOPs. |

Knowledge & Understanding

- Understanding of Personal Protective Equipment (PPE).

- Understand the Uses of various tools and equipment.

- Describe manual transmission system including clutch mechanism (paddle, linkage).

- Describe hydraulic clutch fluid conditions and leakage.

- Describe gear mechanism (linkage bushes and bearings shafts of gears), oil level and leakage.

- Describe gear ratio and noise.

- Describe automatic transmission system including mounts for faults, fluid (ATF) level, solenoid by using automotive scanner, electrical controls and Hydraulic pressure of automatic transmission.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of Personal Protective Equipments.

- Select tools & equipments to diagnose fault in manual and automatic transmission system.

- Dismantle/assemble clutch assembly.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
| 1 | Safety Cap |
| 2 | Safety Gloves |
| 5 | Work Overall |
| 6 | Safety Shoes |
| 7 | Hand Wash/toilet soap |
| 9 | First Aid Box |
| 10 | Tool kit |
| 11 | Stethoscope |
| 12 | Gear puller |
| 13 | Two post lift and safety stand |
| 14 | Air compressor |
| 15 | Creeper |
| 16 | Trolley Jack |
| 17 | Special service tool |
| 18 | Multi meter |
| 19 | Scanner |

### 0716MSA1809: Perform Repair/Maintenance of Farm Machinery

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to diagnose and repair Farm Machinery. It encompasses a broad range of diagnostic techniques and repair procedures that are essential for ensuring the efficient and effective operation of Farm Machinery.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange tools and equipment to diagnose fault in farm machinery | - Use PPE as per SOPs. - Arrange tools and equipment to diagnose fault in farm machinery. |
| - Diagnose fault in tractor | - Inspect: - Engine oil - Oil filter - Tyre pressure - Hydraulic pressure - Steering and suspension system, - Power generator - Transmission oil - Air Filter - Battery - Detect fault in tractor as per SOPs. |
| - Replace/ repair faulty components in tractor | - Maintain oil level (engine, transmission, hydraulic). - Maintain tyres (pressure, puncture). - Replace air/oil filter. - Repair/replace generator & electrical accessories. - Maintain battery (electrolyte, charging). - Clean, grease & lubricate the components. |
| - Diagnose fault in combine harvester | - Inspect: - Engine oil - Oil filter - Tyre pressure - Hydraulic pressure - Steering and suspension system - Power generator - Air Filter - Battery - Reel, header, auger, threshing cylinder. - Grand tank storage. - Detect fault in combine harvester as per SOPs. |
| - Replace/repair faulty components in combine harvester | - Maintain oil level (engine, transmission, hydraulic). - Maintain tyres (pressure, puncture). - Replace air/oil filter. - Repair/replace generator & electrical accessories. - Maintain battery (electrolyte, charging). - Maintain/replace sharpness of header. - Maintain auger. - Maintain threshing cylinder. - Clean grand tank storage. - Clean, grease & lubricate the components. |
| - Diagnose fault in Reaper | - Inspect cutting bars, knife guard, crop divider, star wheels, transport belt/chain. - Diagnose fault in reaper as per SOPs. |
| - Replace/repair faulty components in Reaper | - Repair/Replace cutting bars by using tools. - Repair/Replace star wheels. - Repair/Replace transport belt/chain. - Clean, grease & lubricate the components. - Test the Repair/Replace components as per SOPs. |

Knowledge & Understanding

- Understanding of Personal Protective Equipment (PPE).

- Understand the Uses of various tools and equipment.

- Describe how to maintain the Tractor.

- Describe how to maintain the Combine harvester.

- Describe how to maintain the Reaper.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Uses of Personal Protective Equipments.

- Select tools & equipments to diagnose faults in Tractor.

- Repair/replace generator & electrical accessories.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
| 1 | Safety Cap |
| 2 | Safety Gloves |
| 5 | Work Overall |
| 6 | Safety Shoes |
| 7 | Hand Wash/toilet soap |
| 9 | First Aid Box |
| 10 | Tool kit |
| 11 | Air compressor |
| 12 | Creeper |
| 13 | Trolley jack |
| 14 | Special service tool |
| 15 | Multi meter |
| 16 | Battery tester |
| 17 | Battery charger |
| 18 | Hydro meter |
| 19 | Oil dispenser |
| 20 | Vernier caliper |
| 21 | Oil cane |
| 22 | Air pressure gauge |
| 23 | Oil filter wrench |
| 24 | Tool tray |

### 0716MSA1810: Perform Repair/Maintenance of Agriculture Implements

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to diagnose and repair Agriculture Implements. It encompasses a broad range of diagnostic techniques and repair procedures that are essential for ensuring the efficient and effective operation of Agriculture Implements.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange tools and equipment to diagnose fault in Agriculture Implements | - Use PPE as per SOPs. - Arrange tools and equipment to diagnose fault in Agriculture Implements. |
| - Diagnose fault in; - Farm Trolley - Land Scrapper - Mold Board Plough - Disc Plough - Cultivator - Rotavator - Ridger - Seed Drill - Seed Planter - Sprayers | - Detect fault in Farm Trolley (wheel drum, bearing, tyres,). - Detect fault in Land Scrapper (nut bolts, Sharp edge) by using tools. - Detect fault in Mold Board Plough (nut bolts). - Detect fault in cultivator (spring, nut bolts, plow tips). - Detect fault in Rotavator (blades, side gear box, gear box reduction, nut bolts). - Detect fault in ridger (mold board, nut bolts). - Detect fault in Seed drill (calibration, nut bolts, pipes). - Detect fault in Seed Planter (Calibration, gear maintenance, nut bolts), - Detect fault in Sprayer (nozzle, pipes, electronics devices, water tank, nut bolts, greasing). |
| - Replace/repair faulty components in - Farm Trolley - Land Scrapper - Mold Board Plough - Disc Plough - Cultivator - Rota veter - Ridger - Seed Drill - Seed Planter - Sprayer | - Repair & maintain Farm Trolley (wheel drum, bearing, tyres,) by using tools & equipments. - Maintain Land Scrapper (nut bolts, Sharp edge) by using tools & equipment. - Maintain Mold Board Plough (nut bolts). - Repair & maintain cultivator (spring, nut bolts, plow tips). - Repair &Maintain Inspect Rotavator (blades, side gear box, gear box reduction, nut bolts). - Repair & maintain ridger (mold board, nut bolts). - Replace & maintain Seed drill (calibration, nut bolts, pipes). - Maintain Seed Planter (Calibration, gear maintenance, nut bolts). - Replace & Maintain Sprayer (nozzle, pipes, electronics devices, water tank, nut bolts, greasing). |
| - Diagnose fault in Land Leveler and Laser Leveler | - Detect fault in land lever (cutting edge, nut bolt). - Detect fault in optional attachments in laser leveler (Laser/GPS, Wheels, cutting edge, frame, blades, hitch). |
| - Replace/repair faulty components in Land Leveler and Laser Leveler | - Replace cutting edge, nut bolt in land lever by using tools & equipments. - Replace/Repair Laser/GPS device by using tools and equipments. - Adjust calibration of laser leveler. - Replace cutting edge in laser leveler by using tools and equipments. - Grease hitch & wheel frame in laser leveler. - Replace/sharpen the blades in land leveler and laser leveler |
| - Carry out Work Performance | - Check final inspection to ensure work is done according to SOPs. - Set farm implement's performance in accordance with the manufacturer’s manual. - Check the calibration as per standards. - Clean the work area and dispose of waste and non-recycle materials. - Check tools and equipment are stored according to SOP's. - Process the Workplace documents according to workplace procedures. |

Knowledge & Understanding

- Understanding of Personal Protective Equipment (PPE).

- Understand the Uses of various tools and equipment.

- Describe function of agriculture implements.

- Understanding the functions and maintenance of land leveler and laser leveler.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of Personal Protective Equipments.

- Replace cutting edge, nut bolt in land lever by using tools & equipment.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
| 1 | Safety Cap |
| 2 | Safety Gloves |
| 5 | Work Overall |
| 6 | Safety Shoes |
| 7 | Hand Wash/toilet soap |
| 9 | First Aid Box |
| 10 | Screw driver |
| 11 | Multi meter |
| 12 | Alignment tools |
| 13 | Cleaning tool |
| 14 | Laser level detector |
| 15 | Bearing puller |
| 16 | Opener spinner set |
| 17 | Ring spinner set |
| 18 | Grease gun |
| 19 | Oil cane |
| 20 | Hammer |
| 21 | Allen key set |
| 22 | Magnetic stick |
| 23 | Work bench |
| 24 | Torque wrench |
| 25 | Special Service tool |

### 0716MSA1811: Perform Repair/Maintenance of Electric Components

Overview:

This competency standard covers the skills and knowledge required to diagnose fault in electric implements, fault in monitoring system and devices, replace/repair faulty components in electric motor and devices, fault in power control unit and devices.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange tools and equipment to diagnose fault in electric Components | - Use PPEs as per SOP's. - Arrange tools and equipment to diagnose fault in electric Components as per SOPs. |
| - Diagnose fault in electric motor & devices | - Inspect motor and devices. - Detect fault in electric motor and devices as per SOPs. |
| - Replace/repair faulty components in electric motor and devices | - Replace/Repair faulty components, as per the SOPs. - Install the repaired/replaced components as per SOPs. |
| - Diagnose fault in battery pack & devices | - Inspect battery pack and devices. - Detect fault in battery pack and devices. |

| - Replace/repair faulty components in battery pack & devices | - Replace/Repair faulty components, as per the SOPs. - Install the repaired/replaced components as per SOPs. |
| --- | --- |
| - Diagnose fault in power control unit and devices | - Inspect Power Control Unit, Convertor/controller and devices. - Detect fault in Power Control Unit using diagnostic tools as per SOPs. |
| - Replace/repair faulty components in power control unit and devices | - Replace/Repair faulty components in Power Control Unit and devices, as per SOPs. - Install repaired/replaced components as per SOPs. |
| - Diagnose fault in charging system | - Inspect charging system - Detect faults in charging system as per SOPs |
| - Replace/repair faulty components in charging system | - Replace/Repair faulty components in charging system, as per SOPs - Install repaired/replaced components as per SOPs |
| - Diagnose fault in monitoring system and devices | - Inspect monitoring system and devices - Detect faults in monitoring system and devices as per SOPs |
| - Replace/repair faulty components in monitoring system and devices | - Replace/Repair faulty components in monitoring system and devices, as per SOPs - Install repaired/replaced components as per SOPs |

| - Carry out work Performance | - Check final inspection to ensure work is done according to SOPs. - Set implement's performance in accordance with the manufacturer’s manual. - Switch on the implement. - Clean work area and dispose of waste and non-recycle materials. - Check tools and equipment are stored according to workplace procedures. - Process the Workplace documents according to workplace procedures. |
| --- | --- |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Proper use and maintenance of diagnostic tools and equipment.

- Standard operating procedures (SOPs) for diagnosing and repairing faults.

- Techniques for cleaning and calibrating equipment.

- Methods for documenting diagnostic and repair processes.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE.

- Carryout measuring of current, Voltage and Resistance of Electrical Components.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
| 1 | Safety Cap |
| 2 | Safety Gloves |
| 3 | Work Overall |
| 4 | Safety Shoes |
| 5 | Hand Wash/toilet soap |
| 6 | First Aid Box |
| 7 | Vehicle lifter hydraulic jack |
| 8 | Combination spanner set |
| 9 | Screw driver set |
| 10 | Hose plier |
| 11 | Air blow gun |
| 12 | Wire bush |
| 13 | Air breeding job |
| 14 | Micro meter |
| 15 | Vernier caliper |
| 16 | Multi meter |
| 17 | Battery Tester |
| 18 | Electric Motor Tester |
| 19 | Charger Tester |
| 20 | Hammer & Mallets |

### 0716MSA1812: Adopt Green Practices at Workplace

Overview:

This competency standard is designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to adopt green practices at workplace. It encompasses a broad range of techniques that are essential to conserve resources. The learner will also be able to practice green skills and become more environmental friendly.

| Competency Units | Performance Criteria |
| --- | --- |
| - Segregate types of waste | - Segregate following wastes: - General waste (everyday waste produced in the workplace e.g. paper, cardboard, and non-hazardous materials). - Construction and demolition waste (produced during construction and demolition activities e.g. concrete, bricks, wood). - Hazardous waste (which poses a threat to human health and the environment, e.g. chemicals, batteries). - Industrial waste (produced through industrial processes e.g. manufacturing, mining, and oil and gas operations). - Handle types of wastes identified during operation as per Standard Operating Procedures according to company policy. - Dispose off wastes as per the Standard Operating Procedures according to company policy. |
| - Adopt waste reduction strategies | - Adopt techniques to reduce waste as per the Standard Operating Procedures. - Implement strategies of waste reduction: - Reuse the material from generated waste at the work place. - Reduce the use of environmental adversely affecting materials |
| - Conserve Resources | - Adopt techniques to conserve resource as per the Standard Operating Procedures - Implement resource of conservation techniques as per the Standard Operating Procedures. |

Knowledge & Understanding

- Identification of various types of waste.

- Importance of waste reduction strategies.

- Identification of Environment adversely affecting materials.

- Principles of six R’s in greening.

- Waste reduction techniques.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Enlist types of wastes.

- Handle at least one type of waste.

- Dispose-off one type of waste.

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
| 1 | Personal Protective Equipment (PPEs). |
| 2 | Waste handling SOP of the organization. |
| 3 | Waste bins/brooms/dustpans/garbage bags. |
| 4 | Shovels (assorted range). |
| 5 | Waste handling toolkits. |
| 6 | Trolley/lifters for handling waste. |
| 7 | Waste recycling equipment (if applicable to relevant industry). |

### 0716MSA1813:.Develop Entrepreneurship Skills

Overview:

This competency standard is designed to designed to equip Auto & Farm Machinery Technician with the skill and knowledge required to apply entrepreneurship skills, concept, market analysis and investment.

| Competency Units | Performance Criteria |
| --- | --- |
| - Interpret Entrepreneurship | - Understand Entrepreneur- Entrepreneurship - Enterprises: conceptual issue Entrepreneurship vs. management, Entrepreneurial motivation. - Understand the functions of entrepreneurs in relation to the enterprise and economy. - Identify source of business ideas |
| - Illustrate Project Preparation & Marketing Analysis | - Understand the qualities of a good entrepreneur, SWOT, and risk analysis. - Develop communication skills between buyer and seller. - Keep record of each business deal (email, etc.). |
| - Interpret Investment Procurement | - Develop project, feasibility, legal formalities. - Understand investment procedure - Loan procurement - Banking processes. |

Knowledge & Understanding

- Entrepreneur- Entrepreneurship - Enterprises: conceptual issue Entrepreneurship vs. management, Entrepreneurial motivation.

- Understand the functions of entrepreneurs in relation to the enterprise and economy.

- Understand source of business ideas

- Understand the qualities of a good entrepreneur, SWOT, and risk analysis.

- Record keeping of each business deal (email, etc.).

- Understanding project, feasibility, legal formalities.

- Understanding investment procedure - Loan procurement - Banking processes.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform SWOT analysis of the given project

- Prepare a project proposal for acquiring loan

Tools and equipment Required

The tools and equipment required for this competency standard are given below:

| SR. NO | NAME |
| --- | --- |
| 1 | Training CDs on entrepreneurship |
| 2 | Training CDs on communication skills |

| #### List of Tools and Equipment Required for the Qualification |  |  |
| --- | --- | --- |
| SR. NO | NAME | QTY. REQUIRED |
|  | Safety shoes | 25 |
|  | Helmet | 25 |
|  | Safety gloves | 25 pairs |
|  | Overall | 25 |
|  | Safety glasses | 25 |
|  | Fire extinguishers | 2 |
|  | First aid box kit | 2 |
|  | Steel Fire bucket | 3 |
|  | Waste bins | 2 |
|  | Air Bleeding Jar/ Air Blow Gun | 2 |
|  | Allen Key Set | 5 |
|  | Air Compressor (Double Stage) | 1 |
|  | Alignment Scissors Lift/Hydraulic lift | 1 |
|  | Automotive Scanner | 1 |
|  | Ball Joint (Rack End) Remover | 5 |
|  | Battery Charger/Tester | 2 |
|  | Bearing Puller | 5 |
|  | Brake Fluid Tester. | 2 |
|  | Brake Tester. | 2 |
|  | Combination Plier. | 5 |
|  | Combination Spanner Set. | 5 |
|  | Tool Kit | 5 |
|  | Compression Gauge. | 2 |
|  | Computers | 2 |
|  | Creeper | 2 |
|  | Dial Gauge/indicator with magnet | 5 |
|  | Diesel Fuel Injection Pump (Inline, Rotary) | 2 |
|  | Diesel Injector Tester | 2 |
|  | Diesel Pump Test Bench | 2 |
|  | Engine Hoist | 2 |
|  | Engine Stand. | 2 |
|  | Exhaust Gas Analyzer | 5 |
|  | Feeler Gauge/pitch gauge/thread gauge | 5 |
|  | Files sets | 6 |
|  | Flaring Tools | 2 |
|  | Fuel Pressure Gauge | 5 |
|  | Gear Puller | 5 |
|  | Grease Gun | 5 |
|  | Ginder (disc/Cutter | 2 |
|  | Hammer Different Size | 5 |
|  | Hand Hacksaw with blades | 5 sets |
|  | Hydraulic Jack with Different Capacities | 4 |
|  | Hydrometer | 5 |
|  | Injector Cleaner | 2 |
|  | Internet Connection | 1 |
|  | Jack Trolley. | 2 |
|  | Laser Leveler Detector | 1 |
|  | Magnet Stick | 2 |
|  | Measuring tape | 5 |
|  | Micro meter | 5 |
|  | Multimedia | 1 |
|  | Multi-meter | 5 |
|  | Oil Cane | 5 |
|  | Oil Pressure Tester | 2 |
|  | Open End Spanner set | 5 |
|  | Plier set | 5 |
|  | Piston Ring Compressor Tool. | 5 |
|  | Radiator Pressure Tester | 5 |
|  | Screw Driver Flat (Set) | 5 |
|  | Screw Driver Philip (Set). | 5 |
|  | Smart LED TV for Classroom | 1 |
|  | Socket Set. | 5 |
|  | Spark Plug Cleaner and Tester. | 2 |
|  | Special Service Tools (SST) | 5 |
|  | Steering Wheel Puller | 2 |
|  | Stethoscope | 2 |
|  | Scriber | 5 |
|  | Tachometer | 2 |
|  | Test Lamp. | 2 |
|  | Timing Light Gun | 2 |
|  | Tire Pressure Gauge | 2 |
|  | Tools Trolley (Complete Set of Hand Tools) | 2 |
|  | Torque Wrench | 2 |
|  | Universal Strap Wrench/ Oil Filter Spanner | 2 |
|  | Vacuum Gauge | 2 |
|  | Valve Spring Compressor Tool. | 2 |
|  | Vehicle equipped with the ABS, power steering and workshop manual | 1 |
|  | Vernier Caliper | 5 |
|  | Washing Tray | 5 |
|  | Wheel Alignment Machine/Balancer | 1 |
|  | Wheel Spanner. | 5 |
|  | Wrench (adjustable, pipe) | 5 |
|  | Personal Protective Equipment (PPEs). | 25 |
|  | Waste handling SOP of the organization. | 1 |
|  | Waste bins/brooms/dustpans/garbage bags. | 10 |
|  | Shovels (assorted range). | 5 sets |
|  | Waste handling toolkits. | 10 sets |
|  | Trolley/lifters for handling waste. | 5 |
|  | Waste recycling equipment (if applicable to relevant industry). | 1 |

#### List of Consumables Required for the Qualification

The consumables required for this competency standard are provided below:

| SR.NO | NAME | QTY. REQUIRED |
| --- | --- | --- |
| 1 | Air/Oil Filter | 5 |
| 2 | Alternator and AC Belts. | 2 sets |
| 3 | Auto Wire/Fuse | 2 sets |
| 4 | Axle Seal Set. | 2 sets |
| 5 | Ball Joints | 5 sets |
| 6 | Bearing Seals | 5 sets |
| 7 | Bolts Kit | 5 sets |
| 8 | Brake Drum | 5 sets |
| 9 | Brake Fluid/Brake Shoes | 5 sets |
| 10 | C,B. Point | 5 |
| 11 | Carburetor Cleaner/Kit | 5 |
| 12 | Clutch Set Assembly | 2 |
| 13 | Cotton Gloves | 25 pairs |
| 14 | Cotton Waste | 5 kg |
| 15 | Dust Bushes | 5 |
| 16 | Diesel fuel | 100 ltr |
| 17 | Disc Pad/Plate | 5 |
| 18 | Distilled Water | 5 bottles |
| 19 | ECT, ECU, MAF, O2, Temp and TP Sensor | 2 sets |
| 20 | Engine Oil | 20 ltr |
| 21 | Fan Belt | 5 |
| 22 | Fuel Injectors | 8 |
| 23 | Gear Oil/transmission | 5 Ltr |
| 24 | Grease | 2 kg |
| 25 | House Pipe. | 4 |
| 26 | Insulation Tap | 12 |
| 27 | Kerosene Oil | 20 Ltr |
| 28 | Lower Control Arm | 5 |
| 29 | Master Cylinder Kit | 5 |
| 30 | Overhauling kit | 5 |
| 31 | Petrol | 100 Ltr |
| 32 | Piston Ring Kit | 5 |
| 33 | Radiator Assembly | 5 |
| 34 | Radiator Coolant | 5 |
| 35 | Radiator Fan Motors | 5 |
| 36 | Sand Paper | 12 |
| 37 | Seals oil kit | 5 |
| 38 | Silicone/ tube | 5 |
| 39 | Steel Ruler | 5 |
| 40 | Spark Plug | 12 |
| 41 | Thermostat Valve | 2 |
| 42 | Thrust Washers Set | 5 |
| 43 | Tie Rod | 5 |
| 44 | Timing Belt and Bearing. | 5 |
| 45 | Universal Joint Cross | 2 |
| 46 | Valve/Valve Guide/ seal Set | 5 |
| 47 | W.D-40 | 5 |
| 48 | Washing Gun/tray | 5 |
| 49 | Water Pump Assembly | 5 |
| 50 | Water Temperature Gauge/Sensor | 5 |
| 51 | Wheel Cylinder Kit | 5 |
| 52 | Wheel Nut and Studs | 5 |
| 53 | Wire Grips (Connectors)/Z link | 5 |
| 54 | Tool board | 2 |
| 55 | Calculator | 5 |
| 56 | Drawing table | 4 |
| 57 | Drawing Board | 5 |
| 58 | Steel foot rule | 5 |
| 59 | Pencils. | 25 |
| 60 | Marking tools. | 5 |
| 61 | Geometry Box | 25 |
| 62 | Gasket sheet | 05 |

# Members of Qualification Development & Review Committee

The following members participated in the qualification development and review process:

| Qualification Development Committee |  |  |  |
| --- | --- | --- | --- |
|  |  | Mr. Rana Imran Sattar | Head of Institute, AMTS Sargodha Convener |
|  |  | Mr. Shahzad Amir Rafiq | Instructor, GCT Sahiwal, Member |
|  |  | Mr. Farman Ali | Instructor, GTTI Bhalwal, Sargodha, Member |
| Qualification Review & Development Committee |  |  |  |
| S. No. | Name |  | Organization |
|  |  | Muhammad Ishaq | Deputy Director NAVTTC , Islamabad |
|  |  | Muhammad Aasim | Deputy Director NAVTTC , Islamabad |
|  |  | Shams Shahbaz | Assistant Director, Operations II PTEVTA |
|  |  | Engr Waqas Hashmi | Assistant Director Curriculum PTEVTA |
|  |  | DACUM PANEL |  |
|  |  | Mr. Muhammad Arslan | Cluster Manager Punjab Small Industries, Lahore |
|  |  | Mr. Aamar Wasim | Instructor Auto & Farm TEVTA Punjab, Bhakkar |
|  |  | Mr. Imran Khan | Assistant Director (Academics) KP-TEVTA, Peshawar |
|  |  | Muhammad Nasir Khan | DACUM Facilitator Ex-DD(VT) SS&C Wing, NAVTTC, Islamabad |